| World Radiocommunication Conference (WRC-12) Geneva, 23 January - 17 February 2012 |  |
| --- | --- |
| PLENARY MEETING | Addendum 5 to Document 5(Add.28)-E |
|  | 2011 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| part 28 Addendum 5 |  |
| Agenda Item 7 Sub part A |  |

Introduction

Proposals within this sub-part are related to issues 4A and 4C of the CPM Report on WRC-12 agenda item 7. These two issues are related to suspension of use of satellite networks. In brief, they are intended to streamline and render more effective the application of No. 11.49 and to introduce similar provisions to No. 11.49 in Appendices 30 and 30A.

Clarification of the application of No. 11.49

A lack of clarity on the application of the 24-month suspension period in No. 11.49 has led to many assignments not being in operation for periods in excess of the intended 24-month period. Europe therefore proposes to clarify the moment at which an assignment should be declared suspended. Consequential to these clarifications is the introduction of an additional provision in Article 22 to deal with the flexibility in physically positioning the satellites.

Suspension mechanism for Regions 1 and 3 List assignments of Appendices 30 and 30A

There is currently no formal suspension period within Appendices 30 and 30A for frequency assignments included in the List whereas there are formal suspension periods defined for unplanned frequency bands (No. 11.49) and for the fixed-satellite service (FSS) allotment Plan contained in Appendix 30B (§ 8.17 of Appendix 30B). Europe therefore proposes to introduce in Appendices 30 and 30A a formal suspension mechanism for assignments included in the Regions 1 and 3 List and feeder-link List. This mechanism is substantially similar to the proposal made by Europe regarding the revision of No. 11.49.

Because these two issues are interrelated, Europe submits proposals related to them in a single set.

Issue 4E

CPM11-2 considered a contribution proposing to align the Appendix 30B provisions with the existing Appendices 30/30A provisions that address an extension in the date of bringing into use of frequency assignments in the case of a launch failure.

Europe proposes such an alignment

For the ease of the reader, the table below links the European proposals with the issues and methods as numbered in the CPM report.

| Topic | Related issue in the CPM text on WRC-12 agenda item 7 | Method from the CPM text that Europe supports, if any | List of European proposals |  |
| --- | --- | --- | --- | --- |
|  |  |  | Intent of the proposal | Proposal number (EUR/5A28/) |
| Clarification of the application of No. 11.49 of the RR | Issue 4C | Based on Method B | Clarifying in which cases and how to apply suspension for non-planned satellite networks | 65-68 |
| Suspension mechanism for Regions 1 and 3 List assignments of Appendices 30 and 30A | Issue 4A | Sole method of the CPM text with modifications to align it with the ECP on No. 11.49 | Introducing a mechanism similar to No. 11.49 for frequency assignments: |  |
|  |  |  | - in the Regions 1 and 3 List of Appendix 30 | 69-72 |
|  |  |  | - in the Regions 1 and 3 feeder-link List of Appendix 30A. | 73-76 |
| Alignment of Appendix 30B with Appendices 30 and 30A in case of launch failure | Issue 4E | Based on Method A | To align Appendix 30B with Appendices 30 and 30A in case of launch failure | 77-84 |

MOD

ARTICLE  11

Notification and recording of frequency 
assignments1, 2, 3, 4, 5, 6, 7     (WRC-07)

Section II  –  Examination of notices and recording of frequency assignments
in the Master Register

MOD	EUR/5A28/65

11.49	Where, for a period exceeding ninety days, the space station of a geostationary satellite network either ceases to operate at the notified orbital location22 or becomes incapable of operating on any of the notified frequency assignments, the notifying administration shall either cancel or suspend the corresponding frequency assignments. Where, for a period exceeding ninety days, a non-geostationary satellite system either ceases to operate within the notified orbital parameters or becomes incapable of operating on any of the notified frequency assignments, the notifying administration shall either cancel or suspend the corresponding frequency assignments. No later than thirty days after the end of this ninety-day period, the notifying administration shall inform the Bureau of either the cancellation of the corresponding frequency assignments or the date on which the corresponding frequency assignments were suspended. The date of suspension of suspended assignments is the starting date of the ninety-day period. Any suspended assignment which is not brought back into use23 within two years from the date of suspension shall be cancelled by the Bureau, who shall inform the notifying administration accordingly.

Reasons: To clarify the application of No. 11.49 by indicating when informing the Bureau in case of suspension, by removing the ambiguity between the 18 months and the 2 years and by explicitly specifying what actions shall be undertaken by the Bureau in case of lack of resumption of use.

adD	EUR/5A28/66

_________________

22 	See also No. 22.18A.

Reasons: To insert a link between No. 11.49 and the new provision No. 22.18A dealing with the positioning window of space stations.

adD	EUR/5A28/67

_________________

23 	See Resolution [EUR/A7/37] (WRC-12) for the definition of resumption of use after suspension of frequency assignments to geostationary-satellite networks in certain radiocommunication services and frequency bands.

Reasons: To insert a link between No. 11.49 and the proposed new Resolution [EUR/A7/37] into Article 11.

ARTICLE  22

Space services1

adD	EUR/5A28/68

Section IIIA  –  Positioning window of space stations

22.18A		§ 6bis	In order to reduce the risk of physical collisions, space stations on board geostationary satellites may use an operating orbital position within  0.4° of the nominal longitude indicated in item A.4.a.1 of Annex 2 to Appendix 4 as long as the satellite network to which the space station belongs does not cause more interference nor request more protection than if the space station would be positioned at the longitude of its nominal orbital position.  Section III of Article 22 shall continue to apply to this operating position.

Reasons: To clarify the limits within which a satellite can be positioned around the nominal orbital position of the satellite network. This flexibility is needed to manage the physical co-positioning of two or more satellites at the same nominal orbital position.

APPENDIX  30 (Rev.WRC-07)*

Provisions for all services and associated Plans and List1 for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
             and 12.2-12.7 GHz (in Region 2)     (WRC-03)

ARTICLE 5     (WRC-03)

Notification, examination and recording in the Master International
Frequency Register of frequency assignments to space stations
in the broadcasting-satellite service18     (WRC-07)

## 5.2	Examination and recording

ADD	EUR/5A28/69

5.2.10	For notified frequency assignments emanating from the Regions 1 and 3 List and recorded in the Master Register, where, for a period exceeding ninety days, the space station of a geostationary satellite network either ceases to operate at the notified nominal orbital location20bis or becomes incapable of operating on any of the notified frequency assignments, the notifying administration shall either cancel or suspend those frequency assignments on which the space station is no longer operating or capable of operating. No later than thirty days after the end of this ninety-day period, the notifying administration shall inform the Bureau of either the cancellation of the corresponding frequency assignments or the date on which the corresponding frequency assignments were suspended. The date of suspension of suspended assignments is the starting date of the ninety-day period.

ADD 	EUR/XXA28/70

5.2.11	Any assignment suspended under § 5.2.10 which is not brought back into use20ter within two years from the date of suspension shall be cancelled by the Bureau from the Master Register and the List, unless the assignment is one to which § 4.1.26 or § 4.1.27 is being applied. The Bureau shall inform the notifying administration accordingly.

ADD 	EUR/5A28/71

_________________

20bis 	See also No. 22.18A.

Reasons: To insert a link between § 5.2.10 and the new provision No. 22.18A dealing with the positioning window of space stations.

ADD 	EUR/5A28/72

_________________

20ter 	See Resolution [EUR/A7/37] (WRC-12) for the definition of resumption of use after suspension of frequency assignments to geostationary-satellite networks in certain radiocommunication services and frequency bands.

Reasons: To introduce a mechanism similar to No. 11.49 for frequency assignments in the Regions 1 and 3 List of Appendix 30 and to insert a link between § 5.2.11 of Article 5 of Appendix 30 and the proposed new Resolution [EUR/A7/37] into Appendix 30.

APPENDIX  30A  (Rev.WRC-07)*

Provisions and associated Plans and List1 for feeder links for the broadcasting-satellite service (11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3) in the frequency bands 14.5-14.8 GHz2 and 17.3-18.1 GHz in Regions 1 and 3,
and 17.3-17.8 GHz in Region 2     (WRC-03)

ARTICLE 5 (Rev.WRC-03)

Coordination, notification, examination and recording in the Master
International Frequency Register of frequency assignments to
feeder-link transmitting earth stations and receiving
space stations in the fixed-satellite service21, 22     (WRC-07)

## 5.2 	Examination and recording

## ADD 	EUR/5A28/73

5.2.10	For notified frequency assignments emanating from the Regions 1 and 3 feeder-link List and recorded in the Master Register, where, for a period exceeding ninety days, the space station of a geostationary satellite network either ceases to operate at the notified nominal orbital location24bis or becomes incapable of operating on any of the notified frequency assignments, the notifying administration shall either cancel or suspend those frequency assignments on which the space station is no longer operating or capable of operating. No later than thirty days after the end of this ninety-day period, the notifying administration shall inform the Bureau of either the cancellation of the corresponding frequency assignments or the date on which the corresponding frequency assignments were suspended. The date of suspension of suspended assignments is the starting date of the ninety-day period.

## ADD 	EUR/5A28/74

## 5.2.11	Any assignment suspended under § 5.2.10 which is not brought back into use24ter within two years from the date of suspension shall be cancelled by the Bureau from the Master Register and the List, unless the assignment is one to which § 4.1.26 or § 4.1.27 is being applied. The Bureau shall inform the notifying administration accordingly.

## ADD 	EUR/5A28/75

_________________

24bis 	See also No. 22.18A.

Reasons: To insert a link between § 5.2.10 and the new provision No. 22.18A dealing with the positioning window of space stations.

## ADD 	EUR/5A28/76

_________________

## 24ter 	See Resolution [EUR/A7/37] (WRC-12) for the definition of resumption of use after suspension of frequency assignments to geostationary-satellite networks in certain radiocommunication services and frequency bands.

Reasons: To introduce a mechanism similar to No. 11.49 for frequency assignments in the Regions 1 and 3 List of Appendix 30A and to insert a link between § 5.2.11 of Article 5 of Appendix 30A and the proposed new Resolution [EUR/A7/37] into Appendix 30A.

APPENDIX  30B  (Rev.WRC-12)

Provisions and associated Plan for the fixed-satellite service
in the frequency bands 4 500-4 800 MHz, 6 725-7 025 MHz,
10.70-10.95 GHz, 11.20-11.45 GHz and 12.75-13.25 GHz

ARTICLE 6     (WRC-12)

Procedures for the conversion of an allotment into an assignment, for
the introduction of an additional system or for the modification of
an assignment in the List 1, 2     (WRC-12)

MOD	EUR/5A28/77

6.28	Should the assignments that were the basis of the unfavourable finding not be brought into use within the period specified in § 6.1 or within the extension period under § 6.31bis, then the status of the assignment in the List shall be reviewed accordingly.

ADD	EUR/5A28/78

6.31bis	The regulatory time-limit in § 6.31 for bringing into use of an assignment to a space station of a satellite network may be extended once by not more than three years due to launch failure in the following cases:

–	the destruction of the satellite intended to bring the assignment into use;

–	the destruction of the satellite launched to replace an already operating satellite which is intended to be relocated to bring another assignment into use; or

–	the satellite is launched, but fails to reach its assigned orbital location.

For this extension to be granted, the launch failure must have occurred at least five years after the date of receipt of the complete Appendix 4 data. In no case shall the period of the extension of the regulatory time-limit exceed the difference in time between the three-year period and the period remaining from the date of the launch failure to the end of the regulatory time-limit. In order to take advantage of this extension, the administration shall have, within one month of the launch failure or one month after 17 February 2012, whichever comes later, notified the Bureau in writing of such failure, and shall also provide the following information to the Bureau before the end of the regulatory time-limit of § 6.31:

–	date of launch failure;

–	due diligence information (except the date of launch) as required in Resolution [EUR/B7/39] (WRC-12), if this resolution applies to the satellite network in which the space station is to operate, for the assignments with respect to the satellite that suffered the launch failure.

MOD	EUR/5A28/79

6.32	Thirty days prior to the date of bringing into use under § 6.31 or § 6.31bis, the Bureau shall dispatch a reminder telegram or fax to the notifying administration which has not brought its assignment into use, bringing the matter to its attention.

MOD	EUR/5A28/80

6.33

When:

i)	an assignment is no longer required; or

ii)	an assignment recorded in the List and brought into use has been suspended for a period exceeding two years and ending after the expiry date specified in § 6.31; or

iii)	an assignment recorded in the List has not been brought into use within the eight-year period following the receipt by the Bureau of the relevant complete information under § 6.1 (or within the extended period in the event of an extension under § 6.31bis), with the exception of assignments submitted by new Member States where § 6.35 and 7.7 apply,

the Bureau shall:

a)	publish in a Special Section of its BR IFIC the cancellation of the related Special Sections and the assignments recorded in the Appendix 30B List;

b)	if the cancelled assignment is the result of a conversion of an allotment without modification, reinstate the allotment in the Appendix 30B Plan;

c)	if the cancelled assignment is the result of the conversion of an allotment with modifications, reinstate the allotment with the same orbital location and technical parameters of the cancelled assignment except for its service area, which shall be the national territory of the administration whose allotment is being reinstated; and

d)	update the reference situation for the allotments of the Plan and the assignments of the List.

MOD	EUR/5A28/81

6.34	When a proposed new or modified frequency assignment has not fulfilled all the requirements for entering the List, in accordance with § 6.23 or 6.25, by the expiry date specified in § 6.31 or § 6.31bis in the event of an extension under that provision, the Bureau shall publish in a Special Section of the BR IFIC the cancellation of the related Special Sections.

MOD	EUR/5A28/82

6.36	Should the assignments mentioned in § 6.35 over the national territory of the administration not be brought into use within the eight years following the receipt by the Bureau of the relevant complete information under § 6.1 or within the extension period under § 6.31bis, they would be retained in the List until the end of the World Radiocommunication Conference immediately following the successful completion of the procedure referred to in § 6.35.

ARTICLE 8     (WRC-12)

Procedure for notification and recording in the Master Register
of assignments in the planned bands for the
fixed-satellite service11, 12     (WRC-12)

MOD	EUR/5A28/83

8.13 	A notice of a change in the characteristics of an assignment already recorded, as specified in Appendix 4, shall be examined by the Bureau under § 8.8 and 8.9 as appropriate. Any changes to the characteristics of an assignment, that has been notified and confirmed as having been brought into use, shall be brought into use within eight years from the date of the notification of the modification. Any changes to the characteristics of an assignment that has been notified but not yet brought into use shall be brought into use within the period provided for in §§ 6.1, 6.31 or 6.31bis of Article 6.     (WRC-12)

MOD	EUR/5A28/84

8.16	All frequency assignments notified in advance of their being brought into use shall be entered provisionally in the Master Register. Any frequency assignment provisionally recorded under this provision shall be brought into use no later than the end of the period provided for in § 6.1 or § 6.31bis in the event of an extension under that provision. Unless the Bureau has been informed by the notifying administration of the bringing into use of the assignment, it shall, no later than 15 days before the end of the regulatory period established under § 6.1 or § 6.31bis, send a reminder requesting confirmation that the assignment has been brought into use within the regulatory period. If the Bureau does not receive that confirmation within 30 days following the period provided under § 6.1 or § 6.31bis in the event of an extension under that provision, it shall cancel the entry in the Master Register. In the event that an extension was requested under § 6.31bis but the Bureau determines that the conditions for an extension under § 6.31bis are not met, the Bureau shall inform the administration of its findings and cancel the entry in the Master Register.     (WRC-12)

Reasons:  To align Appendix 30B with Appendices 30 and 30A in case of launch failure

____________________