CEPT Brief on agenda item 1.13

1.13	to consider the results of ITU-R studies in accordance with Resolution 551 (WRC-07) and decide on the spectrum usage of the 21.4-22 GHz band for the broadcasting-satellite service and the associated feeder-link bands in Regions 1 and 3;

## Issue

Resolution 551 (WRC-07) resolves to invite ITU-R:

1	that ITU-R continue technical and regulatory studies on harmonization of spectrum usage, including planning methodologies, coordination procedures or other procedures, and BSS technologies, in preparation for WRC-11, in the 21.4-22 GHz band and the associated feeder-link bands in Regions 1 and 3, taking into account considering h) and i);

2	that WRC-11 review the results of the study and decide the usage of the 21.4-22 GHz band and the associated feeder-link bands in Regions 1 and 3,

CEPT position

CEPT is opposed to any a priori planning of the band 21.4-22 GHz, proposes to use the procedures of Articles 9 and 11 of the Radio Regulations to address sharing between BSS satellite networks in the 21.4-22 GHz bandproposes to reduce the size of the coordination arc from ±16° to ±6° for applying RR No. 9.7 and supports to improve regulatory mechanisms and transparency and truthfulness of the due diligence procedure, i.e. CEPT supports Methods A, B and C of the CPM report. CEPT proposes some regulatory amendments accordingly.

In order to maintain flexibility in the design of satellite systems, CEPT considers that there should be no limitation on the FSS Earth-to-space bands which may be used for the feeder-links of the BSS in the 21.4-22 GHz. To ensure a balance between uplink and downlink spectrum within the 30/20 GHz range in Regions 1 and 3, CEPT proposes  to allocate to the fixed-satellite service (Earth-to-space) the band 24.65-25.25 GHz in Region 1 and 24.65-24.75 GHz in Region 3. with associated mechanism to avoid heavy constraints for future FS development (i.e. minimum earth station antenna diameter of 3.5m and coordination under 9.17).

CEPT is of the view that terrestrial services in Regions 1, 2 and 3 and the broadcasting-satellite service in Regions 1 and 3 should enjoy the same primary status and proposes to include pfd limits to ensure mutual-coexistence of these services:

- Table 21-4 of Article 21 of the Radio Regulations to protect terrestrial services from the Regions 1 and 3 BSS.

- pfd limit of – 120.4 dBW/(m².MHz) to be met at the border of Regions 1 and 3 countries to protect Regions 1 and 3 BSS Earth station from terrestrial services

If WRC-12 adopts such pfd limits in Article 21 to protect terrestrial services in Regions 1, 2 and 3, CEPT is of the view that such pfd limits should not be retroactively applicable to BSS satellite systems notified before the end of the Conference.

CEPT is of the view that fixed services and the broadcasting-satellite service in Regions 1 and 3 should co-exist under specific conditions on the same territory. Therefore CEPT proposes that administrations in Regions 1 and 3 are encouraged to not deploy stations in the mobile service and are encouraged to limit the deployment of stations in the fixed service to point-to-point links.

Background

CPM 11-1 identified WP 4A of SG4 as the Responsible Group for this Agenda item and WP5C, WP6X, (WP 3M), (WP 4B), (WP 6D), (WP 5A), (WP 6G), (WP 7D) were identified as concerned groups. The ITU-R Working Parties indicated above, was identified based on the ITU-R Study Group structure contained in Document CPM11-1/1, and is subject to review and approval by the respective Study Group. All appropriate regulatory/procedural studies on relevant agenda items will be carried out by the Special Committee on Regulatory/Procedural matters (SC) on the basis of proposals from membership of the ITU and the relevant ITU-R Study Groups.

Method

A priori planning is not necessary because it freezes access according to technological assumptions at the time of planning and then prevents flexible use taking account of real world demand and developments.

In preparations under Agenda Item 1.13, concerns have been raised by some administrations that the number of submissions for BSS networks received by the Bureau already at this point in time could make coordination of future BSS networks more difficult. For this purpose, 7 alternative Methods were proposed (i.e. Methods B to H).

- Method B (proposed by Luxembourg): Apply an improve due diligence process

- Method C (proposed by CEPT): Coordination arc of 6°

- Methods D (proposed by Russia) & E (proposed by Iran): Submissions by administrations having no assignments in the 21.4-22 GHz band may jump over earlier filings with possibility to collocate with orbital positions in Appendixes 30, 30A and/or 30B  provided that these meet specific requirements (e.g. limited to or at least national service and coverage area).

- Method F (proposed by India): Provision to reduce the number of pending filings

- Method G (proposed by Russia): Homogenous parameters - network limited to national coverage

- Method H (proposed by United Arab Emirates): Method B + Homogenous parameters + compatible arc of 4°

According to Table 5-1 of Appendix 5 (see N° 9.7, 8)), coordination between BSS geostationary space stations is necessary if their frequency bands overlap and if the orbital separation is less than or equal to 16°. The value of 16° can be seen as overly conservative considering that a value of 9° is used for BSS between 10 and 15 GHz and 8˚ for FSS networks adjacent to the 21.4-22 GHz band (below and above). According to detailed analysis conducted within CEPT and ITU-R, CEPT is of the view that a coordination arc of 6° for geostationary BSS networks in the 21.4-22 GHz band should be used. Further study is needed to address the requirement for transitional measure at the stage of the notification for networks whose coordination request are submitted before the WRC-12.

CEPT supports Methods B and C of the CPM report.

Associated feeder-link bands

A review of the availability of a new continuous block of 600 MHz of FSS spectrum for providing BSS feeder-links associated to the 22 GHz BSS band have shown that:

- within the existing FSS uplink allocations available in Region 1, it may happen that there is not enough capacity to simultaneously feed the BSS band 21.4-22 GHz and other FSS downlinks at a given orbital position.

- the band 24.65-25.25 GHz appears to be the best candidate band, in particular because the band 24.75-25.25 GHz is already allocated to FSS uplink and used in Region 2 and Region 3 (Documents CPGPTA(2010)015 and CPGPTA(2010)065).  Related FSS/FS compatibility study shows that, under worst-case assumptions, the separation distances from which the required protection criterion (I/N = -10dB) is achieved, range from 27.7 km to 105.1 km in the worst case conditions (i.e. no terrain profile, FS station pointed towards FSS transmit station, azimuths of both FS and FSS stations are aligned). The order of magnitude of these separation distances, and their variation according to the pointing azimuths of the FS and FSS stations (e.g. separation distance of 105.1 km down to 20.5 km when the azimuthal discrimination at the FS station with respect to the FSS earth station is greater than 48° and the azimuthal discrimination at the FSS earth station with respect to the FS station is greater than 48°), suggest that sharing between FS and FSS is possible in the band 24.65-25.25 GHz. In order to ensure that this sharing situation would not represent a heavy constraint for future FS development, CEPT proposes to limit the use of this band to earth station in the fixed-satellite service (Earth-to-space) with a minimum antenna diameter of 3.5m to ensure that the number of FSS earth stations remains small. Furthermore, it is recognized that the frequency band 24.65-25.25 GHz is heavily used for FS capacity today by some administrations which are designed to be in accordance with RR Article 21. Therefore it is essential to keep Table 21-1 and associated footnote 21.2.4 unchanged to avoid constraints towards both current and future FS.

As in other bands, all FSS transmit earth Stations including those used for providing BSS feeder-links associated to the 22 GHz BSS band, should be coordinated with other administrations in accordance with RR 9.17.

Several European Administrations indicated that the Uplink band 24.65-25.25 GHz would not be available on their territories due to heavily FS usage.

Regions 1 & 3 terrestrial services in the band 21.4-22.0 GHz

CEPT is of the view that fixed service and the broadcasting-satellite service in Regions 1 and 3 should co-exist under specific conditions on the same territory. Therefore CEPT proposes that administrations in Regions 1 and 3 should not deploy stations in the mobile service and should limit the deployment of stations in the fixed service to point-to-point links.

CEPT is of the view that protection of terrestrial services from the BSS would then be ensured by pfd limits on the basis of RR Table 21-4 and protection of the BSS from terrestrial services, could be ensured through a pfd limit of – 120.4 dBW/(m².MHz) to be met at the border of Regions 1 and 3 countries (this value is based on the new Recommendation ITU-R BO.[PFD_TERRESTRIAL] agreed to be sent to administration for adoption and approval by the last Study Group 4 (29-30 September 2011)).

Region 2 terrestrial services in the band 21.4-22.0 GHz

CEPT is of the view that terrestrial services in Region 2 and the broadcasting-satellite service in Regions 1 & 3 should have the same status. Therefore, limits should be defined to ensure mutual-coexistence of the services with relevant modifications to the Radio Regulations.

To protect the Region 2 terrestrial services from the Regions 1 & 3 broadcasting-satellite service, a hard pfd limits regime in Article 21, Section V could be implemented. As many terrestrial systems were deployed before WRC-07, CEPT proposes to preliminary consider as pfd limits, the pfd mask proposed in Resolution 525 (Rev. WRC-03). Final pfd mask should not create undue constraints to any BSS networks even in the worst case configuration (i.e. protection of Greenland terrestrial services from a BSS network over Iceland).

To protect the Regions 1 & 3 broadcasting-satellite service from the Region 2 terrestrial services, CEPT is of the view that such protection should be ensure through a pfd limit of – 120.4 dBW/(m².MHz) to be met at the border of Regions 1 and 3 countries (this value is based on the new Recommendation ITU-R BO.[PFD_TERRESTRIAL] agreed to be sent to administration for adoption and approval by the last Study Group 4 (29-30 September 2011)).

List of relevant documents

ITU web page dedicated to the CPM (http://www.itu.int/ITU-R/go/rcpm-wrc-12-studies)

## Actions to be taken

In relation with the new footnote 5.B113, Administrations are encouraged to assess the possible impact of the Aeronautical Mobile application (aircraft to aircraft) on the BSS receivers, leading to possible refinement of the footnote.

## Relevant information from outside CEPT

European Union

Regional telecommunication organisations

APT (August 2011)

APT has the following views on the Agenda item 1.13;

Issue A: Intra-service issues

There is no common view on issue A.

Issue B: Feeder-link capacity

The APT members agreed that in China, Korea (Rep. of), India, Indonesia, Japan, Malaysia and Singapore, the band 24.65 – 24.75 GHz would be also allocated to the fixed-satellite service (Earth-to-space) on a primary basis. This use shall be limited to feeder link earth stations using a minimum antenna diameter of 4.5 m, for the broadcasting-satellite service.

Issue C: Inter-service issues

The APT members agreed that an appropriate regulatory mechanism would be necessary in order to ensure the sharing between BSS in Regions 1 and 3 and terrestrial services in Regions 1, 2 and 3.

Protection of BSS in Regions 1 and 3

The APT members agreed that the pfd limit at the border of the territory of any other administration in Regions 1 and 3, [−114.3] dB(W/(m2 ·1 MHz)), shall be applied to a station in terrestrial services in Regions 1, 2 and 3. And No. 9.19 does not apply.

Protection of terrestrial services in Regions 1, 2 and 3

The APT members agreed that the following pfd limits to be included in RR Article 21 shall be applied to a station in BSS in Regions 1 and 3. And No. 9.11 does not apply.

−115 dB(W/(m2 · MHz)) 			for 0° ≤ θ ≤ 5°

−115 + 0.5(θ − 5) dB(W/(m2 · MHz)) 	for 5° ≤ θ ≤ 25°

−105 dB(W/(m2 · MHz)) 			for θ > 25°

where, θ is angle of arrival above the horizontal plane.

<Transitional arrangement>

Taking into account the provisions in Resolution 525, the pfd limit above shall be applied to any terrestrial station in Regions 1 and 3, including those recorded in the MIFR before the end of WRC-12.

ATU (July 2011)

Issue A:

No common position due to lack of consensus

Issue B:

Method B1 (No additional uplink allocation is needed)

Issue C:

No common position due to lack of consensus

ASMG (October 2011)

- propose that the regulatory mechanism currently in force, i.e. procedures contained in RR Articles 9 and 11 be applied for the use of the above-mentioned frequency band (concept/principle of first-come first-served)

- revising Resolution 49 (Rev.WRC-07),

- proposes measures to improve the due diligence requirements currently contained in Res. 49 (Rev.WRC-07) to obtain better coherence/consistency between networks recorded in the Master International Frequency Register (MIFR) and real satellites in operation

- to limit the number of submissions by one administration

- reduce the coordination arc from ±16° to ±[6]°

CITEL (May 2011)

Inter-American Proposal (Argentine, Brazil, Canada, Colombia, Costa Rica, Chile, Ecuador, El Salvador, Guatemala, Mexico, USA and Uruguay and Venezuela)

- Article 21 limits on Regions 1 and 3 BSS emissions on territories of Region 2only

Sharing between satellite services in Regions 1 and 3 and terrestrial services in Region 2 can be most simply implemented through a pfd limits regime in Article 21, Section V with the following pfd limits:

−115 dB(W/(m2 · MHz)) 			for 0° ≤ θ ≤ 5°

−115 + 0.5(θ − 5) dB(W/(m2 · MHz)) 	for 5° ≤ θ ≤ 25°

−105 dB(W/(m2 · MHz)) 			for θ > 25°

- where, θ is angle of arrival above the horizontal plane.SUP of Resolution 525 (Re. WRC-07)

RCC (September 2011)

## To adopt pfd limits for BSS in the 21.422.0 GHz band which are applied on territories of Regions 1, 2 and 3 countries. Proposed value of pfd limit in accordance with mask (105/115 dBW/m2) will be included in Table 21-4 RR.

- To suppress Resolution 525 as providing the interim procedure for introduction of BSS (HDTV) and super-priority of BSS, which is not supported.

- To adopt definitive permanent procedure in accordance with RR Articles 9 and 11, but this procedure must be supplemented (for countries not having assignments for BSS systems in the 21.422.0 GHz band) with special procedure of guaranteed access to orbital-frequency resource.

- To include either into Article 21 text or in footnote 21.2.1 recommendation to use GSO avoidance for FS station antennas in the 21.422.0 GHz band.

- To support allocation to feeder links in the band 24.6525.25 GHz for BSS in the 21.422.0 GHz band.

- To adopt footnote 5.XXY [5.XXZ] in RR Article 5 defining pfd limit 
[104.3 dBW/m2] for stations of terrestrial services that does not exceed at the territory of administrations using BSS for more than 20% of time of the month.

International organisations

NATO

NATO supports the use of the band 21.4-22 GHz for broadcasting-satellite service and associated feeder-link bands in Regions 1 and 3 as long as the band 20.2 – 21.2 GHz is sufficiently protected against adverse effects from too high power flux-density levels in the band 21.4 – 22.0 GHz.

Additionally terrestrial services in Region 2 must be protected from these satellite systems operating in Regions 1 and 3.

Military Importance:  Low

SFCG

No specific views in this Agenda Item

Regional organisations

EBU

EBU supports flexible use of the band 21.4-22 GHz, thus promoting the availability of services by satellite, such as HDTV and ultra HDTV or 3D TV in the future. EBU is not in favour of a priori planning of satellite services because that could restrict access to spectrum and prevent technological developments.

Eumetnet

No specific views in this Agenda Item

Eurocontrol

No specific views in this Agenda Item

CRAF

No specific views in this Agenda Item

ICAO

No specific views in this Agenda Item

IATA

No specific views in this Agenda Item

ASFCG

No specific views in this Agenda Item