|  |  | CPG15(13)003 Annex 8 |
| --- | --- | --- |
| CPG-15 2nd meeting, Luxembourg |  |  |
| Date issued: 7 September 2012 Source: CPG PTA(2012)024 Annex 12 Subject: Draft Brief on WRC-15 Agenda item 4 |  |  |

Summary

Proposal CPG is invited to consider and approve the attached draft Brief on WRC-15 Agenda item 4.

# Draft CEPT Brief on agenda item 4

4	in accordance with Resolution 95 (Rev.WRC-07), to review the Resolutions and Recommendations of previous conferences with a view to their possible revision, replacement or abrogation;

Resolution 95 (Rev.WRC-07), General review of the Resolutions and Recommendations of world administrative radio conferences and world radiocommunication conferences.

## Issue

Resolution 95 (Rev.WRC-07) resolves to invite future competent world radiocommunication conferences

1	to review the Resolutions and Recommendations of previous conferences that are related to the agenda of the Conference with a view to their possible revision, replacement or abrogation and to take appropriate action;

2	to review the Resolutions and Recommendations of previous conferences that are not related to any agenda item of the Conference with a view to:

–	abrogating those Resolutions and Recommendations that have served their purpose or have become no longer necessary;

–	reviewing the need for those Resolutions and Recommendations, or parts thereof, requesting ITU-R studies on which no progress has been made during the last two periods between conferences;

–	updating and modifying Resolutions and Recommendations, or parts thereof that have become out of date, and to correct obvious omissions, inconsistencies, ambiguities or editorial errors and effect any necessary alignment;

## Preliminary CEPT position

CEPT encourages the constant review of Resolutions and Recommendations from previous conferences and will follow activities, in particular of ITU, associated with this effort.

CEPT proposes to suppress Resolution [TBD]

CEPT proposes to modify Resolution [TBD]

CEPT proposes to suppress Recommendation [TBD]

CEPT proposes to modify Recommendation [TBD]

## Background

The review of Resolutions and Recommendations of previous conferences (Volume 3 of the Radio Regulations Geneva 2012) is a standing agenda item. Therefore conferences shall conclude on whether there is a need for any modification or suppression of their Resolutions or Recommendations. The table in Annex 1 is intended to summarize the possible course of actions to be taken in response of the concerned Resolution or Recommendation.

Furthermore, a reference is made to the relevant WRC agenda item for those Resolutions and Recommendations which are covered by the agenda of WRC-15. This reference is made in the last column of the table combined with an identification of the responsible and concerned CPG Project Team or ECC Working Group. The possible course of action for those Resolutions and Recommendations with direct relevance to any specific agenda item of WRC-15 is only for information purposes. The European proposal on those specific Resolutions and Recommendations can be found in the relevant ECP.

As useful information based on Final Acts WRC-12 it is considered to note that WRC-12 approved for deletion following Resolutions from list submitted by ECP AI 4: 97, 345, 355, 533, 546, 805, 905 as well as Recommendation 104. Resolutions 55, 149 and 906 were modified instead of deletion proposed by ECP. From list of four Resolutions proposed by ECP for modification WRC-12 carried out modification of Resolutions 223, 646 and 716.

Clarification is needed and/or necessary steps should be taken in regard of abrogated Resolution 905 which is referred to in:

- footnote 18 to Article 5 of Appendix 30 (Rev.WRC-12),

- footnote 22 to Article 5 of Appendix 30A (Rev.WRC-12),

- footnotes 1 to Article 6 of Appendix 30B (Rev.WRC-12),

- footnote 11 to Article 8 of Appendix 30B (Rev.WRC-12).

The ITU BR will submit information with remarks and possible course of action to CPM15-2 to be included in the relevant section of the CPM Report to WRC-15

## List of relevant documents

ITU-Reports:

ITU-Recommendations:

Further ITU documents:

Final Acts of WRC-12, Vol. III

ECC, ERC or CEPT-Reports:

CEPT-Recommendations:

## Actions to be taken

Administrations and Project Teams are encouraged to study the table in Annex 1 in detail and to provide contributions with comments to CPG PTA including proposals on a possible course of action on the Resolutions and Recommendations relevant to their work.

## Relevant information from outside CEPT (examples of these are below)

### European Union (date of proposal)

### Regional telecommunication organisations:

APT (date of proposal)

ATU (date of proposal)

Arab Group (date of proposal)

CITEL (date of proposal)

RCC (date of proposal)

### International organisations

IATA

ICAO

IMO

NATO

SFCG

WMO and EUMETNET

### Regional organisations

ESA

EUMETNET

Eurocontrol

## ANNEX 1

Review of Resolutions and Recommendations in response to Resolution 95 (Rev.WRC-07)

| Resolution |  | CEPT Proposal WRC-12 | Comments | Proposed Course of action at WRC-15 | WRC-15 Agenda Item/ PTs |
| --- | --- | --- | --- | --- | --- |
| RES 1 | (Rev.WRC-97) Notification of frequency assignments | NOC | Res. is referred to in No. 26/5.2 of App. 26 |  |  |
| RES 2 | (Rev.WRC-03) Equitable use, by all countries, with equal rights, of the geostationary-satellite and other satellite orbits and of frequency bands for space radiocommunication services | NOC |  |  |  |
| RES 4 | (Rev.WRC-03) Period of validity of frequency assignments to space stations using the geostationary-satellite and other satellite orbits | NOC |  |  |  |
| RES 5 | (Rev.WRC-03) Technical cooperation with the developing countries in the study of propagation in tropical and similar areas | NOC/MOD | Aware a) makes a reference to Geneva-89 Agreement. An update to Geneva-06 may be necessary? |  |  |
| RES 7 | (Rev.WRC-03) Development of national radio frequency management | NOC | The section Draws the attention of the next plenipotentiary conference indicates that this RES has to be modified as PP-06 and PP-10 have taken place?? |  |  |
| RES 10 | (Rev.WRC-2000) Use of two-way wireless telecommunications by the International Red Cross and Red Crescent Movement | NOC |  |  |  |
| RES 11 | (WRC-12) Use of satellite orbital positions and associated frequency spectrum to deliver international public telecommunication services in developing countries |  |  |  | 9.1.3 PTB |
| RES 12 | (WRC-12) Assistance and support to Palestine |  |  |  |  |
| RES 13 | (Rev.WRC-97) Formation of call signs and allocation of new international series | NOC | This Resolution is referred to in No. 19.32. |  |  |
| RES 15 | (Rev.WRC-03) International cooperation and technical assistance in the field of space radiocommunications | NOC | Still relevant, Implemented through liaison with ITU-D |  |  |
| RES 18 | (Rev.WRC-12) Relating to the procedure for identifying and announcing the position of ships and aircraft of States not parties to an armed conflict | NOC | Noting should include LRIT system | MOD |  |
| RES 20 | (Rev.WRC-03) Technical cooperation with developing countries in the field of aeronautical telecommunications | NOC |  |  |  |
| RES 25 | (Rev.WRC-03) Operation of global satellite systems for personal communications | NOC |  |  |  |
| RES 26 | (Rev.WRC-07) Footnotes to the Table of Frequency Allocations in Article 5 of the Radio Regulations | NOC |  |  | 8 PTA |
| RES 27 | (Rev.WRC-12) Use of incorporation by reference in the Radio Regulations | MOD | Noting b) of Resolution 67 (WRC-12) “that ITU-R Recommendations incorporated by reference as they would appear in Volume IV of the Radio Regulations may be limited to a list of titles and their cross-references in the Radio Regulations”, shall be taken into account under AI 9.1.4 |  | 2 PTA |
|  | ANNEX 1 Principles of incorporation by reference |  |  |  |  |
|  | ANNEX 2 Application of incorporation by reference |  |  |  |  |
|  | ANNEX 3 Procedures applicable by WRC for approving the incorporation by reference of ITU-R Recommendations or parts there of |  |  |  |  |
| RES 28 | (Rev.WRC-07) Revision of references to the text of ITU-R Recommendations incorporated by reference in the Radio Regulations | NOC |  |  | 2 PTA |
| RES 33 | (Rev.WRC-03) Bringing into use of space stations in the broadcasting-satellite service, prior to the entry into force of agreements and associated plans for the broadcasting-satellite service | [MOD] |  |  | PTB |
| RES 34 | (Rev.WRC-03) Establishment of the broadcasting-satellite service in Region 3 in the 12.5-12.75 GHz frequency band and sharing with space and terrestrial services in Regions 1, 2 and 3 | SUP |  |  |  |
| RES 42 | (Rev.WRC-12) Use of interim systems in Region 2 in the broadcasting-satellite and fixed-satellite (feeder-link) services in Region 2 for the bands covered by Appendices 30 and 30A | NOC |  |  |  |
|  | ANNEX |  |  |  |  |
| RES 49 | (Rev.WRC-12) Administrative due diligence applicable to some satellite radiocommunication services | MOD | This Resolution is referred to in Nos. A.9.4, A.11.2, 11.44.1 and 11.48 This Resolution is also referred to in No. 6.31bis, Article 6 of App. 30B. A former version of this Resolution is referred to in footnote 2 to Article 6 and in footnote 12 to Article 8 of App. 30B. |  | PTB |
|  | ANNEX 1 |  |  |  |  |
|  | ANNEX 2 |  |  |  |  |
| RES 51 | Transistional arrangements concerning coordination and notification | SUP |  |  |  |
| RES 55 | (Rev. WRC-12) Electronic submission of notice forms for satellite networks, earth stations and radio astronomy stations | SUP | Objective reached |  | PTB |
| RES 58 | (WRC-2000) Transitional measures for coordination between certain specific geostationary fixed-satellite service receive earth stations and non-geostationary fixed-satellite service transmit space stations in the frequency bands 10.7-12.75 GHz, 17.8-18.6 GHz, and 19.7-20.2 GHz where epfd limits apply | MOD/SUP | Necessity of transitional measure unclear. |  |  |
|  | ANNEX 1 |  |  |  |  |
| RES 63 | (Rev.WRC-12) Protection of radiocommunication services against interference caused by radiation from industrial, scientific and medical (ISM) equipment | MOD |  |  |  |
| RES 67 | (WRC-12) Updating and rearrangement of the Radio Regulations |  |  |  | 9.1.4 PTA |
| RES 72 | (Rev.WRC-07) World and Regional preparations for world radiocommunication conferences | NOC |  |  |  |
| RES 73 | (Rev.WRC-2000) Measures to solve the incompatibility between the broadcasting-satellite service in Region 1 and the fixed-satellite service in Region 3 in the frequency band 12.2-12.5 GHz | NOC |  |  |  |
| RES 74 | (Rev.WRC-03) Process to keep the technical bases of Appendix 7 current | NOC |  |  |  |
| RES 75 | (Rev.WRC-12) Development of the technical basis for determining the coordination area for coordination of a receiving earth station in the space research service (deep space) with transmitting stations of high-density applications in the fixed service in the 31.8-32.3 GHz and 37-38 GHz bands | NOC |  |  | PTB |
| RES 76 | (WRC-2000) Protection of geostationary fixed-satellite service and geostationary broadcasting-satellite service networks from the maximum aggregate equivalent power flux-density produced by multiple non-geostationary fixed-satellite service systems in frequency bands where equivalent power flux-density limits have been adopted | NOC/MOD | This Resolution is referred to in No. 22.5K |  |  |
|  | ANNEX 1 |  |  |  |  |
| RES 80 | (Rev.WRC-07) Due diligence in applying the principles embodied in the Constitution | NOC |  |  | 9.3 PTB |
|  | ANNEX 1 |  |  |  |  |
|  | ANNEX 2 |  |  |  |  |
| RES 81 | (WRC-2000) Evaluation of the administrative due diligence procedure for satellite networks | SUP |  |  | PT B |
| RES 85 | (WRC-03) Application of Article 22 of the Radio Regulations to the protection of geostationary fixed-satellite service and broadcasting-satellite service networks from non-geostationary fixed-satellite service systems | NOC/MOD |  |  |  |
| RES 86 | (WRC-07) Implementation of Resolution 86 (Rev. Marrakesh, 2002) of the Plenipotentiary Conference | NOC |  |  | 7 PTB |
| RES 95 | (Rev.WRC-07) General review of the Resolutions and Recommendations of world administrative radio conferences and world radiocommunication conferences | NOC |  |  | 4 PTA |
| RES 98 | (WRC-12) Provisional application of certain provisions of the Radio Regulations as revised by WRC-12 and abrogation of certain Resolutions and Recommendations |  |  |  | 4 PTA |
| RES 111 | (Orb-88) Planning of the fixed-satellite service in the bands 18.1-18.3 GHz, 18.3-20.2 GHz and 27-30 GHz | NOC |  |  |  |
| RES 114 | (Rev.WRC-12) Studies on compatibility between new systems of the aeronautical radionavigation service and the fixed-satellite service (Earth-to-space) (limited to feeder links of the non-geostationary mobile-satellite systems in the mobile-satellite service) in the frequency band 5 091-5 150 MHz | MOD | This Resolution is referred to in Nos. 5.444 and 5.444A (reference shall be updated); Resolves 2 requires action by a WRC prior 2018 |  | 1.7 PTB |
| RES 122 | (Rev.WRC-07) Use of the bands 47.2-47.5 GHz and 47.9-48.2 GHz by high altitude platform stations in the fixed service and by other services | NOC |  |  |  |
| RES 125 | (Rev.WRC-12) Frequency sharing in the bands 1 610.6-1 613.8 MHz and 1 660-1 660.5 MHz between the mobile-satellite service and the radio astronomy service | MOD/SUP | Resolves that a future competent WRC should evaluate frequency sharing in these bands. ITU-R M.1316 latest update was 2005 and contains an indication of relevant further studies. See also RA.769. |  |  |
| RES 140 | (WRC-03) Measures and studies associated with the equivalent power flux-density (epfd) limits in the band 19.7-20.2 GHz | NOC |  |  |  |
| RES 142 | (WRC-03) Transitional arrangements relating to use of the frequency band 11.7-12.2 GHz by geostationary-satellite networks in the fixed-satellite service in Region 2 | NOC/MOD | Transitional measure still necessary? |  | PT B |
| RES 143 | (Rev.WRC-07) Guidelines for the implementation of high-density applications in the fixed-satellite service in frequency bands identified for these applications | NOC | This Resolution is referred to in No. 5.516B (reference shall be updated). |  |  |
| RES 144 | (Rev.WRC-07) Special requirements of geographically small or narrow countries operating earth stations in the fixed-satellite service in the band 13.75-14 GHz | NOC |  |  |  |
| RES 145 | (Rev.WRC-12) Use of the bands 27.9-28.2 GHz and 31-31.3 GHz by high altitude platform stations in the fixed service | NOC | This Resolution is referred to in Nos. 5.537A and 5.543A. |  |  |
| RES 147 | (WRC-07) Power flux-density limits for certain systems in the fixed-satellite service using highly-inclined orbits having an apogee altitude greater than 18 000 km and an orbital inclination between 35° and 145° in the band 17.7-19.7 GHz | NOC |  |  |  |
| RES 148 | (WRC-07) Satellite systems formerly listed in Part B of the Plan of Appendix 30B (WARC Orb-88) |  | Check, whether Satellite systems from former Part B of the Plan are in operation |  |  |
| RES 149 | (Rev.WRC-12) Submissions from new Member States of the Union relating to Appendix 30B of the Radio Regulations | SUP |  |  |  |
| RES 150 | (WRC-12) Use of the bands 6 440-6 520 MHz and 6 560-6 640 MHz by gateway links for high-altitude platform stations in the fixed service |  | This Resolution is referred to in No. 5.457; |  |  |
| RES 151 | (WRC-12) Additional primary allocations to the fixed-satellite service in frequency bands between 10 and 17 GHz in Region 1 |  |  |  | 1.6 PTB |
| RES 152 | (WRC-12) Additional primary allocations to the fixed-satellite service in the Earth-to-space direction in frequency bands between 13-17 GHz in Region 2 and Region 3 |  |  |  | 1.6 PTB |
| RES 153 | (WRC-12) The use of frequency bands allocated to the fixed-satellite service not subject to Appendices 30, 30A and 30B for the control and non-payload communications of unmanned aircraft systems in non-segregated airspaces |  |  |  | 1.5 PTC |
| RES 154 | (WRC-12) Consideration of technical and regulatory actions in order to support existing and future operation of fixed-satellite service earth stations within the band 3 400-4 200 MHz, as an aid to the safe operation of aircraft and reliable distribution of meteorological information in some countries in Region 1 |  |  |  | 9.1.5 PTB |
| RES 205 | (Rev.WRC-12) Protection of the systems operating in the mobile-satellite service in the band 406-406.1 MHz | NOC |  |  | 9.1.1 PTB |
| RES 207 | (Rev.WRC-03) Measures to address unauthorized use of and interference to frequencies in the bands allocated to the maritime mobile service and to the aeronautical mobile (R) service | NOC |  |  |  |
|  | ANNEX Interference mitigation techniques |  |  |  |  |
| RES 212 | (Rev.WRC-07) Implementation of International Mobile Telecommunications in the bands 1 885 – 2 025 MHz and 2 110 – 2 200 MHz | NOC | This Resolution is referred to in Nos. 5.351A and 5.388 (reference shall be updated). |  |  |
| RES 215 | (Rev.WRC-12) Coordination process among mobile-satellite systems and efficient use of the allocations to the mobile-satellite service in the 1-3 GHz range |  |  |  |  |
| RES 217 | (WRC-97) Implementation of wind profiler radars | NOC/MOD | This Resolution is referred to in Nos. 5.162A and 5.291A |  |  |
| RES 221 | (Rev.WRC-07) Use of high altitude platform stations providing IMT in the bands 1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz in Regions 1 and 3 and 1 885-1 980 MHz and 2 110-2 160 MHz in Region 2 | NOC | This Resolution is referred to in No. 5.388A (reference shall be updated). |  |  |
|  | ANNEX Characteristics of a HAPS operating as an IMT base station in the frequency bands given in RES 221 (Rev.WRC-07) |  |  |  |  |
| RES 222 | (Rev.WRC-12) Use of the frequency bands 1 525-1 559 MHz and 1 626.5-1 660.5 MHz by the mobile-satellite service, and procedures to ensure long term spectrum access for the aeronautical mobile-satellite (R) service | MOD | This Resolution is referred to in Nos. 5.353A (reference shall be updated) and 5.357A |  |  |
| RES 223 | (Rev.WRC-12) Additional frequency bands identified for IMT | MOD | This Resolution is referred to in Nos. 5.384A and 5.388 (both references shall be updated) |  |  |
| RES 224 | (Rev.WRC-12) Frequency bands for the terrestrial component of International Mobile Telecommunications below 1 GHz | NOC | This Resolution is referred to in Nos. 5.312A, 5.316A, 5.316B and 5.317A |  |  |
| RES 225 | (Rev.WRC-12) Use of additional frequency bands for the satellite component of IMT | MOD |  |  |  |
| RES 229 | (Rev.WRC-12) Use of the bands 5 150-5 250 MHz, 5 250-5 350 MHz and 5 470-5 725 MHz by the mobile service for the implementation of wireless access systems including radio local area networks | NOC/MOD | This Resolution is referred to in Nos. 5.446A, 5.447 and 5.453 Resolves 3 indicates possible actions regarding the aggregated pfd limits Rec. M.1652-1 is incorporated by reference and was being revised. |  |  |
| RES 232 | (WRC-12) Use of the frequency band 694-790 MHz by the mobile, except aeronautical mobile, service in Region 1 and related studies |  | This Resolution is referred to in No. 5.312A; |  | 1.2 PTD |
| RES 233 | (WRC-12) Studies on frequency-related matters on International Mobile Telecommunications and other terrestrial mobile broadband applications |  |  |  | 1.1 PTD |
| RES 234 | (WRC-12) Additional primary allocations to the mobile-satellite service within the bands from 22 GHz to 26 GHz |  |  |  | 1.10 PTB |
| RES 331 | (Rev.WRC-12) Operation of the Global Maritime Distress and Safety System | MOD |  |  |  |
| RES 339 | (Rev.WRC-03) Coordination of NAVTEX services | NOC |  |  |  |
| RES 343 | (Rev.WRC-12) Maritime certification for personnel of ship stations and ship earth stations for which a radio installation is not compulsory | NOC | This Resolution is referred to in No. 47.27A. |  |  |
|  | ANNEX Examination syllabus for radio operator’s certificates appropriate to vessels using the frequencies and techniques of the Global Maritime Distress and Safety System on a non-compulsory basis |  |  |  |  |
| RES 344 | (Rev.WRC-12) Management of the maritime mobile service identity numbering resource | NOC | Revision of Recommendation ITU-R M.585 has been made. M.585 is incorporated by reference |  |  |
| RES 349 | (Rev.WRC-12) Operational procedures for cancelling false distress alerts in the Global Maritime Distress and Safety System | NOC |  |  |  |
|  | ANNEX Cancelling of false distress alerts |  |  |  |  |
| RES 352 | (WRC-03) Use of the carrier frequencies 12 290 kHz and 16 420 kHz for safety-related calling to and from rescue coordination centres | NOC | This Resolution is referred to in Part B, 5 of App. 17. |  |  |
| RES 354 | (WRC-07) Distress and safety radiotelephony procedures for 2 182 kHz | NOC |  |  |  |
|  | Annex |  |  |  |  |
| RES 356 | (WRC-07) ITU maritime service information registration | NOC |  |  |  |
| RES 358 | (WRC-12) Consideration of improvement and expansion of on-board communication stations in the maritime mobile service in the UHF bands |  |  |  | 1.15 PTC |
| RES 359 | (WRC-12) Consideration of regulatory provisions for modernization of the Global Maritime Distress and Safety System and studies related to e-navigation |  |  |  | WRC-18 AI 2.1 |
| RES 360 | (WRC-12) Consideration of regulatory provisions and spectrum allocations for enhanced Automatic Identification System technology applications and for enhanced maritime radiocommunication |  |  |  | 1.16 PTC |
| RES 405 | Relating to the use of frequencies of the aeronautical mobile (R) service | NOC |  |  |  |
| RES 413 | (Rev. WRC-12) Use of the band 108-117.975 MHz by the aeronautical mobile (R) service | MOD/SUP | This Resolution is referred to in No. 5.197A (reference shall be updated). |  |  |
| RES 416 | (WRC-07) Use of the bands 4 400 – 4 940 MHz and 5 925 – 6 700 MHz by an aeronautical mobile telemetry application in the mobile service | NOC | This Resolution is referred to in No. 5.440A, 5.442 and 5.457C |  |  |
| RES 417 | (Rev.WRC-12) Use of the frequency band 960-1 164 MHz by the aeronautical mobile (R) service | MOD/SUP | Linked with Resolution 413 This Resolution is referred to in No. 5.327A; |  |  |
| RES 418 | (Rev.WRC-12) Use of the band 5 091 – 5 250 MHz by the aeronautical mobile service for telemetry applications | NOC | This Resolution is referred to in No. 5.444B and 5.446C |  |  |
|  | Annex |  |  |  |  |
| RES 422 | (WRC-12) Development of methodology to calculate aeronautical mobile-satellite (R) service spectrum requirements within the frequency bands 1 545-1 555 MHz (space-to-Earth) and 1 646.5-1 656.5 MHz (Earth-to-space) |  |  |  |  |
| RES 423 | (WRC-12) Consideration of regulatory actions, including allocations, to support Wireless Avionics Intra-Communications |  |  |  | 1.17 PTC |
| RES 506 | (Rev.WRC-97) Use by space stations in the broadcasting-satellite service operating in the 12 GHz frequency bands allocated to the broadcasting-satellite service of the geostationary-satellite orbit and no other | NOC |  |  |  |
| RES 507 | (Rev.WRC-12) Establishment of agreements and associated plans for the broadcasting-satellite service | NOC/MOD | This Resolution is linked to RES 33. This Resolution is referred to in No. 11.37.2 |  |  |
| RES 517 | (Rev.WRC-07) Introduction of digitally modulated emissions in the high-frequency bands between 3 200 kHz and 26 100 kHz allocated to the broadcasting service | NOC | This Resolution is referred to in 5.134 (reference shall be updated). Recommendations ITU-R BS.1514 and ITU-R BS.1615 have been updated in 2011. |  |  |
| RES 526 | (Rev.WRC-12) Future adoption of procedures to ensure flexibility in the use of the frequency band allocated to the broadcasting-satellite service (BSS) for wide RF-band high-definition television (HDTV) and to the associated feeder links | MOD |  |  |  |
| RES 528 | (Rev.WRC-03) Introduction of the broadcasting-satellite service (sound) systems and complementary terrestrial broadcasting in the bands allocated to these services within the range 1-3 GHz | NOC/MOD | This Resolution is referred to in Nos. 5.417A,5.418 and 5.393. A former version of this Resolution is referred to in No. 5.345. |  |  |
| RES 535 | (Rev.WRC-03) Information needed for the application of Article 12 of the Radio Regulations | NOC | Still relevant? |  |  |
|  | ANNEX |  |  |  |  |
| RES 536 | (WRC-97) Operation of broadcasting satellites serving other countries | NOC |  |  |  |
| RES 539 | (Rev.WRC-03) Use of the band 2 605-2 655 MHz in certain Region 3 countries by non-geostationary satellite systems in the broadcasting-satellite service (sound) | NOC | This Resolution is referred to in Nos. 5.417A and 5.418 This Resolution is also referred to in Table 5-1, No. 9.11 of App. 5. |  |  |
| RES 543 | (WRC-03) Provisional RF protection ratio values for analogue and digitally modulated emissions in the HF broadcasting service | NOC | This Resolution is referred to in Art. 1.1 and 2.5 of Part C of App. 11. |  |  |
|  | ANNEX |  |  |  |  |
| RES 547 | (Rev. WRC-07) Updating of the “Remarks” columns in the Tables of Article 9A of Appendix 30A and Article 11 of Appendix 30 of the Radio Regulations | NOC |  |  |  |
| RES 548 | (Rev.WRC-12) Application of the grouping concept in Appendices 30 and 30A in Regions 1 and 3 | NOC/MOD |  |  |  |
| ES 549 | (WRC-07) Use of the frequency band 620 – 790 MHz for existing assignments to stations of the broadcasting satellite service | NOC | This Resolution is referred to in Table 5-1, Nos. 9.11and 9.19 of App. 5. |  |  |
| RES 550 | (WRC-07) Information relating to the high-frequency broadcasting service | NOC |  |  |  |
| RES 552 | (WRC-12) Long-term access to and development in the band 21.4-22 GHz in Regions 1 and 3 |  | This Resolution is referred to in Nos. A.9.4, A.11.2, 11.44.1 and 11.48 |  |  |
| RES 553 | (WRC-12) Additional regulatory measures for broadcasting-satellite networks in the band 21.4-22 GHz in Regions 1 and 3 for the enhancement of equitable access to this band |  | This Resolution is referred to in No. A.9.8 This Resolution is also referred to in Table 5-1, No. 9.7, 6bis), of App. 5. |  |  |
| RES 554 | (WRC-12) Application of pfd masks to coordination under No. 9.7 for broadcasting-satellite service networks in the band 21.4-22 GHz in Regions 1 and 3 |  | This Resolution is referred to in No. A.11.7 This Resolution is also referred to in Table 5-1, No. 9.7. 6bis), of App. 5. |  |  |
| RES 555 | (WRC-12) Additional regulatory provisions for broadcasting-satellite service networks in the band 21.4-22 GHz in Regions 1 and 3 for the enhancement of equitable access to this band |  | This Resolution is referred to in No. 5.530D |  |  |
| RES 608 | (WRC-03) Use of the frequency band 1 215-1 300 MHz by systems of the radionavigation-satellite service (space-to-Earth) | NOC | This Resolution is referred to in No. 5.329. |  |  |
| RES 609 | (Rev.WRC-07) Protection of aeronautical radionavigation service systems from the equivalent power flux-density produced by radionavigation-satellite service networks and systems in the 1 164-1 215 MHz frequency band | NOC | This Resolution is referred to in No. 5.328A; |  |  |
|  | ANNEX Criteria for application of RES 609 (Rev.WRC-07) |  |  |  |  |
| RES 610 | (WRC-03) Coordination and bilateral resolution of technical compatibility issues for radionavigation-satellite service networks and systems in the bands 1 164-1 300 MHz, 1 559-1 610 MHz and 5 010-5 030 MHz | NOC | This Resolution is referred to in No. 5.328B |  |  |
|  | ANNEX Criteria for application of RES 610 (WRC-03) |  |  |  |  |
| RES 612 | (Rev.WRC-12) Use of the radiolocation service between 3 and 50 MHz to support oceanographic radar operations | MOD | This Resolution is referred to in Nos. 5.132A, 5.145A and 5.161A |  |  |
| RES 641 | (Rev.HFBC-87) Use of the frequency band 7 000-7 100 kHz | NOC |  |  |  |
| RES 642 | Relating to the bringing into use of earth stations in the amateur-satellite service | NOC |  |  |  |
| RES 644 | (Rev.WRC-12) Radiocommunication resources for early warning, disaster mitigation and relief operations | NOC/MOD |  |  |  |
| RES 646 | (Rev.WRC-12) Public protection and disaster relief | MOD |  |  | 1.3 PTA |
| RES 647 | (Rev.WRC-12) Spectrum management guidelines for emergency and disaster relief radiocommunication | NOC/MOD |  |  | 9.1.7 PTA |
| RES 648 | (WRC-12) Studies to support broadband public protection and disaster relief |  |  |  | 1.3 PTA |
| RES 649 | (WRC-12) Possible allocation to the amateur service on a secondary basis at around 5 300 kHz |  |  |  | 1.4 PTC |
| RES 650 | (WRC-12) Allocation for the Earth exploration-satellite service (Earth-to-space) in the 7-8 GHz range |  |  |  | 1.11 PTA |
| RES 651 | (WRC-12) Possible extension of the current worldwide allocation to the Earth exploration-satellite (active) service in the frequency band 9 300-9 900 MHz by up to 600 MHz within the frequency bands 8 700-9 300MHz and/or 9 900- 10 500 MHz |  |  |  | 1.12 PTA |
| RES 652 | (WRC-12) Use of the band 410-420 MHz by the space research service (space-to-space) |  |  |  | 1.13 PTA |
| RES 653 | (WRC-12) Future of the Coordinated Universal Time time-scale |  |  |  | 1.14 PTA |
| RES 654 | (WRC-12) Allocation of the band 77.5-78 GHz to the radiolocation service to support automotive short-range high-resolution radar operations |  |  |  | 1.18 PTC |
| RES 673 | (Rev.WRC-12) The importance of Earth observation radiocommunication applications | MOD | This Resolution is referred to in No. 29A.1 |  |  |
| RES 703 | (Rev.WRC-07) Calculation methods and interference criteria recommended by ITU-R for sharing frequency bands between space radiocommunication and terrestrial radiocommunication services or between space radiocommunication services | SUP |  |  |  |
| RES 705 | (Mob-87) Mutual protection of radio services operating in the band 70-130 kHz | NOC/MOD |  |  |  |
| RES 716 | (Rev.WRC-12) Use of the frequency bands 1 980-2 010 MHz and 2 170-2 200 MHz in all three Regions and 2 010-2 025 MHz and 2 160-2 170 MHz in Region 2 by the fixed and mobile-satellite services and associated transition arrangements | MOD | A former version of this Resolution is referred to in Nos. 5.389A, 5.389C and 5.390 |  |  |
| RES 729 | (Rev.WRC-07) Use of frequency adaptive systems in the MF and HF bands | MOD/SUP | Resolution is referred to in Table 1 of Annex 1 to App. 4. Should be considered in the light of the results of Agenda Item 1.19 WRC-12. |  |  |
| RES 731 | (Rev.WRC-12) Consideration of sharing and adjacent-band compatibility between passive and active services above 71 GHz | NOC |  |  |  |
| RES 732 | (Rev.WRC-12) Consideration of sharing between active services above 71 GHz | NOC |  |  |  |
| RES 739 | (Rev.WRC-07) Compatibility between the radio astronomy service and the active space services in certain adjacent and nearby frequency bands | NOC | This Resolution is referred to in No. 5.208B |  |  |
|  | ANNEX 1 Unwanted emission threshold levels |  |  |  |  |
| RES 741 | (Rev.WRC-12) Protection of the radio astronomy service in the band 4 990-5 000 MHz from unwanted emissions of the radionavigation-satellite service (space-to-Earth) operating in the frequency band 5 010-5 030 MHz | NOC/MOD | This Resolution is referred to in No. 5.443B. |  |  |
| RES 743 | (WRC-03) Protection of single-dish radio astronomy stations in Region 2 in the 42.5-43.5 GHz band | NOC | This Resolution is referred to in Nos. 5.551H and 5.551I. |  |  |
| RES 744 | (Rev.WRC-07) Sharing between the mobile-satellite service (Earth-to-space) and the fixed and mobile services in the band 1 668.4-1 675 MHz | NOC | This Resolution is referred to in No. 5.379D |  |  |
| RES 748 | (Rev.WRC-12) Compatibility between the aeronautical mobile (R) service and the fixed-satellite service (Earth-to-space) in the band 5 091 – 5 150 MHz | NOC/MOD | This Resolution is referred to in No. 5.444B. |  |  |
| RES 749 | (Rev.WRC-12) Use of the band 790-862 MHz in countries of Region 1 and the Islamic Republic of Iran by mobile applications and by other services | MOD | This Resolution is referred to in Nos. 5.316A, 5.316B and 5.317A. |  |  |
| RES 750 | (Rev.WRC-12) Compatibility between the Earth exploration-satellite service (passive) and relevant active services | NOC | This Resolution is referred to in No. 5.338A. |  |  |
| RES 751 | (WRC-07) Use of the frequency band 10.6-10.68 GHz | NOC | This Resolution is referred to in No. 5.482A. |  |  |
|  | ANNEX Sharing criteria in the band 10.6-10.68 GHz |  |  |  |  |
| RES 752 | (WRC-07) Use of the frequency band 36-37 GHz | NOC | This Resolution is referred to in No. 5.550A. |  |  |
|  | ANNEX Sharing criteria in the band 36-37 GHz |  |  |  |  |
| RES 755 | (WRC-12) Power flux-density limits for transmitting stations in the 21.4-22 GHz band |  | This Resolution is referred to in No. 5.530C |  |  |
| RES 756 | (WRC-12) Studies on possible reduction of the coordination arc and technical criteria used in application of No. 9.41 in respect of coordination under No. 9.7 |  |  |  | 9.1.2 PTB |
| RES 757 | (WRC-12) Regulatory aspects for nanosatellites and picosatellites |  |  |  | 9.1.8 PTA |
| RES 758 | (WRC-12) Allocation to the fixed-satellite service and the maritime mobile satellite service in the 7/8 GHz range |  |  |  | 1.9 PTB |
| RES 804 | Rev.(WRC-12) Principles for establishing agendas for world radiocommunication conferences | NOC |  |  |  |
|  | ANNEX 1 Principles for establishing agendas for WRCs |  |  |  |  |
|  | ANNEX 2 Template for the submission of proposals for agenda items |  |  |  |  |
| RES 806 | (WRC-07) Preliminary agenda for the 2015 World Radiocommunication Conference | SUP | New Resolution was developed during WRC-12, containing the agenda for WRC-15, but this Resolution was not abrogated | SUP |  |
| RES 807 | (WRC-12) Agenda for the 2015 World Radiocommunication Conference |  |  |  |  |
| RES 808 | (WRC-12) Preliminary agenda for the 2018 World Radiocommunication Conference |  |  |  | 10 PTA |
| RES 900 | (WRC-03) Review of the Rule of Procedure for No. 9.35 of the Radio Regulations | SUP |  |  |  |
|  | ANNEX Procedure to be used by the Radiocommunication Bureau for networks examined under the Rule of Procedure on No. 9.35 |  |  |  |  |
| RES 901 | (Rev.WRC-07) Determination of the orbital arc separation for which coordination would be required between two satellite networks operating in a space service not subject to a Plan | NOC | This Resolution is referred to in No. Table 5-1, No. 9.7, 7) and 8) of App 5. |  |  |
| RES 902 | (WRC-03) Provisions relating to earth stations located on board vessels which operate in fixed-satellite service networks in the uplink bands 5 925-6 425 MHz and 14-14.5 GHz | NOC | This Resolution is referred to in Nos. 5.457A, 5.457B, 5.506A and 5.506B. |  |  |
|  | ANNEX 1 Regulatory and operational provisions for ESVs transmitting in the 5 925-6 425 MHz and 14-14.5 GHz bands |  |  |  |  |
|  | ANNEX 2 Technical limitations applicable to ESVs transmitting in the bands 5 925-6 425 MHz and 14-14.5 GHz |  |  |  |  |
| RES 903 | (WRC-07) Transitional measures for certain broadcasting satellite / FSS systems in the band 2 500 – 2 690 MHz | NOC | This Resolution is referred to in No. 21.16.3A |  |  |
| RES 904 | (WRC-07) Transitional measure for coordination between Mobile-Satellite Service (Earth-to-space) and the space research (passive) service in the band 1 668 – 1 668.4 MHz for a specific case | NOC | This Resolution is referred to in No. 5.379B Transitional measures still needed? |  |  |
| RES 906 | (Rev.WRC-12) Electronic submission of notice forms for terrestrial services to the Radiocommunication Bureau and exchange of data between administrations | SUP |  |  |  |
| RES 907 | (WRC-12) Use of modern electronic means of communication for administrative correspondence related to advance publication, coordination and notification of satellite networks including that related to Appendices 30, 30A and 30B, earth stations and radio astronomy stations |  |  |  |  |
| RES 908 | (WRC-12) Electronic submission and publication of advance publication information |  |  |  |  |
| RES 909 | (WRC-12) Provisions relating to earth stations located on board vessels which operate in fixed-satellite service networks in the uplink bands 5 925-6 425 MHz and 14-14.5 GHz |  |  |  | 1.8 PTB |
| RES 957 | (WRC-12) Studies towards review of the definitions of fixed service, fixed station and mobile station |  |  |  | 9.1.6 PTA |