|  |  | CPG15(13)xx Annex 2 |
| --- | --- | --- |
| 2nd Meeting CPG15 Luxembourg, 21-26 January 2013 |  |  |
| Date issued: 7 September 2012 Source: CPG PTA(2012)024 Annex 9 Subject: Draft CEPT Brief for Agenda Item 1.12 (WRC-15) |  |  |

Summary

Proposal CPG is invited to consider and approve the attached Draft Brief on WRC-15 AI 1.12.

NTS

DRAFT CEPT BRIEF ON AGENDA ITEM 1.12 (WRC-15)

1.12	to consider an extension of the current worldwide allocation to the Earth exploration-satellite (active) service in the frequency band 9 300-9 900 MHz by up to 600 MHz within the frequency bands 8 700-9 300 MHz and/or 9 900-10 500 MHz, in accordance with Resolution 651 (WRC 12)

# ISSUE

Resolution 651 (WRC-12) invites WRC-15 to consider the possible extension of the current worldwide allocation to the EESS (active) in the frequency band 9 300-9 900 MHz by up to 600 MHz within the frequency range 8 700- 9 300 MHz and/or 9 900-10 500 MHz while ensuring protection of existing services and taking due account of the safety services allocated in the frequency band 9 000 to 9 300 MHz.

Resolution 651 (WRC-12) also invites ITU-R to conduct compatibility studies addressing

EESS (active) and existing services in the frequency bands 8 700-9 300 MHz and 9 900-10 500 MHz in order to ensure the protection of the existing services, taking into account the constraints as per No. 5.476A;

unwanted emissions from stations operating in the EESS (active) within the frequency band 8 700-9 300 MHz into stations of the space research service operating in the frequency band 8 400-8 500 MHz;

unwanted emissions from stations operating in the EESS (active) within the frequency band 9 900-10 500 MHz into stations of the radio astronomy service, space research service (passive) and EESS (passive) operating in the frequency band 10.6-10.7 GHz.

CPM-15-1 assigned ITU-R Working Party 7C as responsible party for Agenda Item 1.12 in Chapter 2 of the CPM-Report.

CPG has assigned PTA to foster all activities related to this Agenda Item.

# Preliminary CEPT position

The addition of 600 MHz to the existing allocation is supported provided that

- studies show adequate RF compatibility with existing radio services

- Radioastronomy service, the space research services (active and passive), and the Earth-exploration-satellite (passive) service in nearby allocations are adequately protected from unwanted emissions

# Background

During the study cycle for WRC-07, studies were performed by ITU-R under Agenda Item 1.3 (WRC-07) to investigate the conditions for the extension of the EESS (active) allocation by 200 MHz above or below the former allocation 9 500 – 9 800 MHz. Based on the results and conclusions in report ITU-R RS.2094, WRC-07 finally decided to extend the allocation to 9 300- 9 900 MHz. This was possible because the overall sharing conditions were found to be acceptable if certain conditions are obeyed. These conditions are regulated in footnotes no. 5.475A, 5.476A, 5.477, 5.478, 5.478A, and 5.478B to protect other radio service in countries mentioned in the footnotes.

Space-borne radars operating in the EESS (active) in this band have demonstrated their important contributions to a large number of scientific and geoinformation applications which is also recognised in Resolution 673 (rev. WRC-12). The growing demand for higher resolution radar picture raises the need to further increase the transmission bandwidth of the next generation of EESS-radars in this band because the effectively used transmission bandwidth has a direct correspondence to the achievable picture resolution.

Editor’s note: The text in squared brackets is considered preliminary. It should reflect studies to be made. [Sharing studies need to be performed for radio services allocated in both potential frequency ranges 8 700-9 300 MHz and 9 900-10 500 MHz, considered for extension by Resolution 651 (WRC-12).

General sharing conditions for EESS (active)

It is important to note that EESS radars transmit only for very short time intervals, i.e. for less than seven seconds per exposure. The next generation of wideband spaceborne radar systems are assumed with transmission characteristics similar to those already used in the current allocation. It is assumed that the conditions for sharing with Radiolocation, Fixed and other Services within the existing allocation of 9.3-9.9 GHz can be widely extrapolated into the candidate extension bands.

Additional sharing studies are needed to investigate the sharing conditions within the two potential extension bands and with radio services that claim particularly protection from unwanted emissions. These services are:

Sharing EESS (active) with the Radiolocation Service (RLS)

RLS systems already share with EESS (active) systems in the current allocation for many years. Since this sharing already occurs without indications of harmful interference, it is assumed that the conclusions achieved in report ITU-R RS.2094 do also apply to both frequency ranges considered for potential extension of the allocation to the EESS. Particular radars, identified by WP5B liaison and relevant for the sharing studies, will be taken from Recommendation ITU-R M.1796.

Sharing EESS (active) with the Radionavigation Service (RNS)

This service comprises allocations particularly dedicated to the maritime and aeronautical radionavigation services. Some of the systems used in both services are also operated in the current allocation under the radionavigation service.

Sharing EESS (active) with the Fixed Service (FS)

The conditions for sharing with FS have already been determined for the band 9.8-9.9 GHz resulting in footnotes no. 5.477and 5.478B. Further studies are needed to investigate if extrapolation of regulatory conditions would be possible.

Sharing EESS (active) with the Mobile Service (MS)

Band allocations to the MS are in the band 10.0-10.5 GHz and through footnotes no. 5.468 and 5.469 as well as 5.480 and 5.481 in the upper and lower extension range, respectively.

Sharing EESS (active) with the Amateur Radio Service (ARS)

ARS is allocated in the band 10.0-10.5 GHz on a secondary basis.

Protection of the Space Research Service (SRS) from EESS (active) unwanted emissions in the band 8 400-8 500 MHz

Besides sharing with radio services potentially affected by an extension, further studies are needed to investigate compatibility conditions of unwanted emissions from wideband EESS systems into nearby SRS allocation in the frequency band 8 400-8 500 MHz. Initial studies (7C/247) were already performed in the last study cycle for WRC-12. These need to be continued and concluded in this cycle.

Protection of the Radio astronomy Service (RAS) from EESS (active) unwanted emissions in the band 10.6-10.7 GHz

The RF compatibility of unwanted emissions from wideband EESS (active) into the Radioastronomy service allocated in the 10.6-10.7 GHz, which started in the last cycle (7C/247) needs to be further analysed and concluded.

Protection of the Earth Exploration-satellite Service (passive) from EESS out-of-band emissions in the band 10.6-10.7 GHz

Further affected in the band 10.6-10.7 GHz, there are two more radio services SRS (passive) and EESS (passive). As SRS (passive) systems are sensors used around other planets, no interference to SRS (passive) would be possible from space borne SARs, thus, no study required.

Publishing the characteristics of typical next generation EESS (active) spaceborne radars

The characteristics of a typical wideband EESS radar instrument need to be developed as a dedicated new ITU-R Recommendation. ]

# List of relevant documents

## ITU Documentation

ITU-R Recommendations

Remote Sensing: 	ITU-R RS.1029, RS.1166, RS.1280, RS.1859, RS.1883

Space applications and meteorology: 	ITU-R SA.509, SA.609, SA.1157

Radio astronomy: 	ITU-R RA.517, RA.611, RA.769, RA.1513, RA.1631, M.1583

Mobile, radiodetermination, amateur satellite services:

ITU-R M.628, M.629, M.824, M.1044, M.1372, M.1461, M.1732, 
M.1796, M.1849

Spectrum Management: 	ITU-R SM.1138, SM.1535, SM.1541

Fixed Service: 	ITU-R F.699, F.758, F.1108, F.1403, F.1245, F.1336

ITU-R Reports:

ITU-R M.2050, M.2076, M.2081, M.2128

ITU-R RS.2094, RS.2178, RS.2188

## CEPT and/or ECC Documentation

ECC Report 018 “Sharing RAS operating in the band 10.6-10.7 GHz and other services”

ECC Report 173 “Fixed Service in Europe – Current use and future trends post 2011”

ERC Report 040 “Fixed Service system parameters for frequency sharing”

## EU Documentation

[To be determined]

# Actions to be taken

In line with Resolution 651(WRC15), ITU-R Working Party 7C will have to develop the following documents. CEPT will observe the progress and contribute, as appropriate, to the development of the documents.

ITU-R Recommendation

1	Recommendation ITU-R RS.[EESS-9GHz-Char] [”Characteristics of synthetic aperture radars operating in the Earth exploration-satellite service (active) around 9.6 GHz”]

ITU-R Reports

1	Report ITU-R RS.[EESS-9GHz_SpectReq] [“Applications and spectrum requirements of very wideband synthetic aperture radars (SAR) operating in the 9 GHz band allocated to the EESS(active)”]

2	Report ITU-R RS.[EESS-9GHz_OOBE] [“RF compatibility of unwanted emissions from 9 GHz EESS synthetic aperture radars (SAR) into the SRS and RAS operating in the frequency bands 8.4-8.5 GHz and 10.6-10.7 GHz, respectively”]

3	Report ITU-R RS.[EESS-9GHz_RDS/ARNS/MRNS] [“Sharing analyses of wideband SAR operating in the 9 GHz EESS with incumbent radio services in the frequency bands 8.7-9.3 GHz and 9.9-10.5 GHz”]

4	Report ITU-R RS.[EESS-9GHz_FS/MS/ARS] [“Sharing analyses of very wideband EESS SAR transmissions with stations in the fixed, mobile, and amateur radio services operating in the frequency band 9 900-10 500 MHz”]

ITU-R CPM Report – Chapter 2 Agenda Item 1.12

In line with the master study schedule distributed by the Bureau with administrative circular CA/201 the following work plan should guide through the work to be performed for this agenda item.

| Working Party 7C Meetings | Planned achievements under Agenda Item 1.12 (WRC-15) |
| --- | --- |
| 1st meeting 24-28 September 2012 Manta, Ecuador | - Initial work plan - Identify for completeness the applicability of existing recommendations and relevant reports to enable sharing studies - LS to WP5A on recommendations and reports taken assumed relevant for mobile (MS) and amateur radio (satellite) (ARSS) services. - LS in response to 5B and 5C to exchange information on radio determination (RNS/RLS) and fixed service (FS) - Structure of draft CPM-Chapter 2 text - Initiate PDN Recommendation 1 with the main characteristics of SARs planned to operate in the extended band - Initiate PDN Reports 1-4. The structure of these documents should be completed and the available studies included. |
| 2nd meeting 8-12 April 2013 ITU, Geneva | - Finalize DN Recommendation 1 - Progress and if possible complete PDN Report 1 and 2 - Update PDN Reports 3 and 4 to include the results of studies. The sharing scenarios should be completed at this stage. - Update draft CPM text - Update work plan as needed - Response LSs to WP5A, 5B, and 5C to send the available material for comments |
| 3rd meeting 11-17 September 2013 ITU, Geneva | - Finalize reports 1 and 2 - Progress Reports 3 and 4 integrating the comments received by LS from 5A, 5B and 5C - Update Draft CPM text - Update work plan as needed |
| 4th meeting 12-16 May 2014 ITU, Geneva | - Finalize Reports 3 and 4 (Note: there is still time after completion of the draft CPM text to work on these reports if needed) - final CPM text on AI1.12 (under responsibility of WP7C); ready for release to CPM Chapter 2 Rapporteur |
| 5th meeting 29 Sep – 3 Oct 2014 ITU, Geneva | - Complete and note any ITU-R Rec/Rep relevant for recognition at CPM-15 - Consider possible consequential changes to the existing ITU-R documentation as a result of adoption of proposed draft new Recommendations/ Reports |

# Relevant information from outside CEPT

## European Union (date of proposal)

[To be determined]

## Regional telecommunication organisations:

APT (date of proposal)

[To be determined]

ATU (date of proposal)

[To be determined]

Arab Group ASMG (date of proposal)

[To be determined]

CITEL (date of proposal)

[To be determined]

RCC (date of proposal)

[To be determined]

## International organisations

IATA (date of proposal)

[To be determined]

ICAO (30 March 2012)

Oppose any allocation to the Earth exploration-satellite service in the frequency band 9 000 – 9 200 MHz unless

- it has been demonstrated through agreed studies that there will be no impact on aviation use

- no additional constraints are place on the use of the frequency band by aeronautical systems

No change to Nos. 5.337, 5.427, 5.474 and 5.475.

IMO (date of proposal)

[To be determined]

NATO (date of proposal)

[To be determined]

SFCG (20 June 2012)

SFCG supports an extension of the current worldwide allocation to the Earth exploration-satellite (active) service in the frequency band 9 300 – 9 900 MHz by up to 600 MHz. Compatibility with SRS (space-to-Earth) links in the 8 400 – 8 500 MHz band and the EESS (passive) in the 10.6 – 10.7 GHz band will have to be ensured, in accordance with the appropriate protection criteria, taking into account any available mitigation techniques that would reduce the level of unwanted emissions in those two bands.

WMO and EUMETNET (date of proposal)

[To be determined]

CRAF (11 October 2011)

CRAF urges administrations to ensure effective protection of radio observatories and space research stations operating in adjacent bands. Coexistence studies are recommended to suggest suitable hard limits for out of band emissions by EESS; any direct illumination of radio astronomy observatories and space research ground stations by the new generation of EESS should be prohibited.

## Regional organisations

ESA (date of proposal)

[To be determined]

EUMETNET (date of proposal)

[To be determined]

Eurocontrol (date of proposal)

[To be determined]