|  |  | Doc. ECC/CPG-15 005 Annex 06 |
| --- | --- | --- |
| 2nd Meeting CPG-15 |  |  |
| Luxembourg, 22-25 January 2013 |  |  |
| Date issued: | 13 December 2012 |  |
| Source: | CPG PTC |  |
| Subject: | Draft CEPT Brief on WRC-15 Agenda Item 1.18 (WRC-15) |  |
| Summary: |  |  |
| Proposal: |  |  |
| CPG is invited to consider and approve the attached Draft CEPT Brief on AI 1.18 (WRC-15). |  |  |

E OF CONTENTS

DRAFT CEPT BRIEF ON AGENDA ITEM 1.18

1.18	to consider a primary allocation to the radiolocation service for automotive applications in the 77.5 – 78.0 GHz frequency band in accordance with Resolution 654 (WRC12).

# ISSUE

Resolution 654 (WRC 12) invites ITU-R:

to conduct, as a matter of urgency, and in time for consideration by WRC-15, the appropriate technical, operational and regulatory studies, including:

i)        sharing studies and regulatory solutions to consider a primary allocation to the radiolocation service in the band 77.5-78 GHz, taking into account incumbent services and existing uses of the band;

ii)       compatibility studies in the band 77.5-78 GHz with services operating in the adjacent bands 76-77.5 GHz and 78-81 GHz;

iii)     spectrum requirements, operational characteristics and evaluation of ITS safety-related  applications that would benefit from global or regional harmonization.

# Preliminary CEPT position

CEPT supports a primary allocation to the radiolocation service in the frequency band 77.5 to 78.0 GHz to enable automotive short-range high-resolution radar operations

# Background

There has been significant growth in the use of automotive radar systems, and these systems are expected to become relatively common place within a few years because of general demand for increased vehicle safety.

In order to facilitate the development and deployment of automotive radar systems in Europe, the frequency ranges have been allocated at 24 GHz on a temporary basis due to incompatibilities with existing services, and 79 GHz on a permanent basis.

The 79 GHz band (77-81 GHz) is thus considered to be the long-term operating frequency for SRR, having been allocated by European Commission Decision 2004/545/EC and CEPT ECC Decision ECC/DEC/(04)03) in 2004.

Studies have shown that the use of collision avoidance technology can prevent or lessen the severity of significant number of traffic accidents. In certain parts of the world, automotive radars have successfully operated particularly in the 76-77 GHz band, for many years without mitigation methods or deactivation methods and without increased reports of interference to licensed services.

Portions of 76-81 GHz band are allocated to the radio astronomy service (RAS), amateur and amateur-satellite and radiolocation (RLS) services on a primary or secondary basis and to the space research (space-to-Earth) service on a secondary basis. At these frequencies, radio propagation decreases more rapidly with distance than at other frequencies and antennas that can narrowly focus transmitted energy are practical and of modest size. While the limited range of such transmissions might appear to be a major disadvantage for many applications, it does allow the reuse of frequencies within very short distances and, thereby enables a higher concentration of transmitters to be located in a geographical area than is possible at lower frequencies. The attenuation of the transmissions, however, varies with the water content of the atmosphere and some other atmospheric factors.

The ITU Council, in adopting Resolution 1318 (Council 2010), stated that ICTs, including intelligent transport systems (ITS), provide mechanisms for vehicular and passenger safety; and invited members of the union to take practical steps to further national and domestic policies, programs and/or educational initiatives in the use of ICTs to improve global road safety.

Status of the CEPT regulatory framework

- In 2001 ECC amended ECC/DEC/(02)01 “.. on the frequency bands to be designated for the co-ordinated introduction of Road Transport and Traffic Telematic Systems” (RTTT) in which the frequency band 76.0 to 77.0 GHz is designated for vehicular radars. This decision is withdrawn in 2012 and replaced by amended ANNEX 5 in ERC Recommendation 70-03 (ANNEX 5: ROAD TRANSPORT AND TRAFFIC TELEMATICS (RTTT))

- (ECC/DEC/(04)03) ECC Decision of 19 March 2004 on the frequency band 77–81 GHz to be designated for the use of Automotive Short Range Radars

## Regulatory status of radiolocation service in the 76-81 GHz band

Currently, 76-77.5 GHz, and 78-81 GHz bands are allocated to the radiolocation service globally on a primary basis. Obtaining a possible global primary radiolocation service allocation in the 77.5-78 GHz band provides for a harmonized, contiguous band for collision avoidance related automotive radar applications in the 76-81 GHz band. It should be noted that RR No. 5.149 urges administrations to take all practicable steps to protect the radio astronomy service from harmful interference in a number of bands including the 76-86 GHz band.

## The CPM Method described below is supported by the CEPT

A primary allocation to the radiolocation service on a worldwide basis, limited to automotive radar applications, between 77.5 GHz and 78 GHz.

Advantages

Provides worldwide harmonization for safety and collision avoidance related automotive radar applications in the 76-81 GHz band.

Provides a broader manufacturing base and increased volume of equipment (globalization of markets) resulting in economies of scale and expanded equipment availability.

Results in minimal interference to incumbent services due to the nature of these short range automotive radars along with the propagation characteristics of this part of the electromagnetic spectrum,

Disadvantage

In some areas, mitigation methods, such as proper emission power limits, may be needed to avoid potential interference to the radio astronomy service operating in portions of the 76-81 GHz band.

### Method

The regulatory approach under this Method is to add a primary allocation to RLS in the Table of Frequency Allocations of RR Article 5, add a footnote limiting radiolocation operations to automotive applications.  Sharing studies will assess whether mitigation measures such as emission power limits may be needed to avoid potential interference with RAS in the band.

The other consequential action under this method would be to suppress Resolution 654 (WRC-12) since it will become superfluous after the studies completed and the allocation to RLS adopted by WRC-15 Conference.

MOD

ARTICLE 5

Frequency allocations

76-81 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| … |  |  |
| 76-77.5 RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) 5.149 |  |  |
| 77.5-78 AMATEUR AMATEUR-SATELLITE RADIOLOCATION 5.XXX Radio astronomy Space research (space-to-Earth) 5.149 |  |  |
| 78-79 RADIOLOCATION Amateur Amateur-satellite Radio astronomy Space research (space-to-Earth) 5.149 5.560 |  |  |
| 79-81 RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) 5.149 |  |  |

5.560	In the band 78-79 GHz radars located on space stations may be operated on a primary basis in the Earth exploration-satellite service and in the space research service.

5.149	In making assignments to stations of other services to which the band [76-86 GHz] are allocated, administrations are urged to take all practicable steps to protect the radio astronomy service from harmful interference. Emissions from space borne or airborne stations can be particularly serious sources of interference to the radio astronomy service (see Nos. 4.5 and 4.6 and Article 29).     (WRC-07)

ADD	5.XXX

The use of the band 77.5-78 GHz by the radiolocation service is limited to automotive radar applications.

Note:  Sharing studies will assess whether emission power limits are needed to avoid potential interference to existing services. If studies conclude that emission power limits are needed, it would be necessary to add appropriate text referring to the system characteristics including power limits. It is anticipated that automotive radars will standardize their system and operational characteristics in order to harmonize operations on a world-wide basis in the band. WP 5A is developing a Recommendation ITU-R M.[AUTO] which describes the system and operational characteristics including power limits for automotive radars operating in the 76-81 GHz band.

Note, the information above could be taken as input for the first Draft ECP

# List of relevant documents

ITU-Recommendations:

ITU-R M.1452-2 – Millimetre wave vehicular collision avoidance radars and radiocommunication systems for intelligent transport system applications.

ITU-Reports:

ITU-R SM. 2057 - Studies related to the impact of devices using ultra-wideband technology on radiocommunication services

Further ITU documents:

Annex 23 of Doc. 5B/167: Working document towards a preliminary draft new Report ITU-R M.[AUTOMOTIVE RADARS] “Systems characteristics and compatibility of automotive radars operating in the 77.5-78 GHz band for sharing studies”

Annex 9 of Doc. 5B/167: Working document towards DRAFT CPM text “Chapter 3. Aeronautical, maritime and radiolocation issues. AGENDA ITEM 1.18”

Annex 10 of Doc. 5B/167: “Working document towards work plan for WRC-15. AGENDA ITEM 1.18”

Annex 21 of Doc. 5A/198 : Working document towards a preliminary draft new Recommendation ITU-R M.[AUTO] - Systems characteristics of automotive radars operating in the frequency band 76-81 GHz for intelligent transport systems applications

Liaison Statement from WP5A to WP5B (Document 5A/TEMP/56) Information on amateur service systems at 77.5 – 78 GHz

CEPT and/or ECC Documentation (Decisions, Recommendations, Reports):

ECC Report 56, “Compatibility of automative collision warning short range radar operating at 79 GHz with radiocommunication services”, Stockholm, October 2004.

ERC Recommendation 70-03 (9th October 2012) Annex 5 RTTT

ETSI standard EN 302 264, Electromagnetic compatibility and Radio spectrum Matters (ERM); Short Range Devices; Road Transport and Traffic Telematics (RTTT); Short Range Radar equipment operating in the 77 GHz to 81 GHz band, 2009, which is under revision (LS from ETSI to WP5A, Document 5A/159-E)

EU Documentation (Directives, Decisions, Recommendations, other), if applicable:

2004/545/EC: Commission Decision of 8 July 2004 on the harmonisation of radio spectrum in the

79 GHz range for the use of automotive short-range radar equipment in the Community (notified

under document number C(2004) 2591)(Text with EEA relevance)

Official Journal L 241 , 13/07/2004 P. 0066 – 0067

# Actions to be taken

## Sharing studies:

To estimate the impact of automotive applications within the RLS in the band 77.5 – 78.0 GHz which may use parameters provided in the PDN Rec ITU-R M [AUTO] with regard to the Amateur and Amateur Satellite Services (primary allocation) and Radio Astronomy and Space Research Services (secondary allocation).

# Relevant information from outside CEPT (examples of these are below)

## European Union (date of proposal)

## Regional telecommunication organisations:

APT (date of proposal)

ATU (date of proposal)

Arab Group (date of proposal)

CITEL (22 - 26 October 2012)

Preliminary views:

U.S. 
The United States supports ITU-R sharing, compatibility and regulatory studies between vehicular radars and all services that operate in the 76-81 GHz region of the spectrum. Based on the outcome of those studies, the United States will consider supporting a primary allocation to the radiolocation service in the 77.5-78 GHz band for automotive radars.

CAN
Canada will support a primary allocation to the radiolocation service in the frequency band 77.5-78 GHz for automotive short-range high-resolution radars if ITU-R studies show sharing is feasible with existing incumbent services in the band and in the adjacent bands 76-77.5 GHz and 78-81 GHz and fixed service incumbents operating in the bands 71-76 GHz and 81-86 GHz are not impacted upon a positive outcome of the agenda item.

RCC (date of proposal)

## International organisations

IATA (date of proposal)

ICAO (28 November 2012)

Preliminary Position

No impact on aeronautical services has been identified.

IMO (date of proposal)

NATO (date of proposal)

SFCG (date of proposal)

WMO and EUMETNET (date of proposal)

## Regional organisations

ESA (date of proposal)

Eurocontrol (date of proposal)