|  |  | CPG15(13)009 Annex 2 |
| --- | --- | --- |
| CPG-15 |  |  |
| Draft brief for WRC 15 agenda item 1.2 |  |  |

ABLE OF CONTENTS

DRAFT CEPT BRIEF ON AGENDA ITEM 1.2

1.2	to examine the results of ITU R studies, in accordance with Resolution 232 (WRC-12), on the use of the frequency band 694-790 MHz by the mobile, except aeronautical mobile, service in Region 1 and take the appropriate measures;

# ISSUE

WRC-15 added No. 5.312A into the Table of Frequency Allocations. This footnote states that in Region 1, the use of the band 694-790 MHz by the mobile, except aeronautical mobile, service is subject to the provisions of Resolution 232 (WRC-12). See also Resolution 224 (Rev. WRC-12).

Resolution 232 resolves

1	to allocate the frequency band 694-790 MHz in Region 1 to the mobile, except aeronautical mobile, service on a co-primary basis with other services to which this band is allocated on a primary basis and to identify it for IMT;

2	that the allocation in resolves 1 is effective immediately after WRC-15;

3	that use of the allocation in resolves 1 is subject to agreement obtained under No. 9.21 with respect to the aeronautical radionavigation service in countries listed in No. 5.312;

4	that the lower edge of the allocation is subject to refinement at WRC-15, taking into account the ITU-R studies referred to in invites ITU-R below and the needs of countries in Region 1, in particular developing countries;

5	that WRC-15 will specify the technical and regulatory conditions applicable to the mobile service allocation referred to in resolves 1, taking into account the ITU-R studies referred to in invites ITU-R below,

and invites ITU-R

1	to study the spectrum requirement for the mobile service and for the broadcasting service in this frequency band, in order to determine as early as possible the options for the lower edge referred to in resolves 4;

2	to study the channelling arrangements for the mobile service, adapted to the frequency band below 790 MHz, taking into account:

–	the existing arrangements in Region 1 in the bands between 790 and 862 MHz and defined in the last version of Recommendation ITU-R M.1036, in order to ensure coexistence with the networks operated in the new allocation and the operational networks in the band 790-862 MHz,

–	the desire for harmonization with arrangements across all Regions,

–	the compatibility with other primary services to which the band is allocated, including in adjacent bands;

3	to study coexistence between the different channelling arrangements which have been implemented in Region 1 above 790 MHz, as well as the possibility of further harmonization;

4	to study the compatibility between the mobile service and other services currently allocated in the frequency band 694-790 MHz and develop ITU-R Recommendations or Reports;

5	to study solutions for accommodating applications ancillary to broadcasting requirements;

6	to report, in time for WRC 15, the results of these studies,

# Preliminary CEPT position

CEPT will support studies according to Resolution 232.

## Spectrum requirements in thIS BAND (694-790 MHz)

TBD

## options for refinement of the lower edge of the allocation in resolves 1 of Resolution 232 (WRC-12)

### General considerations

The development of the options for refinement of the lower edge of the allocation in resolves 1 of Resolution 232 (WRC-12) will need to be based on either one or, most likely, a combination of the following approaches; reference should also be made to No. 5.312A:

The lower edge is set by taking into account a trade-off between the spectrum requirements for the mobile service (e.g. JTG 4-5-6-7/50) and for the broadcasting service (e.g. JTG 4-5-6-7/54) in different countries

The lower edge is set taking into account the studies of the channelling arrangements for the mobile service in the band in question;

The lower edge is set taking into account the results of sharing and compatibility studies ensuring coexistence of services;

The lower edge is set on the basis of the needs of CEPT countries.

Furthermore, the CEPT notes the importance of global harmonisation to foster economies of scale.

### Options

[Options as developed by CEPT to be listed: see also Annex 6 to minutes of CPG PTD #2]

These are suggestions that have been included in inputs to CPG PTD, that need a full set of technical sharing studies before they can be agreed in CEPT.

Lower edge of the mobile allocation: 694 MHz. Mobile transmissions start at 698 MHz

Lower edge of the mobile allocation: 694 MHz. Mobile transmissions start at 703 MHz

Lower edge of the mobile allocation: 698 MHz. Mobile transmissions start at 703 MHz

TBD, taking into account the current proposed channelling arrangements and further inputs that may be received, other options may be further considered.

- One criterion in relation to these options is protection of DTT channel 48.

## Channelling arrangements

TBD

## sharing studies

TBD

# Background

During the Study Period 2008-2012, ITU-R studied compatibility between the mobile service and other radiocommunication services allocated in the 790-862 MHz band under WRC-12 agenda item 1.17. The outcome of that agenda item was a revision of Resolution 749 (Rev. WRC-12). In response to proposals into WRC-12 from administrations, that Conference additionally developed Resolution 232 (WRC-12) which covers the 694-790 MHz band, with possible refinement of the lower frequency boundary at WRC-15.

The Table of Frequency Allocations has the following allocations and footnotes in Region 1 in the range 694-790 MHz:

- Primary allocation to the broadcasting service

- 5.296	Additional allocation:  in Albania, Germany, Saudi Arabia, Austria, Bahrain, Belgium, Benin, Bosnia and Herzegovina, Burkina Faso, Cameroon, Congo (Rep. of the), Côte d'Ivoire, Croatia, Denmark, Djibouti, Egypt, United Arab Emirates, Spain, Estonia, Finland, France, Gabon, Ghana, Iraq, Ireland, Iceland, Israel, Italy, Jordan, Kuwait, Latvia, The Former Yugoslav Republic of Macedonia, Libya, Liechtenstein, Lithuania, Luxembourg, Mali, Malta, Morocco, Moldova, Monaco, Niger, Norway, Oman, the Netherlands, Poland, Portugal, Qatar, the Syrian Arab Republic, Slovakia, the Czech Republic, the United Kingdom, Sudan, Sweden, Switzerland, Swaziland, Chad, Togo, Tunisia and Turkey, the band 470-790 MHz, and in Angola, Botswana, Lesotho, Malawi, Mauritius, Mozambique, Namibia, Nigeria, South Africa, Tanzania, Zambia and Zimbabwe, the band 470-698 MHz are also allocated on a secondary basis to the land mobile service, intended for applications ancillary to broadcasting. Stations of the land mobile service in the countries listed in this footnote shall not cause harmful interference to existing or planned stations operating in accordance with the Table in countries other than those listed in this footnote.     (WRC-12)

- 5.300	Additional allocation:  in Saudi Arabia, Cameroon, Egypt, United Arab Emirates, Israel, Jordan, Libya, Oman, Qatar, the Syrian Arab Republic, Sudan and South Sudan, the band 582-790 MHz is also allocated to the fixed and mobile, except aeronautical mobile, services on a secondary basis.     (WRC-12)

- 5.311A	For the frequency band 620-790 MHz, see also Resolution 549 (WRC-07).     (WRC-07)

- 5.312	Additional allocation:  in Armenia, Azerbaijan, Belarus, the Russian Federation, Georgia, Kazakhstan, Uzbekistan, Kyrgyzstan, Tajikistan, Turkmenistan and Ukraine, the band 645-862 MHz, in Bulgaria the bands 646-686 MHz, 726-758 MHz, 766-814 MHz and 822-862 MHz, in Romania the band 830-862 MHz, and in Poland, the band 830-860 MHz until 31 December 2012 and the band 860-862 MHz until 31 December 2017, are also allocated to the aeronautical radionavigation service on a primary basis.     (WRC-12)

- 5.312A	In Region 1, the use of the band 694-790 MHz by the mobile, except aeronautical mobile, service is subject to the provisions of Resolution 232 (WRC-12). See also Resolution 224 (Rev. WRC-12). (WRC-12)

The 700 MHz frequency band allows cost effective implementation of IMT for countries in particular those with large areas and with low population density.

Terrestrial broadcasting service provides a cost-efficient solution for high-quality media delivery [including] [in particular] to population in rural areas.

[The use of the 700 MHz frequency band for broadcasting varies considerably across Region 1 countries. Many European countries make extensive use of the 700 MHz band for terrestrial broadcasting at the present. Making the 700 MHz band available for other services would require extensive replanning activities].

[However, many European countries are also using other delivery mechanisms for broadcasting content. There are also other delivery mechanisms for broadband.]

CEPT is of the view that studies conducted within ECC should be aimed at searching for the most effective and available solution to provide broadcasting and mobile services, [which could be achieved by a balanced joined use of several delivery mechanisms]. Studies are required onto the most effective delivery mechanisms for different environments.

It is important to take into account future development of the broadcasting and mobile services. It should be noted that both services can deliver digital content to the end user in all environments. [When identifying the requirements for broadcasting and mobile services in radio spectrum it is advisable to take into account that some types of content would be more efficiently delivered via broadcasting service in rural areas, and some types of content – via mobile service. When identifying the requirements for broadcasting service, it is advisable to avoid taking into account the resource for the organization of interactive channels in areas which will be covered by mobile networks with broadband data transmission capabilities. When identifying the requirements for mobile communications, it is advisable to avoid taking into account resources for the transmission of mass media content which would be more efficiently delivered via broadcasting service] Sufficient radio spectrum should be available for the development of different services that may use different radio technologies. In this respect, it would be useful to develop a European long-term strategy for UHF band, beyond WRC-15.

The potential use of 700 MHz, among other bands, for PPDR is discussed within CEPT FM 49; PPDR is also under agenda item 1.3 which is the responsibility of CPG PTA.

# List of relevant documents

## ITU-R Circular letter

Administrative Circular CA/201 (Annex 10): CPM15-1 Decision on the Establishment and Terms of Reference of Joint Task Group 4-5-6-7

## ITU-R Recommendations and Reports

Aeronautical radionavigation service:

Recommendation ITU-R M.1830: Technical characteristics and protection criteria of aeronautical radionavigation service systems in the 645-862 MHz frequency band

Broadcasting service:

Recommendation ITU-R BT.1368: Planning criteria, including protection ratios, for digital terrestrial television services in the VHF/UHF bands

Recommendation ITU-R BT.1895: Protection criteria for terrestrial broadcasting systems

Report ITU-R BT.2215: Measurements of protection ratios and overload thresholds for broadcast TV receivers

Mobile service:

Recommendation ITU-R M.1036-4: Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations (RR)

Report ITU-R M.2039: Characteristics of terrestrial IMT-2000 systems for frequency sharing/interference analyses

## CEPT and/or ECC Documentation (Decisions, Recommendations, Reports)

None identified at present.

## EU Documentation (Directives, Decisions, Recommendations, other), if applicable

None identified at present.

# Actions to be taken in CEPT

Develop positions and, if necessary, conduct studies in relation to the following tasks:

  - Mobile service

    - Study the channelling arrangements for the mobile service, adapted to the frequency band below 790 MHz according to invites ITU-R 2 in Resolution 232

    - Develop options for the refinement of the lower edge of the band 694-790 MHz in Region 1, taking into account the spectrum requirements of the mobile service and broadcasting service in CEPT

  - Broadcasting service:

    - Study the impact of channelling arrangements for the mobile service on the broadcasting service in adjacent spectrum

    - Study sharing and compatibility between the mobile service in Region 1 and the broadcasting service in the GE06 planning area in the frequency band 694-790 MHz, taking into account the assumptions on channelling arrangements provided by Working Party 5D and the impact of systems using these channelling arrangements on the broadcasting service in adjacent frequency bands

  - Aeronautical radionavigation service:

    - Study the compatibility between the mobile service and the aeronautical radionavigation service allocated in the frequency band 694-790 MHz

  - Applications ancillary to broadcasting:

    - Study solutions for accommodating applications ancillary to broadcasting requirements in Region 1

Contribute to ITU-R JTG 4-5-6-7 and WP 5D, and ensure that the CEPT position is adequately reflected in the draft CPM text

# Relevant information from outside CEPT

## European Union (date of proposal)

    - No information received

## Regional telecommunication organisations:

APT (date of proposal)

ATU (date of proposal)

    - The ITU has done substantial work for sub-Saharan Africa for TV services to fit in 470-694 MHz.

    - [Editor’s note: add a reference to BR presentation on this subject, which is on the WP 6A Sharepoint site.]

Arab Group (date of proposal)

CITEL (date of proposal)

RCC (date of proposal)

## International organisations

IATA (date of proposal)

ICAO (date of proposal)

IMO (date of proposal)

NATO (date of proposal)

SFCG (date of proposal)

WMO and EUMETNET (date of proposal)

## Regional organisations

ESA (date of proposal)

EUMETNET (date of proposal)

Eurocontrol (date of proposal)