CPG(13)010 Annex 12

DRAFT CEPT BRIEF ON AGENDA ITEM 1.9

1.9	to consider, in accordance with Resolution 758 (WRC-12):

1.9.1	possible new allocations to the fixed-satellite service in the frequency bands 7 150-7 250 MHz (space-to-Earth) and 8 400-8 500 MHz (Earth-to-space), subject to appropriate sharing conditions;

1.9.2	the possibility of allocating the bands 7 375-7 750 MHz and 8 025-8 400 MHz to the maritime-mobile satellite service and additional regulatory measures, depending on the results of appropriate studies;

# ISSUE

“to conduct technical and regulatory studies on the possible new allocations to the FSS in the frequency bands 7 150-7 250 MHz (space-to-Earth) and 8 400-8 500 MHz (Earth-to-space) in order to ensure compatibility with existing services, with a view to extending the current worldwide allocation to the FSS in the bands 7 250-7 750 MHz (space-to-Earth) and 7 900-8 400 MHz (Earth-to-space);

to conduct the appropriate regulatory studies to ensure that any new FSS allocation referred to in resolves 1 above is limited to FSS systems operated from a fixed known location in order to enable compatibility with systems of other services, taking into account that the operational requirements in the bands 7 150-7 250 MHz (space-to-Earth) and 8 400-8 500 MHz (Earth-to-space) do not encompass small VSAT-like FSS earth stations;

to conduct technical and regulatory studies on the possibility of allocating the bands 7 375-7 750 MHz (space-to-Earth) and 8 025-8 400 MHz (Earth-to-space), or parts thereof, to the maritime-mobile satellite service, while ensuring compatibility with existing services”

# PRELIMINARY CEPT position

CEPT supports the on-going ITU-R studies with a view of making a new allocation to the FSS in the bands 7 150-7 250 MHz (space-to-Earth) and 8 400-8 500 MHz (Earth-to-space) and a new allocation to the MMSS in the bands 7 375-7 750 MHz (space-to-Earth) and 8 025-8 400 MHz (Earth-to-space), condition to the fact that not putting undue constraints to and to ensure protection of the services already allocated in these frequency bands.

Regarding 1.9.2, CEPT is also of the opinion that a more detailed understanding of the requirements/justification and technical parameters of maritime-mobile satellite service (MMSS) are required before it can give a preliminary view on the CEPT position.

# Background

The frequency bands 7 250-7 750 MHz (space-to-Earth) and 7 900-8 400 MHz (Earth-to-space) are currently allocated worldwide to the fixed-satellite service (FSS) and the frequency bands 7 250-7 375 MHz (space-to-Earth) and 7 900-8 025 MHz (Earth-to-space) are also allocated to the mobile-satellite service on a primary basis, subject to agreement obtained under No 9.21 (see No. 5.461). Regarding the FSS and maritime-mobile-satellite service (MMSS), some administrations have reported a shortfall of spectrum available for their current and future applications in these bands. FSS additional bandwidth requirements for data transmission on the next-generation satellites are estimated to be around a maximum of 100 MHz.

WRC-15 Agenda Item 1.9 and Resolution 758 (WRC-12) invite ITU-R to conduct technical and regulatory studies on a possible new allocations to the FSS in the frequency bands 7 150-7 250 MHz (s-E) and 8 400-8 500 MHz (E-s) and to the MMSS in the frequency bands 7 375-7 750 MHz (s-E) and 8 025-8 400 MHz (E-s).

Working Parties 4A and 4C have been identified by the Conference Preparatory Meeting as the responsible ITU-R groups for the studies on WRC-15 agenda item 1.9.1 and 1.9.2 respectively. At their first meetings, these groups sent liaison statements to gather technical characteristics and protection criteria needed to perform the appropriate sharing studies with existing services. They also established work plans for the band-by-band studies as well as for the development of the draft CPM text.

# List of relevant documents

WRC-15 agenda item 1.9.1: Documents 4A/54 (Liaison statement from WP 5C), 61 (Annex 22) (work plan)

WRC-15 agenda item 1.9.2: Documents 4C/39 (Annex 5) (work plan), 41 (Liaison statement from 5C), 47 (Liaison statement from 5A), 55 (Liaison statement from 4A). Recommendation ITU-R SA.1277 and SA.1027

# Actions to be taken

## Analyse the potential the potential for sharing between the FSS (s-E) and the EESS (E-s) considering WRC15 AI 1.11 proposal for an allocation to the EESS (E-s) in the 7-8 GHz range.

## In order to facilitate the technical studies and in support of ITU-R studies, WP4C and WP4A sent liaison statements to WP4A, 5A, 5B, 5C, and 7B in order to obtain the corresponding technical characteristics (power, antenna gain) as well as the MMSS and the FSS characteristics and the degree of deployment (density of systems per km2 per country, region and/or any other adequate representative figure) of the corresponding services in the corresponding frequency bands. Administrations are kindly requested to provide the corresponding characteristics.

Regarding 1.9.1, CEPT also proposes to:

Study uplinks at 8 GHz

- Study FSS protection distance needs of 8 GHz Fixed and Mobile Services.

- Study FSS distance to protect SRS Earth stations.

Study downlinks at 7 GHz

- Study and confirm compatibility of FSS to Fixed and Mobile Services.

- Study and confirm compatibility of FSS to Space Research Service.

Regarding 1.9.2, CEPT also proposes to:

Study uplinks at 8GHz

- Study MMSS protection distance needs of 8 GHz Fixed and Mobile services, perhaps adapting the solutions of  Res 902 6GHz FSS ESVs as an example framework

- Study MMSS distance to protect EESS Earth stations – considering ITU-R SA.1277

- Study the feasibility of MMSS protection to MetSat.

Recommendation ITU-R SA.1277 has information on the required EESS protection distances from MetSat FS FSS and mobile stations. CEPT needs to confirm results for similar studies for the required protection distances from MMSS Earth stations. Document PTB(2012)010rev1 presents results of such studies with separation distances in the order of 400 km

CEPT needs the MMSS earth station and satellite parameters (Tx, Gain, Minimum elevation angle, BW etc.).

Study downlinks at 7GHz

- Study the feasibility of the Maritime Mobile Satellite Service 7375-7750 MHz using the same PFD as FSS to protect FS and MS.

- Study and confirm compatibility of MMSS to METSAT.

- Study and confirm on compatibility of FS to MMSS that as FS max power of +55dBW is in RR   –  MMSS to not claim protection from FS

- Study and confirm on compatibility of MS to MMSS that as MS max power of +55dBW  is in RR – MMSS to not claim protection from MS

# Relevant information from outside CEPT

## European Union (date of proposal)

## Regional telecommunication organisations:

APT (date of proposal)

ATU (date of proposal)

Arab Group (date of proposal)

CITEL (date of proposal)

RCC (date of proposal)

## International organisations

IATA (date of proposal)

ICAO (date of proposal)

IMO (date of proposal)

NATO (date of proposal)

SFCG (date of proposal)

WMO and EUMETNET (date of proposal)

## Regional organisations

ESA (date of proposal)

EUMETNET (date of proposal)

Eurocontrol (date of proposal)