CPG(13)010 Annex 12

DRAFT CEPT BRIEF ON AGENDA ITEM 1.10

1.10	to consider spectrum requirements and possible additional spectrum allocations for the mobile-satellite service in the Earth-to-space and space-to-Earth directions, including the satellite component for broadband applications, including International Mobile Telecommunications (IMT), within the frequency range from 22 GHz to 26 GHz, in accordance with Resolution 234 (WRC-12);

# ISSUE

“to complete, for WRC 15, sharing and compatibility studies towards additional allocations to the mobile-satellite service in the Earth-to-space and space-to-Earth directions, within portions of the bands between 22 GHz and 26  GHz, while ensuring protection of existing services within these bands as well as taking into account No. 5.340 and No. 5.149”.

# Preliminary CEPT position

CEPT sees some difficulties for allocations within these bands and is unlikely to support additional allocations. At present CEPT does not see a need for additional spectrum requirements and therefore CEPT sees a need for justification for possible spectrum allocations for the MSS in the band 22-26 GHz.

# Background

During the study period 2003-2007 ITU-R has studied traffic forecasts for the satellite component of IMT for the time period 2010-2020.  Two scenarios, a low traffic and a high traffic scenario in the frequency range 1-6 GHz had been studied.  The results of the studies are contained in Report ITU-R M.2077 and provided the technical basis for related spectrum requirements identified in the CPM-Report on WRC-07 Agenda Item 1.4.  The Report indicates a shortfall of spectrum available for the satellite component of IMT in the Earth-to-space direction of between 19 and 90 MHz and in the space-to-Earth direction of between 144 and 257 MHz for the year 2020, depending on the traffic scenario.

The above frequency range has been indicated as a preferred range for the satellite component of IMT with a maximum data rate of 144 kbit/s.  Nowadays, data rates of this order can be considered modest because current terrestrial mobile systems using 3G technologies (such as HSDPA) are providing data rates of up to 7.2 Mbit/s (download) per user.  Higher data rates are likely to be introduced in the future, particularly when terrestrial IMT-Advanced systems are deployed.

Current mobile earth stations (MESs) in this frequency range are able to provide data rates of between about 64 kbit/s and 500 kbit/s and make use of directional antennas between about 15 cm and 80 cm in diameter.  These services are available to ships, aircraft and land vehicles.

The provision of additional frequencies for the MSS in bands between 22-26 GHz would allow, over time, a migration of some classes of MES currently operating in the 1-3 GHz range to these new bands.  This would create some additional capacity in the 1-3 GHz range to accommodate the predicted growth in handheld type devices which are more difficult to accommodate in higher frequency bands. In order to reach small terminals MSS requires high pfd levels and regulatory measures need to be developed to ensure the possibility of coordinating the use of such pfd levels.

The frequency band from 22-26 GHz is allocated to a large number of radio services on primary and secondary basis with numerous major utilisations within CEPT, as given in ERC Report 25. Regarding the allocations to radio services on a primary basis the following services can be found in the Table of Allocations of Article 5 of the Radio Regulations as amended at WRC-15 (see Table 1):

Table 1

| Frequency Band | Radio Service |
| --- | --- |
| 24-24.05 GHz | Amateur |
| 24-24.05 GHz | Amateur-Satellite |
| 22.21-22.5 GHz (passive), 23.6-24 GHz (passive), 25.5-27 GHz (s-E) | Earth Exploration-Satellite |
| 22-23.6 GHz, 24.25-25.25 GHz (Regions 1 and 3), 25.25-27 GHz | Fixed |
| 24.65-24.75 GHz (E-s) (Regions 1 and 3); 24.75-25.25 GHz (E-s) | Fixed-Satellite (Earth-to-space) |
| 22.55-23.55 GHz, 24.45-24.75 GHz, 25.5-27 GHz | Inter-Satellite |
| 22-23.6 GHz; 24.25-25.25 GHz (Region 3), 25.25-27 GHz | Mobile |
| 22.21-22.5 GHz, 23.6-24 GHz | Radio Astronomy |
| 24.05 24.25 GHz, 24.25-24.65 GHz (Regions 2 and 3) | Radiolocation |
| 24.65-24.75 (E-s) (Region 2) | Radiolocation-Satellite |
| 22.21-22.5 GHz (passive), 22.55-23.15 GHz (E-s), 23.6-24 GHz (passive), 25.5 -27 GHz (s-E) | Space Research |

For radio services on a secondary basis the Table of Allocations in Article 5 provides the following entries (see Table 2):

Table 2

| Frequency Band | Radio Service |
| --- | --- |
| 24.05-24.25 GHz | Amateur |
| 24.05-24.25 GHz (active) | Earth Exploration-Satellite |
| 25.25-27 GHz (E-s) | Standard Frequency and Time Signal-Satellite |

In addition, it is stated in the Radio Regulations that in making assignments to stations of other services to which the bands 22.01-22.21 GHz, 22.21-22.5 GHz, 22.81-22.86 GHz and 23.07-23.12 GHz are allocated, administrations are urged to take all practicable steps to protect the radio astronomy service from harmful interference. Emissions from spaceborne or airborne stations can be particularly serious sources of interference to the radio astronomy service. (see provision 5.149)

In accordance with the NATO Joint civil/military Frequency Agreement 2002 as of 22.08.2005, the frequency band 24.05-24.25 GHz is used for radiolocation purposes and in the frequency band 25.25-27.5 GHz there is a military requirement for planned fixed and mobile systems.

Estimated future spectrum requirements in the 22-26 GHz range

[TBD]

General sharing considerations

For the protection of the incumbent services it is necessary to carry out sharing and compatibility studies with characteristics of potential MSS systems to facilitate the identification of frequency bands in the range 22-26 GHz suitable for additional allocations for the MSS.

When identifying candidate bands for MSS up- and downlinks the following issues may need to be taken into consideration:

Within CEPT a number of ECC Decision, Recommendations and Reports have been developed to harmonise the use of a wide range of applications in the frequency range 22-26 GHz. Furthermore, for the Member States of the European Union a number of Commission Decisions and CEPT Reports have been developed. The relevant documents can be found in the section list of relevant documents.

Sharing with the Amateur Service

Although only low activity in the amateur service in the frequency bands 24-24.05 GHz and 24.05-24.25 GHz could be observed, there are characteristics of amateur systems for Morse on-off keying in Table 1 and characteristics of amateur analogue voice systems in Table 3 of Recommendation ITU-R M.1732 available which can be used in sharing and compatibility studies. It should be noted that the latter frequency band is allocated to the amateur service on a secondary basis.

In Region 1, and globally for Earth-Moon-Earth communication, the frequency 24.0481 GHz is used as the narrow-band calling frequency. In the United States of America the frequency 24.1921 GHz is used as the narrow-band calling frequency.

Sharing with the Amateur-Satellite Service

The frequency band 24-24.05 GHz is allocated to the amateur-satellite service on a primary basis. No activity in this service could be observed in the frequency band 24-24.05 GHz. However, characteristics of amateur-satellite systems in the Earth-to-space direction for different modes of operation can be found in Tables 5 and 6 of Recommendation ITU-R M.1732 which can be used in sharing and compatibility studies. A satellite project is planned in this band in the near future.

Sharing with the Earth Exploration-Satellite Service

The frequency bands 22.21-22.5 GHz and 23.6-24 GHz are allocated to the earth-exploration service (EESS) (passive). In the latter frequency band all emissions are prohibited (see provision 5.340 of the Radio Regulations). Both frequency bands are allocated on a primary basis.

Protection criteria for inter-orbit links in the EESS in the frequency band 25.5-27 GHz to be used in sharing studies can be can be found in Recommendation ITU R SA.1155.  Interference criteria for stations in the EESS in the frequency band 25.5-27 GHz (s-E) using spacecraft in the geostationary orbit are provided in Recommendation ITU-R SA.1160.

Furthermore, the frequency band 24.05-24.25 GHz is allocated to the earth-exploration service (active) on a secondary basis.

Sharing with the Fixed Service

The frequency bands 22-23.6 GHz (23 GHz band) and 24.25-27 GHz (26 GHz band), with the exception of the band 24.25-25.25 GHz in Regions 2, are allocated to the fixed service on a primary basis. Within ITU-R, a range of recommendations provide information on the characteristics and the protection criteria of the fixed service in the relevant frequency bands. Recommendation ITU-R F.758 contains the appropriate fixed service characteristics for sharing and compatibility studies between fixed service and other services. Recommendations ITU-R F.699, ITU-R F.1245, and ITU-R F.1336 provide fixed system antenna patterns to be used in relevant studies. Recommendations ITU-R F.637 and ITU-R F.748 provide the requisite channelling arrangements for fixed systems between 22-26 GHz.

ERC Technical Recommendation T/R 13-2 recommends a range of channel arrangements for the harmonised use frequencies by paired radio-relay systems with different carrier spacing in the frequency bands 22.0-22.6 GHz and 23.0-23.6 GHz, including those making use of frequencies in the centre-gap in the frequency bands 22.6-22.75 GHz and 22.84-23.0 GHz, as well as unpaired radio-relay systems in the frequency band 22.75-22.84 GHz.

In Europe, but also in a number of countries outside Europe, the frequency bands are extensively used by fixed links to provide the infrastructure requirements for existing 2G and 3G mobile networks and to develop broadband fixed wireless networks. It can be seen from ECC Report 173 that in the frequency bands 22-23.6 GHz and 24.5-26.5 GHz there are many thousands of operational FS links across CEPT. These FS links would preclude operation of MSS earth stations across these bands.

Sharing with the Fixed-Satellite Service

The frequency bands 24.65-24.75 GHz in Regions 1 and 3 and 24.75-25.25 GHz in all Regions are allocated to the fixed-satellite service (Earth-to-space) on a primary basis for feeder links in the broadcasting-satellite service (BSS). They are commonly used to provide feeder links for the BSS networks in all Regions and for some multimedia systems in Region 2. Feeder links for BSS are characterized by a relatively small number of large earth stations in known locations. Earth station antennas in the frequency band 24.65-25.25 GHz in Region 1 and earth stations in the frequency band 24.65-24.75 GHz in Region 3 are limited to a minimum diameter of 4.5 m.

The following recommendations and reports are applicable to the FSS in these bands:

- Recommendation ITU-R S.465 – Reference radiation pattern of earth station antennas in the fixed satellite service for use in coordination and interference assessment in the frequency range 2 to 31 GHz.

- Recommendation ITU-R S.1323 – Maximum permissible levels of interference in a satellite network (GSO/FSS; non-GSO/FSS; non-GSO/MSS feeder links) in the fixed satellite service caused by other co-directional FSS networks below 30 GHz.

- Recommendation ITU-R S.1432 – Apportionment of the allowable error performance degradations to fixed-satellite service (FSS) hypothetical reference digital paths arising from time invariant interference for systems operating below 30 GHz.

- Recommendation ITU-R S.1716 – Performance and availability objectives for fixed satellite service telemetry, tracking, and command systems.

- Report ITU-R BO.2071 – BSS system parameters between 17.3 GHz and 42.5 GHz and associated feeder links.

Sharing with the Inter-Satellite Service

The bands 22.55-23.55, 24.45-24.75 and 25.25-27.0 GHz are allocated to the inter-satellite service (ISS) on a primary basis.

The following recommendations and reports are relevant for the protection of the inter-satellite service in these bands:

- Recommendation ITU-R SA.1019 – Preferred frequency bands and transmission directions for data relay satellite systems.

- Recommendation ITU-R SA.1155 – Protection criteria related to the operation of data relay systems.

- Recommendation ITU-R SA.1276 – Orbital locations of data relay satellites to be protected from the emissions of fixed service systems operating in the band 25.25 27.5 GHz.

- Recommendation ITU-R SA.1414 – Characteristics of data relay satellite systems.

- Recommendation ITU-R SA.1743 – Maximum allowable degradation to radiocommunication links of the space research and space operation services arising from interference from emissions and radiations from other radio sources.

- Report ITU-R SA.2192 – Compatibility between the space research service (Earth to space) and the non-GSO-to-non-GSO systems on the inter-satellite service in the band 22.55 23.55 GHz.

- Recommendation ITU-R S.1899 – Protection criteria and interference assessment methods for non-GSO inter-satellite links in the 23.183-23.377 GHz band with respect to the space research service.

Sharing with the Mobile Service

The frequency ranges 22-23.6 GHz and 25.25-27 GHz are allocated to the mobile service on a primary basis in Regions 1 to 3, while the frequency range 24.25-25.25 GHz is allocated to the mobile service on a primary basis in Region 3 only. For the protection of the mobile service the following recommendations and reports are applicable:

- Recommendation ITU-R F.758 “System parameters and considerations in the development of criteria for sharing or compatibility between digital fixed wireless systems in the fixed service and systems in other services and other sources of interference”;

- Recommendation ITU-R F.1102 “Characteristics of fixed wireless systems operating in frequency bands above about 17 GHz”;

- Recommendation ITU-R F.1336 “Reference radiation patterns of omnidirectional, sectoral and other antennas in point-to multipoint systems for use in sharing studies in the frequency range from 1 GHz to about 70 GHz”;

- Recommendation ITU-R F.1404 “Minimum propagation attenuation due to atmospheric gases for use in frequency sharing studies between systems in the fixed service and systems in the broadcasting-satellite, mobile-satellite and space science services”;

- Recommendation ITU-R F.1704 “Characteristics of multipoint-to-multipoint fixed wireless systems with mesh network topology operating in frequency bands above about 17 GHz”;

- Report ITU-R F.2086 “Technical and operational characteristics and applications of broadband wireless access in the fixed service”.

Recommendation ERC/REC 25-10 identifies the frequency range 21.2-24.5 GHz with the preferred sub-bands 21.2-21.4 GHz, 22.6-23.0 GHz and 24.25-24.5 GHz for the use by cordless cameras.

Sharing with the Radio Astronomy Service

The frequency bands 22.21-22.5 GHz and 23.6-24 GHz are allocated on a primary basis to the radio astronomy service (RAS). Furthermore RR No. 5.149 urges administrations when making assignments to stations of others radio services in the frequency bands 22.01-22.21 GHz, 22.21-22.5 GHz, 22.81-22.86 GHz and 23.07-23.12 GHz to take all practicable steps to protect the RAS from harmful interference in these bands. Member States shall also recognise that no emissions are allowed in the frequency band 23.6-24 GHz (see RR 5.340).

The frequency band 22.21-22.5 GHz is used by the RAS for both continuum observations as well as spectroscopic line observations of the water molecule, whose spectroscopic band in this frequency range is one of the most important for radio astronomy (see Recommendation ITU-R RA.314, Table 40 and the List of Important Spectral Lines of the International Astronomical Union). The water molecule transitions in this band are observed using both single-dish and VLBI techniques.

Recommendation ITU-R RA.769 specifies the protection criteria for radio astronomical observations and gives threshold levels of detrimental interference for primary radio astronomy bands.

For the frequency bands 22.21-22.5 GHz and 23.6-24 GHz, the pfd threshold limit given in Recommendation ITU-R RA.769 for single-dish line observations made using a channel bandwidth (one of the spectrometer channels) of 250 kHz is –162 dB(W/m2) and -161 dB(W/m²), respectively. Pfd threshold limits of –146 dB(W/m2) and -147 dB(W/m²) are defined for single-dish continuum observations in these bands, across the entire 290 MHz bandwidth or 400 MHz, respectively.

VLBI observations, where signals from widely separated antennas are recorded and correlated after the observations, are much less susceptible to local interference. This is reflected in the threshold pfd level for VLBI observations in this band, –129 dB(W/m2), for a bandwidth of 250 kHz. However, within the footprint of a satellite there may be several radio astronomy stations that may experience the same interference.

For detrimental interference from non-GSO systems, the protection criteria and the relevant methodologies are described in Recommendations ITU-R RA.769 and ITU-R RA.1513, as well as in Recommendation ITU-R S.1586 for FSS systems and in Recommendation ITU-R M.1583 for MSS and RNSS systems.

The thresholds of detrimental interference levels to the RAS as defined and calculated in Recommendation ITU-R RA.769 are protection criteria above which radio astronomical data are degraded and may be eventually obliterated. In principle, under rather idealised circumstances, if these levels are very slightly exceeded then it may be possible to compensate at the radio astronomy observatory by increased observing time. In doing so, the channel capacity of the telescope is reduced, with a corresponding reduction in scientific throughput. If the level of interference, under the assumptions of Recommendation ITU-R RA.769 (e.g. antenna performance, etc.), becomes 10 dB or more above the Recommendation ITU-R RA.769 definition, then increased observing time will no longer be effective in ensuring that valid scientific data are provided to the scientist. The radio astronomy station will be unable to operate in an affected frequency band, and its ability to provide service will have been lost if no appropriate mitigation techniques can be applied.

Sharing with the Radiolocation Service

The frequency bands 24.05-24.25 GHz and 24.25-24.65 GHz in Regions 2 and 3 are allocated to the radiolocation service.

Sharing with the Radiolocation-Satellite Service

In Region 2 the frequency band 24.65-24.75 (Earth-to-space) is allocated the radiolocation-satellite service on a primary basis.

Sharing with the Space Research Service

The frequency bands 22.12-22.5 GHz and 23.6-24 GHz are allocated to the space research service (SRS)(passive) on a primary basis while the frequency band 22.55-23.15 GHz (Earth-to-space) and 25.5-27 GHz (space-to-Earth) are allocated to the SRS.

The allocations to the SRS (Earth-to-space) the frequency band 22.55-23.15 GHz and 25.5 -27 GHz (space-to-Earth) support various missions including referenced lunar missions, Lagrangian missions and other near-Earth space research missions. The number of SRS earth stations transmitting in this band will be small. Rather than building new SRS earth stations, upgrading selected existing SRS earth stations will predominate. The upgrade of earth stations depends on a number of factors, including the type of mission to be supported. The number of SRS earth station sites capable of supporting referenced lunar and/or L2 missions is not expected to exceed ten to fifteen on a global basis over the next few decades.

Sharing with the Standard Frequency and Time Signal-Satellite Service

The frequency band 25.25-27 GHz is allocated to the standard frequency and time signal-satellite on a secondary basis.

Specific applications operated in this range [to be moved to some other place in the text]

Within Europe the frequency range 21.65-26.65 GHz is assigned to short range radar (SRR) applications. The frequency range 21.65-24.25 GHz is available until 30 June 2013 while the frequency range 24.25-26.65 GHz is available until 01 January 2018. The latter date is extended by 4 years for automotive short range radars applications for which type-approval application has been submitted and has been granted before 01 January 2018. Furthermore, the frequency range 24.05-27 GHz is assigned to tank level probing radar applications.

Preliminary Conclusions on Sharing and Compatibility Issues

- MSS earth station transmissions would not be feasible in 22-23.6/24.5-26 GHz because of the extensive FS use within CEPT;

- Not feasible in 23.6-24 GHz because of the passive allocations and this band needs to be protected from out-of-band emissions

- MSS earth station transmissions would have to protect the radiolocation service (24.05-24.25 GHz  and it would not be compatible with fixed service (24.25-24.45 GHz)

- FSS Earth to Space transmissions and FS operate across 24.65-25.25 GHz, these preclude MSS at least for Earth to space transmissions.

- Extremely difficult compatibility issues, co-ordination and operation against NGSO Inter-satellite links 22.55-23.55, 24.55-24.75,  and 25.5-27 GHz

Within 22-26 GHz, no areas can be identified where MSS services could operate, without causing interference to current services or constraining their development. Noting that MSS would require separate up and downlink allocations, these links needing to be separated by several hundred megahertz and that previous ITU-R studies of MSS spectrum requirements identified several hundred megahertz requirements, which were unsatisfied by WRC12 Agenda item 1.25.

Therefore, considering the existing extensive use of the current allocations and the incompatibilities with current services, the likely hood for any practical MSS allocations in the bands 22-26 GHz, is very low. The initial assessment to CEPT is that it should not support MSS allocations in these bands but it will continue to review any new ITU-R studies from other regions to assess the potential impact on existing services within CEPT.

# List of relevant documents

## 4.1	ITU Documentation

4C/39		Report of the 9th Meeting of Working Party 4C

[ITU-R S.2223	Technical and operational requirements for GSO FSS earth stations on mobile platforms in bands from 17.3 to 30.0 GHz]

## 4.2	CEPT and/or ECC Documentation

CEPT Report 38	in response to the EC Permanent Mandate on the ”Annual update of the technical annex of the Commission Decision on the technical harmonisation of radio spectrum for use by short range devices” (as of 11 March 2011)

CEPT Report 37	Report from CEPT to the European Commission in response to Part 2 of the Mandate on “Automotive Short-Range Radar systems (SRR)” (as of 25 June 2010)

CEPT Report 36	Report from CEPT to the European Commission in response to Part 1 of the Mandate on ”Automotive Short Range Radar systems (SRR)” (as of 25 June 2010)

ECC/DEC/(11)02	on industrial Level Probing Radars (LPR) operating in frequency bands 6 - 8.5 GHz, 24.05 - 26.5 GHz, 57 - 64 GHz and 75 - 85 GHz (as of 11 March 2011);

ECC/DEC/(04)10	The frequency bands to be designated for the temporary introduction of Automotive Short Range Radars (SRR) (as of 1 June 2012);

ECC/REC/(11)01 Guidelines for assignment of frequency blocks for fixed wireless systems in the bands 24.5-26.5 GHz, 27.5-29.5 GHz AND 31.8-33.4 GHz

ERC/REC 25-10	Frequency ranges for the use of temporary terrestrial audio and video SAP/SAB links (Incl. ENG/OB);

ERC/REC 70-03	(Tromsø 1997 and subsequent amendments) Relating to the use of short range devices (SRD) (as of 07 May 2012)

REC T/R 13-02	Preferred channel arrangements for fixed service systems in the frequency range 22.0 - 29.5 GHz;

ECC Report 173	Fixed service in Europe; current use and future trends post 2011, March 2012

ERC Report 25	The European table of frequency allocations and applications in the frequency range 9 kHz to 3000 GHz (ECA Table), Lille 2011

FM38(2005)INFO2	NATO Joint civil/military Frequency Agreement (NJFA) 2002

## 4.3	EU Documentation

Commission Implementing Decision 2011/485/EU of 29 July 2011 amending Decision 2005/50/EC on the harmonisation of the 24 GHz range radio spectrum band for the time-limited use by automotive short-range radar equipment in the Community (OJ L 198, 30.07.2011, p. 71);

Commission Decision 2010/368/EU of 30 June 2010 amending Decision 2006/771/EC on harmonisation of the radio spectrum for use by short-range devices (OJ L 166, 01.07.2010, p. 33);

Commission Decision 2006/771/EC of 9 November 2006 on harmonisation of the radio spectrum for use by short-range devices (OJ L 312, 11.11.2006, p. 66);

Commission Decision 2005/50/EC of 17 January 2005 on the harmonisation of the 24 GHz range radio spectrum band for the time-limited use by automotive short-range radar equipment in the Community (OJ L 21, 25.01.2005, p. 15);

# Actions to be taken

- Justification for additional spectrum for the MSS in the frequency band 22-26 GHz;

- Identification of candidate bands, if necessary;

- Sharing studies to demonstrate the protection of incumbent services;

# Relevant information from outside CEPT

## European Union (date of proposal)

N/A

## Regional telecommunication organisations:

APT (date of proposal)

N/A

ATU (date of proposal)

N/A

Arab Group (date of proposal)

N/A

CITEL (date of proposal)

N/A

RCC (date of proposal)

N/A

## International organisations

IATA (date of proposal)

N/A

ICAO (date of proposal)

N/A

IMO (date of proposal)

N/A

NATO (date of proposal)

N/A

SFCG (date of proposal)

N/A

WMO and EUMETNET (date of proposal)

N/A

## Regional organisations

ESA (date of proposal)

N/A

EUMETNET (date of proposal)

N/A

Eurocontrol (date of proposal)

N/A

Comparable technical specifications to those given in this ECC Decision are given in EC Decision 2004/545/EC for 79 GHz and in EC Decision 2005/50/EC, amended by 2011/485/EU, for 24 GHz. EU Member States and, if so approved by the EEA Joint Committee, Iceland, Liechtenstein and Norway are obliged to implement these EC Decisions.