CPG(13)10 Annex 12

DRAFT CEPT BRIEF ON AGENDA ITEM 1.6

1.6	to consider possible additional primary allocations:

1.6.1	to the fixed-satellite service (Earth-to-space and space-to-Earth) of 250 MHz in the range between 10 GHz and 17 GHz in Region 1;

1.6.2	to the fixed-satellite service (Earth-to-space) of 250 MHz in Region 2 and 300 MHz in Region 3 within the range 13-17 GHz;

and review the regulatory provisions on the current allocations to the fixed-satellite service within each range, taking into account the results of ITU-R studies, in accordance with Resolutions 151 [COM6/4] (WRC-12) and 152 [COM6/5] (WRC-12), respectively;

Resolution 151 [COM6/4] (WRC-12)

Additional primary allocations to the fixed-satellite service in frequency bands between 10 GHz and 17 GHz in Region 1 (WRC-15 AI 1.6.1)

Resolution 152 [COM6/5] (WRC-12)

Additional primary allocations to the fixed-satellite service in the Earth-to-space direction in frequency bands between 13 GHz and 17 GHz in Region 2 and Region 3 (WRC-15 AI 1.6.2).

# ISSUE

Resolution 151 (WRC-12)

“The WRC-12, considering

a)	that the existing unplanned bands for the fixed-satellite service (FSS) in the 10-15 GHz range are extensively used for a large variety of applications, and these applications have triggered a rapid rise in the demand for this frequency range …

g)	that there is a need to resolve the shortage of spectrum in Region 1 and Regions 2 and 3 …, such that the rapid growth of spectrum demand in considering a) could be met and the limited spectrum resources could be used in an efficient and economical way in accordance with the principle of Article 44 of the ITU Constitution …,

resolves to complete, for WRC 15:

i)	studies of possible bands for a new primary allocation to the fixed-satellite service of 250 MHz in both directions in Region 1 within the bands 10-17 GHz, with particular focus on the frequency range that is contiguous (or near contiguous) to the existing fixed-satellite service allocations, taking into account sharing and compatibility studies, while protecting the existing primary services in the band(s); …”

# Preliminary CEPT position

TBD

# Background

Due to the rapid growth in demand for fixed-satellite services worldwide, WRC-12 has considered an issue on additional primary spectrum allocations to this service. WRC-12 adopted two new resolutions (Resolutions 151 (WRC-12) and 152 (WRC-12)) proposing to consider, as a matter of urgency, possible additional primary allocations to unplanned FSS to address the asymmetry in Earth-to-space and space-to-Earth FSS allocations in Regions 2 and 3, and also to consider possible additional primary allocations to the unplanned FSS (Earth-to-space and space-to-Earth) in Region 1.

It should be noted that a world-wide allocation to FSS has a significant advantage over a regional one. For example, unified FSS allocation in Regions 1 and 3 is important in terms of planning and designing of satellite networks, as well as achieving effective coverage area.

Also, in terms of improving spectral efficiency and convenience of communication system architecture, it is desirable that an additional spectrum for FSS be allocated in a continuous part of the spectrum that is contiguous to the existing fixed-satellite service allocations

A number of studies should be conducted for the purpose of possible allocation of additional spectrum of 250 MHz to the fixed-satellite service (Earth-to-space and space-to-Earth) in frequency bands between 10 and 17 GHz in Region 1 and of 250 MHz to the fixed-satellite service (Earth-to-space) in Region 2 and of 300 MHz in Region 3 within the range 13-17 GHz.

The General working methods for compatibility studies are given in section 5, and Timetable of PT B is given in the Annex to the document.

Frequency bands 10.5-10.6 GHz, 10.6-10.7 GHz, 13.25-13.4 GHz, 15.35-15.4 GHz and 15.4-15.7 GHz were considered in the previous study period (See ITU-R Reports  М.2221, М.2170, M.2229). Following protection criteria were used in the sharing studies mentioned above:

-	10.5-10.6 GHz - interference pfd level for RLS systems is required to be -146 dBW/m2 in 1 MHz;

-	10.6-10.7 GHz - interference pfd level for RAS systems is required to be -180 dBW/m2 in 1 MHz;

-	13.25-13.4 GHz - interference pfd level for aircraft RLS systems is required to be -134 dBW/m2 in 1 MHz (or -158 dBW/m2 in 4 kHz);

-	in the frequency band 15.35 – 15.4 GHz  interference at the RAS receiver input in 50 MHz shall not exceed -202 dBW (or -219 dBW/MHz).

# List of relevant documents

Doc. 4A/61, Annex 09 - Working document towards a preliminary draft new Report ITU-R S.[R2R3.FSS] - Assessment on use of spectrum in the 13-17 GHz band for the fixed-satellite service in Regions 2 and 3 (GSO), begins to provide material needed to support WRC-15 Agenda item 1.6.2, to consider possible additional primary allocations to the unplanned fixed-satellite service (FSS) Earth-to-space spectrum in the 13-17 GHz band in Region 2 and Region 3.

Doc. 4A/61, Annex 11 - Working document towards a preliminary draft new Report ITU-R S.[R1.FSS] - Assessment on use of spectrum in the 10-17 GHz band for the fixed-satellite service in Region 1 (GSO), begins to provide material needed to support WRC-15 Agenda item 1.6.1, to consider possible additional primary allocations to the unplanned fixed-satellite service (FSS) Earth-to-space and space-to-Earth spectrum in the 10-17 GHz band in Region 1.

Doc. 4A/61, Annex 19 - Working document - Work plan for WRC-15 Agenda item 1.6. This Work Plan would need to be revised and updated, as appropriate, at every Working Party 4A meeting.Doc PTB(2012)013 (France) – Proposed revision of the working document towards a preliminary draft new report ITU-R S.[R1.FSS],

Doc PTB(2012)014 (France) – Compatibility analysis between EESS (active) and FSS (Earth-to-space) within the frequency band 13.25-13.75 GHz.

# Actions to be taken

The following studies should be conducted for the purpose of possible allocation of additional spectrum of 250 MHz to the fixed-satellite service (Earth-to-space and space-to-Earth) in frequency bands between 10 and 17 GHz in Region 1, and of 250 MHz to the fixed-satellite service (Earth-to-space) in Region 2, and of 300 MHz in Region 3 within the range 13-17 GHz.

General list of necessary studies:

1)	contributions justifying the need for an additional FSS allocation in Region 1 within the range between 10 and 17 GHz.

Detailed study of the current allocations. Current allocations and use of each band by existing services shall be analyzed in the frequency range under the study, Annex 2.

2)	Results of the studies performed in previous study periods for the bands under the study shall be analyzed. Use of the available study materials would prevent from unnecessary duplication of study efforts.

3)	Technical characteristics of FSS systems which may operate in frequency bands 10-17 GHz and 13-17 GHz shall be identified.

4)	The technical characteristics of network/station in the services already having allocations in frequency bands 10-17 GHz and 13-17 GHz shall be identified and sharing and compatibility studies shall be conducted in order to ensure protection from interference to:

a)	the existing services already having allocations in the above-mentioned frequency bands from the FSS;

b)	the FSS from the existing services.

5)	Based on the results of the sharing and compatibility studies and contributed materials, candidate frequency bands may be identified for additional FSS allocations in the 10-17 GHz range in Region 1, and in 13-17 GHz range in Region 2 and Region 3 in accordance with WRC 15.1 Agenda item 1.6.

# Relevant information from outside CEPT

TBD