E OF CONTENTS

DRAFT CEPT BRIEF ON AGENDA ITEM 1.4 (WRC-15)

1.4	to consider possible new allocation to the amateur service on a secondary basis within the band        5 250-5 450 kHz in accordance with Resolution 649 (WRC-12)

# ISSUE

Resolution 649 (WRC-12) invites WRC-15 to consider, based on the results of the ITU-R studies, the possibility of making an allocation of an appropriate amount of spectrum, not necessarily contiguous, to the amateur service on a secondary basis within the band 5 250-5 450 kHz.

CEPT has identified the following elements, relevant for this agenda item:

to study spectrum requirements for a secondary allocation to the amateur service within the band 5 250-5 450 kHz;

to conduct sharing studies on the impact to other services currently allocated in the band referred to in invites ITU R 1 and in the adjacent bands;

# Preliminary CEPT position

TBD

# Background

WRC-12 adopted Agenda item 1.4 for WRC-15, to consider the possibility of making an allocation of an appropriate amount of spectrum, not necessarily contiguous, to the amateur service on a secondary basis within the band 5 250-5 450 kHz.

- This part of the spectrum is interesting to radio amateurs because it bridges the propagation gap between 3.8 and 7.0 MHz. Particularly at higher latitudes, there are many times when the maximum usable frequency (MUF) is below 7.0 MHz, but is too far above the next lowest amateur frequency band 3.5 – 3.8 MHz for communication to be supported in that band, using typical amateur antennas and power levels. As amateur communication increasingly uses digital rather than analogue modes of emission, intersymbol distortion caused by multipath propagation becomes a more important factor, requiring an operating frequency as near as possible to the MUF.

Propagation characteristics of this frequency band.

It is recognized that sky-wave propagation of as much as 10,000 km is possible on these frequencies, as per Recommendation ITU-R F.1795. Practical considerations of operating a link over such ranges depend on atmospheric noise and interference caused by stations that may be deployed several thousand kilometers from the given radio communication link.

Editors note: one administration provided preliminary results of compatibility studies between proposed amateur stations and systems in the fixed service.

Regulatory and procedural considerations.

For reference, the applicable portion of the Table of Frequency Allocations, including the modifications made at WRC-12, is provided below.

5 060-5 680 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 060-5 250 FIXED Mobile except aeronautical mobile 5.133 |  |  |
| 5 250-5 275 FIXED MOBILE except aeronautical mobile Radiolocation 5.132A 5.133A | 5 250-5 275 FIXED MOBILE except aeronautical mobile RADIOLOCATION 5.132A | 5 250-5 275 FIXED MOBILE except aeronautical mobile Radiolocation 5.132A |
| 5 275-5 450 FIXED MOBILE except aeronautical mobile |  |  |
| 5 450-5 480 FIXED AERONAUTICAL MOBILE (OR) LAND MOBILE | 5 450-5 480 AERONAUTICAL MOBILE (R) | 5 450-5 480 FIXED AERONAUTICAL MOBILE (OR) LAND MOBILE |
| 5 480-5 680 AERONAUTICAL MOBILE (R) 5.111 5.115 |  |  |

5.111	The carrier frequencies 2 182 kHz, 3 023 kHz, 5 680 kHz, 8 364 kHz and the frequencies 121.5 MHz, 156.525 MHz, 156.8 MHz and 243 MHz may also be used, in accordance with the procedures in force for terrestrial radiocommunication services, for search and rescue operations concerning manned space vehicles. The conditions for the use of the frequencies are prescribed in Article 31.

The same applies to the frequencies 10 003 kHz, 14 993 kHz and 19 993 kHz, but in each of these cases emissions must be confined in a band of ± 3 kHz about the frequency.     (WRC-07)

5.115	The carrier (reference) frequencies 3 023 kHz and 5 680 kHz may also be used, in accordance with Article 31, by stations of the maritime mobile service engaged in coordinated search and rescue operations.  (WRC 07)

5.132A	Stations in the radiolocation service shall not cause harmful interference to, or claim protection from, stations operating in the fixed or mobile services. Applications of the radiolocation service are limited to oceanographic radars operating in accordance with Resolution 612 (Rev.WRC 12). (WRC-12)

5.133	Different category of service:  in Armenia, Azerbaijan, Belarus, the Russian Federation, Georgia, Kazakhstan, Latvia, Lithuania, Moldova, Uzbekistan, Kyrgyzstan, Tajikistan, Turkmenistan and Ukraine, the allocation of the band 5 130-5 250 kHz to the mobile, except aeronautical mobile, service is on a primary basis (see No. 5.33). (WRC-12)

5.133A	Alternative allocation: in Armenia, Austria, Belarus, Moldova, Uzbekistan and Kyrgyzstan, the frequency bands 5 250-5 275 kHz and 26 200-26 350 kHz are allocated to the fixed and mobile, except aeronautical mobile, services on a primary basis. (WRC-12)

# List of relevant documents

ITU-Recommendations:

Recommendation ITU-R F.162-3 Use of directional transmitting antennas in the fixed service operating in bands below 30 MHz;

Recommendation ITU-R F.240-7 Signal-to-interference protection ratios for various classes of emission in the fixed service below about 30 MHz;

Recommendation ITU-R F.339-7 Bandwidths, signal-to-noise ratios and fading allowances in complete systems;

Recommendation ITU-R SM.1541-4 Unwanted emissions in the out-of-band domain;

Recommendation ITU-R M.1732-1 Characteristics of systems operating in the amateur and amateur-satellite services for use in sharing studies;

Recommendation ITU-R F.1761 Characteristics of HF fixed radiocommunication systems;

Recommendation ITU-R F.1762 Characteristics of enhanced applications for high frequency (HF) radiocommunication systems;

Recommendation ITU-R F.1821 Characteristics of improved digital high frequency (HF) radiocommunication systems ;

Recommendation ITU-R M.1795 Technical and operational characteristics of SF/HF land mobile systems;

Recommendation ITU-R P.533-11 Methodology for prediction of HF-links operational characteristics;

ITU-Reports:

Working Document towards Preliminary Draft New Report ITU-R M.[5 MHZ CHAR], “Characteristics of amateur radio stations in the range 5 250-5 450 kHz for sharing studies”.

Working Document towards Preliminary Draft New Report ITU-R М.[AMATEUR]

Working Document towards Preliminary Draft New Report ITU-R M.[HF-SPECTRAL OCCUPANCY]

Working Document towards the Draft CPM text for WRC-15 Agenda item 1.4

Other ITU documents:

Amateur and amateur-satellite services Handbook

Work plan for WRC-15 Agenda item 1.4

Resolution 649 (WRC-12)

CEPT and/or ECC Documentation (Decisions, Recommendations, Reports):

EU Documentation (Directives, Decisions, Recommendations, other), if applicable:

# Actions to be taken

- Develop spectrum requirements for the amateur service;

- Specify technical and operational characteristics for amateur communication systems in the proposed band;

- Specify technical characteristics and protection criteria for systems of existing services in the proposed band;

- Conduct compatibility studies based on characteristics, protection criteria and interference scenario’s;

- Develop the CEPT position for this Agenda Item

# Relevant information from outside CEPT (examples of these are below)

## European Union (date of proposal)

## Regional telecommunication organisations:

APT (date of proposal)

ATU (date of proposal)

Arab Group (date of proposal)

CITEL (date of proposal)

RCC (date of proposal)

## International organisations

IATA (date of proposal)

ICAO (28 November 2012)

Preliminary position

To ensure that any allocation made to the amateur service shall not cause harmful interference to the operation of aeronautical systems operating under the allocation to the aeronautical mobile (R) service in the adjacent frequency band 5 450 – 5 480 kHz in Region 2.

IMO (date of proposal)

NATO (date of proposal)

SFCG (date of proposal)

WMO and EUMETNET (date of proposal)

## Regional organisations

ESA (date of proposal)

Eurocontrol (date of proposal)