OF CONTENTS

DRAFT CEPT BRIEF ON AGENDA ITEM 1.15

AI 1.15 	to consider spectrum demands for on-board communication stations in the maritime mobile service in accordance with Resolution 358 (WRC-12)

# ISSUE

Resolves part of Resolution 358 (WRC-12) invites WRC-15 to consider, based on the results of ITU-R studies, the need to possibly identify additional UHF channels within the bands already allocated to the maritime mobile service for on-board communication stations.

Invites part of Resolution 358 (WRC-12), invites ITU-R to conduct, in time for WRC-15, studies to determine the spectrum requirements and potential frequency bands for on-board communication stations, taking into account the protection of services to which the frequency band is currently allocated.

# Preliminary CEPT position

TBD

# Background

Only six frequencies, in the bands between 450 and 470 MHz, are currently identified in No. 5.287 for on-board communication stations

5.287 In the maritime mobile service, the frequencies 457.525 MHz, 457.550 MHz, 457.575 MHz, 467.525 MHz, 467.550 MHz and 467.575 MHz may be used by on-board communication stations. Where needed, equipment designed for 12.5 kHz channel spacing using also the additional frequencies 457.5375 MHz, 457.5625 MHz, 467.5375 MHz and 467.5625 MHz may be introduced for on-board communications. The use of these frequencies in territorial waters may be subject to the national regulations of the administration concerned. The characteristics of the equipment used shall conform to those specified in Recommendation ITU-R M.1174-2. (WRC-07)

# List of relevant documents

ITU-Documentation (Recommendations, Reports, other):

Annex 03 to Document 5B/167 	working document towards CPM Report on Agenda item 1.15

Annex 04 to Document 5B/167 (16 November, 2012) – Draft Work plan- Agenda item 1.15

CEPT and/or ECC Documentation (Decisions, Recommendations, Reports):

EU Documentation (Directives, Decisions, Recommendations, other), if applicable:

# Actions to be taken

Identification and justification of the requirements for additional spectrum for in-board communications:

The following questions need to be answered by the Administrations prior to the next PT C meeting:

  - whether the number of existing frequencies for on board communications is sufficient;

  - whether the frequencies in use for on board frequencies are sufficiently protected via the Radio Regulations and

  - whether existing frequencies allocated for on board frequencies are severely congested in European

- ports and what a possible solution might be.

In case of justified needs requirements the identification of the possible (candidate) frequency bands (frequencies) for on-board communications

  - Compatibility studies to determine the technical and regulatory measures to guarantee the protection from interference;

- i.	to the services which are allocated in the specific (candidate) frequency bands from maritime mobile stations;

- ii.	maritime mobile service in the candidate bands (on-board communications) from the existing services.

# Relevant information from outside CEPT (examples of these are below)

## European Union (date of proposal)

## Regional telecommunication organisations:

APT (date of proposal)

ATU (date of proposal)

Arab Group (date of proposal)

CITEL (22 – 26 October 2012)

Preliminary views:

CANADA: Studies to determine spectrum requirements and possible technology improvement must be carried out to ascertain any additional spectrum requirements.

RCC (date of proposal)

## International organisations

IATA (date of proposal)

ICAO (28 November 2012)

Preliminary Position

No impact on aeronautical services has been identified from WRC-15 Agenda Items 1.15

IMO (8-12 October, 2012)

Preliminary Position

IMO supports the upgrade of the regulatory status of these frequencies to the exclusive use by the maritime mobile service.

NATO (date of proposal)

SFCG (date of proposal)

WMO and EUMETNET (date of proposal)

## Regional organisations

ESA (date of proposal)

Eurocontrol (date of proposal)