OF CONTENTS

DRAFT CEPT BRIEF ON AGENDA ITEM 1.16

1.16	to consider regulatory provisions and spectrum allocations to enable possible new Automatic Identification System (AIS) technology applications and possible new applications to improve maritime radiocommunication in accordance with Resolution 360  (WRC 12);

# ISSUE

The Automatic Identification System (AIS) is a proven maritime data system, with a large number of ships equipped and a supporting terrestrial and satellite infrastructure established;

AIS is used in the ship movement service for collision avoidance, it enables the identification of stations using this system, provides information about a ship and its cargo. It provides a means for ships to exchange ship data, including identification, position, course and speed, with other nearby ships and coast stations.

AIS has the capability for data exchange by application-specific messages for complementary information for navigation. Within recent years the AIS use is increasing rapidly, as well as Application Specific Message (ASM), and a potential overloading, of the current AIS1 and AIS2 (Appendix 18 of the Radio Regulations) need to be considered.

The decision of WRC 12 to assign new channels of the appendix 18 to digital communication makes the implementation and use of new digital communication means possible. The establishment of the maritime AIS technology, the VHF data exchange and certain satellite communication components on these new frequencies offers potential enhancements to VHF maritime safety communications on a global basis to satisfy the increasing need for maritime radiocommunications for enhanced maritime safety

# Preliminary CEPT position

- CEPT is of the view that the implementation of new digital communication such as AIS type technology, VHF data exchange (VDE) and certain satellite communication components would enhance the maritime radio communications for maritime safety.

- CEPT is of the view that no modifications should be required to existing AIS equipment on board existing vessels and that the integrity of the original operational purpose of AIS as the primary function on the existing AIS frequencies should be protected

# Background

Resolves part of Resolution 360 (WRC-12) invites ITU-R:

to consider, based on the results of ITU-R studies, modifications to the Radio Regulations, including possible spectrum allocations, to enable new AIS terrestrial and  satellite applications, while ensuring that these applications will not degrade the current AIS operations and other existing services;

- The ship borne automatic identification system (AIS) mandated under Chapter V of the international convention for the safety of life at sea (SOLAS) has become well accepted by the maritime community and is also being used by thousands of ships not subject to carriage requirement under the SOLAS Convention.

- AIS is supported by a large shore based VHF infrastructure as well as being able to be detected by satellite. AIS is routinely used by ships for navigation and crew familiarity is a positive factor. AIS messages can be sent with a priority #1 (highest) through #4 (lowest).

- AIS 1 and AIS 2 should be reserved for “Navigation Safety/Collision Avoidance” purposes (as a SOLAS requirement) and therefore the Application-Specific Messages (ASM) and other “non-critical communications” should be moved to new channels of RR Appendix 18 to avoid deleterious loading of the AIS VDL. This problem increases as more different types of equipment using AIS technology are developed, more vessels are equipped and more AIS applications are developed and implemented.

- Taking into account the channels identified by WRC-12 as described in above, new digitalized channels could be used with modulation technique described in Recommendation ITU-R M.1842-1, and could be used for future VHF digital data, and ship-to-shore data exchange.

- These may be used as discrete data communications channels, or a number may be combined into a single wide-bandwidth channel.

- Where a number of the 25 kHz channels are combined, a typical scheme might have a 100 kHz bandwidth, allowing a much higher data throughput than a single 25 kHz channel. The use of the six VHF data channels plus two further channels (which have been identified for “possible testing of future AIS applications”) for an international scheme to be known as “VHF data exchange” (VDE).

to consider, based on the results of ITU-R studies, additional or new applications for maritime radiocommunication within existing maritime mobile and mobile-satellite service allocations, and if necessary to take appropriate regulatory measures,

- Satellite communications is an effective means to deliver information in a broadcast or multicast mode to a large number of ships, i.e. efficiently addressing many vessels using only minimal parts of the scarce maritime radio spectrum resource. A satellite downlink channel is able to address a single message to thousands of ships simultaneously within its footprint.

- Suitable frequencies for a satellite VDE downlink channel, as part of the VDE concept - allow the reception of the satellite channel with low-cost receive-only equipment, or can make extensive use of the existing VHF infrastructure on the ships and require only minor modifications.

- A satellite downlink channel would allow to pass information to ships which are out of reach of the terrestrial shore infrastructure, therefore extending the geographical reach of services such as remote areas e.g. polar regions.

With agenda item 1.16, WRC-15 is mandated to decide on technical, regulatory and operational solution to enable and establish new AIS Applications and digital data communication between ship and land and satellite down links in the existing maritime mobile and mobile-satellite service allocations including Appendix 18.

Invites part of Resolution 360 (WRC-12) invites ITU-R:

- to conduct, as a matter of urgency, studies that identify potential regulatory actions to accommodate emerging maritime mobile service and mobile-satellite service AIS requirements;

- to conduct, as a matter of urgency, studies on additional or new applications for maritime radiocommunication within maritime mobile and mobile-satellite service allocations, and to identify potential regulatory actions to accommodate emerging maritime radiocommunication requirements;

- to complete studies in time for WRC-15 taking into account existing systems and services that share the bands

# List of relevant documents

ITU-Documentation (Recommendations, Reports, other)

Recommendation ITU-R M.1084 – Interim solutions for improved efficiency in the use of the band 156-174 MHz by stations in the maritime mobile service.

Recommendation ITU-R M.1371 – Technical characteristics for an automatic identification system using time-division multiple access in the VHF maritime mobile band.

Recommendation ITU-R M.1842 – Characteristics of VHF radio systems and equipment for the exchange of data and electronic mail in the maritime mobile service RR Appendix 18 channels.

ITU-Reports:

Annex 6 to document 5B/167 – WP 5B Chairman’s report (November 2012) – Draft Workplan on AI 1.16

Annex 5 to document 5B/167 – WP 5B Chairman’s report (November 2012) - Working document towards Draft CPM Report on AI 1.16

Other ITU documents:

5B/106 and 5B/153

CEPT and/or ECC Documentation (Decisions, Recommendations, Reports)

EU Documentation (Directives, Decisions, Recommendations, other), if applicable

# Actions to be taken

- Study the loading of AIS 1 and AIS 2.

- Study the content of the existing ASM applications messages and study the implication of these ASM over the VDL.

- Consideration of the additional or new maritime communications applications within the existing maritime mobile and maritime mobile-satellite allocations, such as VDE (terrestrial and satellite).

- Compatibility and sharing studies to determine the technical and regulatory measures for the new  terrestrial and satellite applications, while ensuring that these applications will not degrade the current AIS operations and other existing services.

# Relevant information from outside CEPT (examples of these are below)

## European Union (date of proposal)

## Regional telecommunication organisations:

APT (date of proposal)

ATU (date of proposal)

Arab Group (date of proposal)

CITEL (22-26 October 2012)

Preliminary view

Canada: 
With additional applications being developed in the maritime mobile service, and taking into account other existing services, Canada supports studies to determine the need for additional spectrum and if required, what frequency band would be appropriate for additional applications using AIS technology.

RCC (date of proposal)

## International organisations

IATA (date of proposal)

ICAO (28 November 2012)

Preliminary position

To ensure that any change to the regulatory provisions and spectrum allocations resulting from this agenda item do not adversely impact on the capability of search and rescue aircraft to effectively communicate with vessels during disaster relief operations.

IMO (8-12 October 2012)

IMO’s position is that modifications should not be required to existing AIS equipment on board existing vessels, but rather allow for new applications using AIS technology to evolve, supported by communication primarily on the new frequencies identified by WRC-12, while protecting the integrity of the original operational purpose of AIS as the primary function on the existing AIS frequencies.

NATO (date of proposal)

SFCG (date of proposal)

WMO and EUMETNET (date of proposal)

## Regional organisations

ESA (date of proposal)

Eurocontrol (date of proposal)