BLE OF CONTENTS

DRAFT CEPT BRIEF ON AGENDA ITEM 1.11

Agenda item 1.11: to consider a primary allocation for the Earth exploration-satellite service (Earth-to-space) in the 7-8 GHz range, in accordance with Resolution 650 (WRC-12)

# ISSUE

This agenda item will consider a new primary allocation to the Earth exploration-satellite service (Earth-to-space) in the 7-8 GHz range in order to complement telemetry operations of Earth exploration-satellite service (space-to-Earth) in the 8 025-8 400 MHz band.

Resolution 650 (WRC-12), “Allocation for the Earth exploration satellite service (Earth to space) in the 7/8 GHz range”, resolves to invite ITU-R:

to study spectrum requirements in the 7-8 GHz range for EESS (Earth-to-space) telecommand operations in order to complement telemetry operations of EESS (space-to-Earth) in the 8 025-8 400 MHz band;

to conduct compatibility studies between EESS (Earth-to-space) systems and existing services, with priority to the band 7 145-7 235 MHz, and then within other portions of the 7-8 GHz range only if the band 7 145-7 235 MHz is found not to be suitable;

to complete the studies as a matter of urgency, taking into account the present use of the allocated band, with a view to presenting, at the appropriate time, the technical basis for the work of WRC-15,

# Preliminary CEPT position

CEPT supports the on-going studies with a view of having a primary allocation to EESS(E-s) in the 7-8 GHz range while ensuring an adequate protection of other space and terrestrial services in the 7-8 GHz band, and in particular of the Space Research Services (SRS) within the band 7 145 – 7 235 MHz. Other frequencies in the range 7-8 GHz should be considered if sharing with SRS is not found to be feasible.

# Background

The high concentration of satellites (several hundreds) using the band 2025-2110 MHz (E-to-s) and 2200-2290 MHz (s-to-E) for Tracking, Telemetry and Control (TT&C) is making the satellites coordination in these bands rather difficult.

Among the various satellite services using this band, the Earth exploration-satellite service (EESS) currently can only use the allocation at 2 025-2 110 MHz for the Earth-to-space transmissions, because no other Earth-to-space allocations are available at higher frequencies. The EESS (s-to-E) allocations at higher frequencies (8025-8400 MHz and 25.5-27 GHz) do not have any corresponding EESS (E-to-s) allocation and therefore in practice can be used only for payload data transmission and not for TT&C.

An EESS (E-s) allocation in the 7-8 GHz range would allow its use for TT&C in combination with the existing EESS (space-to-Earth, s-E) allocation in the band 8 025-8 400 MHz, thereby alleviating the congestion problem in S-Band, mitigating the frequency coordination problem, and eventually leading to a simplified on-board architecture and operational concept for future EESS missions. By this way some EESS new missions that already have to use the 8025-8400 MHz band for payload downlink, may use this band at these higher frequencies also for TT&C. It is to be noted that only some EESS missions may be able to migrate to the higher frequency for TT&C. And also these ones may still need using the bands 2025 – 2110 MHz and 2200-2290 MHz during LEOP (Launch and Early Orbital Phase) and in case of emergency. Nevertheless a new EESS allocation would allow limiting the use of these very congested bands at 2 GHz in a way similar to what is done for many telecommunications satellites.

The frequency range 7145-7235 MHz was identified at the WRC 12 as the most promising frequency range for this potential new EESS (E-to-s) allocation. This range is sufficiently close to the existing EESS (s-to-E) allocation at 8025-8400 MHz and has been shared for more than 15 years between the SRS (E-s) worldwide and furthermore with the Space Operation Service (E-s) in the Russian Federation, and systems of the Fixed/Mobile services with no specific sharing problems being reported.

The technical characteristics of potential new EESS (E-s) systems operating in the 7-8 GHz frequency range would be similar to those of SRS near Earth systems, but with lower transmit power requirements and smaller antenna size limited to a diameter between 11 and 15 m. Since the EESS (E-to-s) station will operate in a similar manner to the SRS stations but will require power levels typically much lower than the ones used by SRS in this band, sharing with terrestrial services in principle should be feasible.

Preliminary assessment on the bandwidth required for the EESS (E-s) in the 7-8 GHz are presented in the document CPGPTA(2012)008.

Preliminary analysis of compatibility between the proposed EESS (E-s) and the existing SRS missions in the 7145-7235 MHz band are presented in the document CPGPTA(2012)007.

Preliminary analysis of compatibility between the proposed EESS (E-s) and the existing FS stations are presented in the document CPGPTA(2012)010.

# List of relevant documents

Recommendation ITU-R SA.609-2 “Protection criteria for radiocommunication links for manned and unmanned near-Earth research satellites”

Recommendation ITU-R SA.1157-1 “Protection criteria for deep-space research”

Recommendation ITU-R SA 514-3 “Interference criteria for command and data transmission systems operating in the Earth exploration satellite and meteorological satellite services”

# Actions to be taken

- To prepare proposals to ITU Working document towards a preliminary draft new Report or Recommendation. The following areas should be covered as part of this action:

  - Determination of technical and operational characteristics of EESS earth stations transmitting in the band 7-8 GHz, with particular focus on the band 7 145-7  235 MHz

  - Determination of technical and operational characteristics FS, MS, FSS, SOS, METSAT and SRS systems operating in the band 7-8 GHz

  - Analyse the potential for interference to the existing and planned FS, SOS, MS, FSS, METSAT and SRS systems and determine the appropriate sharing criteria

  - Definition of spectrum requirements of new EESS systems

  - Examination of possible regulatory actions to satisfy the spectrum requirements of the new EESS systems

  - Analyse the potential for sharing between the EESS (E-s) and FSS (s-E) considering WRC15 AI 1.9.1 proposal for an allocation to the FSS (s-E) in the 7-8 GHz range once the FSS (s-E) parameters are defined. This parameters should come from ITU WP 4A documentation.

- To prepare proposals to ECP

- To prepare proposals to the draft CPM Report

# Relevant information from outside CEPT (examples of these are below)

## European Union (date of proposal)

## Regional telecommunication organisations:

APT (date of proposal)

ATU (date of proposal)

Arab Group (date of proposal)

CITEL (date of proposal)

RCC (date of proposal)

## International organisations

IATA (date of proposal)

ICAO (date of proposal)

IMO (date of proposal)

NATO (date of proposal)

SFCG (June 2012 )

SFCG supports the primary allocation to EESS (E-s) in the 7-8 GHz range. For this allocation, suitable protection of other space science services will have to be ensured. SFCG considers that within the band 7 145 – 7 235 MHz, the 7190-7235 MHz portion is preferable considering:

a) the potential for harmful interference to SRS (deep space) spacecraft during their low-Earth-orbit critical events in the 7 145-7 190 MHz band,

b) the potential for damage to EESS spacecraft receivers from high power SRS (deep space) Earth-to-space links in the 7 145-7 190 MHz band, and

c) the operating conditions of EESS missions that would be similar to those of SRS near Earth missions in the 7 190 – 7 235 MHz band.

SFCG considers that other frequencies in the range 7-8 GHz should be considered if sharing with SRS is not found to be feasible.

WMO (date of proposal)

## Regional organisations

CRAF (January 2013)

CRAF supports the protection of the SRS (passive). The lower frequency range 7190 – 7235 MHz is favoured for new allocations.

ESA (date of proposal)

EUMETNET (November 2012)

EUMETNET supports a new allocation to EESS (Earth-to-space) in the 7-8 GHz frequency range, provided that compatibility with meteorological applications is ensured.

Eurocontrol (date of proposal)