|  | Doc. SE19(13)xx |
| --- | --- |
| #### 61st meeting of PT SE 19 #### Copenhagen (ECO), Denmark, 8-9 January 2013 |  |
| #### Date issued: 8 January 2013 #### Source: SE19 Subject: Liaison Statement from SE19 to ECC PT1 on the filter characteristics of FS P-P and P-MP in the 3.5 GHz band |  |

Dear Didier

During it last meeting (8-9 January 2013), SE19 considered the informal questions sent by the ECC PT1 drafting group on BEM activity at 3.5 GHz regarding the filter characteristics of FS P-P and P-MP systems.

Question 1: Could you precise whether the harmonised standard implies conformity with the Block Edge Mask defined in EC Decision 2008/411 (the BEM defined by SE19 in ECC Recommendation (04)05) or whether additional filters are required to meet the BEM?

SE19 has collected the view of several participants coming from different areas of industry and it was noted that the BEM is not reflected in any of the harmonised standards (EN 302 774 on broadband wireless BSs (P-MP), EN 302 326-2 (FWA P-MP) and EN 302 217-2-2 for P-P).

It was confirmed that additional filters or other technical solutions are required to implement the BEM of ECC Recommendation (04)05, but in fact since it was understood that BEM are not typically used, the deployed BS do not implement additional filters. For FDD or TDD, measures like power restriction or introduction of guard band may be used. In addition, for TDD, the coordination can be done by the synchronisation in this band.

Question 2: In the case that additional filters are required on the transmitter side, are such filters also applied to the receiver side in order to balance the additional requirements on Tx leakage as defined in the BEM?

For TDD systems, the filter that is intended to mitigate the TX leakage is additional (i.e. it is not built in the equipment) and it is filtering both the Tx and the Rx path.

For FDD P-MP systems, separate external filters will be needed for fulfilling the BEM for each of the Tx and Rx path. For FDD P-P systems additional filtering was never considered because these systems were not required to fulfill the BEM.

SE19 would like to take the opportunity of this LS to inform ECC PT1 that even though the usage of the conventional (i.e. LOS) P-P seems to be declining in this band, there is a renewed interest in P-P Non-LOS backhauling in this band (See ECC Report 173).

Br

Jean-Philippe

SE19 chairman