CEPT Report 49

Report from CEPT to the European Commission in response to the Mandate <title> (Arial 12pt)

“Technical conditions regarding spectrum harmonisation for terrestrial wireless systems in the 3400-3800MHz frequency band”

Report approved on DD Month YYYY by the ECC(Arial 9pt bold)

# Executive summary

TBD

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	5

2	Definitions	6

3	task 1 of the mandate (BEM)	7

3.1	Justification for the need to revise the existing BEM	7

3.2	Development of the New BEM	7

4	task 2 of the mandate (channelling arrangements)	8

4.1	In response to task 2 of the mandate, CEPT is providing the following information:Channelling arrangements in the 3400-3600 MHZ and 3600-3800 Mhz bands	8

4.1.1	Background information	8

4.1.2	EC context	9

4.1.3	General justification for harmonized frequency arrangements	9

4.1.4	Justification of channeling arrangements in ECC/DEC/(11)06 [4]	9

4.1.4.1	Block size	9

4.1.4.2	Sub-bands 3400-3600 MHz and 3600-3800 MHz are treated independently	9

4.1.4.3	Channeling arrangement for the sub-band 3600-3800 MHz	9

4.1.4.4	Channeling arrangements for the sub-band 3400-3600 MHz	10

4.2	Key principles related to the coordination of MFCN and FSS	10

4.2.1	FSS and BWA	10

4.2.2	Key principles for FSS and MFCN	11

5	Conclusions	12

ANNEX 1: Technical analysis for the justification of new BEM	13

ANNEX 2: Comparison of 3400–3800 MHz ECC deliverables	20

ANNEX 3: cept mandate	22

ANNEX 4: List of reference	23

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| BEM | Block Edge Mask |
| BS | Base stations |
| BWA | Broadband Wireless Access systems |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| ECC | Electronic Communications Committee |
| EC | European Commission |
| ECO | European Communications Office |
| ENG | Electronic News Gathering |
| ERO | European Radiocommunication Office |
| EU | European Union |
| E-UTRA | Evolved Universal Terrestrial Radio Access |
| FSS | Fixed Satellite Service |
| FWS | Fixed Wireless Systems |
| IMT | International Mobile Telecommunications |
| LTRC |  |
| MFCN | Mobile/Fixed Communications Networks |
| MFW | Multipoint Fixed Wireless |
| OB | Outside Broadcasting |
| PMP |  |
| PP FS |  |
| TDD | Time Division Duplex |
| UL/DL | Uplink/Downlink |
| WAPECS |  |

# Introduction

The European Commission has issued a Mandate to CEPT on technical conditions regarding spectrum harmonization for terrestrial wireless systems in the 3400-3800 MHz frequency band to review and amend the technical conditions for the harmonized use of the 3400-3800 MHz frequency band in order to adapt them to the latest developments in technology by preserving flexibility of use in line with the WAPECS approach, including the updating of the Block Edge Mask (BEM) and introducing harmonized frequency arrangements.

CEPT is mandated to undertake the following tasks:

- “Assess and justify any need to revise the common minimal (least restrictive) technical conditions, including BEM, which underlie the harmonised use of in the 3400-3800 MHz frequency band in the EU and, if necessary, identify modified conditions in view of accommodating developments in wireless broadband access technology in particular larger bandwidths. These conditions should be sufficient to avoid interference, facilitate cross-border coordination, and ensure co-existence with other existing systems and services in the same band and adjacent bands.”

- “Assess and justify any need to introduce channelling arrangements in addition to (1) and, if necessary, develop a harmonised solution that is sufficiently precise for the development of EU-wide equipment.”

CEPT has developed a roadmap to structure the work in response to this mandate to address the following issues:

- assess and justify any need to revise the common minimal (least restrictive) technical conditions including BEM

- identify modified conditions in view of accommodating developments in wireless broadband access technology in particular larger bandwidths

- assess and justify any need to introduce channeling arrangements in addition to the LTRC (BEM)

- if necessary, develop a harmonized solution that is sufficiently precise for the development of EU-wide equipment.

The work within CEPT in response to this mandate is carried out by ECC Project Team 1 (ECC PT1).

# Definitions

| Term | Definition |
| --- | --- |

# task 1 of the mandate (BEM)

## Justification for the need to revise the existing BEM

Regarding the work on issue (a), ECC agreed on the justification for the need to revise the common minimal (least restrictive) technical conditions including BEM. The justification being the following:

In 2004 ECC adopted ECC/REC/ (04)05 [2] on “Guidelines for accommodation and assignment of Multipoint Fixed Wireless systems in frequency bands 3.4-3.6 GHz and 3.6-3-8 GHz” and in 2007 ECC/DEC/(07)02 Error! Reference source not found. on “Availability of frequency bands between 3400-3800 MHz for the harmonised implementation of Broadband Wireless Access systems (BWA)”. In 2008 the BEM contained in ECC/REC/(04)05 [2] were included in the Commission Decision 2008/411/EC [3] (on the harmonization of the 3400-3800 MHz frequency band for terrestrial systems capable of providing electronic communications services in the Community).

WRC-07 identified the band 3400-3600 MHz for IMT, so ECC developed band plans for MFCN systems including IMT (see ECC/DEC/(11)06 [4]).

ECC analyzed the existing BEM contained in ECC/REC/(04)05 [2], which were developed for PMP FWS systems in 2004 and concluded that it is not suitable for the introduction of MFCN systems including IMT in the 3400-3600 MHz band, due to the following reasons:

- The BEM available have been designed to ensure co-existence between PMP FWS applications only.

- The BEM were derived with the assumption of an internal guard band (half a channel width).

- The effect of blocking was not considered for establishing the BEM (which may lead to more stringent masks).

- The BEM may not even be suitable when PMP FWS are based on adjacent TDD blocks.

- The BEM is developed under the assumption that a high gain antenna leads to a lower probability of interference than a low gain antenna. While that might be appropriate for Fixed Wireless Systems, it is certainly inappropriate for other types of MFCN systems.

- The ETSI SEM (for 3GPP band class 7/38) do not allow an operation to fit to the BEM. It is anticipated that the SEMs of IMT-Advanced systems would not allow an operation to fit the BEM of the ECC/REC/(04)05 [2] as well (due to their large bandwidths).

- The existing 3.5 GHz BEM is justified in cases, where there is no commonly agreed band plan and maximum flexibility is needed (like the case of BWA deployments). When band plans are available and adopted, there is no need for the unnecessarily tight BEM but it should be adjusted to the more harmonized conditions in order to facilitate affordable equipment, maximise the spectrum efficiency (e.g. by reduced guard bands) and thus maximize the available amount of spectrum.

The technical analysis is provided in Annex 1 to this report.

## Development of the New BEM

For the purposes of this report the term BWA refers to legacy BWA systems licenced under existing 3400 – 3600 MHz licencing regimes (ECC/REC/(04)05, Commission Decision 2008/411/EC or even prior). The term MFCN is a collective term for radio communication systems including BWA, fixed point to point and point to multipoint services and for the purposes of this report refers to radio communication systems which will be deployed according to the new BEM defined in this report.

CEPT has carried out studies (see ECC Report XXX) to determine appropriate LRTCs for MFCN. In this report the BEMs for different categories of base stations were derived from MCL analysis and simulations. Four different categories of base stations were considered: Macro, Micro, Pico and Femto. Since they have different characteristics, different BEMs were defined for each category of base station.

For each category of base station, the corresponding power levels of the BEM were defined for a number of different regions: In-block power (PIB) where the operator may use the full power for the corresponding base station category, baseline power (PBL) was defined for other operators’ blocks (FDD UL and FDD DL as well as TDD), power in the transitional regions (PTR) was defined for the regions up to 10 MHz adjacent to an operator’s own full-power block and power in the guard bands in case the FDD arrangement is used in 3400 – 3600 MHz: 3400 – 3410 MHz, 3490 – 3510 MHz and 3590 – 3600 MHz

Inblock limits:

Table 28 contains the e.i.r.p. in-block limits for different categories of base stations.

Table 28: In-block power limits (dBm e.i.r.p.)

| Macro BS | Not obligatory |
| --- | --- |
| Micro BS | [47 dBm |
| Pico BS | 24 dBm] |
| Femto/Home BS | 20 dBm |

Baseline limits:

- Baseline power limits (dBm/MHz e.i.r.p.)

|  | FDD DL region (limit per antenna) | FDD UL region (limit per cell) | TDD regions (limit per cell) |
| --- | --- | --- | --- |
| Macro BS | 2 | -42 | -42 |
| Micro BS | -9 | -45 | -45 |
| Pico BS | -27 | -43 | -43 |
| Femto/Home BS | -32 | -41 | -41 |

Transitional regions limits:

- Transitional region power limits (dBm/5MHz e.i.r.p.) per antenna for different off-sets from lower or upper block-edge

|  | +/- (0-5) MHz | +/- (5 - 10) MHz |
| --- | --- | --- |
| Macro BS | 16.3 | 11 |
| Micro BS | [5.6 | -1.4 |
| Pico BS | -16 | -20] |
| Femto/Home BS | -21.6 | -25 |

Guard band limits:

In the harmonised frequency arrangement guard bands are only required where FDD channelization is adopted. The permitted emission levels for these three different guard bands are as follows:

•	3400 – 3410 MHz: tbd

•	3490 – 3510 MHz: tbd

•	3590 – 3600 MHz: tbd

Coexistence studies for other services than MFCN have been carried out for both inband and out of band scenarios. The inband services considered were FSS, FS and BWA and the out of band services civil and military Radiolocation.

The conclusions are as follows:

FS

MFCN applies to all Mobile and Fixed networks including point to point Fixed links.

FSS

Due to the varying characteristics of different types of FSS earth stations and their deployment, no single separation distance, guard band or signal strength limit can be provided to guarantee co-existence with MFCN. Successful co-existence should be achieved through co-ordination on a case-by-case basis, assuming FSS earth stations locations are known. However, some general observations can be made. Separation distances for co-existence vary considerably depending on type of equipment and deployment (e.g. tilt and clutter), but can be large. UE impact earth stations less than BS, so separation that prevents interference from BS will also protect earth stations from UE interference. There are several mitigation techniques that can be applied, in particular site shielding of earth stations. Interference from FSS satellites to MFCN may exceed the acceptable interference level, but in most cases only by a small margin.

Radiolocation:

Due to the varying characteristics of different types of radar stations and their deployment, no single separation distance, guard band or signal strength limit can be provided to guarantee co-existence with MFCN. Successful co-existence should be achieved through co-ordination on a case-by-case basis. However, some general observations can be made for non-overlapping adjacent channels. For airborne radars the required separation distance is approximately 0 km, depending on the radar type and antenna type. For land-based/shipborne radars the required separation distance is less than 1 km, depending on the radar type and antenna type. A frequency separation analyses concludes that for a 5 km separation, and considering IMT-Advanced interference to radars, the required frequency separation varies between 14 and 65 MHz, depending on radar type and scenario.

There are mitigation techniques which can reduce the separation distance or frequency separation required. In particular, for adjacent channel/adjacent band interference, improved receiver performance and decreased unwanted emissions can be efficient.

Regarding interference from radars to MFCN networks, installation of systems closer than ca. 5 km from the radar should be coordinated. It is necessary to establish a protection distance of approximately 11 km in some areas. Considering blocking effects, the radar may impact MFCN systems up to a distance of 30 km.

The analysis did not take into account the fact that radar antennas rotate and therefore only affect a particular MFCN base station or UE intermittently.

Adjacent band limit in the case of adjacent band usage by military systems:

In some CEPT countries military radiolocation systems that are deployed below 3400 MHz need a fixed limit for protection. According to [reference] this protection level is set as [x dBm].

[This protection requirement leads to [a] restricted block[s] in the case of TDD deployment and to a more stringent limit for the adjacent guardband in the case of FDD deployment]

The detailed study results can be found in ECC Report XXX.

# task 2 of the mandate (channelling arrangements)

## In response to task 2 of the mandate, CEPT is providing the following information:Channelling arrangements in the 3400-3600 MHZ and 3600-3800 Mhz bands

The aim of this section is to assess and justify the need to introduce channelling arrangements in addition to the BEM developed as task 1 of the mandate. The channelling arrangements should be sufficiently precise to enable the development of EU-wide equipment.

CEPT has approved ECC/DEC/(11)06 [4] that precisely provides channelling arrangements in the 3400-3600 MHz and 3600-3800 MHz frequency bands. The reasoning and justification that led to the improvement of the regulatory framework in terms of channelling arrangements is used as basis for this section.

### Background information

Any harmonised frequency arrangements for the 3400-3800 MHz band should facilitate high data rate mobile/fixed communications networks (MFCN) including International Mobile Telecommunications (IMT) services supported by larger channel bandwidths as an evolution to the existing framework without the consequential requirement for a replacement of systems based on the existing regulatory framework. It aims at providing the basis to the mobile industry and administrations to respond to the growth of mobile broadband and technological developments for wider channel bandwidths and increased data rates.

Since WRC-07, the 3400-3600 MHz band has been allocated on a primary basis to the mobile, except aeronautical mobile, service and identified for IMT in almost all CEPT member countries.

The term IMT covers IMT-2000 and IMT-Advanced systems. A wide range of systems are defined: 6 IMT-2000 radio interfaces and 2 IMT-Advanced radio interfaces ensuring a competitive environment.

Recommendation ITU-R M.1036 [5] (on frequency arrangements for implementation of the terrestrial component of IMT) has been revised to include, among others, the arrangement(s) for the 3400-3600 MHz band.

By beginning of 2012, ITU-R agreed on the IMT-Advanced technologies in cooperation with standardisation organisations paving the way for future mobile broadband usage going beyond IMT-2000.

The ECO (formerly ERO) carried out a survey in 2008 (ECC PT1(09)109R1 Annex 16 [8]) which found diverse implementation of BWA/FWA within 3400-3800 MHz in CEPT countries, including some IMT systems. This is reflected in various licensing coverage (national, regional) and various frequency blocks choices (different portions of the 3400-3800 MHz). Moreover, this survey showed that paired blocks are used or planned to be used in TDD mode in some countries.

In so far as is practicable, the frequency arrangements in ECC/DEC/(11)06 [4] are intended to be technology neutral and capable of facilitating competitive provision of services using a range of technologies and modes (fixed, nomadic and mobile) with sufficient flexibility to accommodate current wireless broadband services deployed in the band.

When developing these channelling arrangements, ECC considered the following CEPT regulatory framework that is in force for broadband and fixed wireless access systems (BWA/FWA) in the 3400-3800 MHz band:

The ECC/REC/(04)05 [2] that offers guidelines for accommodation and assignment of multipoint fixed wireless systems in the frequency bands 3400-3600 MHz and 3600-3800 MHz;

The ECC/DEC/(07)02 [1] on availability of frequency bands between 3400-3800 MHz for the harmonised implementation of Broadband Wireless Access systems (BWA). This Decision refers to ECC/REC/(04)05 [2] for frequency arrangements. The revision of this Decision is scheduled for 2012.

See comparison of ECC Deliverables in Annex 2 of this report.

### EC context

The existing Commission Decision 2008/411/EC [3] on the harmonisation of the 3400-3800 MHz frequency band for terrestrial systems capable of providing electronic communications services in the Community is based on the results of studies in response to EC mandates that are documented in CEPT Reports 15 and 19 (which defines least restrictive technical conditions for 3400-3800 MHz).

Under the scope of the EC mandate (task 1) CEPT is conducting additional analysis to determine whether the existing least restrictive technical conditions (BEM) are suitable also for the high data rate IMT services supported by larger channel bandwidths as foreseen in the context of this ECC Decision.

### General justification for harmonized frequency arrangements

It was recognised by the ECC that implementation of MFCN including IMT systems providing high data rate applications in the band 3400-3800 MHz based on a harmonised frequency arrangement will maximise the opportunities and benefits for end users and society, will benefit capital expenditure for operators, reduce development and implementation costs of manufacturing equipment and will secure future long terms investments by providing economies of scale. Harmonised frequency arrangement facilitates economies of scale resulting in the availability of affordable equipment. A harmonised frequency arrangement will reduce complexity in cross border coordination. Global roaming is facilitated by common frequency arrangements and measures for free circulation for IMT terminals. The opportunity to utilize larger channel bandwidths will assist the provision of high data rates for IMT (especially with IMT-Advanced).

### Justification of channeling arrangements in ECC/DEC/(11)06 [4]

#### Block size

ECC/DEC/(11)06 [4] chose to use block sizes of 5 MHz. It was considered that spectrum licensed for MFCN is generally assigned in multiples of 5 MHz, except where this is not possible, e.g. due to the presence of existing users. This block size enables (by combination of adjacent blocks) to utilize larger channel bandwidths creating the possibility to provide high data rates for IMT (especially with IMT-Advanced). Channel bandwidths such as 10, 20 and 40 MHz or more that could be accommodated in the bands 3400-3600 MHz and 3600-3800 MHz will enable higher data rates.

#### Sub-bands 3400-3600 MHz and 3600-3800 MHz are treated independently

The two sub-bands are treated as independent bands considering that they are treated differently in the Radio Regulations context and that the incumbent use of spectrum for each sub-band varies. For instance use of these two sub-bands for Fixed Satellite Service (FSS) is not the same (the band 3600-3800 MHz is used for FSS more intensively than the band 3400-3600 MHz).

#### Channeling arrangement for the sub-band 3600-3800 MHz

A TDD band plan has been chosen for this sub-band. It was considered that TDD may allow more flexible accommodation of current use of the frequency bands by other services. There is more flexibility to create “holes” in the band to protect incumbent users, as these holes are not replicated in the UL/DL band as is the case for FDD. For example TDD allows more efficient spectrum use when taking into account existing fixed satellite usage in case of geographical sharing. This is especially relevant to the 3600-3800 MHz band since this band is more intensively used for FSS than the band 3400-3600 MHz.

The TDD arrangement is based on a block size of 5 MHz starting at the lower edge of 3600 MHz. If blocks need to be offset to accommodate other uses, the raster should be 100 kHz. Narrower blocks can be defined adjacent to other users, to allow full use of spectrum. It has to be noted that TDD in one extreme case also covers downlink only operation.

Title XXXX

#### Channeling arrangements for the sub-band 3400-3600 MHz

The ECC Decision places the two band plans (FDD and TDD) for the 3400-3600 MHz band on the same level due to the balance of the positions expressed. However it was also decided that the frequency arrangements in the 3400-3600 MHz should be subject to review no later than end 2013 with the aim to identify a preferred frequency arrangement.

It was however noted in ECC/DEC/(11)06 [4] that although there are licensed paired frequency arrangements in many CEPT countries, TDD systems are currently used in a number of those countries in the band 3400-3600 MHz due to the better availability of TDD systems.

Below is the frequency arrangement based on TDD duplex mode. The block size is 5 MHz starting at the lower edge of 3400 MHz. If blocks need to be offset to accommodate other users, the raster should be 100 kHz. Narrower blocks can be defined adjacent to other users, to allow full use of spectrum. It has to be noted that TDD in one extreme case also covers downlink only operation.

XXXX

Below is the frequency arrangement based on FDD. The block size is 5 MHz starting at the lower edge of 3410 MHz. The sub-band 3410-3490 MHz is used for the uplink, the sub-band 3510-3590 MHz is used for the downlink. The resulting duplex gap is 20 MHz (3490-3510 MHz). If blocks need to be offset to accommodate other uses, the raster should be 100 kHz. Narrower blocks can be defined adjacent to other users, to allow full use of spectrum.

XXXXX

## Key principles related to the coordination of MFCN and FSS

### FSS and BWA

There are currently 170 fixed satellite earth stations authorized within the EU Member States (deployed on 78 sites). As such, they are protected by Member States against harmful interference.

For MFCN and FSS coordination, similar principles can be used than as for BWA and FSS. Indeed in the case of BWA, the “central stations” (or master) are coordinated with the FSS earth stations. This implies that all the (fixed) terminal stations, operating under the control of central stations are consequently coordinated under the umbrella of the central stations (this typically requires to slightly extend the coordination distances). The same idea can be applied to MFCN where the BWA terminal stations are now replaced with mobile terminal stations that also operate under the control of the base stations (which need to be coordinated with the FSS stations).

### Key principles for FSS and MFCN

The following key principles related to the coordination between Mobile/Fixed Communication Network stations and Fixed-Satellite Service (FSS) Earth stations should be implemented at national level in order to ensure coordination between these systems:

- Frequency coordination is primarily concerned with local implementation, local propagation conditions and local licensed use of the shared band. This is best dealt with by national administrations;

- Some administrations have effective co-ordination arrangements in place. The implementation of these guidelines is at the discretion of the national administrations to the extent this may help them;

- The key objectives of co-ordination processes are maximising efficient use of the available spectrum for the benefit of the EU whilst protecting existing licensed uses of the band;

- Coordination processes and associated protection should only apply to registered/licensed spectrum users;

- Data exchange and coordination processes are mutual and reciprocal to all band users;

- Data on registered use of the band should be available to all users under relevant legal protections and confidentiality obligations;

- The coordination process must be both accurate and fast to enable all operators to efficiently plan spectrum utilisation and network deployments;

- Operators should have access to registered band usage to maximise the successful coordination of spectrum through propagation modelling without physical measurement at the planning stage;

- All parties are responsible for the efficient use of spectrum. In deploying new MFCN stations and new FSS Earth stations, operators should be cognisant of the need to minimise constraints on the other service;

- These guidelines primarily relate to co-ordination within national boundaries. For the situation where MFCN  and FSS stations are within the territories of different administrations, the use of these guidelines within bilateral agreements may help to expedite cross border co-ordination;

- All parties should undertake reasonable efforts to successfully complete the coordination exercise as quickly as possible;

- Either party has the inherent right to refer the co-ordination to the relevant NRA(s) if agreement cannot be reached.

# Conclusions

TBD

Technical analysis for the justification of new BEM

[Editor’s Note: The format/layout of this Annex needs to be aligned with the rest of the document by the ECO before submitting the document to the ECC (Source: ECC PT1(11)162 Annex 20)]

- Technical conditions for PMP FWS base stations (extracted from ECC/REC/(04)05 – Annexes 2 and 3) [2]

  - Maximum e.i.r.p., defined in Annex 2.

The following paragraphs have been extracted form Annex 2:

“Maximum e.i.r.p. density limits are set by administrations in their national licensing conditions in order to define pfd levels for co-ordination distances between different geographical areas or for cross-border agreements or sharing with other services. Transmit output power and e.i.r.p.  levels for Multipoint FWS systems are more driven by trade-offs between the required service coverage and other operational considerations. e.i.r.p.  density depends also on the system bandwidth that in modern PMP FWS might be flexibly changed.”

Maximum e.i.r.p. within a block:

Title (style: ECC Table title)

| Station Type | Max e.i.r.p. spectral density (dBW/MHz) (Including tolerances and ATPC range, Note 1) |
| --- | --- |
| Central Station (CS) (and Repeater Station(RS) down-links) | +23 Note 2 |
| Note 2: CS e.i.r.p. density value given in the table is considered suitable for conventional 90 deg sectorial antennas. Administrations may consider to adjust this value if other type of antennas are used (e.g. decrease the limit for omni-directional antennas, or increase when narrow-sector or adaptive antennas are used) |  |

“For further enhancing the efficiency, administrations may allow operators to apply mutual co-ordination at the block edge and at the service border edge for potential further relaxation of the above e.i.r.p. limits, depending on requirements for protecting other services or systems, such as PP FS. This could be reached, for instance, by taking advantage of mitigation techniques such as the shielding effect, limiting the height of Central Stations, or for stations that are located far from the service area boundary.”

  - Reference Block Edge Mask, defined in Annex 3.

The following paragraphs have been extracted form Annex 3:

“The block edge mask given in this annex was developed to ensure co-existence between PMP FWS applications only; different considerations would be required where the adjacent system is not a PMP FWS system, but for example ENG/OB or other.”

“The floor level in the mask provided in this annex has been based on co-existence studies reported in ECC Report 33 [7][7]; where the PMP FWS co-existence studies were mostly made with statistical tools and assumptions of typical radio systems, their deployment and service performance objectives. The reference points of the transition slope were chosen based on consideration of practical filters and various modulation envelopes. These studies and considerations may be subject to refinement as operational experience and system characteristics evolve. Therefore the block edge mask based upon these studies may also be subject to refinement.”

“Emissions from one operator’s frequency block into another operator’s frequency-adjacent block will need to be controlled. This was done in few other frequency bands by establishing fixed guard bands between the assignments. However, taking due account of the possible variety of broadband systems considered in this recommendation, different network and service requirements, and considering the expected broadening of the required bandwidth, it would be impossible to uniquely and efficiently set such guard bands and it is recommended that coordination and interference mitigation techniques be implemented between operators.”

“Also adjacent block receiver rejection concurs to a reduced interference scenario, however the study in Report 33 did not consider the effect of receiver selectivity since the technology neutrality assumption did not allow deciding on its typical parameters. Therefore it is not in the scope of this recommendation to set limits for it; nevertheless it is expected that ETSI standards will adequately cover the issue.”

“It should be also noted that when TDD or mixed FDD/TDD systems are placed in immediately adjacent blocks, the probability of occurrence of worst cases of interference between CSs is quite higher than in situations where only FDD are deployed. Therefore, even if the mask proposed in this annex would offer a suitably low probability of interference for such cases, when TDD systems are concerned additional mitigation techniques (geographic separation of stations, natural/physical shielding, etc) and/or additional co-ordination (including networks synchronisation) between operators should be implemented as far as possible.”

Definition of the block edge mask:

Central Station Block Edge Spectral Density Mask

XXXX

| Frequency offset break pointsfor the CS mask | Definition (% of the size of the assigned block, Note ) |
| --- | --- |
| A | 20% |
| B | 35% |

Note: X% of the smaller of adjacent blocks, if blocks are of unequal size

Tabular description of Central Station Block Edge Spectral Density Mask

| Frequency offset | CS Transmitter Output Power Density Limits(dBW/MHz) |
| --- | --- |
| In-band (within assigned block) | See Annex 2 |
| ΔF=0 | -36 |
| 0<ΔF<A | -36 - 41·(ΔF/A) |
| A | -77 |
| A<ΔF<B | -77 - 12·((ΔF-A)/(B-A)) |
| ΔF≥B | -89 |

- ETSI requirements for LTE

The relevant document to consider is ETSI EN 301 908-14 V5.2.1 (2011-05) [6]: Evolved Universal Terrestrial Radio Access (E-UTRA) Base Stations (BS).

It should be noted that the bands 3400-3600 MHz and 3600-3800 MHz are not yet part of the E-UTRA Base Station operating bands ; see table 1-1 of <Title of the document>that document, copied below as Table 4::

Title XXX

| E-UTRA band | Direction of transmission | E-UTRA Base Station operating bands |
| --- | --- | --- |
| 1 | Transmit | 2 110 MHz to 2 170 MHz |
|  | Receive | 1 920 MHz to 1 980 MHz |
| 3 | Transmit | 1 805 MHz to 1 880 MHz |
|  | Receive | 1 710 MHz to 1 785 MHz |
| 7 | Transmit | 2 620 MHz to 2 690 MHz |
|  | Receive | 2 500 MHz to 2 570 MHz |
| 8 | Transmit | 925 MHz to 960 MHz |
|  | Receive | 880 MHz to 915 MHz |
| 20 | Transmit | 791 MHz to 821 MHz |
|  | Receive | 832 MHz to 862 MHz |
| 33 | Transmit and Receive | 1 900 MHz to 1 920 MHz |
| 34 | Transmit and Receive | 2 010 MHz to 2 025 MHz |
| 38 | Transmit and Receive | 2 570 MHz to 2 620 MHz |

The closest E-UTRA band from the 3.5 GHz band is band 7 (and 38). Therefore the comparison between the ETSI mask and the CEPT BEM has been made on that basis although this represents a tightening of the SEM.

  - e.i.r.p. defined by ETSI in band 7 and 38

ETSI currently defines no in-band e.i.r.p. limit, nor output power values.

  - Spectrum Emission Mask for band 7 and 38

The following tables were extracted from document ETSI EN 301 908-14 [6]. Three different types of base stations have been defined: wide area, local area and home.

Wide Area BS operating band unwanted emission limits
for 5 MHz, 10 MHz, 15 MHz and 20 MHz channel bandwidth (E-UTRA bands 7 and 38)

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0,05 MHz f_offset < 5,05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5,05 MHz f_offset < min(10,05 MHz, f_offsetmax) | -12,5 dBm | 100 kHz |
| 10 MHz f fmax | 10,5 MHz f_offset < f_offsetmax | -15 dBm (see note) | 1 MHz |
| NOTE: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Local Area BS operating band unwanted emission limits for
 5 MHz, 10 MHz, 15 MHz and 20 MHz channel bandwidth

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0,05 MHz f_offset < 5,05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5,05 MHz f_offset < min(10,05 MHz, f_offsetmax) | -35,5 dBm | 100 kHz |
| 10 MHz f fmax | 10,05 MHz f_offset < f_offsetmax | -37 dBm (see note) | 100 kHz |
| NOTE: The requirement is not applicable when fmax < 10 MHz. |  |  |  |

Home BS operating band unwanted emission limits for 5 MHz, 10 MHz, 
15 MHz and 20 MHz channel bandwidth

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0,05 MHz f_offset < 5,05 MHz |  | 100 kHz |
| 5 MHz f < 10 MHz | 5,05 MHz f_offset < 10,05 MHz | -40,5 dBm | 100 kHz |
| 10 MHz f fmax | 10,5 MHz f_offset < f_offsetmax | P – 52 dB, 2 dBm ≤ P ≤ 20 dBm -50 dBm, P < 2 dBm (see note) | 1 MHz |
| NOTE: For Home BS, the parameter P is defined as the aggregated maximum power of all transmit antenna ports of Home BS |  |  |  |

Note: for home BS, with frequency offset ≥ 10 MHz, an output power of 20 dBm has been chosen for the purpose of this contribution ; the corresponding minimum requirement is therefore -32 dBm/MHz.

- Analysis of these technical conditions

  - Maximum e.i.r.p.

As there is no value specified in the ETSI harmonised standard, the comparison with the value mentioned in the CEPT Recommendation is not possible. However, a short analysis is provided below:

The e.i.r.p. value as contained in ECC/REC/(04)05 [2] = 23 dBW/MHz=30 dBW/ 5 MHz=60 dBm/5 MHz

This value is similar to what would be expected for a macro base station (also in the order of 60 dBm/5 MHz).

Conclusion: the maximum e.i.r.p. (in-band value) set up in ECC/REC/(04)05 [2] is compatible with typical in-band e.i.r.p. mobile deployments.

  - BEM versus SEM

- The three following figures show the comparisons of BEM and SEM for BEM based on a 5 MHz block assignment, as well as 10 MHz and 20 MHz.

| 5 MHz |  |
| --- | --- |

xxxxxxxx

| 10 MHz |  |
| --- | --- |

XXXXXXX

| 20 MHz |  |
| --- | --- |

XXXXXX

It can be seen from the above figures that the SEM exceed the BEM for any value of block assignment.

- ETSI requirements for BWA

The relevant document to consider is ETSI EN 302 774 V1.1.1 (2011-05): Broadband Wireless Access Systems (BWA) in the 3 400 MHz to 3 800 MHz frequency band (Base Stations).

Taking the same approach to compare the SEM of BWA and the current given BEM for the 3.4-3.6 GHz band, the conclusions are the same as described in the section 3b.

Comparison of 3400–3800 MHz ECC deliverables

This annex provides an overview of the current ECC framework for the band 3400-3800 MHz including a comparison of ECC Decision (07)02, ECC Recommendation (04)05 and ECC Decision (11)06 that confirms the consistency of ECC framework according to evolution of market needs and the need to maintain this consistency in the future.

At this stage, there are three relevant deliverables for the band 3400-3800 MHz:

- ECC/DEC/(07)02 “Availability of frequency bands between 3400-3800 MHz for the harmonised implementation of Broadband Wireless Access systems (BWA)”

- ECC/REC/(04)05 “Guidelines for accommodations and assignments of multipoint fixed wireless systems 3.4-3.6 GHz and 3.6-3.8 GHz”

- ECC/DEC/(11)06 : “Harmonised frequency arrangements for mobile/fixed communications networks (MFCN) (including IMT) operating in the bands 3400 - 3600 MHz and 3600- 3800 MHz”

scope of deliverables

ECC/REC/(04)05 covers Point multipoint fixed wireless systems (Point-to-Multipoint Fixed Wireless Systems (PMP FWS). It has a more narrow scope than the other two ECC deliverables.

ECC/DEC/(07)02 on Broadband Wireless Access systems (BWA) covers Fixed, Nomadic and, also, Mobile Wireless Access (MWA). It provides in its Annex considerations for an implementation of a flexible usage mode for BWA in 3400-3600 MHz and/or in 3600-3800 MHz on the basis initially of a fixed and nomadic usage. In particular, these considerations refers to ECC Recommendation (04)05 and state that the technical conditions in ECC Decision (04)05 may be used for implementation of flexible usage mode. Moreover ECC/DEC/(07)02 mentions that the introduction of MWA usage mode will be subject to additional requirements for deployment of mobile TS Mobile Wireless Access (annex 1 §3 of the Decision).

ECC/DEC/(11)06, focusing primarily on a mobile usage includes a forward looking approach. The harmonised frequency arrangements for the 3400-3600 MHz and 3600-3800 MHz bands are intended to facilitate high data rate International Mobile Telecommunications (IMT) services supported by larger channel bandwidths as an evolution to the existing framework without the consequential requirement for a replacement of systems based on the existing regulatory framework.

coexistence with incumbent users

For ECC/DEC/(07)02, the designation of spectrum within 3400-3800 MHz for BWA should take due consideration of incumbent users (see decides 1 and 3).

Although the wording is different, ECC/DEC/(11)06 also designates spectrum for MFCN on a non-exclusive basis (“without prejudice to the protection and continued operation of other existing users in these bands”). It is assumed that transition from legacy terrestrial systems to future terrestrial systems will be managed at national level.

Band plan and duplex mode

ECC/DEC/(07)02 does not provided a definite duplex mode or any band plan.

ECC/DEC/(11)06 provides two possible band plans for the band 3400-3600 MHz, one TDD and one FDD. The band plan for 3600-3800 MHz is TDD.

ECC/REC/ (04)05 lets the possibility to have a mix of FDD and TDD blocks (i.e. recommends paired blocks that can be used either for FDD or for TDD). The guidelines for these flexible arrangements are:

- The bands 3400-3600 MHz and 3600-3800 MHz are treated as separate bands,

- 100 MHz duplex separation for paired blocks,

- in the case of paired FDD blocks the lower block of the two paired FDD blocks is used for uplink,

- that 3400 MHz to 3410 MHz is not included in the band plan.

block size

ECC Recommendation (04)05 The blocks are designed to fit 3.5 MHz and 7 MHz channels (4 of them per block). The preferred size for blocks are multiple of these channels and may include internal guard bands. The resulting sizes for paired spectrum are (2x17.5 MHz, 2x21 MHz, 2x35 MHz, 2x42 MHz) and for unpaired spectrum (35 MHz, 42 MHz, 70 MHz and 84 MHz).

In case of external guard bands the sizes of the blocks may be reduced.

ECC Decision (11)06 The block sizes are multiples of 5 MHz.

emission requirements

ECC Decision (07)02 refers to ECC Recommendation (04)05 for emission levels. But since it covers also MWA which is not covered by ECC Recommendation (04)05 additional requirements are provided for mobile terminal stations (in block emission level and spacing of the carrier from the block edge to protect adjacent networks).

- For the technical requirements it refers in its annex to ECC Recommendation (04)05: “As a starting point, the guidance given in ECC Recommendation (04)05 on technical conditions for implementation of flexible usage mode, to be set in the technology neutral BWA licence process, shall be considered”.

- For mobile terminals, the annex of ECC Decision (07)02 provides additional requirements

In the case of adjacent band TDD/FDD systems additional mitigation techniques should be considered (geographic separation of stations, natural/physical shielding, and/or additional co-ordination including networks synchronisation)

ECC Recommendation (04)05 provides emission requirements in the form of Block Edge Masks (BEM).

- For the Central Station (CS) BEM are provided with an “in block” limit (annex 2) and “out of block” limits (annex 3).

- For the terminal stations (NB: which are fixed in the context of ECC Recommendation (04)05) only an “in block” limit is provided (annex 2). The equipment requirements in the relevant harmonised standards are considered to provide sufficient protection for adjacent networks, so that “out of block” BEM limits for terminals are not needed.

ECC Decision (11)06 There is no emission technical requirement.

Least restrictive technical conditions suitable for IMT systems with larger channel bandwidth are developed separetly.

Harmonization

ECC Decision (07)02 does not contain a harmonised band plan, since it refers to ECC/REC/ (04)05 for detailed frequency arrangements, which itself allows for flexibility and a mix of duplex modes.

ECC Decision (11)06 provides one harmonised band plan for the band 3600-3800 MHz (TDD) and two harmonised band plans for the band 3400-3600 MHz (FDD and TDD). ECC decided that the band plans for the band 3400-3600 MHz should be subject to review no later than end 2013 with the aim to identify a preferred band plan.

cept mandate

[To be added]

List of reference

- ECC Decision (07)02 on availability of frequency bands between 3400-3800 MHz for the harmonised implementation of Broadband Wireless Access systems (BWA)

- ECC Recommendation (04)05 onGuidelines for accommodations and assignments of multipoint fixed wireless systems 3.4-3.6 GHz and 3.6-3.8 GHz

- Commission Decision 2008/411/EC on the harmonization of the 3400-3800 MHz frequency band for terrestrial systems capable of providing electronic communications services in the Community).

- ECC Decision (11)06 on Harmonised frequency arrangements for mobile/fixed communications networks (MFCN) (including IMT) operating in the bands 3400-3600 MHz and 3600-3800 MHz

- Recommendation ITU-R M.1036

- ETSI EN 301 908-14: Harmonized EN for IMT-2000, Evolved Universal Terrestrial Radio Access (E-UTRA) (BS) covering the essential requirements of article 3.2 of the R&TTE Directive

- ECC Report 33 on the analysis of the coexistence of Point-to-Multipoint FWS cells in the 3.4-3.8 GHz band

- ECC PT1(09)109R1 Annex 16: “ERO Information Document on the use  of the bands 3400-3600 MHz and 3600-3800 MHz  in CEPT”, Final Minutes of the 32nd ECC PT1 meeting (27-29 April 2009)

For one to four antennas

For cross-border coordination with non-EU administrations not listed in the 5.430A footnote of RR the provisions of this footnote should be taken into account.