ECC PT1 Resolution of Comments on “Draft CEPT report 48 …”

1 	General Comments

| Comment number | Section number/ Clause | Paragraph Figure/ Table | Type of comment (General/ Technical/Editorial) | COMMENTS | Proposed change | Resolution analysis |
| --- | --- | --- | --- | --- | --- | --- |
| France/0 |  |  | General | France welcomes the opportunity to provide comments on the CEPT report 48. At high level, France agrees with the draft CEPT report 48 and its general conclusions on the MCA scenario for 1.8GHz and the 2GHz bands. This CEPT report refers heavily on the ECC 187 report (COMPATIBILITY STUDY BETWEEN MOBILE COMMUNICATION SERVICES ON BOARD AIRCRAFT (MCA) AND GROUND-BASED SYSTEMS) which was publically consulted on the 19.09.12 We anticipate that few changes should be required on the 1.8GHz scenario to reflect modifications on the report ECC 187 done during the resolution meeting. Any slight modifications around the conclusions shall dully be justified and discussed before any amendments of the CEPT report are approved. MCA scenario at 2.6 GHz: We understand that since the publication of the CEPT report 48 for public consultation, further work on the report ECC 187 based on the result of public consultation has been taking place. The conclusion on the MCA scenario at 2.6 GHz has been radically changed. In consequence, France considers that this new scenario at 2.6 GHz should be carefully considered and publically discussed in details before any decision on its inclusion in CEPT 48. At this stage, such MCA 2.6 GHz scenario cannot be included in the CEPT 48. | - | Comment taken into account. The section related to the 2.6 GHz has been amended to be in-line with the decisions at WGSE regarding ECC Report 187. |
| OnAir/0 |  |  | General | OnAir has considered the draft ECC report 48 “Report from CEPT to the European Commission in response to the Second Mandate to CEPT on mobile communication services on board aircraft (MCA)”. Below some editorial comments are provided. Additionally, following the public consultation on the ECC Report 187 on which the CEPT report 48 is based, revisions of some compatibility study results were received. Those modifications were agreed and adopted at the last SE7 meeting, and therefore included in the ECC report 187. It seems that those new elements should be also included in the CEPT Report 48 to be in line with the ECC report 187. | - | Noted |
| OnAir/1 |  |  | General | In some tables, the height above ground is expressed in meter while in other, in kilometer. It may be relevant to express all heights above ground in meter. | Proposed to express all heights in meter | Accepted |
| OnAir/2 | 0 |  | Editorial | During the public consultation on the ECC Report 187, it was shown that the compatibility between Radar and LTE in the 2.6 GHz band is feasible. Therefore, the compatibility study between LTE connectivity system onboard aircraft and ground network was carried out. The result of this compatibility study has to be included in the “executive summary” Additionally, a revision of the 2100 MHz band was received during the public consultation of the ECC Report 187 and agreed by SE7 to be included in the above mentioned Report. Therefore, the results of this study should be updated. Results of compatibility study in 1800 MHz has to be corrected | See attachment within ECC PT1(13)012 | Included apart from comments on 2.6 GHz The section related to the 2.6 GHz has been amended to be in-line with the decisions at WGSE regarding ECC Report 187. |
| OnAir/3 | 3.1 | Table 4 | Editorial | To add “RAS (2655-2690 MHz)” in the “2600 MHz” row and “in-band sharing” column, | See attachment within ECC PT1(13)012 | Accepted |
| OnAir/4 | 3.1 | Table 5 | Editorial | To add “RAS (2655-2690 MHz)” in the “2600 MHz” row and “sharing with ground based systems” column, | See attachment within ECC PT1(13)012 | Accepted |
| OnAir/5 | 3.2.1 |  | Editorial | In the second paragraph, the relative gain of the antenna should be read as “-1.84” and not “-0.34” | The relative antenna gain is -1.84 dBi | Accepted |
| OnAir/6 | 3.2.4 | Table 10 | Editorial | Additional text in the title of Table 10 to mention to what the result refer to | “simulation result for scenario 4(impact of UMTS connectivity system onboard aircraft to ground network) | Noted and modified text included |
| OnAir/7 | 3.2.4 | Table 11 | Editorial | Additional text in the title of Table 11 to mention to what the result refer to | “simulation result for scenario 4(impact of NCU to the ground LTE network) | Noted and modified text included |
| OnAir/8 | 3.2.5 and 3.2.6 |  | Technical/ editorial | During the public consultation, SE7 received comments with respect to the SEAMCAT simulation: - when the simulation was performed with SEAMCAT, the aircraft attenuation was not taken into account in the interfering transmitter power. - Additionally, when the simulation was performed with SEAMCAT for the busy days, the distribution of the aircraft height refers to the “normal day”. And a revision of the compatibility study (change of power level of UE and number of UE onboard). These proposals were agreed by SE7 and included into the ECC report 187. Therefore, it is proposed to update the section 3.2.5 and 3.2.6 in order to be in line with the ECC report 187 | Update tables in section 3.2.5 and 3.2.6 See attachment within ECC PT1(13)012 | Accepted |
| OnAir/9 | 3.2.6 |  | Technical/ editorial | To add a new table which provides the maximum EIRP defined outside the aircraft that the UMTS UE can transmit according to the height of the aircraft above ground. | See attachment within ECC PT1(13)012 | Accepted |
| OnAir/10 | 4.2.1 |  | Editorial | In the second paragraph, the relative gain of the antenna should be read as “-1.84” and not “-1.34” | The worst case elevation angle is 48 °, corresponding to an antenna gain of -1.834 dBi. | Accepted |
| OnAir/11 | New Section |  | Technical/ Editorial | During the public consultation, it was shown that compatibility between radars and onboard LTE is feasible (studies were reviewed and accepted by SE7 and included in the ECC report 187). Therefore, compatibility study between LTE onboard and ground network was performed. | It is propose to add a new section containing the result of the compatibility study | The section related to the 2.6 GHz has been amended to be in-line with the decisions at WGSE regarding ECC Report 187. |
| OnAir/12 | 3.5.2 |  | Technical / editorial | In the initial compatibility study, it was assumed that the maximum gain is at 90° elevation angle which is not the case. Based on tthe ECC report 174, it shows that the maximum elevation angle of meteorology radar antenna is 37° and the maximum gain for ATC radar is not at 90° elevation angle but rather between 0° and about 10 °. New result was received during the public consultation on the ECC report 187 and agreed to be included in the ECC report 187. | To update the value of the pv-rx and I+N/N in Table 24 “power density at victim receiver (radar) from 3000 m to 10 000 m “ and Table 25 “I+N/N at victim receiver” | The section related to the 2.6 GHz has been amended to be in-line with the decisions at WGSE regarding ECC Report 187. |
| OnAir/13 | 3.5.2 |  | Editorial | From the previous comment, to update the summary below Table 25 “I+N/N at victim receiver” | See attachment within ECC PT1(13)012 | The section related to the 2.6 GHz has been amended to be in-line with the decisions at WGSE regarding ECC Report 187. |
| OnAir/14 | 4 |  | Editorial | During the public consultation on the ECC Report 187, it was shown that the compatibility between Radar and LTE in the 2.6 GHz band is feasible. Therefore, the compatibility study between LTE connectivity system onboard aircraft and ground network was carried out. The result of this compatibility study has to be included in the “conclusion” Additionally, a revision of the 2100 MHz band was received during the public consultation of the ECC Report 187 and agreed by SE7 to be included in the above mentioned Report. Therefore, the results of this study should be updated. Results of compatibility study in 1800 MHz has to be corrected | See attachment within ECC PT1(13)012 | 2.6 GHz conclusions have been amended in-line with ECC Report 187 from the WGSE meeting |