- Table 15 Acceptable e.i.r.p. levels to avoid BS-BS interference, dBm/MHz e.i.r.p.

| Victim Interferer | MFCN Outdoor macro BS | MFCN Outdoor micro BS | MFCN Indoor pico BS | MFCN Indoor femto BS |
| --- | --- | --- | --- | --- |
| MFCN Outdoor macro BS | -41.9 | -15.6 | -15.9 | -15.9 |
| MFCN Outdoor micro BS | -29.6 | -44.9 | -22.0 | -22.0 |
| MFCN Indoor pico BS | -40.9 | -33.0 | -43.5 | -33.5 |
| MFCN Indoor femto BS | -40.9 | -33.0 | -33.5 | -33.5 |

OOB e.i.r.p. calculations

OOB e.i.r.p. = acceptable out-of-block e.i.r.p. emissions, i.e. emissions into the frequency block of the interfered base station measured after the transmitting antenna in the direction of the antenna boresight.

The baseline BEM level calculated here is based on Minimum Coupling Loss analysis for interference between base stations belonging to different operators, reflecting the need for worst-case analysis in the BS-BS interference scenarios. For each type of base station, such an MCL analysis is carried out for all other types of base stations. The strictest requirement obtained for each type of base station can then be used as the baseline level for OOB e.i.r.p. based on BS-BS interference. For BS – UE interference see Section X. Section Y summarizes the BEM requirements based on all these calculations and simulations.

Protection levels based on I/N = -6 dB:

Macro BS (NF 5 dB): -115 dBm/MHz

Micro BS (NF 8 dB): -112 dBm/MHz

Pico BS (NF 13 dB): -107 dBm/MHz

Femto BS (NF 13 dB): -107 dBm/MHz.

Using the acceptable interference as defined above, OOB e.i.r.p. can be determined from the following equations:

Iacc = OOB e.i.r.p. – Tx tilt/Tx antenna decoupling – Propagation Loss – wall penetration loss + Grx – Rx tilt/Rx antenna decoupling

OOB e.i.r.p. = Iacc + Tx tilt/Rx antenna decoupling + Prop loss + wall penetration loss – Grx + Rx tilt/Rx antenna decoupling

No feeder loss is assumed. All calculations are done for a bandwidth of 1 MHz. Error! Reference source not found. shows the minimum horizontal distance between different types of base stations. See Section X for antenna heights for different base stations.

Minimum horizontal distance between two Base Stations of different networks
 for the MCL calculations

| Minimum Horizontal Distance | MACRO | MICRO | PICO | FEMTO |
| --- | --- | --- | --- | --- |
| MACRO | 70 m | 30 m | 30 m | 30 |
| MICRO | 30 | 30 m | 15 m | 15 |
| PICO | 30 | 15 | 10 m | 10 |
| FEMTO | 30 | 15 | 10 | 10 |

BASELINE OOB E.I.R.P. LIMIT FOR MACROCELL BS [EDITOR’S NOTE: ALEX, CAN YOU PLEASE ALSO CHECK THE FORMAT FOR THE HEADINGS IN ANNEXES! I CANNOT CHANGE THEM TO THE ECC ANNEX HEADINGS WITHOUT CREATING A MESS…]

Macrocell BS to macrocell BS

It is assumed that the antennas of the two macro base stations are on the same level, and that there is an antenna decoupling loss of 4.8 dB at each antenna due to downtilt, 6 degrees, of the antennas.

Macro BS to Macro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -115.0 |
| Tx Downtilt Loss (dB) | 4.8 |
| PL (dB) | 80.5 |
| Wall penetration loss (dB) | 0 |
| - Rx Ant. Gain (dBi) | - 17 |
| Downtilt Loss (dB) | 4.8 |
| OOB e.i.r.p. Level (dBm/MHz) | -41.9 |

Macrocell BS to Microcell BS

In the co-existence scenario between Macro BS and micro BS, Macro BS antenna height is 30 m and micro BS antenna height is 6m. As a consequence of this height difference there is an additional antenna decoupling loss at both antennas, which is calculated with the Recommendation ITU-R F.1336 [8] sector antenna and omni antenna models, peak side lobes in both cases.

Macro BS to Micro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -112.0 |
| Tx antenna decoupling (dB) | 12.9 |
| PL (dB) | 75.2 |
| Wall penetration loss (dB) | 0 |
| - Rx Ant. Gain (dBi) | - 6 |
| Rx antenna decoupling (dB) | 14.3 |
| OOB e.i.r.p. Level (dBm/MHz) | -15.6 |

Macrocell BS to Picocell/Femto BS

In the calculation given in X for the co-existence scenario from macro BS to pico/femto BS, it is supposed that the pico/femto BS is placed roughly level with the base station, so that there is a worst case assumption of the main lobe of the macro BS antenna pointing directly at the pico/femto base station. It is assumed that there is a wall in-between the macro base station antenna and the antenna of the pico/femto cell.

Macro BS to Pico/Femto BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -107.0 |
| Tx antenna decoupling (dB) | 0 |
| PL (dB) | 73.1 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | - 0 |
| Rx antenna decoupling (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -15.9 |

Baseline OOB e.i.r.p. limit for Microcell BS

Microcell BS to Macrocell BS

Similarly to the Macro – micro case, antenna decoupling due to the vertical antenna diagrams of macro and micro have been applied.

Micro BS to Macro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -115.0 |
| Tx antenna decoupling (dB) | 14.3 |
| PL (dB) | 75.2 |
| Wall penetration loss (dB) | 0 |
| - Rx Ant. Gain (dBi) | - 17 |
| Rx antenna decoupling (dB) | 12.9 |
| OOB e.i.r.p. Level (dBm/MHz) | -29.6 |

Microcell BS to Microcell BS

The calculation of the baseline OOB e.i.r.p. level for Micro BS for the co-existence scenario Micro BS to Micro BS is summarized in the table below. As seen from the simulations (ref), there is an “interference margin” in the UL of micro cells, so we can assume there is an additional margin which has not been taken into account in the table below.

Micro BS to Micro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -112.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 73.1 |
| Wall penetration loss (dB) | 0 |
| - Rx Ant. Gain (dBi) | -6 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -44.9 |

Microcell BS to Picocell/Femtocell BS

The calculation of the baseline OOB e.i.r.p. level for Micro BS for the co-existence scenario Micro BS to Picocell/femtocell BS is summarized in Error! Reference source not found.. For this co-existence scenario, since Picocell/femtocell BS antennas are placed inside of building, an indoor penetration factor of 18 dB is used in the calculation of potential interference from the outdoor microcell BS to the indoor picocell/femtocell BS. No antenna decoupling has been assumed in these calculations, although there is a minor difference in micro and pico/femto BS antenna height even if the pico/femto base stations are located on the ground floor of the building.

Micro BS to Pico/Femto BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -107.0 |
| Tx antenna decoupling (dB) | 0 |
| PL (dB) | 67 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | - 0 |
| Rx antenna decoupling (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -22 |

baseline e.i.r.p. level for picocell bs

Picocell BS to Macrocell BS

The calculation of the baseline OOB e.i.r.p. level for pico BS with the co-existence scenario Pico BS to Macrocell BS is summarized in the table below. In the calculation, by considering picocell BS is inside of the building and macrocell BS is in outdoor area, an indoor penetration factor of 18 dB is used. No antenna decoupling loss is assumed, as the pico cell may be on the same level as the macrocell BS antenna.

Pico BS to Macro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -115.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 73.1 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | -17 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -40.9 |

Picocell BS to Microcell BS

The calculation of the baseline OOB e.i.r.p. level for pico BS with the co-existence scenario Pico BS to Microcell BS is summarized in Table xxx. In the calculation, by considering that the picocell BS is inside the building and the microcell BS is in an outdoor area, an indoor penetration factor of 18 dB is used.

Pico BS to Micro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -112.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 67.0 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | -6 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -33.0 |

Picocell BS to Picocell BS

The calculation of the baseline OOB e.i.r.p. level for picocell BS with the co-existence scenario Pico BS to Picocell/Femtocell BS is summarized in Error! Reference source not found.. In the calculation, free space propagation model is used in the pathloss calculation. It is assumed that there is no wall between the base stations.

Pico BS to Pico BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -107.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 63.5 |
| Wall penetration loss (dB) | 0 |
| - Rx Ant. Gain (dBi) | 0 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -43.5 |

Picocell BS to Femtocell BS

For the pico – femto scenario it is assumed that there is a wall of indoor type in-between the base station antennas, corresponding to 10 dB penetration loss.

Pico BS to Femto BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -107.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 63.5 |
| Wall penetration loss (dB) | 10 |
| - Rx Ant. Gain (dBi) | 0 |
| Rx Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -33.5 |

Baseline OOB e.i.r.p. limit for Femtocell BS

Femtocell BS to Macrocell BS

The calculation of the baseline OOB e.i.r.p. level for Femto BS with the co-existence scenario Femtocell BS to Macrocell BS is summarized in Error! Reference source not found.. In the calculation, an 18 dB indoor penetration loss is used.

Macro BS to Macro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -115.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 73.1 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | -17 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -40.9 |

Femtocell BS to Microcell BS

The calculation of the baseline OOB e.i.r.p. level for Femto BS with the co-existence scenario Femtocell BS to Microcell BS is summarised in Error! Reference source not found..

Femto BS to Micro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -112.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 67.0 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | -6 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -33.0 |

Femtocell BS to Picocell/Femtocell BS

The calculation of the baseline OOB e.i.r.p. level for Femto BS with the co-existence scenario Femto BS to Femtocell/Picocell BS is summarized in the table below.

Femto BS to Pico/Femto BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -107.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 63.5 |
| Wall penetration loss (dB) | 10 |
| - Rx Ant. Gain (dBi) | 0 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -33.5 |

Conclusions

Table ?? contains a summary of the results from the sections above. The most restrictive scenario for each type of base stations has been highlighted yellow. In Section X, these results and those from BS – UE and UE – BS interference analysis are used to derive the BS BEM.

OOB e.i.r.p. levels based on BS-BS interference, dBm/MHz e.i.r.p.

| Victim Interferer | MFCN Outdoor macro BS | MFCN Outdoor micro BS | MFCN Indoor pico BS | MFCN Indoor femto BS |
| --- | --- | --- | --- | --- |
| MFCN Outdoor macro BS | -41.9 | -15.6 | -15.9 | -15.9 |
| MFCN Outdoor micro BS | -29.6 | -44.9 | -22 | -22 |
| MFCN Indoor pico BS | -40.9 | -33.0 | -43.5 | -33.5 |
| MFCN Indoor femto BS | -40.9 | -33.0 | -33.5 | -33.5 |