| ECC PT1 |  | ECC PT1(13)XXX |
| --- | --- | --- |
| ECC PT1 – XO # 3 meeting CG on 3.5 GHz |  |  |
| ECO – Online Meeting – 21 March 2013 |  |  |
| Date issued: | 19th March 2013 |  |
| Source: | Ericsson Sweden |  |
| Subject: | LRTC for 3.4 – 3.8 GHz, simulation results for Indoor Scenarios |  |
| N Password protection required? (Y/N) |  |  |

Summary:

At ECC PT1 – XO # 2 meeting CG on 3.5 GHz, it was agreed to evaluate the impact of macro BS on indoor deployments of micro and pico BS for 3.5 GHz. This document encapsulates a comprehensive simulation analysis in the downlink, where the macro BS interferes the UEs connected to micro or pico BS that are deployed indoors. In particular, the following scenarios are considered: - Macro Vs Indoor Micro (Manhattan Grid Layout) Scenario - Case 1: All victim micro UEs are located indoors - Case 2: 50% of the victim micro UEs are located indoors and 50% are located on the streets in the Manhattan grid - Macro Vs Indoor Pico (Manhattan Grid Layout) Scenario - Case 1: All victim pico UEs are located indoors Note: Since pico cells are mainly intended for indoor coverage, we only consider the scenario where all the pico UEs are located indoors

Background:

The Correspondence Group on LRTCs for 3.4 – 3.8 GHz need to progress its work in order to meet the deadline established in the mandate from the European Commission.

Simulation Methodology:

Simulations are performed using the well known Monte-Carlo simulation methodology elaborated in [1]. The simulation parameters employed had been agreed at ECC PT1 – XO # 1 meeting CG on 3.5 GHz, unless stated otherwise. In general, the simulations are performed using the following procedure:

- Run the system under observation (victim system) independently without the impact of any aggressor in the adjacent band with the simulation parameters as mentioned in Table 1. This provides the baseline performance of the system (SINR, throughput, etc.)

- Introduce the aggressing system in the adjacent band without any additional isolation (i.e. restricted channel) and evaluate the impact on the victim system in terms of performance (throughput) degradation of the link

- Introduce a restricted channel (additional isolation) in between the two systems and repeat step 2 to identify the required restricted channel for acceptable performance of the victim system

Simulation Results:

References:

[1] – 3GPP TR 36.942

[2] – Report  ITU-R  M.2135-1, “Guidelines for evaluation of radio interface technologies for 	IMT-Advanced”, Section 1.3.1, Table A1-2

[3] - Recommendation ITU-R F.1336-3

[4] – 3GPP TR 25.942 Section 5.1.3.1.2