| Plenary |  |  | Doc. ECC(12)016rev1 |
| --- | --- | --- | --- |
| 31st Meeting |  |  |  |
| Tallinn, 29th May – 1st June 2012 |  |  |  |
| Date issued: | 25th May 2012 |  |  |
| Source: | ECO |  |  |
| Subject: | ECO Bulletin on ongoing/new issues in other regions or organisations |  |  |
| N Password protection required? (Y/N) |  |  |  |
| Summary: |  |  |  |
| This bulletin provides a summary update on aspects of the progress in the USA and the Asia Pacific Region of several issues already familiar to the ECC. These are inter-alia: 1. FCC – Incentive auctions to free broadcast spectrum in UHF 2. PPDR – FCC as well as APT as well as showing relation to WRC-15 new agenda items; 3. APT Wireless Group (AWG) latest activities, in particular the reports on trends in mobile communications and cognitive radio (two new APT Reports); 4. Major revision (4th) of IEEE 802.11 (has possibly an effect on some work items in the ECC such as industrial wireless applications); 5. MBANS (Medical Body Area Network System) – new FCC regulation finalised; 6. Medical Micro Power Networks (addition frequency opportunities for ULP-AMI) – new FCC regulation, report and order; 7. FCC Development to make MSS spectrum available for terrestrial broadband services; 8. Worldwide overview of Cognitive Radio solution regulations in TVWS available in EFIS; 9. 79 GHz Short Range Radar regulations worldwide set into place; 10. Overview UWB regulations outside Europe; 11. DA2GC (Direct-Air-to-Ground-Communication Systems) The more general review of activities in other regions has not identified developments which suggest new items of work to be considered (indeed, most of the items are already under progress in the ECC as well). An exception may be item 7 which might also show a tendency/shift on the market for MSS applications that has not been under discussion so far inside of the ECC and which might give an increased possibility to also use some of these MSS frequencies by terrestrial services in the future. Another new development is that the FCC started a process for a consultation (starting 30 May 2012) based on a Qualcomm proposal, to install in the band 14.-14.5 GHz (FSS Uplink) on secondary basis a terrestrial air-ground mobile service. This bulletin summarises the issues reported, with some original documents embedded electronically for reference. |  |  |  |
| Proposal: |  |  |  |
| This bulletin is to note by the ECC. More detailed input on some of the subjects covered is being input to the groups dealing with the respective subject. |  |  |  |
| Background: |  |  |  |
| The Office brings to each ECC meeting a bulletin on activities in radio communications in other world regions, where a regulatory dimension is raised (e.g. by innovative services or technology). The primary objective is to identify whether the ECC needs to investigate further or consider possible new actions. A secondary but more frequently addressed objective is to enable comparison to be made with the regulatory approach in other regions to subjects already treated by the ECC (including, where relevant, to the work of the CPG). |  |  |  |

# FCC – Incentive Auctions

Recent congressional approval of incentive auctions are “a piece of the puzzle to meet the spectrum crunch”, Julius Genachowski, chairman of the U.S. FCC told the National Association of Broadcasters trade show on April 16, 2012. “Developing this first-of-its-kind auction will be complex, involving many challenging questions” of economics, engineering, and law”. The Congressional action on incentive auctions codifies the voluntary nature of contributing spectrum, authorises the FCC to offer three unprecedented opportunities to share in auction proceeds: 1) contributing all 6 MHz; 2) channel sharing, and 3) moving from UHF to VHF. The action also preserves must-carry rights for channel sharing stations. It directs the FCC to make all reasonable efforts to preserve broadcaster coverage and it creates a $1.75 billion repacking fund to pay moving costs of broadcasters who don’t participate, as well as other incumbents.

# Public Protection and Disaster Relief

The 2012 WRC held in Geneva, Switzerland - 23 January-17 February 2012 adopted three new Resolutions which might also be relevant to this subject: COM6/8, COM5/10 and COM6/11. They were adopted under Conference Agenda 8.2 of the WRC-12 to appear as Agenda Items under WRC-15 and are as follows:

- to consider additional spectrum allocations to the mobile service on a primary basis and identification of additional frequency bands for International Mobile Telecommunications (IMT) and related regulatory provisions, to facilitate the development of terrestrial mobile broadband applications, in accordance with Resolution COM6/8 (WRC 12);

- to examine the results of ITU R studies, in accordance with Resolution COM5/10 (WRC 12), on the use of the frequency band 694-790 MHz by the mobile, except aeronautical mobile, service in Region 1 and take the appropriate measures;

- to review and revise Resolution 646 (Rev.WRC 12) for broadband public protection and disaster relief (PPDR), in accordance with Resolution COM6/11 (WRC 12).

Driven by many Asian countries the WRC adopted a resolution specific to PPDR. WRC-15 will review and revise Resolution 646 for broadband public protection and disaster relief (PPDR).

Resolution 646 is from WRC 2003, and specifies frequency bands for Narrowband, Wideband and Broadband usage. Region 1 did not specify any band for broadband.

What happens in Region 2 – Americas ? (here: the FCC)

On Friday February 16, 2012, the U.S. Congress passed a bill authorising the Federal Communications Commission (FCC) to reallocate the 700 MHz D-Block spectrum for dedicated public safety broadband use. In conjunction with the previously allocated public safety broadband block spectrum, U.S. public safety will now have a dedicated 20 MHz of broadband spectrum in 2 x 10 MHz contiguous blocks. Also of note, the U.S. Congress did not require the 700 MHz public safety narrowband spectrum to be returned for re-farming and authorised the FCC to determine whether the narrowband (NB) spectrum should be allocated in the future for public safety broadband use (BB) (potentially resulting in 30 MHz of broadband spectrum in 2 x 15 MHz contiguous blocks).

Some key elements of the legislation:

•Reallocates the 10 MHz D Block to Public Safety, and along with the Public Safety Spectrum Trust (PSST) 10 MHz, provides a total of 20 MHz broadband public safety spectrum;

•Establishes a Network Construction Trust fund of $7 billion (to build the nationwide BB-PPDR network);

•Establishes a temporary FCC Interoperability Board;

•Does not affect the 700 MHz public safety narrowband (i.e. P25 voice) spectrum.

What happens in Region 3 – Asia – Pacific?

APT wireless Group (AWG) has been working on the Digital Dividend issue and has developed two reports on the situation in APT member countries as well as preferred frequency arrangements for the use of the band 698-806 MHz, becoming available as part of the Digital Dividend.

APT Report: APT/AWF/REP-11 on "UHF Band Usage and Considerations for Realizing the UHF Digital Dividend"

APT Report: APT/AWF/REP-14 on "Harmonized Frequency Arrangements for the Band 698-806 MHz“

APT Report APT/AWF/Rep-24 on "Implementation Issues Associated with Use of the Band 698-806 MHz by Mobile Services"

APT Report: APT/AWF/REP-27 on "PPDR Applications Using IMT Based Technologies and Networks" (April 2012)

Many Asian countries are already actively moving to implement the results of these studies. For example, it is understood that Australia and India could be seeking to commence awarding licenses in the band in 2012.

APT plans also a workshop on 10-12 July 2012 on Disaster Management Communications.

# Other APT Reports

APT published in April 2012 two new Reports:

APT Report 28 on "Survey of Trend and Forecast on Mobile Communications in APT Countries"

APT Report 29 on “APT Survey Report on "Cognitive Radio Systems (CRS) and Software Defined Radio (SDR)"

Japan seems to be in the lead in Asia concerning the implementation of CRS in frequency regulations.

The Draft Report of the 12th Meeting of the APT Wireless Group (AWG-12), 10 – 13 April 2012, Xiamen, People’s Republic of China is attached here to give an overview of the APT work:

# IEEE – Fourth Update of 802.11

IEEE announced and published on 7th May 2012 the fourth revision of the standard to be released since its initial publication in 1997

(Press release: http://standards.ieee.org/news/2012/802.11-2012.html).

This amendment includes enhancements and consolidates 10 (ten) 802.11 standards such as 802.11n or 802.11p (latter one is supporting 5.9 GHz ITS) to the base standard that were approved since IEEE 802.11’s last full revision in 2007. Operation in the 3650-3700 MHz band, vehicular environments (ITS; 802.11p, now integrated in 802.11, is the base standard used by the European ITS standardisation, although with some adaptations), mesh networking, security, broadcast/multicast and unicast data delivery, interworking with external networks and network management have been included.

This has also effect on operations such as for industrial wireless applications in the 2.4 GHz band which can be operated with higher QoS requirements than before by using the 802.11 base standard.

(item already under discussion in WGFM SRD/MG)

# MBANS

The FCC Chairman announced that MBANS were on the agenda for the next open meeting scheduled for May 24, 2012 (thus indicating to publish very soon an order, i.e. new MANS regulations):

Medical Body Area Networks (MBANs) Report and Order and Further Notice of Proposed Rulemaking: The Commission will consider a Report and Order and Further Notice of Proposed Rulemaking to establish service rules and an allocation for Medical Body Area Networks and seek comment on the selection of an MBAN coordinator, dated 24 May 2012:

Background information:

The frequencies under consideration at the FCC are in 2360-2400 MHz. The concept on the proponent side is supported by Philips, GE Healthcare and the American Society Healthcare Association (latter one also the dedicated national database operator in the USA). The FCC will limit operations in the 2360-2390 MHz band to medical professionals in hospitals and a notification and coordination duty is to be followed. This is an automatic process and includes that all MBANS master equipment should be notified in a national database. MBANS masters again will provide electronic keys to all MBANS client equipment within the operating distance (thus ensuring the location and radius of operation). Hence, MBANS could be seen as a specific case of a professional SRD application applying a “geo-location database concept” including the possibility to quickly react in case of interference at a given location.

(item under discussion in WGFM SRD/MG)

# Addition frequency opportunities for ULP-AMI (new FCC regulation)

The Report and Order dated 9 May 2012 amends the MICS (Medical Implant Communication Systems) rules to allow them to operate in the 413-419 MHz, 426-432 MHz, 438-444 MHz, and 451-457 MHz frequency bands on a secondary basis. The ULP-AMI must be frequency agile so that they can switch among the four frequency bands so as to continue operating when use of a particular frequency band becomes too heavy. The requirements also limit the power (1mW e.i.r.p.) and length of time (low duty cycle and fast listen-before-talk) of the ULP-AMI transmissions.

(no such work item in the ECC so far)

# FCC Development to make MSS spectrum available for terrestrial broadband services

This is a very interesting development against the background of high-profile spectrum requirements benig stated for broadband services in Europe.   The details of the FCC proposals can be found at the following link. (release dated 21 March 2012): http://www.fcc.gov/document/fcc-proposes-40-mhz-additional-spectrum-mobile-broadband:

Service Rules for Advanced Wireless Services in the 2000-2020 MHz and 2180-2200 MHz Bands, WT Docket No. 12-70; Fixed and Mobile Services in the Mobile Satellite Service Bands at 1525-1559 MHz and 1626.5-1660.5 MHz, 1610-1626.5 MHz and 2483.5-2500 MHz, and 2000-2020 MHz and 2180-2200 MHz, ET Docket No. 10-142; Service Rules for Advanced Wireless Services in the 1915-1920 MHz, 1995-2000 MHz, 2020-2025 MHz and 2175-2180 MHz Bands, WT Docket No. 04-356.

Background:

In the MSS (today this is predominantly services towards ships (ESV) and planes (AES)), a clear market tendency is to provide services with higher data speeds, which in turn favours the use of Ku-/Ka-Band rather than L-Band/S-Band.  ECO has seen for example a rapid increase in the number of ESV notifications under ECC/DEC/(05)10 (Ku-Band, increase from 9 to 131 notifications by 30 April 2012 in last 12 months). Ship customers nowadays invest in Ku-band earth station equipment (and possibly also Ka-Band in the future when the Ka-Band satellites currently under construction will become available). Even satellite operators having no Ku-Band capacity of their own, lease Ku-and capacity to bridge the time until their own e.g. Ku-/Ka-Band new satellite(s) become operational in order to not lose their customers “fleet-by-fleet”).

More than a dozen of Ka-band satellites for longitudes over Europe (Inmarsat, SES, Eutelsat, Telenor and other) are under procurement and operators are late to respond to the market and wait eagerly that they can put them in operation. They also push for the new Ka Band licence-exempt Decision under development in the FM44 (ESOMP).

In addition, there is a new work item to find additional capacities in Ku-Band covered by a new work item for WRC-15.

The FCC seems to take into account this development away from the traditional L-Band/S-Band MSS capacity and considers opening of some frequencies on the US landmass also for terrestrial broadband services. This development/tendency (a certain shift from L-Band / S-Band MSS towards Ku-Band and Ka-Band) has also been mentioned on 10 May 2012 at the EC workshop on spectrum inventory/review in Bruxelles.

(subject as such has not yet been discussed somewhere in the ECC but relation to some earlier studies about PMSE applications in the MSS L-Band as well as question about potential usage of the 2 GHz MSS band in the future noting that one licensed operator has not procured space segment for it).

The FCC voted 3-0 to issue a notice of proposed rulemaking that explores how the S-band of MSS spectrum, which the FCC has renamed "AWS-4," should be designed so that the satellite spectrum can be repurposed for terrestrial use. The operator “Dish” currently owns 40 MHz of S-band spectrum--specifically from 2000-2020 MHz and 2180-2200 MHz--and is seeking waivers from the FCC to use it for terrestrial mobile broadband. Additionally, the FCC will look at repurposing the 1695-1710 MHz band for commercial mobile broadband, which could mean Dish's spectrum holdings might get shifted.

# Cognitive Radio solutions in TVWS:

A worldwide extensive overview of TVWS (TV White Space) activities (IEEE, PAWS, regulation in many jurisdictions) provided by Industry Canada has been installed in EFIS and is available in the spectrum inventory section under <<TVWS worldwide ECO 18 January 2012>>, ECO-ECC Document type.  See also Doc. ECC (12)022, which reports on the ECC’s own recent workshop on Cognitive Radio (including TVWS in its scope), which included contributions from outside Europe.

(files due to their size not embedded here)

# 79 GHz Short Range Radar

The implementation of 79 GHz SRR regulations worldwide can be seen under: http://www.79ghz.eu/ (there is a EU funded project for implementing 79 GHz sensor technology in cars).

In the last 12 months, a considerable number of countries have been added who introduced regulations for 79 GHz SRR: Canada, USA, Mexico, China, India, Japan, Korea, Singapore.

(This information is in relation to WRC-15, AI 1.18: to consider a primary allocation to the radiolocation service for automotive applications in the 77.5 – 78 GHz range in accordance with Resolution COM6/23 (WRC-12).

# UWB Regulations outside Europe (included for information due to the new Fifth EC mandate on UWB to the ECC)

The embedded file shows an overview of the UWB regulations outside of Europe.

Before the FCC, the following rule changes are under discussion:

- Automotive radar: withdrawal of the so-called stand still mode for 76-77 GHz

- Fixed radars at 76-77 GHz: same discussion as in Europe (no fixed installed radars allowed yet). The only fixed installations under discussion in the USA are applications at airports while WGFM/ SRDMG recently discussed radars at railway level crossings. A general request does not exist, like in Europe (final ETSI SRdoc on the subject did not contain any fixed installed radars).

- TLPR/LPR regulation in final drafting phase, similar to the already existing ECC regulations.

(All items under discussion in SRD/MG)

# DA2G

The North American Direct-Air-To-Ground network provider “Gogo” has initiated before the SEC (Securities and Exchange Commission) a process for filing an IPO (Initial Public Offering) to find investors contributing 100 million USD to the company. The related information to be provided for this IPO for potential investors shows the accumulated debt in the initial years of operations to be between 300 to 400 million USD while the company in 2012 also reached the break even. “Gogo” announced in the paper also their potential plans for services in China and India. In China, test trials are already being performed with the Chinese regulator and the Civil Aviation Administration of China (CAAC) being involved. It is expected that China will publish a regulation later in 2012 (very likely to use higher frequencies above 1 GHz). Meanwhile, in North America, the DA2GC system has been established with more than 1.500 commercial aircraft and 5000 business aircraft equipped so far.

(DA2GC is under discussion in FM48)

In addition, the FCC started a process for a consultation (starting 30 May 2012) based on a Qualcomm proposal, to install in the band 14.-14.5 GHz (FSS Uplink) on secondary basis a terrestrial air-ground mobile service. Qualcomm's estimate is that 150-250 ground stations would be necessary to cover the continental USA. Emissions toward aircraft would be pointed primarily northwards and thus exclude radiating into the geostationary arc and therefore avoid interference towards the FSS Ku-Band. In addition, aircraft earth stations (AES) are operated today primarily in Ku-Band (notified under ECC/DEC/(05)11 to the ECO) as well which could facilitate more compact common implementations. The existing Gogo-service has been criticised in the past for being in a “capacity-trap” due to the existing DA2G spectrum awarded until 2016 in the USA being only 3 MHz wide in total. This move could overcome this problem.

Also on the agenda are plans to make 25 MHz in the 2.3 GHz WCS band "more usable" and an initiative to open more Wi-Fi spectrum by freeing up an additional 120 MHz in the 5 GHz band, the FCC Chairman said.