Comments on ECC Deliverable under public consultation

“Draft CEPT Report 44 “In response to the EC Permanent Mandate on the ”Annual update of the technical annex of the Commission Decision on the technical harmonisation of radio spectrum for use by short range devices”

Sources:

Administration/Company/Entity: LPRA

Name and Appointment of contributor: Saad Mezzour – LPRA Chairman

1 	General Comments

The LPRA membership and in particular the Active Medical Implant membership has reviewed the draft CEPT Report 44 under public consultation and requests that the following comments be considered.

CEPT Report 44 in Section 3 page 9 contains several paragraphs describing the relationship between SRDs and “designated services” operating in the various bands recognized by the ITU-R and CEPT as listed in CEPT Report 25.  In general, the presumption is that SRDs operate without rights and with obligations.  They are not a recognized and defined radio service as generally listed in the ITU-R radio regulations and may be allowed only under the rule of “no harmful interference and no protection”.  A consequence of this is that neither the radio regulations nor the RTTE Directive give SRDs protection, including from other SRDs.  Thus the guiding principle for SRD access to bands with allocated services, primary and or secondary, is that compatibility studies must be made that show the potential a specific SRD and application have for causing interference to the services operating that band.

The SRD-MG through CEPT appears be proposing to violate the principles stated in the ITU-R Radio Regulations and the position stated in the current RTTE Directive by proposing that SRDs have equal access rights to frequency bands and because of this, have to protect each other to a reasonable and defined extent.  One has only to ask a simple question to realize such a policy is unworkable.  For example, if two different SRD proponents wish to use the same band and both are compatible with the existing services but are not compatible with each other, which one gets access to the band?  In another example, if SRD No 1 has been given access to a band and SRD No. 2 wishes to use the band but is not compatible with the use of the band by SRD No. 1, is SRD No. 2 denied access to the band?  If SRD No. 2 is denied access to the band, the effective status of SRD No.1 is elevated to that of a “service”. Of course the winners and losers could be picked by the authorities, but that would certainly lead to a potential abuse of regulatory power. What criteria would be used to pick winners and/or losers?

Unfortunately the above thought is carried forth in the discussion in section 5.2.12 on Active Medical Implants and peripherals.  In this section the CEPT report references the fact that the 401-402, 402 – 405, and 405 – 406 MHz bands have been removed from ERC/REC7-03 Annex 12 following the amendment of ERC/DEC /(01)17.  The amended ERC/DEC/(01)17 in Decides 1 states the following: “to harmonise the use of the frequency band 401 - 406 MHz for ULP-AMI communication systems on a secondary basis to meteorological radio services.  Thus ULP-AMI is allowed to use the band on a secondary basis to the METAIDS primary users.  CEPT Report 44 continues for over one and a half pages to discuss the previous SRD status of ULP-AMI and the usage conditions attendant to ULP-AMI but then it makes the following statement at 3 different places in the report: “It is recommended to add a footnote in the EC Decision which states that “ERC Decision (01)17 defines that protection of active medical implant communication systems from short range device applications shall be ensured in the 401-406 MHz frequency band”.  It is not explained what the purpose of the above statement is although Decides 4 of ERC/DEC/(01)17 is referred to.  Decides 4 states the following: “that protection of ULP-AMI communication systems from short range device applications shall be ensured in the 401-406 MHz frequency band”.  Decides 4 is a clear statement of the intent of the ECC Decision is to treat ULP-AMI operating in the 401-406MHz as a true secondary service and nothing more.  All services in any band are protected from SRDs desiring to use the band.

The SRD/MG also refused to recommend to the EC to remove the 401 – 406 MHz band from the EC Decision on SRDs as requested by ETSI, claiming it was not clear what benefit this would give to ULP-AMI users.  Leaving the band in the EC Decision will create a clear division between the regulatory path within the ECC and the EC bodies. The CEPT report should be modified by the SRD/MG and if protocol consideration prevent this action by the WGFM and ECC, then it should be sent back to the SRD/MG with clear instructions they should change their position and ask the EC to remove the band from the EC Decision on SRDs.

2	Proposals related to the ECC Deliverables

| Comment number | Section number/ Clause | Paragraph Figure/ Table | Type of comment (General/ Technical/Editorial) | COMMENTS | Proposed change |
| --- | --- | --- | --- | --- | --- |
| LPRA/1 | Section 3 page 9 |  | General | Section 3 page 9 contains several paragraphs describing the relationship between SRDs and “designated services” operating in the various bands recognized by the ITU-R and CEPT as listed in CEPT Report 25. In general, the presumption is that SRDs operate in general without rights and with obligations. They are not a recognized and defined as radio service as generally listed in the ITU-R radio regulations and may be allowed only under the rule of “no harmful interference and no protection”. A consequence of the above as stated is that neither the radio regulations nor the RTTE Directive give SRDs protection, including from other SRDs. | Add a specific section to the CEPT report 44 stating that the guiding principle for new SRD to access to bands allocated to services, primary, secondary and existing SRD users, is that compatibility studies must be made that show the potential a new SRD comer and application have for causing interference to the services and other users operating in that band. |
| LPRA/2 | Section 3 page 9 | 4th paragraph | General | The 4th paragraph on page 9 states the following “This means that neither the radio regulations nor the essential requirement of the R&TTE Directive give SRDs protection, including from other SRDs.”. It is not clear if it is protection from authorised or non-authorised SRD. | Replace the referred text with “This means that neither the radio regulations nor the essential requirement of the R&TTE Directive give SRDs protection, including from other authorized SRDs » |
| LPRA/3 | Section 3 page 9 | 5th Paragraph | General | The fifth paragraph on page 9 states the following “It is however seen to be the role of the regulators to ensure that SRDs do have equal access to frequency bands and would therefore have to protect each other to a reasonable and defined extent”. Same comment as LPRA/2 | Replace the referred text with “It is however seen to be the role of the regulators to ensure that authorized SRDs do have equal access to frequency bands and would therefore have to protect each other to a reasonable and defined extent”. |
| LPRA/4 | Section 3 page 9 | Last Paragraph | General | The last paragraph on page 9 states the following “This was usually interpreted as to avoid harmful interference to other short-range devices or share the spectrum equally between the SRD but the new ECC Report 181 [9] offers a more comprehensive methodology to accomplish this”. This sentence means that the interpretation is to avoid harmful interference only to other short range devices without ensuring protection to primary or secondary services. | Replace the referred text with “This was usually interpreted as to avoid harmful interference to Radio Services and other short-range devices or share the spectrum equally between the SRD but the new ECC Report 181 [9] offers a more comprehensive methodology to accomplish this”. |
| LPRA/5 | Section 4 page 10 | 4th Paragraph | General | The 4th paragraph in Section 4 states the following: “This increased the transparency for industry to find the relevant UWB regulation in one place. UWB-technology based SRD applications operating mainly in the frequency ranges 3.1-4.8 GHz and 6-9 GHz are covered by a different EC Decision [5], although the technology itself is used for short range wireless applications in support of broadband communication applications or the provision of location information with accurate resolution based on the usage of ultra-wideband(s).” The same as “UWB applications operating mainly in the frequency ranges 3.1-4.8 GHz and 6-9 GHz are covered by a different EC Decision” ULP-AMI applications operating in the 401-406MHz band needs to be covered by a different EC Decision | CEPT needs to propose to the Commission in the CEPT report 44 to remove the frequency band 401-406MHz used by ULP-AMI applications and cover these application in a separate EC Decision |
| LPRA/6 | Section 4.1.1 page 12 | Last Paragraph | General | The last paragraph in Section 4.1.1 states the following: “Finally, it needs to be emphasised that it is ultimately the responsibility of manufacturers to build short-range devices in a way to protect such devices against harmful interference to the extent possible and to minimise the risk of interference from radiocommunication services as well as from other short-range devices operating.” Same comment as LPRA/2 | Replace the referred text with “Finally, it needs to be emphasised that it is ultimately the responsibility of manufacturers to build short-range devices in a way to protect such devices against harmful interference to the extent possible and to minimise the risk of interference from radiocommunication services as well as from other authorised short-range devices operating in the band.” |
| LPRA/7 | Section 5 page 16 | 3rd Pragraph of Step 1 section | General | The Step 1 defines as “If "application category" = description of typical use or typical values of the technical characteristics (A)” | Clarify what is the action for step 1 “Replace the referred text with “If "application category" = description of typical use or typical values of the technical characteristics (A)” |
| LPRA/8 | Table 1 | Entry for Active Medical Implants | General | The column “Removal of application category required / possible” for the Animal implantable devices states that “No, an unlimited number of devices could cause interference to the services in this band.” The frequency bands used by Active medical implants used in humans have the same if not more justification for protection needs as the one for Animal implantable devices. | Replace the justifications for the column “Removal of application category required / possible” for the entry Active Medical implants by: “No, an unlimited number of devices could cause interference to the services and other users in this band.” To align with the same justification used for Animal implantable Devices. |
| LPRA/9 | Section 5.2.12 |  | General | The entire section 5.2.12 provides a significant argumentation and justification for the request from the Active Medical Industry, the LPRA and the ETSI to remove the frequency band 401-406MHz used by Active Medical Implants from the EC Decision on SRD. LPRA still request CEPT to consider all arguments and justification provided to remove the 401-406MHz band used by Active Medical Implants from the EC Decision on SRD. | Propose to the Commission in the CEPT report 44 to remove the frequency band 401-406MHz used by ULP-AMI applications and cover these application in a separate EC Decision |