| SE24 Meeting M66 Montegrotto, 27-28 August 2012 | M66_17R0_SE24 |
| --- | --- |
| - Date issued: 20th August 2012 - Source: BNetzA and Josef Schuermann - Status: For consideration - Subject: Test report of a measurement campaign for 13.56 MHz RFID Systems |  |

Summary: BNetzA has conducted a field test in conjunction with RFID Manufacturer FEIG and INFINEON in order to: - Characterize the emitted spectrum from lower emission/wideband and narrower band/higher emissions RFID systems which are covered by the proposed RFID emission mask. - Determine interference effects and required protection distances for Short Wwave Broadcasting as the most critical service affected

Proposal: Discuss and conclude the measurement campaign results, submit a recommendation to the next WGSE meeting in September.

Background: - ETSI has submitted to ECC the TR 103 059 which describes the need for a revised Transmitter mask in order to meet new RFID applications in applications as defined in ISO/IEC 14443, 15693 and 18000-3 for applications as NFC, e-Passports, Banking/payments, Libraries, Logistics ect. - Preliminary tests in of document M65_24R1_SE24 provided a first assessment of the potential interference but indicated the need for further investigations.