# Annex to STG(13)21 blocking interference

Content

Annex to STG(13)21 blocking interference	1

0	Correction of the calculation (see ticket #997)	2

0.1	Current implementation	2

0.2	Proposed changes	2

1	Consideration of the emission mask	4

1.1	Example	4

1.2	Current implementation	4

1.3	Trial to repair	5

1.4	Proposal	5

# Correction of the calculation (see ticket #997)

## Current implementation

Table 1: comparison of current implementation and needed changes

| mode | Calculation (corrected) | Comments | Currently implemented |
| --- | --- | --- | --- |
| Eq1 (mode: user defined): (no change) | Avr = Bmask | Here Bmask is similar to a “blocking response” (as used in STG(12)25) in dB (positive values) | Avr = Bmask |
| Eq2 (mode: protection ratio): | Avr_BlockingRatio=Bmask + C/(N+I) + (N+I)/N | Here Bmask is in dB (positive and negative values) | Avr_BlockingRatio=Bmask + C/(N+I) + 3 |
| Eq3 (mode: sensitivity): | Avr_sensitivity = Bmask + C/(N+I) – sensitivity_receiver - I/N | Here Bmask is similar to a “blocking level” i.e. the maximum acceptable interfering power in dBm | Avr_sensitivity = Bmask + C/(N+I) – sensitivity_receiver |
| Note: Bmask is the input from the user and extracted from the standard. |  |  |  |

## Proposed changes

Source on package org.seamcat.model.core

Class BlockingInterference.java

Proposed change for Eq2 (yellow marked)

Proposed change for Eq3 (yellow marked)

# Consideration of the emission mask

## Example

For whatever reasons the user may decide to define the transmitted power by the emission mask as shown in Figure 1 below:

Figure 1: transmit power defined by the emission mask

Note: in order to avoid a misinterpretation it is proposed the remove “(kHz)” from the above legend.

## Current implementation

- SEAMCAT gets the value of the input field “Power” taking account of its distribution (getTxTrialPower).

- SEAMCAT applies then the power control (if selected).

- SEAMCAT considers the antenna gains of ILT (rItVrAntGain) as well as VLR (rVrItAntGain).

- SEAMCAT does not consider any bandwidth in terms of power spectral density.

This leads to an erroneous calculation by not taking account of the defined power of (on the above example) 68 dBm/5 MHz.

## Trial to repair

Adding the above code to the method iRSSLinkBudgetBlock was the first trial to solve the problem. Unfortunately, in the environment of this method only the ArrayList “points” is made available, which is normalized to the bandwidth of 1 MHz and additionally, the call of the method getRefBandwidth() returns zero.

Note: the interface to the post processing plugin makes the origPoints available, i.e. one could correct the total power there.

## Proposal

As the reference bandwidth of the interfering transmitter is already required for the calculation of the co-channel part of the emission mask, it is proposed to change the calculation of the blocking interference in such a way, that the emission mask including its reference bandwidth is considered in general.