|  |  | Doc. STG(13)22 |
| --- | --- | --- |
| #### WGSE STG |  |  |
| #### 34th Meeting of SE STG #### Copenhagen, Denmark, 17 – 18 April 2013 |  |  |
| #### Date issued: | #### 17 April 2013 |  |
| #### Source: | #### Bundesnetzagentur |  |
| #### Subject: | #### Default values used by creating a new SEAMCAT workspace |  |

| Password protection required? (Y/N) | N |
| --- | --- |

# Introduction

The actual default values in SEAMCAT are useful to run the workspace without any warning messages. If the ticket #995 is applied, the consistency check is disabled. If the user causes one of the offered options the calculated values have to have in line with the consistency check.

# Proposal

The calculations of the values are:

CN+II=0dB=CN=Sensitivity - Noise Floor

The actual default value for the noise floor (N) is: -110 dBm

The actual default value for the sensitivity (C) is: -103 dBm

CI=CN+IN

N+IN =10∙log⁡10IN10+1

The noise floor (N) can be derived with:

N=10∙log10k∙T∙B+30

Per default B is 200 kHz, this leads to a noise floor of: -121 dBm

The values in the interference criteria panel should be not changed:

With the calculated noise floor of 121 dBm and the default value for C/(N+I) we get the following values:

Sensitivity= Noise Floor+CN+II=0dB=-121 dBm+16 dB=-105 dBm

Therefore it is proposed to change the default values for the noise floor and sensitivity.

Noise floor 	= -121dBm

Sensitivity 	= -105dBm

k = Boltzmann constant; T = 290K; B = Bandwidth