#### Post processing plugin ShowLBTapproachR3

This plugin is not intended to offer a solution for simulating some kind of time domain behaviour with SEAMCAT, it only wants to show how some of such a behaviour could comparatively easily be simulated using a plugin. It is up to those requesting the consideration of the time domain during SEAMCAT simulations, to decide if it and, which of it could be implemented by defining also the parameters needed and their values.

The source code is available as annex to this description. It should be noted that it has not been written by a professional Java programmer

Note: 
the plugin itself could “serve” more than one interfering system (interfering link) but it does not distinguish between those in terms of assigning each a different behaviour. It has also to be noted that the plugin does neither consider any antenna pattern nor any transmitter masks nor other propagation models than Extended Hata. It assumes always a co-channel outdoor scenario. The author is of the opinion that it is at that stage sufficient to show the principle. Further details might be a matter of subsequent versions.

Important note: 
this plugin assumes the duty cycle as the active fraction of an interval time, i.e.

with

and with DC in percent.

Description of „How it works”

As a post processing plugin it is called after each snapshot of the SEAMCAT simulation. Therefore it can only “manipulate” simulated results.

It offers these options:

- Simulate a variable packet length

- Simulate an LBT between the interfering transmitters

- Simulate a sensing between each interfering transmitter and the transmitter of the victim link

- Export some information about the simulation as an XML file
the corresponding XSD schema (simulateLBT.xsd) is separately available through the files attached

Figure 1: Configuration panel of the plugin

The main concept of simulating the time domain with this plugin is as follows:

Figure 2: principle of distributing packets randomly

#### Procedure:

- Collect data of the interfering transmitter
the information about each interfering transmitter (available through the plugin interface) is copied as an object “LBTcompontent” onto the list “simulatedTX” by setting at the same time the transmitter to “inactive” and assigning a random time within the interval time at which the transmission of the packet starts as well as the corresponding time the transmission of the packet stops.

- Perform “duty cycle”
The plugin defines a random time “slot” within the interval time and looks at each LBTcompontent whether at that time its packet is in transmission mode. If so, this LBTcompontent is set to “active” and copied onto the list “simulatedTXcollision”.

Option 1: simulate LBT between the interfering transmitters:
For all transmitters on the list “simulatedTXcollision” an LBT procedure is performed (TX1 vs, TX2, TX3 … TXn; TX2 vs. TX3, TX4  … TXn and so on) by checking whether the power received exceeds the threshold. If the limit is exceeded, this transmitter is set to “inactive”, labelled as “dropped due to LBTilt” and copied onto the list “simulatedTXdropped” and then removed from the list “simulatedTXcollision”, otherwise it remains “active”.

Option 2: simulate sensing of the ILT (transmitter of the victim link):
For each transmitter on the list “simulatedTXcollision” a sensing of the VLT is performed. In case the received power exceeds a limit (different from the ILT), this transmitter is set to “inactive”, labelled as “dropped due to LBTvlt” and copied onto the list “simulatedTXdropped” and then removed from the list “simulatedTXcollision”, otherwise it remains “active”.

- Consider the transmitters set to inactive  
The plugin checks which of the transmitters are inactive and decreases the corresponding interfering power (iRSS) by 200 dB, i.e. it “switches it off” and updates the resulting iRSS for this snapshot accordingly.

- Export information about the simulation (optional)
An XML file is generated containing this information:

the XML file could be imported into e.g. Microsoft Excel for post simulation analysis like this one:

Changes to version 3a:

Source:	Karl Koch; adhoc@heiseka.de

Note: the parameter “LBT listen time” is not used by this version of the plugin

Note: the length of the packet is given as input parameter and defers randomly from each other as a fraction of the input value for the packet length in case “simulate variable packet length” is selected. 
The unit [ms] is to be understood as an example and is never considered as such.

Added for version 3a: earliest start and latest stop of all occurrences are recorded.