SE(13)032A2

ECC Report <No>

COMPATIBILITY STUDIES – MSS TERMINALS TRANSMITTING TO A SATELLITE IN THE BAND 
1980 – 2010 MHz AND ADJACENT CHANNEL UMTS SERVICES

Month YYYY (Arial 9pt bold)

(last updated: Month YYYY) (Arial 9pt)

# Executive summary

The aim of this Report is to verify whether the conclusions of the ERC Report 065 are still valid when taking into account the characteristics of MSS terminals operating in the 1980 – 2010 MHz band contained in EN 302 574-2 and EN 302 574-3, considering also MSS terminals operating in a Complementary Ground Component (CGC).

The studies in the ERC Report 65 [3], relate to narrow band satellite transmissions conforming to ETSI TBR 42. Furthermore, the 2 GHz MSS network previously considered was MEO whereas the network under consideration of this Report is GSO.

UE terminals operating within a satellite/CGC systems are assumed to have a maximum output power of +24dBm, when operating to CGC base station networks, in conformance with ETSI EN 302 574-2 [4]. They are assumed to be built and to operate in similar ways as terrestrial ECN networks and to provide similar applications/services. Therefore, MSS terminal operated in a CGC mode are not studied in detail within this report.

This Report studies in detail potential interference from MSS UT transmitting to the satellite when in the vicinity of a base station or a UT of an ECN network operating in the 1920 – 1980 MHz and the 2010 – 2025 MHz bands.

Deterministic results show that when an MSS UT is near to a victim ECN BS, in the absence of any mitigation technique, the interference caused is above the recommended protection criterion based on I/N. As a consequence of these deterministic results, a complementary statistical analysis was also performed by using the SEAMCAT tool for studying the interference caused by MSS UT into ECN macro base stations and ECN UT.

From the results of the analysis, the following conclusions apply:

- The results for interference from MSS UTs into ECN MSs are all acceptable, and hence no further action is necessary to address adjacent band compatibility at the frequency boundary at 2010 MHz.

- The results for interference from MSS UTs into ECN BSs show that wideband MSS UTs cause acceptable loss of capacity to ECNs in the adjacent band. Although there is one case where the interference exceeds the capacity loss criterion, the average capacity loss for the whole network remains under that criterion. Hence, for the wideband MSS terminals no further action is necessary.

- The results for interference from MSS UTs into ECN BSs show that for the narrowband MSS UTs, there are several cases where the average capacity loss of the Reference cell and the whole network exceeds the criterion. For these cases, a more careful analysis was required. Such analysis showed that provided that a guardband of 300 kHz on the 1980 MHz band edge is maintained, no further action is necessary.

A guardband of 300 kHz on the 1980 MHz edge and a guardband of 500 kHz on the 2010 MHz edge, respectively, are adequate to protect ECN.

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	5

2	FREQUENCY USAGE	8

2.1	2 GHz band	8

3	SHARING SCENARIOS	9

4	ASSUMPTIONS AND METHODOLOGY	12

4.1	UMTS Base Stations characteristics and parameters	12

4.1.1	Antenna gain patterns:	12

4.1.2	Miscellaneous:	13

4.2	UMTS User Terminals characteristics and parameters	14

4.3	MSS User Terminals within the band 1980-2010 MHz, characteristics and system parameters	14

4.4	Frequency plans at the band edges	16

4.5	Propagation models	17

4.6	Methodology for deterministic analysis	17

4.7	Methodology for SEAMCAT analysis	18

5	analysis and results	21

5.1	ACIR for wideband MSS UT	21

5.2	ACIR for narrowband MSS UT	22

5.3	Calculation of the minimum required distance between an MSS UT and an ECN BS	23

5.4	Conclusion for the deterministic analysis	24

5.5	Statistical Study	25

5.6	Mitigation techniques to reduce interference from MSS to electronic communication networks	27

6	conclusions	29

ANNEX 1: FIGURES RELATED TO THE DETERMINISTIC ANALYSIS	31

ANNEX 2: Calculated ACIR value variation with frequency offset	39

ANNEX 3: FIGURES RELATED TO THE STATISTICAL ANALYSIS	41

ANNEX 4: List of reference	50

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| ACIR | Adjacent Channel Interference Ratio |
| ACLR | Adjacent Channel Leakage power Ratio |
| ACS | Adjacent Channel Selectivity |
| BS | Base Station |
| BTS | Base Transceiver Station |
| BW | Bandwidth |
| CDMA | Code Division Multiple Access |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| CGC | Complementary Ground Component |
| C/I | Carrier to Interference ratio |
| DEC | Decision |
| DECT | Digital Enhanced Cordless Telecommunications |
| ECA | European Common Allocation |
| ECC | European Communications Committee |
| ECC PT1 | European Communications Committee Project Team 1 |
| ECN | Electronic Communication Network |
| ERC | European Radiocommunications Committee |
| e.i.r.p. | Effective isotropic radiated power |
| ERP | Effective Radiated Power |
| ETSI | European Telecommunications Standards Institute |
| FDD | Frequency Division Duplex |
| IEEE | The Institute of Electrical and Electronics Engineers |
| IMT | International Mobile Telecommunications |
| ITU-R | International Telecommunication Union - Radiocommunication |
| MCL | Minimum Coupling Loss |
| MES | Mobile Earth Stations |
| MS | Mobile Station |
| MSS UT | Mobile Satellite System - User Terminal |
| Rx | Receiver |
| SEAMCAT | Spectrum Engineering Advanced Monte-Carlo Analysis Tool |
| TDD | Time Division Duplex |
| Tx | Transmitter |
| UE | User Equipment |
| UMTS | Universal Mobile Telecommunications System |
| WG FM | Working Group Frequency Management |
| 3GPP | The3rd Generation Partnership Project |

# Introduction

ECC Decision (06)09 [1] designates the bands 1980-2010 MHz and 2170-2200 MHz to Mobile Satellite Services (MSS), which may incorporate Complementary Ground Component (CGC). It also states that “mobile satellite systems operating in accordance with this Decision shall ensure compatibility with terrestrial systems operating in the mobile service in the adjacent bands below 1980 MHz and between 2010 MHz and 2170 MHz;”. It is, therefore, important to determine the extent of any interference issues between MSS/CGC and adjacent band IMT services.

The frequency allocations in the bands 1980-2010 MHz and 2170-2200 MHz and their respective adjacent bands are given in Table 1. The relevant adjacent bands are 1900-1980 MHz, 2010-2025 MHz and 2110-2170 MHz.

ECC Decision (06)01 [2] designates the adjacent bands mentioned above to IMT2000/UMTS. The band 1900 - 1980 MHz is designated for FDD uplink (mobile to base). The band 2110-2170 MHz is designated for FDD downlink (base to mobile) and the band 2010-2025 MHz is designated for either TDD or FDD uplink.

ERC Report 65 [3] (completed in 1999) contains comprehensive analyses of compatibility between UMTS and several other services in the 2GHz band. These other services include MSS in the bands 1980-2010 MHz and 2170-2200 MHz. However, the report did not cover the use of CGC base stations or user terminals accessing them.

Although not covered by ERC Report 65 [3], from the ETSI standards ETSI EN 302 574-1 [5] and ETSI EN 301 908-3 [6] it can be seen that the out of band emissions for CGC base stations are similar to those of UMTS 3GPP that already exist, since a similar technology is used. Furthermore, it is expected that the base stations of the two systems would use similar power levels and network deployment. Hence, the adjacency issues pertaining to these CGC base stations would essentially be identical to those that currently exist between different mobile network operators within the bands below 1980 MHz.

The analysis of adjacencies of 3GPP equipment and the use of new deployed FDD/TDD technology has been widely considered in Appendix 4 of CEPT Report 19 [8]] (for the band 2500-2690 MHz) and the issues are well explained and documented although no legacy equipment/system was available at the time the Report was published. The difference between the adjacencies for MSS CGC base stations at 2170-2200 MHz and the adjacencies for IMT base stations (2500-2690 MHz) are small. They both use IMT equipment built to ETSI standards that have out of band emission masks based on the same 3GPP standards described in CEPT Report 19 [8]].

CEPT Reports 19 [8] and 39 [9] also includes regulatory solutions based on the use of restricted blocks to mitigate against interference between base stations as it may occur at the 2010 MHz boundary., Considering that TDD networks are not largely deployed within the CEPT countries today, if ECN TDD Base Stations use the band 2010-2025 MHz and CGC base stations are deployed in the future in CEPT countries, further compatibility studies will be needed. Furthermore CGC BS parameters are quite similar to those of ECN FDD base stations, and it is believed that there is no compatibility issue between CGC base stations and ECN FDD base stations.

Table 1: Frequency allocations relevant to the bands 1980-2010 MHz and 2170-2200 MHz

| Allocation to services |  |  |  |
| --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |
| 1 930-1 970 FIXED MOBILE 5.388A5.388B |  | 1 930-1 970 FIXED MOBILE 5.388A5.388B Mobile-satellite (Earth-to-space) | 1 930-1 970 FIXED MOBILE 5.388A5.388B |
| 5.388 |  | 5.388 | 5.388 |
| 1 970-1 980 FIXED MOBILE 5.388A5.388B 5.388 |  |  |  |
| 1 980-2 010 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.3885.389A5.389B5.389F |  |  |  |
| 2 010-2 025 FIXED MOBILE 5.388A5.388B 5.388 |  | 2 010-2 025 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.3885.389C5.389E | 2 010-2 025 FIXED MOBILE 5.388A5.388B 5.388 |
| Allocation to services |  |  |  |
| Region 1 |  | Region 2 | Region 3 |
| 2 120-2 160 FIXED MOBILE 5.388A5.388B |  | 2 120-2 160 FIXED MOBILE 5.388A5.388B Mobile-satellite (space-to-Earth) | 2 120-2 160 FIXED MOBILE 5.388A5.388B |
| 5.388 |  | 5.388 | 5.388 |
| 2 160-2 170 FIXED MOBILE 5.388A5.388B 5.388 |  | 2 160-2 170 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.3885.389C5.389E | 2 160-2 170 FIXED MOBILE 5.388A5.388B 5.388 |
| 2 170-2 200 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A 5.3885.389A5.389F |  |  |  |
| 2 200-2 290 SPACE OPERATION (space-to-Earth) (space-to-space) EARTH EXPLORATION-SATELLITE (space-to-Earth) (space-to-space) FIXED MOBILE 5.391 SPACE RESEARCH (space-to-Earth) (space-to-space) 5.392 |  |  |  |

In the case of MSS/CGC user terminals, two new ETSI standards have recently been developed. One of these relates to MSS/CGC terminals with a bandwidth in the range 1 - 5 MHz The other one relates only to MSS user terminals with a bandwidth in the range 55 kHz - 1 MHz. The studies in the above mentioned ERC Report 65 [3], however, relate to narrow band satellite transmissions conforming to ETSI TBR 42. Furthermore, the 2 GHz MSS network previously considered was MEO whereas the network currently under consideration is GSO; therefore, different types of terminals and power levels may be used. This report then considers further compatibility studies using the new assumptions listed above, to determine whether the principal findings of ERC Report 65 [3] are still applicable. The wideband MSS User Terminals are assumed to use CDMA.

User Equipment (UE) terminals operating in ECN networks below 1980 MHz can have terminal powers operating up to 24 dBm. The satellite network operating in association with a Complementary Ground Component (CGC) provides additional terrestrial coverage and, therefore, the density of MSS terminals could exceed that considered in ERC Report 65 [3]. From a standard point of view, class 1 and 2 MSS terminals can connect to both satellite and CGC base stations; UE terminals operating within a satellite/CGC systems are assumed to have a maximum output power of +24dBm, when operating to CGC base station networks, in conformance with ETSI EN 302 574-2 [5] power class 3 and CGC networks are assumed to be built in similar ways as terrestrial ECN networks and provide similar applications/services. Therefore, these are not studied within this report. The SEAMCAT files used for the calculations are available in a zip-file at the www.ecodocdb.dk  in the same section where this Report is available.

# FREQUENCY USAGE

## 2 GHz band

The different services in 2GHz and adjacent bands are illustrated in Figure 1:. The more detailed situation for the MSS uplink based on ECC Decision 06(01) [2] dated 24th March 2006 is illustrated in Figure 2:. ECC Decision (06)01 was revised on 2nd November 2012 and it has maintained the 1920 – 1980 MHz band designation for FDD use. However, the 2010 – 2025 MHz band is now under separate review.

1980

1920

1900

TDD

FDD UL

MSS/CGC

2010

2025

2110

MSS/CGC

2170

2200

FDD

TDD

MHz

FS, SS(E-S)

FS, SS(S-E)

DECT

Figure 1: Services/systems around the 2 GHz bands

1979.7

2010.5

2025

1995

2110

2170

2185

2200

MSS operator 2

MSS operator 1

CGC/MSS Uplink

1980

2010

2169.7

MHz Band edge

Guard band relates to UMTS ONLY

1979.7

2010.5

2025

1995

2110

2170

2185

2200

CGC/MSS Uplink

1980

2010

2169.7

MHz Band edge

Guard band relates to UMTS ONLY

Figure 2: Information about the systems relevant for the studies in this Report

# SHARING SCENARIOS

The purpose of this Report is to analyse the compatibility between ECN and MSS UTs compliant with the ETSI standards 302 574-2 [5] (Wide band terminals) and 302 574-3 [5] (Narrow band terminals) in the 1980-2010 MHz band and ECN operating in 1900-1980 MHz, and 2010-2025 MHz, frequency bands. Both these standards specify new conditions for the MSS UTs such as output power, ACLR and others; in general, these technical characteristics are different from those considered in ERC Report 65 [3]. This report contains technical studies limited to the case of MSS UTs transmitting to a satellite in the band specified above. The studies involve MSS terminals with Tx power higher than those considered in ERC Report 65 [3].

There are two different frequency borders that are studied, at 1980 MHz and at 2010 MHz.

Consequently, two different scenarios are defined, each one dealing with an edge of the 1980-2010 MHz band:

Scenario A: MSS UT transmitting in the band 1980-2010 MHz  ECN BS receiving in the band 1920-1980 MHz – FDD operation

Scenario A.1: Wideband MSS

Scenario A.1.1: Wideband MSS UT -> ECN BS in rural environment (Macro cell)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area  with radius of  16.9 km

- 5 High gain terminals, transmitting at full power to the satellite, deployed in an area operating co-frequency (assuming CDMA) with radius of 16.9 km

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area with radius of  16.9 km

- 5 Low gain terminals, transmitting at full power to the satellite, deployed in an area operating co-frequency (assuming CDMA) with radius of 16.9 km

ECN terminals per ECN cell determined by SEAMCAT for the fully loaded (6 dB noise rise corresponding 75% cell load) system

Scenario A.1.2: Wideband MSS UT -> ECN BS in urban environment (Macro cell)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

Scenario A.1.3: Wideband MSS UT -> ECN BS in urban environment (Micro cell – studied with a deterministic approach only)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

Scenario A.1.4: Wideband MSS UT -> ECN BS in urban environment (Pico cell – studied with a deterministic approach only)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

- 1 Low gain terminal; transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

Scenario A.2: Narrowband MSS

Scenario A.2.1: Narrowband MSS UT -> ECN BS in rural environment (Macro cell)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area operating co-frequency radius of 16.9 km (no CDMA used)

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area operating co-frequency with radius of 16.9 km (no CDMA used)

Scenario A.2.2: Narrowband MSS UT-> ECN BS in urban environment (Macro cell)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

Scenario A.2.3: Narrowband MSS UT -> ECN BS in urban environment (Micro cell – studied with a deterministic approach only)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

Scenario A.2.4: Narrowband MSS UT -> ECN BS in urban environment (Pico cell – studied with a deterministic approach only)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

Scenario B: MSS UT transmitting in the band 1980-2010 MHz  ECN MS receiving in the band 2010-2025 MHz – TDD operation

Scenario B.1: Wideband MSS

Scenario B.1.1: Wideband MSS UT -> ECN MS in rural environment (Macro cell)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area with radius of  16.9 km

- 5 High gain terminals, transmitting at full power to the satellite, deployed in an area operating co-frequency (assuming CDMA) with radius of 16.9 km

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area with radius of  16.9 km

- 5 Low gain terminals, transmitting at full power to the satellite, deployed in an area operating co-frequency (assuming CDMA) with radius of 16.9 km

Scenario B.1.2: Wideband MSS UT -> ECN MS in urban environment (Macro cell)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

Scenario B.2: Narrowband MSS

Scenario B.2.1: Narrowband MSS -> ECN MS in rural environment (Macro cell)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area operating co-frequency with radius of 16.9 km (no CDMA used)

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area operating co-frequency with radius of 16.9 km (no CDMA used)

Scenario B.2.2: Narrowband MSS -> ECN MS in urban environment (Macro cell)

- 1 High gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

- 1 Low gain terminal, transmitting at full power to the satellite, deployed in an area with radius of 1.69 km

It should be noticed that the assessment of the interference of MSS UT transmitting in the band 1980-2010 MHz into ECN BS receiving in the band 2010-2025 MHz and operating in TDD mode is the same as that studied in Scenario 1, since the assumptions to be made are very similar.

For the two scenarios listed above, the wide and narrow band MSS UTs are studied both with deterministic analysis, and Monte Carlo simulations; the latter approach is performed by using the SEAMCAT simulation software.

Moreover, a Minimum Coupling Loss (MCL) method is used to analyse the interference between stations without taking statistical aspects into account, providing the necessary attenuation required between MSS and ECN to enable interference-free operation under specified conditions.

# ASSUMPTIONS AND METHODOLOGY

Figure 3: below illustrates the interfering paths studied in this Report. In general, an MSS UT is supposed to transmit to a satellite in the 1980 – 2010 MHz band; therefore the following systems may be interfered due to unwanted emissions and the adjacent channel selectivity of the receiver:

- ECN FDD Base Stations receiving in the 1920 – 1980 MHz band;

- ECN TDD Base Stations receiving in the 2010 – 2025 MHz band;

- ECN TDD User Terminals receiving in the 2010 – 2025 MHz band.

Wanted signal paths

Interference paths

CGC BS

TDD MS

TDD BS

FDD BS

FDD MS

MSS UT

Figure 3: Interference scenarios

In the paragraphs below, the assumptions and the methodologies used for deriving the results in Section 5 are listed.

## UMTS Base Stations characteristics and parameters

The following parameters are applicable to the BS operating both in FDD mode (in the 1920-1980 MHz band) and in TDD mode (in the 2010-2025 MHz band).

### Antenna gain patterns:

The gain patterns for the ECN BS used in the calculations are listed in the following Table 2:

Table 2: Parameters for BS antenna

| Type of analysis | Antenna gain pattern |
| --- | --- |
| Deterministic analysis | Macro and micro BS: Recommendation ITU-R F.1336-2, recommends 3.1 [10] (“Peak side-lobe pattern”) Pico BS: Omni antenna |
| Statistical analysis with single MSS interferer | Macro BS: Recommendation ITU-R F.1336-2, recommends 3.1 [10] (“Peak side-lobe pattern”) |
| Statistical analysis with multiple MSS interferers | Macro BS: Recommendation ITU-R F.1336-2, recommends 3.2 [10] (“Average side-lobe pattern”) |

### Miscellaneous:

The parameters listed in the following Table 3: are applicable to the ECN BS and have been used in the calculations:

Table 3: Parameters for ECN BS

|  | Macro BS | Micro BS | Pico BS |
| --- | --- | --- | --- |
| BS output power at antenna connector (dBm) | 43 |  |  |
| BS Antenna Gain (dBi) | 18 | 5 | 0 |
| Feeder loss (dB) | 3 | 1 | 0 |
| Reference sensitivity (dBm/ 3.84 MHz) | -121 | -111 | -107 |
| Wanted signal mean power (dBm/ 3.84 MHz) | -115 | -105 | -101 |
| Channel bandwidth (MHz)* | 5 | 5 | 5 |
| 1st channel ACS (dB)(± 5 MHz) | 46 | 46 | 46 |
| 2nd channel ACS (dB) (± 10 MHz) | 58 | 53 | 54 |
| Maximum power interfering signal 1st ch. (dBm) | -62.7 | -52.7 | -48.7 |
| Maximum power interfering signal 2nd ch. (dBm) 1 | -50.7 | -45.7 | -40.7 |
| Noise Figure (dB) | 5.4 | 15.4 | 19.4 |
| Typical cell radius (km) | 6.0 (rural) 1.0 (urban) | 0.315 | 0.04 |
| Antenna height (m) | 45 (rural) 30 (urban) | 5 | 2 |
| Antenna down tilt (deg) | 3 | 0 | 0 |

*Note, 5 MHz is the carrier separation, the effective channel bandwidth is 3.84 MHz

Noise figure (dB) was derived based on a desensitization value of 6 dB (3GPP TS 25.104) using the following formula: ACS_relative = ACS_test – Noise_floor – 10 log10 (10M/10 – 1) (see ITU-R Report 2039 [4]);

Maximum power interfering signal was calculated based on a desensitization value of 1 dB (corresponding to I/N= -6 dB) and used for the deterministic study.

## UMTS User Terminals characteristics and parameters

The parameters listed in the following Table 4: are applicable to the ECN User Terminals:

Table 4: Parameters for ECN UT

| Parameter | Value |
| --- | --- |
| Maximum output power (dBm) | 24 |
| Dynamic power control range (dB) | 74 |
| Antenna gain (dBi) | 0 (omni) |
| Feeder loss (dB) | 0 |
| Reference sensitivity (dBm) | -117 |
| Channel bandwidth (MHz)* | 5 |
| 1st channel ACS (dB)(± 5 MHz) | 33 |
| Maximum power interfering signal 1st ch. (dBm) | -72.1 |
| Noise Figure (dB) | 9 |
| Antenna height (m) | 1.5 |

*Note, 5 MHz is the carrier separation, the effective channel bandwidth is 3.84 MHz

## MSS User Terminals within the band 1980-2010 MHz, characteristics and system parameters

This Section lists the parameters valid for MSS User Terminals.

In general, two types of UT are identified: wideband (operating with carrier bandwidths of 1 MHz or greater) and narrowband (operating with carrier bandwidths of less than 1 MHz). Each of them is referring to the ETSI standard EN 302 574-2 [4] and EN 302 574-3 [4], respectively.

For each of these two types, a given MSS UT can be a “High” or “Low” gain terminal the first type refers to UTs equipped with a directional antenna, while the antenna of the second type of terminals can be assumed isotropic. The following Table 5: and Table 6: resumes the relevant parameters and assumptions applicable:

Table 5: Parameters for MSS UT

| Parameter | Wideband High Gain | Narrowband High Gain | Wideband Low Gain | Narrowband Low Gain |
| --- | --- | --- | --- | --- |
| Maximum antenna gain (dBi) | 15 | 15 | 0 | 0 |
| Antenna gain pattern | As per rec. 4.1 of Rec. ITU-R F.1336-2 [10] | As per rec. 4.1 of Rec. ITU-R F.1336-2 [10] | Isotropic | Isotropic |
| Maximum output power at antenna connector (dBm) | 33 | 30 | 39 | 39 |
| Minimum antenna elevation (deg) | 5 | 5 | - | - |
| Typical antenna elevation (deg) | 20 | 20 | - | - |
| Carrier bandwidth | 5 MHz | 200 kHz | 5 MHz | 200 kHz |
| Antenna height (m) | 1 | 1 | 1.5 | 1.5 |

Table 6: Power classes for wideband terminals (from ETSI EN 302 574-2[4])

| Power Class 1 |  | Power Class 1bis |  | Power Class 2 |  | Power Class 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power (dBm) | Tol (dB) | Power (dBm) | Tol (dB) | Power (dBm) | Tol (dB) | Power (dBm) | Tol (dB) |
| +39 | +2.7/-2.7 | +33 | +1/-3 | +27 | +1/-3 | +24 | +1.7/-3.7 |

The characteristics of the Adjacent Channel Leakage Ratio (ACLR) applicable to wideband terminals can be derived from Section 4.2.7 of the ETSI standard EN 302 574-2 [5] mentioned above. The relevant parameters are resumed in the following Table 7:

Table 7: Unwanted emission mask and ACLR for wideband MSS terminals (from ETSI EN 302 574-2)[5]

|  | 1st adjacent channel |  | 2nd adjacent channel |  |
| --- | --- | --- | --- | --- |
| Type of terminal | ACLR (dB) | absolute (dBm/5 MHz) | ACLR (dB) | absolute (dBm/5 MHz) |
| High Gain | 42 | +6 | 52 | -4 |
| Low Gain | 42 | -3 | 52 | -13 |

Nevertheless, the same information for narrowband terminals is not contained in the ETSI standard EN 302 574-3 [4]. The best information that can be extrapolated from the document is contained in its Sections 4.2.2 and 4.2.3, dealing with the maximum e.i.r.p. spectral density of the unwanted emissions from the UE within and outside the 1980-2010 MHz band (shown in Table 8:). Taking also into account the information contained in the previous Table 6:, an ACLR can therefore be calculated. The relevant results are shown in Table 9: and Table 10:.

Table 8: OOB emission mask for narrowband MSS terminals (calculated from ETSI EN 302 574-3 [5]) – values are absolute power in dBm in the Adjacent Channels 5 MHz wide dBm

| Guard band (kHz) | 1st adjacent channel | 2nd adjacent channel | 3rd adjacent channel |
| --- | --- | --- | --- |
| 0 | 34.0 | -13.0 | -17.8 |
| 100 | 14.7 | -13.0 | -17.8 |
| 200 | 14.7 | -13.0 | -17.8 |
| 300 | 14.6 | -13.0 | -17.8 |
| 400 | 14.6 | -13.0 | -17.8 |
| 500 | 14.5 | -13.0 | -17.8 |

Table 9: Calculated ACLR for narrowband high gain MSS terminals (calculated from ETSI EN 302 574-3[5]) – values are in dB and refer to Adjacent Channels 5 MHz wide

| Guard band (kHz) | 1st adjacent channel | 2nd adjacent channel | 3rd adjacent channel |
| --- | --- | --- | --- |
| 0 | 11.0 | 58.0 | 62.8 |
| 100 | 30.3 | 58.0 | 62.8 |
| 200 | 30.3 | 58.0 | 62.8 |
| 300 | 30.4 | 58.0 | 62.8 |
| 400 | 30.4 | 58.0 | 62.8 |
| 500 | 30.5 | 58.0 | 62.8 |

Table 10: Calculated ACLR for narrowband low gain MSS terminals (calculated from ETSI EN 302 574-3 [5]) – values are in dB and refer to Adjacent Channels 5 MHz wide

| Guard band (kHz) | 1st adjacent channel | 2nd adjacent channel | 3rd adjacent channel |
| --- | --- | --- | --- |
| 0 | 5.0 | 52.0 | 56.8 |
| 100 | 24.3 | 52.0 | 56.8 |
| 200 | 24.3 | 52.0 | 56.8 |
| 300 | 24.4 | 52.0 | 56.8 |
| 400 | 24.4 | 52.0 | 56.8 |
| 500 | 24.5 | 52.0 | 56.8 |

## Frequency plans at the band edges

This section outlines the frequency plans at the band edges for both the MSS and ECN systems.

There are two ETSI standards for the MSS/CGC user terminals, one using narrowband signals for satellite use and the other for wideband signals for satellite or CGC use. Both ETSI standards have a range of carrier bandwidths within which manufacturers and operators can develop their networks. The narrow band terminal bandwidths are from 55 kHz to 1 MHz while the wideband terminals bandwidths are from 1 MHz to 5 MHz.

Taking into account the parameters listed in Table 5: for the 1980 MHz band edge, the frequency plan applicable to the MSS user terminals could be:

Table 11: Frequency plan for the 1980 MHz band edge – MSS

| 1980 MHz MSS Wideband: 5 MHz UT centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 1982.5 | 1987.5 | Etc. |

T1980

| 1980 MHz MSS Narrowband: 200 kHz UT centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 1980.1 | 1980.3 | Etc. |

SS Wideband: 5 MHz UT centre frequencies (MHz)

Similarly, for the 2010 MHz band edge, the frequency plan applicable to the MSS user terminals could be:

Table 12: Frequency plan for the 2010 MHz band edge – MSS

| 2010 MHz MSS Wideband: 5 MHz UT centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 2007.5 | 2002.5 | Etc. |

| 2010 MHz MSS Wideband: 200kHz UT centre frequencies (MHz) |  |  |
| --- | --- | --- |
| 2009.9 | 2009.7 | Etc. |

ECC/DEC/(06)01 [2] defines in Annex 1 the centre frequencies of the carriers nearest to the MSS band edges. Taking into account the channel bandwidth specified in Tables 3 and 4, two guard bands (spacing between the edges of the two services) are identified: a guardband of 300 kHz on the 1980 MHz edge and a guardband of 500 kHz on the 2010 MHz edge. The frequency plan Illustrated in the following Table 13: and Table 14: can then be outlined accordingly.

Table 13: Frequency plan for the 1980 MHz band edge – ECN

| 1980 MHz ECN FDD 5 MHz BScentre frequencies (MHz) – guard band of 300 kHz |  |  |
| --- | --- | --- |
| 1977.2 | 1972.2 | Etc. |

Table 14: Frequency plan for the 2010 MHz band edge – ECN

| 2010 MHz ECN TDD: 5 MHz BS/UT centre frequencies (MHz) – guard band of 500 kHz |  |  |
| --- | --- | --- |
| 2013 | 2018 | Etc. |

## Propagation models

The propagation model to be used in the calculations varies depending on the particular scenario considered. The following Table 15: provides the right propagation model for each scenario:

Table 15: Propagation models

| Scenario | Propagation Model | Notes |
| --- | --- | --- |
| MSS UT into ECN Macro BS in rural environment | Extended Hata – Rural | Outdoor and victim receiver “above roof”. |
| MSS UT into ECN Macro BS in urban environment | Extended Hata – Urban | Outdoor and victim receiver “above roof”. For deterministic study: Urban Hata. For statistical study use the Extended HATA model integrated in Seamcat. |
| MSS UT into ECN Micro/Pico BS in urban environment | Equation (5) in Annex 1 of Recommendation ITU-R P.1411-5 | Outdoor and victim receiver “below roof” Median value of LoS propagation model within street canyons. |
| MSS UT into ECN UT | Free-space | To be used in deterministic analysis. |
| MSS UT into ECN UT | IEEE 802.11 - C | To be used in statistical analysis. |

## Methodology for deterministic analysis

This approach consists in determining the minimum distance at which an MSS User Terminal can generate an unacceptable interference into a particular ECN Base Station; the same approach can also be used for calculating the minimum required guard band to be put in place.

In order to determine the interference due to both the in-band and out-of-band emissions of an MSS UT, a methodology aligned with that presented in the ECC Report 131 [8] – “Derivation of a block edge mask (BEM) for terminal stations in the 2.6 GHz frequency band (2500-2690 MHz)” has been used.

Figure 4: below illustrates the basic concept of Adjacent Channel Interference Ratio (ACIR). The ACIR is a parameter quantifying the interference caused in the adjacent channel as a result of two different phenomena:  on the one hand, interference is caused to the victim receiver by out-of-band emissions from the adjacent system (blue shaded area on the right of the figure). The Adjacent Channel Leakage Ratio (ACLR) quantifies the degree to which this takes place.

A second source of interference is the victim receiver’s ability to reject signals in the adjacent channel otherwise known as the Adjacent Channel Selectivity (ACS). This is illustrated by the red shaded area (left hand shaded area) in Figure 3:

When considering the adjacent channel interference between two systems operating in adjacent frequencies to each other, the ACLR of the interferer and the ACS of the victim receiver should be combined to give the overall ACIR using the formula below:

ACIR = 1/(1/ACLR + 1/ACS)								(1)

The ACIR is then a measure of the degree of isolation between systems operating in adjacent frequencies to each other and represents the degree of protection afforded to the receiver.

The formula in (1) shows that where one of the factors in the equation is much less than the other then it will tend to limit the overall ACIR. This is also evident from Figure 4:, where the total interfering power is the combination of the two shaded areas. If one is very much greater than the other then it will dominate the ACI performance between the two systems. If, for example, the ACLR is 10 dB lower than the ACS, then the overall ACIR will only be 0.4 dB worse than the ACLR.

Figure 4: Description of ACIR, ACS and ACLR

To assess adjacent band compatibility between MSS UTs and ECN, it is necessary to determine the ACIR, taking into account the values of ACS and ACLR listed in the previous Table 4:, Table 7: and Table 8:.

In the calculations it is assumed that the boresights of the MSS UT and ECN BS (or UT) antenna are always lying on the same plane, when these are directional. Yet in this case, results have been calculated for 4 different antenna elevation angles (5, 10, 15 and 20 deg); this is why they are sometimes shown as a range in the sections which follow.

## Methodology for SEAMCAT analysis

This section presents the methodology of simulations addressing those applicable scenarios established in this report to analyse potential interference to ECN networks from the operation of MSS UE.

Parameters of modelled land-based UMTS systems were established in accordance with the main parameters set out in Table 3: and Table 4:. In addition to those, several other secondary yet important parameters needed for SEAMCAT simulations were assumed as follows:

Table 16: Additional parameters used to define victim UMTS systems

| Paremeter | Value |
| --- | --- |
| Voice activity factor | 1 (Note 1) |
| Receiver noise figure: Macro BS | 5.4 dB |
| Voice bit rate | 12.2 kbps (Note 2) |
| Link Level Data sets | W-CDMA/UMTS: SEAMCAT: 1900MHz; 1 % FER |
| Target network noise rise for CDMA Uplink | 6 dB |
| UMTS BS’ adjacent channel selectivity/blocking rejection | See Table 3 |
| UMTS cell radius | See Table 3 |
| Cell type | 3 - sector antenna |
| Initial UMTS capacity, MS per sector | Generated and optimized by SEAMCAT |
| Propagation model for links inside victim CDMA system | See explanations in Table 15: |

Note 1: 	 Please visit the SEAMCAT on-line manual at http://tractool.seamcat.org/wiki/Manual for the explanation of how the voice activity factor is taken into account for the calculations of results;

Note 2: 	only voice communication channel is modelled in SEAMCAT CDMA module for certain simplification reasons (i.e. providing stable, non-bursty communication).

It should be further noted that interference impacts CDMA system differently than a “traditional” system. In non-CDMA system, the victim is passive with regards to the external interference, and interference criterion (C/I) considered as a trigger for interference occurrence.

It should be further noted that interference impacts CDMA systems and “traditional” systems in different ways. In non-CDMA systems, the victim is passive with regards to the external interference, and a criterion based on the C/I ratio is considered as the trigger for determining the occurrence of interference.

But when the victim is a CDMA system, it may use its inherent power tuning mechanism to try to compensate for the interference received, up to a point when relevant network resources reach their limits and the victim system starts to disconnect some of the earlier associated users. The interference here is therefore measured not in terms of probability of exceeding the C/I criterion, but in terms of probability of exceeding a certain capacity loss. In order to model this power tuning process correctly, the SEAMCAT tool builds a cluster of 19 CDMA sites (57 cells) and further complements it for the effect of “endless network” by applying a certain “wrap-around” technique.

This scenario describes the possibility that MSS UT’s are interfering into victim CDMA BS receivers (uplink). The physical outline of this scenario was derived by randomly positioning 1 (or 5) UT within an area with radius of 16.9 km for rural environment and with radius of 1.69 km for urban environment from the central (reference) cell of an ECN network. A snapshot taken from the status window of a SEAMCAT simulation is shown below in Figure 5. The red dots indicate the location of the MSS UTs, while the green dots indicate the location of the ECN BS.

When considering MSS UT operating in the same location as that where the interfered UMTS network is located, results are presented:

- CDF of the capacity loss and the average of the capacity loss of ECN Network over the whole deployment area;

- CDF of the capacity loss in only one cell within the ECN Network deployed in the simulation area;

- CDF of the capacity loss of the cell within which the MSS user terminal is located for every snapshot.

At each snapshot SEAMCAT randomly positions 1 (or 5) interfering transmitter(s), depending on the considered scenario.

Figure 5: Outline of SEAMCAT simulations

# analysis and results

The deterministic calculations are performed according to the scenarios defined in the previous Section 3, using the parameters and the methodology contained in Section 4. At the same time, without considering the 300/500 kHz guard bands that exist at the 1980 and 2010 MHz borders, the interference generated by an UMTS terminal equipped with an omni-directional antenna and transmitting a maximum power of 24 dBm was selected as the criterion to trigger a deeper analysis through a statistical approach. Therefore, for a given scenario, if results obtained through a deterministic approach are considered acceptable, no statistical analysis is performed for that particular scenario.

Where required, statistical studies are performed using the SEAMCAT tool. However in the case of Micro and Pico cells, the SEAMCAT tool (or any other freely available tool) is unable to model such use adequately, since it is not currently possible to model a realistic deployment of Micro and Pico cells as deployed within a typical real network. Consequently, even where statistical analysis of interference to a Micro or Pico cell might be required, such analysis is not performed.

## ACIR for wideband MSS UT

The calculated first and second adjacent channel ACIR values for wideband MSS UT with respect to the ECN Base Stations for the 1980 and 2010 MHz boundaries are shown in the Table 17: below.

Table 18: contains, instead, the ACIR value with respect to the ECN User Terminals for the 2010 MHz boundary.

When assessing the ACIR with a guard band applied at one edge of the band, the values of ACLR and ACS have appropriately been re-calculated taking into account the values available for both the 1st and the 2nd adjacent channels. In Annex 2 are figures showing how ACIR values vary with frequency offset (guard).

Table 17: ACIR for wideband MSS User Terminals vs. ECN Base Stations

| Boundary | UMTS channel centre frequency (MHz) | Scenario | ACLR (dB) first adjacent channel | ACS (dB) first adjacent channel | ACIR (dB) first adjacent channel | ACIR (dB) second adjacent channel |
| --- | --- | --- | --- | --- | --- | --- |
| 1980 MHz or 2010 MHz | 1977.5 (FDD) or 2012.5 (TDD) | A.1 | 42 | 46 | 40.5 | 51.0 |
| 1980 MHz | 1977.2 (300 kHz guard band) (FDD) | A.1.1 and A.1.2 | 42.2 | 46.3 | 40.8 | 51.3 |
|  |  | A.1.3 | 42.2 | 46.2 | 40.7 | 49.6 |
|  |  | A.1.4 | 42.2 | 46.2 | 40.8 | 50.0 |
| 2010 MHz | 2013 (500 kHz guard band) (TDD) | A.1.1 and A.1.2 | 42.4 | 46.4 | 41.0 | 51.3 |
|  |  | A.1.3 | 42.4 | 46.4 | 40.9 | 49.7 |
|  |  | A.1.4 | 42.4 | 46.4 | 40.9 | 50.1 |

Table 18: ACIR for wideband MSS User Terminals vs. ECN User Terminals

| Boundary | UMTS channel centre frequency (MHz) | Scenario | ACLR (dB) first adjacent channel | ACS (dB) first adjacent channel | ACIR (dB) first adjacent channel | ACIR (dB) second adjacent channel |
| --- | --- | --- | --- | --- | --- | --- |
| 2010 MHz | 2012.5 (TDD) | B.1 | 42 | 33 | 32.5 | 32.5 |
| 2010 MHz | 2013 (500 kHz guard band) (TDD) | B.1 | 42.4 | 33 | 32.5 | 32.5 |

## ACIR for narrowband MSS UT

The calculated first and second adjacent channel ACIR values for narrowband MSS UT with respect to the ECN Base Stations for the 1980 and 2010 MHz boundaries are shown in the Table 19 below.. The ACS of ECN BS was defined for a wideband interferer, and it is assumed that the same value applies to the narrow band interferer of 200 kHz. As it can be seen in table 19, the dominant contribution to ACIR is the narrowband MSS UT ACLR.

Table 20 contains, instead, the ACIR value with respect to the ECN User Terminals for the 2010 MHz boundary.

When assessing the ACIR with a guard band applied at one edge of the band, the values ACS have appropriately been re-calculated taking into account the values available for both the 1st and the 2nd adjacent channels. In Annex 2 are figures showing how ACIR values vary with frequency offset (guard).

Table 19: ACIR for narrowband MSS User Terminals vs. ECN Base Stations

| Boundary | UMTS channel centre frequency (MHz) | Scenario | ACLR (dB) first adjacent channel | ACS (dB) first adjacent channel | ACIR (dB) first adjacent channel | ACIR (dB) second adjacent channel |
| --- | --- | --- | --- | --- | --- | --- |
| 1980 MHz or 2010 MHz | 1977.5 (FDD) or 2012.5 (TDD) | High Gain UT | 11 | 46 | 11 | 52.5 |
|  |  | Low Gain UT | 5 | 46 | 5 | 49.9 |
| 1980 MHz 1980 MHz or 2010 MHz | 1977.2 (300 kHz guard band) (FDD) 1977.5 (FDD) or 2012.5 (TDD) | A.2.1.a and A.2.2.a | 30.4 | 46.3 | 30.3 | 55 |
|  |  | A.2.1.b and A.2.2.b | 24.4 | 46.3 | 24.4 | 51 |
|  |  | A.2.3.a | 30.4 | 46.2 | 30.3 | 51.8 |
|  |  | A.2.3.b | 24.4 | 46.2 | 24.4 | 49.5 |
|  |  | A.2.4.a | 30.4 | 46.2 | 30.3 | 52.5 |
|  |  | A.2.4.b | 24.4 | 46.2 | 24.4 | 49.9 |
| 2010 MHz | 2013 (500 kHz guard band) (TDD) | A.2.1.a and A.2.2.a | 30.5 | 46.4 | 30.4 | 55 |
|  |  | A.2.1.b and A.2.2.b | 24.5 | 46.4 | 24.5 | 51 |
|  |  | A.2.3.a | 30.5 | 46.4 | 30.4 | 52.5 |
|  |  | A.2.3.b | 24.5 | 46.4 | 24.5 | 49.5 |
|  |  | A.2.4.a | 30.5 | 46.4 | 30.4 | 52.5 |
|  |  | A.2.4.b | 24.5 | 46.4 | 24.5 | 49.9 |

Table 20: ACIR for narrowband MSS User Terminals vs. ECN User Terminals

| - Boundary | UMTS channel centre frequency (MHz) | Scenario | ACLR (dB) first adjacent channel | ACS (dB) first adjacent channel | ACIR (dB) first adjacent channel | ACIR (dB) second adjacent channel |
| --- | --- | --- | --- | --- | --- | --- |
| 2010 MHz | 2012.5 (TDD) | B.1 – High Gain MSS UT | 11 | 33 | 11 | 33 |
|  |  | B.1 – Low Gain MSS UT | 5 | 33 | 5 | 32.9 |
| 2010 MHz | 2013 (500 kHz guard band) (TDD) | B.1 – High Gain MSS UT | 30.5 | 33 | 28.6 | 33 |
|  |  | B.1 – Low Gain MSS UT | 24.5 | 33 | 23.9 | 32.9 |

The ACS of ECN UT was defined for a wideband interferer, and it is assumed that the same value applies to the narrow band interferer of 200 kHz. As it can be seen in Table 20:, the dominant contribution to ACIR is the narrowband MSS UT ACLR.

## Calculation of the minimum required distance between an MSS UT and an ECN BS

The following Table 21: shows the results obtained through a deterministic analysis of the adjacent channel interference scenarios listed in Section 3. Annex 1 contains the calculated deterministic figures and Table 21: presents the results as the minimum required separation distance to fulfil the I/N requirement. For the MSS UTs equipped with a directive antenna, will the minimum required separation distance depend on the elevation angle. Some results are presented as a range, in order to take into account of the off-axis angle between the boresight of the UT antenna and that of the ECN base station.

Table 21: Deterministic Study Results (minimum separation distance in km)

|  |  | Minimum Required distance (km) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scenario | Elevation angle 5 deg |  | Elevation angle 10 deg | Elevation angle 15 deg | Elevation angle 20 deg |
| A.1.1.a | 1.5-5.7 |  | 1.5-5.4 | 1.5-4.8 | 1.5-4.2 |
| A.1.1.c | 3.6 |  | 3.6 | 3.6 | 3.6 |
| A.1.2.a | 0.1-0.6 |  | 0.1-0.6 | 0.1-0.5 | 0.1-0.5 |
| A.1.2.b | 0.4 |  | 0.4 | 0.4 | 0.4 |
| A.1.3.a | 0.2-0.7 |  | 0.2-0.7 | 0.2-0.6 | 0.2-0.5 |
| A.1.3.b | 0.5 |  | 0.5 | 0.5 | 0.5 |
| A.1.4.a | 0.1-0.3 |  | 0.1-0.3 | 0.1-0.2 | 0.1-0.2 |
| A.1.4.b | 0.2 |  | 0.2 | 0.2 | 0.2 |
| A.2.1.a | 2.3-9.2 |  | 2.3-8.6 | 2.3-7.7 | 2.3-6.6 |
| A.2.1.b5 | 10.0 |  | 10.0 | 10.0 | 10.0 |
| A.2.2.a5 | 0.2-1.0 |  | 0.2-0.9 | 0.2-0.8 | 0.2-0.7 |
| A.2.2.b5 | 1.0 |  | 1.0 | 1.0 | 1.0 |
| A.2.3.a5 | 0.3-1.1 |  | 0.3-1.0 | 0.3-0.9 | 0.3-0.8 |
| A.2.3.b5 | 1.4 |  | 1.4 | 1.4 | 1.4 |
| A.2.4.a5 | 0.1-0.4 |  | 0.1-0.4 | 0.1-0.4 | 0.1-0.3 |
| A.2.4.b5 | 0.6 |  | 0.6 | 0.6 | 0.6 |
| B.1.1.a | 1-13.2 |  | 1-11.7 | 1-9.5 | 1-7.2 |
| B.1.1.c | 4.9 |  | 4.9 | 4.9 | 4.9 |
| B.1.2.a | 1-13.2 |  | 1-11.7 | 1-9.5 | 1-7.2 |
| B.1.2.b | 4.9 |  | 4.9 | 4.9 | 4.9 |
| B.2.1.a | 1-14.6 |  | 1-12.9 | 1-10.6 | 1-8.0 |
| B.2.1.b | 13.1 |  | 13.1 | 13.1 | 13.1 |
| B.2.2.a | 1-14.6 |  | 1-12.9 | 1-10.6 | 1-8.0 |
| B.2.2.b | 13.1 |  | 13.1 | 13.1 | 13.1 |

Note: for a given base station, when results are indicated in range, the higher distance is the worst case for that particular base station.

## Conclusion for the deterministic analysis

The approach followed in Section 5.1 of this study shows the interference that could be generated by the MSS User Terminals’ In-Band emissions to ECN BSs and ECN Uts out of band emissions in the 2 GHz band.

Calculations of Section 5.3 related to the analysis of Scenario 1 determine the region within which an MSS UT transmitting to the satellite is generating interference greater than the maximum power shown in Table 3:. The following Table 22: resumes the results obtained:

Table 22: Deterministic study results

| Cell type | Environment | Range (km) | Worst case |
| --- | --- | --- | --- |
| Macro | Urban | 0.1-1.0 | Narrowband low gain UT in rural environment @ 5 deg el |
| Macro | Rural | 1.5-10.0 | Narrowband low gain UT in rural environment @ 5 deg el |
| Micro | Urban | 0.2-1.4 | Narrowband low gain UT in urban environment @ 5 deg el |
| Pico | Urban | 0.1-0.6 | Narrowband low gain UT in urban environment @ 5 deg el |
| ECN UT | Urban/Rural | 1-14.6 | Narrowband high gain UT in rural environment @ 5 deg el |

Table 22: shows that when the interfered system is a Macro BS, the minimum required distance between it and a transmitting narrowband MSS UT can be greater than the typical ECN cell radius; for a given base station, when results are indicated in range, the higher distance is the worst case for that particular base station.

The required separation distances between MSS UT to ECN Micro and Pico BS can be larger than the ECN microcellular/picocellular cell ranges.

Given the results for the macro cell BS and for the ECN UT summarised in the table above, further analysis is conducted using SEAMCAT to assess the real risk of interference.  The SEAMCAT analysis is contained in the next section.

## Statistical Study

Taking into account the assumptions in Section 4 and in the subsections above, simulations have been run using a Monte - Carlo approach, using the ECO SEAMCAT software. The following additional parameters have been used for defining the various workspaces:

Number of snapshots for a single simulation = 2000 events;

Voice activity factor = 1.

High gain MSS UT elevation angle = 20 deg.

- The following additional parameter has been used for defining the various workspaces relative to the interference assessment of MSS UT to ECN Uplinks:

    - Target cell noise rise = 0.8 dB.

It should be noted that, for simplifying the simulation work, in the scenarios for the low gain, narrowband UTs, the ACLR values used in SEAMCAT were those given in Table 9 which are actually those applicable to the high gain narrowband UTs (i.e. 11 dB ACLR in the first adjacent channel).

The following vectors have then been used for determining the required statistics:

“Capacity Loss (active users), system” for determining the system capacity loss;

“Capacity Loss (active users), Reference cell” for determining the capacity loss in the Reference cell;

“Capacity Loss (active users), Worst cell” for determining the capacity loss in the Worst cell for every snapshot

See Figure 5 for the representation of the interference scenario.

All the figures relative to the CDF of capacity loss are provided in Annex 3. ECN Uplink and Downlink average capacity loss are summarized in Tables 23 and 24, respectively.

Table 23:  Results of the statistical study (Scenario A)

| Scenario | Interferer | whole Network (Average capacity loss) | Reference Cell (Average Capacity loss) | Worst Cell (Average Capacity loss) |
| --- | --- | --- | --- | --- |
| A.1.1.a | Wideband, rural, 1 highgain UT | 0.3% | 0.4% | 11.5% |
| A.1.1.b | Wideband, rural, 5 highgain UTs | 0.9% | 1 % | 37.6% |
| A.1.1.c | Wideband, rural, 1 lowgain UT | 0.7% | 1.74% | 29.7% |
| A.1.1.d | Wideband, rural, 5 lowgain UTs | 3.2% | 6.9% | 81.2% |
| A.1.2.a | Wideband, urban, 1 highgain UT | 0.1% | 0.6% | 5.5% |
| A.1.2.b | Wideband, urban, 5 high gain UTs | 0.3% | 2 % | 12.5% |
| A.2.1.a | Narrowband, rural, 1 high gain UT | 7.4% | 11.1 % | 96.4% |
| A.2.1.b | Narrowband, rural, 1 low gain UT | 23 % | 41 % | 98.7% |
| A.2.2.a | Narrowband, urban, 1 high gain UT | 3 % | 11.7% | 82.2% |
| A.2.2.b | Narrowband, urban, 1 low gain UT | 11 % | 39.3% | 96.7% |

Table 24: Results of the statistical study (Scenario B)

| Scenario | Interferer | whole Network (Average capacity loss) | Reference Cell (Average Capacity loss) | Worst Cell (Average Capacity loss) |
| --- | --- | --- | --- | --- |
| B.1.2.a | Wideband, urban, 1 high gain UT | 0 % | Never interfered | Not Applicable |
| B.1.2.b | Wideband, urban, 1 low gain UT | 1.7 % | 0.01% | N.A. |
| B.2.1.a | Narrowband, rural, 1 high gain UT | 1.8 % | Never interfered | N.A. |
| B.2.1.b | Narrowband, rural, 1 low gain UT | 1.8 % | Never interfered | N.A. |
| B.2.2.a | Narrowband, urban, 1 high gain UT | 1.7 % | 0.1 % | N.A. |
| B.2.2.b | Narrowband, urban, 1 low gain UT | 4.3 % | 0.4 % | N.A. |

The SEAMCAT Monte-Carlo simulation results for different defined scenarios are summarised in Table 23 and Table 24. In the simulations, the interfering MSS terminals (1 or 5) are randomly distributed within the simulation area composed of 57 cells, as shown in Figure 5.

The whole network average capacity loss is the system capacity loss averaged over the number of simulation snapshots (2000) and over the number of 57 cells (19 tri-sector sites).

The Reference cell is the cell located in the centre of the 57 cells. The figure for the capacity loss in the Reference cell is then taken as the average capacity loss over the 2000 samples.

The Worst cell is the most impacted cell at each simulation snapshot and it can in fact be a different cell for each snapshot. The figure for the capacity loss in the Worst cell is then taken as the average capacity loss over the 2000 samples.

The simulation results given in the Table 23 and Table 24 show that:

For the same type of MSS terminals, the ECN UL capacity loss is dependent on the number of interferers, For example, the system capacity loss in scenario A.1.1.b (5 MSS UT) is higher than that of scenario A.1.1.a (1 MSS UT).

The average ECN UL whole network capacity loss varies from 0.1% to 23% depending on the scenario.

For the ECN UL case, the Reference cell average capacity loss varies from 0.6% to 41%, depending on the scenario.

For the ECN UL case, the Worst cell average capacity loss varies from 5% to 98%, depending on the scenario.

For the ECN DL case, the average capacity loss (system and ref cell) is always below 4%.

## Mitigation techniques to reduce interference from MSS to electronic communication networks

In general, the interference from MSS terminals into ECN BS and UTs comes from two contributions: the out-of-band emissions of MSS terminals (dependent on their ACLR) and the selectivity/blocking capability of the ECN BS and UTs receiver (dependent on their ACS).

If it was necessary to reduce the harmful interference caused by MSS UT transmitting to a satellite into ECN, the following possible mitigation techniques (or a combination of them) could be used.

Table 25: Possible mitigation techniques

| Mitigation technique | Comments |
| --- | --- |
| Reducing the power of the interfering MSS UT | Reducing the interfering transmitter output power will reduce the overall interference level. Applying this as a general method might have negative impact on the provision of the MSS service, especially, if the power reduction is applied to all MSS UT. The level of the required reduction is dependent on the specific situation. |
| Adjusting BS antenna characteristics (height, pattern, tilt and direction) taking into account local conditions | May lead to a reduction of coverage and transmitted power in some directions, and thereby reduce throughput. 2 GHz band FDD ECN network is in commercial service, it is very difficult to modify the network. More suitable for situations where an MSS terminal with directional antenna is permanently (or for relatively long periods of time) located at a fixed position. Will require joint planning between the two services. |
| Definition of an affected area around each potentially affected base station | The affected area would differ depending on the environment and output power from MSS terminal. |
| Additional filtering in MSS UTs in order to increase their ACLR | A measure to minimize the unwanted emissions from the MSS terminal, and, thereby, to reduce the minimum coupling loss (MCL) required to avoid interference to the ECN, if it is dominated by the ACLR. A filter might have some impact on the MSS link budget (increased insertion loss). Does not solve any blocking problem in ECN network. |
| Additional filtering in public mobile networks base station, ACS | A measure to reduce the power received from the MSS terminal in band emission, possibly increasing the ACIR if it is dominated by the ACS and reducing the required MCL to avoid interference to the base station of public mobile network. A filter might have some impact on the link budget (increased insertion loss, contribution to receiver noise figure), 2 GHz FDD ECN networks are in commercial use, it is difficult to install additional filters to the ECN FDD BS. Does not solve MSS terminal unwanted emission problem. |
| Frequency separation, guard band between MSS terminal and the terrestrial ECN | An additional guard band between the MSS and the terrestrial ECN will reduce MSS terminal unwanted emission into ECN network. Similar methods that is already introduced from terrestrial ECN side. A guard band will also reduce blocking problem in ECN network. Reduce the already limited spectrum resources of both systems. More a regulatory rather than a technical measure to avoid interference. |
| Usage of the MSS frequencies near frequency edges for CGC. | With same output powers from MSS terminals and power control algorithm as public mobile networks the interference would not be different than today’s interference from an existing public network on other side in frequency |

# conclusions

Results shown in the previous sections address potential issues when an MSS UT transmitting to the satellite is in the vicinity of a base station or a UT of an ECN network.

Deterministic results show that when an MSS UT is near to a victim ECN BS, in the absence of any mitigation technique, the interference caused is above the recommended protection criterion based on I/N. Nevertheless, it should also be taken into account that ECN networks are built with more than one base station. Table 21 shows, in some cases, a range of distances in which interference would be caused to the victim ECN BS; in those cases, the lower distance may mean that the interferer is pointing towards another base station.

The deterministic study of micro and pico base stations reveal that the minimum distance that gives protection from interference is between 200 meters and 1400 meters for a micro base station and between 100 meters and 600 meters for a pico base station. In most cases these distances exceeds the normal cell radius of the (pico or micro) base station.

As a consequence of these deterministic results, a statistical analysis was also performed as complementary analysis.

A statistical analysis of interference into micro and pico base stations has not been performed in this study due to the current limitations of the SEAMCAT tool in modelling networks composed of different type of cells in a single simulation.

Statistical analysis has however been performed with the SEAMCAT tool for studying the interference effects into ECN macro base stations and ECN UT.

The report takes and uses the criteria for assessment of the statistical results; MSS terminal interference to the UMTS network is considered sufficiently low provided that the capacity loss is no more than 5 % on average over the different snapshots.

The mobile communities had a concern that the worst performance in a cell should not be averaged over the entire network, as it may hide the fact that a particular cell may be interfered (i.e. Worst cell analysis was proposed). However, the results in Table 23 for the “Worst Cell (average capacity loss)” is essentially a deterministic worst case result, assuming that the nearest cell to the MSS terminal is always the reference point, this is not a realistic  assumption as terminals are mobile. Therefore, the Worst Cell column should not be used as a basis   for the conclusion of the statistical studies results, even though these results do suggest high values of capacity loss for a different cell each time.

Therefore, the 5% interference criterion specified above is applied to the capacity loss of the whole network and the Reference cell for drawing conclusions from this analysis.

From the results summarised in Table 23 and Table 24, the following conclusions apply:

The results for scenario B (interference from MSS UTs to ECN MSs) are all acceptable, and hence no further action is necessary to address adjacent band compatibility at the frequency boundary at 2010 MHz.

The results for scenario A show that wideband MSS UTs cause acceptable loss of capacity to ECNs in the adjacent band. Although there is one case (scenario A.1.1.d) where the interference to the Reference cell exceeds the 5% capacity loss criterion, the average capacity loss for the whole network remains under 5%.  Also, some of the mitigation factors identified above, together with the assumption that the MSS UT and ECN equipment may perform better than the conditions specified in the ETSI standards, would reduce interference in a real situation below the 5% criterion for the Reference cell.  Hence, for the wideband MSS terminals no further action is necessary.

For the narrowband MSS UTs, there are several cases where the average capacity loss of the Reference cell and the whole network exceeds 5%.  For these cases, more careful analysis is required, discussed below.

From the derivation of the ACIR as shown in Table 19, it can be seen that the ACIR is dominated by the relatively poor ACLR in the first adjacent channel of the MSS UTs.  The baseline SEAMCAT assumptions are based on ACLR values of 11 dB and 5 dB for the high gain and low gain narrowband terminals respectively.  It should be noted that the ACLR of the narrowband UTs is not specified in the applicable ETSI standard, and only the unwanted emission levels are specified in terms of maximum e.i.r.p. of the unwanted emissions.  As a consequence, MSS UTs are predicted to have high OOB emission levels, irrespective of the e.i.r.p.  of the UT.  This situation overestimates the actual OOB emission levels of some MSS UTs.

In order to assess the sensitivity of the results to the ACLR value, additional simulations have been run, taking as reference the scenario for which the results of the statistical analysis are the worst (Scenario A.2.1.b, in this case). The results quantify the amount of additional ACLR (on top of 11 dB, in this case) in the first adjacent channel that would be needed to have a capacity loss of the whole system lower than 5%.

Figure 6 shows the results.

Figure 6: Additional 1st ch ACLR required vs system capacity loss

In this example, additional ACLR of 15 dB is required to reduce the average system capacity loss to 5%, bringing the total required 1st channel ACLR to 26 dB.  Taking into account the 300 kHz guardband which exists between the ECN frequency assignments and the MSS band at 1980 MHz, the ACLR for the low gain narrowband MSS UTs is at least 24.4 dB.  Although this is short of the target by 1.6 dB, the assumption that MSS UT may perform better than the conditions specified in the ETSI standards would allow the results for this case to be considered acceptable.

In the case of the high gain narrowband UT, the required additional ACLR would be less than the 15 dB shown above, since the results for the high gain case are better than those for the low gain case.  However, assuming that an ACLR of 26 dB is required, Table 9 shows that an ACLR exceeding this value is obtained with a guardband of 300 kHz.

Hence provided a guardband of 300 kHz on the 1980 MHz band edge is maintained with respect to the ECN networks, the interference may be considered acceptable.

FIGURES RELATED TO THE DETERMINISTIC ANALYSIS

Figure 7: Excess interference generated by the MSS UT into the ECN system

| Scenario | Graph of the excess interference generated by the MSS UT into the ECN system |
| --- | --- |
| - Scenario A.1.1.a | Offset = 90 deg Offset =0deg |
| - Scenario A.1.1.c |  |
| - Scenario A.1.2.a | Offset =0deg Offset = 90 deg |
| - Scenario A.1.2.b |  |
| - Scenario A.1.3.a | Offset = 90 deg Offset =0deg |
| - Scenario A.1.3.b |  |
| - Scenario A.1.4.a | Offset = 90 deg Offset =0deg |
| - Scenario A.1.4.b |  |
| - Scenario A.2.1.a (no guard-band) | Offset = 90 deg Offset =0deg |
| - Scenario A.2.1.a (300 kHz guard-band) | Offset =0deg Offset = 90 deg |
| - Scenario A.2.1.b (300 kHz guard-band) |  |
| - Scenario A.2.2.a (300 kHz guard-band) | Offset = 90 deg Offset =0deg |
| - Scenario A.2.2.b (300 kHz guard-band) |  |
| - Scenario A.2.3.a (300 kHz guard-band) | Offset = 90 deg Offset =0deg |
| - Scenario A.2.3.b (300 kHz guard-band) |  |
| - Scenario A.2.4.a (300 kHz guard-band) | Offset = 90 deg Offset =0deg |
| - Scenario A.2.4.b (300 kHz guard-band) |  |
| - Scenarios B.1.1.a and B.1.2.a | Offset = 90 deg Offset =0deg |
| - Scenario B.1.1.c and B.1.2.b |  |
| - Scenario B.1.2.a and B.2.2.a (500 kHz guard-band) | Offset = 90 deg Offset =0deg |
| - Scenario B.2.1.b and B.2.2.b (500 kHz guard-band) |  |

Calculated ACIR value variation with frequency offset

Figure 9: ACIR variation due to frequency separation (guard band) for Macro BS and wideband/narrowband MES

Figure 10: ACIR variation due to frequency separation (guard band) for Micro BS and wideband/narrowband MES

FIGURES RELATED TO THE STATISTICAL ANALYSIS

The following results are obtained with SEAMCAT v.4.0.1 Beta 11, taking into account the scenarios and assumptions described in Sections 4 and 5.

The negative capacity loss that sometimes is shown when dealing with Ref cells is the consequence of the handover mechanism of the CDMA algorithm implemented in SEAMCAT. Graphs of PDF’s for System, Reference Cell and Worst Cell are presented in Table 26 - Table 41 below:

Table 26: Scenario A.1.1.a

Table 27: Scenario A.1.1.b

Table 28: Scenario A.1.1.c

Table 29: Scenario A.1.1.d

Table 30: Scenario A.1.2.a

Table 31: Scenario A.1.2.b

Table 32:  Scenario A.2.1.a

Table 33: Scenario A.2.1.b

Table 34: Scenario A.2.2.a

Table 35: Scenario A.2.2.b

Table 36: Scenario B.1.2.a

|  | Ref. Cell never Interfered |
| --- | --- |

Table 37: Scenario B.1.2.b

Table 38: Scenario B.2.1.a

|  | No interference affecting Ref. Cell |
| --- | --- |

Table 39: Scenario B.2.1.b

|  | No interference affecting Ref. Cell |
| --- | --- |

Table 40: Scenario B.2.2.a

Table 41: Scenario B.2.2.b

List of reference

- ECC Decision (06)09 on the designation of the bands 1980-2010 MHz and 2170-2200 MHz for use by systems in the Mobile-Satellite Service including those supplemented by a Complementary Ground Component (CGC);

ECC Decision (06)01 on the harmonised utilisation of spectrum for terrestrial IMT-2000/UMTS systems operating within the bands 1900-1980 MHz, 2010-2025 MHz and 2110-2170 MHz;

ERC Report 65 on adjacent band compatibility between UMTS and other services in the 2 GHz band;

ITU-R Report 2039 on characteristics of terrestrial IMT-2000 systems for frequency sharing/interference analyses;

ETSI EN 302 574 for satellite earth stations for MSS operating in the 1980-2010 MHz E/s and 2170-2200 MHz s/E frequency bands;

ETSI EN 301 908-3 for IMT-2000 third-generation cellular networks;

ETSI EN 301 442 for Mobile Earth Stations (MESs), including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) in the 2 GHz bands under the Mobile Satellite Service (MSS);

CEPT Report 19 on least restrictive technical conditions for WAPECS frequency bands;

CEPT Report 39 on developing  least restrictive technical conditions for 2 GHz bands;

Recommendation ITU-R F.1336-2 on reference radiation patterns of omnidirectional, sectoral and other antennas in point-to multipoint systems for use in sharing studies in the frequency range from 1 GHz to about 70 GHz (see recommends 3.1 and 3.2);

ECC Report 131 on derivation of a Block Edge Mask (BEM) for terminal stations in the 2.6 GHz frequency band (2500-2690 MHz)

The value is used in the SEAMCAT simulation for scenario B1 and B2.

Maximum power interfering signal is calculated with a required I/N of -6 dB

If necessary a guard band may be introduced. Any necessary guard band here should be added to guard band requirements in Chapter 5

It should be noted that there is no information available as far as the ACS of ECN UT on the 2nd adjacent channel; therefore, as a conservative approach, the same ACS as the 1st adjacent is here used.

Results are obtained applying a 300 kHz guard band

The Worst cell capacity loss is not applicable when the interfered system is an ECN User Terminal