SE(13)014A3

ECC Report <No>

Compatibility studies in the band 5725 – 5875 MHz between SRD equipment for wireless industrial applications using technologies different from UltraWideBand (UWB) and other systems

Approved DD Month YYYY (Arial 9pt bold)

[last updated: DD Month YYYY) (Arial 9pt) [date of the latest update]]

# Executive summary

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	7

2	Overview of industrial applications using technologies different from Ultra-WideBand (UWB)	8

3	INDUSTRIAL APPLICATIONS USING TECHNOLOGIES DIFFERENT FROM ULTRA-WIDEBAND (UWB) PARAMETERS AND DEPLOYMENT SCENARIOS	13

3.1	Technical Parameters	13

3.1.1	Transmitter Parameters	13

3.1.1.1	Transmitter output power	13

3.1.1.2	Transmitter masks	13

3.1.1.3	Antenna Characteristics	15

3.1.1.4	Bandwidth	16

3.1.2	Receiver parameters	16

3.1.2.1	Receiver sensitivity	16

3.1.2.2	Other receiver parameters	17

3.1.3	Channel access parameters	17

3.1.4	Conclusion of technical parameters	17

3.2	Deployment Scenarios	17

3.2.1	Operation of wireless industrial applications	18

3.2.2	Geographical deployment	19

4	OTHER SERVICES IN THE BAND 5725 - 5875 MHZ	21

4.1	Radiolocation Service	23

4.1.1	Technical characteristics	23

4.1.2	Operational characteristics of Radiolocation systems	24

4.1.3	Protection criteria	24

4.2	Road Transport and Traffic Telematics (RTTT) Systems	26

4.2.1	Parameters	26

4.2.2	Protection Criteria	28

4.3	Fixed Service	28

4.3.1	Point-to-Point Links	29

4.3.2	Broadband Fixed Wireless Access (BFWA) Systems	29

4.3.2.1	Parameters	29

4.3.3	Protection criteria [to be deleted?]	32

4.4	FIXED SATELLITE (E-S) SERVICE (FSS)	32

4.5	AMATEUR AND AMATEUR-SATELLITE (S-E) SERVICES	33

4.5.1	Characteristics for the Amateur Service	34

4.5.2	Characteristics for the Amateur-Satellite Service	35

4.6	INTELLIGENT TRANSPORT SYSTEMS (ITS)	35

4.6.1	Parameters	35

4.6.2	Protection criteria	39

4.7	Short Range Devices (SRD)	39

4.7.1	General (non-specific) SRD	39

4.7.2	Tank Level Probing Radar (TLPR)	40

5	COMPATIBILITY STUDIES	46

5.1	Radiolocation Service	46

5.1.1	Step 1: MCL Calculations	46

5.1.1.1	Methodology for calculating interference from WIA into Radar	46

5.1.1.2	Determination of required separation distance	49

5.1.1.3	Step 2	50

5.1.1.4	DFS consideration	50

5.1.2	The use of mitigation techniques as a method to enable sharing between Radiolocation service and WIA systems in the 5.8 GHz band	54

5.1.3	Conclusion	54

5.2	Road Transport and Traffic Telematics (RTTT) Systems	54

5.2.1	Compatibility between interferer WIA (indoor) and victim RTTT	55

5.2.2	COMPATIBILITY BETWEEN interferer RTTT and victim WIA (indoor)	57

5.2.3	COMPATIBILITY BETWEEN interferer WIA (outdoor) and victim RTTT	60

5.2.4	Compatibility between interferer RTTT and victim WIA (outdoor)	61

5.2.5	Conclusions	63

5.2.5.1	Conclusion for interferer WIA and victim RTTT	63

5.2.5.2	Conclusion for interferer RTTT (RSU) and victim WIA	64

5.3	BFWA	65

5.3.1	MCL calculations	65

5.3.2	Sensing procedure	66

5.3.3	SEAMCAT simulations	69

5.3.3.1	WIA 20 MHz	72

5.4	Fixed Satellite (E-s) Service (FSS)	75

5.5	Amateur Service and Amateur-satellite (s-E) Service	75

5.5.1	Compatibility between WIA and the Amateur Service	75

5.5.1.1	Step 1: MCL calculations	75

5.5.1.2	Step 2: SEAMCAT simulations	78

5.5.1.3	Conclusions	78

5.5.2	Compatibility between WIA and the Amateur-Satellite Service	78

5.5.2.1	Step 1: MCL calculations	78

5.5.2.2	Step 2: SEAMCAT simulations	80

5.5.2.3	Conclusions	81

5.6	Compatibility between WIA and General (non-specific) short range devices	81

5.6.1	Assumptions	81

5.6.2	Impact of WIA devices on SRD	81

5.6.2.1	Step 1: MCL calculation	81

5.6.2.2	Conclusion on WIA sharing with SRDs	84

5.7	INTELLIGENT TRANSPORT SYSTEMS (ITS)	84

5.7.1	Compatibility between interferer WIA (indoor) and victim ITS	84

5.7.2	COMPATIBILITY BETWEEN interferer ITS and victim WIA (indoor)	86

5.7.3	COMPATIBILITY BETWEEN interferer WIA (outdoor) and victim ITS	88

5.7.4	Compatibility between interferer ITS and victim WIA (outdoor)	89

5.7.5	Conclusion	91

5.7.5.1	Conclusion for interferer WIA and victim ITS	91

5.7.5.2	Conclusion for interferer ITS and victim WIA	92

6	Initial consideration on the license regime	93

6.1	Coexistence management for license exempt usage	93

6.2	Light licensing procedures	93

ANNEX 1: TYPICAL WIA ANTENNA EXAMPLES	94

ANNEX 2: List of referenceS	97

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| AFA | Adaptive Frequency Agility |
| ACK | Acknowledgement |
| APC AS | Adaptive Power Control Amateur Service |
| BC CD | Best case scenario Collision Detection |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| CS | Carrier Sensing |
| CSMA | Carrier Sensing Multiple Access |
| CW | Continuous Wave |
| CR | Cognitive Radio |
| DAA | Detect and Avoid |
| DC | Duty Cycle |
| DFS | Dynamic Frequency Selection |
| DP | Combination of Data volume and Power consumption |
| dRSS | desired Received Signal Strength |
| DS | Combination of Data volume and size issues |
| DSI | Digital Spectrum Investigation |
| DSSS | Direct Sequence Spread Spectrum |
| EC | European Commission |
| ECC | Electronic Communications Committee |
| e.i.r.p. | Equivalent Isotropic Radiated Power |
| ERP | Effective Radiated Power |
| FHSS | Frequency Hopping Spread Spectrum |
| FWA | Fixed Wireless System |
| RFID | Radio Frequency Identification |
| GSE | Group spectrum efficiency |
| ITS | Intelligent Transport System |
| ITU-R | International Telecommunication Union-Recommendation |
| LBT | Listen Before Talk |
| LC | Combination of Latency and Cost issues |
| LDC | Low Duty Cycle |
| LHCP | left-hand circular polarization |
| LO | Local Oscillator |
| LP | Level Probing |
| LS | Combination of Latency and Size issues |
| M2M | Machine to Machine |
| MCL | Minimum Coupling Loss |
| OSI | Open Systems Interconnection |
| RAKE | Radio Activated Key Entry |
| RC | Combination of Reliability and cost issues |
| RHCP | right-hand circular polarization |
| RKE | Remote Keyless Entry |
| RP | Combination of Reliability and Power consumption |
| RSPG | Radio Spectrum Policy Group |
| RS | Combination of Reliability and Size issues |
| SAE | Single system Absolute Efficiency |
| SAW | Surface Acoustic Wave |
| SGRE | Single system in a Group Relative Efficiency |
| SNR | Signal to Noise Ration |
| SRD | Short Range Device |
| SRE | Single system Relative Efficiency |
| SUE | Spectrum usage Efficiency |
| TDMA | Time Duplex Modulation Access |
| TPMS | Tyre Pressure Monitoring System |
| TRP | Total radiated Power |
| TCXOs | Temperature Compensated Oscillators |
| TLPR | Tank Level Probing Radar |
| ULP | Ultra Low Power |
| UWB | Ultra Wide Band |
| VLDC | Very Low Duty Cycle |
| WC WGFM | Worst case scenario Working Group Frequency Management |
| WIA | Wireless Industrial Applications |
| WT | Wanted Transmitter |
| [Editor’s note: check the lit] |  |

# Introduction

Industrial automation requires "robust" wireless technologies to be used for their critical wireless links in industrial applications. Wireless applications allow savings of often complex and expensive cables, cable protection and plugs, the increased mobility and flexibility as well as the wear and tear free transmission medium. These advantages are particularly high in the area of:

- monitoring and mobile worker communication;

- wireless sensors and actuators at moving parts;

- setups that require flexibility in terms of tool or machine reconfigurations.

Different functions can be mastered substantially more efficient by a wireless network of data acquisition terminals, robotic type equipment or automated guided vehicles. Those wireless applications are intended to be operated in the frequency range 5725 to 5875 MHz and are further described in an SRdoc [1].The current regulations allows for the deployment of SRDs in the frequency range from 5725 to 5875 MHz but limited to 25 mW.

Table 1: Regulatory parameters related to Annex 1 to ERC Rec. 70-03 [2]

| Frequency Band | Power / Magnetic Field | Spectrum access and mitigation requirement | Channel spacing | ECC/ERC Decision | Notes |
| --- | --- | --- | --- | --- | --- |
| i: 5725 - 5875 MHz | 25 mW (14 dBm) e.i.r.p | No requirements | No spacing | - | - |

This report investigates the possible deployment of Wireless Industrial Application (WIA) using technologies different from Ultra-Wide Band in the frequency range 5725 to 5875 MHz and their possible impact on the others systems / services allocated/identified in this frequency range.

[note: consider updating the introduction to reflect the case of manufacturing cells which can be deployed in the existing regulations.]

# Overview of industrial applications using technologies different from Ultra-WideBand (UWB)

In a larger industrial plant, if a chemical or oil-/and gas industry process plant ("process automation") or e.g. an automotive discrete manufacturing plant (discrete or "factory automation"), there are and will be always many different wireless systems and technologies for different purposes in parallel to each other (partly or completely overlapping).

The subdivision of such systems into three main classes can be typically done according to Table 2 into:

cell or sub-unit automation;

factory hall or plant sub-unit automation; and

plant level automation.

The classes are described below in more detail.

Cell (or subunit) automation:

Lowest control system level, can be a part of a line in an automotive plant or a normal discrete manufacturing cell or a subunit in process automation (e.g. a reactor with a local control to which sensors and actuators are connected). Typically lower range (e.g. 10 m to 30 m range) but most demanding for latency and robustness, are capable to live with fast movements, integrated antennas and many obstacles (nearly complete shielding).

One such cell unit has one wireless system with in average 30 devices.

Up to 10 such units/manufacturing cells can be in close proximity, so that their interference area overlaps.

The area related local device density at 10 m range therefore is typically 10x30 devices per 10x10 m² or 0,33 to 3 devices per m² (at 30 m to 10 m range respectively).

The cell automation data packets as such are typically quite small and have 16 octets on air (e.g. 4 octets of user data, 12 octets for addressing, control and error protection) and have to be sent every 50 ms in each direction.

Figure 1: Example of a 1 plant with 10 production halls and 50 manufacturing cells

Factory hall (or plant subunit) automation:

Medium Control System level, where e.g.:

- whole production lines or moving applications (e.g. moving through a factory hall in discrete manufacturing e.g. automated guided vehicles, rail hanging power screwdrivers), or

- whole production units in process automation:

    - Cover a larger area (e.g. 100 m x 100 m). This is solved by an industrial WLAN or a mesh type technology (TDMA schemes used) to safely cover a larger area with.

    - In average 100 devices and still low latencies.

Also here the master is in need of a higher duty cycles and high power to cover the range without line of sight:

Up to 5 such independent systems can be within range of each other (are within "interference" range).

The area related local device density at 100 m range therefore is approx. 5 x 100 devices per 100 x 100 m² or 0,022 per m² at 100 m range.

The hall/subunit automation data packets as such are typically medium size with 200 octets on air (e.g. 140 octets of user data, 60 octets for addressing, networking, control and error protection) and have to be sent every 200 ms in each direction.

Figure 2: Example of hall wide networks, up to locally parallel

Plant level wide applications:

Control system level covering up to the whole plant, typically an industrial mesh technology:

Able to cover e.g. 1 km x 1 km but typically with mesh technology to increase robustness against typical industrial influences (moving obstacles, interference/coexistence).

One such mesh system can have up to 1000 connected devices, but each device only having to cover a smaller range (100 m) and the mesh covers the larger distances needed, without excessive power needs.

There may be up to 3 independent such mesh networks operating in parallel in the whole plant.

Up to max. 50 devices of the 3 clusters can be within range of each other (are within "interference" range).

The area related local device density at 100 m range therefore is approx. 50 devices per 100x100 m² or

0,025 per m² at 100 m range.

- The plant level automation data packets are typically medium size with 105 octets on air (e.g. 50 octets of user data, 55 octets for addressing, networking, control and error protection) and have to be sent every 500 ms in each direction.

Figure 3: Example of plant wide networks, up to 3 parallel

All of these 3 levels are operated in parallel (partially or completely overlapping interference area), and often by different operators and connected to different Control Systems. Each of the many wireless systems has to be allowed to switch on and off and vary the number of connected active devices and data amount transferred, depending of the needs of the many different production cells/subunits/ lines in order to maximise individually production, quality, safety and do service, troubleshooting and installation work on the productions units.

Parallel means that at most parts of the plant, the three "wireless" classes operate overlapping, preferably in the same frequency band for maximal flexibility of coexistence management reasons, to increase spectrum efficiency and to have the same needed spectrum properties like industrial-interference-free, power efficiency (range) and bending/damping by obstacles.

Normally, in total more than 70 MHz of spectrum would be required to support applications from the 3 classes to operate seamless within a given environment without any restriction. This would still require a clever site planning which is not always possible as the different wireless solutions are from different vendors. However, recent and future developments of more intelligent sharing mechanisms will facilitate automatic seamless operation of these 3 classes even in less spectrum is made available.

Figure 4: Example of a combination of 3 classes

The total estimation of needed spectrum to share can be estimated from Table 2 by adding up the spectrum estimated for the three classes to be in the order of 76 MHz of continuous spectrum.

Table 2 shows the extreme low latency and application availability requirements of many industrial wireless applications.

Table 2: Industrial application

| Characteristic | Manufacturing cell | Factory hall | Plant level |
| --- | --- | --- | --- |
| Indoor/outdoor application | Indoor | Mostly indoor | Mostly outdoor |
| Spatial dimension LxWxH [m³] | 10 x 10 x 3 | 100x100x10 | 1000 x 1000 x 50 |
| Number of devices (typically) | 30 | 100 | 1000 |
| Number of parallel networks (= clusters) | 10 | 5 | 5 |
| Number of such clusters per plant | 50 | 10 | 1 |
| Min. Number of locally parallel devices | 300 | 500 | 250 |
| Network type | Star | Star/Mesh | Mesh |
| S, Typ. packet size [on air, byte] | 16 | 200 | 105 |
| Maximum allowable latency (end-to-end) incl. jitter/retransmits [ms] | 5 ± 10 % | 20 ± 10 % | 20 ± 10 % |
| DC, Max. on-air duty cycle related to media utilisation (see clause 5) / DC?? | 20 % | 20 % | 20 % |
| T, Update Time [ms] | 50 ± 10 % | 200 ± 10 % | 500 ± 10 % |
| Packet Loss Rate (involved outside latency) | 10-9 | 10-9 | 10-9 |
| E, Spectral efficiency (typically) [Bit/s/Hz] | 1 | 1,18 | 0,13 |
| B, Resulting min. Bandwidth requirement [MHz] | 8 | 34 | 34 |
| Application availability | Exceeds 99,999 % (less than 300 s accumulated outage per annum). |  |  |
| Typical operational distance | Depends on individual use case and frequency of operation. |  |  |
| NOTE: To account for up and downlink information, 2 telegrams have been assumed. |  |  |  |

Additional technical details are provided in section 3 (see also the SRdoc [1]).

[Editor’s note: the concept of DC / DC, Max. on-air duty cycle related to media utilisation

(see clause 5) may need to take into account the current discussion within ETSI – check the reference document. Discussions are ongoing on this subject.]

# INDUSTRIAL APPLICATIONS USING TECHNOLOGIES DIFFERENT FROM ULTRA-WIDEBAND (UWB) PARAMETERS AND DEPLOYMENT SCENARIOS

## Technical Parameters

### Transmitter Parameters

#### Transmitter output power

E.i.r.p. up to 400 mW (26 dBm) is required.

#### Transmitter masks

The following figures provide the transmitter spectrum masks for 1 MHz, 3 MHz and 20 MHz systems respectively.

Figure 5: Transmitter spectral mask for 1 MHz bandwidth

- Figure 6: Transmitter spectral mask for 3 MHz bandwidth

Figure 7: Transmitter Spectral Mask for 20 MHz bandwidth

#### Antenna Characteristics

No restrictions on antenna characteristics, according to the description in section 6.3.2 of ETSI TR 102 889-2 V1.1.1 (2011-08) [1].

The default antenna type is omnidirectional (collinear dipole antenna). Therefore, Recommendation ITU-R F.1336-3 [3] should be used. Following section (1.a) in section formula (2.1) Rec. ITU-R F.1336-3, the expression of antenna gain in dBi at elevation angle Θ in degrees is given by:

where:

Θ: 	absolute value of the elevation angle relative to the angle of maximum gain (degrees)

Θ3: 	the 3 dB beamwidth in the vertical plane (degrees)

k:	sidelobe factor  for all antennas operating in the 3-70 GHz range, the parameter k should be 0.

G(Θ) :	gain relative to an isotropic antenna (dBi)

G0 :	the maximum gain in the azimuth plane (dBi)

The WIA antenna is shown in ANNEX 1:, Figure 31. The hinge mounting allows various orientations. Therefore, in Seamcat simulations, a random antenna tilt between 0º and 90º (for azimuths between 0º and 360º) is taken into account.

Figure 8: Antenna pattern according ITU-R F.1336

Additional information on the antenna pattern is given in ANNEX 1:.

[Ben: ITU-R Rec. F.1336-3 has several formulas for antenna diagram and side-lobe levels. A reference to the right formula number (2.1) in ITU-R F.1336-3 would be appropriate, as well as a motivation of that choice. The motivation of the choice is not 100% clear to me, so I have no suggestion for an appropriate text.]

#### Bandwidth

As different technologies are used, the typical occupied bandwidth for a single device varies between 1 MHz and 20 MHz. Frequency Hopping as well as non-frequency hopping technologies is used. The parallel use of all tree classes requires a continuous spectrum of 76 MHz.

### Receiver parameters

#### Receiver sensitivity

Most of the technologies used in the industrial applications are based on specifications such as IEEE 802.11 [4], IEEE 802.15.1 [5] and IEEE 802.15.4 [6]. Therefore, the relevant receiver parameters including the receiver sensitivity can be found in those standards.

#### Other receiver parameters

The performance of certain sharing mechanisms may require certain specific receiver requirements. The sharing study will show if additional receiver parameters are required.

### Channel pattern

The establishment of coexistence is a combination of technical and/or organizational measures to ensure the interference-free operation of wireless industrial applications in their environments. Coexistence between wireless industrial automation communication systems can be achieved by uncoupling the dimensions frequency; time and space. Consequential the WIA systems with 1 MHz, 3 MHz and 20 MHz use separate frequency bands.

Figure 9:	 Possible channelization

The use of WIA systems with 1 MHz, 3 MHz and 20 MHz bandwith depends on the requirement of industrial application. Figure 9 depict three different examples of WIA channelization.

It is possible to use the following channel by WIA systems with 1 MHz bandwidth:

Channel centre frequency = Channel starting frequency + 1 × n (MHz)			Eq. 1

n = 0,…, 140

Channel starting frequency is defined as 5.730 GHz.

The possible channel centre frequency for the WIA systems with 3 MHz and 20 MHz bandwidth can calculate with equation 2:

Channel centre frequency = Channel starting frequency + 5 × nch (MHz)			Eq. 2

nch = 147,…, 173

Channel starting frequency is defined as 5 GHz.

For WIA systems with 3 MHz bandwidth is it possible to use the channels 146 to 151, 153 to 167 and 169 to 174.

For WIA systems with 20 MHz bandwidth is it possible to use the non-overlapping channels 148, 155, 159, 163 and 172 or 173.

The first 2.5 MHz at the edges of the band are not planned to be used.

Table 3: Possible channels

| Channel No. | Channel centre frequency [MHz] | Channel No. | Channel centre frequency [MHz] | Channel No. | Channel centre frequency [MHz] |
| --- | --- | --- | --- | --- | --- |
| 145 | 5725 | 156 | 5780 | 167 | 5835 |
| 146 | 5730 | 157 | 5785 | 168 | 5840 |
| 147 | 5735 | 158 | 5790 | 169 | 5845 |
| 148 | 5740 | 159 | 5795 | 170 | 5850 |
| 149 | 5745 | 160 | 5800 | 171 | 5855 |
| 150 | 5750 | 161 | 5805 | 172 | 5860 |
| 151 | 5755 | 162 | 5810 | 173 | 5865 |
| 152 | 5760 | 163 | 5815 | 174 | 5870 |
| 153 | 5765 | 164 | 5820 | 175 | 5875 |
| 154 | 5770 | 165 | 5825 |  |  |
| 155 | 5775 | 166 | 5830 |  |  |

### Channel access parameters

For maximised spectrum efficiency, including sharing among all wireless industrial applications present, a spectrum sharing mechanism appropriate for industrial applications is required.

An example of that is Frequency Agility. Frequency Agility is the ability of a system to operate according to frequency or channel assignments of a centralized or distributed control mechanism, which will define the configuration of all devices within an industrial site or subarea thereof. Configurations may change over time depending on the application requirements. If non-contiguous spectrum is assigned, then the Frequency Agility feature is supposed to operate across all assigned sub-bands.

### Conclusion of technical parameters

The considered WIA systems may typically use 1 MHz, 5 MHz, 10 MHz or 20 MHz channelization, which is necessary to obtain sufficiently high data rates.

Table 4: Technical parameters of WIA system

| Parameter | System for Manufacturing Cell | System for Production Hall | System for Plant |
| --- | --- | --- | --- |
| max e.i.r.p. | 25 mW (14 dBm) | 400 mW (26 dBm) | 400 mW (26 dBm) |
| Receiver sensitivity | -70 dBm in 1 MHz | -70 dBm in 1 MHz - 90 dBm in 3 MHz -88 dBm @ 6 MBit/s in 20 MHz | - 90 dBm in 3 MHz -88 dBm in 20 MHz |
| Channel bandwidth (MHz) | 1 MHz | 1 MHz, 3 MHz and 20 MHz | 3 MHz and 20 MHz |
| Channel spacing (MHz) | 1 MHz | 1 MHz, 5 MHz and 20 MHz | 5 MHz and 20 MHz |
| Min. Number of hopping channels | no requirements | - | no requirements |
| Max. Number of hopping channels | BWTotal/1 MHz | - | BWTotal/5 MHz |
| Topology33 | Star | Star/Mesh | Mesh |
| Duty Cycle | 20 % | 20 % | 20 % |
| Antenna type | N/A | N/A | N/A |
| Antenna gain | N/A | N/A | N/A |
| Antenna pattern | N/A | N/A | N/A |
| Polarisation | N/A | N/A | N/A |
| Antenna height (m) | 0,5 m to 3 m | 0,5 m to 10 m | 0,5 m to 50 m |

The manufacturing cells can be deployed under the existing regulatory framework of Rec. 70-03 Annex 1.

## Deployment Scenarios

All of these three wireless classes (cell or sub-unit automation; factory hall or plant sub-unit automation and plant level automation) are operated in parallel. Parallel means that at most parts of the plant, the three "wireless" classes operate overlapping, preferably in the same frequency band for maximal flexibility of coexistence management reasons, to increase spectrum efficiency and to have the same needed spectrum properties like industrial-interference-free, power efficiency (range) and bending/damping by obstacles.

That means the available spectrum is occupied by the three wireless classes of industrial applications. The medium utilisation is 100 %. The instance of coexistence manager guarantees the parallel work of different wireless devices and systems, therefore only one device use a certain piece of available spectrum (bandwidth) at a certain place and at a certain time.  The e.i.r.p. is equal over the spectrum and time and the aggregate interference effect will not increase.

Table 4: Parameters for deployment scenario

| Frequency Band | Maximum mean power | Duty Cycle | Channel spacing | Remark |
| --- | --- | --- | --- | --- |
| 5725 to 5850 MHz | 26 dBm | 100 % | No requirements | - |
| 5850 to 5875 MHz | 26 dBm | 100 % | No requirements | - |

[Note: there are still some ongoing discussion relating to the DC subject.]

### Geographical deployment of WIA

The following table provides an estimation of the number of wireless industrial automation devices other than UWB (WIA) over Germany. The information relating to Germany is then extended to the whole Europe.

Table 6: Deployment of WIA in Europe

| Parameters | Values |
| --- | --- |
| Population / Germany | 80 000 000 |
| Population / Frankfurt am Main | 690 000 |
| Plants / Frankfurt am Main | 4 |
| Plants / Germany | 533 |
| Wireless Devices / Germany | 12 266 667 |
| Area Germany km² | 357 000 |
| Wireless devices Germany/km² | 34.4 |
| Automation ratio EU/Germany | 3 |
| Wireless Devices EU | 36 800 000 |
| area EU km² | 10 180 000 |
| device density EU/km² | 3.6 |

# OTHER SERVICES IN THE BAND 5725 - 5875 MHZ

Table 7 provides the extract from the ITU Radio Regulations [7] for the bands used through this report.

Table 7: Extract of Article 5 of the ITU Radio Regulations

| Region 1 | Region 2 | Region 3 |
| --- | --- | --- |
| 5 725-5 830 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur 5.150 5.451 5.453 5.455 5.456 | 5 725-5 830 RADIOLOCATION Amateur 5.150 5.453 5.455 |  |
| 5 830-5 850 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) 5.150 5.451 5.453 5.455 5.456 | 5 830-5 850 RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) 5.150 5.453 5.455 |  |
| 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE 5.150 | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Amateur Radiolocation 5.150 | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Radiolocation 5.150 |

Footnotes of RR Art. 5 relevant for CEPT countries:

5.150	The following bands: ... 5 725-5 875 MHz (centre frequency 5 800 MHz), and ... are also designated for industrial, scientific and medical (ISM) applications. Radio communication services operating within these bands must accept harmful interference which may be caused by these applications. ISM equipment operating in these bands is subject to the provisions of No. 15.13.

5.451	Additional allocation:  in the United Kingdom, the band 5 470-5 850 MHz is also allocated to the land mobile service on a secondary basis. The power limits specified in Nos. 21.2, 21.3, 21.4 and 21.5 shall apply in the band 5 725-5 850 MHz.

5.455	Additional allocation:  in Armenia, Azerbaijan, Belarus, Cuba, the Russian Federation, Georgia, Hungary, Kazakhstan, Latvia, Moldova, Mongolia, Uzbekistan, Kyrgyzstan, Tajikistan, Turkmenistan and Ukraine, the band 5 670-5 850 MHz is also allocated to the fixed service on a primary basis.     (WRC-03)

The following table provides an overview ERC Report 25 [8] for the bands 5725 – 5875 MHz.

Table 8: Allocation / Identification of spectrum according to ERC Report 25 [8]

| Frequency range | European Common Allocation | ECC/ERC harmonisation measures | Application | European footnotes | Standard | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 5725 - 5830 MHz | FIXED-SATELLITE (E/S) RADIOLOCATION Amateur Mobile 5.150 EU2 EU22 |  | Amateur |  | EN 301 783 |  |
|  |  | ECC/REC/(06)04 | BFWA |  | EN 302 592 | Within the band 5725-5875 MHz |
|  |  |  | Defence systems |  |  | Tactical and weapon system radars |
|  |  |  | ISM |  | EN 300 440 | Within the band 5725-5875 MHz |
|  |  | ERC/REC 70-03 | Non-Specific SRD |  | EN 302 372 | Within the band 5725-5875 MHz |
|  |  | ERC/REC 70-03 | Radiodetermination applications |  |  | Within the band 4500-7000 MHz for TLPR application |
|  |  | ECC/DEC/(02)01 ERC/REC 70-03 | RTTT |  |  | Within the band 5795-5805 MHz. RTTT in the band 5805-5815 MHz on a national basis |
|  |  |  | Weather Radars |  |  | Ground based and airborne |
| 5830 - 5850 MHz | FIXED-SATELLITE (E/S) RADIOLOCATION Amateur Amateur Satellite (S/E) Mobile 5.150 EU2 EU22 |  | Amateur Satellite (S/E) | EU23 |  | Within the band 5830-5850 MHz |
|  |  | ECC/REC/(06)04 | BFWA |  | EN 302 502 | Within the band 5725-5875 MHz |
|  |  |  | Defence systems |  |  | Tactical and weapon system radars |
|  |  |  | ISM |  |  | Within the band 5725-5875 MHz |
|  |  | ERC/REC 70-03 | Non-Specifics SRDs |  | EN 300 440 | Within the band 5725-5875 MHz |
|  |  | ERC/REC 70-03 | Radiodetermination applications |  | EN 302 372 | Within the band 4500-7000 MHz for TLPR application |
|  |  |  | Weather radars |  |  | Ground based and airborne |
| 5850 - 5925 MHz | FIXED FIXED-SATELLITE (E/S) MOBILE 5.250 | ECC/REC/(06)04 | BFWA | EN 302 502 |  | Within the band 5725-5875 MHz |
|  |  |  | FSS |  | EN 301 443 | Priority for civil networks |
|  |  |  | ISM |  |  | Within the band 5725-5875 MHz |
|  |  | ECC/DEC/(08)01 ECC/REC/(08)01 | ITS |  | EN 302 571 | Within the band 5875-5925 MHz. Within the band 5855-5875 MHz |
|  |  | ERC/REC 70-03 | Non-Specific SRDs |  | EN 300 440 | Within the band 5725-5875 MHz |
|  |  | ERC/REC 70-03 | Radiodetermination applications |  | EN 302 372 | Within the band 4500-7000 MHz for TLPR application |

EU2: Civil-military sharing.

EU22: The band 5250-5850 MHz is utilised for a variety of radiodetermination applications falling within the radionavigation and radiolocation services. This band will be subject to further detailed consideration.

EU23 In the sub-bands 5660-5670 MHz (earth to space), 5830-5850 MHz (space to earth) and 10.45-10.50 GHz the amateur-satellite additionally operates on a secondary and non interference basis to other services. In making assignments to other services, CEPT administrations are requested wherever possible to maintain these allocations in such a way as to facilitate the reception of amateur emissions with minimal power flux densities.

The following services and systems are covered within this study following the approach considered in previous ECC Reports ([ref to ECC Report 101]):

4.1Radiolocation Service

4.2 Road Transport and Traffic Telematic (RTTT) Systems

4.3 Fixed Service (Point-to-Point Links) and Broadband Fixed Wireless Access (BFWA)

4.4 Fixed-Satellite (E-s) Service (FSS)

4.5 Amateur Service, Amateur-satellite (s-E) Service

4.7 Intelligent Transport Systems (ITS)

4.8	Short Range Devices including:

  - Non-specific Short Range Devices and

  - Tank Level Probing Radar (TLPR) applications

4.9 Direct Air To Ground Communications [Editor’s note: for these systems we may need to wait for the output of the studied undertaken within CEPT.]

## Radiolocation Service

The bands between 5725 MHz and 5850 MHz are allocated to the Radiolocation service on a primary basis.

### Technical characteristics

Recommendation ITU-R M.1638 [9] provides characteristics of radars operating under the Radiolocation services in the frequency range 5250-5850 MHz. Within this range, the band between 5725 MHz and 5850 MHz is used by many different types of radars on fixed land-based, ship borne and transportable platforms. It should be noted that most of these radars are designed to operate not only in the 5725-5850 MHz band but in a larger portion of the band 5250 – 5850 MHz.

Table 9 contains technical characteristics of representative systems deployed in this band. This includes a subset of the radars contained in Recommendation ITU-R M.1638 [9], which are relevant for the frequency band 5725 – 5850 MHz (radars L, M, N, O and Q) and three additional radars operated by administrations within CEPT (X, Y and Z). This information is generally sufficient for calculation to assess the compatibility between these radars and other systems.

Frequency hopping is one of the most common Electronic-Counter-Counter-Measures (ECCM). Radar systems that are designed to operate in hostile electronic attack environments use frequency hopping as one of its ECCM techniques. This type of radar typically divides its allocated frequency band into channels. The radar then randomly selects a channel from all available channels for transmission. This random occupation of a channel can occur on a per beam position basis where many pulses on the same channel are transmitted or on a per pulse basis. This important aspect of radar systems should be considered and the potential impact of frequency hopping radar should be taken into account in sharing studies.

### Operational characteristics of Radiolocation systems

There are numerous radar types, accomplishing various missions, operating within the Radiolocation service throughout the whole range 5250 – 5850 MHz, and specifically within the 5725 – 5850 MHz band. Test range instrumentation radars are used to provide highly accurate position data on space launch vehicles and aeronautical vehicles undergoing developmental and operational testing. These radars are typified by high transmitter powers and large aperture parabolic reflector antennas with very narrow pencil beams. The radars have auto-tracking antennas which either skin-track or beacon-track the object of interest. Periods of operation can last from minutes up to 4-5 hours, depending upon the test program. Operations are conducted at scheduled times 24 hours/day, 7 days/week.

Shipboard sea and air surveillance radars are used for ship protection and operate continuously while the ship is underway as well as entering and leaving port areas. These surveillance radars usually employ moderately high transmitter powers and antennas which scan electronically in elevation and mechanically a full 360 degrees in azimuth. Operations can be such that multiple ships are operating these radars simultaneously in a given geographical area. Other special-purpose radars are also operated in the band 5250 – 5850 MHz.

### Protection criteria

The de-sensitising effect on radars operated in this band from other services of a CW or noise-like type modulation is predictably related to its intensity. In any azimuth sectors in which such interference arrives, its power spectral density can simply be added to the power spectral density of the radar receiver thermal noise, to within a reasonable approximation. If power spectral density of radar-receiver noise in the absence of interference is denoted by N0 and that of noise-like interference by I0, the resultant effective noise power spectral density becomes simply I0+N0. An increase of about 1 dB for the Radiolocation radar would constitute significant degradation. Such an increase corresponds to an (I+N)/N ratio of 1.26, or an I/N ratio of about –6 dB. These protection criteria represent the aggregate effects of multiple interferers, when present. The tolerable I/N ratio for an individual interferer depends on the number of interferers and their geometry, and needs to be assessed in the course of analysis of a given scenario. The aggregation factor can be very substantial in the case of certain communication systems, in which a great number of stations can be deployed.

Table 9: Characteristics of Radiolocation systems

| Characteristics | Radar l | Radar M | Radar N | Radar O | Radar Q | Radar X (Note 1) | Radar Y (Note 1) | Radar Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Function | Instrumentation |  |  | Surface and air search |  |  |  | Search |
| Platform type | Ground |  |  |  | Ship | Ground /Vehicle |  |  |
| Tuning range (MHz) | 5350-5850 |  | 5400-5850 |  | 5450-5825 | 5400 – 5850 |  | 5250-5850 |
| Modulation | None |  | Pulse / chirp pulse | Chirp pulse | None |  |  | Non-Linear FM |
| Tx power into antenna (kW) | 2800 | 1200 | 1000 | 165 | 285 | 12 | 12 | 70 |
| Pulse width (µs) | 0.25, 1.0, 5.0 | 0.25, 0.5, 1.0 | 0.25-1 (plain) 3.1-50 (chirp) | 100 | 0.1, 0.25, 1.0 | 4-20 | 4-20 | 3.5, 6, 10 |
| Pulse rise/fall time (µs) | 0.02-0.5 | 0.02-0.05 | 0.02-0.1 | 0.5 | 0.03/0.05/ 0.1 | No detail | No detail | N/A |
| Pulse repetition rate (pps) | 160, 640 | 160, 640 | 20-1 280 | 320 | 2 400/1 200/ 750 | 1000-7800 | 1000-7800 | 2500/3750 |
| Chirp bandwidth (MHz) | N/A | N/A | 4.0 | 8.33 | N/A | No detail | No detail |  |
| RF emission bandwidth –3 dB –20 dB (MHz) | 0.5-5 | 0.9-3.6 6.4-18 | 0.9-3.6 6.4-18 | 8.33 9.9 | 5.0/4.0/1.2 16.5/12.5/7.0 | 5 | 5 |  |
| Antenna pattern type | Pencil |  |  |  | Fan | N/A |  |  |
| Antenna type | Parabolic |  | Phased Array |  | Travelling wave feed horn array | N/A |  | Phased Array |
| Antenna polarization | Vertical/Left-hand circular |  |  |  | Horizontal | Vertical |  | Horizontal |
| Antenna main beam gain (dBi) | 54 | 47 | 45.9 | 42 | 30.0 | 35 | 35 | 31.5 |
| Antenna elevation beam width (degrees) | 0.4 | 0.8 | 1.0 | 1.0 | 28.0 | N/A | N/A | 43.8 |
| Antenna azimuthal beam width (degrees) | 0.4 | 0.8 | 1.0 | 1.0 | 1.6 | N/A | N/A | 1.75 |
| Antenna horizontal scan rate (degrees/s) | N/A (Tracking) |  |  |  | 90 | N/A (tracking) | 180/360 | 120/180 |
| Antenna horizontal scan type | N/A (Tracking) |  |  |  | 30-270 ° Sector | N/A |  |  |
| Antenna vertical scan rate (degrees/s) | N/A (Tracking) |  |  |  | N/A |  |  |  |
| Antenna vertical scan type (degrees) | N/A (Tracking) |  |  |  | Fixed | N/A |  |  |
| Antenna sidelobe (SL) levels (dB) | -20 | -20 | -22 | -22 | -25 | -40 | -40 | N/A |
| Antenna height (m) | 20 | 8-20 | 20 | 20 | 40 | 10 | 10 | 6 – 13 |
| Receiver IF 3 dB bandwidth (MHz) | 4.8, 2.4, 0.25 | 4, 2, 1 | 2-8 | 8 | 1.2, 10 | 4 | 4 | N/A |
| Receiver noise figure (dB) | 5 | 5 | 11 | 5 | 10 | 5 | 5 | 13 |
| Minimum discernable signal (dBm) | –107 | –100 | –107,–117 | –100 | -94 (short) -102 (wide) | -103 | -103 | -108 |

Note 1: Radars X and Y can operate both in fixed frequency and in hopping mode: the following parameters have to be taken into account in the different compatibility studies in the band 5725-5875 between industrial applications using technologies different from UWB and Radiolocation service.	
	Frequency hopping characteristics	
		Frequency band: 			5250 to 5850 MHz or 5470 to 5850 MHz	
		type of frequency hopping:  	random	
		hopping rate: 			300 to 1500 Hz	
		number of frequency:  		1 frequency /10 MHz

## Road Transport and Traffic Telematics (RTTT) Systems

ECC Decision (02)01 [10] designates the frequency bands 5795-5805 MHz, with possible extension to 5815 MHz, for RTTT. The band 5795-5805 MHz is for use by initial road-to-vehicle systems, in particular road toll systems, with an additional sub-band, 5805-5815 MHz, to be used on a national basis to meet the requirements of multi-lane road junctions.

### Parameters

The regulatory parameters (maximum power levels) for RTTT are given in Annex 5 of ERC Recommendation 70-03 [2]. The RTTT parameters used in this Report are taken from the EN 300 674 [11] developed by ETSI and the EN12253 [12] developed by CENELEC. It should be noted that the EN 300 674 deals with both Road Side Units (RSU) and On-Board Units (OBU) and is divided in two parts, the part 1 providing general characteristics and test methods, the part 2 containing the essential requirements under article 3.2 of the R&TTE Directive [13].

Table 10: Summary of characteristics of the RTTT systems

|  | Road Side Units | On Board Units |
| --- | --- | --- |
| Frequency range (MHz) | 5795 and 5815 |  |
| e.i.r.p. | 2 W (33 dBm) standard for -35° ≤ θ ≤ 35° 18 dBm for θ > 35° 8 W (39 dBm) optional | Maximum re-radiated sub-carrier e.i.r.p.: -24 dBm (Medium data rate) -14 dBm (High data rate) |
| Antenna gain | 10 – 20 dB (assumed front-to-back ratio of 15 dB) | 1 – 10 dB (assumed front-to-back ratio of 5 dB) |
| Transmitter Bandwidth | 1 MHz | 500 kHz |
| Receiver bandwidth | 500 kHz | 200 MHz – 1.4 GHz (not used) |
| Polarization | left circular | left circular |
| Receiver sensitivity (at the receiver input) | -104 dBm (BPSK) | -60 dBm |
| Co-channel C/I (dB) | 6 for 2-PSK, 9 for 4-PSK, 12 for 8-PSK | Not defined |

Figure 10 depicts the RTTT frequency utilization for 1,5 MHz sub-carrier frequency, according the EN 300 674. The location of downlink channels from RSU to OBU and the location of uplink channels from OBU to RSU become visible.

Figure 10: RTTT frequency utilization for 1,5 MHz sub-carrier frequency, according the EN 300 674

The transmit power limits of RTTT downlink, uplink and out of band emissions are depicted in Figure 11.

Figure 11: e.i.r.p. limits of RTTT

Furthermore the typically antenna pattern of RSU is depict in Figure 12 [EN12253]

Figure 12: Antenna pattern RSU

### Protection Criteria

On Board Unit (OBU)

The OBU requires a -60 dBm signal in order to function at all and to understand commands from the RSU.  Assuming negligible re-radiation loss and a signalling distance of 8 m, the received signal strength at the OBU should be -59 dBm or higher . This corresponds to power density of -56 dBm/MHz. Assuming that simple BPSK is used, the required margin is 6 dB and thus the protection criterion for the OBU would be -62 dBm/MHz on-axis and -57 dBm/MHz off-axis.

Road Site Unit (RSU)

The RSU, when operating in BPSK mode requires a 6 dB margin over its receiver sensitivity: this gives -107 dBm at the receiver input or density of -98 dBm/MHz at the input to an antenna with a -9 dB off-axis gain. Since the RSU antenna points at the road surface, no on-axis gain is taken into consideration.

## Fixed Service

In this frequency range, two types of Fixed Service links may be deployed Point-to-Point links and Point-to-Multipoint links as Broadband Fixed Wireless Access (BFWA) systems.

ECC Report 173 [14] indicates that in “the lightly licensed 5.8 GHz frequency band FWA (fixed and nomadic) operation continues to be possible on a national basis under the framework set by ECC Recommendation ECC/REC(06)04 [15] and ETSI Harmonised Standard EN 302 502 [16]. Coexistence considerations result in a low EIRP constraints and a need to implement a demanding Dynamic Frequency Selection (DFS) feature for the protection of primary Radiodetermination service.”

“The band is unlicensed/light licensed, from 5.725 to 5.850 GHz.

From 5.85 to 5.95, some use is indicated, mostly for P-MP. The licensing regime appears to be mostly link-based. Few countries indicated a significant use, including the Russian Federation (1400 P-P links, 600 P-MP Base Stations, infrastructure and broadcasting).

National frequency plans have been declared in majority of answers.”

### Point-to-Point Links

ITU-R Recommendation F.383-7 [17] defines the channel arrangements for the lower 6 GHz band. Depending on which channel arrangements are chosen, the frequency range may extend from 5850 – 6425 MHz. ERC Recommendation 14-01 [18] defines the CEPT harmonised channel plans for Radio-frequency channel arrangements for high capacity analogue and digital radio-relay systems operating in the band 5925 MHz – 6425 MHz.

The harmonised CEPT arrangements are based on recommends 1 of Recommendation F.383-7 [17], which do not extend below 5925 MHz. There is a limited deployment of Point-to_point links,therefore this case is not considered in the compatibility studies.

### Broadband Fixed Wireless Access (BFWA) Systems

ECC Report 101 [19] indicated that Broadband Fixed Wireless Access (BFWA) is used here to refer to wireless systems that provide local connectivity for a variety of applications and using a variety of architectures, including combinations of access as well as interconnection. ECC Report 68 [20] depicts the different architectures of BFWA and provides the relevant information on these different kinds of networks including technical parameters to ensure compatibility with other systems. The Table 11 below gives the main parameters for two BFWA architectures, Point to Multipoint (P-MP) and Mesh.

The 5.725-5.875 GHz band should be able to provide sufficient spectrum for commercial BFWA operations, even though exclusive frequency allocations and channel co-ordination is not envisaged in this band.

[Editor’s note: we probably need to reflect some of the material received in response to the ECC Questionnaire on FWA.]

#### Parameters

[The following table provides an overview of the characteristics for BFWA systems [ref to ECC Rep. 101].

Table 11: Interferer and victim technical parameters

| Device | Unit | BFWA P-MP | BFWA Mesh |
| --- | --- | --- | --- |
| e.i.r.p. | dBm | 36 | 36 |
| Bandwidth | MHz | 20 | 20 |
| Antenna Gain | dBi | 18 | 10 |
| Human losses | dB | 0 | 0 |
| Sidelobe attenuation | dB | 15 | 15 |
| TPC | dB | 10 | 10 |
| Sensitivity (at the antenna input) | dBm | -86 | -86 |
| Protection criterion | C/I | 6 (BPSK) | 6 (BPSK) |

]

The technical parameters of the BFWA systems were provided by a BFWA operator in Germany. A P-MP system with a Central Station (sector antenna 120°) and a Terminal Station (direction antenna, 22dBi) was assumed to be representative for the majority of BFWA systems

Figure 13: Typical BFWA P-MP deployment scenario (from ECC Report 68)

The technical parameters of the BFWA system used in this study can be found in Table 12.

Table 12: BFWA system parameters

| Parameter | BFWA P-MP |
| --- | --- |
| Topology | Receiver Sectored Central Station (CS) Transmitter Terminal Stations (TS). |
| Channel bandwidth | 20 MHz |
| Duplex/ Access scheme | TDD/TDMA |
| Max Tx power dBm TS (conducted) | 14 dBm |
| Max e.i.r.p. | 36 dBm |
| Max Power density spectral (dBm/MHz) e.i.r.p. | 23 dBm/MHz |
| Adaptive power control dB | 12 dB |
| Antenna CS (Rx antenna-> Victim Link Receiver) | 16 dBi, sector antenna 120°, 4° down tilt See Figure 14 and Figure 15 |
| Antenna height CS | 30 m |
| Antenna TS (Tx antenna-> Victim Link Transmitter) | 22 dBi: See Figure 16 and Figure 17 |
| Antenna height TS | 10 m |
| Receiver sensitivity 64-QAM | -68 dBm |
| Fading Margin | 10 dB |
| Minimum wanted power at TS and BS (Sensitivity – fade margin) | -58 dBm |
| Protection ratio 64 QAM | C/(I+N) 24 dB C/N=C/I=27 dB |
| Receiver Noise floor | -95 dBm |
| Link lenght | Typical 1 km |
| Path loss model | Spherical diffraction (outdoor, outdoor) see Annex 1 |

Figure 14: Typical P-MP Central station antenna pattern ((Ubiquiti Networks, www.ubnt.com))

Figure 15: Typical P-MP Central station (Ubiquiti Networks, www.ubnt.com)

Figure 16: Typical BFWA Terminal Station antenna pattern (Ubiquiti Networks, www.ubnt.com)

Figure 17: Typical BFWA TS antenna (Ubiquiti Networks, www.ubnt.com)

### Protection criteria [to be deleted?]

## FIXED SATELLITE (E-S) SERVICE (FSS)

As shown in Table 13, FSS deployments use the whole band 5725 – 5875 MHz and it is used by transmitting earth stations in the Earth-to-space direction operating only to satellites in geostationary orbits. In the 125 MHz portion of the band up to 5850 MHz, this is a Region 1 allocation only (i.e. only Europe, Africa, and some of the northernmost countries in Asia). Above 5850 MHz the band is part of the heavily utilised FSS global uplink band and most of the currently operating satellites (INTELSAT & New Skies for instance) have receive transponders in this upper portion of the band.

Table 13 provides details of the selection of satellites that have been taken as representative of those requiring protection in the visible portion of the geostationary orbit from Europe. The parameters shown are those required in sharing studies with the industrial applications using technologies different from UWB systems. In these frequency bands, the satellite beams cover very large areas of the Earth (using global, hemispherical, zonal or regional beams) as can be seen by the satellite footprint coverage plots in Annex 6. These gain contour plots are used to determine the receive gain in the direction of the non UWB devices.

Table 13: Sample Satellite Data taken from ITU filings for the band 5725 – 5875MHz

| Satellite | Sub-satellite longitude | Part of Frequency range 5725-5875 MHz used | Satellite Maximum Receive Gain Gsat(dBi) | Space Station Receiving System Noise Temperature Tsat (Kelvin) |
| --- | --- | --- | --- | --- |
| A | 5o West | Whole band | 34 | 773 |
| B | 14o West | Whole band | 26.5 | 1200 |
| C | 31.5o West | > 5850 MHz | 32.8 | 700 |
| D | 3o East | Whole band | 34 | 773 |
| E | 18o West | >5850MHz | 32.8 | 700 |
| F | 53o East | Whole band | 26.5 | 1200 |
| G | 59.5o East | Whole band | 34 | 1200 |
| H | 66o East | >5850 MHz | 34.7 | 700 |
| I | 359o East | >5850 MHz | 32.8 | 700 |

Figure 18 shows the basic sharing scenario between FWA terminals and the FSS service.  The studies reported on in Section 5.4 address the aggregate emissions of a large number of FWA terminals into the satellite receivers.

Figure 18: FSS/FWA Sharing Scenario in the band 5725-5875 MHz

## AMATEUR AND AMATEUR-SATELLITE (S-E) SERVICES

The amateur and amateur-satellite (s-E) services have harmonised allocations in all three ITU Regions in the frequency range 5725 – 5850 MHz with secondary status as follows:

Table 14: Allocations for Amateur Services

| Frequency | Service |
| --- | --- |
| 5725 - 5830 MHz | Amateur |
| 5830 - 5850 MHz | Amateur Amateur Satellite (space-to-Earth) |

The operational characteristics of amateur stations and amateur-satellite stations vary significantly. However based on the IARU Region-1 VHF Managers Handbook they can be categorised as:-

- Weak signal reception of Narrowband Terrestrial and EME (Moonbounce) operation in the sub-band 5760 - 5762 MHz, including propagation beacons.

- Data and multimedia systems (point to-point links and area repeaters) in other parts of the band

- Low-power satellite downlinks within 5830 – 5850 MHz (typically from LEO Cubesat satellites)

### Characteristics for the Amateur Service

Recommendation ITU-R M.1732-1 [21] provides characteristics of stations operating in the amateur service for use in sharing studies:-

Table 15: Examples of Amateur Service characteristics in the band 5725-5850 MHz

| Parameter | CW-Morse, EME | SSB Voice | FM Voice | Digital Voice and Multimedia |
| --- | --- | --- | --- | --- |
| Transmitter Power(1) (dBW) | 3 – 20 (typically: 13) | 3 – 20 (typically: 13) | 3 – 20 (typically: 10) | 1-10 (typically: 6) |
| Transmitter Feeder Loss (dB) | 1 – 2 | 0 – 10 | 0 - 10 | 1 – 6 |
| Antenna gain (dBi) | 0 – 40 | 0 – 40 | 0 - 40 | 0-30 |
| Typical e.i.r.p.(dBW) | 1 -45 | 1 – 45 | 1 – 45 | 10-40 |
| Antenna polarisation | Horizontal, Vertical, | Horizontal, Vertical | Horizontal, Vertical | Horizontal, Vertical. |
| Receiver IF bandwidth (kHz) | 0.4 | 2.7 | 9 15 | 2.7, 6, 16, 130, 10500 |
| Receiver Noise Figure(2) (dB) | 1 – 7 (typically: 1 ) | 1 – 7 (typically 1) | 1 – 3 (typically: 2) | 2 |

(1) Maximum powers are determined by each administration.

(2) Receiver noise figures for bands above 50 MHz assume the use of low-noise preamplifiers.

In order to simplify studies, it is assumed a typical terrestrial CW/SSB amateur station uses a 0.9 m (33 dBi) dish directional antenna, 13 dBW (20 Watt transmitter), and has a 1 dB NF receiver optimised for the 5760-5762 MHz band.

The determination of the maximum tolerable interference level from emissions of a single WIA device at the Amateur Service (AS) receiver is based on ECC Report 172 [22], where it is said that this level should be lower than N + (I/N) where N is the AS receiver inherent noise level and I/N the interference to noise ratio. The interference to noise ratio can be taken as –10 dB as given in ECC Report 172 [22].

### Characteristics for the Amateur-Satellite Service

Recommendation ITU-R M.1732-1 [21] provides characteristics of stations operating in the amateur-satellite service for use in sharing studies:

Table 16: Characteristics of Amateur-Satellite systems in the space-to-Earth direction

| Parameter | CW-Morse | SSB, FM, Digital Voice, Data |
| --- | --- | --- |
| Transmitter Power (1) (dBW) | 10 | 10 |
| Transmitter Feeder Loss (dB) | 0.2 – 1 | 0.2 – 1 |
| Transmitting antenna gain (dBi). | 0 – 6 | 0 |
| Typical e.i.r.p.(dBW) | 0 – 15 | 9 – 15 |
| Antenna polarisation | Horizontal, Vertical, LHCP, RHCP | Horizontal, Vertical, LHCP, RHCP |
| Receiver IF bandwidth (kHz) | 0.4 | 2.7, 16, 50, 100 |
| Receiver Noise Figure (2) (dB) | 1 – 3 (typically: 1) | 1 – 3 (typically: 2) |

(1) Maximum powers are determined by each administration.

(2) Receiver noise figures for bands above 50 MHz assume the use of low-noise preamplifiers.

Most current amateur satellites are typically picosats (also called ‘cubesats’) that occupy slightly elliptical Sun-Synchronous low earth orbits (LEO) of 600-800km altitude. These smaller satellites have relatively low power and antenna gain.

For sharing studies 0 dBW Tx Power, 0.5 dB feeder loss and 3 dB antenna gain for a patch antenna are assumed for the satellite; and the amateur receiving groundstation is assumed to be similar to the amateur service.

It should be noted that most amateur satellite downlinks are coordinated and harmonised in a sub-band at 5840-5842 MHz.

## INTELLIGENT TRANSPORT SYSTEMS (ITS)

Radiocommunication systems in the 5 GHz range can today offer communications with a high data rate, ranges up to 1 000 m, low weather-dependence, and global compatibility and interoperability for ITS communication.

The connectivity required by the applications can be summarized as:

Inter-Vehicles  (IVC) (this includes multi-hop routing involving several vehicles):

- Linear (e.g. for convoys of vehicles);

- Vehicle cluster covering several lanes (e.g. for lane management, overtaking assist).

Vehicle to Roadside (uplink) V2R and roadside to vehicle R2V (downlink):

- One vehicle to beacon;

- Beacon to one vehicle;

- Beacon to many vehicles (broadcast, short range and long range);

- Beacon to selected vehicles.

Cluster of vehicles communication, including to roadside beacon.

This is further described in ECC Report 101 [19]. ITS use for non-safety application the frequency range from 5855 MHz to 5875 MHz.

### Parameters

ECC Report 101 [19] considers the following list of parameters for ITS systems.

Table 17: Systems parameters (not exhaustive)

| Parameter | Value | Comments |
| --- | --- | --- |
| Frequency stability | 1ppm | This figure takes account of the frequency tolerance allowed by IEEE 802.11, together with the expected Doppler variation from a vehicle closing speed of 400 km/h. |
| Maximum radiated power (e.i.r.p.) | Equipment classes: A 10 dBm B 20 dBm C 33 dBm | Transmitter power control (TPC) with a 30 dB range. |
| Antenna beam shape/gain | RSU: 10 dBi OBU: 5; 8 dBi | See Figure 20 and Figure 21 |
| Polarization | TBD | Circular and linear polarisations each have certain benefits. Some degree of rejection of emissions from oppositely travelling vehicles may be required. |
| Modulation scheme | BPSK QPSK 16QAM 64QAM | This is the standard set within IEEE 802.11 |
| Data rates | 3/4.5 /6/9/ 12/18 /24/27 Mbit/s | This is the standard set within IEEE 802.11. As an option two channels may be combined to produce double data rates (up to 54 Mbit/s). Default data rate is 6Mbit/s. |
| Channel Bandwidth | 10 MHz, option 20 MHz | This is the standard set within EEE 802.11. |
| Communication mode | Half-duplex, broadcast | Half-duplex and broadcast are believed to be adequate for the applications considered to date. |
| Receiver sensitivity | -92dBm/MHz | Based on a -82 dBm for a bandwidth of 10 MHz |
| Protection criterion | C/I=6dB | For a classical BPSK signal |

Communication channels will be open for the applications within the respective usage category (either road safety related or not, i.e. used for traffic management).

The required power levels (e.i.r.p.) range from 3 dBm to 33 dBm to achieve communication distances of up to 1000 m.

The technical parameters used for interference assessment are given in the following table.

Table 18: Technical requirements of ITS devices

| Characteristics | Value | Unit |
| --- | --- | --- |
| Receiver bandwidth | 10 | MHz |
| Receiver sensitivity | -82 | dBm |
| Antenna gain (see note 1) | 8 | dBi |
| Receiver sensitivity at antenna input | -100 | dBm/MHz |
| C/I | 6 | dB |
| Allowable Interfering Power at receiver antenna input | -106 | dBm/MHz |
| Transmitter Characteristics |  |  |
| Bandwidth | 10 | MHz |
| Txout, e.i.r.p | 33 | dBm |
| Txout e.i.r.p per MHz | 23 | dBm/MHz |
| Assumed value for TPC | 8 | dB |
| Net Txout e.i.r.p | 15 | dBm/MHz |
| Antenna Gain | 8 | dBi |

Table 19: ITS Channel utilization

| Channel type | Centre frequency | CH No. | Channel spacing | Default data rate | TX power limit | TX power density limit |
| --- | --- | --- | --- | --- | --- | --- |
| G5CC | 5 900 MHz | 180 | 10 MHz | 6 Mbit/s | 33 dBm EIRP | 23 dBm/MHz |
| G5SC2 | 5 890 MHz | 178 | 10 MHz | 12 Mbit/s | 23 dBm EIRP | 13 dBm/MHz |
| G5SC1 | 5 880 MHz | 176 | 10 MHz | 6 Mbit/s | 33 dBm EIRP | 23 dBm/MHz |
| G5SC3 | 5 870 MHz | 174 | 10 MHz | 6 Mbit/s | 23 dBm EIRP | 13 dBm/MHz |
| G5SC4 | 5 860 MHz | 172 | 10 MHz | 6 Mbit/s | 0 dBm EIRP | -10 dBm/MHz |
| G5SC5 | As required in EN 302 571 for the band 5 470 MHz to 5 725 MHz |  | several | dependent on channel spacing | 30 dBm EIRP (DFS master) | 17 dBm/MHz |
|  |  |  |  |  | 23 dBm EIRP (DFS slave) | 10 dBm/MHz |

Unwanted emission levels are given by Rec. ITU-R SM.1541 [23] for the out of band domain and SM.329 [24] and ERC Recommendation 74-01 [25] for the spurious domain. Table 20 provides unwanted emissions for ITS, which are considered in the following sections of this report.

Table 20: Attenuation (dBr) below the e.i.r.p. (dBm/MHz)

|  | e.i.r.p. (dB/MHz) | ±4,5 MHz (dBr) | ±5 MHz (dBr) | ±5,5 MHz (dBr) | ±10 MHz (dBr) | ±25 MHz (dBr) |
| --- | --- | --- | --- | --- | --- | --- |
| Class A | 0 | 0 | -10 | -20 | -28 | -40 |
| Class B | 10 | 0 | -16 | -20 | -28 | -40 |
| Class C | 23 | 0 | -26 | -32 | -40 | -50 |

Figure 19: Power spectral density of ITS device

Typical antenna pattern for the two ITS devices RSU and OBU are shown in Figure 20 and Figure 21. It is an essential design feature of communications between vehicles, or between vehicles and local infrastructure beacons, that they are directed more or less horizontally in a typical omni-directional pattern with typically 8 dBi gain in the horizontal plane.

Figure 20: Antenna pattern RSU (antenna gain: 10 dBi)

Figure 21: Antenna pattern OBU (antenna gain: 8 dBi)

### Protection criteria

## Short Range Devices (SRD)

### General (non-specific) SRD

As specified in Annex 1 of ERC Recommendation 70-03 [2], the frequency band 5725-5875 MHz is used by non-specific SRD. This use should comply with the technical characteristics as shown below.

Table 21: Technical characteristics of SRD

| Frequency Band | Power | Antenna | Channel Spacing | Duty Cycle (%) |
| --- | --- | --- | --- | --- |
| 5725-5875 MHz | 25 mW e.i.r.p | Integral (no external antenna socket) or dedicated | No channel spacing - the whole stated frequency band may be used | No duty cycle restriction |

In addition to these regulatory technical characteristics, assumptions on some parameters had to be made in order to carry out compatibility studies. Three kinds of SRD are considered for the interference assessment (see the following table) as in previous CEPT studies [20] or [19] ?.

Table 22: Assumed SRD parameters

| Parameter | SRD I | SRD II | SRD III | Comments |
| --- | --- | --- | --- | --- |
| Typical bandwidth BW (MHz) | 0.25 MHz | 20 MHz | 8MHz | Note 1, Note 2. |
| TX Power, dBm e.i.r.p. | +14 | +14 | +14 |  |
| Ant. Gain, dBi | 2 to 20 | 2 to 24 | 2 |  |
| Ant. Polarization | Circular | Circular | Vertical |  |
| Receiver sensitivity, dBm | -110 | -91 | -84 |  |
| Receiver noise dBm/MHz | -114 | N/A | N/A |  |
| Protection criterion, dB | I/N=0dB | C/I=8dB | C/I=20dB |  |
| SRD Noise figure F | 9.00 dB | N/A | N/A |  |
| FkTB | -105 dBm/MHz | N/A | N/A |  |
| Max OoB RX interference, dBm | -35 | -35 | -35 | E.g. limit for Rx blocking |
| Duty cycle : % | Up to 100% | Up to 100% | 100% |  |
| RX wake-up time (if applicable) | 1 sec | 1 sec | N/A | For battery operated equipment |
| Note 1: The given bandwidths are for non-spread spectrum modulation. Note 2: For spread spectrum modulation (FHSS, DSSS and other types) the bandwidth can be up to 100 MHz |  |  |  |  |

### Tank Level Probing Radar (TLPR)

Tank Level Probing Radar (TLPR) are installed in closed metallic tanks or reinforced concrete tanks, or similar enclosure structures made of comparable attenuating material, holding a substance, liquid or powder. TLPR utilizes bandwidths that extend over multiple frequency bands allocated to numerous radio services. The emission limits for TLPR are specified outside the tank (see EN 302 372 [26]).

The power levels for these bands are -41.3 dBm/MHz e.i.r.p. but in reality the power levels outside the actual tank are much lower than this maximum value. The value of -41.3 dBm/MHz e.i.r.p. is based on a standard test tank of 500 litre. The reason for doing so is as follows. Levels outside an installed tank are in many cases difficult if not impossible to measure because average is ranging from 8 to 100,000 cubic meters. In practice therefore a test tank of 500 litre is used that fits for example in an anechoic chamber. The practical outside levels of an installed tank however are lower (varying from 20dB to 30dB thanks the cavity attenuation) than the -41.3 dBm/MHz limit the test tank emits.

Given the specificities of TLPRs, in particular of typical deployment scenarios (low numbers, at fixed location, in industrial plant areas…), CEPT administrations had considered in the past that compatibility studies for TLPR are not necessary (see CEPT Report 26 [28]). A similar approach was adopted in this report and no compatibility studies were developed.

# COMPATIBILITY STUDIES

The section details the compatibility studies between the industrial applications using other technologies different from UWB (WIA) systems detailed in section 4 and other radio communications services and systems which were detailed in section 5.

## Radiolocation Service

This section of the report examines the prospects of co-channel sharing between radar systems and the industrial applications using other technologies different from UWB operating in frequency band 5725 – 5850 MHz. Information and technical characteristics of the considered radars can be found in section 4.1. This section provides basic calculations of the interference level from a single WIA device into radars and identifies the need for mitigation techniques which are described in subsequent sections.

### Step 1: MCL Calculations

#### Methodology for calculating interference from WIA into Radar

The determination of the maximum tolerable interference level from emissions of a single WIA device at the radar receiver is based on Recommendation ITU-R M.1461, where it is said that this level should be lower than N + (I/N) where N is the radar receiver inherent noise level and I/N the interference to noise ratio. The interference to noise ratio can be taken as –6 dB as given in Recommendations ITU-R M.1461 [24] and ITU-R M.1638 [25].

Interference from WIA into Radars

In this section,  the method used to calculate the potential interference to Radiolocation devices is based on the Minimum Coupling Loss (MCL) required between radars and WIA systems. The separation distances can be calculated using the free space propagation model.

MCL=PT_WIA+Corr-IRadar

where

MCL		Minimum Coupling Loss in dB

PT_WIA		Maximum Transmit Power, before antenna and feeders (WIA) in dBW

Corr		Bandwidth correction factor:

if BWRadar smaller than BWWIA : 10logBWRadarBWWIA

if BWRadar larger than BWWIA: 0

and where:

BWRadar		Receiver Noise Bandwidth (Radar) in Hz

BWWIA		Transmitter Bandwidth (WIA) in Hz

IRadar		Maximum Permissible Interference at Receiver after antenna and feeder (Radar) in dB

The MCL is then converted into the required propagation loss LP as follows:

LP=MCL+GT_WIA-LT_WIA+GR_Radar-LR_Radar-LWall Loss

where

GT_WIA		Gain of the WIA antenna in dBi

LT_WIA		WIA feeder loss in dB

GR_Radar		Gain of Radar antenna in dBi

LR_Radar		Radar feeder loss in dB

LWall Loss		Wall loss in dB, 15 dB for indoor systems and 0 dB for outdoor systems

The required separation distances d (in meters) were calculated, assuming free space propagation, from:

d=λ4π∙10LP20

where λ is the wavelength given in meters.

For these calculations, basic assumptions have been chosen for the WIA parameters transmit power and antenna gain, leading to an e.i.r.p. of 26 dBm:

- in a bandwidth of 1 MHz and 20 MHz for the indoor case (WIA I and WIA II)

- In a bandwidth of 3 MHz and 20 MHz for the outdoor case (WIA III and WIA IV)

Table 23: Separation distances WIA I – Radar systems

| Parameter |  | I | M | N | O | Q | X & Y | Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Radar | Tx power into antenna peak (kW) | 2800 | 1200 | 1000 | 165 | 285 | 12 | 70 |
|  | Tx power into antenna peak (dBm) | 94.5 | 90.8 | 90.0 | 82.2 | 84.5 | 70.8 | 78.5 |
|  | Receiver IF3dB bandwidth (MHz) | 4.8 | 4 | 8 | 8 | 10 | 4 | 1 |
|  | Antenna main beam gain | 54 | 47 | 45.9 | 42 | 30 | 35 | 31.5 |
|  | Antenna height (m) | 20 | 15 | 20 | 20 | 40 | 10 | 10 |
|  | Radar feeder loss (dB) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | E.i.r.p radar (dBm) | 148.5 | 137.8 | 135.9 | 124.2 | 114.5 | 105.8 | 110.0 |
|  | Mini discernible signal (dBm) | -107 | -100 | -107 | -100 | -102 | -103 | -108 |
|  | Receiver noise figure (dB) | 5 | 5 | 11 | 5 | 10 | 5 | 13 |
|  | Noise level NBW (dBm) | -107.2 | -108.0 | -104.9 | -104.9 | -104.0 | -108.0 | -114.0 |
|  | Noise level N (dBm) | -102.2 | -103.0 | -93.9 | -99.9 | -94.0 | -103.0 | -101.0 |
|  | Noise level protection criterion (dBm) | -108.2 | -109.0 | -99.9 | -105.9 | -100.0 | -109.0 | -107.0 |
| WIA I | WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
|  | WIA feeder loss | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Bandwidth (MHz) | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Calculations | Bandwidth correction factor (dB) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Minimum coupling loss (dB) | 134.2 | 135.0 | 125.9 | 131.9 | 126.0 | 135.0 | 133.0 |
|  | Wall Loss (dB) | 15.0 | 15.0 | 15.0 | 15.0 | 15.0 | 15.0 | 15.0 |
|  | Required propagation loss (dB) | 173.2 | 167.0 | 156.8 | 158.9 | 141.0 | 155.0 | 149.5 |
|  | Frequency (MHz) | 5800 | 5800 | 5800 | 5800 | 5800 | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 |
|  | Free space distance (km) | 1873 | 916 | 286 | 364 | 46 | 230 | 122 |

Table 24: Separation distances WIA II – Radar systems

| Parameter |  | I | M | N | O | Q | X & Y | Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Radar | Tx power into antenna peak (kW) | 2800 | 1200 | 1000 | 165 | 285 | 12 | 70 |
|  | Tx power into antenna peak (dBm) | 94.5 | 90.8 | 90.0 | 82.2 | 84.5 | 70.8 | 78.5 |
|  | Receiver IF3dB bandwidth (MHz) | 4.8 | 4 | 8 | 8 | 10 | 4 | 1 |
|  | Antenna main beam gain | 54 | 47 | 45.9 | 42 | 30 | 35 | 31.5 |
|  | Antenna height (m) | 20 | 15 | 20 | 20 | 40 | 10 | 10 |
|  | Radar feeder loss (dB) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | E.i.r.p radar (dBm) | 148.5 | 137.8 | 135.9 | 124.2 | 114.5 | 105.8 | 110.0 |
|  | Mini discernible signal (dBm) | -107 | -100 | -107 | -100 | -102 | -103 | -108 |
|  | Receiver noise figure (dB) | 5 | 5 | 11 | 5 | 10 | 5 | 13 |
|  | Noise level NBW (dBm) | -107.2 | -108.0 | -104.9 | -104.9 | -104.0 | -108.0 | -114.0 |
|  | Noise level N (dBm) | -102.2 | -103.0 | -93.9 | -99.9 | -94.0 | -103.0 | -101.0 |
|  | Noise level protection criterion (dBm) | -108.2 | -109.0 | -99.9 | -105.9 | -100.0 | -109.0 | -107.0 |
| WIA II | WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
|  | WIA feeder loss | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Bandwidth (MHz) | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Calculations | Bandwidth correction factor (dB) | -6.2 | -7.0 | -4.0 | -4.0 | -3.0 | -7.0 | -13.0 |
|  | Minimum coupling loss (dB) | 128.0 | 128.0 | 122.0 | 128.0 | 123.0 | 128.0 | 120.0 |
|  | Wall Loss (dB) | 15.0 | 15.0 | 15.0 | 15.0 | 15.0 | 15.0 | 15.0 |
|  | Required propagation loss (dB) | 167.0 | 160.0 | 152.9 | 155.0 | 138.0 | 148.0 | 136.5 |
|  | Frequency (MHz) | 5800 | 5800 | 5800 | 5800 | 5800 | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 |
|  | Free space distance (km) | 917 | 410 | 181 | 230 | 33 | 103 | 27 |

Table 25: Separation distances WIA III – Radar systems

| Parameter |  | I | M | N | O | Q | X & Y | Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Radar | Tx power into antenna peak (kW) | 2800 | 1200 | 1000 | 165 | 285 | 12 | 70 |
|  | Tx power into antenna peak (dBm) | 94.5 | 90.8 | 90.0 | 82.2 | 84.5 | 70.8 | 78.5 |
|  | Receiver IF3dB bandwidth (MHz) | 4.8 | 4 | 8 | 8 | 10 | 4 | 1 |
|  | Antenna main beam gain | 54 | 47 | 45.9 | 42 | 30 | 35 | 31.5 |
|  | Antenna height (m) | 20 | 15 | 20 | 20 | 40 | 10 | 10 |
|  | Radar feeder loss (dB) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | E.i.r.p radar (dBm) | 148.5 | 137.8 | 135.9 | 124.2 | 114.5 | 105.8 | 110.0 |
|  | Mini discernible signal (dBm) | -107 | -100 | -107 | -100 | -102 | -103 | -108 |
|  | Receiver noise figure (dB) | 5 | 5 | 11 | 5 | 10 | 5 | 13 |
|  | Noise level NBW (dBm) | -107.2 | -108.0 | -104.9 | -104.9 | -104.0 | -108.0 | -114.0 |
|  | Noise level N (dBm) | -102.2 | -103.0 | -93.9 | -99.9 | -94.0 | -103.0 | -101.0 |
|  | Noise level protection criterion (dBm) | -108.2 | -109.0 | -99.9 | -105.9 | -100.0 | -109.0 | -107.0 |
| WIA III | WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
|  | WIA feeder loss | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Bandwidth (MHz) | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Calculations | Bandwidth correction factor (dB) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -4.8 |
|  | Minimum coupling loss (dB) | 134.2 | 135.0 | 125.9 | 131.9 | 126.0 | 135.0 | 128.2 |
|  | Wall Loss (dB) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Required propagation loss (dB) | 188.2 | 182.0 | 171.8 | 173.9 | 156.0 | 170.0 | 159.7 |
|  | Frequency (MHz) | 5800 | 5800 | 5800 | 5800 | 5800 | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 |
|  | Free space distance (km) | 10530 | 5152 | 1609 | 2049 | 259 | 1294 | 398 |

Table 26: Separation distances WIA IV – Radar systems

| Parameter |  | I | M | N | O | Q | X & Y | Z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Radar | Tx power into antenna peak (kW) | 2800 | 1200 | 1000 | 165 | 285 | 12 | 70 |
|  | Tx power into antenna peak (dBm) | 94.5 | 90.8 | 90.0 | 82.2 | 84.5 | 70.8 | 78.5 |
|  | Receiver IF3dB bandwidth (MHz) | 4.8 | 4 | 8 | 8 | 10 | 4 | 1 |
|  | Antenna main beam gain | 54 | 47 | 45.9 | 42 | 30 | 35 | 31.5 |
|  | Antenna height (m) | 20 | 15 | 20 | 20 | 40 | 10 | 10 |
|  | Radar feeder loss (dB) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | E.i.r.p radar (dBm) | 148.5 | 137.8 | 135.9 | 124.2 | 114.5 | 105.8 | 110.0 |
|  | Mini discernible signal (dBm) | -107 | -100 | -107 | -100 | -102 | -103 | -108 |
|  | Receiver noise figure (dB) | 5 | 5 | 11 | 5 | 10 | 5 | 13 |
|  | Noise level NBW (dBm) | -107.2 | -108.0 | -104.9 | -104.9 | -104.0 | -108.0 | -114.0 |
|  | Noise level N (dBm) | -102.2 | -103.0 | -93.9 | -99.9 | -94.0 | -103.0 | -101.0 |
|  | Noise level protection criterion (dBm) | -108.2 | -109.0 | -99.9 | -105.9 | -100.0 | -109.0 | -107.0 |
| WIA IV | WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
|  | WIA feeder loss | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Bandwidth (MHz) | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Calculations | Bandwidth correction factor (dB) | -6.2 | -7.0 | -4.0 | -4.0 | -3.0 | -7.0 | -13.0 |
|  | Minimum coupling loss (dB) | 128.0 | 128.0 | 122.0 | 128.0 | 123.0 | 128.0 | 120.0 |
|  | Wall Loss (dB) | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Required propagation loss (dB) | 182.0 | 175.0 | 167.9 | 170.0 | 153.0 | 163.0 | 151.5 |
|  | Frequency (MHz) | 5800 | 5800 | 5800 | 5800 | 5800 | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 |
|  | Free space distance (km) | 5159 | 2304 | 1017 | 1296 | 183 | 579 | 154 |

#### Determination of required separation distance

The results of Table 5.1.1 below show that, with all the radars under consideration, the necessary separation distances are determined by the value of the radio-horizon He which is calculated with the following formula:

Hekm=4.12∙hWIA+hRadar

where hWIA and hRadar correspond to the antenna heights of the WIA and radar respectively.

Figure 22: Radar and WIA horizon

Table 27: Separation distance - horizon

| Characteristic | Factory hall | Plant level |
| --- | --- | --- |
| Range of antenna high WIA | 2 to 6 m | 1.5 to 50 m |
| Typical Antenna high WIA | 4 m | 2 m to 50 m |
| He (hRadar=10 m) | 21.2 km | 18.9 km to 42.2 km |
| He (hRadar=15 m) | 24.2 km | 21.8 km to 45.1 km |
| He (hRadar=20 m) | 26.7 km | 24.3 km to 47.6 km |
| He (hRadar=40 m) | 34.3 km | 31.9 km to 55.2 km |

Note: manufacturing cells are not considered since they are intended to be deployed in the framework of the existing regulations Rec. 70-03 Annex 1 – band i.

With the assumed antenna heights for hWIA and hRadar, the required separation distance is in the order of 20 to 35 km in the indoor case and 20 to 55 km for the outdoor case for single device with a very low duty cycle of 0.02 %.

#### Step 2: aggregated impact

Assumptions

Parameters

Attenuation wall: 10dB [note: 15 dB used in other sections of the report]

Radar antenna height: 10m

Same polarisation for victim and interferer

Methodology

To achieve cohabitation without constraint, WIA signal (considered as an interferer) seen by radar receiver should not be upper than radar interference criteria (Ic). Calculations are realised between WIA and radar with the following equation:

Necessary Attenuation Loss = EIRP_WIA + Gant_rad – 10 x log(BW_WIA/BW_rad) – Ic

Necessary attenuation loss can be expressed in terms of separation distance between the two systems. Distances are calculated with Free Space propagation model.

Then, propagation loss has to be converted into a separation distance between radar and WIA (using a propagation model).

Configuration

Considering the WIA system as the interferer, the intent is to study the compatibility between WIA and radars in two configurations.

| Configuration 1: Factory hall (or plant sub unit) |  |
| --- | --- |
| WIA 10 cell networks with 50 devices per cell (indoor) or WIA in a factory hall 5 parallel network with 100 devices per cell (mostly indoor) | C band radar |

Figure 23: Cohabitation synoptic of WIA and military radar

| Configuration 2: Combination of 3 classes (manufacturing cell, factory hall, plant level) |  |
| --- | --- |
| 750m 500 m |  |
| example or industrial area |  |

Figure 24: cohabitation synoptic of WIA in an industrial area and military radar

The number of WIA seen by a radar is function of the distance between radar and WIA location and the radar antenna beam width. An aproximative number of WIA seen by a radar (in the example above, i.e. in a particular azimuth) is given in the table below:

Table 27: Number of WIA seen by a radar [format table]

| Antenna | Radar l | - Radar M | - Radar N | - Radar O | Radar Q | Radar X Y | Radar Z |
| --- | --- | --- | --- | --- | --- | --- | --- |
| elevation beam width | 0.4 | 0.8 | 1.0 | 1.0 | 28.0 | N/A | 43.8 |
| azimuthal beam width | 0.4 | 0.8 | 1.0 | 1.0 | 1.6 | N/A | 1.75 |
| Aperture at 10km (A_1) | 70m | 140m | 174m | 174m | 280m |  | 305 |
| A_1 x 750m | 0,05km² | 0,1km² | 0,12km² | 0,12km² | 0,21km² |  | 0,22km² |
| Number of devices | 50 | 105 | 127 | 127 | 210 |  | 228 |
| Aperture at 20km (A_2) | 140m | 280m | 348m | 348m | 560m |  | 610 |
| A_2 x 750m | 0,1km² | 0,21km² | 0,25km² | 0,25km² | 0,42km² |  | 0,44km² |
| Number of devices | 100 | 210 | 254 | 254 | 420 |  | 456 |

In this example, number of devices seen by a radar is bounded by 50 and 456.

Results

WIA in configuration 1

In the first configuration,, this case is based on a point-to-point link budget on a cochannel frequency basis between only one device and a radar. It is assumed that at least one transmitter uses the same frequency than radar.

Necessary Attenuation Loss = EIRP_WIA + Gant_rad – 10 x log(BW_WIA/BW_rad) – Ic

WIA (Indoor) interference into radar receiver

EIRP_WIA = 26dBm -10dB = 16dBm

Table 29: WIA indoor link budget [format table]

|  |  | Radar Q | Radar X & Y | Radar Z |
| --- | --- | --- | --- | --- |
| Necessary Attenuation Loss | WIA 1 (1MHz) | 154dB | 160dB | 161,5dB |
|  | WIA 2 (2MHz) | 154dB | 160dB | 158,5dB |
|  | WIA 3 (20MHz) | 151,5dB | 154dB | 148,5dB |
| Separation Distance | WIA 1 (1MHz) | 204km | 408km | 490km |
|  | WIA 2 (2MHz) | 204km | 408km | 343km |
|  | WIA 3 (20MHz) | 153km | 204km | 109km |

Table 4:

WIA (Outdoor) interference into radar receiver

EIRP_WIA = 26dBm

Table 29: WIA outdoor link budget [format table]

|  |  | Radar Q | Radar X & Y | Radar Z |
| --- | --- | --- | --- | --- |
| Necessary Attenuation Loss | WIA 1 (1MHz) | 164dB | 170dB | 171,5dB |
|  | WIA 2 (2MHz) | 164dB | 170dB | 168,5dB |
|  | WIA 3 (20MHz) | 161,5B | 164dB | 158,5dB |
| Separation Distance | WIA 1 (1MHz) | 652km | 1300km | 1540km |
|  | WIA 2 (2MHz) | 652km | 1300km | 1540km |
|  | WIA 3 (20MHz) | 490km | 652km | 346km |

In these two cases (§ 3.1.1 and 3.1.2), several devices can transmit in the same time:

In the first case, up to 10 cells with 50 devices per cell can be used,

In the second case up to 5 parallel networks with 100 devices per cell in the second case can be used.

However only one WIA device is taken into account in the calculation, because there are no sufficient details about “intelligent sharing mechanism” (§2 overview of industrial applications using technologies different from UWB).

WIA in configuration 2

In this second configuration, 2 cases of cohabitation are studied:

Radar on a fixed frequency

One WIA manufacturing cell, one factory hall, and one Plant level.

This case can be considered as studied in §3.1, if it is assumed that only one device transmit on radar frequency.

As written in the draft report, “the aggregate interference effects will not increase”.

This statement has to be justified, for example when several hundreds devices are used. The number of devices on the same frequency has to be identified, in order to calculate the corresponding aggregate level.

Radar on a hopping mode

In §3.2 of draft ECC report, it is written, the aggregate interference effects will not increase, eirp is equal all over the spectrum and time.

That means all channels in the 5725-5850MHz band will be occupied, and a radar in a hopping mode will not find one available channel in this part of its frequency band. A calculation to evaluate the collision probability, between the WIA in a frequency agility mode and a radar in a hopping mode is necessary to confirm that radar receiver will see WIA signal(s) in each of its channel.

#### Step 3

DFS consideration (taken from ECC Report 110:

SEAMCAT simulations

#### DFS consideration

This section provides the methodology based on link budget analysis, that is used for the determination of the DFS detection threshold to protect radars. Such a methodology has been used to determine DFS detection thresholds for RLAN and BFWA at 5 GHz and is also considered in this report for the BBDR detection threshold.

The threshold is determined from two link budget analyses, (1) and (2) whose description is provided below. This is based on the assumption of a symmetrical propagation path between the interfering system with DFS (this system is quoted as Int in this Annex, it can be either BBDR, BFWA or RLAN) and the radar (RL) and also that the transmitter and receiver bandwidths of the radar are the same:

(1): The link budget gives the propagation losses PL to limit the interference level coming from the interfering system Int towards the radar receiver below the noise level minus 6dB (I/N=-6dB). Let d be the separation distance.

(2): The link budget gives the propagation losses PL to allow the interferer Int to detect at the distance d the presence of a radar. Therefore, the interference level coming from the radar towards the receiver of the Int system will be used as the detection threshold at the antenna connector (Th).

Note that:

: spectral density of the interferer (dBm/MHz)

: Antenna gain of the interferer

: Bandwidth of the interferer

: power of the interferer (dBm)

: Ambient noise (dBm/MHz) with noise temperature of T=290°K,

a reference bandwidth Bo=1 MHz and F the noise figure in dB

the maximum allowable level of interference on the RL

: spectral desity of the radar (dBm/MHz)

: Antenna gain of the radar

: Bandwidth of the radar

: power of the radar (dBm)

: Detection threshold at the antenna connector (dBm/MHz)

: Detection threshold at the antenna connector (dBm) in the bandwidth of the interferer

PL: Propagation losses

If BInt>BRadar

(1)

(2)

=>

If BInt<BRadar

(1)

(2)

=>

Finally, these calculations lead to the same formula

in dBm over the bandwidth of the interferer

Note: The ECC Report 68 indicated that an appropriate detection threshold ThFWA should be -69dBm (close to -67.18dBm which can be calculated using this method) over the BFWA bandwidth.

Calculations for WIA, applying the same equations as for FWA give the following results.

Table 29:  DFS threshold WIA – 1 MHz [table format]

| Characteristics of the Radars | L | M | N | O | Q | X & Y | Z |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tx power into antenna peak (kW) | 2800 | 1200 | 1000 | 165 | 285 | 12 | 70 |
| Tx power into antenna peak (dBm) | 94,47 | 90,79 | 90,00 | 82,17 | 84,55 | 70,79 | 78,45 |
| Radar bandwidth (MHz) | 4,8 | 4 | 8 | 8 | 10 | 4 | 1 |
| Noise figure (dB) | 7 | 4 | 2,3 | 3 | 3 | 5 | 13 |
| Characteristics of a WIA device |  |  |  |  |  |  |  |
| WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
| Bandwith (MHz) | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Antenna gain (dBi) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| WIA spectral density power (dBm/MHz) at the antenna connector | 22,99 | 22,99 | 22,99 | 22,99 | 22,99 | 22,99 | 22,99 |
| Characteristics of the DFS | L | M | N | O | Q | X & Y | Z |
| Noise level (dBm/MHz) | -106,98 | -109,98 | -111,68 | -110,98 | -110,98 | -108,98 | -100,98 |
| DFS Detection threshold Th (dBm) | -44,51 | -51,19 | -53,68 | -60,80 | -58,43 | -70,19 | -54,53 |

Table 30: DFS threshold WIA – 3 MHz [format table]

| Characteristics of the Radars | L | M | N | O | Q | X & Y | Z |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tx power into antenna peak (kW) | 2800 | 1200 | 1000 | 165 | 285 | 12 | 70 |
| Tx power into antenna peak (dBm) | 94,47 | 90,79 | 90,00 | 82,17 | 84,55 | 70,79 | 78,45 |
| Radar bandwidth (MHz) | 4,8 | 4 | 8 | 8 | 10 | 4 | 1 |
| Noise figure (dB) | 7 | 4 | 2,3 | 3 | 3 | 5 | 13 |
| Characteristics of a BFWA device |  |  |  |  |  |  |  |
| WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
| Bandwith (MHz) | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Antenna gain (dBi) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| WIA spectral density power (dBm/MHz) at the antenna connector | 22,99 | 22,99 | 22,99 | 22,99 | 22,99 | 22,99 | 22,99 |
| Characteristics of the DFS | L | M | N | O | Q | X & Y | Z |
| Noise level (dBm/MHz) | -106,98 | -109,98 | -111,68 | -110,98 | -110,98 | -108,98 | -100,98 |
| DFS Detection threshold Th (dBm) | -39,73 | -46,41 | -48,91 | -56,03 | -53,66 | -65,41 | -49,76 |

Table 31: DFS threshold WIA – 20 MHz [table format]

| Characteristics of the Radars | L | M | N | O | Q | X & Y | Z |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tx power into antenna peak (kW) | 2800 | 1200 | 1000 | 165 | 285 | 12 | 70 |
| Tx power into antenna peak (dBm) | 94,47 | 90,79 | 90,00 | 82,17 | 84,55 | 70,79 | 78,45 |
| Radar bandwidth (MHz) | 4,8 | 4 | 8 | 8 | 10 | 4 | 1 |
| Noise figure (dB) | 7 | 4 | 2,3 | 3 | 3 | 5 | 13 |
| Characteristics of a WIA device |  |  |  |  |  |  |  |
| WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
| Bandwith (MHz) | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Antenna gain (dBi) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| WIA spectral density power (dBm/MHz) at the antenna connector | 22,99 | 22,99 | 22,99 | 22,99 | 22,99 | 22,99 | 22,99 |
| Characteristics of the DFS | L | M | N | O | Q | X & Y | Z |
| Noise level (dBm/MHz) | -106,98 | -109,98 | -111,68 | -110,98 | -110,98 | -108,98 | -100,98 |
| DFS Detection threshold Th (dBm) | -31,50 | -38,18 | -40,67 | -47,79 | -45,42 | -57,18 | -41,52 |

Same level for the DFS threshold for indoor case and outdoor case if:

- The wall attenuation is compensated on the two paths.

- Scenarios: outdoor device linked with indoor device are not considered (note: it is not planned but can not be completely excluded…)

- Check the EN to check the characteristics which are used in the standard

- Mostly mobile radar systems. Fixed radar systems, but limited.

### The use of mitigation techniques as a method to enable sharing between Radiolocation service and WIA systems in the 5.8 GHz band

To be done:

Indoor/Outdoor use

Mitigation techniques: Dynamic frequency selection (DFS); Listen before talk (LBT), Adaptive frequency hopping (AFH);

### Conclusion

It can be concluded that an indoor use or mitigation techniques are required to enable the sharing between WIA systems and radars.

It is assumed that the indoor outdoor wall loss is 15 dB as in  ECC Report 101 [9] . Therefore the WIA systems for an industrial environment with a transmitter maximum e.i.r.p. of 400 mW are complied with the limit values of generic use SRD as defined in [10].

## Road Transport and Traffic Telematics (RTTT) Systems

WIA and RTTT systems can operate on the same channel, therefore coexistence problems can be expected at both sites.

[note: we also need to cover the situation when the mobile unit of the RTTT system is entering in a factory]

The following propagation model is used in this section:

The calculations developed in the different compatibility studies used the same propagation model as in ECC Report 68.

LFS=20log⁡λ4πdd≤d020logλ4πd0-10n0logdd0d0≤d≤d120logλ4πd0-10n0logd1d0-10n1logdd0d>d1

Another propagation model is also proposed in the ITS SRdoc ETSI TR 102 492-1 [2] [3]. Assumptions are a first breakpoint distance do at 15 m and exponent beyond no=2.7 separation distances presented below will investigate both cases.

Table 34: Parameters of propagation

|  | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- |
| Breakpoint distance d0 (m) | 64 | 128 | 256 | 15 |
| Pathloss factor n0 beyond the first break point | 3.8 | 3.3 | 2.8 | 2.7 |
| Breakpoint distance d1 (m) | 128 | 256 | 1024 | 1024 |
| Pathloss factor n1 beyond the first break point | 4.3 | 3.8 | 3.3 | 2.7 |

This section provides results of calculation for the separation distances to protect SRD in the band 5725 to 5875 MHz from WIA. The characteristics of SRD systems are provided in section 4.7.1. The WIA characteristics are provided in section 3.

### Compatibility between interferer WIA (indoor) and victim RTTT

The compatibility between interferer WIA (indoor) with 1 MHz bandwidth and the victim RTTT RSU is listed in Table 2. The final result of compatibility consideration is a required separation distance between interferer WIA an victim RTTT.

Table 35:	Interferer WIA I bandwidth 1 MHz, victim RTTT RSU (format)

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: WIA-I: 1 MHz |  |  |  |  |  |  |
| Bandwidth | 1 | MHz |  |  |  |  |
| Tx out, eirp | 26 | dBm | 26 | 26 | 26 | 26 |
| effect of TPC (dB) | 0 | dB | 0 | 0 | 0 | 0 |
| Wall loss | 15 | dB | -15 | -15 | -15 | -15 |
| Antenna Gain | 0 | dBi |  |  |  |  |
| Bandwidth correction factor (ItBW > VrBW) | 3 | dB | -3 | -3 | -3 | -3 |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 8 | 8 | 8 | 8 |
| Reception part: RSU RTTT |  |  |  |  |  |  |
| Receiver bandwidth | 0,5 | MHz | 0,5 | 0,5 | 0,5 | 0,5 |
| Receiver sensitivity | -104 | dBm | -104 | -104 | -104 | -104 |
| Antenna gain | 10 | dBi | 10 | 10 | 10 | 10 |
| C min per MHz at antenna input |  | dBm/MHz | -114 | -114 | -114 | -114 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -117 | -117 | -117 | -117 |
| MAIN LOBE WIA - MAIN LOBE RTTT |  |  |  |  |  |  |
| Separation distance WIA RTTT |  | M | 618 | 1157 | 2358 | 1432 |
| MAIN LOBE WIA - SIDE LOBE RTTT |  |  |  |  |  |  |
| Sidelobe attenuation (dB) | 20 | dB | 20 | 20 | 20 | 20 |
| Separation distance WIA RTTT |  | M | 212 | 344 | 528 | 260 |

The compatibility consideration between interferer WIA (indoor) with 20 MHz bandwidth and the victim RTTT RSU is listed in Table 4.

Table 36: Interferer WIA III bandwidth 20 MHz, victim RTTT RSU (format table)

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: WIA III 20 MHz |  |  |  |  |  |  |
| Bandwidth | 20 | MHz |  |  |  |  |
| Tx out, eirp | 26 | dBm | 26 | 26 | 26 | 26 |
| effect of TPC | 0 | dB |  |  |  |  |
| Wall loss | 15 | dB | -15 | -15 | -15 | -15 |
| Antenna Gain | 0 | dBi |  |  |  |  |
| Bandwidth correction factor (ItBW > VrBW) | 16 | dB | -16 | -16 | -16 | -16 |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | -5 | -5 | -5 | -5 |
| Reception part: RSU RTTT |  |  |  |  |  |  |
| Receiver bandwidth | 0,5 | MHz | 0,5 | 0,5 | 0,5 | 0,5 |
| Receiver sensitivity | -104 | dBm | -104 | -104 | -104 | -104 |
| Antenna gain | 10 | dBi | 10 | 10 | 10 | 10 |
| C min per MHz at antenna input |  | dBm/MHz | -111 | -111 | -111 | -111 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -117 | -117 | -117 | -117 |
| MAIN LOBE WIA - MAIN LOBE RTTT |  |  |  |  |  |  |
| Separation distance WIA RTTT |  | M | 308 | 526 | 939 | 472 |
| MAIN LOBE WIA - SIDE LOBE RTTT |  |  |  |  |  |  |
| Sidelobe attenuation RTTT | 20 | dB | 20 | 20 | 20 | 20 |
| Separation distance WIA RTTT |  | M | 103 | 145 | 181 | 86 |

### COMPATIBILITY BETWEEN interferer RTTT and victim WIA (indoor)

The compatibility consideration between interferer RTTT RSU (outdoor) and the victim WIA (indoor) with 1 MHz bandwidth is listed in Table 5.

Table 37: Interferer RTTT RSU; victim WIA I bandwidth 1 MHz (format table)

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: RTTT RSU |  |  |  |  |  |  |
| Bandwidth | 1 | MHz |  |  |  |  |
| Tx out, eirp | 33 | dBm | 33 | 33 | 33 | 33 |
| effect of TPC (dB) | 0 | dB |  |  |  |  |
| Wall loss | 15 | dB | -15 | -15 | -15 | -15 |
| Antenna Gain | 0 | dBi |  |  |  |  |
| Bandwidth correction factor (ItBW > VrBW) | 0 | dB |  |  |  |  |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 18 | 18 | 18 | 18 |
| Reception part: WIA 1 |  |  |  |  |  |  |
| Receiver bandwidth | 1 | MHz | 1 | 1 | 1 | 1 |
| Receiver sensitivity | -82 | dBm | -82 | -82 | 82 | -82 |
| Antenna gain | 0 | dBi | 0 | 0 | 0 | 0 |
| C min per MHz at antenna input |  | dBm/MHz | -82 | -82 | -82 | -82 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -88 | -88 | -88 | -88 |
| MAIN LOBE RTTT - MAIN LOBE WIA I |  |  |  |  |  |  |
| Separation distance RTTT WIA |  | m | 224 | 366 | 575 | 284 |
| SIDE LOBE RTTT - MAIN LOBE WIA I |  |  |  |  |  |  |
| Sidelobe attenuation (dB) | 20 | dB | 20 | 20 | 20 | 20 |
| Separation distance RTTT WIA |  | m | 72 | 96 | 111 | 52 |

The compatibility consideration between interferer RTTT RSU (outdoor) and the victim WIA (indoor) with 20 MHz bandwidth is listed in Table 7.

Table 38: Interferer: RTTT RSU; Victim: WIA III bandwidth 20 MHz (format table)

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: WIA I 1 MHz |  |  |  |  |  |  |
| Bandwidth | 1 | MHz |  |  |  |  |
| Tx out, eirp | 33 | dBm | 33 | 33 | 33 | 33 |
| effect of TPC | 0 | dB |  |  |  |  |
| Wall loss | 15 | dB | -15 | -15 | -15 | -15 |
| Antenna Gain | 0 | dBi |  |  |  |  |
| Bandwidth correction factor (ItBW > VrBW) | 0 | dB |  |  |  |  |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 18 | 18 | 18 | 18 |
| Reception part: RSU RTTT |  |  |  |  |  |  |
| Receiver bandwidth | 20 | MHz | 20 | 20 | 20 | 20 |
| Receiver sensitivity | -88 | dBm | -88 | -88 | -88 | -88 |
| Antenna gain | 0 | dBi |  |  |  |  |
| C min per MHz at antenna input |  | dBm/MHz | -101 | -101 | -101 | -101 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -107 | -107 | -107 | -107 |
| MAIN LOBE WIA - MAIN LOBE RTTT |  |  |  |  |  |  |
| Separation distance WIA RTTT |  | m | 619 | 1159 | 2363 | 1435 |
| MAIN LOBE WIA - SIDE LOBE RTTT |  |  |  |  |  |  |
| Sidelobe attenuation RTTT | 20 | dB | 20 | 20 | 20 | 20 |
| Separation distance WIA RTTT |  | m | 212 | 345 | 530 | 261 |

### COMPATIBILITY BETWEEN interferer WIA (outdoor) and victim RTTT

The compatibility consideration between interferer WIA (outdoor) with 3 MHz and the victim RTTT RSU (outdoor) is listed in Table 8.

Table 39: Interferer WIA II bandwidth 3 MHz, victim RTTT RSU (format table)

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: WIA I 1 MHz |  |  |  |  |  |  |
| Bandwidth | 3 | MHz |  |  |  |  |
| Tx out, eirp | 26 | dBm | 26 | 26 | 26 | 26 |
| effect of TPC | 0 | dB |  |  |  |  |
| Wall loss | 0 | dB |  |  |  |  |
| Antenna Gain | 0 | dBi |  |  |  |  |
| Bandwidth correction factor (ItBW > VrBW) | -10 | dB | -10 | -10 | -10 | -10 |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 18 | 18 | 18 | 18 |
| Reception part: RSU RTTT |  |  |  |  |  |  |
| Receiver bandwidth | 0,5 | MHz | 0,5 | 0,5 | 0,5 | 0,5 |
| Receiver sensitivity | -104 | dBm | -104 | -104 | -104 | -104 |
| Antenna gain | 10 | dBi | 10 | 10 | 10 | 10 |
| C min per MHz at antenna input |  | dBm/MHz | -111 | -111 | -111 | -111 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -117 | -117 | -117 | -117 |
| MAIN LOBE WIA - MAIN LOBE RTTT |  |  |  |  |  |  |
| Separation distance WIA RTTT |  | m | 1069 | 2151 | 4813 | 3425 |
| MAIN LOBE WIA - SIDE LOBE RTTT |  |  |  |  |  |  |
| Sidelobe attenuation RTTT | 20 | dB | 20 | 20 | 20 | 20 |
| Separation distance WIA RTTT |  | m | 366 | 640 | 1192 | 622 |

The compatibility consideration between interferer WIA (outdoor) with 20 MHz and the victim RTTT RSU (outdoor) is listed in Table 9.

Table 40: Interferer WIA III bandwidth 20 MHz, victim RTTT RSU (format table)

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: WIA III 20 MHz |  |  |  |  |  |  |
| Bandwidth | 20 | MHz |  |  |  |  |
| Tx out, eirp | 26 | dBm | 26 | 26 | 26 | 26 |
| effect of TPC | 0 | dB |  |  |  |  |
| Wall loss | 0 | dB |  |  |  |  |
| Antenna Gain | 0 | dBi |  |  |  |  |
| Bandwidth correction factor (ItBW > VrBW) | -16 | dB | -16 | -16 | -16 | -16 |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 10 | 10 | 10 | 10 |
| Reception part: RSU RTTT |  |  |  |  |  |  |
| Receiver bandwidth | 0,5 | MHz | 0,5 | 0,5 | 0,5 | 0,5 |
| Receiver sensitivity | -104 | dBm | -104 | -104 | -104 | -104 |
| Antenna gain | 10 | dBi | 10 | 10 | 10 | 10 |
| C min per MHz at antenna input |  | dBm/MHz | -111 | -111 | -111 | -111 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -117 | -117 | -117 | -117 |
| MAIN LOBE WIA - MAIN LOBE RTTT |  |  |  |  |  |  |
| Separation distance WIA RTTT |  | m | 688 | 1305 | 2709 | 1696 |
| MAIN LOBE WIA - SIDE LOBE RTTT |  |  |  |  |  |  |
| Sidelobe attenuation RTTT | 20 | dB | 20 | 20 | 20 | 20 |
| Separation distance WIA RTTT |  | m | 236 | 389 | 622 | 308 |

### Compatibility between interferer RTTT and victim WIA (outdoor)

The compatibility consideration between interferer RTTT RSU (outdoor) and the victim WIA (outdoor) with 3 MHz is listed in Table 10.

Table 41: Interferer RTTT RSU; victim WIA II bandwidth 3 MHz (format table)

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: RTTT RSU |  |  |  |  |  |  |
| Bandwidth | 1 | MHz |  |  |  |  |
| Tx out, eirp | 33 | dBm | 33 | 33 | 33 | 33 |
| effect of TPC (dB) | 0 | dB |  |  |  |  |
| Wall loss | 0 | dB |  |  |  |  |
| Antenna Gain | 0 | dBi |  |  |  |  |
| WIA Bandwidth / RTTT Bandwidth ratio | 0 | dB |  |  |  |  |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 33 | 33 | 33 | 33 |
| Reception part: WIA 1 |  |  |  |  |  |  |
| Receiver bandwidth | 3 | MHz | 3 | 3 | 3 | 3 |
| Receiver sensitivity | -92 | dBm | -92 | -92 | -92 | -92 |
| Antenna gain | 0 | dBi |  |  |  |  |
| C min per MHz at antenna input |  | dBm/MHz | -97 | -97 | -97 | -97 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -103 | -103 | -103 | -103 |
| MAIN LOBE RTTT - MAIN LOBE WIA |  |  |  |  |  |  |
| Separation distance RTTT WIA |  | m | 1102 | 2225 | 5006 | 3594 |
| SIDE LOBE RTTT - MAIN LOBE WIA |  |  |  |  |  |  |
| Sidelobe attenuation (dB) | 20 | dB | 20 | 20 | 20 | 20 |
| Separation distance RTTT->ITS |  | m | 378 | 662 | 1240 | 653 |

The compatibility consideration between interferer RTTT RSU (outdoor) and the victim WIA (outdoor) with 20 MHz is listed in Table 11.

Table 42: Interferer RTTT RSU; victim WIA II bandwidth 20 MHz (format table)

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: RTTT RSU |  |  |  |  |  |  |
| Bandwidth | 1 | MHz |  |  |  |  |
| Tx out, eirp | 33 | dBm | 33 | 33 | 33 | 33 |
| effect of TPC (dB) | 0 | dB |  |  |  |  |
| Wall loss | 0 | dB |  |  |  |  |
| Antenna Gain | 0 | dBi |  |  |  |  |
| WIA Bandwidth / RTTT Bandwidth ratio | 0 | dB |  |  |  |  |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 33 | 33 | 33 | 33 |
| Reception part: WIA 1 |  |  |  |  |  |  |
| Receiver bandwidth | 20 | MHz | 20 | 20 | 20 | 20 |
| Receiver sensitivity | -88 | dBm | -88 | -88 | -88 | -88 |
| Antenna gain | 0 | dBi | 0 | 0 | 0 | 0 |
| C min per MHz at antenna input |  | dBm/MHz | -101 | -101 | -101 | -101 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -107 | -107 | -107 | -107 |
| MAIN LOBE RTTT - MAIN LOBE WIA |  |  |  |  |  |  |
| Separation distance RTTT WIA |  | m | 1383 | 2877 | 6729 | 5159 |
| SIDE LOBE RTTT - MAIN LOBE WIA |  |  |  |  |  |  |
| Sidelobe attenuation (dB) | 20 | dB | 20 | 20 | 20 | 20 |
| Separation distance RTTT WIA |  | m | 474 | 856 | 1667 | 937 |

### Conclusions

#### Conclusion for interferer WIA and victim RTTT

The antenna main beam of RTTT RSU is directed to the lane (show Figure 5). Therefor only the RTTT side loop is consider for coexistence between WIA and RTTT.

Figure 23: Sharing RSU on the road

The protection of RTTT receivers can be achieved with a maximum separation distances of 528 metres with respect to WIA systems operating indoor with 1 MHz bandwidth. The separation distance is much smaller than 528 meters for all other indoor WIA systems 357 m for WIA with 3 MHz bandwidth and 181 m for WIA with 20 MHz bandwidth. The detailed values are listed in Table 11.

Table 43:	Required separation distance interferer WIA (indoor) and victim RTTT RSU (format table)

|  | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- |
| WIA I --> RTTT main loop | m | 618 | 1157 | 2358 | 1432 |
| WIA III --> RTTT main loop | m | 308 | 526 | 939 | 472 |
| WIA I --> RTTT side loop | m | 212 | 344 | 528 | 260 |
| WIA III --> RTTT side loop | m | 103 | 145 | 181 | 86 |

For outdoor WIA applications is the required separation distance to RTTT receiver much high. The maximum required separation distance between RTTT and WIA system with 3 MHz bandwidth is 828 m. Even for the WIA systems with 20 MHz bandwidth, a separation distance of 622 m is required.

Table 44: Required separation distance interferer WIA (outdoor) and victim RTTT RSU (format table)

|  | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- |
| WIA II --> RTTT main loop | m | 828 | 1611 | 3450 | 2280 |
| WIA III --> RTTT main loop | m | 688 | 1305 | 2709 | 1696 |
| WIA II --> RTTT side loop | m | 284 | 479 | 828 | 414 |
| WIA III --> RTTT side loop | m | 236 | 389 | 622 | 308 |

#### Conclusion for interferer RTTT (RSU) and victim WIA

The RTTT system has also an influence on all WIA systems. The separation distance amounts 111 m for indoor WIA systems with 1 MHz bandwidth, 374 m for indoor WIA systems with 3 MHz bandwidth and 530 m for WIA systems with 20 MHz bandwidth. The detailed values depicted in Table 13.

Table 45: Required separation distance interferer RTTT RSU and victim WIA (indoor) (format table)

|  | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- |
| RTTT main loop --> WIA I | m | 224 | 366 | 575 | 284 |
| RTTT main loop --> WIA III | m | 619 | 1159 | 2363 | 1435 |
| RTTT side loop --> WIA I | m | 72 | 96 | 111 | 52 |
| RTTT side loop --> WIA III | m | 212 | 345 | 530 | 261 |

The influence from RTTT to outdoor WIA systems is much higher, because there is no positive wall attenuation. The required separation distance between interferer RTTT RSU side loop and outdoor WIA with 3 MHz bandwidth is between 378 m and 1240 m depending on propagation model. The required separation distance between interferer RTTT RSU and outdoor WIA with 20 MHz bandwidth is between 474 m and 1667 m depending on propagation model.

Table 46: Required separation distance interferer RTTT RSU and victim WIA (outdoor) (format table)

|  | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- |
| RTTT main loop --> WIA II | m | 1102 | 2225 | 5006 | 3594 |
| RTTT main loop --> WIA III | m | 1383 | 2877 | 6729 | 5159 |
| RTTT side loop --> WIA II | m | 378 | 662 | 1240 | 653 |
| RTTT side loop --> WIA III | m | 474 | 856 | 1667 | 937 |

The final results of required separation distance shown that RTTT and WIA systems interact at same dimension. The installation and long-life operation of industrial wireless communication networks require a coexistence management according to “IEC/TS 62657-2 Industrial communication networks – Wireless communication network - Part 2: Coexistence management” [ref]. This means that the frequency band of WIA systems must check for potential interferer. If a interferer (e.g. RTTT) is detected, the occupied/interferer frequency band can’t use for WIA real-time applications.

[note: if the reference to the IEC is kept, a reference to the appropriate section should be inserted alternatively,  the relevant process considered in the IEC document could be described in the current document.]

## BFWA

The technical parameters of the WIA systems used in this study are summarised in Table 2.

Table 47: BFWA system parameters (format table)

| Parameter | WIA parameter |
| --- | --- |
| Channel bandwidth | 1MHz, 3 MHz, 20 MHz |
| Duplex/ Access scheme |  |
| Max Tx power dBm (conducted) | 21 dBm |
| Antenna | Dipol 5dBi See Figure 6 |
| TX mask | See Annex 2 |
| Antenna height Tx and Rx | 0.5 to 5 m |
| Link lenght | 5 to 100m |
| Path loss model used in SEAMCAT | IEEE plugin (Model_C_IEEE_802_11_rev2) see Annex 1 |

Figure 24: dipol antenna (5dBi), vertical antenna pattern WIA (horizontal pattern omni)

### MCL calculations

Table 3 shows required separation distances based on the parameters provided in the previous sections.

Table 48: MCL calculations (format table)

The obvious consequence is that under worst case conditions (Free Space loss) the BFWA link can be affected within a radius around the WIA between 3 and 10 km. This situation is much less critical in urban environments. This scenario will be further analysed in the following sections.

### Sensing procedure

In this section the requirements forWIA systems (Interfering transmitter IT, transmitting to its wanted receiver WR) to detect BFWA systems (Wanted transmitter WT transmitting to the victim receiver VR) is analysed. IT is able to monitor the WT, which is the basis for the sensing mechanism which is called LBT in this section.

The following abbreviations and definitions are valid in this annex:

- Dimensions: r/m, P/dBm, S/dBm, SIR/dB, f/GHz, All antennas 0dBi

- VR Victim receiver (BFWA)

  - N: Noise floor kTBF of VR (-95 dBm/20MHz)

  - F: Noise figure of VR, (BFWA 6 dB)

  - S: Signal strength received at the VR from WT (Pwt)

  - SNR: signal to noise ratio, or C/N at VR

  - SIRmin: Signal to interference ratio, or C/I at VR (27 dB)

  - WT Wanted transmitter (victim link, BFWA)

  - Pwt Transmit power of WT (CS) (BFWA 14 dBm)

  - Gs Antenna gain WT (22 dBi)

- IT Interfering Transmitter (WIA)

  - Pit Transmit power of IT (SRD 21 dBm)

  - Antenna gain IT 5dBi

  - WR Wanted receiver (Interfering Link)

  - Plbt: LBT power received at WR from WT (Pwt)

  - Pthr: power threshold for the LBT mechanism at IT

- I: Interfering power at VR,

- n: Path loss exponent n (e.g. n=2 free space loss)

- Rint: radius around VR; inside interference can occur (S-I<SIRmin)

- Rsig: radius around VR; inside the victim link works wit S-N<SNRmin

- Rdet: radius around WT; inside the IT can detect the WT

- Wall: wall attenuation dB

The following figure explains the investigated scenario. Within a radius of Rint around the VR the IT can exceed the protection objective of the VR (e.g. C/I). Within a radius of Rdet around the WT the IT can detect the WT (Threshold is exceeded).

In the light blue area in the following figure 7 LBT is working effectively. The red area is the so called “hidden node”, where the IT is not able to detect the WT.

Figure 25: Title?

The situation is simplified because each BFWA station is assumed to transmit and receive simultaneously (TDD).

Figure 28: Illustration of the scenario

The formulas given hereafter are the basis for the analysis.

Signal strength at the BFWA receiver:

S (at VR)= N + SNR    = 	Pwt (BFWA)+ Gsbfwa+Gebfwa- PL(Rsig)		(1)

The interference power at the BFWA receiver:

I (at VR)= 	S - SIRmin = 	Pit (SRD)+Gswia(p)+Gebfwa(p)-Wall- PL(Rint) 		(2)

The threshold power at the SRD:

Pthr (at SRD)	= 		Pwt (BFWA)+Gsbfwa(p)+Gewia(p)-Wall- PL(Rdet)	(3)

Path loss model:

PL	=	32.5+10*n*log(R/m)+20*log(f/GHz)					(4)

(1)+(4) -> 10n*log(Rsig) = Pwt+ Gsbfwa+Gebfwa -N-SNR-32.5-20logf			(5)

(2)+(4) -> 10n*log(Rint) = Pit+ Gswia(p)+Gebfwa(p)-Wall-N-SNR+SIRmin-32.5-20logf (6)

(3)+(4) -> 10n*log(Rdet) = Pwt+ Gsbfwa(p)+Gewia(p)-Wall-Pthr-32.5-20logf	(7)

Relation Rdet/Rint: (7)-(6) -> 10n*log(Rdet/Rint)=Pwt-Pit-Pthr+N+SNR-SIRmin	(8)

Under the assumption Rdet=Rint the WI system should be perfectly able to detect the BFWA system and thus with formula (8) it follows:

. 				Pthr=Pwt-Pit+N+SNR-SIRmin				(9)

Table 49 shows the required LBT threshold values using formula (9), with variable WIA bandwidth (1MHz, 3 MHz, 20 MHz) and 3 different SNR values (27dB, 37 dB, 47 dB).

Table 49: LBT threshold [table format]

In the above calculation it is assumed that all links having the same path loss model. For the case of a specific sensing antenna on top of the WIA plant, with better propagation conditions for the detection path (propagation exponent n2) the situation is more complicated and can not be reduced to a simple formula, but the situation can be improved

10n1*log(Rint) = Pit+ Gswia(p)+Gebfwa(p)-Wall-N-SNR+SIRmin-32.5-20logf		(10)

10n2*log(Rdet) = Pwt+ Gsbfwa(p)+Gesensing- Pthr-32.5-20logf				(11)

Table 50 shows the interference and detection ranges using the above formulas (10) and (11).

Table 50: Interference and detection ranges [table format]

Table 5: interference and detection range comparison

The calculations in Table 50 shows that a sensing antenna improves the situation because the detection distance is always higher as the interference distance; only for unrealistic combination of a outdoor WIA usage (0dB wall loss) and the same propagation conditions in the sensing and interfering path the advantages of a sensing antenna are disappearing. This issue will be analysed in more detail in the next section.

### SEAMCAT simulations

In this section a probabilistic analysis is provided using the SEAMCAT tool (www.seamcat.org).

The IEEE plugin was used in SEAMCAT in order to be able to consider also an urban-like propagation model (Model_C_IEEE_802_11_rev2, Date  17/10/2012), see also Annex 1 and http://tractool.seamcat.org/wiki/Manual/PropagationModels/IEEE_802.11_Model_C_modified).

The following configuration was used in the simulations:

- Breakpoint distance 5m for urban environments

- Breakpoint distance 50m for suburban environments

- Breakpoint distance 500m for rural environments

It is assumed that mainly TDMA access techniques are used in this environment, that means the spectrum usage appears like one WIA device (1MHz, 3MHz and 20 MHz) being active at the same time on each WIA channel, but for 100% of time.

The BFWA Victim (20 MHz BW) sees either 1x 20 MHz WIA systems, or 6 x 3 MHz WIA or 20 x 1MHz WIA systems

The below figures are explaining the SEAMCAT scenario.

Figure 27: WIA spectrum utilisation

Figure 28: The SEAMCAT scenario

Interfering Link

Interfering Link

Wanted Link

Wanted Link

Figure 29: Two Snapshots from the SEAMCAT simulations

Figure 30: Scenario outline 20 MHz

#### WIA 20 MHz

Table 6 summarises the SEAMCAT simulations without any consideration of mitigation techniques. The only difference between scenario 1, 2 and 3 are the used propagation conditions in the interfering path (Interfering Link Transmitter to Victim Link Receiver):

- Scenario 1 and 3: Urban propagation conditions (see Annex 1)

- Scenario 2: Suburban/rural propagation conditions (see Annex 1)

Table 52: Seamcat results 20 MHz WI systems without mitigations [table format]

|  | Scenario 1 | Scenario 2 | Scenario 3 |
| --- | --- | --- | --- |
| Victim link |  |  |  |
| Frequency | 5750 MHz |  |  |
| Victim link receiver | BFWA CS |  |  |
| Reception bandwidth | 20.000 kHz |  |  |
| Propagation model | Spherical diffraction outdoor- outdoor (see Annex 1) |  |  |
| C/(I+N) | 24 dB |  |  |
| Coverage radius | 1000 m |  |  |
| Interfering link |  |  |  |
| Frequency | 5750 MHz |  |  |
| Type | WIA 20MHz |  |  |
| Tx power | 21 dBm |  |  |
| antenna | 5 dBi |  |  |
| ILT-> ILR path | IEEE 802.11 plugin Breakpoint distance 5m (see Annex 1) |  |  |
| ILT->VLR path | IEEE 802.11 plugin Breakpoint distance 5m (see Annex 1) | IEEE 802.11 plugin Breakpoint distance 50m (see Annex 1) | IEEE 802.11 plugin Breakpoint distance 10m (see Annex 1) |
| ILT->VLR | None, Interfering link distributed around the VLR - 1 active device - Distance between VLR and ILT from 100m to 1000m |  |  |
| dRSS | -56 dBm | -56 dBm | -56 dBm |
| iRSS | -120 dBm +/- 15dB | -91 dBm +/- 15dB | -110 dBm +/- 15dB |
| Prob that dRSS/iRSS<C/(I+N) | 0% | 25.8% | 1.2% |

The results from Table 6 are confirming the MCL calculations in section 1.2, that in urban environments (breakpoint distance 5m/10m in the path loss model) no problem is expected but in rural and suburban environments (break point distance 50m) harmful interference can occur.

Based on the propagation path loss from the worst case scenario 2 the effect of the mitigation techniques APC (Table 7) and LBT (Table 8) are analysed below.

Table 53: Seamcat results 20 MHz WI systems, APC consideration [table format]

|  | Scenario 4 | Scenario 5 | Scenario 6 |
| --- | --- | --- | --- |
| Victim link |  |  |  |
| Frequency | 5750 MHz |  |  |
| Victim link receiver | BFWA CS |  |  |
| Reception bandwidth | 20.000 kHz |  |  |
| Propagation model | Spherical diffraction outdoor- outdoor (see Annex 1) |  |  |
| C/(I+N) | 24 dB |  |  |
| Coverage radius | 1000 m |  |  |
| Interfering link |  |  |  |
| Frequency | 5750 MHz |  |  |
| Type | WIA 20MHz |  |  |
| Tx power | 21 dBm |  |  |
| antenna | 5 dBi |  |  |
| ILT-> ILR path | IEEE 802.11 plugin Breakpoint distance 5m (see Annex 1) |  |  |
| APC | 20 dB, threshold -87dBm | 10 dB, threshold -87dBm | 20dB, threshold -77dBm |
| ILT->VLR path | IEEE 802.11 plugin Breakpoint distance 50m (see Annex 1) |  |  |
| ILT->VLR | None, Interfering link distributed around the VLR - 1 active device - Distance between VLR and ILT from 100m to 1000m |  |  |
| dRSS | -56 dBm |  |  |
| iRSS | -108 dBm +/- 15dB | -101 dBm +/- 15dB | -102 dBm +/- 17dB |
| Prob that dRSS/iRSS<C/(I+N) | 2.5% | 8.3% | 10% |

Table 7 shows that APC is efficient to improve the situation and with an APC threshold of -87 dBm plus a dynamic range of 20dB the impact can be essentially improved (25.8% without and 2.5% with APC).

Table 54: Seamcat results 20 MHz WI systems, LBT consideration [table format]

|  | Scenario 7 | Scenario 8 | Scenario 9 |
| --- | --- | --- | --- |
| Victim link |  |  |  |
| Frequency | 5750 MHz |  |  |
| Victim link receiver | BFWA CS |  |  |
| Reception bandwidth | 20.000 kHz |  |  |
| Propagation model | Spherical diffraction outdoor- outdoor |  |  |
| C/(I+N) | 24 Db |  |  |
| Coverage radius | 1000 m |  |  |
| Interfering link |  |  |  |
| Frequency | 5750 MHz |  |  |
| Type | WIA 20MHz |  |  |
| Tx power | 21 dBm |  |  |
| antenna | 5 dBi |  |  |
| ILT-> ILR path | IEEE 802.11 plugin Breakpoint distance 5m (see Annex 1) |  |  |
| LBT | - path (VLT->ILT): IEEE 802.11 plugin, Breakpoint distance 50m path (see Annex 1) - threshold -97 dBm/20MHz | - path (VLT->ILT): IEEE 802.11 plugin, Breakpoint distance 500m path (see Annex 1) - threshold -97 dBm/20MHz | - path (VLT->ILT): IEEE 802.11 plugin, Breakpoint distance 500m path (see Annex 1) - threshold -90 dBm/20MHz |
| ILT->VLR path | IEEE 802.11 plugin Breakpoint distance 50m (see Annex 1) |  |  |
| ILT->VLR | None, Interfering link distributed around the VLR - 1 active device - Distance between VLR and ILT from 100m to 1000m |  |  |
| sRSS | -101 dBm +/- 10dB | -86 dBm +/- 9dB | -86 dBm +/- 9dB |
| dRSS | -56 dBm | -56 dBm | -56 dBm |
| iRSS | -94 dBm +/- 14dB | -99 dBm +/- 9dB | -97 dBm +/- 12dB |
| Prob that dRSS/iRSS<C/(I+N) | 11% | 0.2% | 2.5% |
|  |  | The changed propagation condition between the BFWA CS and the WIA can only be considered by a separate sensing antenna on top of the building | The changed propagation condition between the BFWA CS and the WIA can only be considered by a separate sensing antenna on top of the building |

The LBT simulations in Table 8 are confirming the results in the manual analysis in section 1.3 that a simple LBT mechanism within the WIA devices is not very efficient and improves the situation only marginal (26% without LBT, 11% with LBT and a very low threshold value of -97dBm). More efficient would be a sensing antenna to be installed on top of the industrial plant, with the advantage of better propagation conditions to the victim link and the possibility to choose a higher gain antenna for the sensing purpose.

The simulations in this section are only provided for 20 MHz bandwidth for the WIA devices, which is a best case scenario. The same simulations can be conducted as appropriate for WIA with 1MHz and 3 MHz BW, but the results are expected to be more critical (e.g. in worst case 1MHz systems could appear like 20x 20 MHz systems, resulting in a 20 times higher risk of interference). One simple solution without any further study for the other WIA bandwidths would be to limit the power spectral density to 13dBm/1MHz e.i.r.p..

:

## Fixed Satellite (E-s) Service (FSS)

To be done:

## Amateur Service and Amateur-satellite (s-E) Service

### Compatibility between WIA and the Amateur Service

The determination of the maximum tolerable interference level from emissions of a single WIA device at the Amateur Service (AS) receiver is based on ECC Report 172 [11], where it is said that this level should be lower than N + (I/N) where N is the AS receiver inherent noise level and I/N the interference to noise ratio. The interference to noise ratio can be taken as –10 dB as given in ECC Report 172 [11].

#### Step 1: MCL calculations

The same approach as in section 5.1.1 is considered.

Table 55: Separation distances WIA I – Radio Amateur Service

| Parameter |  | CW Voice | SSB Voice | FM Voice | Digital Voice and Multimedia |
| --- | --- | --- | --- | --- | --- |
| Radio Amateur | Tx power into antenna peak (dBm) | 43.0 | 43.0 | 43.0 | 43.0 |
|  | Receiver IF3dB bandwidth (kHz) | 0.4 | 2.7 | 9 | 2.73 |
|  | Antenna main beam gain | 33 | 33 | 33 | 33 |
|  | Feeder loss (dB) | 1 | 1 | 1 | 2 |
|  | E.i.r.p Amateur Service (dBm) | 76.0 | 76.0 | 76.0 | 76.0 |
|  | Receiver noise figure (dB) | 1 | 1 | 1 | 2 |
|  | Noise level NBW (dBm) | -148.0 | -139.7 | -134.4 | -139.6 |
|  | Noise level N (dBm) | -147.0 | -138.7 | -133.4 | -137.6 |
|  | Noise level protection criteria (dBm) | -153.0 | -144.7 | -139.4 | -143.6 |
| WIA I | WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 |
|  | WIA feeder loss | 0 | 0 | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 | 0 | 0 |
|  | Bandwidth (MHz) | 1 | 1 | 1 | 1 |
| Calculations | Bandwidth correction factor (dB) | -34.0 | -25.7 | -20.5 | -25.6 |
|  | Minimum coupling loss (dB) | 145.0 | 145.0 | 145.0 | 144.0 |
|  | Wall Loss (dB) | 15.0 | 15.0 | 15.0 | 15.0 |
|  | Required propagation loss (dB) | 162.0 | 162.0 | 162.0 | 160.0 |
|  | Frequency (MHz) | 5800 | 5800 | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 | 0.05 | 0.05 |
|  | Free space distance (km) | 516 | 516 | 516 | 410 |

Table 56: Separation distances WIA II – Radio Amateur Service

| Parameter |  | CW Voice | SSB Voice | FM Voice | Digital Voice and Multimedia |
| --- | --- | --- | --- | --- | --- |
| Radio Amateur | Tx power into antenna peak (dBm) | 43.0 | 43.0 | 43.0 | 43.0 |
|  | Receiver IF3dB bandwidth (kHz) | 0.4 | 2.7 | 9 | 2.73 |
|  | Antenna main beam gain | 33 | 33 | 33 | 33 |
|  | Feeder loss (dB) | 1 | 1 | 1 | 2 |
|  | E.i.r.p Amateur Service (dBm) | 76.0 | 76.0 | 76.0 | 76.0 |
|  | Receiver noise figure (dB) | 1 | 1 | 1 | 2 |
|  | Noise level NBW (dBm) | -148.0 | -139.7 | -134.4 | -139.6 |
|  | Noise level N (dBm) | -147.0 | -138.7 | -133.4 | -137.6 |
|  | Noise level protection criteria (dBm) | -153.0 | -144.7 | -139.4 | -143.6 |
| WIA II | WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 |
|  | WIA feeder loss | 0 | 0 | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 | 0 | 0 |
|  | Bandwidth (MHz) | 20 | 20 | 20 | 20 |
| Calculations | Bandwidth correction factor (dB) | -47.0 | -38.7 | -33.5 | -38.6 |
|  | Minimum coupling loss (dB) | 132.0 | 132.0 | 132.0 | 131.0 |
|  | Wall Loss (dB) | 15.0 | 15.0 | 15.0 | 15.0 |
|  | Required propagation loss (dB) | 149.0 | 149.0 | 149.0 | 147.0 |
|  | Frequency (MHz) | 5800 | 5800 | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 | 0.05 | 0.05 |
|  | Free space distance (km) | 115 | 115 | 115 | 92 |

Table 57: Separation distances WIA III – Radio Amateur Service

| Parameter |  | CW Voice | SSB Voice | FM Voice | Digital Voice and Multimedia |
| --- | --- | --- | --- | --- | --- |
| Radio Amateur | Tx power into antenna peak (dBm) | 43.0 | 43.0 | 43.0 | 43.0 |
|  | Receiver IF3dB bandwidth (kHz) | 0.4 | 2.7 | 9 | 2.73 |
|  | Antenna main beam gain | 33 | 33 | 33 | 33 |
|  | Feeder loss (dB) | 1 | 1 | 1 | 2 |
|  | E.i.r.p Amateur Service (dBm) | 76.0 | 76.0 | 76.0 | 76.0 |
|  | Receiver noise figure (dB) | 1 | 1 | 1 | 2 |
|  | Noise level NBW (dBm) | -148.0 | -139.7 | -134.4 | -139.6 |
|  | Noise level N (dBm) | -147.0 | -138.7 | -133.4 | -137.6 |
|  | Noise level protection criteria (dBm) | -153.0 | -144.7 | -139.4 | -143.6 |
| WIA III | WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 |
|  | WIA feeder loss | 0 | 0 | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 | 0 | 0 |
|  | Bandwidth (MHz) | 3 | 3 | 3 | 3 |
| Calculations | Bandwidth correction factor (dB) | -38.8 | -30.5 | -25.2 | -30.4 |
|  | Minimum coupling loss (dB) | 140.2 | 140.2 | 140.2 | 139.2 |
|  | Wall Loss (dB) | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Required propagation loss (dB) | 172.2 | 172.2 | 172.2 | 170.2 |
|  | Frequency (MHz) | 5800 | 5800 | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 | 0.05 | 0.05 |
|  | Free space distance (km) | 1677 | 1677 | 1677 | 1332 |

Table 58: Separation distances WIA IV – Radio Amateur Service

| Parameter |  | CW Voice | SSB Voice | FM Voice | Digital Voice and Multimedia |
| --- | --- | --- | --- | --- | --- |
| Radio Amateur | Tx power into antenna peak (dBm) | 43.0 | 43.0 | 43.0 | 43.0 |
|  | Receiver IF3dB bandwidth (kHz) | 0.4 | 2.7 | 9 | 2.73 |
|  | Antenna main beam gain | 33 | 33 | 33 | 33 |
|  | Feeder loss (dB) | 1 | 1 | 1 | 2 |
|  | E.i.r.p Amateur Service (dBm) | 76.0 | 76.0 | 76.0 | 76.0 |
|  | Receiver noise figure (dB) | 1 | 1 | 1 | 2 |
|  | Noise level NBW (dBm) | -148.0 | -139.7 | -134.4 | -139.6 |
|  | Noise level N (dBm) | -147.0 | -138.7 | -133.4 | -137.6 |
|  | Noise level protection criteria (dBm) | -153.0 | -144.7 | -139.4 | -143.6 |
| WIA IV | WIA e.i.r.p (dBm) outdoor | 26 | 26 | 26 | 26 |
|  | WIA feeder loss | 0 | 0 | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 | 0 | 0 |
|  | Bandwidth (MHz) | 20 | 20 | 20 | 20 |
| Calculations | Bandwidth correction factor (dB) | -47.0 | -38.7 | -33.5 | -38.6 |
|  | Minimum coupling loss (dB) | 132.0 | 132.0 | 132.0 | 131.0 |
|  | Wall Loss (dB) | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Required propagation loss (dB) | 164.0 | 164.0 | 164.0 | 162.0 |
|  | Frequency (MHz) | 5800 | 5800 | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 | 0.05 | 0.05 |
|  | Free space distance (km) | 649 | 649 | 649 | 516 |

[During the web meeting 04/10/2012, it was proposed to consider C/I as interference criterion for the FM and digital systems.

Murray and Marko to go on exchanging on the protection of the systems given in the two last columns]

#### Step 2: SEAMCAT simulations

[During the web meeting 04/10/2012, it was proposed to consider C/I as interference criterion for the FM and digital systems.

To simplify a co-frequency situation, assume an average antenna height of 25 metres for a terrestrial amateur station, as used in ECC Report 172 [11].

During the web meeting 04/10/2012, it was proposed to develop SEAMCAT simulations. In order to develop such simulations additional parameters are needed:

- Radio Amateur antenna patterns (ITU-R Recommendations F.699 or F1245).

- WIA characteristics:

  - Distribution of power

  - Deployment

  - Duty Cycle

#### Conclusions

In co-channel case where the antenna main lobes are pointing at each other, the required MCL between WIA and stations in the Amateur Service can be significant.

It should be noted that the most sensitive amateur service operations are globally harmonised at 5760-5762 MHz (CW, SSB etc), where long distance reception occurs close to the noise floor. A reduction of the power density of WIA would reduce the risk of interference to Radio Amateur Service. Mitigations techniques in this frequency range include:

- [Avoid  the most sensitive channels

- A reduction of the power density (using a larger bandwidth)]

[Note: during the web meeting 04/10/2012, it was indicated that a I/N of -6dB is also considered for some of the links, this may need to be reflected in the conclusions.]

### Compatibility between WIA and the Amateur-Satellite Service

#### Step 1: MCL calculations

The same approach as for the Radio Amateur is considered. The following tables provide the results of the calculated separation distances.

Table 59: Separation distances WIA I – Radio Amateur-Satellite Service (space-to-Earth)

| Parameter |  | CW Voice | SSB Voice |
| --- | --- | --- | --- |
| Radio Amateur | Tx power into antenna peak (dBm) | 43.0 | 43.0 |
|  | Receiver IF3dB bandwidth (kHz) | 0.4 | 2.7 |
|  | Transmitter antenna main beam gain | 30.0 | 30.0 |
|  | Receiver antenna main beam gain | 0.4 | 2.7 |
|  | Feeder loss (dB) | 3 | 3 |
|  | E.i.r.p Amateur Service (dBm) | 33 | 33 |
|  | Receiver noise figure (dB) | 0.5 | 0.5 |
|  | Noise level NBW (dBm) | 32.5 | 32.5 |
|  | Noise level N (dBm) | 1 | 2 |
|  | Noise level protection criteria (dBm) | -148.0 | -139.7 |
| WIA I | WIA e.i.r.p (dBm) outdoor | 26 | 26 |
|  | WIA feeder loss | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 |
|  | Bandwidth (MHz) | 1 | 1 |
| Calculations | Bandwidth correction factor (dB) | -34.0 | -25.7 |
|  | Minimum coupling loss (dB) | 145.0 | 144.0 |
|  | Wall Loss (dB) | 15.0 | 15.0 |
|  | Required propagation loss (dB) | 162.5 | 161.5 |
|  | Frequency (MHz) | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 |
|  | Free space distance (km) | 547 | 488 |

Table 60: Separation distances WIA II – Radio Amateur-Satellite Service (space-to-Earth)

| Parameter |  | CW Voice | SSB Voice |
| --- | --- | --- | --- |
| Radio Amateur | Tx power into antenna peak (dBm) | 43.0 | 43.0 |
|  | Receiver IF3dB bandwidth (kHz) | 0.4 | 2.7 |
|  | Transmitter antenna main beam gain | 30.0 | 30.0 |
|  | Receiver antenna main beam gain | 0.4 | 2.7 |
|  | Feeder loss (dB) | 3 | 3 |
|  | E.i.r.p Amateur Service (dBm) | 33 | 33 |
|  | Receiver noise figure (dB) | 0.5 | 0.5 |
|  | Noise level NBW (dBm) | 32.5 | 32.5 |
|  | Noise level N (dBm) | 1 | 2 |
|  | Noise level protection criteria (dBm) | -148.0 | -139.7 |
| WIA II | WIA e.i.r.p (dBm) outdoor | 26 | 26 |
|  | WIA feeder loss | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 |
|  | Bandwidth (MHz) | 20 | 20 |
| Calculations | Bandwidth correction factor (dB) | -47.0 | -38.7 |
|  | Minimum coupling loss (dB) | 132.0 | 131.0 |
|  | Wall Loss (dB) | 15.0 | 15.0 |
|  | Required propagation loss (dB) | 149.5 | 148.5 |
|  | Frequency (MHz) | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 |
|  | Free space distance (km) | 122 | 109 |

Table 61: Separation distances WIA III – Radio Amateur-Satellite Service (space-to-Earth)

| Parameter |  | CW Voice | SSB Voice |
| --- | --- | --- | --- |
| Radio Amateur | Tx power into antenna peak (dBm) | 43.0 | 43.0 |
|  | Receiver IF3dB bandwidth (kHz) | 0.4 | 2.7 |
|  | Transmitter antenna main beam gain | 30.0 | 30.0 |
|  | Receiver antenna main beam gain | 0.4 | 2.7 |
|  | Feeder loss (dB) | 3 | 3 |
|  | E.i.r.p Amateur Service (dBm) | 33 | 33 |
|  | Receiver noise figure (dB) | 0.5 | 0.5 |
|  | Noise level NBW (dBm) | 32.5 | 32.5 |
|  | Noise level N (dBm) | 1 | 2 |
|  | Noise level protection criteria (dBm) | -148.0 | -139.7 |
| WIA III | WIA e.i.r.p (dBm) outdoor | 26 | 26 |
|  | WIA feeder loss | 0 | 0 |
|  | Antenna Gain (dBi) | 0 | 0 |
|  | Bandwidth (MHz) | 20 | 20 |
| Calculations | Bandwidth correction factor (dB) | -47.0 | -38.7 |
|  | Minimum coupling loss (dB) | 132.0 | 131.0 |
|  | Wall Loss (dB) | 0.0 | 0.0 |
|  | Required propagation loss (dB) | 164.5 | 163.5 |
|  | Frequency (MHz) | 5800 | 5800 |
|  | Wavelength (m) | 0.05 | 0.05 |
|  | Free space distance (km) | 688 | 613 |

[During the web meeting 04/10/2012, it was proposed to consider C/I as interference criterion for the FM and digital systems.]

#### Step 2: SEAMCAT simulations

During the web meeting 04/10/2012, it was proposed to develop SEAMCAT simulations. In order to develop such simulations additional parameters are needed:

- Radio Amateur antenna patterns (ITU-R Recommendations F.699 or F.1245).

- WIA characteristics:

  - Distribution of power

  - Deployment

  - Duty Cycle

#### Conclusions

In co-channel case where the antenna main lobes are pointing at each other, the required MCL between WIA and stations in the Amateur-Satellite Service can be significant.

It should be noted that the most sensitive Amateur-Satellite Service operations are globally harmonised at 5840-5842 MHz (CW, SSB etc), where long distance reception occurs close to the noise floor. A reduction of the power density of WIA would reduce the risk of interference to Radio Amateur Service. Mitigations techniques in this frequency range include:

- [Avoid  the most sensitive channels

- A reduction of the power density (using a larger bandwidth)]

[Note: during the web meeting 04/10/2012, it was indicated that a I/N of -6dB is also considered for some of the links, this may need to be reflected in the conclusions.]

## Compatibility between WIA and General (non-specific) short range devices

### Assumptions

The analysis was based on the following assumptions and parameters:

The SRD antenna is omni-directional;

It is considered that the majority of SRD are likely to be operated indoors and that the number of SRD currently using this band is very low.

The SRD is typically used indoors, thus a 15 dB wall loss was assumed;

There is clear separation between indoor WIA and indoor SRD; thus a 30 dB wall loss was assumed

### Impact of WIA devices on SRD

#### Step 1: MCL calculation

The same propagation model as in section 5.2 is considered for the MCL calculations. Characteristics of SRD are given in section 4.

Table 62: WIA characteristics

| Characteristic | Manufacturing cell | Factory hall | Plant level | Plant level |
| --- | --- | --- | --- | --- |
|  | WIA I | WIA II | WIA III | WIA IV |
| Indoor/Outdoor | Indoor | mostly indoor | Outdoor | Outdoor |
| Transmit power (dBm) | 26 | 26 | 26 | 26 |
| Bandwidth (MHz) | 1 | 20 | 3 | 20 |
| Transmit power (dBm/MHz) | 26 | 13 | 21 | 13 |

The separation distance depends from the required attenuation between the interferer WIA and the victim SRD.

[Note: in the following calculations, the bandwidth correction factor should be changed.]

Table 63: Interference assessment for SRD type I with maximum antenna gain

| SRD Type I (Antenna Gain max. 20 dBi) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristic | Unit | Propagation model |  |  |  |
|  |  | Urban | Suburban | Rural | ETSI |
| Protection criterion | I/N (dB) | 0,0 | 0 | 0 | 0 |
| Allowable Interfering power level | dBm/MHz | -105,0 | -105,0 | -105,0 | -105,0 |
| Interference power level (at the antenna port) | dBm/MHz | -125,0 | -125,0 | -125,0 | -125,0 |
| Wall loss (building penetration) | LWall (dB) | 15 | 15 | 15 | 15 |
| Required Attenuation (WIA I) | LFS (dB) | 121 | 121 | 121 | 121 |
| Required Attenuation (WIA II) | LFS (dB) | 108 | 108 | 108 | 108 |
| Separation distance WIA I | d (m) | 500 | 909 | 1786 | 1020 |
| Separation distance WIA II | d (m) | 249 | 414 | 721 | 336 |

Table 64: Interference assessment for SRD type I with minimum antenna gain

| SRD Type I (Antenna Gain max. 2 dBi) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristic | Unit | Propagation model |  |  |  |
|  |  | Urban | Suburban | Rural | ETSI |
| Protection criterion (C/I) | I/N (dB) | 0,0 | 0 | 0 | 0 |
| Allowable Interfering power level I | dBm/MHz | -105,0 | -105,0 | -105,0 | -105,0 |
| Interference power level (at the antenna port) | dBm/MHz | -107,0 | -107,0 | -107,0 | -107,0 |
| Wall loss (building penetration) | LWall (dB) | 15 | 15 | 15 | 15 |
| Required Attenuation (WIA I) | LFS (dB) | 103 | 103 | 103 | 103 |
| Required Attenuation (WIA II) | LFS (dB) | 93 | 93 | 93 | 93 |
| Separation distance WIA I | d (m) | 191 | 305 | 509 | 220 |
| Separation distance WIA II | d (m) | 112 | 167 | 253 | 94 |

Table 65: Interference assessment for SRD type II with maximum antenna gain

| SRD Type II (Antenna Gain max. 24 dBi) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristic | Unit | Propagation model |  |  |  |
|  |  | Urban | Suburban | Rural | ETSI |
| Protection criterion (C/I) | C/I (dB) | 8,0 | 8,0 | 8,0 | 8,0 |
| Allowable Interfering power level I | dBm/MHz | -112,0 | -112,0 | -112,0 | -112,0 |
| Interference power level (at the antenna port) | dBm/MHz | -136,0 | -136,0 | -136,0 | -136,0 |
| Wall loss (building penetration) | LWall (dB) | 15 | 15 | 15 | 15 |
| Required Attenuation (WIA I) | LFS (dB) | 132 | 132 | 132 | 132 |
| Required Attenuation (WIA II) | LFS (dB) | 122 | 122 | 122 | 122 |
| Separation distance WIA I | d (m) | 901 | 1772 | 3851 | 2608 |
| Separation distance WIA II | d (m) | 527 | 967 | 1917 | 1111 |

Table 66: Interference assessment for SRD type II with minimum antenna gain

| SRD Type II (Antenna Gain max. 2 dBi) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristic | Unit | Propagation model |  |  |  |
|  |  | Urban | Suburban | Rural | ETSI |
| Protection criterion (C/I) | C/I (dB) | 8,0 | 8,0 | 8,0 | 8,0 |
| Allowable Interfering power level I | dBm/MHz | -112,0 | -112,0 | -112,0 | -112,0 |
| Interference power level | dBm/MHz | -114,0 | -114,0 | -114,0 | -114,0 |
| Wall loss (building penetration) | LWall (dB) | 15 | 15 | 15 | 15 |
| Required Attenuation (WIA I) | LFS (dB) | 110 | 110 | 110 | 110 |
| Required Attenuation (WIA II) | LFS (dB) | 100 | 100 | 100 | 100 |
| Separation distance WIA I | d (m) | 277 | 467 | 830 | 399 |
| Separation distance WIA II | d (m) | 162 | 255 | 413 | 170 |

Table 67: Interference assessment for SRD type III

| SRD Type III |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Characteristic | Unit | Propagation model |  |  |  |
|  |  | Urban | Suburban | Rural | ETSI |
| Protection criterion (C/I) | C/I (dB) | 20 | 20 | 20 | 20 |
| Allowable Interfering power level I | dBm/MHz | -113,0 | -113,0 | -113,0 | -113,0 |
| Interference power level | dBm/MHz | -115,0 | -115,0 | -115,0 | -115,0 |
| Wall loss (building penetration) | LWall (dB) | 15,0 | 15,0 | 15,0 | 15,0 |
| Required Attenuation (WIA I) | LFS (dB) | 111 | 111 | 111 | 111 |
| Required Attenuation (WIA II) | LFS (dB) | 101 | 101 | 101 | 101 |
| Separation distance WIA I | d (m) | 293 | 497 | 891 | 436 |
| Separation distance WIA II | d (m) | 172 | 271 | 443 | 186 |

[Note: the outdoor calculations should be included for WIA III and WIA IV.]

Consider adding calculations with the SRDs operating above their sensitivity

Consider developing SEAMCAT simulations

#### Conclusion on WIA sharing with SRDs

Refer to side lobe calculations…

[Given that SRD devices and WIA systems are likely to use channelisation with maximum bandwidths in the region of 20 MHz it is feasible to introduce FWA systems into the same environment as SRD devices for the following reasons:

The majority of SRD devices are likely to be operated indoors;

The number of SRD devices currently using this band is very low.]

## INTELLIGENT TRANSPORT SYSTEMS (ITS)

WIA and ITS systems can operate on the same channel, therefor coexistence problems can be expected at both sites.

Regarding coexistence studies, two main types of potential problems have been identified:

-	interference from WIA (indoor / outdoor) on the ITS On-board unit (OBU),

-	interference from the ITS OBU to WIA (indoor/outdoor),

[Note: we should also insert some consideration to address the case where OBU is entering in a factory.]

The same propagation model as in section 5.2 is considered for the MCL calculations.

### Compatibility between interferer WIA (indoor) and victim ITS

The compatibility between interferer WIA (indoor) with 1 MHz bandwidth and the victim ITS is listed in Table 4. The final result of compatibility consideration is a required separation distance between interferer WIA an victim ITS.

Table 68: Interferer WIA I bandwidth 1 MHz, victim ITS [table format]

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: WIA-I: 1 MHz |  |  |  |  |  |  |
| Bandwidth | 1 | MHz |  |  |  |  |
| Tx out, eirp | 26 | dBm | 26 | 26 | 26 | 26 |
| effect of TPC (dB) | 0 | dB | 0 | 0 | 0 | 0 |
| Wall loss | 15 | dB | 15 | 15 | 15 | 15 |
| Antenna Gain | 0 | dBi |  |  |  |  |
| Bandwidth correction factor ( ItBW > VrBW) | 0 | dB |  |  |  |  |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 11 | 11 | 11 | 11 |
| Reception part: ITS |  |  |  |  |  |  |
| Receiver bandwidth | 10 | MHz | 10 | 10 | 10 | 10 |
| Receiver sensitivity | -82 | dBm | -82 | -82 | -82 | -82 |
| Antenna gain | 8 | dBi | 8 | 8 | 8 | 8 |
| C min per MHz at antenna input |  | dBm/MHz | -100 | -100 | -100 | -100 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -106 | -106 | -106 | -106 |
| MAIN LOBE WIA - MAIN LOBE ITS |  |  |  |  |  |  |
| Separation distance WIA ITS |  | m | 403 | 714 | 1351 | 725 |
| MAIN LOBE WIA - SIDE LOBE ITS |  |  |  |  |  |  |
| Sidelobe attenuation (dB) | 12 | dB | 12 | 12 | 12 | 12 |
| Separation distance WIA ITS |  | m | 212 | 345 | 529 | 261 |

The compatibility consideration between interferer WIA (indoor) with 3 MHz bandwidth and the victim ITS is listed in Table 5.

The compatibility consideration between interferer WIA (indoor) with 20 MHz bandwidth and the victim ITS is listed in Table 6.

Table 69: Interferer WIA III bandwidth 20 MHz, victim ITS [table format]

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: WIA III 20 MHz |  |  |  |  |  |  |
| Bandwidth | 20 | MHz |  |  |  |  |
| Tx out, eirp | 26 | dBm | 26 | 26 | 26 | 26 |
| effect of TPC | 0 | dB |  |  |  |  |
| Wall loss | -15 | dB | -15 | -15 | -15 | -15 |
| Antenna Gain | 0 | dBi |  |  |  |  |
| Bandwidth correction factor ( ItBW > VrBW) | -3 | dB | -3 | -3 | -3 | -3 |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 8 | 8 | 8 | 8 |
| Reception part: ITS |  |  |  |  |  |  |
| Receiver bandwidth | 10 | MHz | 10 | 10 | 10 | 10 |
| Receiver sensitivity | -82 | dBm | -82 | -82 | -82 | -82 |
| Antenna gain | 8 | dBi | 8 | 8 | 8 | 8 |
| C min per MHz at antenna input |  | dBm/MHz | -100 | -100 | -100 | -100 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -107 | -107 | -107 | -107 |
| MAIN LOBE WIA - MAIN LOBE ITS |  |  |  |  |  |  |
| Separation distance WIA ITS |  | m | 343 | 595 | 1095 | 561 |
| MAIN LOBE WIA - SIDE LOBE ITS |  |  |  |  |  |  |
| Sidelobe attenuation ITS | 12 | dB | 12 | 12 | 12 | 12 |
| Separation distance WIA ITS |  | m | 181 | 287 | 413 | 202 |

### COMPATIBILITY BETWEEN interferer ITS and victim WIA (indoor)

The compatibility consideration between interferer ITS (outdoor) and the victim WIA (indoor) with 1 MHz bandwidth is listed in Table 7.

Table 70: Interferer ITS; victim WIA I bandwidth 1 MHz  [table format]

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: ITS |  |  |  |  |  |  |
| Bandwidth | 10 | MHz |  |  |  |  |
| Tx out, eirp | 33 | dBm | 33 | 33 | 33 | 33 |
| effect of TPC (dB) | 8 | dB | -8 | -8 | -8 | -8 |
| Wall loss | -15 | dB | -15 | -15 | -15 | -15 |
| Antenna Gain | 8 | dBi |  |  |  |  |
| Bandwidth correction factor ( ItBW > VrBW) | -10 | dB | -10 | -10 | -10 | -10 |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 0 | 0 | 0 | 0 |
| Reception part: WIA 1: 1 MHz |  |  |  |  |  |  |
| Receiver bandwidth | 1 | MHz | 1 | 1 | 1 | 1 |
| Receiver sensitivity | -82 | dBm | -82 | -82 | 82 | -82 |
| Antenna gain | 0 | dBi | 0 | 0 | 0 | 0 |
| C min per MHz at antenna input |  | dBm/MHz | -82 | -82 | -82 | -82 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -88 | -88 | -88 | -88 |
| MAIN LOBE ITS - MAIN LOBE WIA I |  |  |  |  |  |  |
| Separation distance ITS WIA |  | m | 81 | 110 | 131 | 61 |
| SIDE LOBE ITS - MAIN LOBE WIA I |  |  |  |  |  |  |
| Sidelobe attenuation (dB) | 12 | dB | 12 | 12 | 12 | 12 |
| Separation distance ITS WIA |  | m | 39 | 48 | 49 | 22 |

The compatibility consideration between interferer ITS (outdoor) and the victim WIA (indoor) with 3 MHz bandwidth is listed in Table 8.

Table 71: Interferer ITS; victim WIA II bandwidth 3 MHz [table format]

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: ITS |  |  |  |  |  |  |
| Bandwidth | 10 | MHz |  |  |  |  |
| Tx out, eirp | 33 | dBm | 33 | 33 | 33 | 33 |
| effect of TPC | 8 | dB | -8 | -8 | -8 | -8 |
| Wall loss | 15 | dB | -15 | -15 | -15 | -15 |
| Antenna Gain | 8 | dBi |  |  |  |  |
| Bandwidth correction factor ( ItBW > VrBW) | -5 | dB | -5 | -5 | -5 | -5 |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 5 | 5 | 5 | 5 |
| Reception part: WIA II 3 MHz |  |  |  |  |  |  |
| Receiver bandwidth | 3 | MHz | 3 | 3 | 3 | 3 |
| Receiver sensitivity | -92 | dBm | -92 | -92 | -92 | -92 |
| Antenna gain | 0 | dBi |  |  |  |  |
| C min per MHz at antenna input |  | dBm/MHz | -97 | -97 | -97 | -97 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -103 | -103 | -103 | -103 |
| MAIN LOBE ITS - MAIN LOBE WIA II |  |  |  |  |  |  |
| Separation distance ITS WIA |  | m | 243 | 402 | 652 | 324 |
| SIDE LOBE ITS - MAIN LOBE WIA II |  |  |  |  |  |  |
| Sidelobe attenuation ITS | 12 | dB | 12 | 12 | 12 | 12 |
| Separation distance ITS WIA |  | m | 128 | 186 | 243 | 116 |

The compatibility consideration between interferer ITS (outdoor) and the victim WIA (indoor) with 20 MHz bandwidth is listed in Table 9.

Table 72: Interferer: ITS; Victim: WIA III bandwidth 20 MHz [table format]

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: ITS |  |  |  |  |  |  |
| Bandwidth | 10 | MHz |  |  |  |  |
| Tx out, eirp | 33 | dBm | 33 | 33 | 33 | 33 |
| effect of TPC | 8 | dB | -8 | -8 | -8 | -8 |
| Wall loss | 15 | dB | -15 | -15 | -15 | -15 |
| Antenna Gain | 8 | dBi |  |  |  |  |
| Bandwidth correction factor ( ItBW > VrBW) | 0 | dB |  |  |  |  |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 10 | 10 | 10 | 10 |
| Reception part: WIAIII – 20 MHz |  |  |  |  |  |  |
| Receiver bandwidth | 20 | MHz | 20 | 20 | 20 | 20 |
| Receiver sensitivity | -88 | dBm | -88 | -88 | -88 | -88 |
| Antenna gain | 0 | dBi |  |  |  |  |
| C min per MHz at antenna input |  | dBm/MHz | -101 | -101 | -101 | -101 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -107 | -107 | -107 | -107 |
| MAIN LOBE WIA - MAIN LOBE ITS |  |  |  |  |  |  |
| Separation distance WIA ITS |  | m | 404 | 714 | 1352 | 726 |
| MAIN LOBE WIA - SIDE LOBE ITS |  |  |  |  |  |  |
| Sidelobe attenuation ITS | 12 | dB | 20 | 20 | 20 | 20 |
| Separation distance WIA ITS |  | m | 212 | 345 | 530 | 261 |

### COMPATIBILITY BETWEEN interferer WIA (outdoor) and victim ITS

The compatibility consideration between interferer WIA (outdoor) with 3 MHz and the victim ITS RSU (outdoor) is listed in Table 10.

Table 73: Interferer WIA II bandwidth 3 MHz, victim ITS [table format]

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: WIA I 1 MHz |  |  |  |  |  |  |
| Bandwidth | 3 | MHz |  |  |  |  |
| Tx out, eirp | 26 | dBm | 26 | 26 | 26 | 26 |
| effect of TPC | 0 | dB |  |  |  |  |
| Wall loss | 0 | dB |  |  |  |  |
| Antenna Gain | 0 | dBi |  |  |  |  |
| Bandwidth correction factor ( ItBW > VrBW) | 0 | dB |  |  |  |  |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 26 | 26 | 26 | 26 |
| Reception part: ITS |  |  |  |  |  |  |
| Receiver bandwidth | 10 | MHz | 10 | 10 | 10 | 10 |
| Receiver sensitivity | -82 | dBm | -82 | -82 | -82 | -82 |
| Antenna gain | 8 | dBi | 8 | 8 | 8 | 8 |
| C min per MHz at antenna input |  | dBm/MHz | -100 | -100 | -100 | -100 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -106 | -106 | -106 | -106 |
| MAIN LOBE WIA - MAIN LOBE ITS |  |  |  |  |  |  |
| Separation distance WIA ITS |  | m | 900 | 1771 | 3848 | 2605 |
| MAIN LOBE WIA - SIDE LOBE ITS |  |  |  |  |  |  |
| Sidelobe attenuation ITS | 12 | dB | 12 | 12 | 12 | 12 |
| Separation distance WIA ITS |  | m | 474 | 856 | 1666 | 936 |

The compatibility consideration between interferer WIA (outdoor) with 20 MHz and the victim ITS (outdoor) is listed in Table 11.

Table 74: Interferer WIA III bandwidth 20 MHz, victim ITS [table format]

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: WIA III 20 MHz |  |  |  |  |  |  |
| Bandwidth | 20 | MHz |  |  |  |  |
| Tx out, eirp | 26 | dBm | 26 | 26 | 26 | 26 |
| effect of TPC | 0 | dB |  |  |  |  |
| Wall loss | 0 | dB |  |  |  |  |
| Antenna Gain | 0 | dBi |  |  |  |  |
| Bandwidth correction factor ( ItBW > VrBW) | -3 | dB | -3 | -3 | -3 | -3 |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 23 | 23 | 23 | 23 |
| Reception part: ITS |  |  |  |  |  |  |
| Receiver bandwidth | 10 | MHz |  |  |  |  |
| Receiver sensitivity | -82 | dBm | -82 | -82 | -82 | -82 |
| Antenna gain | 8 | dBi | 8 | 8 | 8 | 8 |
| C min per MHz at antenna input |  | dBm/MHz | -100 | -100 | -100 | -100 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -106 | -106 | -106 | -106 |
| MAIN LOBE WIA - MAIN LOBE ITS |  |  |  |  |  |  |
| Separation distance WIA ITS |  | m | 766 | 1476 | 3119 | 2015 |
| MAIN LOBE WIA - SIDE LOBE ITS |  |  |  |  |  |  |
| Sidelobe attenuation ITS | 12 | dB | 12 | 12 | 12 | 12 |
| Separation distance WIA ITS |  | m | 403 | 713 | 1350 | 724 |

### Compatibility between interferer ITS and victim WIA (outdoor)

The compatibility consideration between interferer ITS (outdoor) and the victim WIA (outdoor) with 3 MHz is listed in Table 12.

Table 75: Interferer ITS; victim WIA II bandwidth 3 MHz [table format]

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: ITS |  |  |  |  |  |  |
| Bandwidth | 1o | MHz |  |  |  |  |
| Tx out, eirp | 33 | dBm | 33 | 33 | 33 | 33 |
| effect of TPC (dB) | 8 | dB | 8 | 8 | 8 | 8 |
| Wall loss | 0 | dB |  |  |  |  |
| Antenna Gain | 8 | dBi |  |  |  |  |
| Bandwidth correction factor ( ItBW > VrBW) | -5 | dB | -5 | -5 | -5 | -5 |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 20 | 20 | 20 | 20 |
| Reception part: WIA 1 |  |  |  |  |  |  |
| Receiver bandwidth | 3 | MHz | 3 | 3 | 3 | 3 |
| Receiver sensitivity | -92 | dBm | -92 | -92 | -92 | -92 |
| Antenna gain | 0 | dBi | 0 | 0 | 0 | 0 |
| C min per MHz at antenna input |  | dBm/MHz | -97 | -97 | -97 | -97 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -103 | -103 | -103 | -103 |
| MAIN LOBE ITS - MAIN LOBE WIA |  |  |  |  |  |  |
| Separation distance ITS WIA |  | m | 543 | 998 | 1989 | 1163 |
| SIDE LOBE ITS - MAIN LOBE WIA |  |  |  |  |  |  |
| Sidelobe attenuation (dB) | 12 | dB | 12 | 12 | 12 | 12 |
| Separation distance ITS WIA |  | m | 285 | 482 | 835 | 418 |

The compatibility consideration between interferer ITS (outdoor) and the victim WIA (outdoor) with 20 MHz is listed in Table 13.

Table 76: Interferer ITS; victim WIA III bandwidth 20 MHz [table format]

| LINK BUDGET | Value | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- | --- |
| Emission part: ITS |  |  |  |  |  |  |
| Bandwidth | 10 | MHz |  |  |  |  |
| Tx out, eirp | 33 | dBm | 33 | 33 | 33 | 33 |
| effect of TPC (dB) | -8 | dB | -8 | -8 | -8 | -8 |
| Wall loss | 0 | dB |  |  |  |  |
| Antenna Gain | 8 | dBi |  |  |  |  |
| Bandwith correction ItBW > VrBW | 0 | dB |  |  |  |  |
| Net Tx Out eirp (spurious level) |  | dBm/MHz | 25 | 25 | 25 | 25 |
| Reception part: WIA III |  |  |  |  |  |  |
| Receiver bandwidth | 20 | MHz |  |  |  |  |
| Receiver sensitivity | -88 | dBm | -88 | -88 | -88 | -88 |
| Antenna gain | 0 | dBi | 0 | 0 | 0 | 0 |
| C min per MHz at antenna input |  | dBm/MHz | -101 | -101 | -101 | -101 |
| Protection criterion |  |  |  |  |  |  |
| Criterion C/I | 6 | dB | 6 | 6 | 6 | 6 |
| Allowable Interfering power level 'I' at receiver antenna input |  | dBm/MHz | -107 | -107 | -107 | -107 |
| MAIN LOBE ITS - MAIN LOBE WIA |  |  |  |  |  |  |
| Separation distance ITS WIA |  | m | 901 | 1772 | 3851 | 2608 |
| SIDE LOBE ITS - MAIN LOBE WIA |  |  |  |  |  |  |
| Sidelobe attenuation (dB) | 20 | dB | 20 | 20 | 20 | 20 |
| Separation distance ITS WIA |  | m | 474 | 856 | 1667 | 937 |

### Conclusion

#### Conclusion for interferer WIA and victim ITS

The protection of ITS receivers can be achieved with a separation distance of 1351 m to WIA systems with a bandwidth less than 10 MHz in the main loop of ITS antenna. The separation distance between victim ITS and interferer WIA with 20 m ranged between 343 m and 1095 m depending on considered propagation model.

The result looks much better for the case of interferer WIA and victim ITS side loop. For interferer WIA with a bandwidth less than 1o MHz ranged the separation distance between 212 m and 529 m. The separation distance ranged between 181 m and 412 in the case of interferer WIA with 20 MHz bandwidth and victim ITS side loop. The detailed values are listed in Table 14.

Table 77: Required separation distance interferer WIA (indoor) and victim ITS [table format]

|  | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- |
| WIA I --> ITS main loop | m | 403 | 714 | 1351 | 725 |
| WIA II --> ITS main loop | m | 403 | 714 | 1351 | 725 |
| WIA III --> ITS main loop | m | 343 | 595 | 1095 | 561 |
| WIA I --> ITS side loop | m | 212 | 345 | 529 | 261 |
| WIA II --> ITS side loop | m | 212 | 345 | 529 | 261 |
| WIA III --> ITS side loop | m | 181 | 287 | 413 | 202 |

For outdoor WIA applications is the required separation distance to protect ITS receivers much higher. In the case of interferer bandwidth less than 1o MHz, the separation distance between interferer and victim ITS main loop ranged between 900 m and 3848 m for depend on the considered propagation model. In the case of WIA system with 20 MHz bandwidth and victim ITS main loop the separation distance ranged between 766 m and 3119 m. The detail results listed in Table 15.

Table 78: Required separation distance interferer WIA (outdoor) and victim ITS [table format]

|  | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- |
| WIA II --> ITS main loop | m | 900 | 1771 | 3848 | 2605 |
| WIA III --> ITS main loop | m | 766 | 1476 | 3119 | 2015 |
| WIA II --> ITS side loop | m | 474 | 856 | 1666 | 936 |
| WIA III --> ITS side loop | m | 403 | 713 | 1350 | 724 |

#### Conclusion for interferer ITS and victim WIA

The ITS system has also an influence on all WIA systems. The separation distance ranged between 81 m and 131 m depending on environment for indoor WIA systems with 1 MHz bandwidth, ranged between 243 m and 652 for indoor WIA systems with 3 MHz bandwidth and for WIA systems with 20 MHz bandwidth the separation distance to interferer ITS ranged between 404 m and 1352 m. The detailed values depicted in Table 16.

Table 79: Required separation distance interferer ITS and victim WIA (indoor) [table format]

|  | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- |
| ITS main loop --> WIA I | m | 81 | 110 | 131 | 61 |
| ITS main loop --> WIA II | m | 243 | 402 | 652 | 324 |
| ITS main loop --> WIA III | m | 404 | 714 | 1352 | 726 |
| ITS side loop --> WIA I | m | 39 | 48 | 49 | 22 |
| ITS side loop --> WIA III | m | 128 | 186 | 243 | 116 |
| ITS side loop --> WIA III | m | 212 | 345 | 530 | 261 |

The influence from ITS to outdoor WIA systems is much higher, because the positive wall attenuation be away. The required separation distance between interferer ITS and WIA with 3 MHz bandwidth ranged between 543 m and 1989 m, depending on propagation model. The required separation distance between interferer ITS and outdoor WIA with 20 MHz bandwidth ranged between 901 m and 3851 m depending on propagation model.

The ITS side loop less influence the victim WIA. Nevertheless the required separation distance between interferer (ITS side loop) and victim WIA with 3 MHz bandwidth ranged between 285 m and 835 m. For victim WIA with 20 MHz bandwidth the required separation distance to interferer ITS side loop ranged between 474 m and 1667 m depending on considered propagation model.

Table 80: Required separation distance interferer ITS and victim WIA (outdoor)

|  | Units | Urban | Suburban | Rural | ETSI |
| --- | --- | --- | --- | --- | --- |
| ITS main loop --> WIA II | m | 543 | 998 | 1989 | 1163 |
| ITS main loop --> WIA III | m | 901 | 1772 | 3851 | 2608 |
| ITS side loop --> WIA II | m | 285 | 482 | 835 | 418 |
| ITS side loop --> WIA III | m | 474 | 856 | 1667 | 937 |

The final results of required separation distance shown that ITS and WIA systems interact at similar distances.

# Initial consideration on the license regime

## Coexistence management for license exempt usage

It is shown in the previous sections that it is not a simple task to detect the emission of the victim link in order to avoid those frequencies; a specific sensing antenna with better propagation conditions to the surrounding and thus to the potentially affected victim system, would be required. A threshold value of -87 dBm together with an omnidirectional 10dBi (only in azimuth) could be sufficient to detects the BFWA usage. That means if the sensing antenna is receiving any signal with a level above -87 dBm, the band where the threshold is exceeded needs to be avoided.

## Light licensing procedures

Here the avoidance of the BFWA channels could be enforced with a practical coordination procedure between the Administration and the BFWA and WIA operators.

A similar proposal is documented in ECC Report 167 (“Practical implementation of registration/coordination mechanism for UWB LT2 systems”) and a regulatory proposal is given in ECC/REC(11)09.

This procedure is imaginable and definitely an issue in the future as it is similar to cognitive radio. But it is not expected to be available in short or mid-term.

# CONCLUSIONs

[to be developed]

TYPICAL WIA ANTENNA EXAMPLES

Figure 31: Example of dual band antenna (Siemens AG)

Figure 32: Antenna pattern of dual band antenna (Siemens AG)

Figure 33: Antenna pattern of dual band antenna (Hirschmann Automation and Control GmbH)

| Figure 34: H-Plane field pattern (WIMo GmbH) | Figure 35: E-plane field pattern (WIMo GmbH) |
| --- | --- |

Path loss models

Figure 13: comparison IEEE model with 5m breakpoint distance with spherical diffraction and LOS/exp3.5

Figure 14: in SEAMCAT used IEEE model compared to LOS and exponent 3.5

Figure 15: IEEE 802.11 plugin for SEAMCAT

Figure 16: comparison of other indoor path loss models

List of referenceS

ETSI TR 102 889-2 V1.1.1: Electromagnetic compatibility and Radio spectrum Matters (ERM); System Reference Document; Short Range Devices (SRD); Part 2: Technical characteristics for SRD equipment for wireless industrial applications using technologies different from Ultra-Wide Band (UWB)

ERC/REC 70-03: Short Range Devices (SRD)

Recommendation ITU-R F:1336-3: Title?

IEEE 802.11: IEEE Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

IEEE 802.15.1: IEEE Standard for Information technology-- Local and metropolitan area networks-- Specific requirements-- Part 15.1a: Wireless Medium Access Control (MAC) and Physical Layer (PHY) specifications for Wireless Personal Area Networks (WPAN)

IEEE 802.15.4: IEEE Standard for Local and metropolitan area networks--Part 15.4: Low-Rate Wireless Personal Area Networks (LR-WPANs)

Radio Regulations (www.itu.int)

ERC Report 25: The European table of frequency allocations and utilisations covering the frequency range 9 kHz to 3000 GHz

Recommendation ITU-R M.1638: “Characteristics of and protection criteria for sharing studies for radiolocation, aeronautical radionavigation and meteorological radars operating in the frequency bands between 5 250 and 5 850 MHz”

ECC Decision (02)01: ECC Decision of 15 March 2002 on the frequency bands to be designated for the coordinated introduction of Road Transport and Traffic Telematic Systems

EN 300 674: Dedicated Short Range Communication (DSRC) transmission equipment (500 kbit/s / 250 kbit/s) operating in the 5,8 GHz Industrial, Scientific and Medical (ISM) band; Part 2: Harmonized EN under article 3.2 of the R&TTE Directive

EN12253: Road transport and traffic telematics. Dedicated short-range communication. Physical layer using microwave at 5.8 GHz

R&TTE Directive: Radio and telecommunications terminal equipment

ECC Report 173: Fixed Service in Europe

ECC/REC(06)04: Use of the band 5725-5875 MHz BFWA

EN 302 502: Broadband Radio Access Networks (BRAN); 5,8 GHz fixed broadband data transmitting systems; Harmonized EN covering essential requirements of article 3.2 of the R&TTE Directive

ITU-R Recommendation F.383-7: Radio-frequency channel arrangements for high-capacity fixed wireless systems operating in the lower 6 GHz (5 925 to 6 425 MHz) band

ERC Recommendation 14-01: Radio-frequency channel arrangements for high capacity analogue and digital radio-relay systems operating in the band 5925 MHz - 6425 MHz

ECC Report 101: Compatibility Studies in the band 5855– 5925 MHz between Intelligent Transport System (ITS) and other Systems

ECC Report 68: Compatibility studies in the band 5725-5875 MHz between Fixed Wireless Access (FWA) systems and other systemsHenley, A.; Bond, S.: Radiocommunications Agency Radio Technology & Compatibility Group - Building Shielding Loss at 5GHz. Amended Report, RTCG Project no. 424, 27th October 1997

Recommendation ITU-R M.1732-1: Characteristics of systems operating in the amateur and amateur-satellite services for use in sharing studies

ECC Report 172, Broadband Wireless Systems Usage in 2300-2400 MHz

Recommendation ITU-R SM. 1541 Title?

Recommendation ITU-R SM. 329 Title?

ERC Recommendation 74-01 Title?

EN 302372 : TLPR Title?

ECC Report 110: Compatibility studies between Broad-Band Disaster Relief (BBDR) and other systems

CEPT Report 26: Report from CEPT to the European Commission in response to the Permanent Mandate to CEPT regarding the “annual update of the technical annex of the Commission Decision on the technical harmonisation of radio spectrum for use by short range devices”

Recommendation ITU-R M.1461: Procedures for determining the potential for interference between radars operating in the radiodetermination service and systems in other services

Recommendation ITU-R M.1638: Characteristics of and protection criteria for sharing studies for radiolocation, aeronautical radionavigation and meteorological radars operating in the frequency bands between 5 250 and 5 850 MHz

Nobles and F. Halsall: "Radiowave Propagation Measurements within a Building at 2GHz, 5GHz and 17GHz"; Wireless '97 Conference paper, Calgary, July 1997.

EN 300 440 v.1.6.1, Electromagnetic compatibility and Radio spectrum Matters (ERM); Short range devices; Radio equipment to be used in the 1 GHz to 40 GHz frequency range; Part 1: Technical characteristics and test methods. 2010-08

INTERNATIONAL AMATEUR RADIO UNION REGION 1, VHF MANAGERS HANDBOOK. Version 6.00, December 2011

ITU-R Report SM.2154: Short-range radiocommunication devices spectrum occupancy measurement techniques

ECO Report XX on Summary of the responses to the questionnaire on the use of the frequency band 863-870 MHz and preliminary conclusions of the SRD/MG

European Commission Directive 2009/140/EC on the authorisation of electronic communications networks and services

Recommendation ITU-R SM.1046-2: Definition of spectrum use and efficiency of a radio system

Taking Sides on Technology Neutrality - Chris Reed Professor of Electronic Commerce Law, Centre for Commercial Law Studies, Queen Mary University  of London.2007

B.A. Witvliet, "PhD Research Plan - Spectrum Access Mechanism for Licence Exempt radio systems", Radiocommunications Agency Netherlands, 2010.

- ERC Report 101 on Comparison of the minimum coupling loss method, enhanced minimum coupling loss method, and the Monte-Carlo simulation

EN 300 220, ElectroMagnetic Compatibility and Radio Spectrum Matters (ERM);Short Range Devices (SRD);Radio equipment to be used in the 25 MHz to 1 000 MHz frequency range with power levels ranging up to 500 mW; Part 1: Technical characteristics and test methods

Tanenbaum, “Computer Networks” 4th Ed, 2003, Pearson Education Inc.

prEN13757-4: Communication systems for meters and remote reading of meters - Part 4: Wireless meter readout (Radio Meter reading for operation in the 868-870 MHz SRD band)

ETSI TR 102 886: Technical characteristics of Smart Metering (SM) Short Range Devices (SRD) in the UHF Band;System Reference Document, SRDs, Spectrum Requirements for Smart Metering European access profile Protocol (PR-SMEP)

FM22(09)077 Report on monitoring 863-870 MHz

EC Decision on on harmonisation of the radio spectrum for use by short-range devices

ETSI TR 103 056: Short Range Devices (SRD); Technical characteristics for SRD equipment for social alarm and alarm applications

CEPT Report 14 - Develop a strategy to improve the effectiveness and flexibility of spectrum availability for Short Range Devices (SRDs)

The value for the transmitter output power is higher than the specified value of 250 mW (24 dBm) in the technical report ETSI TR 102 889-2 V1.1.1 [1]. Explanation: The specified transmitter output power in [1] refers to the 2.4 GHz frequency band. In the frequency band from 5.725 to 5.875 GHz, the electromagnetic wave propagation conditions are worse; therefore a higher level of transmitter output power is needed to meet the requirements of industrial wireless applications also within this frequency band.

The receiver sensitivity of the RSU is -104 dBm for BPSK. The free space loss over 8 m is 18dB, antenna gain is assumed to be 15 dB at the RSU and 5dB at the OBU; the 1 m loss factor is 47 dB

Recommendation ITU-R M.1732-1 [21] the specific 5.7GHz band is combined with other amateur bands. Therefore some adjustments have been made in order to be more representative of 5.7GHz equipment

According IEEE 802.11-2012