SE(13)035A1

ECC Report <No>

Least Restrictive Technical Conditions suitable for MFCN, including IMT, for the frequency range 3.4 – 3.8 GHz

Month YYYY (Arial 9pt bold)

[last updated: DD Month YYYY) (Arial 9pt) [date of the latest update]]

# Executive summary (style: heading 1)

A BEM for the bands 3400 – 3600 MHz and 3600 – 3800 MHz was introduced through decision 2008/411/EC. However it is recognised that this BEM is not based on a harmonised frequency arrangement and that it is primarily intended for fixed services. Consequently the European Commission issued a mandate to undertake further studies on technical conditions, including BEM, for the 3400 – 3600 MHz and 3600 – 3800 MHz bands.

For the purposes of this report the term BWA refers to legacy BWA systems licenced under existing 3400 – 3600 MHz licencing regimes prior to XXX. The term MFCN is a collective term for radio communication systems including BWA, fixed point to point and point to multipoint services and for the purposes of this report refers to radio communication systems which should comply with the BEM defined in this report.

CEPT has thus carried out studies to determine appropriate LRTCs for MFCN. In this report the BEMs for different categories of base stations were derived from MCL analysis and simulations. Four different categories of base stations were considered: Macro, Micro, Pico and Femto. Since they have different characteristics, different BEMs are defined for each category of base station.

For each category of base station, the corresponding power levels of the BEM are defined for a number of different regions: In-block power (PIB) where the operator may use the full power for the corresponding base station type, baseline power (PBL) is defined for other operators’ blocks (FDD UL and FDD DL as well as TDD), power in the transitional regions (PTR) is defined for the regions up to 10 MHz adjacent to an operator’s own full-power block and power in the guard bands in case the FDD arrangement is used in 3400 – 3600 MHz: 3400 – 3410 MHz, 3490 – 3510 MHz and 3590 – 3600 MHz

Inblock limits:

Table 28 contains the e.i.r.p. in-block limits for different categories of base stations.

Table 28: In-block power limits (dBm e.i.r.p.)

| Macro BS | Not obligatory |
| --- | --- |
| Micro BS | [47 dBm |
| Pico BS | 24 dBm] |
| Femto/Home BS | 20 dBm |

Baseline limits:

- Baseline power limits (dBm/MHz e.i.r.p.)

|  | FDD DL region (limit per antenna) | FDD UL region (limit per cell) | TDD regions (limit per cell) |
| --- | --- | --- | --- |
| Macro BS | 2 | -42 | -42 |
| Micro BS | -9 | -45 | -45 |
| Pico BS | -27 | -43 | -43 |
| Femto/Home BS | -32 | -41 | -41 |

Transitional regions limits:

- Transitional region power limits (dBm/5MHz e.i.r.p.) per antenna for different off-sets from lower or upper block-edge

|  | +/- (0-5) MHz | +/- (5 - 10) MHz |
| --- | --- | --- |
| Macro BS | 16.3 | 11 |
| Micro BS | [5.6 | -1.4 |
| Pico BS | -16 | -20] |
| Femto/Home BS | -21.6 | -25 |

Guard band limits:

In the harmonised frequency arrangement guard bands are only required where FDD channelization is adopted. The permitted emission levels for these three different guard bands are as follows:

•	3400 – 3410 MHz: tbd

•	3490 – 3510 MHz: tbd

•	3590 – 3600 MHz: tbd

Coexistence studies for other services than MFCN have been carried out for both inband and out of band scenarios. The inband services considered are FSS, FS and BWA and the out of band services are civil and military Radiolocation.

The conclusions are as follows:

FS

MFCN applies to all Mobile and Fixed networks including point to point Fixed links.

FSS

Due to the varying characteristics of different types of FSS earth stations and their deployment, no single separation distance, guard band or signal strength limit can be provided to guarantee co-existence with MFCN. Successful co-existence should be achieved through co-ordination on a case-by-case basis, assuming FSS earth stations locations are known. However, some general observations can be made. Separation distances for co-existence vary considerably depending on type of equipment and deployment (e.g. tilt and clutter), but can be large. UE impact earth stations less than BS, so separation that prevents interference from BS will also protect earth stations from UE interference. There are several mitigation techniques that can be applied, in particular site shielding of earth stations. Interference from FSS satellites to MFCN may exceed the acceptable interference level, but in most cases only by a small margin.

Radiolocation:

Due to the varying characteristics of different types of radar stations and their deployment, no single separation distance, guard band or signal strength limit can be provided to guarantee co-existence with MFCN. Successful co-existence should be achieved through co-ordination on a case-by-case basis. However, some general observations can be made for non-overlapping adjacent channels. For airborne radars the required separation distance is approximately 0 km, depending on the radar type and antenna type. For land-based/shipborne radars the required separation distance is less than 1 km, depending on the radar type and antenna type. A frequency separation analyses concludes that for a 5 km separation, and considering IMT-Advanced interference to radars, the required frequency separation varies between 14 and 65 MHz, depending on radar type and scenario.

There are mitigation techniques which can reduce the separation distance or frequency separation required. In particular, for adjacent channel/adjacent band interference, improved receiver performance and decreased unwanted emissions can be efficient.

Regarding interference from radars to MFCN networks, installation of systems closer than ca. 5 km from the radar should be coordinated. It is necessary to establish a protection distance of approximately 11 km in some areas. Considering blocking effects, the radar may impact MFCN systems up to a distance of 30 km.

The analysis did not take into account the fact that radar antennas rotate and therefore only affect a particular MFCN base station or UE intermittently.

Adjacent band limit in the case of adjacent band usage by military systems:

In some CEPT countries military radiolocation systems that are deployed below 3400 MHz need a fixed limit for protection. According to [reference] this protection level is set as [x dBm].

[This protection requirement leads to [a] restricted block[s] in the case of TDD deployment and to a more stringent limit for the adjacent guardband in the case of FDD deployment]

TABLE OF CONTENTS

0	Executive summary (style: heading 1)	2

1	Introduction	8

2	Definitions	9

2.1	MFCN (including IMT)	9

2.1.1	Base station parameters	9

2.1.2	User equipment parameters	10

2.1.3	Deployment parameters	10

2.1.4	Additional parameters for statistical studies and MCL calculations	11

2.1.5	Base Station antenna model for MFCN networks	12

2.2	BWA	15

2.3	FSS	17

2.4	Radiolocation	18

3	Propagation models	19

3.1	Free Space model	19

3.2	ITU-R Report M.2135	19

3.3	Street level propagation	20

3.4	Indoor propagation	21

3.5	indoor - outdoor penetration	21

3.6	UE – UE propagation [necessary?]	22

3.7	application of propagation models	22

4	DERIVATION OF BS AND UE BLOCK EDGE MASKS	24

4.1	Interference scenarios	24

4.2	BS to BS interference: MCL Analysis	24

4.2.1	Acceptable e.i.r.p. levels	24

4.3	Macro – Macro: Simulation Analysis	24

4.4	Macro – Micro: Simulation Analysis	25

4.4.1	Macro Aggressor	25

4.4.2	Micro Aggressor	26

4.5	Macro – pico/femto: Simulation Analysis	26

4.6	Micro – Micro: Simulation Analysis	26

4.7	Micro – pico/femto: Simulation Analysis	27

4.8	pico/femto – pico/femto: Simulation Analysis	27

4.9	UE to UE interference	27

4.10	Base station BEM	28

4.10.1	Block Edge Mask elements	28

4.10.2	Base station categorisation	29

4.10.3	In-block region	29

4.10.4	Baseline regions	29

4.10.5	Transitional Region	30

4.10.6	Guard Bands	31

4.11	UE BEM	31

4.12	Use of transitional region	31

4.13	Mitigation techniques for intra-mfcn interference	32

4.13.1	Synchronization and alignment of UL/DL transmissions in TDD spectrum	32

4.13.2	Additional filtering	32

4.13.3	Restricted blocks / Guard bands	32

4.13.4	Site coordination	32

5	Inter-service interference	34

5.1	Coexistence between MFCN and existing BWA systems	35

5.2	coexistence between MFCN and FSS systems	35

5.2.1	Conclusion on FSS co-existence	35

5.3	coexistence between MFCN and Radiolocation systems	36

5.3.1	Conclusion on Radiolocation co-existence	36

5.4	protection of adjacent band services	37

6	Cross-border coordination	38

7	Conclusion	39

ANNEX 1: OOB e.i.r.p. calculations	42

ANNEX 2: INTRA-MFCN INTERFERENCE ANALYSIS – SIMULATION SET 1	48

ANNEX 3: Intra-MFCN interference analysis – simulation set 2   [Editor’s note: the simualtion results from the eco have to be added to this annex]	55

ANNEX 4: UE BEM DISCUSSION FROM CEPT REPORT 39	56

ANNEX 5: mfcn – fss co-existence	57

7.1	IMT In-band parameters	62

7.2	IMT Out-of-band parameters	63

7.3	Results	63

ANNEX 6: coexistence between MFCN and RADIOLOCATION services	66

ANNEX 7: List of reference	73

Note on the Table of Contents (delete after reading)

This is automatically styled and compiled from the headings, subheadings and page numbers from the document that follows. To update the Table of Contents move cursor within the table and press F9.

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| BEM | Block Edge Mask |
| BS | Base Station |
| BWA | Broadband Wireless Access |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| DEC | Decision |
| DL |  |
| EC | European Commission |
| ECC | Electronic Communications Committee |
| ECN | Electronic Communication Network |
| e.i.r.p. | equivalent isotropic radiated power |
| FDD | Frequency division duplex |
| FS | Fixed Service |
| IMT | International Mobile Telecommunications |
| LOS | Line-of-sight |
| LTE | Long Term Evolution |
| MFCN | Mobile/Fixed Communications Networks |
| NLOS | Non-line-of-sight |
| OOB | Outside Broadcasting |
| REC | Recommendation |
| REP | Report |
| Rx | Receiver |
| SEM | Spectrum Emission Mask |
| TDD | Time Division Duplex |
| Tx | Transmitter |
| UE | User Equipment |
| UL |  |
| UMTS | Universal Mobile Telecommunications System |
| WRC-07 | World Radio Conference in 2007 |

# Introduction

In 2004 ECC adopted ECC/REC/(04)05 [1] on “Guidelines for accommodation and assignment of Multipoint Fixed Wireless systems in frequency bands 3.4 - 3.6 GHz and 3.6 - 3.8 GHz” and in 2007 ECC/DEC/(07)02 [2] on “availability of frequency bands between 3400-3800 MHz for the harmonised implementation of Broadband Wireless Access systems (BWA)”. In 2008 the Block Edge Masks (BEM) contained in ECC/REC/(04)05 [1] were included in the European Commission Decision 2008/411/EC [3] (on the harmonisation of the 3 400-3 800 MHz frequency band for terrestrial systems capable of providing electronic communications services in the Community).

WRC-07 identified the band 3.4 - 3.6 GHz for IMT, and subsequently ECC adopted ECC/DEC/(11)06 [4] which contains the harmonised frequency arrangements for MFCN systems including IMT for 3.4 – 3.6 and 3.6 – 3.8 GHz.

As the BEM contained in ECC/REC/(04)05 [1] were developed for PMP FWS systems in 2004 it is not suitable for the introduction of MFCN systems including IMT in the 3.4 - 3.6 and 3.6 – 3.8 GHz band. Consequently ECC proposed in 2011 to develop a new Report on suitable BEM for this frequency range. CEPT has since received a Mandate from the European Commission to undertake studies on technical conditions, including BEM, in the 3.4 - 3.6 and 3.6 – 3.8 GHz bands. The mandate also requests that attention be paid to co-existence with existing systems in the same band and adjacent bands.

The proposal to apply the BEM approach was based on the fact that it has been able to fulfil the objectives set out in several WAPECS Mandates from the European Commission and it was therefore decided to use this approach as a working assumption for the development of the least restrictive technical conditions for the 3.4-3.8 GHz range. Co-existence with other services, co-channel or adjacent channel and applications is not necessarily guaranteed by the BEM for MFCN, as other methods may be more efficient depending on co-existence scenario, such as frequency or distance separation, or specific site engineering.

The BEM is a ‘regulatory mask’, and should not be confused with Spectrum Emission Masks (SEM) for base stations and user equipment employed by SDOs. The BEM concept does not in itself define the means by which the equipment in an operator’s network meet the BEM.

For user equipment, the BEM proposed by this Report is restricted to in-block power, which is in line with previous decisions from the EC on UE BEMs. UE aspects are taken into consideration however when deriving the BS BEM and in the analysis of interference to and from other services.

# Definitions

This section provides the parameters and characteristics of the systems that are deployed in the 3.4 - 3.8 GHz band or whose deployment is foreseen in the near future and which have been included in the compatibility studies in section 4.

## MFCN (including IMT)

The parameters presented below represent typical characteristics for MFCN equipment and networks. Examples of specific technologies that may be deployed are LTE (refs in 36-series and ETSI) and WiMAX (refs 802-series and ETSI). Relevant for the analysis in this report is also the Multi Standard Radio specification of 3GPP (refs in 37-series and ETSI(?)).[MFCN applies to all Mobile and Fixed networks including point to point Fixed links.] [editor’s note: ECO to check the definition for MFCN]

### Base station parameters

The following table includes parameters for Macrocell, Microcell, Picocell and Femtocell base stations for typical [MFCN/mobile] base stations. [editor’s note: check with other reports how MFCN was defined and used]

Base station parameters

|  | Macrocell (Wide area BS) | Microcell (Wide area BS) | Picocell (Local area BS) | Femtocell (Home area BS) |
| --- | --- | --- | --- | --- |
| Maximum output power (BS) | 46 dBm/[x dBm for 5MHz]10 MHz, 20 MHz, 40 MHz (for one transmit antenna port) | 35dBm/[x dBm for 5MHz]10 MHz, 20 MHz, 40 MHz (for one transmit antenna port) | 24 dBm/[x dBm for 5MHz]10MHz, 20 MHz, 40 MHz MHz (for one transmit antenna port) | 20 dBm/[x dBm for 5MHz]10MHz, 20 MHz, 40 MHz MHz (for one transmit antenna port) |
| ACS (BS) | 45 dB | [42/45 dB] | 45 dB | 44 dB |
| ACLR |  |  |  |  |
| BS feeder loss | 0 dB | 0 dB | 0 dB | 0 dB |
| e.i.r.p. | 63 dBm/[x dBm for 5MHz]10 MHz, 20 MHz, 40 MHz | 41 dBm/[x dBm for 5MHz]10 MHz, 20 MHz, 40 MHz | 24 dBm/[x dBm for 5MHz]10 MHz, 20 MHz, 40 MHz | 20 dBm/[x dBm for 5MHz]10 MHz, 20 MHz, 40 MHz |
| Signal/Channel bandwidth | 5 MHz, 10 MHz, 20 MHz, 40 MHz | 5 MHz, 10 MHz, 20 MHz, 40 MHz | 5 MHz, 10 MHz, 20 MHz, 40 MHz | 5 MHz, 10 MHz, 20 MHz, 40 MHz |
| Noise figure (BS) | 5 dB | 8dB | 13 dB | 13 dB |
| N=F.k.T.B(BS) | Values for 5 MHz -99 dBm/10 MHz -96 dBm/20 MHz -93 dBm/40 MHz =-109 dBm/MHz | -96 dBm/10 MHz -93 dBm/20 MHz -90 dBm/40 MHz =-106 dBm/MHz | -91 dBm/10 MHz -88 dBm/20 MHz -85 dBm/40 MHz =-101 dBm/MHz | -91 dBm/10 MHz -88 dBm/20 MHz -85 dBm/40 MHz =-101 dBm/MHz |
| I/N protection criterion for MCL analysis | -6 dB | -6 dB | -6 dB | -6 dB |

For detailed information on unwanted emissions for specific technologies, see Annex X.

### User equipment parameters

The following table includes parameters for user equipment in Macrocell, Microcell, Picocell and Femtocell environments of a typical [MFCN/mobile] network. [editor‘s note: same as above on MFCN]

User equipment parameters

| Parameter | Value |
| --- | --- |
| Maximum output power (UE) | 23 dBm |
| ACS (UE) | Value for 5MHz 33 dB (for 10 MHz channel), 27dB (for 20 MHz channel BW) [editor’s note: add footnote to explain 40 MHz case] |
| Antenna Type | Isotropic |
| Antenna height (UE) | 1.5m |
| Signal/Channel bandwidth | 5 MHz, 10 MHz, 20 MHz, 40 MHz |
| Noise figure (UE) | 9 dB |
| N=F.k.T.B(UE) | Value for 5MHz -95 dBm/10 MHz -92 dBm/20 MHz -89 dBm/40 MHz =-105 dBm/MHz |
| I/N protection criterion for MCL analysis | -6 dB |

[table agreed but the yellow]

### Deployment parameters

The following table includes typical deployment parameters for MFCN [see above on MFCN]Macrocell, Microcell, Picocell and Femtocell base stations.

Deployment parameters

|  | Macrocell (Wide area BS) | Microcell (Wide area BS) | Picocell (D) (Local area BS) | Femtocell (D) (Home area BS) |
| --- | --- | --- | --- | --- |
| Intersite distance within the same network | 350m | - | - | - |
| Cell range / 3GPP | 233m | - | - | - |
| Cell radius for omnicells | - | [50m] | [25m] | [10m] |
| Antenna Type | ITU-R F.1336 Sector antenna with peak side lobes for worst-case analysis (k = 0.7), and with average side lobes for statistical analysis (k = 0.2) | ITU-R F.1336 Omni antenna with peak side lobes for worst-case analysis (k = 0), and with average side lobes for statistical analysis (k = 0) | isotropic | isotropic |
| BS max antenna gain | 17 dBi | 6 dBi | 0 dBi | 0 dBi |
| Antenna downtilt | 6° | 0° | 0° | 0° |
| 3dB horizontal beamwidth | 65° | N.A. | N.A. | N.A. |
| Antenna height (BS) | 30 m | 6 m | 3 m | 1 m |

### Additional parameters for statistical studies and MCL calculations

The following table includes additional parameters that are needed for statistical studies.

Additional parameters for statistical studies

|  | Macrocell (Wide area BS) | Microcell (Wide area BS) | Picocell (D) (Local area BS) | Femtocell (D) (Home area BS) |
| --- | --- | --- | --- | --- |
| Minimum number of interfering BSs | 57 | 19 | 20 | 20 |
| Number of victim BSs | 57 | 19 [editor’s note: move row to SEAMCAT section in the annex] | 1 | 1 |
| Number of active users on the uplink (transmitting at the same time) | 3 | 3 | 3 | 3 |
| Intersite correlation factor | 0.5 | 0.5 [editor’s note: move row to SEAMCAT section in the annex] | - | - |
| Pathloss correlation- standard deviation | 8 dB | 8 dB | - | - |
| MCL between BS to UE | 70 dB | 53 dB (3m Free Space) | 50 dB (2m Free Space) | 50 dB |
| MCL between UE to UE | 43.5 dB (1m Free Space) | 43.5 dB | 43.5 dB | 43.5 dB |
| Bit rate mapping | As defined in TR 36.942 | As defined in TR 36.942 [editor’s note: move row to SEAMCAT section in the annex] | As defined in TR 36.942 | As defined in TR 36.942 |
| Handover margin | 3dB | 3dB [editor’s note: move row to SEAMCAT section in the annex] | Not applicable | Not applicable - |
| Maximum level for interference at victim receiver | IRX= -115 dBm/MHz | IRX= -112 dBm/MHz | IRX= -107 dBm/MHz | IRX= -107 dBm/MHz |
| SINR minimum of the BS | -10dB | -10dB [editor’s note: move row to SEAMCAT section in the annex] | -10dB | -10dB |

[Table agreed but the yellow]

Minimum transmit power (UE)	-40 dBm

The following table includes the minimum horizontal distance between two base stations of different networks that were used in the MCL calculations.

Minimum horizontal distance between two BS of different networks for the MCL calculation

| Direct Horizontal Distance | MACRO | MICRO | PICO | FEMTO |
| --- | --- | --- | --- | --- |
| MACRO | 70 m | 30 m | 30 m | 30 m |
| MICRO | 30 m | 30 m | 15 m | 15 m |
| PICO | 30 m | 15 m | 10 m | 10 m |
| FEMTO | 30 m | 15 m | 10 m | 10 m |

### Base Station antenna model for MFCN networks

Recommendation ITU-R F.1336-3 [16] is used for the macro and micro base station antenna patterns. For micro base stations the antenna pattern is assumed to be omnidirectional in the horizontal plane (Section 2 of F.1336-3), whereas for macro base stations three sector base stations are assumed (Section 3 of F.1336-3).

For statistical analysis the antenna patterns representing average side lobes are used, whereas for worst-case analysis (Minimum Coupling Loss), the antenna patterns representing peak side lobes are used.

The parameter k determines the side-lobe levels, and is set to different values depending on frequency and antenna type (sector vs omni) as follows:

- k = 0 for average and peak side lobe patterns for omni antennas (valid for 3 – 70 GHz)

- for sectoral antennas and peak side lobe patterns k = 0.7 ( valid for 1 to 6 GHz)

- for sectoral antennas and average side lobe patterns k = 0.2 ( valid for 1 to 6 GHz)

[Annex 8 of Recommendation ITU-R F.1336-3 [16] contains an alternative side lobe model to improve the sectoral antenna reference radiation patterns in the main text of the Recommendation.]

The vertical antenna patterns (average and peak side lobes) of a 3.5 GHz omni antenna with peak gain 6 dBi are presented in Figure X. The horizontal and vertical antenna patterns (average and peak side lobes) of a 3.5 GHz sector antenna with 3 dB beamwidth of 65 degrees and 17 dBi antenna gain derived from Recommendation ITU-R F.1336-3 [16] are plotted in Figure 1.

ITU-R Recommendation F.1336-3 omni antenna patterns, vertical dimension, for 6 dBi maximum gain. Average (blue) and peak (red) side lobes.

Horizontal pattern	Vertical pattern

ITU-R Recommendation F.1336-3 sector antenna patterns, 17 dBi maximum gain, 65 degrees 3 dB beamwidth. Average (blue) and peak (red) side lobe patterns.

## BWA

[The parameters in the following table are typically used for P-P and P-MP networks according to ECC Report 100. Editor’s note: Wait until the definition for MFCN is clear and think about how to reflect the “currently used” and the “new” BWA after that.]

The following table includes parameters for different types of BWA deployment, applicable for both FDD and TDD (Source: ECC Report 100 [17]).

BWA systems characteristics

| Parameter | Value | Unit | Remarks |
| --- | --- | --- | --- |
| Channel bandwidth | 1.75…14 | MHz |  |
| FDD; duplex spacing | 100 | MHz | This is the preferred duplex spacing value; in particular cases, 50MHz can be also used |
| TX peak output power, CS | 35 | dBm | In some scenarios the CS power may need to be up to 43dBm, to cope with Nomadic deployment |
| TX peak output power, TS-Fixed | 22 | dBm | The typical TS power is limited by cost and limitation of the CS power: the OFDMA/sub-channelisation gain compensates for the power difference. In some scenarios the TS power may need to be up to 30dBm. |
| TX peak output power, TS-Nomadic | 20 | dBm |  |
| Power Control reduction for outdoor units | 14 | dB |  |
| OFDMA/channelisation up-link gain | 3…15 | dB |  |
| UL/DL ratio, TS-Fixed | 0.01…1 |  | For FDD, max. 1:1 |
| UL/DL ratio, CS | 0.3…1 |  | For FDD, max. 1:1 |
| CS sector antenna gain | 17 | dBi | Assuming 60° and 90° antennas |
| CS omni-directional antenna gain | 9 | dBi |  |
| Adaptive antenna gain improvement | 20*logN | dBi | N=number of antennae (N=4 typically), assuming beam forming |
| Roof-top TS-Fixed antenna gain | 20 | dBi |  |
| Roof-top TS-Fixed antenna beam-width | 20 | Degrees |  |
| Window TS-Fixed antenna gain | 10 | dBi |  |
| Indoor TS directional antenna gain | 9 | dBi |  |
| TS omni-directional antenna gain for nomadic use | 3…5 | dBi |  |
| TS omni-directional antenna gain for mobile use | 0 | dBi |  |
| % rooftop TSs | 10-50 | % |  |
| % window TSs | 10-30 | % |  |
| % mobile TSs | 10-30 | % |  |
| % indoor TS-Fixed + TS-Nomadic | 30-70 | % | A bias to Nomadic use is anticipated |
| Number of channel in reuse pattern | 4 |  |  |
| Receiver sensitivity (CS) | -96…-74 | dBm | Evaluated for 7MHz NF=5dB; SNR=2.5…24.5dB, for different modulation/coding variants; 2dB-implementation loss |
| Receiver sensitivity (TS) | -94…-72 | dBm | Evaluated for 7MHz NF=7dB; SNR=2.5…24.5dB, for different modulation/coding variants; 2dB-implementation loss |

Table XX includes recommended parameters for point-to-point FS system (extracted from revised ITU-R F.758-4 – output of WP 5C) and Table XX recommended parameters for point-to-multipoint FS system (based on revised ITU-R F.758-4).

Recommended parameters for point-to-point FS system 
(extracted from revised ITU-R F.758-4 – output of WP5C)

| Frequency range (GHz) | 3.600-4.200 |  | 3.700-4.200 |
| --- | --- | --- | --- |
| Reference ITU-R Rec. | F.635 |  | F. 382 |
| Modulation | 64-QAM | 512-QAM | QPSK |
| Channel spacing and receiver noise bandwidth (MHz) | 10,30, 40, 60, 80, 90 | 10,30, 40, 60, 80, 90 | 28, 29 [editor’s note : check bold text] |
| Maximum Tx output power range (dBW) | −1 | 7 | 0 |
| Maximum Tx output power density range (dBW/MHz) (1) | −16…−11 | −9.0 | −15 |
| Minimum feeder/multiplexer loss range (dB) | 0 | 3 | 3 |
| Maximum antenna gain range (dBi) | 42 | 40 | 37 |
| Maximum e.i.r.p. range (dBW) | 41 | 44 | 38 |
| Maximum e.i.r.p. density range (dBW/MHz) (1) | 26…31 | 28 | 23 |
| Receiver noise figure (dB) | 3 | 2 | 4 |
| Receiver noise power density typical (=NRX ) (dBW/MHz) | −141 | −142 | −140 |
| Normalized Rx input level for 1 × 10–6 BER (dBW/MHz) | −114.5 | −106.5 | −126.5 |
| Nominal long-term interference power density (dBW/MHz) (2) | −141 + I/N | −142 + I/N | −140 + I/N |

(3) There are two modulations (QPSK and 4FSK) described and QPSK is selected.

Recommended parameters for point-to-multipoint FS system 
(based on revised ITU-R F.758-4)

| Frequency range (GHz) |  | 3.40-3.80 |  |
| --- | --- | --- | --- |
| Reference ITU-R Recommendation |  | F.1488 |  |
| Modulation format | Central Stations QPSK through 64-QAM(7) |  | Terminal Stations QPSK |
| Channel spacing and receiver noise bandwidth (MHz) | 25(5) 1.75, 3.5, ...14(9) |  | 25(5) 1.75, 3.5, ...14(9) |
| Tx output power range (dBW) | 5…13 |  | −6…0 |
| Tx output power density range (dBW/MHz)(1) | −6.46…10.6 |  | −17.5…−2.43 |
| Feeder/multiplexer loss range (dB) | 2 |  | 0 |
| Antenna type and gain range (dBi) | 10 (omni)… 18 (sector) |  | 6 (omni) indoor & outdoor… 18 (directional) outdoor |
| e.i.r.p. range (dBW) | 21…29 |  | 8…18 |
| e.i.r.p. density range (dBW/MHz)(1) | 9.54…26.5 |  | −3.46…15.6 |
| Receiver noise figure typical (dB) | 3 |  | 3 |
| Receiver noise power density typical (=NRX ) (dBW/MHz) | −141 |  | −141 |
| Normalized Rx input level for 1 × 10–6 BER (dBW/MHz) | −127.5…−114.5 |  | −127.5 |
| Nominal long-term interference power density (dBW/MHz) (2) | −141 + I/N |  | −141 + I/N |

NOTE – The intended set of parameters for two reference system for sharing/coexistence studies are presently not or only partially available; administrations are invited to contribute. On a provisional basis, the parameters reported in Annex 3 for the same bands may be used. (7) The modulation format is usually changed dynamically according the propagation impairment. (8) Recommendation ITU-R F.701 recommends only a basic pattern of 0.5 MHz (or its integer multiple). The values of 5, 5.5 and 6 MHz are proposed as most common channel spacings for these systems. (9) Recommendation ITU-R F.1488 recommends only a basic pattern of 0.25 MHz (or its integer multiple). The values of 1.75, 3.5, …14 MHz are proposed as most common channel spacings for these systems.

In addition, this type of equipment should comply with the essential requirements of the ETSI Standard EN 302 326-2 [18]. In particular, the transmitter spectrum density masks considered in this Report are taken from the EN 302 326-2 [18].

## FSS

The parameters for FSS systems can be found in ECC Report 100 [17] and ITU-R Report M.2109 [19]. Since no new sharing or compatibility study was done for this report for the reasons stated in the section XX, the parameters are not reproduced here. For further details on co-existence with FSS, including FSS parameters, see Annex 6, which contains a summary of previous studies.

## Radiolocation

The parameters for FSS systems can be found in ECC Report 100 [17], ECC Report 174 [20] and ITU-R Report M.2111 [21]. Since no new sharing or compatibility study was done for this report for the reasons stated in the section XX, the parameters are not reproduced here. For further details on co-existence with radiolocation, including radiolocation parameters, see Annex 7, which contains a summary of previous studies.

# Propagation models

## Free Space model

This is a basic propagation model, which describes the theoretical minimum propagation path loss between transmitter and receiver antennas in free space, when direct line of sight (LOS) is assumed. This propagation model is valid for all frequencies above 30 MHz:

where:

f = frequency [MHz],

d = distance between transmitter and receiver [km].

## ITU-R Report M.2135

[editor’s note: add information on LOS per site restriction]

The propagation models in ITU-R Report M.2135 [reference] are based on the work in Winner II (Wireless World Initiative New Radio phase II), and are valid for the frequency range 2 – 6 GHz.

The models cover different propagation scenarios for indoor and outdoor environments in urban, suburban and rural settings. The model that has been used in this report is the one for urban macro cells, which takes into account both LoS and NLoS propagation. The upper limit on distance (5 km) does not prevent it from being used in this context due to the small cell radius used in the simulations.

The path loss is calculated as follows:

LoS:

PL = 22.0 log10(d) + 28.0 + 20 log10(fc),  = 410, m < d < d′BP (1)

= 4, d′BP < d < 5 000 m(1), hBS = 25 m(1), hUT = 1.5 m(1)

NLoS:

PL = 161.04 – 7.1 log10 (W) + 7.5 log10 (h) – (24.37 – 3.7(h/hBS)2) log10 (hBS) +

(43.42 – 3.1 log10 (hBS)) (log10 (d)  3) +20 log10(fc) – (3.2 (log10 (11.75 hUT))2  4.97)

= 6

10 m < d < 5 000 m

h = avg. building height
W = street width

hBS =  25 m, hUT  = 1.5 m,
W = 20 m, h = 20 m.

The applicability ranges:
5 m < h < 50 m
5 m < W < 50 m 
10 m < hBS < 150 m 
1 m < hUT < 10 m

Footnote 1: Break point distance d′BP  = 4 h′BS h′UT fc/c, where fc is the centre frequency (Hz), c = 3.0  108 m/s is the propagation velocity in free space, and h′BS and h′UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h′BS and h′UT are computed as follows:

h′BS = hBS – 1.0 m, h′UT = hUT – 1.0 m

where: hBS and hUT are the actual antenna heights, and the effective environment height in urban environments is assumed to be equal to 1.0 m.

The LoS probabilities are given by the equation below. Note that probabilities are used only for system level simulations.

PLOS = min (18/d,1)  (1 – exp (–d / 63)) + exp (–d / 63), d is measured in meters.

## Street level propagation

This propagation model is used between micro cell base stations and outdoor UEs. The model is presented in [A Recursive model … ] and is also used in 3GPP, 25.942 [ref].

The proposed model is a recursive model that calculates the path loss as a sum of LOS and NLOS segments. The shortest path along streets between the BS and the UE has to be found within the Manhattan environment.

The path loss in dB is given by the formula:

where:

-	dn is the "illusory" distance;

-	l is the wavelength;

-	n is the number of straight street segments between BS and UE (along the shortest path).

The illusory distance is the sum of these street segments and can be obtained by recursively using the expressions  and  where c is a function of the angle of the street crossing. For a 90° street crossing the value c should be set to 0,5. Further, sn-1 is the length in meters of the last segment. A segment is a straight path. The initial values are set according to: k0 is set to 1 and d0 is set to 0. The illusory distance is obtained as the final dn when the last segment has been added.

The model is extended to cover the micro cell dual slope behavior, by modifying the expression to:

.

where:

.

Before the break point xbr the slope is 2 [unit missing], after the break point it increases to 4 [unit missing]. The break point xbr is set to 300 m. x is the distance from the transmitter to the receiver.

[Editor’s note: investigate the origin of the simplified W-I for above rooftop propagation]

To take into account effects of propagation going above rooftops it is also needed to calculate the pathloss according to the shortest geographical distance. This is done by using the COST Walfish-Ikegami Model and with antennas below rooftops:

L = 24 + 45 log (d+20).

Where:

-	d is the shortest physical geographical distance from the transmitter to the receiver in metros.

The final pathloss value is the minimum between the path loss value from the propagation through the streets and the path loss based on the shortest geographical distance, plus the log-normally distributed shadowing (LogF) with standard deviation of 10 dB should be added:

Pathloss_micro = min (Manhattan pathloss, macro path loss) + LogF.

## Indoor propagation

P.1238 [editor’s note: needs further elaboration]

## indoor - outdoor penetration

Alternatives: Cost 231 or M:2135 or WINNER II.

[Editor’s note: After the XO meeting in December only information on propagation models that were actually used in the studies will be retained.]

The ITU Recommendation ITU-R P.1238-7 for indoor path loss model is formally expressed as:

Ltotal=  20 log10 f    N log10 d    Lf  (n)  –  28dB	(1)

where:

L = the total path loss. Unit: decibel (dB).

N = distance power loss coefficient;

f = frequency (MHz);

d = separation distance (m) between the base station and portable terminal (where d> 1 m);

Lf = floor penetration loss factor (dB);

n = number of floors between base station and portable terminal (n 1).

Power loss coefficients, N, for indoor transmission loss calculation

| Frequency | Office |
| --- | --- |
| 3.5 GHz | 27 |

Floor penetration loss factors, Lf (dB) with n being the number of floors penetrated, for indoor transmission loss calculation (n ≥ 1)

| Frequency | Office |
| --- | --- |
| 3.5 GHz | 27 |

[editor’s note: if we assume the values below, why have the 2 tables above?]

The following wall indoor penetrations are assumed for the studies at 3.5 GHz:

- 18dB => first windows penetration

- 28dB => deep indoor penetration

## UE – UE propagation [necessary?]

IEEE 802.11 “Model C” [23], reference ECC Report 131 [24], for “NLoS” in Hot Spot environments.

For the studies of UE to UE using IEEE_C propagation model it was agreed to use an attenuation of 0db before and 10 dB after the break point for the scenario between Macrocell and Picocell. For Picocell scenarios a break point of 10m was agreed.

## application of propagation models

For worst-case analysis (BS – BS Minimum Coupling Loss calculations) the Free Space propagation is used, with added wall penetration loss where applicable (between outdoor and indoor base stations), as detailed in Annex X.

For statistical analysis to investigate BS – UE, UE – BS and UE – UE interference, Tables XX – YY below describe which propagation models have been used. Simulations are carried out for an urban scenario which is assumed to be the most important for this frequency range, and also the most challenging in terms of interference.

Propagation between BS and UE

| Scenario | Propagation model used |
| --- | --- |
| Macro BS – outdoor UE | ITU-R Report M.2135 |
| Macro BS – indoor UE | tbd |
| Outdoor Micro BS to outdoor UE | Recursive method/25.942 |
| Outdoor Micro BS to indoor UE | tbd |
| Indoor Pico BS to indoor UE | P.1238 |
| Indoor Pico BS to outdoor UE | tbd |
| Indoor Femto BS to indoor UE | P.1238 |
| Indoor Femto BS to outdoor UE | tbd |

# DERIVATION OF BS AND UE BLOCK EDGE MASKS

This section contains summaries of the Intra-MFCN interference studies that were taken as the basis for the BEM(s). Detailed information on some of the simulations can be found in the corresponding annexes (numbers). Co-existence with other services is considered separately in Section 5.

## Interference scenarios

For the derivation of the BEM, interference in all combinations of Macro-, Micro-, Pico- and Femtocells was considered between base stations, between base stations and UEs as well as between UEs.

Due to the static nature of the interference in the scenario Macrocell BS versus Macrocell BS only worst case analysis with the calculation of minimum coupling loss was performed. For all other interference scenarios Monte Carlo Simulations were done additionally to take into account the mobility and intermittent interference from terminals.

No need to distinguish between FDD and TDD interference …

[editor’s note: more text is needed to explain that aspect]

## BS to BS interference: MCL Analysis

### Acceptable e.i.r.p. levels

Table XX contains a summary of the results of the BS to BS MCL analysis that are presented in detail in Annex X. For each type of base station the most restrictive scenario has been highlighted in bold. The e.i.r.p. value for each scenario corresponds to the acceptable e.i.r.p. level that can be transmitted in the interfered base stations uplink channel. It has been assumed that the receiver properties are good enough to make transmitter leakage the dominant source of interference.

Acceptable e.i.r.p. levels to avoid BS-BS interference, dBm/MHz e.i.r.p.

| Victim Interferer | MFCN Outdoor macro BS | MFCN Outdoor micro BS | MFCN Indoor pico BS | MFCN Indoor femto BS |
| --- | --- | --- | --- | --- |
| MFCN Outdoor macro BS | -41.9 | -13.7 | -15.9 | -15.9 |
| MFCN Outdoor micro BS | -27.7 | -44.9 | -15.9 | -15.9 |
| MFCN Indoor pico BS | -40.9 | -33.0 | -43.5 | -33.5 |
| MFCN Indoor femto BS | -40.9 | -33.0 | -33.5 | -33.5 |

The values are derived per cell. [add some text here about number of antenna etc.]

## Macro – Macro: Simulation Analysis

The results in this section are presented in detail in Annex X.

The following two tables show the average and 5% level throughput degradation for uplink and downlink interference when two macro cellular systems are operated in the same geographical area on adjacent channels.

Uplink and Downlink UE Throughput degradation

| Additional Isolation (dB) | UPLINK |  |  | DOWNLINK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average throughput Degradation | 5% throughput Degradation |  | Average throughput Degradation | 5% throughput Degradation |  |
| -13 | 13.143 % |  | 31.240 % | 9.502 % |  | 52.995 % |
| -8 | 5.704 % |  | 10.941 % | 4.829 % |  | 26.280 % |
| 0 | 0.891 % |  | 1.683 % | 1.263 % |  | 6.406 % |
| 2 | 0.316 % |  | 0.607 % | 0.811 % |  | 3.515 % |
| 7 | 0.185 % |  | 0.185 % | 0.282 % |  | 1.131 % |
| 12 | 0.105 % |  | 0.010 % | 0.093 % |  | 0.650 % |
| 17 | 0.067 % |  | 0.001 % | 0.029 % |  | 0.411 % |

Table X shows the UL throughput degradation for the average and cell edge (5% level) for BS-to-BS interference. The significant need for additional isolation is clearly visible.

BS-to-BS scenario, UL throughput degradation

| ACLR offset X (dB) | BS-to-BS Case (Victim Uplink) |  |
| --- | --- | --- |
|  | Average throughput Degradation | 5% Degradation |
| 0 | 100 % | 100 % |
| 2 | 100 % | 100 % |
| 7 | 100 % | 100 % |
| 12 | 99.927 % | 100 % |
| 17 | 87.548 % | 100 % |
| 22 | 61.755 % | 53.232 % |
| 27 | 35.215 % | 23.355 % |
| 32 | 15.422 % | 8.547 % |
| 37 | 5.577 % | 2.768 % |

## Macro – Micro: Simulation Analysis

The results in this section are presented in detail in Annex X.

In this section results are presented for an interference scenario where a macro and a manhattan type micro system are operating in the same geographical area on adjacent channels.

### Macro Aggressor

The results presented in this section are for the case when the macro system is operating as the aggressor and the micro cells placed in the Manhattan grid (as shown in Figure X) are the victim.

Uplink and Downlink UE Throughput degradation

| Additional Isolation (dB) | UPLINK |  |  | DOWNLINK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average throughput Degradation | 5% throughput Degradation |  | Average throughput Degradation | 5% throughput Degradation |  |
| -13 | 19.50 % |  | 30.119 % | 4.096 % |  | 5.892 % |
| -8 | 10.146 % |  | 11.746 % | 1.523 % |  | 2.630 % |
| 0 | 3.022 % |  | 1.900 % | 0.627 % |  | 1.572 % |
| 2 | 2.029 % |  | 1.337 % | 0.168 % |  | 0.0647 % |
| 7 | 0.796 % |  | 0.0407 % | 0.0536 |  | 0.0204 |
| 12 | 0.281 % |  | 0.008 % | 0.0169 % |  | 0.0064 % |
| 17 | 0.092 % |  | 0.0027 % | 0.0053 % |  | 0.002 % |

### Micro Aggressor

This section presents the results for the macro-micro scenario where the micro system is operating as the aggressor and the macro system is the victim.

One important thing to note here is that the results contained in Table xx are for one reference cell in the macro system, which is overlapped completely by the micro (Manhattan) grid (see Figure X). For the DL, only the UEs in this reference macro cell are considered and for the UL case, the BS of this reference cell is considered for evaluation.

Uplink and Downlink UE Throughput degradation

| Additional Isolation (dB) | UPLINK |  |  | DOWNLINK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average throughput Degradation | 5% throughput Degradation |  | Average throughput Degradation | 5% throughput Degradation |  |
| -13 | 1.838 % |  | 0.1991 % | 3.122 % |  | 33.88 % |
| -8 | 0.6703 % |  | 0.0630 % | 1.617 % |  | 31.73 % |
| 0 | 0.3766 % |  | 0.0106 % | 0.468 % |  | 12.278 % |
| 2 | 0.0729 % |  | 0.0063 % | 0.314 % |  | 7.665 % |
| 7 | 0.0232 % |  | 0.0019 % | 0.1168 % |  | 2.558 % |
| 12 | 0.0073 % |  | 0.0006 % | 0.0393 % |  | 0.823 % |
| 17 | 0.0023 % |  | 0.0002 % | 0.0127 % |  | 0.261 % |

[editor’s note: put in some text on negative additional isolation to explain the idea]

## Macro – pico/femto: Simulation Analysis

[editor’s note: include input from ECO and possibly from Ericsson]

## Micro – Micro: Simulation Analysis

The results in this section are presented in detail in Annex 3.

The micro vs micro case governs the scenario where two systems are being operated in a Manhattan structure.

Uplink and Downlink UE Throughput degradation

| Additional Isolation (dB) | UPLINK |  |  | DOWNLINK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average throughput Degradation | 5% throughput Degradation |  | Average throughput Degradation | 5% throughput Degradation |  |
| -13 | 3.193 % |  | 1.277 % | 2.159 % |  | 6.210 % |
| -8 | 1.299 % |  | 0.445 % | 0.763 % |  | 2.093 % |
| 0 | 0.289 % |  | 0.142 % | 0.138 % |  | 0.242 % |
| 2 | 0.182 % |  | 0.084 % | 0.0828 % |  | 0.188 % |
| 7 | 0.062 % |  | 0.026 % | 0.0264 % |  | 0.102 % |
| 12 | 0.020 % |  | 0.008 % | 0.0083 % |  | 0.101 % |
| 17 | 0.006 % |  | 0.002 % | 0.0026 % |  | 0.084 % |

## Micro – pico/femto: Simulation Analysis

[editor’s note: Simulations to be included after simulations were run by ECO]

## pico/femto – pico/femto: Simulation Analysis

[editor’s note: include input from ECO and possibly from Ericsson]

## UE to UE interference

The interference between UEs belonging to different FDD operators will be very limited due to the duplex gap and efficient filtering of duplex filters for both transmitters and receivers.

Interference from TDD UEs to FDD UEs and vice versa will also be limited provided equipment is designed properly. For instance, 3GPP has defined an additional requirement of -50 dBm/MHz as inter-band protection level (Table 6.6.3.2-1):

- Band 22 (FDD 3410-3590 MHz) UE is specified with the following requirements:

- -50 dBm/MHz for the protection of band 43 (TDD 3600-3800 MHz)

- -50 dBm/MHz over 3525-3590 MHz and -40 dBm/MHz over 3510-3525 MHz for the protection of other operators in the band 22.

- Band 42 (TDD 3400-3600 MHz) UE  is specified with the following requirements:

- -50 dBm/MHz for the protection of band 43 (TDD 3600-3800 MHz) with some exceptions due to technical feasibility constraint.

- Band 43 (TDD 3600-3800 MHz) UE is specified with the following requirements:

- -50 dBm/MHz for the protection of band 42 and/or band 22 with some exceptions due to technical feasibility constraint.

UE to UE interference will be strongest between unsynchronized TDD networks. When a UE is transmitting at the same time as a UE using an adjacent channel in the vicinity is receiving, interference may be strong, and within a given frequency band the TDD bands, there is no additional requirement on UE OOB emissions. Table X provides the UE OOB emission levels for various frequency offsets (the 20 MHz channel spectrum mask). A mitigating effect is that such scenarios may be relatively rare, except from in hot spots.

UE OOB emission levels

| Frequency offset (MHz) | dBm/MHz |
| --- | --- |
| 0 - 1 | -5,8 |
| 1 - 5 | -10 |
| 5 - 10 | -13 |
| 10 - 15 | -13 |
| 15 - 20 | -13 |
| 20 - 25 | -25 |
| > 25 | -30 |

This report only provides the in-block power for UEs, and the UE to UE interference is not studied further here. See also Annex X for a further discussion on this issue.

## Base station BEM

[Editor’s note: further discussions on antenna gain (also for micro, pico) and multiple antenna columns/elements needed in several sections below]

Different types of Base Stations may be used in the 3.4 – 3.8 GHz range, and since they have different characteristics, different BEMs are defined in this section for Macro, micro, pico and femto/Home Base Stations respectively.

Whenever there are bilateral agreements between operators these levels may be relaxed, for instance in case of adjacent TDD blocks that are synchronized and have aligned DL/UL transmissions.

Between TDD two blocks there is a transitional region, sometimes referred to as restricted channel. The size of this transitional region is determined by filter requirements, see Section X.

The BEM has not been explicitly constructed to protect other services/applications in the band. That is managed by other means, such as geographical or frequency separation, or special site arrangements. However, sometimes the application of the BEM to base stations will improve co-existence possibilities.

### Block Edge Mask elements

For each type of base station, the corresponding power levels of the BEM are defined for a number of different regions:

•	In-block power (PIB) where the operator may use the full power for the corresponding base station type

•	Baseline power (PBL) is defined for other operators’ blocks (FDD UL and FDD DL as well as TDD)

•	Power in the transitional regions (PTR) is defined for the regions up to 10 MHz adjacent to an operator’s own full-power block, see further below.

•	Power in the guard bands in case the FDD arrangement is used in 3400 – 3600 MHz: 3400 – 3410 MHz, 3490 – 3510 MHz and 3590 – 3600 MHz

[editor’s note: drawing would help the understanding of this section!]

### Base station categorisation

For the purposes of this report the base stations are categorised by the following power limits at the antenna connector in table X. These categories correspond to placement above or just at rooftop level (macro), on the side of a wall (micro), on a wall or at the ceiling indoors (pico) or in a home environment (femto).

[editor’s note: add some text to explain that the values in table 1 do not contradict the values in the following table, but that they are for a different purpose]

Power limits for different types of base stations

| Type of BS | Power limit at the antenna connector |
| --- | --- |
| Macro BS | No limit |
| Micro BS | [38 dBm] |
| Pico BS | 24 dBm |
| Femto BS | 20 dBm |

### In-block region

Table 28 contains the e.i.r.p. in-block limits for different types of base stations.

Table 28: In-block power limits (dBm e.i.r.p.

| Macro BS | Not obligatory |
| --- | --- |
| Micro BS | [47 dBm |
| Pico BS | 24 dBm] |
| Femto/Home BS | 20 dBm |

[note: Ericsson proposes to delete the table ot to remove/increase upper limits for Micro and Pico]

For the macro base station, an in-block e.i.r.p. limit is not obligatory. In case an upper limit is desired by an administration, a value of 68 dBm e.i.r.p. may be applied.

[The Micro BS e.i.r.p. limit of 47 dBm is calculated by using a 38 dBm limit for the power at the antenna connector together with an antenna with a maximum gain of 9 dBi.]

### Baseline regions

For each type of base station there are two baseline limits corresponding to on the one hand blocks in the FDD DL region, and on the other blocks in the FDD UL and TDD regions. These levels are presented in Table X. The FDD DL baseline levels have been derived from requirements for BS-to-UE and UE-to-BS interference requirements, whereas the FDD UL/TDD baseline levels have been derived from the BS-to-BS interference analysis in Section X. The limits provided are per antenna in the FDD DL region and per cell in the FDD UL/TDD regions.

When the power limit is defined per cell, a simplistic method for calculating the total power used in the cell is to add up the power from each antenna column/element. This will overestimate the interference power, as for the most part the antenna powers will not add up constructively, and since the usage of advanced antennas may be considerably different (narrow lobes, dynamic lobes, etc.). For limits defined per cell 3 dB should be removed from the total power due to the different polarisation.

[Editor’s note: Switch to 5 MHz measurement bandwidth, as that has been used for other BEMs.]

Baseline power limits (dBm/MHz e.i.r.p.)

|  | FDD DL region (limit per antenna) | FDD UL region (limit per cell) | TDD regions (limit per cell) |
| --- | --- | --- | --- |
| Macro BS | 2 | -42 | -42 |
| Micro BS | -9 | -45 | -45 |
| Pico BS | -27 | -43 | -43 |
| Femto/Home BS | -32 | -41 | -41 |

Note that in FDD DL region the limit is per antenna, whereas in the FDD UL and TDD region it is per cell. This is because, blah, blah, blah

### Transitional Region

This region is defined to enable the reduction of power from the in-block power to the baseline level in the FDD DL and the TDD regions. In the case of FDD downlink block transitional regions, the size is 10 MHz (which may be outside the FDD DL band). For TDD block transitional regions, the size is limited by the distance to the adjacent operator’s full-power block, or the distance to the lower or upper edge of the TDD band, in either case not more than 10 MHz (see Section X).

The requirements for different types of base stations are defined in 5 MHz bandwidth, 0 – 5 MHz and 5 – 10 MHz offset from the upper and lower edges of an operator’s block. The FDD DL transitional levels have been derived from requirements for BS-to-UE and UE-to-BS interference requirements, whereas the TDD transitional levels have been derived from BS-to-BS interference requirements. The requirements are summarized in Table X.

[Note: Add paragraph with explanation that this in combination with ACS of interfered operator is enough]

Transitional region power limits (dBm/5MHz e.i.r.p.) per antenna for different off-sets from lower or upper block-edge

|  | +/- (0-5) MHz | +/- (5 - 10) MHz |
| --- | --- | --- |
| Macro BS | 16.3 | 11 |
| Micro BS | 5.6 | -1.4 |
| Pico BS | -16 | -20 |
| Femto/Home BS | -21.6 | -25 |

### Guard Bands

In the harmonised frequency arrangement guard bands are only required where FDD channelization is adopted.

[editor’s note: add channel arrangement picture]

The permitted emission levels for these three different guard bands are as follows:

- 3400 – 3410 MHz: FDD UL baseline level

- 3490 – 3510 MHz: FDD UL baseline level in 3490 – 3500 MHz, FDD DL baseline level in 3500 – 3510 MHz

- 3590 – 3600 MHz: FDD DL baseline level

For the FDD DL and TDD blocks adjacent to guard bands, the transitional requirements apply for the first 10 MHz outside the block.

## UE BEM

This report provides a recommended upper limit of 25dBm e.i.r.p. for the in-block power of the terminals, which is in line with previous Decisions of the Commission, see [refs for 2.6 GHz, 2 GHz, prev 3.4 – 3.8 GHz].

Since any possible additional requirements on UEs are not included in the relevant EC decisions, these requirements have to be taken into account by ETSI when developing harmonised standards. Close cooperation between ETSI and CEPT as well as SDOs may be necessary to ensure that any additional requirements on UEs are taken into account in the harmonized standards.

CEPT Report 39, Sections XX, contains a more detailed discussion about responsibilities of different organizations regarding UE BEMs, which is also provided in Annex X.

## Use of transitional region

The analysis above shows that in the case of unsynchronized TDD networks, it is necessary to have a certain frequency separation, 5 or 10 MHz, between two full-power TDD blocks. This transitional region, sometimes referred to as a restricted channel, may then be used for low power communication. Synchronization of networks and alignment of uplink and downlink transmissions of adjacent TDD operators would allow full powers to be used in the transitional regions, as there would be no BS to BS or UE to UE interference. This is not discussed any further in this section.

It is assumed here that the transitional regions are used by MFCN services. It is for further study whether other services could be deployed.

BS to UE and UE to BS interference will not cause any problems, as the interference scenarios are the same as those studied in the simulation analysis in Section X.

For BS to BS interference, two interference aspects must be taken into account. To begin with, transmitter leakage from a BS into an adjacent block of another operator from a base station using the transitional region should not exceed the TDD region baseline level. Secondly, in-block power of the base station must be limited so as to avoid excessive receiver leakage for the adjacent block operator. The analysis for deriving power limits in the transitional region, Section X, shows that the transitional region power for 0-5 MHz offset is sufficiently low to avoid interference problems, and can thus be used as in-block power as well. It is here assumed that the characteristics of the interfered receiver are aligned with the assumptions on typical equipment as presented in Section X. That the acceptable power is lower than that of a terminal is a result of having static BS to BS interference as opposed to intermittent UE to BS interference.

[For UE to UE interference … to be continued.]

## Mitigation techniques for intra-mfcn interference

## There are a number of ways to reduce interference between MFCN networks deployed in the same or adjacent geographical areas. These mitigation techniques may be used to meet the requirements of the block edge masks or to obtain additional interference reduction when the block edge masks do not provide sufficient protection.

### Synchronization and alignment of UL/DL transmissions in TDD spectrum

When TDD spectrum is used without synchronization and alignment of UL/DL transmission, there could be BS to BS and UE to UE interference. In particular BS to BS interference is known to require special treatment, as is also obvious from Sections X and Y above, containing MCL and simulation analysis of such scenarios for different types of base station deployments. Indeed, additional filtering is required, and due to the roll-off region of such filters, it is not possible to allocate adjacent full-power blocks without a certain separation, see further Section X (filter requirements). Usage of unsynchronized TDD systems thus has two drawbacks, additional equipment and loss of spectrum for full-power deployment.

These drawbacks can be removed by synchronization of TDD operator’s networks, and by alignment of UL/DL transmissions. The interference will then only be from BS to UE and from UE to BS. These are the same interference scenarios as for an FDD allocation, and consequently no additional filters or frequency separation is necessary, provided that Tx and Rx leakage characteristics of the TDD equipment is similar to that of FDD systems.

Synchronization is technically feasible for outdoor cells (using GNSS like GPS), and the main technical challenge comes from indoor femtocells cases. However for this kind of scenario, it may be that synchronization between operators is not necessary, considering the expected average distance, probability of interference (i.e. two femtocells on adjacent channel close to each other, wall penetration loss, etc.).

The one remaining drawback of such an arrangement is the lack of flexibility in terms of split between UL and DL transmissions. Unless substantial geographical separation between different deployment areas is available, this UL/DL alignment between operators may also be necessary between different geographical areas.

[Reference to TDD synch report, if finished simultaneously]

### Additional filtering

Additional filtering can be applied to base stations on both the transmitter and the receiver side to reduce leakage to and from adjacent channels. Indeed, the solution with 5 or 10 MHz separation between unsynchronized full-power TDD blocks requires such additional Tx and Rx filters for the kind of typical MFCN characteristics presented in Section X.

### Restricted blocks / Guard bands

In the case of unsynchronised adjacent band networks all kind of interference scenario may occur. The scenarios that are not dealt with by standardisation are the BS to BS interference and the TS to TS interference. For BS to BS interference, the situation requires additional filters, but also a frequency separation between full-power blocks of different operators, to allow filter roll-off. This separation may be achieved by specifically assigned channels in-between full-power blocks, or by operator-internal assignment of spectrum that is used with lower power or not at all.

### Site coordination

Site coordination enables limitation of BS to BS interference in the case where the base stations are deployed in close proximity to each other. Measures that can be applied are for instance choice of antenna tilt or azimuthal direction, horizontal or vertical antenna separation (ref ITU-R Report), and general selection of antenna placement.

The BEM power limits have been calculated based on specific assumptions regarding physical separation of interfering and interfered antennas, which are not necessarily always satisfied in reality. Deriving BEM conditions from an absolute worst case would place unrealistically strict requirements on all BS equipment. For deployment scenarios where the BEM assumptions do not hold, site coordination may provide effective measures to ensure sufficiently low interference.

# Inter-service interference

Table 31 contains the allocations for 3.3 – 4.2 GHz in the Radio Regulations (“Release 2012”) for Regions 1-3, with footnotes concerning Europe summarized below the table. The services present in 3.4 – 3.8 GHz or in adjacent bands considered for co-existence analysis in this report are thus Radiolocation, Fixed Service and Fixed Satellite Service.

Service with allocations on secondary basis, such as amateur radio and radiolocation above 3.4 GHz, are not studied here. Furthermore ECC Report 100 [17] contains an analysis of co-existence between BWA and ENG/OB.

The co-existence analysis is in general not based on the BS BEM, but rather the basic characteristics of the MFCN networks, see Section X, in order to provide the appropriate information for those cases when due to bilateral operator agreements the requirements on base stations have been relaxed.

Radio Regulations

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 3 300-3 400 RADIOLOCATION | 3 300-3 400 RADIOLOCATION Amateur Fixed Mobile | 3 300-3 400 RADIOLOCATION Amateur |
| 5.149 (SM : not this band) 5.429 5.430 (SM : not this part of the world) | 5.149 | 5.1495.429 |
| 3 400-3 600 FIXED FIXED-SATELLITE (space-to-Earth) Mobile 5.430A Radiolocation 5.431 | 3 400-3 500 FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile 5.431A Radiolocation 5.433 5.282 | 3 400-3 500 FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile 5.432B Radiolocation 5.433 5.282 5.432 5.432A |
|  | 3 500-3 700 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile Radiolocation 5.433 | 3 500-3 600 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.433A Radiolocation 5.433 |
| 3 600-4 200 FIXED FIXED-SATELLITE (space-to-Earth) Mobile |  | 3 600-3 700 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile Radiolocation 5.435 |
|  | 3 700-4 200 FIXED FIXED-SATELLITE (space to-Earth) MOBILE except aeronautical mobile |  |

Footnote 5.429 is an additional allocation of 3300 – 3400 MHz to fixed and mobile on a primary basis for some countries in Regions 1 and 2, however none of those in Europe. It concerns European countries only in the sense that countries bordering the Mediterranean shall not claim protection for their fixed and mobile services from the radiolocation service and is therefore not studied any further in this report.

Footnote 5.430 A states that the band 3 400-3 600 MHz is allocated to the mobile, except aeronautical mobile, service on a primary basis for a number of European (all CEPT?) and other countries including subject to agreement obtained under No. 9.21 with other administrations and is identified for International Mobile Telecommunications (IMT). However this identification does not preclude the use of this band by any application of the services to which it is allocated and does not establish priority in the Radio Regulations. “Before an administration brings into use a (base or mobile) station of the mobile service in this band, it shall ensure that the power flux-density (pfd) produced at 3 m above ground does not exceed −154.5 dB(W/(m2  4 kHz)) for more than 20% of time at the border of the territory of any other administration. This limit may be exceeded on the territory of any country whose administration has so agreed.”

Footnote 5.431 provides an additional allocation in Germany, Israel and the United Kingdom, where the band 3 400-3 475 MHz is also allocated to the amateur service on a secondary basis. It does thus not require protection and is not studied in this report.

## Coexistence between MFCN and existing BWA systems

[editor’s note: Include material from the “sanity check”]

[editor’s note: Compatibility studies show that additional protection of the FS system is required (Excel file from JPK, add this as annex)

Coordination may be necessary

Usage of additional filters may not always solve the issue]

[Additional filters or other technical solutions are required to implement the BEM of ECC Recommendation (04)05, but in fact since it was understood that BEM are not typically used, the deployed BS do not implement additional filters. For FDD or TDD, measures like power restriction or introduction of guard band may be used. In addition, for TDD, the coordination can be done by the synchronisation in this band.]

Cross-technology TDD synchronization might be possible and could result in some benefits for coexistence.

## coexistence between MFCN and FSS systems

Co-existence between the existing BWA/Mobile Services and FSS has been studied in ECC Report 100 [17] and ITU-R Report M.2109 [19]. These reports are summarized in Annex X, and conclusions are drawn below.

### Conclusion on FSS co-existence

Due to the varying characteristics of different types of FSS earth stations (e.g. bandwidths, antenna diameter, antenna gain) their deployment (antenna height, elevation angle) and the terrain surrounding them, as well as differences in characteristics of different BWA or MFCN systems, no single separation distance, guard band or signal strength limit can be provided to guarantee co-existence with MFCN. Successful co-existence should be achieved through co-ordination on a case-by-case basis. However, some general observations can be made:

- Co-channel co-existence is not possible when FSS earth stations are deployed ubiquitously since then no minimum separation distance can be guaranteed.

- Separation distances for co-existence vary considerably depending on type of equipment and deployment (e.g. tilt and clutter), but can be large.

- BWA TS/MFCN UE impact earth stations less than CS/BS, so separation that prevents interference from CS/BS will also protect earth stations from TS/UE interference.

- LNB of satellite receivers need to be considered for adjacent frequency band operation.

- There are several mitigation techniques that can be applied, in particular site shielding of earth stations.

- Interference from FSS satellites to MFCN may exceed the acceptable interference level, but in most cases only by a small margin.

It is noted that the results above are primarily based on co-existence with MFCN macro cells only. Micro, pico and femto cell co-existence will result in considerably lower separation distances due to lower power and shielding offered by houses in the vicinity of the base stations.

## coexistence between MFCN and Radiolocation systems

Co-existence between MFCN and Radiolocation has been studied in ECC Reports 100 [17] and ECC Report 174 [20] and ITU-R Report M.2111 [21]. The results from these studies are summarized in Annex Y and conclusions are drawn below.

### Conclusion on Radiolocation co-existence

Due to the varying characteristics of different types of radar stations, their deployment (antenna height, elevation angle) and the terrain surrounding them, as well as differences in characteristics of different BWA/MFCN systems, no single separation distance, guard band or signal strength limit can be provided to guarantee co-existence with MFCN. Successful co-existence should be achieved through co-ordination on a case-by-case basis. However, some general observations can be made:

Sharing studies of MFCN interference to different types of radars, assuming non-overlapping adjacent channel analysis and with IMT-Advanced unwanted emissions of -17 dBm/MHz, have shown the following:

  - For airborne radars the required separation distance is approximately 0 km, depending on the radar type and antenna type.

  - For land-based/shipborne radars the required separation distance is less than 1 km, depending on the radar type and antenna type.

A frequency separation analyses concludes that for a 5 km separation, and considering IMT-Advanced interference to radars, the required frequency separation varies between 14 and 65 MHz, depending on radar type and scenario.

[Editor’s note: note that adjacent here means higher interference than -17 dBm/MHz. Need to express that somehow.]

There are mitigation techniques which can reduce the separation distance or frequency separation required. In particular, for adjacent channel/adjacent band interference, improved receiver performance and decreased unwanted emissions can be efficient.

Regarding interference from radars to MFCN networks, the following observations have been made:

- Installation of BWA systems closer than ca. 5 km from the radar should be coordinated;

- In order to guarantee a limited C/I degradation of the P-MP BWA system, it is necessary to establish a protection distance of approximately 11 km in some areas (this value may be much less in some directions);

- Considering the degradation for blocking effect, the radar can have impact in the BWA systems until 30 km (this value may be much less in some directions).

A radar system radiates directional beams and, for instance, a victim BWA CS in a rotation period of the radar will only be affected x percentage of time. This probability was not considered in the main studies and in this manner the minimum separation distances obtained between the systems are somewhat pessimistic. Measurements of continuous versus intermittent interference indicate that radar pulses cause less considerably less damage than a continuous wave interference with the same power.

## protection of adjacent band services

In some CEPT countries military radiolocation systems that are deployed below 3400 MHz need a fixed limit for protection. According to [reference] this protection level is set as [x dBm].

[This protection requirement leads to [a] restricted block[s] in the case of TDD deployment and to a more stringent limit for the adjacent guardband in the case of FDD deployment]

[editor’s note: Find and include the limits for the adjacent band systems from the existing regulation]

# Cross-border coordination

This section describes the basic idea of how to manage interference between MFCN networks across borders (or between different regions within one country), i.e. interference between operators using overlapping frequencies in adjacent geographical areas.

For the case when networks on either side of a boundary are coordinated in the sense that the same frequency arrangement is used, and that TDD operators are synchronized and use the same uplink-downlink configuration, cross-border coordination between MFCN networks is a well-known problem. For detailed descriptions of how cross-border coordination is managed in CEPT see the relevant cross-border Recommendations REFS (ERC Recommendation 01-01, ECC Recommendation (05)08, ECC Recommendation (08)02, ECC Recommendation (11)04, ECC Recommendation (11)05). Considering the system characteristics of the MFCN networks, see Section X, expected to be deployed in 3.4 – 3.8 GHz, the general methodology should apply also for this frequency range:

- Apply the appropriate field strength (or pfd) trigger levels from the appropriate CEPT cross-border Recommendation to protect MFCN equipment. These field strengths are typically defined for a height 3 meters above ground level, at the border and possibly also some distance into the adjacent country/region.

- A propagation model is selected, e.g. ITU-R Recommendation P.1546, and the field strength at the border (or some distance into the other country) is calculated for e.g. 10% time and 50% of locations. Coordination is then required when base stations cause field strengths exceeding the trigger levels.

- A detailed field strength analysis can then be carried out to incorporate more details from the deployment and the detailed topography of the region in question.

Modifications are introduced to the interfering network to ensure that the field strength (pfd) levels are sufficiently low on the other side of the border Cross-border coordination requires special care when different frequency allocations (FDD vs TDD) are used on either side of a border or when TDD operators on either side of the border do not synchronize their systems and choose the same uplink-downlink configuration, due to BS-BS interference. Such interference may appear in the 3.4 – 3.8 range due to the multiple frequency arrangements and the TDD allocations. Although the same principles apply as for the case above, trigger levels are considerably lower and may lead to substantially increased separation distances.

The restrictions on field strength levels across the border may thus constrain the deployments of operators in proximity of border areas. From this point of view, there is a clear benefit of harmonization and even synchronization across borders. In addition, harmonized band plans would also help to mitigate interference between terminal stations.

It should be noted that there are ongoing studies within CEPT which will detail the various field strength values that may be used for technology neutral co-ordination of dissimilar systems. Cross-border coordination in the band 3400 – 3800 MHz will be subject to an ECC Recommendation as for other cross-border coordination in other bands.

# Conclusion

There are 4 different categories of base stations: Macro, Micro, Pico and Femto. Since they have different characteristics, different BEMs are defined for each category of base station.

For each category of base station, the corresponding power levels of the BEM are defined for a number of different regions: In-block power (PIB) where the operator may use the full power for the corresponding base station type, baseline power (PBL) is defined for other operators’ blocks (FDD UL and FDD DL as well as TDD), power in the transitional regions (PTR) is defined for the regions up to 10 MHz adjacent to an operator’s own full-power block and power in the guard bands in case the FDD arrangement is used in 3400 – 3600 MHz: 3400 – 3410 MHz, 3490 – 3510 MHz and 3590 – 3600 MHz

[editor’s note: add picture on channelling arrangement and BEM general]

In the following sections the power limits for the different regions are defined.

Inblock limits:

The following table contains the e.i.r.p. in-block limits for different categories of base stations.

In-block power limits (dBm e.i.r.p.)

| Macro BS | Not obligatory |
| --- | --- |
| Micro BS | [47 dBm |
| Pico BS | 24 dBm] |
| Femto/Home BS | 20 dBm |

Baseline limits:

- Baseline power limits (dBm/MHz e.i.r.p.)

|  | FDD DL region (limit per antenna) | FDD UL region (limit per cell) | TDD regions (limit per cell) |
| --- | --- | --- | --- |
| Macro BS | 2 | -42 | -42 |
| Micro BS | -9 | -45 | -45 |
| Pico BS | -27 | -43 | -43 |
| Femto/Home BS | -32 | -41 | -41 |

Transitional regions limits:

- Transitional region power limits (dBm/5MHz e.i.r.p.) per antenna for different off-sets from lower or upper block-edge

|  | +/- (0-5) MHz | +/- (5 - 10) MHz |
| --- | --- | --- |
| Macro BS | 16.3 | 11 |
| Micro BS | [5.6 | -1.4 |
| Pico BS | -16 | -20] |
| Femto/Home BS | -21.6 | -25 |

Guard band limits:

In the harmonised frequency arrangement guard bands are only required where FDD channelization is adopted. The permitted emission levels for these three different guard bands are as follows:

•	3400 – 3410 MHz: tbd

•	3490 – 3510 MHz: tbd

•	3590 – 3600 MHz: tbd

Coexistence studies for other services than MFCN have been carried out for both inband and out of band scenarios. The inband services considered are FSS, FS and BWA and the out of band services are civil and military Radiolocation.

The conclusions are as follows:

FS

MFCN applies to all Mobile and Fixed networks including point to point Fixed links.

FSS

Due to the varying characteristics of different types of FSS earth stations and their deployment, no single separation distance, guard band or signal strength limit can be provided to guarantee co-existence with MFCN. Successful co-existence should be achieved through co-ordination on a case-by-case basis, assuming FSS earth station locations are known. However, some general observations can be made. Separation distances for co-existence vary considerably depending on type of equipment and deployment (e.g. tilt and clutter), but can be large. UE impact earth stations less than BS, so separation that prevents interference from BS will also protect earth stations from UE interference. There are several mitigation techniques that can be applied, in particular site shielding of earth stations. Interference from FSS satellites to MFCN may exceed the acceptable interference level, but in most cases only by a small margin.

Radiolocation:

Due to the varying characteristics of different types of radar stations and their deployment, no single separation distance, guard band or signal strength limit can be provided to guarantee co-existence with MFCN. Successful co-existence should be achieved through co-ordination on a case-by-case basis. However, some general observations can be made for non-overlapping adjacent channels. For airborne radars the required separation distance is approximately 0 km, depending on the radar type and antenna type. For land-based/shipborne radars the required separation distance is less than 1 km, depending on the radar type and antenna type. A frequency separation analyses concludes that for a 5 km separation, and considering IMT-Advanced interference to radars, the required frequency separation varies between 14 and 65 MHz, depending on radar type and scenario.

There are mitigation techniques which can reduce the separation distance or frequency separation required. In particular, for adjacent channel/adjacent band interference, improved receiver performance and decreased unwanted emissions can be efficient.

Regarding interference from radars to MFCN networks, installation of systems closer than approximately 5 km from the radar should be coordinated. It is necessary to establish a protection distance of approximately 11 km in some areas. Considering blocking effects, the radar may impact MFCN systems up to a distance of 30 km.

The analysis did not take into account the fact that radar antennas rotate and therefore only affect a particular MFCN base station or UE intermittently.

Adjacent band limit in the case of adjacent band usage by military systems:

In some CEPT countries military radiolocation systems that are deployed below 3400 MHz need a fixed limit for protection. According to [reference] this protection level is set as [x dBm].

[This protection requirement leads to [a] restricted block[s] in the case of TDD deployment and to a more stringent limit for the adjacent guardband in the case of FDD deployment]

OOB e.i.r.p. calculations

OOB e.i.r.p. = acceptable out-of-block e.i.r.p. emissions, i.e. emissions into the frequency block of the interfered base station measured after the transmitting antenna in the direction of the antenna boresight.

The baseline BEM level calculated here is based on Minimum Coupling Loss analysis for interference between base stations belonging to different operators, reflecting the need for worst-case analysis in the BS-BS interference scenarios. For each type of base station, such an MCL analysis is carried out for all other types of base stations. The strictest requirement obtained for each type of base station can then be used as the baseline level for OOB e.i.r.p. based on BS-BS interference. For BS – UE interference see Section X. Section Y summarizes the BEM requirements based on all these calculations and simulations.

Protection levels based on I/N = -6 dB:

- Macro BS (NF 5 dB): -115 dBm/MHz

- Micro BS (NF 8 dB): -112 dBm/MHz

- Pico BS (NF 13 dB): -107 dBm/MHz

- Femto BS (NF 13 dB): -107 dBm/MHz

Using the acceptable interference as defined above, OOB e.i.r.p. can be determined from the following equations:

Iacc = OOB e.i.r.p. – Tx tilt/Tx antenna decoupling – Propagation Loss – wall penetration loss + Grx – Rx tilt/Rx antenna decoupling

OOB e.i.r.p. = Iacc + Tx tilt/Rx antenna decoupling + Prop loss + wall penetration loss – Grx + Rx tilt/Rx antenna decoupling

No feeder loss is assumed. All calculations are done for a bandwidth of 1 MHz. Table 32 shows the minimum horizontal distance between different types of base stations. See Section X for antenna heights for different base stations.

Table 32: Minimum horizontal distance between two Base Stations of different networks
 for the MCL calculations

| Minimum Horizontal Distance | MACRO | MICRO | PICO |
| --- | --- | --- | --- |
| MACRO | 70 m | 30 m | 30 m |
| MICRO | 30 | 30 m | 15 m |
| PICO | 30 | 15 | 10 m |
| FEMTO | 30 | 15 | 10 |

BASELINE OOB E.I.R.P. LIMIT FOR MACROCELL BS [EDITOR’S NOTE: ALEX, CAN YOU PLEASE ALSO CHECK THE FORMAT FOR THE HEADINGS IN ANNEXES! I CANNOT CHANGE THEM TO THE ECC ANNEX HEADINGS WITHOUT CREATING A MESS…]

Macrocell BS to macrocell BS

It is assumed that the antennas of the two macro base stations are on the same level, and that there is an antenna decoupling loss of 4.8 dB at each antenna due to downtilt, 6 degrees, of the antennas.

Table 33: Macro BS to Macro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -115.0 |
| Tx Downtilt Loss (dB) | 4.8 |
| PL (dB) | 80.5 |
| Wall penetration loss (dB) | 0 |
| - Rx Ant. Gain (dBi) | - 17 |
| Downtilt Loss (dB) | 4.8 |
| OOB e.i.r.p. Level (dBm/MHz) | -41.9 |

Macrocell BS to Microcell BS

In the co-existence scenario between Macro BS and micro BS, Macro BS antenna height is 30 m and micro BS antenna height is 6m. As a consequence of this height difference there is an additional antenna decoupling loss at both antennas, which is calculated with the Recommendation ITU-R F.1336 [8] sector antenna and omni antenna models, peak side lobes in both cases.

Table 34: Macro BS to Micro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -112.0 |
| Tx antenna decoupling (dB) | 12.9 |
| PL (dB) | 75.2 |
| Wall penetration loss (dB) | 0 |
| - Rx Ant. Gain (dBi) | - 6 |
| Rx antenna decoupling (dB) | 16.2 |
| OOB e.i.r.p. Level (dBm/MHz) | -13.7 |

Macrocell BS to Picocell/Femto BS

In the calculation given in X for the co-existence scenario from macro BS to pico/femto BS, it is supposed that the pico/femto BS is placed roughly level with the base station, so that there is a worst case assumption of the main lobe of the macro BS antenna pointing directly at the pico/femto base station. It is assumed that there is a wall in-between the macro base station antenna and the antenna of the pico/femto cell.

Table 35:Macro BS to Pico/Femto BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -107.0 |
| Tx antenna decoupling (dB) | 0 |
| PL (dB) | 73.1 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | - 0 |
| Rx antenna decoupling (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -15.9 |

Baseline OOB e.i.r.p. limit for Microcell BS

Microcell BS to Macrocell BS

Similarly to the Macro – micro case, antenna decoupling due to the vertical antenna diagrams of macro and micro have been applied.

Table 36: Micro BS to Macro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -115.0 |
| Tx antenna decoupling (dB) | 16.2 |
| PL (dB) | 75.2 |
| Wall penetration loss (dB) | 0 |
| - Rx Ant. Gain (dBi) | - 17 |
| Rx antenna decoupling (dB) | 12.9 |
| OOB e.i.r.p. Level (dBm/MHz) | -27.7 |

Microcell BS to Microcell BS

The calculation of the baseline OOB e.i.r.p. level for Micro BS for the co-existence scenario Micro BS to Micro BS is summarized in the table below. As seen from the simulations (ref), there is an “interference margin” in the UL of micro cells, so we can assume there is an additional margin which has not been taken into account in the table below.

Table 37: Micro BS to Micro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -112.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 73.1 |
| Wall penetration loss (dB) | 0 |
| - Rx Ant. Gain (dBi) | -6 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -44.9 |

Microcell BS to Picocell/Femtocell BS

The calculation of the baseline OOB e.i.r.p. level for Micro BS for the co-existence scenario Micro BS to Picocell/femtocell BS is summarized in Error! Reference source not found.. For this co-existence scenario, since Picocell/Femtocell BS antennas are placed inside of building, an indoor penetration factor of 18 dB is used in the calculation of potential interference from the outdoor microcell BS to the indoor picocell/femtocell BS. No antenna decoupling has been assumed in these calculations, although there is a minor difference in micro and pico/femto BS antenna height even if the pico/femto base stations are located on the ground floor of the building.

Table 38: Micro BS to Pico/Femto BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -107.0 |
| Tx antenna decoupling (dB) | 0 |
| PL (dB) | 73.1 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | - 0 |
| Rx antenna decoupling (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -15.9 |

baseline e.i.r.p. level for picocell bs

Picocell BS to Macrocell BS

The calculation of the baseline OOB e.i.r.p. level for pico BS with the co-existence scenario Pico BS to Macrocell BS is summarized in Table 39 In the calculation, by considering picocell BS is inside of the building and macrocell BS is in outdoor area, an indoor penetration factor of 18 dB is used. No antenna decoupling loss is assumed, as the pico cell may be on the same level as the macrocell BS antenna. .

Table 39: Pico BS to Macro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -115.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 73.1 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | -17 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -40.9 |

Picocell BS to Microcell BS

The calculation of the baseline OOB e.i.r.p. level for pico BS with the co-existence scenario Pico BS to Microcell BS is summarized in Table xxx. In the calculation, by considering that the picocell BS is inside the building and the microcell BS is in an outdoor area, an indoor penetration factor of 18 dB is used.

Table 40: Pico BS to Micro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -112.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 67.0 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | -6 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -33.0 |

Picocell BS to Picocell BS

The calculation of the baseline OOB e.i.r.p. level for picocell BS with the co-existence scenario Pico BS to Picocell/Femtocell BS is summarized in Error! Reference source not found.. In the calculation, free space propagation model is used in the pathloss calculation. It is assumed that there is no wall between the base stations.

Table 41: Pico BS to Pico BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -107.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 63.5 |
| Wall penetration loss (dB) | 0 |
| - Rx Ant. Gain (dBi) | 0 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -43.5 |

Picocell BS to Femtocell BS

For the pico – femto scenario it is assumed that there is a wall of indoor type in-between the base station antennas, corresponding to 10 dB penetration loss.

Table 42: Pico BS to Femto BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -107.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 63.5 |
| Wall penetration loss (dB) | 10 |
| - Rx Ant. Gain (dBi) | 0 |
| Rx Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -33.5 |

Baseline OOB e.i.r.p. limit for Femtocell BS

Femtocell BS to Macrocell BS

The calculation of the baseline OOB e.i.r.p. level for Femto BS with the co-existence scenario Femtocell BS to Macrocell BS is summarized in Table 43. In the calculation, an 18 dB indoor penetration loss is used.

Table 43: Macro BS to Macro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -115.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 73.1 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | -17 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -40.9 |

Femtocell BS to Microcell BS

The calculation of the baseline OOB e.i.r.p. level for Femto BS with the co-existence scenario Femtocell BS to Microcell BS is summarised in Table 44.

Table 44: Femto BS to Micro BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -112.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 67.0 |
| Wall penetration loss (dB) | 18 |
| - Rx Ant. Gain (dBi) | -6 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -33.0 |

Femtocell BS to Picocell/Femtocell BS

The calculation of the baseline OOB e.i.r.p. level for Femto BS with the co-existence scenario Femto BS to Femtocell/Picocell BS is summarized in Table 45

Table 45: Femto BS to Pico/Femto BS OOB e.i.r.p. analysis

| F (MHz) | 3600 |
| --- | --- |
| Protection level (dBm) at BS Rx | -107.0 |
| Tx Downtilt Loss (dB) | 0 |
| PL (dB) | 63.5 |
| Wall penetration loss (dB) | 10 |
| - Rx Ant. Gain (dBi) | 0 |
| Downtilt Loss (dB) | 0 |
| OOB e.i.r.p. Level (dBm/MHz) | -33.5 |

Conclusions

Table ?? contains a summary of the results from the sections above. The most restrictive scenario for each type of base stations has been highlighted yellow. In Section X, these results and those from BS – UE and UE – BS interference analysis are used to derive the BS BEM.

Table 46: OOB e.i.r.p. levels based on BS-BS interference, dBm/MHz e.i.r.p.

| Victim Interferer | MFCN Outdoor macro BS | MFCN Outdoor micro BS | MFCN Indoor pico BS | MFCN Indoor femto BS |
| --- | --- | --- | --- | --- |
| MFCN Outdoor macro BS | -41.9 | -13.7 | -15.9 | -15.9 |
| MFCN Outdoor micro BS | -27.7 | -44.9 | -15.9 | -15.9 |
| MFCN Indoor pico BS | -40.9 | -33.0 | -43.5 | -33.5 |
| MFCN Indoor femto BS | -40.9 | -33.0 | -33.5 | -33.5 |

INTRA-MFCN INTERFERENCE ANALYSIS – SIMULATION SET 1

Simulation Methodology

Simulations are performed using the well known Monte-Carlo simulation methodology elaborated in [1]. The simulation parameters employed had been agreed at ECC PT1 – XO # 1 meeting CG on 3.5 GHz, unless stated otherwise. In general, the simulations are performed using the following procedure:

- Run the system under observation (victim system) independently without the impact of any aggressor in the adjacent band with the simulation parameters as mentioned in the Table below. This provides the baseline performance of the system (SINR, throughput, etc.)

- Introduce the aggressing system in the adjacent band without any additional isolation (i.e. restricted channel) and evaluate the impact on the victim system in terms of performance (throughput) degradation of the link

- Introduce a restricted channel (additional isolation) in between the two systems and repeat step 2 to identify the required restricted channel for acceptable performance of the victim system

The table below summarizes the parameters used in the simulation analysis. Parameters that are in addition to those listed in Section X are highlighted yellow.

Table 47:Simulation Parameters

| Simulation Parameters |  |
| --- | --- |
| Parameter | Value |
| Bandwidth | 10 MHz |
| Frequency | 3.5 GHz |
| Handover margin | 3 dB |
| Parameters for Macro Deployment |  |
| Nr. of sites | 19 |
| Nr. of cells per site | 3 cells/site |
| Nr. of active users per cell | 3 |
| Inter-Site Distance (ISD) | 500 m (3GPP Case 1) |
| Propagation Model | ITU-R M-2135 [2] |
| Inter-site fading correlation | 0.5 |
| Maximum Coupling Gain | -70 dB (i.e. ~= 30m from BS) |
| BS antenna type | ITU-R F1336 [3] sectorized, with K=0.7 and averaged side-lobes |
| BS antenna gain | 17 dBi |
| BS antenna height | 30 m |
| BS noise figure | 5 dB |
| BS antenna tilt | 6 degrees |
| Horizontal 3 dB beam-width | 65 degrees |
| UE antenna type | Omni (3 dimensional) |
| UE antenna gain | 0 dBi |
| UE antenna height | 1.5 m |
| Max BS transmit power | 46 dBm |
| Max UE transmit power | 23 dBm |
| UE noise figure | 9 dB |
| Uplink Power Control | Pset 1 (from [1] in section 5.1.1.6, table 5.3) |
| Parameters for Micro Deployment |  |
| Model | Manhattan Structure [4] |
| Nr. of city blocks | 8 |
| Block size | 80 m |
| Road width | 20 m |
| Nr. of sites | 32 |
| Nr. of cells per site | 1 cell/site |
| Nr. of active users per cell | 3 |
| Propagation Model | Manhattan Propagation (section 5.1.4.3 [4]) and for detailed modelling [5] |
| Maximum Coupling Gain | -53 dB (i.e. ~= 3m from BS) |
| Max BS transmit power | 35 dBm |
| BS noise figure | 8 dB |
| BS antenna type | ITU-R F1336 Omni, with K=0 and averaged side-lobes |
| BS antenna gain | 6 dBi |
| BS antenna height | 6 m |
| UE Parameters | Same as for macro deployment |
| Uplink Power Control | Pset1 for micro cells [6] Plxile = 105, Gamma = 1 |

Simulation Results

Macro Vs Macro Scenario

The figure below shows the deployment structure for the macro vs macro scenario, where the aggressor and the victim system are off-setted by a distance that is equal to the cell radius.

Figure 3: Macro vs Macro Deplyment scenario

Table 48 shows the average throughput degradation for

- Uplink: when the uplink transmissions of the aggressing system’s UEs cause interference to the uplink transmissions of the victim links.

- Downlink: when the downlink transmissions of the aggressing system’s BS cause interference to the downlink transmissions of the victim links.

Table 48: Uplink and Downlink UE Throughput degradation

| Additional Isolation (dB) | UPLINK |  |  | DOWNLINK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average throughput Degradation | 5% throughput Degradation |  | Average throughput Degradation | 5% throughput Degradation |  |
| -13 | 13.143 % |  | 31.240 % | 9.502 % |  | 52.995 % |
| -8 | 5.704 % |  | 10.941 % | 4.829 % |  | 26.280 % |
| 0 | 0.891 % |  | 1.683 % | 1.263 % |  | 6.406 % |
| 2 | 0.316 % |  | 0.607 % | 0.811 % |  | 3.515 % |
| 7 | 0.185 % |  | 0.185 % | 0.282 % |  | 1.131 % |
| 12 | 0.105 % |  | 0.010 % | 0.093 % |  | 0.650 % |
| 17 | 0.067 % |  | 0.001 % | 0.029 % |  | 0.411 % |

For the UL scenario, the transmit power of the UEs are based on the power control algorithm previously agreed in 3GPP in [1] for macro UEs is illustrated in the following figure:

Figure 4: Uplink transmit power of the UEs

Table 49 articulates the throughput degradation for the average and cell edge UEs. For BS-BS type of interference, the significant need for additional isolation is clearly visible.

Table 49: BS-to-BS scenario, UL throughput degradation

| ACLRl offset X (dB) | BS-to-BS Case (Victim Uplink) |  |
| --- | --- | --- |
|  | Average throughput Degradation | 5% Degradation |
| 0 | 100 % | 100 % |
| 2 | 100 % | 100 % |
| 7 | 100 % | 100 % |
| 12 | 99.927 % | 100 % |
| 17 | 87.548 % | 100 % |
| 22 | 61.755 % | 53.232 % |
| 27 | 35.215 % | 23.355 % |
| 32 | 15.422 % | 8.547 % |
| 37 | 5.577 % | 2.768 % |

Marco Vs Manhattan Micro Scenario

This section imparts the simulation analysis for a macro-micro deployment, where the micro cells are placed in a Manhattan grid (see [4] and [5] for details).

Figure 5: Macro-Micro(Manhattan) deployment scenario

It is to be noted that, for the uplink scenario, the power control of the UEs previously agreed in 3GPP in [11] (for macro UEs) and in [6] (for micro UEs) has been employed and the power transmitted by the UEs is illustrated in the following figure:

Figure 6: Uplink transmit power of the UEs

Macro Aggressor

The results presented in this section are for the case when the macro system is operating as the aggressor and the micro cells placed in the Manhattan grid (as shown in Figure 5) are the victim.

Table 50: Uplink and Downlink UE Throughput degradation

| Additional Isolation (dB) | UPLINK |  |  | DOWNLINK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average throughput Degradation | 5% throughput Degradation |  | Average throughput Degradation | 5% throughput Degradation |  |
| -13 | 19.50 % |  | 30.119 % | 4.096 % |  | 5.892 % |
| -8 | 10.146 % |  | 11.746 % | 1.523 % |  | 2.630 % |
| 0 | 3.022 % |  | 1.900 % | 0.627 % |  | 1.572 % |
| 2 | 2.029 % |  | 1.337 % | 0.168 % |  | 0.0647 % |
| 7 | 0.796 % |  | 0.0407 % | 0.0536 |  | 0.0204 |
| 12 | 0.281 % |  | 0.008 % | 0.0169 % |  | 0.0064 % |
| 17 | 0.092 % |  | 0.0027 % | 0.0053 % |  | 0.002 % |

Micro Aggressor

This section presents the results for the macro-micro scenario where the micro system is operating as the aggressor and the macro system is the victim.

One important thing to note here is that the results contained in Table 51 are for one reference cell in the macro system, which is overlapped completely by the micro (Manhattan) grid (see Figure 5). For the DL, only the UEs in this reference macro cell are considered and for the UL case, the BS of this reference cell is considered for evaluation.

Table 51: Uplink and Downlink UE Throughput degradation

| Additional Isolation (dB) | UPLINK |  |  | DOWNLINK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average throughput Degradation | 5% throughput Degradation |  | Average throughput Degradation | 5% throughput Degradation |  |
| -13 | 1.838 % |  | 0.1991 % | 3.122 % |  | 33.88 % |
| -8 | 0.6703 % |  | 0.0630 % | 1.617 % |  | 31.73 % |
| 0 | 0.3766 % |  | 0.0106 % | 0.468 % |  | 12.278 % |
| 2 | 0.0729 % |  | 0.0063 % | 0.314 % |  | 7.665 % |
| 7 | 0.0232 % |  | 0.0019 % | 0.1168 % |  | 2.558 % |
| 12 | 0.0073 % |  | 0.0006 % | 0.0393 % |  | 0.823 % |
| 17 | 0.0023 % |  | 0.0002 % | 0.0127 % |  | 0.261 % |

Micro (Manhattan) Vs Micro (Manhattan) Scenario

The micro vs micro case governs the scenario where two systems are being operated in a Manhattan structure as shown in Figure 5 (the dots in the figure represent BS). The propagation models employed are well known in the prior art (including [4] and [5]).

Figure 7: Micro Vs Micro (Manhattan) deployment scenario

The transmit powers for the UL scenario are illustrated in Figure 6.

Figure 8: Transmit power for the UL scenario

Table 52: Uplink and Downlink UE Throughput degradation

| Additional Isolation (dB) | UPLINK |  |  | DOWNLINK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average throughput Degradation | 5% throughput Degradation |  | Average throughput Degradation | 5% throughput Degradation |  |
| -13 | 3.193 % |  | 1.277 % | 2.159 % |  | 6.210 % |
| -8 | 1.299 % |  | 0.445 % | 0.763 % |  | 2.093 % |
| 0 | 0.289 % |  | 0.142 % | 0.138 % |  | 0.242 % |
| 2 | 0.182 % |  | 0.084 % | 0.0828 % |  | 0.188 % |
| 7 | 0.062 % |  | 0.026 % | 0.0264 % |  | 0.102 % |
| 12 | 0.020 % |  | 0.008 % | 0.0083 % |  | 0.101 % |
| 17 | 0.006 % |  | 0.002 % | 0.0026 % |  | 0.084 % |

Intra-MFCN interference analysis – simulation set 2  
[Editor’s note: the simualtion results from the eco have to be added to this annex]

UE BEM DISCUSSION FROM CEPT REPORT 39

The following is an excerpt from CEPT Report 39 [reference], Section 2.4, discussing the BEM in relation to ETSI harmonized standards.

Technical conditions applying to terminal equipment

Another concern is about the management within the EU of interference between terminals. Since they are not included in the relevant EC decisions, CEPT assumes that these conditions have to be taken into account with care when developing harmonised standards by ETSI. There may be an issue because within the EU, mobile terminals are generally exempted from individual licensing and also because network operators are required to connect terminal stations having an appropriate interface and meeting the essential requirements of Article 3 of the R&TTE Directive [reference] (in the context of spectrum masks, the relevant provision is Article 3.2, relating to harmful interference). To ensure that interference between terminals is managed effectively it is therefore extremely important that ETSI takes account of relevant ECC work on WAPECS bands – amending their harmonised standards as necessary. It has to be noted that some administrations assume that interference between terminals will be successfully handled by ensuring conformity to the R&TTE Directive – if ETSI does not take this issue into account in the development of harmonised standards then this may not be a safe assumption.

The R&TTE Directive relates to both placing equipment on the market and putting it into service. In the past, there has generally been a one-to-one correspondence between harmonized standard, application/technology and frequency band (i.e., one applicable harmonized standard for an application or technology in a particular frequency band), and the national measures for license exemption have almost always been based on this standard. In other words, the spectrum emission mask for the terminal relative to the nominal channel edge will be the same as the block edge mask relative to the block edge, or more stringent.

However, this one-to-one correspondence may not necessarily apply under the WAPECS concept. There might be different criteria for putting equipment into service, associated with different operational restrictions. Without the appropriate directions given in the harmonised standards to ensure compliance, this could lead to a non-compliance with the CEPT sharing criteria. Therefore it is important to ensure that the development of harmonised standards takes account of the sharing criteria developed by CEPT for terminals in order to avoid such non-compliances.

- Only few administrations referred to additional technical conditions for terminal equipment on the basis of CEPT or ECC reports.

- One administration refers explicitly to these technical conditions even in the licensing process.

This is clearly an area for which the RSPG opinion on streamlining is particularly relevant. CEPT should cooperate with ETSI to ensure that development of harmonised standards will include instructions on how the CEPT sharing criteria can be met by equipment.”

mfcn – fss co-existence

SUMMARY OF FSS CO-EXISTENCE ANALYSIS IN ECC REPORT 100

The following is a summary of Section 5.4 of ECC Report 100 [17].

The BWA system characteristics in the analysis are as follows (Table 5.4.1 of Report 100):

Table 53: Basic BWA characteristics used for the sharing with FSS

|  | BWA CS |  | BWA TS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CS-1 (critical case) | CS-2 (typical) | TS-1 (critical case) | TS-2 (typical) |  | TS-3 (“Omni”) |
| TX peak output power (dBm) | 43 (for nomadic) | 35 | 30 |  | 22 | 20 |
| channel bandwidth (MHz) | 7 | 7 | 7 |  | 7 | 7 |
| feeder loss (dB) | 1 | 1 | 1 |  | 1 | 1 |
| Power control (dB) | 0 | 0 | 0-30 dB (12 dB) |  | 0-30 dB (12 dB) | 0-30 dB (12 dB) |
| peak antenna gain (dBi) | 17 | 17 | 20 |  | 10 | 0 |
| antenna gain pattern | Rec. ITU-R F.1336, | Rec. ITU-R F.1336, | Rec. ITU-R F.1336 |  | Rec. ITU-R F.1336 | Omni |
| antenna elevation (deg) | 0 | 0 | 0 |  | 0 | 0 |
| antenna height a.g.l. (m) | 50 | 30 | 20 |  | 10 | 1.5 |
| noise figure (dB) | 5 | 5 | 7 |  | 7 | 7 |
| receiver noise in reference bandwidth of 4 kHz (dBW) | -163.0 | -163.0 | -161.0 |  | -161.0 | -161.0 |
| Number of co-channel TSs per CS | n/a | n/a | 16 with 25% activity factor |  | 16 with 25% activity factor | 16 with 25% activity factor |

The characteristics are not identical to those expected for the MFCN networks studied, but similar enough to make the results relevant. CS-1 bandwidth is 7 MHz and the antenna height is 50 m instead of 30 m, but is otherwise the same as for a MFCN base station according to Table Y. CS-2 also has 7 MHz bandwidth and 35 dBm output power, but is otherwise the same as a MFCN base station. As for the TSs, the bandwidth and TX output power is slightly lower than for a MFCN UE, but the distance is very small.

Earth Station parameters for six different types are presented in Table 54 (Table 5.4.2 from ECC Report 100[17]).

Table 54: ES parameters

|  | ST-1 | ST2 | ST3 | ST4 | ST5 | ST-6 |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna Diameter (m) | 4.5 | 4.5 | 8 | 8 | 32 | 32 |
| Gain (dBi) | 42.6 | 42.6 | 47.7 | 47.7 | 59.8 | 59.8 |
| Antenna Diagram | ITU-R S.465 | ITU-R S.465 | ITU-R S.465 | ITU-R S.465 | ITU-R S.465 | ITU-R S.465 |
| Antenna Height (m) | 3 | 3 | 5 | 5 | 25 | 25 |
| Noise temperature (K) | 70 | 70 | 82 | 82 | 70 | 70 |
| Elevation angle (°) | 4 | 33 | 4 | 33 | 4 | 33 |
| Azimuth (°) | 104 | 190 | 104 | 190 | 104 | 190 |

Interference from BWA CSs into FSS ES receivers is summarized in Table 55 (Table 5.4.3 of Report 100). The results are expressed as mitigation distances, “which is defined as the geographical area delimited by the distance on a given azimuth and elevation from an ES, sharing the same frequency band with terrestrial stations, within which there is a potential for the level of permissible interference to be exceeded and co-ordination is necessary to ensure successful operation between terrestrial stations and ES.”

The results are for co-channel interference, from a single MFCN BS, and for a “generic scenario” without terrain profile included in the propagation calculations. The separation distances correspond to I/N values no lower than -10 dB for 20% of the time. No short-term interference has been considered here. For such an analysis a terrain model must be incorporated (see further below)

Based on the comparison between the BWA parameters in this study and those expected for MFCN BSs, mitigation distances for MFCN can be expected to be somewhere in-between those of CS-1 and CS-2. Note that in reality operation of BWA stations within the mitigation distances may be possible due to the influence of the terrain and clutter.

Table 55: Summary of mitigation distances

| Type of FSS ES | Interfering BWA station CS-1 | Interfering BWA station CS-2 |
| --- | --- | --- |
|  | Distance (km) | Distance (km) |
| ST 1 | 122 | 71 |
| ST 2 | 53 | 43 |
| ST 3 | 119 | 68 |
| ST 4 | 55 | 44 |
| ST 5 | 128 | 76 |
| ST 6 | 67 | 56 |

Sensitivity to variations in three different parameters are presented in Figure 9 (Figure 5.4.4 of Report 100 [17]): off-axis angle, elevation angle and ES antenna diameter. Off-axis angle and elevation angle of the ES may influence mitigation distances considerably.

Figure 9: Influence of the FSS ES and BWA CS parameters on the mitigation area

Examples of ES co-existence based on propagation with terrain profile and incorporating short-term interference are also provided. The parameters and results from two of those are presented in Tables X and Y (from Section 5.4.2.4 of Report 100 [17]).

Table 56: Details of two combined ES sites used in detailed analysis

| Brookmans Park |  |
| --- | --- |
| Location | N51:43:44, W0:10:39 |
| Antenna height a.g.l. (m) | 5 |
| Antenna gain (dBi) | 47.7 |
| Antenna elevation (deg) | 31 |
| Antenna azimuth (deg) | 180 |
| Delta N | 45 |
| Goonhilly |  |
| Location | N50:02:55, W5:10:46 |
| Antenna height a.g.l. (m) | 25 |
| Antenna gain (dBi) | 59.8 |
| Antenna elevation (deg) | 32 |
| Antenna azimuth (deg) | 173 |
| Delta N | 45 |

Table 57: Maximum mitigation distances (in km) required to protect site specific FSS ES receivers without the additional clutter loss

| Type of interfering BWA/BWA station | FSS ES Antenna 8 m diameter (47.7 dBi gain) at Brookmans Park |  |  | FSS ES Antenna1 32 m diameter (59.8 dBi gain) at Goonhilly |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Long Term Propagation | Short Term Propagation | Maximum mitigation distance | Long Term Propagation | Short Term Propagation | Maximum mitigation distance |
| CS-1 | 100 | 3002 | 300 | 115 | 3202 | 3202 |
| CS-2 | 80 | 2252 | 2252 | 100 | 2702 | 2702 |

Aggregation of interference to ESs by multiple base stations has also been studied, with the result that depending on BWA deployment, the increase in distance may be between 15 and 25%.

The analysis of BWA TS interference to ESs show that in all cases a co-ordination between the CS and the ES is sufficient to protect the FSS ES from both the BWA CS and the BWA TS, due to the considerably shorter separation distance required for TSs.

Two types of adjacent band interference mechanisms were studied, unwanted emissions from BWA stations and saturation of ES LNBs, assuming that they have been made to receive in the entire 3400 – 4200 MHz band. Separation distances due to the first type of interference are summarized in Table X (Table 5.4.10 of Report 100) and those due to the second in Tables Y and Z (Tables 5.4.11 and 5.4.12 of Report 100).

Table 58: Summary of required separation distance between BWA CS or TS and FSS ES

|  |  | Required Separation Distance (km) |  |
| --- | --- | --- | --- |
| Type of BWA Station | FSS ES antenna off-axis angle |  |  |
| CS-1 and CS-2 | 5° | 1.087-4.33 |  |
|  | 15° | 0.277-1.1 |  |
|  | 30° | 0.117-0.464 |  |
| TS-1 | 5° | 13.7 |  |
|  | 15° | 3.48 |  |
|  | 30° | 1.47 |  |
| TS-2 (Indoor) (Note1) | 5° | 0.77 |  |
|  | 15° | 0.196 |  |
|  | 30° | 0.083 |  |
| TS-3 (Mobile) | 5° | 1.37 |  |
|  | 15° | 0.348 |  |
|  | 30° | 0.147 |  |

Table 59: Required separation distance between BWA CS and FSS ES to avoid LNB saturation

|  | CS-1 |  |  | CS-2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Arrival angle of BWA signal at FSS E/S | 5 | 15 | 30 | 5 | 15 | 30 |
| FSS E/S antenna off-axis gain (dBi)1 | 14.5 | 2.6 | -4.9 | 14.5 | 2.6 | -4.9 |
| BWA e.i.r.p. (dBm) | 60 |  |  | 52 |  |  |
| LNB Saturation Level (dBm) | 50 |  |  |  |  |  |
| Excess over LNB Saturation Level (dB) | 124.5 | 112.6 | 105.1 | 116.5 | 104.6 | 97.1 |
| Frequency (MHz) | 3700 |  |  |  |  |  |
| Required Separation Distance (km) | 10.89 | 2.76 | 1.16 | 4.33 | 1.10 | 0.46 |

Table 60: Required separation distance between BWA TS and FSS ES to avoid LNB saturation

|  | TS-1 |  |  | TS-2 (Indoor)2 |  |  | TS-3 (Mobile) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arrival angle of BWA signal at FSS E/S | 5 | 15 | 30 | 5 | 15 | 30 | 5 | 15 | 30 |
| FSS E/S antenna off-axis gain (dBi)1 | 14.5 | 2.6 | -4.9 | 14.5 | 2.6 | -4.9 | 14.5 | 2.6 | -4.9 |
| BWA e.i.r.p. (dBm) | 50 |  |  | 32 |  |  | 20 |  |  |
| LNB Saturation Level (dBm) | 50 |  |  |  |  |  |  |  |  |
| Excess over LNB Saturation Level (dB) | 114.5 | 102.6 | 95.1 | 96.5 | 84.6 | 77.1 | 84.5 | 72.6 | 65.1 |
| Frequency (MHz) | 3700 |  |  |  |  |  |  |  |  |
| Required Separation Distance (km) | 3.44 | 0.87 | 0.37 | 0.43 | 0.11 | 0.05 | 0.11 | 0.03 | 0.01 |

Interference from FSS spacecraft into BWA stations may exceed the required interference criterion by a few dB in few cases however the probability of such cases is expected to be low.

Summary of FSS co-existence analysis in ITU-R Report M.2109

This Report provides a summary of the sharing studies between IMT-Advanced systems and geostationary satellite networks in the fixed-satellite service (FSS) in the 3 400-4 200 and 4 500-4 800 MHz frequency bands.

Table 61 below contains the FSS parameters used in the analysis. In addition, the following parameters were used:

- Antenna diameters: 2.4 m and 11m (feeder link).

- Antenna heights: 30 m (urban case) and 3m (rural case).

Table 61: Typical downlink FSS parameters in the 4 GHz band

| Parameter |  | Typical value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Range of operating frequencies |  | 3 400-4 200 MHz, 4 500-4 800 MHz |  |  |  |  |  |  |  |
| Earth station off-axis gain towards the local horizon (dBi)(1) |  | Elevation Angle(2) |  | 5° | 10° | 20° | 30° | 48° | >85° |
|  |  | Off-axis gain |  | 14.5 | 7.0 | –0.5 | –4.9 | –10 | 0 |
| Antenna reference pattern |  | Recommendation ITU-R S.465 (up to 85°) |  |  |  |  |  |  |  |
| Range of emission bandwidths |  | 40 kHz – 72 MHz |  |  |  |  |  |  |  |
| Receiving system noise temperature |  | 100 K |  |  |  |  |  |  |  |
| Earth station deployment |  | All regions, in all locations (rural, semi-urban, urban) (3) |  |  |  |  |  |  |  |
| 1) The values were derived by assuming a local horizon at 0° of elevation. (2) 5° is considered as the minimum operational elevation angle. (3) FSS antennas in this band may be deployed in a variety of environments. Smaller antennas (1.8 m-3.8 m) are commonly deployed on the roofs of buildings or on the ground in urban, semi-urban or rural locations, whereas larger antennas are typically mounted on the ground and deployed in semi-urban or rural locations. |  |  |  |  |  |  |  |  |  |

## IMT In-band parameters

Table 62 contains the IMT-Advanced parameters used in the analysis.

Table 62: IMT-Advanced base station parameters

| Parameter | Value | Value considered in the simulations |
| --- | --- | --- |
| e.i.r.p. density range: macro base station scaled to 1 MHz bandwidth | 39 to 46 dBm/MHz | 46 dBm/MHz |
| e.i.r.p. density range: micro base station scaled to 1 MHz bandwidth | 15 to 22 dBm/MHz | 22 dBm/MHz |
| Maximum e.i.r.p.(1) (Transmitter output power + antenna gain – feeder loss) | 59 dBm (macro base station) 35 dBm (micro base station) |  |
| Antenna type (Tx/Rx) (the gain is assumed to be flat within one sector) | Sectored for macrocell omni for microcell |  |
| Receiver thermal noise (including noise figure) | –109 dBm/MHz |  |
| Protection criterion (I/N) interference to individual base station | –6 dB or –10 dB(2) |  |
| Protection criterion (I/N) vs satellite systems | –10 dB |  |
| (1) e.i.r.p. range of values assume range of frequency bandwidth between 20 and 100 MHz. (2) This value has to be used when assessing compatibility between a non primary allocated system and a primary allocated system (e.g. between UWB and IMT-Advanced). |  |  |

Table 63: IMT-Advanced mobile station parameters

| Parameter | Value | Value to be considered in the simulations |  |
| --- | --- | --- | --- |
| Maximum Tx PSD range output power(1) | 4 to 11 dBm/MHz | 7.5(2) dBm/MHz |  |
| Maximum e.i.r.p. | 24 dBm |  |  |
| Receiver thermal noise (dBm/MHz) (Including noise figure) | –109 to –105 dBm/MHz |  |  |
| Protection criterion (I/N) | –6 dB |  |  |
| (1) With reference signal bandwidth between 20 and 100 MHz. (2) A median value is selected considering the effect of automatic transmit power control (ATPC). |  |  |  |

Table 64: IMT-Advanced network parameters

| Parameter | Value |
| --- | --- |
| Macro cell antenna gain | 20 dBi |
| Micro cell antenna gain | 5 dBi |
| Macro cell feeder loss | 4 dB |
| Micro cell feeder loss | 0 dB |
| Antenna pattern for vertical sharing | Rec. ITU-R F.1336(1) |
| Mobile station antenna gain | 0 dBi |
| Base station Antenna downtilt (Micro) | 0 degree |
| Base station Antenna downtilt (Macro) | 2 degrees |
| Base station antenna height (Micro) | 5 m |
| Base station antenna height (Macro) | 30 m |
| Mobile station antenna height (mobile station) | 1.5 m |
| Intersite distance (Micro) | 600 m |
| Intersite distance (Macro) | 5 km |
| Intersite distance (Macro) for urban case | 1,5 km |
| Active users density (Dense Urban/Macro) | 18/km² |
| Active users density (Dense Urban/Micro) | 115/km² |
| Active users density (Suburban/Macro) | 15/km² |
| Active users density (Suburban /Micro) | 19/km² |
| Frequency reuse pattern | 1(2) and 6(3) |

(1) Recommendation ITU-R F.1336 [16] has generally been used in the studies. However, STUDY 2 of this report has used the Recommendation ITU-R F.1336-2 (see § 8.2.2). (2) The same frequency is used by all sectors. (3) Except STUDY 6 in § 8.1.2, all the other studies have only applied 1.

## IMT Out-of-band parameters

The following values were assumed to define the spectrum mask, valid for the bandwidths between 20 MHz and 100 MHz, where the 3rd adjacent channel and above has been calculated based on spurious emission:

Table 65: IMT-Advanced out-of-band parameters

| Offset | ACLR limit |
| --- | --- |
| 1st adjacent channel | 45 dB |
| 2nd adjacent channel | 50 dB |
| 3rd adjacent channel and above | 66 dB |

## Results

11 different studies were carried out with varying assumptions on propagation, single vs aggregate interference and compliance with FSS and IMT parameters above. Table 65 presents the results in terms of required separation distances for both long-term and short-term interference for the case of flat terrain (generic study). Upper and lower bounds are provided, based on the different studies. The differences in results depend on assumptions about FSS ES antenna elevation angles, propagation models, interference apportionment, BS downtilt, etc.

Analysis was also carried out for specific cases, i.e. with terrain information included in the propagation calculations. The results are similar to those for the generic case, but, as expected, with a somewhat higher variance in separation distances, as terrain may both shelter from interference and reduce the propagation loss.

Table 66: Separation distances (km) for generic (flat terrain) interference analysis

|  | Macro BS | Micro BS | Mobile Station |
| --- | --- | --- | --- |
| Co-channel Long-term Single interferer | 33-70 | 15-50 | 0 – 1.5 |
| Co-channel Long-term Aggregate interference | 51-61 | 46-58 | 0-1.5 |
| Co-channel Short-term Single interferer | 34-430 | N.A. | 1.5 |
| Adjacent channel Long-term Single interferer | 0.07-80 | 2-51 | 0.5-32.5 |
| Adjacent channel Long-term Aggregate interference | 0.35-45 | 4-35 | N.A. |
| LNA/LNB saturation Long-term Single interferer | 10-30 | 0.6-2 | 0.17-0.55 |

Different mitigation techniques were also investigated:

- Sector disabling. One way to reduce the transmitting output power level could be to disable the antenna sector that points towards the FSS earth station, noting that such an area would be covered through the use of other frequency bands by IMT-Advanced systems. Compared with normal full active sector mode, the application of this mitigation technique has shown that the separation distance ranges are reduced by between 0 and 49% in generic studies (without terrain horizon profile) and between 0 and 83% for one specific site (with terrain horizon profile).

- MIMO. By using this technique, a gain reduction in the base station transmit antenna diagram is generated towards the interfered FSS earth station. By using the MIMO technique, the minimum separation distance is 35 m in case of an IMT-Advanced base station and single FSS receiving earth station under the assumption of 0o direction of earth station (DOE) estimation error which implies that null beam to the FSS receiving earth station is formulated perfectly. In the case of an IMT-Advanced base station and 3 FSS receiving earth stations, the minimum separation distance increases up to 3.5 km under the same assumptions. Other results have shown that under the assumption of 8° DOE estimation error, the minimum separation distances is 22 km, but this still reduces the minimum separation distance by approximately 50% in the considered case.As for the sector disabling technique, this approach would require the use of other frequencies to cover the area where the base transmit antenna gain is reduced.

- Site shielding. In Recommendation ITU-R SF.1486 [25] , interference attenuation effect, in a range about 30 dB, due to the site shielding isolation obtained by providing physical or natural shielding at the FSS earth stations is described. If such shielding isolation is taken into account, the required separation distance to protect FSS earth station receivers from IMT-Advanced transmitters can be reduced. However, the required distance separation between IMT-Advanced transmitter and a FSS receiving earth station using site shielding has to be evaluated on a site-by-site basis and is dependent on characteristics and location of each site. The possibility of applying site shielding may not be guaranteed for all sites.

- Antenna downtilting. A possible mitigation technique to improve sharing is antenna downtilting at the IMT-Advanced base stations. One study shows that for one specific site in urban macro environment, the required separation distance is decreased by approximately 30% and 50% for the long-term and short-term interference criteria, respectively, when the antenna downtilt at IMT-Advanced transmitter is changed from 2° to 7°. However, the impact of this technique may vary for different locations and results may be different at other locations.

- Dynamic spectrum allocation. If information can be made available to IMT-advanced networks what FSS channels are used at a specific point in time, free spectrum may be used dynamically. This may be achieved with a database that is updated dynamically.

- Usage of beacon: A beacon that is transmitted from the FSS earth station locations may provide dynamic information on its spectrum usage, and could thus provide information to IMT-Advanced systems on unused spectrum.

With respect to co-channel interference from FSS into IMT-Advanced, studies have provided a range of margins relative to the required I/N criterion (from 9 to –11 dB) depending on the assumptions (particularly the type of IMT-Advanced base station considered and the FSS space station e.i.r.p. density). As a result, the IMT-Advanced base and mobile stations may experience interference from emissions of authorized satellite networks.

coexistence between MFCN and RADIOLOCATION services

The sections below summarize results obtained in previous studies of ECC and ITU-R related to adjacent band co-existence between MFCN/BWA above 3400 MHz and Radiolocation, which in Region 1 has a primary allocation in 3300 – 3400 MHz.

According to the EFIS database, the Radiolocation band below 3400 MHz is used for military and civil (including airborne) Radiolocation. Furthermore it may be used for meteorological purposes, although there is no allocation for that in the Radio Regulations. Although the radar and MFCN parameters may not be identical to what was assumed in the studies below, the results should give a good overall view of co-existence characteristics between MFCN networks and the Radiolocation service.

Summary of Radiolocation co-existence analysis in ECC Report 100

The following is a summary of Section 5.5 and Annexes 6 and 7 of ECC Report 100.

For the purpose of studies, representative characteristics of radar systems can be found in Recommendation ITU-R M.1465 [26] “Characteristics of, and protection criteria for radars operating in the radiodetermination service in the frequency band 3 100-3 700 MHz”. These typical characteristics are provided in the Table 67 below.

Table 67: Table of characteristics of radiolocation systems in the band 3 100-3 700 MHz

| Param | Land-based systems |  |  | Ship systems |  |  | Airborne system |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A |  | B |  | A |  |  | A |  |
| Use | Surface and air search |  | Surface search |  | Surface and air search |  |  | Surface and air search |  |
| Modulation | P0N/Q3N |  | P0N |  | P0N | Q7N |  | Q7N |  |
| Tuning range (GHz) | 3.1-3.7 |  |  |  | 3.5-3.7 | 3.1-3.5 |  | 3.1-3.7 |  |
| TX power into antenna (kW) (Peak) | 640 | 1000 |  |  | 850 | 4000 |  | 1000 |  |
| Pulse width (s) | 160-1000 | 1.0-15 |  |  | 0.25, 0.6 | 6.4-51.2 |  | 1.25(1) |  |
| Repetition rate (kHz) | 0.020-2 | 0.536 |  |  | 1.125 | 0.152-6.0 |  | 2 |  |
| Compression ratio | 48000 | Not applicable |  |  | Not applicable | 64-512 |  | 250 |  |
| Type of compression | Not applicable | Not applicable |  |  | Not applicable | CPFSK |  | Not applicable |  |
| Duty cycle (%) | 2-32 | 0.005-0.8 |  |  | 0.28, 0.67 | 0.8-2.0 |  | 5 |  |
| TX bandwidth (MHz) (–3 dB) | 25/300 | 2 |  |  | 4, 16.6 | 4 |  | > 30 |  |
| Antenna gain | 39 | 40 |  |  | 32 | 42 |  | 40 |  |
| Antenna type | Parabolic |  |  |  | Parabolic | PA |  | SWA |  |
| Beamwidth (H,V) (degrees) | 1.72 | 1.05, 2.2 |  |  | 5.8, 4.5 | 1.7, 1.7 |  | 1.2, 3.5 |  |
| Vertical scan type | Not applicable | Not applicable |  |  | Not applicable | Random |  | Not applicable |  |
| Maximum vertical scan (degrees) | 93.5 | Not applicable |  |  | Not applicable | 90 |  |  60 |  |
| Vertical scan rate (degrees/s) | 15 | Not applicable |  |  | Not applicable |  |  | Not available |  |
| Horizontal scan type | Not applicable | Rotating |  |  | Rotating | Random |  | Rotating |  |
| Maximum horizontal scan (degrees) | 360 |  |  |  | 360 |  |  | 360 |  |
| Horizontal scan rate (degrees/s) | 15 | 25.7 |  |  | 24 | Not applicable |  | 36 |  |
| Polarization | RHCP | V |  |  | H | V |  | Not available |  |
| Rx sensitivity (dBm) | Not available | -112 |  |  | -112 | Not available |  | Not available |  |
| S/N criteria (dB) | Not available | 0 |  |  | 14 | Not available |  | Not available |  |
| Rx noise figure (dB) | 3.1 | Not available |  |  | 3 | Not available |  | 3 |  |
| Rx RF bandwidth (MHz) (–3 dB) | Not available | 2.0 |  |  | Not available |  |  | Not available |  |
| Rx IF bandwidth (MHz) (–3 dB) | 380 | 0.67 |  |  | 8 | Matched to emission |  | 1 |  |
| Deployment area (1 000 km2) | 32 | 1468 |  |  | 188 | 511 |  | Worldwide |  |
| Number of systems per area | 1 | 6 |  |  | 1-2 | 7 |  | 36 |  |
| (1) 100 ns compressed. CPFSK: continuous-compression FSK; PA: phased array; SWA: slotted waveguide array |  |  |  |  |  |  |  |  |

This study includes the assessment on the impact from radar systems operating below 3.4 GHz on BWA operating in the band 3400-3800 MHz. The results are from a detailed case study that represents a specific case of co-existence of radars vs. BWA, summarized below.

The main results of the studies are:

- From the co-ordination study results it appears that the installation of BWA systems closer than ca. 5 km from the radar should be coordinated;

- In order to guarantee a limited C/I degradation of the P-MP BWA system, it is necessary to establish a protection distance of approximately 11 km in some areas (this value may be much less in some directions);

- Considering the degradation for blocking effect, the radar can have impact in the BWA systems until 30 km (this value may be much less in some directions).

A radar system radiates directional beams and, for instance, a victim BWA CS in a rotation period of the radar will only be affected x percentage of time. This probability was not considered in the main studies and in this manner the minimum separation distances obtained between the systems are somewhat pessimistic.

Separate measurements of continuous versus intermittent interference indicate that radar pulses cause less considerably less damage than a continuous wave interference with the same power.

From the various discussions in this issue it is clear that the principal way for assuring co-existence of radars vs. BWA is the co-ordination on a case-by-case basis, but then some additional (generic) case studies could be used to illustrate the extent of the problem.

Summary of Radiolocation co-existence analysis in ITU-R Report M.2111

The scope of this study is co-existence between IMT-Advanced and Radiolocation, using the same band, 3.4 – 3.7 GHz. Adjacent channel analysis is carried out, providing results that are relevant for the scenario with Radiolocation MFCN in adjacent bands, below and above 3.4 GHz.

IMT-Advanced parameters

Major parameters such as antenna gains and heights are based on Report ITU-R M.2039, and the required parameters for calculation of aggregated path loss, such as deployment density at each zone, are introduced and listed in Table 68. Mobile terminal parameters are listed in Table 69. [editor’s note: the table numbers and the text have to be adjusted accordingly in this section]

Table 68: IMT-Advanced base station parameters

| Attribute | Value |  |
| --- | --- | --- |
|  | Macro cell | Micro cell |
| Cell size (radius) (m) | Suburban 2 000(1) Rural 3 000(1) | Urban 1 000(1) |
| Base station density for aggregate interference calculation (km2) | Suburban 0.08(1) Rural 0.035(1) Airborne radar: 0.052(1) | Urban 0.32(1) |
| Transmission bandwidth (MHz) | 25 | 25 |
| Transmitter power (dBm) | 43 | 38 |
| Transmission spectrum density (dBm/MHz) | 29 | 24 |
| Antenna gain (dBi) | 17 | 5 12(2) |
| Cell configuration | 120° sector | 120° sector |
| Antenna height (M) | 30 | 10 20(2) |
| Tilt of antenna (degree down) | 2.5 7(2) | 0 20(2) |
| Receiver noise figure (dB) | 5(1) | 5(1) |
| Allowable interference level (I/N = –6 dB) (dBm/MHz) | –115 | –115 |
| OOB emission level (dBm/MHz) | –17(3) | –17(3) |

NOTE 1 – Pico cell was not used in this assessment because Pico cell is usually used as an indoor solution and it is not expected to cause significant outdoor interference due to building penetration loss. (1) Parameters for aggregated interference assessment. (2) Includes optimization. (3) With regard to OOB emission level, additional attenuation of 10 dB is assumed.

Table 69: IMT-Advanced mobile terminal parameters

| Attribute | Value |
| --- | --- |
| Typical transmission spectrum density (dBm/MHz) | 13 |
| Antenna gain (dBi) | 0 |
| Antenna height (m) | 1.5 |
| Receiver noise figure (dB) | 9 |
| Allowable interference level (Primary to primary or secondary to secondary I/N = –6 dB) (dBm/MHz) | –113 |
| OOB emission level (dBm/MHz) | –17 |

Radiolocation Parameters

Recommendation ITU-R M.1465 [26] – Characteristics of and protection criteria for radars operating in the radiodetermination service in the frequency band 3 100-3 700 MHz, contains technical characteristics of radar systems. Radar parameters are listed in Table 70.

Table 70: Radar parameters

| Attribute | Value |  |  |
| --- | --- | --- | --- |
|  | Land-based radar B | Shipborne radar A | Airborne radar |
| Tuning range (GHz) | 3.1 ~ 3.7 | 3.1 ~ 3.5 | 3.1 ~ 3.7 |
| Tx power into antenna (peak) (MW) | 1 | 0.85 | 1 |
| Antenna gain (dBi) | 40 | 32 | 40 |
| Antenna type | Parabolic | Parabolic | SWA |
| Beamwidth (H,V) (degree) | 1.05, 2.2 | 1.5/5.8 ~ 45 | 1.2, 3.5 |
| Horizontal scan type | Rotating | Rotating | Rotating |
| Maximum vertical scan (degree) | Not applicable | Not applicable | ± 60 |
| Antenna height (m) | 10 | 30 | >7 000 |
| Receiver IF bandwidth (MHz) | 0.67 | 8 | 1 |
| Receiver noise figure (dB) | Not available | 3 | 3 |
| Estimated allowable interference level (I/N = –6 dB) (dBm/MHz) | –117 | –117 | –117 |
|  | Land-based radar B | Shipborne radar A | Airborne radar |
| Deployment area (1 000 km2) | 1 468 | 188 | Worldwide |
| Number of systems per area (Integer) | 6 | 1-2 | 36 |

NOTE 1 – Total deployment area of all radars excluding airborne radar is 2 199 000 km2. It takes only 0.4% of the total earth surface. This deployment density was based upon a previous version of Recommendation ITU-R M.1465 however the in force version does not provide the information to derive the conclusion of 0.4%. NOTE 2 – Line of sight distance between airborne radar and macro base station antenna is 365 km. Total deployment area including the interfering area to the airborne radar would be at most 3% of the total earth surface when all radars listed in Recommendation ITU-R M.1465 [26] are activated simultaneously. This deployment density was based upon a previous version of Recommendation ITU-R M.1465 however the in force version does not provide the information to derive the conclusion of 3%.

Since both Recommendations ITU-R M.1461 [28] and ITU-R M.1465 [26] note that signal from other service resulting in an I/N ratio of –6 dB or below is acceptable to the radar systems, an I/N of is used for the protection criteria for the radars analysed.

Antenna radiation pattern estimation

ITU-R Recommendations which describe the antenna radiation patterns used in this assessment are listed in Table 71.

Because Recommendation ITU-R M.1465 [26] defines only technical characteristics of radar systems, and there is no existing radar antenna reference pattern currently available in ITU-R, the pattern in Recommendation ITU-R M.1652 [27], Annex 6, Appendix 1 is used in this analysis.

Table 71: ITU-R Recommendations for antenna pattern estimation

| Antenna type | RPE referenced Rec. |
| --- | --- |
| IMT-Advanced base station sector antenna | F.1336-1, K = 0 Sector |
| IMT-Advanced mobile terminal antenna | F.1336-1, K = 0 Omni |
| Land-based radar B parabolic | M.1652, Annex 6, Appendix 1 |
| Shipborne radar A fan beam | M.1652, Annex 6, Appendix 1 |
| Airborne radar SWA antenna | M.1652, Annex 6, Appendix 1 |

Table 72 lists required separation distances for adjacent channel interference scenarios where IMT-Advanced is interfering radars. OOB emission levels listed in Table 68 and Table 69 were used.

Table 72: Separation distances required to protect radar receivers for adjacent channel interference

| Transmitting | Required separation horizon distance R0 (km) |  |  |
| --- | --- | --- | --- |
|  | Land-based radar B | Shipborne radar A | Airborne radar |
| Base station M.2039 Antenna | 3.3 | 1.1 | 0 |
| Antenna tilt etc. | 1.4 | <1 | 0 |
| Mobile terminal | <1 | <1 | 0 |

Another set of results provides information on required frequency separation between radar and IMT-Advanced channels, given a certain distance separation. Assuming a 5 km separation distance and a 25 MHz IMT-Advanced channel, this frequency separation is 14 – 21 MHz for the airborne radar studied, and 28 – 65 MHz for the shipborne radars. It should be noted that the assumptions in this ITU-R Report about adjacent channel performance for IMT-Advanced differs considerably from the specifications of e.g. LTE, and that the results thus may be pessimistic.

The analysis for radar interference to IMT-Advanced equipment does not incorporate the aspect of intermittent radar interference. Furthermore the IMT-Advanced characteristics are not up to date. The results are thus omitted here.

A number of different mitigation techniques were also studied. The technique that would be most relevant for the scenario with adjacent band interference would be additional filtering to improve receiver performance and decrease unwanted emissions. The possible improvements are not quantified in the report.

Conclusion

Sharing studies between IMT-Advanced and different radars assuming non-overlapping adjacent channel analysis, with IMT-Advanced unwanted emissions of -17 dBm/MHz, the following holds:

- For airborne radar the required separation distance is approximately 0 km, depending on the radar type and antenna type.

- For land-based/shipborne radar the required separation distance is less than 1 km, depending on the radar type and antenna type.

The frequency separation analyses concluded that for IMT-Advanced interference to radars, the frequency separation varies between 14 and 65 MHz, depending on radar type and scenario.

There are mitigation techniques which can reduce the separation distance or frequency separation required. In particular, for adjacent channel/adjacent band interference, improved receiver performance and decreased unwanted emissions can be efficient.

Summary of Aeronautical radionavigation … and met radar co-existence analysis in ECC Report 174

[Editor’s note: Radiolocation is not the same as ARNS, but met radar may exist in Europe. Check inclusion of the results into the report]

List of reference

- ECC/REC/(04)05  on “Guidelines for accommodation and assignment of Multipoint Fixed Wireless systems in frequency bands 3.4 - 3.6 GHz and 3.6 - 3.8 GHz”

ECC/DEC/(07)02 on “availability of frequency bands between 3400-3800 MHz for the harmonised implementation of Broadband Wireless Access systems (BWA)”

Commission Decision 2008/411/EC (on the harmonisation of the 3 400-3 800 MHz frequency band for terrestrial systems capable of providing electronic communications services in the Community)

ECC/DEC/(11)06 which contains the harmonised frequency arrangements for MFCN systems including IMT for 3.4 – 3.6 and 3.6 – 3.8 GHz

3GPP standard 36.104 (Version XXX-XX)

3GPP TR 36.942

Report  ITU-R  M.2135-1, “Guidelines for evaluation of radio interface technologies for IMT-Advanced”

Recommendation ITU-R F.1336-3

3GPP TR 25.942

IEEE: A recursive method for street micro cell path loss calculation, Jan-Erik Berg, Ericsson

3GPP TR 37.809

WMF-T23-005-R015v05 (Releases 1.0 and 1.5)

WMF-T23-005-R020v01 (Release 2)

ETSI Standard EN 302 774

ETSI Standard EN 301 908 Part 20 revision

Recommendation ITU-R F.1336-3

ECC Report 100

ETSI Standard EN 302 326-2

ITU-R Report M.2109

ECC Report 174

ITU Report M.2111

ITU-R Report M.2135

IEEE 802.11 “Model C”

ECC Report 131

Recommendation ITU-R SF.1486

Recommendation ITU-R M.1465

Recommendation ITU-R M.1652

Recommendation ITU-R M.1461

For one to four antennas

The difference between picocells and femtocells lies in the scenarios they are used in: picocell BS can be deployed in indoor line-of-sight scenarios, whereas femtocell BS are only deployed in indoor non line-of-sight scenarios.

extracted from 3GPP TR 36.824 for LTE macro BS (5 dB) andfrom 3GPP TR 36.931 for LTE pico BS (13 dB)

For one to four antennas

For one to four antennas