
	[image: image1.wmf]ECC

Electronic Communications Committee

CEPT


	Doc. ECC/SE(13)037


	Working Group SE
	

	63rd Meeting of WG SE

Lübeck, Germany, 28 January – 1 February 2013
	

	
	

	Date issued: 
	22 January 2013

	Source:
	ECC PT1

	Subject:
	LS to SE 24 on LTE UE emission masks


	Password protection required? (Y/N)


	

	

	
	[image: image2.wmf]ECC

Electronic Communications Committee

CEPT



	To: 

Ralf Kallenborn

Chairman WG SE Project Team SE24  

E-mail: ralf.kallenborn@bnetza.de   

C.c. 
Chairman WGSE

Mr. Karl Loew

E-mail: karl.loew@bnetza.de

	
	

	
	Date: Jan.  18th, 2013

	Source:
ECC PT1



	Subject:
LTE UE spectrum mask in compatibility study

	


Dear Ralf,
ECC PT1 would like to thank SE24 for the liaison statement on identified ambiguities with definition of LTE UE transmitter mask. ECC PT1 would like to propose the following guidance on this matter:

For the SE24 compatibility study’s, the UE spectrum mask from core spec. ETSI TS136.101 should be used. ECC PT1 would also like to point out that in TS136.101 ACLR has been specified to address co-existence with adjacent systems. If appropriate, ACLR from the core spec TS136.101 can also be used in the SEAMCAT simulations. 

Concerning the UE EIRP power, the understanding of ECC-PT1 is that the UE Tx power is specified at antenna connector. Referring to ECC Decision (09)03, ECC PT1 proposes to use 23 dBm EIRP for LTE UE in SE24’s compatibility study. In the ECC Decision, it is clearly indicated that 23 dBm is EIRP for fixed terminal and TRP for mobile and nomadic terminal stations.  For isotropic antennas TRP is equivalent to EIRP.   
ECC PT1 notes that the external antennas described in ECC PT1(13)027 Annex 2 are not supplied by mobile operators, and ECC PT1 understands that such antennas are not supplied or endorsed by the mobile operators concerned. Indeed, their use may well be contrary to national regulations implementing the Commission Decision. They therefore should not be taken into account in coexistence studies.
Regarding the applicability of the measurement uncertainty in the test requirement in EN302.908-13, ECC PT1 can clarify that the measurement uncertainty is not taken into account twice. The guidance on the inclusion of measurement uncertainty in the test requirement can be found in Recommendation ITU-R M.1545. 

Best regards,

Didier Chauveau

Chairman CEPT ECC PT1
Chauveau@anfr.fr
N








1
WG SE – Lübeck, Germany – 28 January – 1 February 2013
1/2

