SE(13)044A1

ECC Report 187

COMPATIBILITY STUDY BETWEEN MOBILE COMMUNICATION SERVICES ON BOARD AIRCRAFT (MCA) AND GROUND-BASED SYSTEMS

Month YYYY (Arial 9pt bold)

# Executive summary

This report considers the technical impact on ground-based systems of introducing a new Mobile Communication service onboard aircraft based on UMTS or LTE technologies operating at height of at least 3,000 meter above ground in the 1800 MHz (1710-1785 MHz for the uplink and 1805-1880 MHz for the downlink) and in the 2600 MHz (2500-2570 MHz for uplink and 2620-2690 MHz for downlink) as of LTE and in the 2100 MHz (1920-1980 MHz for uplink and 2110-2170 MHz for downlink) as of UMTS.

This report described additional studies on the compatibility of a MCA system with terrestrial networks, when the aircraft is at least 3000 m above ground. The studies demonstrated that harmful interference to terrestrial networks will not occur provided that the following technical conditions are met:

  - In the 1800 MHz connectivity (LTE technology): the e.i.r.p. defined outside the aircraft, resulting from the LTE terminal transmitting at 5 dBm/5 MHz inside the aircraft must not exceed the values as provided  in the table below:

| Height above ground (m) | Maximum e.i.r.p, defined outside the aircraft, resulting from the ac-UE in (dBm/5 MHz) |
| --- | --- |
| 3000 | 1.7 |
| 4000 | 3.9 |
| 5000 | 5 |
| 6000 | 5 |
| 7000 | 5 |
| 8000 | 5 |

- the transmit power of ac-NodeB must not exceed the maximum e.i.r.p. defined outside the aircraft as provided in the table below:

| Height above ground (m) | Maximum e.i.r.p, defined outside the aircraft, resulting from the ac-NodeB (dBm/5 MHz) |
| --- | --- |
| 3000 | 1.0 |
| 4000 | 3.5 |
| 5000 | 5.5 |
| 6000 | 7.1 |
| 7000 | 8.4 |
| 8000 | 9.6 |

In the 2100 MHz connectivity band (UMTS technology):

- The transmit power of the ac-Node B must not exceed the maximum e.i.r.p defined outside the aircraft as given in the ECC/DEC(06)07;

- The transmit power of the UMTS terminal must not exceed -6 dBm/3.84 MHz and the maximum number of users should not exceed 20.

- The e.i.r.p.of the ac-UE defined outside the aircraft must not exceed the following values as shown in the table below:

Table 2: xx

| Height above ground (km) | Max permitted e.i.r.p.(dBm/3.84 MHz) |
| --- | --- |
| 3 | 3.1 |
| 4 | 5.6 |
| 5 | 7 |
| 6 | 7 |
| 7 | 7 |
| 8 | 7 |

In the 2600 MHz connectivity band (LTE technology);

Table 3: xxx

| Minimum operational height above ground (km) | Maximum permitted e.i.r.p. produced by the onboard LTE device (dBm/5 MHz) | Maximum permitted e.i.r.p produced by the onboard nodeB (dBm/5 MHz) |
| --- | --- | --- |
| 3 | 6.9 | 2.1 |
| 4 | 9.4 | 4.6 |
| 5 | 10 | 6.5 |
| 6 | 10 | 8.1 |
| 7 | 10 | 9.5 |
| 8 | 10 | 10.6 |

It should be noted that the connectivity service in the 2600 MHz band can only be provided at the heights above 4500 m due to compatibility requirements with radars.

Additionally, compatibility with the adjacent band Radio astronomy service primary allocation at 2690-2700 MHz can be achieved assuming that the out-of-band emission outside the aircraft is lower than – 66.4 dBm/10 MHz at 3000 metres. To achieve compatibility with the RAS secondary allocation in the shared band at 2655-2690 MHz would require the same limit on emissions. It should be mentioned that the ECC/DEC/(06)07 [1] already provided a maximum e.i.r.p. that could be delivered by the NCU outside the aircraft in the band 2620-2690 MHz

With respect to the controlled NCU bands, the studies have shown that there is no change in the power levels defined outside the aircraft for the frequency bands at 460 MHz, 900 MHz, 1800 MHz and 2100 MHz as provided in the Commission Decision 2008/294/EC [6] and no change to the 2600 MHz power level for the NCU defined in ECC/DEC/(06)07 [1] – value based on the studies contained in the Annex G of ECC Report 093 [2].

In the 800 MHz band, the e.i.r.p. of the NCU should not exceed the value contained in the below table:

Table 4: NCU e.i.r.p.

| Height above ground (km) | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equivalent e.i.r.p. (dBm/10 MHz) | -0.87 | 1.63 | 3.57 | 5.15 | 6.49 | 7.65 | 8.68 | 9.59 |

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	7

2	SCOPE OF STUDIES	7

2.1	Frequency bands	7

2.2	Identification OF SHARING COMPATIBILITY STUDIES	8

2.3	IDENTIFICATION OF SCENARIOS	8

3	Aircraft height distribution	18

4	Ground- based public mobile network parameters used for modelling LTE at 800 MHz, 1800 MHz, 2100 MHz and 2600 mhZ	18

5	Ground- based public mobile network parameters used for modelling WIMAX at 1800 MHz	20

6	sYSTEM PARAMETERS OF THE CONSIDERED ADJACENT BANDS	20

6.1	RADIO ASTRONOMY SERVICE (RAS) PARAMETERS	20

6.2	RAS antennas and thresholds of interference	21

6.3	Radar parameters	22

7	compatibility analysis	24

7.1	Analysis related to onboard LTE connectivity at 1800 MHz	24

7.1.1	Scenario 1: Impact of g-base station on ac-UE at 1800 MHz	24

7.1.2	Scenario 2: Impact of ac-UE on g-base station at 1800 MHz	25

7.1.3	Estimation of the maximum power level emitted by the onboard nodeB in the 1800 MHz	25

7.1.4	Scenario 5	27

7.2	Analysis related to onboard connectivity at 2100 MHz	27

7.2.1	SEAMCAT simulations	30

7.3	ANALYSIS RELATED TO ONBOARD CONNECTIVITY AT 2600 MHZ	36

7.3.1	Scenario 1: Impact of g-base station on ac-UE at 2600 MHz	36

7.3.2	Scenario 2: Impact of ac-UE on g-base station at 2600 MHz	36

7.3.3	Estimation of the maximum power level emitted by the onboard nodeB in the 2600 MHz	37

7.3.4	Scenario 5	38

7.4	COMPATIBILITY ANALYSIS AT 800 MHZ	39

7.4.1	Scenario 1: Impact of g-NodeB on ac-UE	39

7.4.2	Scenario 2: Impact of ac-UE on g-NodeB at 800 MHz	39

7.4.3	Scenario 3: Impact of the NCU on g-UE at 800 MHz	40

7.4.4	Scenario 4: impact of multiple NCU on g-UE at 800 MHz	41

7.5	COMPATIBILITY ANALYSIS AT 900 MHZ	42

7.5.1	Scenario 1: Impact of g-NodeB on ac-UE at 900 MHz	42

7.5.2	Scenario 2: Impact of ac-UE on g-NodeB	42

7.5.3	Scenario 3: Impact of the NCU on g-UE	43

7.5.4	Scenario 4: Impact of the NCU on g-UE	44

7.6	COMPATIBILITY ANALYSIS AT 1800 MHZ	44

7.6.1	Scenario 1: Impact of g-base station on ac-UE at 1800 MHz	44

7.6.2	Scenario 2: Impact of ac-UE on g-base station at 1800 MHz	45

7.6.3	Scenario 3: Impact of the NCU on g-UE at 1800 MHz	45

7.6.4	Scenario 4: Impact of the NCU on g-WIMAX UE at 1800 MHz	46

7.7	COMPATIBILITY ANALYSIS AT 2100 MHZ	46

7.7.1	Scenario 1: Impact of g-NodeB on ac-UE at 2100 MHz	46

7.7.2	Scenario 2: Impact of ac-UE on g-NodeB at 2100 MHz	47

7.7.3	Scenario 3: Impact of the NCU on g-UE at 2100 MHz	48

7.7.4	Scenario 4: Impact of the NCU on g-UE at 2100 MHz	48

7.8	PROTECTION OF ADJACENT SERVICES	49

7.8.1	RADIO ASTRONOMY SERVICES IN THE 2690 – 2700 MHZ	49

7.8.2	RADAR SERVICES OPERATING ABOVE 2700 MHZ	50

8	Conclusions	53

ANNEX 1: Example of system activation	55

ANNEX 2: List of reference	57

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| ACLR | Adjacent Channel leakage ratio |
| ac-Node B/BTS | Aircraft base station |
| ac-UE/MS | Mobile terminal onboard an aircraft |
| BS | Base Station |
| e.i.r.p. | equivalent isotropic radiated power |
| FDD | Frequency division duplex |
| g-Node B/BTS | Ground base station |
| g-UE/MS | Ground mobile terminal |
| GSM | Global System for Mobile communication |
| GSMOBA | GSM onboard aircraft |
| ITU | International Telecommunication Union |
| LTE | Long Term Evolution |
| MCA | Mobile Communication services on board Aircraft |
| MCFN | Mobile/ Fixed Communication Network |
| MCL | Minimum Coupling Loss |
| NCU | Network Control Unit |
| PSD | Power Spectral Density |
| RAS | Radio Astronomy Service |
| RB | Resource block |
| SEAMCAT | Spectrum Engineering Advanced Monte-Carlo Analysis Tool |
| UE | User Equipment |
| UMTS | Universal Mobile Telecommunications System |
| WiMAX | Worldwide Interoperability for Microwave Access |
| WCDMA | Wideband Code Division Multiple Access |

# Introduction

ECC Report 093 [2] initially considered the technical compatibility between GSM equipment on board aircraft (GSMOBA) and ground-based public mobile networks.  This report addresses the impact on ground-based systems and networks of introducing new Mobile Communication services on board Aircraft (MCA) systems based on UMTS or LTE connectivity operating at a height of at least 3000 m above ground level in the following frequency bands:

1710-1785 MHz for uplink (terminal transmit, base station receive) and 1805-1880 MHz for downlink (base station transmit, terminal receive);

1920-1980 MHz for uplink (terminal transmit, base station receive) and 2110-2170 MHz for downlink (base station transmit, terminal receive);

2500-2570 MHz for uplink (terminal transmit, base station receive) and 2620-2690 MHz for downlink (base station transmit, terminal receive).

This report provides an outline of the different operational scenarios considered with respect to the introduction of new MCA systems based on UMTS and LTE connectivity. MCA means public mobile services intended for the airline passengers independent of the technology (e.g. GSM, UMTS, LTE) implemented in the aircraft, defined in the EC Decision 2008/294/EC [6].

# SCOPE OF STUDIES

ECC Report 093 [2] identifies the frequency bands and the expected operational scenarios for assessing compatibility issues of operating MCA systems based on the GSM technology with ground-based public mobile networks.  When considering the compatibility issues arising from the operation of UMTS and LTE systems onboard aircraft, the analysis needs to be repeated assuming the UMTS or LTE base station onboard the aircraft.  Similarly the impact on ground-based public LTE and UMTS networks are included in the study.

All other parameters relating to ground-based public networks as ‘victim’ links or ‘interfering’ links remain unchanged from the GSM case.

## Frequency bands

The updated analysis includes the frequency bands as contained in the Table below when considering emissions from mobile terminals onboard aircraft as well as from onboard pico-cell (connectivity bands) and to prevent interaction with ground-based public mobile systems (controlled bands).

Table 5: Frequency bands

| Connectivity bands: | 1710-1785 MHz (uplink)/ 1805-1880 MHz (downlink) (LTE1800) 1920-1980 MHz (Uplink)/ 2110-2170MHz (downlink) (UMTS2100) 2500-2570 MHz(uplink) / 2620-2690 MHz (downlink) (LTE 2600) |
| --- | --- |
| Controlled bands: | 791-821 MHz (LTE 800) 925-960 MHz (LTE 900) 1805-1880 MHz (LTE1800) 2500-2690 MHz (LTE 2600) |

The reason for the analysis of those new connectivity bands is that they are seen as the primary bands for UMTS and LTE in Europe; hence such cellular technologies will be widely supported by the onboard customers’ terminals.

## Identification OF SHARING COMPATIBILITY STUDIES

Tables 3 and 4 identifies the compatibility studies to be performed

Table 6: Identifications of sharing studies between onboard connectivity system and 
ground-based systems

| Band | Technology on board aircraft | In-band sharing with ground-based systems | Adjacent-band sharing with ground-based systems |
| --- | --- | --- | --- |
| 1800 MHz | GSM* | GSM, LTE |  |
| 1800 MHz | LTE | GSM, LTE |  |
| 2100 MHz FDD | UMTS | UMTS |  |
| 2600 MHz FDD 2600 MHz TDD | LTE | LTE RAS (2655-2690 MHz) | Radio Astronomy Service (RAS) (2690-2700 MHz), Radars (2700-2900 MHz) |

*This technology was already studied in ECC Report 093 [2].

Table 7: Identification of sharing studies between ground-based network and the NCU

| Band | Sharing with ground-based systems | Adjacent-band sharing with ground-based systems |
| --- | --- | --- |
| 450 MHz | CDMA450, FlashOFDM |  |
| 800 MHz | LTE |  |
| 900 MHz | GSM, UMTS, LTE, WiMAX |  |
| 1800 MHz | GSM, UMTS, LTE, WiMAX |  |
| 2100 MHz FDD | UMTS, LTE |  |
| 2600 MHz FDD | UMTS, LTE, RAS (2655-2690 MHz) | Radio Astronomy Service (RAS) (2690-2700 MHz), Radars (2700-2900 MHz) |
| 2600 MHz TDD | # UMTS, WiMAX, LTE, RAS (2655-2690 MHz) |  |

- The NCU (Network Control Unit) is a part of the MCA system designed to ensure by raising the noise floor inside the cabin that mobile terminals within the cabin cannot access to the ground-based public networks and that those compatible with the onboard technology do not transmit any signal without being controlled by the MCA system, i.e. the onboard Node B or onboard BTS.

## IDENTIFICATION OF SCENARIOS

The considered MCA (UMTS / LTE) system is designed to ensure that a mobile terminal onboard an aircraft (ac-UE) is unable to communicate with ground-based public mobile networks, whilst providing onboard connectivity to ac-UE in the LTE1800, UMTS2100 or LTE 2600 frequency bands.

The new analysis studies the impact of the:

Network control unit (NCU) emissions in the ground-based downlink (base station transmit  mobile station receive link) (the new bands for control) ;

Aircraft base station (ac-NodeB) emissions in the ground-based downlink (base station transmit  mobile station receive link), at 1800 MHz (LTE) 2100 MHz (UMTS) and 2600 MHz (LTE) only;

Mobile terminal on aircraft (ac-UE) emissions in the ground-based uplink (mobile station transmit  base station receive link), at 1800 MHz (LTE), 2100 MHz (UMTS) and 2600 MHz (LTE).

Figure 1: MCA and ground-based cellular system interference scenario

The following six scenarios are studied when needed:

Scenario 1: Impact of ground base station (g-NodeB) to the ac-UE. This scenario, using a minimum coupling loss (MCL) approach, identifies the conditions in which the mobile terminal on aircraft (ac-UE) will have visibility of the ground-based networks. Note that the NCU and aircraft base station (ac-NodeB) are not taken into account in this scenario.

Scenario 2: Impact of the ac-UE to g-NodeB. This scenario, using both MCL approach and SEAMCAT analysis, assessed in which conditions the ac-UE will have the ability to connect to ground-based networks, and in that case, the impact on other ground-based links. Note that the NCU and ac-NodeB are not taken into account in this scenario.

Scenarios 3 and 4: Impact of onboard NCU and ac-NodeB emissions to the downlink of ground-based networks, for single (Scenario 3) and multiple (Scenario 4) aircraft respectively;

Scenarios 5 and 6: Impact of ac-UE emissions to the uplink of ground-based networks, for single (Scenario 5) and multiple (Scenario 6) aircraft respectively.

Table 8: Modelling scenarios

| Scenario # | Interferers | Interfered system |
| --- | --- | --- |
| 1 | g-NodeB | ac-UE |
| 2 | ac-UE | g-NodeB |
| 3 | NCU and ac-NodeB | Ground-based network downlink |
| 4 | Multiple aircraft NCU and ac-NodeB | Ground-based network downlink |
| 5 | ac-UE | Ground-based network uplink |
| 6 | Multiple aircraft ac-UE | Ground-based network uplink |

The SEAMCAT scenario definition and elements had been used to define the scenarios necessary to assess the impacts between the two systems (ground-based mobile network vs. MCA (UMTS or LTE)), as shown in Figure 2.

Figure 2: SEAMCAT Scenario Definition

Scenario 1: Impact of g-NodeB on ac-UE (MCA not active)

This scenario assesses in which conditions the ac-UE will have visibility of the ground-based networks, by using MCL calculations. It was identified as a starting point for the study and the results will be used as inputs for Scenarios 3 and 4.

The scenario assumes one g-NodeB (using various cellular bands), and the MCA systems are disregarded, i.e. both ac-NodeB and NCU are inactive.

Figure 3: Scenario 1, where g-NodeB signal is received by onboard mobile terminals

Table 9: General summary of Scenario 1 (three new bands in addition to what is 
already covered in ECC Report 093 [2])

| Number of aircraft |  | 1 |  |
| --- | --- | --- | --- |
| Altitude of the aircraft above ground level |  | 3000 m to 10000 m |  |
| Elevation |  | Various angles from g-NodeB |  |
| Interfering transmitter |  | Single g-NodeB |  |
| Position of transmitter |  | Static |  |
| Transmitter frequencies |  | 800 MHz, 1800 MHz, 2100 MHz, 2600 MHz |  |
| Technologies |  | LTE |  |
| Path loss between aircraft and ground networks |  | Free space path loss |  |
| Victim receiver |  | Single ac-UE |  |
| Criteria |  | Received power by ac-UE from g-NodeB compared to ac-UE sensitivity as function of height above ground level |  |
| Aim |  | Assess if an onboard terminal will have visibility of ground-based networks |  |
| Modelling approach |  | MCL |  |
| Cellular technology covered |  | - LTE 800 - LTE 1800 - LTE 2600 - LTE 2100 - LTE 900 - WiMAX 1800 |  |

Scenario 2: Impact of ac-UE on g-NodeB (UMTSOBA/LTEOBA not active)

This scenario assesses in which conditions the onboard ac-UE will have the ability to connect to ground-based networks, by using both MCL calculation and the resulting potential impact on other ground-based links. The scenario consists of one victim link (ground-based uplink), and a single onboard ac-UE, with MCA system disregarded, i.e. both ac-NodeB and NCU inactive.

Figure 4: Scenario 2, where ac-UE signal is received by g-NodeB

Table 10: General summary of Scenario 2 (three new bands in addition to what is 
already covered in ECC Report 093 [2])

| Number of aircraft | 1 |
| --- | --- |
| Altitude of the aircraft above ground level | 3000 m to 10000 m |
| Elevation | Various angles from a g-NodeB |
| Interfering Transmitter | Single ac-UE |
| Interfering Transmitter power | Full power depending on the frequency band |
| Transmitter frequency | 800 MHz, 1800 MHz, 2100 MHz, 2600 MHz |
| Technologies | UMTS (WCDMA), CDMA2000, LTE |
| Path loss between aircraft and ground networks | Free space path loss |
| Victim receiver | Single g- NodeB |
| Criteria | Received power by a g- NodeB from ac-UE compared to the g- NodeB’s sensitivity |
| Aim | Assess whether an ac-UE can communicate with the ground-based network |
| Modelling approach | MCL |
| Cellular technology covered | - LTE 800 - LTE 1800 - LTE 2600 - UMTS 2100 - LTE 2100 - LTE 900 - WiMAX 1800 |

Scenario 3: NCU/ac-NodeB impact on the ground-based communication link (g-BTS/NodeB to g-MS/UE (downlink)) from a single aircraft

This scenario assesses the impact of onboard NCU (and ac-NodeB) emissions on the ground-based UE receivers, by using both MCL calculations and SEAMCAT simulations. This scenario consists of a single interfering link (the NCU and ac-NodeB emissions directed to ac-UE) whose emissions could impact a single victim link (ground-based downlink). NCU is operating and there is onboard connectivity at 1800 MHz (LTE), 2100 MHz (UMTS) and 2600 MHz (LTE).

Figure 5: Scenario 3: NCU/ac-NodeB interfering ground-based victim downlink
 (g-BTS/NodeB to g-MS/UE)

Table 11: General summary of Scenario 3 (NCU transmissions in three new bands (LTE) and 
onboard LTE 1800, UMTS 2100 and LTE 2600 Node B)

| Number of aircraft | 1 |
| --- | --- |
| Altitude of the aircraft above ground level | 3000 m to 10000 m |
| Elevation | Various angles from ground-based link |
| Interfering Transmitter (1) | ac-NodeB (Leaky cable) |
| Transmitter frequency (1) | 1800 MHz, 2100 MHz and 2600 MHz |
| Interfering Transmitter (2) | NCU (Leaky cable) |
| Transmitter frequency (2) | 800MHz, 1800 MHz, 2100 MHz, 2600 MHz |
| Victim receiver | Single g-MS/UE |
| Wanted transmitter | Single g-BTS/NodeB |
| Victim link | g-BTS/NodeB to g-MS/UE |
| Position of victim receiver | Typical outdoor distribution illustrating noise-limited network (rural area) |
| Path loss between aircraft and ground networks | Free space path loss |
| Criteria | Interference criterion I: C/(N+I) Interference criterion II: (I/N) |
| Aim | To determine the probability of the ac-NodeB/NCU interfering with the g-BTS/NodeB to g-MS/UE communication link. |
| Modelling approach | MCL, SEAMCAT |
| Simulation cases | - NCU interferer g-UE LTE 800 - NCU interferer g-UE WiMAX 1800 - NCU interferer g-UE LTE 2600 - NCU interferer g-UE LTE 2100 - NCU interferer g-UE LTE 900 - ac-NodeB UMTS 2100 interferer g-UE LTE 2100 - ac-NodeB LTE 2600 interferer g-UE LTE 2600 |

Scenario 4: NCU/ac-NodeB impact on the ground-based communications link (g-BTS/NodeB to g-MS/UE (downlink)) from multiple aircraft

This scenario assesses the impact of MCA in several aircraft, resulting from their onboard NCU (and ac-NodeB) emissions, on the ground-based UE receiver, by using SEAMCAT simulations.

The scenario consists of multiple MCA interfering links (multiple aircraft) where emissions of their NCU and/or ac-NodeB could impact a victim link (ground-based downlink). NCUs are operating and there is onboard connectivity (at 1800 MHz (LTE), 2100 MHz (UMTS) and 2600 MHz (LTE)) in all modelled aircraft.

Figure 6: Scenario 4: NCU/ac-NodeB interfering ground-based victim downlink 
(g-BTS/NodeB to g-MS/UE) from multiple aircraft

Table 12: General summary of Scenario 4 (NCU transmissions in three new bands (LTE) and onboard LTE 1800, UMTS 2100 and LTE 2600 Node B)

| Number of aircraft | Airport distribution |
| --- | --- |
| Altitude of the aircraft above ground level | Altitude, position and direction distribution |
| Elevation | Various angles from ground-based link |
| Interfering Transmitter (1) | ac-NodeB (Leaky cable) |
| Transmitter frequency (1) | 1800 MHz, 2100 MHz and 2600 MHz |
| Interfering Transmitter (2) | NCU (Leaky cable) |
| Transmitter frequency (2) | 800 MHz, 1800 MHz, 2100 MHz, 2600 MHz |
| Victim receiver | Single g-MS/UE |
| Position of victim receiver | Typical MS/UE distribution |
| Wanted transmitter | g-BTS/NodeB |
| Position of wanted receiver | Typical outdoor distribution illustrating noise-limited network (rural area) |
| Victim link | g-BTS/NodeB to g-MS/UE |
| Path loss between aircraft and ground networks | Free space path loss |
| Criteria | Interference criterion I: C/(N+I) Interference criterion II: (I/N) |
| Aim | To determine the probability of the ac-BTS interfering with the g-BTS/NodeB to g-MS/UE communication link for multiple aircraft. |
| Modelling approach | SEAMCAT |
| Simulation cases | - NCU interferer g-UE LTE 800 - NCU interferer g-UE WiMAX1800 - NCU interferer g-UE LTE 2600 - NCU interferer g-UE LTE 900 - NCU interferer g-UE LTE 2100 - ac-NodeB interferer g-UE UMTS 2100 - ac-NodeB interferer g-UE LTE 2600 |

Scenario 5: ac-UE impact on the ground-based communications link (g-UE to g-NodeB (uplink)) from a single aircraft

This scenario assesses the impact of onboard ac-UE emissions on the ground-based BTS/NodeB receiver, by using both MCL calculations and SEAMCAT simulations.

This scenario considers ac-UE as an interferer whose emissions could have impact on a single victim link (ground-based uplink). NCU is operating and there is onboard connectivity (at 1800 MHz (LTE), 2100 MHz (UMTS) or 2600 MHz (LTE)).

Figure 7: Scenario 5: ac-UE interfering ground-based uplink (g-MS/UE to g-BTS/NodeB)

Table 13: General summary of Scenario 5

| Number of aircraft | 1 |
| --- | --- |
| Altitude of the aircraft above ground level | 3000 m to 10000 m |
| Elevation | Various angles from ground-based link |
| Interfering Transmitter | Single ac-UE |
| Transmitter frequency | 1800 MHz, 2100 MHz and 2600 MHz |
| Victim receiver | 1 g-NodeB |
| Position of victim receiver | Fixed |
| Wanted transmitter | 1 g-UE |
| Position of wanted transmitter | Typical distribution illustrating noise-limited network (rural area) |
| Victim link | g-UE to g-NodeB |
| Path loss between aircraft and ground networks | Free space path loss |
| Criteria | Interference criterion I: C/(N+I) Interference criterion II: (I/N) |
| Aim | To determine the probability of the ac-UE interfering with a g-MS to g-NodeB and g-UE to g-NodeB communication link |
| Modelling approach | MCL, SEAMCAT |
| Simulation cases | - ac-UE UMTS 2100 interferer on g-UE g-NodeB UMTS 2100 - ac UE UMTS 2100 interferer on g-UE g-NodeB LTE 2100 - ac-UE LTE 2600 interferer on g-UE g-NodeB LTE 2600 |

Scenario 6: ac-UE impact on the ground-based communication link (g-UE to g-NodeB (uplink)) from multiple aircraft

This scenario assesses the impact of onboard ac-UE emissions on the ground-based BTS/NodeB receivers, by using SEAMCAT simulations.

The scenario consists of a multiple interfering links (multiple aircraft) where emissions of their ac-UEs could impact a victim link (ground-based uplink).

Figure 8: Scenario 6: ac-UE interfering ground-based uplink (g-UE to g-NodeB) from multiple aircraft

Table 14: General summary of Scenario 6

| Number of aircraft | Airport distribution |
| --- | --- |
| Altitude of the aircraft above ground level | Altitude, position and direction distribution |
| Elevation | Various angles from ground-based link |
| Interfering Transmitters | Assumed average number of mobiles transmitting per aircraft: 6 |
| Transmitter frequency | 1800 MHz (LTE), 2100 MHz (UMTS) and 2600 MHz (LTE) |
| Victim receiver | Single g-NodeB |
| Position of victim receiver | Fixed |
| Wanted transmitter | Single g-UE |
| Position of wanted transmitter | Typical distribution illustrating noise-limited network (rural area) |
| Victim link | g-UE to g-NodeB |
| Path loss between aircraft and ground networks | Free space path loss |
| Criteria | Interference criterion I: C/(N+I) Interference criterion II: (I/N) |
| Aim | To determine the probability of the ac-UE interfering with the g-UE to g-NodeB communication links for multiple aircraft near an airport. |
| Suggested modelling approach | SEAMCAT |
| Simulation cases | - ac-UE Interferer on g-UE g-NodeB UMTS 2100 - ac-UE Interferer on g-UE g-NodeB LTE 2600 |

# Aircraft height distribution

In the section 7.4.1.of the ECC Report 093 [2], the number of interferers was defined based on input data from two radar surveillance plots of the London area in busy air traffic hours. With respect to the NCU, the number of interferers to be considered is 18 for normal busy day while it is 33 for extreme busy day. The distribution of aircraft will be as defined in Table 15 (Table 14 of the ECC Report 093 [2]).

Table 15: Typical height distribution and total number of aircraft simultaneously present

| Altitude above sea level (m) | Percentage during busy hours of busy days | Percentage during busy hours of normal days |
| --- | --- | --- |
| 3000 – 4000 | 25% | 28% |
| 4000 – 5000 | 12% | 21% |
| 5000 – 6000 | 11% | 18% |
| 6000 – 7000 | 8% | 6% |
| 7000 – 8000 | 6% | 8% |
| 8000 – 9000 | 9% | 5% |
| 9000 – 10 000 | 11% | 5% |
| 10 000 – 11 000 | 8% | 4% |
| 11 000+ | 10% | 6% |
| Total | 100% | 100% |
| Total Number of aircraft simultaneously present | 146 | 80 |

# Ground- based public mobile network parameters used for modelling LTE at 800 MHz, 1800 MHz, 2100 MHz and 2600 mhZ

The following table provides the parameters used in the studies:

Table 16: LTE parameters in the 800 MHz band

| Parameter |  | LTE |  |
| --- | --- | --- | --- |
|  |  | UE | BS |
| Transmitter e.i.r.p. | dBm / channel | 23 | 64 |
| Receiver bandwidth | MHz | 4.5, 9, 13.5 and 18 | 4.5, 9, 13.5 and 18 |
| Channel bandwidth | MHz | 5, 10, 15 and 20 | 5, 10, 15 and 20 |
| Masking factor | dB |  |  |
| Reference system noise figure (taken from values quoted in standards) | dB | 9 | 5 |
| Reference noise level (taken from values quoted in standards) | dBm / channel | -98 in 5 MHz -95 in 10 MHz -92 in 20 MHz | -102 in 5 MHz -99 in 10 MHz -96 in 20 MHz |
| Reference receiver sensitivity (taken from values quoted in standards) | dBm / channel | –100 in 5 MHz –97 in 10 MHz –94 in 20 MHz | -101.5 |
| Interference criterion I (C/(N+I) ) | dB |  |  |
| Interference criterion II (I/N) | dB | -6 |  |
| Channel spacing | MHz | 5,10,20 | 5,10,20 |
| Maximum antenna gain | dBi | 0 | 15 |
| Antenna height | m | 1.5 | 45 |
| Feeder loss | dB | 0 | 3 |
| Cell radius | km | 8.633 |  |

Table 17: LTE parameters in the 1800 MHz, 2100 MHz and 2600 MHz band

| Parameter |  | LTE |  |
| --- | --- | --- | --- |
|  |  | UE | BS |
| Antenna input Power | dBm / channel | 23 | 43 in 5 MHz 46 in 10, 15, 20 MHz |
| Receiver bandwidth | MHz | 4.5, 9, 13.5 and 18 | 4.5, 9, 13.5 and 18 |
| Channel bandwidth | MHz | 5, 10, 15 and 20 | 5, 10, 15 and 20 |
| Masking factor | dB |  |  |
| Reference system noise figure (taken from values quoted in standards) | dB | 9 | 5 |
| Reference noise level (taken from values quoted in standards) | dBm / channel | -98 in 5 MHz -95 in 10 MHz -92 in 20 MHz | -102 in 5 MHz -99 in 10 MHz -97 in 15 MHz -96 in 20 MHz |
| Reference receiver sensitivity (taken from values quoted in standards) | dBm / channel | –100 in 5 MHz –97 in 10 MHz –95.2 in 15 MHz –94 in 20 MHz | -101.5 |
| Interference criterion I (C/(N+I) ) | dB |  |  |
| Interference criterion II (I/N) | dB | -6 |  |
| Channel spacing | MHz | 5,10,20 | 5,10,20 |
| Maximum antenna gain | dBi | 0 | 17 |
| Antenna height | m | 0 | 30 |
| Cell radius | km | 8 |  |

# Ground- based public mobile network parameters used for modelling WIMAX at 1800 MHz

The following table provides the parameters used in the studies:

Table 18: WIMAX parameters in the 1800 MHz band

| Parameters |  | WIMAX 1800 |  |
| --- | --- | --- | --- |
|  |  | BS | UE |
| Carrier separation | MHz | 5. 10 |  |
| Tx Power (Maximum) | dBm | 43 | 23 |
| Antenna gain | dBi | 15 to 17 | 0 |
| Feeder loss | dB | 3 | 1 |
| Antenna height | m | 45 (Rural) 30 (Urban) | 1.5 |
| Antenna down-tilt | ° | 3 | - |
| BS-UE MCL | dB | 80 (Rural) 70 (Urban) | - |
| Receiver Bandwidth (MHz) | MHz | 4.75 for 5 MHz channel 9.5 for 10 MHz channel | 4.75 for 5 MHz channel 9.5 for 10 MHz channel |
| Receiver Thermal Noise Level (dBm) | dBm | -102.2 for 5 MHz channel -99.2 for 10 MHz channel | -99.2 for 5 MHz channel -96.2 for 10 MHz channel |
| Receiver reference sensitivity (dBm) | dBm | -101.3 for 5 MHz channel -98.3 for 10 MHz channel | -97.8 for 5 MHz channel -94.8 for 10 MHz channel |
| ACLR_1 (dB) (5MHz for 5 MHz channel) (10MHz for 10 MHz channel) ACLR_1 (dB) (UTRA BW 3.84 MHz) | dB | 45 45 | 30 33 |
| ACLR_2 (dB) (10 MHz for 5 MHz channel) (20 MHz for 10 MHz channel) | dB | 50 | 44 |

# sYSTEM PARAMETERS OF THE CONSIDERED ADJACENT BANDS

## RADIO ASTRONOMY SERVICE (RAS) PARAMETERS

Radio Astronomy is a passive service, so does not cause interference to other users of the radio spectrum. It uses state of the art receiver systems and is highly susceptible to interference from air and space borne transmitters. Developments over the last 20 years mean that radio astronomical observations are often made on a coordinated basis worldwide and, since radio astronomy is also dependent on naturally occurring phenomena, the operational frequencies it uses cannot be moved within the spectrum. Additionally, radio astronomy cannot operate effectively with levels of interference that would be tolerable in commercial systems. Consequently, its coexistence with other services in adjacent and shared bands needs careful management. From the RAS perspective, the bands near 2.7 GHz are important because it can produce receivers with extremely low noise characteristics and there is a low galactic background; they are mainly used for continuum observations in many European observatories either as single dish or for coordinated simultaneous measurements across Europe.

In view of deploying MCA systems on board aircraft, compatibility with RAS in the band 2690-2700 MHz should be examined. However, the RAS also has an allocation on a secondary basis in the band 2655-2690 MHz; an allocation to MCA will imply sharing with the existing RAS operations in this band.

The frequency bands 2655-2690 and 2690-2700 MHz are allocated to the Radio Astronomy Service supported by RR footnotes 5.149 and 5.340 respectively, which state that:

5.149: “In making assignments to stations of other services to which the bands: …., 2655-2690 MHz, .... are allocated, administrations are urged to take all practicable steps to protect the radio astronomy service from harmful interference. Emissions from space-borne or airborne stations can be particularly serious sources of interference to the radio astronomy service (see Nos. 4.5 and 4.6 and Article 29).”

5.340: “All emissions are prohibited in the following bands: 1 400-1 427 MHz, 2 690-2 700 MHz, except those provided for by No. 5.422, ...”

## RAS antennas and thresholds of interference

The RAS uses very high gain antennas and unfiltered receivers having large fractional bandwidths in order to detect cosmic radio waves; observations of the highest sensitivity are obtained when radio astronomers make use of the widest possible bandwidth. In most cases, interference to the radio astronomy station will be received through the antenna side lobes, so the very narrow, high gain, main beam response to the interference is not usually considered. In fact for most interference calculation purposes it has become the practice to model the RAS observatory antenna is as having a gain of 0 dBi in all directions. This has been encapsulated in Recommendation ITU-R RA.769: “Protection criteria used for radio astronomical measurements”; the data in the tables in this Recommendation are regarded as the generally applicable interference threshold criteria for the protection of high sensitivity radio astronomy observations.

RAS protection requirements

Recommendation ITU-R RA.769-2 [4] provides the protection criteria for radio astronomical measurements. The appropriate value for the band 2690-2700 MHz is 207 dBW/10MHz or -177 dBm/10MHz, which also applies to all systems operated in the adjacent band 2655-2690 MHz at, or near the location of the radio telescope.

Parameters for radio astronomy stations

ECC Report 045 [12] provides the relevant parameters for the radio astronomy stations in Europe using the 2690-2700 MHz band.

Table 19: RAS stations operating at the 2690-2700 MHz band in Europe

| Country | Place | Latitude N | Longitude E | Heigth above sea level (m) | Diameter (m) | Minimum elevation (°) |
| --- | --- | --- | --- | --- | --- | --- |
| Czech Republic | Ondrejov1 | 49°54'38" | 14°47'01" | 525 | 3 7,5 | 0 0 |
| France | Nançay | 47°23'26" | 02°12'00" | 180 | 200 x 40 | 3.6 |
| Germany | Effelsberg | 50°31'32" | 06°53'00" | 369 | 100 | 7 |
| Netherlands | Westerbork | 52°55'01" | 06°36'15" | 16 | 14 x 25 | 0 |
| Russia | Kalyazin Pushchino Zelenchukskaya | 57o 13'22" 54o 49'00" 43o 49'53" | 37o 54'01" 37o 40'00" 41o 35'32" | 195 200 1000 | 64 22 32 | 0 6 -5 |
| Switzerland | Bleien1) | 47°22'36" | 08°33'06" | 469 | 7 | 5 |
| United Kingdom | Cambridge | 52°09'59" | 00°02'20" | 24 | 60 x 5 | 0 |
| United Kingdom | Jodrell Bank | 53°14'10" | -02°18'26" | 78 | 76 32 13 | -1 0 0 |
| Typical maximum antenna gain: 69.0 dBi |  |  |  |  |  |  |

1  solar observations

## Radar parameters

The following table provides the technical characteristics of radars in the band above 2700 MHz.

Table 20: Radar parameters for the frequency band above 2700 MHz

| Parameter | Unit | ATC and defense |  |  |  |  | Meteorology |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Type 1 | Type 2 |  | Type 3 |  | Type 4 |
| Category |  | Frequency hopping | 2 to 4 frequencies |  |  |  | Single frequency |
| Maximum antenna gain | dBi | >40 | 34 |  |  |  | 43 |
| Antenna pattern |  | Not given | Vertical pattern cosecant-squared (see figure 5) |  |  |  | Recommendation ITU-R F.1245-2 [10] |
| Antenna height | m | 5-40 (normal 12) |  |  |  |  | 7-21 (normal 13) |
| Polarization |  | Circular |  |  |  |  | H/V |
| Feeder loss | dB | <1 |  | Not given |  |  | 2 |
| Minimum elevation angle | ° | Not given | 2 (see Recommendation ITU-R M.1851 [11] ) |  |  |  | 0.5 |
| Protection level (Note 1) | dBm/MHz | -122 |  |  |  |  |  |
| 1 dB compression point (Note 2) | dBm | -20 (see Recommendation ITU-R M.1464 [5]) |  |  |  |  | 10 |
| Transmission power | kW | 1000 | 400 | 30 |  |  | 794 |
| Reference bandwidth | kHz | 2500 | 1000 | 800 |  | 800 | 1000 |
| 40 dB bandwidth | MHz | 9.5 | 20 | 4 |  | 2 | 25 |
| Out of band roll off | dB/decade | 20 | 20 | 20 |  |  | 40 |
| Spurious level | dBc | -60 | -60 | -60 |  |  | -60 for old radars and -75 to -90 for new radars |
| Pulse repetition rate | Hz | <300 | ~1000 | 825 |  |  | 250 – 1200 (See Rec ITU-R M.1849 [7] |
| Pulse duration | µs | 20 and 100 | 1 | 1 |  | 100 | 0.8-2 |
| Rise and fall time | % of pulse length | 1% | 10% | 0.169 |  | Not given | 10% |
| Antenna rotation | RPM | 6-12 | 12-15 | 15 |  |  | See Rec ITU-R M.1849 [7] |
| Scan in elevation |  | Not given | Fixed |  |  |  | See Rec ITU-R M.1849 [7] |

Note 1: This protection level is derived from measurements as explained in Recommendation ITU-R M.1464-1 [5]).

Note 2: ECC Report 174 [3] gives blocking levels that can be more stringent than 1 dB compression point.

Figure 9 provides the ATC radar antenna pattern in elevation

Figure 9: ATC radar antenna pattern in elevation

Figure 10 provides the typical elevation angle of meteorological radars over time. It shows that the elevation angle goes from 0.5° to about 37°. For the compatibility study, the elevation angle of 37° will be considered.

Figure 10: Meteorological radars, typical elevation variation over time

# compatibility analysis

ECC Report 093 [2] considers the technical compatibility between GSM equipment on board aircraft and ground-based public mobile networks.  The additional compatibility studies performed here address the impact on ground-based public mobile networks of introducing a MCA system based on the UMTS / LTE technology operating at a height of at least 3000 metres above ground level in the following frequency bands:

1710-1785 MHz for uplink (terminal transmit, base station receive) / 1805-1880 MHz for downlink (base station transmit, terminal receive);

1920-1980 MHz for uplink (terminal transmit, base station receive) / 2110-2170 MHz for downlink (base station transmit, terminal receive);

2500-2570 MHz for uplink (terminal transmit, base station receive) / 2620-2690 MHz for downlink (base station transmit, terminal receive).

It will as well develop the technical conditions to protect ground-based cellular systems operating in the frequency band 832-862 MHz / 791-821 MHz.

## Analysis related to onboard LTE connectivity at 1800 MHz

### Scenario 1: Impact of g-base station on ac-UE at 1800 MHz

This scenario assesses in which conditions the ac-UE will have visibility of the terrestrial LTE1800 networks, by using MCL calculations.

The worst case elevation angle is 48 °, corresponding to an antenna gain of -1.84 dBi.

Table 21: Impact of g-LTE base station on ac-UE at 1800 MHz

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / base station (km) | Path loss (dB) | Ant. Gain (dBi) at given angle | LTE1800 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | e.i.r.p. (dBm) | Max. received power in aircraft, Pmax_rec:ac-MS (dBm/ch) | Margin (dB) |
| 3000 | 48 | 4.04 | 109.9 | -1.84 | 41.16 | -73.7 | -26.3 |
| 4000 | 48 | 5.38 | 112.4 | -1.84 | 41.16 | -76.2 | -23.8 |
| 5000 | 48 | 6.73 | 114.3 | -1.84 | 41.16 | -78.1 | -21.9 |
| 6000 | 48 | 8.07 | 115.9 | -1.84 | 41.16 | -79.7 | -20.3 |
| 7000 | 48 | 9.42 | 117.2 | -1.84 | 41.16 | -81.1 | -18.9 |
| 8000 | 48 | 10.76 | 118.4 | -1.84 | 41.16 | -82.2 | -17.8 |
| 9000 | 48 | 12.1 | 119.4 | -1.84 | 41.16 | -83.2 | -16.8 |
| 10000 | 48 | 13.45 | 120.3 | -1.84 | 41.16 | -84.2 | -15.8 |

A negative margin means that an extra isolation is necessary to remove the visibility of the ground networks.

### Scenario 2: Impact of ac-UE on g-base station at 1800 MHz

This scenario assesses in which conditions the onboard ac-UE will have the ability to connect to terrestrial networks.

Table 22: impact of ac-UE on g-base station at 1800 MHz

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / g_UE (km) | Path loss (dB) | Rx Ant. Gain (dBi) at given angle | LTE1800 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | UE e.i.r.p (dBm) | Max. received power on ground, Pmax_rec:_g_node B (dBm/ch) | Margin (dB) |
| 3000 | 48 | 4.04 | 109.9 | -1.84 | 23 | -93.7 | -7.8 |
| 4000 | 48 | 5.38 | 112.4 | -1.84 | 23 | -96.2 | -5.3 |
| 5000 | 48 | 6.73 | 114.3 | -1.84 | 23 | -98.1 | -3.4 |
| 6000 | 48 | 8.07 | 115.9 | -1.84 | 23 | -99.7 | -1.8 |
| 7000 | 48 | 9.42 | 117.2 | -1.84 | 23 | -101.1 | -0.4 |
| 8000 | 48 | 10.76 | 118.4 | -1.84 | 23 | -102.2 | 0.7 |
| 9000 | 48 | 12.1 | 119.4 | -1.84 | 23 | -103.2 | 1.7 |
| 10000 | 48 | 13.45 | 120.3 | -1.84 | 23 | -104.2 | 2.7 |

A negative margin shows that it is possible that an UE could connect to a ground-based mobile network.

### Estimation of the maximum power level emitted by the onboard nodeB in the 1800 MHz

Based on the ECC/DEC/(06)07 [1] and taken into account the fact that the GSM mobile terminal will transmit 0 dBm, then it is possible to determine the minimum aircraft attenuation as shown in;

Table 23; aircraft attenuation

| Height above ground (m) | Aircraft attenuation (dB) |
| --- | --- |
| 3000 | 3.3 |
| 4000 | 1.1 |
| 5000 | -0.5 |
| 6000 | -1.8 |
| 7000 | -2.9 |
| 8000 | -3.8 |

From Table 23, it is possible to estimate the e.i.r.p. outside the aircraft with the following formula:

e.i.r.p. (dBm/Channel) = Max received signal + Radiation factor – aircraft attenuation + 5 dB (this value was used as initial assumption in the ECC Report 093 [2]).

Then, from the calculated e.i.r.p. the increase of noise level will be estimated.

Table 24: MCL calculation

| height above ground (km)  | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- |
| Max received signal level (dBm/5MHz) | -73.7 | -76.2 | -78.1 | -79.7 | -81.1 | -82.2 |
| Radiation Factor (Large Aircraft) (dB) | 70 | 70 | 70 | 70 | 70 | 70 |
| Aircraft Attenuation (dB) | 3.3 | 1.1 | -0.5 | -1.8 | -2.9 | -3.8 |
| Equivalent e.i.r.p. (as point of source) (dBm/5MHz) | -1.0 | -1.3 | -1.6 | -1.9 | -2.2 | -2.4 |
| Free Space Propagation Losses (dB) | 107.3 | 109.8 | 111.7 | 113.3 | 114.6 | 115.8 |
| Maximum Received Noise by g-MS (dBm) | -108.3 | -111.1 | -113.4 | -115.2 | -116.8 | -118.2 |
| System Noise Level, reference values (dB/bw) | -100 | -100 | -100 | -100 | -100 | -100 |
| Increase of the noise floor at g-MS with respect to reference values (dB) | 0.60 | 0.33 | 0.20 | 0.13 | 0.09 | 0.06 |

Editor’s note: radiation factor in the table above is extracted from ECC Report 093 [3] (Table 21).

From Table 36 and Table 24, it is then possible to calculate the required attenuation in order to get the 1 dB increase noise floor at the ground UE:

Table 25: Calculation of maximum e.i.r.p.

| Height above ground (km) | MCL, 1 dB increased noise floor |  |  | Maximum e.i.r.p. produced by the ac-nodeB (dBm/5 MHz) | Maximum e.i.r.p. produced by the ac-nodeB (dBm/200 kHz) |
| --- | --- | --- | --- | --- | --- |
|  | MS attenuation (dB) | Ac-nodeB power (dBm) | Required attenuation (dB) |  |  |
| 3 | 3.3 | -1 | -2.43 | 1.43 | -12.55 |
| 4 | 1.1 | -1.3 | -5.22 | 3.92 | -10.06 |
| 5 | -0.5 | -1.6 | -7.50 | 5.9 | -8.08 |
| 6 | -1.8 | -1.9 | -9.36 | 7.46 | -6.52 |
| 7 | -2.9 | -2.2 | -10.94 | 8.74 | -5.24 |
| 8 | -3.8 | -2.4 | -12.36 | 9.96 | -4.02 |

Based on the result of the maximum e.i.r.p., defined outside the aircraft and produced by the ac-NodeB in 1800 MHz, it can be seen that the limit contained in the ECC/DEC/(06)07 [1] in the band 1800 MHz remains.

### Scenario 5

Table 26: MCL calculation for ac-UE1800 MHz to terrestrial LTE networks

| height above ground (km) | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance g-nodeB/ ac-UE (km) | 4.04 | 5.38 | 6.73 | 8.07 | 9.42 | 10.76 | 12.1 | 13.45 |
| UE power level (dBm/5 MHz) | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |
| Aircraft Attenuation (dB) | 3.3 | 1.1 | 0 | 0 | 0 | 0 | 0 | 0 |
| e.i.r.p. outside the aircraft (dBm/5 MHz) | 1.7 | 3.9 | 5 | 5 | 5 | 5 | 5 | 5 |
| Free Space Propagation Losses (dB) | 109.4 | 111.9 | 113.8 | 115.4 | 116.7 | 117.9 | 118.9 | 119.8 |
| Terrestrial LTE antenna Gain (dBI) | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 |
| Maximum Received Noise by g-NodeB (dBm/5 MHz) | -109.5 | -109.8 | -110.6 | -112.2 | -113.5 | -114.7 | -115.7 | -116.6 |
| System Noise Level, reference values (dBm/5 MHz) | -102 | -102 | -102 | -102 | -102 | -102 | -102 | -102 |
| Increase of the noise floor at g-NodeB with respect to reference values (dB) | 0.71 | 0.67 | 0.56 | 0.40 | 0.30 | 0.23 | 0.18 | 0.15 |

Table 26 shows that the increase of noise remains below 1 dB.

## Analysis related to onboard connectivity at 2100 MHz

The ECC Report 093 [2] already contains results for scenarios 1 and 3. Those results are calculated using the same parameters as identified in Table 17. Therefore, those studies are not repeated for this report. The e.i.r.p. level used for the scenario 4 is the ECC limit (i.e 1 dBm / 3.84 MHz).

### SEAMCAT simulations

- Parameters adopted for Scenario 4, 5 & 6

Link parameters

Table 27: Interfering Link Parameters for Scenario 4

| Ac-Node B |  | e.i.r.p. |  | 1dBm/3.84MHz |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Antenna Height |  | Refer to Table 15 |  |
|  |  | Antenna peak gain |  | 0 dBi |  |
| Ac-UE |  | Receiver Sensitivity |  | -119dBm/3.84MHz |  |
|  |  | Antenna Height |  | Refer to Table 15 |  |
|  |  | Antenna peak gain |  | 0 dBi |  |

Table 28: Victim Link Parameters for Scenario 4

| Number of users per cell (defined by SEAMCAT) |  | 37 |  |
| --- | --- | --- | --- |
| Cell Radius |  | 6 km |  |
| Handover margin |  | 4 dB |  |
| Minimum coupling loss |  | 70 dB |  |
| Radiation pattern |  |  | # Recommendation ITU-R F.1336-3 Section 3.2 [8] |  |  |
| Antenna gain |  | 18 dBi |  |
| Simulation radius |  | 56 km |  |

Table 29: Interfering Link Parameters for Scenario 5 and 6

| Ac-UE |  | e.i.r.p. |  | -6 dBm/3.84MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Antenna Height |  | Refer to Table 15 |  |  |
|  |  | Antenna peak gain |  | 0 dBi |  |  |
| Ac-BTS |  | Receiver Sensitivity |  | -121 dBm/3.84MHz |  |  |
|  |  | Antenna Height |  | Refer to Table 15 |  |  |
|  |  | Antenna peak gain |  | 0 dBi |  |  |

Table 30: Victim link Parameters for Scenario 5 & 6

| Number of users per cell (defined by SEAMCAT) |  | 35 |  |
| --- | --- | --- | --- |
| Cell Radius |  | 6 km |  |
| Handover margin |  | 4 dB |  |
| Minimum coupling loss |  | 70 dB |  |
| UE Maximum Transmit Power |  | 24 dBm |  |
| UE power Control Range |  | 72.0 dB |  |
| Radiation pattern |  | Recommendation ITU-R F.1336-3 Section 3.2 [8] |  |
| Antenna gain |  | 18dBi |  |
| Simulation radius |  | Scenario 5 : Refer to (Distance g-BTS to ac-UE) Scenario 6 : 200km |  |

Number of Interferers

In Section 7.4.1. of the ECC Report 093 [2], the number of interferers was defined based on the input data from two radar surveillance plots of the London area in busy air traffic hours. With respect to the NCU, the number of interferers to be considered is 18 for normal busy day while it is 33 for extreme busy day. The distribution of aircraft is as defined in Table 15 (Table 14 of the ECC Report 093 [2]). Considering frequency reuse and following the same development in page 30 of ECC Report 093[2], the number of interferers used in scenario 4 is calculated as followed:

Number of UMTS channel available onboard aircraft: 12 UMTS channels (assume full access to the whole 3G spectrum for connectivity)

Number of UMTS channel onboard narrow body aircraft: 2

Number of UMTS channel onboard wide body aircraft: 5

Weighted average frequency allocated (assuming 27% wide body aircraft)

= (0.27*5*5)+(0.73*2*5) MHz= 14.05 MHz

Frequency re-use factor = frequency allocated / spectrum pool available:

= 14.05/60=23.4 %

Therefore, for Scenario 4:

Number of ac-BTS for a normal day = 18*23.4%= 4

Number of ac-BTS for a busy day = 33*23.4% = 8

The numbers of interferers used in the simulations for a normal and busy day are 4 and 8 respectively.

For Scenario 5: The number of interferers for a single aircraft is the number of simultaneous ac-UE users we consider the case of 6.

For Scenario 6: The number of interferers for a single aircraft remains the same as Scenario 5, however, the number of interference links to the victim link is now 4 and 8 respectively for the normal day and busy day case.

- SEAMCAT Simulation Results for Scenario 4

In this scenario, we study the impact of the ac-BTS on the terrestrial UMTS networks on the downlink communications link between the g-BTS and g-UE. The e.i.r.p. used is as defined in the ECC/DEC/(06)07, i.e. 1 dBm/ 3.84 MHz at 3000 metre. Table 31 provides the simulation results for Scenario 4.

Table 31: SEAMCAT simulation results for Scenario 4

| Description of the case |  |  | Reference cell | CDMA system |
| --- | --- | --- | --- | --- |
|  |  |  | Average capacity loss | Average capacity loss |
| Scenario 4 (2100 MHz) | Multiple ac-BTS to terrestrial UMTS network | Normal day (4 interferers) | 0% | 3.72 % |
|  | Multiple ac-BTS to terrestrial UMTS network | Busy day (8 interferers) | 0 % | 2.35% |

Note: in the above Table, the aircraft distribution for a normal day is greater than that for a busy day at aircraft height above ground at 3000 to 5000 metres (see Table 15).

- SEAMCAT Simulation Results for Scenarios 5 and 6

In the scenarios 5 and 6 it is studied the impact of the ac-UE (of one and multiple aircrafts) on the ground-based UMTS networks on the uplink communications link between the g-UE to the g-BTS.

In this scenario, the impact of the ac-UE (of a single aircraft) on the ground-based base station (g-Node-B) was studied. The e.i.r.p. values for ac-UE considered here is -6dBm and the numbers of ac-UE on board considered here are 20.

Prec_g-Node B = e.i.r.p.ac-UE – LAircraft – Lprop + Gg-Node B

Prec_g-Node B : Power received at the g-NodeB (dBm)

e.i.r.p.ac-UE : e.i.r.p. of the ac-UE when the NCU is active (dBm)

LAircraft  : Attenuation due to aircraft (dB)

Lprop : Propagation loss between aircraft and g-UE (dB)

Gg-Unode B: Antenna gain of the g-Node B (dBi)

The increase in noise floor at the g-UE receiver is given by:

| (dB) |  |
| --- | --- |
| : | Noise power level of the g-node B |
| : | Interference received by the g-node B |

|  | : Noise level of the g-Node B |
| --- | --- |
|  | : Interference received by g-Node B |

The table below assesses the change to interference level of user terminal on board aircraft at different height above ground level.

For the purposes of this analysis the following assumptions are used:

Number of simultaneous users: 20;

ac-UE e.i.r.p.: -6dBm.

Table 32: Impact of onboard (UMTS 2100) terminal (ac-UE) on ground-based base 
station (g-NodeB) noise level

| Height above ground (km) | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance g-node B / ac-UE (km) | # 4 | # 94.6 | # 114.1 | # 132.5 | # 150.1 | # 167 | # 183 | # 198.4 |
| Power of onboard UE (dBm) | -6 | -6 | -6 | -6 | -6 | -6 | -6 | -6 |
| UE Antenna Gain (dBi) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Simultaneous Users (dB) | 13.0 | 13.0 | 13.0 | 13.0 | 13.0 | 13.0 | 13.0 | 13.0 |
| ttenuation due to the aircraft (dB) | 5.00 | 5.00 | 5.00 | 5.00 | 5.00 | 5.00 | 5.00 | 5.00 |
| Path Loss (dB) | 111.2 | 113.7 | 115.6 | 117.2 | 118.5 | 119.7 | 120.7 | 121.6 |
| Terrestrial BTS Antenna Gain (dBi) | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 |
| Power Received at g-BTS (dBm) | -111.0 | -113.5 | -115.4 | -117.0 | -118.3 | -119.5 | -120.5 | -121.4 |
| System Noise Level (typical operators) | -104 | -104 | -104 | -104 | -104 | -104 | -104 | -104 |
| Increase of the noise floor at g-nodeB with respect to typical values (dB) | 0.80 | 0.47 | 0.30 | 0.21 | 0.16 | 0.12 | 0.10 | 0.08 |

Tables bellow shows the SEAMCAT results for different values of e.i.r.p. of ac-UE for different number of ac-UE.

Table 33: SEAMCAT simulation results for Scenario 5 with number of ac-UE = 20

| Height above ground (km) | Average Capacity Loss |  |
| --- | --- | --- |
|  | e.i.r.p. ac-UE= -6dBm |  |
|  | Reference cell | CDMA system |
| 3 | 3.74% | 0.00% |
| 5 |  | 0.03% |  | 0.00% |  |
| 8 | 0.03% | 0.00% |

In this scenario, the impact of the ac-UE (of multiple aircrafts) on the terrestrial UMTS networks on the uplink communications link between the g-UE to the g-BTS was studied.  Table 34 shows the SEAMCAT results for Scenario 6 with e.i.r.p. of ac-UE = -6dBm and with the number of ac-UE = 20.

Table 34: SEAMCAT results for Scenario 6 with the number of ac-UE = 20

| Description of the case |  |  | Average Capacity Loss |  |
| --- | --- | --- | --- | --- |
|  |  |  | e.i.r.p. ac-UE= -6dBm |  |
|  |  |  | Reference cell | CDMA system |
| Scenario 6 (2100 MHz) | Multiple ac-UE to terrestrial UMTS network | Normal day | 0.22% | 0 % |
|  | Multiple ac-UE to terrestrial UMTS network | Busy day | 0.38% | 0% |

The results show that the average capacity loss remains below 5%.

Table 35: xxx

| Height above ground (km) | MCL, 1 dB increased noise floor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Aircraft attenuation (dB) | Ac-UE power (dBm) | Mulitple user factor (dB) | Required attenuation (dB) | Effective attenuation (dB) | Max permitted e.i.r.p. (dBm/channel) |
| 3 | 5 | -6 | 13 | -1.1 | 3.9 | 3.1 |
| 4 | 5 | -6 | 13 | -3.6 | 1.4 | 5.6 |
| 5 | 5 | -6 | 13 | -5.5 | 0 | 7 |
| 6 | 5 | -6 | 13 | -7.1 | 0 | 7 |
| 7 | 5 | -6 | 13 | -8.5 | 0 | 7 |
| 8 | 5 | -6 | 13 | -9.6 | 0 | 7 |

## ANALYSIS RELATED TO ONBOARD CONNECTIVITY AT 2600 MHZ

### Scenario 1: Impact of g-base station on ac-UE at 2600 MHz

This scenario assesses in which conditions the ac-UE will have visibility of the terrestrial LTE2600 networks, by using MCL calculations.

The worst case elevation angle is 48 °, corresponding to an antenna gain of -1.84 dBi.

Table 36: Impact of g-LTE base station on ac-UE at 2600 MHz

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / base station (km) | Path loss (dB) | Ant. Gain (dBi) at given angle | LTE2600 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | e.i.r.p. (dBm) | Max. received power in aircraft, Pmax_rec:ac-MS (dBm/ch) | Margin (dB) |
| 3000 | 48 | 4.04 | 113.0 | -1.84 | 41.16 | -76.8 | -23.2 |
| 4000 | 48 | 5.38 | 115.5 | -1.84 | 41.16 | -79.3 | -20.7 |
| 5000 | 48 | 6.73 | 117.4 | -1.84 | 41.16 | -81.3 | -18.7 |
| 6000 | 48 | 8.07 | 119.0 | -1.84 | 41.16 | -82.8 | -17.2 |
| 7000 | 48 | 9.42 | 120.3 | -1.84 | 41.16 | -84.2 | -15.8 |
| 8000 | 48 | 10.76 | 121.5 | -1.84 | 41.16 | -85.3 | -14.7 |
| 9000 | 48 | 12.1 | 122.5 | -1.84 | 41.16 | -86.4 | -13.6 |
| 10000 | 48 | 13.45 | 123.4 | -1.84 | 41.16 | -87.3 | -12.7 |

A negative margin means that an extra isolation is necessary to remove the visibility of the ground networks.

### Scenario 2: Impact of ac-UE on g-base station at 2600 MHz

This scenario assesses in which conditions the onboard ac-UE will have the ability to connect to terrestrial networks.

Table 37: impact of ac-UE on g-base station at 2600 MHz

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / g_UE (km) | Path loss (dB) | Rx Ant. Gain (dBi) at given angle | LTE2600 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | UE e.i.r.p. (dBm) | Max. received power on ground, Pmax_rec:_g_node B (dBm/ch) | Margin (dB) |
| 3000 | 48 | 4.04 | 113.0 | -1.84 | 23 | -96.8 | -4.7 |
| 4000 | 48 | 5.38 | 115.5 | -1.84 | 23 | -99.3 | -2.2 |
| 5000 | 48 | 6.73 | 117.4 | -1.84 | 23 | -101.3 | -0.2 |
| 6000 | 48 | 8.07 | 119.0 | -1.84 | 23 | -102.8 | 1.3 |
| 7000 | 48 | 9.42 | 120.3 | -1.84 | 23 | -104.2 | 2.7 |
| 8000 | 48 | 10.76 | 121.5 | -1.84 | 23 | -105.3 | 3.8 |
| 9000 | 48 | 12.1 | 122.5 | -1.84 | 23 | -106.4 | 4.9 |
| 10000 | 48 | 13.45 | 123.4 | -1.84 | 23 | -107.3 | 5.8 |

A negative margin shows that it is possible that an UE could connect to a ground-based mobile network.

### Estimation of the maximum power level emitted by the onboard nodeB in the 2600 MHz

e.i.r.p. (dBm/Channel) = Max received signal + Radiation factor – aircraft attenuation + 5 dB (value used as initial assumption).

Then, from the calculated e.i.r.p., the increase of noise level will be estimated.

Table 38: MCL calculation

| height above ground (km)  | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- |
| Max received signal level (dBm/5MHz) | -76.8 | -79.3 | -81.3 | -82.8 | -84.2 | -85.3 |
| Radiation Factor (Large Aircraft) (dB) | 71 | 71 | 71 | 71 | 71 | 71 |
| Aircraft attenuation (dB) | 3.3 | 1.1 | -0.5 | -1.8 | -2.9 | -3.8 |
| Equivalent e.i.r.p. (as point of source) (dBm/5MHz) | -4.1 | -4.4 | -4.8 | -5 | -5.3 | -5.5 |
| Free Space Propagation Losses (dB) | 110.4 | 112.9 | 114.8 | 116.4 | 117.8 | 118.9 |
| Maximum Received Noise by g-MS (dBm) | -114.5 | -117.3 | -119.6 | -121.4 | -123.1 | -124.4 |
| System Noise Level, reference values (dB/bw) | -100 | -100 | -100 | -100 | -100 | -100 |
| Increase of the noise floor at g-MS with respect to reference values (dB) | 0.15 | 0.08 | 0.05 | 0.03 | 0.02 | 0.02 |

From Table 36Table 38, it is then possible to calculate the required attenuation in order to get the 1 dB increase noise floor at the ground UE:

Table 39: Calculation of maximum e.i.r.p.

| Height above ground (km) | MCL, 1 dB increased noise floor |  |  | Maximum e.i.r.p. produced by the ac-nodeB (dBm/5 MHz) | Maximum e.i.r.p. produced by the ac-nodeB (dBm/4.75 MHz) |
| --- | --- | --- | --- | --- | --- |
|  | MS attenuation (dB) | Ac-nodeB power (dBm) | Required attenuation (dB) |  |  |
| 3 | 3.3 | -4.1 | -10.6 | 4.5 | 4.28 |
| 4 | 1.1 | -4.4 | -11.4 | 7 | 6.78 |
| 5 | -0.5 | -4.8 | -13.7 | 8.9 | 8.68 |
| 6 | -1.8 | -5 | -15.5 | 10.5 | 10.28 |
| 7 | -2.9 | -5.3 | -17.2 | 11.9 | 11.68 |
| 8 | -3.8 | -5.5 | -18.5 | 13 | 12.78 |

Based on the result of the maximum e.i.r.p., defined outside the aircraft and produced by the ac-NodeB in 2600 MHz, it can be seen that the limit contained in the ECC/DEC/(06)07 [1] in the band 2600 MHz remains.

### Scenario 5

Table 40: MCL calculation for ac-UE2600 to terrestrial LTE networks

| height above ground (km) | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance g-nodeB/ ac-UE (km) | 4.04 | 5.38 | 6.73 | 8.07 | 9.42 | 10.76 | 12.1 | 13.45 |
| UE power level (dBm/5 MHz) | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Aircraft Attenuation (dB) | 3.3 | 1.1 | 0 | 0 | 0 | 0 | 0 | 0 |
| e.i.r.p. outside the aircraft (dBm/5 MHz) | 6.7 | 8.9 | 10 | 10 | 10 | 10 | 10 | 10 |
| Free Space Propagation Losses (dB) | 113.0 | 115.5 | 117.4 | 119.0 | 120.3 | 121.5 | 122.5 | 123.4 |
| Terrestrial LTE antenna Gain (dBI) | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 | -1.8 |
| Maximum Received Noise by g-NodeB (dBm/5 MHz) | -108.1 | -108.4 | -109.2 | -110.8 | -112.1 | -113.3 | -114.3 | -115.2 |
| System Noise Level, reference values (dBm/5 MHz) | -102 | -102 | -102 | -102 | -102 | -102 | -102 | -102 |
| Increase of the noise floor at g-NodeB with respect to reference values (dB) | 0.95 | 0.90 | 0.75 | 0.54 | 0.40 | 0.31 | 0.25 | 0.20 |

Table 40 shows that the increase of noise remains below 1 dB.

The following table provides the minimum fuselage attenuation required in order not to exceed the 1 dB increase of noise floor, assuming the onboard LTE UE transmits at 10 dBm/channel.

Table 41: Minimum required attenuation

| Height above ground (m) | Minimum required effective attenuation of signals to and from the ac-UE (dB) |
| --- | --- |
| 3000 | 3.1 |
| 4000 | 0.6 |
| 5000 | 0 |
| 6000 | 0 |
| 7000 | 0 |
| 8000 | 0 |

## COMPATIBILITY ANALYSIS AT 800 MHZ

The following parameters are used in the calculation:

  - Node B

e.i.r.p :  64 dBm/10 MHz

The Node B sensitivity:  -101.5 dBm

Antenna gain : 15 dBi

  - UE

e.i.r.p.: 23 dBm/10 MHz

The sensitivity of the UE is -97 dBm/10 MHz

Antenna gain of the UE is 0 dBi

  - Aircraft attenuation

5 dB

### Scenario 1: Impact of g-NodeB on ac-UE

The worst case elevation angle considered for the study at 800 MHz is 48° whatever the height above ground of the aircraft. The antenna gain is -0.34 dBi.

Table 42: Margin for protection of ac-UE from terrestrial networks

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / base station (km) | Path loss (dB) | Ant. Gain (dBi) at given angle | LTE 800 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | e.i.r.p. (dBm) | Max. received power in aircraft, Pmax_rec:ac-MS (dBm/ch) | Margin(dB) |
| 3000 | 48 | 4.04 | 102.6 | -0.34 | 48.66 | -58.92 | -38.08 |
| 4000 | 48 | 5.38 | 105.1 | -0.34 | 48.66 | -61.42 | -35.58 |
| 5000 | 48 | 6.73 | 107.0 | -0.34 | 48.66 | -63.35 | -33.66 |
| 6000 | 48 | 8.07 | 108.6 | -0.34 | 48.66 | -64.94 | -32.06 |
| 7000 | 48 | 9.42 | 109.9 | -0.34 | 48.66 | -66.24 | -30.76 |
| 8000 | 48 | 10.76 | 111.1 | -0.34 | 48.66 | -67.44 | -29.56 |
| 9000 | 48 | 12.10 | 112.1 | -0.34 | 48.66 | -68.44 | -28.56 |
| 10000 | 48 | 13.45 | 113.0 | -0.34 | 48.66 | -69.34 | -27.66 |

A negative margin means that an extra isolation is necessary to remove the visibility of the ground networks.

### Scenario 2: Impact of ac-UE on g-NodeB at 800 MHz

This scenario assesses in which conditions the onboard ac-UE will have the ability to connect to terrestrial networks.

Table 43: Impact of ac-UE on g-NodeB at 800 MHz

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / g_UE (km) | Path loss (dB) | Rx Ant. Gain (dBi) at given angle | LTE 800 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | e.i.r.p. (dBm) | Max. received power on ground, Pmax_rec:_g_node B (dBm/ch) | Margin(dB) |
| 3000 | 48 | 4.04 | 102.6 | -0.34 | 23 | -84.94 | -16.56 |
| 4000 | 48 | 5.38 | 105.1 | -0.34 | 23 | -87.44 | -14.06 |
| 5000 | 48 | 6.73 | 107.0 | -0.34 | 23 | -89.34 | -12.16 |
| 6000 | 48 | 8.07 | 108.6 | -0.34 | 23 | -90.94 | -10.56 |
| 7000 | 48 | 9.42 | 109.9 | -0.34 | 23 | -92.24 | -9.26 |
| 8000 | 48 | 10.76 | 111.1 | -0.34 | 23 | -93.44 | -8.06 |
| 9000 | 48 | 12.1 | 112.1 | -0.34 | 23 | -94.44 | -7.06 |
| 10000 | 48 | 13.45 | 113.0 | -0.34 | 23 | -95.34 | -6.16 |

A negative margin shows that it is possible that an UE could connect to a ground-based mobile network.

### Scenario 3: Impact of the NCU on g-UE at 800 MHz

This scenario assesses the impact of onboard NCU emissions on the ground-based UE receivers, by using MCL calculations.

Table 44: Impact of a signal NCU to terrestrial LTE network

| Height above ground (km) | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max received Signal Level (dBm/channel) inside aircraft | -58.92 | -61.44 | -63.34 | -64.94 | -66.24 | -67.44 | -68.44 | -69.34 |
| Radiation Factor (Large Aircraft) (dB) | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
| Aircraft Attenuation for leaky feeder transmission (dB) | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Equivalent e.i.r.p. (as point of source) (dBm/10 MHz) | -4.92 | -7.44 | -9.34 | -10.94 | -12.24 | -13.44 | -14.44 | -15.34 |
| Free Space Propagation Losses (dB) | 100.00 | 102.50 | 104.44 | 106.02 | 107.36 | 108.52 | 109.55 | 110.46 |
| Maximum Received Noise by g-UE (dBm/channel) | -104.92 | -109.94 | -113.78 | -116.96 | -119.60 | -121.96 | -123.99 | -125.80 |
| System Noise Level, reference values (dBm/channel) | -95 | -95 | -95 | -95 | -95 | -95 | -95 | -95 |
| Increase of the noise floor at g-UE with respect to reference values (dB) | 0.42 | 0.14 | 0.06 | 0.03 | 0.02 | 0.01 | 0.01 | 0.00 |

From the results of Table 44, it is then possible to calculate, for different height above ground of the aircraft what the equivalent e.i.r.p. of the NCU should be to get 1 dB increase of noise floor at ground UE. These values are contained in Table 45.

Table 45: maximum e.i.r.p. of the NCU

| Height above ground (km) | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equivalent e.i.r.p. (dBm/10 MHz) | -0.87 | 1.63 | 3.57 | 5.15 | 6.49 | 7.65 | 8.68 | 9.59 |

SEAMCAT results

- The following parameters used in SEAMCAT are as follow:

the total number of resource blocks (RB) is fixed and depends on the LTE channel bandwidth. In the case of a 10 MHz channel bandwidth, the maximum number of RBs is 50

the number of active users is 1 or 3

the number of RB allocated to a terminal depends on the number of terminals operating at the same time within a cell/sector.

the cell radius is 8.633 km.

- The Table 46 below provides the results of SEAMCAT simulations for average capacity loss

Table 46: Average capacity loss

| Situation |  | Reference Cell |  | OFDMA System |  |
| --- | --- | --- | --- | --- | --- |
| Description of the case |  | Average capacity loss | Average bitrate loss | Average capacity loss | Average bitrate loss |
| NCU transmitting in the 800 MHz band over terrestrial LTE networks | Transmitter placed randomly within a radius of 17 km at 3 km above ground | 0 % | 0.001 % | 0 % | 0 % |
|  | Transmitter placed randomly within a radius of 28 km at 5 km above ground | 0 % | 0.02 % | 0 % | 0.001 % |
|  | Transmitter placed randomly within a radius of 45 km at 8 km above ground | 0% | 0.002% | 0% | 0.001% |
|  | Transmitter placed randomly within a radius of 56 km at 10 km above ground | 0% | 0.002% | 0% | 0.001% |

### Scenario 4: impact of multiple NCU on g-UE at 800 MHz

The maximum number of resource blocks for a 10 MHz channel bandwidth is 50. In the simulation, 3 users were defined.

The cell radius for rural case is 8.66 km.

Table 47 provides the result for the scenario 4.

Table 47: simulation result for scenario 4

| Description of the case |  |  | Reference cell |  | OFDMA system |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Average capacity loss | Average bitrate loss | Average capacity loss | Average bitrate loss |
| Scenario 4 (800 MHz) | Multiple NCU to terrestrial LTE network | Normal day (18 interferers) | 0% | 0.006% | 0 % | 0,003 % |
|  | Multiple NCU to terrestrial LTE network | Extreme busy day (33 interferers) | 0% | 0.01% | 0 % | 0,004 % |

The result shows that the average capacity loss remains below 1%.

## COMPATIBILITY ANALYSIS AT 900 MHZ

### Scenario 1: Impact of g-NodeB on ac-UE at 900 MHz

This scenario assesses in which conditions the ac-UE will have visibility of the terrestrial networks, by using MCL calculations.

The excel file attached to this document provides details of calculation for elevation angles going from 0° to 90°.

From the calculation for different elevation angles, the worst case elevation angle considered for the study at 900 MHz is 48° whatever the height above ground of the aircraft. The relative antenna gain is -1.84dBi.

Table 48: Impact of g-NodeB on ac-UE at 900 MHz

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / base station (km) | Path loss (dB) | Ant. Gain (dBi) at given angle | LTE 900 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | e.i.r.p. (dBm) | Max. received power in aircraft, Pmax_rec:ac-MS (dBm/ch) | Margin(dB) |
| 3000 | 48 | 4.04 | 103.9 | -1.84 | 41.16 | -67.7 | -32.3 |
| 4000 | 48 | 5.38 | 106.4 | -1.84 | 41.16 | -70.2 | -29.8 |
| 5000 | 48 | 6.73 | 108.3 | -1.84 | 41.16 | -72.1 | -27.9 |
| 6000 | 48 | 8.07 | 109.9 | -1.84 | 41.16 | -73.7 | -26.3 |
| 7000 | 48 | 9.42 | 111.3 | -1.84 | 41.16 | -75.1 | -24.9 |
| 8000 | 48 | 10.76 | 112.4 | -1.84 | 41.16 | -76.2 | -23.8 |
| 9000 | 48 | 12.10 | 113.4 | -1.84 | 41.16 | -77.2 | -22.8 |
| 10000 | 48 | 13.45 | 114.3 | -1.84 | 41.16 | -78.1 | -21.9 |

A negative margin means that an extra isolation is necessary to remove the visibility of the ground networks.

### Scenario 2: Impact of ac-UE on g-NodeB

This scenario assesses in which conditions the onboard ac-UE will have the ability to connect to terrestrial networks.

Table 49: impact of ac-UE on g-NodeB at 900 MHz

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / g_UE (km) | Path loss (dB) | Rx Ant. Gain (dBi) at given angle | LTE 900 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | e.i.r.p. (dBm) | Max. received power on ground, Pmax_rec:_g_node B (dBm/ch) | Margin(dB) |
| 3000 | 48 | 4.04 | 103.9 | -1.84 | 23 | -87.74 | -13.76 |
| 4000 | 48 | 5.38 | 106.4 | -1.84 | 23 | -90.24 | -11.26 |
| 5000 | 48 | 6.73 | 108.3 | -1.84 | 23 | -92.14 | -9.36 |
| 6000 | 48 | 8.07 | 109.9 | -1.84 | 23 | -93.74 | -7.76 |
| 7000 | 48 | 9.42 | 111.3 | -1.84 | 23 | -95.14 | -6.36 |
| 8000 | 48 | 10.76 | 112.4 | -1.84 | 23 | -96.24 | -5.26 |
| 9000 | 48 | 12.1 | 113.4 | -1.84 | 23 | -97.24 | -4.26 |
| 10000 | 48 | 13.45 | 114.3 | -1.84 | 23 | -98.14 | -3.36 |

A negative margin shows that it is possible that an UE could connect to a ground-based mobile network.

### Scenario 3: Impact of the NCU on g-UE

In this frequency band, the ECC/DEC/(06)07 [1] provides the maximum e.i.r.p. defined outside the aircraft. At the first stage, the minimum value needed to screen the LTE ground network should be defined and calculate what the increase of noise floor will be.

Table 50: MCL result of impact of the NCU on g-UE

| Height above ground (km) | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max received Signal Level (dBm/channel) inside aircraft | -67.7 | -70.2 | -72.1 | -73.7 | -75.1 | -76.2 | -77.2 | -78.1 |
| Radiation Factor (Large Aircraft) (dB) | 64 | 64 | 64 | 64 | 64 | 64 | 64 | 64 |
| Aircraft Attenuation for leaky feeder transmission (dB) | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Equivalent e.i.r.p. (as point of source) (dBm/channel) | -13.7 | -16.24 | -18.14 | -19.74 | -21.14 | -22.24 | -23.24 | -24.14 |
| Equivalent e.i.r.p. (as point of source) (dBm/200 kHz) | -27.72 | -30.22 | -32.12 | -33.72 | -35.12 | -36.22 | -37.22 | -38.12 |
| Free Space Propagation Losses (dB) | 100.00 | 102.50 | 104.44 | 106.02 | 107.36 | 108.52 | 109.55 | 110.46 |
| Maximum Received Noise by g-UE (dBm) | -113.74 | -118.74 | -122.58 | -125.76 | -128.50 | -130.76 | -132.79 | -134.60 |
| System Noise Level, reference values (dB/channel) | -98 | -98 | -98 | -98 | -98 | -98 | -98 | -98 |
| Increase of the noise floor at g-UE with respect to reference values (dB) | 0.114 | 0.036 | 0.015 | 0.007 | 0.004 | 0.002 | 0.001 | 0.001 |

The above table shows that the increase of noise floor at ground UE remains below 1 dB. It also shows that the value needed to screen the ground LTE 900 cellular network is below the e.i.r.p. limit defined in the ECC/DEC/(06)06 [9].

Instead of performing all the SEAMCAT simulations starting from the result contained in the above table, it is proposed to use the e.i.r.p. limit as contained in the ECC/DEC/(06)07 [1] and to perform only the scenario 4 in which several interferers will be taken into account. The result of this simulation will indicate whether this e.i.r.p. will have an impact on the ground LTE 900 network.

### Scenario 4: Impact of the NCU on g-UE

The e.i.r.p. used is the one as defined in the ECC/DEC/(06)07 [1], i.e.. -19 dBm/200 kHz at 3,000 MHz.

The maximum number of resource blocks for a 5 MHz channel bandwidth is 25, and a typical number of active users is 1 or 3.

The cell radius for rural case is 8.633 km.

Table 51 provides the result for the scenario 4.

Table 51: Simulation result for scenario 4

| Description of the case |  |  | Reference cell |  | OFDMA system |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Average capacity loss | Average bitrate loss | Average capacity loss | Average bitrate loss |
| Scenario 4 (900 MHz) | Multiple NCU to terrestrial LTE network | Normal day (18 interferers) | 0% | 0.005% | 0 % | 0,003 % |
|  | Multiple NCU to terrestrial LTE network | Extreme busy day (33 interferers) | 0 % | 0.009% | 0% | 0.004% |

## COMPATIBILITY ANALYSIS AT 1800 MHZ

### Scenario 1: Impact of g-base station on ac-UE at 1800 MHz

This scenario assesses in which conditions the ac-UE will have visibility of the terrestrial WIMAX networks, by using MCL calculations.

The excel file attached to this document provides details of calculation for elevation angles going from 0° to 90°. The worst case elevation angle is 48 °, corresponding to an antenna gain of -1.34 dBi.

Table 52: Impact of g-WIMAX base station on ac-UE at 1800 MHz

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / base station (km) | Path loss (dB) | Ant. Gain (dBi) at given angle | WIMAX1800 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | e.i.r.p. (dBm) | Max. received power in aircraft, Pmax_rec:ac-MS (dBm/ch) | Margin (dB) |
| 3000 | 48 | 4.04 | 109.9 | -1.34 | 38.66 | -76.2 | -18.6 |
| 4000 | 48 | 5.38 | 112.4 | -1.34 | 38.66 | -78.7 | -16.1 |
| 5000 | 48 | 6.73 | 114.3 | -1.34 | 38.66 | -80.6 | -14.2 |
| 6000 | 48 | 8.07 | 115.9 | -1.34 | 38.66 | -82.2 | -12.6 |
| 7000 | 48 | 9.42 | 117.2 | -1.34 | 38.66 | -83.6 | -11.2 |
| 8000 | 48 | 10.76 | 118.4 | -1.34 | 38.66 | -84.7 | -10.1 |
| 9000 | 48 | 12.10 | 119.4 | -1.34 | 38.66 | -85.7 | -9.1 |
| 10000 | 48 | 13.45 | 120.3 | -1.34 | 38.66 | -86.7 | -8.1 |

A negative margin means that an extra isolation is necessary to remove the visibility of the ground networks.

### Scenario 2: Impact of ac-UE on g-base station at 1800 MHz

This scenario assesses in which conditions the onboard ac-UE will have the ability to connect to terrestrial networks.

Table 53: impact of ac-UE on g-base station at 1800 MHz

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / g_UE (km) | Path loss (dB) | Rx Ant. Gain (dBi) at given angle | WIMAX1800 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | UE e.i.r.p. (dBm) | Max. received power on ground, Pmax_rec:_g_node B (dBm/ch) | Margin (dB) |
| 3000 | 48 | 4.04 | 109.9 | -1.34 | 22 | -94.21 | -4.09 |
| 4000 | 48 | 5.38 | 112.4 | -1.34 | 22 | -96.70 | -1.60 |
| 5000 | 48 | 6.73 | 114.3 | -1.34 | 22 | -98.64 | 0.34 |
| 6000 | 48 | 8.07 | 115.9 | -1.34 | 22 | -100.22 | 1.92 |
| 7000 | 48 | 9.42 | 117.2 | -1.34 | 22 | -101.56 | 3.26 |
| 8000 | 48 | 10.76 | 118.4 | -1.34 | 22 | -102.72 | 4.42 |
| 9000 | 48 | 12.1 | 119.4 | -1.34 | 22 | -103.74 | 5.44 |
| 10000 | 48 | 13.45 | 120.3 | -1.34 | 22 | -104.66 | 6.36 |

A negative margin shows that it is possible that an UE could connect to a ground-based mobile network.

### Scenario 3: Impact of the NCU on g-UE at 1800 MHz

In this frequency band, the ECC/DEC/(06)07 [1] provides the maximum e.i.r.p. defined outside the aircraft. At the first stage, the minimum value needed to screen WIMAX ground network should be defined and calculate what the increase of noise floor will be.

Table 54: MCL result of impact of the NCU on g-UE

| Height above ground (km)  | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max received Signal Level (dBm/channel) inside aircraft | -76.2 | -78.7 | -80.6 | -82.2 | -83.6 | -84.7 | -85.7 | -86.7 |
| Radiation Factor (Large Aircraft) (dB) | 70 | 70 | 70 | 70 | 70 | 70 | 70 | 70 |
| Aircraft Attenuation for leaky feeder transmission (dB) | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Equivalent e.i.r.p. (as point of source) (dBm/10 MHz) | -22.21 | -24.70 | -26.64 | -28.22 | -29.56 | -30.72 | -31.74 | -32.66 |
| Free Space Propagation Losses (dB) | 107.29 | 109.78 | 111.72 | 113.31 | 114.65 | 115.81 | 116.83 | 117.74 |
| Maximum Received Noise by g-UE (dBm/channel) | -129.50 | -134.48 | -138.37 | -141.53 | -144.21 | -146.52 | -148.57 | -150.40 |
| System Noise Level, reference values (dB/9.5 MHz) | -96.2 | -96.2 | -96.2 | -96.2 | -96.2 | -96.2 | -96.2 | -96.2 |
| Increase of the noise floor at g-MS with respect to reference values (dB) | 0.002 | 0.001 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |

The above table shows that the increase of noise floor at ground UE remains below 1 dB.

Instead of performing all the SEAMCAT simulations starting from the result contained in the above table, it is proposed to use the e.i.r.p. limit as contained in the ECC/DEC/ (06)07 [1] and to perform only the scenario 4 in which several interferers will be taken into account. The result of this simulation will indicate whether this e.i.r.p. will have an impact on the ground WIMAX network.

### Scenario 4: Impact of the NCU on g-WIMAX UE at 1800 MHz

The e.i.r.p. used is the one as defined in the ECC/DEC//(06)07 [1], i.e.. -13 dBm/200 kHz at 3,000 MHz.

The distribution of aircraft is the same as contained in the section 5.5.4.of this report.

The cell radius for rural case is 6 km.

Table 55 provides the result for the scenario 4.

Table 55: simulation result for scenario 4

| Description of the case |  |  | Reference |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Irss mean (dBm) | iRSS 95th percentile (dBm) | Criterion I: P(C/(N+I)< 9 dB) (%) | Criterion II: P(I/N>-6 dB) (%) |
| Scenario 4 (1800 MHz) | Multiple NCU to terrestrial WIMAX network | Normal day (18 interferers) | -106.4 | -102.9 | 0.08 | 2.51 |
|  | Multiple NCU to terrestrial WIMAX network | Extreme busy day (33 interferers) | -103.8 | -101.2 | 0.96 | 16.7 |

## COMPATIBILITY ANALYSIS AT 2100 MHZ

### Scenario 1: Impact of g-NodeB on ac-UE at 2100 MHz

This scenario assesses in which conditions the ac-UE will have visibility of the terrestrial networks, by using MCL calculations.

From the calculation for different elevation angles, the worst case elevation angle considered for the study at 2100 MHz is 48° whatever the height above ground of the aircraft. The relative antenna gain is -0.34 dBi

Table 56: Impact of g-NodeB on ac-UE at 2100 MHz

| Height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / base station (km) | Path loss (dB) | Aircraft height above ground (m) | LTE 2100 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | e.i.r.p. (dBm) | Max. received power in aircraft, Pmax_rec:ac-MS (dBm/ch) | Margin(dB) |
| 3000 | 48 | 4.04 | 111.2 | -1.84 | 44.16 | -72.0 | -25.0 |
| 4000 | 48 | 5.38 | 113.7 | -1.84 | 44.16 | -74.5 | -22.5 |
| 5000 | 48 | 6.73 | 115.6 | -1.84 | 44.16 | -76.4 | -20.6 |
| 6000 | 48 | 8.07 | 117.2 | -1.84 | 44.16 | -78.0 | -19.0 |
| 7000 | 48 | 9.42 | 118.5 | -1.84 | 44.16 | -79.4 | -17.6 |
| 8000 | 48 | 10.76 | 119.7 | -1.84 | 44.16 | -80.5 | -16.5 |
| 9000 | 48 | 12.1 | 120.7 | -1.84 | 44.16 | -81.5 | -15.5 |
| 10000 | 48 | 13.45 | 121.6 | -1.84 | 44.16 | -82.5 | -14.5 |

A negative margin means that an extra isolation is necessary to remove the visibility of the ground networks.

### Scenario 2: Impact of ac-UE on g-NodeB at 2100 MHz

This scenario assesses in which conditions the onboard ac-UE will have the ability to connect to terrestrial networks.

Table 57: impact of ac-UE on g-NodeB at 2100 MHz

| Aircraft height above ground (m) | Worst case elevation angle (deg) | Distance aircraft / g_UE (km) | Path loss (dB) | Rx Ant. Gain (dBi) at given angle | LTE 2100 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | e.i.r.p. (dBm) | Max. received power on ground, Pmax_rec:_g_node B (dBm/ch) | Margin(dB) |
| 3000 | 48 | 4.04 | 111.2 | -1.84 | 23 | -95.0 | -6.5 |
| 4000 | 48 | 5.38 | 113.7 | -1.84 | 23 | -97.5 | -4.0 |
| 5000 | 48 | 6.73 | 115.6 | -1.84 | 23 | -99.4 | -2.1 |
| 6000 | 48 | 8.07 | 117.2 | -1.84 | 23 | -101.0 | -0.5 |
| 7000 | 48 | 9.42 | 118.5 | -1.84 | 23 | -102.4 | 0.9 |
| 8000 | 48 | 10.76 | 119.7 | -1.84 | 23 | -103.5 | 2.0 |
| 9000 | 48 | 12.10 | 120.7 | -1.84 | 23 | -104.5 | 3.0 |
| 10000 | 48 | 13.45 | 121.6 | -1.84 | 23 | -105.5 | 4.0 |

A negative margin shows that it is possible that an UE could connect to a ground-based mobile network.

### Scenario 3: Impact of the NCU on g-UE at 2100 MHz

In this frequency band, the ECC/DEC/(06)07 [1] provides the maximum e.i.r.p. defined outside the aircraft. At the first stage, the minimum value needed to screen the LTE ground network should be defined and calculate what the increase of noise floor will be.

Table 58: MCL result of impact of the NCU on g-UE

| Height above ground (km) | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max received Signal Level (dBm/channel) inside aircraft | -72.0 | -74.5 | -76.4 | -78.0 | -79.4 | -80.5 | -81.5 | -82.5 |
| Radiation Factor (Large Aircraft) (dB) | 71 | 71 | 71 | 71 | 71 | 71 | 71 | 71 |
| Aircraft Attenuation for leaky feeder transmission (dB) | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| Equivalent e.i.r.p. (as point of source) (dBm/channel) | -11.0 | -13.5 | -15.4 | -17.0 | -18.4 | -19.5 | -20.5 | -21.5 |
| Free Space Propagation Losses (dB) | 108.6 | 111.1 | 113.0 | 114.6 | 116.0 | 117.1 | 118.1 | 119.0 |
| Maximum Received Noise by g-UE (dBm) | -119.6 | -124.6 | -128.5 | -131.6 | -134.3 | -136.6 | -138.7 | -140.5 |
| System Noise Level, reference values (dB/channel) | -95 | -95 | -95 | -95 | -95 | -95 | -95 | -95 |
| Increase of the noise floor at g-UE with respect to reference values (dB) | 0.015 | 0.005 | 0.002 | 0.001 | 0.001 | 0.000 | 0.000 | 0.000 |
| Equivalent e.i.r.p. (as point of source) (dBm/ 200 kHz) | -28.01 | -30.49 | -32.44 | -34.02 | -35.36 | -36.51 | -37.53 | -38.45 |

The above table shows that the increase of noise floor at ground UE remains below 1 dB. It also shows that the value needed to screen the ground LTE 2100 cellular network is below the e.i.r.p. limit defined in the ECC/DEC/(06)07 [1].

Instead of performing all the SEAMCAT simulations starting from the result contained in the above table, it is proposed to use the e.i.r.p. limit as contained in the ECC/DEC/(06)07 [1] and to perform only the scenario 4 in which several interferers will be taken into account. The result of this simulation will indicate whether this e.i.r.p. will have an impact on the ground LTE 2100 network.

### Scenario 4: Impact of the NCU on g-UE at 2100 MHz

The e.i.r.p. used is the one as defined in the ECC/DEC/(06)07 [1], i.e. 1 dBm/ 3.84 MHz at 3000 m.

Table 59: victim link parameters

| SINR minimum |  | -10 dB |  |
| --- | --- | --- | --- |
| Max subcarriers per base station |  | 48 |  |
| Number of subcarriers per mobile |  | 16 |  |
| Handover margin |  | 1dB |  |
| Minimum coupling loss |  | 80 dB |  |
| User per BS |  | 3 |  |
| Antenna gain |  | 18 dBi |  |
| Radiation pattern |  | # Recommendation ITU-R F.1336-3 section 3.2 [8] |  |
| Cell radius |  | 6 km |  |

Table 60: interfering link parameters

| Power level | 5.16 dBm/10 MHz |
| --- | --- |
| Antenna height | See Table 15 |
| Number of interferer | 18 for normal day 33 for busy day |
| Simulation radius | 56 km |

Table 61 provides the result for the scenario 4.

Table 61: simulation result for scenario 4

| Description of the case |  |  | Reference cell |  | OFDMA system |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Average capacity loss | Average bitrate loss | Average capacity loss | Average bitrate loss |
| Scenario 4 (2100 MHz) | Multiple NCU to terrestrial LTE network | Normal day (18 interferers) | 0% | 0.005% | 0 % | 0.003 % |
|  | Multiple NCU to terrestrial LTE network | Extreme busy day (33 interferers) | 0 % | 0.009% | 0% | 0.005% |

## PROTECTION OF ADJACENT SERVICES

### RADIO ASTRONOMY SERVICES IN THE 2690 – 2700 MHZ

RAS protection requirements

For the bands in question, the appropriate threshold of interference level of spectral power flux density taken from Table 1 (continuum observations) of Recommendation ITU-R RA.769-2 [4] is -247 dB(W/m2.Hz), which equates to a maximum interference power level in a notional 10 MHz bandwidth of -177 dBm. This threshold of interference level is also based on an assumed observational integration time of 2000 s. Continuum observations made with single-dish telescopes commonly undertaken in European observatories are well characterised by these parameters.

With the assumptions noted, the worst case scenario for interference at the RAS observatory will be from an aircraft flying directly over the observatory at the minimum height at which the system is allowed to operate; from ECC Report 093 [2] this is assumed to be 3000 m.

ECC Report 093 [2] considers the possibility of obtaining an equivalent emitted power from the aircraft treated as a point source – i.e. effectively the power level ‘outside the aircraft’. This is useful in the RAS situation; the path loss between the aircraft and the observatory can be calculated and the threshold level of interference detrimental to RAS operation is given in the paragraph above.

Since the aircraft is in line of sight of the observatory, at these frequencies the path loss ‘L’ may be calculated to a reasonable approximation based on the free space path loss equation (i.e. For 3000 m Height above ground at 2695 MHz, L = 110.6 dB). For the scenario stated, the power ‘Pext’ outside the aircraft at 3000 m falling into the band must therefore be less than:

Pext = -177 + 110.6 = -66.4 dBm/10 MHz

This is the ‘single entry’ worst case and requires modification subject to the likely density of aircraft around the observatory, which could produce a significant continuous additional background noise level. This situation is under consideration.

### RADAR SERVICES OPERATING ABOVE 2700 MHZ

The impact of MCA system operating in 2500-2690 MHz band on radar system above 2700 MHz band was assessed. This analysis assumes radar performance parameters identical to ECC Report 174 [3]. Those parameters are presented in Table 20.

AC-nodeB

The AC-nodeB related technical characteristics are gathered from ECC Report 093 [2]

According to ECC Report 093 [2], there are 3 suggested aircraft attenuation levels as shown in Table 62.

Table 62: Values of "attenuation due to the aircraft"

| Case | Ac-BTS/NCU signal attenuation (dB) |
| --- | --- |
| A (low) | 5 |
| B (medium) | 10 |
| C (high) | 15 |

In this report, the worst case scenario applies with the least attenuation due to the aircraft, i.e. 5 dB.

- Compatibility study between ac-node B (LTE 2600) and Radar system in the band 2700-2900 MHz

In this scenario, the impact of the ac-node B on the Radar systems was studied. Table 20 provides the parameters needed for the radio frequency interference analysis. Here, the increase in noise floor at the victim receiver (radar) is calculated for the different types (Type 1 – 4) of radar.

where,

: Power spectral density (dBm/MHz) received at victim receiver

: Power spectral density (dBm/MHz) transmitted at interfering transmitter (1.9 dBm/4.75 MHz)

: Antenna gain at the interfering transmitter (dBi) – this value is not considered as it is already contained in the

: Antenna gain at the victim receiver (dBi)

: Adjacent Channel Leakage Ratio (45 dB, 3GPP TS 36.104)

: Aircraft attenuation

: Free space path loss

CB : 4G channel bandwidth (4.75 MHz, worst case)

Further,

where,

: I+N/N ratio at victim receiver

: Reference noise floor at victim receiver (dBm/MHz)

The power spectral density (PSD) (dBm/MHz) received at the victim receiver,  is calculated for distance between interfering transmitter (ac-BTS) and victim receiver (radar) from 3000 m to 10000 m.

The results are summarised in Table 63 and the respective I+N/N ratios at the victim receiver are summarised in Table 64 taking into account = -122 dBm/MHz (from Table 20).

Results

Table 63: Power Spectral Density at victim receiver (radar) from 3000 m to 10000 m and (I+N)/N

| Distance (m) |  | Free Space Path Loss (from onboard equipment to victim receiver) |  | Pv-Rx |  |  | (I+N)/N | Pv-Rx |  |  | (I+N)/N |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Type 1 |  | Type 2 and 3 |  |  |
| 3000 |  | 110.36 |  | -141.3 |  | 0.051 |  |  | -147.3 |  |  | 0.013 |
| 4000 |  | 112.86 |  | -143.8 |  | 0.029 |  |  | -149.8 |  |  | 0.007 |
| 5000 |  | 114.80 |  | -145.7 |  | 0.018 |  |  | -151.7 |  |  | 0.005 |
| 6000 |  | 116.38 |  | -147.3 |  | 0.013 |  |  | -153.3 |  |  | 0.003 |
| 7000 |  | 117.72 |  | -148.6 |  | 0.009 |  |  | -154.6 |  |  | 0.002 |
| 8000 |  | 118.88 |  | -149.8 |  | 0.007 |  |  | -155.8 |  |  | 0.002 |
| 9000 |  | 119.90 |  | -150.8 |  | 0.006 |  |  | -156.8 |  |  | 0.001 |
| 10000 |  | 120.82 |  | -151.7 |  | 0.004 |  |  | -157.7 |  |  | 0.001 |

With respect to the radar type 4, the worst case is when the aircraft is at 37° elevation angle from the radar, and the elevation angle of the radar antenna is at 37°.

Table 64: Power Spectral Density at victim receiver (radar) from 3000 m to 10000 m and (I+N)/N for radar type 4

| Distance (m) | Free Space Path Loss (from onboard equipment to radar) | Pv-Rx | I+N/N |
| --- | --- | --- | --- |
| 3000 | 114.81 | -128.68 | 0.84 |
| 4000 | 117.31 | -131.18 | 0.50 |
| 4500 |  | 118,33 |  | -132,20 |  | 0,40 |  |
| 5000 | 119.25 | -133.12 | 0.32 |
| 6000 | 120.83 | -134.70 | 0.23 |
| 7000 | 122.17 | -136.04 | 0.17 |
| 8000 | 123.33 | -182.19 | 0.00 |
| 9000 | 124.35 | -138.22 | 0.10 |
| 10000 | 125.26 | -139.13 | 0.08 |

From the protection criteria for Radar I/N = -10dB (Recommendation ITU-R M.1464-1) [5] it is derived the criterion I+N/N = 0.41dB. Reviewing the results in Table 52 and Table 64, it shows that the increase of noise floor at the victim receiver (i.e. radar) never exceeds the protection level for Type 1,2 and 3. For type 4, connectivity could be offered above 4500 meters above ground. As such, it can be concluded that the band 2620-2690 MHz can be used for connectivity.

# Conclusions

This report described additional studies on the compatibility of a MCA system with terrestrial networks, when the aircraft is at least 3000 m above ground. The studies demonstrated that harmful interference to terrestrial networks will not occur provided that the following technical conditions are met:

In the 1800 MHz connectivity (LTE technology): the e.i.r.p. defined outside the aircraft, resulting from the LTE terminal transmitting at 5 dBm/5 MHz inside the aircraft must not exceed the values as provided  in the table below:

| Height above ground (m) | Maximum e.i.r.p, defined outside the aircraft, resulting from the ac-UE in (dBm/5 MHz) |
| --- | --- |
| 3000 | 1.7 |
| 4000 | 3.9 |
| 5000 | 5 |
| 6000 | 5 |
| 7000 | 5 |
| 8000 | 5 |

- the transmit power of ac-NodeB must not exceed the maximum e.i.r.p. defined outside the aircraft as provided in the table below:

| Height above ground (m) | Maximum e.i.r.p, defined outside the aircraft, resulting from the ac-NodeB (dBm/5 MHz) |
| --- | --- |
| 3000 | 1.0 |
| 4000 | 3.5 |
| 5000 | 5.5 |
| 6000 | 7.1 |
| 7000 | 8.4 |
| 8000 | 9.6 |

in the 2100 MHz connectivity band (UMTS technology);

- The transmit power of the ac-Node B must not exceed the maximum e.i.r.p defined outside the aircraft as given in the ECC/DEC(06)07;

- The transmit power of the UMTS terminal must not exceed --6 dBm/3.84 MHz and the maximum number of users should not exceed 20.

- The e.i.r.p.of the ac-UE defined outside the aircraft must not exceed the following values as shown in the table below:

Table 66: xxx

| Height above ground (km) | Max permitted e.i.r.p. (dBm/channel) |
| --- | --- |
| 3 | 3.1 |
| 4 | 5.6 |
| 5 | 7 |
| 6 | 7 |
| 7 | 7 |
| 8 | 7 |

In the 2600 MHz connectivity band (LTE technology);

Table 67: xxx

| Minimum operational height above ground (m) | Maximum permitted e.i.r.p. produced by the onboard LTE device (dBm/5 MHz) | Maximum permitted e.i.r.p. (dBm/5 MHz) |
| --- | --- | --- |
| 3000 | 6.9 | 2.1 |
| 4000 | 9.4 | 4.6 |
| 5000 | 10 | 6.5 |
| 6000 | 10 | 8.1 |
| 7000 | 10 | 9.5 |
| 8000 | 10 | 10.6 |

It should be noted that the connectivity service in the 2600 MHz band can only be provided at the heights above 4500 m due to compatibility requirements with radars.

Additionally, compatibility with the adjacent band Radio astronomy service can be achieved assuming that the out-of-band emission outside the aircraft is lower than – 66.4 dBm/10 MHz at 3000 metres. To achieve compatibility with the RAS secondary allocation in the shared band at 2655-2690 MHz would require the same limit on emissions. It should be mentioned that the ECC/DEC/(06)07 [1] already provided a maximum e.i.r.p. that could be delivered by the NCU outside the aircraft in the band 2620-2690 MHz

With respect to the controlled NCU bands, the studies have shown that there is no change in the power levels defined outside the aircraft for the frequency bands at 460 MHz, 900 MHz, 1800 MHz and 2100 MHz as provided in the Commission Decision 2008/294/EC [6] and no change to the 2600 MHz power level for the NCU defined in  ECC/DEC/(06)07[1] – value based on the studies contained in the Annex G of ECC Report 093 [2].

In the 800 MHz band, the e.i.r.p. of the NCU should not exceed the value contained in the below table:

Table 68: NCU e.i.r.p. limits

| Height above ground (km) | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equivalent e.i.r.p. (dBm/10 MHz) | -0.87 | 1.63 | 3.57 | 5.15 | 6.49 | 7.65 | 8.68 | 9.59 |

Example of system activation

The ECC/DEC/(06)07 [1] provides the technical parameters related to the MCA system. Such onboard service could only be provided to airline passengers during the cruise phase and, above 3.000 meter above the ground, as represented in Figure 11.

Descent

Taxi out & take-off

Cruise

Climb

Approach

Landing & taxi in

Service active

Min 3,000m above ground

Figure 11: Provision of GSM/GPRS services to passengers onboard aircraft

The height above sea level (i.e. altitude), the actual position (longitude and latitude) of the aircraft is given to the MCA system via input from the aircraft avionics.

The MCA system had access to a geographical database where the ground elevation height is registered according to GPS locations. The granularity of the ground height elevation map is a square of 10 km * 10 km and each square provides the highest elevation point (height above sea level).

Based on the information (GPS location and altitude) received from the avionics system, the MCA system will subtract the altitude parameter from the avionics input with the ground elevation height corresponding to the map location in order to calculate the effective height above ground for that 10 km * 10 km area.

An example is shown in the diagram below:

Figure 12: Example of Height Calculation of “Mobile OnAir” system over mountainous terrain

This approach ensures that the MCA system does not transmit lower than 3,000 metres height above ground. The 10 km granularity taking the highest point smoothes out variations in terrain whilst ensuring conformance to height limits.

On comparing this approach to the actual height elevation of Austria, for example, this actually means that in some places where Alp mountains reach 3,200 metres elevation above the sea level, the minimum altitude to activate the system would be at above 6,200 metres above sea level.

List of reference

- ECC Decision (06)07 on the harmonised use of airborne GSM systems in the frequency bands 1710-1785 and 1805-1880 MHz

ECC Report 093 on Compatibility between GSM equipment on board aircraft and terrestrial networks”

ECC Report 174 on Compatibility between the mobile service in the band 2500-2690 MHz and the radiodetermination service in the band 2700-2900 MHz”

Recommendation ITU-R RA.769-2 on Protection criteria used for radio astronomical measurements

Recommendation ITU–R M.1464-1 on Characteristics of radiolocation radars, and characteristics and protection criteria for sharing studies for aeronautical radionavigation and meteorological radars in the radiodetermination service operating in the frequency band 2 700-2 900 MHz”

Commission Decision 2008/294/EC on harmonised conditions of spectrum use for the operation of mobile communication services on aircraft (MCA services) in the Community

Recommendation ITU-R M.1849 Technical and operational aspects of ground-based meteorological radars

Recommendation ITU-R F.1336-3 Reference radiation pattern for omnidirectional, sectoral and other antennas in point-to-multipoint systems for use in sharing studies in the frequency range from 1 GHz to about 70 GHz

ECC Decision (06)06 on the availability of frequency bands for the introduction of Narrow Band Digital Land Mobile PMR/PAMR in the 80 MHz, 160 MHz and 400 MHz bands

Recommendation ITU-R F.1245-2 Mathematical model of average and related radiation patterns for line-of-sight point-to-point fixed wireless system antennas for use in certain coordination studies and interference assessment in the frequency range from 1 GHz to about 70 GHz

Recommendation ITU-R M.1851 Mathematical models for radiodetermination radar systems antenna patterns for use in interference analyses

ECC Report 045 on Sharing and adjacent band compatibility between UMTS/IMT-2000 in the band 2500-2690 MHz and other services