	[image: image1.wmf]ECC

Electronic Communications Committee

CEPT


	Doc. ECC/SE(13) TEMP012


	Working Group SE
	

	63rd Meeting of WG SE

Lübeck, Germany, 28 January – 1 February 2013
	

	
	

	Date issued: 
	22 January 2013

	Source:
	Germany

	Subject:
	ITU-R-Contribution WGSE on CRS: Studies on cognitive radio systems (White Space Devices) in Europe


	Password protection required? (Y/N)


	


	Radiocommunication Study Groups
	[image: image2.png]




	
	

	
	

	Received:
Day Month Year
Subject:
 Resolution ITU-R 58
	Document 1A/XXX-E

	
	Document 1B/XXX-E

Day Month Year

	
	English only

	[Germany (Federal Republic of), Portugal] 



	STUDIES on Cognitive Radio Systems (White Space Devices) in Europe 


	


Introduction

Resolution ITU-R 58 (RA-12) invites for studies on cognitive radio systems (CRS). Studies from a more general perspective are falling into the scope of Study Group 1 (spectrum management). Working Party 1A will contribute on spectrum engineering techniques (see 1A/39 Annex 11) and Working party 1B on spectrum management methodologies and economic strategies (see 1B/42 Annex 14). 

Within its framework CEPT / ECC / WGSE performed the following comprehensive technical studies for cognitive radio systems deployed in the white space devices of the frequency band 470-790 MHz which are also related to Resolution ITU-R 58: ECC Report 159 [1]; ECC Report 185 [2] and ECC Report 186 [3].

· 
· 
· 
Definitions used in the above mentioned ECC Reports 
White Space:
(From CEPT Report 24) 
‘White Space’ is a label indicating a part of the spectrum, which is available for a radiocommunication application (service, system) at a given time in a given geographical area on a non-interfering /non-protected basis with regard to other services with a higher priority on a national basis.
Cognitive Radio:
(From ITU-R Report SM.2152 ) 
Cognitive radio system (CRS): A radio system employing technology that allows the system to obtain knowledge of its operational and geographical environment, established policies and its internal state; to dynamically and autonomously adjust its operational parameters and protocols according to its obtained knowledge in order to achieve predefined objectives; and to learn from the results obtained.

White space device:
White space devices (WSDs) are devices that can use White Space spectrum without causing harmful interference to protected services by employing required cognitive capabilities.
Incumbent radio services/systems
In the reports mentioned above is analysed the protection of the following incumbent radio services/systems:
· Terrestrial Broadcasting Service (BS) including DVB-T in particular.

· Program Making and Special Event (PMSE) systems including radio microphones in particular.

· Radio Astronomy Service (RAS) in the 608-614 MHz band.

· Aeronautical Radio Navigation Service (ARNS) in the 645–790 MHz band.

· Mobile Service (MS) below 470 MHz and above 790 MHz.
Summary of the above mentioned ECC Reports 
The ECC Report 159 [1], as the starting point of the investigations on CRS, carried out studies on the compatibility of WSD with some of the incumbent radio services / systems listed above. These studies were focussed on stand alone, sensing based WSD. This report concluded a list of areas requiring further work, summarized as follows:
· Areas related to WSD characteristics

· Technical considerations on the protection of the broadcasting service
· Technical considerations on the protection of PMSE
· Specification and implementation of the requirements for the geo-location database approach
In following to this, the output of the ECC Report 185 [2], which complements the analysis of the ECC Report 159 [1], is as follows:

a) Classification of White Space Devices (WSD) and possible approaches to set up fixed maximum permitted power limits for WSDs, 

b) Considerations on the collaborative spectral sensing to overcome site-specific bad channel conditions;

c) Protection of broadcasting: 
Complementary analysis on the sensitivity of the basic parameters (location probability, coverage assessment) and the performance of DTT receivers in the presence of WSDs;

d) Protection of PMSE 
e) Protection of ARNS
f) Impact of WSD interference on mobile services operating in adjacent bands to 470-790 MHz

g) Some Examples of national investigations on spectrum potentially available for WSDs

As a completion of the ECC Report 185 [2], the ECC Report 186 [3] was developed to analyse the technical and operational requirements for the operation of WSD under the geo-location database, namely:

:

a) Considerations on location accuracy of WSD
b) general principles and operational requirements to WSD operating under the master/slave concept

c) Database management

d) Examples of the translation process in the geo-location database to protect incumbent radio services/systems
e) Considerations on the combination of sensing and geo-location
It should be noted that these reports are mainly for guidance for administrations within the CEPT possibly planning to introduce cognitive radio systems within the frequency range 470 – 790 MHz on a national basis.

It should also be noted that a harmonization of the requirements of the geo-location database and its interfaces is in progress within the framework of ETSI.

Proposal

ITU-R SG 1, WP 1A and WP1B are invited to consider these studies and use relevant generic parts for their work requested by Resolution ITU-R 58 and, if felt necessary, also to inform those Study Groups addressed by resolves 2 of the Resolution ITU-R 58.

References

[1]
ECC Report 159:  editorial note: all links to be updated accordingly
TECHNICAL AND OPERATIONAL REQUIREMENTS FOR THE POSSIBLE OPERATION OF COGNITIVE RADIO SYSTEMS IN THE ‘WHITE SPACES’ OF THE FREQUENCY BAND 470-790 MHz

[2]
ECC Report 185:

Further definition of technical and operational requirements for the operation of white space devices in the band 470-790 MHz  

[3]
ECC Report 186:

Technical and operational requirements for the operation of white space devices under geo-location approach
Abbreviations

CEPT

European Conference of Postal and Telecommunications Administrations

ECC

Electronic Communications Committee

WG SE
Working Group on Spectrum Engineering
DTT

Digital Terrestrial Television

DVB-T

Digital Video Broadcasting – Terrestrial

ETSI

European Telecommunications Standards Institute

N








� This contribution has been developed and agreed within the framework of CEPT /ECC/ WGSE





1
WG SE – Luebeck, Germany – 28 January – 1 February 2013
5/5

