ECC Report 191

Adjacent band compatibility between MFCN and PMSE audio applications in the 1800 MHz range

Month YYYY (Arial 9pt bold)

# executive summary

## Studies included in the report

The purpose of this Report is to find conditions for operation of PMSE audio equipment (wireless microphones) in the frequency band 1785-1805 MHz. This frequency range corresponds to the duplex gap of the 1710-1785/1805-1880 MHz mobile band.

This report considers interference between PMSE equipment operating in the band 1785-1805 MHz and public mobile network equipment operating in the bands 1710-1785 MHz and 1805-1880 MHz.  The studies in this report consider mainly the interference scenarios where audio PMSE equipment interfere mobile, i.e. PMSE equipment interferes with mobile base station receiving below 1785 MHz and PMSE equipment interferes with mobile terminal receiving above 1805 MHz. The report considers a total of 16 scenarios corresponding to a specific combination of the following options:

Indoor/outdoor,

PMSE interferes MFCN or MFCN interferes PMSE,

MFCN BS or MFCN UE,

MFCN equipment is LTE or GSM.

## Methods carried out in the report

The report is based on several methods.

Method 1: Monte-Carlo simulations carried out with the SEAMCAT tool, provided in ANNEX 1:

Method 2: Minimum Coupling Loss (MCL) analysis, provided in ANNEX 2:

Method 3: Analysis based on the method adopted in CEPT Report 30 [3], provided in ANNEX 3:

## Results

### Results of the studies

The Method 1 (Monte-Carlo simulations, ANNEX 1) assumes specific PMSE equipment parameters (e.g. Spectrum emission mask, power, system bandwidth) and aims at deriving the needed restricted frequency range between the audio PMSE equipment and the mobile equipment in adjacent bands for PMSE using these parameters.

The methods in ANNEX 2: and ANNEX 3: aim at deriving a Block Edge Mask, with specific in-band and out of band allowed emission levels, without preliminary assumption on PMSE power, spectrum emission mask or system bandwidth.

### Limitations of the studies

It should be noted that the analyses are limited to cases where there is an interference risk (both audio PMSE equipment and mobile terminal/base station are in operation), without taking into account the probability of such scenario which is related to the market penetration of audio PMSE equipment and mobile systems. The analyses also consider MFCN equipment operating at the edge of their band, which may not always be the case, especially in the case of GSM.

The scenarios corresponding to mobile equipment (both terminals and base stations) interfering with audio PMSE equipment is not fully studied in this report. Preliminary considerations are provided through Monte-Carlo simulations, without providing a full analysis as they only consider analogue audio PMSE and do not consider the effect of duplex filtering in mobile base station and terminals. The conclusions of this report do not guarantee that audio PMSE equipment will be able to operate in the band, but only that, provided they can operate, they would not create interference to mobile systems in adjacent bands.

### BEM proposal for PMSE audio applications in the frequency band 1785-1805 MHz

Table 1: BEM for handheld microphone

|  | Frequency Range | Handheld e.i.r.p. | Reasoning |
| --- | --- | --- | --- |
| OOB | < 1785 MHz | -17 dBm/200kHz | LTE UE spectrum emission mask |
| Restricted frequency range | 1785-1785,2 MHz | 4 dBm/200kHz | GSM blocking |
|  | 1785,2-1803,6 MHz | 13 dBm/channel |  |
|  | 1803,6-1804,8 MHz | 10 dBm/200kHz* | slow increase of LTE UE selectivity |
| Restricted frequency range | 1804,8-1805 MHz | -14 dBm/200kHz | GSM blocking |
| OOB | > 1805 MHz | -37 dBm/200kHz | OOB calculation, in line with ERC/REC 74-01 |

* with a limit of 13 dBm/channel

Table 2: BEM for body worn microphone

|  | Frequency Range | Body worn e.i.r.p. | Reasoning |
| --- | --- | --- | --- |
| OOB | < 1785 MHz | -17 dBm/200kHz | LTE UE spectrum emission mask |
|  | 1785-1804,8 MHz | 17 dBm/channel |  |
| Restricted frequency range | 1804,8-1805 MHz | 0 dBm/200kHz | GSM blocking |
| OOB | > 1805 MHz | -23 dBm/200kHz | OOB calculation* |

* For the body worn case the body loss is 14 higher than for the handheld case, therefore the -23 dBm for body worn is equivalent to -37 dBm for handheld.

### Discussions of results for scenarios

The studied interference scenarios may not happen in many real cases, because the PMSE device cannot go in operation when suffering interference from base station or UE. See Table 20 in Annex 1, section A1.3. Further studies may be performed in this respect taking also into account the performance of current MFCN (e.g. considering the impact of the duplex filter) and PMSE equipment. Depending on the scenarios to be studied, the duplex filters may reduce the MFCN out of band emissions and thereby also the interference impact on PMSE devices.

It is suggested that in the indoor environment, the physical location of any pico station should be considered in relation to the PMSE installation in order to minimise mutual interference by use of wall loss or other mitigation techniques, which would reduce interaction.

Further studies may also be required to study the case of digital PMSE devices with different resistance to interference compared to analogue devices.

TABLE OF CONTENTS

0	executive summary	2

0.1	Studies included in the report	2

0.2	Methods carried out in the report	2

0.3	Results	2

0.3.1	Results of the studies	2

0.3.2	Limitations of the studies	2

0.3.3	BEM proposal for PMSE audio applications in the frequency band 1785-1805 MHz	3

0.3.4	Discussions of results for scenarios	3

1	Introduction	7

2	Frequency usage and associated technical conditions	8

2.1	Frequency usage	8

2.2	Frequency environment	9

3	Parameters and scenarios for studies	10

3.1	MFCN and PMSE parameters	10

3.2	Scenarios	16

4	Results of COMPATIBILITY STUDIES	19

4.1	Methodology	19

4.2	Results	19

4.2.1	Handheld PMSE	19

4.2.2	Body Worn PMSE	20

5	CONCLUSION	22

5.1	Studies included in the report	22

5.2	Methods carried out in the report	22

5.3	Results	22

5.3.1	Results of the studies	22

5.3.2	Limitations of the studies	22

5.3.3	BEM proposal for PMSE audio applications in the frequency band 1785-1805 MHz	23

5.3.4	Discussions of results for scenarios	23

ANNEX 1: seamcat Simulation for Indoor and Outdoor environment	24

ANNEX 2: Derivation of a BEM based on Minimum Coupling Loss analysis	30

ANNEX 3: Adaptation of CEPT Report 30 derivation	41

ANNEX 4: Derivation of a BEM based on mobile UE emission limit requirements	48

ANNEX 5: Setup procedure for an interference free operation of wireless microphone and in-ear monitor links	50

ANNEX 6: Impact of MFCN Bandwidth on PMSE tx - MFCN LTE rx studies	52

ANNEX 7: Considerations on Receiver Blocking Response and Receiver Blocking Level	54

ANNEX 8: A mixed scenario	59

ANNEX 9: List of references	71

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| 3GPP | 3rd Generation Partner Project |
| ACS | Adjacent Channel Selectivity |
| BEM | Block Edge Mask |
| BS | Base Station |
| BW | Bandwidth |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| DEC | Decision |
| DL | Downlink |
| DPLC | Distance Power Loss Coefficient |
| ECC | Electronic Communications Committee |
| ECN | Electronic Communications Network |
| e.i.r.p. | effective isotropic radiated power |
| EN | European Norm |
| ERC | European Radiocommunications Committee |
| ERP | Effective Radiated Power |
| ETSI | European Telecommunications Standards Institute |
| E-UTRA | Evolved Universal Terrestrial Radio Access |
| FDD | Frequency Division Duplex |
| FFT | Fast Fourier Transform |
| FSPL | Free Space Propagation Loss |
| GSM | Global System for Mobile communications |
| IEEE | Institute of Electrical and Electronics Engineers |
| IMD | Inter-modulation Distortion |
| IMT | International Mobile Telecommunication |
| ITU | International Telecommunication Union |
| LTE | Long Term Evolution |
| MCL | Minimum Coupling Loss |
| MFCN | Mobile/Fixed Communications Networks |
| N/A | Not Available |
| NF | Noise Figure |
| NLOS | Non Line Of Sight |
| OOB | Out-Of-Band |
| PI | Power of Interference |
| PMSE | Programme Making and Special Events |
| RB | Resource Block |
| REC | Recommendation |
| RFR | Restricted Frequency Range |
| RR | Radio Regulations |
| SEAMCAT | Spectrum Engineering Advanced Monte Carlo Analysis Tool |
| TR | Technical Report |
| TS | Technical Specification |
| TS | Terminal Station |
| UE | User Equipment |
| UL | Uplink |

# Introduction

In Europe, the band 1785-1800 MHz is harmonised for radio microphones, as detailed in the ERC/REC 70-03 Annex 10 [1], with limitations for the in-band power: 20 mW/ 13 dBm for handheld microphones and 50 mW / 17 dBm for body worn microphones. The European Commission has mandated CEPT to study the extension of this band by another 5 MHz to 1805 MHz.

The report considers 16 scenarios corresponding to different interference cases: indoor/outdoor, PMSE interfering with MFCN or MFCN interfering with PMSE, MFCN UE or MFCN BS, LTE or GSM as MFCN technology.

Studies have been performed with 3 different methods, including Monte-Carlo simulations (using SEAMCAT) and Minimum Coupling Loss (MCL) analyses.

The Report is structured as follows:

In Chapter 2, the frequency usages are described;

In Chapter 3, the assumptions, scenarios considered and simulation environments are presented;

In Chapter 4, the results are provided;

In Chapter 5, conclusions are drawn;

In Annexes, simulation and calculation results are presented for different methods, considerations about Mobile UE emission limits are provided, setup procedure for PMSE is described, the impact of MFCN bandwidth on studies is analysed and methods to derive receiver blocking response and receiver blocking level are specified.

# Frequency usage and associated technical conditions

## Frequency usage

Table 3 shows an overview of main band usage in and around the 1710-1880 MHz band. More details about the European Common Allocations and the relation to European Standards are found in subsequent sections.

Table 3: Overview of band usage in and around the 1785-1805 MHz band

|  | 1710 MHz 1785 MHz 1800 MHz 1805 MHz 1880 MHz |  |  |  |
| --- | --- | --- | --- | --- |
| RR Region 1 Allocation and RR footnotes relevant to CEPT and Frequency Band | FIXED MOBILE 5.384A 5.149 5.341 5.385 5.386 5.387 | FIXED MOBILE 5.384A 5.386 5.387 | FIXED MOBILE 5.384A 5.386 | FIXED MOBILE 5.384A 5.386 |
| European Common Allocation | FIXED MOBILE 5.384A 5.149 5.341 5.385 | FIXED MOBILE | MOBILE Fixed | FIXED MOBILE 5.384A |
| Major Utilisation | GSM 1800 ERC/DEC/(95)03 ECC/REC/(05)08 EN 301 502 EN 301 511 IMT ECC/DEC/(06)13 ECC/REC/(08)02 EN 301 908 MCV ECC/DEC/(08)08 MCA ECC/DEC/(06)07 EN 302 480 | Radio microphones and Assistive Listening devices ERC/REC 70-03 EN 300 422 Wireless Audio Applications ERC/REC 70-03 EN 300 422 Mobile applications See NOTE 1 | See NOTE 1 | GSM 1800 ERC/DEC/(95)03 ECC/REC/(05)08 EN 301 502 EN 301 511 IMT ECC/DEC/(06)13 ECC/REC/(08)02 EN 301 908 MCV ECC/DEC/(08)08 MCA ECC/DEC/(06)07 EN 302 480 |

NOTE 1: This band is identified for IMT in the RRs, but within CEPT this band is not planned for the harmonised introduction of IMT

## Frequency environment

The FDD duplex gap extends from 1785 MHz to 1805 MHz as illustrated in Figure 1.

Figure 1: The FDD duplex gap and potential interference

Different interference scenarios are proved in this band and the potential interferences are to be investigated. In the next section the different scenarios are described and technical parameters are listed.

# Parameters and scenarios for studies

## MFCN and PMSE parameters

Table 4: Parameters for an LTE UE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Channel Bandwidth | MHz | 10 |  |
| Transmission bandwidth (BW) | MHz | 9 | ETSI TS 136.101, Table 7.3.1-2 [7] Sensitivity for a 10 MHz channel is defined for 50 Resource Blocks (RB) ETSI TS 136.211, Section 6.2.3 1 Resource Block corresponds to 180 kHz |
| Reference Sensitivity | dBm | -94 | ETSI TS 136.101, Table 7.3.1-1 |
| Noise Figure (NF) | dB | 9 | 3GPP TR 36.824 |
| Noise Floor (N, after FFT processing) | dBm | -95.4 | 10.log(k.T.BW.1000) + NF This is the noise floor at the output of the FFT, i.e. affecting the transmission bandwidth. (See ANNEX 6:) |
| Standard Desensitization DSTANDARD | dB | 13 | ETSI TS 136.101, Table 7.6.3.1-1 |
| Standard Blocking Level IOOB-STANDARD | dBm | -55 | ETSI TS 136.101, Table 7.6.3.1-1 at 212.5 kHz from the channel edge |
| Blocking Response | dB | -27.7 then decrease by 0.8 dB every 200 kHz | CEPT Report 30, Section A5.2.2 decrease of 8 dB at 2 MHz offset assumed |
| Target Desensitization DTARGET | dB | 3 | SE7(12)061 |
| Target Blocking Level IOOB-TARGET | dBm | -67.8 then increase by 0.8 dB every 200 kHz | at 212.5 kHz from the channel edge |
| Antenna height | m | 1.5 |  |
| Body loss | dB | 3 |  |
| Antenna gain | dBi | -4 | Average value Omni directional |
| Maximum transmit power | dBm | 23 | ETSI TS 136.101, Table 6.2.2-1 |
| Out-of-band emissions (Monte-Carlo Simulations) | dB |  | ETSI TS 136.101, Table 6.6.2.1.1-1 values relative to 23 dBm |
| Duplexer impact |  |  | Not taken into account |

Table 5: Parameters for a GSM UE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Bandwidth (BW) | MHz | 0.2 | ETSI TS 145.005, Section 2 channel spacing is 200 kHz |
| Reference Sensitivity | dBm | -102 | ETSI TS 145.005, Table 6.2-1a |
| Noise Figure (NF) | dB | 8 | 3GPP TR 45.050 |
| Noise Floor (N) | dBm | -113 | 10.log(k.T.BW.1000) + NF |
| Standard Desensitization DSTANDARD | dB | 3 | ETSI TS 145.005, Section 5.1.3 DTARGET = DSTANDARD |
| Blocking Protection Ratio | dB | -9 for 1804.8-1805 MHz - 41 for 1804.6-1804.8 MHz -49 for 1804.4-1804.6 MHz | ETSI TS 145.005, Table 6.3-1 |
| Standard Blocking Level IOOB-STANDARD | dBm | -90 for 1804.8-1805 MHz -58 for 1804.6-1804.8 MHz -50 for 1804.4-1804.6 MHz | IOOB-STANDARD = Sens + D - Protection_Ratio |
| Blocking Response | dB | -23 for 1804.8-1805 MHz -55 for 1804.6-1804.8 MHz -63 for 1804.4-1804.6 MHz |  |
| Antenna height | m | 1.5 |  |
| Body loss | dB | 3 |  |
| Antenna gain | dBi | -4 | Average value Omni directional |

Table 6: Parameters for an LTE macro BS (wide area)

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Channel Bandwidth | MHz | 10 |  |
| Transmission bandwidth (BW) | MHz | 4.5 | ETSI TS 136.104, Table 7.2.1-1 Sensitivity for a 10 MHz channel is defined for 25 Resource Blocks (RB) ETSI TS 136.211, Section 6.2.3 1 Resource Block corresponds to 180 kHz |
| Reference Sensitivity | dBm | -101.5 | ETSI TS 136.104, Table 7.2.1-1 |
| Noise Figure (NF) | dB | 5 | 3GPP TR 36.824 |
| Noise Floor (N, after FFT processing) | dBm | -102.4 | 10.log(k.T.BW.1000) + NF over 25 RB This is the noise floor at the output of the FFT, i.e. affecting the transmission bandwidth. (See ANNEX 6:) |
| Standard Desensitization DSTANDARD | dB | 6 | ETSI TS 136.104, Table 7.5.1-1 |
| Standard Blocking Level IOOB-STANDARD | dBm | -49 | ETSI TS 136.104, Table 7.5.1-1 |
| Blocking Response | dB | -48.7 |  |
| Target Desensitization DTARGET | dB | 1 |  |
| Target Blocking Level IOOB-TARGET | dBm | -59.7 |  |
| Antenna height | m | 30 |  |
| Antenna gain (with cable loss) | dBi | 15 |  |
| Maximum transmit power | dBm | 46 |  |
| Out-of-band emissions (Monte-Carlo Simulations) | dB |  | ETSI TS 136.104, Table 6.6.3.2.2-1 values relative to 46 dBm |
| BS filter impact |  |  | Not taken into account |
| Vertical antenna pattern (Monte-Carlo Simulations) | dB | A down-tilt of 3° is assumed | SEAMCAT 4.0.0, Library Antenna, 3GPP Tri-Sector Antenna |
| Horizontal antenna pattern | dB | Omni directional | Envelope of a 3-sector-antenna |

Table 7: Parameters for a GSM macro BS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Bandwidth (BW) | MHz | 0.2 | ETSI TS 145.005, Section 2 channel spacing is 200 kHz |
| Reference Sensitivity | dBm | -104 | ETSI TS 145.005, Table 6.2-1b |
| Noise Figure (NF) | dB | 8 | 3GPP TR 45.050 |
| Noise Floor (N) | dBm | -113 | 10.log(k.T.BW.1000) + NF |
| Standard Desensitization DSTANDARD | dB | 3 | ETSI TS 145.005, Section 5.1.2 |
| Blocking Protection Ratio | dB | -9 for 1804.8-1805 MHz -41 for 1804.6-1804.8 MHz -49 for 1804.4-1804.6 MHz | ETSI TS 145.005, Table 6.3-1 |
| Standard Blocking Level IOOB-STANDARD | dBm | -92 for 1804.8-1805 MHz -60 for 1804.6-1804.8 MHz -52 for 1804.4-1804.6 MHz | IOOB-STANDARD = Sens + D - Protection_Ratio |
| Blocking Response | dB | -21 for 1804.8-1805 MHz -53 for 1804.6-1804.8 MHz -61 for 1804.4-1804.6 MHz |  |
| Target Desensitization DTARGET | dB | 1 |  |
| Target Blocking Level IOOB-TARGET | dBm | -98 for 1804.8-1805 MHz -66 for 1804.6-1804.8 MHz -58 for 1804.4-1804.6 MHz |  |
| Antenna height | m | 30 |  |
| Antenna gain (with cable loss) | dBi | 15 |  |

Table 8: Parameters for an LTE pico BS (local area)

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Channel Bandwidth | MHz | 10 |  |
| Transmission bandwidth (BW) | MHz | 4.5 | ETSI TS 136.104, Table 7.2.1-2 Sensitivity for a 10 MHz channel is defined for 25 Resource Blocks (RB) ETSI TS 136.211, Section 6.2.3 1 Resource Block corresponds to 180 kHz |
| Reference Sensitivity | dBm | -93.5 | ETSI TS 136.104, Table 7.2.1-2 |
| Noise Figure (NF) | dB | 13 | 3GPP TR 36.931 |
| Noise Floor (N, after FFT processing) | dBm | -94.4 | 10.log(k.T.BW.1000) + NF over 25 RB This is the noise floor at the output of the FFT, i.e. affecting the transmission bandwidth. (See ANNEX 6:) |
| Standard Desensitization DSTANDARD | dB | 6 | ETSI TS 136.104, Table 7.5.1-1 |
| Standard Blocking Level IOOB-STANDARD | dBm | -41 | ETSI TS 136.104, Table 7.5.1-1 |
| Blocking Response | dB | -48.7 |  |
| Target Desensitization DTARGET | dB | 1 |  |
| Target Blocking Level IOOB-TARGET | dBm | -51.7 |  |
| Antenna height | m | 3 |  |
| Antenna gain (with cable loss) | dBi | 0 |  |
| Maximum transmit power | dBm | 24 | ETSI TS 136.104, Table 6.2-1 |
| Out-of-band emissions (Monte-Carlo Simulations) | dB |  | ETSI TS 136.104, Table 6.6.3.2A-1 values relative to 24 dBm |
| BS filter impact |  |  | Not taken into account |
| Vertical antenna pattern | dB | Omni directional |  |
| Horizontal antenna pattern | dB | Omni directional |  |

Table 9: Parameters for a GSM pico BS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Bandwidth (BW) | MHz | 0.2 | ETSI TS 145.005, Section 2 channel spacing is 200 kHz |
| Reference Sensitivity | dBm | -95 | ETSI TS 145.005, Table 6.2-1b |
| Noise Figure (NF) | dB | 8 | 3GPP TR 45.050 |
| Noise Floor (N) | dBm | -113 | 10.log(k.T.BW.1000) + NF |
| Standard Desensitization DSTANDARD | dB | 3 | ETSI TS 145.005, Section 5.1.2 |
| Blocking Protection Ratio | dB | -9 for 1804.8-1805 MHz -41 for 1804.6-1804.8 MHz -49 for 1804.4-1804.6 MHz | ETSI TS 145.005, Table 6.3-1 |
| Standard Blocking Level IOOB-STANDARD | dBm | -83 for 1804.8-1805 MHz -51 for 1804.6-1804.8 MHz -43 for 1804.4-1804.6 MHz | IOOB-STANDARD = Sens + D - Protection_Ratio |
| Blocking Response | dB | -30 for 1804.8-1805 MHz -62 for 1804.6-1804.8 MHz -70 for 1804.4-1804.6 MHz |  |
| Target Desensitization DTARGET | dB | 1 |  |
| Target Blocking Level IOOB-TARGET | dBm | -89 for 1804.8-1805 MHz -57 for 1804.6-1804.8 MHz -49 for 1804.4-1804.6 MHz |  |
| Antenna height | m | 3 |  |
| Antenna gain (with cable loss) | dBi | 0 |  |

Table 10: Parameters for handheld PMSE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Bandwidth (BW) | MHz | 0.2 |  |
| Antenna height | m | 1.5 |  |
| Body loss | dB | 1 around 0° 7 elsewhere |  |
| Maximum e.i.r.p. | dBm | 13 | ERC/REC 70-03, Annex 10 |
| Transmitter mask (Monte-Carlo Simulations) | dBm |  | ETSI EN 300.422 (revised) |

Table 11: Parameters for body worn PMSE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Bandwidth (BW) | MHz | 0.2 |  |
| Antenna height | m | 1.5 |  |
| Body loss | dB | 15 |  |
| Maximum e.i.r.p. | dBm | 17 | ERC/REC 70-03, Annex 10 |
| Transmitter mask (Monte-Carlo Simulations) | dBm |  | ETSI EN 300.422 (revised) |

Table 12: Parameters for PMSE receivers

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Bandwidth (BW) | MHz | 0.2 |  |
| Reference Sensitivity | dBm | -90 | ETSI TR 102.546, Section B.4.1.3 |
| Noise Figure (NF) | dB | 6 | ETSI TR 102.546, Section B.3.1 |
| Noise Floor (N) | dBm | -115 | 10.log(k.T.BW.1000) + NF |
| Standard Desensitization DSTANDARD | dB | 3 | DTARGET = DSTANDARD |
| Blocking Response | dB |  | ETSI TR 102.546 Attachment 2, Applicable Receiver Parameter for PWMS below 1 GHz |
| Antenna height | m | 3 |  |
| Antenna gain | dBi | 0 | Omni directional |

Note: How to calculate the receiver blocking response and the target receiver blocking level is described in Annex 7.

## Scenarios

In the following table the relevant scenarios are listed.

Table 13: Overview of scenarios

| Scenario | Outdoor/ Indoor |  | Interferer | Victim | Distance (MCL) | Distance range (Monte-Carlo Simulations) | Propagation model |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Outdoor |  | PMSE | GSM UE | 15 | 15..100 m | IEEE 802.11 Model C, break-point at 5m |
| 2 |  |  | PMSE | LTE UE |  |  |  |
| 3 |  |  | GSM UE | PMSE |  |  |  |
| 4 |  |  | LTE UE | PMSE |  |  |  |
| 5 |  |  | PMSE | GSM macro BS | 100 | 100..350 m | Extended Hata, Urban |
| 6 |  |  | PMSE | LTE macro BS |  |  |  |
| 7 |  |  | GSM macro BS | PMSE |  |  |  |
| 8 |  |  | LTE macro BS | PMSE |  |  |  |
| 9 | Indoor |  | PMSE | GSM UE | 5 | 5..50 m | IEEE 802.11 Model C, break-point at 5m |
| 10 |  |  | PMSE | LTE UE |  |  |  |
| 11 |  |  | GSM UE | PMSE |  |  |  |
| 12 |  |  | LTE UE | PMSE |  |  |  |
| 13 |  |  | PMSE | GSM pico BS | 5 | 5..50 m | IEEE 802.11 Model C, break-point at 5m |
| 14 |  |  | PMSE | LTE pico BS |  |  |  |
| 15 |  |  | GSM pico BS | PMSE |  |  |  |
| 16 |  |  | LTE pico BS | PMSE |  |  |  |

Note 1: In the distance range of an event area, e.g. theatre or outdoor show, people are present across the propagation link between a transmitter and a receiver and may cause additional loss (of up to 20 to 30 dB), as a result of body loss or multi-path interference due to body scattering. Thus, the propagation model IEEE 802.11 (Model C) is used as in ECC Report 131 [5].

Note 2: An outdoor show is typically a concert performance

- A part from the indoor and the outdoor scenarios, a comprehensive mixed scenario for a densely populated urban environment is given for information, see Annex 8.

The set-up of distance ranges in the table above in the simulations is illustrated in Figure 2 below.

Figure 2: Illustration of the distance range

PMSE should be operated only if a check of quality of service in the radio environment is performed before and resulted in sufficient quality. The PMSE setup indicates whether enough PMSE channels with no interference are available to guarantee the needed quality of service. This procedure is described in ANNEX 5:.

Figure 3 and Figure 4 below illustrate the outdoor and indoor scenarios.

.

Figure 3: Outdoor interference scenario

Figure 4: Indoor interference scenario

# Results of COMPATIBILITY STUDIES

## Methodology

In order to address a compatibility study for PMSE in the 1785-1805 MHz band different methods have been used:

Method 1 - Simulations: Given that PMSE applications will be used in very different environments, it is impossible to predict in advance in which radio environmental PMSE is operated, as well as the spatial location of the devices to each other and the number of different devices. Therefore, a Monte Carlo analysis is applied using the simulator tool SEAMCAT (version 4.0.0).

Method 2 - Minimum Coupling Loss (MCL) calculation: It is simple to use and does not require a computer for implementation in order to have the results for the worst case analysis. The result generated is isolation in dB, which are converted into the required physical separation between PMSE and MFCN systems.

Method 3 - Analysis based on the method adopted in CEPT Report 30 [3]

- The full descriptions of Methods 1 to 3, including derivations, can be found in Annexes 1 to 3. In addition, Annex 4 identifies the current emission limits in the band considered for terminals with similar characteristics as audio PMSE devices.

## Results

### Handheld PMSE

The results provided by Methods 1, 2 and 3 for handheld PMSE are summarised in Table 14.

Table 14: Summary of results for different scenarios, Handheld PMSE

| Scenario | Indoor /Outdoor | Interferer | Victim | Method 1 Unwanted/Blocking (%) | Method 2 | Method 3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Outdoor | PMSE | GSM UE | Not available | RFR: 1804.8-1805 MHz OOBE: -37 dBm/200kHz | Not available |
| 2 | Outdoor | PMSE | LTE UE | 0/1.05 | RFR: 1803.6-1805 MHz OOBE: -36 dBm/200kHz | RFR: 1803.8-1805 MHz OOBE=-42 dBm/200 kHz |
| 3 | Outdoor | GSM UE | PMSE | Not available | Not available | Not available |
| 4 | Outdoor | LTE UE | PMSE | 89.89/1.30 | Not available | Not available |
| 5 | Outdoor | PMSE | GSM macro BS | 0.09/0.13 | RFR: 1785-1785.2 MHz OOBE: -20 dBm/200kHz | Not available |
| 6 | Outdoor | PMSE | LTE macro BS | 0.01/0.03 | RFR: None required OOBE: -17 dBm/200kHz | RFR: 1785-1785.2 MHz OOBE=-29 dBm/200 kHz |
| 7 | Outdoor | GSM macro BS | PMSE | Not available | Not available | Not available |
| 8 | Outdoor | LTE macro BS | PMSE | 59.52/26.27 | Not available | Not available |
| 9 | Indoor | PMSE | GSM UE, indoor | Not available | RFR: 1804.4-1805 MHz OOBE: -53.5 dBm/200kHz | Not available |
| 10 | Indoor | PMSE | LTE UE, indoor | 0/5.92 | RFR: 1800-1805 MHz OOBE: -52.5 dBm/200kHz | Not available |
| 11 | Indoor | GSM UE | PMSE | Not available | Not available | Not available |
| 12 | Indoor | LTE UE | PMSE | 99,87/18,73 | Not available | Not available |
| 13 | Indoor | PMSE | GSM pico BS | 22.56/0.01 | RFR: 1785-1786.6 MHz OOBE: -60 dBm/200kHz | Not available |
| 14 | Indoor | PMSE | LTE pico BS | 2.27/3.17 | Not allowed in the band (RFR > 20 MHz) | Not available |
| 15 | Indoor | GSM pico BS | PMSE | Not available | Not available | Not available |
| 16 | Indoor | LTE pico BS | PMSE | 99,92/40,46 | Not available | Not available |

### Body Worn PMSE

The results provided by Methods 1, 2 and 3 for Body Worn PMSE are summarized in .

Table 15: Summary of results for different scenarios, Body Worn PMSE

| Scenario | Indoor /Outdoor | Interferer | Victim | Method 1 Unwanted/Blocking (%) | Method 2 | Method 3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Outdoor | PMSE | GSM UE | Not available | RFR: 1804.8-1805 MHz OOBE: -23 dBm/200kHz | Not available |
| 2 | Outdoor | PMSE | LTE UE | 0/0.37 | RFR: none required OOBE: -22 dBm/200kHz | RFR: 1804.8-1805 MHz OOBE=-35 dBm/200 kHz |
| 3 | Outdoor | GSM UE | PMSE | Not available | Not available | N/A |
| 4 | Outdoor | LTE UE | PMSE | 89.89/1.30 | Not available | Not available |
| 5 | Outdoor | PMSE | GSM macro BS | 0.01/0.02 | RFR: none required OOBE: -6 dBm/200kHz | Not available |
| 6 | Outdoor | PMSE | LTE macro BS | 0.02/0.07 | RFR: none required OOBE: -3 dBm/200kHz | RFR: 1785-1785.2 MHz OOBE=-21 dBm/200 kHz |
| 7 | Outdoor | GSM macro BS | PMSE | Not available | Not available | Not available |
| 8 | Outdoor | LTE macro BS | PMSE | 59.52/26.27 | Not available | Not available |
| 9 | Indoor | PMSE | GSM UE, indoor | Not available | RFR: 1804.6-1805 MHz OOBE: -39.5 dBm/200kHz | Not available |
| 10 | Indoor | PMSE | LTE UE, indoor | 0/5.84 | RFR: 1802.4-1805 MHz OOBE: -38.5 dBm/200kHz | Not available |
| 11 | Indoor | GSM UE | PMSE | Not available | Not available | Not available |
| 12 | Indoor | LTE UE | PMSE | 99,87/18,73 | Not available | Not available |
| 13 | Indoor | PMSE | GSM pico BS | 66.29/33.56 | RFR: 1785-1785.4 MHz OOBE: -46 dBm/200kHz | Not available |
| 14 | Indoor | PMSE | LTE pico BS | 0.15/1.47 | RFR: None required but e.i.r.p. limited to 15 dBm OOBE: -41 dBm/200kHz | Not available |
| 15 | Indoor | GSM pico BS | PMSE | Not available | Not available | Not available |
| 16 | Indoor | LTE pico BS | PMSE | 99,92/40,46 | Not available | Not available |

# CONCLUSION

## Studies included in the report

The present report studies the conditions for operation of PMSE audio equipment (wireless microphones) in the frequency band 1785-1805 MHz. This frequency range corresponds to the duplex gap of the 1710-1785/1805-1880 MHz mobile band.

This report considers interference between PMSE equipment operating in the band 1785-1805 MHz and public mobile network equipment operating in the bands 1710-1785 MHz and 1805-1880 MHz, The studies in this report consider mainly the interference scenarios where audio PMSE equipment interfere mobile, i.e. PMSE equipment interferes with mobile base station receiving below 1785 MHz and PMSE equipment interferes with mobile terminal receiving above 1805 MHz. The report considers a total of 16 scenarios corresponding to a specific combination of the following options:

Indoor/outdoor,

PMSE interferes MFCN or MFCN interferes PMSE,

MFCN BS or MFCN UE,

MFCN equipment is LTE or GSM.

## Methods carried out in the report

The report is based on several methods.

Method 1: Monte-Carlo simulations carried out with the SEAMCAT tool, provided in ANNEX 1:

Method 2: Minimum Coupling Loss (MCL) analysis, provided in ANNEX 2:

Method 3: Analysis based on the method adopted in CEPT Report 30 [3], provided in ANNEX 3:

## Results

### Results of the studies

The Method 1 (Monte-Carlo simulations, ANNEX 1) assumes specific PMSE equipment parameters (e.g. Spectrum emission mask, power, system bandwidth) and aims at deriving the  needed restricted frequency range between the audio PMSE equipment and the mobile equipment in adjacent bands for PMSE using these parameters.

The methods in ANNEX 2: and ANNEX 3: aim at deriving a Block Edge Mask, with specific in-band and out of band allowed emission levels, without preliminary assumption on PMSE power, spectrum emission mask or system bandwidth.

### Limitations of the studies

It should be noted that the analyses are limited to cases where there is an interference risk (both audio PMSE equipment and mobile terminal/base station are in operation), without taking into account the probability of such scenario which is related to the market penetration of audio PMSE equipment and mobile systems. The analyses also consider MFCN equipment operating at the edge of their band, which may not always be the case, especially in the case of GSM.

The scenarios corresponding to mobile equipment (both terminals and base stations) interfering with audio PMSE equipment is not fully studied in this report. Preliminary considerations are provided through Monte-Carlo simulations, without providing a full analysis as they only consider analogue audio PMSE and do not consider the effect of duplex filtering in mobile base station and terminals. The conclusions of this report do not guarantee that audio PMSE equipment will be able to operate in the band, but only that, provided they can operate, they would not create interference to mobile systems in adjacent bands.

### BEM proposal for PMSE audio applications in the frequency band 1785-1805 MHz

Table 16: BEM for handheld microphone

|  | Frequency Range | Handheld e.i.r.p. | Reasoning |
| --- | --- | --- | --- |
| OOB | < 1785 MHz | -17 dBm/200kHz | LTE UE spectrum emission mask |
| Restricted frequency range | 1785-1785,2 MHz | 4 dBm/200kHz | GSM blocking |
|  | 1785.2-1803.6 MHz | 13 dBm/channel |  |
|  | 1803.6-1804.8 MHz | 10 dBm/200kHz* | slow increase of LTE UE selectivity |
| Restricted frequency range | 1804.8-1805 MHz | -14 dBm/200kHz | GSM blocking |
| OOB | > 1805 MHz | -37 dBm/200kHz | OOB calculation, in line with ERC/REC 74-01 |

*with a limit of 13 dBm/channel

Table 17: BEM for body worn microphone

|  | Frequency Range | Body worn e.i.r.p. | Reasoning |
| --- | --- | --- | --- |
| OOB | < 1785 MHz | -17 dBm/200kHz | LTE UE spectrum emission mask |
|  | 1785-1804.8 MHz | 17 dBm/channel |  |
| Restricted frequency range | 1804.8-1805 MHz | 0 dBm/200kHz | GSM blocking |
| OOB | > 1805 MHz | -23 dBm/200kHz | OOB calculation* |

* For the body worn case the body loss is 14 higher than for the handheld case, therefore the -23 dBm for body worn is equivalent to -37 dBm for handheld.

### Discussions of results for scenarios

The studied interference scenarios may not happen in many real cases, because the PMSE device cannot go in operation when suffering interference from base station or UE. See Table 20 in Annex 1, section A1.3. Further studies may be performed in this respect taking also into account the performance of current MFCN (e.g. considering the impact of the duplex filter) and PMSE equipment. Depending on the scenarios to be studied, the duplex filters may reduce the MFCN out of band emissions and thereby also the interference impact on PMSE devices.

It is suggested that in the indoor environment, the physical location of any pico station should be considered in relation to the PMSE installation in order to minimise mutual interference by use of wall loss or other mitigation techniques, which would reduce interaction.

Further studies may also be required to study the case of digital PMSE devices with different resistance to interference.

seamcat Simulation for Indoor and Outdoor environment

The relevant scenarios used for PMSE can be classified into 2 basic types: outdoor and indoor. The analyses were based on Monte Carlo simulation to cover the various deployment situations of PMSE in the different environments: SEAMCAT (version 4.0.0). The parameters are used for the studies were presented in chapter 3. These results compared with the MCL analyses gives the possibility to drive a BEM for PMSE, therefore the same assumptions was made for this Annex and Annex 2.All Scenarios does not consider power control of the UE or BS, this means it is always assumed that the MFCN receive only the minimum amount of power and transmit always the maximum power.

pmse Body worn

Table 18: PMSE Body worn

| Unwanted/Blocking (exceedance probability) [%] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency distance between the PMSE channel edge and the band edge of the MFCN [kHz] |  |  |  |  |  |  |  |  |  |  |  |
| Scenario | Interferer | Victim | I/N [dB] | 200 | 400 | 600 | 800 | 1000 | 1200 | 1400 | Outdoor |
| 2 | PMSE | LTE UE(10MHz) | 0 | 0/0.37 | 0/0.12 | 0/0 | - | - | - | - |  |
| 2 | PMSE | LTE UE(5MHz) | 0 | 0/0.81 | 0/0.41 | 0/0.26 | 0/0.13 | 0/0 | - | - |  |
| 6 | PMSE | LTE BS | -6 | 0.02/0.07 | - | - | - | - | - | - |  |
| 5 | PMSE | GSM BS | -6 | 0.01/0.02 | 0/0 | - | - | - | - | - |  |
| 10 | PMSE | LTE UE | 0 | 0/13.50 | 0/11.95 | 0/10.18 | 0/9.25 | 0/8.43 | 0/7.10 | 0/5.84 | Indoor |
| 14 | PMSE | LTE Pico BS | -6 | 0.15/1.47 | - | - | - | - | - | - |  |
| 13 | PMSE | GSM Pico BS | -6 | 66.29/33.56 | 43.38/18.98 | 33.89/0 | 26.22/0 | 24.82/0 | 25.00/0 | 25.37/0 |  |

- The Scenarios 6 and 14 are only made with one frequency separation, because there is no additional selectivity for the BS receiver in the PMSE band. The results do not change, if the frequency separation is increased.

PMSE hand held

| Unwanted/Blocking (exceedance probability) [%] |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Frequency distance between the PMSE channel edge and the band edge of the MFCN [kHz] |  |  |  |  |  |  |  |  |
| Scenario | Interferer | Victim | I/N [dB] | 200 | 400 | 600 | 800 | 1000 | 1200 | 1400 | Outdoor |
| 2 | PMSE | LTE UE(10MHz) | 0 | 0/1.05 | 0/0.74 | 0/0.67 | 0/0.40 | 0/0.37 | 0/0.32 | 0/0.13 |  |
| 2 | PMSE | LTE UE(5MHz) | 0 | 0/2.08 | 0/1.59 | 0/1.15 | 0/0.87 | 0/0.79 | 0/0.51 | 0/0.48 |  |
| 6 | PMSE | LTE BS | -6 | 0.01/0.03 | - | - | - | - | - | - |  |
| 5 | PMSE | GSM BS | -6 | 0.09/0.13 | 0.02/0.2 | 0/0 | - | - | - | - |  |
| 10 | PMSE | LTE UE | 0 | 0/11.28 | 0/10.01 | 0/8.93 | 0/7,99 | 0/7.26 | 0/6.65 | 0/5.92 | Indoor |
| 14 | PMSE | LTE Pico BS | -6 | 2.27/3.17 | - | - | - | - | - | - |  |
| 13 | PMSE | GSM Pico BS | -6 | 58.05/30.24 | 39.98/16.76 | 30.94/0.01 | 23.47/0 | 22.93/0 | 22.79/0 | 22.56/0.01 |  |

Table 19: PMSE Hand held

- The Scenarios 6 and 14 are only made with one frequency separation, because there is no additional selectivity for the BS receiver in the PMSE band. The results do not change, if the frequency separation is increased.

pmse receiver

Table 20: PMSE Receiver

| Unwanted/Blocking (exceedance probability) [%] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency distance between the PMSE channel edge and the band edge of the MFCN [MHz] |  |  |  |  |  |  |
| Scenario | Interferer | Victim | I/N [dB] | 0,3 | 19,7 |  |
| 4 | LTE UE | PMSE | 0 | 89.89/1.30 | 5.28/1.49 | Outdoor |
| 8 | LTE BS | PMSE | 0 | 59.52/26.27 | 17.20/26.87 |  |
| 16 | LTE Pico BS | PMSE | 0 | 99,92/40,46 | 55.88/41.02 | Indoor |
| 12 | LTE UE | PMSE | 0 | 99,87/18,73 | 30,32/18,61 |  |

The results show the exceedance probabilities for worst case scenarios, the usability could be make feasible by several mitigation techniques and additional factors e.g.

favouring structural properties of a building

additional wall losses

duplex filter installed in the BS and UE

Power control of the UE is not taken into account.

Take into account this additional factors, further studies are needed to investigate the impact in the results.

conclusion

The results show that a PMSE setup is required to ensure the needed QoS for PMSE, in the presence of a MFCN (PMSE is able to operate only within environments where the operational channels provide sufficient high QoS, see Annex 5). If this is guaranteed, PMSE can operate without causing any harmful impact to MFCN and vice versa.

For the indoor case it is in the responsibility of the user of the PMSE and the site owner to guarantee the usability of PMSE and the MFCN (Pico BS or UE).

It must be highlighted that the indoor scenarios consider no additional wall loss or other favouring structural properties of the building. This is also the case for the outdoor scenarios if there is a building or other structure to generate additional losses, they might have the consequences of lower exceedance probabilities.

According to ECC Report 131 [5] a probability of ≤ 5%, of exceeding the limits is considered, if the desensitization is 1 dB or 3 dB.

Monte-carlo simulations with modified assumptions

In this Section simulation results for interference from PMSE to MFCN are presented. These can be found in the tables in Section below.

The main assumptions used in this Section are:

Only hand-held PMSE is considered since it is the limiting case;

PMSE bandwidth is 200 kHz;

For outdoor cases where the base station is the victim, the minimum and maximum distances are 60 and 300 m, respectively;

For the outdoor PMSE to MFCN terminal station case, the IEEE802.11 propagation model with a 20 m break point has been used;

For the indoor cases, the IEEE802.11 propagation model with a 5 m break point has been used;

Antenna patterns:

    - -1, -7, -20 dB, with maximum gain in a 5 degree sector (See Table 10);

    - Constant -1 dB. This case corresponds to a PMSE device on a stand, i.e., without any human interaction;

The probability for unwanted plus blocking interfering power exceeding the I/N criteria has been considered;

Required guard band for 5% exceedance of I/N has been calculated with the assumption of 0.8 dB improvement of blocking response for every 200 kHz.

Detailed assumptions can be found in the Section 3.1.

Table 21: Parameters used in simulations

| Scenario | Interferer | Victim | rmin (m) | rmax (m) |  | Propag. Model | Environment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | PMSE | LTE UE | 10 | 100 |  | IEEE 802.11C1 | Outdoor |
| 4 | LTE UE | PMSE | 10 | 100 |  | IEEE 802.11C1 |  |
| 6 | PMSE | LTE BS | 60 |  | 300 | Extended Hata |  |
| 8 | LTE BS | PMSE | 60 | 300 |  | Extended Hata |  |
| 5 | PMSE | GSM BS | 60 | 300 |  | Extended Hata |  |
| 10 | PMSE | LTE UE | 5 | 50 |  | IEEE 802.11C1 | Indoor |
| 14 | PMSE | LTE Pico BS | 5 | 50 |  | IEEE 802.11C1 |  |
| 16 | LTE Pico BS | PMSE | 5 | 50 |  | IEEE 802.11C1 |  |
| 12 | LTE UE | PMSE | 5 | 50 |  | IEEE 802.11C1 |  |

Note: For MCL calculation the minimum distance between victim receiver and interferer is rmin.

1 The break point for outdoor environment is 20m and for the indoor 5m.

|  |  |  |  | Unwanted + Blocking (exceedance probability) [%] |  |  |  |  |  |  | Guard band req’rd for 5% exceed. [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency distance between the PMSE channel edge and the band edge of the MFCN [kHz] |  |  |  |  |  |  |  |  |
| Scenario | Interferer | Victim | I/N [dB] | 200 | 400 | 600 | 800 | 1000 | 1200 | 1400 |  | Outdoor |
| 2 | PMSE | LTE UE(5 MHz) | 0 | 33.8 |  | 29.7 |  | 25,9 |  | 21.7 | 3.6 |  |
| 2 | PMSE | LTE UE (10 MHz) | 0 | 25,7 |  | 22.5 |  | 19.2 |  | 16.3 | 2.8 |  |
| 6 | PMSE | LTE BS | -6 | 0.9 | - | - | - | - | - | - | - |  |
| 5 | PMSE | GSM BS | -6 | 1.9 | 0.3 | 0.1 | - | - | - | - | - |  |
| 10 | PMSE | LTE UE (5 MHz) | 0 | 38.9 |  | 33.8 |  | 29,7 |  | 24.9 | 4.5 | Indoor |
| 10 | PMSE | LTE UE (10 MHz) | 0 | 29.3 |  | 26.3 |  | 22.4 |  | 18.5 | 3.7 |  |
| 14 | PMSE | LTE Pico BS | -6 | 18.2 | - | - | - | - | - | - | - |  |
| 13 | PMSE | GSM Pico BS | -6 | 98 |  | 66.7 |  | 54.8 |  | 54.3 | - |  |

Table 22: PMSE Hand Held (Antenna pattern: constant -1 dB)

Table 23: PMSE Hand Held (Antenna pattern: -1, -7, -20 dB)

|  |  |  |  | Unwanted + Blocking (exceedance probability) [%] |  |  |  |  |  |  | Guard band req’rd for 5% exceed. [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Frequency distance between the PMSE channel edge and the band edge of the MFCN [kHz] |  |  |  |  |  |  |  |  |
| Scenario | Interferer | Victim | I/N [dB] | 200 | 400 | 600 | 800 | 1000 | 1200 | 1400 |  | Outdoor |
| 2 | PMSE | LTE UE(5 MHz) | 0 |  |  |  |  |  |  | 5.8 | 1.6 |  |
| 2 | PMSE | LTE UE (10 MHz) | 0 |  |  | 6.4 | 5.2 | 4.8 |  | 3.2 | 1.0 |  |
| 6 | PMSE | LTE BS | -6 | 0.12 | - | - | - | - | - | - | - |  |
| 5 | PMSE | GSM BS | -6 | 0.2 | 0.0 | 0.0 | - | - | - | - | - |  |
| 10 | PMSE | LTE UE (5 MHz) | 0 |  |  |  |  |  |  | 7.1 | 1.8 | Indoor |
| 10 | PMSE | LTE UE (10 MHz) |  | 9.2 |  | 7.1 |  | 6.0 | 5.2 | 4.7 | 1.4 |  |
| 14 | PMSE | LTE Pico BS | -6 | 4.6 | - | - | - | - | - | - | - |  |
| 13 | PMSE | GSM Pico BS | -6 | 53.4 | 34.8 | 27.1 | 21.8 | 20.9 | 20.7 | 21.0 |  |  |

Conclusion to section A1.5

It is concluded that a significant guard band is needed between PMSE devices and the MFCN downlink.

Derivation of a BEM based on Minimum Coupling Loss analysis

One simple BEM derivation method is to conduct a Minimum Coupling Loss analysis based on the interfered receiver sensitivity/blocking parameters, the loss of the propagation channel over the assumed protection distance and other relevant attenuation factors.

Results analysis and BEM

Details on calculation method and assumptions are provided in Section A2.2.

Results from blocking calculations

Blocking calculations result in in-block e.i.r.p. limits. When the maximum e.i.r.p. acceptable from a microphone is lower than the e.i.r.p. allowed by ERC/REC 70-03 [1], then a guard-band is required.

The table below summarizes the results.

Table 24: Results of blocking MCL analysis

| Scenario | Victim | Handheld PMSE: guard-band requirement | Body worn PMSE: guard-band requirement |
| --- | --- | --- | --- |
| 1 | GSM UE, outdoor | 200 kHz (1804.8-1805 MHz) | 200 kHz (1804.8-1805 MHz) |
| 2 | LTE UE, outdoor | 1.4 MHz (1803.6-1805 MHz) | No guard-band required |
| 5 | GSM macro BS, outdoor | 200 kHz (1785-1785.2 MHz) | No guard-band required (A) |
| 6 | LTE macro BS, outdoor | No guard-band required | No guard-band required |
| 9 | GSM UE, indoor | 600 kHz (1804.4-1805 MHz) | 400 kHz (1804.6-1805 MHz) |
| 10 | LTE UE, indoor | 5 MHz (1800-1805 MHz) | 2.6 MHz (1802.4-1805 MHz) |
| 13 | GSM pico BS, indoor | 1.6 MHz (1785-1786.6 MHz) | 400 kHz (1785-1785.4 MHz) |
| 14 | LTE pico BS, indoor | Not allowed (C) | No guard-band required but ei.r.p. limited to 15 dBm (B) |

(A) A guard-band of 200 kHz would be required for separation distance lower than 95m.

(B) With a separation distance of 6m, body worn microphones can transmit at full e.i.r.p. (17 dBm).

(C) Improvements brought by BS filter are not taken into account.

Results from out-of-block calculations

Out-of-block calculations result in out-of-block e.i.r.p. limits.

The table below summarizes the results.

Table 25: Results of out-of-block MCL analysis

| Scenario | Victim | Handheld PMSE: OOB emission level | Body worn PMSE: OOB emission level |
| --- | --- | --- | --- |
| 1 | GSM UE, outdoor | -37 dBm/200kHz | -23 dBm/200kHz |
| 2 | LTE UE, outdoor | -36 dBm/200kHz | -22 dBm/200kHz |
| 5 | GSM macro BS, outdoor | -20 dBm/200kHz | -6 dBm/200kHz |
| 6 | LTE macro BS, outdoor | -17 dBm/200kHz | -3 dBm/200kHz |
| 9 | GSM UE, indoor | -53.5 dBm/200kHz | -39.5 dBm/200kHz |
| 10 | LTE UE, indoor | -52.5 dBm/200kHz | -38.5 dBm/200kHz |
| 13 | GSM pico BS, indoor | -60 dBm/200kHz | -46 dBm/200kHz |
| 14 | LTE pico BS, indoor | -55 dBm/200kHz | -41 dBm/200kHz |

BEM

Based on these calculations, the following Block Edge Mask for PMSE audio terminals operating outdoor in 1785-1805 MHz band can be justified:

Table 26: Block Edge Mask for PMSE audio terminals operating in 1785-1805 MHz

| Frequency range | Emission limit (handheld) |  | Frequency range | Emission limit (body worn) |
| --- | --- | --- | --- | --- |
| 1710-1785 MHz | -20 dBm/200kHz |  | 1710-1785 MHz | -10 dBm/200kHz |
| 1785.2-1803.6 MHz | 13 dBm/channel (A) |  | 1785.2-1804.8 MHz | 17 dBm/channel (A) |
| 1805-1880 MHz | -37 dBm/200kHz |  | 1805-1880 MHz | -23 dBm/200kHz |

(A) In-band emission limits are those defined in ERC/REC 70-03 Annex 10.

For an indoor use, a BEM approach cannot ensure a systematic coexistence between PMSE and MFCN. On a case by case basis, coexistence can be enabled thanks to appropriate separation distance from possible pico BS and UE, wall absorption and other appropriate regulatory measures adopted at national level.

MCL calculations

Calculation tables for in-block e.i.r.p. (blocking case)

Outdoor, UE, Scenarios 1 and 2:

For an outdoor UE (scenarios 1 and 2), the maximum e.i.r.p.

Mic_ e.i.r.p.max,in-block = Blocking_Level + Path_Loss - UE_Antenna_Gain + UE_Body_Loss + Mic_Body_Loss

where path loss is calculated according to IEEE 802.11 Model C propagation model and for a separation distance of 15m.

Table 27: Calculation of maximum allowed microphone e.i.r.p. for a GSM UE 
(Scenario 1)

| Frequency range | MHz | 1805-1804.8 | 1804.8-1804.6 |
| --- | --- | --- | --- |
| Offset from the edge | MHz | 0-0.2 | 0.2-0.4 |
| Victim UE characteristics |  |  |  |
| Narrow-band blocking level | dBm | -90 | -58 |
| Attenuation calculation |  |  |  |
| Path loss | dB | 68.2 | 68.2 |
| Antenna gain | dBi | -4 | -4 |
| UE body loss | dB | 3 | 3 |
| Handheld Microphone |  |  |  |
| Microphone body loss | dB | 1 | 1 |
| Max e.i.r.p. acceptable | dBm | -13.8 | 18.2 |
| Microphone e.i.r.p. allowed | dBm | 13 | 13 |
| Conclusion |  | Guard band | Ok |
| Body worn Microphone |  |  |  |
| Microphone body loss | dB | 15 | 15 |
| Max e.i.r.p. acceptable | dBm | 0.2 | 32.2 |
| Microphone e.i.r.p. allowed | dBm | 17 | 17 |
| Conclusion |  | Guard band | Ok |

Table 28: Calculation of maximum allowed microphone e.i.r.p. for an LTE UE 
(Scenario 2)

| Frequency range | MHz | 1804.8-1804.6 | 1803.8-1803.6 | 1803.6-1803.4 |
| --- | --- | --- | --- | --- |
| Offset from the edge | MHz | 0.2-0.4 | 1.2-1.4 | 1.4-1.6 |
| Victim UE characteristics |  |  |  |  |
| Narrow-band blocking level | dBm | -67.4 (Note 1) | -63.4 | -62.6 |
| Attenuation calculation |  |  |  |  |
| Path loss | dB | 68.2 | 68.2 | 68.2 |
| Antenna gain | dBi | -4 | -4 | -4 |
| UE body loss | dB | 3 | 3 | 3 |
| Handheld Microphone |  |  |  |  |
| Microphone body loss | dB | 1 | 1 | 1 |
| Max e.i.r.p. acceptable | dBm | 8.8 | 12.8 | 13.6 |
| Microphone e.i.r.p. allowed | dBm | 13 | 13 | 13 |
| Conclusion |  | Guard band | Guard band | Ok |
| Body worn Microphone |  |  |  |  |
| Microphone body loss | dB | 15 | 15 | 15 |
| Max e.i.r.p. acceptable | dBm | 22.8 | 26.8 | 27.6 |
| Microphone e.i.r.p. allowed | dBm | 17 | 17 | 17 |
| Conclusion |  | Ok | Ok | Ok |

Note 1: At 212.5 kHz offset, the blocking level is -67.8 dBm. Since a linear slope of 8 dB / 2 MHz is assumed, at 300 kHz offset the blocking level is -67.8 + 8.[(0.3-0.2125)/2] = -67.8 + 0.35 = -67.45 dBm.

Outdoor, macro BS, Scenarios 5 and 6:

For an outdoor macro BS (Scenarios 5 and 6), the maximum e.i.r.p. acceptable from a microphone is given by the following formula:

Mic_e.i.r.p.max,in-block = Blocking_Level + Path_Loss - BS_Antenna_Gain + BS_Antenna_Discrimination + Mic_Body_Loss

where path loss is calculated according to Extended Hata propagation model and for a separation distance of 100m.

Table 29: Calculation of maximum allowed microphone e.i.r.p. for a GSM macro BS 
(Scenario 5)

| Frequency range | MHz | 1785-1785.2 | 1785.2-1785.4 |
| --- | --- | --- | --- |
| Offset from the edge | MHz | 0-0.2 | 0.2-0.4 |
| Victim BS characteristics |  |  |  |
| Narrow-band blocking level | dBm | -98 | -66 |
| Attenuation calculation |  |  |  |
| Path loss | dB | 100.9 | 100.9 |
| Antenna gain (w/ cable loss) | dBi | 15 | 15 |
| Antenna discrimination | dB | 15 | 15 |
| Handheld Microphone |  |  |  |
| Microphone body loss | dB | 1 | 1 |
| Max e.i.r.p. acceptable | dBm | 3.9 | 35.9 |
| Microphone e.i.r.p. allowed | dBm | 13 | 13 |
| Conclusion |  | Guard band | Ok |
| Body worn Microphone |  |  |  |
| Microphone body loss | dB | 15 | 15 |
| Max e.i.r.p. acceptable | dBm | 17.9 | 49.9 |
| Microphone e.i.r.p. allowed | dBm | 17 | 17 |
| Conclusion |  | Ok (Note 1) | Ok |

Note 1: A guard-band of 200 kHz would be required for separation distance lower than 95m.

Table 30: Calculation of maximum allowed microphone e.i.r.p. for an LTE macro BS 
(Scenario 6)

| Frequency range | MHz | 1785.4-1789.8 |
| --- | --- | --- |
| Offset from the edge | MHz | 0.4-4.8 |
| Victim BS characteristics |  |  |
| Narrow-band blocking level | dBm | -59.7 |
| Attenuation calculation |  |  |
| Path loss | dB | 100.9 |
| Antenna gain (w/ cable loss) | dBi | 15 |
| Antenna discrimination | dB | 15 |
| Handheld Microphone |  |  |
| Microphone body loss | dB | 1 |
| Max e.i.r.p. acceptable | dBm | 42.1 |
| Microphone e.i.r.p. allowed | dBm | 13 |
| Conclusion |  | Ok |
| Body worn Microphone |  |  |
| Microphone body loss | dB | 15 |
| Max e.i.r.p. acceptable | dBm | 56.1 |
| Microphone e.i.r.p. allowed | dBm | 17 |
| Conclusion |  | Ok |

Indoor, UE, Scenarios 9 and 10:

For an indoor UE (Scenarios 9 and 10), the maximum e.i.r.p. acceptable from a microphone is given by the following formula:

Mic_ e.i.r.p.max,in-block = Blocking_Level + Path_Loss - UE_Antenna_Gain + UE_Body_Loss + Mic_Body_Loss

where path loss is calculated according to IEEE 802.11 Model C propagation model and for a separation distance of 5m.

Table 31: Calculation of maximum allowed microphone e.i.r.p. for a GSM UE 
(Scenario 9)

| Frequency range | MHz | 1805-1804.8 | 1804.8-1804.6 | 1804.6-1804.4 | 1804.4-1803.4 |
| --- | --- | --- | --- | --- | --- |
| Offset from the edge | MHz | 0-0.2 | 0.2-0.4 | 0.4-0.6 | 0.6-1.6 |
| Victim UE characteristics |  |  |  |  |  |
| Narrow-band blocking level | dBm | -90 | -58 | -50 | -43 |
| Attenuation calculation |  |  |  |  |  |
| Path loss | dB | 51.5 | 51.5 | 51.5 | 51.5 |
| Antenna gain | dBi | -4 | -4 | -4 | -4 |
| UE body loss | dB | 3 | 3 | 3 | 3 |
| Handheld Microphone |  |  |  |  |  |
| Microphone body loss | dB | 1 | 1 | 1 | 1 |
| Max e.i.r.p. acceptable | dBm | -30.5 | 1.5 | 9.5 | 16.5 |
| Microphone e.i.r.p. allowed | dBm | 13 | 13 | 13 | 13 |
| Conclusion |  | Guard band | Guard band | Guard band | Ok |
| Body worn Microphone |  |  |  |  |  |
| Microphone body loss | dB | 15 | 15 | 15 | 15 |
| Max e.i.r.p. acceptable | dBm | -16.5 | 15.5 | 23.5 | 30.5 |
| Microphone e.i.r.p. allowed | dBm | 17 | 17 | 17 | 17 |
| Conclusion |  | Guard band | Guard band | Ok | Ok |

Table 32: Calculation of maximum allowed microphone e.i.r.p. for an LTE UE 
(Scenario 10)

| Frequency range | MHz | 1804.8-1804.6 | 1802.6-1802.4 | 1800.2-1800 | 1800-1789.8 |
| --- | --- | --- | --- | --- | --- |
| Offset from the edge | MHz | 0.2-0.4 | 2.4-2.6 | 4.8-5 | 5-5.2 |
| Victim UE characteristics |  |  |  |  |  |
| Narrow-band blocking level | dBm | -67.4 (Note 1) | -56.6 | -47.0 | -46.2 |
| Attenuation calculation |  |  |  |  |  |
| Path loss | dB | 51.5 | 51.5 | 51.5 | 51.5 |
| Antenna gain | dBi | -4 | -4 | -4 | -4 |
| UE body loss | dB | 3 | 3 | 3 | 3 |
| Handheld Microphone |  |  |  |  |  |
| Microphone body loss | dB | 1 | 1 | 1 | 1 |
| Max e.i.r.p. acceptable | dBm | -7.9 | 2.9 | 12.5 | 13.3 |
| Microphone e.i.r.p. allowed | dBm | 13 | 13 | 13 | 13 |
| Conclusion |  | Guard band | Guard band | Guard band | Ok |
| Body worn Microphone |  |  |  |  |  |
| Microphone body loss | dB | 15 | 15 | 15 | 15 |
| Max e.i.r.p. acceptable | dBm | 6.1 | 16.9 | 26.5 | 27.3 |
| Microphone e.i.r.p. allowed | dBm | 17 | 17 | 17 | 17 |
| Conclusion |  | Guard band | Guard band | Ok | Ok |

Note 1: At 212.5 kHz offset, the blocking level is -67.8 dBm. Since a linear slope of 8 dB / 2 MHz is assumed, at 300 kHz offset the blocking level is -67.8 + 8.[(0.3-0.2125)/2] = -67.8 + 0.35 = -67.45 dBm.

Results for scenario 10 take into account the additional duplex filter rejection of 2 dB at 2 MHz offset, as assumed in CEPT Report 30 [3], Section A5.2.2.

Indoor, pico BS, Scenarios 13 and 14:

For an indoor pico BS (Scenarios 13 and 14), the maximum e.i.r.p. acceptable from a microphone is given by the following formula:

Mic_ e.i.r.p.max,in-block = Blocking_Level + Path_Loss - BS_Antenna_Gain + BS_Antenna_Discrimination + Mic_Body_Loss

where path loss is calculated according to IEEE 802.11 Model C propagation model and for a separation distance of 5m.

Table 33: Calculation of maximum allowed microphone e.i.r.p. for a GSM pico BS 
(Scenario 13)

| Frequency range | MHz | 1785-1785.2 | 1785.2-1785.4 | 1785.4-1785.6 | 1785.6-1786.6 | 1786.6-1788 |
| --- | --- | --- | --- | --- | --- | --- |
| Offset from the edge | MHz | 0-0.2 | 0.2-0.4 | 0.4-0.6 | 0.6-1.6 | 1.6-3 |
| Victim BS characteristics |  |  |  |  |  |  |
| Narrow-band blocking level | dBm | -89 | -57 | -49 | -47 | -37 |
| Attenuation calculation |  |  |  |  |  |  |
| Path loss | dB | 52.1 | 52.1 | 52.1 | 52.1 | 52.1 |
| Antenna gain (w/ cable loss) | dBi | 0 | 0 | 0 | 0 | 0 |
| Antenna discrimination | dB | 0 | 0 | 0 | 0 | 0 |
| Handheld Microphone |  |  |  |  |  |  |
| Microphone body loss | dB | 1 | 1 | 1 | 1 | 1 |
| Max e.i.r.p. acceptable | dBm | -35.9 | -3.9 | 4.1 | 6.1 | 16.1 |
| Microphone e.i.r.p. allowed | dBm | 13 | 13 | 13 | 13 | 13 |
| Conclusion |  | Guard band | Guard band | Guard band | Guard band | Ok |
| Body worn Microphone |  |  |  |  |  |  |
| Microphone body loss | dB | 15 | 15 | 15 | 15 | 15 |
| Max e.i.r.p. acceptable | dBm | -21.9 | 10.1 | 18.1 | 20.1 | 30.1 |
| Microphone e.i.r.p. allowed | dBm | 17 | 17 | 17 | 17 | 17 |
| Conclusion |  | Guard band | Guard band | Ok | Ok | Ok |

Table 34: Calculation of maximum allowed microphone e.i.r.p. for an LTE pico BS 
(Scenario 14)

| Frequency range | MHz | 1785.4-1789.8 |
| --- | --- | --- |
| Offset from the edge | MHz | 0.4-4.8 |
| Victim BS characteristics |  |  |
| Narrow-band blocking level | dBm | -51.7 |
| Attenuation calculation |  |  |
| Path loss | dB | 52.1 |
| Antenna gain (w/ cable loss) | dBi | 0 |
| Antenna discrimination | dB | 0 |
| Handheld Microphone |  |  |
| Microphone body loss | dB | 1 |
| Max e.i.r.p. acceptable | dBm | 1.3 (Note 1) |
| Microphone e.i.r.p. allowed | dBm | 13 |
| Conclusion |  | Guard band |
| Body worn Microphone |  |  |
| Microphone body loss | dB | 15 |
| Max e.i.r.p. acceptable | dBm | 15.3 (Note 2) |
| Microphone e.i.r.p. allowed | dBm | 17 |
| Conclusion |  | Limited e.i.r.p. |

Note 1: With a separation distance of 11.50m, handheld microphones can transmit at full e.i.r.p. (13 dBm).

Note 2: With a separation distance of 6m, body worn microphones can transmit at full e.i.r.p. (17 dBm).

Calculation tables for out-of-band e.i.r.p. (out-of-band emissions case)

The in-band interference level is given in the formula below:

In-band_Interference_Level = Thermal_Noise + Noise_Figure + INR

- Thermal_Noise = 10 log (kB.T.BW.1000),

where kB is the Boltzmann constant, T = 290 K, and BW is the bandwidth considered in Hz

- INR = 10 log (10D/10 - 1)

where D is the desensitization of the victim receiver (BS or UE)

Outdoor, UE, Scenarios 1 and 2:

For an outdoor UE (scenarios 1 and 2), the maximum out-of-band emissions e.i.r.p. acceptable from a microphone is given by the following formula:

Mic_ e.i.r.p.max,oob = In-band_Interference_Level + Path_Loss - UE_Antenna_Gain + UE_Body_Loss + Mic_Body_Loss

where path loss is calculated according to IEEE 802.11 Model C propagation model and a separation distance of 15m.

Table 35: Calculation of maximum allowed out-of-band microphone e.i.r.p. for a GSM UE 
(Scenario 1)

| Victim UE characteristics |  |  |
| --- | --- | --- |
| Interferer power allowed | dBm/200kHz | -113.0 |
| Attenuation calculation |  |  |
| Path loss | dB | 68.2 |
| Antenna gain | dBi | -4 |
| UE body loss | dB | 3 |
| Handheld Microphone |  |  |
| Microphone body loss | dB | 1 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -36.8 |
| Body worn Microphone |  |  |
| Microphone body loss | dB | 15 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -22.8 |

Table 36: Calculation of maximum allowed out-of-band microphonee.i.r.p. for an LTE UE
(Scenario 2)

| Victim UE characteristics |  |  |
| --- | --- | --- |
| Interferer power allowed | dBm/200kHz | -112.0 |
| Attenuation calculation |  |  |
| Path loss | dB | 68.2 |
| Antenna gain | dBi | -4 |
| UE body loss | dB | 3 |
| Handheld Microphone |  |  |
| Microphone body loss | dB | 1 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -35.8 |
| Body worn Microphone |  |  |
| Microphone body loss | dB | 15 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -21.8 |

Outdoor, macro BS, Scenarios 5 and 6:

For an outdoor macro BS (scenarios 5 and 6), the maximum out-of-band emissions e.i.r.p. acceptable from a microphone is given by the following formula:

Mic_e.i.r.p.max,oob = In-band_Interference_Level + Path_Loss - BS_Antenna_Gain + BS_Antenna_Discrimination + Mic_Body_Loss

where path loss is calculated according to Extended Hata propagation model and a separation distance of 100m.

Table 37: Calculation of maximum allowed out-of-band microphone e.i.r.p. for a GSM macro BS (Scenario 5)

| Victim BS characteristics |  |  |
| --- | --- | --- |
| Interferer power allowed | dBm/200kHz | -119 |
| Attenuation calculation |  |  |
| Path loss | dB | 100.9 |
| Antenna gain (w/ cable loss) | dBi | 15 |
| Antenna discrimination | dB | 15 |
| Handheld Microphone |  |  |
| Microphone body loss | dB | 1 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -17.1 |
| Body worn Microphone |  |  |
| Microphone body loss | dB | 15 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -3.1 |

Table 38: Calculation of maximum allowed out-of-band microphone e.i.r.p. for an LTE macro BS (Scenario 6)

| Victim BS characteristics |  |  |
| --- | --- | --- |
| Interferer power allowed | dBm/200kHz | -122 |
| Attenuation calculation |  |  |
| Path loss | dB | 100.9 |
| Antenna gain (w/ cable loss) | dBi | 15 |
| Antenna discrimination | dB | 15 |
| Handheld Microphone |  |  |
| Microphone body loss | dB | 1 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -20.1 |
| Body worn Microphone |  |  |
| Microphone body loss | dB | 15 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -6.1 |

Indoor, UE, Scenarios 9 and 10:

For an indoor UE (scenarios 9 and 10), the maximum out-of-band emissions e.i.r.p. acceptable from a microphone is given by the following formula:

Mic_e.i.r.p.max,oob = In-band_Interference_Level + Path_Loss - UE_Antenna_Gain + UE_Body_Loss + Mic_Body_Loss

where path loss is calculated according to IEEE 802.11 Model C propagation model and a separation distance of 5m.

Table 39: Calculation of maximum allowed out-of-band microphone e.i.r.p. for a GSM UE 
(Scenario 9)

| Victim UE characteristics |  |  |
| --- | --- | --- |
| Interferer power allowed | dBm/200kHz | -113.0 |
| Attenuation calculation |  |  |
| Path loss | dB | 51.5 |
| Antenna gain | dBi | -4 |
| UE body loss | dB | 3 |
| Handheld Microphone |  |  |
| Microphone body loss | dB | 1 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -53.5 |
| Body worn Microphone |  |  |
| Microphone body loss | dB | 15 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -39.5 |

Table 40: Calculation of maximum allowed out-of-band microphone e.i.r.p. for an LTE UE 
(Scenario 10)

| Victim UE characteristics |  |  |
| --- | --- | --- |
| Interferer power allowed | dBm/200kHz | -112.0 |
| Attenuation calculation |  |  |
| Path loss | dB | 51.5 |
| Antenna gain | dBi | -4 |
| UE body loss | dB | 3 |
| Handheld Microphone |  |  |
| Microphone body loss | dB | 1 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -52.5 |
| Body worn Microphone |  |  |
| Microphone body loss | dB | 15 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -38.5 |

Indoor, pico BS, Scenarios 13 and 14:

For an indoor pico BS (scenarios 13 and 14), the maximum out-of-band emissions e.i.r.p. acceptable from a microphone is given by the following formula:

Mic_e.i.r.p.max,oob = In-band_Interference_Level + Path_Loss - BS_Antenna_Gain + BS_Antenna_Discrimination + Mic_Body_Loss

where path loss is calculated according to IEEE 802.11 Model C propagation model and for a separation distance of 5m.

Table 41: Calculation of maximum allowed out-of-band microphone e.i.r.p. for a GSM pico BS (Scenario 13)

| Victim BS characteristics |  |  |
| --- | --- | --- |
| Interferer power allowed | dBm/200kHz | -113 |
| Attenuation calculation |  |  |
| Path loss | dB | 52.1 |
| Antenna gain (w/ cable loss) | dBi | 0 |
| Antenna discrimination | dB | 0 |
| Handheld Microphone |  |  |
| Microphone body loss | dB | 1 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -59.9 |
| Body worn Microphone |  |  |
| Microphone body loss | dB | 15 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -45.9 |

Table 42: Calculation of maximum allowed out-of-band microphone e.i.r.p. for an LTE pico BS (Scenario 14)

| Victim BS characteristics |  |  |
| --- | --- | --- |
| Interferer power allowed | dBm/200kHz | -108 |
| Attenuation calculation |  |  |
| Path loss | dB | 52.1 |
| Antenna gain (w/ cable loss) | dBi | 0 |
| Antenna discrimination | dB | 0 |
| Handheld Microphone |  |  |
| Microphone body loss | dB | 1 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -54.9 |
| Body worn Microphone |  |  |
| Microphone body loss | dB | 15 |
| Max out-of-band e.i.r.p. | dBm/200kHz | -40.9 |

MCL calculations for PMSE towards LTE

Results

This subsection contains MCL calculation results for PMSE towards LTE based on the parameters defined in the subsection below, for two different propagation models.

Table 43: Minimum distance calculation in kilometers

| Propagation Model | PMSE Body Loss | PMSE -> LTE UE with frequency separation of: |  | PMSE -> LTE pico BS with frequency separation of: |  | PMSE -> LTE Macro BS with frequency separation of: |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 KHz | 200KHz | 0 KHz | 200KHz | 0 KHz | 200KHz |
| FSPL | 1 dB | 0.213 | 0.031 | 0.855 | 0,035 | 8.550 | 0.345 |
|  | 7 dB | 0.107 | 0.016 | 0.429 | 0,017 | 4.285 | 0.218 |
| IEEE 802.11C | 1 dB | 0.043 | 0.014 | 0.095 | 0,015 | N/A |  |
|  | 7 dB | 0.029 | 0.010 | 0.064 | 0,010 |  |  |
| Extend Hata for Metropolitan areas | 1 dB | N/A |  | N/A |  | 0.332 | 0.054 |
|  | 7 dB |  |  |  |  | 0.224 | 0.041 |

Parameters

This subsection contains the parameters used in the MCL calculations.

Table 44: Parameters for the LTE system

| Parameter | Unit | UE | Pico BS | Macro BS |
| --- | --- | --- | --- | --- |
| ACS | dB | 33 | 46 | 46 |
| Frequency | MHz | 1810 | 1780 | 1780 |
| Noise figure | dB | 9 | 13 | 5 |
| Noise level | dBm | -95,4 | -94,4 | -102,4 |
| I/N | dB | 0 | -6 | -6 |
| Body loss | dB | 3 | 0 | 0 |
| Tx Bandwidth | MHz | 9 | 4,5 | 4,5 |
| Antenna Gain | dBi | -4 | 0 | 15 |

Table 45: Parameters for PMSE

| Parameter | Unit | UE | Pico BS |
| --- | --- | --- | --- |
| Frequency offset | KHz | 0 | 200 |
| Bandwidth | KHz | 200 | 200 |
| Frequency | MHz | 1785.1 | 1785.3 |
|  |  | 1804.9 | 1804.7 |
| ACLR | dB | 16.3 | 48.8 |
| Antenna Gain | dBi | 0 |  |
| Body Loss | dB | 1 |  |
|  |  | 7 |  |
| Transmit Power | dBm | 13 |  |

Adaptation of CEPT Report 30 derivation

The method described in CEPT Report 30 [3] has been adjusted to the band 1800 MHz. The interference from low power applications to ECN (on the basis of section A5.2 of CEPT Report 30) has been considered.

The impact of the unwanted emissions and the blocking from PMSE to ECN has been considered.

For this method, it is assumed that, in practice, a device’s rejection of an adjacent-channel interferer is 3 dB better than that implied by the 3GPP blocking specifications.

Table 46: Calculation of maximum interfering power

| Specification | Narrow band blocking on MFCN TS | Narrow band blocking on MFCN BS |
| --- | --- | --- |
| Source | TS 36.101 section 7.6.3 | TS 36.104 section 7.5 |
| Blocking requirement | -55 dBm | -49 dBm |
| Associated desensitization | 13 dB | 6 dB |
| Target desensitization | 3 dB | 1 dB |

Improved ACS for narrow-band interfering signal according to frequency offset.

For an offset of 2 MHz from the victim’s channel-edge, the narrowband selectivity is naturally greater than for a 0 MHz offset. Starting from a narrowband selectivity at zero offset, a linear slope of 17/5 dB/MHz would result in a higher narrowband ACS value at higher offset.

Duplex filter rejection according to frequency offset.

It is assumed that an MFCN TS receiver duplex filter provides an additional rejection of 2 dB at 2 MHz offset from the channel-edge for narrow band signals (<1 MHz).

A linear slope of 2/2 dB/MHz for the duplex filter rejection was considered

The Table below shows that a 1 MHz guard band would be needed to protect LTE TS from hand-held radiomicrophone emissions. Indeed, for such a guard band, the in-band e.i.r.p. would be 13 dBm (which is the in-band hand-held radiomicrophone e.i.r.p.) and in this case, the out-of-block e.i.r.p. above 1805 MHz should be -31 dBm/10 MHz.

Nevertheless a 1.2 MHz guard band would be preferable to protect TS LTE from hand-held radiomicrophone emissions. Indeed, for this guardband, the out-of-block e.i.r.p. above 1805 MHz should be -27 dBm/10 MHz (higher than -31 dBm/10 MHz) which is equivalent to  -44 dBm/200 kHz to be compared to the -38.9 dBm/200 kHz of method 4 (see Table  below).

At the end, the better compromise would indeed probably to consider a 1.2 MHz guard band as it enables higher radiomicrophone out-of-block emissions than for a 1 MHz guard band.

Table 47: Interfering link budget calculations

|  |  | BEM hand held PMSE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Interferer |  |  |  |  |
|  |  | PMSE | PMSE | PMSE | PMSE | PMSE |  |
|  |  | Victim : | Victim : | Victim : | Victim : | Victim : |  |
| Parameter | Units | LTE TS | LTE TS | LTE TS | LTE TS | LTE TS | Comment |
| Interferer frequency | MHz | 1805-1804,8 | 1804,8-1804,6 | 1804-1803,8 | 1803,8-1803,6 | 1803,6-1803,4 |  |
| 3GPP specs for victim |  | Narrow band blocking on LTE TS | Narrow band blocking on LTE TS | Narrow band blocking on LTE TS | Narrow band blocking on LTE TS | Narrow band blocking on LTE TS |  |
| Interferer power | dBm | -55,00 | -55,00 | -55,00 | -55,00 | -55,00 | Puw (Specified) |
| Implied desensitisation | dBm | 13,00 | 13,00 | 13,00 | 13,00 | 13,00 | D (Specified) |
| Imax 13 dB | dBm | -82.66 | -82.66 | -82.66 | -82.66 | -82.66 |  |
| Target desensitisation | dB | 3,00 | 3,00 | 3,00 | 3,00 | 3,00 | Performance criterion |
| Receiver NF | dB | 9,00 | 9,00 | 9,00 | 9,00 | 9,00 | NF |
| Thermal noise floor (9 MHz) | dBm | -95,43 | -95,43 | -95,43 | -95,43 | -95,43 | Pn = 10log(kTB) + NF + 30 |
| Receiver selectivity | dB | 27,66 | 28,3 | 31 | 31,7 | 32,4 | receiver selectivity=ACS(fg=0)+(17/5)*fg with ACS(fg=0)=Iinterf power-Imax 13 dB |
| Victim's performance |  |  |  |  |  |  |  |
| Performance beyond specs | dB | 3,00 | 3,00 | 3,00 | 3,00 | 3,00 | Gdevice |
| Guard-band | MHz | 0 | 0,2 | 1 | 1,2 | 1,4 | fg (Guardband at victim's boundary) |
| Duplexer attenuation | dB | 0,00 | 0,6 | 3,2 | 3,8 | 4,5 | Gduplex (given the guard-band fg)=(16/5)*fg |
| Receiver selectivity | dB | 31 | 32 | 37 | 39 | 40 | ACS = ACS + Gdevice + Gduplex |
| Geometry |  |  |  |  |  |  |  |
| Horizontal distance | m | 15,00 | 15,00 | 15,00 | 15,00 | 15,00 |  |
| Interferer height | m | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 |  |
| Victim height | m | 1,5 | 1,5 | 1,5 | 1,5 | 1,5 |  |
| Link budget |  |  |  |  |  |  |  |
| Interferer body-loss | dB | -1,00 | -1,00 | -1,00 | -1,00 | -1,00 | Gb,i |
| Hata path loss | dB | -68,2 | -68,2 | -68,2 | -68,2 | -68,2 | Gpl |
| Victim body loss | dB | -3,00 | -3,00 | -3,00 | -3,00 | -3,00 | Gb,v |
| Victim ant. elevation pattern | dB | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | dGa,v |
| Victim antenna gain | dBi | 0,00 | 0,00 | 0,00 | 0,00 | 0,00 | Ga,v |
| Coupling loss | dB | -72 | -72 | -72 | -72 | -72 | G = Gb,i + Gpl + Gwl + Gb,v + Ga,v+dGa,v |
| Interferer in-block e.i.r.p. | dBm | 7,00 | 8,00 | 13,00 | 13,00 | 13,00 | Pib |
| Interferer out-of-block e.i.r.p. | dBm/(10 MHz) | -35 | -32 | -31 | -27 | -26 | Linear: Poob = PI/G - Pib/ACS |

Table 48: Interfering link budget calculations

|  |  | BEM body worn PMSE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Interferer | Interferer | Interferer | Interferer |  |
|  |  | PMSE | PMSE | PMSE | PMSE |  |
|  |  | Victim : | Victim : | Victim : | Victim : |  |
| Parameter | Units | LTE TS | LTE TS | LTE TS | LTE TS | Comment |
| Interferer Frequency | MHz | 1804,9-1804,7 | 1804,7-1804,5 | 1804,1-1803,9 | 1803,9-1803,7 |  |
| 3GPP specs for victim |  | Narrow band blocking on LTE TS | Narrow band blocking on LTE TS | Narrow band blocking on LTE TS | Narrow band blocking on LTE TS |  |
| Interferer power | dBm | -55,00 | -55,00 | -55,00 | -55,00 | Puw (Specified) |
| Implied desensitisation | dBm | 13,00 | 13,00 | 13,00 | 13,00 | D (Specified) |
| Imax 13 dB | dBm | -82.66 | -82.66 | -82.66 | -82.66 |  |
| Target desensitisation | dB | 3,00 | 3,00 | 3,00 | 3,00 | Performance criterion |
| Receiver NF | dB | 9,00 | 9,00 | 9,00 | 9,00 | NF |
| Thermal noise floor (9 MHz) | dBm | -95,43 | -95,43 | -95,43 | -95,43 | Pn = 10log(kTB) + NF + 30 |
| Receiver selectivity | dB | 27,66 | 28,4 | 31,7 | 32,4 | receiver selectivity=ACS(fg=0)+4*(fg-0.1) with ACS(fg=0)=Iinterf power-Imax 13 dB |
| Victim's performance |  |  |  |  |  |  |
| Performance beyong spec |  | 3 | 3 | 3 | 3 | Gdevice |
| Guard-band | MHz | 0.1 | 0.3 | 1.1 | 1.3 | fg (Guardband at victim's boundary) |
| Duplexer attenuation | dB | 0,00 | 0,2 | 0.8 | 1 | Gduplex (given the guard-band fg)=(2/2)*(fg-0.1) |
| Receiver selectivity | dB | 31 | 32 | 34.2 | 35 | ACS = ACS + Gdevice + Gduplex |
| Geometry |  |  |  |  |  |  |
| Horizontal distance | m | 15,00 | 15,00 | 15,00 | 15,00 |  |
| Interferer height | m | 1,5 | 1,5 | 1,5 | 1,5 |  |
| Victim height | m | 1,5 | 1,5 | 1,5 | 1,5 |  |
| Link budget |  |  |  |  |  |  |
| Interferer body-loss (body worn) | dB | -15,00 | -15,00 | -15,00 | -15,00 | Gb,i |
| Hata path loss | dB | -61,03 | -61,03 | -61,03 | -61,03 | Gpl |
| Victim body loss | dB | -3,00 | -3,00 | -3,00 | -3,00 | Gb,v |
| Victim ant. elevation pattern | dB | 0,00 | 0,00 | 0,00 | 0,00 | dGa,v |
| Victim antenna gain | dBi | -4,00 | -4,00 | -4,00 | -4,00 | Ga,v |
| Coupling loss | dB | -83,03 | -83,03 | -83,03 | -83,03 | G = Gb,i + Gpl + Gwl + Gb,v + Ga,v+dGa,v |
| Interferer in-block e.i.r.p. | dBm | 17,00 | 17,00 | 17,00 | 17,00 | Pib |
| Interferer out-of-block e.i.r.p. | dBm/(10 MHz) | -18.5 | -16.5 | -15.4 | -13.8 | Linear: Poob = PI/G - Pib/ACS |

Table 49: Interfering link budget calculations

|  |  | BEM hand held | BEM bodyworn |  |
| --- | --- | --- | --- | --- |
|  |  | Interferer | Interferer |  |
|  |  | PMSE | PMSE |  |
|  |  | Victim : | Victim : |  |
| Parameter | Units | LTE BS | LTE BS | Comment |
| Interferer Frequency | MHz | 1785.1-1785.3 | 1785.1-1785.3 |  |
| 3GPP specs for victim |  | Narrow band blocking on LTE TS | Narrow band blocking on LTE TS |  |
| Interferer power | dBm | -49 | -49 | Puw (Specified) |
| Implied desensitisation | dBm | 6 | 6 | D (Specified) |
| Imax 6 dB | dBm | -94.7 | -94.7 |  |
| Target desensitisation | dB | 1 | 1 | Performance criterion |
| Receiver NF | dB | 5,00 | 5,00 | NF |
| Thermal noise floor (9 MHz) | dBm | -99.4 | -99.4 | Pn = 10log(kTB) + NF + 30 |
| Receiver selectivity | dB | 45.7 | 45.7 | receiver selectivity=ACS(fg=0)+4*(fg-0.1) with ACS(fg=0)=Iinterf power-Imax 13 dB |
| Victim's performance |  |  |  |  |
| Performance beyong spec |  | 3 | 3 |  |
| Guard-band | MHz | 0.1 | 0.1 | fg (Guardband at victim's boundary) |
| Receiver selectivity | dB | 48.7 | 48.7 | ACS = ACS + Gdevice |
| Geometry |  |  |  |  |
| Horizontal distance | m | 60 | 60 |  |
| Interferer height | m | 1,5 | 1,5 |  |
| Victim height | m | 30 | 30 |  |
| Link budget |  |  |  |  |
| Interferer body-loss | dB | -7 | -15 | Gb,i |
| Hata path loss | dB | -85.6 | -85.6 | Gpl |
| Victim ant. elevation pattern | dB | -15,00 | -15,00 | dGa,v |
| Victim antenna gain | dBi | 15 | 15 | Ga,v |
| Coupling loss | dB | -92.6 | -100.6 | G = Gb,i + Gpl + Gwl + Gb,v + Ga,v+dGa,v |
| Interferer in-block e.i.rp. | dBm | 13 | 17 | Pib |
| Interferer out-of-block e.i.r.p. | dBm/(10 MHz) | -12.7 | -4.7 | Linear: Poob = PI/G - Pib/ACS |

Derivation of a BEM based on mobile UE emission limit requirements

The deployment scenario for the BEM corresponds to low power devices with deployment topology similar to UE. It should be noted that UE specifications already include specific requirements to avoid UE to UE or UE to BS interference for equipment operating in the 1710-1785/1805-1880 MHz mobile band. Protection of mobile service can simply be insured through extension of such requirements to other equipment operating in the centre gap.

The introduction of UMTS, LTE and WiMAX terminals in the band 1710-1785/1805-1880 MHz has been studied in ECC Report 82 [25] and CEPT Report 40 [12] and has been found satisfactory. Any system creating no more interference than GSM/UMTS/LTE/WiMAX terminals in the band 1710-1785/1805-1880 MHz should therefore clearly be compatible with services in this band.

Emissions in the 1710-1785 MHz band

Maximum e.i.r.p. for a GSM UE is 30 dBm; and the one for an LTE UE is 23 dBm. These values are above the 17 dBm that is the maximum e.i.r.p. for a body worn wireless microphone.

The GSM specification 3GPP TS 45.005 (see Table 4.2.1.3-b1) Table 50, provides the following spectrum emission mask:

Table 50: GSM UE spectrum emission mask

| Offset from centre frequency (in kHz) | dBc |  |
| --- | --- | --- |
| 0 | 0.5 | 30.5 dBm / 200 kHz |
| 100 | 0.5 | 30.5 dBm / 200 kHz |
| 200 | -30 | 0 dBm / 200 kHz |
| 250 | -33 | -3 dBm / 200 kHz |
| 400 | -60 | -30 dBm / 200 kHz |
| 600..1800 | -60 | -30 dBm / 200 kHz |
| 1800..6000 | -65 | -35 dBm / 200 kHz |

The LTE specification 3GPP TS 36.101 (see Table 6.6.2.1.1-1) Table 51, provides the following spectrum emission mask.

Table 51: LTE UE spectrum emission mask

| Offset from the edge (in MHz) | dBm |  |
| --- | --- | --- |
| 0..1 | -18 dBm / 30 kHz | -9.8 dBm / 200 kHz |
| 1..5 | -10 dBm / MHz | -17 dBm / 200 kHz |

As mobile deployments can occur in adjacent channels, and compared to the wireless microphone spectrum emission mask (see Section 3.1), it is clear that an emission level of -10 dBm / MHz in 1710-1785 MHz does not create undue interference to existing networks in that band.

Emissions in the 1805-1880 MHz band

The ERC/REC 74-01, Annex 2 Table 2.1, [2] (as ITU-R SM.329-11) indicates that unwanted emissions in the spurious domain from land mobile terminals and radio microphones should be limited to -30 dBm / MHz above 1 GHz. This limit is quoted in the LTE specification 3GPP TS 36.101 (see Table 6.6.3.1-2).

Therefore, PMSE audio equipment can be allowed to transmit in 1805-1880 MHz at a power of -30 dBm / MHz. As a result, an emission limit of -37 dBm / 200 kHz can be justified.

According to the wireless microphone spectrum emission mask (see section 3.1), this requirement is met:

at 135 kHz from the edge for a 200 kHz channel microphone;

at 252 kHz from the edge for a 400 kHz channel microphone:

at 333 kHz from the edge for a 600 kHz channel microphone.

Table 52: Limits for Terminal stations (In-block power limits for terminal stations)

|  | Maximum mean power (including Automtic Transmitter Power Control (ATPC) range) |
| --- | --- |
| Total radiated power (TRP) | 31 dBm/5 MHz |
| e.i.r.p. | 35 dBm/5 MHz |
| NB: e.i.r.p. should be used for fixed or install terminal stations and the TRP should be used for the mobile or nomadic terminal stations. TRP is a measure of how much power the antenna actually radiates. The TRP is defined as the integral of the power transmitted in different directions over the entire radiation sphere. |  |

Conclusion

Based on previous CEPT studies and the spurious emission requirement, the following Block Edge Mask for PMSE audio terminals operating in 1785-1805 MHz can be justified:

Table 53: Block Edge Mask for PMSE audio terminals operating in 1785-1805 MHz

| Frequency range | Emission limit (e.i.r.p.) |
| --- | --- |
| 1710-1785 MHz | -17 dBm / 200 kHz |
| 1785-(Note 1) MHz | 17 dBm / channel |
| (Note 1)-1805 MHz | -10 dBm / 200 kHz |
| 1805-1880 MHz | -37 dBm / 200 kHz |
| Note 1: - 1804.8 MHz for a 200 kHz channel - 1804.7 MHz for a 400 kHz channel - 1804.6 MHz for a 600 kHz channel |  |

Note: In order to prevent interferences, a guard-band of 200 kHz is usually implemented between two mobile networks when at least one of them uses GSM technology. This is similar to the results obtained here.

Setup procedure for an interference free operation of wireless microphone and in-ear monitor links

In this annex the setup procedure for an interference free operation is described. This procedure is performed before the PMSE user can go online and during the use of the equipment. The procedure is used for analogue systems and in principle also for digital systems.

Secure interference free operation of wireless microphone and in-ear monitor links

Microphone user sets up his equipment.

First thing that is switched on is the receiver and listening to its output. This is either done through the connected PA system via the mixing console or through a headphone directly connected to the receiver headphone socket.

If there is no signal audible the receiver’s frequency can be used for operation.

There may or may not be an audible signal at the receiver output. If there is an audible signal, the frequency is already occupied and cannot be used. However, it is possible that the frequency is occupied but there is no audio signal coming from the receiver. Most analogue PMSE systems use a "tone key" system which helps prevent unwanted signals and noise from being output by the receiver. Unless the unwanted PMSE transmission is using the same tone key frequency, the user will not be able to hear it. However, the unwanted signal may still cause interference if it is strong enough. Likewise, digital PMSE systems use various modulation and coding schemes which are typically incompatible. It is quite possible that a channel could be occupied by a different type of digital system and no audio would be heard from the receiver. The same thing would happen if an analogue PMSE system picked up a digital PMSE transmission, and vice versa.

The user also monitors the RF and Audio Level indicators to prove the audible output of the link. If there is no indication at these two level meters the frequency is not in use and can be used for interference free operation: the transmitter can be switched on at the receiver’s frequency.

The most basic method for determining whether a given frequency is in use is by monitoring the receiver's RF indicator. However, the lack of an indication does not necessarily mean that interference-free operation is assured because these RF indicators are not precision devices and there could still be a residual signal present on the frequency.

If there is a signal audible or the two level meter show signals a different frequency will be adjusted at the receiver and the same test has to be done again.

Some receivers do this check already alone in “sound-check-mode” or similar. In this mode the spectrum is scanned and frequencies for operation are indicated. The user can choose out of the offered frequencies one for his operation.

Many recent PMSE models incorporate some type of built-in scanning capability that will help the user determine which channels are usable. The operation of this function may range from basic to highly sophisticated, rivalling the performance of a dedicated spectrum analyser. In general, these scanners are useful for finding "open" frequencies.

Certain manufacturers offer for their receiver systems an Ethernet link so that the receiver can be connected to a notebook. These manufacturers also offer on their homepage free of charge software that makes the receiver to operate as a spectrum scanner – similar to the above mentioned “sound-check-mode”. This time all the results can be seen on the notebooks screen and a frequency gap for the operation can be chosen.

Several manufacturers have enabled their PMSE systems to be connected to a computer to provide enhanced scanning and frequency management capabilities. A variety of interfaces are used, including Ethernet and USB. For multichannel operation of wireless links the manufacturers offer sets of frequencies which are calculated to be free of interference. These frequencies have to be chosen for the operation for maximum security of the wireless link. In the vast majority these frequencies are stored as presets in both: receiver and transmitter. PMSE user coordination on site

The preselected compatible channel groups provided by PMSE

Manufacturers are selected to be free from Inter-modulation Distortion (IMD) interference. This does not mean that they are free from all sources of interference; only interference caused by interaction between multiple systems. The use of these frequencies should help ensure that a multi-channel PMSE system can be operated reliably. They do not affect whether the link is "secure" in the sense that the transmissions are protected from outside interference or interception. PMSE user coordination on site

PMSE users coordinate themselves on site for interference free operation. The current analogue systems allow listening to the links of other users and making it easy to get into personal contact with these operators. In this personal contact the operation time or the coordinating of the frequency use will be coordinated personally.

For the reasons given above, either analogue or digital systems necessarily allow users to monitor the links of other users. Therefore, this technique is not commonly used or relied upon as a means of frequency coordination at multi-user events.

It has to be noticed that the coordination has to be modified with digital systems as the finding of the “interfering” operator will be more complicated. Solutions for this, like monitoring of signal strength indicator, have to be discussed.

For large events, there will typically be an assigned frequency coordinator who also monitors frequency use. PMSE system operators are normally required to register their systems with the coordinator to prevent interference. Coordinators typically use spectrum analysers to monitor and control spectrum usage.

Impact of MFCN Bandwidth on PMSE tx - MFCN LTE rx studies

Introduction

The LTE standard specifies several operating bandwidths (ETSI TS 136.101 - Table 5.6-1 and ETSI TS 136.104 - Table 5.6-1). The standard also differentiates between the Channel Bandwidth and the Transmission Bandwidth (e.g. for a specific test channel). The relationship between the Channel Bandwidth and the Transmission Bandwidth is provided in the Table 54 for both uplink and downlink.

Table 54: Channel Bandwidth and Transmission Bandwidth in ETSI TS 136.101 and ETSI TS 136.104

| Link | Channel BW | Transmission BW (Sensitivity) |
| --- | --- | --- |
| Uplink | 10 | 4.5 MHz (25RB, ETSI TS 136.104, Table 7.2.1-x) (Sensitivity Wide Area BS = -101.5 dBm, ETSI TS 136.104 Table 7.2.1-1 Sensitivity Local Area BS = -93.5 dBm, ETSI TS 136.104 Table 7.2.1-2) |
| Downlink | 10 | 9 MHz (50RB, ETSI TS 136.101, Table A.2.2.1.1-1) (Sensitivity = -94 dBm, ETSI TS 136.101 Table 7.3.1-1) |

The selection of an assumed bandwidth has an impact on the noise level assumed in the study, since the noise level N is directly related to the bandwidth by the formula:

N = 10.log10(k.T) + 10.log10(BW) + NF

where k = Boltzmann constant; T = 290 K; BW = Bandwidth and NF = Noise Figure.

The Table below indicates the different values of N, for NF(LTE UE) = 9 dB, NF(LTE macro BS) = 5 dB and NF(LTE pico BS) = 13 dB.

Table 55: Noise levels for Channel and Transmission Bandwidths

| Link | Channel BW / Noise | Transmission BW (MHz) / Noise (dBm) |
| --- | --- | --- |
| Uplink | 10 MHz -99 dBm (Macro) -91 dBm (Pico) | 4.5 MHz -102.5 dBm (Macro) -94.5 dBm (Pico) |
| Downlink | 10 MHz / -95 dBm | 9 MHz / -95.5 dBm |

The present Annex demonstrates that the studies on the interference from PMSE to MFCN (LTE) are not impacted by the choice of either the Channel Bandwidth of the Transmission Bandwidth.

PMSE Out-of-Band emission limits

In OOBE studies, we compare the level of OOBE in the receiver’s bandwidth with the noise level on the same bandwidth, ensuring that a target ratio of interference on noise is respected, i.e. (OOBE – N) is constant and independent of the receiver bandwidth.

Assuming 2 different receiver bandwidths BW1 and BW2, we would have:

N1 = 10.log10(k.T) + NF + 10.log10(BW1)

and

N2 = 10.log10(k.T) + NF + 10.log10(BW2)

Given that OOBE - N is constant:

OOBE1 - N1 = OOBE2 - N2

which implies that

OOBE1 - 10.log10(BW1) =  OOBE2 - 10.log10(BW2)

Demonstrating that the normalized values of OOBE1 and OOBE2 (in dBM/MHz) are strictly identical.

Therefore, the choice of a specific receiver bandwidth has no impact on the acceptable level of OOBE derived.

Blocking

Assuming 2 different receiver bandwidths BW1 and BW2, we would have:

N1 = 10.log10(k.T) + NF + 10.log10(BW1)

and

N2 = 10.log10(k.T) + NF + 10.log10(BW2)

From the derivation of the blocking response derivation in ANNEX 3:

Blocking Response = N + 10.log10[10^(DSTANDARD/10) - 1] - IOOB-STANDARD

In ETSI TS 136.104 and ETSI TS 136.101, IOOB-STANDARD and DSTANDARD are independent from the consideration of Channel Bandwidth of Transmission Bandwidth. Therefore:

Blocking Response (N2) = Blocking Response (N1) + (N2-N1)

In turn, in order to find the absolute level for blocking, it is necessary to calculate the IIB-TARGET based on the noise floors (N1 and N2) and a Margin which is a function of DTARGET:

IIB-TARGET(N1) = N1 + Margin(DTARGET),

and

IIB-TARGET(N2) = N2 + Margin(DTARGET).

Therefore:

| IOOB-TARGET(N2) | = | IIB-TARGET(N2) - Blocking Response (N2) |
| --- | --- | --- |
|  | = | N2 + Margin(DTARGET) – [Blocking Response(N1) + (N2-N1)] |
|  | = | N1 + Margin(DTARGET) – Blocking Response (N1) |
|  | = | IOOB-TARGET(N1) |

Which demonstrates that the Blocking Level derived is independent from the consideration of either the Channel Bandwidth or the Transmission Bandwidth.

Considerations on Receiver Blocking Response and Receiver Blocking Level

Preliminary

The blocking parameters of a standard are specified for specific values testing parameters which do not always correspond to typical operational values. In particular, blocking values in LTE standards are specified for desensitization values (e.g. 6 dB for BS, 13 dB for MS) which may not correspond to the desired operational value for specific studies (e.g. 1 dB for BS and 3 dB for MS). It is therefore sometimes necessary to translate the Blocking Level defined by the standard for a desensitization DSTANDARD into the corresponding Blocking Level for another desensitization DTARGET.

The analysis in Section A7.3 is conducted for a standard that provides a maximum power (Blocking Level) of an interfering signal received outside of the in-band, for a given frequency offset between the wanted signal and the interfering signal. This absolute blocking level is specified in the standard for a specific desensitization DSTANDARD. This is the case for the narrowband blocking specifications in LTE standards (ETSI TS 136.101 and ETSI TS 136.104). Ii is also the case for the blocking specifications for the 4th adjacent channel and the following ones in GSM (ETSI TS 145.005).

The analysis in Section A7.4 is conducted for a standard that provides a protection ratio. This is the case for the blocking specifications for the first 3 adjacent channels in GSM (ETSI TS 145.005).

Definitions

| Abbreviation | Explanation |
| --- | --- |
| Blocking Level | Maximum power (Maximum IOOB) of an interfering signal outside of the in-band, for a given frequency offset between the wanted signal and the interfering signal, given in dBm |
| Blocking Response | Receiver filter attenuation of signals outside of receiver’s channel/band, given in dB. It is derived by the following equation: Blocking Response = IIB - IOOB |
| CSTANDARD | Wanted signal level defined by the standard for the blocking specification |
| D | Desensitization of the receiver in the presence of an interfering signal, given in dB. It corresponds to the ‘noise rise’ due to the interfering signal and is derived by the following equation in dB: D = 10.log10[(10^(N/10) + 10^(IIB/10))] - N |
| DSTANDARD | Desensitization defined by the standard for the blocking specification |
| DTARGET | Target desensitization for a specific interference study |
| IIB | IOOB in-band equivalent interfering signal |
| IIB-STANDARD | IOOB-STANDARD equivalent in-band interfering signal |
| IIB-TARGET | IOOB-TARGET equivalent in-band interfering signal |
| IOOB | Interfering signal at the RF input of a receiver, outside of the receiver’s bandwidth. |
| IOOB-STANDARD | Allowed power of an interfering blocking signal as specified by the standard (for DSTANDARD). |
| IOOB-TARGET | Allowed power of an interfering blocking signal for DTARGET. |
| N | Noise floor, given in dBm. N is derived from the following equation in dB: 10.log10(k.T.BW) + NF, where k = Boltzmann constant, T = 290 K, BW = Bandwidth, NF = Noise figure |
| SENSITIVITY | Minimum power of the wanted signal defined by the standard for appropriate reception in the absence of interference |

CSTANDARD is referred to in different standards and documents as:

Useful signal (ETSI TS 145.005 - Chapters 5.1.2 and 5.1.3)

Wanted signal mean power (ETSI TS 136.104 - Table 7.5.1-1)

Pw (ETSI TS 136.101 - Table 7.6.3.1-1)

Prefsens + desensitization (ETSI TS 136.104 - Table 7.5.1-1, ETSI TS 136.101 - Table 7.6.3.1-1)

“C”

CSTANDARD is specified for a given sensitivity and a given desensitization.

IOOB-STANDARD is referred to in different standards as:

Blocking signal level (ETSI TS 145.005 - Table 5.1-2a)

Puw (ETSI TS 136.101 - Table 7.6.3.1-1)

Interfering signal mean power (ETSI TS 136.104 - Table 7.5.1-1)

IOOB-STANDARD is specified for a given frequency offset, a given sensitivity and a given desensitization DSTANDARD.

IOOB-TARGET is derived for a given frequency offset, a given SENSITIVITY and a given desensitization DTARGET.

SENSITIVITY is referred to in different standards as:

Reference sensitivity level (ETSI TS 145.005 - Tables 6.2-1x),

Reference sensitivity (ETSI TS 136.101 - Table 7.3.1-1),

Reference sensitivity power level (ETSI TS 136.104 - Tables 7.2.1-1 and 7.2.1-2),

Prefsens (ETSI TS 136.101 - Table 7.3.1-1, ETSI TS 136.104 - Tables 7.2.1-1 and 7.2.1-2).

When the blocking level (Maximum IOOB) is given by the standards

Derivation of the Receiver Blocking Response

Goal

When an interfering signal IOOB is applied to the RF input of a receiver outside of the receiver’s bandwidth, the receiver will be interfered due to the non-perfect selectivity of the receiver’s filter. However, the receiver’s filter attenuate the interfering signal IOOB into an ‘equivalent in-band interfering signal’ IIB. In other words, the performance of the receiver are left unchanged in presence of the interfering signal IOOB at the given frequency offset, or in presence of the interfering signal IIB in the receiver’s bandwidth.

The receiver Blocking Response is defined as the receiver filter attenuation of signals outside of receiver’s channel/band (in dB):

Blocking Response = IIB - IOOB

The present section derives the receiver Blocking Response from the Blocking Level specified in the standard.

Derivation

Starting from:

DSTANDARD = 10.log10[10^(N/10) + 10^(IIB-STANDARD/10)] - N

IIB-STANDARD can be then derived from the following equation in dB:

IIB-STANDARD = N + 10.log10[10^(DSTANDARD/10) - 1]

IIB-STANDARD - N = 10.log10[10^(DSTANDARD/10) - 1]

For example,

For DSTANDARD = 16 dB, (IIB-STANDARD – N) = 15.9 dB (ETSI TS 136.101 - Table 7.6.3.1-1, Channel Bandwidth = 5 MHz).

For DSTANDARD = 13 dB, (IIB-STANDARD – N) = 12.7 dB (ETSI TS 136.101 - Table 7.6.3.1-1, Channel Bandwidth = 10MHz).

For DSTANDARD = 6 dB, (IIB-STANDARD – N) = 4.7 dB (ETSI TS 136.104 - Table 7.5.1-1, Wide Area BS and Local Area BS).

For DSTANDARD = 3 dB, (IIB-STANDARD – N) = 0 dB (ETSI TS 145.005 - Chapters 5.1.2 and 5.1.3).

Blocking Response is then derived by the following equation:

| Blocking Response | = | IIB-STANDARD - IOOB-STANDARD |
| --- | --- | --- |
|  | = | N + (IIB-STANDARD - N) - IOOB-STANDARD |
|  | = | N + 10.log10[10^(DSTANDARD/10) - 1] - IOOB-STANDARD |

Graphical representation

Figure 5: Derivation of the Blocking Response from the Blocking Level specified by the standard

Derivation of the receiver Blocking Level

Goal

From the Blocking Response, it is possible to derive the receiver Blocking Level for another value of desensitization.

In order to derive this new receiver Blocking Level, we assume that the Blocking Response is constant and fully linear over the complete range of desensitization from 0 to DSTANDARD.

Derivation

IIB-TARGET can be derived from the following equation:

IIB-TARGET = N + 10.log10[10^(DTARGET/10) - 1]

IOOB-TARGET can be derived from the following equation:

IOOB-TARGET = IIB-TARGET - Blocking Response

Graphical representation

Figure 6: Derivation of the Blocking Level for a desired DTARGET

When a protection ratio is given by the standards

This is the case for GSM, with regards to the 3 first adjacent channels.

In ETSI TS 145.005, Table 6.3-1, the CSTANDARD - IOOB-STANDARD is specified in dB for a desensitization DSTANDARD of 3 dB (see chapter 5.1.2 in ETSI TS 145 005).

First adjacent channel, [C/Ia1]Linear = C – Ia1= -9 dB,

Second adjacent channel, [C/Ia2]Linear4 = C –Ia2 = -41 dB,

Third adjacent channel, [C/Ia3]Linear4 = C – Ia3 = -49 dB.

These are the “blocking protection ratios”.

Derivation

Let’s consider for instance a BS and the first adjacent channel.

| CSTANDARD | = | SENSITIVITY + DSTANDARD |
| --- | --- | --- |
|  | = | -104 dBm + 3 dB |
|  | = | -101 dBm |

| IOOB-STANDARD | = | CSTANDARD - Blocking Protection Ratio |
| --- | --- | --- |
|  | = | -101 dBm - (-9 dB) |
|  | = | -92 dBm |

| Blocking Response | = | N + 10.log10[10^(DSTANDARD/10) -1] - IOOB-STANDARD |
| --- | --- | --- |
|  | = | -113 + 0 - (-92) |
|  | = | -21 dB |

The Receiver Blocking Level IOOB-TARGET for a desensitization DTARGET of 1 dB can be derived from the following equation:

| IOOB-TARGET | = | N + 10.log10[10^(DTARGET/10) -1] - Blocking Response |
| --- | --- | --- |
|  | = | -113 - 6 + 21 |
|  | = | -98 dBm |

A mixed scenario

Simulations of an outdoor event

There are a multitude of scenarios possible for PMSE equipment. It could for example be an outdoor event, or an indoor event in an urban or rural environment. The simulation scenario considered has focussed on the analysis of a real PMSE venue within a densely populated urban environment with both indoor and outdoor users of the mobile network.

- One suitable site has been identified in the city centre of Stockholm, Sweden. It is an example of a highly populated urban environment in close proximity to terrestrial MCNs. It is a square (Sergels Torg), which is commonly used for outdoor events. Next to the square is the house of culture (Kulturhuset). A view of the square and the house of culture is shown in  Figure 7: Sergels Torg and the house of culture in Stockholm, Sweden. One particular (of several) existing base station will be used in the SEAMCAT simulations. The base station covers the square, and the house of culture, and has approximately the same view as the photo in Figure 8, only directed more to the right.

Figure 7: Sergels Torg and the house of culture in Stockholm, Sweden

Three UL scenarios were simulated, two for GSM and one for LTE. The two GSM UL scenarios simulated were based on an SINR approach. These scenarios were as follows:

Scenario 1: Outdoor BTS, outdoor radio mic (on the square), outdoor MS (on the square)

Scenario 2: Outdoor BTS, outdoor radio mic (on the square), indoor MS (inside the house of culture)

The LTE UL scenario simulated was as follows and was done on the basis of BS desensitization:

Scenario 3: Outdoor BS and an outdoor radio mic (on the square)

Simulation set-up

The geometry of the Sergels Torg simulation is shown in Figure 8.

33m

33m

46m

10m

20.5m

BTS 1: 18dBi, 6-16 deg downtilt

1.5m

Event A

outdoor-outdoor

outdoor-indoor

20m

Figure 8: The Sergels Torg simulations

In the three scenarios, the simulations were first performed using an extended Hata propagation model for the interfering radio microphone to base station link, in order to account for a multi-path environment, and then repeated using a free space path loss model to account for the case in which there is a direct line of sight path between the radio microphone and the base station. In all wanted uplink cases (i.e. MS to BS) only the extended Hata model was considered. In the LTE simulation, simulations results were generated for EUTRA signals bandwidths of 1.4 MHz and 5 MHz.

Results

GSM

The criterion of C/(I+N) was used to measure the probability of interference. The results represent the probability of a C/(N+I) value less than 8 dB being encountered by the BTS under the various simulation conditions.

Table 57 and Table 58 contain the simulation results for the two GSM scenarios:

Radio mic outside (on the square), MS outside (Scenario 1)

Radio mic outside, MS inside (the building) (Scenario 2)

Table 56 tabulates the results for the scenario in which the path between the BTS and radio microphone is a multipath propagation path characterised by the Extended Hata model and Table 58 tabulates the results for the scenario in which there exists a line of sight path (Free Space Path Loss Model) between the BTS and radio microphone.

Table 57: GSM Simulation results for multipath propagation between BTS and the Radio Microphone

| Probability of less than 8 dB SINR occurring under conditions 1 dB desensitization (GSM) (Extended Hata Model for BTS – Radio Mic Propagation Path) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| GSM | MS Location |  |  |  |  |
|  | Outdoor | Indoor |  |  |  |
| Body Loss (dB) | Guard Band | Guard Band | Guard Band | Guard Band | Guard Band |
|  | 0 kHz | 0 kHz | 200 kHz | 400 kHz | 600 kHz |
| 0 | 1.2 % | 16.73 % | 3.37 % | 0.33 % | 0.08 % |
| 8 | 0.29 % | 7.51 % | 1.08 % | 0.09 % | 0 % |
| 16 | 0.04 % | 2.61 % | 0.22 % | 0.01 % | 0 % |

Table 58: GSM Simulation results for line of sight propagation between BTS 
and the Radio Microphone

| Probability of less than 8 dB SINR occurring under conditions 1 dB desensitization (GSM) (Free Space Path Loss Model for BTS – Radio Mic Propagation Path) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| GSM | MS Location |  |  |  |  |
|  | Outdoor | Indoor |  |  |  |
| Body Loss (dB) | Guard Band | Guard Band | Guard Band | Guard Band | Guard Band |
|  | 0 kHz | 0 kHz | 200 kHz | 400 kHz | 600 kHz |
| 0 | 11.95% | 68.04% | 28.64% | 5.10% | 0.84% |
| 8 | 3.77% | 45.57% | 12.05% | 1.19% | 0.07% |
| 16 | 0.92% | 24.57% | 4.09% | 0.14% | 0% |

The relationship between radio microphone carrier frequency and guard band for a 600 kHz bandwidth signal is described by Table 59.

Table 59: Relationship between Radio Microphone Carrier Frequency, Frequency Offset 
and Guard Band for a 600 kHz signal bandwidth

| Radio Microphone Signal Carrier Frequency (MHz) | Carrier Frequency Offset from Upper Band Edge of Band 3 Uplink (kHz) | Guard Band between upper band edge of Band 3 uplink and lower radio microphone signal frequency (kHz) |
| --- | --- | --- |
| 1785.3 | 300 | 0 |
| 1785.5 | 500 | 200 |
| 1785.7 | 700 | 400 |
| 1785.9 | 900 | 600 |

The results of Table 57 and Table 58 are depicted graphically in Figure 9.

Figure 9: Probability of less than 8 dB SINR occurring under conditions of 1dB desensitization (GSM)

The probability of blocking in the case of 0 dB body loss, 0 kHz guard band, BTS and MS outdoor and multipath propagation between the BTS and radio microphone is 1.2 %. When the MS is indoors it can be seen that a probability of blocking equal to 1.2 % can be arranged by ensuring a guard band of at least 350 kHz. The probability of blocking in the case of 0 dB body loss, 0 kHz guard band, BTS and MS outdoor and line of sight propagation between the BTS and radio microphone is 11.95 %. Again when the MS is indoors it can be seen that a probability of blocking equal to 11.95 % can be arranged by ensuring a guard band of at least 375 kHz. Furthermore, in this case, the probability can be reduced to 1.2 % by ensuring a guard band of 600 kHz.

LTE

The criterion of N/(N+I) (i.e. the desensitization of the BS) was used to measure the probability of interference. The results represent the probability of a desensitization of greater than 1 dB being encountered by the BS under the various simulation conditions. A desensitization of 1 dB corresponds to an I/N ratio of -6 dB.

Table 53 and Table 54 contain the simulation results for the LTE scenario with an EUTRA signal bandwidth of 1.4 MHz.

Table 60: LTE Simulation results for multipath propagation between BS and the Radio Microphone and EUTRA signal Bandwidth of 1.4 MHz

| Probability of the occurrence of > 1 dB desensitization for LTE 1.4 MHz (Extended Hata Model for BTS – Radio Mic Propagation Path) |  |  |  |  |
| --- | --- | --- | --- | --- |
| EUTRA (1.4 MHz) |  |  |  |  |
| Body Loss (dB) | Guard Band | Guard Band | Guard Band | Guard Band |
|  | 0 kHz | 200 kHz | 400 kHz | 600 kHz |
| 0 | 88.13 | 46.32 | 10.46 | 4.96 |
| 8 | 68.11 | 19.21 | 2.07 | 0.82 |
| 16 | 39.35 | 4.58 | 0.32 | 0.14 |

Table 61: LTE Simulation results for line of sight propagation between BS and the Radio Microphone and EUTRA signal Bandwidth of 1.4 MHz

| Probability of the occurrence of > 1 dB desensitization for LTE 1.4 MHz (Free Space Path Loss Model for BTS – Radio Mic Propagation Path) |  |  |  |  |
| --- | --- | --- | --- | --- |
| EUTRA (1.4 MHz) |  |  |  |  |
| Body Loss (dB) | Guard Band | Guard Band | Guard Band | Guard Band |
|  | 0 kHz | 200 kHz | 400 kHz | 600 kHz |
| 0 | 100% | 100% | 98.97% | 80.03% |
| 8 | 100% | 100% | 54.03% | 31.46% |
| 16 | 100% | 83.21% | 4.4% | 0% |

Table 61 tabulates the results for the scenario in which the path between the BS and radio microphone is a multipath propagation path characterised by the Extended Hata model and Table 62 tabulates the results for the scenario in which there exists a line of sight path (Free Space Path Loss Model) between the BS and radio microphone.

The results of Table 60 and Table 61 are depicted graphically in Figure 10.

Figure 10: Probability of the occurrence of > 1 dB desensitization for LTE 1.4 MHz

Figure 10 shows that the probability of greater than 1 dB desensitization occurring for a 1.4 MHz bandwidth EUTRA signal can be reduced to the 1 % probability threshold by ensuring a guard band of 550 kHz. The exceptions to this can be seen to be the case of 0 dB body loss and multipath propagation between the BS and the radio microphone and the cases of 0 dB and 8 dB body loss for line of sight propagation between the BS and the radio microphone. As can be seen from the figure, these cases would require a larger guard band.

Table 63 and Table 64 contain the simulation results for the LTE scenario with an EUTRA signal bandwidth of 5 MHz.

Table 62: LTE Simulation results for multipath propagation between BS and the Radio Microphone and EUTRA signal Bandwidth of 5 MHz

| Probability of the occurrence of > 1 dB desensitization for LTE 5 MHz (Extended Hata Model for BTS – Radio Mic Propagation Path) |  |  |  |  |
| --- | --- | --- | --- | --- |
| EUTRA (5 MHz) |  |  |  |  |
| Body Loss (dB) | Guard Band | Guard Band | Guard Band | Guard Band |
|  | 0 kHz | 200 kHz | 400 kHz | 600 kHz |
| 0 | 73.93 | 25.27 | 4.48 | 3.06 |
| 8 | 46.29 | 7.74 | 0.96 | 0.53 |
| 16 | 19.81 | 1.82 | 0.15 | 0.09 |

Table 63: LTE Simulation results for line of sight propagation between BS and the Radio Microphone and EUTRA signal Bandwidth of 5 MHz

| Probability of the occurrence of > 1 dB desensitization for LTE 5 MHz (Free Space Path Loss Model for BTS – Radio Mic Propagation Path) |  |  |  |  |
| --- | --- | --- | --- | --- |
| EUTRA (5 MHz) |  |  |  |  |
| Body Loss (dB) | Guard Band | Guard Band | Guard Band | Guard Band |
|  | 0 kHz | 200 kHz | 400 kHz | 600 kHz |
| 0 | 100% | 100% | 77.23% | 62.58% |
| 8 | 100% | 94.5% | 29.43% | 15.35% |
| 16 | 100% | 44.86% | 0% | 0% |

Table 62 tabulates the results for the scenario in which the path between the BS and radio microphone is a multipath propagation path characterised by the Extended Hata model and Table 63 tabulates the results for the scenario in which there exists a line of sight path (Free Space Path Loss Model) between the BS and radio microphone.

The results of Table 62 and Table 63 are depicted graphically in Figure 11

Figure 11: Probability of the occurrence of > 1 dB desensitization for LTE 5 MHz

Figure 11 also shows that the probability of greater than 1 dB desensitization occurring for a 5 MHz bandwidth EUTRA signal can be reduced to the 1% probability threshold by ensuring a guard band of 400 kHz. Again the exceptions to this can be seen to be the case of 0 dB body loss and multipath propagation between the BS and the radio microphone and the cases of 0 dB and 8 dB body loss for line of sight propagation between the BS and the radio microphone. As can be seen from the figure, these cases would require a larger guard band.

Conclusion

The results for GSM indicate that a 600 kHz guard band is sufficient to ensure that, irrespective of the propagation mechanism between the Radio Microphone and the BTS and body loss, the situation in which the BTS is outdoors and the MS is indoors is equivalent to the case in which both BTS and MS are outdoors. In the scenario considered this equates to a probability of blocking of below 1.2 % when considering a required SINR of 8 dB and assuming a desensitization of 1 dB.

For the cases simulated, the results for LTE also indicate that a 600 kHz guard band is required to reduce the probability of the BS experiencing a desensitization of > 1 dB to below 1 %. This assumes that an I/N ratio of -6 dB is an adequate value for an LTE system operating in a dense urban environment. The exceptions to this are for the case of 0 dB body loss and multipath propagation between the BS and the radio microphone and the cases of 0 dB and 8 dB body loss for line of sight propagation between the BS and the radio microphone. These cases would require a larger guard band.

The simulation scenario considered has focused on the analysis of a real PMSE venue within a densely populated urban environment. As such it is considered that this system will be subject to noise from UEs and BS associated with the same system as the BS under analysis. Under these circumstances a higher I/N can be tolerated than that which can be tolerated in a rural implementation. In a rural implementation, due to the lack of other neighbouring interferers, the effect of an interferer will be more predominant as the system will be required to operate with more stringent requirements on I/N. For example, an I/N of -10 dB may be a more appropriate threshold. In this case, in order to avoid exceeding a lower desensitization threshold, either a larger guard band or greater separation distance would be required between the BS and the radio microphone.

simulation details

simulation parameters

The frequency allocation for the BTS, the radio mic and the MS are shown in Figure 12

Victim Receiver (Vr)

BTS RX

1710-1785MHz

iRSS

Interfering Transmitter (It)

Radio Mic TX

1785-1805MHz

Wanted Transmitter (Wt)

MS TX

1710-1785MHz

Figure 12: Frequency allocations in the simulations

The following parameter set-ups were used in the simulations:

Table 64: GSM1800 BTS (Vr) Parameters for SEAMCAT

| Parameter | Units | BTS (Vr) |
| --- | --- | --- |
| Frequency (ARFCN 885) | MHz | 1784.8 |
| Receiver bandwidth | kHz | 200 |
| Reference System noise figure (taken from [4]) | dB | 4 |
| Reference Noise level (taken from [4]) | dBm/channel | -117 |
| Reference Receiver Sensitivity (taken [4]) | dBm/channel | -108 |
| Antenna Height | m | 20.5 |
| Maximum Antenna Gain | dBi | 18 |

Table 65: GSM1800 MS (Wt) Parameters for SEAMCAT

| Parameter | Units | MS (Wt) |
| --- | --- | --- |
| Frequency (ARFCN 885) | kHz | 1784.8 |
| Tx Power | dBm/channel | 30 |
| Antenna Height | m | 1.5 |
| Maximum Antenna Gain | dBi | 0 (omni assumed) |

Table 66: LTE1800 BS (Vr) Parameters for SEAMCAT

| Parameter | Units | BTS (Vr) |  |
| --- | --- | --- | --- |
| Frequency | MHz | 1784.2 | 1782.4 |
| Receiver bandwidth | MHz | 1.4 | 5 |
| Noise Bandwidth | MHz | 1.08 | 4.5 |
| Reference System noise figure (taken from [4]) | dB | 5 | 5 |
| Reference Noise level | dBm | -108.55 | -102.35 |
| Antenna Height | m | 20.5 | 20.5 |
| Maximum Antenna Gain | dBi | 18 | 18 |

Table 67: Radio Mic (It) Parameters for SEAMCAT

| Parameter | Value |
| --- | --- |
| Maximum power | 20 mW (13 dBm) Hand Held |
| Frequency | 1785 – 1805 MHz |
| It Simulation Frequency | 1785.3 MHz |
| Antenna Height | 1.5 m |
| Bandwidth | 600 kHz |
| Body Loss | 0 dB, (radio mic on stand) 8 dB 16 dB |
| Maximum Antenna Gain | -6 dBi (omni assumed) |

Simulation assumptions

The BTS (Vr) antenna pattern was assumed to be as defined in [23].

Scenario 1 propagation models (Multipath propagation path between BTS and Radio Mic)

BTS and Radio Mic outdoor

Model:	Extended Hata

General Environment:	Urban

Local Environment (receiver):	Outdoor

Local Environment (transmitter):	Outdoor

Propagation Environment:	Above Roof (due to the height of the BTS transmitter)

BTS and MS outdoor

Model:	Extended Hata

General Environment:	Urban

Local Environment (receiver):	Outdoor

Local Environment (transmitter):	Outdoor

Propagation Environment:	Above Roof (due to the height of the BTS transmitter)

Scenario 1 propagation models (Line of sight propagation path between BTS and Radio Mic)

BTS and Radio Mic outdoor

Model:	Free Space Path Loss

Variation Std. Dev.:	1 dB

BTS and MS outdoor

Model:	Extended Hata

General Environment:	Urban

Local Environment (receiver):	Outdoor

Local Environment (transmitter):	Outdoor

Propagation Environment:	Above Roof (due to the height of the BTS transmitter)

Scenario 2 propagation models (Multipath propagation path between BTS and Radio Mic)

BTS and Radio Mic outdoor

Model:	Extended Hata

General Environment:	Urban

Local Environment (receiver):	Outdoor

Local Environment (transmitter): 	Outdoor

Propagation Environment: 	Above Roof (due to the height of the BTS transmitter)

BTS and MS indoor

Model:	Extended Hata

General Environment:	Urban

Local Environment (receiver):	Outdoor

Local Environment (transmitter):	Indoor

Propagation Environment:	Above Roof (due to the height of the BTS transmitter)

Scenario 2 propagation models (Line of sight propagation path between BTS and Radio Mic)

BTS and Radio Mic outdoor

Model:	Free Space Path Loss

Variation Std. Dev.:	1 dB

BTS and MS indoor

Model:	Extended Hata

General Environment:	Urban

Local Environment (receiver):	Outdoor

Local Environment (transmitter):	Indoor

Propagation Environment:	Above Roof (due to the height of the BTS transmitter)

All parameters relating to indoor propagation were allowed to default to their default settings in SEAMCAT

Scenario 3 propagation models (Multipath propagation path between BS and Radio Mic)

BS and Radio Mic outdoor

Model:	Extended Hata

General Environment:	Urban

Local Environment (receiver):	Outdoor

Local Environment (transmitter):	Outdoor

Propagation Environment:	Above Roof (due to the height of the BS transmitter)

Scenario 3 propagation models (Line of sight propagation path between BS and Radio Mic)

BS and Radio Mic outdoor

Model:	Free Space Path Loss

Variation Std. Dev.:	1 dB

Radio microphone emission mask

The ETSI standard EN 300 422 [6] describes the PMSE operation above 1 GHz for three different channel bandwidths (200, 400 and 600 kHz). The relevant spectrum masks are still in discussion, but the 600 kHz channel bandwidth emission mask was used in these simulations, see Figure 13. The center frequency for the Radio mic channel was chosen to be 1785.3 MHz

Figure 13: Emission mask with 600 kHz channel bandwidth

List of references

ERC Recommendation 70-03: “Relating to the use of short range devices (SRD)”, Version of 22 August 2011

ERC Recommendation 74-01: “Unwanted emissions in the spurious domain”, Version of 2011

CEPT Report 30: ”The identification of common and minimal (least restrictive) technical conditions for 790-832 MHz for the digital dividend in the European Union“, ECC, 30 October 2009

ERC Report 42: “Handbook on radio equipment and systems radio microphones and simple wide band audio links”, Version of October 1996

ECC Report 131: “DERIVATION OF A BLOCK EDGE MASK (BEM) FOR TERMINAL STATIONS IN THE 2.6 GHz FREQUENCY BAND (2500-2690 MHz)”, Version of January  2009

ETSI EN 300 422: ”Wireless microphones in the 25 MHz to 3 GHz frequency range”

ETSI TS 136 101: “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

ETSI TS 136 104: “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

ETSI EN 301 357: “Electromagnetic compatibility and Radio spectrum Matters (ERM); Technical characteristics and test methods for analogue cordless wideband audio devices using integral antennas operating in the CEPT recommended 863 MHz to 865 MHz frequency range”

ETSI TR 102 546: “Electromagnetic compatibility and Radio spectrum Matters (ERM); Technical characteristics for Professional Wireless Microphone Systems (PWMS); System Reference Document”

ETSI TS 137 104: “Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception”

CEPT Report 40: "Compatibility study for LTE and WiMAX operating within the bands 880-915 MHz / 925-960 MHz and 1710-1785 MHz / 1805-1880 MHz (900/1800 MHz bands)", ECC, 12 November 2010

3GPP TS 36.101 v10.4.0: "E-UTRA User Equipment (UE) radio transmission and reception"

3GPP TS 36.104 v10.4.0: "E-UTRA Base Station (BS) radio transmission and reception"

3GPP TS 25.101 v10.3.0: "UTRA User Equipment (UE) radio transmission and reception (FDD)"

3GPP TS 45.005 v10.2.0: "GSM/EDGE Radio transmission and reception"

3GPP TR 25.816 v8.0.0: "UMTS 900 MHz Work Item Technical Report"

3GPP TR 25.885 v1.0.0: "UMTS1800/1900 Work Items Technical Report"

3GPP TR 36.942 v10.2.0: "E-UTRA Radio Frequency (RF) system scenarios"

ETSI EN 301 840: "Digital radio microphones operating in the CEPT Harmonized band 1785 MHz to 1800 MHz"

ITU-R P.1411-5: "Propagation data and prediction methods for the planning of short-range outdoor radiocommunication systems and radio local area networks in the frequency range 300 MHz to 100 GHz"

Extended Hata – SRD model, http://tractool.seamcat.org/raw-attachment/wiki/Manual/PropagationModels/ExtendedHata/Hata-and-Hata-SRD-implementation_v2.pdf

ITU-R F.1336-2 “Reference radiation patterns of omnidirectional, sectoral and other antennas in point-to-multipoint systems for use in sharing studies in the frequency range from 1 GHz to about 70 GHz”

ERC Report 25: "The European table of frequency allocations and utilisations in the frequency range 9 kHz to 3000 GHz", amended Lille 2011.

ECC Report 82: "Compatibility study for UMTS operating within the GSM 900 and GSM 1800 frequency bands"

In this case BEM is taken to be specific to PMSE applications under study.

k = Boltzmann constant; T = 290 K; BW = Bandwidth; NF = Noise figure

See ECC Report 131 Annex 2 [5]

These ACLR values are obtained from the PMSE spectrum mask in SEAMCAT

It should be noted that the standard refers to C/Ia which is an equation in the linear domain, but specifies the value in dB, i.e. in the logarithmic domain.