Coexistence between the radionavigation-satellite and the amateur services in the frequency range 1 240 - 1 300 MHz

ECC Report <RNSS_AS>

[approved DD Month YYYY]

# Executive summary

The executive summary will be dealt with when the report will be finalized

The frequency range 1 240-1 2600 MHz, used by GLONASS systems of the Russian Federation and frequency range 1260-1300 used by the the European GALILEO system as well as by the Chinese Beidou and the Japanese QZSS (and planned to be used by the Korean KPS)for the global provisioning of  radionavigation-satellite service (RNSS), occupies the frequency band 1 240-1 300 MHz which is also allocated to the amateur and amateur-satellite services on a secondary basis in the ITU Radio Regulations. Further, this band is shared with primary allocations to the radiolocation (RLS), radionavigation (RNS) on a co-primary basis and with the Earth exploration-satellite service (EESS (active)) on a co-secondary basis.

With the implementation of the new RNSS system applications or services, and considering the fact that RNSS signals are characterised by uubiquitous, wideband, very low received power and continuous emissions, it has become necessary to clarify more specifically the operating conditions for certain applications (operating modes) in the amateur and amateur satellite services to ensure the continued use of this band and to ensure an appropriate long-term development of both services.

The allocation in the frequency band 1 240-1 300 MHz to the amateur service is important to the radio amateurs worldwide. The allocation closes the gap between the 'neighbouring' allocations 430-440 MHz and 2 300-2 450 MHz to the radio amateur service, as the physical conditions are very specific for propagation monitoring and communication experiments.

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	7

2	AMATEUR SERVICES	8

2.1	Amateur services in 1 240 – 1 300 MHz	8

2.1.1	Introduction	8

2.1.2	Characteristics of amateur signal emissions	8

2.1.3	IARU band plan	9

2.1.4	Amateur Station Categorisation	11

2.1.4.1	2.2.4.1 Home Station	12

2.1.4.2	2.2.4.2 Temporary ”Portable” Station	13

2.1.4.3	Permanent Installation	13

2.1.5	Typical Amateur Station Type Characterisitcs	14

2.1.5.1	Home Station	14

2.1.5.2	Temporary ”Portable” Station	15

2.1.5.3	Permanent Installation	15

2.1.6	Typical Usage Patterns	15

3	Scenarios for coexistence evaluation	18

3.1	Frequency overlap betweeN, RNSS and amateurs	18

3.2	SELECTED REPRESENTATIVE SCENARIOS	18

3.2.1	Scenario 1:	18

3.2.2	Scenario 2:	19

3.3	Propagation model:   [For Further Discussion – This needs to account for path terrain and clutter at the receiver end]	19

3.3.1	Time variability effects	20

3.3.2	Space variability effects	20

4	RNSS systems	23

4.1	typical RNSS applications in the 1240-1300 MHz BAND	23

4.1.1	High Precision applications	23

4.1.1.1	Government: Transportation and environmental management	23

4.1.1.2	Agriculture	23

4.1.1.3	Construction – Heavy and civil engineering	24

4.1.1.4	Automotive navigation	24

4.1.1.5	-Surveying and mapping	24

4.1.1.6	Utilities, energy, mining, oil and natural gas	24

4.1.2	Authentication applications	24

4.1.2.1	Maritime	25

4.1.2.2	Road	25

4.1.2.3	Timing	25

4.2	Galileo Emissions and Protection requirements	27

4.2.1	Galileo  system description	27

4.2.2	GALILEO E6 Signal Baseline	27

4.2.2.1	High Accuracy Service (HAS)	28

4.2.2.2	Commercial Authentication Service (CAS)	29

4.2.3	GALILEO Protection requirements	29

4.3	GLONASS EMISSIONS and protection requirements	31

4.3.1	GLONASS  system description	31

4.3.1.1	GLONASS Signal description	31

4.3.1.2	Signals with frequency division multiple access	31

4.3.2	GLONASS Protection requirements	33

4.4	OTHER RNSS	33

5	Methodology	34

6	Simulations	36

7	Measurements	37

8	Quantification of interference and options for RFI mitigation	38

8.1	Analysis of protection criteria	38

8.2	[RFI mitigation on GALILEO E6-B receiver]	38

8.3	Mitigation in amateur emissions	38

8.4	Statistics of use of band by amateur services (time and location)	38

9	Conclusions	39

ANNEX 1: List of references	40

ANNEX 2: AMSS regulations and deployment in Switzerland	42

ANNEX 3: Parameters of unmanned Amateur Radio Stations in Germany, Frequency Range 1240 – 1300 MHz	45

ANNEX 4: A survey of the number of active (transmitting) amateur stations using the band 1240 – 1300 MHz at the busiest times in some countries within the CEPT region	50

ANNEX 5: Amateur radio repeater and beacon stations using the band 1240 – 1300 MHz  in some countries within the CEPT region – May 2020	55

9.1	Amateur radio Repeater and Beacon Stations	55

9.2	licensing and assignment procedures	55

9.3	VOICE AND DATA REPEATERS	55

9.3.1	Repeater frequencies in 1240-1300MHz	55

9.3.2	Data Repeater frequencies	56

9.3.3	Voice and Data Repeater assignments – a survey of some CEPT countries	56

9.4	Amateur TV (ATV) repeaters	57

9.4.1	ATV Repeater frequencies in 1240-1300MHz	57

9.4.2	Repeater Architecture	57

9.4.3	Repeater Trends	58

9.4.4	Repeater Assignments – a survey of some CEPT countries	58

9.5	Propagation beacons	59

9.5.1	Propagation Beacon Frequencies in 1240-1300 MHz	59

9.6	References (move to reference section)	59

ANNEX 6: Measurements made in Germany	62

9.6.1.1	SSB voice (Test Cases G1-07, G1-08, G1-13)	67

LIST OF ABBREVIATIONS

| Abbreviation | Explanation (style: ECC Table Header red font) |
| --- | --- |
| ARNS | Aeronautical Radionavigation Service (ITU RR) |
| ATV | Amateur television (usually analogue FM emissions) |
| BOC | Binary Offset Carrier |
| BPS | Bits per second |
| BPSK | Binary phase shift keying |
| CAS | Commercial Authentication Service (GALILEO E6) |
| CBOC | Composite Binary Offset Carrier modulation |
| CDMA | Code Division Multiple Access |
| C/NAV | Commercial Navigation message (provided in E6-B signal) |
| cps | Chips per second |
| CRC | Cyclic Redundancy Check |
| CW | Continuous wave (in amateur service also used for Morse telegraphy communication |
| DATV | Digital Amateur TV (applying DVB-S and DVB-S2 Standards) |
| ECC | Electronic Communications Committee |
| FEC | Forward Error correction |
| FSK | Frequency Shift Keying |
| HAS | High-Accuracy Service (GALILEO E6) |
| IARU-R1 | International Amateur Radio Union (Region 1); |
| ICD | European Union, “Galileo Open Services Signal in Space Interface Control Document (OS SIS ICD)”; http://ec.europa.eu/growth/sectors/space/galileo/ |
| ISU | Interference Suppression Unit |
| ITU-R | ITU Radiocommunications Sector |
| GNSS | Global Navigation Satellite Service |
| OS | Open Service (GALILEO) |
| PSA | Precision Step Attenuator |
| PSD | Power Spectral Density |
| RFI | Radio Frequency interference |
| RNSS | Radionavigation-Satellite Service (ITU RR) |
| RR | ITU-R Radio Regulations |
| SIS | Signal-In-Space |
| MGM sps | Machine Generated Mode Symbols per second |

# Introduction

This section will be dealt with when the report will be finalised

The frequency range 1 240-1 260 MHz, used by the Russian Federation GLONASS system and the frequency range  1 260-1 300 MHz, used by the European GALILEO system as well as by the Chinese Beidou and the Japanese QZSS (and planned to be used by the Korean KPS) for the provisioning of radio navigation -satellite service (RNSS), occupies the frequency band 1 240- 1 300 MHz which is also allocated  to the amateur and amateur-satellite services on a secondary basis in the ITU Radio Regulations. This band is further shared with primary allocations to the radiolocation (RLS), radionavigation (RNS) on a co-primary basis and with the Earth exploration-satellite service (EESS (active)) a co-secondary basis.

With the implementation of the GALILEO system, it has become necessary to clarify more specific the operating conditions for certain applications (operating modes) in the amateur and amateur satellite services to ensure the continued use of this band and to ensure an appropriate long-term development of both services.

For radio amateurs worldwide the allocation to the amateur service in the frequency band 1 240-1 300 MHz, also known as the “23cm-band”, is an important frequency band between the allocations in 430-440 MHz and 2 300-2 450 MHz as the physical conditions are very specific for propagation monitoring and communication experiments.

This report analyses conditions for the coexistence of the RNSS receiver processing the signals with emissions from stations operating in the amateur and amateur-satellite services allocated in the frequency range 1 240-1 300 MHz. It is based on two measurement campaigns (2, 3).

# AMATEUR SERVICES

## Amateur services in 1 240 – 1 300 MHz

### Introduction

The ITU Radio Regulations allocate the frequency band 1 215 – 1 300 MHz to the amateur service and the amateur-satellite service with a secondary status. ITU RR no. 1.56 define the amateur service as: “A radiocommunication service for the purpose of self-training, intercommunication and technical investigations carried out by amateurs, that is, by duly authorized persons interested in radio technique solely with a personal aim and without pecuniary interest.” According to this, the amateur service incorporates a large portfolio of different applications in terms of signals and usage scenarios.

### Characteristics of amateur signal emissions

The characteristics of typical applications are shown in Table 2.

Table 2: Amateur service applications  and usage scenarios in the frequency range 1 240-1 300 MHz

| Application | Typical Bandwidth | Details | Remarks |
| --- | --- | --- | --- |
| Digital Voice | 12,5 kHz (Tetra: 25 kHz) | DMR, D-Star DV, NXDN, Tetra, APCO 25, C4FM | simplex and repeaters (interconnected by the Internet), mobile/ handheld/ stationary usage DMR: TDMA Access (two timeslots – 50% duty cycle) |
| Digital Data | 12,5 – 150 kHz | Packet Radio (AFSK 1k2, FSK 9k6), D-Star, Digital Data 128 kbit/s | signal bursts, automatic stations, mobile/ stationary usage |
| Morse Code | 500 Hz | CW (100 WPM) | Moon bounce (high power), Beacons (24/7 automatic stations), Contests (Activity weekends), stationary usage |
| Analogue Wide | 12,5 kHz | FM | simplex and repeaters (some interconnected by the Internet – Echolink), mobile/handheld/stationary usage |
| Analogue Narrow | 2 700 Hz | SSB | simplex and linear transponders, Contests (Activity weekends), stationary usage |
| MGM (machine generated mode) | 6 – 2 700 Hz | RTTY, SSTV, PSK31, WSPR | simplex, operator controlled (no automatic stations), Contests (Activity weekends), satellites (only CO-65 in space), stationary usage |
| Analogue ATV | 16 – 18 MHz | FM-ATV | simplex and repeaters, stationary usage |
| Digital ATV | 2 – 8 MHz | DATV (DVB-T, DVB-S) | simplex and repeaters, stationary usage |

### IARU band plan

As an affair of the worldwide amateur service's self-administration, the International Amateur Radio Union (IARU) coordinates the interests of its Member Organisations. The three IARU Regions are organized to broadly mirror the structure of the ITU and its related regional telecommunications organizations. In any case, national regulatory provisions prevail and may lead to different regulations.

Currently IARU recommends the band plan shown in Table 3 below for the amateur allocation in the frequency range 1 240 – 1 300 MHz in IARU Region 1.

The usage of the frequency range by the amateur and amateur satellite services is driven by the varied operational and experimental interests of the radio amateur users themselves. To support this the regional band plan is developed to maintain order, avoid conflict and interference between applications, provide understanding of the frequencies for specific activities and form a basis for intra and inter service coordination when required. The regional band plan is not mandatory but is strongly recommended for adoption by the individual national societies.  In some cases it may be adopted to some extent in national regulations and it may be adjusted on a national basis to facilitate national coordination and sharing with other services in the band. Respecting the band plan is common practice and necessary to facilitate successful radio contacts especially between countries and for inter-regional communications.

The band plan is reviewed periodically and may be adjusted to reflect new technologies and evolving applications. External influences driven by the requirements to share with other services can also be taken into account. The band plan is published by the IARU in the periodically updated IARU R1 VHF Handbook.

Table 3: IARU Region 1 band plan for 1 240 - 1 300 MHz

| Frequency | Max. Bandwidth | Mode | Usage |
| --- | --- | --- | --- |
| 1240 - 1240.5 | 2700 Hz | All Mode | (reserved for future) |
| 1240.5 - 1240.75 | 500 Hz | Telegraphy MGM | Beacons (reserved for future) |
| 1240.75 - 1241 | 20 kHz | FM/Digital voice | (reserved for future) |
| 1241 - 1243.25 | 20 kHz | All Mode | 1240.000-1241.000 Digital communications 1242.025-1242.250 Repeater output, ch. RS1 -- RS10 1242.250-1242.700 Repeater output, ch. RS11 -- RS28 1242.725-1243.250 Packet radio duplex, ch. RS29 -- RS50 |
| 1243.25 - 1260 | Bandwidth limits according to national regulations. | ATV Digital ATV | 1258.150-1259.350 Repeater output, ch. R20 -- R68 |
| 1260 - 1270 | Bandwidth limits according to national regulations. | Satellite Service |  |
| 1270 - 1272 | 20 kHz | All Mode | 1270.025-1270.700 Repeater input, ch. RS1 -- RS28 1270.725-1271.250 Packet Radio duplex, ch. RS29 -- RS50 |
| 1272 - 1290.994 | Bandwidth limits according to national regulations. | ATV Digital ATV |  |
| 1290.994 - 1291.481 | 20 kHz | FM Digital voice Repeater INPUT | RM0 (1291.000) -- RM19 (1291.475) 25 kHz spacing |
| 1291.494 - 1296 | Bandwidth limits according to national regulations. | All Mode | 1293.150-1294.350 Repeater input, R20 (1291.475) - R68 (1294.350) |
| 1296 - 1296.15 | 500 Hz | Telegraphy MGM | 1296.00-1296.025 Moonbounce 1296.138 PSK31 centre of activity |
| 1296.15 - 1296.8 | 2700 Hz | Telegraphy MGM SSB | 1296.200 Narrow-band centre of activity 1296.400-1296.600 Linear transponder input 1296.500 Image center (SSTV, FAX etc) 1296.600 Narrow band data center (MGM, RTTY ...) 1296.600-1296.700 Linear transponder output 1296.741-1296.743 experimental MGM (500 Hz) 1296.750-1296.800 Local Beacon (10W ERP max) |
| 1296.8 - 1296.994 | 500 Hz | Telegraphy MGM | BEACONS EXCLUSIVE (b) |
| 1296.994 - 1297.481 | 20 kHz | FM Digital voice Repeater OUTPUT | RM0 (1297.000) -- RM19 (1297.475) 25 kHz spacing |
| 1297.494 - 1297.981 | 20 kHz | FM (c) Digital voice (e) | 1297.500-1297.975 SIMPLEX channels 25 kHz spacing SM20 - SM39 1297.500 FM activity centre 1297.725 Digital Voice Calling 1297.900-1297.975 Simplex FM Internet Voice gateways |
| 1298 - 1299 | 20 kHz | All Mode | General mixed analogue or digital use 25 KHz spacing channels 1298.005 RS1 1298.9755 RS1 |
| 1299 - 1299.75 | 20 kHz | All Mode | Arranged as 5x150 kHz channels for high speed Digital Data (DD) usage: Centers: 1299.075, 1299.225, 1299.375, 1299.525, 1299.675 MHz (+/- 75 kHz) |
| 1299 - 1299.75 | 20 kHz | All Mode | 8x25kHz channels (avaiable for FM/DV use): Centers: 1299.775, 1299.975 |

### Amateur Station Categorisation

There are many applications in the amateur service but the stations and their usage patterns can be broadly categorised into three types:

Home Station.

Temporary “Portable” Station.

Permanent installations (away from an individual’s home address).

Table [x]: Summarising the Amateur and Amateur Satellite Applications against the Station Types

| Application | Station Type |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  | Home | Temporary | Installation |  |  |
|  |  |  | Repeater | Beacon |  |
| Voice (Analogue) SSB | Yes | Yes |  |  | Long distance tropospheric weak signal ops. (incl EME). |
| Voice (Analogue) NBFM | Yes | Yes | Yes |  | Local neighbourhood communications Satellite comms. |
| Voice (Digital) | Yes |  | Yes |  | Local neighbourhood communications |
| Telegraphy | Yes | Yes |  | Yes | Long distance tropospheric weak signal ops. (incl EME). |
| Analogue Television | Yes | Yes | Yes |  | Legacy technology, deployments reducing. |
| Digital Television | Yes | Yes | Yes |  | State of the art technology, deployments increasing. |
| MGM | Yes | Yes |  | Yes | Long distance tropospheric weak signal ops. (incl EME). |
| Data | Yes | Yes (Mobile) | Yes |  | Local neighbourhood communication links. |

The operational details are considered specific to the band 1240-1300 MHz as they are dependent upon the nature of the equipment needed and operator skills to operate in this band as well as the propagation characteristics.

It should also be borne in mind that often nationally specific conditions can lead to variations in the operating pattern in particular the frequencies used by permanent installations.

#### 2.2.4.1 Home Station

This would be installed at the usual station licence holder home location. The majority of home station usage is for narrow band terrestrial ad-hoc communications with other similarly equipped amateur stations. However since the propagation characteristics are more challenging than those in lower UHF and VHF bands the band would not be the first choice for casual contacts or group/club on-air gatherings. Most stations are better equipped in the lower frequency bands for this type of operation. Casual random calls under flat propagation conditions are rare as high antenna gain and narrow beam widths preclude useful ”broadcast” calls. The highest level of home station activity occurs during (usually competitive) scheduled activity periods that take place on a publicised regular basis during weekday evenings and a number of weekends throughout the year. Generally analogue narrow band Morse, SSB and MGM applications are used in the range 1296 – 1298 MHz

Enhanced propagation conditions tend to be variable and can occur randomly throughout the year. They may last from minutes to days depending on the mechanism at work. These can encourage operation although activity levels will be less compared with the more popular lower UHF and VHF bands. Generally analogue narrow band Morse, SSB and MGM applications are used in the range 1296 – 1298 MHz.

In order to extend the communication distance that can be achieved, this band is popular amongst a few stations suitably equipped to overcome the losses inherent in an earth-moon-earth (EME) reflected path. Of course these are only possible when the moon is visible and require high performance low noise stations with larger antenna systems that may not be compatible with all locations. These weak signal contacts most commonly use analogue narrow band Morse and MGM applications in the range 1296 – 1298 MHz. MGM applications are most commonly used for random contacts.

This band is a popular choice for Amateur TV (ATV) applications due to the bandwidth available. Nowadays digital ATV (DATV) is encouraged and becoming the most popular application. As discussed above, random activity is quite rare but again activity (and contest) periods are scheduled as a focus for operation and experimentation. Simplex TV operation tends to occur around 1255 MHz.

Operation through amateur satellites takes place within the uplink only band at 1260-1270 MHz and these can include tele-command activities as well as direct narrow band voice and low rate data communications. There are four currently active in this band at present but this is a lively area of interest.

#### 2.2.4.2 Temporary ”Portable” Station

Often the propagation constraints experienced for a home station (usually due to local clutter) can be overcome in part by temporarily siting a station in an advantageous position (usually high ground)  away from the home location and usually in a rural setting. Again the majority of usage is for terrestrial ad-hoc communications with other amateur stations for short duration narrow band contacts usually associated with scheduled (competitive) activity periods.

ATV activity is also possible, although random activity is rarely seen outside scheduled activity (and contest) periods which act as a focus for operation and experimentation.

EME activity is unlikely as there is no advantage to be gained.

#### Permanent Installation

Permanent installations include specific voice repeaters, ATV (and occasionally data) repeaters and propagation beacons. As permanent stations, these are licensed in their own right for their specific location, operating frequency and output power (as ERP). The licence is usually associated with a licensed ”keeper” of the installation. Propagation beacons usually operate 24/7 and will typically emit a narrowband FSK signal with call sign ID and location information in the range 1296.8 – 1296.994 MHz.

Voice repeaters usually re-transmit narrow band analogue and digital voice traffic when activated with a signal on the input frequency and are mostly associated with extending mobile user coverage. Propagation at these frequencies does not lend itself to reliable wide area repeater coverage so activity is far less than in lower UHF and VHF bands (and fewer commercial radios are suitable for mobile installations). The most common installations transmit around 1297 - 1298 MHz although a few experimental systems may operate in other parts of the band.

Data and TV repeater stations transmit wider bandwidth amateur signals and often transmit test signals when not being accessed by a user station on the input channel. This band is the most popular for amateur TV repeaters which tend to operate with input and output frequencies in the range 1242 – 1260 MHz. Actual assignments can be nationally dependant. There are cases where alternative output frequencies are used to facilitate national inter-service coordination (e.g. UK TV repeater output frequencies are in the range 1300 – 1325 MHz).

### Typical Amateur Station Type Characterisitcs

There is no standard amateur station. The following antenna types and power levels are typical based on published information about the activity periods and operating contests. In general home and temporary stations would use highly directional antennas.

#### Home Station

Most Home Stations will use a single directional beam antenna, however in a few cases multiple beam antennas can be combined to increase the array gain. This is more usual for EME operators for whom high antenna gain is essential for overcoming the high path and reflection loss. A higher performance EME station might use a medium size dish antenna.

| Antenna Type | Gain Typical | 3dB Beam width |
| --- | --- | --- |
| Single Yagi beam (23 to 55 element) | 18 to 21 dB | 18 to 10 deg |
| Multiple Yagi (for EME) Dish antenna (for EME) | 21 dB (4m) 32 dB | 10 deg 4 deg |

Analysis of a typical activity period results (non EME) shows that around 15% of active stations used a multi-antenna array.

Analysis of the same activity period showed the following spread of transmitter power levels (NB: 100% = 34 stations only):

| Power Range (Watts) | % Stations |
| --- | --- |
| Up to 10 | 47% |
| 11-25 | 9% |
| 26 - 100 | 26% |
| 101 - 300 | 12% |
| Over 300 | 6% |

For EME operation experiences have shown that a minimum performance station could expect to make MGM based contacts using around 50W of power into a multiple antenna beam array. Higher performance stations are likely to require at least around 10dB more EIRP through a combination of power level and increased antenna gain.

#### Temporary ”Portable” Station

As with Home Stations there is a spread of the same antenna types that might be used.  Similarly analysis of the same activity period as above show that only 15% of stations used multi-antenna arrays. (The actual number was 3).

Analysis of the same activity period shows the following spread of transmitter power levels (NB: 100% = 13 stations only):

| Power Range (Watts) | % Stations |
| --- | --- |
| Up to 10 | 61.5% |
| 11-25 | 7.5% |
| 26 - 100 | 7.5% |
| 101 - 300 | 15% |
| Over 300 | 7.5% |

#### Permanent Installation

Most permanent installations (beacons and repeaters) are less directional and are generally intended to provide coverage over an area. They are usually licensed to operate at a specific ERP.

| Antenna Type | Gain Typical | 3dB Beamwidth |
| --- | --- | --- |
| Various (e.g. Alford slot, Colinear array, horn, flat panel, big wheel...) | Up to around 13 dB | Omni to 60 deg |

| ERP Range (Watts) | % NB Beacons | % Repeaters (ERP) |
| --- | --- | --- |
| Up to 10 | 69% | 16% |
| 11-25 | 8% | 76% |
| 26 - 100 | 20% | 8% |
| 101 - 300 | 1% | 0% |
| Over 300 | 1% | 0% |

### Representative antenna heights

The following antenna heights are representative of typical amateur station installations.

–	Typical antenna height for a home station = 12 m above ground level.

–	Typical antenna height for a temporary station = 3 m to 15 m above ground level.

–	Typical height for a permanent installation station = 25 m above ground level.

Permanent installation stations are often installed at an advantageous location so as to take advantage of elevated local terrain or tall structures in order to increase the effective antenna height.

### Typical Usage Patterns

For all home and temporary “portable” station applications, narrow-band or wideband, the highest number of actively transmitting amateur stations can be found during the scheduled operating and radiosport contest periods. Table 4 summarises the total scheduled operating and contest periods scheduled in one region for a typical year. As these activities are usually formalised in the amateur operator calendars, the published national results can be consulted to determine the number of transmitting stations that were active during any one activity or contest period.

Table 4: Scheduled operating periods and active operating station numbers

| Usage type | Annual scheduled operating periods | Total active stations per scheduled operating period | Active temporary stations per scheduled operating period |
| --- | --- | --- | --- |
| Narrow-band activity period and radiosport | Total, on average 108 hours over a year | From 9 to 140 maximum depending on the country reviewed. | 15 to 20 maximum depending on the country reviewed. |
| EME activity | 5 × 24-hour contest periods | Up to 10 maximum depending on the country reviewed. (Maximum < 70 across the European area) | None |
| Wideband (typically ATV) activity period and radiosport | Total, on average 120 hours over a year | From 1 to 24 maximum depending on the country reviewed. (Maximum < 100 across the European area) | 10 maximum depending on the country reviewed. |

Permanent installation stations present a different scenario when considering the operational time. Unmanned amateur radio stations are more or less in continuous operation, while manned stations only transmit intermittently. Propagation beacon and repeater station directories from a representative region can be consulted to develop the summary presented in Table 5.

Table 5: Permanent Installation station operating periods in a typical year

| Usage type | Annual operation | Active installations |
| --- | --- | --- |
| Narrow-band propagation beacons | Transmitting continuously usually. | From 4 to 20 depending on the country reviewed. Region 1 = 88 in total. |
| Narrow-band repeaters | Low and only when activated on the input frequency by a user station. May transmit more regularly if a beacon mode is present. | From 9 to 19 depending on the country reviewed. |
| ATV repeaters (the users are usually home stations) | Low and only when activated on the input frequency by a user station in a random and sporadic manner. May transmit more regularly if a beacon mode is present. | From 10 to 18 depending on the country reviewed. 5 to 10 users within the local coverage area transmitting one at a time. |

### Activity factors of amateur transmitting stations in the 1 240-1 300 MHz band

Activity factor considers the amount of time that any particular station is transmitting during any operational period of activity. All applications involve two-way communication requiring periods of reception as well as transmission. It is usual practice for any home station or temporary portable station to spend more time receiving than transmitting.

Maximum Activity Factor for home station and temporary “portable” stations = 50% and typically less.

Any permanent installation station operating in a beacon mode will exhibit a 100% activity factor.

### User density of amateur transmitting stations in the 1 240-1 300 MHz band

1)	Home station and temporary “portable” station

–	For narrow-band activity periods the maximum density of transmitting stations = 0.0002 stations/km2.

–	For wideband activity periods the maximum density of transmitting stations = 0.0001 stations/km2.

–	For EME operations the maximum density of transmitting stations = 0.000 013 stations/km2.

Recognising that not all active stations may submit a record of their activities, a 33% uplift has been added to the total active stations per scheduled operating period from Table 7.

2)	Permanent installation

–	For narrow-band data and voice repeaters the average density of transmitting stations = 0.000 3 stations/km2.

–	For wideband ATV repeaters, the average density of transmitting stations = 0.000 1 stations/km2.

–	For propagation radio beacon stations, the average density of transmitting stations = 0.000 1 stations/km2.

# Scenarios for coexistence evaluation

This chapter should go after the chapter on RNSS

The list of scenarios may not be exhaustive

## Frequency overlap betweeN, RNSS and amateurs

The text and the figure should be updated adding the other systems as well

Figure 3 shows the spread GALILEO E6-signal and the corresponding portions with applications of the amateur service. Except the very wide-band modes of FM-ATV and various options for D-ATV (DVB-T/DVB-S) the remainder of applications falls into three categories which may show different grades of impact, if at all, on the reception and decoding of the GALILEO signals.

## SELECTED REPRESENTATIVE SCENARIOS

Amateur Transmitting Parameters for coexistence evaluation

Two scenarios are proposed to cover the majority of expected amateur and amateur satellite applications in the 1240-1300MHz band.

### Scenario 1:

Directional antenna radiating away from a home or temporary station site, the following parameters are considered.

These parameters can be considered to cover the home and temporary “portable” station situations with an amateur operator using narrow band telegraphy and telephony applications including MGM modes. Satellite uplink operation and EME operation can be taken into account by considering antenna off axis gain. Wideband modes such as DATV are included within this scenario. Nominal values are highlighted and a range of reasonable values can be considered if sensitivity analysis is needed.

### Scenario 2:

Omni-Directional antenna radiating away from a permanent station site.

These parameters cover permanent installations such as beacon and repeater station outputs (narrow band and wide band) using antennas intended for area coverage (omni/low gain).

| Parameter | Scenario 1 (directional) |  | Scenario 2 (omni) |  |
| --- | --- | --- | --- | --- |
|  | Nominal | Range | Nominal | Range |
| Tx Power (dBW) | 10 | up to 24 | 10 | up to 13 |
| Antenna Gain (dBi) | 20 | up to 30 | 6 | up to 10 |
| Antenna 3dB beam width (deg) | 18 | - | N/A | - |
| Height over ground (m) | 12 | - | 25 | - |
| Antenna off axis gain (dB) | -20 | - | N/.A |  |

## Propagation model: 		[For Further Discussion – This needs to account for path terrain and clutter at the receiver end]

Other propagation models, suitable for maritime and aeronautical receivers should be added

The goal is to estimate the dimension of interference areas. For doing so, several propagation models can be used; such as ITU-R P.452, ITU-R P.1546 or other models used for mobile radio planning, such as Okumura-Hata. The CEPT Monte-Carlo analysis SEAMCAT tool (See ECC Report 252 Annex 17) includes these models and additionally the Extended Okamura-Hata model.

For each of the models cited above, the following considerations ca be made:

Okumura-Hata is relatively easy to use and it is well known for its general reliability, but it has the limitation that ideally the transmitting station should be at least 30 m above the ground. The model is intended to calculate the median loss over a pixel of terrain at distance d from the transmitter. Because of this, if the model is used to calculate interference area, a margin factor should be inserted in the analysis in order to take into account the spatial variability of the electromagnetic field. If one applies Okumura-Hata, he must be aware of the limitations on its parameters, including a maximum distance of 10 km, a minimum BS antenna height of 30 m. On the other side, since Okumura-Hata is conceived for frequencies up to 1500 MHz, its frequency range suites the case at hand. . Within the ECO SEAMCAT tool this is implemented as “Extended Okumura-Hata” and can typically be used for mobile services and other services working in non-LOS/cluttered environments up to 3 GHz, for link lengths preferably below 40 km.

ITU-R P. 1546 does not have the limitation on antenna height of the TX (it can be as low as 10 m), but it has a limitation on the minimum distance, that should be at least 1 km. This model has the advantage of having built in location and time probabilities, so that it can be directly used for the analysis of interference areas.

On the other side, ITU-R ITU-1812 is more versatile because it does not have significant limitations on the TX antenna height and the minimum distance, and it considers antenna probability. The minimum distance at which it can be used is 250 m.

For the models ITU-R P.1546 and ITU-R 1812 a tested Matlab implementation is available to ITU members.

The ECO hosts a WiKi page at https://wiki.cept.org/display/SH/A17+Propagation+models that provides useful information on the models implemented in the SEAMCAT tool.

A useful comparison of the P.1546 model and the Extended Hata model can be found at https://wiki.cept.org/display/SH/A17.1.3+Using+the+Extended+Hata+vs.+P.1546+models.

- [Make these Wiki references into footnotes]

For reference, the following figure gives the curves of propagation loss, along a path of 100 km, over rural terrain at 1300 MHz, for a transmitting antenna height of 10 meters, a receiving antenna height of 1.5, and a rural terrain. The two curves refer to two location probabilities, namely of 50% and 1%.

Figure 1

If one is interested in the magnitude of interference at very short distances, in the order of magnitude of a few hundreds of meters, more detailed analyses should be conducted, where the case of light of sight and free space propagation is also considered.

### Time variability effects

Due to the variation in the atmospheric conditions and propagation conditions, such as ducting, the interfering signal can show time variability.  These phenomena are taken into account by the models ITU-R. P. 1546 and ITU-R. P. 1812, however, these time effects are mostly relevant over long distances, while at short distances they tend to be negligible.

### Space variability effects

The other aspect to be considered is the space variability of the electromagnetic field. By the way it is conceived, a propagation model such as Okumura-Hata gives the estimated median value of the received power in a given pixel of terrain. Inside this pixel of terrain, you can still have slow fading and fast fading. The effect of local statistical variations f the EM field also needs to be taken into account.

In order to appreciate this fact, consider a pixel of terrain 50x50 m wide. Assume that the maximum tolerable interfering power for the RNSS receiver is PintMAX. In order to declare that the pixel is free from interference it is not sufficient to verify that the interfering received power from the radio amateur station, calculated with the chose propagation model, is equal or below PintMAX. For instance, when its value is exactly equal to PintMAX this means that 50% of the locations inside the pixel will be still be above this value. For this reason, the analysis of interference shall be conducted in such a way that, for a given pixel to be declared interference free, the interfering EM field shall below the reference threshold for, say, X=99% of its locations.

It is therefore necessary to have statistical model of the spatial variability of the EM field for a given pixel. In general, such a variation is composed of a slow variation (shadow fading) and fast variation (fast fading), that it due to multipath effects.

A characterization of the spatial variability of the field strength in various frequency band and for different propagation scenarios (the clutter at the location of the RX plays a fundamental role), is implicitly contained in the curves of ITU-R. 1546 and it is also treated in ITU-R. P1812, but it is excluded by Hata. Should one use Hata, an additional margin for fading should be added, in the same way it is added when Hata is used for mobile radio planning.

If one considers, for instance ITU-R. P.1812, for an outdoor location, at 1.5 meters, in the frequency band 1300 MHz, the standard deviation of spatial variation of the received power (that is assumed to be lognormal) is 5.5 dB for the 1300 MHz (see section 4.8 and Table 6). A value of 5.5 dB and the assumption of a lognormal fading clearly refer to the slow variation of the field, while the fast variation is not considered.

The following table gives a comparison of the propagation models presented above.

| Model | Scope | Limitations | Clutter loss consideration | Advantages | Drawbacks |
| --- | --- | --- | --- | --- | --- |
| ITU-R P.1546 | Point to area | Tx height ∈[10;3000] m Distance ∈[1;1000] m | The clutter heights around the transmitter and the receiver are represented by two parameters | Can be used without specific terrain data. Works with sea paths. Implemented in SEAMCAT. |  |
| ITU-R P.1812 | Point to area | Distance > 250 m | 1) At each point on the profile between the transmitter and receiver point a clutter height is added to the terrain height. This profile is used in the diffraction calculation (the antenna heights are also adjusted depending on the clutter at each end). 2) A terminal clutter correction is also applied, dependant on the clutter at the transmit and receiving end; this is the same as in P. 1546 | Representative of terrain, works with sea paths | Requires detailed terrain height data along the path. Not implemented in SEAMCAT |
| Okumura-Hata | PtP, broadcast | Tx height > 30 m Mobile station height ∈[1; 10] m Distance ∈[1;10] km | Fit on measured data from urban environment, not terrain specific | Easy to use | Not path specific |
| Extended Okumura-Hata in SEAMCAT | Mobile services and other services working in non-LOS/cluttered environment . Urban, sub-urban and open area environments considered. |  | Statistical variation of path loss included | Easy to use and implemented in SEAMCAT. Low-height mobile terminals moving in cluttered environment. | Not path specific |

# RNSS systems

RNSS systems are in use around the globe, with billion RNSS devices in use. There are currently several systems that transmit signals within the frequency range 1240-1300 MHz. The band 1240-1260 MHz is currently used by the Russian Federation Glonass system while the band 1260-1300 MHz is used by the European Galileo system as well as by the Chinese Beidou and the Japanese QZSS. The same band is also planned to be used by the Korean KPS. Specific inputs are provided for Galileo within section 3.3, for Glonass within section 3.4 and for other RNSS within section 3.5, respectively.

Before going into the specific of each system, it is worth go give an overview of the RNSS applications that are typically provided within the band.

## typical RNSS applications in the 1240-1300 MHz BAND

This section should be reviewed making it more technical, EC to propose an update for the next meeting

Although the globally installed base of RNSS devices is greatly dominated by smartphones, followed a distant second by automobiles, the number of RNSS devices in use for professional applications continues to grow and serve a critical role in national economies, public safety, science, etc. Billions of people globally benefit from these high-end RNSS devices on a day-to-day basis, e.g. enjoying the production of sustainable and cost-effective agriculture, using efficiently coordinated transport networks, and leveraging RNSS-synchronized telecommunication networks.

RNSS is used for several type of applications including high precision applications.

### High Precision applications

#### Government: Transportation and environmental management

Commercial high-precision RNSS is used widely by national and local governments, including within the departments for administration of transportation, agriculture, forest and land management, and security as well as by emergency services/first responders and other departments and bureaus.

Local government uses of high-precision RNSS include mapping, surveying and other transportation uses, Geographic Information Systems (GIS) for asset management, emergency preparedness, disaster response and E911 mapping, public sector water, wastewater and electric utilities, public works, environmental management, dam and structure monitoring, environmental health, insurance rating districts, flood zones, tax appraisals, the provision of geodetic control networks, and other functions.

High-precision RNSS is used in response and disaster planning to capture the location of critical infrastructure for utilities, transportation and emergency services. By combining RNSS measurements with elevation models, planners can identify areas susceptible to flooding or other damage. The information is stored in Geographic Information Systems (GIS) where it can be accessed by emergency managers and response organizations.

RNSS is being increasingly used as an essential enabling technology to monitor and provide early warnings for natural phenomena such as earthquakes, landslides, volcanoes and flood hazards.

#### Agriculture

Precision agriculture uses high accuracy, real-time RNSS on-board agricultural machinery to manage distribution of fertilizer and pesticides, and planting and harvesting of crops.

Using RNSS precision guidance, farmers can plant rows closer together and with greater precision, to increase crop yields and reduce waste due to overlaps or gaps. When used on harvesting machines, the collection of RNSS precise positioning data, combined with information about crop yields, is applied to seeding and fertilization plans for the following season’s crops. The RNSS positioning adds precision to weed and insect control, allowing farms to decrease the use of potentially toxic pesticides and herbicides by as much as 80 percent. Precision agriculture requires 24/7 delivery of continuous real-time position accuracies with Key Performance Indicators (KPI) from 1 cm to 10 cm during agricultural operations. This positional capability enables the grower to operate a range of farm machinery, including at night that carefully follows precision farming plans requiring repeatable KPI throughout the growing cycle, from tilling through harvesting. Many precision agriculture receivers require a real-time differential data stream, often delivered by integrated MSS receiver equipment.

#### Construction – Heavy and civil engineering

RNSS construction machine control systems consist of rugged, high-precision RNSS receivers mounted on construction machines of various types. With reference to a computer model of a job grading plan, the RNSS system is required to determine the precise position of the machine’s blade continuously (24/7) to within one inch or less using the on-board computer to continuously compare the blade’s precise position to the design plan. By watching a display in the machine’s cab, the operator controls the machine to produce the desired results.

#### Automotive navigation

RNSS technology is the enabling technology for all automotive navigation. Integration with other sensors and communications networks is common, making RNSS integral to the development of smart cars and autonomous vehicles.

#### -Surveying and mapping

High-precision RNSS is used in many surveying functions necessary for civil engineering and architectural design, production and maintenance of maps and Geographic Information Systems (GIS), land management and title transactions, and management of critical assets such as utility infrastructure, pipelines, dams, roads, rail and waterways. High-precision RNSS is also used to provide services to cities and counties for tax appraisal purposes and flood zone mapping.

#### Utilities, energy, mining, oil and natural gas

High-precision RNSS is used by electric, gas and water utilities to map and manage their widely dispersed assets, in the avoidance and management of major power, water, or gas outages, in vegetation management, rapid location of damaged equipment, in pipeline integrity inspections and in tasks related to environmental and safety compliance. In Energy and Natural Resources, RNSS is used extensively in the construction of sustainable energy projects such as wind farms and solar power sites, seismic exploration and production of domestic oil and gas reserves, mine surveying, measurement and safety monitoring, pipeline construction, pipeline integrity and safety monitoring, drill location and environmental monitoring, measurement and compliance.

### Authentication applications

Nowadays, the use of RNSS as a primary source of PVT can be found in an increasing number of products and services. According to [16], 5.8 bln RNSS devices were in use in 2017. By 2025, this number is forecasted to increase to more than 9 bln. In this context, a failure or loss of signal due to outside influence can result in a range of consequences, as highlighted also in [17].

Due to the extreme low power, RNSS, can be harmed by several threats, intentional and unintentional. Normally these threats are classified in literature is three main categories:

Jamming is the act of directing electromagnetics energy toward communication (and navigation) system disrupting or preventing signal reception. It can be categorized as denial of service, since the GNSS is still available but its signal is interfered;

Spoofing is deliberate transmission of counterfeit RNSS signals with intention to manipulate GNSS receiver into providing false Position, Velocity and Time (PVT) information. The introduction of authentication functionalities is aiming to prevent to the extent technically possible this kind of manipulation.

Meaconing means recording and rebroadcasting of authentic RNSS signals in order to confuse a navigation system or user. In order to o mitigate these risks, several RNSS provide different authentication capabilities for civil use.

Several applications domain have been identified for authentication applications with the corresponding economic impact.

#### Maritime

Authentication applications are expected to bring the majority of benefits to autonomous vessels being able to provide continuous authentication that is required for real-time operations which is considered fundamental by this sector.

The potential benefits of this kind of applications have been ranked as medium/low for fishing vessels, with an added value for this application mainly in case of a depending on the cost of the service.

#### Road

Road presents some similarities with the maritime, especially for connected and automated vehicles:

Authentication applications are expected to positively impact mainly CAD (Connected and Automated Driving), due to its real time authentication capability.

#### Timing

For timing and synchronization, the experts think that a concrete impact can come from authentication applications for all the applications.

The table below provides a summary of the potential gross impacts of authentication applications, distinguishing between
the two scenarios (free vs fee) and highlighting the type of benefit created.

| Domain | Application | Impact (tot - 2025-2035) |
| --- | --- | --- |
| Maritime | Autonomous vessels | 5.8 M€ |
|  | Fishing vessels | 6.1 M€ |
| Road | Connected and Automated Driving (CAD) | 1000 M€ |
|  | Dangerous and Valuable Goods (DVG) tracking | 0.5 M€ |
|  | Fleet management (FM) | 0.5 M€ |
|  | Road User Charging (RUC) | 0.5 M€ |
| Timing & Synchronization | Telecommunication applications | 13.3 M€ |
|  | Energy applications | 0.4 M€ |
|  | Finance applications | 2.3 M€ |
| Consumer Platform | Smartphones, Mobiles and PDAs (payments) | 1.67 M€ |
|  | Total | 1.65 b€ |

## Galileo Emissions and Protection requirements

### Galileo  system description

The Galileo system consists of a constellation of 30 satellite positions (24 transmitting satellites and six in-orbit active spares) with ten satellites positioned on each of three 56° inclined equally spaced orbital planes. Each satellite transmits navigation signals on three carrier frequencies. These signals are modulated with a structured bit stream, containing coded ephemeris data and navigation messages, and have sufficient bandwidth to produce the necessary navigation precision without recourse to two-way transmission or Doppler integration. The system provides accurate timing and position determination in three dimensions anywhere on or near the surface of the Earth. One of the three carrier frequencies is within the frequency band, 1260-1300Mhz, with a central frequency at 1278.75Mhz, and the signal baseline transmitted at that frequency is described in the following section.

### GALILEO E6 Signal Baseline

The Galileo E6 signal baseline is transmitted at the carrier frequency 1278.75 MHz. The E6 signal baseline includes two different signals, the E6-A signal carrying the “Public Regulated Service (PRS)”, and the signal commonly known as “Commercial Service (CS)”, with the components E6-B and -C. The E6-BC signal, which is specified in [1] and [15], includes the newly defined High Accuracy Service (HAS) and Commercial Authentication Service (CAS), as described below. In the following table some basic parameters for the E6-BC signal baseline are provided.

| Service | E6 HAS | E6 CAS |
| --- | --- | --- |
| Carrier Frequency | 1278.75 MHz |  |
| Signal frequency range | 1260-1300 MHz |  |
| Multiple Access Technique | CDMA |  |
| Spreading Modulation | BPSK(5) |  |
| Code Frequency | 5.115 MHz |  |
| Primary PRN Code Length | 5115 |  |
| Data Rate | 1000 sps | - |
| Minimum Received Power | -155.25 dBW |  |
| Polarization | RHCP |  |

Figure 2: Power Spectral Density (PSD) of the Galileo E6 signal baseline

The signal component E6-B carrier the C-NAV data message supporting the High-accuracy service (HAS) and the Commercial Authentication Service (CAS), while E6-C is a data-less pilot component, implementing CAS. The multiplexing of the two signal components is represented in the figure next.

Figure 1: Multiplexing of the GALILEO E6B+C signal

#### High Accuracy Service (HAS)

The Galileo High Accuracy Service (HAS) is transmitted on the E6-B signal component in the frequency band 1260-1300 MHz, as described above. HAS is aimed at market applications requiring higher accuracy performance than that offered by the Open Service [1], and provides added value services targeting 20 cm accuracy or better on a free charge basis, with content and format of data publicly and openly available on a global scale.

The High Accuracy Service complements the current offer of high accuracy solutions for demanding transport-related applications such as automated driving and drones. It also represents a strong entry level solution in precision agriculture and a useful service for mapping and GIS related applications.

#### Commercial Authentication Service (CAS)

The Galileo Commercial Authentication Service (CAS) is transmitted on the E6-C component in the frequency band 1260-1300 MHz, as described above. CAS provides the highest level of robustness for RNSS users by fully encrypting the E6-C component to protect against most spoofing attacks by helping to detect the occurrence of spoofing and allowing equipment and users to take mitigating action. The provision of E6-C can also support the adoption of triple-frequency receivers which are more resilient to interference.

GALILEO CAS is therefore contributing to ensuring the safety and the security of several applications, such as transport and automotive, which are progressively evolving towards higher digitalization, automation and connectivity. Similarly, the synchronization of critical infrastructures, as well as the mobile-based transaction leveraging on RNSS for user authentication, will benefit from using such a secure signal.

### GALILEO Protection requirements

The Galileo receiver which has to be protected is a ground-based receiver that will track the E6-BC signal. The function of this receiver is to track either or both the wide-band E6-B (data) and E6-C (pilot) signal components. In the case E6-B is concerned, the receiver will also demodulate data transmitted on this signal component which provides, among other information, Precise Point Positioning (PPP) corrections. In the case the E6-C is processed, the receiver will implement advanced authentication, including the decryption of the spreading code.

The characteristics of this type of receiver that processes the E6-BC signal are provided in the table below.

| Parameter | Value |
| --- | --- |
| Signal frequency range (MHz) | 1 278.75 21 |
| Maximum receiver antenna gain in upper hemisphere (dBi) | 3 (circular) |
| Maximum receiver antenna gain in lower hemisphere (dBi) | –6 (circular) |
| RF filter 3 dB bandwidth (MHz) | 40.92 |
| Pre-correlation filter 3 dB bandwidth (MHz) | 40.92 |
| Receiver system noise temperature (K) | 722 |

Within the following table, the technical characteristics and protection criteria (maximum aggregate interference thresholds) for the above receiver are provided, both for narrow-band and wide-band interference (narrow-band continuous interference is considered to have a bandwidth less than 1 kHz, wideband continuous interference is considered to have a bandwidth greater than 1 MHz).

| Tracking mode threshold power level of aggregate narrow-band interference at the passive antenna output (dBW) | -134.5 |
| --- | --- |
| Acquisition mode threshold power level of aggregate narrow-band interference at the passive antenna output (dBW) | N/A |
| Tracking mode threshold power density level of aggregate wideband interference at the passive antenna output (dB(W/MHz)) | -140 |
| Acquisition mode threshold power density level of aggregate wideband interference at the passive antenna output (dB(W/MHz)) | N/A 10 |

## GLONASS EMISSIONS and protection requirements

### GLONASS  system description

The GLONASS system is comprised of 24 satellites located in three orbital planes with eight satellites in each plane. The planes are separated from each other by 120 degree longitude. The orbit inclination angle is 64.8 degrees. The satellites are equally spaced by 45 degrees in a plane by argument of latitude. Their rotation period is 11 h 15 min. The height of the orbit is 19 100 km.

Each satellite transmits navigation signals in three frequency bands: L1 (1.6 GHz), L2 (1.2 GHz) and L3 (1.1 GHz). The satellites transmit two types of signals: with frequency division multiple access and code division multiple access. Signals with frequency division multiple access are differentiated by carrier frequency; the same carrier frequency may be used by antipodal satellites located in the same plane. Navigation signals are modulated with a continuous bit stream (which contains information about the satellite ephemeris and time), and also a pseudo-random code for pseudo-range measurements. Signals with code division multiple access have the same carrier frequency and are differentiated by code. These signals are modulated by structured binary sequence that contains coded data about ephemerides and time. A user receiving signals from four or more satellites is able to determine the three location coordinates and the three velocity vector constituents with high accuracy. Navigational determinations are possible when on or near the Earth’s surface.

#### GLONASS Signal description

#### Signals with frequency division multiple access

The carrier frequencies of navigation signals with frequency division multiple access vary by an integer multiple of 0.4375 MHz in the L2 band.

Since 2006 new satellites in the GLONASS system use 14 to 20 carrier frequencies in different bands. In the L2 band carrier frequencies from 1 242.9375 MHz (lowest) to 1 248.6250 MHz (highest) are used. Nominal values of carrier frequencies of radionavigation signals used in the GLONASS system are given in table below.

| K (No. of carrier frequency) | FKL2 (MHz) |
| --- | --- |
| 06 | 1 248.6250 |
| 05 | 1 248.1875 |
| 04 | 1 247.7500 |
| 03 | 1 247.3125 |
| 02 | 1 246.8750 |
| 01 | 1 246.4375 |
| 00 | 1 246.0000 |
| −01 | 1 245.5625 |
| −02 | 1 245.1250 |
| −03 | 1 244.6875 |
| −04 | 1 244.2500 |
| −05 | 1 243.8125 |
| −06 | 1 243.3750 |
| −07 | 1 242.9375 |

Two phase-shift keying (by 180 degrees of the phase) navigation signals shifted in phase by 90 degrees (in quadrature) are transmitted at each carrier frequency. They are a standard accuracy (SA) signal and a high accuracy (HA) one.

The SA signal structure is a pseudo-random sequence which is Modulo-2 added to a continuous digital data stream transmitted with a 50 bit/s rate. The pseudo-random sequence has a chip rate of 0.511 MHz and its period is 1 ms.

The HA signal is also a pseudo-random sequence Modulo-2 added to a continuous data stream. The pseudo-random sequence chip rate is 5.11 MHz.

Digital data include information about the satellite’s ephemerides, clock time and other useful information.

Transmitted signals are elliptically right-hand polarized with an ellipticity factor no worse than 0.7 . The minimum guaranteed power of a signal at the input of a receiver (assumes a 0 dBi gain antenna) is specified as −161 dBW (−131 dBm) for both SA and HA signals.

In L2 frequency range there are two classes of emissions that are used in the GLONASS system: 1M02G7X and 10M2G7X. Characteristics of these signals are given in table below.

| Frequency range | Emission class | Tx bandwidth (MHz) | Maximum peak power of emission (dBW) | Maximum spectral power density (dB(W/Hz)) | Antenna gain (dB) |
| --- | --- | --- | --- | --- | --- |
| L2 | 10M2G7X 1M02G7X | 10.2 1.02 | 14 14 | −53 −43 | 10 |

The power spectrum envelope of the navigation signal is described by the function (sin x/x)2, where:

in which:

f:	frequency considered

fc:	carrier frequency of the signal

ft:	chip rate of the signal.

The main lobe of the spectrum forms the signal’s operational spectrum. It occupies a bandwidth equal to 2(t. The lobes have a width equal to (t.

3.3.2.2	Signals with code division multiple access

The carrier frequency of GLONASS navigation L2 signal with code division multiple access is 1 248.06 MHz.

The signal with code division multiple access in L2 band includes four components. 
These components are formed by BPSK(1), BOC (1,1) and BOC (5,2.5) modulations.

Two CDMA signals emitted at a single carrier frequency 1 248.06 MHz at different quadrature with 90 degrees shift. Each signal consists of two components with a time-division multiplexing. The data transfer speed in two signals is 125 bit/s and 250 bit/s respectively.

The minimum guaranteed power of a signal at the input of a receiver (assumes a 3 dBi gain antenna with elevation no less than 5 degrees) is specified as −158,5 dBW (−128,5 dBm).

The GLONASS system uses three classes of emission in L2 frequency range: 2M05G7X, 4M10G7X, 15M4G7X. The characteristics of these signals are shown in table below.

| Frequency range | Emission class | Tx bandwidth (MHz) | Maximum peak power of emission (dBW) | Maximum spectral power density (dB(W/Hz)) | Antenna gain (dB) |
| --- | --- | --- | --- | --- | --- |
| L2 | 2M05G7X 4M10G7X 15M4G7X 15M4G7X | 2.05 4.1 15.4 15.4 | 14 14 14 14 | –45.6 –48.2 –52.8 –52.8 | 12.5 |

### GLONASS Protection requirements

The GLONASS receiver may use CDMA and/or FDMA RNSS signals and may operate on multiple carrier frequencies at a time. The term “FDMA” refers to a frequency-division multiple access modulation technique in which all the RNSS satellites use the same modulation code, but each satellite transmits on a different carrier frequency. The term “CDMA” refers to a code-division multiple access modulation technique in which all RNSS satellite signals are transmitted on the same carrier frequency but with different modulation codes.

Frequency range for GLONASS FDMA signals is 1246 MHz + 0.4375 * K ± 5.11,
where K = -7,..., + 6.

For CDMA signals of the GLONASS system, the frequency range is 1,248,06    7,7 MHz.

Characteristics of RNSS receivers (space-to-Earth) are presented below.

| Parameter | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Type 1 |  | Type 2 |  | Type 3 |  |
|  | FDMA | CDMA | FDMA | CDMA | FDMA | CDMA |
| Maximum receiver antenna gain in upper hemisphere (dBi) | 7 (circular) |  | 6 |  | 6 |  |
| Maximum receiver antenna gain in lower hemisphere (dBi) | - 10 (circular) |  | 6 |  | 6 |  |
| RF filter -3 dB bandwidth (MHz) | 30 |  | 30 |  | 30 |  |
| Pre-correlation filter -3 dB bandwidth (MHz) | 20 | 25 | 20 | 25 | 20 | 25 |
| Receiver system noise temperature (K) | 400 |  | 645 |  | 645 |  |

Within the following table, the technical characteristics and protection criteria (maximum aggregate interference thresholds) for the above receiver are provided, both for narrow-band and wide-band interference (narrow-band continuous interference is considered to have a bandwidth less than 1 kHz, wideband continuous interference is considered to have a bandwidth greater than 1 MHz).

| Parameter | Value |  |  |
| --- | --- | --- | --- |
|  | Type 1 | Type 2 | Type 3 |
| Tracking mode threshold power level of aggregate narrow-band interference at the passive antenna output (dBW) | -149 Note 1 Note 2 | -193 Note 1 | -158 Note 1 |
| Acquisition mode threshold power level of aggregate narrow-band interference at the passive antenna output (dBW) | -155 Note 1 Note 2 | -199 Note 1 | -164 Note 1 |
| Tracking mode threshold power density level of aggregate wideband interference at the passive antenna output (dB(W/MHz)) | -140 Note 1 Note 2 | -150 Note 1 | -139 Note 1 |
| Acquisition mode threshold power density level of aggregate wideband interference at the passive antenna output (dB(W/MHz)) | -146 Note 1 Note 2 | -156 Note 1 | -145 Note 1 |

NOTE 1. – For FDMA and CDMA signals processing, narrow-band continuous interference is considered to have less than a 1 kHz bandwidth, and wideband continuous interference is considered to have greater than a 500 kHz bandwidth. Thresholds for interference bandwidths between 1 kHz to 1 MHz (or for FDMA to 500 kHz) are undefined and may require further study.

Note 2. – This threshold should account for the aggregate power of all interference. The threshold value does not include any safety margin.

## OTHER RNSS

# Methodology

In order to assess the potential for coexistence the following steps were used:

1) Use an agreed calculation process for determining the losses expected between a transmitting amateur station and the RNSS receivers. This needs to take into account the effect of terrain between the transmitter and receiver and the local clutter that may be around the receiver.

2) Based on the protection criteria for the RNSS receivers given in chapter 4 and the typical amateur operating scenarios in chapter 2, determine initial expected separation distances.

3) Consider how the results of the measurement campaigns and other technical investigations should be taken into account in order to reflect the specifics of the amateur band plans and application characteristics.

4) Consider the probability for the potential for receiving an interfering signal at the RNSS receiver.

5) Consider other mitigation measures that might be available.

Measurement and technical investigation Results +

Amateur Bandplan Specifics +

Amateur Application Bandwidths +

Coexistence criteria (including refinements to the RNSS criteria)

5) Coexistence Potential

RNSS protection Levels

4) Probability

3) Account for Measurement Results

Path Loss +

Terrain Loss +

Clutter Loss

Likelihood of interference

Amateur Transmitting Parameters

Amateur Application Parameters

- Overall Propagation Loss

2) Separation Distances –

Amateur OperationalMitigations

Technical Mitigation Measures

Amateur Usage Locations

Amateur Usage Patterns

# Simulations

## Initial Study to Evaluate potential Intereference Areas (ref: SE40(21)013)

As directed in SE40 meeting #72, the initial study presented in contribution SE40(21)013 is placed here for the purposes of comment collection and possible refinement.

### Introduction

The aim of the study is to provide an assessment of the geographical extension of the interference cause by stations of the radio amateur service into Galileo E6 Receivers.

The methodology consists considering several types of radio amateur stations and calculating the area around them where the protection criterion of the Galileo E6 receiver would not be met.

### Parameters of the radio amateur stations

The parameters of the radio amateur stations are given in chapter 2.1.5 of this document

### Propagation models

The propagation loss has been calculated using Recommendation ITU-R P.1546. The setting of the model are: type of terrain: land, area type: sub-urban. The height of the RNSS RX above ground is 1.5 m. Two values have been used for the clutter height: 10 m and 0 m (open land).

It has to be noted that with a clutter height of 10 m and a receiver at 1.5 m above ground the direction of arrival of the main propagation path is usually, via a diffraction for the clutter top. . This means the elevation of arrival of the main propagation path could usually be above 5°. If this the case, the assumption of using – 6 dB as the gain for the RNSS receiver could be too optimistic, therefore for a clutter height of 10 m, the antenna gain of the RNSS receiver has been set equal to 3 dBi. On the other side, when the clutter height is set to 0 m, the RNSS receiver gain is set to -6dB (see J. Pahl, Interference analysis - Modelling radio systems for spectrum management, Wiley, 2016).

The polarization loss considered in this study is 3 dB.

### Time variability effect

Due to the variation in the atmospheric conditions and propagation conditions, such as ducting, the interfering signal can show time variability.  These phenomena are taken into account by ITU-R. P. 1546. However, these time effects are mostly relevant over long distances, while at short distances they tend to be negligible. In the calculations it was decided to use 1% and 50%  of time for paths.

### Location variability effects

The other aspect to be considered is the spatial variability of the electromagnetic field. By the way it is conceived, a propagation usually gives the estimated median value of the received power in a given pixel of terrain. This is the case, for instance, of the curves given by ITU-R P.1546.

Inside this pixel of terrain, you can still have slow fading and fast fading. The effect of local statistical variations f the EM field also needs to be taken into account.

In order to appreciate this fact, consider a pixel of terrain 50x50 m wide. Assume that the maximum tolerable interfering power for the RNSS receiver is PintMAX. In order to declare that the pixel is free from interference it is not sufficient to verify that the interfering received power from the radio amateur station, calculated with the chose propagation model, is equal or below PintMAX. For instance, when its value is exactly equal to PintMAX this means that 50% of the locations inside the pixel will be still be above this value. For this reason, the analysis of interference shall be conducted in such a way that, for a given pixel to be declared interference free, the interfering EM field shall below the reference threshold for, say, X=99% of its locations.

It is therefore necessary to have statistical model of the spatial variability of the EM field for a given pixel. In general, such a variation is composed of a slow variation (shadow fading) and fast variation (fast fading), that it due to multipath effects.

A characterization of the spatial variability of the field strength in various frequency band and for different propagation scenarios (the clutter at the location of the RX plays a fundamental role), is described in section 12, Annex 5 of ITU-R P.1546-6.

ITU-R P.1546 gives curves of basic propagation loss for different location probability. The curve of propagation loss given for 50% of probability means that, for that given pixel, 50% of the locations will actually have a propagation loss lower than the value given by the model and 50% a propagation loss higher than that. If, on the other side on considers the curves referring to a location probability equal to 1%, this means that for a given pixel and a corresponding propagation loss, for 99% of the locations inside that pixel the propagation loss will be actually higher (and, therefore, interference lower).

In other words, if one calculate the contour of the interfered area with the model set at 50% location probability, the contour will be the focus where , for a pixel of terrain, say, 50x50 m wide, half of the surface will be interfered and half interference-free. Inside the contour, of course, the interference probability will be higher, and outside lower. On the other side, if one traces the contour with the model set at 1% location probability, the contour will be the focus where, for a pixel 50x50 wide, 99% of the locations inside that pixel will be interference free. Outside the contour the interference probability will be progressively higher, outside, progressively lower.

In this study both contours at 50% and 1% location probability are provided.

### Protection requirements of Galileo

The protection requirements of Galileo, together with parameters of the RNSS receiver, are specified in Recommendation ITU-R M.1902. This recommendation is being reviewed by WP 4C, inter alia, with respect to the Galileo protection requirements. However, it was agreed by WP 4C that the values provided in the PDNR can already be used in the studies.

The preliminary protection requirements are the following:

For a narrow band interfering signal (Bw< 128 kHz) interfering the Galileo received in tracking mode: -134.5 dBW at the output of the RX antenna.

For a wideband interfering signal (Bw> 1 MHz) interfering the Galileo received in tracking mode: -140 dBW/MHz at the output of the RX antenna.

The characteristics of the Galileo receiver are copied in the table below.

Table 1. Galileo RX parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Polarization | Circular |  |
| Antenna gain upper hemisphere | 3 dB |  |
| Antenna gain lower hemisphere | -6 dB | To be used for elevations up to 5° |

For the study, the values of -134.5 dBW -140 dBW/MHz have been  used, corresponding to the narrowband interferer and the wideband analogue ATV interferer, respectively. For the choice of the RNSS receiver antenna gain, see the section about the propagation model.

### Parameters of the radio amateur stations

Parameters for the radio amateur station vary significantly, both in terms of transmission power and type of signal.

Based on the information contained in chapter 2.1.5 the following ‘typical stations’ have been considered.

#### Home stations

Two set of parameters for the home stations have been considered (the power varying for each of the two types). They are given in the tables below.

Table 2. Parameters for station: Home station 1

| Parameters | Value |
| --- | --- |
| Antenna | Single Yagi, 18 dBi gain, 18° 3 dB aperture |
| TX power | 1 W, 100 W, 300 W |
| Antenna height above ground | 12 meters |
| Polarization | Linear |

The following figures give the graphical representation of the antenna diagram of the Single Yagi antenna with 18 dBi gain and 18° 3dB-beamwidth obtained using ITU-R F. 1336.

Figure 2. Antenna diagrams (F. 1336), Yagi antenna 18 dBi, 18° half beamwidth (home station 1).

Figure 3. 3D representation of the antenna diagram. Colorbar units is dBi.

Table 3. Parameters for station: Home station 2

| Parameters | Value |
| --- | --- |
| Antenna | Dish (4m), 32 dBi gain, 4° 3 dB aperture |
| TX power | 50 W |
| Antenna height above ground | 3 meters |
| Polarization | Linear |

Figure 4. Antenna diagram, home station type 2.

Figure 5. 3D representation of the antenna diagram. Colorbar units is dBi.

#### Permanent installation

This class of station is constituted by stations not installed at the home location of the radio amateur, used as relay and beacons. For this station it is considered only one type of station, whose parameters are given below.

Table 4. Permanent installation parameters

| Parameters | Value |
| --- | --- |
| Antenna | 10 dBi gain, 60° 3dB beamwidth |
| TX power | 25W |
| Antenna height above ground | 25 meters |
| Polarization | Linear |

Figure 6. Antenna diagram, permanent installation.

Figure 7. 3D representation of the antenna diagram. Colorbar units is dBi.

### Type of service and frequency overlap with the RNSS

As indicated above, ITU-R M.1902 prescribes two protection criteria, differing depending on whether the interfering signal is narrowband or wide band. In ITU-R M.1902, however, the protection criterion for system 6 (corresponding to Galileo E6) does not depend, also in the case of a narrowband interferer, on where the interfering signal inside the useful band of the Galileo signal. Also, the protection criteria in the Recommendation do not take into account modulation and other emission parameters such as duty cycle.

As a result of this assumption, the in-band interference analysis conducted using ITU-R M.1902 leads to two type cases only: narrowband interfering signal and wideband interfering signal.

### Results of simulations

#### Home station-1

The following set of figures represents the interfering received power for a home station type 1 whose parameters are contained in Table 1, for different Tx powers and for different location probabilities (50% and 1%). It has to be noted that ITU-R P.1546 is not defined for distances less than 1 Km, and this is reflected in the circular area around the transmitter, with radius 1 km (in dark blue in the figures), where the Loss is not calculated.

(a)                                                                        (b)

Figure 8. Home station 1, sub-urban environment, Tx power 1 W, location probability 50%, clutter height 0 m (RNSS RX gain – 6 dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 9. Home station 1, sub-urban environment, Tx power 1 W, location probability 1%, clutter height 0 m (RNSS RX gain – 6 dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 10. Home station 1, sub-urban environment, Tx power 100 W, location probability 50%, clutter height 0 m (RNSS RX gain – 6 dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 11. Home station 1, sub-urban environment, Tx power 100 W, location probability 1%, clutter height 0 m (RNSS RX antenna gain -6dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 11. Home station 1, sub-urban environment, Tx power 300 W, location probability 50%, clutter height 0 m (RNSS RX antenna gain -6dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 11. Home station 1, sub-urban environment, Tx power 300 W, location probability 1%, clutter height 0 m (RNSS RX antenna gain -6dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 11. Home station 1, sub-urban environment, Tx power 1 W, location probability 50%, clutter height 10 m (RNSS RX antenna gain 3dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 12. Home station 1, sub-urban environment, Tx power 1 W, location probability 1%, clutter height 10 m (RNSS RX antenna gain 3dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 12. Home station 1, sub-urban environment, Tx power 300 W, location probability 50%, clutter height 10 m (RNSS RX antenna gain 3dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 12. Home station 1, sub-urban environment, Tx power 300 W, location probability 1%, clutter height 10 m (RNSS RX antenna gain 3dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

#### Home station-2

The following figures refer to the home station, type 2.

(a)                                                                        (b)

Figure 13. Home station  2, sub-urban environment, Tx power 50W, location probability 50%, clutter height 0 m (RNSS RX antenna grain – 6dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 14. Home station 2, sub-urban environment, Tx power 50W, location probability 1%, clutter height 0 m (RNSS RX antenna grain -6 dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 15. Home station 2, sub-urban environment, Tx power 50W, location probability 50%, clutter height 10 m (RNSS RX antenna grain 3 dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 16. Home station 2, sub-urban environment, location probability 1%, clutter height 10 m (RNSS RX antenna grain 3 dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

#### Permanent installation

(a)                                                                        (b)

Figure 17. Permanent installation, sub-urban environment, broadband interferer, Tx power 25 W, location probability 50%, clutter height 0 m (RNSS RX antenna gain – 6dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 18. Permanent installation, sub-urban environment, broadband interferer, Tx power 25 W, location probability 1%, clutter height 0 m (RNSS RX antenna gain -6 dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 19. Permanent installation, sub-urban environment, broadband interferer, Tx power 25 W, location probability 50%, clutter height 10 m (RNSS RX antenna gain 3dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

(a)                                                                        (b)

Figure 20. Permanent installation, sub-urban environment, broadband interferer, Tx power 25 W, location probability 1%, clutter height 10 m (RNSS RX antenna gain 3 dBi) for: (a) Bw< 128 kHz with -134.5 dBW using Recommendation ITU-R P.1546, (b) Bw> 1 MHz with -140 dBW/MHz using Recommendation ITU-R P.1546.

The simulations indicate interference areas around radio amateur stations with an extent of several km, depending on the case.

Interferences around the amateur radio station are higher, by around 10 dB, for narrowband signals. For wideband interfering signals, if a -140 dBW/MHz value at the output of the RX antenna is considered then the protection criteria is lower for all types of antennas, for all power values and for all location probabilities. For higher location probabilities the protection criteria is also higher as it should be expected. If the TX power is increased, then the area of interference also increases making it more difficult to protect the RNSS systems against possible interference. Same conclusion can be made for higher clutter, the interference around the Amateur Radio station increases.

Three types of radio amateur radio stations have been considered and it was observed that the higher the gain of the station the higher the interference area. It was also observed that the highest interference area, till around 65 km, was obtained for Home station 2 where the maximum gain of the station was considered 32 dBi. It can also be noted that if the amateur radio station has a lower 3dB beam width then the interference is at its highest in the direction of the maximum gain. But, for antennas with higher 3dB beam with the interference starts to become more consistent in other directions too and not only in the direction of the maximum gain.

Further analysis could consider frequency dependent protection criteria and specific applications where very directive antennas are pointed in direction that reduce the interference, such as moon-bounce communications.

# Measurements

This chapter presents a discussion of the results of measurements presented in the Annexes.

[TBD]

## Measurements executed in italy by EC JRC

Within ANNEX 7 an extensive analysis of multiple consecutive interference events taking place in May/June 2021 in the area of Varese (Italy) caused by an Amateur Radio Repeater active in the region is presented. The annex reports spectrum measurements as well as an assessment of the impact of the interference on some high-grade GNSS receivers and resulting in an extensive degradation.

As reported in the annex conclusions, the interfering event has been reported to the competent authorities in Italy (Ministero dello Sviluppo Economico, MISE).

Following the evidence presented in ANNEX 7, EC JRC is running an extensive testing campaign using a synthetic laboratory setup and a wider range of GNSS receivers. This campaign aims at covering a wider range of power levels, in order to assess the effect of closer or farther distances from the source, as well as other typical AS emission types and carrier frequencies. Also, the effect of the interference will be assessed when the receivers are in a mobile sub-urban scenario, exploiting data collected in June when the emission was present. The result of this campaign will be presented through successive contributions to this report and will complement the results provided in this contribution.

# Quantification of interference and options for RFI mitigation

## Analysis of protection criteria

## [RFI mitigation on GALILEO E6-B receiver]

## Mitigation in amateur emissions

## Statistics of use of band by amateur services (time and location)

# Conclusions

List of references

European Union, “Galileo Open Services Signal in Space Interface Control Document (OS SIS ICD)”; http://ec.europa.eu/growth/sectors/space/galileo/; 2016

"Compatibility between Amateur Radio Services and Galileo in the 1260-1300 MHz Radio Frequency Band", Issue 1.1, European Commission, Joint Research Centre 2015 (FM44(19)017)

"Compatibility between Amateur Radio Services and GALILEO in the 1260- 1300 MHz Radio Frequency band". JRC Scientific and Policy Reports, Issue 1.1 2015

"Coexistence measurements: amateur radio service and Galileo E6 in the frequency range 1260 – 1300 MHz", Measurement report G 001/00261/18 and G 531/00282/18 BNetzA, Germany (PTA(19)032)

Recommendation ITU-R M.1787-3: Description of systems and networks in the Radio Navigation-Satellite Service (space-to-Earth and space-to-space) and technical characteristics of transmitting space stations operating in the bands 1 164-1 215 MHz, 1 215-1 300 MHz and 1 559-1 610 MHz”, International Telecommunication Union, Geneva, 2018

The International Amateur Radio Union, IARU Region 1 band plan for 1240 - 1300 MHz (Landshut, 2017); https://iaru-r1.org/index.php/spectrum-andband-plans/uhf/23-centimeter; last accessed: 2019-01-21

“Binary Offset Carrier Modulations for Radionavigation” John Betz, Journal of the Institute of Navigation (IoN), Vol. 48, No 4, Winter 2001/2002, USA,

“Effect of Narrowband Interference on GPS Code Tracking Accuracy”, John Betz, ION NTM 2000, 26-28 January 2000, Anaheim, CA

”Effect of Partial-Band Interference on Receiver Estimation of C/No: Theory”, J. Betz, MITRE Report, 2006

“GNSS Disruptions,” in GNSS technology and applications series, Understanding GPS/GNSS: Principles and applications, E. D. Kaplan and C. J. Hegarty, Eds., Boston, London: Artech House, 2017, pp. 549–617

„A Dual-Frequency Interference Suppression Unit for E1/E5a Designed with National Instruments Software Defined Radios", Kraus T., Sailer S., Eissfeller B., Presentation at ION GNSS+2015, 17.09.2015, Tampa, Florida, USA

“Modul zur Unterdrückung von Störsignalen für globale Navigationssysteme mit NI SDR”, Kraus T., Sailer S., Eissfeller B., In: R. Jamal, R. Heinze (Hrsg.): Virtuelle Instrumente in der Praxis 2015. Begleitband zum 20. VIP-Kongress, VDE Verlag 2015, pp. 43-46

"Auswerten von CPU-gestützten Algorithmen zur Störsignalunterdrückung im GNSS-Bereich und FPGA-Implementierung eines SDRs", Sailer S. Diplomarbeit, Hochschule Mittweida, 2015

COMMISSION IMPLEMENTING DECISION (EU) 2017/224 of 8 February 2017 setting out the technical and operational specifications allowing the commercial service offered by the system established under the Galileo programme to fulfil the function referred to in Article 2(4)(c) of Regulation (EU) No 1285/2013 of the European Parliament and of the Council

"Galileo E6-B/C Codes" Technical Note; January 2019

"The Particular Importance of Galileo E6C", J. Curran, T. Melgrad, insideGNSS, vol. September/October 2016, pp. 57-63

"GNSS Antennas: An Introduction to Bandwidth, Gain pattern, Polarization and all that", G.J.K. Moernaut, D. Orban, GPS World

"State of the Art leading Edge Geodetic Antennas from Leica Geosystems", Leica Geosystems, Leica GNSS Reference antennas (White Paper)

"GNSS antennas RF design considerations for u-blox GNSS receivers Application note UBX-15030289-R03", u-blox AG, October 2019

"Antenna Survey 2018", GPS World

"Galileo E6-B/C Codes" Technical Note; January 2019 - available at www.gsc-europa.eu

GNSS Market Report, GSA Issue 5 2017

Global Navigation Space Systems: reliance and vulnerabilities, The Royal Academy of Engineering, 2011

Russian Space Systems, "Global Navigation Satellite System Interface Control Document (GLONASS ICD), Navigational radiosignal in bands L1, L2"; 2008

[25] Russian Space Systems, "Global Navigation Satellite System Interface Control Document (GLONASS ICD), General Description of Code Division Multiple Access Signal System"; 2016

Parsons, The Mobile Radio Propagation Channel, 2nd Edition, Wiley.

AMSS regulations and deployment in Switzerland

In 2019, Switzerland had 8 570 146 residents in an area of 41 285 km2. In the frequency range 1 240–1 300 MHz, there are 41 unmanned amateur radio stations registered in OFCOM’s database (see attached spreadsheet). It is possible that other such facilities are in operation due to ancient permits. The database does not contain systems that were put into operation before 1998 and have not been changed since, or the changes have not been approved by OFCOM. However, the number of such systems is expected to be very small.

| Unmanned Amateur Radio Stations in Switzerland in the Frequency Range 1 240-1 300 MHz |  |
| --- | --- |
| Ratio of unmanned amateur radio stations to the total population | 4.784∙10-6 4.8 per million residents |
| Unmanned amateur radio station density | 1.0 per 1 000 km2 |

Table 1: Unmanned amateur radio stations in Switzerland
in the frequency range 1 240-1 300 MHz

Unmanned stations are more or less in continuous operation, while manned stations only transmit sporadically. However, there is no database about the manned stations.

In Switzerland, the frequency range 1240–1300 MHz is assigned to the amateur and amateur-satellite service on a secondary basis with the following additional restrictions:

1 240–1 260 MHz: Special permissions are required. The use of this band by the amateur-satellite service is prohibited.

1 260–1 270 MHz: The use of this band by the amateur-satellite service is limited to the Earth-to-space direction (according to RR footnote 5.282).

1 270–1 300 MHz: Special permissions1 are required. The use of this band by the amateur-satellite service is prohibited.

The maximum power of the amplifier is 1 000 W peak envelope power (PEP) in all cases. Amateur radio equipment sold in Switzerland has to be compliant to ETSI EN 301 783.

As already noted in earlier SE40 meetings, the amateur radio parameters cover a wide range. It is very difficult, if not impossible, to make statements about their distributions due to their non-normal and asymmetrical behaviour. Thus, descriptive statistics is more helpful. In Table 2, some of the content in OFCOM’s database is summarized to “five-number summaries”:

the sample minimum (smallest observation)

the first quartile (Q1) or 25th percentile

the median or 50th percentile

the third quartile (Q3) or 75th percentile

the sample maximum (largest observation)

| Parameters of Unmanned Amateur Radio Stations in the Frequency Range 1 240-1 300 MHz | Min | Q1 (25th) | Median (50th) | Q3 (75th) | Max |
| --- | --- | --- | --- | --- | --- |
| Tx frequency [MHz] | 1 240.025 | 1 255.000 | 1 260.300 | 1 298.725 | 1 299.875 |
| Rx frequency [MHz] | 1 240.025 | 1 240.763 | 1 270.750 | 1 294.100 | 1 299.600 |
| Tx power [dBW] | -3.0 | 10.0 | 13.0 | 16.0 | 23.0 |
| Antenna gain [dBi] | 2.2 | 8.1 | 11.2 | 12.7 | 21.2 |
| Losses [dB] | 0.0 | 0.0 | 2.0 | 2.8 | 3.0 |
| Bandwidth [kHz] | 0.5 | 16.0 | 16.0 | 37.5 | 20 000.0 |
| Height above ground [m] | 2.0 | 7.5 | 12.0 | 20.0 | 50.0 |
| Height above sea level [m] | 257 | 739 | 946 | 1 464 | 3 574 |

Table 2: Parameters of unmanned amateur radio stations in Switzerland
in the frequency range 1 240-1 300 MHz

Linear polarizations are mainly used, but occasionally circular polarizations can also be found. The information available on the antenna types used is only very general and can be seen in Table 3.

| Antenna Types of Unmanned Amateur Radio Stations in the Frequency Range 1 240-1 300 MHz |  |
| --- | --- |
| Directional antennas | 53.7% |
| Non-directional antennas | 46.3% |

Table 3: Antenna types of unmanned amateur radio stations in Switzerland
in the frequency range 1 240-1 300 MHz

OFCOM’s database also contains information about the types of traffic modes used (Table 4).

| Traffic Modes of Unmanned Amateur Radio Stations in the Frequency Range 1 240-1 300 MHz |  |
| --- | --- |
| Semi-duplex | 75.6% |
| Simplex | 14.6% |
| Duplex | 4.9% |
| Broadcast | 4.9% |

- Table 4: Traffic modes of unmanned amateur radio stations in Switzerland
in the frequency range 1 240-1 300 MHz

Parameters of unmanned Amateur Radio Stations in Germany, Frequency Range 1240 – 1300 MHz

NATIONAL OVERVIEW

At the end of 2019 Germany had 83 166 711 residents in an area of 357 582 km2. Approximately 64 000 citizens possess an amateur license (licenses for stations not included, as club stations, unmanned stations need a separate license) although only a small proportion of those amateurs will be active and transmitting in the band 1240-1300 MHz.

308 licences for automatic stations in the band 1240-1300 MHz existed at the end of July 2020. Several licenses may be issued for the same location, one for each transmitting station. Naturally, the frequency usage is spatially dense in areas where many people reside.

Table 1: Unmanned amateur stations in Germany, frequency range 1240 – 1300 MHz

| Parameter | Value |
| --- | --- |
| Ratio of unmanned amateur radio stations to the total population | 3,703 • 10-6 |
| Unmanned amateur station density | 0,861 per 1000 km2 |

ASSIGNMENT PROCEDURES FOR UNMANNED STATIONS IN THE 23 CM RANGE

According to the regulations in Germany, any automated or remote controlled station (unmanned station) of the amateur service has to be licensed. Each of those stations gets an individual callsign (on a single site, several stations can exist). On course of this assignment, a site-specific examination is done to check the availability of the assigned frequencies by the regulator, thus identifying and preventing interferences by co-/ or adjacent channel use (either by other amateur stations or other frequency users). Individual obligations can be made to ensure interference-free operation.

Following specific regulations apply:

The responsible operator for unmanned controlled stations needs a license equivalent to the CEPT full-license.

For unmanned controlled stations: maximum equivalent radiated power (ERP) of 15 W PEP.

For manned stations: maximum equivalent radiated power (ERP) of 750 W PEP.

Maximum occupied bandwidth is 2 MHz, except:

7 MHz for television emissions, digital or amplitude modulated;

18 MHz for television emissions, frequency modulated;

The term ‘occupied bandwidth’ refers to the 99% bandwidth.

No unmanned operation in the frequency range 1247 – 1263 MHz.

Maximum EIRP of 5 Watt in the frequency range 1247 – 1263 MHz (manned stations).

Frequency range 1260 – 1270 MHz is allowed for amateur satellite service on a secondary basis (other secondary assignments have priority above the amateur satellite service), link direction: earth to space.

The frequencies are checked for conformance with the radio amateur’s own band plan during the application phase; exceptions are only accepted in well-found cases. In the case of ATV applications, the centre frequency affects the available bandwidth: while operation in the middle of the sub-band (1280 MHz) can be granted with a bandwidth up to 16 MHz, on the centre frequency 1288 MHz one only apply for up to 8 MHz and on centre frequency 1291 MHz the operation is limited to a maximum bandwidth of 6 MHz.

The mandatory license for a new station is limited to one year. This gives some additional flexibility in case of issues. After the first year, the license holder can apply for a renewal of the license which is then given for three years. The licenses can be renewed on application.

STATIONS PARAMETERS

The following sections summarize the key parameters that can be derived from the electronic part of the database.

Frequency band usage

The illustration in Figure 1 and the corresponding numbers in Table 2 show the distribution of amateur stations sorted by the sub-bands as given in the radio amateur’s band plan.

One can see that the most interest leans toward the band’s edge. Nonetheless, a certain interest can be noted in the sub-band for the digital links (1291.494 – 1296 MHz)

Figure 1: Number of unmanned amateur stations grouped by sub-band in the 23 cm frequency range

Table 2: Number of unmanned station (frequency range 23 cm) grouped by frequency sub-band

| Frequency range / MHz | Count |
| --- | --- |
| 1240 – 1243.25 | 75 |
| 1243.25 – 1260 | 1 |
| 1260 – 1270 | 2 |
| 1270 – 1272 | 1 |
| 1272 – 1290.994 | 37 |
| 1291.494 – 1296 | 17 |
| 1296 – 1296.15 | 0 |
| 1296.15 – 1296.8 | 1 |
| 1296.8 – 1296.994 | 25 |
| 1296.994 – 1297.981 | 6 |
| 1298 – 1300 | 143 |

Occupied bandwidth

Another issue of interest is the occupied bandwidth. As already implicit stated by the frequency band usage, most stations focus on bandwidth conservative usages. The number show that more than 80% of the issued licenses have applied for up to 150 kHz. The data is given in the following Table 3 and illustrated in Figure 2.

Table 3: Cumulative bandwidth distribution of unmanned amateur stations in the 23 cm range

| Bandwidth class / kHz | Count | Cumulated | Percentage |
| --- | --- | --- | --- |
| ≤ 1 | 18 | 18 | 6% |
| 1 … 6.25 | 15 | 33 | 11% |
| 6.25 … 12,5 | 40 | 73 | 24% |
| 12.5 … 25 | 128 | 201 | 65% |
| 25 … 50 | 42 | 243 | 79% |
| 50 … 150 | 11 | 254 | 82% |
| 150 … 6000 | 18 | 272 | 88% |
| 6000 … 12000 | 12 | 284 | 92% |
| 12000 … 16000 | 24 | 308 | 100% |

Note: the class 150 … 6000 seems an odd choice, considering the huge frequency range involved. Actually, it contains:
1x250 kHz; 1x2000 kHz; 16x6000 kHz

Figure 2: Unmanned amateur stations, 23 cm frequency range: bandwidth distribution

Licensed Equivalent radiated power

The values for the equivalent radiated power (ERP) in dBW are summarised below. The majority of licensees (approx. 60 %) have applied for the maximum ERP of 11,76 dBW (15 W).

Table 4: unmanned amateur station, 23 cm frequency range: ERP distribution

| ERP range / dBW | Count |
| --- | --- |
| ≤ 0 | 4 |
| 5 | 49 |
| 10 | 69 |
| 11,76 | 186 |

A survey of the number of active (transmitting) amateur stations using the band 1240 – 1300 MHz at the busiest times in some countries within the CEPT region

[I added the IARU INFO Doc 001 text following the suggestion of the meeting #68. But it needs further work because formatting is mangled by the ECC template  - like the hyperlinks.

- However I did not copy across the IARU specific Conclusions from the White Paper]

Introduction

As well as taking into account the technical parameters associated with the amateur transmitters, it will be key to consider the number and geographical spread of active amateur transmitters that could interfere with the Galileo service users at any specific time. Although there are many tens of thousands of licensed amateurs in most large European countries and many hundreds that take an interest or may be equipped for the 23cm band, only a fraction of those are actively transmitting in the band at any one specific time.

This paper refers to published information and surveys the number of active transmitting stations recorded from the perspective of home and temporary portable simplex stations using narrow band and wide band modes. It does not deal with operation into repeater stations or the output signals from repeater stations (both narrow band and wideband ATV).

Readily available data from a number of CEPT countries has been consulted but of course stations are operational in all CEPT countries.

Amateur Activity Periods in 1240-1300MHz

In order to incentivise radio station development, regular national and regional contests and activity periods are organised throughout the year by the local national societies and interest groups . These activity periods are identified for narrowband terrestrial simplex communication applications as well as for more specialised activities like earth-moon-earth communications or broadband amateur TV.

These contest and activity periods attract by far the largest number of simultaneous users (and therefore transmissions) onto the 23cm band compared with other times when random transmissions might occur.

As contests require adjudication, the active station logs are submitted to a central source (usually the national radio society or contest organiser) and summaries are published in result tables. These summary tables can be consulted to estimate the number of active stations (and therefore transmitters) over the activity period.

Narrow Band Activity in the range 1296 – 1298 MHz

In many countries monthly activity periods are scheduled during a specific weekday evening usually lasting around 2.5 hours. In addition there are two main Europe-wide contest sessions scheduled during the spring and autumn time that last for 24 hours. The results from these periods can be surveyed to identify the busiest sessions in order to evaluate the maximum number of stations active on the band.

Active stations during the busiest period in some CEPT countries:

| Country | Active Stations |
| --- | --- |
| UK | 100 |
| Germany | 139 |
| France1 | 88 - 127 |
| Italy | 36 |
| Netherlands | 19 |
| Switzerland | 9 |

Note 1: Over each of the last 5 years. Power level data not available.

However not every active station will submit their activity log for adjudication and by way of an example:

The 23cm UK society contest manager reported that 155 different callsigns were active throughout 2019 at some point.

The Dutch society VHF manager indicated that 87 different callsigns were active during 2019 as a whole.

To be conservative the numbers in the table above could be increased by 50%.

Resources consulted:

GB - https://www.rsgbcc.org/cgi-bin//vhfresults.pl?Contest=1.3GHz%20UKAC&year=2019

CH - https://www.uska.ch/amateurfunkpraxis/contest/schweizer-contest-uhfvhf/

IT - http://www.ari.it/index.php?option=com_content&view=article&id=6051&Itemid=352&lang=it

DE - https://www.darc.de/der-club/referate/conteste/ukw-conteste-start/archiv-ukw-conteste/

Earth-Moon-Earth (EME) Activity in the range 1296 – 1298 MHz

There are five major activity contest periods scheduled each year by interest groups in Europe, USA and Italy. Each scheduled period is 24hrs although the moon will only be visible for around 15 hours from any single location.

Again these activity periods are the focus for activity and result in the busiest times on the band.

Active 23cm EME stations in the CEPT countries represented in the results:

| Country | Active Stations |
| --- | --- |
| Russian Federation | 7 |
| Czech Republic | 5 |
| Sweden | 5 |
| Germany | 4 |
| Fr / IT/ Poland | 3 each |
| + 8 more countries | 1 each |

In total 38 active stations across the CEPT region are noted for the specific event analysed. In addition another 19 stations across the CEPT region are noted as “multiband”. These stations will be active on frequencies in the lower VHF and UHF ranges as well as the 23cm band.

Resource:

https://contests.arrl.org/ContestResults/2018/EME-2018-FinalFullResults.pdf

Wide Band Activity (ATV) around 1260 MHz

There is one major regional activity contest period scheduled each year by the amateur TV community in Europe. This is a 30 hour event over a weekend. In addition some national societies organise scheduled activity weekends once a month.

Again these activity periods are the focus for activity and represent the busiest times on the band.

Recorded number of active stations by CEPT country:

| Country | Active Stations |
| --- | --- |
| Italy | 24 |
| Netherlands | 24 |
| UK | 15 |
| Sweden | 5 |
| Spain | 4 |
| France / Germany | 2 each |
| Switzerland | 1 |

In total 77 active stations across the CEPT region are noted for the 30hr regional activity contest. Using the UK as an example, the published results show that 8 of the 15 active stations were temporary “portable” stations.

Resource:

https://wiki.batc.org.uk/images/c/ce/IARU_Region_1_ATV_Contest_2018_Results.pdf

Amateur radio repeater and beacon stations using the band 1240 – 1300 MHz  in some countries within the CEPT region – May 2020

The IARU consulted the national amateur radio society VHF/Microwave Managers in a sample of CEPT countries for national information about the number of repeater and beacon stations assigned frequencies in the band 1240-1300 MHz.

This annex presents that information against the backdrop of the IARU Region1 band plan [1].

Amateur radio Repeater and Beacon Stations

As well as individual radio amateur stations the band 1240-1300 MHz is also occupied by stations operating as repeaters or beacons. These are always individually licensed for a particular location and operating frequency. Their assignment is coordinated on a national basis. In general a repeater station is established at an advantageous radio location to receive surrounding less well sited individual stations on a specific input frequency and relay (i.e. re-transmit) their traffic on an alternative specific output frequency from the better site. This increases the range for less well-sited individual stations. Repeaters may relay voice, amateur TV or data traffic. Voice and TV repeaters might carry either analogue or digital traffic.

Beacons are established for the purposes of monitoring propagation conditions in the band and providing a reliable off-air signal for test purposes.

licensing and assignment procedures

Repeater or beacon stations are usually licensed in their own right as an extension or addition to a specific radio amateur’s personal licence who then becomes the designated “keeper” for that station acting as the official point of contact. Importantly, the keeper (and designated deputies) has the responsibility to close down the transmitting station in a timely manner at the request of the authorities. Repeater station assignments are co-ordinated within the amateur service at a national level by the interested amateur parties usually before the application for a licence is submitted to the national regulatory body. Propagation beacons may additionally be co-ordinated regionally.

National regulatory bodies are often responsible for co-ordinating a licence application with other primary service spectrum users in the band with whom the amateur service is already sharing. This can lead to departures from the generic IARU band plan to take account of other national spectrum services.

VOICE AND DATA REPEATERS

Repeater stations generally operate in a paired frequency duplex mode. There is an input frequency for the receiver and an output frequency for the transmitter.  When not relaying traffic (in stand-by) the repeater output is silent apart from a periodic identification signal. However some stations do revert to a beacon mode when in stand-by. The transmitter is usually activated on receipt of an appropriate signal on the repeater input frequency. Other features can include “watch-dog” timers to time-out the transmissions if a signal persists on the input channel for longer than a specified time (usually a few minutes at the most).

The legacy analogue voice narrow-band FM mode remains common but digital mode usage is expanding. Certain manufacturers and other groups have developed a small number of digital voice modes with varying degrees of popularity. For the 23cm band the most common mode is ‘D-STAR’ (voice at 4800bps MSK) [2]. Other modes are DMR [3] and Fusion [4] but these two were not evident in the survey sample.

Repeater frequencies in 1240-1300MHz

The IARU R1 band plan includes the following sub-bands (identified here as sub-bands ‘a’ to ‘f’) that may be used for repeater operation. Not all frequencies are assigned in every country and the actual frequencies assigned can vary on a national basis.

All Mode (max bandwidth 20kHz):

Sub-Band a

1242.025-1242.700 MHz Repeater output, ch. RS1 – RS28 paired with:

1270.025-1270.700 MHz Repeater input, ch. RS1 -- RS28

Sub-Band b

1242.725-1243.250 MHz Digital communications, ch.RS29 - RS50 paired with:

1270.725-1271.250 MHz Digital communication, ch.RS29 – RS50

Sub-Band c

1293.150-1294.350 MHz Repeater input, paired with:

1258.150-1259.350 MHz Repeater output, ch.R20 - R68

FM Voice / Digital Voice (max bandwidth 20kHz with 25kHz channel spacing):

Sub-Band d

Repeater input – ch.RM0 (1291.000 MHz) - RM19 (1291.475 MHz) paired with:

Repeater output – ch.RM0 (1297.000 MHz) - RM19 (1297.475 MHz)

Additionally - All Mode, General mixed analogue or digital use in 25 kHz channels:

Sub-band e

1298.025MHz (RS1) to 1298.975MHZ (RS39)  [e]

Data Repeater frequencies

Amateur non-voice data ‘Packet Radio’ modes operate through narrow bandwidth traffic nodes and repeaters having similar bandwidth requirements to voice repeater stations. The digital voice mode ‘D-STAR’  has an associated  ‘DD mode’ for higher rate data traffic (128kbs - TDD) that requires a single wider channel of 150 kHz.

The IARU band plan includes these options in different sub-bands in the ‘all mode’ section from 1298 MHz to 1300 MHz for digital mode usage depending on bandwidth.

Sub-band f

1298.000 MHz to 1299.000MHz in 25 kHz channels

1299.000 MHz to 1299.750 MHz in 150 kHz channels

1299.750 MHz to 1300.000 MHz in 25 kHz channels

Voice and Data Repeater assignments – a survey of some CEPT countries

The data in the table below summarises the information received from a sample of IARU Region 1 VHF managers on the number of voice and data repeater stations licensed to operate in the IARU band plan repeater sub-bands in a number of countries. Whether the repeater stations are actually in-service at the time of the survey would require deeper analysis.

Voice and Data Repeater assignments – in some CEPT countries

|  | Voice and Data Repeaters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sub-Band | a |  | B |  | c |  | d |  | e |  | f |  |
| Direction1 | Tx | Rx | Tx | Rx | Tx | Rx | Tx | Rx | Tx | Rx | Tx | Rx |
| Belgium | - | 4 | - | - | - | - | 1 | 1 | 4 | - | 3 | 3 |
| Denmark | - | - | - | - | - | - | 4 | 4 | - | - | - | - |
| France | - | 1 | - | - | - | - | 5 | 5 | - | - | 2 | 2 |
| Italy2, | 4 | 12 | - | - | - | - | 35 | 18 | - | - | - | - |
| Netherlands3,4 | 3 | 11 | 1 | - | - | - | - | - | 13 | - | 2 | 2 |
| Switzerland5,6 | - | 3 | 1 | 2 | 6 | 7 | - | - | 4 | - | 9 | 7 |
| UK | - | - | - | - | - | - | 9 | 9 | - | - | - | - |

Notes:

1	Direction: Tx indicates a repeater transmitting output frequency and Direction Rx indicates a repeater receive input frequency.

2	Two voice repeaters are transmitting on the input frequency in sub-band a (Rx) not in alignment with the IARU plan. One voice repeater is transmitting in 1258.900 MHz, not aligned with the IARU band plan.

3	National database doesn’t indicate the input receiving frequency or the duplex split. On line resources for the specific repeater stations consulted. Four remain unknown.

4	In addition two repeaters receive in channels above 1297.700MHz.

5	Source CEPT WGSE PTSE40 Document SE40(20)010R1.

6	In addition seven repeaters are transmitting and eight are receiving on frequencies that are at variance with the IARU band plan including the ‘All Mode’ sections, the satellite section and the beacon section.

Amateur TV (ATV) repeaters

ATV Repeater frequencies in 1240-1300MHz

The IARU R1 band plan identifies the following sub-bands for analogue and digital TV repeaters. Not all frequencies are assigned in every country and the actual usage varies on a national basis.

1243.250 -1260.000 MHz  - Identified here as ‘Sub-band a’

1272.000 -1290.994 MHz – Identified here as ‘Sub-band b’

In some cases, for national reasons, frequencies outside these ranges may be assigned for ATV repeater operation.

Repeater Architecture

There is no standard TV repeater station and the architecture complexity and mode(s) of operation (e.g analogue or digital standard) are a free choice for the station proposer (unless prohibited by national licence conditions). The features can be chosen to support the interests of local groups of amateur station operators. However the licence might reflect the technology choice in which case regulatory action might be needed if the mode of operation is changed.

An ATV repeater station may exhibit any of the following operational characteristics and features:

Input and output frequencies that are either in-band or cross-band with input or output frequencies in other bands (e.g. commonly 2.3 GHz or 10 GHz).

The repeater station may have more than one input frequency and more than one output frequency.

Older technology analogue ATV repeaters employ frequency modulation.

Analogue TV repeaters are assigned a wider bandwidth channel – usually 12 to 16 MHz.

Newer technology digital ATV repeaters are usually based on adaptations of commercially standardised air interfaces (see trends below).

Digital TV repeaters are assigned narrower channels as low as 6 or 8 MHz.

The repeater station may handle only analogue TV signals or digital TV signals (or both).

The repeater may re-modulate analogue TV signals onto a digital carrier.

The repeater may support control functions (e.g. access request, output mode...) signalled in other frequency bands (e.g. VHF amateur bands).

The repeater station may be flexible to handle various digital TV modes (e.g. symbol rate, coding, error correction etc.).

The repeater may operate in a beacon mode when not in use (e.g. a test card or video loop).

The repeater may be completely de-activated when activity is unlikely (e.g. overnight) to reduce power consumption.

Repeater Trends

Legacy analogue TV repeaters continue to operate but modern installations deploy spectrally efficient digital TV repeaters transmitting DVB-S/MPEG-2 signals (usually 2Msym/sec or 4Msym/sec). This is actively encouraged by the most forward-looking national interest groups who work hard to develop the appropriate hardware and operating practices. Use of these air interfaces reduces the transmission bandwidth and improves the inter-service co-ordination potential. Further experimentation continues to increase the spectrum efficiency of amateur TV signals and it has been shown possible to transmit HD MPEG-4 signals with symbol rates less than 333kSym/sec in a reduced bandwidth (500 kHz).

Repeater Assignments – a survey of some CEPT countries

The data in the table below summarises the information received from a sample of IARU Region 1 VHF managers on the number of ATV repeater stations licensed to operate in the ATV repeater sub-bands identified in the IARU band plan for a sample of CEPT countries. Whether the repeater stations are actually in-service at the time of survey would require deeper analysis.

ATV Repeater Assignments in some CEPT countries

|  | ATV Repeaters |  |  |  |
| --- | --- | --- | --- | --- |
| Sub-Band | a |  | b |  |
| Direction1 | Tx | Rx | Tx | Rx |
| Belgium | - | 5 | 6 | 3 |
| Denmark | - | - | - | - |
| France | 20 | 1 | 4 | 5 |
| Germany2,3 | - | N/K | 53 | N/K |
| Italy4 | 11 | 1 | - | 1 |
| Netherlands | 1 | 4 | 8 | 1 |
| Switzerland | 4 | 1 | 2 | - |
| UK5 | - | 21 | - | 4 |

Notes:

1		Direction Tx indicates a repeater transmitting output frequency and Direction Rx indicates a repeater receive input frequency.

2		In Germany 16 digital ATV repeaters are transmitting on a centre frequency of 1291 MHz and these are included in  “sub-band	 b”.

3		Receiver input frequencies not provided in data.

4		In addition, seven TV repeaters are transmitting between 1240 MHz and 1243 MHz just below sub-band a and one is transmitting	 at 1267 MHz between sub-bands a and b. Ten TV repeaters are receiving just below sub-band a in 1240- 1243 MHz.

5		In the UK there are 25 TV repeaters transmitting outside the band between 1304 MHz and 1322 MHz. This is a national agreement.

Propagation beacons

Propagation Beacon Frequencies in 1240-1300 MHz

Propagation beacons are built and installed at a remote location by radio amateurs to provide stable and accurate off-air signal sources for receiver system testing and importantly to provide an indication of the radio propagation conditions over longer paths. The beacon might be installed to operate with an omni-directional or directional antenna. Usually the beacon emits a narrow band continuous wave signal with an identification (call sign) and location information message repeated on a regular basis using closely spaced FSK. In some cases amateur digital mode signals are employed enabling automated monitoring of the beacon reception. Most beacons are transmitting continuously.

The IARU R1 band plan identifies the following frequency sub-bands for propagation beacons:

1296.750-1296.800 Local Beacon (10W ERP max)

1296.800 - 1296.994 Beacons exclusive

The data in the table below summarises the number of beacon stations licensed to operate in the IARU band plan propagation beacon sub-bands for a sample of CEPT countries.

Beacon Station Numbers in some CEPT countries

|  | Beacons |
| --- | --- |
| Belgium | 5 |
| Denmark | 3 |
| France | 15 |
| Germany | 20 |
| Italy | 13 |
| Netherlands | 4 |
| Switzerland | 2 |
| UK | 11 |

References (move to reference section)

[1]	IARU R1 VHF Handbook	https://www.iaru-r1.org/wp-content/uploads/2019/08/Latest-VHF_Handbook.pdf

[2]	D-STAR: https://icomuk.co.uk/What-is-D-STAR

[3]	DMR: https://en.wikipedia.org/wiki/Digital_mobile_radio

[4]	Fusion: http://systemfusion.yaesu.com/what-is-system-fusion/

An example coverage map for 23cm band ATV Repeaters in the UK

Measurements made in Germany

The content of this annex is still open for discussion

Methodology and measurement setup

GALILEO E6 was represented by a constellation of ten E6 signals. One signal served as the victim while the other nine were considered as system internal noise, as they are discriminate by the spreading codes within the receiver.

Instead of performing go/no-go tests to a given threshold, the measurements were performed parametrically  by applying a wider range of RFI power level while measuring the decrease of the post-correlation C/No of the used GNSS receiver.

The measurement setup is shown in Figure 4. One can see two separate signal paths, one for the GALILEO E6B/C signal and one for the currently tested amateur radio signal. The RFI signals were applied separately, one after the other. Victim and interfering signals from both sides are added at controlled power level and fed into the GNSS receiver.

All receiver input signals are also available to a set of monitoring and measurement equipment (devices 11, 12 and 13). The receiver, like all other active elements in the test set-up, except the RSA (device 11), are interconnected and controlled via LAN. Some test cases include a special Interference Suppression Unit (ISU, device number 24) which is then inserted in front of the GNSS receivers input. Precision step attenuators in both paths enable controlled setting of signal levels. This concept was preferred as it assures reproducible test conditions of the RF power levels. This allowed to compensate the power loss due to the ISU insertion, making sure that the victim receiver is always supplied with the same C/No.

The amateur radio equipment was located in a separate room about 10 m apart from the laboratory hosting the GNSS test set-up. All parts involved in the generation of the amateur radio signals were real life devices (no measurement signal generators involved). All signals were delivered to the main measurements room via high performance coaxial cable to enable unambiguous RF power level conditions at the GNSS receiver input. This separation proofed successfully the rejection of radiation that was measurable in the close vicinity of some transmitters.

Details on each source of amateur signals are provided in [4]. A commercial GNSS simulator generates 10 GALILEO E6B/C-signals and 10 GPS-L1-C/A signals. The GPS L1-signal is used as a time marker. The simulator also adds controlled noise to simulate a defined C/No-condition in the GNSS receiver.

Figure 4: Measurement set-up

The scenario is as follows: the GNSS signal path is levelled in such a way that a C/N0 at the receiver's output of about 45 dBHz is achieved. The power and noise levels within the signal chain are measured. Changes to the levels in the GNSS signal path were only done via the corresponding step attenuator (noise level in that chain is dominated by the GNSS simulator and the line driver amplifier (device number 14) to maintain a constant C/No at the RF domain.

For each amateur signal, the full level at the lowest setting of the step attenuator in the amateur path (device number 18) was measured. Afterwards, the interfering level was set via the step attenuator only. This keeps the relation of the signal and the generated noise and unwanted emissions (spurious resp. out of Band domain) for any power level fed to the victim receiver. Device number 15 to 17 was applied to limit the max. power offered to the precision step attenuator.

The measurement procedure was as follows: the receiver was allowed to settle with the wanted signal active only. Then, a strong level from the unwanted signal was applied to enforce a strong dip in the C/No at the receiver; the interferer was disabled again for a few seconds (allowing the receiver to settle again) and finally the interferer was slowly ramped up in power. During the testing time, the C/No-values were logged, synchronized with the attenuator settings in the interfering signal path.

For any major type of radio amateur signal, a test case investigating the worst case conditions was performed, where the amateur radio signal was placed on the E6-centre frequency (as long as long as the device allowed that setting). Furthermore, frequency offsets as low as possible (according to the IARU-bandplan) were introduced.

Measurement evaluation

Each test case produced files originally created by the receiver's internal measurement feature that were exported to a RINEX file. An evaluation tool transforms the C/No vs. time information of the PSA steps into a representation of C/No vs. the absolute power values at the input of an 0 dBi GNSS-antenna.

The mapping from the receiver input to the input of a 0 dBi antenna is as follows: 
Given the noise level NBWNoise in the E6-signal’s system bandwidth BWNoise of 40.92 MHz and an assumed noise figure of the active receive antenna (NFAnt)dB of 2 dB, the gain of the antenna (GAnt)dB can be derived from the thermal noise level (-174 dBm/Hz). This gain is also applied to the amateur radio signal level, resulting in the level at the input of a 0 dBi antenna.

The evaluations also include a modelled result curve according to the equation of [10, p. 556], where the interference resistance factor Q has been chosen in such a way to smooth the model’s result close to the measured values.

Test cases and measurement results

There is a huge variety of radio amateur applications as indicated in section 2.1. For that, a pre-selection process categorised the emissions into four Groups, representing the diversity of all potential RF emissions:

G1: signal bandwidth < 1 kHz (Morse, SSB voice);

G2: signal bandwidth up to 15 kHz (FM voice);

G3: signal bandwidth up to 200 kHz (high speed data)

G4: signal bandwidth 1 … 16 MHz (Amateur TV)

Test case overview

The test cases were chosen in such a way that all available amateur signal groups were used. The carrier frequencies were varied to measure at least (if possible) the interfering signal on the Galileo E6-centre frequency  (worst case scenario) and with the least possible frequency separation from the E6-centre frequency, if IARU's band plan for region I is respected (bound to the transmitter's available channel selection options).

The following Table XXX gives an overview of all measurements performed. Within the table, the columns have the following meaning:

Group: indicates the group of emission classes the interfering signal belongs to.

Topic: contains a short description of the test’s settings.

Interfering freq.: centre frequency of the amateur radio signal, in MHz

Offset from E6-centre: the difference between the amateur radio signal’s centre frequency and the E6-centre frequency, in MHz.

Level type (amateur radio): refers to the detector/measurement type used in measuring the RFI at 0 dB attenuation.

Table 1: Measurements Galileo E6 vs. amateur radio - overview of test cases performed

| Group | Topic | Interfering freq./MHz | Offset from E6-centre/MHz | Level type (amateur radio) |
| --- | --- | --- | --- | --- |
| 1 | Morse, IARU band plan | 1296.20 | 17.45 | PEP |
| 1 | Morse, centre frequency | 1278.75 | 0.00 | PEP |
| 1 | SSB voice, IARU band plan | 1296.20 | 17.45 | PEP |
| 1 | SSB voice, centre frequency | 1278.75 | 0.00 | PEP |
| 1 | SSB voice, centre frequency, ISU mitigation | 1278.75 | 0.00 | PEP |
| 2 | FM voice, IARU band plan | 1297.50 | 18.75 | PEP |
| 2 | FM voice, centre frequency | 1278.75 | 0.00 | PEP |
| 2 | FM voice, centre frequency, AIM+ filter | 1278.75 | 0.00 | PEP |
| 2 | FM voice, centre frequecy, ISU mitigation | 1278.75 | 0.00 | PEP |
| 3 | FSK 128 kbps, IARU band plan | 1299.21 | 20.46 | PEP |
| 3 | FSK 128 kbps, centre frequency | 1278.75 | 0.00 | PEP |
| 3 | FSK 128 kbps, centre frequency ISU mitigation | 1278.75 | 0.00 | PEP |
| 4 | FM-ATV, IARU band plan | 1280.00 | 1.25 | PEP |
| 4 | DVB-T 1, IARU band plan | 1288.00 | 9.25 | RMS |
| 4 | DVB-T 1 MHz, centre frequency | 1278.75 | 0.00 | RMS |
| 4 | DVB-T 1 MHz, centre frequency, ISU mitigation | 1278.75 | 0.00 | RMS |
| 4 | DVB-T 4 MHz, IARU frequency | 1286.00 | 7.25 | RMS |
| 4 | DVB-T 4 MHz, centre frequency | 1278.75 | 0.00 | RMS |
| 4 | DVB-S 6 MHz, centre frequency | 1278.75 | 0.00 | RMS |
| 4 | DVB-T 1 MHz, frequency sweep | various | various | RMS |
| 4 | DVB-T 1 MHz, center frequency | 1278.75 | 0.00 | RMS |

Measurement results

The following sections detail the result for the individual test case, sorted by the interfering signal. In any section, two plots are given - one that shows the total test result over the whole RFI power tested and a second one that concentrates on the CN0 degradation in a range of -5 … 0 dBHz relative to the 45 dBHz baseline. 
The figures relate the CN0 as reported by the victim receiver to the interfering power supplied by the output of an 0 dBi antenna. . The relationship between the power supplied to the receiver's input port and the equivalent power at the reference antenna is given above.

Morse (Test Cases G1-03, G1-04)

The results for the Morse signal are shown in Figure 25. The C/No plots indicate strong variations. It can be seen that a frequency separation of 17.45 MHz from the centre frequency relaxes the C/No-degradation by approx. 30 dB. Figure 26 shows the changes in C/No relative to a reference of 45 dBHz.
Table 5 shows that at a C/No degradation of 1dB, an additional e.g. 0.5 dB increases the margin of interference power by 2 dB.

Figure 1: Processed results summary for Morse signal

Figure 2: Result summary showing C/N0 changes relative to 45 dBHz for Morse signal

| ΔC/N0 | Morse 1296.20 MHz | Morse 1278.75 MHz | Morse 1278.75 MHz with ISU |
| --- | --- | --- | --- |
| -1 dB | -68,8 dBm | -98,8 dBm | N/A |
| -1,5 dB | -66,7 dBm | -96,8 dBm | N/A |
| -5 dB | -59,6 dBm | -89,6 dBm | N/A |
| Q-factor | 42 dB | 12 dB | N/A |

Table 2: Key values for Morse signal: maximum signal power at 0 dBi antenna to certain C/N0 degradation allowance

#### SSB voice (Test Cases G1-07, G1-08, G1-13)

The results for the SSB voice signal are shown in Figure 27. Figure 28 shows the changes in C/N0 relative to a reference of 45 dBHz. It can be seen that a frequency separation of 17.45 MHz away from the centre frequency relaxes the C/N0-degradation by almost approx. 40 dB. 
If the ISU is not inserted in the centre frequency scenario, the C/N0 degrades fast.

Table 6 shows that at a C/N0 degradation of 1dB, an additional e.g. 0.5 dB increases the margin of interference power by 2dB.

Furthermore, the significant improvement by the ISU is clearly visible: although the ISU has to mitigate the RFI on Galileo’s E6 centre frequency, it allows even more interference signal power than in the offset frequency case where the ISU is not inserted. Considering that the SSB signal has a certain similarity to the Morse signal (both are narrow band, “pulse-type” emissions; but the amplitude variations in the signal envelope differ) the ISU produces a significant improvement on RFI mitigation.

Figure 3: Result summary for SSB voice signal

Figure 4: Result summary showing C/N0 changes relative to 45 dBHz for SSB voice signal

| ΔC/N0 | SSB voice 1296.20 MHz | SSB voice 1278.75 MHz | SSB voice 1278.75 MHz with ISU |
| --- | --- | --- | --- |
| -1 dB | -58,3 dBm | -95,3 dBm | -53 dBm |
| -1,5 dB | -56,3 dBm | -93,3 dBm | -48,6 dBm |
| -5 dB | -49,1 dBm | -86,1 dBm | -39,1 dBm |
| Q-factor | 52,5 dB | 15,5 dB | 64 dB |

Table 3: Key values for SSB voice signal: maximum signal power at 0 dBi antenna to certain C/N0 degradation allowance

FM voice (Test Cases G2-03, G2-04, G2-04 w/WB, G2-05)

The results for the FM voice signal are shown in Figure 29. Figure 30 shows the changes in C/N0 relative to a reference of 45 dBHz. Apparently, a frequency offset of 18.75 MHz from the centre frequency improves the C/N0-degradation by more than 40 dB.

No difference can be seen between the receiver operated with and without the receiver’s built-in AIM+ mitigation.
The ISU compensates the interfering signal on the centre frequency at least as good as a frequency offset from the centre frequency, i.e. operates on frequencies in accordance with IARU Band plan.

Table 7 shows that at a C/N0 degradation of 1 dB, an additional e.g. 0.5 dB increases the margin of interference power by 2 dB.

Figure 5: Result summary for FM voice signal

Figure 6: Result summary showing C/N0 changes relative to 45 dBHz for FM voice signal

| ΔC/N0 | FM voice 1297.50 MHz | FM voice 1278.75 MHz | FM voice 1278.75 MHz with ISU |
| --- | --- | --- | --- |
| -1 dB | -61,5 dBm | -106,8 dBm | -61,5 dBm |
| -1,5 dB | -59,5 dBm | -104,8 dBm | -52,2 dBm |
| -5 dB | -52,2 dBm | -97,6 dBm | -40 dBm |
| Q-factor | 49,3 dB | 4 dB | 64 dB |

Table 4: Key values for FM voice signal: maximum signal power at 0 dBi antenna to certain C/N0 degradation allowance

FSK 128 kbps (Test Cases G3-03, G3-04, G3-06)

The results for the FSK are shown in Figure 31. Figure 30 shows the changes in C/N0 relative to a reference of 45 dBHz. A frequency separation of 20.46 MHz off the centre frequency improves the C/N0-degradation by more than 50 dB. The ISU compensates the interfering signal on the centre frequency almost as good as compared to the signal operated with an offset from the centre frequency.

Table 8 shows that at a C/N0 degradation of 1dB, an additional e.g. 0.5 dB increases the margin of interference power by 2 dB.

Figure 7: Result summary for FSK signal

Figure 8: Result summary showing C/N0 changes relative to 45 dBHz for FSK signal

| ΔC/N0 | FSK 1299.21 MHz | FSK 1278.75 MHz | FSK 1278.75 MHz with ISU |
| --- | --- | --- | --- |
| -1 dB | -48,3 dBm | -102,3 dBm | -57,7 dBm |
| -1,5 dB | -46,3 dBm | -100,3 dBm | -50,7 dBm |
| -5 dB | -39,1 dBm | -93,1 dBm | -39,7 dBm |
| Q-factor | 62,5 dB | 8,5 dB | 64 dB |

Table 5: Key values for FSK signal: maximum signal power at 0 dBi antenna to certain C/N0 degradation allowance

FM-ATV (Test Case G4-01)

The result for the FM-ATV signal is shown in Figure 33. Figure 34 shows the changes in C/N0 relative to a reference of 45 dBHz. The signal was offset by 1.25 MHz from the centre frequency. It can be seen that the modulation’s constant envelope prevents strong fluctuations in the C/N0-indication.

Table 9 shows that at a C/N0 degradation of 1dB, an additional e.g. 0.5 dB increases the margin of interference power by 2dB.

Further tests regarding frequency offsets were not performed: according to the IARU Band plan, this type of signal normally uses that frequency.

Figure 9: Result summary for FM-ATV

Figure 10: Result summary showing C/N0 changes relative to 45 dBHz for FM-ATV

| ΔC/N0 | FM ATV 1280.00 MHz | FM ATV 1278.75 MHz | FM ATV 1278.75 MHz with ISU |
| --- | --- | --- | --- |
| -1 dB | -106,8 dBm | N/A | N/A |
| -1,5 dB | -104,8 dBm | N/A | N/A |
| -5 dB | -97,6 dBm | N/A | N/A |
| Q-factor | 4 dB | N/A | N/A |

Table 6: Key values for FM ATV signal: maximum signal power at 0 dBi antenna to certain C/N0 degradation allowance

DVB-T ATV with 1MHz bandwidth (Test Cases G4-05, G4-06, G4-06x)

The result for the DVB-T ATV-signal with a bandwidth of 1 MHz is shown in Figure 35. Figure 36 shows the changes in C/N0 relative to a reference of 45 dBHz. In the offset case, the interfering signal’s centre frequency was shifted 9.25 MHz away from E6 centre frequency. No strong fluctuations can be seen as compared to e.g. the Morse or FSK signal.

The result shows that a frequency offset relaxes the interference situation. Furthermore, the ISU performs differently as compared to the narrow band emissions, albeit for the power levels tested it still prevents the C/N0 to degrade to less than 38 dBHz. This might be due to the following reasons:

The current state of the ISU’s development is to perform best on small band signals.

The ISU removes those parts of the spectrum that have been identified as an unwanted signal. The wider the removed spectrum, the more power of the Galileo spectrum is also removed. Because of this removal, an inherent drop of C/N0 by approx. 2 dB occurs. This can be seen at RFI level of approx. -71 dBm.

The bump in the ISU-curve is due it's detection threshold: as long as the incoming signal is not rated as interference, the ISU does nothing at all.

Table 10 shows that at a C/N0 degradation of 1dB, an additional e.g. 0.5 dB increases the margin of interference power by 2 dB.

Figure 11: Result summary for DVB-T ATV, 1 MHz bandwidth

Figure 12: Result summary showing C/N0 changes relative to 45 dBHz for DVB-T ATV, 1 MHz bandwidth

| ΔC/N0 | DVB-T ATV 1 MHz 1280.00 MHz | DVB-T ATV 1 MHz 1278.75 MHz | DVB-T ATV, 1 MHz, 1278.75 MHz with ISU |
| --- | --- | --- | --- |
| -1 dB | -79,3 dBm | -105,8 dBm | - |
| -1,5 dB | -77,3 dBm | -103,8 dBm | - |
| -5 dB | -70,1 dBm | -96,6 dBm | - |
| Q-factor | -31,5 dB | 5 dB | - |

Table 7: Key values for DVB-T ATV signal (1 MHz bandwidth): maximum signal power at 0 dBi antenna to certain C/N0 degradation allowance

DVB-T ATV with 1MHz bandwidth, variable centre frequency offsets (Test Case G4-18)

In addition, further tests were performed to study the influence of various carrier frequencies for the DVB-T signal. For that, the DVB-T signal level was kept constant, while the frequency was shifted through a portion of GALILEO’s signal spectrum. The result is shown in Figure 37 with absolute C/N0 values on the left Figure and C/N0-values relative to the value at Δf=15 MHz in the right Figure).

The figure can be read as follows: the GALILEO signal is the more affected the closer the interferer is to its centre frequency. This complies largely with results reported in the previous sections. The spectral zero at the offset -5 MHz is not as vulnerable as the centre frequency, but more sensitive than the zero at -15 MHz.

The test was not extended to the positive frequency offset range, since the spectra of the E6B/C signals involved are symmetric around their centre frequency. This general finding is considered sufficient for an initial estimation of the RF compatibility situation.

Figure 13: Influence of different centre frequencies of the DVB-T signal. Left hand side: absolute C/N0 values; right hand side: C/N0-values relative to the values at the frequency offset of -15 MHz

DVB-T ATV with 4 MHz bandwidth (Test Cases G4-09, G4-10);
DVB-S ATV with 6 MHz bandwidth (Test Case G4-14)

The results for the DVB-T ATV-signal with a bandwidth of 4 MHz and the DVB-S signal (bandwidth: 6 MHz) are shown in Figure 38. Figure 39 shows the changes in C/N0 relative to a reference of 45 dBHz. In the offset case (DVB-T only), the RFI carrier frequency was shifted 7.25 MHz away from E6 centre frequency. No strong fluctuations can be seen as compared to e.g. the Morse or FSK signal. When comparing this Figure to the 1 MHz DVB-T case, one has to remember that the offset in this case is smaller (interferer is closer to the centre frequency) and the signal is wider (interferer’s power is spread wider across the spectrum, reaching even further to the centre frequency). Apart from that, the DVB-S signal has a constant envelope, in contrast to the DVB-T OFDM signal.

The result shows that a frequency offset relaxes the interference situation, but far less than in the 1 MHz bandwidth case.

Table 11 shows that at a  C/N0 degradation of 1dB, an additional e.g. 0.5 dB increases the margin of interference power by 2dB.

Figure 14: Result summary for DVB-T ATV, 4 MHz bandwidth and DVB-S ATV, 6 MHz bandwidth

Figure 15: Result summary showing C/N0 changes relative to 45 dBHz for DVB-T ATV, 4 MHz bandwidth and DVB-S ATV, 6 MHz bandwidth

| ΔC/N0 | DVB-T ATV 4 MHz 1286.00 MHz | DVB-T ATV 4 MHz, and DVB-S 6 MHz 1278.75 MHz | DVB-T ATV, 4 MHz,and DVB-S 6 MHz 1278.75 MHz with ISU |
| --- | --- | --- | --- |
| -1 dB | -98,8 dBm | -108,3 dBm | N/A |
| -1,5 dB | -96,8 dBm | -106,3 dBm | N/A |
| -5 dB | -89,6 dBm | -99,1 dBm | N/A |
| Q-factor | 12 dB | 2,5 dB | N/A |

Table 8: Key values for DVB-T ATV signal (4 MHz Bandwidth) and DVB-S (6 MHz Bandwidth):  maximum signal power at 0 dBi antenna to certain C/N0 degradation allowance

Test for independence of initial C/N0  (Group G4-20)

To study if the initial set C/N0-ratio has an influence on the measurement result, the following test was performed: the level of the simulated GALILEO signals was set in such a way that every two satellite signals had the same power. The noise power level remained unchanged with respect to the previous measurements.
Then, the DVB-T signal with 1 MHz bandwidth was applied as RFI on the E6 centre frequency. The test sequence started again (setting a trigger point, returning to the initial signal power level and gradually increasing the RFI power). The result is shown in Figure 40 below.

It can be seen that all lines run in parallel to each other. This means that the absolute value of the initial C/N0 has no influence on C/N0 degradation as long as that C/N0 is far above the receiver's sensitivity to capture the wanted signal.

Figure 16: C/N0 vs. time with different GALILEO signal power levels, interfered by a DVB-T signal (1 MHz bandwidth, E6 centre frequency)

Summary

Measurements were performed to study the effects on the post-correlation C/N0 when different amateur radio signals arrive at a 0 dBi antenna of a GALILEO E6B/C DS SS CDMA receiver. Many different signal types and frequency combinations were investigated to cover not only worst case, but also typical situations.

The tests performed considered all signals that are usually emitted in the amateur service on frequencies in accordance with the IARU band plan as well as on the E6 centre frequency.

The findings can be summarised as follows:

The worst case occurs when an interfering signal is applied on the E6 centre frequency

Vice versa: frequency separation from centre frequency yields significantly higher tolerable levels for the interfering signal (e.g. when using frequencies i.a.w. the IARU Band plan)

A non-constant envelope of the interfering signal leads to high scattering of the receivers observed C/N0.

Although DVB-T OFDM-signals also employ a non-constant envelope, they are fast enough to avoid such fluctuations and have apparently less dynamic amplitude changes than, for example, an on/off-keyed narrow band signal such as used in Morse telegraphy

The used Interference Suppression Unit (ISU) can significantly reduce the impact of interfering signals, particularly for narrowband signals (B < 150 kHz).

Even if the ISU’s performance was not tested against the Morse signal, one can expect a similar behaviour as seen in the e.g. SSB signal case.

Against the wider amateur TV signals, the used ISU did not lead to a strong receiver immunity.

The wider the signal to be supressed (i.e. filtered), the more energy is taken from the E6 signal. This reduction of the RF signal's CN0 leads to a worse CN0 at the receiver's output.

It is to be noted that the used receiver’s input filter is smaller than specified in [1] (30 MHz instead of 40.92 MHz). Nonetheless, the RF level measurements respect the full channel bandwidth.

GALILEO E6B probability of bit error conditions under varying C/No

Relation between Eb/No and C/No

In the Galileo system, the following relation holds between the carrier to noise and energy per bit to noise ratios:

CN0|dB= EbN0|dB+10lg⁡(Rd∙r∙D)

where the symbol transmission rate is given by Rd , the channel code rate r and the power distribution between the data and pilot channel D.

The E6B signal has a gross bit rate of 1000 symbols per second with one symbol representing one bit. The error correction scheme is three-fold: a cyclic redundancy check (CRC), a half-rate convolutional forward error correction (FEC) and block interleaving. The net bit rate is approx. 500 bps. The power distribution in the RF channel is equal between the pilot and data channel. This leads to:

CN0|dBHz= EbN0|dBHz+10lg1000∙0.5∙12-1

CN0|dBHz= EbN0|dBHz+30 dB

BER for E6B-C

In an AWGN channel with an assumed coherent receiver (perfect tracking performance in terms of time and frequency), the BER is given by

Pb=12erfc(EbN0)

Using Matlab's® BER-Tool, the following figures can be derived for the uncoded and coded (hard- and soft decision decoder) case. The simulations are verified by measurements on two receivers [16].

In the following, a hard decision decoder is assumed.

Figure 17: BER for Galileo E6-B/C with no channel coding and convolutional decoding with hard and soft decision in AWGN channel, no implementation losses assumed

Relation between minimum power level of Galileo E6-B/C and C/N0

[1] states that the Galileo satellites provide the minimum signal level of -125 dBm for user elevation angles above 5 degrees (nadir) at the output of an ideally matched (RHCP) 0 dBi antenna.

The gain pattern of an antenna is a function of azimuth and elevation. For GNSS receive antennas, the azimuth pattern is typically uniform and can be neglected.

The gain pattern for the elevation is dependent on the model. In the following, two different elevation patterns of (geodetic) antennas are sketched.

Figure 18: Example of elevation gain patterns of two different geodetic antennas [17], [18]

Patterns related to more consumer-oriented products -- focussed on the performance of GPS on L1 -- can be taken from [19]. For the two antennas shown, one can see that the 0 dBi-line is reached at a boresight angle of approx. 60°. At 5° above nadir, both antennas are at about -8 dBi.

The noise figure of GNSS receivers is typically dominated by the noise of the active antennas. A typical value of the antenna noise figure F is 2 dB [20].

In total, the expectable Carrier-to-Noise level for a satellite 5° above nadir is given by:

CN0min,dBHz= Pmin,dBm-gAnt-N0,dBm/Hz+FdB

CN0min,dBHz= -125 dBm-8 dBi-(-174dBmHz+2 dB)=39 dBHz

Quantification of interference and options for RFI mitigation

Analysis of protection criteria

Protection criteria determined by CN0 degradation down to a certain BER

The difference between the reception at the minimum signal level and a target BER is the degradation allowance due to any kind of RFI or signal propagation related phenomena.
If one assumes the minimum CN0 of 39 dBHz as stated in section 7.4.3 and allows a minimum BER of 1e-2 (coupled to an CN0 of 34 dBHz, c.f. section 7.4.2), then a margin of 5 dB is allowed.

RFI mitigation on GALILEO E6-B receiver

There is no agreement on whether the following text or parts of it could be moved to the main body of the report.

Galileo E6 receivers typically employ a pulse-blanking mechanism to handle the pulses from aeronautical large-area radar installations. Those blankers typically feature a level triggered disable circuit to interrupt the potential high level pulses into the receiver's sensitive circuits. The resulting loss of signal is in the time span of nanoseconds, which neither causes the loss of a bit (gross data rate of 1000 bps) nor the loss of the receiver's synchronization.

On the other hand, amateur radio signals -- even if keyed -- are in the range of at least milliseconds. Employing receivers with circuits to supress amateur radio emissions seems to be an attractive option if one considers that modern receivers sometimes are equipped with jammer mitigation capabilities. Along with certain sensing strategies, those jammers work by adaptive filtering in time or frequency domain of suspect signals. Even more advanced schemes for high grade receive systems incorporate steered antennas to mitigate interference signals.

With those considerations in mind, the recommendation of mitigation circuits seems a viable option, for not only the robustness against amateur radio signals is enhanced.

Measurements from EC JRC on interference event from AS to GNSS in Italy

Evidence of interference events from AS to Galileo receivers are provided in this annex referring to multiple events observed in May/June 2021 in the region of Varese (Italy), and assessed by the Joint Research Centre (JRC) of the European Commission. Following the events reported below and using them as a reference, further measurements are ongoing within the JRC laboratories in the effort to characterize the effect of different AS emission types (at various carrier frequencies and power levels) on multiple GNSS receivers.

In June 2021 the Joint Research Centre of the European Commission executed a number of data collections within its premises (located in Ispra, in the vicinity of Varese, Italy). Those data collections dealt with the testing of Galileo High Accuracy Service (HAS), currently in a pre-operational testing phase of its Signal in Space (SiS) , transmitted in the E6 band (1260-1300 MHz). In the context of the testing preparation, JRC scientists are performing a number of data collections under diverse reception conditions, including static open sky and mobile sub-urban reception. The scope is to test a bench of Galileo E6 receivers in order to confirm adequate processing of the E6-BC signals (acquisition, tracking) as well as adequate demodulation of HAS data (carried by the E6-B signal component). A bench of several receivers enabled to process E6-BC signals from different manufacturers and targeting different market segments is being setup within the JRC Testing and Demonstration Hub for the EU GNSS Programmes.

Analysing some of the data collected (consisting of recorded IF samples from all Galileo satellites in view over E1 and E6 bands), a strong narrow-band emission within the E6 band has been identified. The concerned emission is a narrow-band signal observed at a frequency of 1297.3 MHz. A screenshot of the emission as visualized on a spectrum analyser is provided in Figure 11.

Figure 32:  Spectrum Analyser displaying interfering narrow-band signal at 1297.3 MHz

Note that the emission is characterised by a strong power, being more than 40 dB above the noise floor.

The emission has been analysed and it was identified to be an FM modulated signal transmitted by an Amateur Radio Repeater. The repeater was identified through the Ministerial identifier transmitted through the signal, which included also a code specifying its position (Maidenhead locator system). Recordings of the audio messages and identifiers of the Radio Repeater are available in the case of need.

The emission was observed to last about 15 seconds, and repeating regularly every 2/3 minutes. This behaviour seems compatible with the repeater transmitting its Ministerial identifier in the absence of an input signal making use of the channel.

Following this observation, an analysis has been performed to check the possible presence of this emission in other times beyond the one of the data collection (June 14th, 15:30-16:00 UTC). JRC has a permanent spectrum monitoring station deployed on its premises and logging spectrum measurements 24/7. Reviewing some of those data some interesting cases were identified. It is important to specify that the analysis, looking only at data collected in May/June 2021, did not pretend to be exhaustive and was not looking to all data available but just focusing to identify some more relevant events. Other events might have happened that are not reported here.

In the following figures spectrograms from the JRC Spectrum Monitoring station are provided for some relevant cases that were identified. Some of the identified events are of the same kind as the one previously discussed, and therefore characterised by short emissions (some seconds) repeated over time, compatible with the repeater transmitting its identifier. Those are reported the following three figures.

Figure 33 Spectrogram 1240-1300 MHz – 2021.06.14, 15:19-15:53 UTC (Case 1)

Figure 34 Spectrogram 1240-1300 MHz – 2021.06.14, 5:38-6:12 UTC (Case 2)

Figure 35 Spectrogram 1240-1300 MHz – 2021.06.13, 16:32-17:06 UTC (Case 3)

Other cases have been observed during which instead the emission is continuous over several minutes, possibly corresponding to an effective transmission of a message through the repeater (Figure 37, Figure 38, Figure 36). In these cases it is interesting to observe the presence a second emission at 1291 MHz, received with a lower power, and compatible with a possible uplink to the repeater.

Figure 36 Spectrogram 1240-1300 MHz – 2021.06.14, 17:02-17:36 UTC (Case 4)

Figure 37 Spectrogram 1240-1300 MHz – 2021.06.11, 21:50-22:24 UTC (Case 5)

Figure 38 Spectrogram 1240-1300 MHz – 2021.05.09, 13:02-13:36 UTC (Case 6)

In all the figures above it is interesting to observe the contemporary presence of emissions from radars, whose typical spectral patterns (pulsed signals) are well visible between 1255 and 1270 MHz. There are two airports in the area (the International Airport of Malpensa and the Military Airport of Cameri), both about 25 km southern of Ispra. It is important to mention that those emissions are managed in most of the receivers through pulse blanking, as radars are co-primary in the band and must be tolerated to a certain extent. Thanks to this blanking mitigation techniques most of the interference effect is removed before the correlation. Still, the receiver performance is affected by this operation as, by definition, the blankers are causing also a loss of “useful power”. As such, the necessary coexistence with the co-primary service in the band is already “costing” to GNSS receivers a substantial loss and has to be accounted for within the system margins.

During the events reported above there were two E6 capable receivers logging 24/7 GNSS data. Logs from a third receiver are available for the event on June 14th. An analysis has been performed to observe the impact of the different events on the post-correlation C/N0. The results are provided in the following figures. For obvious reasons the receivers are anonymized and marked as “Receiver A”, “Receiver B” and “Receiver C”. It is important to mention that the three receivers are all high-end, multi-band, multi-GNSS receivers, very well-known and widely employed for professional applications in the high-accuracy domain worldwide.

For what matters the Case 1 (2021.06.14, 15:19-15:53 UTC, Figure 33), during which the interference is present only in short intervals, logs from three receivers are available. The C/N0 for an interval of 30 minutes are represented in the three figures next:

Figure 39 C/N0, Receiver A, E6-B signal, all satellites in view (Case 1)

Figure 40 C/N0, Receiver B, E6-BC signal, all satellites in view (Case 1)

Figure 41 C/N0, Receiver C, E6-BC signal, all satellites in view (Case 1)

As it can be seen, even if the interfering signal is lasting few seconds (and repeating itself every few minutes), this is causing evident drops in the carrier to noise ratio estimated by the different receivers operating during the event. The receiver A has an instantaneous degradation of about 2 dB on all the satellites when the interfering signal is present, while the degradation on Receiver B and Receiver C is much more evident, in the order of about 10 dB and 20 dB respectively, again on all satellites in view.

In order to provide evidence that this effect is coming from the interference (beyond the perfect alignment of the distortions with the presence of the interfering signal), in the following figure the C/N0 estimated by Receiver B while processing E1 signals (1575 MHz) is provided for comparison.

Figure 42 C/N0, Receiver B, E1-BC signal, all satellites in view (Case 1)

As it can be seen in Figure 42, the C/N0 of the receiver while processing Galileo E1-BC signals is absolutely nominal.

Looking at the Case 6 (Figure 38), which is the one presenting the most extensive interfering event with more than 15 minutes of almost uninterrupted transmission from the repeater, the C/N0 measured over one hour at the two receivers operating during the event is represented in the two figures next:

Figure 43 C/N0, Receiver A, E6-B signal, all satellites in view (Case 6)

Figure 44 C/N0, Receiver B, E6-BC signal, all satellites in view (Case 6)

As it can be easily seen, the effect of the presence of the interfering signal is very evident on both receivers, and causing again a drop on the C/N0 of about 3 dB in the case of Receiver A and about 10 dB for the receiver B for all satellites in view.

Provided that different receivers are affected to a different extent (typically as a function of their front-end filter bandwidth), it is evident that all those cases represent unacceptable degradation on all the receivers considered, and affecting all satellites in view, independently from their actual elevation. Indeed the measured degradation is exceeding by far the commonly accepted indicator for an ongoing interference in the terminology of GNSS (as reported in the previous section) which is the decrease of the carrier-to-noise density C/N0 at the output of the tracking loop for every single satellite’s signal by 1 dB.

As previously mentioned, the concerned Amateur Radio Repeater has been identified through its Ministerial identifier transmitted in an FM voice message and it is located in Campo dei Fiori (Varese), a hill dominating the area and located at about 15 km from the JRC Ispra site. The area is represented in the following figure, including markers for the JRC site location (Ispra), the approximate repeater location (Parco Naturale Regionale Campo dei Fiori) and the minimum concerned area (a circle of radius equal to the distance between JRC and the repeater). It is useful in this context to mention that the concerned area includes the main city of Varese and comprises a population which is well above the 100.000 inhabitants.

Figure 45 Area Affected by the emission (as a minimum)

The interfering events above described have been reported to the competent authorities in Italy (Ministero dello Sviluppo Economico, MISE) on 21st June 2021.

Regions 2 and 3 comprise the same areas as Regions defined by the ITU; IARU coordinates the band plan for the frequency band 1 240- 1 300 MHz

This corresponds to 16 stations

This corresponds to 8 stations

The analysed results were published by the national radio amateur societies in several European countries.

A comprehensive discussion of fast and slow fading can be found, for instance, in Parsons, The Mobile Radio Propagation Channel, 2nd Edition, Wiley.

The minimum received power on the surface of the Earth is measured at the output of an isotropic 0 dBic receiver antenna for any elevation angle equal or above 5°

The Galileo protection requirements are those provided in the Preliminary Draft Revision of Recommendation ITU-R M.1902-1, 4C/109-Annex5

The maximum lower hemisphere gain value applies for 5° elevation

Signal acquisition is performed using the E1-BC signal. The E1-BC signal is in the 1 559-1 610 MHz RNSS frequency band. Further details for these signals are provided in Annex 3 (Galileo) of Recommendation ITU-R M.1787. Appropriate acquisition threshold are provided in Recommendation ITU-R M.1903 Annex 2, Table 2-2, “High-precision” column.

Minimum receiver antenna gain at 5 degrees elevation angle is –5,5  dBi.

Authorizations in this frequency range are only granted on an individual basis after evaluation by OFCOM (CH).

https://www.gsc-europa.eu/sites/default/files/sites/all/files/Galileo_HAS_Info_Note.pdf

https://www.euspa.europa.eu/newsroom/news/euspa-launches-high-accuracy-service-call

JRC125180, JRC Testing and Demonstration Hub for the EU GNSS Programmes, 2021, ISBN 978-92-76-39185-2