| WGSE PT45 meeting |  | Doc. SE45(18)xxx |
| --- | --- | --- |
| Maisons-Alfort, France, 02-03 October 2018 |  |  |
| Date issued: | 2018.09.25 |  |
| Source: | HPE |  |
| Subject: | Calculation of the total and active populations per time zone |  |
| N Group membership required to read? (Y/N) |  |  |
| Summary: |  |  |
| This contribution provides calculations of the world’s total population per time zone and the proportion of the population that is active, i.e. awake during a given hour, for example the busy hour. |  |  |
| Proposal: |  |  |
| To include this study and apply its results in the respective sections of the draft ECC report on 6 GHz RLAN. |  |  |
| Background: |  |  |
| SE45 studies the conditions of compatibility with incumbent services for RLANs deployed in the band 5925 MHz – 6425 MHz. This document is submitted to SE45 in response to contribution SE45(18)071 “Preliminary results for sharing studies between RLANs and FSS”. |  |  |

# Busy Hour Time zoneconsiderations

In contribution SE45(18)071R1 [1] a “Time zone adjustment factor” was proposed, the reasoning being that when a busy hour population is distributed over a large geographical area, the proportion of active inhabitants will vary over the different time zones.

In this document [1], it is assumed that in time zones UTC to UTC+3 the entire population is active, in the UTC+4 zone 70% are active, in the UTC+5 and UTC+6 zones 50% of the population are active, and in zones UTC+7 to UTC+12 10% of the population are active.

In the contribution it is acknowledged that that countries belonging to a time zone are not considered independently, but the adjustment factor is applied by longitude interval, with each time zone being 15° (longitude) wide. The result of this calculation is shown in Table 1.

Table 1: Total and ”active” population per time zone as presented in SE45(18)071

| Continent | Population | Population with TZ adjustment factor |
| --- | --- | --- |
| Africa | 1.522.250.000 | 1.521.133.027 |
| Asia | 4.799.910.000 | 1.562.689.523 |
| Europe | 742.544.000 | 715.780.431 |
| Oceania | 45.075.000 | 4.416.116 |
| North and Central America | 623.246.000 | 230.612.386 |
| South America | 452.588.000 | 347.825.348 |
| Total | 8.185.613.000 | 4.382.456.831 |

Considering that the busy hour generally starts at 8 p.m. the assumptions on population activity made above are not consistent with typical human activity patterns.  In [2] it was shown that transitions between active and non-active times are not as smooth as assumed above but rather steep (see Figure 1).

Figure 1: Sleep: Daily rhythm of persons aged 20 to 74 on weekdays [2]

While activity patterns vary between countries this variation is essentially limited to the hour that people wake up and go to bed. These hours vary only by up to one hour, not only within Europe (as shown above) but globally (see Figure 2).

Figure 2: Wake and bed times in 20 surveyed countries [3]

Based on the above information we calculated the number of persons that are active globally at the time of the busy hour in a specific time zone. As reference zone and time we chose UTC and 8 p.m.

It must be noted that ”active” here does not mean that people are actively using RLAN or the Internet but simply that they are not sleeping.

As first step we determined the global population distribution by time zone.

# Population distribution by time zone

## Methodology

A list of the world’s countries and the corresponding time zone information was obtained from [4]. There are, however, several countries representing a considerable share of the world’s population that span several time zones. These are: Australia, Brazil, Canada, Democratic Republic of the Congo, France, Indonesia, Kazakhstan, Kiribati, Mexico, Federated States of Micronesia, Russia, and the United States of America.

Owing to its overseas territories, France is the country with the largest number of time zones. As the majority of the countries population is concentrated in the UTC+1 zone, the other territories have not been considered in this calculation.

For all other multi-time zone countries with the exception of Micronesia and Kiribati detailed information on population distribution per time zone was collected from a variety of sources [5] [6] [7] [8] [9] [10] [11] [12] [13] [14] [15] [16] [17] [18]. The two aforementioned countries which both span two time zones were placed in the UTC+11 and UTC+12 zones, resp. The resulting error is negligible; overall, more than 99.9% of the global population have been accounted for in their actual time zones.

The total and active population figures shown below were calculated for the year 2025, based on the latest available forecasts provided by the United Nations [19]. The following modfications of the UN source data were made:

- EU Member State Cyprus was moved from the ’Asia’ to the ’Europe’ category

- The population of Russia in time zones UTC+6 and beyond was moved from the ’Europe’ to the ’Asia’ category

The active share of the population for each hour of the day was derived from [2] and applied to the populations in the various time zones. As stated above UTC was chosen as reference time zone and  08:00 p.m. as the reference local time.

The resulting values for total and active populations per time zone are presented in Table 2.

Table 2: Total and active population per time zone

| Time zone | Total population | Active share of population | Active (awake) population |
| --- | --- | --- | --- |
| UTC-11:00 | 321.811 | 93% | 299.284 |
| UTC-10:00 | 1.643.483 | 99% | 1.627.048 |
| UTC-09:00 | 617.375 | 99% | 611.201 |
| UTC-08:00 | 64.341.715 | 99% | 63.698.298 |
| UTC-07:00 | 44.143.670 | 99% | 43.702.234 |
| UTC-06:00 | 276.929.769 | 99% | 274.160.472 |
| UTC-05:00 | 316.882.373 | 99% | 313.713.549 |
| UTC-04:30 | 77.428.606 | 99% | 76.654.320 |
| UTC-04:00 | 73.524.210 | 99% | 72.788.968 |
| UTC-03:30 | 548.421 | 99% | 542.937 |
| UTC-03:00 | 260.247.765 | 99% | 257.645.287 |
| UTC-02:00 | 3.085 | 99% | 3.054 |
| UTC-01:00 | 601.854 | 99% | 595.835 |
| UTC | 304.449.308 | 99% | 301.404.815 |
| UTC+01:00 | 898.136.074 | 95% | 853.229.271 |
| UTC+02:00 | 607.728.530 | 80% | 486.182.824 |
| UTC+03:00 | 666.518.917 | 35% | 233.281.621 |
| UTC+03:30 | 86.729.781 | 20% | 17.345.956 |
| UTC+04:00 | 43.615.175 | 17% | 7.414.580 |
| UTC+05:00 | 310.916.346 | 10% | 31.091.635 |
| UTC+05:30 | 1.473.178.946 | 3% | 44.195.368 |
| UTC+05:45 | 31.813.598 | 2% | 636.272 |
| UTC+06:00 | 204.262.928 | 2% | 4.085.259 |
| UTC+06:30 | 57.001.494 | 2% | 1.140.030 |
| UTC+07:00 | 435.985.694 | 2% | 8.719.714 |
| UTC+08:00 | 1.686.163.103 | 4% | 67.446.524 |
| UTC+09:00 | 212.898.187 | 10% | 21.289.819 |
| UTC+09:30 | 2.150.802 | 17% | 365.636 |
| UTC+10:00 | 35.652.955 | 40% | 14.261.182 |
| UTC+11:00 | 1.357.748 | 70% | 950.423 |
| UTC+12:00 | 6.407.310 | 82% | 5.253.994 |
| Total | 8.182.201.033 | 39% | 3.204.337.410 |

It shows that at 3.2 billion the resulting number of active persons is approximately 1.2 billion below that calculated in SE45(18)071R1 [1] (see Table 1).

The main difference lies in the considerably lower activity levels in Asia and Africa at the time of the UTC busy hour (see Table 3).

Table 3: Total and active population by continent

| Region | Population (2025) | Active (awake) population |
| --- | --- | --- |
| Africa | 1.517.706.140 | 1.123.644.338 |
| Asia | 4.825.636.804 | 403.547.494 |
| Europe | 721.026.895 | 594.889.828 |
| Latin America | 691.493.304 | 684.578.371 |
| Northern America | 382.428.768 | 378.604.480 |
| Oceania | 43.909.122 | 19.072.898 |
| Grand Total | 8.182.201.033 | 3.204.337.410 |

Globally, the number of active persons reaches its peak of 7.8 billion when the time is 20:00 in the UTC+7 time zone.

# References

| [1] | France, »SE45(18)071R1 Preliminary results for sharing studies between RLANs and FSS,« 2018. |
| --- | --- |
| [2] | Eurostat, »How Europeans spend their time, Everyday life of women and men,« 2004. |
| [3] | O. J. Walch, A. Cochran og D. B. Forger, »A global quantification of sleep schedules using smartphone data,« Science Advances, årg. 2, nr. 5, 6 May 2016. |
| [4] | The Guardian, »Time zones around the world, in every country,« [Online]. Available: https://www.theguardian.com/news/datablog/2009/sep/14/time-zones-countries-world-gmt. |
| [5] | Countries-of-the-World.com, »US time zones by state,« [Online]. Available: https://www.countries-ofthe-world.com/time-zones-usa.html. |
| [6] | Unted States Census Bureau, »State Population Totals and Components of Change: 2010-2017,« 08 05 2018. [Online]. Available: https://www.census.gov/data/tables/2017/demo/popest/state-total.html. |
| [7] | GeoLounge, »Which Country Has the Most Time Zones?,« 18 March 2016. [Online]. Available: https://www.geolounge.com/country-time-zones/. |
| [8] | Wikipedia, »List of federal subjects of Russia by population,« [Online]. Available: https://en.wikipedia.org/wiki/List_of_federal_subjects_of_Russia_by_population. [Senest hentet eller vist den 04 09 2018]. |
| [9] | TimeTemperature.com, »RUSSIA TIME ZONES - RUSSIA CURRENT TIMES,« [Online]. Available: https://www.timetemperature.com/europe/russia_time_zones.shtml. [Senest hentet eller vist den 04 09 2018]. |
| [10] | MetricMaps, »Canada Population by Time Zone,« 05 January 2018. [Online]. Available: https://metricmaps.org/2018/01/05/canada-population-by-time-zone-2/. |
| [11] | Wikipedia, »Time in Brazil,« [Online]. Available: https://en.wikipedia.org/wiki/Time_in_Brazil. [Senest hentet eller vist den 04 09 2018]. |
| [12] | Statoid, »States of Mexico,« 26 July 2016. [Online]. Available: http://www.statoids.com/umx.html. |
| [13] | Statoids, »Provinces of Indonesia,« 01 November 2015. [Online]. Available: http://www.statoids.com/uid.html. |
| [14] | Wikipedia, »List of provinces of the Democratic Republic of the Congo,« [Online]. Available: https://en.wikipedia.org/wiki/List_of_provinces_of_the_Democratic_Republic_of_the_Congo. [Senest hentet eller vist den 04 09 2018]. |
| [15] | Wikipedia, »Ranked list of states and territories of Australia,« [Online]. Available: https://en.wikipedia.org/wiki/Ranked_list_of_states_and_territories_of_Australia. [Senest hentet eller vist den 04 09 2018]. |
| [16] | GreenwichMeanTime.com, »Australia,« [Online]. Available: https://greenwichmeantime.com/time-zone/australia/. [Senest hentet eller vist den 04 09 2018]. |
| [17] | Wikipedia, »Regions of Kazakhstan,« [Online]. Available: https://en.wikipedia.org/wiki/Regions_of_Kazakhstan. [Senest hentet eller vist den 04 09 2018]. |
| [18] | Wikipedia, »Time in Kazakhstan,« [Online]. Available: https://en.wikipedia.org/wiki/Time_in_Kazakhstan. [Senest hentet eller vist den 04 09 2018]. |
| [19] | United Nations Department of Economic and Social Affairs, »2018 Revision of World Urbanization Prospects,« 16 May 2018. [Online]. Available: https://www.un.org/development/desa/publications/2018-revision-of-world-urbanization-prospects.html. |

The original table did not include totals.