ECC Report 252

SEAMCAT

Handbook

Edition 3

Approved 29 April 2016 – Rev June 2017

About the CEPT and ECO

The European Conference of Postal and Telecommunications Administrations (CEPT) is an organisation where policy makers and regulators from 48 countries across Europe collaborate to harmonise telecommunication, radio spectrum and postal regulations.

The European Communications Office (ECO) is the Secretariat of the CEPT. It provides advice and support to CEPT to help it to develop and deliver its policies and decisions in an effective and transparent way. Its core duties are to provide a European centre of expertise in electronic communications, to contribute to the work of the three CEPT committees and to manage CEPT’s day-to-day activities.

ECO further supports CEPT member countries and other stakeholders providing a forum to debate and advance European communications policy for the benefit of all Europe’s citizens.

Acknowldegment

ECO would like to thank many participating companies and organisations to the SEAMCAT MoU, who made the creation of the SEAMCAT tool possible. Special thanks go to the active members of CEPT Working Group Spectrum Engineering (WG SE), SEAMCAT Management Committee (SMC) (1997-2002), and SEAMCAT Technical Group (STG) (since 2003), whose many contributions formed the main basis of the work in the SEAMCAT project in general and the content of this document in particular.

For further enquiries

Any enquiries to the material in this document as well to the functioning of the SEAMCAT software may be addressed to ECO at:

ECO / Project SEAMCAT

Nyropsgade 37, 4th floor

1602 Copenhagen, Denmark

Tel: +45 33 89 63 00

Fax: +45 33 89 63 30

E-mail: seamcat@eco.cept.org

TABLE OF CONTENTS

Quick Start in SEAMCAT	17

1	INtroduction	28

1.1	Background to spectrum engineering	28

1.2	Basic RF terminology	28

1.2.1	Receiver thermal noise	28

1.2.2	Noise figure and Noise factor	29

1.2.3	Receiver noise floor	29

1.2.4	Receiver sensitivity	29

1.2.5	Wanted signal: dRSS	30

1.2.6	Interfering signal: iRSS	30

1.2.7	Bandwidth correction factor	30

1.2.8	Desensitisation	30

1.2.9	Blocking	31

1.2.10	Adjacent channel selectivity (ACS)	31

1.2.11	In-band, out-of-band, spurious, unwanted emission	31

1.2.12	Co-channel	32

1.3	Monte-Carlo basics	32

1.4	Interference calculations in generic systems	34

1.4.1	Introduction	34

1.4.2	An illustration with C/I as interference criterion	34

1.4.3	Methodology associated to the interference criterion (C/I, C/(I+N), (N+I)/N, I/N)	36

1.4.4	Interference criteria relationship	37

1.4.5	Unwanted emissions	37

1.4.6	Receiver blocking	38

1.4.7	Intermodulation	39

1.4.8	Overloading	39

1.4.9	Combined interference mechanism	39

1.4.10	Interference calculation	39

1.5	Applicability of SEAMCAT to Spectrum Engineering Problems	40

1.6	Understanding radio jargon	40

1.7	SEAMCAT software	41

1.7.1	Installation using .jar package	41

1.7.2	SEAMCAT home directory and configuration panel 	41

1.7.3	Java Source code	42

1.7.4	Multi processing power in SEAMCAT	42

1.8	The SEAMCAT community	42

1.8.1	SEAMCAT Technical Group (STG)	42

1.8.2	SEAMCAT community – more than STG	43

1.9	Reference material and ECC/CEPT Reports	44

2	GeTting around in SEAMCAT	45

2.1	Quick buttons toolbar	45

2.2	‘Welcome to SEAMCAT’ window	45

2.3	Typical SEAMCAT work flow	46

2.4	Simulation workspace	46

2.4.1	Scenario workspace	46

2.4.2	Results Workspace	47

2.5	Create or update a simulation workspace  	47

2.6	Batch operation 	49

2.7	Running and stoping a simulation  	50

2.8	Pausing and RESUMING a simulation	51

2.9	Running from the command line	51

2.10	Consistency check 	51

2.11	Simulation control	52

2.12	Simulation results overview	54

2.13	Saving options in SEAMCAT	54

2.13.1	Saving workspace and batch 	54

2.13.2	Saving results – file extension	55

2.14	Simulation report 	55

2.15	Logging function for debugging	56

2.16	Play/replay an event 	58

2.17	Testing propagation models 	59

2.18	Testing distribution function 	61

2.19	Testing unwanted emission function 	61

2.20	Multiple vectors comparison 	62

2.21	Pocket calculator 	62

2.22	Online manual help content 	63

2.23	Report enhancementS and bugs 	63

2.24	Installing plugins into SEAMCAT	64

3	Example of runninG a simulation	66

3.1	Setting the scenario	66

3.2	Calculating the dRSS	67

3.2.1	Victim link	67

3.2.2	System to be a victim	67

3.2.3	Receiver	68

3.2.4	Transmitter	70

3.2.5	Positioning the VLT vs VLR	70

3.2.6	Selecting the propagation model	72

3.2.7	Calculating the dRSS by hand	73

3.2.8	Export/import your system to library	73

3.2.9	Selecting the victim in the Scenario	73

3.3	Calculating the iRSS	74

3.3.1	Interfering Links	74

3.3.2	System to be an interferer	75

3.3.3	Transmitter	75

3.3.4	Receiver	76

3.3.5	Positioning of the ILT vs ILR	76

3.3.6	Positioning of the VLR vs ILT	77

3.3.7	Calculating the iRSS by hand	78

3.3.8	Emission, reference, VLR bandwidth relationship and bandwidth correction factor	79

3.4	Launching a simulation	80

3.5	Extract results vectors (dRSS, iRSS etc…)	81

3.6	Calculating the probability of interference	82

3.6.1	Compatibility calculation mode	82

3.6.2	Translation calculation mode	83

3.6.3	Example of probability of interference calculation	83

4	Example of calculations	85

4.1	iRSSunwanted	85

4.2	iRSSblocking	86

4.2.1	User-defined mode	86

4.2.2	Sensitivity mode	87

4.2.3	Protection ratio	88

4.3	iRSSintermodulation	88

4.4	iRSSoverlaoding	89

4.5	Coverage radius (VLR-VLT)	92

4.6	Simulation radius (ILT-VLR)	94

4.7	Uniform distribution of ILT vs VLR	96

4.7.1	Uniform density mode - simulation radius calculation	96

4.7.2	“None” mode	100

4.8	Protection distance	101

4.9	Power Control	105

4.10	Antenna gain	108

5	Generic module	110

5.1	Generic system tab	110

5.1.1	System	110

5.1.2	General	110

5.2	Receiver	111

5.2.1	Receiver identification	111

5.2.2	Antenna pointing	112

5.2.3	Antenna patterns identification	113

5.2.4	Reception characteristics	113

5.2.5	Interference criteria	114

5.3	Transmitter	115

5.3.1	Transmitter identification	116

5.3.2	Transmitter antenna pointing	116

5.3.3	Antenna patterns identification	116

5.3.4	Emission characteristics	116

5.4	Transmitter to Receiver Path	118

5.4.1	Relative location	118

5.4.2	Coverage radius	120

5.4.3	Local environment	122

5.4.4	Propagation Model	124

6	Spectrum sensing and Cognitive Radio	125

6.1	Introduction	125

6.2	Simulating spectrum sensing	125

6.3	Input system parameters	126

6.4	Scenario parameters	127

6.5	Examples	127

6.5.1	All the channels are available	128

6.5.2	All the channels are blocked	129

6.5.3	Some of the channels are blocked and some are available	129

7	Cellular Network simulation	131

7.1	Introduction	131

7.2	CDMA overview	131

7.3	OFDMA overview	132

7.4	TDD vs FDD simulation	132

7.5	Cellular network positioning	132

7.5.1	System	132

7.5.2	System layout – reference cell selection	137

7.5.3	System layout preview	140

7.5.4	Mobile station	140

7.5.5	Base station	141

7.6	CDMA/OFDMA commonalities	142

7.6.1	Pathloss and Effective Pathloss	142

7.6.2	Measure interference from entire cluster	142

7.6.3	Wrap around feature and implementation	142

8	CDMA module	145

8.1	Introduction	145

8.2	CDMA system tab	145

8.2.1	System	145

8.2.2	General	145

8.3	General settings	146

8.3.1	CDMA general settings	147

8.3.2	Local environment	148

8.3.3	Receiver settings	148

8.3.4	Transmitter settings	148

8.3.5	CDMA uplink	149

8.3.6	CDMA downlink	150

8.3.7	CDMA capacity	150

8.3.8	Propagation Model	152

8.4	Positioning	152

8.5	CDMA Link level data	152

8.5.1	CDMA link level system mapping	152

8.5.2	CDMA DL Power Control Methodology (VOICE ONLY)	152

8.5.3	CDMA uplink Power Control Methodology in SEAMCAT (VOICE ONLY)	153

8.6	CDMA Downlink - simulation algorithm	153

8.6.1	Simulation Methodology	153

8.6.2	Traffic Generation	154

8.6.3	Soft Handover	155

8.6.4	Power Control	155

8.6.5	Success rate	158

8.7	CDMA Uplink - simulation algorithm	158

8.7.1	Power Control	158

8.7.2	Soft and Softer Handover	159

8.7.3	Voice Activity Factor	160

8.7.4	System loading	160

8.7.5	Outage calculation	161

8.7.6	CDMA UL cell selection	161

8.8	Capacity for CDMA system	161

8.8.1	CDMA Uplink capacity finding	161

8.8.2	CDMA Downlink capacity finding	162

9	OFDMA module	164

9.1	Introduction	164

9.2	OFDMA system tab	164

9.2.1	System	165

9.2.2	General	165

9.3	General settings	165

9.3.1	OFDMA general settings	167

9.3.2	Local environment	168

9.3.3	Receiver settings	168

9.3.4	Transmitter settings	168

9.3.5	OFDMA uplink	168

9.3.6	OFDMA UE Distribution	169

9.3.7	OFDMA downlink	169

9.3.8	OFDMA Capacity	169

9.3.9	Propagation model	170

9.4	Positioning	170

9.5	Link-to-system level mapping	170

9.6	Achieved bit rate	170

9.7	DL C/I calculation	170

9.8	UL C/I calculation	172

9.9	UL calculation of the UE frequencies	173

9.10	OFDMA UL power control	173

9.11	Pathloss Correlation	174

10	Scenario	176

10.1	Setting the scenario	176

10.2	Multiple Interferers generation	176

10.2.1	Generation of multiple interferer links with different systems	176

10.2.2	Auto-generation of multiple interfering links 	177

10.2.3	Generation of interferers with the same characteristics	180

10.3	Interfering Link Transmitter to Victim Link Receiver Path (ILT -> VLR)	181

10.3.1	Relative positioning of interfering link (Generic system)	182

10.3.2	Relative positioning of interfering link (Cellular system)	188

10.3.3	Interferers density	189

10.3.4	Pathloss correlation	189

10.3.5	Propagation Model	189

10.4	Interfering link transmitter to victim link transmitter path (spectrum sensing)	190

10.4.1	Spectrum sensing characteristics	190

10.4.2	Detection of threshold	191

10.4.3	Probability of failure	192

10.4.4	Adjacent channel scenario – e.i.r.p. max. in-block limit	192

10.4.5	Propagation Model	193

11	Event Processing	195

11.1	Event processing concept	195

11.2	Event processing GUI	195

11.3	Loading an EPP to a simulation	196

11.4	Configuring an EPP	197

12	Results	198

12.1	Scenario outline	198

12.1.1	Simulation Summary	198

12.1.2	Simulation status	198

12.1.3	Generic system outline	198

12.1.4	Cellular system outline	199

12.2	Overview of output results	201

12.3	Generic model Results	202

12.3.1	Generated signals results	202

12.3.2	Calculated radius	202

12.3.3	Intermodulation results	202

12.3.4	Overloading results	203

12.4	Spectrum sensing results	204

12.4.1	sRSS vector	204

12.4.2	WSD frequency	206

12.4.3	WSD e.i.r.p.	206

12.4.4	Victim frequency vector	207

12.4.5	Average e.i.r.p. per frequency	207

12.4.6	Average active WSDs per frequency vector	207

12.5	CDMA output results	208

12.5.1	CDMA capacity finding	208

12.5.2	CDMA results	209

12.5.3	CDMA results for cell selection algorithm	211

12.6	OFDMA output results	211

12.7	EPP results	211

12.8	Cellular Structure	212

12.8.1	Plot configuration	213

12.8.2	Summary of event #n	214

12.8.3	Inspect selected element	215

12.9	Interference calculations	218

12.9.1	Introduction	218

12.9.2	Interference calculation GUI	219

12.9.3	Interference Calculation Engine Control	221

13	Library of scenario elements	222

13.1	Library overview	222

13.2	Import / export a library element in/from a workspace  	222

13.3	Edit and share library element 	224

13.4	Library elements	226

13.4.1	System elements	226

13.4.2	Spectrum emission mask elements	227

13.4.3	Receiver blocking mask elements	228

13.4.4	Receiver elements	229

13.4.5	Transmitter elements	230

13.4.6	CDMA Link level data	231

13.5	Plugins Library	233

13.5.1	Antenna plugin elements	233

13.5.2	Coverage radius plugin elements	234

13.5.3	Propagation model plugin	234

13.5.4	Event Processing Plugin	235

14	Plugin development	236

14.1	SEAMCAT source code	236

14.2	Background and Terminology	236

14.2.1	Understanding abstraction level	236

14.2.2	Simulation & Engine	237

14.2.3	Packages	239

14.2.4	System & Results	240

14.2.5	Debug your code	243

14.3	Commonalities to puglins	245

14.3.1	@Config – Annotation of Input interface	246

14.3.2	Consistency check - consistencyCheck()	247

14.3.3	Logger functionalities for plugins	253

14.4	Propagation Model Plugin (PMP)	254

14.5	Antenna Plugin (AP)	256

14.5.1	AP – static pre-set of the antenna pattern	256

14.5.2	AP – dynamic calculation of antenna gain	258

14.6	Coverage Radius Plugin (CRP)	260

14.7	Event Processing Plugin (EPP)	262

14.7.1	Overview	262

14.7.2	Core EPP	262

14.7.3	Post processing UI as optional part of the EPP	264

14.8	FAQ on plugin development	268

ANNEX 1: Data elements	270

ANNEX 2: Interference Calculation Algorithm	277

ANNEX 3: Relationship between C/I, C/(N+I), (N+I)/N and I/N	281

ANNEX 4: dRSS calculation	283

ANNEX 5: iRSS calculation	285

ANNEX 6: sRSS calculation	291

ANNEX 7: Unwanted emissions mask	292

ANNEX 8: Receiver selectivity and blocking attenuation	302

ANNEX 9: : Examples of deriving parameter values from radio standards	313

ANNEX 10: Distribution	317

ANNEX 11: : Antenna	323

ANNEX 12: : Relative positioning	334

ANNEX 13: Coverage and simulation radius	341

ANNEX 14: : Power control function (for non-CDMA only)	345

ANNEX 15: : Cellular algorithms	347

ANNEX 16: : Spectrum sensing algorithm	366

ANNEX 17: : Propagation models	370

ANNEX 18: Plugin: Compilation and debuging	420

ANNEX 19: : Consistency check (examples)	441

ANNEX 20: : Debug information	443

ANNEX 21: : Reference material and workspaces ECC/CEPT reports	445

ANNEX 22: : dB conversion table	446

ANNEX 23: How to make the most of the SEAMCAT figures	447

ANNEX 24: : Example - TETRA MS interferers the BS of the analogue FM PMR	449

1.	SYSTEMS:	453

2. SCENARIO	457

2.1. VICTIM SYTEM	457

3. RESULTS	458

ANNEX 25: : Answers to the Tetra vs FM PMR example	459

1. SYSTEMS:	459

ANNEX 26: : List of references	464

ANNEX 27: : Glossary	466

Index	468

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| 3GPP | 3rd Generation Partnership Project |
| ACS | Adjacent Channel Selectivity |
| ACIR | Adjacent Channel Interference Ratio |
| ACLR | Adjacent Channel Leakage Ratio |
| AP | Antenna Plugin |
| BS | Base Station |
| CDF | Cumulative Distribution Function |
| CDMA | Code Division Multiple Access |
| CDMAE | CDMA Engine |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| CRP | Coverage Radius Plugin |
| CR | Cognitive Radio |
| DL | Downlink |
| dRSS | desired Received Signal Strength |
| DTH | Direct To Home |
| ECC | Electronic Communications Committee |
| ECO | European Communication Office |
| EPP | Event Processing Plugin |
| ETSI | European Telecommunications Standards Institute |
| E-UTRA | Evolved Universal Terrestrial Radio Access |
| FDD | Frequency Division Duplex |
| GSM | Global System for Mobile communication |
| GUI | Graphical User Interface |
| ICE | Interference Calculation Engine |
| ILR | Interfering link receiver |
| ILT | Interfering link transmitter |
| iRSS | interfering Received Signal Strength |
| It | Interefering link transmitter |
| ITU-R | International Telecommunication Union – Radiocommunication Sector |
| LTE | Long Term Evolution |
| MCL | Minimum Coupling Loss |
| NF | Noise Figure |
| OFDM | Orthogonal Frequency-Division Multiplex |
| OFDMA | Orthogonal Frequency-Division Multiple Access |
| PDF | Probability Distribution Function |
| PMP | Propagation model plugin |
| RF | Radio Frequency |
| Rx | Receiver |
| SEAMCAT | Spectrum Engineering Advanced Monte Carlo Analysis Tool |
| SINR | Signal to Infererence plus Noise Ratio |
| sRSS | sensing Received Signal Strength |
| STG | SEAMCAT Technical Group |
| sws | SEAMCAT WorkSpace |
| TDD | Time Division Duplex |
| TETRA | Terrestrial Trunked Radio |
| TS | Terminal Station |
| Tx | Transmitter |
| UE | User Equipment |
| UI | User Interface |
| UMTS | Universal Mobile Telecommunications System |
| VLR | Victim Link Receiver |
| VLT | Victim Link Transmitter |
| WG SE | Working Group Spectrum Engineering |
| WSD | White Space Device |

Executive summary

Introduction

SEAMCAT (Spectrum Engineering Advanced Monte Carlo Analysis Tool) is a statistical simulation model that uses a method of analysis called Monte Carlo to assess the potential interference between different wireless systems such as broadcasting, point to point, point to multipoint, radar, Mobile networks, aeronautical and satellites.

This ECC Report is intended as a handbook on SEAMCAT for new users; to guide them from installation to carrying out advanced simulations, therefore the users will be refered to as “you”. It is also intended as a reference for more experienced users by providing working guidance on different scenarios and advanced functions.

SEAMCAT is a radio spectrum system oriented software tool which allows you to build your own libraries (such as antennas, spectrum masks, propagation models, radio systems) or use those provided by other users to ease the effort to build complete scenarios for investigation. SEAMCAT is distributed with a predefined set of libraries, so that you do not need to reinvent the wheel every time you have studies to perform.

SEAMCAT is designed so that play/replay feature and plugins can help radio spectrum engineers to understandhow simulations are run or to extend algorithms.

The fundamental concept of SEAMCAT is to provideeasy sharing of radio spectrum compatibility studies (parameters, methods and results) within the CEPT by using a common interface. SEAMCAT is an open source tool and therefore freely available. SEAMCAT is widely used within the CEPT project teams, and also outside the CEPT globally by regulators, industry and academia. Scenarios can be studied, recreated and reconsidered by any of the interested parties using SEAMCAT. The only area for discussion arethe input parameters and the scenarios under investigation. Studies for investigation may cover and are not limited to:

Sharing and compatibility studies on different equipment operating in the same or adjacent frequency bands;

Assessing the impact of using different systems transmit and receive masks;

Evaluation of limits applied e.g. for unwanted emissions, blocking or intermodulation products.

SEAMCAT version 5and upward is a tool based on multicore processing allowing fast computation.

You can join the SEAMCAT community (Section ‎1.8) to contribute to the development of SEAMCAT and its libraries..

This ECC Report provides updated information compared to the material given in ERC Report 68 ‎[9] which specified the original SEAMCAT alorithms.

Methodology

The radio spectrum is a limited resource and can only be used optimally if compatibility is assured between radio systems located in the same or adjacent frequency bands. The important criterion for radio compatibility is the difference between the wanted/desired signal level (victim link system) and any interfering signal levels in the victim link receiver’s input. The most significant interference mechanisms are unwanted emissions from transmitters as well as blocking and intermodulation in the victim link receiver.

There are different approaches ‎[19] to analyse these criteria, such as the Monte Carlo method as provided by SEAMCAT, and the Minimum Coupling Loss (MCL) method.

The Monte Carlo method is a statistical methodology to simulate processes by randomly taking values from a distribution. The term "Monte Carlo method" was coined in the 1940s by physicists working on nuclear weapon projects in the Los Alamos National Laboratory ‎[11]. This approach was used for solving statistical problems and the code name given to the work was Monte Carlo.

A Monte Carlo simulation as used in this report is a statistical technique based upon the consideration of many independent events in time, space and frequency. For each event, or simulation trial, a scenario is built up using a number of different random variables that define the systems to be simulated (e.g. the location of the interferers with respect to the victim, the victim link's wanted signal strength, the channels/frequnecies of the victim and interferer use). If a sufficient number of simulation trials are considered then the probability of a certain event occurring can be evaluated with a high level of accuracy by taking the average result over all trials.

The flexibility in the model and its’ supporting software implementation allow a quick yet reliable consideration of spatial and temporal distributions of the received signals and the resulting statistical probability of interference in a wide variety of scenarios.

Structure of this ECC Report

Section ‎1 to ‎2 of this document provides a general overview and description of the software.

Section ‎3 to ‎4 guides the beginner, step by step, on how to use SEAMCAT providing details about the software and the graphical interface.

Section ‎5 to ‎9 introduces and defines the various radio system input parameters that can be simulated in SEAMCAT.

Section ‎10 to ‎12 presents the various tabs, scenarios, event processing and results that are used when performing a spectrum sharing or compatibility study.

Section ‎13 addresses the library functionaility of SEAMCAT for simulating common transmitters, receivers, masks, antennas, propagation models, etc

Section ‎14 guides you through the plugin development that is made available to you so that you can develop your own algorithms and libraries compatible with the SEAMCAT run time environment.

‎ANNEX 1: to ‎ANNEX 27: provides more details of the algorithms, interfaces and principles of sharing studies.

# Quick Start in SEAMCAT

Download and Installation of SEAMCAT from www.seamcat.org

Platform independent (jar package)

(Windows, Linux, MAC)

Creating, opening and saving a workspace scenario (.sws file)

new

open

save

Select the .sws file

Workspace scenario (.sws file)

Set the characteristics of the systems to investigate (generic Tx, Rx and path between Tx/Rx or cellular general settings, positioning)

Set the scenario, i.e. select the systems to use for the victim and interferer(s) as well as the path between the victim and the interfer(s), number of events, and option to run in debug mode

The EPP (Event Processing Plugin) allows you to process intermediary SEAMCAT results for each simulated event

Worskspace storing the scenario only

Fields are editable

File saved as .sws (SEAMCAT Workspace Scenario)

Systems that can be simulated in SEAMCAT

Generic Systems

Cellular Systems

Land Mobile

Short Range Devices

Terrestrial broadcasting

Point-to-Point and Point-to-Multipoint fixed systems

Earth based stations (e.g. DTH receivers) of satellite systems

CDMA (UMTS, CDMA 2000x, etc)

OFDMA LTE

Victim Link

Transmitter

(VLT)

Victim Link

Receiver

(VLR)

Interfering

Link

Transmitter

(ILT)

dRSS

iRSS

Victim System

Interferer System(s)

Interfering

Link

Receiver

(ILR)

Generic System Parametrisation

Receiver characteristics: (sensitivity, bandwidth, antenna, blocking response, interference criteria, etc.)

Transmitter characteristics (power, antenna, spectrum emission mask, etc.)

Position and propagation model information between the Tx and the Rx

Note: The frequency is overwritten at the “Scenario” tab level

Import a system from library

Export a system to library

Duplicate a system

Delete a system

CDMA/OFDMA System Parametrisation

CDMA/OFDMA settings (Rx/Tx settings, links, capacity, local environments, propagation model, pathloss correlation, etc.)

Cellular positioning (System settings, grid layout, mobile, base station)

Note: The frequency is overwritten at the “Scenario” tab level

Victim

Receiver

(VLR)

Interfering

Link

Transmitter

(ILT)

iRSS

Interfering link

Scenario Parametrisation

Select one victim from a list defined under the “system” tab

Select interferer system(s) from a list defined under the “system” tab

Set the path characteristics between the victim and interfering system (Relative positioning, density of interferers, pathloss correlation, propagation model)

Frequency in “Scenario” overwrites settings in “Systems”

Add an interfering link to the scenario

Duplicate an interfering link

Delete a link

Generate multiple interfering links

Change the system type

Victim System

Interferer System(s)

Workspace Results (.swr file)

Outline of the simulation with summary of the dRSS and iRSS calculation, graphical overview of the Tx/Rx position for the victim and interferer(s)

List of all the results

Replay featurefor each event

Worskspace storing the results

Read only scenario

File saved as .swr (SEAMCAT Workspace Result)

Interference calculation engine

Workspace Results - Outline

Summary of the dRSS and iRSS results

Progress bar to track the current task status and the system memory

Display the victim and interfering link element for each snapshot

Quick overview of the dRSS and iRSS during run time

Workspace Results - Results

Play/Replay feature for one event

Output vector from the SEAMCAT simulation

Input parameters used for these specific simulation results (read only)

Statistics on the simulation process, time consumptions, number of processor used

Display any output results of an EPP

Workspace Results – Interference Calculations

Compatibility: Probability of interference as a single result

Translation: Probability of interference as a function of parameters

Control of the Interference Calculation Engine

Compatibility: Probability of interference

Translation: Probability of interference

as a function of parameters

17.5 % of probability of interference

Workspace Results – Interference Calculations

Compatibility: Probability of interference as a single result

Translation: Probability of interference as a function of parameters

Control of the Interference Calculation Engine

Compatibility: Probability of interference

Translation: Probability of interference

as a function of parameters

17.5 % of probability of interference

Batch environment: Run multiple workspaces simultaneously

Start a new batch

Set of workspacesincluded in the batch

Batch control: add new or existing workspace, export the workspace (save), delete and duplicate

Open a batch

Save a batch

Close a batch

Run a batch

SEAMCAT batch file

Batch to be saved with workspace scenario (.sbj file)

Batch results

Launch the analysis of the interference calculation

SEAMCAT Batch Result file

To be saved as .sbr file

List of main Interference results

Workspace results to be saved as part of the batch result file

Results: dRSS and iRSS vectors

Compare vectors

Select the vectors to display from a list

from one workspace

from many workspaces

from external files

Libraries: Edit, Select

Built in system specific libraries

Built in and plugin libraries

Install plugins

Edit libraries

Library selection from the workspace

Libraries: Import, Export

Import system components from library to workspace

Export from workspace to library

Import system from library to workspace

Export system from workspace to library

Import/Export libraries

Import/Export systems

Import/Export systems components

Plugin generation

Large range of input parameters to implement

No limitation on the number of inputs

Configuration of  various tyoes of plugin

Menu overview

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

| ID | Description |
| --- | --- |
| 1 | Create a new workspace |
| 2 | Create a new batch job |
| 3 | Open any SEAMCAT workspace file (.sws, .swr) or batch job (.sbj, .sbr) |
| 4 | Save a workspace or a batch file under the current working directory |
| 5 | Save as a workspace or a batch with a selectable filename |
| 6 | Save all the workspaces, batch files at once |
| 7 | Close the selected workspace or batch |
| 8 | Import library |
| 9 | Export library |
| 10 | Access the configuration of SEAMCAT (Home selection, Server settings) |
| 11 | History of open files |
| 12 | Exit SEAMCAT (this will close the SEAMCAT application) |
| 13 | Allows the tool bar to be displayed |
| 14 | Allows the status bar to be displayed |
| 15 | System library |
| 16 | Spectrum emission mask library |
| 17 | Receiver blocking mask library |
| 18 | Receiver library (Generic system only) |

26

27

28

29

30

31

32

33

34

35

36

37

| ID | Description |
| --- | --- |
| 19 | Transmitter library (Generic system only) |
| 20 | CDMA link level data library |
| 21 | Antenna plugin library |
| 22 | Coverage radius plugin library |
| 23 | Propagation model plugin library |
| 24 | Event processing plugin library |
| 25 | Install .jar plugins to SEAMCAT environment |
| 26 | To detect erroneous or inconsistent values in the scenario definition before running a simulation |
| 27 | Generate a SEAMCAT simulation report |
| 28 | Start a workspace or a batch simulation |
| 29 | Check the built-in distribution function |
| 30 | Check the built-in and plug-in propagation model |
| 31 | Check the unwanted emission function |
| 32 | Access pocket calculator feature |
| 33 | Plot and compare multiple vectors (results from current and other workspaces) |
| 34 | Help contents – redirect to the on-line manual |
| 35 | Procedure to report an error |
| 36 | About SEAMCAT |
| 37 | Enable/disable the display of the welcome window |

# INtroduction

## Background to spectrum engineering

The radio spectrum is a limited resource and can only be used optimally if compatibility is assured between wireless systems located in the same or adjacent frequency bands. For example, an important criterion for radio compatibility is the difference between the wanted and unwanted signal levels in the victim link receiver input. This parameter can be used to derive a separation in physical position or frequency between the victim and interfering systems . Considering only the adjacent bands, the most significant interference mechanisms are unwanted emissions from interfering transmitters as well as blocking and intermodulation in the victim link receiver.

A statistical simulation model based on the Monte-Carlo method has been developed within the CEPT Working Group Spectrum Engineering, named SEAMCAT® (Spectrum Engineering Advanced Monte-Carlo Analysis Tool). This software implementation allows quick yet reliable considerations of spatial distributions of the received signals and the resulting statistical probability of interference in a wide variety of scenarios. By adapting the operational conditions of radio systems with respect to the probability of exceeding the protection criterion, the most efficient use of the radio spectrum can be identified.

SEAMCAT is software that simulates radio spectrum systems, allowing the implementation of user customized libraries and the use of those provided by other users to ease the effort to build complete scenarios. These libraries may contain predefined antenna patterns, spectrum emission masks, propagation models, complete radio systems, etc.

SEAMCAT version 5 and upward provides multicore processing allowing for fast computations.

It is possible to join the SEAMCAT community to contribute to the development of the software and libraries, as explained in Section ‎1.8.

## Basic RF terminology

The following terminology has been developed to support the SEAMCAT simulation tool. While it is broadly in line with what is in use in ITU-R, ETSI, and CEPT, it may be expected that for other needs or documents, ETSI and CEPT/ECC will use different definitions, for example definitions more focused on specific equipment characteristics or aspects of such products.

### Receiver thermal noise

The thermal noise (in Watts) expressed in power level is defined as:

N0 (Watts) = kBTB	(Eq. 1)

where:

kB is the Boltzmann's constant 1.38 x 10-23 in joules per kelvin (J/K),

T is the receiver absolute temperature in Kelvin (K),

B is the bandwidth in Hertz (Hz) over which the noise is measured.

It can be seen that the noise power of eq1 is dependent on the temperature and on the bandwidth. This figure is then normally expressed in terms of dBm and is defined as:

N0 (dBm) = -173.977 + 10*log10(systemBandwidth (Hz))	(Eq. 2)

The noise power spectral density of -173.977 dBm/Hz implies a temperature of 290 K.

### Noise figure and Noise factor

Noise figure (NF) and noise factor (F) are measures of degradation of the signal-to-noise ratio (SNR), caused by components in a radio frequency (RF) signal chain. It is a number by which the performance of an amplifier or a radio receiver can be specified, with lower values indicating better performance.

The noise factor is defined as the ratio of the output noise power of a device to the portion thereof attributable to thermal noise. The noise factor is thus the ratio of actual output noise to that which would remain if the device itself did not introduce noise, or the ratio of input SNR to output SNR.

The noise figure is simply the noise factor expressed in decibels (dB).

NF = 10*log10(F) 	(Eq. 3)

### Receiver noise floor

Any practical measurement will be subject to some form of noise or unwanted signal (thermal noise or interfering signals). The noise floor limits the smallest measurement that can be taken with certainty since any measured amplitude cannot on average be less than the noise floor.

The noise floor (N) is the level of noise introduced by the receiver system below which the signal that is being captured cannot be isolated from the noise, and is defined in dBm as:

N (dBm) = N0 (dBm) + NF (dB) 	(Eq. 4)

or in linear domain:

N (Watts) = 10^((-173.977 + 10*log10(systemBandwidth (Hz)) + NF)/10)	 (Eq. 5)

As an example, in SEAMCAT, for a receiver with a noise figure of 9 dB and a bandwidth of 5 MHz, the noise floor will be

-173.977 dBm/Hz +60 dB/MHz +10*log10(5 MHz) = -98 dBm

Another example is that for a noise figure of 4 dB and a bandwidth of 200 kHz with thermal noise = kTB = -121 dBm, the noise floor is -117 dBm.

In SEAMCAT, the input is the distribution of the strength of the noise floor. This parameter is used for the probability calculation when the criteria is C/(N+I), (I/N) or (I+N)/N.

### Receiver sensitivity

The sensitivity of a receiver is normally taken as the minimum input signal (Smin) required to produce an output signal with a specific signal-to-noise (S/N) ratio. S/N is a required minimum ratio, if N is increased, then S must also be increased to maintain the S/N ratio. The threshold value is chosen high enough above the mean noise level so that the probability of random noise peaks exceeding the threshold, and causing false alarms, is acceptably low. It is defined (in log domain/dB) as

Sensitivity = Noise Floor + Receiver protection ratio	(Eq. 6)

In SEAMCAT, the following equation (in log domain/dB) is applied:

Sensitivity = Noise Floor + C/(N+I) 	(Eq. 7)

Where C/(N+I) is the carrier (or signal) to interference plus noise ratio as input to SEAMCAT. The S/N is equal to the C/(N+I) in the absence of any interferer.

In SEAMCAT, this is used in the calculation of the receiver attenuation in Sensitivity mode. See Annex ‎A8.5 for further details.

### Wanted signal: dRSS

The victim’s wanted signal strength also called desired Received Signal Strength (dRSS), corresponding to the carrier level (C), is calculated as a simple link budget between the victim link receiver (VLR) and the victim link transmitter (VLT) as described in ‎ANNEX 4:.

### Interfering signal: iRSS

The interfering Received Signal Strength (iRSS), corresponding to the interference level (I), is calculated as a link budget between the VLR and the interfering link transmitter (ILT) as described in ‎ANNEX 5:.

The various interference mechanisms resulting in different iRSS (i.e. unwanted, blocking and intermodulation) are decribed in Section ‎1.4.5.

### Bandwidth correction factor

When the bandwidth of the interferer and the victim are different, SEAMCAT automatically applies a bandwidth correction factor to calculate the unwanted emission power for a specific bandwidth.

The following example introduces an interferer transmitting 2 W. This is equivalent to 33 dBm (see conversion in Table 100). The amount of energy that a VLR receives in its bandwidth can be derived according to these two cases illustrated in Figure 1.

Figure 1: Illustration of the bandwidth correction factor

Further examples of correction bandwidth can be found in section ‎3.3.8.

### Desensitisation

Desensitisation (D) of the receiver in the presence of an interfering signal, given in dB, corresponds to the ‘noise rise’ or ‘noise augmentation’ due to the interfering signal and is derived by the following equation in dB:

D = 10.log10[(i + n)/n]	(Eq. 8)

and is equivalent in the linear domain to:

D = 10.log10[(10(N/10) + 10 (I/10))] – N	(Eq. 9)

To ensure proper operation, the receiver is designed to include a margin equal or lower than D which allows it to tolerate a certain level of interference (I) in the listened channel. This can be caused by co-channel and/or non-co-channel interference sources.

When running a radio network or a radio link, the objective is to maintain the signal to interference and noise ratio SINR equal to the sensitivity to noise ratio. This is illustrated in the following figure.

Figure 2: Desensitisation

An equivalent expression of the desensitisation, (N+I)/N, is expressed as follows in terms of interference to noise ratio I/N in dB.

I/N = 10.log10(10D/10 - 1)	(Eq. 10)

noting that in linear domain it is equivalent to:

i / n = [(i + n) / n] - 1 = 10D/10 – 1	(Eq. 11)

### Blocking

The term “Blocking” is used in SEAMCAT to describe the capability of the victim receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted input signal on a different frequency than the one of the wanted signal.

It has to be noted that most standards and specifications distinguish between the receiver selectivity given as Adjacent Channel Selectivity (ACS) for frequency offsets close to the centre frequency of the VLR, and Blocking (a term often associated with desensitisation) for frequency offsets far away from the centre frequency of the VLR. It is assumed that the receiver blocking performance is flat over the bandwidth of the interfering signal.

### Adjacent channel selectivity (ACS)

The receiver selectivity is often given as Adjacent Channel Selectivity (ACS) – i.e. when the concept of "channel" has been defined for the system being considered. As the wording suggests, this parameter defines the requirement in case one single ILT is set at the centre frequency of the (first) adjacent channel. The values for the second and third adjacent channel are occasionally also defined by a standard. However, standards generally implicitly define the interfering signal as of the same system type as the victim, i.e. using the same bandwidth and  generally the same or at least a similar modulation scheme.

### In-band, out-of-band, spurious, unwanted emission

In-band emission is understood as relating to the necessary bandwidth.

The ITU-R Radio Regulations define the followings in Nos. 1.152, 1.144, 1.145 and 1.146:

1.152 Necessary bandwidth: For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

1.144 Out-of-band emission: Emission on a frequency or frequencies immediately outside the necessary bandwidth which results from the modulation process, but excluding spurious emissions.

1.145 Spurious emission: Emission on a frequency or frequencies which are outside the necessary bandwidth and the level of which may be reduced without affecting the corresponding transmission of information. Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products, but exclude out-of-band emissions.

1.146 Unwanted emissions: Consist of spurious emissions and out-of-band emissions

These definitions are illustrated in the following figure (see also Figure 1 in ECC Recommendation (02)05 ‎[20]):

OOB Emissions

Spurious Emissions

OOB Domain

(OOB predominate)

Spurious Domain

(Spurious predominate)

250 %

of the necessary bandwidth

50 %

of the necessary bandwidth

Frequency

Unwanted Emissions

Figure 3: Il lustration of the OOB and Spurious Domains

### Co-channel

A co-channel interference scenario can be illustrated as shown in figure below. In this case a part or the whole component of the interference is within the receiver bandwidth of the victim receiver.

Figure 4: Co-channel interference scenario

## Monte-Carlo basics

The statistical methodology used as a basis for SEAMCAT is the Monte Carlo method. Statistical simulation methods may be contrasted to conventional analytical methods, which are typically applied to ordinary or partial differential equations that describe some underlying physical or mathematical system. In many applications of the Monte Carlo method the physical process is simulated directly and there is no need to even write down the differential equations that describe the behaviour of the system.

The Monte-Carlo simulation method is based upon the principle of taking samples of random variables from a given distribution. Before the simulation the distributions need to be defined for all relevant parameters of the wireless systems to be modelled (e.g. antenna heights, powers, operating frequencies, positions of the transceivers, etc.). Fixed values can be specified for parmaters which do not vary in the scenario (e.g. systems with specific frequnecies or heights). The technical specifications of the receiver and transmitter are generally extracted from relevant equipment standard (e.g. standards produced by ETSI, 3GPP, IEEE etc.).

SEAMCAT uses these distributions to generate random events using samples of the above mentioned distributions. For each event, SEAMCAT stores the signal strength of the interfering and the desired signals calculated in dedicated data arrays. As a very final step, the probability of interference can be calculated by comparing the wanted and unwanted signals at the victim link receiver in each event to the relevant interference criterion (e.g.  C/I).

The only requirement is that the physical or mathematical parameter can be described by a probability density function (PDF). Once the PDFs of the relevant parameters are known, the Monte Carlo simulation can proceed by randomly sampling them. Many simulation trials are performed with different random samples for each trial and the desired result is taken as an average over the number of observations. In many practical applications one can predict the statistical error in this average result and hence an estimate of the number of Monte Carlo trials that are needed to achieve a given error.

SEAMCAT models one single victim link receiver (VLR) connected to a victim link transmitter (VLT) operating amongst a population of one or more interfering link transmitters (ILT) which are linked to an interfering link receiver (ILR). These interferers may belong to the same system as the victim, a different system or a mixture of both. The locations of the interferers are distributed around the victim either completely randomly or with some relation to the location of victim in a manner that can be specified by the user.

Figure 5 illustrates the terminology of the various elements that are simulated for (a) ‘generic’ systems (i.e. non-cellular) and (b) cellular systems.

- (b)

Figure 5: Terminology used in SEAMCAT (a) generic systems and (b) cellular systems

It is common practice to use a uniform random distribution for the locations of the transceivers. The density of interferers is set in line with the environment being modelled, i.e. an urban environment should have a higher density than a rural environment. Only a proportion of the interferers are active at any instance. This proportion may depend for example on the day of the week as well as the time of day. Figure 6 illustrates how the interferers and victim may appear for one simulation trial. Also illustrated is the victim link transmitter providing the victim’s wanted signal (dRSS: desired Received Signal Strength).

Figure 6: A typical victim and interferer scenario for a Monte Carlo simulation trial

For cellular simulations, the generation of events includes additional complexity - several iterations of a power control loop (CDMA) may be needed within each event, and some initial pre-calculations are required, such as the calculation of non-interfered (nominal) capacity. For victim CDMA systems the interference criterion is the excess outage, i.e. the percentage of previously served users disconnected as a result of the interference impact. For victim OFDMA systems the interference criterion is the bitrate loss, i.e. the percentage of bit-rate lost compared to a non interfered victim network.

## Interference calculations in generic systems

### Introduction

In this section the interference calculations for ‘generic’ systems are described. Cellular systems (OFDMA and CDMA) use a different interference calculation method based on throughput and capacity loss which are described in more detail in ‎ANNEX 15:

### An illustration with C/I as interference criterion

The C/I ratio available at the victim receiver’s input is computed using both the iRSS (Interfering Received Signal Strength) and the dRSS (desired Received Signal Strength),. Figure 7 illustrates the various signal levels used to determine whether or not interference is occurring.

Figure 7(a) represents the situation in case of no interference - the VLR is receiving the dRSS with some safe margin above its sensitivity level. The victim’s signal level is the sum of the sensitivity and wanted signal margin l.

Figure 7 (b) illustrates the case of tolerable interference. The interference power iRSS adds to the noise floor power resulting in an increase of the noise floor. The example introduces an increase of 1 dB of the noise floor caused by an iRSS 6 dB below the noise floor. As a result the wanted signal margin is also reduced by 1 dB assuming a constant wanted signal power. However since the original wanted signal margin is much larger the interference is tolerable –i.e. the C/I ratio available at the receiver’s input is larger than the S/N required for the operation of the system.

Figure 7(c) shows the case of interference which can not be tolerated – i.e.  the operation of the system is impaired. The power sum of all the interfering signals including the noise floor of the receiver results in an insufficient wanted signal margin -, i.e. the C/I ratio available at the receivers input is less than the S/N ratio required for the intended operation.

Figure 7: Levels used to determine whether or not interference is occurring

As a receiver cannot distringuish between various sources of interference or noise, the sum of all interefing signals including the receiver noise floor has to be taken into account. The C/I ratio available at the receiver’s input must be greater than the S/N required for the operation of the system if the interference is to be avoided. SEAMCAT checks for this condition and records whether or not degradation due to interference is occurring. This is illustrated further in Figure 8

The Monte Carlo technique works by considering many independent events in time (or in space). For each instant a scenario is built up using a number of different random variables, i.e. where the interferer is located with respect to the victim, the signal strength of the wanted signal, which channels the victim and interferer are using etc. If a sufficient number of simulation trials are run then the probability of a certain event occurring can be calculated with a high level of accuracy.

In this way, the tool is able to quantify the probability of interference between radio systems and is able to help determine appropriate frequency arrangement rules or identify suitable limits for transmitter/receiver performance.

You can select the interfering modes (unwanted and blocking) as well as the interference criteria of your choice in SEAMCAT as shown in Figure 97 (p. 83).

- For each random event where

dRSS > sensitivity:

noise Floor (dBm)

desired signal value (dBm) (dRSS)

interfering signal (dBm) (iRSS)

C/Itrial > C/Itarget?

Interference (dB)

- if C/Itriali >C/Itarget:”no interference” event

- if C/Itriali <C/Itarget: “interfered”

- finally, after cycle of Nall events:

overall Pinterference= 1- (Nno interference/Nall)

Desired signal strength: dRSS -> (C)

Interfering signal strength: iRSS -> (I)

random

event

sensitivity

Figure 8: Illustrative summary of the interference criteria computation

### Methodology associated to the interference criterion (C/I, C/(I+N), (N+I)/N, I/N)

Four interference criteria are considered within SEAMCAT:

C/I	: Carrier to interference ratio;

C/(I+N)	: Carrier to interference plus noise ratio;

(N+I)/N	: Desensitisation;

I/N	: Interference to noise ratio.

All  these criteria can be specified as an input to your simulation. (Figure 9), but a single criteria needs to be chosen for the interference calculation ( Figure 10). Multiple interference calculations are possible on the same set of results if more than one criterion are used separately.  In the example below the  criterion for interference to occur for the VLR isa carrier to interference ratio (C/I) less than the minimum allowable value of 19 dB-

These parameters are also used in the evaluation of the two blocking modes (Protection ratio and Sensitivity, see section ‎1.4.5).

| Figure 9: Interference criteria values that you provide as input to your simulation | - Figure 10: Selection of the interference criteria used for the evaluation of interference (from the Interference Calculation Engine control panel) |
| --- | --- |

### Interference criteria relationship

C/I may vary typically from 9 dB (e.g. for QPSK) to 26 dB or higher (e.g. for 64QAM…). By introducing artificial noise iRSS on top of the noise floor (I/N), C/I is then desensitised by (N+I)/N resulting in C/(N+I). Note that the desensitisation is exactly the factor (N+I)/N (also = 1+I/N).

Further details of the relationship are given in ‎ANNEX 3:.

Considering that

and 	(Eq. 12)

and assuming a C/I of 19 dB, the following examples may be considered:

I/N = 0 dB, results in (N+I)/N = 3 dB and considering C/I = 19 dB, then C/(N+I) = C/I - 3 dB = 16 dB

I/N = -6 dB, results in (N+I)/N  1 dB and considering C/I = 19 dB, then C/(N+I) = C/I - 7 dB = 12 dB

I/N = -10 dB, results in (N+I)/N  0.4 dB and considering C/I = 19 dB, then C/(N+I) = C/I - 10 dB = 9 dB

I/N = -20 dB, results in (N+I)/N = 0.04  0.1 dB and  C/I = 19 dB, then C/(N+I) = C/I - 20 dB = -1 dB

|  | Note: | In case C/(I+N) is chosen as the protection criterion: if I/N ≤ -20 dB, the impact of the interferer is negligible compared to the noise floor (i.e. C/(I+N) ≈ C/N); if I/N > 10 dB, then C/(I+N) ≈ C/I (i.e. the interferer is more dominant than the noise). |
| --- | --- | --- |

### Unwanted emissions

The level of unwanted emissions  (i.e. consisting of the out-of-band emissions and the spurious emissions ‎[10] of the ILT) falling within the VLR receiver bandwidth (Figure 11) is determined using the interferer’s transmit mask, the receiver bandwidth of the VLR, the interferer-to-victim frequency separation, the gains of the antennas and the propagation loss. The receiver experiences the unwanted power directly as additional noise in terms of I+N. There is no possibility in terms of filtering with which the receiver could reduce this impact by itself.

Note that the receiver bandwidth is taken into account in the unwanted calculation.

Further details on the unwanted emission mask are provided in ‎ANNEX 6:. Details on the iRSSunwanted calculation are given in ‎ANNEX 5:. The unwanted emission is also sometimes quantified using the term Adjacent Channel Leakage Ratio (ACLR) (see Annex ‎A15.7).

Interfering Link Transmitter Emission Mask

Victim Link Receiver

fVLR

fILT

Rx Bandwidth

Tx Bandwidth

Unwanted emissions

Figure 11: Illustration of the interference due to the unwanted emissions (i.e. the unwanted emissions of ILT falling in the receiver bandwidth of VLR)

### Receiver blocking

The level of interference determined by the interferer’s transmit power, the antenna gains and propagation loss, is further decreased due to the receiver blocking performance for a given interferer/victim frequency separation. Details on the iRSSblocking calculation are given in ‎ANNEX 5:

Note that from SEAMCAT 5.0.1 onwards, the blocking attenuation is computed at the ILT frequency and that the ILT bandwidth is nowconsidered (see ‎ANNEX 8:). There are 3 ways to calculate the blocking response which are described in more detail in ‎ANNEX 8::

User Defined (dB): 	AttBlocking = BlockUD (dB)	(Eq. 13)

Protection Ratio (dB): 	AttBlocking = BlockPR (dB) + C/(N+I) (dB) + (N+I)/N (dB) - I/N (dB)	(Eq. 14)

Sensitivity Mode (dBm): AttBlocking = BlockSens (dBm) – SensitivityVLR (dBm) + C/(N+I) (dB) – I/N (dB)	(Eq. 15)

Rejection

of the receiver

Rx Bandwidth

Tx Bandwidth

Victim Link Receiver Blocking Mask

Interfering Link Transmitter Emission Mask

fVLR

fILT

Blocking

Figure 12: Illustration of the blocking of the victim link receiver (i.e. total emission power of ILT reduced by the blocking attenuation (selectivity) function of the VLR

### Intermodulation

The intermodulation interference, i.e. the power of intermodulation products, reduced by the intermodulation attenuation function of the VLR can be used in interference calculations. See ‎ANNEX 5: for further details.

### Overloading

Overloading threshold is the minimum interfering signal levelat which the receiver loses its ability to discriminate against interfering signals at frequencies other than that of the wanted signal. See Annex ‎A2.2 for the use of overloading in interference calculation and Annex ‎A5.4. for the iRSSoverloading calculation.

### Combined interference mechanism

The combination of the unwanted emissions and receiver blocking can also be studied simultaneously in SEAMCAT as depicted in Figure 13. See Annex ‎A2.3 for further details.

Rx Bandwidth

Tx Bandwidth

Unwanted emissions

Victim Link Receiver Blocking Mask

Interfering Link Transmitter Emission Mask

fVLR

fILT

Blocking

Figure 13: Illustration of the combined unwanted emissions and 
the receiver blocking mechanism in SEAMCAT

### Interference calculation

SEAMCAT calculates the probability of interference for generic (i.e. non-cellular) victim systems. Each sample of dRSS and iRSS generated during a simulation is compared against the relevant signal-to-noise criterion (specified in the scenario, such as C/N, C/N+I etc). The probability of interference is calculated for all events where the dRSS is greater than the sensitivity of the victim link receiver (dRSS > sens). This probability can be calculated for two different modes, as illustrated in Figure 271 of section ‎12.9:

Compatibility: This mode provides a single-figure estimate of the probability of interference in a given interference scenario;

Translation: This mode calculates probability of interference as a function of changing one of the following parameters:

Transmitter power of the interefering link transmitter;

Blocking response level of the victim link receiver;

Intermodulation rejection level for the victim link receiver.

## Applicability of SEAMCAT to Spectrum Engineering Problems

SEAMCAT can address virtually all radio interference scenarios on terrestrial paths in both co-channel (sharing) and adjacent frequency (compatibility) interference studies. This flexibility is achieved by the way the system parameters are defined as variable (or constant) through their distribution functions. It is therefore possible to model even very complex situations by relatively simple variations of some elementary functions. A number of various wireless services can be modelled using SEAMCAT, such as:

Broadcasting: terrestrial systems and Earth stations of satellite systems;

Fixed Services: Point-to-Point and Point-to-Multipoint fixed systems;

Mobile Services: Land Mobile Systems, Short Range Devices and Earth based components of satellite systems.

Section ‎1.9 presents various radio systems that have been simulated in various ECC Reports.

In general, SEAMCAT may be used to address the following spectrum engineering issues:

Sharing studies between different radio systems operating in the same frequency band;

Compatibility studies between different radio systems operating in different frequency bands;

Evaluation of transmitter and receiver masks;

Evaluation of limits for certain system parameters, such as transmitter unwanted emissions (spurious and out-of-band), and receiver blocking or intermodulation levels.

SEAMCAT assumes a flat Earth model for calculating path geometries and propagation losses. This limits the range of considered standard interference scenarios to terrestrial configurations and non-path-specific propagation models. Certain aeronautical and space-to-Earth paths can also be modelled if suitable propagation models are used.

## Understanding radio jargon

Many common terms in RF engineering are used differently depending on the specific community where they are used. The following gives a non-exhaustive example of the variety of terms that can be found.

C (i.e. the wanted signal level) is referred to in different radio standards and documents as follows. It is specified for a given sensitivity and a given desensitisation.

Useful signal (ETSI TS 145.005 - Chapters 5.1.2 and 5.1.3);

Wanted signal mean power (ETSI TS 136.104 - Table 7.5.1-1);

Pw (ETSI TS 136.101 - Table 7.6.3.1-1);

Prefsens + desensitisation (ETSI TS 136.104 - Table 7.5.1-1, ETSI TS 136.101 - Table 7.6.3.1-1) ;

“C”.

IOOB  (i.e. the allowed power of an interfering blocking signal as specified by the standard) is referred to in different standards as follows. It is specified for a given frequency offset, a given sensitivity and a given desensitisation.

Blocking signal level (ETSI TS 145.005 - Table 5.1-2a);

Puw (ETSI TS 136.101 - Table 7.6.3.1-1);

Interfering signal mean power (ETSI TS 136.104 - Table 7.5.1-1).

Sensitivity is referred to in different standards as:

Reference sensitivity level (ETSI TS 145.005 - Tables 6.2-1x);

Reference sensitivity (ETSI TS 136.101 - Table 7.3.1-1);

Reference sensitivity power level (ETSI TS 136.104 - Tables 7.2.1-1 and 7.2.1-2);

Prefsens (ETSI TS 136.101 - Table 7.3.1-1, ETSI TS 136.104 - Tables 7.2.1-1 and 7.2.1-2).

## SEAMCAT software

### Installation using .jar package

SEAMCAT is an open source project developed using Java which means it can run on any operating system supported by Java. The SEAMCAT installation is started by clicking on the install link from www.seamcat.org and it is free of charge.

The .jar package is platform independent so that you can run SEAMCAT on either Mac OS, Linux or Microsoft Windows operating systems. You need to check on the website that you have the latest version installed.

It is required that you have the Java runtime environment (JRE) installed on your computer, available to download free of charge from www.java.com. The latest version is recommended.

### SEAMCAT home directory and configuration panel

When SEAMCAT (5 and upward) is run, a folder (SEAMCAT home directory) will be automatically created so that every time a new version of SEAMCAT is run, a subfolder with the version number will be created as shown in Figure 14.

Figure 14: Illustration of the seamcat-app content

Each SEAMCAT version will have the structure as shown in Figure 15.

Figure 15: Content of a work directory

Table 1: Description of the content of the SEAMCAT home directory

| Name of file/folder | Type | Description |
| --- | --- | --- |
| logfiles | Folder | Contains the log file (.txt) generated when debug mode is used |
| reports | Folder | Contains the report files (.xml/.html/.xlsx) generated by the Report feature |
| temp | Folder | Contains temporary files used by SEAMCAT |
| workspaces | Folder | Default folder for creating new workspaces and saving result workspaces. This can be changed from the File menu |
| Default-library.sli | File | Contains the default library of SEAMCAT (systems, masks, antennas, etc.) |
| seamcat.log | File | Text file containing all the log information reported by SEAMCAT (errors, warnings etc.). Note that the file may be appended with information everytime SEAMCAT is launched. This folder may be deleted from time to time if so wished to clear computer space or reset settings. When deleted, it is automatically created next time SEAMCAT is launched. |
| settings.xml | File | Contains the settings of your SEAMCAT configuration including all settings related to external plugins |

The location of the home directory can be modified in the SEAMCAT configuration panel accessible via general menu option <File/Configuration> () also using <CTRL+G>. Further information is given in Annex ‎A20.1.

### Java Source code

The source code for SEAMCAT is made available for scrutiny. The only condition for obtaining a copy of the source code is to sign a special Source Code User Licence Agreement and deliver it to ECO by email (seamcat@eco.cept.org). The procedure is described at: http://www.cept.org/eco/groups/eco/seamcat-source-code.

### Multi processing power in SEAMCAT

As of version 5, SEAMCAT is able to run in multiprocessor environments to increase the computation speed. The more processors a computer has, the less time a simulation will take. Since the Monte Carlo simulation method considers that each event is independent from each other, events can therefore be distributed across all the processor cores in themachine in order to decrease the computation time. The EPP runs in parallel for each event.

If debug mode is used SEAMCAT runs on only one processor since parallel processing of the events would make the log file unreadable.

## The SEAMCAT community

### SEAMCAT Technical Group (STG)

The SEAMCAT project is an ongoing WG SE (Working Group Spectrum Engineering) activity (Figure 16). The daily maintenance of the project and the SEAMCAT software is entrusted to the ECO. The project is funded by the signatories of the ECO council. The SEAMCAT Technical Group (STG) acts as the supervising committee and source of technical expertise. For further information on STG visit the CEPT web page: http://www.cept.org/ecc/groups/ecc/wg-se/stg.

Figure 16: SEAMCAT project organigram and steakholders

### SEAMCAT community – more than STG

SEAMCAT allows building customized library of components (systems, antennas etc.) or use those created by someone else. It is possible to use predefined antenna patterns, spectrum emission masks, and propagation models etc. which are available in the SEAMCAT library, so that reuse of scenarios, library items, etc. can be used  in new studies. You can join the SEAMCAT community to contribute to this activity.

The SEAMCAT community is a forum where active SEAMCAT users can report feedback on the use and improvement of the tool (see Section ‎2.23). This includes the members of STG but also users from universities and organisations all around the world. This community has extended itself well beyond the CEPT borders as depicted in Figure 17.

It is possible to contribute to the community by sharing workspaces, populating libraries, creating and sharing plugins (EPPs, propagation plugins, etc...) and contribute to ECC and CEPT reports with SEAMCAT simulations. Any plugins are automatically embedded, so it’s only  necessary to share workspaces by email.

The SEAMCAT application is open source and it is available free of charge.

The SEAMCAT community is accessible via different media:

SEAMCAT email reflector (seamcat@list.cept.org):

To subscribe: mailto:seamcat-request@list.cept.org?subject=Subscribe;

To unsubscribe: mailto:seamcat-request@list.cept.org?subject=unsubscribe;

STG-ECO forum (http://www.cept.org/ecc/groups/ecc/wg-se/stg/client/forum).

Figure 17: Usage of SEAMCAT within and outside the CEPT (downloads in 2014)

## Reference material and ECC/CEPT Reports

Existing workspaces (i.e. .sws files) which have been generated as part of some ECC report or CEPT reports activities are available at the following address www.ecodocdb.dk. These files are can be opened with the latest SEAMCAT version, with the following restrictions:

- Post processing plugins created in previous versions are not compatible in version 5, as they were moved into Event Processing Plugins

The local environments, which were placed together with the propagation models in previous versions, are to be found in version 5 in the system link tab (ILT –> ILR path, VLT –> VLR path). These settings are not migrated from previous versions.

See ‎ANNEX 21: for an overview of studies per technology where SEAMCAT was used from various ECC deliverables. An example is shown in Figure 18 for the ECC Report 122.

Figure 18: ECC database where .sws file used in producing ECC or CEPT report are stored 
(see ‎ANNEX 21: on p. 445)

# GeTting around in SEAMCAT

## Quick buttons toolbar

A ‘quick buttons’ toolbar with access to the most commonly used features is avaialable at the top of the SEAMCAT interface window. The function of each button is shown in a tooltip whenever your mouse is moved over it. Certain  buttons are inactive when the function is not applicable, e.g. the stop simulation button is active only during the run of a simulation. Inactive buttons are displayed dimmed.

Table 2: Overview of the buttons on the quick menu toolbar

|  | new workspace |  | export library |  | check the built-in and plugin propagation model |
| --- | --- | --- | --- | --- | --- |
|  | new batch |  | start event generation (workspace or batch) |  | check the unwanted emission function |
|  | open (workspace or batch) |  | stop event genetration (workspace or batch) |  | compare multiple vectors |
|  | Save (save as, save all) |  | generate a report (xml or html file) |  | pocket calculator |
|  | close workspace or batch |  | check the consistency of the workspace |  | on-line manual help contents |
|  | import library |  | check the built-in distribution function |  | report bugs or enhancement |

## ‘Welcome to SEAMCAT’ window

Every time SEAMCAT is launched, a welcome dialog box is displayed which allows the selection of various options (the welcome dialog box can be disabled under File/Configuration). The first step is typically to create a new workspace or open a previously created workspace. This is done by selecting ‘New’, ‘Open’ or ‘Recently Used’ (to open the most recent workspaces from the displayed list). ‘New Batch’ can be used to specify multiple sequential simulation runs using new or existing workspaces. ‘Calc’ launches the SEAMCAT calculator which is a convenient tool for common basic engineering calculations.

- Figure 19: Welcome pop up window

When a new workspace is created, it will initially be automatically assigned a name 'Workspace xyz', where xyz is the serial number of the newly created workspace. The workspace name may be changed at any time afterwards using Save or Save As. The file in your file system can also be renamed - SEAMCAT will recognise the new name when loading the workspace.

Several workspaces can be open at the same time – different workspaces are shown as tabs at the top of the user interface. The size and position of the SEAMCAT subpanel graphical user interface (GUI) is minatained when switching from one workspace to another for ease of  comparison between workspaces.

Figure 20: Several workspaces in different tabs

## Typical SEAMCAT work flow

When using SEAMCAT for a compatibility analysis, a typical workflowwould be asfollows:

- Create or update a simulation workspace;

- Set the simulation control parameters (number of events, debug mode or not);

- Run a simulation;

- Analyse the event generation results, modify the input scenario if necessary and re-run the event generation;

- Perform one or more interference calculation runs;

- Generate a simulation report.

Figure 21: Typical work flow when using SEAMCAT

In addition to these basic steps futher ones may be useful for different purposes:

Save workspace as an XML compliant file for later (repeated) work or for sharing with other users;

Create or update elements in the equipment library for repeatedly used equipment types;

Import a library;

Use of batch job, to serialise your work;

Share your workspace with others;

Use of an Event Processing Plugin for advanced scenario specific calculations.

## Simulation workspace

### Scenario workspace

A scenario workspace can be seen as a working environment for a given study where all input fields are editable. It consists of the following three tabs:

Systems: This allows setting the characteristic of the systems to investigate (generic Tx, Rx and path between Tx/Rx or cellular general settings, positioning);

Scenario: This sets the simulation scenario, i.e. the selection of the systems to be used as victim or interferer(s) as well as the path between the victim and the interfer(s), the number of events and optional selection of debug mode;

Event processing: The event processing plugin (EPP) environment allows the processing of intermediary SEAMCAT results for each simulated event. Some EPPs are available built in to the tool, but it is also possible to create customized EPPs.

Figure 22: Illustration of the workspace scenario (Highest hierarchy level)

### Results Workspace

When a simulation is performed, SEAMCAT automatically generates a separate workspace to contain the results which appears as a separate tab entitled ’Results[Workspace xyz]’ and stores read only information.

Figure 23: Illustration of the workspace results (Highest hierarchy level)

It is not possible to edit any of the input fields in the results tab. It is however possible to select ‘Export workspace’ to generate and open a new workspace scenario entitled ‘Exported Workspace xyz‘ :

Figure 24: Illustration of exporting a results workspace into a scenario workspace

The results workspace consists of the following tabs depending on the scenario set-up:

Outline:the outline of the simulation can be seen with both a summary of the dRSS and iRSS calculation and a graphical overview of the Tx/Rx positions for the victim and interferer(s);

Results: gives access to a list of all results generated by SEAMCAT and from the EPPs used in the simulation;

Interference calculations: gives access to the Interference calculation engine;

CDMA Capacity Finding: This panel is only available when a CDMA system is used in the scenario. It indicates the non-interfered capacity (i.e. number of UEs) when the network is gradually filled up with UEs while measuring a specific system outage depending on uplink (UL) or downlink (DL) configuration;

Cellular Structure: This panel is only available when a cellular system (CDMA or OFDMA) is used in the scenario. It shows the results of the cellular specific simulation – i.e. layout of the cells, position of users and indication of dropped users.

Figure 25: Results workspace with a Cellular system (the “CDMA capacity finding” is only available with CDMA simulation)

## Create or update a simulation workspace

To start,  create a new workspace (see 1 in Figure 25 or open an existing one (2).

Figure 26: Illustration of how to create or open a workspace

A simulation scenario is a part of a scenario workspace, which defines all technical parameters of modeled links as well as some related modelling assumptions, such as physical (mutual) placement of transceivers, propagation modelling settings and so on. Within SEAMCAT a simulation scenario is based upon the concept of links. A link stands for a pairing of one transmitter with one receiver operating together in a given wireless system. Such a link is also called a “system link”.

When a new workspace is created, all of its scenario elements are populated with default values (representing a hypothetical interference case between two systems at 900 MHz, as shown in Figure 27). Therefore after creating a new workspace the first step should be to review and modify the scenario parameters so that the resulting simulation scenario reflects the configuration of the wireless systems to be simulated.

To create or update a simulation workspace, the following steps need to be followed:

- Set the technical characterisitic of the various systems that you want to investigate. In the example of Figure 26 the first system is Generic and the second one is an OFDMA UL system. Do not forget to export the systems used to the library environment so that they can be reused  at a latter point of time.

Figure 27: Setting various systems (import/export to library environment)

Figure 28: Overview of the various systems which can be simulated in SEAMCAT 
(i.e. Generic and cellular)

- Set the scenario (see section ‎10);

Set the path characteristics between the victim and the interferer (relative position, interferer’s density, path loss correlation and propagation model). Links can also be added, duplicated, etc...;

The number of events to simulate can be set in the simulation control panel. It is also possible to choose  whether to run a simulation in the debug mode or not;

Having reviewed the scenario, the simulation can be run.

## Batch operation

The SEAMCAT's Batch function allows automation of repetitive compatibility studies by scheduling several SEAMCAT simulations to be done in one run of the programme. This is done by setting up a simulation schedule with instructions on how to change certain variable scenario input parameters in several consecutive simulations. One typical example of using SEAMCAT batch functionality would be to perform multiple simulations in order to study interference probability on identical scenarios with only small frequency changes between them.

A new batch can be created directly from the quick menu button or you can open an existing batch file using the usual open icon. The following menu button can be used for the batch

Table 3: Overview of the batch operation menu buttons

|  | new batch |  | close workspace or batch |
| --- | --- | --- | --- |
|  | open (workspace or batch) |  | start event generation (workspace or batch) |
|  | save |  | stop event genetration (workspace or batch) |

The batch environment, as shown in Figure 28 consists of 3 panels (#1), identification, control and Workspaces.

Figure 29: Batch environment

The control panel provides the following buttons dedicated to workspaces:

Table 4: Overview of batch control panel

|  | New workspace to add in the batch |  | Remove a workspace from the batch |
| --- | --- | --- | --- |
|  | open a workspace to the batch |  | Duplicate a workspace inside a batch |
|  | Save a workspace |  |  |

When several workspaces are opened in the batch mode environment, parameters can be directly modified in the workspace with an unlimited number of times by adding new simulations to the batch schedule until the intended variation of input parameters is fully covered by all entries.

It is also possible to save workspace results automatically after completing their simulation.

Figure 30: Workspaces panel, where you can load as many workspaces as your memory can handle

Batch results (Figure 29 #1) are available when the batch simulation is started. A result tab (#2) is by default active indicating the progress of the batch work (#3) and provides interference calculation engine capability (#5). When pressing the analyse button the ICE is activated and the probability of interference is displayed (#6).

The results vectors for each workspace are also available (#4) for scrutiny.

Figure 31: Batch results environment

If the “debug mode” is selected and a batch is run, the batch will stop until the log window is closed.

## Running and stoping a simulation

A simulation is started when clicking on the Start button in the simulation control part of the Scenario tab (). The simulation starts and all other commands/buttons are, meanwhile, disabled except the stop button (). SEAMCAT automatically performs the consistency check of the scenario parameters before starting the event generation (see Section ‎2.10).

The simulation runs until the number of events has been performed or if the simulation is stopped prematurely by pressing the stop button.

Figure 32: Simulation control  window in the scenario tab .

## Pausing and RESUMING a simulation

A simulation can be paused when clicking on the Pause button in the simulation control part of the Scenario tab (). The simulation pauses and all other commands/buttons are, meanwhile, disabled except the stop button (). The simulation is resumed by clicking on the Resume button ().

Figure 33: Simulation control inside the Outline window in the Results tab during run time

## Running from the command line

A command line launch of SEAMCAT (i.e. without GUI) is possible using the same seamcat.jar file that has  been downloaded.  From the command line with a workspace file called "myWorkspace.sws" the following command will launch the workspace and save the results in myWorkspace.swr by default.

java -cp seamcat.jar org.seamcat.CommandLine myWorkspace.sws

Several options are possible. The result file name can be changed by using the option: result=otherName.swr and the number of events can be specified by using the option: events=12345. The example below uses all these parameters:

java -cp seamcat.jar org.seamcat.CommandLine Default-ws.sws result=commandLine.swr events=200000

## Consistency check

The consistency check function is run automatically before the start of an event in order to detect erroneous or inconsistent values in the scenario definition (see Figure 32). You can also trigger it manually as illustrated in Figure 34. The aim of the consistency check is to prevent runtime exceptions during the simulation due to incorrect input values.

Figure 34: consistency check button in the scenario tab

The consistency check includes:

Warnings if odd configuration are detected for random parameters, such as negative antenna height, etc;

Consistency between inter-dependent parameters such as: C/I, C/(N+I), (N+I)/N, and I/N;

Verification of the application range of propagation models.

In case it fails, the scenario consistency check will pop up a message window like the below

Figure 35: Scenario consistency check message

These warnings may be ignored in some cases, but you should be aware of the following:

Consistency checks are generally connected to non valid values which cause runtime exceptions during the simulation

Each runtime exception generates an appropriate error message which is recorded on the seamcat.log file (located on your SEAMCAT home directory):

- Note that the yellow marked header format can be changed as described in Annex 20.

Logging the error messages takes time, hence it significantly increases the simulation time;

Depending on the scenario (in particular the number of events simulated) the amount of the seamcat.log file might readily grow up to some tens of Mega Byte per simulation run.

It is therefore highly recommended to solve the reported issues prior to run the simulation. Further information on the meaning of consistency check and SEAMCAT error message is available in ‎ANNEX 19:.

Section ‎14.3.2 presents the implementation of the consistency check in the source code and for plugin development and its graphical results.

## Simulation control

The simulation control setting “number of events” is used to determine the number of events you want to simulate. The “Debug mode” feature is described in Section ‎2.15. These control settings are accessible from the Scenario tab.

Figure 36: Simulation control

## Simulation results overview

Once a simulation is complete a workspace result (see Section ‎2.4.2) is automatically created within SEAMCAT where it is possible to extract any results vector. Depending on the nature of the victim system (i.e. generic, CDMA or OFDMA), the results vectors may vary. Further details about the nature of the results can be found in Section ‎12.

The bottom of the results panel contains the scenario statistics of the simulation’s run (number of processor utilised, total simulation duration etc…). The statistics display is irrespective of the radio system simulated.

Figure 37 Overview of the results panel using a cellular victim system and the statistics panel

## Saving options in SEAMCAT

### Saving workspace and batch

Workspaces can be saved <CTRL+S> or “save as” (either scenario or results)/batch by using the default option SEAMCAT offers in the File menu. The file extension is appended automatically. These are SEAMCAT-specific file extensions. The .sws, .swr, sbj and sbr format are equivalent to a .zip file. Table 5 provides an overview and a description of the files contained per extension.

When closing SEAMCAT without previously having saved or closed the workspace, a prompt window will appear asking whether any changes to the workspace should be saved.

Figure 38: Illustration of how to create or open a workspace and its associated file name

Figure 39: Saving options in SEAMCAT

Table 5: Saving options in SEAMCAT

| SEAMCAT file type | SEAMCAT extension | Files contained |
| --- | --- | --- |
| SEAMCAT Workspace Scenario | .sws | Scenario.xml |
| SEAMCAT Workspace Results | .swr | Scenario.xml and results.xml |
| SEAMCAT batch job | .sbj | Batch.xml |
| SEAMCAT batch results | .sbr | Batch.xml and workspace xy_results.xml |

### Saving results – file extension

Saving workspace results will store all simulation calculations in the workspace. Vector results can also be independently saved using one of the three different file extensions: tab separated text, xls spreadsheet and xlsx spreadsheet. The various extension options are illustrated in Figure 40.

Figure 40: Example of saving options of the vector results

## Simulation report

Any simulations and their results overview can be presented in a report. This report is automatically generated by SEAMCAT. Select the Report command from the Workspace menu, click on the Report icon of the toolbar () or press <CTRL+R> to activate the Report options window:

Figure 41: SEAMCAT report generator panel

In this dialog window, the contents of the generated report can be configured, by checking or un-checking following items:

Scenario: It indicates whether all scenario parameters should be included or not;

Pre Simulation results: Pre-simulation is a result group where a simulation can store results that are calculated before the simulation starts. This is used primarily for the cellular simulations were complex calculations are conducted before the simulation begins (e.g. capacity findings for CDMA). For some simulation this can be empty. Currently the generic simulation has no need for presimulation results and therefore leaves the group empty;

Simulation results: It indicates whether simulation control parameters should be included or not;

Expand results: allows expanding tabular data by checking or unchecking the Expand tabular data option.

There are three formats for the report:

XML File (default);

HTML File;

Excel File (.xls and .xlsx).

When clicking on “Generate”, a file is automatically generated in the “reports” folder under the SEAMCAT home directory (see Figure 15).

For all other data types the output is similar to their display in the workspace tree view except that couples of values defining user-defined random data, functions, and radiation patterns are fully expanded.

Figure 42: Example of a report generated in SEAMCAT in a html format

## Logging function for debugging

SEAMCAT allows two options to generate two distinct log files. An automatic SEAMCAT log that monitors the behaviour of the application, and a user debug log to check how the computation is performed.

seamcat.log: This log file is automatically generated by SEAMCAT and is intended to be used to debug the Java application mainly in case of a crash of the application or when an exception in the code occurs. You can select/browse the filename and directory from the SEAMCAT options and also the log level.

Figure 43: SEAMCAT configuration

Figure 44: Seamcat.log log file automatically generated by SEAMCAT

Debug Mode: Running a simulation in “debug mode” Figure 45 is used for detailed check of the simulation process. Selecting this option, SEAMCAT will generate a log file where all interim results of simulations including what random values were generated for parameters defined as functions or distributions. This information is very useful for further inspection. The log file will be automatically stored in the sub-directory "Logfiles" of the main SEAMCAT directory. The debug option reduces the number of events to 5 events for the sake of limiting the size of log file. The generated log file may be reviewed using standard reading tools, such as Notepad, MS Windows, etc.

Figure 45: Selection of the debug mode

The output of the file (filename_date_number.log) is indicated at the bottom’s right corner of the SEAMCAT application window (see Figure 46) as well as in the first line of the log display as shown in Figure 106.

Figure 46: Filename and directory output for the debug log file

An example fragment of such a debug mode log file is shown in Figure 47. The settings for generating the log file in the SEAMCAT configuration panel can be modified as described in ‎ANNEX 20:.

Figure 47: Example of a debug mode log file

## Play/replay an event

In the Results tab there is access to a list of all the results from SEAMCAT and from the EPPs used in the simulation giving the option to “Play/replay” one event, and a log of the calculation will be made available.

When inspecting the dRSS or iRSS vector, an event may be identified and further investigated by using thereplay feature. For that, identify the event number to replay, insert this number and click on the ”play” button . A new tab called <Replay [event = x]> will be created and will contain an “Event Result” panel on the left and a “Log Trace” on the right hand side. This log file is automatically saved under the “logfiles” folder in your home directory.

Figure 48: Illustration of the play/replay feature

Figure 49: Illustration of the Replay tab with “Event Result” on the left and “Log Trace” on the right

Doing cellular system simulation the play/replay function allows re-runnning a specific event and redisplaying a cellular structure layout as shown in Figure 50. Inspecting an event allows the validation of the configuration of the simulated workspace.

Figure 50: Example of the play/replay feature for a cellular network

## Testing propagation models

A sanity check on the suitability of a propagation model can be done by clicking on <Tools/Test Propagation Models> in the main menu of SEAMCAT, by clicking on the icon () of the toolbar or by pressing <CTRL+SHIFT+M>. With this feature, it is possible to evaluate the result while using a certain propagation model. .

Figure 51: Access to testing propagation model

For that, choose the propagation models to compare them. It is possible to compare 2 or more propagation models with this interface.

Figure 52 shows that the SEAMCAT resulting attenuation from the free space is 101.47 dB where the free space path loss is defined as:

,	(Eq. 16)

where d is given in km and f is given in MHz. This results in

L = (32.44 + 10 log(8) + 20 log(1000)) = 101.5 dB

Figure 52: Illustration on to check the propagation model results

If one parameter is chosen as a distribution (for instance distance), then the button <generate sorted samples > is activated. Pressing it enables you to plot your results according to a sorted X-axis from min to max.

Finally click on <Generate samples> or <generate sorted samples> (depending which one is active) to view the results. If no results are displayed, check the Java console for any error messages.

Figure 53 Example comparing 2 propagation loss using the “generate sorted samples”. You can display the vector in linear or logarithmic scale

## Testing distribution function

A sanity check on distributions can be done with the test distribution function from the menu bar of SEAMCAT. The result of generating random values using a particular distribution type can be tested by selecting the <Test Distributions> command from the Workspace menu, by clicking on the icon of the toolbar () or by pressing <CTRL+SHIFT+D>. This could be e.g. used to analyse the statistical qualities of generated random variables.

Figure 54: Access to testing unwanted emission function

Figure 55: Illustration of the distribution check routine

## Testing unwanted emission function

A sanity check on the unwanted emission calculation can be done by using the <Test Rel. Unwanted> emission function in the menu bar of SEAMCAT, by clicking on the icon of the toolbar () or by pressing <CTRL+SHIFT+U>. The definition of unwanted emissions mask can be checked by calculating the actual unwanted emissions for given mask in a given VLR bandwidth applying a certain ILT-VLR frequency offset:

Figure 56: Access to testing unwanted emission function

This feature allows to directly extract the uwanted emission by entering the frequency separation, the receiver bandwidth and the spectrum mask as shown in Figure 57.

Figure 57: Illustration of the unwanted emission function check routine

## Multiple vectors comparison

All the vectors (or a subset) that have been generated for one workspace or more can be seen at once. For this, the <Compare Vectors> command is available by clicking this icon  or by pressing <CTRL+SHIFT+V> to open the dialog box in Figure 58.

It allows displaying multiple vectors that are resulting from the simulations (batch or not);

It allows adding external vectors from a past simulation;

It allows changing the text of the legend by double clicking on the vector.

Figure 58: Illustration of the compare vectors feature

## Pocket calculator

SEAMCAT contains a builtin pocket calculator. Select the command < Tool/Pocket Calculator> from the Workspace menu, click on the pocket calculator icon of the toolbar () or CTRL+SHIFT+C to activate the Pocket calculator window:

Figure 59: SEAMCAT pocket calculator

The calculator can also be accessed from the graphical user interface when a scalar is to be entered as shown in the example of Figure 60 with the wall loss attenuation of the Extended Hata propagation model.

Figure 60 Access to the calculator from the GUI panel

## Online manual help content

SEAMCAT provides a direct access to the online manual by clicking on the following symbol . Each panel contains a specific link where detailed information on the feature or algorithm can be found. In case of need for further information, clarification, or in order to supply additional information or improvements to the manual that could be useful for other users of SEAMCAT, please contact the ECO project manager at seamcat@eco.cept.org

## Report enhancementS and bugs

The SEAMCAT tool is thoroughly tested but in case of need to report bugs or enhancements,

send an email to seamcat@eco.cept.org by describing the issue and attach relevant files (i.e. workspace files, screen shots, system log file etc…). Further information on the log file can be found in section ‎2.15.

It is advised to use the STG template for this, which can be downloaded by clicking on the “bug report” button:

- Figure 61: Bug report

## Installing plugins into SEAMCAT

From the SEAMCAT library menu it is possible to install a library as a .jar file. By creating a new .jar entry and adding the file SEAMCAT will now include this plugin to be selectable when configuring scenarios.

Figure 62: Installing plugins

After installation the installed plugin can be seen in the SEAMCAT library. Any plugins can be loaded from this interface (e.g. Propagation Model Plugin, Event Processing Plugin, coverage radius etc…). Further description on the library is available in section ‎13.

Figure 63 Library jar installer

# Example of runninG a simulation

## Setting the scenario

Running a simulation consists essentially in setting the input parameters of the victim and the interfering system according to your requirement. Similarly, you need to consider the requirement of the geographical location of the various systems. For this example, let’s consider a fixed link, which is interfered by another fixed link as illustrated in Figure 64.

The Victim link transmitter and the Victim link receiver compose the “Victim link” and the Interefering link transmitter and the Interfering link receiver compose the “Interfering Link”.

Wanted

Transmitter

(VLT)

Victim

Receiver

(VLR)

Wanted

Receiver

(ILR)

Interfering Link

Transmitter

(ILT)

dRSS

iRSS

Victim link

Interfering link

Figure 64 Example of Application of SEAMCAT

## Calculating the dRSS

Define Victim link receiver (VLR)

- Operating frequency

- Receiver bandwidth

- Antenna characteristics

- Interference criteria

- Noise floor

Define Victim link transmitter (VLT)

- Tx power

- Antenna characteristics

Position the VLT vs VLR

In this section, you will learn how to ...

Calculate the dRSS

Test propagation model

Launch simulation

Extract dRSS vectors

✓

✓

✓

✓

✓

✓

✓

### Victim link

The victim parameter characteristics summarised in Table 6 should be entered into SEAMCAT.

Table 6: Characteristics of the victim link pair of receiver and transmitter

| Parameters | Value | Units |
| --- | --- | --- |
| Operating Frequency | 1000 | MHz |
| Transmitter power | 30 | dBm |
| Receiver bandwidth | 200 | kHz |
| Tx and Rx antenna type | Omni directional |  |
| Tx and Rx antenna gain | 9 | dBi |
| Tx and Rx antenna height | 30 | m |
| C/I protection criteria | 19 | dB |
| C/(N+I) protection criteria | 16 | dB |
| (N+I)/N | 3 | dB |
| I/N | 0 | dB |
| Noise floor | -110 | dBm |

Each simulation workspace contains one and only one victim link.

### System to be a victim

In order to set up a workspace, the first step is to set the system that will be used as victim: set the characteristics of the receiver (that will be the victim link receiver (VLR)) and the transmitter (that will be the victim link transmitter (VLT)).

System characteristics can be directly edited in the system tab. This is illustrated in the figure below.

Figure 65 Access to the system environment

For example, a generic system can be selected from the “import system from library button” as shown Figure 67.

Figure 66: Import system from library button

Figure 67: Selecting a system from library

First, let set the frequency of the victim. The frequency can be changed from 900 MHz (default value) to 1000 MHz as described in Figure 68. In this example a constant value is considered, but in principle any type of distribution can be selected.

Figure 68: Example of setting up the operating frequency

### Receiver

Set the victim link receiver by selecting the receiver tab.

Figure 69: Selecting the receiver tab

The parameters should be field according to the value of Table 6.

The noise floor is –110 dBm. (#1 of Figure 70);

The reception bandwidth is 200 kHz;  (#2 of Figure 70);

To simulate an omni-directional antenna, leave blank the horizontal pattern or select the omni directional antenna from the library (#3). The antenna peak gain is to be set to a value of 9 dBi; (#4) (see Figure 71);

The antenna height is 30 m; (#5 of Figure 72);

The interference criteria of the victim link receiver are a C/I of 19 dB, a C/(N+I) of 16 dB, a  (N+I)/N criteria of 3 dB and a I/N of 0 dB; (#6 of Figure 73).

Figure 70 Setting the reception characteristic

Figure 71: Setting the antenna pattern from the library and setting the peak gain to 9 dBi

Figure 72: Setting the antenna height

Figure 73: Setting the interference criteria

### Transmitter

Set now the victim link transmitter by selecting the transmitter tab

Figure 74: Selecting the transmitter tab

The parameters should be filled as follows:

The power is 30 dBm; (#1 of Figure 75)

An omni-directional antenna of 9 dBi is used;

The antenna height is 30 m.

Figure 75: Example of setting up the victim link transmitter

### Positioning the VLT vs VLR

Define now the positions of transmitters and receivers of the victim link.

Figure 76: Selecting the Tx to Rx path tab

SEAMCAT allows defining the locations of the Victim link receiver and the Victim link transmitter in a fixed manner (correlated) or following some distribution (uncorrelated) as summarised in Figure 77. The input parameters are detailed in section ‎5.4, and the algorithms are detailed in ‎ANNEX 13:.

Uncorrelated cases mode

User-defined Radius

Correlated cases mode

Correlated distance (origin = VLT)

Noise limited Network

Traffic limited Network

VLT ↔ VLR

location

Figure 77: Summary of the VLT ↔ VLR location capability in SEAMCAT

In this exercise, set the distance between the VLT and the VLR as fixed (correlated distance) (x = y = 2 km) as illustrated in Figure 78.

Victim Link

Transmitter

(VLT)

Victim

Receiver

(VLR)

dRSS

Victim link

(0,0)

(2 km,2 km)

Delta X = 2 km

Delta Y = 2 km

x

y

Figure 78: Distance between the Victim link transmitter and the Victim link receiver (Victim Link)

It is assumed that the Tx and Rx are both outdoor as shown in Figure 79.

Figure 79: Example of setting up the outdoor/indoor ratio

For this, select the correlated distance option of SEAMCAT as shown in Figure 80. In SEAMCAT, the origin of the coverage radius (see ‎ANNEX 13:) is the transmitter (this is also reminded in the GUI) of the link.

Figure 80: Illustration of the correlated distance in SEAMCAT

### Selecting the propagation model

Since both the transmitter and the receiver characteristics and the location between the two have been defined, the propagation model needed for the simulation can be now selected.

To simplify this task, let us assume that the free space model is used to calculate the attenuation between the Victim link receiver (VLR) and the Victim link transmitter (VLT).

In addition, the Variation should be disabled as shown in Figure 81.

Figure 81: Example of setting up the free space propagation model for Step 1

### Calculating the dRSS by hand

Using the Free space equation, the power received by the Victim link receiver (dRSS) can be easily derived:

dRSS = Pe + Ge + Gr - L	(Eq. 17)

dRSS = 30 + 9 + 9 - (32.5 + 10 log(8) + 20 log(1000))

dRSS = -53.5 dBm

Keep this calculation in mind, as it will be compared with what SEAMCAT calculates in the following sections.

### Export/import your system to library

When setting the system iscomplete, it is possible to export it to the library, so it can be reused in a future point of time..

Figure 82: Example of importing/exporting a system to library

### Selecting the victim in the Scenario

Now it’s time to create the victim link. The only thing needed is to select asystem to be used as  victim link as shown in (#1) of Figure 83 under the “scenario” tab.

Note that the frequency field at the “scenario” tab level overwrites what was predefined at “system” tab level (#2).

Figure 83: Setting the system of your choice to be the victim link in the “scenario” tab

## Calculating the iRSS

In this section, you will learn how to ...

Position the VLR vs It (i.e. Victim link vs Interfering link)

✓

Calculate the iRSS

✓

Relation between Emission bandwidth, reference bandwidth and VLR bandwidth

✓

Define Interefering link transmitter (It)

- Tx power

- Antenna characteristics

- Set the emission bandwidth

✓

### Interfering Links

The characteristics of the interefering system summarised in Table 7 should be entered into the SEAMCAT simulation scenario. For this example  only one interfering link will be simulated.

Table 7: Characteristics of the interfering link pair of transmitter and receiver

| Parameters | Value | Units |
| --- | --- | --- |
| Operating frequency | 1000 | MHz |
| Transmitter power | 33 | dBm |
| Emission bandwidth | 200 | kHz |
| Reference bandwidth | 200 | kHz |
| Tx antenna type | Omni directional |  |
| Tx antenna gain | 11 | dBi |
| Tx antenna height | 30 | m |

### System to be an interferer

Contrary to the victim link, is possible to set more than one interfering link in your simulation. A given simulation workspace must contain at least one interfering link.

First edit the technical characteristics of the system(s) that will be used as interferer(s). It is possible to generate many links with the same system or with different systems. Figure 84 presents an example with 3 interfering links having each a different technical characteristic.

Figure 84: Example of generating multiple Interfering links

The frequency of 1000 MHz will be finally set at the “scenario” tab level (#1).

Figure 85: Setting up the operating frequency of the interfering link

### Transmitter

Set now the victim link transmitter by selecting the transmitter tab:

the Interefering link transmitter uses a Power of 33 dBm; (#1 of Figure 86)

the Interefering link transmitter uses an omni-directional antenna of 11 dBi gain; (#2)

the Interefering link transmitter uses an antenna height of 30m; (#3)

the Interefering link transmitter uses an emission bandwidth of 200 kHz and a reference bandwidth Bref = 200 kHz. (#4)

The emission bandwidth of 200 kHz is defined through the emission mask (see Figure 86). This interefering link transmitter emission mask is defined in dBc. Then, enter an attenuation given in a reference bandwidth, the corresponding power is derived using the following equation:

P(dBm/Bref) = Pe (dBm)+Att(dBc/Bref)	(Eq. 18)

Where Pe is the power of the Interefering link transmitter within the emission bandwidth (also know as the in-band power). The sign of Att(dBc/Bref) is explained in section ‎A7.5. Then, in this example, within the emission bandwidth  (200kHz- offset between –0.1 MHz and 0.1 MHz), the power is 33 dBm, if the reference bandwidth is supposed to be equal to the emission bandwidth then Att = 0 dBc/Bref, this gives:

33 (dBm/200kHz) = 33 +0 (dBc/Bref)

The attenuation in dBc should be taken equal to 0 dBc/200 kHz (the link between the mask given in a reference bandwidth and the mask defined in 1 MHz is explained in ‎ANNEX 6:).

Figure 86: Setting up the interefering link transmitter

Figure 87: Spectrum emission mask settings

### Receiver

For this exercise, the Power Control is not activated, therefore there is no need to fill in any information about the Interfering link receiver.

### Positioning of the ILT vs ILR

Positioning of the ILT vs ILR is similar to positioning VLT vs VLR as described in Section ‎3.2.5.

This is required when the power control in the interfering link is simulated. It is also important to set it in case the geometry of the scenario is dependant on the position of the ILR (for instance ILR being at the center of the ILT distribution, or the victim link being fixed with respect to the ILR),

### Positioning of the VLR vs ILT

SEAMCAT allows defining the relative location between the Victim link receiver and the Interefering link transmitter as illustrated in Figure 88. The input parameters are detailed in section ‎10.3, and the algorithms are detailed in ‎ANNEX 13:.

Uncorrelated cases mode

None

(nactive interferers)

ILT /VLR

Correlated cases mode

ILT/VLT

VLT /VLR

ILR/VLT

Uniform density (nactive interferers)

Closest

(single interferer)

VLR ↔ ILT

location

Figure 88: Summary of the VLR ↔ ILT location capability in SEAMCAT

In this exercise, the distance between the Interefering link transmitter and the Victim link receiver as shown in Figure 89 is fixed by using the correlated distance as illustrated by Figure 90.

Victim Link

Receiver

(VLR)

Interfering Link

Receiver

(ILR)

Interfering

Transmitter

(ILT)

Victim Link

Transmitter

(VLT)

dRSS

iRSS

Victim link

Interfering link

Delta X = 4 km

Delta Y

= 4 km

Figure 89: Distance between the Interefering link transmitter and the Victim link receiver

Figure 90: Excample of setting up the distance between the Interefering link transmitter and the Victim link receiver in SEAMCAT. Here the ILT is positioned relative to the VLR

### Calculating the iRSS by hand

Figure 91: Illustration of SEAMCAT display (only 1 snapshot) of the various pair of transmitter and receiver and the dRSS and iRSS relationship

The attenuation between the Interefering link transmitter and the Victim link receiver is simulated by using the free space model (Variation should be disabled). When the simulation is finished, SEAMCAT presents the positioning of the various pairs of transmitters and receivers as shown in Figure 91(only one snapshot is illustrated).

Using these assumptions, it is possible to derive the interfering power received by the Victim link receiver iRSS:

iRSS = Pe + Ge + Gr - L 	(Eq. 19)

iRSS = 33 (dBm) + 11 + 9 - (32.5 + 10 log(32) + 20 log(1000))

iRSS = -54.5 dBm

### Emission, reference, VLR bandwidth relationship and bandwidth correction factor

The proposed exercise considers the case where the VLR bandwidth is the same as the emission bandwidth and the reference bandwidth.

This section aims at illustrating the interaction between the emission bandwidth (ItBW), reference bandwidth (Bref), VLR bandwidth (VLRBW) and any bandwidth correction factor. This section provides examples where these values are different and its’ effect on the iRSS calculation.

Relationship between Emission bandwidth and Reference bandwidth

For a fixed VLRBW = 200 kHz, and fixed ItBW= 200 kHz, the attenuation Att(dBc/Bref) will be different depending on the values of the Bref in order to achieve the same interference power level.

Case 1, ItBW > Bref :

Bref = 100 kHz, with Att = -3 dBc/Bref, iRSS =  -51.48 dBm;

As mentioned in Section ‎A7.5 on p.298, if the reference bandwidth is lower than the emission bandwidth then the attenuation must be defined with negative sign;

Case 2, ItBW = Bref :

Bref = 200 kHz, with Att = 0 dBc/Bref, iRSS =  -54.49 dBm;

If the reference bandwidth is equal to the emission bandwidth then the attenuation should be set  as zero.

Case 3, ItBW < Bref :

Bref = 400 kHz, with Att = 3 dBc/Bref, iRSS =  -57.5 dBm;

If the reference bandwidth is larger than the emission bandwidth then the attenuation must be defined with positive sign;

Relationship between Emission bandwidth and Victim link receiver bandwidth

For a fixed Bref = 200 kHz and a fixed ItBW= 200 kHz, depending on the size of the VLRBW a bandwidth correction factor is applied or not.

Case 4, ItBW = VLRBW:

VLRBW = 200 kHz, iRSS =  -54.49 dBm;

Case 5, ItBW > VLRBW:

VLRBW = 100 kHz, iRSS =  -57.5 dBm;

As shown in ‎ANNEX 22: on p. 446, when the ItBW > VLRBW, the interfering power in the VLR is reduced due a bandwidth correction factor automatically applied in SEAMCAT. As a results, the iRSS value decreases compare to a case where ItBW = VLRBW.

Case 6, ItBW < VLRBW:

VLRBW = 400 kHz, iRSS =  -54.49 dBm;

As illustrated in ‎ANNEX 22:, when the ItBW < VLRBW, there is no bandwidth correction factor applied to the interfering emitted power since the all the energy is “seen” by the VLR. Therefore the iRSS is equal to the case where ItBW = VLRBW. Note also that since the emission bandwidth of the interferer is smaller than the victim, SEAMCAT will complain that the spectrum emission mask of the interferer is not defined with the range of the victim bandwidth, therefore you need to increase the out of band emission.

Figure 92: Illustration of the emission spectrum mask with respect to the VLR bandwidth in case 6

Case 7: This is the same as case 6 (VLR = 400 kHz and Bref = 200 kHz) except that the  spectrum emission mask (emission bandwidth 200 kHz) has slopes on both sides (Figure 93) which generates higher interference compared to the case 6  and the iRSS = -53.44 dBm.

Figure 93: Illustration of case 7 and the “extra” interfering energy to the VLR due to 
the slope in the emission mask

## Launching a simulation

A simulation is now ready to be launched. It should be noted that prior to beginning the simulation, SEAMCAT checks the consistency and suitability of certain input parameters (see ‎ANNEX 19:).

Since the default operating frequency of the Interefering link transmitter is 900 MHz and the unwanted emission mask is not within the range of the Victim link receiver, SEAMCAT automatically complains and displays a warning window.

Figure 94: Consistency check warning being prompted because the operating 
frequency of the interferer is not set-up properly

Therefore, it’s better to select “No - Cancel simulation and let me correct errors” and define the operating frequency (1000 MHz) of the interfering link. These parameters do not affect the calculation of the dRSS. It is only to ensure that SEAMCAT does not experience any exceptions when running.

## Extract results vectors (dRSS, iRSS etc…)

You can extract the dRSS vectors from the “Results” panel in the newly create workspace results in Figure 95.

Figure 95: Results panel to extract any of the vectors generated by SEAMCAT

Figure 96: Example of the dRSS vector

When double clicking on a result vector field (Figure 36 (#1)) the display panel as shown in Figure 96, offers three options to display the results in vector (limited to 20.000 events), CDF and PDF format.

In addition,  some statistical information can be displayed such as average, median, standard deviation (StdDev), variance, min and max. Further information about these statistics (i.e. how they are computed) are to be found in Section ‎A1.5.

## Calculating the probability of interference

You will learn how to ...

Compatibility calculation mode

✓

Extract the Probability of Interference in ....

✓

Translation calculation mode

✓

After the simulation of all events has completed, SEAMCAT will have calculated and stored the dRSS and iRSS vectors. Now it is possible to evaluate the probability of interference for the simulated scenario using the interference calculation control panel as shown in Figure 97.

Figure 97: Interference calculation tabsheet

Any of the following parameters can be selected from the panel when calculating the probability of interference:

Calculation mode: compatibility or translation;

Which type of interference signal is considered for calculation: unwanted, blocking, overloading, intermodulation or a combination of them;

Interference criterion: C/I, C/(N+I), (N+I)/N or I/N:

### Compatibility calculation mode

It is then possible to derive the C/I (i.e. dRSS/iRSS equivalent to dRSS-iRSS in dB):

dRSS/iRSS = -53.5-(-54.5) = 1 dB

Since the resulting C/I is below the protection criteria (19 dB), the probability of interference calculated by SEAMCAT (compatibility calculation mode) is equal to 1 as shown in .

### Translation calculation mode

When the Translation mode is chosen, it is possible to calculate and display a chart of the probability of interference as a function of one of the following input parameters:

Output power of Interefering link transmitter;

Blocking response level of Victim link receiver;

Intermodulation response level of Victim link receiver.

The translation function, as shown as (#1) Figure 98 in, allows investigation of the probability of interference for varying power supplied (#2) to the interefering link transmitter. The power supplied (#3) to the interefering link transmitter should be equal to 15 dBm, which is 18 dB below the value used in the simulation (33 dBm). Effectively the C/I will be increased by 18 dB and reaches the level of 19 dB.

Figure 98: Translation function

[Editor’s note: Figure above to be modified]

### Example of probability of interference calculation

The protection criteria to be used for the calculation of the probability of interference and the type of interference to be considered (unwanted and/or blocking) can be equally chosen (see Figure 97).

Using the unwanted mode, it is possible to derive the C/I:

dRSS/iRSSunwanted = -53.5- (-77.5) = 24 dB

Since the resulting C/I is above the protection criteria (19 dB), the probability of interference calculated by SEAMCAT (Interference calculation) is equal to 0).

The same conclusion is reached by using the C/(N+I) criteria. It should be noted that SEAMCAT performs a consistency check (‎ANNEX 19:) between the interference criteria (‎ANNEX 3:).

It is also possible to derive the (N+I)/N= -77.5-(-100)= 22.5 (since I>>N). Since the (I+N)/N which is obtained is above the protection criteria (3 dB), the probability of interference calculated by SEAMCAT (Interference calculation) is equal to 1).

The same conclusion is reached by using the I/N criteria.

Using the blocking interference mode (protection ratio) it is then possible to derive the C/I:

dRSS/iRSSblocking = -53.5 - (-113.5) = 60 dB

Since the C/I obtained is above the protection criteria (19 dB), the probability of interference calculated by SEAMCAT (Interference calculation) is equal to 0).

It is also possible to derive the (N+I)/N= -113.5-(-100)= -13.5. Since the (N+I)/N which is obtained is below the protection criteria (3 dB), the probability of interference calculated by SEAMCAT (Interference calculation) is equal to 0  ).

(

Then, by taking into account the sum of the two signal types, the probability of interference becomes equal to 1 (due to the unwanted emissions)

# Example of calculations

This section presents examples on how to use SEAMCAT to calculate Interference mechanism described in ‎ANNEX 5:.

## iRSSunwanted

The following parameters should be changed in the simulation: (i) the interferer operates at 1000.3 MHz and (ii) outside the emission bandwidth, the attenuation is -23 dBc/200 kHz.

The corresponding power may derived using the known equation:

P(dBm/Bref)=Pe (dBm)+Att(dBc/Bref)	(Eq. 20)

Then, in this example, outside the emission bandwidth (offset between –0.1 MHz and -1 MHz and between 0.1 MHz and 1 MHz), the power is equal to:

P = 33 +(–23(dBc/Bref)) = 10 (dBm/200 kHz)

The complete unwanted emission mask is provided in .

Figure 99: Unwanted emission mask

Using the previous assumptions, it is possible to derive the interfering power received by the Victim link receiver within its bandwidth as described in ‎ANNEX 5: on page 285. This is called the iRSSunwanted:

iRSSunwanted = P + Ge + Gr - L 	(Eq. 21)

iRSSunwanted = 10 (dBm/200 kHz) + 11 + 9 - (32.5 + 10 log(32) + 20 log(1000))

iRSSunwanted = -77.5 dBm

This can be checked by running a simulation and displaying the iRSSunwanted signal as depicted in Figure 100.

|  |  | In this example there is no bandwidth correction factor to be applied to the calculation of the iRSSunwanted since the VLR bandwidth and the ILT reference bandwidth have the same value (i.e. 200 KHz). Examples of correction bandwidth can be found in section ‎3.3.8 |
| --- | --- | --- |

Figure 100: Mean iRSSunwanted

## iRSSblocking

For this exercise, the blocking response from the receiver has a positive sign as shown in Figure 101. Detailed information on the calculation of the iRSSblocking can be found in section ‎A5.2 of ‎ANNEX 5:.

Figure 101:Definition of the receiver blocking response

### User-defined mode

In this case, the Blocking is provided in dB and represents the attenuation of the receiver at a given frequency offset (see Figure 12). The resulting receiver attenuation equals the user-defined input values. Then, the iRSSblocking at the interferer operating frequency  may be calculated as follows.

(Note: The ILT bandwidth is not considered in the iRSSblocking  calculation):

iRSSblocking (fit) = Pe + Ge + Gr - L - Att (fit)	(Eq. 22)

iRSSblocking = 33 (dBm) + 11 + 9 - (32.5 + 10 log(32) + 20 log(1000)) - 40(dB)

iRSSblocking = -94.5 dBm

This can be checked by running a simulation and displaying the iRSSblocking in case of User-defined mode calculated by SEAMCAT. See the figure below.

Figure 102: Mean iRSSblocking in case of User-defined mode

### Sensitivity mode

In this calculation mode the function blockMax Interf Signal (f) that you entered represents the absolute power level (in dBm) of maximum interfering signal (maximum acceptable interfering power), which might be tolerated by the receiver at a given frequency separation (see Figure 12).

In this case SEAMCAT calculates the receiver attenuation, Attenuation (f), to be applied to the interfering signal by using the following expression:

Attenuation(f) = blockMax Interf Signal (f) (dBm) - sensVLR (dBm) + C/(N+I) (dB) – I/N (dB)	(Eq. 23)

where:

f fILT - fVLRis the frequency separation;

sensVLR is the sensitivity of the VLR (dBm) as defined in the simulation scenario.

To achieve a realistic value, you may define the sensitivity (sensVLR) as (see the figure below):

Sensitivity = Noise Floor + C/(N+I)

Sensitivity = -110 dBm + 16 = -94 dBm

Figure 103: Setting up the sensitivity in SEAMCAT

Then the attenuation may be evaluated:

Attenuation (f ) = 40 + 94 + 16 - 0 = 150 dB

iRSSblocking = Interfering Signal Level (f it )= -54.5 - 150 = -204.5 dBm

This can be checked by running a simulation and displaying the iRSSblocking in case of Sensitivity mode calculated by SEAMCAT see Figure 104.

Figure 104: Mean iRSSblocking in case of Sensitivity mode

### Protection ratio

This mode is identical to the “sensitivity” mode since the only difference is that the Blocking value (relative to the noise floor) is provided in dB. The software processes the information using exactly the same method to obtain the value of the receiver attenuation (see Figure 12).

The function blockProtection Ratio(f) that you entered represents the protection ratio, i.e. the ratio of maximum acceptable level of interfering signal to the wanted signal level, at a given frequency separation.

In this case SEAMCAT calculates the receiver attenuation aVLR(f) to be applied to the interfering signal by using the following expression (see Figure 105):

Attenuation (f) = blockProtection Ratio(f) (dB) + C/(N+I) (dB) + (N+I)/N (dB) – I/N (dB)	(Eq. 24)

Attenuation (f  ) = 40 + 16 + 3 - 0 = 59 dB

iRSSblocking = Interfering Signal Level (f it ) = -54.5 - 59 = -113.5 dBm

Figure 105: Mean iRSSblocking in case of Protection-ratio mode

## iRSSintermodulation

The following graphics presents the input paremeters to activate when the intermodulation mechanism is to be investigated. The intermodulation rejection mask is expressed in dB versus frequency offset (MHz). Results details can be found in Section ‎12.3.3.

Figure 106: Activation of the intermodulaion feature in SEAMCAT

## iRSSoverlaoding

The following graphics presents the input paremeters to activate when the overloading mechanism is to be simulated.

Figure 107: Activation of the overloading feature in SEAMCAT

The overloading mask is expressed in power (dBm) versus frequency offset (MHz) and is defined as a function in SEAMCAT.

Figure 108: Setting of the overloading mask in SEAMCAT

The filtering of the receiver is expressed in power (dB) versus frequency offset (MHz) and is defined as a function in SEAMCAT (see Figure 109). It is set by default to a constant value of zero.

Note that if the blocking attenuation mode in user-defined and the overloading feature have been selected, then a consistency check will remind you that the actual blocking response and the receiver filter are the same element and they should not be accounted twice.

Figure 109: example of filling the Rx filter function

When a simulation is run the following results are extractable (see Section ‎12.3.4 for further details)

As an example, this means that for the following overloading mask presented in Figure 3 with a victim frequency of 908 MHz and an interfering frequency of 900 MHz, the delta frequency (i.e. frequency offset) is -8 MHz with an overloading treshold of -8.5 dB and a Rx filter value of 9 dB. This can be found from the log file as shown below in Figure 110.

Figure 110: Output from logfile

## Coverage radius (VLR-VLT)

In this section the Free Space Model is used to derive the attenuation on the different paths and the Victim Link and Interfering Link operate at the same frequency – 1000 MHz – co-channel interference. Figure 111 presents an illustration of the various radii (i.e. coverage radius and simulation radius) that are used in SEAMCAT and with reference to the various pairs of transmitters and receivers used for a simulation.

Victim Link

Receiver

(VLR)

(Origin of simulation)

Interfering Link

Transmitter

(ILT)

(Origin of

interfering

coverage)

Victim Link

Transmitter

(VLT)

(Origin of

victim

coverage)

dRSS

Victim

Receiver

iRSS

Victim link

Coverage radius

Interfering Link

Receiver

(ILR)

Interfering link

Coverage radius

Simulation radius

dRSS

Interfering link receiver

Figure 111: Illustration of the Coverage Radius and the Simulation Radius with respect to the pairs of transmitters and receivers of the victim and interfering links

The distance between the VLT and the VLR is referred to as the coverage radius () (see ‎ANNEX 13:). In the case of mobile applications, the number of terminals that may transmit in a given cell of the network is given by:

(Eq. 25)

The calculation of the coverage radius can be found in ‎ANNEX 13:.

The cluster characterises the frequency re-use of a mobile network (see Figure 112).

Figure 112: Frequency cluster

The victim link coverage radius (i.e. centred on the victim link transmitter) may then be calculated by using the formula below. Figure 113´presents how to set-up SEAMCAT.

(Eq. 26)

Figure 113: Calculation of the victim link coverage radius

= 1.42 km 	(Eq. 27)

Figure 115 presents the results of the dRSS vector and the results in the coverage radius of the victim link.

The dRSS for a receiver located at the edge of the coverage area may be calculated:

dRSS = 30 (dBm) + 9 + 9 - (32.5 + 10 log(1.43^2) + 20 log(1000)) = -47.5 dBm

Figure 114: Resulting dRSS distribution

Figure 115: Calculation results of the victim link coverage radius

Figure 116: Illustration of the VLR and VLT positioning based on the traffic limited network coverage radius (ILT-red, ILR-green, VLR-yellow, VLT-blue)

## Simulation radius (ILT-VLR)

The distance between the Victim link receiver and the Interefering link transmitter is referred to as the simulation radius (see ‎ANNEX 13:). It can be defined as shown in Figure 117.

Figure 117: Definition of the minimum distance

Using this feature, the interefering link transmitter is located between 1 km (0.1 x 10 km) and 10 km (1 x 10 km) from the Victim link receiver as shown in the SEAMCAT display in Figure 118.

Figure 118: SEAMCAT display of the minimum distance

(ILT-red, ILR-green, VLR-yellow, VLT-blue)

If the interefering link transmitter is located at 10 km, it is possible to derive the iRSSunwanted:

iRSSunwanted = 33 (dBm) + 11 + 9 - (32.5 + 10 log(10 x 10) + 20 log(1000))

iRSSunwanted = 53 - 32.5 = 20.5 - 60 - 20 = -59.5 dBm

If the interefering link transmitter is located at 1 km:

iRSSunwanted = 33 (dBm) + 11 + 9 - (32.5 + 10 log(1) + 20 log(1000))

iRSSunwanted = 53 - 32.5 = 20.5 - 60 = -39.5 dBm

The iRSSunwanted extends from -59.5 dBm to -39.5 dBm as confirmed by Figure 119.

Figure 119: iRSSunwanted

## Uniform distribution of ILT vs VLR

You may define a uniform deployment density of terminals/transmitters per km2 (see ‎ANNEX 13:). This can either be done by using the “Uniform Density” mode combined with the calculation of a simulation radius as described in section ‎4.7.1 and ‎A13.2 or by using the “none” mode combined with the choice of uniform path polar distance as described in section ‎4.7.2.

### Uniform density mode - simulation radius calculation

The number of active transmitters that will be uniformly located within the simulation radius is given by:

(Eq. 28)

Figure 120 presents the GUI with the input value.

Figure 120: Setting up the simulation radius in SEAMCAT
(Note that the results of the simulation radius is displayed only after running simulation)

The simulation radius is calculated by using the following formula:

(Eq. 29)

In the example of Figure 120, this gives:

= 0.178 km

Figure 121: iRSS using the simulation radius for 1 active transmitter (R=0.178)

Figure 122: Calculated simulation radius with 1 active transmitter

Then for a single interefering link transmitter located at the edge of the simulation radius (R = 0.178 km), the iRSSunwanted may be calculated:

iRSSunwanted = 33 (dBm) + 11 + 9 - (32.5 + 10 log(0.178^2) + 20 log(1000)) = -24.5dBm

When increasing the number of active transmitters to 10 (see Figure 123), the simulation radius becomes:

= 0.564 km

Then, for a single interefering link transmitter located at the edge of that simulation radius (R = 0.564 km), the iRSSunwanted resulting from this terminal may be calculated as:

iRSSunwanted = 33 (dBm) + 11 + 9 - (32.5 + 10 log(0.564^2) + 20 log(1000)) = -34.5dBm

If 10 active terminals are located at the edge of the simulation radius, the iRSSunwanted may be calculated in the following way:

iRSSunwanted = -34.6 (dBm) + 10 log(10) = -24.5 dBm

Then the iRSSunwanted should be above -24.5 dBm as shown on Figure 124.

Figure 123: Setting up the simulation radius in SEAMCAT with a different 
number of active transmitters

Figure 124: iRSS using the simulation radius calculated for 10 active interferers (R=0.564 km)

Figure 125: Calculated simulation radius with 10 active transmitters

### “None” mode

When you select the “None” mode (see ‎ANNEX 13:), he can also define a Uniform density of terminal/transmitter by using the Uniform polar distance defined within the path distance factor and a uniform distributed path azimuth (0 to 360 deg).Uniform polar distance leads to uniformly distributed terminals in the circular area (area) and the Uniform distance leads to uniformly distributed terminals along the radius (line).

Then using a user-defined radius of 0.178 km and 1 interefering link transmitter (see Figure 126), it is possible to reproduce the results of section ‎4.7.1. Therefore, the same results as those given in Figure 121 are found as shown in Figure 127.

Figure 126: Uniform density using “None” mode

Figure 127: iRSSunwanted / Uniform polar feature / 1 interefering link transmitter and simulation radius of 0.178 km (same results as in Figure 121)

Using a user-defined radius of 0.564 km and 10 interefering link transmitters, the same results as those given in Figure 124 are reached as shown in Figure 128.

Figure 128: iRSSunwanted / Uniform polar feature / 10 interefering link transmitter 
(same results as in Figure 121)

## Protection distance

If a minimum protection distance between the victim link receiver and interefering link transmitter is introduced then the simulation radius may then be calculated by using the following formula:

(Eq. 30)

Note: The calculation of the iRSS at each trial will be performed only if the location of the ILT satisfies the following condition .

Figure 129 presents how to set-up SEAMCAT (See also ‎A13.2 for further details).

Figure 129: Setting up the protection distance related to the simulation radius with 1 active interferer

The simulation radius (see Figure 131) may then be calculated by using the following formula:

= 0.531 km

Figure 130: Calculated simulated radius

Figure 131: Calculation of the simulation radius and the iRSSunwanted using the protection distance (for 1 active interferer)

If the distance between the VLR and the ILT is equal to the protection distance (0.5 km), then:

iRSSunwanted = 33 (dBm) + 11 + 9 - (32.5 + 10 log(0.5 x 0.5) + 20 log(1000)) = -33.5dBm

If the distance between the VLR and the ILT is equal to the simulation radius (0.531 km), then:

iRSSunwanted = 33 (dBm) + 11 + 9 - (32.5 + 10 log(0.531 x 0.531) + 20 log(1000)) = -34 dBm

These results are in line with those depicted on Figure 131. We can then increase the number of active transmitters to 10 as shown in Figure 132.

Figure 132: Setting up the protection distance related to the simulation radius with 10 active interferer

The simulation radius may then be calculated by using the following formula (see Figure 133 for the result):

= 0.75 km

Figure 133: Calculation of the Rsimu and the iRSSunwanted using the protection distance 
(for 10 active interferer)

If the distance between the VLR and a single ILT is equal to the protection distance, then:

iRSSunwanted = 33 (dBm) + 11 + 9 - (32.5 + 10 log(0.5 x 0.5) + 20 log(1000)) = -33.5 dBm

If the distance between the VLR and a single ILT is equal to the simulation radius, then:

iRSSunwanted = 33 (dBm) + 11 + 9 - (32.5 + 10 log(0.75 x 0.75) + 20 log(1000)) = -37 dBm

If 10 transmitters are deployed around the VLR then:

iRSSunwanted = -37 (dBm) + 10 log(10) = -27 dBm and iRSSunwanted = -33.5 (dBm) + 10 log(10) = -23.5 dBm

-27 dBm < iRSSunwanted < -23.5 dBm (see Figure 133)

## Power Control

Note: The power control is used only in this section. When considering other sections, the power control feature should be deactivated.

A power control feature is implemented within SEAMCAT. When this feature is activated it introduces a variation of the interefering link transmitter power. More details on the implementation of the power control in SEAMCAT is presented in ‎ANNEX 14:.

When using the scenario provided in section ‎3 (Victim Link and Interfering Link operate at the same frequency 1000 MHz but with a interfering link receiver antenna gain should be equal to 11 dBi), it becomes possible to define the distance between the Victim link receiver and the Interefering link transmitter.

For simplification, we consider that the Victim link receiver and the Interefering link transmitter are defined using the following assumptions (1 km distance between the Victim and the Interefering link transmitter). This is illustrated in Figure 134.

Figure 134: Setting up the distance between the Interfering and the Victim pair of 
transmitter and receiver

If the power control is not activated then the iRSSunwanted is (see Figure 135):

iRSSunwanted = 33 (dBm) + 11 + 9 - (32.5 + 10 log(1) + 20 log(1000)) = -39.5 dBm

Figure 135: iRSSunwanted without power control

The same assumption is used for the distance between ILT and the ILR (i.e. x = 0 km and y = 1 km) where the ILT is the (0,0) origin as shown in Figure 136.

Figure 136: Setting up the distance between the Interefering link transmitter and the Interfering link receiver to evaluate the dRSSInterfering link receiver.

The power received by the Interfering link receiver (dRSSInterfering link receiver) is then:

dRSSInterfering link receiver = 33 (dBm) + 11 + 11 - (32.5 + 10 log(1) + 20 log(1000)) = -37.5 dBm

|  | You should not confuse the dRSSInterfering link receiver and the dRSSVictim link receiver. Figure 111 illustrates their differences. |
| --- | --- |

The power control can be input as shown in Figure 137.

Figure 137: Setting up the power control for the interfering link.

Using these assumptions, the results of the simulation are the same as previously since the threshold (-35 dBm) is above the dRSSInterfering link receiver(-37.5 dB). This corresponds to the Case 1 described in ‎ANNEX 14:.

Now, if the threshold is decreased from –35 dBm to –50 dBm (see Figure 138)

Figure 138: Reduction of the minimum threshold in the PC

Since, the power control feature is activated the gain of the power control is determined according to the guidance given in ‎ANNEX 14: on page 345. Since the dRSSInterfering link receiver is –37.5 dBm and:

-50 dBm (PC threshold) + 2 * PC step (5 dB) < –37.5 dBm < -50 dBm (PC threshold) + 3 * PC step (5 dB)

The gain of the power control (git PC) is 10 dB (Case (i+1) in ‎ANNEX 14:). This means that the iRSSunwanted will be decreased by 10 dB, i.e. :

iRSSunwanted = -39.5 dBm - git PC  = -49.5 dBm

Finally, if the threshold is decreased from –50 dBm to –70 dBm, the power control feature is activated. Since the dRSSInterfering link receiver is –37.5 dBm, this results in:

-70 dB (PC threshold) + 20 (PC dyn) < –37.5 dBm

The gain of the power control is 20 dB (Case (n_step + 2) in ‎ANNEX 14:). This means that the iRSSunwanted will be decreased by 20 dB, i.e. :

iRSSunwanted = -39.5 dBm - git PC  = -59.5 dBm

## Antenna gain

For simplification, we consider that the Victim link receiver and the Interefering link transmitter are defined using the following assumptions (again 1 km distance between the Interefering link transmitter and the Victim link receiver).

The iRSSunwanted is then:

iRSSunwanted = 33 (dBm) + 11 + Gr - (32.5 + 10 log(1) + 20 log(1000)) = -48.5 dBm + Gr

Figure 139: Definition of the antenna gain

In order to investigate the evolution of the iRSS versus the antenna radiation pattern with fixed location of the pairs of transmitters and receivers (i.e. to get random angle arrival and consequently random gain of the antenna radiation pattern), it is possible to rotate artificially the antenna pattern defined in Figure 139 in the azimuth domain. This can be done by rotating the antenna from 0 to 360 by applying a uniform distribution from 0 to 360 deg to the main beam direction (0 deg).

Figure 140: Setting up the rotation in the azimuth domain

The receiver antenna gain extends from 9 dB + (0  to -50 dB) meaning that it varies from +9 dB to -41 dB depending on the azimuth angle (Figure 140), the iRSSunwanted is then :

-89.5 dBm < iRSSunwanted < -39.5 dBm

Results generated by SEAMCAT are presented in Figure 141.

Figure 141: iRSSunwanted

# Generic module

As mentioned earlier in this manual, virtually all radio interference scenarios on terrestrial paths can be addressed in both co-channel (sharing) and adjacent frequency (compatibility) interference studies. A number of various radiocommunications services can be modelled using the generic module:

Broadcasting: Terrestrial systems and earth stations (e.g. DTH receivers) of satellite systems;

Fixed services: Point-to-point and point-to-multipoint fixed systems;

Mobile Services: Land mobile systems, short range devices and earth based components of satellite systems.

This flexibility is achieved by the way the system parameters are defined as constant values or random variables  through their distribution functions. It is therefore possible to model even very complex situations by relatively simple variations of some elementary functions. This section explains the use of these parameters in SEAMCAT calculations, where appropriate.

## Generic system tab

This tab allows you to update all scenario parameters of for a generic system .

In the upper part of the tab, you have three panels.

Figure 142: Generic system

### System

You can name and write some description of the victim system you want to simulate.

### General

You can enter the frequency. The frequency value is overwritten at the “Scenario” tab level.

Table 8: Generic system – General panel

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Frequency | f | Distribution or Scalar | MHz | Distribution of the centre frequency of the system |

You have access to 3 sub tabs to set the generic system:

Receiver: This will be used either as VLR or ILR;

Transmitter: This will beused either as VLT or ILT;

Transmitter to Receiver Path (i.e. either VLT  VLR or ILT  ILR).

## Receiver

It can be the VLR or the ILR as illustrated in Figure 143.

Figure 143: Receiver illustration as VLR or ILR

The receiver consists of 5 panels (Figure 144); Receiver identification, antenna pointing, antenna patterns identification, reception characteristics and interference criteria.

Figure 144: Receiver GUI

### Receiver identification

This panel is a common interface that is reused in all other radio systems.

Table 9: Receiver identification

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Library | - | Library | - | Allows to import/export the receiver characteristics from/to the library to/from the workspace |
| Name | - | Free text | - | Freely chose a name and a description for the receiver. Remember that the settings can be exported to the library, so it is important that the description is accurate enough when reusing or sharing it.. |
| Description | - | Free text | - |  |

### Antenna pointing

This panel is a common interface that is reused in other radio system. It contains all information relative to the antenna other than the radiation pattern.

Table 10: Receiver antenna pointing GUI

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Antenna height | h | Distribution or Scalar | m | See ‎ANNEX 11: |
| Azimuth ref. 0 deg is pointing to the Tx | - | Boolean | - | When selected, the antenna (i.e. for an antenna azimuth distribution of 0°) is pointing by default at the VLT. If not selected, it looks EAST. |
| Antenna azimuth: Antenna alignment horizontal tolerance | H | Distribution or Scalar | degree | This is the angle between the Rx main beam and the direction to Tx. E.g. if antenna azimuth=0, the Rx and Tx antennas are strictly aligned in the horizontal plane, see ‎ANNEX 11: |
| Elevation ref. 0 is pointing to the Tx | - | Boolean | - | When selected, the antenna (i.e. for an antenna elevation distribution of 0°) is tilted by default towards the VLT. If not selected, it is set horizontal |
| Antenna elevation: Antenna alignment vertical tolerance | V | Distribution or Scalar | degree | This is the vertical angle between the Rx main beam and the direction towards Tx. E.g. if antenna elevation=0, the Rx and Tx antennas are strictly aligned in the vertical plane, see ‎ANNEX 11: on p. 323. |

### Antenna patterns identification

This panel is a common interface that is reused in all other radio system. It contains all information relative to the antenna radiation pattern:

Table 11: Antenna pattern identification

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Library | - | Library | - | Allows to import/export the antenna pattern from/to the library to/from the workspace |
| Name | - | Free text | - |  |
| Description | - | Free text | - |  |
| Antenna peak gain | G | Scalar | dBi | Antennae are implemented as plugins, therefore see ‎ANNEX 11: as separate guidance on the the radiation pattern. |
| Horizontal | - | Pattern | - | Pattern selection. See working range in ‎ANNEX 11:. |
| Vertical | - | Pattern | - | Pattern selection. See working range in ‎ANNEX 11:. |

### Reception characteristics

This panel consists in setting of the receiver characteristics of the generic system:

Table 12: Reception characteristics GUI

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Noise floor: define a distribution of the noise floor | N | Distribution or Scalar | dBm | See section ‎1.2.3 for further details |
| Blocking mode | - | Boolean | - | Blocking mode and associated blocking mask |
| Blocking response: Receiver frequency response (receiver blocking performance) | blocking | Function (X,Y) (MHz) | dBm or dB depend. on mode | Receiver mask attenuation (positive or negative values depending on the chosen blocking mode, see below) versus frequency, see ‎ANNEX 8: |
| Intermodulation rejection: Intermodulation response (intermodulation interference) | intermod | Function (X,Y) (MHz) | dB | Receiver mask at the intermodulation frequency. (see Annex ‎A5.3 for further details) |
| Receive power dynamic range | Pcmax | Scalar | dB | Used in the calculation of the dRSS. It is the maximum range of the received power that the VLR can accept, in terms of the maximum receive power over the VLR’s sensitivity threshold. If the trialled dRSS value exceeds sens+Pcmax , the dRSS is set to the latter value. See ‎ANNEX 14:. |
| Sensitivity | sens | Scalar | dBm/VLR reception bandwidth | Sensitivity of the receiver. See Section ‎1.2.4 |
| Reception bandwidth: Operating bandwidth | B | Scalar | kHz | Bandwidth of the receiver. |
| Overloading | - | Boolean | - | When selected, the overloading calclulation is performed. See Annex ‎A5.4 for further details. |
| Overloading threshold | Oth | Function (X,Y) (MHz) | dBm | It is the maximum interfering signal level close to which the receiver loses its ability to discriminate against interfering signals at frequencies differing from that of the wanted signal |
| Receiver filter | Rx_filter | Function (X,Y) (MHz) | dB | Filtering of the receiver (if any). The filtering of the receiver is by default 0 dB (similarly to the default blocking filtering value). It is used in connection with the overloading calculation. |

### Interference criteria

Section ‎1.4 presented the concept of interference criteria (C/I, C/(N+I), (N+I)/N, I/N) when the victim is a generic system. The consistency between these values falls under the responsibility of the user. It should be noted that only one criterion is used at a time in the final interference calculation.

It is important to remember that these parameters are also used in the evaluation of the two blocking modes (Protection ratio and Sensitivity) as presented in See section ‎1.4.5.

SEAMCAT performs a consistency checking between the interference criteria as explained in ‎ANNEX 3:.

Table 13: Interference criteria panel GUI

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Interference criteria | C/I C/(N+I) (N+I)/N I/N | Scalar | dB | At least one of these criteria should be defined: ( C/I, C/(N+I), (N+I)/N, I/N). Then, one of these criteria is chosen for each interference probability calculation. |
| Calculate interference criteria | - | calculator | - | This feature allows the evaluation of the consistency between the entered interference criteria ( C/I, C/(N+I), (N+I)/N, I/N) and proposes alternative values that can be selected to ensure consistency between them. |

The “calculate interference criteria” is a user friendly calculator for the interference criteria. It avoids conspicuous calculations, increases the transparency and avoids consistency check warnings because of the use of inconsistent sets of values.

The calculation and selection of a consistent set of interference criteria is implemented. The calculator opens with the existing values in the interference criteria fields of the workspace. A checkbox (default: ON) allows to force the consistency of the C/(N+I) value with the workspace, i.e. the values ‘Noise floor’ and ‘Sensitivity’. The relation between the values is given in the formula C/(N+I) [dB] = Sensitivity [dBm] – Noise floor [dBm]

The Interference Criteria Calculator displays all possible sets of consistent interference criteria rounded to two decimals. The same set is displayed only once. As a consequence of rounding, the displayed sets are differ at least 0.01 dB in one of the four values.

When you select the column of values you want and click ok, the values are copy/pasted to the “Interference criteria” panel.

Figure 145: : Interference criteria calculator GUI

## Transmitter

It can be the ILT or the VLT as illustrated in .Figure 146

Figure 146: Transmitter illustration as ILT or VLT

It consists in 4 panels (Figure 147); Transmitter identification, antenna pointing, antenna patterns identification, emission characteristics.

Figure 147: Transmitter GUI

### Transmitter identification

This is the same panel as in section ‎5.2.1 so that transmitter characteristics can be imported/exported from/to the library to/from the workspace and you can freely chose a name and a description.

### Transmitter antenna pointing

It contains all information relative to the antenna other than the radiation pattern. It is the same panel as for the receiver antenna pointing (Section ‎5.2.2). It points, in azimuth and in elevation, towards the Receiver.

### Antenna patterns identification

It contains all information relative to the antenna radiation pattern. It is similar to the receiver antenna patterns identification (Section ‎5.2.3).

### Emission characteristics

This panel consists in setting of the emission characteristics of your generic system.

Table 14: Emission characteristics GUI

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Power | P | Scalar or Distribution | dBm | This is the transmitter power supplied of the generic system |
| Interfere is CR: |  | Boolean |  | When the CR button is checked then it allows to set the emission characteristics of the VLT and ILT (used for the sRSS calculation only. See Section ‎6) |
| Emission mask: | emission_rel(f) | Function (X,Y) (MHz) | dBc/ reference bandw. (MHz) | Define the mask of the transmitter, in the emission bandwidth and out of the emission bandwidth. It is the unwanted signal level from the ILT. (See ‎ANNEX 7:) |
| Unwanted emissions floor: Noise floor signal level | emission_floor(f) | F unction (X,Y) (MHz) | dBm/ reference bandw. (MHz) | Define the minimum strength of the unwanted emissions. So the unwanted emissions equal to Max(PTx + Unwanted emission, Unwanted emissions floor) (see Annex ‎A7.4) |
| Power control |  |  |  | If Power control is checked, the 3 following parameters have to be defined. This Power control is used to limit the output power of the transmitter (see ‎ANNEX 14:) |
| Power control step size | PC step | Scalar | dB |  |
| Min threshold | PC threshold | Scalar | dBm/ emission bandw | If the received power is lower than this threshold, then no power control takes place |
| Dynamic range | PC dyn | Scalar | dB | If the received power is higher than Pc treshold + Pc dyn then the full power control takes place, i.e. the power is decreased by Pc dyn |

## Transmitter to Receiver Path

3 elements form the path between the VLR and the VLT or the ILR and ILT as illustrated in Figure 148.

Figure 148: Transmitter to Receiver Path illustration

Figure 149: Transmitter to Receiver Path GUI

### Relative location

When the Correlated distance option is checked, the positions of the receiver and transmitter are geographically fixed with respect to each other (e.g. co-located or constantly spaced base stations). The transmitter is considered a reference centre.

When the correlated distance is unchecked, the receiver is randomly moving around the transmitter. There are 2 primary options to define type of mutual placement of VLR with respect to VLT.

See ‎ANNEX 12: for further details on the algorithm and conventions.

Figure 150: Relative location panel

Table 15: Relative location GUI

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Correlation distance | - | Boolean | - | When checked, only the Delta X and Y are editable. |
| Delta X | X | Distribution | Km | Horizontal distance between the transmitter and receiver. It can be used to shift horizontally the distributed receivers. |
| Delta Y | Y | Distribution | Km | Vertical distance between the transmitter and receiver. It can be used to shift vertically the distributed receivers. |
| Path azimuth | - | Distribution | Degree | Horizontal angle for the location of the Rx respect to the Tx. If constant, the Rx’s location will be on a straight line. If not, the location of the Rx will be on an angular area. (See Annex ‎A1.1) |
| Path distance factor | - | Distribution | - | Distance factor to describe path length between the Tx and the Rx. If the path factor is constant, the Rx will be located on a circle around the Tx. (See Annex ‎A1.1) |
| Use of polygon | - | Boolean | - | When checked, it is possible to select a shape of a polygon. The default shape is the hexagon. |
| Shape of the polygon | - | Boolean | - | You can select between, heptagon, octagon , hexagon (default shape)pentagon , rectangle and triangle |
| Turn CCW | - | Distribution | degree | Allows rotating counter-clock wise the selected polygon |

### Coverage radius

A coverage radius is calculated for both the victim link and the interfering link. It is the for the victim link (VLR-VLT) and the for the interfering link (ILR-ILT) (see Annex ‎A13.1). The receivers will be randomly deployed within the area centred on the transmitter and delimited by the coverage radius if the non-correlated option is selected.

Three different modes are available for calculating the maximum radius .

User-defined radius allows directly entering the maximum radius (See Annex ‎A13.1.1);

Figure 151: User-defined coverage radius dialog box

Table 16: Description on User-defined coverage radius

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Coverage radius | Rmax | Scalar | km | The coverage radius defines the coverage of the system, i.e. the maximum distance between an ILT and a ILR or between a VLT and a VLR. The origin point of the coverage radius is logically the VLT or the ILT. |

The noise-limited network option will calculate the coverage radius based on the formula for noise-limited network. If this option is chosen, a set of input boxes will appear below allowing the user to enter specific parameters required for this calculation. In this case it is considered that the coverage of the transmitter is limited only by propagation losses and other elements in thelink budget, with received signal operating at the sensitivity limit. The details of the calculation are given in Annex ‎A13.1.2.

Figure 152: Noise limited network coverage radius dialog box

The coverage radius in the noise-limited network is defined by the parameters of Table 17. Note that the input parameters for the Noise-limited network interface are set to zero by default in order to independently define the radius from some parameters set elsewhere in the link.

Table 17: Description of the Noise limited network coverage radius user interface

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Reference antenna height (receiver): | h0 | Scalar | m | The height used for coverage radius calculations. If a distribution is used to define the real height, the coverage radius would be different in each trial, here the value may be fixed. |
| Reference antenna height (transmitter): | h0 | Scalar | m | The height used for coverage radius calculations. |
| Reference frequency | fVLR | Scalar | MHz |  |
| Reference power | PVLT | Scalar | dBm |  |
| Minimum distance |  |  | km |  |
| Maximum distance |  |  | km |  |
| Availability |  |  | % |  |
| Fading standard deviation |  |  | dB |  |
| Reference percentage of time |  |  | % |  |

The traffic-limited network option will calculate the coverage radius, based on the formula for traffic-limited network. If this option is chosen, a set of input boxes will appear below allowing user to enter specific parameters required for this calculation (See Annex ‎A13.1.3).

Figure 153: Traffic limited network coverage radius dialog box

Table 18: Description of the traffic limited network coverage radius user interface

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Density |  | Scalar | 1/km2 | Maximum number of active transmitters per km2 |
| Number of channels |  | Scalar |  | Number of frequency channels of the system |
| Number of users per channel |  | Scalar |  | Number of MS per frequency channel |
| Frequency cluster |  | Scalar |  | Size of a group of frequency channels. See Figure 176 for illustrative details. |

The consistency of this parameter should be verified against the sensitivity, so that if a receiver is placed at given distance (e.g. at the maximum coverage radius) the received power is higher than the sensitivity for a reasonable percentage of occurrences (availability).

### Local environment

The percentage of transceivers being indoor and outdoor can be selected thanks to this panel.. It will work in combination with the chosen propagation model that you will select. By default the transmitter and receiver are located outdoor. For each elements of the link, it is possible to add  or remove  a probablity of indoor.

Figure 154: Example of setting up the outdoor/indoor ratio

You can edit the field by double cliking

Figure 155: Graphical interface to edit the probability, wall loss and associated standard deviation

Table 19: Local environment and wall loss

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Local environment: Receiver | Indoor/ outdoor | - | - | Environment of the receiver antenna: outdoor, indoor It is used for both VLR and ILR |
| Local environment: Transmitter | Indoor/ outdoor | - | - | Environment of the transmitter antenna: outdoor, indoor It is used for both VLT and ILT |
| Probability | - | Scalar | % | Probability that a Tx or Rx is located indoors or outdoors |
| Wall loss | or | Scalar | dB | Attenuation of external walls separating indoor and outdoor propagation environments. This parameter is associated to the selected propagation model |
| Std. dev. | or | Scalar | dB | Wall loss stdandard deviation (indoor - outdoor) Wall loss standard deviation associated to the selected propagation model |

Note that when opening a workspace created prior to SEAMCAT version 5, all settings are mapped to the current SEAMCAT version running on your machine. As the parameter local environment didn’t exist before version 5, a warning may appear indicating that “local environments are skipped for multiple interefering links”. This means that SEAMCAT was not able to automatically set the paramters of the local environments (most likely due to a scenario with multiple interferers). Therefore, there is a need to edit the local environment manually.

Figure 156: Migration warning on the local environment

### Propagation Model

A suitable propagation model can be selected to be applied when calculating signal loss along the path between transmitters and receivers. Further information on propagation models are presented in detail in ‎ANNEX 17:.

# Spectrum sensing and Cognitive Radio

## Introduction

Cognitive radio is a technology which is being developed to bring greater efficiency, speed and reliability to users of wireless devices. The number of high powered wireless devices coming on to the market is increasing exponentially placing an unprecedented demand on the limited radio spectrum.  Cognitive radio technology is seen as a potential solution to this problem as it identifies unused frequencies in a local area and switches a device to this temporarily to provide users with uninterrupted and faster mobile services.

In the following, CRS refers to Cognitive Radio Systems. One typical example of usage of CRS is in the so called “white spaces” (WS), which refers to frequency spectrum which is potentially available at a given time for further

utilisation within frequency spectrum originally planned another service (e.g. Broadcasting). Devices operating in the white spaces using cognitive radio features are called White Space Devices (WSD) ‎[16].

## Simulating spectrum sensing

The manual sets out how SEAMCAT can be used for spectrum sensing where the interfering devices (ILT) try to detect the presence of protected services (e.g. the VLT) transmitting in each of the potentially available channels. Spectrum sensing essentially involves conducting a measurement within a candidate channel to determine whether any protected service is present and transmitting.  When a channel is determined to be vacant, sensing is typically applied to adjacent channels to identify what constraints there might be on transmission power, if any.

A key parameter for spectrum sensing is the detection threshold that is used by a cognitive device to detect the presence or the absence of a protected service’s transmission. If it detects no emission above this threshold in a channel, the white space device (WSD, i.e. the It) is allowed to transmit, otherwise the WSD keeps silent or look into other channels. You can study this phenomenon in SEAMCAT, which enables multiple cognitive radio systems. The cognitive radio feature mainly introduces the detection threshold and the selection of the operating frequency of the WSD.

In SEAMCAT, the CRSs are assumed to be the interferers. A SEAMCAT workspace will contain only 1 victim system and 1 or many interferers. It is possible to assess the aggregated impact of interferers that can be either CR devices or not. The scenario allows the impact of spectrum sensing to be investigated where a cognitive radio device is activated nearby a victim system. Both the victim and the interfering dialogue interface should be filled to enable spectrum sensing in SEAMCAT. Figure 157 illustrates that the introduction of spectrum sensing requires an extra budget link called sensing Received Signal Strength (sRSS) which represents the signal which is transmitted by the VLT and is sensed by the It. Note that the It acts as a transceiver, meaning that when the energy is sensed though the bandwidth of the sensing device (i.e. the It), it is acting as a receiving device.

The sRSS (considering the unwanted mask of the DTT) at the channel m is calculated as described in ‎ANNEX 6:.

It is assumed that the frequency of the interfering cognitive radio device is dependent on the frequency range defined for the victim. This means that when the CR module is activated, the interfering frequency function dialogue box is de-activated (#4 of Figure 231). Depending on how the victim frequency is defined (i.e. constant, discrete or distributed between fmin and fmax). SEAMCAT only allows the use of the following distributions: Constant, User defined, Uniform, User defined (stair).

sRSS1

(dBm)

iRSS 1

(from WSD 1)

(dBm)

dRSS

(from DTT)

(dBm)

VLT

ILR1

VLR

It1

Victim link

Interfering

Link #1

sRSS2

ILR2

It2

Interfering

Link #2

iRSS 2

ILR3

It3

Interfering

Link #3

iRSS 3

sRSS3

Figure 157: Illustration of 3 cognitive radio systems (WSD) and a victim system (sRSS is in blue)

## Input system parameters

For the sensing feature to be activated, both the victim and interferer transmitters have to have the “interferer is CR” selected (Figure 158). This will activate the spectrum sensing algorithm (see ‎ANNEX 16:) and will allow you to set the transmitting characteristic of the victim link transmitter, i.e. the energy that the cognitive radio spectrum sensing device is to sense. You will be able to set the emission mask and the emission floor.

Figure 158: Selection of the CR algorithm

Table 20: Victim Link transmitter

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Interferer is CR |  | switch |  | When the CR button is checked then it allows to set the emission characteristics of the VLT (used for the sRSS calculation only) |
| Emission mask: Unwanted signal level (VLT) |  | Function (X,Y) (MHz) | dBm/reference bandw. (MHz) | Define the mask of the transmitter, in the emission bandwidth and out of the emission bandwidth. Negative values in the relative mask should be chosen in a way that the integration over the emission bandwidth results in the total emitted power. If constant mask, there is no emission outside of the bandwidth. |
| Unwanted emissions floor: Noise floor signal level |  | Function (X,Y) (MHz) | dBm/ reference bandw. (MHz) | Define the minimum strength of the unwanted emissions. So the unwanted emissions equal to Max(Pwt + Unwanted emission, Unwanted emissions floor) |

Note that the settings of the emissions mask and the emission floor is the same as for the interfering link (i.e. same units).

Figure 159: Activate the CR in the VLT (Victim Link) to consider its emission characteristics 
(i.e. used in the sRSS calculation)

## Scenario parameters

In addidtion to the input system parameters, you also need to set charateristics of the spectrum sensing algorithm as described in Section ‎10.4.

## Examples

At the end of the simulation, SEAMCAT provides a set of output vectors as presented in Section ‎12.4. The following subsections present examples on how to interpret the results.

### All the channels are available

In such a scenario, the detection threshold is taken to a value of 0 dB (Figure 160) much higher than the sRSS level (average = -82.09 dBm) (Figure 161). Therefore, no victim system has been detected and the WSDs are allowed to transmit in any of the specified channels (Figure 162) per event.

| Figure 160: Selection of a high detection threshold | Figure 161: The sRSS values are well below the detection threshold, so no victim system have been detected |
| --- | --- |

In such a case the e.i.r.p.used in the simulation is the Txpower (=-33 dBm) + Gmax (=0 dBi), meaning that the in-block e.i.r.p. limit does not apply. This means that whatever the frequency selected by the WSD its e.i.r.p.. is the same (Figure 164) (here assuming that the Power Control at the It is OFF).

| Figure 162: The WSD can transmit anywhere in the victim frequency range | Figure 163: e.i.r.p. set to -33 dBm as set in the It, i.e. Txpower (=-33 dBm) + Gmax (=0 dBi), meaning that there were no limit applied to the It Tx power |
| --- | --- |

Figure 165 illustrates that on average there are 1.17 WSDs active at 1000.5 MHz per event, 1.23 WSDs in 1001.5 MHz, 1.34 WSDs in 1002.5 MHz and 1.26 WSDs in 1003.5 MHz for the same out of 5 which were input to the simulation. In this case all the WSDs were active (but in different frequencies) since the sum equal to 5 (i.e. none of the WSDs have been turned off).

| Figure 164: e.i.r.p. is the same irrespective of the frequency | Figure 165: Illustration of the number of WSDs per frequency |
| --- | --- |

### All the channels are blocked

In such a scenario, the detection threshold is taken to a lower value compared to the sRSS and none of the WSDs are transmitting. In SEAMCAT, the Tx power is set to -1000 dBm (Figure 166). The WSD frequency is the same as the victim frequency per event.

Figure 166: all the WSDs are off i.e. -1000 dBm

Note that in this specific case, the average e.i.r.p. and average WSDs per frequency can not be calculated and therefore can not be displayed as shown in Figure 168.

Figure 167: All the WSDs are off i.e. -1000 dBm

### Some of the channels are blocked and some are available

This is a typical example. In such a scenario, the detection threshold is taken to a value of -80 dB (flat over the frequency range). This mean that when considering the sRSS of Figure 168, for some of the events, the sRSS will be above that threshold and therefore some WSDs will be considered as off (which explains the -1000 dBm value in Figure 169 (b)) and the WSD frequencies are randomly distributed.

Figure 168: sRSS for 5 WSDs per event

| (a) WSD frequency | (b) WSD e.i.r.p. |  |
| --- | --- | --- |

Figure 169: Output results of the WSD in (a) frequency and (b) e.i.r.p.

In this case, one can see that all the channels were occupied in Figure 170. The WSDs were allowed to transmit with an e.i.r.p. of 33 dBm in the 3 middle channels (1000.3 MHz to 1000.7 MHz) while for the side channels less power was allowed (i.e. 24 dBm for 1000.1 MHz and 16.89 dBm for 1000.9 MHz).

Figure 170 (b) also indicates that not all the simulated WSDs were active (those having an e.i.r.p. of -1000 dBm), and that per event about only 1 WSD was active.

| (a) average e.i.r.p. | (b) average number of active WSDs |  |
| --- | --- | --- |

Figure 170: output results of the WSD in (a) frequency and (b) e.i.r.p.

# Cellular Network simulation

## Introduction

Whereas traditional simulation of non-CDMA or non-OFDMA systems is carried out in SEAMCAT by taking two pairs of transmitters-receivers and estimating signals received between them separately (i.e. without any form of feed-back influence), the simulation of cellular systems requires a much more complex process of power controlling in a fully loaded system, including impact from two tiers of neighbouring cells and, for victim cellular systems, the attempt by the system to level-out the interference impact.

There are two different types of Monte-Carlo model that could be employed: a ‘static’ model, also referred to as a ‘snapshot’ model, and a ‘dynamic’ model. In a cellular network, connections will frequently arrive and leave the network during a given period of time. This causes fluctuating traffic and power levels. Principally, it is possible to carry out a dynamic Monte-Carlo taking into account this fluctuating traffic in real time. It can account for dynamical statistical characteristics; however it is extremely time consuming to run. In cases, where many scenarios need to be investigated, such long runtimes could become restrictive. Therefore, the snapshot model is preferred in such cases and selected for SEAMCAT. This model sets up a random distribution of users based on one instant in time in connection with a network configuration and considered service characteristics. A set of statistics which accurately reflects these scenarios is derived by simulating several such snapshots.

To investigate the coexistence of a mobile radio network with another radio technology in SEAMCAT, a snapshot of both victim and interfering systems is modeled at each event generation in SEAMCAT, which generates transmit power, interference levels as well as the probability of link success of the victim system for a given number of users at a time instant. It captures a snapshot of the UE powers in the network and the number of user links which can be successfully carried given these powers. In order to analyze the impact of the interfering network on the victim one, the success rates of the victim network in the presence and absence of the interfering system are compared.

The term UE or MS are used interchangeably in this manual. You should review and modify the input parameters of the cellular network for the particular scenario that is being simulated and more detailed can be found in publications such as ‎[6], ‎[7] or ‎[8].

## CDMA overview

When simulating CDMA systems, SEAMCAT performs power controlling in a fully loaded system of all the MSs so that the impact from the neighbourhing two tiers is included (inter-cell interference), for victim CDMA systems, and the system level-out the inter-cell and external interference impact.  The balancing of the overall system is performed by the CDMA power control algorithm.

When the CDMA system is the interferer, the UEs are balanced only once, the transmit powers and positions of relevant transmitters (BS or MS, depending on scenario) are used as interefering link transmitters for calculating the iRSS. If the CDMA system is a victim, the power balancing is done first without any external interferer, where the number of connected CDMA UEs is estimated, and then the external interferer is introduced and the CDMA system is re-balanced again. Afterwards the number of served users, compared with that before the introduction of interference was introduced, allows estimating impact of interference in terms of excess outage brought to the system. The combination of the above is applied when both victim and interferer systems are CDMA.

The term "CDMA" is used in this manual and in SEAMCAT environment in general to refer to any radio technology that employs the Code Division Multiple Access modulation scheme. The specific CDMA standard (e.g. CDMA2000-1X, or W-CDMA/UMTS) can be selected by incorporating the appropriate link level curves into the simulation scenario. Furthermore, at present, only the interference impact of/on "voice" can be studied using SEAMCAT.

## OFDMA overview

The simulation of OFDMA systems is similar to that of the CDMA systems, except that after the overall two-tiers cellular system structure (incl. wrap-around) is built and populated with mobiles, OFDMA replaces the CDMA power tuning process with an iterative process of assigning a variable number of traffic sub-carriers and calculating the overall carried traffic per base station.

The OFDMA module has been designed for a Long Term Evolution (LTE) network from 3GPP TR36.942 ‎[12]. Therefore E-UTRA RF coexistence studies can be performed with Monte-Carlo simulation methodology.

## TDD vs FDD simulation

Note that TDD (Time Division Duplex)/FDD (Frequency Division Duplex) simulations are scenario dependent meaning that the direction of the interferer (UL or DL) to the victim will be studied. In this context, the time dependency is not simulated as it is not important. Therefore, SEAMCAT can already simulate this, since the worst case scenario is to be considered i.e. a FDD UL or DL scenario should give similar results as a TDD UL or DL simulation. Note however that to simulate TDD systems, the correct TDD characteristics would have to be filled in.

## Cellular network positioning

5 panels characterised the positioning of a cellular system. This panel is the same whether a CDMA (UL/DL) or OFDMA (UL/DL) is simulated.

Figure 171: Cellular network positioning GUI

### System

Initialy macro-cellular environment was implemented in SEAMCAT, but with time more flexibility was given to the tool to reproduce various topology options in cellular network (Figure 176). Cell sites are laid out in a hexagonal grid. Sites with omni-directional antennas are placed in the middle of the cells as depicted in Figure 172 and sites with tri-sector antennas are placed at the edge of the cells, where each site covers three cells. Figure 173 shows one of these cell sites (small hexagons in dashed lines) and that the arrows demonstrate the antenna orientation of each cell. The BS to BS distance (also refered as inter-site distance in the literature) is D. The cell radius R is equal to D/sqrt(3) in the omni-antenna case and is equal to D/3  in the tri-sector antenna case. Both suburban scenario and urban scenario can be modeled with this cell configuration. The scenarios differ only in propagation conditions and in the cell radius.

A wrap around cluster is used to reduce the number of cells required in the simulations and consequently to enable faster simulation run times. The number of cell sites in the cluster is assumed to be 19 (19 cells in the case of omnia-antenna and 57 cells in the case of tri-sector antenna), which appears to be appropriate for SEAMCAT simulation (see Section ‎7.6.3 for further details on wrap-around technique).

| Figure 172: Macro-Cellular CDMA Network Deployment with Omni Antenna | Figure 173: Macro-Cellular CDMA Network Deployment with Tri-Sector Antenna |
| --- | --- |

Therefore SEAMCAT supplements a single considered CDMA / OFDMA cell with its Base Station (BS) two tiers of virtual cells to form a 19 cell (57 cell for tri-sector deployment) cluster, which is then populated with a certain number of mobile stations (MS) and a power control algorithm is then applied for balancing overall system, see Figure below:

Figure 174: 19 cells omni setup

CDMA and OFDMA module shares common platform like the positioning of the cellular layout. The celular topology in SEAMCAT is composed of the “Cell layout” and the “Cell radius”a shown in Figure 176.

Figure 175: Cellular network positioning GUI

In the “Cell Layout” you can select 2 tiers, 1 tier or single cell layout. In addition, you can select between Omni directional (single sector),  tri-Sector (3GPP) and tri-Sector (3GPP2).

The “Cell Radius” (km) is the size of the cell and defines also the BS to BS distance (i.e. inter-site distance).

Figure 176: Overview of the topology options in cellular network

Two types of hexagonal grids are used to represent cellular layout, there is the 3GPP (http://www.3gpp.org/) and the 3GPP2 (http://www.3gpp2.org/). The differences are illustrated in Figure 177 (3GPP) and in Figure 178 (3GPP2). The fundamental principal of the two approaches is that they share the same commonality for the BS to BS. Based on this same value, it is possible to extract the relationship of the cell range and cell radius between the two approaches.

Within the CEPT work, it is more common to use the 3GPP hexagonal grid, ECC Repport 82 ‎[6] and ECC Repport 96 ‎[7].

Figure 177 presents an example of the 3GPP approach:

| Figure 177: 3GPP illustration of the Cell Radius, Cell Range and BS to BS distance. |  |
| --- | --- |
| where: | Cell Radius = R1 Cell Range = 2R1 BS to BS distance = 3R1 |

Figure 178 (a) and (b) illustrate the 3GPP2 approach for tri-sector and omni-sector respectively:

| (a) (b) Figure 178: 3GPP2 illustration of the Cell Radius, Cell Range and BS to BS distance for (a) tri-sector case and (b) omni-sector case |  |  |
| --- | --- | --- |
| where: | Cell Radius = R Cell Range = h = sqrt(R2-R2/4) BS to BS distance = 2h | (Eq. 31) |

What is important is that the BS to BS station distance be the same between the 3GPP and the 3GPP2 approach, i.e. where 3R1 = 2h which is equivalent to R = sqrt(3) R1.

From there it is possible to extract the cell radius in SEAMCAT.

Table 21: Example of the distances relationship between 3GPP and SEAMCAT

|  | Urban Case | Rural Case |
| --- | --- | --- |
| SEAMCAT cell radius (R)= | 433 m | 4330 m |
| SEAMCAT cell range (h)= | 375 m | 3750 m |
| Distance BS to BS (2h = 3 R1) = | 750 m | 7500 m |
| 3GPP cell range (2R1) = | 500 m | 5000 m |
| 3GPP cell radius (R1) = | 250 m | 2500 m |

In summary, according to Figure 179 below, the Table 22 shows the current different definitions for sector, cell and radii:

Table 22: Different definitions for sector, cell and radii

| Parameter | 3GPP TR 36.942 | ECC Report 252 and others | Recommendation ITU-R M.2101 Report ITU-R M.2292 |
| --- | --- | --- | --- |
| Sector | 1 hexagon | 1 hexagon | 1 hexagon |
| Cell | 3 hexagon | 3 hexagon | 1 hexagon |
| Cell radius | X | X | Y = 2*X |
| Cell range | Y = 2*X | Y = 2*X | Not defined |
| BS to BS distance | Z = 3*X | Z = 3*X | Z = 3*X |

Figure 179: Different definitions for sector, cell and radii

### System layout – reference cell selection

A singe cell consists of several MSs connected to their serving BS. The reference cell is a single cell that is surrounded by two tiers of virtual cells to form a 19 cells (or 57 cells for tri-sector deployment) cluster. This cells clutter is then populated with a certain number of MSs. The reference cell is by default at the center of the network, but you can modify it by selecting any cells you want. Part of configuring a CDMA or OFDMA network is selecting the reference cell. In SEAMCAT it is possible to choose between two network configurations (3GPP and 3GPP2, see Figure 176).

The reference cell in Figure 180 is used to calculate the effects of interference and to measure results and all non reference cells are used to provide a proper interference background to the reference cell.  You can click on the cell that should be used as reference cell when gathering results. The red cell is the current selection.

Figure 180: System layout - reference cell selection

Table 23: System layout GUI

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Center of infinite network | - | Boolean | - | Quick access to predefined selection of reference cell. This only changes the selected reference cell – no other simulation parameter is changed. |
| Left hand side of network | - | Boolean | - | Position the reference cell on the left hand side of the network. Can be used to reproduce border network layout. |
| Right hand side of network | - | Boolean | - | Position the reference cell on the right hand side of the network. Can be used to reproduce border network layout. |
| Measure interference from entire cluster | - | Boolean | - | See section ‎7.6.2 |
| Generate wrap-around | - | Boolean | - | See section ‎7.6.3 |

Normally the considered cellular system (CDMA or OFDMA) is modelled as endless network using the so called wrap-around technique. Alternatively, you may specify that the modelled cellular cell is laying at the edge of the network, in this case the cellular system will be modelled as if extending to one side only. The latter case may be suitable for simulation of geographically separated victim and interfering systems, like in cross-border scenarios as illustrated in Figure 181.

Figure 181

Figure 181: Example on how to set up the system layout to reproduce a border coordination scenario

### System layout preview

You have the possibility to see a preview of the network you are simulating. You can click on the cell that should be used as reference cell when gathering results. The red cell is the current selection.

Figure 182: System layout preview

### Mobile station

Figure 183: Cellular system – Mobile station GUI

Table 24: Cellular system – Mobile station parameters

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Antenna height | HMS | Distribution or Scalar | m | Height of user terminal in meters. Note that the assumed antenna height definition (above ground, above local clutter, effective antenna height) should correspond to the selected propagation model. |
| Antenna gain | GTx , GTx | Distribution or Scalar | dB | An omni directional antenna pattern is assumed. Depending on the link direction, it can be either the gain of the Tx (UL) or the Rx (DL) |
| Mobility |  | Distribution or Scalar | Km/h | Distribution of speed among the users.Theese speeds have to conform to the speed options in the selected Link Level Data (Section ‎8.5). For simplicity SEAMCAT assumes four different speeds, assigned to mobile users with uniform probability: 0 km/h - No movement, 3 km/h - Walking, 30 km/h - Urban driving, 100 km/h - Motorway driving |

### Base station

Figure 184: Cellular system – Base station GUI

Table 25: Cellular system – Base station parameters

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Antenna height | HBS | Distribution or Scalar | m | Distribution used to determine height of BS. Note that the assumed antenna height definition (above ground, above local clutter, effective antenna height) should correspond to the selected propagation model |
| Antenna tilt | - | Distribution or Scalar | degree | Equivalent to a physical tilt of an antenna on a mast, (-) sign is a downtilt, (+) sign is an uptilt. See ‎ANNEX 11: for further details and illustration. |
| Antenna pattern | - | Library | - | See Section ‎5.2.3 |

## CDMA/OFDMA commonalities

### Pathloss and Effective Pathloss

Path loss between each user and BS needs to be calculated within the cellular layout. In SEAMCAT, there is a distinction between the raw pathloss and the effective pathloss. The effective pathloss considers the minumum coupling loss (MCL) as defined in 3GPP. The MCL is the parameter describing the minimum loss in signal between BS and UE or UE and UE in the worst case and is defined as the minimum distance loss including antenna gains measured between antenna connectors. Note that the effective path loss includes shadowing.

The effective pathloss is defined such as:

(Eq. 32)

where:

GTx : antenna gain at the transmitter (Tx) in dBi.

GRx : antenna gain at the receiver (Rx) in dBi.

The MCL is an input parameter to SEAMCAT. Typical values of MCL can be found in 3GPP documents (3). By default this value is 70 dB (i.e. typical value for Macro cell Urban Area BS <-> UE for frequency of 2000 MHz, e.g., there is a difference between 900 MHz and 2500 MHz with respect to MCL.) when defining the victim or interferer OFDMA system, but the default MCL value for generic interferer is set to 0 dB when assessing the interference between victim and interferer (ILT -> VLR path).

### Measure interference from entire cluster

For a CDMA network used as an interferering network, when the “Measure interference from entire cluster” button is checked, all the transmitters of the CDMA network are used when simulating the interference (i.e. all 19/57 BS or all UEs in all the cells) to simulate the external interference. When it is not checked, it is only the reference cell which is the interferer. This feature only applies when a CDMA network is the source of interference.

When the interferer is OFDMA, it is assumed that the interference comes from the entire cluster and never from the reference cell only. This is true for both the downlink and the uplink. You can not select the option on the interface.

### Wrap around feature and implementation

To analyze the behavior of a cellular network without inducing any artifacts due to boundary effects limitations, it is necessary to consider an infinite cellular network. In this case one cannot perform simulation techniques because the network model is not finite. It is necessary to apply a way of simulating and analyzing the infinite network using a finite model. Wrap-around is a model developed for this purpose.

By embedding a finite repeat pattern (cluster) from the infinite hexagonal lattice on a torus, we define in fact a mapping of all the clusters forming the lattice into a generic cluster. In other words, the cell layout is wrap-around to form a toroidal surface. In order to be able to perform this mapping, the number of cells in a cluster has to be a rhombic number , defined by two “shifting” parameter i and j as

(Eq. 33)

A toroidal surface is chosen because it can be easily formed from a rhombus by joining the opposing edges. In SEAMCAT ,  with i=3 and j=2 is used. To illustrate the cyclic nature of the wrap-around cell structure, the cluster of 19 cells is repeated 8 times at rhombus lattice vertices as shown in Figure 185. Note that the original cell cluster remains in the center while the 8 clusters evenly surround this center set. From the figure, it is clear that by first cutting along the blue lines to obtain a rhombus and then joining the opposing edges of the rhombus a toroid can be formed. Furthermore, since the toroid is a continuous surface, there are an infinite number of rhombus lattice vertices but only a few selected have been shown to illustrate the cyclic nature.

In the wrap-around model considered, the signal or interference from any mobile station to a given cell is treated as if that mobile station is in the first 2 rings of neighboring cells. The distance from any mobile station to any base station can be obtained as follows:

- Define a coordinate system such that the center of cell 1 is at (0,0).

- The path distance and angle used to compute the path loss and antenna gain of a mobile station at (x,y) to a base station at (a,b) is the minimum of the following:

Distance between (x,y) and (a,b);

Distance between (x,y) and

Distance between (x,y) and

Distance between (x,y) and

Distance between (x,y) and

Distance between (x,y) and

Distance between (x,y) and  ,

where D is the inter-site distance.

Figure 185: Wrap-around with ’9’ clusters of 19 cells showing the toroidal nature of the wrap-around surface

In the “ploting options” panel, you can toggle wrap-around plotting to allow easier selection of correct cell.

# CDMA module

## Introduction

SEAMCAT allows the simulation of cellular networks employing Code Division Multiple Access modulation scheme. The specific CDMA standard (e.g. CDMA2000-1X, or W-CDMA/UMTS) can be selected by incorporating the appropriate link level curves into the simulation scenario. Only the interference impact of/on "voice" users can be studied using SEAMCAT. The CDMA examples given in this manual are based on 3GPP UMTS specifications.

CDMA and OFDMA share common panels and calculation. You are invited to consult Section ‎7.5 for the cellular topology, Reference Cell etc.. Section ‎7.6 for the Pathloss and Effective Pathloss definition etc.

## CDMA system tab

Whether you want to simulate CDMA UL (uplink) or CDMA DL (downlink), you can import  from the system library (Figure 186 – (a)), the cellular system you want (Figure 186 – (b)). You can also export  the characteristics of your CDMA network to the library for later reuse.

(a)

(b)

Figure 186: Selection of a CDMA cellular network from the system library

### System

You can name and write some description of the CDMA system you want to simulate. It is the same layout as when a generic system is selected.

### General

You can enter the frequency. The frequency value is overwritten at the “Scenario” tab level.

Table 26: General system panel

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Frequency | f | Distribution or Scalar | MHz | Distribution of the centre frequency of the system |

It is used to define the necessary parameters for the modelling of CDMA systems. The CDMA interface has been split into 2 tabs.

## General settings

The General settings tabsheet contains a range of CDMA system parameters as well as some parameters that depend on the modelled direction of CDMA link (uplink vs. downlink). 7 panels characterised the CDMA system. The below graphic represent the CDMA UL when a victim.

Figure 187: CDMA UL general settings

The only differences with CDMA DL are the following 2 panels (CDMA Downlink and CDMA Capacity).

Figure 188: CDMA DL general settings difference with CDMA UL

### CDMA general settings

Table 27: CDMA general settings input parameters

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Receiver Noise Figure |  | Scalar | dB | Equipment-specific noise figure of receiver. It is used to calculate the noise floor. See Section‎1.2.2 |
| Handover margin |  | Scalar | dB | Specifies the maximum difference between the links in users active list. The actual active-list selection is based on pathloss calculations. |
| Call drop threshold |  | Scalar | dB | Threshold to determine call drops. It is used by the power control to determine if a user should be dropped when not meeting exact target requirement. |
| Voice bit rate |  | Scalar | kbps | it is used to calculate the processing gain. See Section ‎8.7.1. processingGain = 10*log10(systemBandwidth (MHz) / voiceBitRate (kbps) * 1000); |
| Reference bandwith |  | Scalar | MHz | Bandwidth of the system. It is the same for either UL or DL. |
| Voice activity factor | - | - | - | It is set to 1, i.e. 100% (all voice users that are generated are active). It is not editable. |
| Minimum Coupling Loss |  | Scalar | dB | The minimum path loss. It is used in the calculation of the effective path loss depicted in section ‎7.6.1 |
| Link Level Data |  | Function (X,Y) |  | Drop-down selection of Link level data look-up functions from Library. It is user's responsibility to choose an appropriate set of data. See Section ‎8.5 for further details |

### Local environment

You can choose the suitable indoor and outdoor ratio for the mobile station to be used by the propagation model. Further details are presented in Section ‎5.4.3.

### Receiver settings

This content of this panel depends whether CDMA system is a victim or an interfering system. If the CDMA is a victim, you will have to set the blocking mask. It is a shared interface with OFDMA.

Figure 189: Cellular panel receiver settings

Table 28: Receiver settings of a cellular system

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Blocking mask/ACS: Receiver frequency response (receiver blocking performance) | blocking | Function (X,Y) (MHz) | dB | It is similar to the blocking response depicted in Figure 12 except that it is to be set as user defined mode only. In case that the blocking mask is defined with negative values, the parameters Standard desensitisation and I/N_target (described below) are used to compute the blocking mask used in the simulations. |
| Standard desensitisation |  |  | dB | It is value of desensitisation of the receiver as defined in the standards. |
| Target I/N |  |  | dB | It is the protection criteria used for the simulation. |

### Transmitter settings

This content of this panel depends whether CDMA system is a victim or an interfering system.  If the CDMA is an interferer, you will have to set the spectrum emission mask and the emissions floor. It is a shared interface with OFDMA.

Figure 190: Cellular panel transmitter settings

When defining the Emission Mask (not as BEM) the units of the user defined mask are: Offset (MHz); Mask Values (dBc); Reference Bandwidth (kHz).

When defining the Emission mask as a Block Edge Mask (BEM), SEAMCAT sets to 0 dBi the peak gain of the transmitter antenna and uses the power entries of the Mask as e.i.r.p (already including the gain of the antenna). So, if the Emission mask is defiend as BEM, the units of the user defined mask are: Offset (MHz); Mask Values (dBm e.i.r.p.); Reference Bandwidth (kHz).

### CDMA uplink

This is only available if CDMA UL selected.

Table 29: CDMA UL input parameters for the power control

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Target network noise rise |  | Scalar | dB | Specific level of noise that the network is willing to handle, when this level is reached it starts removing UEs to reduce its noise level |
| cell noise rise selection |  | Boolean | - | Select the algorithm that allow the cell selection based on a noise rise increased. If selected, then the measure of the noise rise per each cell is considered and the algorithm, recursively, tries to identify the number of affected cells due to a single source/cluster of interferers and remove users. If not selected, then the measure of the noise rise over the whole network is considered (See Section ‎8.7.6 for details) |
| Target cell noise rise |  | Scalar | dB | Only available when Cell noise rise selection is active. It is set to 0.1 dB by default. The “cell noise rise” algorithm will assess whether to drop users from any cell in which the noise rise exceeds the threshold indicated above. The default value of 0.1 dB has been chosen to ensure that the analysis does not disregard any cases of interfered cells, since users may also be dropped as the consequence of a low noise rise. |
| MS maximum transmit power |  | Scalar | dB | Maximum transmit power of the MS (i.e. the UE) |
| MS power control range |  | Scalar | dB | Span of the fluctuation of the power |
| PC convergence precision |  | Scalar | dB | In the uplink, each mobile station perfectly achieves the target C/I, Eb/N0_target, during the power control loop convergence, assuming that the maximum transmit (TX) power, max_MS_Tx_Pw, is not exceeded. Those mobile stations not able to achieve Eb/N0_target after convergence of the power control loop are considered in outage (i.e. they are dropped). The power control loop is considered to converge when all mobile stations are within the max_MS_Tx_Pw and their Tx power is adjusted by less than the “PC convergence precision” value for the last power balancing iteration. |

### CDMA downlink

This panel is available only if CDMA DL selected.

Table 30: CDMA DL input parameters for the power control

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Success threshold |  | Scalar | dB | Threshold to determine perfect link quality. |
| Base Station Pilot Channel Fraction | pilot_frac | Scalar | - | Fraction of max BS power allocated to pilot. |
| Base Station Overhead Channel Fraction | Overhead_frac | Scalar | - | Fraction of max BS power allocated to overhead channels (paging, etc.). |
| Base Station maximum Broadcast Power |  | Scalar | dBm | maximum Broadcast Power |
| Base Station maximum traffic channel fraction |  | Scalar | - | Fraction of the maximum allowable broadcast power (per traff. chan. per BS). The maximum allowable traffic channel power is compared to the calculated transmit traffic channel power levels with respect to the Ec/Ior link level data for iterative adjustment in the DL power control. |

### CDMA capacity

The capacity of the simulated system (i.e. how many mobiles per cell should be generated in the system) is dependent on all other settings and cannot always be easily deducted from these. Therefore SEAMCAT has a feature that allows for automatic determination of capacity. This is also known as simulation of non interfered capacity and is enabled by default.

Figure 191: CDMA UL system - Determination of the optimum number of UEs (CDMA capacity)

In CDMA UL, the number of optimised users is being re-calculated for each event. It is recommended to run the "simulate non-interfered capacity" so that SEAMCAT can provide a "best" optimised value, this will optimise the computation time afterwards. If you are using another number you risk to create an overhead in your computation time without any change in the output results.

Figure 192: CDMA DL system - Determination of the optimum number of UEs (CDMA capacity)

Table 31: CDMA Capacity settings parameters (UL and DL)

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Simulate non interfered capacity | - | Boolean | - | Toggles automatic capacity finding. If the option Simulate non-interfered capacity is checked, then the system will automatically simulate the 'optimal' number of the mobiles for given system configuration (type of system, bandwidth, cell sizes, etc). The optimum finding algorithm is developed to establish the loading that would correspond to approx. 80% of maximum system capacity. If this option is unchecked, you are free to set a constant user-defined average number of mobile users per cell especially if the optimal capacity for the current scenario is known (this is often the case when running consecutive simulations with the same system) there is no need to simulate – as the simulation process can be quite lengthy. When this checkbox is disabled SEAMCAT uses the value entered in 2 – “Users per cell” as the capacity per cell. |
| Init users per cell | - | Scalar | - | If capacity simulation is enabled this indicates the starting point of the simulation. Selecting the right starting point can speed up the capacity finding. If capacity simulation is disabled the value in this field is the actual value used by SEAMCAT. SEAMCAT does NOT change this input value into the result of the simulation! Users per cell is equal to UE per Base Station. SEAMCAT consider each Base station as its own cell. |
| Delta users per cell |  | Scalar | - | When SEAMCAT tries to find the optimal capacity it adjust the number of UEs per cell starting with this value. A proper value here can speed up capacity finding. |
| Number of trials |  | Scalar | - | When finding the optimal capacity SEAMCAT runs this (i.e. Number of trials) many snapshots of every value of UEs per cell before deciding whether or not the current value is the optimal capacity. Generally larger numbers mean greater precision but also longer time needed by the algorithm. |
| Target noise rise precision |  | Scalar | dB | Uplink only – the precision used when comparing the noise rise of the filled system with target noise rise set under the “CDMA Uplink” panel |
| Tolerance of initial outage |  | Scalar | % | Downlink only – The tolerance of initial outage is the percentage of UEs that can be dropped before SEAMCAT determines that the tested number of UEs cannot fit into the system (i.e. 20 user_per_cell * 19 BS = 380 UEs, if 5% or less of 380 UEs are dropped, the system is considered able to handle/service 20 UEs per cell). SEAMCAT will adjust the value of UEs per cell untill a value is found which in 80% of the specified number of trials is able to handle the tested number of UEs per cell.This parameter allows for UEs in “extreme” pathloss situations to be “ignored” from the optimal capacity finding. |

### Propagation Model

You can choose the suitable propagation model to be applied when calculating signal loss along the transmitter and the receiver path. A choice and settings of propagation models are presented in ‎ANNEX 17:.

## Positioning

See section ‎7.5, common to CDMA and OFDMA on the positioning of BSs and MSs.

## CDMA Link level data

### CDMA link level system mapping

Power control is a crucial mechanism in CDMA mobile radio networks, which needs to be modeled in SEAMCAT. It is a complex process involving various layers of signaling, measurement and modulation/demodulation procedures. It is not feasible to model signaling, link and chip level details of CDMA power control in network level simulations performed by SEAMCAT due to the complexity and CPU time constraints. Hence, it is necessary to adopt the two-step approach employed widely in the industry for the simulation of CDMA based systems.

The first step utilizes link level simulations that model fast fading channels, power control procedures and actual chip level algorithms to generate outputs that map channel power requirements to link quality (e.g. frame erasure rate, FER). Such simulations involve the knowledge of intricate details of the CDMA signaling procedures and modulation/demodulation methods.

Major CDMA vendors develop link level simulations and contribute their results to the standard bodies. Since the link level results are independent of most system level variations (cell sizes, amplifier ratings, antenna types, etc.), they are applicable to a wide variety of network configurations. The second step in the simulation of CDMA involves system level simulations that actually model the CDMA network on a macro scale. Since the required channel power vs. link performance data is available from the link level results, transmit power levels for CDMA channels can be calculated and utilized in the system level modelling of a CDMA network.

The approach described above enables the reuse of link level data to model various network configurations. Furthermore, through the use of the link level data, an accurate power control model is implicitly included in the system level simulations that run at moderate complexity.

The built-in CDMA Link level Data used in DL and UL can be found in Section ‎13.4.6. SEAMCAT allows you to load your own library too.

### CDMA DL Power Control Methodology (VOICE ONLY)

Figure 193 presents the dependency between the condition of a user in the network (the so called geometry), mobile speed and soft handover state of the UE that are needed to map a particular link quality to the channel power requirement.

Figure 193: Power Control Module (high level)

All these factors determine the appropriate mapping of a particular link quality to the channel power requirement. For example, stationary users may require less power than moving users to attain the same link quality. Similarly, users connected to several BS’s at the same time (soft handover) may require less power than users connected to a single BS to achieve the same link quality. Furthermore, users in favorable locations (high geometry) may again require less power than users that are in unfavorable locations (low geometry). Hence, link level data includes different mappings (look up tables) between link quality and required power for different mobile speeds, geometries and soft handover states. Furthermore, in order to remove the dependency on the total BS power (may vary from system to system), the power requirements are reported as normalized power fractions (fraction of the total BS power).

Consequently, the link level data is used in modelling power control in a variety of conditions such as different mobile speeds, geometrical user distributions, soft handover characteristics and amplifier output power ratings. In CDMA Downlink, the link level is a function of Ec/Ior.

### CDMA uplink Power Control Methodology in SEAMCAT (VOICE ONLY)

Performance characteristics of individual links to be used in the power control module of SEAMCAT are generated a priori from link level simulations. This usually includes several mappings between requested link quality (e.g. block error rate, BLER) and required transmit power of mobile stations/base stations. For generating such mappings in form of “look up tables”, link level simulations involve multipath fading, physical layer transceiver algorithms, e.g. modulation/demodulation and coding/decoding, as well as power control procedures. Different mulipath fading channels (e.g. the ITU channel models) are used to model various configurations, e.g. indoor, outdoor, pedestrian, vehicular, etc.

In CDMA UL, the sum of received C/I values in two sectors should meet the C/I requirements specified by the link level simulation data. In CDMA Uplink, the link level is a function of Eb/No.

## CDMA Downlink - simulation algorithm

### Simulation Methodology

The main goal of the downlink power control in SEAMCAT is to calculate the total BS output power and the success rate (% of calls with no link quality degradation) for a given snapshot of the system. BS output power is a key parameter in the scenarios where CDMA is the interferer. Success rate, on the other hand, is crucial in CDMA victim scenarios. One possible way to analyze the impact of other system interference on CDMA is to compare the success rates in the presence and absence of external interference.

A snapshot of the mutually existing systems is modeled at each event generation in SEAMCAT. Hence, at each event generation the power control algorithm should also be run for the CDMA cell, whether it is the victim or the interferer. This is illustrated in Figure 194. The setup block is inherited from the higher layers of SEAMCAT and consists of initializing the system parameters. The next step involves the generation of traffic for power control, calculation of appropriate path losses within the CDMA cell layout and determination of soft handover states. Power control is then performed by utilizing the link level data via an iterative process. Finally, necessary outputs are generated and fed into the interference calculation modules in SEAMCAT.

Figure 194: DL Power Control Simulation Methodology – Overview

For simplicity, the CDMA downlink power control methodology is described for omni-cells. However, extension to multi-sector cells is straightforward. In a multi-sector configuration, each sector should be treated in the same way a cell is treated in the omni configuration.

### Traffic Generation

While the BS output power and the outage ratio is likely to be calculated for a single CDMA cell, accurate modelling of power control requires the consideration of inner-system interference generated by the surrounding tiers of CDMA cells. The significance of other cell interference in CDMA requires that at least two tiers surrounding the cell of interest be considered. However, BS power and outage statistics will only be collected from the center cell, which has the most accurate interference background (two surrounding tiers). Cells surrounding the center cell will not be visible to the higher levels of SEAMCAT and will only be used to generate the inner-system other-cell interference background for the center cell (Figure 195).

Figure 195: Cell layout for power control

The power control simulation time increases with the number of cells for which power control algorithm is run. One way to reduce the simulation time is to simulate only the center cell and the first tier around it with actual power control algorithms and use artificial interference generators for the second tier as shown in Figure 195. More specifically, the BS’s in the center cell and the first tier go through the power control algorithms and calculate the precise power they need to transmit. Whereas, the BS’s in the second tier are assigned an output power level to generate interference into the center cell and the first tier. If the output power level set for the second tier is reasonable, this approach will speed up the simulation considerably without sacrificing much from accuracy. Possible methods to determine the appropriate artificial interference level will be addressed later in the paper. Nevertheless, if more accurate results are desired, the second tier can also be simulated using actual power control. In that case, a third tier with artificial interference generators would further increase accuracy by presenting the second tier with a more realistic interference background. However, given the considerations on complexity, the layout shown in Figure 195presents the most appropriate balance between simulation speed and accuracy.

Since higher levels in SEAMCAT consider only a single CDMA cell, the cell layout shown in Figure 195 may need to be generated separately in the power control module. It is expected that the UE placement be done consistently with SEAMCAT’s existing algorithms. However, once the UEs are placed, their mobility assignment should also be done. Actual mobility of the UEs cannot be simulated easily in a static simulation, but the effects of mobility on the channel power can be modeled in a limited sense. While the UEs will be treated at fixed locations within each snapshot, each will be assigned a speed to determine their channel conditions (fast fading), which will be used in the determination of their channel power requirements. This allows the flexibility to simulate various system configurations (fixed, highway, pedestrian, etc.).

### Soft Handover

A user may simultaneously be connected to multiple BS’s in CDMA based systems (soft handover). Since soft handover affects the amount of power transmitted by each BS to a certain user, it is necessary to determine whether the UE is served by a single BS or multiple BS’s. The actual determination of the soft handover state of a user and the corresponding channel power requirements may get complicated. Hence, a simplified soft handover algorithm is presented next, which captures the essence of soft handover effects while avoiding implementation of complex algorithms.

Base stations that are connected to a user are included in the “active set” of that user. A base station is initially selected to be in the “active set” based on the strength of its pilot signal versus the interference background. Each base station broadcasts a certain fixed percentage of its maximum power on the pilot channel. The interference background consists of the non-orthogonal energy received on the other channels of the base stations within the active set and the total broadcast power of the base stations that are not in the active set. The BS selection criterion, “pilot Ec/Io” is then defined as

(Eq. 34)

with the following definitions:

Ec is the chip energy received from ith BS;

Io is the spectral density of total received interference;

pilot_frac is the fraction of BS power allocated to pilot;

Pmax,I is the maximum receivable power from ith BS (max BS transmit power*path loss);

W is the system bandwidth;

Pj is the total received power from jth BS;

F is the mobile station noise figure;

N0 is the thermal noise power density;

Iext is the external interference (out of system).

Based on this selection criterion, the following simplified soft handover algorithm can be employed to assign soft handover states to each user:

For each user:

- Add the BS with the strongest corresponding Ec/Io to the active set;

- Add the BS with the second strongest corresponding Ec/Io to the active set if its Ec/Io is within 4 dB of the strongest Ec/Io.

Then the soft handover state of a user becomes the number of BS’s in its active set, which is either one or two. Note that in actual systems, the active set of a user may have more than 2 BS’s. However, in order to develop a unified methodology that can simulate various implementations of CDMA based systems and to avoid overwhelming complexity, this simplified approach is suggested. Several standards (including UMTS) present similar methodologies for simulations.

### Power Control

As far as SEAMCAT is concerned, the actual CDMA power control algorithm looks merely like a black box that maps link quality to channel power. However, the mapping is not simply one-to-one. Depending on the conditions of the mobile user, the same link quality can map to different channel power requirements. A key parameter that determines the condition of a user is called the “geometry”.  The higher the geometry, the more favorable the UE’s condition is. The geometry is defined as:

(Eq. 35)

with the following definitions:

Pactive is the total power received from BS’s in the active set;

No is the thermal noise;

Pother is  the total power received from BS’s not in the active set;

Iext is the external Interference (out of system).

The fractional power levels found in the link level data are defined for each user (channel) as:

(Eq. 36)

with the following definitions:

Ptraff_active is the total received traffic channel power from BS’s in the active set

Ptotal_active is the total power received from BS’s in the active set

Ptotal_active is the sum of the total received power from the BS’s in the active set including their pilot, overhead and all traffic channels. Whereas Ptraff_active includes only the traffic channel power that is received from the BS’s in the active for the particular user.  In other words, a user’s Ec/Ior shows the fraction of the total received power that is used for voice communication with that user. Based on this definition, the amount of traffic channel power received from a BS for a particular user can be derived from the Ec/Ior requirements reported in the link level data.

If user has only 1 BS in the active set (simplex), the power received from the BS is:

Ptraff=Ptotal_active x Ec/Ior  	(Eq. 37)

If user has 2 BS’s in the active set (2-way soft handover), power received from one of the BS’s is then:

Ptraff=(Ptotal_active x Ec/Ior)/2	(Eq. 38)

Note that symmetry between the two soft handover legs (links with BS’s in the active set) is assumed. Therefore, when a user is connected to two BS’s, it receives equal power from each link. The determination of the traffic channel power levels for each user cannot be done in a single step. The inherent assumption in equations 37 and 38 is that Ptotal_active is known. However, Ptotal_active itself is the sum of the pilot, overhead and all traffic channel power levels received from the BS’s in the active set. Therefore, an iterative process is required to determine the individual traffic channel received power levels.

Figure 196: Power Control Loop

Figure 196 shows how the power control loop operates. The initial step is to initialize each BS in the cell layout (figure 1) by assigning total broadcast power levels. A figure around 70% of maximum BS power is appropriate. Note that for the simulated BS’s, the total BS power will be updated at each iteration by the power control loop. After enough iterations, the power levels will converge to the correct values.

Once the initialization is complete, geometry and soft handover state for each user can be calculated based on the initial values of the BS broadcast levels. Then the Ec/Ior requirement for each active user can be obtained from the link level data using its mobile speed assignment, calculated geometry and soft handover state. Equations 37 and 38 can then be used to get the received traffic channel power levels for each user. Path loss information can then be used to determine the corresponding transmit channel power levels. However, the calculated transmit traffic channel power levels should be checked against the maximum allowable traffic channel power and transmit/receive levels should be adjusted if necessary. As a result of such an adjustment, a user may not meet its Ec/Ior requirement. Based on a “call drop threshold”, such a user may be removed from the system if it meets the following criterion:

Achieved Ec/Ior < Ec/Ior requirement – Call drop threshold (dB) 	(Eq. 39)

The call drop threshold is set such that dropping a call is limited to extreme circumstances (thresholds less than 2dB are not recommended) and kept mostly as a safety measure to avoid a single user hogging the BS resources. In an actual system, calls are not dropped at the instant they fail to meet their link quality target. The system will tolerate quality degradation up to certain durations and at the same time avoid a single user to sacrifice the overall system performance by consuming all the BS resources (max. traff. chan. pow. setting). In fact, for systems that employ sufficient control of maximum traffic channel power, call drops may be avoided completely within the power control loop. Eventually, users not meeting their Ec/Ior target will be evaluated when the success rate of the system is calculated.

Once the transmit traffic channel levels are calculated, the broadcast power of each BS should accordingly be updated. If the total broadcast power of a BS turns out to be greater than its maximum allowable level, all traffic channels served by that BS should be scaled down so that the maximum BS power constraint is met. The scaling factor that should be applied to the traffic channel power levels can easily be calculated as:

(Eq. 40)

where:

Pmax is the maximum allowable BS power

Pcalculated is the actual calculated BS broadcast power  (including pilot and overhead).

Scaling is only done if Pcalculated > Pmax and it is done only on the traffic channels; pilot and overhead power levels remain at a constant percentage of the maximum allowable BS power. For channels that go through the scaling, achieved Ec/Ior levels may not match the required Ec/Ior levels. Therefore, call drop criterion (if used) shown in equation 6 should also be checked after the scaling. The process is outlined in Figure 196.

This process describes a single iteration of the power control loop. After all the traffic channel power levels are determined and the BS levels are updated, the process should be repeated (with the new, more accurate BS broadcast levels). Convergence of the traffic channel power levels should be checked at the end of each iteration. The loop can be terminated once the traffic channel power of every simulated user in the network converges to the desired precision.

Signaling and other errors in power control are considered in the link level simulations. System level simulations do not consider additional errors and assume that each user is served with the required power level that is determined from link level data, provided that the BS has enough power to do so and the maximum traffic channel limit is not exceeded.

### Success rate

The power control loop terminates when every BS broadcast power converges and traffic channel power level for every user is calculated. Therefore, both the BS output power and the success rate for the cell of interest (center cell in Figure 195) can be calculated. BS output power is the sum of the power in pilot, overhead and all traffic channels. Success rate is the percentage of calls that do not suffer quality degradation. The following process can be used to calculate both output metrics:

i.	Power control loop is terminated (traffic power converges for every user)

ii.	Final BS transmit power levels are calculated (sum of all traffic, pilot and overhead)

iii.	Total BS broadcast power for the cell of interest is determined

(For each active user in the cell of interest)

iv.	Final geometry is calculated based on BS power levels calculated in ii.

v.	Traffic Ec/Ior target is determined based on geometries calculated in iv.

vi.	Achieved Ec/Ior is calculated based on BS power levels calculated in ii.

vii.	Success criterion is checked

(Eq. 41)

viii.	Success rate is determined for the cell of interest

Success Threshold is usually a small figure such as 0.5dB. Users who miss their Ec/Ior targets by more than the threshold suffer link quality degradation. Note that if call drops occurred within the power control loop, they should also be considered when success rate is determined:

(Eq. 42)

## CDMA Uplink - simulation algorithm

The center cell site only is used to calculate the effects of interference. In spite of this fact, it is essential to consider the intra-system interference caused by other cells in the cluster for an accurate modelling of power control. The precise transmit power of all active mobile stations in the wrap-around cluster has to be calculated in the uplink power control loop.

### Power Control

In CDMA networks, closed-loop fast transmit power control (TPC) is supported in uplink. The base station estimates the signal-to-interference ratio (C/I), measured in bit energy-to-noise density ratio Eb/N0, and compares it to a target value (Eb/N0_target). If the estimated C/I is below Eb/N0_target, the base station commands the mobile station to increase the transmit power; if the measured C/I is above Eb/N0_target, it commands the mobile station to lower its power. The fast transmit power control works at a frequency of f Hz (1500 Hz for WCDMA and 800 Hz in CDMA2000 1x), thus the TPC commands are transmitted at 1/f s time intervals (0.667 ms for WCDMA and 1.25 ms for CDMA2000 1x).

In reality, the fast TPC is not ideal because of issues such as

inaccuracies in the C/I estimates;

transmit power control signaling errors;

delay in the transmit power control loop.

Links level simulations take these errors into account and reflect their impacts on the link quality figures in the look up tables to be input to the power control module of SEAMCAT.  Therefore, we assume a simple C/I based fast closed-loop TPC of traffic channels for uplink in the following.

In the uplink, each mobile station perfectly achieves the target C/I, Eb/N0_target, during the power control loop convergence, assuming that the maximum transmit (TX) power, max_MS_Tx_Pw, is not exceeded. Those mobile stations not able to achieve Eb/N0_target after convergence of the power control loop are considered in outage.

The local-mean Signal-to-interference power ratio in the uplink, (C/I)UL, is calculated by multiplying the received signal power S by the processing gain G, and dividing the result by the total interference power Itotal

(Eq. 43)

with

(Eq. 44)

Iintra is the intra-cell interference power, i.e. the interference generated by those mobile stations served by the same base station as the considered mobile station. Iinter is the inter-cell interference power from other radio cells. Iout is the interference power coming from the interfering system. N0 is thermal noise (as well as spurious interference) contained in the receiver bandwidth, W, and b is an interference reduction factor due to the use of interference mitigation signal processing techniques in the uplink, e.g. Multi User Detection. No such interference mitigation technique is assumed in these considerations, therefore b = 0.

Assuming a mobile station power control range in the order of MS_PC_Range dB; the minimum TX power is therefore max_MS_Pw_Tx – MS_PC_Range dBm.

### Soft and Softer Handover

The handover model proposed is a simplified soft handover. We assume that all base stations transmit with the same pilot power in downlink. Therefore, PL_fading (path loss plus the shadow fading) is the only criterion for selecting the base stations belonging to the active set of a mobile station.

We assume that active set for a mobile station consists of two base stations; the base station with the strongest signal, i.e. the lowest PL_fading, and the base station with the second strongest signal if its signal strength is within Handover_Margin dB of the strongest signal (in other words its PL_fading  is within Handover_Margin dB of the lowest PL_fading).

In the case that base stations with omni-antenna are used at the cell sites, selection combining among the base stations in active set is performed and the base station with the strongest signal is selected as the serving base station of the mobile station. In the event of base stations with tri-sector antenna, similar procedure is applied, if the two sectors in the active set belong to different cell sites, else a maximal ratio combining is realized by summing the received signal powers. In the later case, the sum of received C/I values in two sectors should meet the C/I requirements specified by the link level simulation data. Because during softer handover, the mobile station is usually in the overlapping coverage area of two adjacent sectors of the base station, it is reasonable to assume that it has symmetric links to both sectors in the active set. As a consequence, each sector needs to fulfill one half of the C/I requirement.

### Voice Activity Factor

The voice activity factor is the measure of how long the non-silence period is to the overall time for voice communication as it reflects the fact that speech users are silent or speaking. In SEAMCAT, It is assumed that all connected users are speaking constantly during a simulated event. It is therefore set to 1 (i.e. 100%).

### System loading

The following procedures can be used for system loading during simulation and preparation of simulation outputs.

System loading

To determine the number of active mobile stations Act_MS in the network:

- Set up:

- Average traffic load in terms of a predefined number of users per cluster: N_UL

- standard deviation of log-normal shadowing σshadowing

- voice activity factor Act_Factor (fixed to 100%)

- target maximum noise rise over the thermal noise in the network  η_target

- target C/I (Eb/N0_target) to fulfill service requirement depending on configuration and mobility (provided by link level simulations)

- maximum transmit power of mobile station max_MS_Pw_Tx

- power control range – MS_PC_Rang:

- In the case that the CDMA uplink is the victim link, add the received power from the interfering system to the thermal noise power

For each event:

- put down uniformly mobile stations at pseudo-random locations across the network and distribute speed among them

- Add a new mobile station in the set of active users in the network

  - compute average path-loss from the mobile station to the base station of each cell

  - generate a log-normal pseudo-random value to add to each of the path losses to model shadow fading

  - perform a pseudo-random weighted coin-toss to determine voice activity, where 1 occurs with probability Act_Factor

  - compute required received power at the base station to meet Eb/N0_target, given interference from pre-existing mobiles and other sources ( and )

  - compute required transmit power of the mobile station

  - adjust the required transmit powers of the all existing mobile stations perturbed by addition of the new mobile station

  - continue the adjustment until the convergence of power control loop is achieved. A convergence criterion could be that the variation of two consecutive transmit powers of each mobile station is within a predefined threshold.

  - compare the number of active mobile stations, Act_MS, with N_UL

  - if  Act_MS ≥ N_UL terminate the addition of a new mobile station in the network

  - else measure the average noise rise over the thermal noise η and compare it with the target noise rise limit η_target

  - if η_target is reached, terminate the addition of a new mobile station in the network

  - else add a new mobile station and go to step II

### Outage calculation

Two conditions are counted as outage.

- A mobile station, which is not able to transmit the required amount of power to meet the received Eb/N0_target due to maximum power limitations. This mobile is counted as part of the specified traffic load N_UL. However, the mobile is assumed to be transmitting no power.

In the case of Act_MS < N_UL, no more mobile stations can be added to the set of active users because of noise rise limits.  In this event, N_UL - Act_MS outages are counted.

For each event, the number of optimised users is being re-calculated

### CDMA UL cell selection

For the CDMA UL, there are two algorithms selectable in SEAMCAT:

Recommended algorithm when the interferer is a cellular network or affecting many cells in a network: the noise rise (which is measured per cell) is averaged over the whole network. This way, the UEs with highest power over the whole victim network are removed in order to compensate the noise rise due to external interference (Section ‎8.7.4).

Recommended algorithm when the interferer affects one or a few cells in a network (e.g. a strong interferer located close to a small part of the victim network): the noise rise is calculated per cell. This algorithm works as follows:

- The cells with highest noise rise are selected.

- Recursively, cell per cell, the UEs with highest power in the cell are removed in order to level out the network noise rise (see Annex ‎A15.3 for further details on the algorithm)

This algorithm allows investigating per event how many cells are being affected (see Section ‎12.5.3).

## Capacity for CDMA system

When starting a simulation involving one or more CDMA systems SEAMCAT will begin by checking if any of the CDMA systems have the “Simulate non interfered capacity” feature enabled (this is the default setting). The purpose of the “Simulate non interfered capacity” feature is to find the non interfered capacity system (i.e. how many mobiles per cell should be generated in the system before the introduction of an external interference) with the current configuration.

For each CDMA system needing to have its optimal capacity simulated the screen shown in Figure 197 and Figure 198 will appear for uplink and for downlink systems respectively.

### CDMA Uplink capacity finding

In CDMA uplink, the capacity is found by gradually filling system with users until a certain average noise rise with the specified threshold noise rise is reached. The Noise rise is measured as the linear average of dB values – across all 19/57 base-stations. After every trial SEAMCAT calculates the average noise rise over the total number of trials and if this value is above the threshold restarts the simulation with a lower value of users per cell.

Figure 197: Uplink non-interfered capacity finding

Table 32: Elements of the uplink non-interfered capacity finding screen

| ID | Description |
| --- | --- |
| 1 | This dial indicates the number of trials completed with the current capacity. This dial will range from 0 to the number of trials entered as value “4” on Figure 191. |
| 2 | This dial indicates the current number of users being tested. Range is dynamic and the dial is mainly intended as an easy visual indicator of values being tested. |
| 3 UL | The needle shows the current value of average noise rise across the trials run. The red area indicates the noise rise is too low (too few users in the system) – the green area is the target noise rise (plus/minus) the tolerance specified. The yellow area indicates the average noise rise is too high (too many users in the system). |
| 4 UL | The bar chart gives information on previous values tested. The Y axis is the average noise rise and the X axis is the number of users per cell being tested. |

### CDMA Downlink capacity finding

In CDMA downlink, the capacity is found by gradually filling system with users while measuring system outage. For every number of users a certain number of trials are run and then the number of “successful” trials is compared to a predefined success criterion. Only the number of trials is configurable by user – success criterion is fixed at 80%. This means that optimal capacity of a downlink system is defined as the capacity which the system is able serve without any outage in 80% of trials. This step can be quite time consuming. See ‎ANNEX 15: to get a detailed description on how SEAMCAT determines “optimal capacity” for DL CDMA– using the values from Figure 192.

SEAMCAT tries to detect when no more trials with a tested value is needed to adjust to the next value. As an example, if 40 users per cell are being tested for 20 trials and the first 5 trials are unsuccessful it is not possible to reach 80% success rate and there is no need to simulate the last 15 trials. Regardless of the result of the last 15 trials SEAMCAT will conclude that fewer users per cell are needed. If more than 80% success rate is reached before all trials are simulated, there is no need to simulate the rest of the trials.

Figure 198: Downlink non-interfered capacity finding

Table 33: Elements of the downlink non-interfered capacity finding screen

| ID | Description |
| --- | --- |
| 1 | This dial indicates the number of trials completed with the current capacity. This dial will range from 0 to the number of trials entered as value “4” on Figure 191. SEAMCAT does not always simulate all the trials – target is 80% success and if for example 5 out of 20 have already failed – SEAMCAT does not simulate the rest of the trials. |
| 2 | This dial indicates the current number of users being tested. Range is dynamic and the dial is mainly intended as an easy visual indicator of values being tested. |
| 3 DL | The red area indicates that less than 80% has been reached (i.e. too many users in the system – or not all trials yet complete) – the green area is 80% (16 trials with the default settings). The yellow area indicates that more than 80% of the trials are successful (too few users in the system). SEAMCAT stops when this dial stops in the green area after all trials completed. |
| 4 DL | The bar chart gives information on previous values tested. The Y axis is the number of successful trials and the X axis is the number of users per cell being tested. |

# OFDMA module

## Introduction

The current OFDMA module has been designed for a Long Term Evolution (LTE) network from 3GPP ‎[12]. Therefore E-UTRA RF coexistence studies can be performed with Monte-Carlo simulation methodology.

The general simulation assumptions are presented in this section to provide a guideline on how to perform coexistence simulations. The OFDMA DL (downlink) LTE algorithm implemented in SEAMCAT assumes a 100% loaded system and each user is allocated with a fixed number of resource blocks. This is equivalent to modelling a Round Robin scheduler with full buffer traffic model and a frequency reuse of 1/1 (i.e. Single Frequency Network is assumed). The OFDMA algorithm as implemented in SEAMCAT takes into account the intra system interference into the reference cell, caused by UEs located in adjacent cells and using the same RBs but also caused by UEs located in the reference cell which are using different RBs. The OFDMA UL (uplink) LTE algorithm implemented in SEAMCAT is similar to the OFDMA DL LTE algorithm with one exception. In the OFDMA UL system it is possible to  load the system with a set number of resource blocks rather than only 100% load like in the OFDMA DL system (see ‎9.3.6).

The network layout is similar to the one used for CDMA. The methodology assumes that the UEs are deployed randomly in the whole network region according to a uniform geographical distribution. The wrap around technique is employed to remove the network deployment edge effects.

Note that if the OFDMA is a DL interferer, the OFDMA is simulated as in “traditional” simulation with the BSs transmitting at full power. This decreases the simulation time of a full OFDMA simulation. In OFDMA DL interferer case, only the position of the BSs will be calculated because full transmit power is assumed. For all other simulations (including UL) scenarios full OFDMA network simulation is required. Consequently, some of the input parameter of the GUI interface have been grey-out when the OFDMA DL interferer case is selected.

Since it is arguable that some simulation assuming a rural environment would not need to assume full power transmission (i.e. full loaded network) when the system is DL and interferer, you may need to manipulate either the input power or the spectrum mask (or both) in order to simulate the DL interferer case for rural deployment.

## OFDMA system tab

Whether you want to simulate OFDMA UL or OFDMA DL, you can import  from the system library (Figure 199– (a)), the cellular system you want (Figure 199– (b)). You can also export  the characteristics of your OFDMA network to the library for later reuse.

(a)

(b)

Figure 199: Selection of a OFDMA cellular network from the system library

### System

You can name and write some description of the OFDMA system you want to simulate. It is the same layout as when a generic system is selected.

### General

You can enter the frequency. The frequency value is overwritten at the “Scenario” tab level.

Table 34: General system panel

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Frequency | f | Distribution or Scalar | MHz | Distribution of the centre frequency of the system |

It is used to define the necessary parameters for the modelling of OFDMA systems. The OFDMA interface has been split into 2 tabs, general settings and positioning.

## General settings

The General settings tabsheet contains a range of OFDMA system parameters as well as some parameters that depend on the modelled direction of OFDMA link (uplink vs. downlink). 8 panels characterise the OFDMA DL system while 9 panels characterise the OFDMA UL system. The below graphic represent the OFDMA DL.

Figure 200: OFDMA DL general settings GUI

There are two differences between OFDMA DL and UL. The first difference is the definition of transmit power. OFDMA DL does not have a power control algorithm. The second difference is the OFDMA UE Distribution panel available only for OFDMA UL.

Figure 201: Transmit power in OFDMA DL

Figure 202: UE distribution in OFDMA UL

### OFDMA general settings

They are similar for the OFDMA DL and UL.

Table 35: OFDMA general settings input parameters

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Max RBs per Base Station | - | Scalar | - | Number of available Resource Blocks (RBs) per BS |
| Number of RBs per mobile | - | Scalar | - | Number of RBs per UE. Note the ratio of Max RBs per Base Station/ Number of RBs per mobile gives the number of active users per serving BS. This can however be overridden in OFDMA UL by using “User defined active MS per BS” where the number of active MS can be specified. |
| Handover Margin |  |  | dB | Specifies the maximum difference, in dB, between the links in users active list. The actual active-list selection is based on pathloss calculations. |
| Minimum Coupling Loss |  | Scalar | dB | The minimum path loss. It is used in the calculation of the effective path loss depicted in section ‎7.6.1 |
| System bandwidth |  | Scalar | MHz | When the OFDMA is the interferer and depending on the direction of the interfering OFDMA link to be simulated, you should pay attention to the emission bandwidth of the unwanted emission mask and the system bandwidth (see Section ‎9.3.4) |
| Receiver Noise Figure |  | Scalar | dB | Equipment-specific noise figure of receiver. It is used to calculate the noise floor. See Section‎1.2.2 |
| Bandwidth of RBs |  |  | MHz |  |
| OFDMA LTE Link-to-system level mapping |  | Function (X,Y) | - | The traffic (i.e. bit rate) per UE is a look up table used to map throughput in terms of spectral efficiency (bps per Hz) with respect to calculated SINR (= C/(I+N)) (dB) (Signal to Interference-plus-Noise Ratio) level. A drop-down selection of Link level data look-up 2 dimensions functions from Library. The OFDMA Link level data has the same formats for uplink and downlink but with different values as depicted in Section ‎9.5. This link level data (bitrate mapping) is user selectable and can be modified depending on the simulation to perform. You are responsible to choose an appropriate set of data. |

### Local environment

You can choose the suitable indoor and outdoor ratio for the mobile station to be used by the propagation model. Further details are presented in Section ‎5.4.3.

### Receiver settings

This content of this panel depends whether OFDMA system is a victim or an interfering system. If the OFDMA is a victim, you will have to set the blocking mask. It is the same input as for CDMA, see Section ‎8.3.3.

### Transmitter settings

This content of this panel depends whether OFDMA system is a victim or an interfering system.  If the OFDMA is an interferer, you will have to set the spectrum emission mask and the emissions floor. It is the same input as for CDMA, see Section ‎8.3.4.

When the OFDMA is the interferer and depending on the direction of the interfering OFDMA link to be simulated, the the emission bandwidth of the unwanted emission mask and the system bandwidth may be different.

In DL, the unwanted emission mask corresponds to the BS transmitting over all the RBs (i.e. the emission bandwidth is the same as the System Bandwidth input)

In UL, the emission bandwidth (i.e. in-band part of the unwanted emission mask) corresponds to the UE transmitting over a number of RBs (i.e. the emission bandwidth is equal to the RB bandwidth x Number of RBs requested per user) while the system bandwidth is equal to the total RBs x RB bandwidth. Note that the system bandwidth is input to SEAMCAT and approximately RB bandwidth x the total Number of RBs (i.e. Max. subcarriers per BS input).

Figure 203: Illustration of the emission spectrum mask in UL for a LTE transmission

### OFDMA uplink

This panel is available only if OFDMA UL is selected and is the input parameters to the UL OFDMA power control described in Section ‎9.10.

Table 36: OFDMA UL power control input parameters

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Maximum allowed transmit power of mobile |  | Scalar | dBm | Transmit power of the UE |
| Minimum transmit power of mobile |  | Scalar | dBm | Minimum transmit power used in the power control. |
| Power Scaling Threshold |  | Scalar |  | Used in the calculation of the path loss limit for the power control. It is a limit threshold compared to the value of the CDF used in the power control. With this power control scheme, the 1-“ Power Scaling Threshold” percent of UEs will transmit at Pmax, i.e. are not power controlled |
| Balancing factor (0<γ<1) |  | Scalar |  | Balancing factor for UEs with bad channel and UEs with good channel. |
| Estimate coupling loss percentile | CLx-ile | calculator |  | This calculator allows you to estimate the coupling loss percentile used in the simulation |

### OFDMA UE Distribution

### This panel is available only if OFDMA UL is selected.

Table 37: OFDMA UL UE distribution input parameters

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| User defined active MS per BS |  | Scalar |  | Number of active UEs per BS. Each UE will be assigned the number of RBs specified in the OFDMA general settings parameter “Number of RBs per mobile”. |
| Frequency Scheduling |  |  |  | A list of possible frequencies with corresponding probabilities that each frequency is used. |

### OFDMA downlink

This panel is available only if OFDMA DL is selected. There is no power control for OFDMA DL.

Table 38: OFDMA DL BS transmit power definition

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Base Station Maximum transmit power |  | Scalar | dBm | Transmit power of the BS |

### OFDMA Capacity

The panel is similar for the OFDMA DL and UL.

Table 39: OFDMA capacity definition (i.e. number of UE from which active UEs will be selected)

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Users per BS |  | Scalar |  | Defines how many mobiles per cell should be generated in the system. For each base station (BS), each UE will be added to served UE list of that BS. Depending on the propagation or handover conditions, a UE will either remain connected to the BS or will be disconnected. THIS IS NOT the number of active users. The number of active users is the ratio of Max subcarriers per Base Station/ Number of subcarriers per mobile as described in Table 35. The number of active users can also be overridden for OFDMA UL using “User defined active MS per BS” described in Table 37. |

### Propagation model

You can choose the suitable propagation model to be applied when calculating signal loss along the transmitter and the receiver path. A choice and settings of propagation models are presented in ‎ANNEX 17:.

## Positioning

See section ‎7.5, common to CDMA and OFDMA on the positioning of BSs and MSs.

## Link-to-system level mapping

A look up table is used to map throughput in terms of spectral efficiency (bps per Hz) with respect to calculated SINR (= C/(I+N)) (dB) level. This link level data (bitrate mapping) is user selectable and can be modified depending on the simulation to perform.

Figure 204: Throughput vs SINR for Baseline E-UTRA Coexistence Studies (source: ‎[12])

## Achieved bit rate

The achieved bit rate is calculated as follows:

(Eq. 45)

The achieved aggregate bit rate is calculated over all the mobiles connected to the reference cell.

## DL C/I calculation

The relationship between the contributors of the interference in a OFDMA network is illustrated in Figure 205

IT

Iext = iRSS

dRSS

VLT

VLR

Ref. cell

adj. cell

Iinter

BS1

BS2

Figure 205: Illustration of the interference mechanism in the OFDMA module

Figure 205 illustrates the interference mechanism in the OFDMA module where the inter-system or also called self interference is noted “Iinter” and the interference from an “external” interference system is referred to as “Iext”. In the SEAMCAT OFDMA implementation, the term “BS” and “cell” have the same meaning.

The C/I calculation in DL is calculated as

(Eq. 46)

where C(j,k) is the received power at the k-th user from the serving BS, i.e., the j-th BS

(Eq. 47)

and where  is the power of resource block and effective_pathloss is defined as in Section ‎7.6.1.

I(j,k) is the sum of the interference power

(Eq. 48)

where

Iinter (j,k) is the adjacent cell interference (i.e. from the same victim system, i.e. denoted inter-system interference;

Iext(j,k) is the interference from external interfering system(s) in adjacent channel, i.e interference power into this resource block including ACIR;

N is the noise floor.

,			(Eq. 49)

(Eq. 50)

Iext(j,k) is the interference to the the victim UEj,k from N interfering link transmitters (ILT). Note that the ACIR (Adjacent Channel Interference Ratio) is implicitly taken into account when both unwanted and blocking mechanism are summed in the computation. The unwanted and blocking are defined as follow

(Eq. 51)

for each of the victim UEj,k’s frequency where the DL information is received. The ILT can be any generic systems or a BS/UEs of a cellular system.

(Eq. 52)

at the victim system frequency. Where NRB is the number of RBs (i.e. subcarriers) requested per UE, and M is the maximum number of RBs per BS.

N=10-173.977+10log10NRB×RBsBandwidth+NoiseFigureUE10	 (Eq. 53)

where NoiseFigureUE is the noise figure of a UE.

## UL C/I calculation

The C/I calculation in UL is calculated so that C(j,k) is the received power from the UEj,k at the j-th BS.

(Eq. 54)

where Pt is the transmit power of the UE in dBm (see UL Power control below) and effective_pathloss is defined as in Section ‎7.6.1..

The total interference to a UL cellular system is derived from

(Eq. 55)

where

Iinter is the interference coming from UEs of the same system but from adjacent cells, i.e. the inter-system interference from other cells. Since a fully orthogonal system is assumed, only UEs  which transmit in the same frequency subcarriers will introduce interference to each other, hence only UEs in other cells with the same k index are considered;

Iext(j,k) is the interference from external interfering system(s) in adjacent channel;

N is the noise floor.

(Eq. 56)

(Eq. 57)

Iext(j,k) is the interference to the the victim BSj from N interfering link transmitters (ILT), where

(Eq. 58)

for each of the victim BSj’s frequency where the UL information is received. The ILT can be any generic systems or a BS/UEs of a cellular system.

(Eq. 59)

at the victim system frequency. Where NRB is the number of RBs (i.e. subcarriers) requested per UE, and M is the maximum number of RBs per BS.

N=10-173.977+10log10NRB×RBsBandwidth+NoiseFigureBS10 			(Eq. 60)

where NoiseFigureBS is the noise figure of a BS.

## UL calculation of the UE frequencies

The frequency of the UE in UL is calculated as follow

FUE=Fsystem-BWsystem2+Diff2+NRBUE×BWRB2×indexlink×2+1 		(Eq. 61)

with Diff taking into account for any difference between the BWsystem and the effective bandwidth (NRBBS×BWRB) so that

Diff= BWsystem-NRBBS×BWRB				(Eq. 62)

where

FUE: Centre frequency of the UE;

Fsystem: Frequency of the system (i.e. the network);

BWsystem: Bandwidth of the system;

NRBUE: Number of Resource Blocks (RB) per mobile;

NRBBS: Number of RB for the BS;

BWRB: Bandwidth of the RB;

NUE: Number of UEs in the system (calculated as NRB_BS/NRB_UE);

Indexlink: Index of the specific link UE to serving BS (input to SEAMCAT). Index = [0, NUE-1].

Figure 206: Illustration of the calculation of the UE frequencies in UL

## OFDMA UL power control

In OFDMA UL, the power control is applied to the active users (i.e. the mobile users with specific RBs) so that the UE Tx power is adjusted with respect to the effective path loss (i.e. based on the MCL) to the BS it is connected to. In 3GPP ‎[12], the UL power control is defined so that the UE transmit power is set such as:

(Eq. 63)

where:

Pt is the UE Tx power in dBm;

Pmax is the maximum transmit power in dBm;

Rmin is the minimum power reduction ratio to prevent UEs with good channels to transmit at very low power level. Rmin is set by Pmin / Pmax.

CL is the effective path loss in dB for the UE from its serving BS;

CLx-ile is the x-percentile effective path loss (plus shadowing) value. CLx-ile is defined here as the value in the CDF, which is greater than the effective path loss of x percent of the MSs in the cell from the BS (i.e. it corresponds to the parameter “power Scale Threshold”. It is set by default to 0.9, but you can change it;

γ is assumed to equal to 1 by default in SEAMCAT.

With this power control scheme, the 1-x percent of UEs that have a path-loss greater than CLx-ile will transmit at Pmax, i.e. are not power controlled. Annex ‎A15.6 provides further information about the implementation and the usage of the OFDMA UL power control.

## Pathloss Correlation

The concept of a simple correlation model for shadow fading has been widely adopted in LTE co-existence studies mostly employed in uplink case. The propagation attenuation is modelled as the product of the path loss and the shadow fading. The shadow fading is well approximated by a log-normal distribution. Let z denotes shadow fading in dB with zero mean and variance σ2. Then the shadow fading of path from one UE to the i-th BS is expressed as

zi=a*x + b*yi , 	(Eq. 64)

where a2+b2=1 and x and yi are independent Gaussian distributed variables, both with zero mean and variance σ2 . yi and yj for are independent as well.

Figure 207 presents how to set-up the pathloss correlation in SEAMCAT (only available for OFDMA). The panel is similar for the OFDMA DL and UL.

Figure 207: Illustration of the pathloss correlation – input parameters

Figure 208: Illustration of the pathloss correlation principle

Thus, the correlation coefficient of the shadow fading from one UE to two different BSs, i.e., the i-th and j-th BS, is

EzizjEzi2=a2	(Eq. 65)

In most LTE studies, a=b=12 is assumed ‎[12]. For cellular systems with three-sector antennas, the shadowing correlation between sites (equivalent to BS in Omni antenna system) is of 0.5 and correlation between sectors of the same site is consequently of 1.

# Scenario

## Setting the scenario

A typical scenario consists of one victim link which describes the communication system being interfered and at least one interfering link which describes the interfering system(s) that may cause interference to the victim link. CDMA or OFDMA systems are modeled differently, using special algorithm that creates a grid of multiple cells.

Set the scenario by selecting what system will be the victim and what system will be the interferer. Remember that setting the frequency at the “scenario level” overwrite any settings at the “System level”.

Figure 209: Setting the simulation scenario

Table 40: Simulation scenario – Victim System

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Frequency | fVLT or fILT | Distribution or Scalar | MHz | Distribution of the centre frequency of the victim system or the interfering system links. |

The simulation control is explained in Section ‎2.11.

## Multiple Interferers generation

You have 3 options to generate multiple interferers in SEAMCAT.

### Generation of multiple interferer links with different systems

This option allows SEAMCAT to generate multiple interferers which may have the same or different technical characteristics from each other.  The following menu buttons are available in the interfering system links control panel.

|  | Add an interfering systems link to the scenario |  | Generate multiple interfering links |
| --- | --- | --- | --- |
|  | Duplicate an interefering link |  | Change the selcted system type |
|  | Delete a link |  | On-line manual help |

Figure 210: Interfering system links control

Figure 211: Generation of multiple interferer links with different 
technical characteristics from each other

The feature “to position with” allows the deployment of a second type of interferer (for instance interfering link 1) for which the transmitters will be located at the same location as the transmitters of another type of interferers (i.e. interfering link 2 or interfering link 3). This feature is of interest since it allows deploying these two interferers at the same location (i.e. with the same coordinates) and these two transmitters could be transmitting at the same time while having different transmitter characteristics (e.g. emission mask, antenna radiation pattern…) or with a relative X, Y position set by DeltaX and DeltaY.

Figure 212: Possibility to position interferer links with one another

If for one interfering link (e.g. interfering link 1) the number of active transmitter is one, then for any extra interfering links, only one Tx is simulated. When the “to position with” feature is selected, any values are grey shaded and only one transmitter is simulated.

### Auto-generation of multiple interfering links

This option corresponds to duplicate n times a specific interfering links on a circle or on a hexagonal grid as illustrated below in (a) and (b) respectively. It is available by clicking on (). These interferers have the same characteristics as the reference interfering link. It has the purpose of automatically generating a regular pattern of interfering links.

Figure 213: (a) Circular and (b) hexagonal layout

Figure 214: Auto-generation of multiple interfering links

The multiple generate feature graphical interface consists of 3 parts:

Selection of the reference interferer

Relative position of this reference interferer to the victim link

Layout preview of the new interferers

(a) Selection of the reference interferer

You can choose an interfering link that will be used to clone the new interferers.

Figure 215: Selection of the reference interferers

In the Generate Multiple Interfering Link dialog box, when selecting ok, 6 new interferers will now be present in the workspace, centered to the selected interferer (i.e. interfering link 1).

Figure 216: Generated new interferes, centered on interfering link 1

(b) Relative position of this reference interferer to the victim link

You can adjust the position of the interferers with respect to either the VLT (Victim link transmitter) or the VLR (Victim link receiver). When the generate multiple feature is run the relative positioning of interfering link mode (i.e. in the victim receiver to interfering transmitter path tab) is by default overwritten. In this case the center of the interferers is set to (1,1) to VLT.

Figure 217: Relative position of the interferer to the VLR or VLT

(c) Layout preview of the new interferers

As a results the preview will display the following illustration

Figure 218: Layout preview of the relative position of the interferer to the VLR or VLT

In the appearing dialog window, you may select the parameters described in .

Table 41: Generate Multiple Interfering Link GUI input parameters

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Circular or hexagonal layout | - | - | - | General circular or hexagonal layout |
| Number of tiers of generated multiple cells | - | Scalar | - | You can generate as many tiers as you want |
| Number of links in the first tier | - | Scalar | - | You can set the total number of links in the first tier |
| Intersite distance | D | Scalar | km | Distance between 2 BSs |
| Displacement angle | θ | Scalar | degree | Angle between the horizontal and the first BS (counter clockwise) |
| Angle offset |  | Scalar | degree | Angle offset of the displacement angle |

(d) Illustrative example of the generation of multiple interfering link

The below figure shows an example of scenario that may be used for estimating intra‐service interference to a victim base station of cellular system from mobile transmitters operating in the next tier of co‐channel cells of the same system.

The displacement angle is calculated automatically by the dialogue window by evenly spacing the specified number of cells around the 360 deg arc, but you may amend this angle. e.g. in order to achieve placement of multiple cells in a sector of less than 360 deg. The parameter angle offset may be used to specify the offset of an azimuth towards the first interfering cell with regard to the x‐axis, as seen from the centre cell.

During the multiple link generation, the intersite distance parameter (0.433 km in the example), in combination with the specified initial offset angle, will overwrite the original coordinates (Delta X/Delta Y) in the Vr‐It path tab setting of the Interfering links.

Figure 219: Example where 1 tier is used to position 4 interferers in a square shape 
(i.e. corners of a building) with the Vr positioned outside the square

### Generation of interferers with the same characteristics

Within one interfering link, you can define a number of active interfering transmitters when the mode "None" or "Uniform density" is selected. These active transmitters have the same technical characteristics (i.e. a simple duplicate) and they are deployed spatially independently according to the mode selected. The iRSS result is stored as one vector (of size number of events) where for each event the iRSS value is the simple power summation of the number of active transmitters.

The number of active transmitters is directly used to compute the simulation radius (see Annex ‎A13.2).

Figure 220: Generation of multiple interferers with the same characteristics and 
using a specific deployment mode

## Interfering Link Transmitter to Victim Link Receiver Path (ILT -> VLR)

The ILT to VLR path can have several combinations as shown in Figure 222. Four panels characterised the path between the ILT and ILR.

Figure 221: ILT to VLR path combination with generic and cellular system

Figure 222: Transmitter to Victim Link Receiver Path (ILT -> VLR)

### Relative positioning of interfering link (Generic system)

The relative position of the Victim Receiver (VLR) and the Interfering Transmitter (ILT) depends on the various options presented below. There is a unique simulation radius (Rsimu) contrary to the 2 coverage radius (one for the victim and one for the interferer link). This is illustrated below in Figure 223 for a generic system interfering with a second generic system.

See ‎ANNEX 12: for further details on the algorithm and conventions.

Figure 223: Example of the simulation radius (VLR with ILT)

Depending on the system simulated several positioning options are possible when the generic system is the interferer and the victim is a generic system and cellular system as shown in Figure 224 and Figure 227 respectively.

Figure 224: Relative positioning of a generic interfering link with a generic victim system

Each interfering signal calculation results from the contribution of

None: nactive interefering link transmitters located in a circular area with the simulation radius. You define yourself the radius. The random placement of the interefering link transmitters in this area is defined by the path azimuth and the path distance factor parameters.

- See Annex ‎A13.2.1 for detailed algorithm.

Table 42: ILT-VLR path - none mode (generic vs generic)

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Reference component | - |  | - | Positioning of the distributed component which is either the ILT or the ILR |
| Position relative to | - | - | - | Positioning of the Reference component relative to either the VLR or the VLT |
| Delta X | ∆X | Distribution or Scalar | km | Horizontal distance between the transmitter and receiver. It can be used to shift horizontally the distributed receivers. |
| Delta Y | ∆Y | Distribution or Scalar | km | Vertical distance between the transmitter and receiver. It can be used to shift vertically the distributed receivers. |
| Set ILR at he center of the ILT distribution | - | Boolean | - | set the distance factor distribution of the ILT with regards to the VLR. It overwrites the settings in the transmitter to reveicer path of the interferer. |
| Path azimuth |  | Distribution or Scalar | Deg | Horizontal angle for the location of the ILT respect to the victim link. If constant, the Rx’s location will be on a straight line. If not, the location of the Rx will be on an angular area. (See Annex ‎A1.1) |
| Path distance factor |  | Distribution or Scalar |  | Distance factor to describe path length between the ILT and VLR. This factor will be multiplied by Rsimu to obtain the coverage area. Therefore, the trialled distance between ILT and VLR will be Rsimu *Path factor. E.g. if user enters a distribution 0…1, then the distance will be between 0 and Rsimu. If the path factor is constant, the ILT will be located on a circle around the VLR which means that the distance between the ILT and VLR will not change |
| Simulation radius | Rsimu |  | km | User defined |
| Number of active transmitter | nactive | Scalar |  | If nactive>1, this will result in spatially-independent generation of the specified number of Its, whereas the resulting total iRSS strength will be obtained by simple power summation of the individual iRSS signal values. |
| Minimum coupling loss | MCL | Distribution or Scalar | dB | The minimum path loss. It is used in the calculation of the effective path loss (Section ‎7.6) |
| Protection distance | d0 | Distribution or Scalar | (km) | minimum protection distance between the victim link receiver and interefering link transmitter (Section ‎A13.2.3) |
| Use of polygon |  |  |  | You are also able to select a polygon shape as an alternative to the default circle. A various selection of polygon is available. You are able to rotate counter-clock wise (ccw) the polygon shape. |
| Co-locate |  |  |  | This feature allows deploying two interferers at the same location and their two transmitters could be transmitting at the same time while having different transmitter characteristics (e.g. emission mask, antenna radiation pattern…) |

Uniform density: Each interfering signal calculation results from the contribution of nactive interefering link transmitters uniformly located in a circular area. The parameters are taken from the system settings (see section ‎A13.2.2.)

Figure 225: Transmitter density and traffic

- The simulation radius is automatically derived from the density and activity of transmitters (see the Interferers density panels) in Annex ‎A13.2.2.

Table 43: ILT-VLR path - Uniform density mode (generic vs generic)

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Reference component | - |  | - | Positioning of the distributed component which is either the ILT or the ILR |
| Position relative to | - | - | - | Positioning of the Reference component relative to either the VLR or the VLT |
| Delta X | ∆X | Distribution or Scalar | km | Horizontal distance between the transmitter and receiver. It can be used to shift horizontally the distributed receivers. |
| Delta Y | ∆Y | Distribution or Scalar | km | Vertical distance between the transmitter and receiver. It can be used to shift vertically the distributed receivers. |
| set ILR at he center of the ILT distribution | - | Boolean | - | set the distance factor distribution of the ILT with regards to the VLR. It overwrites the settings in the transmitter to reveicer path of the interferer. |
| Path azimuth |  | Distribution or Scalar | Deg | Horizontal angle for the location of the ILT respect to the victim link. If constant, the Rx’s location will be on a straight line. If not, the location of the Rx will be on an angular area. (See Annex ‎A1.1) |
| Number of active transmitter | nactive | Scalar |  | number of active interferers in the simulation (nactive should be sufficiently large so that the (n+1)th interferer would bring a negligible additional interfering power). If nactive>1, this will result in spatially-independent generation of the specified number of Its, whereas the resulting total iRSS strength will be obtained by simple power summation of the individual iRSS signal values. |
| Simulation radius | Rsimu |  | km | Note: the simulation radius value is readable only after each simulation |
| Interferes density |  |  |  | A simulation radius is calculated, Rsimu. Interefering link transmitters will be randomly deployed within the area centred on the Victim link receiver and delimited by the simulation radius Rsimu. If a protection is defined then Interefering link transmitters will be randomly deployed within the area centred in the Victim link receiver and delimited by the protection distance and the simulation radius Rsimu . See Table 47 for information on the input parameter and Annex ‎A13.2.2 for the calculation. |
| Minimum coupling loss | MCL | Distribution or Scalar | dB | The minimum path loss. It is used in the calculation of the effective path loss (Section ‎7.6) |
| Protection distance | d0 | Scalar | (km) | minimum protection distance between the victim link receiver and interefering link transmitter (Section ‎A13.2.3) |
| Co-locate |  |  |  | This feature allows deploying two interferers at the same location and their two transmitters could be transmitting at the same time while having different transmitter characteristics (e.g. emission mask, antenna radiation pattern…) |

Closest interferer: Each interfering signal calculation results from the contribution of just one interefering link transmitter. The parameters are taken from the system settings (see section ‎A13.2.4)

Figure 226: Transmitter density and traffic

- This ILT is randomly placed in a circular area with a simulation radius derived from the density of interferers. See Annex ‎A13.2.4 for detailed alogorithm.

Table 44: ILT-VLR path - Closest interferer mode (generic vs generic)

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Reference component | - |  | - | Positioning of the distributed component which is either the ILT or the ILR |
| Position relative to | - | - | - | Positioning of the Reference component relative to either the VLR or the VLT |
| Delta X | ∆X | Distribution or Scalar | km | Horizontal distance between the transmitter and receiver. It can be used to shift horizontally the distributed receivers. |
| Delta Y | ∆Y | Distribution or Scalar | km | Vertical distance between the transmitter and receiver. It can be used to shift vertically the distributed receivers. |
| set ILR at he center of the ILT distribution | - | Boolean | - | set the distance factor distribution of the ILT with regards to the VLR. It overwrites the settings in the transmitter to reveicer path of the interferer. |
| Path azimuth |  | Distribution or Scalar | Deg | Horizontal angle for the location of the ILT respect to the victim link. If constant, the Rx’s location will be on a straight line. If not, the location of the Rx will be on an angular area. (See Annex ‎A1.1) |
| Number of active transmitter | nactive | Scalar |  | number of active interferers in the simulation (nactive should be sufficiently large so that the (n+1)th interferer would bring a negligible additional interfering power). If nactive>1, this will result in spatially-independent generation of the specified number of Its, whereas the resulting total iRSS strength will be obtained by simple power summation of the individual iRSS signal values. |
| Simulation radius | Rsimu |  | km | Note: the simulation radius value is readable only after each simulation |
| Interferes density |  |  |  | The distance between the Victim link receiver and the Interefering link transmitter follows a Rayleigh distribution, where the standard deviation is given by . See Table 47 for information on the input parameter and Annex ‎A13.2.4 for the calculation. |
| Minimum coupling loss | MCL | Distribution or Scalar | dB | The minimum path loss. It is used in the calculation of the effective path loss (Section ‎7.6) |
| Protection distance | d0 | Scalar | (km) | minimum protection distance between the victim link receiver and interefering link transmitter (Section ‎A13.2.3) |
| Co-locate |  |  |  | This feature allows deploying two interferers at the same location and their two transmitters could be transmitting at the same time while having different transmitter characteristics (e.g. emission mask, antenna radiation pattern…) |

Correlated : It is called the correlated mode. It means that the positions of the receiver and transmitter are geographically fixed with respect to each other (e.g. co-located or constantly spaced base stations). In the following four cases of fixed placement, the relative location of the two pair of transmitter and receiver is described by dX/dY displacement, with the origin being either on the Transmitter or Receiver of the victim link depending on the option selected;

Table 45: ILT-VLR path – Correlated mode (generic vs generic)

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Reference component | - |  | - | Positioning of the distributed component which is either the ILT or the ILR |
| Position relative to VLT or VLR | - |  | - | Positioning of the fixed interefer transmitter (ILT) or receiver (ILR) with the origin beingReference component relative to either onthe VLR or the victim link transmitter (VLT) or receiver (VLR) on the option selected. |
| Delta X | ∆X | Distribution or Scalar | km | Horizontal distance between the transmitter and receiver. It can be used to shift horizontally the distributed receivers. |
| Delta Y | ∆Y | Distribution or Scalar | km | Vertical distance between the transmitter and receiver. It can be used to shift vertically the distributed receivers. |
| Minimum coupling loss | MCL | Distribution or Scalar | dB | The minimum path loss. It is used in the calculation of the effective path loss (Section ‎7.6) |

In the case the victim system is a cellular system (CDMA or OFDMA, either UL or DL), the options are slightly changed as shown below, where Position relative to is always the BS of the reference cell.

Figure 227: Relative positioning of a generic interfering link with a cellular victim system

### Relative positioning of interfering link (Cellular system)

The relative position of the Victim Receiver (VLR) and the Interfering cellular system depends on the various options presented below.

Figure 228: Relative positioning of a cellular interfering link with a generic victim system

Cor. (interfering BS ref. cell): in which case the relative location is explicitely defined by the dX/dY values given in the scenario and the reference is the BS ref.cell. It is a similar mode as described in Table 45 where the BS ref.cell of the cellular interferer is position with respect to the VLT or VLR depending on the selection;

Dyn (interfering BS ref.cell): this dynamic distance mode provides a a relative location that follows a uniform distribution in the distance and angle domain.

Table 46: ILT-VLR path – Correlated mode (cellular vs generic)

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Position relative to VLT or VLR | - | Boolean | - | Positioning of the fixed interefer transmitter (ILT) or receiver (ILR) with the origin being either on the victim link transmitter (VLT) or receiver (VLR) on the option selected. |
| Delta X | ∆X | Distribution or Scalar | km | Horizontal distance between the transmitter and receiver. It can be used to shift horizontally the distributed receivers. |
| Delta Y | ∆Y | Distribution or Scalar | km | Vertical distance between the transmitter and receiver. It can be used to shift vertically the distributed receivers. |
| Path azimuth | - | Distribution or Scalar | Deg | Horizontal angle for the location of the interfering BS ref.cell respect to the VLR or VLT |
| Path distance | - | Distribution or Scalar | km | Path length between the interfering BS ref.cell respect to the VLR or VLT . |
| Minimum coupling loss | MCL | Distribution or Scalar | dB | The minimum path loss. It is used in the calculation of the effective path loss (Section ‎7.6) |

Figure 229: Relative positioning of a cellular interfering link with a cellular victim system

Cor. (victim BS ref.cell  interfering BS ref.cell): It is the same mode as described in Table 45 but where the BS of the reference cell of the victim cellular network is the reference position of the BS of the reference cell of the interfering cellular network.

Dyn. (victim BS ref.cell  interfering BS ref.cell): It is the same mode as described in Table 48, but where the BS of the reference cell of the victim cellular network is the reference position of the BS of the reference cell of the interfering cellular network.

### Interferers density

The panel is activated if "Uniform density" or/and "closest interferer" mode is selected. See Annex ‎A13.2.2 for more details on the calculation.

Figure 230: Interferers density panel (only in "Uniform density" and "closest interferer” mode)

Table 47: Setting up the interferes density

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Density of transmitters | densit | Scalar | 1/km2 | Maximum number of active transmitters per km2 |
| Probability of transmission | Pit | Scalar | % |  |
| Activity | activityit | Function (X,Y) | 1/h | Temporal activity variation as a function of the time of the day (hh/mm/ss) |
| Time | time | Scalar | hour | Time of the day. If the activity function (above), here it should be specified which hour (from the defined range of function) should be considered in a simulation |

### Pathloss correlation

The panel is activated if the victim is either OFDMA UL or OFDMA DL. It is decribed in more details in Section ‎9.11.

### Propagation Model

You can choose the suitable propagation model to be applied when calculating signal loss between the ILT and the VLR. A choice and settings of propagation models are presented in ‎ANNEX 17:.

## Interfering link transmitter to victim link transmitter path (spectrum sensing)

### Spectrum sensing characteristics

When the spectrum sensing is activated, the tab “Interfering link transmitter to victim link transmitter path” will become editable (#1 of Figure 231) and you can set the input parameters of the CR algorithm (#2). Note that the frequency of the interferer is disabled (#3). The purpose of the CR algorithm in SEAMCAT automatically calculates the number of possible channels the WSD will operate in based on the operating frequency range of the victim system and its victim link receiver bandwidth (#4).

You can not simulate “OFDMA/CDMA” as a victim and have a CR interferer. The implementation only considers generic versus generic

Figure 231: Example of the Cognitive Radio GUI selection - Input settings

When an interferer is set as a CR, the emission characteristics (i.e. transmitted power, emission mask and unwanted emission mask) have to be entered (see Section ‎6.3) and the spectrum sensing characteristics presented in Figure 232 have to be entered.

Figure 232: Setting the spectrum sensing characteristics in the Victim link 
transmitter to Interefering link transmitter Path

Table 48: Spectrum sensing characteristics

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Detection threshold: |  | Function (X,Y) or Scalar (offset) | dBm | Define the detection threshold for the spectrum sensing in a offset function. Either a constant value (i.e. flat over the spectrum) or as a user defined function as shown in #1 of Figure 232 illustrates the setting of the detection threshold (a) as a constant or (b) as a function. Figure 233 (c) illustrates where the offset refers to. Note the user-defined function is defined as offset with the victim frequency being the reference. The offset 0 is refered to the Victim frequency. |
| Probability of failure: |  | Scalar | % | You can select this function as shown in #2 of Figure 232. The probability of failure is given in percentage. In the illustration below a probability of failure of 10 % is entered. Positive value from 0 to 100. |
| Sensing reception bandwidth |  | Scalar | kHz | Define the bandwidth of the sensing device (i.e. ILT). It is used in the calculation of the sRSS: This is a constant value given in kHz as shown in #3 of Figure 232. |
| e.i.r.p. max In-block limit |  | Function (X,Y) (offset) | Offset (MHz)/ Mask (dBm)/ Ref.BW (kHz) | Define the E.I.R.Pmax In-block limit to protect the victim system as an offset function where the offset 0 is refered to the selected interfering frequency. The outcome of the algorithm set the allowed power at the ILT. It has the following components [offset, Mask, Ref.BW] where Offset in MHz is equivalent to the “DTT in use at” columns, Mask in dBm is the “In-block CR EIRPmax limit” and Ref. BW is the bandwidth of the DTT as shown in #4 of Figure 232. Note that SEAMCAT will normalise any value entered in the table to 1 MHz and convert back to the victim bandwidth. |

### Detection of threshold

In relation to the spectrum sensing results, if this CR detect that a victim system is in the vicinity it will select an appropriate operating frequency and it will lower its emission based on an e.i.r.p. max in block limit defined in the spectrum sensing characteristics. Figure 233 presents an example of the detection threshold (a) as a constant or (b) as a function and illustrates in (c) where the offset refers to and its evolution from event to event.

(a)

(b)

(c)

Figure 233: Example of the detection threshold (a) as a constant or (b) as a function and illustrates in (c) where the offset refers to

### Probability of failure

This feature is input selectable (by default, it is de-activated).The probability of failure may account for the failure in selecting wrongly a non_available channel for one event.

This means that when a failure appears, a channel which was initially selected as occupied by a victim DTT becomes “wrongly” available for the WSD to transmit. This results in a “conflict” situation.

For instance with a defined pfailure , that means that for x total of WSD (initial input to SEAMCAT) there is x*pfailure WSDs which will transmit in the victim frequency without power constraint. pfailure is an input parameter.

### Adjacent channel scenario – e.i.r.p. max. in-block limit

In the case where the WSDs are not allowed to transmit in the same operating frequency as for the victim DTT device, the WSDs can decide to transmit in the adjacent bands or channels. This scenario is illustrated in Figure 234 In this example the WSDs have sensed that in the channel 6 there is a victim system (here a DTT), therefore the WSDs will choose other channels to transmit.

The maximum permitted in-block and out-of-block e.i.r.p. of autonomous CRs would be specified as a function of the guard band with respect to DTT channels used in the local proximity of the CR. The available guard band would be identified by comparison of the detected DTT signal powers against a fixed detection threshold.

Figure 234: Illustration of WSD1 detecting a victim device in channel 6 and as a consequence decides to operate in channel 3 which is available

The purpose of the SEAMCAT simulation is to investigate the level of interference created by WSDs to the DTT victim device. Therefore the iRSSunwanted and iRSSblocking for a WSD will be computed.

As a reminder, the e.i.r.p. (Equivalent isotropically radiated power) is defined as:

e.i.r.p.max = PILT -Lc+ GmaxIt→VLR 					(Eq. 66)

where Lc is the cable loss in dB. We will neglect Lc.

Extract the Tx power = e.i.r.p. max - GmaxIt→VLR and calculate the iRSSunwanted and the iRSSblocking from the WSD to the victim DTT device. As a result, the interference calculation can be performed on the summation of the iRSSunwanted per channel and iRSSblocking per channel in the case where there are multiple active WSDs per channel. The determination of the e.i.r.p. max in-block limit is illustrated in Annex ‎A16.2.

An example of In-block input values (dBm), is presented in Table 49 and Figure 235 illustrates how to set this parameter in SEAMCAT.

Table 49: Example of In-block CR e.i.r.p. max. emission limits as a function of guard band with respect to a victim DTT with channel bandwidth of 8 MHz (source SE43(10)18)

| DTT in use at | In-block CR e.i.r.p. max limit (dBm) |
| --- | --- |
| co-channel | - |
| n 1 | -12.8 |
| n 2 | 3.2 |
| n 3 | 11.2 |
| n 4 | 16.2 |
| n 5 | 20.2 |
| n 6 | 16.2 |
| n + 6 | 21.2 |
| n 7 | 22.2 |
| n 8 | 23.2 |
| n 9 | 4.2 |
| n + 9 | 23.2 |
| n 10 | 24.2 |
| > n 11 | 25.2 |

Figure 235: GUI of the In-block CR e.i.r.p. max limit (dBm)

### Propagation Model

You can choose the suitable propagation model to be applied when calculating signal loss along the transmitter and the receiver path. A choice and settings of propagation models are presented in ‎ANNEX 17:.

# Event Processing

## Event processing concept

The Event Processing tab is the place in SEAMCAT where you can set the Event Processing Plugin (EPP). The EPP is a plugin that is given access to the EventResults, one by one, as they are produced by the simulation - an EPP can be viewed as an adjacent simulation. An EPP is allowed to inspect the scenario and the EventResult and based on this observation it can produce its own results. Once an EPP is installed in SEAMCAT it can be configured for any scenario possible.

Figure 236 depicts the basic flow of SEAMCAT for a single thread example i.e. One event result (Ri) is calculated at a time in sequence.

Figure 236: Flow of SEAMCAT

Each of the produced event results can be ‘processed’ by a plugin in the sense that it can make additional computation based on the event result and scenario settings. This has huge potential. Basically there is no limits to how this can be used!

Figure 237: EPP Flow in SEAMCAT

EPPs have a dedicated thread to run in, and therefore run at the same time as the simulation runs as depicted in Figure 237. EPP inputs are exactly the same for other plugin inputs, but the EPP results are very different. The EPP can produce results like Single values, Vectors, Vector groups, Scatter diagrams, Barcharts.

## Event processing GUI

The Event processing GUI consists of 2 main parts as shown in Figure 238. On the left hand side you can add () or duplicate () an EPP to the simulation, duplicate an EPP, remove () an EPP from the simulation and export () an EPP.

There exist some predefined EPPs builtin stored in the library environment (Section ‎13.5.4) but you can also install your own EPP into SEAMCAT by loading your own .jar file (Section ‎2.24). The list of builtin EPPs is not exhaustive and will increase with time and needs of SEAMCAT comunity.

A dedicated section on how to develop your own EPP is given in Section ‎14.

Figure 238: Event processing GUI

## Loading an EPP to a simulation

To use EPP in a simulation go to the event processing tab and click the plus sign (Figure 239).

Figure 239: Using EPP in a workspace

You will be prompted with a list of available EPPs and their preview as shown in Figure 240.

Figure 240: List of available EPPs

You can add as many EPPs as you want to your simulation.

## Configuring an EPP

The EPP can be configured as wanted for this particular simulationas soon as it is loaded to the simulation workspace. In this example the EPP called “Demo 1: Collect intermediary results” is loaded. With this EPP, you can select the intermediary vector results that you want to display after simulation.

Figure 241: Configuring an EPP

In case an EPP is used in a simulation, it can return a set of result. The EPP results are described in Section ‎12.7.

# Results

## Scenario outline

### Simulation Summary

The simulation summary (left of Figure 242) provides a quick access to the mean, median and standard deviation of the dRSS, iRSS unwanted and iRSSblocking

The mean is computed using the simulated results in dB. The median is computed using the simulated results converted to linear values and then converted again to dB. The standard deviation (StdDev) is computed using the simulated results in dB. This is further explained in Annex ‎A1.5.1

### Simulation status

The progress bar (right of Figure 242) reflects the percentage of events simulated with respect to overall number of events specified in the simulation control window in the scenario tab.

Figure 242: Simulation status

### Generic system outline

During the event generation, all involved transceivers (ILT, ILR, VLR and VLT) are plotted in graphical display window placed on the simulation outline tab.

Figure 243: Example of “Scenario outline” for a generic system 1 vs generic system 
2 type of simulation

Figure 244: dRSS, IRSS samples

The dRSS and the iRSS vectors are shown for 100 events maximum to avoid overloading the memory and to speed the computation time. The iRSS is shown as a summation of all the interferers.

### Cellular system outline

Once SEAMCAT has determined the number of UEs per cell either through simulation of optimal capacity (see Figure 197 for UL and Figure 198 for DL) or by a value that you specify, the actual simulation of snapshots begins. As with all scenarios this causes SEAMCAT to show the scenario outline (Figure 245 for UL and Figure 246 for DL).

Figure 245: Example: CDMA Uplink as victim – Base-stations are shown as VLR 
(several snapshots are displayed)

Figure 246: Example: CDMA Downlink as victim – Base-stations are shown as VLT (1 snapshot displayed here only)

Only CDMA base-stations will appear in the Scenario outline graph. Dependent on the scenario the base-stations positioning will appear as shown in Table 50.

Table 50: Role of the cellular element in the scenario outline

| Scenario configuration | BS role | UE role | Positioning | Illustration |
| --- | --- | --- | --- | --- |
| CDMA Downlink is victim | Victim link transmitter | Victim link receiver | Reference cell is positioned in (0,0) |  |
| CDMA Uplink is victim | Victim link receiver | Victim link transmitter | Reference cell is positioned in (0,0) |  |
| CDMA Downlink is interferer | Interefering link transmitter | Interfering link receiver | Relative to victim and reference cell |  |
| CDMA Uplink is interferer | Interfering link receiver | Interefering link transmitter | Relative to victim and reference cell |  |

## Overview of output results

Table 51 presents a summary of the ouput results for various victim systems.

Table 51: Interference criteria summary for various victim systems

| Victim system | Intereference criteria |
| --- | --- |
| Generic module (i.e. non CDMA/OFDMA) | Probability of interference based on C/I, C/(I+N), (N+I)/N, I/N |
| CDMA | Capacity loss (i.e. number of voice users being dropped) |
| OFDMA | Bitrate loss (i.e. number of bit rate lost compared to a non interfered victim network) |

## Generic model Results

After running a simulation with generic system, each of these signals is displayed with indicated array size (i.e. the number of valid generated events), unit and then the type (vector, double etc..).

Double-clicking on one of the vectors, the signal dialog window (see Annex ‎A1.5) will plot the signal as well as its statistical features.

Figure 247: List of the output results for generic vs generic simulation

### Generated signals results

dRSS folder (‎ANNEX 4:) with dRSS vector;

iRSS unwanted folder (‎ANNEX 5:) with as many iRSSunwanted vector for each interfering link and one vector for the sum;

iRSS blocking folder (‎ANNEX 5:) with as many iRSSblocking vector for each interfering link and one vector for the sum.

### Calculated radius

The calculated radius folder contains the values of the coverage and/or simulation radii calculated for the event generation when set to non-correlated (See ‎ANNEX 13:).

### Intermodulation results

iRSS intermodulation folder (Figure 248) with as many iRSSintermod vector for each combinatory couple in case there are more than one interfering link and one vector for the sum.

Figure 248: Intermodulation results folder

### Overloading results

As a result of the equation described in Annex ‎A5.4, for each event and at each frequency, the sum of the iRSSoverloading (iRSSsum_overloading) will be compared to the overloading treshold Oth(fi)  at that frequency (fi). The difference, delta (in dB), will be stored.

When delta ≥ 0, then the receiver is overloaded at that fi (i.e. the rest of the frequency do not matter);

When delta < 0, then the receiver is not overloaded at fi.

If only one frequency is present, the iRSSsum_overloading is compared to the Oth(fi) and the difference will be stored for that event.  If more than one frequency is present, the highest delta value will be indicated in the vector (i.e. irrespective of the frequency) , such that

for each event j (where dRSS > sensitivity){

for Frequency = i to number of total frequencies{ delta_max(j) = max(delta(fi)); } 	(Eq. 67)

}

Figure 249: Overloading results folder

The output delta overloading vector is of the dimension delta x number of events

Figure 250: Delta overloading result vector

## Spectrum sensing results

When the cognitive radio mode is activated, SEAMCAT resturns the output vector shown in Figure 251 and described in Table 52.

Figure 251: Output vector for the CR

Table 52: Output results for CR simulation

| # | Item | Description |
| --- | --- | --- |
| 1 | sRSS | sRSS value calculated at the selected WSD frequency (i.e. where the WSD is allowed to transmit) for each of the event |
| 2 | WSD frequency | The actual selected frequency at which the WSDs are allowed to transmit as the result of the spectrum sensing algorithm |
| 3 | WSD e.i.r.p. | The actual selected e.i.r.p. at which the WSDs are allowed to transmit as the result of the spectrum sensing algorithm |
| 4 | Victim frequency | frequency at which the victim device transmits per event |
| 5 | Average e.i.r.p. per event x active WSDs (for each frequency) | average e.i.r.p. per event for all the active WSDs transmitting at a certain frequency |
| 6 | Average Active WSD per event (for each frequency) |  |

At the end of the simulation, SEAMCAT provides a set of output vectors as above. Then you can perform the interference probability calculation as previously described.

### sRSS vector

The sRSS vector (dBm) is merely the sRSS value calculated at the selected WSD frequency (i.e. where the WSD is allowed to transmit) for each of the event. Figure 252 displays the vector for each WSD. You can select any or all of the WSD (#1). The same applies to the CDF and density graphs (#2).

Figure 252: sRSS vector output

You are able to save the results (#4) as .txt file for further post-processing.

Figure 253: Saving format of the sRSS vector for post-processing

### WSD frequency

This vector represents the actual selected frequency (MHz) at which the WSDs are allowed to transmit as the result of the spectrum sensing algorithm.

| (a) |  | (b) |
| --- | --- | --- |

Figure 254: Frequencies where the WSDs are transmitting as (a) vector display and (b) CDF

Figure 255: Density of the frequencies used by the WSDs

### WSD e.i.r.p.

The actual selected e.i.r.p. (dBm) at which the WSDs are allowed to transmit as the result of the spectrum sensing algorithm. This vector displays the e.i.r.p. for each of the WSD for all the events. In the example presented in Figure 256, the WSDs are transmitting with a e.i.r.p. between -10 and 20 dBm and -1000 dBm (equivalent to WSD switch off).

Figure 256: Example of the density of the e.i.r.p. per WSD

### Victim frequency vector

The victim frequency vector (MHz) represents the frequency at which the victim device transmits per event.

### Average e.i.r.p. per frequency

It presents the average e.i.r.p. per event (dBm vs MHz) for all the active WSDs transmitting at a certain frequency such that:

(Eq. 68)

Let us assume a different example from above, where 4 channels have been identified for the WSD to operate. Figure 257 shows that on average 33 dBm, for one event, was transmitted by the active WSDs at 1000.5 MHz , 8.82 dBm at 1001.5 MHz etc...

Figure 257: Average e.i.r.p. per frequency

### Average active WSDs per frequency vector

It provides the average number of active WSD per event for a specific frequency:

(Eq. 69)

Re-using the example of Figure 257, Figure 258 indicates that, with for instance 5 WSDs set as input parameters, an average of 0.63 WSDs were active at 1000.5 MHz (with 33 dBm e.i.r.p. Figure 257), 1.29 WSDs were active at 1001.5 MHz (with 8.82 dBm e.i.r.p. Figure 257), 1.57 active WSDs at 1002.5 MHz and 1.51 active WSDs at 1003.5 MHz. It can also be noted that in this particular example, the sum of the active WSDs across the selected frequencies is 5, meaning that all the simulated WSDs have been active and none have been turn off.

Figure 258: Average active WSDs per frequency

## CDMA output results

### CDMA capacity finding

During the pre-simulation part, the system estimates the load of the network as shown in Figure 259. The results are presented as in Figure 260.

Figure 259: Example of the CDMA UL capacity finding status

Figure 260: Pre-simulated results (victim case)

### CDMA results

Once SEAMCAT has completed the simulation, the results are shown as displayed in Figure 261, when the CDMA network is the victim. This figure presents the difference between the 2 steps power balancing process (1-initial power balancing, 2- power balancing after introduction of an external interference), that is to say " non-interfered capacity " is the number of UEs in the victim network prior to adding an external interferer and "interfered capacity" is the number of UE in the victim network after adding the external interference.

Units: number of connected UEs;

Initial capacity: Number of connected UEs before any external interference is considered;

Interfered capacity: Results after External interference is applied;

Excess outage, users: How many UEs were dropped due to external interference;

Outage percentage: Percentage of UEs dropped due to external interference.

When the CDMA system is the interfering link, the total received power at the receiver in the victim link, due to the transmit power of all the active mobile stations in the three cells of the center cell site of the CDMA cluster, adjusted for spectral masks, etc., is counted as the interfering power in the victim link.  Therefore, it is not necessary to keep track of any capacity loss in this case, unless the victim link is also a CDMA system.

Figure 261: Example of the CDMA results display panel

Figure 262: Example of the CDMA results display panel

SEAMCAT is able to calculate (for CDMA) 3 losses:

- Loss of UEs for the whole network based on the before and after number of UE

System capacity loss = 100 - (interfered_capacity/ non-interfered_capacity)*100 		(Eq. 70)

- Loss of UEs for the reference cell based on the before and after number of UE

Ref cell capacity loss = 100 - (interfered_capacity/ non-interfered_capacity)*100 		(Eq. 71)

- Loss of UEs for the whole network based on the total number of dropped UE

System capacity loss = total_dropped_UE_system/total_simulated_UE*100 		7.

It is quite important to understand that there is not a 1-to-1 map between non-interfered active/interfered users and the dropped users. Dropped user can occurs at many level of the algorithm, it can be due to:

“Unable to connect during first initialisation of UE” during the initialization (long before the step 2 balancing) but still it will be registered;

During the balancing. For CDMA UL (no cell selection activated), when the noise rise is balanced, i.e. the number of UE is acceptable is reached after introduction of external interferers,  then the BS estimates the signal-to-interference ratio (C/I), measured in bit energy-to-noise density ratio Eb/N0, and compares it to a target value (Eb/N0_target). If the difference between the estimated C/I and the Eb/N0_target, is higher than a call drop threshold, then the UE is dropped;

“Eb/No requirement does not meet while scaling the channel power” during the scaling power.

### CDMA results for cell selection algorithm

When the CDMA “cell selection” algorithm is simulated, the following output vectors are also available for scrutiny:

Average network noise rise (initial without external interference): value at step 1 (i.e. before the algorithm - see Annex ‎A15.3.2) Average network noise rise (initial): value at step 5 (i.e. before the algorithm - see Annex ‎A15.3.2) Average network noise rise (resulting): value at step 10 (i.e. after the algorithm - see Annex ‎A15.3.2) Capacity loss in the whole network (for each event, calculate the capacity loss in %). Capacity loss in the reference cell per event Capacity loss in the worst cell per event (the first strongest cell: selectedCell[1]). The cell-ID can be different from event to event but the capacity loss is to be extracted). The number of cells affected per event.

## OFDMA output results

The results of the OFDMA simulation are given in terms throughput loss of the OFDMA victim. Figure 263 presents an overview of the simulation results. It consists of the achieved bitrate (with or without external interference) for the reference cell or the whole system.

A summary of the bit rate loss expressed in percentage for both the reference cell and the entire OFDMA network (i.e. the whole system) is also available. The percentage calculation is performed for each snapshot and the mean of the percentage over all the snaphsots is deduced.

Figure 263: Example of the OFDMA results display panel

## EPP results

In case an EPP is used and it returns a set of result, they will also be included next to the statistic panel as shown in  Figure 264. The EPP can also produce results like Single values, Vectors, Vector groups, Scatter diagrams, Barcharts.

Figure 264: EPP results

## Cellular Structure

When simulating CDMA or OFDMA systems, you will have access to the additional tab "Cellular structure", which will become active after completion of the simulation. This new tab allows you to inspect the internal details of CDMA/OFDMA cluster based on data on one event.

After a simulation these GUI parts are used to provide access to calculated results but also detailed insight into the last event of the simulation as illustrated in Figure 265, but you can reproduce any event using the play/replay feature (Section ‎2.16).

The “Summary of event #n” and “inspect selected element” panel of Figure 265 are shared components from the CDMA and OFDMA module.

Figure 265: Example of the last event overview of the cellular network (CDMA)

### Plot configuration

The top part of the detailed system information screen contains a range of checkboxes used to control which information is plotted (Figure 266).  A full description of each checkbox is given in Table 53.

Figure 266: Plot configuration

Table 53: Plot configuration elements

| Name | Description |
| --- | --- |
| Users | Plot active UEs across the entire system |
| Dropped users | Plot dropped UEs across the entire system |
| Connection lines | Plot active connections for all active UEs – this only shows if “UEs” are checked |
| TX stats | If system is downlink this toggles the display of the transmit power of each base-station. If system is uplink this toggles the display of the noise rise of each base-station as well as the total interference experienced by that base-station. Also the number of active UEs connected to each base-station is shown – regardless of link direction. |
| Antenna Pattern | Toggles a visual representation of the antenna pattern of the selected base-station. This is mostly interesting in tri-sector scenarios. The plot of the antenna pattern can be used to ensure that the correct sector is selected. |
| Cell centre | Toggles the display of base-station position within the cell. |
| External Interferers | Toggles the display of external interferers. This only has effect when CDMA is victim. |
| Cell ID# | Toggles the display of the internal SEAMCAT cell id next to the cell centre |
| Legend | Toggles the display of the legend in the top left part of the main plot area |

The main part of the CDMA network Details window is used by the main plot. The plot shows a visual representation of the last snapshot and should be used to validate that the input parameters actually corresponds to the system that should be simulated. The plot allows for heavy user interaction. A very basic example is shown Figure 267 below.

Figure 267: Main plot of CDMA network

### Summary of event #n

The “Summary of event#n” panel provide a few metrics on the number of users simulated Figure 268.

Figure 268: Event summary of cellular setup

Table 54: Snapshot summary description

| Name | Description |
| --- | --- |
| Total Users | The total number of UEs in the system (number of BS x UEs per cell) for CDMA (active and dropped) and number of active users for OFDMA. |
| Connected Users (active/ inactive) | Number of UEs connected. In CDMA and OFDMA it is assumed that all users are active. |
| Dropped Users | Number of Ues dropped after power balancing. If CDMA is victim it is the number of Ues dropped after introduction of interference. Note that uplink CDMA drops Ues based on the average noise rise in the system – so it is possible for a single interferer to “shut-down” the entire system (causing all Ues to be dropped). In OFDMA, the purpose is to look at the bitrate/throughput loss and not to look at the number of dropped users, but it is possible to drop users depending on the input set-up. |
| Selected system | If more than one CDMA network is available in the scenario, a dropdown is used to select the system. You can choose to visualise either the victim or the interfering system which has been simulated. When you select the victim system, it is also possible to see the position of the interferer. |
| Selected sector | When tri-sector layout is used, it allows you to see the antenna pattern for the selected sector. Note that the antenna pattern should also be selected in the plot configuration (for visualisation purpose only). |

### Inspect selected element

Detail

When an element of the main plot is selected, its detailed information is shown in a table. Further detailed are presented in Annex ‎A15.1 with respect to CDMA network, Detailed of voice users, cell.

Figure 269: Detailed information of each element of the main plot

Event Vectors

You are able to investigate some intermediary output vectors resulting from the cellular simulation.

For the position information (x,y) of the active UE details, all coordinates are always shown in the SEAMCAT coordinate system which by definition either the VLT or the victim reference cell in (0,0). Therefore, the position of the elements of an interfering CDMA or OFDMA system is based on the victim reference cell and not its "internal" reference cell.

Table 55: CDMA UL (a) and DL (b) snapshot vector for the last event

Table 56: Output vector results for CDMA UL

| Name | Description |
| --- | --- |
| Calculated pathloss | Raw parthloss for all the active links (i.e. active UE to ist serving BS) |
| Distance to first BS | Distance from UE to its serving BS (first refer to cases where tri-sector is active) |
| Effective Pathloss to Ext. interferer (all victims, all interefers) | Effective pathloss between all the victims and all the external interferers |
| Effective pathloss, active links | Effective pathloss between all the victims and there respective serving BSs. Results of the below equation for all the active links |
| External interference, all cells | Sum of the iRSSblocking and iRSSunwanted at each victim cell |
| Geometry | Evaluate the Geometry for the active users for the all network |
| Inter System Interference | Evaluate the interference from your own network |
| Noise rise over the noise floor | Evaluate the Noise rise over the noise floor |
| Number of dropped users per BS | Evaluate the Number of dropped users per BS |
| Rx power, active links | Received power at the victim serving BS (UL) or active UE (DL) from its own system (used for investigating the inter-system interference from other cells) |
| Size of active list | Size of active list |
| Tx power external interferers | Tx power from the interferer |
| Tx power, active users | Tx power from its own system |

Table 57:. Output vector results for CDMA DL (the rest of the vectors are like for the UL)

| Name | Description |
| --- | --- |
| External interference, active users (all cells) | Evaluate the external interference on all the active users for the whole network |
| External interference, active users (ref cell) | Evaluate the external interference on all the active users for the reference cell only |
| External interference, dropped users (all cells) | Evaluate the external interference on all the dropped users for the whole network |
| External interference, dropped users (ref cell) | Evaluate the external interference on all the dropped users for the reference cell only |

Table 58: OFDMA UL (a) and DL (b) snapshot vector for the last event

- (b)

Table 59: Output vector results for OFDMA UL

| Name | Description |
| --- | --- |
| Calculated pathloss | Raw parthloss for all the active links (i.e. active UE to ist serving BS) |
| Distance to first BS | Distance from UE to its serving BS (first refer to cases where tri-sector is active) |
| Effective Pathloss to Ext. interferer (all victims, all interefers) | Effective pathloss between all the victims and all the external interferers |
| Effective pathloss, active links | Effective pathloss between all the victims and there respective serving BSs. Results of the below equation for all the active links |
| External interference –blocking (all victims – all interferers) | iRSSblocking for each of the victim UE interferered by each interferer |
| External interference –unwanted (all victims – all interferers) | iRSSunwanted for each of the victim UE interferered by each interferer |
| External interference –blocking (all victims) | Aggregate external interference iRSSblocking for each of the victim UE. Sum over all the interferers |
| External interference –unwanted (all victims) | Aggregate external interference iRSSunwanted for each of the victim UE. Sum over all the interferers |
| External interference, all cells | Sum of the iRSSblocking and iRSSunwanted at each victim cell |
| Frequency mobiles | Vector of the frequency of the UE (in UL) for each active link |
| Inter System Interference | Evaluate the interference from your own network |
| Rx power, active links | Received power at the victim serving BS (UL) or active UE (DL) from its own system (used for investigating the inter-system interference from other cells) |
| Size of active list | Size of active list |
| Tx power external interferers | Tx power from the interferer |
| Tx power, active users | Tx power from its own system |

Table 60: Output vector results for OFDMA DL (the rest of the vectors are like for the UL)

| Name | Description |
| --- | --- |
| Achieved SINR, active users (ref cell) | Achieved SINR in the ref cell only |
| Achieved SINR, active users (all cells) | Achieved SINR for the all system |
| Achieved bitrate, active users (all cells) | Achieved bit rate for the all system |

Plot settings

The plotting options control how the system is shown in the main plot area and how you select elements from the system. This potting option can be seen on the overview page Figure 270.

Figure 270: Plot settings

You can zoom in and out by using either the mouse wheel or the Zoom Factor slider. When clicking on a displayed item SEAMCAT tries to match the coordinates of the click to a cellular element – selecting the first matched item.

When SEAMCAT tries to match the click to an element it allows for a certain amount of uncertainty when matching the coordinates. This uncertainty is also called click radius to illustrate the effect of the actual click point being in the centre of a circle used to search for CDMA elements. You can adjust the “click radius” and in combination with the zoom this allows for all elements to be selected using the algorithm supplied above.

It is often the case that an element different than desired or no element at all is selected when clicking the plot. This problem is resolved by zooming in and possibly changing the click radius.

## Interference calculations

### Introduction

The Interference Calculation Engine (ICE) is that part of the SEAMCAT architecture which calculates for generic victim receiver the probability of being impacted by the sum of the simulated interference power. The calculated result is commonly called "Interference probability" or " probability of interference", in fact it is the probability of exceeding the limit of one of the interference criteria given for the victim receiver.

Regarding the definition of the Radio Regulations (RR) Article 1.166

The effect of unwanted energy due to one or a combination of emissions, radiations, or inductions upon reception in a radiocommunication system, manifested by any performance degradation, misinterpretation, or loss of information which could be extracted in the absence of such unwanted energy.

Administrations may have to distinguish between permissible interference (RR 1.167) and accepted interference (RR 1.168).

Details of the interference calculation algorithm are given in ‎ANNEX 2:.

### Interference calculation GUI

When the simulation is finished, the dRSS and the iRSS vectors are stored. You may proceed to use the facilities of the Interference Calculation Engine (ICE) in order to evaluate the probability of interference for the simulated scenario.

The probability of interference is calculated by the ICE with the following choice of input parameters:

Calculation mode: compatibility or translation;

Which type of interference signal is considered for calculation: unwanted, blocking, intermodulation, overloading or their combination;

Interference criterion: C/I, C/(N+I), (N+I)/N or I/N.

If more than one interference calculation was done (i.e. with different combination of interference criterion), you may scroll through all of them by using the Previous / Next buttons.

Interference calculation results:

single interference probability value (compatiblity mode);

probability as function of the translation parameter (translation mode).

When the compatiblity mode is chosen, a single-figure estimate of the probability of interference is calculated;

When the Translation mode is chosen, you may calculate and display as chart the probability of interference as function of one of the following input parameters:

Output power of Interfering transmitter;

Blocking response level of Victim receiver;

Intermodulation response level of Victim receiver.

You are able to save the results of the translation mode using the “save translation button”.

Figure 271: Interference calculation panel

Table 61: Parameters to calculate the probability of interference

| ID | Description | Comments |
| --- | --- | --- |
| 1 | Calculation mode/ Compatibility: | Compatibility: Gives the probability of being interfered by the Blocking interference and/or by the Unwanted interference and/or by intermodulation interference. The result is a probability of interference. |
| 2 | Calculation mode/ Translations: | In this case all the following parameters should be independent from frequencies: Receiver blocking response mask, Receiver intermodulation rejection mask, power distribution of interfering transmitter, Unwanted emission floor mask. Calculation of the probability of interference as a function of the reference parameters (Power supplied by the It for the unwanted, Blocking response level of the Vr for the Blocking, And intermodulation rejection level for the Vr). These parameters are varying on user-defined definition domain defined by the number of points where the software has to calculate the probability. The result is a graph. |
| 3 | Signal type | Choose the interference studied: Unwanted and/or Blocking and/or Intermodulation and Overloading in case simulated. |
| 4 | Interference criterion | Choose between C/I, C/(N+I), (N+I)/N, I/N) |
| 5 | Events | Total number of simulated events, in case Translation multiplied by the number of points |
| 6 | Events (dRSS > sensitivity) | It represents the number of events taken into account for the calculation. The accuracy of the calculated results relates to this ratio, i.e. the less this number the higher the inaccuracy. |
| 7 | Calculation control | Delete a result, and see the last results |
| 8 | Translation parameters: If translation was chosen | Number of points between the min and max, where the software will calculate the probability. |
| 9 | Result / Compatibility | Probability of interference: 1 - always interfered, 0 - never interfered |
| 10 | Result / Translation | Gives the graph, showing the resulting probability of interference vs. the selected values of translation parameter. The average of the graph depends of the number of points, but the higher the number is, the longer the calculations are. |
| 11 | Save translation button | You are able to save the results of the translation mode |

### Interference Calculation Engine Control

It allows to the calculation of the probability of interference for several ICE configurations (i.e. different signal types, interference criteria, etc..) for the same simulation. Figure 272 presents how the control box is used.

Figure 272: Use of the Interference Calculation Engine (ICE)

When the translation mode is activated, the overloading feature is deactivated as shown in Figure 273.

Figure 273: When translation is activated, the overloading feature is de-activated

# Library of scenario elements

## Library overview

SEAMCAT library is a useful tool for creating and storing various elements that are part of a wireless system so that they can be re used at a later point of time. The SEAMCAT library contains the following elements:

Systems and System Plugins (SP)

Spectrum Emission Masks (SEM)

Receiver Blocking Masks (RBM)

Receivers

Transmitters

CDMA Link Level Data

Antenna Gain Plugins (AGP)

Coverage Radius Plugins (CRP)

Propagation Model Plugins (PMP)

Event Processing Plugins (EPP)

Note that technical library management functions are available regardless of whether a simulation workspace has been loaded or not. The library can be saved in a separate file too so that you can also share them.

Figure 274: Overview of the SEAMCAT library elements

## Import / export a library element in/from a workspace

Modification of existing (default) library elements as well as creation of new elements can be managed through the relevant options of Library menu. Once stored, the library elements may be then easily inserted into scenario by selecting a particular library element in the Definition field for relevant transceiver in either victim or interfering links, as illustrated in the following picture:

Figure 275: Import / export a library element in/from a workspace

Figure 276: Import / export a library to save or share your work

It is possible to save customised libraries on locally by “exporting” them. The libraries are saved with the extension “.sli”. Any library item can be selected (Figure 277), and the path to the folder where these items will be saved can also be specified (Figure 278).

Figure 277: Export library elements locally

Figure 278: Select where the library should be saved.

Library items can be shared and uploaded into SEAMCAT by using the “import” button.

Figure 279: Import .sli files

Once a library element is imported () into a given scenario, there remains no  link between the original library element and its copy in the workspace. That is, if at some later stage a library element is modified or suppressed  it will not produce any change in replicas of that library element previously pasted into existing workspaces. Upon clicking on the import button, you will have to chose from a drop down list the element that you want as shown below:

A library can be created from a workspace and exported ()to a local library folder. SEAMCAT will automatically place the new element in the right category of the library.

## Edit and share library element

When opening a library item from the workspace menu,  the following interface will appear (as an example, the antenna interface is shown in the figure below).

Figure 280: Example of the antenna interface library

All the library elements follow the same command feature . They allow:

Creating a new antenna, click on

Deleting an existing antenna, select it in the list and click on

Duplicating an existing antenna, select it in the list and click on . Reference of the new antenna is derived from the reference of the selected one by incrementing the integer suffix of that reference, if any, or adding character '1' to the end of the original name.

Reloading the pre-defined libraries (it will delete any entries that you may have made), click ;

Getting on-line help by clicking on

When adding a new elements, enter the name (unique identifier), description of the elements as well as their technical parameters.

Note: Tooltips are available on the buttons to remember their meaning .

Once a broad library of various elements are populated, it might be useful to exchange it with other team members working on studies involving similar wireless systems.

To share the created library, an export file may be created by using command Export Library in menu section File. When selecting this menu option, you will be offered a choice of defining the file name and the entire current library and a library copy will be unloaded to that file.

Figure 281: General import/Export library

The SEAMCAT library exchange files are saved using XML format with a unique file extension .sli (for Seamcat Library). Such library export file may then be distributed within a group of users and may be uploaded by any user via the command Import Library on the File menu.

In case of collision of an imported and an existing library element that have the same name (e.g, there will always be a collision between the DEFAULT library elements saved in an exchange file and those existing in target SEAMCAT copy), a choice will be given to either to overwrite the existing elements or to import new library items after renaming them.

## Library elements

### System elements

To create or modify a library element containing parameters of a particular system, i.e. generic, cellular network like CDMA or OFDMA, select the system library lements command from the Library pull-down menu or directly CTRL+SHIFT+S.

A list of existing pre-defined systems will be selectable as shown in Figure 282 for a generic system and Figure 283 for cellular systems (e.g. CDMA UL network).

Figure 282: Example of generic System elements

Figure 283: Example of CDMA UL System elements

When adding a new system, enter the name (unique identifier) and a description.

Note: It is also possible to create a system from the workspace and export it to a local library.

### Spectrum emission mask elements

To create or modify a library element containing parameters of a particular spectrum mask, select the Spectrum emission masks command from the Library pull-down menu or directly with CTRL+SHIFT+S.

A list of existing pre-defined spectrum emission mask will be selectable as shown in the figure below. This is an example of a LTE UE (Table 6.6.3.1-2 of 3GPP TS 36.101) where the limit for the spurious emission domain (outside ± 12.5 MHz) applies to frequencies above 1000 MHz:

Figure 284: Spectrum emission mask library elements

When adding a new spectrum emission mask, enter the name (unique identifier) and description of its technical parameters.

Note 1: It is possible to quickly scroll up and down and see the differences in the masks.

Note 2: It is possible to create a mask from the workspace and export it to your library environment

Figure 285: Example on how to import/export to/from the workspace/library for 
the spectrum emission mask

### Receiver blocking mask elements

To create or modify a library element containing parameters of a particular blocking mask, select the Receiver blocking masks command from the Library pull-down menu or directly with CTRL+SHIFT+B.

A list of existing pre-defined receiver blocking mask will be selectable as shown in the figure below:

Figure 286: Receiver blocking mask library elements

When adding a new receiver blocking mask, enter the name (unique identifier) and description of its technical parameters .

Note 1: It is possible to quickly scroll up and down and see the differences in the blocking masks.

Note 2: It is possible to create a blocking mask from the workspace and export it to your library environment

Figure 287: Example on how to import/export to/from the workspace/library for 
the receiver blocking mask

### Receiver elements

To create or modify a library receiver element, select the Receivers command from the Library pull-down menu or directly with CTRL+SHIFT+R.

This activates the Receiver Library window which displays a list of existing receiver elements in the library. Please note that it is the same interface as in the workspace, so that it is easy to fill. If no receiver has been previously created , the list will contain only the DEFAULT receiver elements:

Figure 288: Receiver library elements

The parameters of receiver are grouped in 5 different panels:

Receiver identification: to define receiver name (unique identifier), description;

Antenna pointing: It contains all information relative to the antenna other than the radiation pattern (Please read here more about antenna azimuth/elevation settings);

Antenna patterns identification: to define a specific antenna to be associated with that particular receiver. Note that it is possible to select one of the existing antenna library elements or define completely new antenna parameters to be associated only with the given receiver;

Reception characteristics: to define the basic receiver features, such as bandwidth, sensitivity, blocking (selectivity) function, etc.;

Interference criterion: to define the required minimum C/I and/or other interference criterion, as appropriate.

Note 1: It is possible to quickly scroll up and down and see the differences in the receivers.

Note 2: It is possible to create a receiver from the workspace and export it to your library environment

Figure 289: Example on how to import/export to/from the workspace/library for the receiver elements

### Transmitter elements

To create or modify a library transmitter element, select the Transmitters command from the Library pull-down menuor directly with CTRL+SHIFT+T.

This activates the Transmitter Library window which displays a list of existing transmitter elements in the library. Please note that it is the same interface as in the workspace so that it is easy to fill. If you no transmitter has been previously createdthe list will contain only the DEFAULT transmitter elements:

Figure 290: Transmitter library elements

The parameters of transmitter are grouped in 4 different panels:

Transmitter identification: to define name (unique identifier), description;

Antenna pointing: It contains all information relative to the antenna other than the radiation pattern;

Antenna patterns identification: to define a specific antenna to be associated with that particular receiver. Note that you can select one of the existing antenna library elements or define completely new antenna parameters to be associated only with the given receiver;

Emission characteristics: to define the basic transmission features, such as power, spectrum emission mask function, etc..

Note 1: It is possible to quickly scroll up and down and see the differences in the transmitters.

Note 2: It is possible to create a transmitter from the workspace and export it to your library environment

Figure 291: Example on how to import/export to/from the workspace/library for the transmitter elements

### CDMA Link level data

To create or modify CDMA Link level data set, select the CDMA Link level data command from the Library pull-down menu or directly CTRL+SHIFT+L. This activates the CDMA Link level data window which displays a list of existing CDMA Link level data:

Figure 292: CDMA Link level data library elements

Regarding the library sets of CDMA Link level data, the default sets distributed within SEAMCAT library are understood to be sufficient to cover typical CDMA systems expected to be deployed in Europe and their respective frequency bands. any additional link level data may be defined to cover other types of CDMA systems. However, it is recommended that only users with advanced knowledge of CDMA systems develop the new CDMA Link level data for SEAMCAT simulations as several considerations should be kept in mind  before creating new Link level data:

The SEAMCAT CDMA Link level data have different formats for uplink and downlink sections of CDMA link. In particular, the Link level data for uplink provides Eb / No requirements, whereas downlink data define the Ec / Ior requirements;

Check carefully that the newly created CDMA link level data is appropriate for the SEAMCAT's CDMA simulation methodology for the respective section of the link (uplink/downlink).

From the worskpace, select the relevant library item as shown below:

Figure 293: Example on how to import/export to/from the workspace/library for 
the CDMA Link Level data library elements

When adding or editing a data set, a table with data points is displayed and new data points entered will appear on the graph (for Downlink sets). For Downlink sets, every data point must as a minimum contain a geometry value, otherwise it is invalid and it will not be possible to leave the editing dialog.

Basic information about the link-level data set can be viewed and amended in by clicking Basics button in the sidebar of the edit window. This will display a new dialog.

Note: When Ok is clicked in the basics dialog any information changed in this dialog will be saved immediately even if Cancel is subsequently pressed in the overlying data point window.

Figure 294: Example on how to edit a CDMA Link Level data element

## Plugins Library

A detailed description on plugin development is available in section ‎14.

### Antenna plugin elements

To create or modify a library element containing parameters of a particular antenna, select the Antennas command from the Library pull-down menu or directly with CTRL+SHIFT+A.

Since the antenna is implemented as a plugin, it is possible to include a new antenna plugin using the .jar installer and to add it to the list of existing pre-defined antennas. Any of the antenna elements is then selectable as shown in the picture below:

Figure 295: Example of the antenna library

When adding a new antenna, enter the name (unique identifier) and description of the antenna as well as its technical parameters - peak gain and one or more radiation patterns, as necessary. See Annex ‎A11.6 for further details.

Note 1: It is possible to quickly see the antenna pattern with mouse-over capability on top of the pattern button

.

Figure 296: Quick over mouse preview of the pattern

Note 2: If none of the radiation patterns is checked, SEAMCAT will assume this being an omni-directional antenna.

Note 3: It is possible to create an antenna from the workspace and export it to your library environment.

### Coverage radius plugin elements

To create or modify a coverage radius element, select the Coverage radius library command from the Library pull-down menu or directly CTRL+SHIFT+C. This activates the Coverage radius library window which displays a list of existing Coverage radius library:

Figure 297: Example of the coverage radius plugin library

### Propagation model plugin

To create or modify a library element containing parameters of a particular propagation model, select the Propagation model command from the Library pull-down menu or directly with CTRL+SHIFT+P.

Since the propagation models are implemented as plugin, it is possible to include a new propagation model using the .jar installer and to add it to the list of existing pre-defined models. Any of the propagation models is then selectable as shown in the picture below:

Figure 298: Example of the propagation model plugin library

More detailed information on specific propagation models is available in ‎ANNEX 17:.

### Event Processing Plugin

To create or modify a library element containing parameters of a particular event processing plugin, select the event processing plugin command from the Library pull-down menu or directly with CTRL+SHIFT+E.  A list of available built-in event processing plugin is available in Figure 240.

# Plugin development

## SEAMCAT source code

Programming is all about detail so when looking at even simple code it can be bloated with irrelevant stuff, logging, error handling, comments.

Often, nitty gritty details hides the true purpose of a piece of code. So being able to navigate through souce code you need to understand what highlevel and lowlevel abstractions are made when designing and implementing the software.

In SEAMCAT we have abstractions like Systems, Simulations, Antennas and plugins but we need to have a good mental picture of how they relate to each other.

With SEAMCAT you can setup victim, setup a number of interferers, define interference properties, control the number of snapshots, simulate and inspect the collected results after the simulation. This is nothing new to a seasoned SEAMCAT user but that does not really help when the source code is to be navigated.

## Background and Terminology

The following terminology is used throughout this document.

System: A radio system of SEAMCAT. Currently SEAMCAT supports two kinds of systems, i.e. Generic System and Cellular System (with four sub types available: CDMA UpLink, CDMA DownLink, OFDMA UpLink, OFDMA DownLink). However different, all systems has common traits such as frequency, receiver characteristics, transmitter characteristics, positioning, etc.

Victim system: referred to as 'The victim' as there is only one victim for a simulation.

Interfering system: a system that interferes with the victim system.

Interference link: this is a constellation of a victim system and an interfering system. But it also includes configuration outside of systems, i.e. relative positioning of the systems, propagation model, specifics for system types such as user defined DRSS for generic system.

Scenario: a complete configuration of a victim system and a list of interference links (and thereby a list of interfering systems).

Event (or snapshot): a random trial of all distributions in a scenario.

EventResult: from an event the system simulations apply calculations and produce a number of results, which is what we call the event result.

Simulation result: the total result of combining all produced EventResults.

### Understanding abstraction level

We can conceptually view SEAMCAT as a layered cake of abstractions. Knowing these different layers helps a lot in understanding how to navigate the source and how to understand SEAMCAT.

Figure 299: Abstraction level in SEAMCAT

So in a very high level of abstraction, the Monte Carlo simulation runs through the specified number of randomized events, which each produces an EventResult. Lastly it combines all these EventResults into a simulation result which can be inspected in SEAMCAT.

### Simulation & Engine

We start at the highest abstraction: Simulation & Engine. If we forget about the specific system types and multiple interferers we get the conceptualized view of a scenario. A victim system can be interfered by an interfering system.

On a very high level of abstraction an interference simulation consists of

- victim.simulate, i.e. Configure the victim system;

- interferer.simulate, i.e. Configure the interference system;

- and collect the interference results based on the settings of the two systems.

Figure 300: Abstraction level of an interference simulation

A closer look at the 3 steps: get the list of active receivers from the victim. Remember do not think of a specific system, if you for instance think of a Generic system you might say that this is never the case because there can be only one receiver. But there being only one is a special case of being many. And for Cellular systems there can definitely be many receivers (UE or BS). So the highlevel view of the algorithm is that there is a list of active receivers in the victim system.

Now we do the same (but opposite) for the interfering system: get the list of active transmitters. Again the list of active transmitters is the most general or highlevel notion of the algorithm which describes all possible cases (0, 1, or many).

Finally we combine all victim receivers with interfering transmitters and base the calculation of the interference with this as shown in Figure 301.

Figure 301: Abstraction level of IT->VR links

These IT->VR links are really what we are looking for and bases the interference calculations upon.

Given this the following results are possible:

0 – For some reason there was either 0 active receivers or 0 active transmitters in the victim, interfering system (respectively). Therefore we have no IT->VR links;

1 – There is precisely one active receiver and one active transmitter. This is most commonly the case for ”generic-generic” simulations;

Many – There are many IT->VR links either because the systems are generic with many active transmitters or cellular (or maybe it is a future system not invented yet).

We have certainly described a lot about how SEAMCAT works but at no point it was needed to talk about any systems specifics. Of course during a simulation the choise of system will very much affect how it is done, but conceptualized like this there is no need.

On the highest level of abstraction we have the Simulation & Engine. “Simulation” holds the scenario setup and various hook for starting and completing a snapshot while “Engine” runs a simulation by controlling the steps in the simulation

Figure 302: Simulation and Engine abstract representation

The figure below from the engine shows almost exactly the steps described above.

The real implementation is in InterferenceSimulationEngine.java take a look at it. Remind yourself that programming is all about details: there are so many other things going on than the actual loop.

Figure 303: Engine code

The engine asks the simulation for the victim system simulation. So the simulation controls which victim simulation instance is handed to the engine, and the engine then uses that instance for doing its calculation. Therefore the engine is decoupled from the knowledge of which specific victim simulation is running (e.g. Cellular victim).

### Packages

SEAMCAT has different modules. These are used to group certain classes of the code and handle dependencies between them. This is similar to the layers of abstraction where the engine only knows general system types but not specifics. Modules can be used to enforce such dependencies.

There are two main modules: model and application.

Model holds all the abstract definitions that composes a System – Transmitter, Receiver, Antenna, Propagation model, Distribution, etc.;

Application implements the SEAMCAT UI, simulations, Propagation Models, and basically ties everything together. We will descuss this structure in more detail later.

Figure 304: Modules of SEAMCAT

Inside a module we organize the software files in packages. It is similar to a folder and like a folder it contains the individual source code files. The names of the packages is helpful when nagivating the source code.

The packages of the model module.

The names can be helpful when navigating the source code:

Cellular and generic contains the definitions for the cellular and generic systems;

Plugin holds the plugin definition for the mentioned types (Eventprocessing, Coverageradius, propagation etc…);

Distribution, functions, and mathematics holds the distributions and other helper functions;

Simulation and types holds the simulation results and general type definitions (Antenna, Receiver, RadioSystem, etc).

The packages of Application is more messy than the model. So here it is harder to know exactly where to go. The packages to know here is: scenario and simulation.

Scenario contains the implementation of some of the types defined in Model;

Similarly with simulation: here are the specific simulations e.g. CDMADownlinkVictimSimulation.

It is the very nature of code to change, but with a good design and understanding of the abstractions it should be easy to identify part that will change the least. Most of the dynamic parts of the code can be examined with runtime inspection by software development tools. Mechanism vs Policy is a software principle based on the fact that mechanism change infrequently whereas policies change all the time.

Mechanism is here the engine of SEAMCAT and policies are the plugins. The parts that changes behaviour are left open for developers.

| Figure 305: Packages of Model | Figure 306: Packages of Application |
| --- | --- |

### System & Results

We move a step down in the abstraction level and look at system types. So far we have avoided the specific knowledge of the system type for the engine and simulation. This gives us power. Since the engine and simulation has been implemented without knowing the specificity of a system to simulate, we can now extend the system with new types. As long as the general system interface remains the same, adding new system types would not affect the engine or simulation at all!

The system has agnostic properties. With agnostic we mean that we are talking about a system in general – no mention of any specifics. Remember that the Engine is system agnostic – it can calculate the interference regardless of the system type!

So what is a system? Basically a System contains Transmitter(s) and Receiver(s) and link(s) between these. Whenever we have a link a Propagation Model applies i.e. the connection from the transmitter to the receiver will suffer a loss how big this loss will be is determined by the Propagation Model.

Figure 307 is the Model-module code for a RadioSystem – the most abstracted version of a system. These definitions would therefore apply to any specific system but in the abstract definition there is no mention of Generic or Cellular specific properties. So generally speaking a RadioSystem has a frequency which is a distribution. It has a Receiver and a Transmitter. And importantly it defines a Link between the transmitter and receiver.

Figure 307: Model-module code for a RadioSystem

In SEAMCAT we use the workspace to define our interference setup. In code terms we call the workspace a Scenario. The Scenario is how the engine sees the workspace configuration. It is high on the abstraction scale because the engine cannot know anything about the details (and does not need to). It is an abstract immutable representation of the workspace. It is found in Scenario.java and is used by the Engine.

Running a SEAMCAT simulation produces results. The results fall into two categories 1) Simulation results and 2) Intermediate results

Simulation results:

SimulationResult is the result of running a SEAMCAT simulation. It holds all results that will be saved/loaded if the workspace is saved/loaded with results. It is represented by class: SimulationResult.java.

SEAMCAT shows all results in the 'Simulation Results' tab. It basically lists all the groups available in the SimulationResult class as shown in Figure 308.

The simulation result consists of several SimulationResultGroups. Each group can be seen on the tab 'Simulation Results'. A group is a set of results that logically belongs together. All simulation results are shown after the simulation has completed and can be inspected/saved/loaded in SEAMCAT. The group result holds the relevant results for this group. The result types can be of many different types: double, vector, xy-plot, etc. We will get back to these types in the EPP section. Group results can be found in the source file SimulationResultGroup.java.

Intermediate results - EventResult

A simulation is a series of randomized settings based on the scenario. In SEAMCAT it is called an EventResult. The EventResult is a placeholder during the simulation. It contains all information to be used by a simulation, e.g. positions of the elements in the system. Values in EventResult are used to produce the final SimulationResult.

The EventResult consists of many LinkResults. Every connection between a Tx and Rx is represented with a LinkResult. It could be inside the victim system, or it could be an interference link between the victim system and one of the interfering systems: all represented by a LinkResult. It is by far the most important result is the LinkResult. It holds the intermediate results of a particular link (e.g.Tx,Rrx position). It can be seen in the file LinkResult.java.

Figure 308: Simulation results

Accessing data from EventResults

Results produced from a single snapshot of the simulation are stored in an object called EventResult. An EventResult contains both general results and results that are specific to the actual scenario. It is important to understand the basic structure of the EventResult and the general results in order to be able to develop own EPPs.

The most important result produced by the simulations is the LinkResult. A LinkResult represents an actual connection between a receiver and a transmitter. This is completely general so it could be a connection from a mobile station of a cellular network to a receiver of a generic system - we don't know this by looking at the LinkResult but if the scenario was setup with a CDMA UpLink victim and generic system as interferer the described LinkResult could occur.

A LinkResult contains the actual values of this connection, i.e. position of the receiver, position of the transmitter, the distance between them, the frequency, the path loss and effective path loss. By inspecting the LinkResult interface other similar values are documented.

Roughly said, the EventResult contains a collection of LinkResults. But as pointed out, a LinkResult does not tell you where in the scenario it is located. Therefore an EventResult provides a structure that helps the plugin developer navigate the LinkResult collection.

A LinkResult will occur in the following places:

Inside a system: Is it for instance a generic system it will always be from the transmitter to the receiver. Is it a cellular system it depends on the link direction (up or down). In downlink cellular system a link will be from a base station to a mobile station - notice that in this case there are many more possibilities than in the generic case - we could have many base stations and many mobile stations.

Between two systems: When such a LinkResult occurs it is attached with much more information than just the LinkResult. This is because a link between two systems is called an interference link and is exactly what SEAMCAT is about. So a LinkResult between two systems is a special class (extending the LinkResult) called InterferenceLinkResult. It always goes from a transmitter in the interfering system to a receiver in the victim system.

Between two systems (sensing): This is a special link that can occur if the victim system supports Cognitive Radio. In such a case a LinkResult can occur from a interfering system transmitter to a victim system transmitter.

As we now know what different LinkResult that can occur in a simulation, we are suited to understand the structure of an EventResult. The most important method in the class EventResult is the one with the signature: InterferenceLinkResults getInterferenceLinkResult(InterferenceLink link);

Let's break it down in order to fully understand what it does:

the input argument InterferenceLink link: as stated earlier a scenario has a single victim system and a configured list of InterferenceLink(s). Each InterferenceLink identifies a specific interfering system. So the input argument specifies what InterferingLink we are interested in seeing the results from.

return type InterferenceLinkResults: this is a container of many InterferingLinkResults. We will dig down into the details of this class soon.

So given a specific InterferenceLink from the scenario it gives you all the LinkResults (i.e. actual connections) representing this InterferenceLink so going from the interfering system (to a transmitter) of the InterferenceLink and to the victim system (to a receiver).

To summarize, given a scenario of for instance a generic victim system and three configured interference links, each interference link having CDMA UpLink as interference system. One particular EventResult can give you three different InterferingLinkResults, one for each of the interference links in the scenario. To get the InterferenceLinkResults you are interested in you have to use the corresponding InterferenceLink value from the scenario.

The class InterferenceLinkResults

The class InterferenceLinkResults gives the plugin developer the possibility to navigate all LinkResults produced by this sceario's InterferenceLink. It does to by its three methods listed below.

getVictimSystemLinks():  returns a list of LinkResults from inside the victim system. If the victim system is a generic system, it will be a list of one element typically. If the system is of a cellular type, there will most likely be many LinkResults (the default setting for a cellular system is two tier, tri-sector, suggesting 57 base stations).

getInterferingSystemLinks(): similar to the getVictimSystemLinks() for only for the interfering system. Notice that the interfering system is now singular, i.e. the interfering system, because we are inside a specific InterferingLink, which identifies a single interfering system.

getInterferenceLinkResults(): this returns a list of InterferenceLinkResult elements. Notice that the size of this list is always the product of the size of the other two: if getVictimSystemLinks contains n elements and getInterferingSystemLinks contains m elements, getInterferingLinkResults will contain n*m elements.

### Debug your code

A way to understand the code is to simply read through it. But this is very hard indeed and it is recommended to inspect the code when it is running. That way, variables can be inspected and execution flow is directly visible.

An example of a generic scenario with a breakpoint in GenericVictimSimulation.simulate is shown in Figure 309. In Intellij you set breakpoints by clicking in the margin of the file. This will cause a red dot to appear and when running in debug mode the code will freeze when that point is reached.

Tip: to find classes (i.e. to get the GenericVictimSystemSimulation.java) do Ctrl+N.

Figure 309: Code inspection - Setting breakpoints

SEAMCAT is launched in debug mode from Intellij. This is done by opening Seamcat.java and right-clicking on the class name. In the menu that appears choose ‘Debug ‘Seamcat.main()’.

Figure 310: Launch SEAMCAT in debug mode

SEAMCAT will now open and a scenario can be setup. When a scenario has been set up as wanted, the simulation is run by pressing the simulation start button.

Once the execution of the code reaches the point in the code where the breakpoint is set the execution will pause and the breakpoint will light up in Intellij (see Figure 311). Now the developer controls the execution flow.

The button panel below has different control buttons to handle the code flow: step over will go to the next line of code. Step into will go into the method on the line. This has to be tried a number of times before one masters this!

Figure 311: Catching breakpoint

The callstack shows how we got here in the code (Figure 312). This is extremely valuable for inspecting who calls who during the simulation.  Here we see that we are coming from line 114 in GenericVictimSystemSimulation.java. This only apply during an active breakpoint i.e. When the code it temporary suspended.

Figure 312: Callstack

The evaluate can be used to evaluate expression in the current code state. Here we take one of the variables eventResult and evaluates the expression eventResult.getDRSSResult(). This evaluates to the double value 0.0. This only applies when you have an active breakpoint.

Figure 313: Expression evaluation

## Commonalities to puglins

We are now moving further down in the abstraction layers of Figure 299. Plugins (e.g. propagation model) are defined in the model module.  So the application SEAMCAT uses this definition and uses the plugins in the simulation. Plugin developers can write plugins that are then used by SEAMCAT.

Figure 314: Modules of SEAMCAT

Plugins have been designed so that they follow common rules.

### @Config – Annotation of Input interface

The input parameters to be used by plugins are defined through the interface Input{}. Inside this interface the different parameters are configured through the annotation @Config.

Figure 315: Example of an input interface and @Config

Figure 316: display of the example

Each of these annotations is described in detail below:

order
defines the position on the input panel and must be therefore unique.

name
to show the user which the parameter is intended for

general
it has to be noted that without specifying a default vale, as for instance above for the propagation model, the default values of the corresponding types are applied. which are e.g. for

- double:	0.0

distribution: constant 0.0

function: constant 0.0

propagation model: Extended Hata

antenna: Antenna Peak Gain;

Factory
allows you to create your own default setting for distributions, functions, antennas and propagation models. The implemented DataExporter allows you to the create tabular data sets and save these onto you hard disk.

It has to be noted that String selections like shown below have to be implemented as ‘enum’:

- Figure 317: Selection parameter

### Consistency check - consistencyCheck()

The description of the plugins (e.g. PMP, EPP etc..) in the GUI panel informs you about its scope of application and this should be accordingly considered when you define your scenario.

In order to prevent the plugins from reporting physically wrong results the validity of actual parameter values are checked during the calculations (Figure 318). In case parameter values exceed the valid range, appropriate exceptions, e.g. RuntimeException, stops the plugin from doing further calculations.

Figure 318: RuntimeException checker in the calculation codes

This can be avoided if you are informed about such conflicts before starting the simulation by checking the consistency of your plugins like it is done for the rest of the SEAMCAT workspace. Therefore the method consistencyCheck() is available to all the plugins.

The basic principle on that is quite simple: the more extensive the checks the less the risk of running into problems during the simulation.

For this, each type of plugins provides the same method consistencyCheck() to perform the recommended checks:

Figure 319: Common method to perform checks

The referenced parameters are detailed as follow:

ConsistencyCheckContext: provides information about parameters relevant for the type of plugin.

List<Object>:
depending on the type of plugin it contains one or two objects which are either a Radio System or links which can be the system link, the interference link or the sensing link of generic systems. Corresponding details are described in the following sub-sections.

Input
the input parameters of the plugin

Validator:
provides the interface to the scenario check in returning the consistency check messages generated by the plugin

- Figure 320: ConsistencyCheckContext interface

Concerning the ConsistencyCheckContext it is important to be aware of the applicability of plugins to the scenario components:

an antenna plugin (AP) can be applied only to a radio system, i.e. a generic system or a cellular system, hence the Origin is set to SYSTEM

a coverage radius plugin (CRP) can be applied only to a generic system, , hence the Origin is either SYSTEM or in case the system is the interferer INTERFERENCE_LINK

a propagation model plugin (PMP) can be applied to the system link, the interference link and the sensing link, , hence the origin can be SYSTEM or INTERFERENCE_LINK, depending on to which link the PMP is applied

an event processing plugin (EPP) and its post processing user interface (PPUI) cannot be applied to any of the scenario components, hence its ConsistencyCheckContext is set to NULL, except the Scenario which is not available for other plugins

This default information can be replaced or extended if the plugin is nested, i.e. if the plugin is used as an input parameter of another plugin (annotation, see the previous section ‎14.3.1).

In case you apply nested plugins it is highly recommended to check the validity of these nested plugins inside the consistency check of your 'parent' plugin. The below example of an EPP which uses a nested PMP wants only to show the principle of checking input parameters:

- Figure 321: Example of checking the validity of a nested plugin

Example of a consistency check for a PMP

Let assume a fictive propagation model with the following restrictions

antenna heights less than 25 m

frequency range 45 MHz  250 MHz

distances less than 12 km

not applicable to cellular systems

An appropriate consistency check could look like this:

- Figure 322: Example of a consistency check for a PMP

You could place some checks related to sensing characteristics which might have impact on the model. In case it is an interference link, the transmitter is ILT, for sensing links it is the VLT. This part is used only if the model is assigned to a system link, i.e. to the victim and / or to the interferer

The result of applying this model to the system link and the interference link could look like this:

- Figure 323: Output of the PMP consistency check

Example of a consistency check for an AP

Let assume a fictive antenna model with the following restrictions:

antenna heights in the range 20 m … 60 m

frequency range 400 MHz … 1000 MHz

only for cellular systems

This could be checked for instance as shown below:

- Figure 324: Example of a consistency check for a AP

which would generate a consistency warning message like this:

- Figure 325: Output of the AP consistency check

Example of a consistency check for an EPP

Unlike the above plugin types the EPP plugin is not belong to a system or a link and therefore the ConsistencyCheckContext provides only the scenario. The functionality of the other reference parameters Scenario, Input and Validator remains as described above.

Let assume a fictive EPP with these restrictions:

applicable only if the victim system is generic

the coverage radius of the victim system shall not exceed 50 km

the simulation radius of the EPP (set as parameter) must not exceed the coverage radius of the victim system

Such a consistency check could be for instance implemented like this:

- Figure 326: Example of a consistency check for an EPP

and would generate the below warning message:

- Figure 327: Output of the EPP consistency check

Performing the consistency check on the workspace

The consistency check is in general called in order to check the complete scenario when you start the simulation.

In case there is any warning, a corresponding message pops up like this:

Figure 328: Examle of a message triggered at launch when a consistency check is triggered

Ignoring the warning message might cause crashing the simulation.

A second option to show the results of the consistency check is to manually activate it by hitting the Check consistency button as described in Section ‎2.10. This could prevent you from an unintended start of the simulation.

### Logger functionalities for plugins

SEAMCAT provides the option to run a simulation in debug mode. With this, you are informed about all relevant calculation steps and the values used, in a similar manner than for the built-in methods. It is therefore advisable to implement this functionality also onto the your plugin in order to make it more transparent to third party users.

The implementation is simply done with this, but requires the apache log4j-1.2x.jar on your development environment:

Its application could look like this (from the above example):

- Figure 329: Example of logging in your plugin’s code

It would generate the corresponding debug information as depicted in Figure 330. It is integrated automatically to the rest of the SEAMCAT code log file.

- Figure 330: Output results of the plugin logger

## Propagation Model Plugin (PMP)

The propagation model is essential because it is involved in every link. Instead of providing implementations of this it is exposed as a plugin. With a PMP you have the flexibility to define what input fields is needed. In the computation you have access to the user field values and you can access the link settings. Given these inputs your PMP then has to compute the propagation loss and return that.

The interface PropagationModelPlugin provides the information about the link components it is assigned to, i.e. either the system link, the interference link or the sensing link, respectively. This information contains e.g. antenna heights, actual antenna gain, actual frequency, distance between the components, local environment etc. (on the below figure referenced as LinkResult). It will be built into a Java archive (.jar file) and can subsequently be installed in a running SEAMCAT.

The structure of the PMP is similar to other types of plugins. It is composed of the following:

- evaluate(): the core body of the PMP with the mathematical formulation of its physical behavior;

consistencyCheck(): the check of consistency / validity of the input parameters ;

description(): the unique name of the PMP and an optional description (but highly recommended in order to show the user its characteristics and its scope of application);

Input: the input parameters of the PMP (see section ‎14.3.1).

- Figure 331: basic structure of the PMP

The referenced parameters are detailed as follow (only different to the EPP, see section ‎14.7.2):

LinkResult
all information about the link condition that the PMP is assigned to

Boolean b (on the above example already renamed) flag whether variations have to be considered by the model

An example of a PMP (simple Dual-Slop model, similar to IEEE 802.11 Model C) is presented below. Note that the consistency check is separately shown in section ‎14.3.2. The input parameter are presented in Figure 332 and the algorithm is presented in Figure 333. The panel of the simple Dual-Slop model in SEAMCAT is shown in Figure 334.

Figure 332: Description and input interface (The parameter variations can be pre-set by the PMP)

- Figure 333: Algorithm implementation in the method evaluate() (the yellow marked condition lets the PMP stop calculations in case true, due to not consistent with the scope of application)

- Figure 334: PMP configuration panel

## Antenna Plugin (AP)

The interface AntennaGainPlugin provides, similar to the PMP, all information about the link it is assigned to, additionally all information about the parameters relevant for the antenna, for instance the height and azimuth and elevation angles as well as the local environment it is placed at.

The structure of the AP is similar to other types of plugins. It is composed of the following:

- evaluate(): returns the calculated antenna gain

- consistencyCheck(): see section 8.2.2

- description(): the unique name of the PMP and an optional description (but highly recommended in order to show the user its characteristics and its scope of application)

- Input: see section 8.2.1

Figure 335: Basic structure of the AP

The referenced parameters are detailed as follow (only different to the EPP, see section ‎14.7.2, and the PMP, see section 8.6.3):

AntennaResult:
It contains the information about the parameters relevant for the antenna, for instance the height, azimuth and elevation angles as well as the local environment and position it is placed at

Double v:
It is the peak gain of the antenna

There are basically two options to create antenna plugins

Static: 
The plugin predefines static values of the antenna pattern and uses these for the calculation of the gain. You can, in addition, edit (i.e. modify / replace) the values on the system and / or antenna library level.

Dynamic: 
The plugin dynamically calculates the antenna pattern (e.g. ITU-R Recommendation like ITU-R F.1336 etc…) using these for the calculation of the gain.

Simple examples are shown in section ‎14.5.1 (static) and section ‎14.5.2 (dynamic).

### AP – static pre-set of the antenna pattern

The below example is just to show how such a plugin could be done. It is worth to mention that in most cases it would be more efficient to duplicate an existing Horizontal/Vertical antenna of the antenna library, modify the pattern and rename it accordingly.

Figure 336: Simple example for static pattern

Figure 337: Configuration panel of the AP example with static pre-set of the antenna pattern

Figure 338: Example of (random) antenna pattern

### AP – dynamic calculation of antenna gain

In several cases, the antenna pattern depends on operational conditions of the system the antenna is assigned to. For instance, cellular systems apply antennas with pattern dependent on the width of the sector that the antenna shall serve, or for the Fixed Service, the knowledge of the antenna pattern size, frequency is inherent to the workspace that you will simulate. Hence predefined set of pattern may not be adequate, and an antenna plugin that is able to provide the feasibility to generate appropriate pattern for each event would be more suited.

The example below shows an excerpt of the built-in antenna plugin ITU-R F.699 that takes into consideration an equation instead of a preset values.

- Figure 339: simple example for dynamic pattern

- Figure 340: Configuration panel of the example

## Coverage Radius Plugin (CRP)

Following the general plugin approach of SEAMCAT 5, the default coverage radius types 1) User-defined radius 2) Noise Limited and 3) Traffic Limited are implemented as plugins.

You may define your own coverage radius algorithms by doing an appropriate plugin. The coverage radius plugins are applicable only to Generic Systems.

The structure of this Coverage Radius Plugin is similar to other types of plugins. It is composed of the following:

- evaluate(): algorithm of the calculation of the coverage radius;

consistencyCheck(): see section ‎14.3.2;

description(): the unique name of the coverage radius plugin and an optional description (but highly recommended in order to show the user its characteristics and its scope of application);

Input: the input parameters of the Coverage Radius Plugin, see section ‎14.3.1.

Figure 341: Basic structure of the coverage radius plugin

The referenced parameters are detailed as follow (only different to the EPP, the PMP and the AP):

GenericSystem:
contains all information about the system. i.e. its transmitter, receiver and the link conditions in between. It includes also for instance the information whether the system applies a user-defined dRSS, and if its distribution, too.

For simplification the already implemented Traffic Limited plugin is shown as example:

Figure 342: Traffic limited coverage radius as example

Figure 343: Configuration panel of Traffic Limited coverage radius

There are no specific predefined values for the Traffic Limited coverage radius plugin (e.g. set to zero) since generic systems parameters may generally differ from each other.

## Event Processing Plugin (EPP)

### Overview

EPP follows the same principles of module and encapsulation as the PMP since it is the same plugin abstraction layer. Monte Carlo algorithm operates on a high level so that it randomize a setup (event), compute result for event, repeat desired number of times and collect a total result. This is very similar to how SEAMCAT operates on the highest level.

Notice that once an EPP is installed in SEAMCAT it can be configured for any scenario possible. It is therefore bad practice, when programming an EPP to assume anything about the scenario (e.g. the victim system is of a specific type). A re-usable EPP must be implemented in full generality assuming nothing about the scenario, and if something specific is required from the scenario, provide the necessary validation during the consistency check (e.g. if an EPP only works for a generic victim system - it should emit a warning during the consistency check phase allowing the user to abort the simulation).

An EPP has access to the scenario i.e. the complete setup of the simulation. Also it can access its defined input parameters. Interestingly it has an iterator where it can access the event results. An iterator is a Java construct that allows you to ask if it has more elements and get the next element – also this construct can be used in a for-loop. The interface of EPP shows that one must return a type ResultTypes. This is a container for all the results produced by the EPP.

In SEAMCAT, we have immutable state, and therefore, it does not matter at what time different components access it. All will read the same regardless of the interleaving. Suppose the person object was mutable, i.e. the name property could be changed. If there were several parallel threads that could access the same person object it would be really difficult to know what person.getName() would return. Null? "John", "Joe"?. For immutable state, time does not matter, parallel processing is possible.

### Core EPP

The EventProcessingPlugin interface provides all simulated results as EventResult to the EPP and defines its basic structure as shown in Figure 344. It is composed of the following:

- evaluate(): inside the method evaluate the code to be performed by the EPP results generated by the EPP are passed to the workspace by ResultTypes and displayed on the result panel

- consistencyCheck(): it allows the consistency check of the EPP reporting warning messages together with the consistency check of the workspace

- description(): the description is split into  "name of the EPP"," description" (comma separated string variables)

- Input: input parameters of the EPP

- tabs(): the interface to the PPUI (Post Processing UI)

- Figure 344: basic structure of the EPP

The referenced parameters are detailed as follow:

Scenario
settings of the systems used for the workspace. It holds the settings of the interference scenario as specified in the UI. EventResult holds the results data which can be read and used to create new data. More interestingly it also holds intermediate data – this is data not exposed as results in SEAMCAT

EventResult
all simulated results, for instance positions of all components, the path loss between components, the simulated antenna gains, the simulated vectors for dRSS and iRSS values etc… The EventResult holds data for any kind of scenario – i.e. Cellular victim and two generic interferers.

ConsistencyCheckContext List<Object>
due to the general concept of the plugins it is always referenced on the method consistencyCheck and contains only the scenario.

Input
the parameters defined by the EPP and to be used by itself

Validator
the interface to the workspace consistency check; the syntax for a warning message is like this:
validator.error("warning message");

String s
similar to List<Object>, contains the information about the link type, e.g. Victim System Link, and not applicable to EPPs

Description
unique name of the EPP which is displayed on EPP library panel for selection;
optional description, in case empty the syntax has to be (pluginName,"")

ResultTypes
results which the EPP may produce are reported to the result panel of SEAMCAT;
the following types are available:

Single Value Result (double, integer, long, string);

Vector Result  List<Double>);

Vector Group Result (Map of List<Double>);

Bar Chart;

Scatter diagram.

### Post processing UI as optional part of the EPP

The PPUI is considered by the EPP by adding it to a list of PorstProcessingUI as shown on the bult-in Demo 6:

- Figure 345: Example of building a PPUI

You may add as many UIs as you want.  These are displayed as own tabs.

- Figure 346: Example of an EPP generating 2 custom User Interface

A basic CustomUI is composed of the following:

- getTitle(): title of the UI, displayed as name of the corresponding tabulator of the result panel of

SEAMCAT

- buildUI(): this code constructs the panels and the actions to be performed by the custom UI

- panelDefinitions(): It is the definition of the content of the panels of the custom UI

- Figure 347: Basic structure of the custom UI

The core custom UI elements are detailed in the following section.

Panel definition

You can define multiple panel within the panelDefinitions() as shown in Figure 349. The below code corresponds to the sub panels of the custom UI where the input parameters are placed onto.

- Figure 348: Example of code to split the panels

- Figure 349: Example of multiple panel definition

These input interfaces have the same structure and annotation as shown in the above section ‎14.3.1. For the example of Figure 349, the code will look like this:

- Figure 350: Example of code to insert input parameter for the “left” and “right” panels.

Method buildUI

Here you define the design of the custom UI.

- Figure 351: Example of code to split the panels

The above example splits the UI into two horizontally arranged sub panels which can be accessed through e.g. ModelPanel<Left> leftPanel = panels.get("Parameters");

@PostProcessing – Annotation of Custom UI

The post processing annotation generates an action button which lets the custom UI perform the actions defined by the method assigned to this button. The buttons cannot be de-activated ("greyed out") e.g. when building the UI or by the EPP. Hitting the button performs the action regardless any other thing.

Figure 348 presents that (1) the annotation is similar to the input interface and (2) the body of the method to be performed.

Figure 352: Example of @PostProcessing

The post processing annotation allows the following parameters:

Scenario
settings of the systems used for the workspace

ResultTypes
the result types generated by the "parent" EPP

Interface input
the input interface of the "parent" EPP

SimulationResult
all vectors and vector groups generated by the workspace.

The parameters are restricted to these four types. The call order is not relevant. An example of the @post processing feature is presented below where various output display can be selected like “scatter diagram”, “bar chart”, or a “vector group” as shown in Figure 353.

Figure 353: Example of creating 3 “action button” using @PostProcessing

The name of the button is the name of the annotation and an example of code is presented only for the “scatter” button in Figure 354. Similar @PostProcessing can be coded to create the “Bar chart” and “Vector Group”.

Figure 354: Example of creating the scatter button and its associated action

Consistency check of the custom UI

As aforementioned, the post processing buttons are always enabled by default. In cases of more complex calculations to be performed by the custom UI, it might be advisable to prior check the consistency / validity of the input parameters of the custom UI.

Unlike the other plugins, the custom UI, as a result type of the EPP, does not provide its own consistency check. This needs to be designed by the developer of the EPP, as shown in

Figure 355: Example of creating your own “check consistency” in the custom UI, 
as a result type of the EPP

The example in Figure 356 shows a solution to code your own consistency check within @PostProcessing

Figure 356: Example on how to code your own consistency check within @PostProcessing

The flag "consistencyPassed" could be used to block other methods proceeding in case the check failed. If this flag is declared globally private, all methods of the class can access it. This flag is intended to give the other @PostProcessing the information about the consistency check.

The yellow marked code generates the below message:

- Figure 357: Example on how your own consistency would look like

## FAQ on plugin development

Q: if I try to develop a propagation plugin that makes use of FreeSpace or Hata or P528 or any else, how do I do ?

A: you have to go through the Factory like this example for Extended Hata

- Figure 358: Example on accessing a PMP through Factory.propagationModelFactory() 
in the source code

The above  example applies its own input annotations for 'General environment' and 'variations".

You may take the linkResult from the evaluate call of your propagation model or you can generate a new one by creating a new LinkResultImpl(), putting all relevant parameters of the link onto that.

Data elements

This annex lists all the data types used to defined the input parameters within SEAMCAT:

distributions  - for parameters modeled as random variables but also constant;

function - for parameters modeled as user-defined functions (look-up tables, etc);

emission masks - for transmission masks and similar parameters modeled as emission masks;

radiation pattern - a special class of functions for defining antenna radiation patterns;

generated signals - vectors representing the signal values generated during simulation.

Distributions

The user has access to the distribution window from the  button. This window allows defining the statistical distribution function to be associated with the given variable parameter, whose value would be randomly generated each time when starting a new event during simulation. This is the most often used data type in SEAMCAT, as a majority of the parameters in the workspace scenario may be defined via distributions, with those randomly generated parameters providing the statistical essence of Monte-Carlo simulations in SEAMCAT. Further details can be found in ‎ANNEX 10:.

The distribution entry dialog window looks like this:

Figure 359: Example of the distribution GUI

Function entry

The user  has access to the Function window from the  button. Function entry dialog window is used to define function-like parameters of simulation, such as blocking response or intermodulation rejection. This is how a function entry dialog box looks like:

Table 62: Description of the function options in SEAMCAT

| Description | Comments |
| --- | --- |
| Constant | In this case the function always returns the same constant value |
| User-defined | It is defined by a sequence of points (x, y=f(x)) meaning that the input as many values as needed for abscissa and corresponding function response values, paying attention to units shown in the heading of the relevant function definition window |

Figure 360: Example on how to edit a user defined function (i.e. like receiver blocking mask)

To introduce a non-constant function, select User-defined item in the Type group box and enter the points defining the function in form of pairs (x, y=f(x)) in the function table.

Note: the range of entered data depends on the type of defined parameter,. For example, for user-defined blocking function, all entered values y=f(x) should be positive.

The following functions are available in the dialog window:

Add: To add a new pair, click on Add button. Pairs are then automatically sorted by increasing x value;

Delete: To suppress a pair, click on Delete button;

Sym: To symmetrize a function, click on Sym button, this results in generating for each existing pair (x, y) a symmetric counterpart (-x, y), unless it already exists;

Load: click on the Load button to import a set of function values from a text file. This file must be formatted to contain one pair (x, y) per line, TAB separated;

Save: press the Save button to export the current function table into a text file, e.g. for frequently used functions or for sharing with other users.

Emissions mask

The emissions masks dialog window is designed to define parameters of the unwanted emission of the transmitter.

Figure 361: Example on how to edit a function (i.e. like spectrum emission mask)

An emissions mask function is always defined by a sequence of triplets (x, y = f(x), z) displayed in the emissions mask grid where f(x) stands for the relative emitted power (in dBc) at frequency offset x (expressed in MHz), measured in a reference bandwidth z (expressed in kHz). A full description of the calculation of total emissions is available in ‎ANNEX 6:.

Two curves are displayed:

Red curves: It shows the actual curve of the mask, for given reference bandwidth z, that is:

y = f(x)	(Eq. 72)

Blue curves: it shows the curve normalized in a reference bandwidth of 1 MHz, that is:

y = f(x) + 10log(1000/z)	(Eq. 73)

The blue curve is aimed to detect potential discontinuities in the definition of transmission mask, and is also the one which is used in actual simulations when integrating the total power of unwanted emissions, in order to overcome possible differences of victim and interferer bandwidths.

The value of reference bandwidth is therefore extremely important as it has a direct impact on the calculated unwanted emissions level. Unless the reference bandwidth is separately defined as a part of mask definition (e.g. for mask definitions in radio equipment standards, etc), you are advised to enter the total emissions bandwidth of interefering transmitter (i.e. channel width) as a value for the reference bandwidth.

The following functions are available in the dialog window:

Add: To add a triplet click on Add button. Triplets are then automatically sorted by increasing x value,

Delete: To suppress a triplet at marker position, click on Delete button;

Sym: To symmetrize a transmission mask, click on Sym button, this results in generating for each triplet (x, y, z) a symetric triplet (-x, y,z), unless it already exists;

Load: click on the Load button to load a mask from an external text file. This file must contain one triplet (x, y, z) per line, with values TAB separated;

Save: If you want to save the defined mask for future re-use or sharing with other users, click on Save button to export the defined mask grid to a text file.

Antenna

The pattern function is used to describe the radiation patterns of antennas. In the system definition windows, each transmitter and receiver of both victim and interfering links has an "Antenna Patterns Identification" panel, which allows the user to define antenna parameters. The positioning of the antennas and its pointing convention in a scenario are described in ‎ANNEX 11:.

Figure 362: Antenna patterns identification GUI

Figure 363: Defining antenna pattern, using XY coordinates system

Signal vectors

Signal display window is provided for visualisation of various signals produced by the event generation and link calculations. This window is activated when you double-click on the relevant vector element summarised in Section ‎12.

Statistics

For each vector result, a set of statistics is available:

Average: The average is computed using the simulated results in dB;

Median: The median is computed using the simulated results in linear value. Then the Median is expressed in dB;

Standard deviation (StdDev): The StdDev is computed using the simulated results in dB;

Variance:it is the square of the StdDev in dB;

Min: It returns the minimum value of the vector results;

Max: It returns the maximum value of the vector results.

Visualisation

Three visualisation modes are available according to the option selected in the display radio box, i.e. vector, C.D.F or P.D.F. You may save each visualised data by pressing the "Save" button as explained below. Further details on the save extension (.txt, .xls, .xlsx) is also given in Section ‎2.13.

Vector: this mode displays an events plot of the generated signal vector.

Figure 364: Vector GUI and the corresponding .txt

The saved data are formatted in two columns: the first column contains the event numbers and the second column contains the corresponding value of the signal strength for that event (in the units displayed).

Event number	dRSS [dBm]

0.0		-136.11488844442846

1.0		-122.29568787623859

2.0		-79.77991992419228

3.0		-80.61033154758469

4.0		-106.19867659657338

5.0		-122.66942167419278

Figure 365: Format of the vector file

C.D.F.: this mode allows to display the Cumulative Distribution Function (CDF) of the generated signal. The CDF is computed on the generated signal vector raw data, i.e. as expressed in dBm and not on linear values.

Figure 366: CDF GUI and the corresponding .txt file

The saved data are formatted in three columns: the first column contains the event numbers, the second column contains the values of the signal strength in the units displayed, and the third column contains the corresponding value of the cumulative distribution function.

Event number	dRSS [dBm]		Cumulative

991.0		-174.54696055463498	0.0

819.0		-168.29448466100342	6.666666666666667E-5

363.0		-167.2508479840345	1.3333333333333334E-4

2509.0		-166.92020005084856	2.0E-4

7281.0		-166.57288242874336	2.666666666666667E-4

2508.0		-165.99975472502794	3.333333333333333E-4

3110.0		-165.8293404691574	4.0E-4

Figure 367: Format of the CDF file

- P.D.F.: this mode allows displaying the probability density function (PDF) of the generated signal, i.e. a statistical histogram showing the square root of the number of given signal strength values. The user can change the number of bins.

Figure 368: PDF GUI and the corresponding .txt file

The saved data are formatted in two columns: the first column contains the value (in the units displayed) and the second column contains the corresponding probability.

Bin			Probability

-103.29291416807828	0.0116

-102.3364840152386	0.01067

-101.38005386239891	0.01087

-100.42362370955922	0.00787

-99.46719355671954	0.00753

-98.51076340387985	0.0068

Figure 369: Format of the PDF file

Interference Calculation Algorithm

The algorithm takes one-by-one the corresponding dRSS and iRSS vector results, sums the values from several iRSS vectors if there are more than one interfering link or if user have chosen more than one interference type (i.e. unwanted, blocking, and intermodulation), and then compares that composite iRSS value with dRSS value in that snapshot and with sensitivity floor in accordance with the chosen interference criteria (i.e. C/I, C/(I+N), I/N or I/(I+N)).

General case

If the dRSS signal is above the sensitivity floor, it is counted as qualifying snapshot and if then the composite iRSS signal satisfies the criteria (e.g. if dRSS-iRSS>C/I), then the system notes that the given snapshot produced the non-interfered communication attempt.

SEAMCAT calculates the probability of interference (pI) of the victim link receiver as follows:

PI=1-PNI						(Eq. 74)

were pNI is the probability of Non Interference (NI) of the receiver.

When a C/I criterion is considered, pNI is defined as:

(Eq. 75)

since by definition P(A|B)=P(A∩B)/P(B), pNI becomes:

(Eq. 76)

with where P is the number of interferers (i.e. active transmitters).

Monte Carlo method is applied individually to the numerator and to the denominator of the expression of pNI . The result obtained is an estimation of pNI  by using the following equations (p’NI):

(Eq. 77)

with M the number of events (or snapshots) and where

(Eq. 78)

Similarly, when a C/(I+N) criterion is considered, pNI is defined as:

(Eq. 79)

When a (I+N)/N criterion is considered, pNI is defined as:

(Eq. 80)

When a I/N criterion is considered, pNI is defined as:

(Eq. 81)

Overloading case

Nall events are passing the dRSS criterion

For both Case 1 or Case 2, the probability of interference should be calculated in a two steps process:

- Step 1: calculate the probability of interference resulting from the overloading for a given total number of events where the dRSS > sensitivity.

Among Nall events for which the dRSS criterion is met, Noverloaded are interfered due to the overloading, the corresponding probability is:

Poverloading = Noverloaded / Nall or similarly Poverloading = 1 - ( Ngood_overloading /Nall)

- Step 2: for Nall - Noverloaded  the events which were not interfered in step 1, calculate the probability of interference resulting from blocking / unwanted emissions (the reference should be the total number of events where the dRSS > sensitivity).

i.e. for (Nall - Noverloaded) events which passed Step 1, Nblocked  are interfered by unwanted or blocking or by the consideration of both phenomena (applying the current threshold for unwanted and blocking)

Pblocking  = Nblocked / Nall or similarly Pblocking = 1 - ( Ngood_blocking /Nall)

The total probability of interference is the sum of those two probabilities (step 1 and step 2) of interference.

Ptot 	= Poverloading + Pblocking 						(Eq. 82)

= Noverloaded / Nall + Nblocked / Nall

= (Noverloaded +Nblocked)/ Nall

= 1- ((Nall - Noverloaded +Nblocked)/ Nall

Note: overloading is only implemented in the victim receiving interface of “traditional” SEAMCAT, i.e. the overloading is not implemented where the victim is either CDMA or OFDMA. However, the impact of the interference from CDMA or OFDMA system should be taken into account when overloading is activated in the victim dialog box.

Note: overloading is not implemented for translation. When translation is selected in the ICE window, the overloading is button should be shaded to show that it is not active.

Interference calculation combination

The Interference calculation panel allows you to extract the probability of interference from various combinations. They are summarised in Table 63.

Table 63: Summary of the calculation of the probability of interference resulting from the overloading

| Signal type combination | Description |
| --- | --- |
| Unwanted only | Calculate the probability of interference by only considering the iRSSunwanted |
| Blocking only | Calculate the probability of interference by only considering the iRSSblocking |
| Intermodulation only | Calculate the probability of interference by only considering the iRSSintermod |
| Blocking + unwanted | Calculate the probability of interference by considering both the iRSSunwanted and iRSSblocking |
| Blocking + unwanted + intermodulation | Calculate the probability of interference by considering the iRSSunwanted , iRSSblocking and iRSSintermod |
| Overloading only | Calculate the probability of interference resulting from the overloading only for a given total number of events where the dRSS > sensitivity. The resulting probability is Poverloading = 1 - ( Ngood_overloading /Nall) and Ngood_overloading = Nall - Noverloaded where: Nall is the number of event where dRSS > sensitivity, Ngood_overloading is the number of “good” events from overloading (i.e. not overloaded). Noverloaded is the number of event when the receiver is overloaded. e.g. For 100 events simulated, dRSS > sensitivity occurred for Nall (=70 events). Out of Nall, the overloading occurred for Noverloaded (=30 events). Therefore the probability of interference of overloading only is Poverloading =1-(40/70). |
| Overloading + blocking | Calculate the probability of interference resulting from the overloading and from the blocking for a given total number of events where the dRSS > sensitivity. The resulting probability is Poverloading+blocking = 1 - ( (Ngood_overloading+blocking)/Nall ) and Ngood_overloading+blocking = Nall - Noverloaded - Nblocked where: Nall is the number of event where dRSS > sensitivity, Ngood_overloading+blocking is the number of “good” events from the combined overloading and blocking mechanism. Noverloaded is the number of event when the receiver was overloaded. Nblocked is the number of event which were inpaired by blocking (i.e. receiver blocked). It is obtained from the comparison with the iRSSblocking and a specific interference criterion (C/I, I/N etc..) e.g. For 100 events simulated, dRSS > sensitivity occurred for Nall (=70 events). Out of Nall, the overloading occurred for Noverloaded (=30 events). The iRSSblocking is then calculated over the (70-30=) 40 remaining events. Number of good events resulting from blocking, Nblocked, is based on the comparison between the iRSSblocking and a specific interference criterion over these 40 events. Let say that for Nblocked = 25 events out of 40 the receiver is blocked. Therefore the probability of interference of the combined overloading and blocking is Poverloading+blocking= 1 - ( (40 - 25)/70 ) |
| Overloading + blocking + unwanted | Same principle as above except that the iRSScomp (=iRSSblocking+iRSSunwanted) is considered in the comparison to a specific interference criterion |
| Overloading + blocking + unwanted + intermodulation | Same principle as above except but additionally considering iRSSintermod on iRSScomp |
| Overloading + unwanted | Same principle as above except that the iRSSunwatnted is compared to a specific criterion |

Relationship between C/I, C/(N+I), (N+I)/N and I/N

SEAMCAT performs a consistency checking between the interference criteria:

Between C/I and C/N+I: C/I must be higher than C/N+I.

Between C/I, C/(N+I), and (I+N)/I if these 3 parameters are defined.

(Eq. 83)

Between C/I, C/(N+I), and I/N if these 3 parameters are defined.

(Eq. 84)

Between I+N/I and I/N if these two parameters are defined.

(Eq. 85)

For example, we may calculate the (N+I)/N when we know the value of C/I and C/(N+I). If we assume that:

C/I = 19 dB	C/(N+I) = 16 dB

Using the equation:

(Eq. 86)

This gives:

(Eq. 87)

Therefore, if the you provide a value for each of these parameters (C/I, C/(N+I) and (N+I)/N), SEAMCAT using the equation (1) checks the consistency of these parameters.

Taking into account this value of (I+N)/N  and using the equation:

(Eq. 88)

Considering

(Eq. 89)

and that

(Eq. 90)

Therefore, this leads to

(Eq. 91)

The following default values for the C/I, C/(N+I), (N+I)/N and I/N of 19 dB, 16 dB, 3 dB and 0 dB respectively avoid the display of a warning. In case you are using inconsistent interference criteria value, you will see the following warning

Figure 370: Interference criteria inconsistency warning

dRSS calculation

dRSS calculation

For one single event, the instantaneous desired Received Signal Strength, dRSScalc is calculated as

dRSS=PVLToutput-PLVLT→VLR+GVLR→VLT+GVLT→VLR			(Eq. 92)

where

PVLToutput		: Power supplied to the VLT antenna;

GVLT→VLR	: VLT antenna gain in the direction of the VLR;

GVLR→VLT	: VLR antenna gain in the direction of the VLT;

PLVLT→VLR 	: Path loss between the VLT and the VLR.

Power control

If the received signal can not exceed a given value, i.e. depending on the power control implemented in the victim system, the following applies dRSS = min(dRSScalc , sens+Pcmax x ). The power control implemented for the dRSS is described in ‎ANNEX 14:.

User defined dRSS

You can define a specific constant value or distribution of dRSS in the Transmitter to Receiver Path window by selecting user defined dRSS.

Figure 371: User defined dRSS

In this case the SEAMCAT overwrites the algorithms for the dRSS calculation and proceeds directly to calculation of iRSS.

iRSS calculation

For the iRSS calculation i different interfering systems may be considered, each having some n active ILTs. The resulting interfering power in the VLR is computed as described below.

Unwanted calculation

The calculation of the interfering signal strength (iRSS) values for each event for unwanted interference (iRSSunwanted) is as follows where the i-th interferer signal is defined as

iRSSunwantedi=emissionILTfILT-fVLR+gILTPC-maxPLILT→VLR-GILT→VLR+GVLR→ILT,MCL	(Eq. 93)

The calculation is repeated for each defined interfering link so that the total iRSSunwanted is

iRSSunwanted =10×log10i=1n10iRSSunwantedi10		(Eq. 94)

where for each interferer

fILT		: frequency of the ILT;

fVLR		: frequency of the VLR;

emissionILTfILT-fVLR	: relative emission mask which is a function of f = ;

gILTPC		: power control gain for the ILT with the power control function, see ‎ANNEX 14:;

GILT→VLR	: ILT antenna gain in the direction of the VLR;

PLILT→VLR	: path loss between the interefering link transmitter i and the victim link receiver;

GVLR→ILT	: VLR antenna gain in the direction of the ILT.

In addition, relative emission mask is introduced to enable calculations of interference between systems in the same or adjacent bands. The real emission is always greater or equal than the absolute emission floor.

(Eq. 95)

where

PILToutput 		: power supplied to the ILT antenna (before power control);

: relative emission as described in ‎ANNEX 6:;

: absolute emission floor.

Blocking calculation

The calculation of the interfering signal strength (iRSS) values for each event for unwanted interference (iRSSblocking) is as follows where the i-th interferer signal is defined as

iRSSblockingi=PILToutput+ gILTPC-aVLRfILT-fVLR-maxPLILT→VLR-GILT→VLR-GVLR→ILT,MCL

(Eq. 96)

The calculation is repeated for each defined interfering link so that the total iRSSblocking is

iRSSblocking =10×log10i=1n10iRSSblockingi10		(Eq. 97)

where for each interferer:

fVLR	: victim link operating frequency;

fILT	: Interfering link operating frequency. Note that the trial of the dRSS frequency, fVLR, occurs only once for each simulation round, i.e. fVLR is trialled once, together with the VLT, VLR positions, the VLT transmit power and other distributions pertaining to the victim link. These values associated with the dRSS trial round then apply to all n trials of iRSS (where n is the number of interferers);

PILToutput	: power supplied to the ILT antenna (before power control).;

gILTPC 	: power control gain for the ILT with the power control function, see ‎ANNEX 14:.

GILT→VLR	: ILT antenna gain in the direction of the VLR;

GVLR→ILT	: VLR antenna gain in the direction of the ILT;

PLILT→VLR 	: path loss between the interfering link transmitter i and the victim link receiver;

aVLR fILT-fVLR 	: blocking attenuation of the victim link receiver, see ‎ANNEX 8:.´;

MCL	: Minimum coupling loss given by the system parameter definition.

Intermodulation rejection calculation

ETSI defines the intermodulation response rejection as a measure of the capability of the receiver to receive a wanted modulated signal, without exceeding a given degradation due to the presence of two or more unwanted signals with a specific frequency relationship to the wanted signal frequency ‎[18].

In SEAMCAT, it is called the intermodulation rejection, and in the presence of more than one interfering system applying its own frequency distribution, a theoretically unlimited number of intermodulation products exist, caused by the non-linearity of the VLR.  In practice, just the products close to the frequency of the VLR are of importance, of which the products of the so called 3rd order are the most dominant and therefore considered by SEAMCAT. The frequency conditions for the intermodulation products of the 3rd order are

(Eq. 98)

with

fVLR-b2≤f0≤fVLR+b2					(Eq. 99)

where

b 	: is the bandwidth of the VLR;

: the trialed frequency of the victim system;

: the trialed frequencies of pairs of interfering systems in case more than two systems are to be taken into account.

As an example consider a victim system with a desired signal at frequency f0, a channel separation f and interfering signals Ei1 and Ei2  at frequencies f0+n∆f and f0+2n∆f, respectively. The receiver non-linearity produces an intermodulation product Eif of third order at the frequency f0 as shown below in Figure 372.

(Eq. 100)

Figure 372: Illustration of intermodulation product Eif of third order at the frequency f0

The signal strength Eif of the intermodulation product is given by

(Eq. 101)

with some constant k to be determined. For signal levels (measured in dB), Eif becomes iRSSintermodi,j and the equation above reads

iRSSintermodi,j=2iRSSi1+ iRSSi2+20×logk				(Eq. 102)

From the measurement procedure which is described e.g. in the ETSI standard ETSI 300-113, clause 8.8, we can derive the calculation algorithm. The method is similar to the contribution for blocking interference. ETSI 300-113 defines via the intermodulation response Limr the interfering signal levels iRSSi1 = iRSSi2 at which bit errors due to intermodulation just start to be recorded.

Figure 373: Illustration of intermodulation product from ETSI 300-113

This means, for iRSSi1 and iRSSi2, we have an intermodulation product iRSSintermodi,j relative to the noise floor (0 dB), due to the noise augmentation (N+I)/N of 3 dB corresponds to an I/N of 0 dB. Transferring Figure 373 into a formula gives the relation

(Eq. 103)

from which we can derive the calculation algorithm for this example

iRSSintermodi,j=2(iRSSi1-Limr1)+ (iRSSi2-Limr2)-3Lsens+3dB	(Eq. 104)

where for interferer i-th, at frequency x:

iRSSi,x=PILToutput+ gILTPC+maxPLILT→VLR-GILT→VLR-GVLR→ILT,MCL		(Eq. 105)

where

PILToutput	: power supplied to the ILT antenna (before power control);

gILTPC 	: power control gain for the ILT with the power control function, 
see ‎ANNEX 14:;

PLILT→VLR 	: path loss between the interfering link transmitter i and the victim link receiver;

GILT→VLR	: ILT antenna gain in the direction of the VLR;

GVLR→ILT	: VLR antenna gain in the direction of the ILT;

MCL	: Minimum coupling loss given by the system parameter definition.

For any other consistent combination of (N+I)/N and I/N this becomes

iRSSintermodi=2(iRSSi1-Limr1)+ (iRSSi2-Limr2)-3Lsens+N+IN-IN 	(Eq. 106)

For the computation of the intermodulation products in the victim link receiver two different interfering systems (i-th and j-th) are required and the total intermodulation product is

iRSSintermod =10×log10i=1ni=1i≠jn10iRSSintermodi,j10		(Eq. 107)

where:

iRSSintermodi,j	: intermodulation product at the frequency f0;

Limr		: the attenuation given by the input parameter 'Intermodulation rejection'. This attenuation applies inside the bandwidth of the VLR. It is therefore advisable to set a constant value as intermodulation rejection;

Lsens		: the sensitivity of the VLR given by the system parameter definition;

N+IN		: Noise augmentation given by the system parameter definition;

IN		: interference to noise ratio given by the system parameter definition.

In case the intermodulation product does not fit the frequency condition fVLR-b2≤f0≤fVLR+b2 , a value of -1000 dBm is returned in SEAMCAT.

The EPP Demo9 (Figure 374) allows you to see the behaviour of this equation.

Figure 374: EPP Demo 9 – intermodulation internals

Overloading calculation

SEAMCAT calculates the interfering signal resulting from the possible interfering system(s) at the interfering frequency as follows:

For each events{

If (dRSS > sensitivity){

Case 1: single source of interference

iRSSoverloading=PILToutput+ gILTPC-Rx_filter+maxPLILT→VLR-GILT→VLR+GVLR→ILT,MCL 	(Eq. 108)

where:

PILToutput	: power supplied to the ILT antenna (before power control);

gILTPC 		: power control gain for the ILT with the power control function, 
see ‎ANNEX 14:;

PLILT→VLR 	: path loss between the interfering link transmitter i and the victim link receiver;

GILT→VLR	: ILT antenna gain in the direction of the VLR;

GVLR→ILT	: VLR antenna gain in the direction of the ILT;

MCL		: Minimum coupling loss given by the system parameter definition;

Rx_filter	: Filtering of the receiver (if any).

- Note: The filtering of the receiver is by default 0 dB (similarly to the default blocking filtering value).

Case 2: multiple source of interference

2.1 If the interferers are operating at the same frequency, iRSSoverloading is sum of each of the iRSSoverloading calculated for each of the source of interference.

2.2  If the interferers are operating at different frequencies, an iRSSoverloading should be calculated considering all of the sources of interference for each frequency (i.e. the sum of the iRSS overloading is performed at the specific frequency).  Each of the calculated iRSSoverloading should be compared with the different overloading threshold extracted from the overloading mask depending on the considered frequency offset.

}

else

{

skip this event.

}

}

iRSSuwanted, iRSSblocking and iRSSintermodulation calculation flow chart

iRSSblock=10logi=1n10iiRSSblock10

iRSSunwanted=10logi=1n10iiRSSunwanted/10

iRSSintermod=10logi=1nj=1, j≠in10ii,jRSSintermod/10

Calculation of intermodulation products

ii,jRSSintermod

Calculation of:

gILT→VLRfILT, gILT→VLRfVLR, gVLR→ILTfILT, gVLR→ILTfVLR,PLILT→VLRfILT, PLILT→VLRfVLR

Calculation of:

iRSSunwantedi=emissionILTfILT-fVLR+gILTPC-maxPLILT→VLR-GILT→VLR+GVLR→ILT,MCL

iRSSblockingi=PILToutput+ gILTPC-aVLRfILT-fVLR-maxPLILT→VLR-GILT→VLR-GVLR→ILT,MCL

gILTPC=0

no

Power

Control?

Trial of:

hVLR,gILT→VLR, gVLR→ILT, dILT→VLR

Calculation of: RILTmax

Calculation of :

PLILT→VLR, gILTPC

yes

Trial of relevant interferer parameters:

fILT, hILT, θILT→VLR, pILTsupplied, dILT→VLR

Calculation of Rsimu

Interferer i = 1 : n

Calculation of Rmax

Figure 375: irss due to unwanted and blocking calculation

sRSS calculation

For one single event, the instantaneous sensing Received Signal Strength, sRSS is calculated as

sRSSfm=PVLToutputfm-PLVLT→ILT+GVLT→ILT+GILT→VLT		(Eq. 109)

where

PVLToutputfm		: power supplied to the VLT antenna. The bandwidth of the sensing device (It) should be considered in the calculation. Note: The emission bandwidth of the VLT may be different from the ILT receiving bandwidth, therefore when the sRSS is to be calculated, a bandwidth correction factor is needed. PVLT is subject to power control. The emission spectrum mask of the VLT is provided in order to calculate PVLT at frequency fm;

fm		: is the frequency of the sensing device;.

GVLT→ILT	: VLT antenna gain in the direction of the ILT;

GILT→VLT	: ILT antenna gain in the direction of the VLT;

PLVLT→ILT 	: Path loss between the VLT and the ILT.

Unwanted emissions mask

Spectrum emission mask

The spectrum emission mask of a transmitter can be broken up as follows:

Necessary bandwidth = width of the frequency band which is theoretically just sufficient to ensure the transmission of information.

Occupied bandwidth = width of the frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage β/2 of the total mean transmitted power. The value of β/2 shall be taken as 0.5%.

Out-of-band emission = emission on frequencies immediately outside the channel bandwidth which results from the modulation process and non-linearity in the transmitter, but excluding spurious emissions.

Spurious emission = emission on frequencies which are outside the channel bandwidth and the level of which may be reduced without affecting the corresponding transmission of information, thus excluding out-of-band emission.

Unwanted emissions = consist of out-of-band emission and spurious emission.

Figure 376: Illustration of a spectrum emission

The necessary bandwidth shall be considered to determine the maximum co-channel interference level that is acceptable by the receiver and the occupied bandwidth shall be considered to determine in which part of the channel 99% of the transmitted power is concentrated.

Unwanted emission Mask

For the interefering link transmitter, an emission mask is defined as a function of ∆f=f ,fILT and can be defined as maximum power levels emissionILT∆f in reference bandwidth bS∆f.

This mask can also be expressed as the maximum of

the sum of the supplied interfering power pILTsupplied, a relative emission mask (containing the wanted transmission and all unwanted emissions including the emission floor depending on the power control) and the gain power control;

or the absolute emission floor.

The relative emission mask is described by a triplet (frequency offset (MHz), relative emission level (dBc) and reference bandwidth (MHz)). The emission floor is defined later in this Annex.

The interefering link transmitter power pILTsupplied (dBm) at fILT is used for evaluating the link budget with the interfering link receiver (i.e. power control).

ILT emissions mask

Figure 377 shows the principle of the determination of the interfering power. If fILT=fVLR then the interfering frequencies fall exactly in the receiving band of the victim link receiver (co-channel interference).

For simplification within the algorithms the mask function  is normalized to 1 Hz reference bandwidth:

(Eq. 110)

The bandwidth  is the bandwidth used for the emission mask. The total received interfering power  can easily be calculated by integration over the receiver bandwidth from  to

(Eq. 111)

with  denoting the normalized mask in dBm/Hz. Using 1 Hz reference bandwidth the integral can be replaced by a summation

(Eq. 112)

where  is given in dBm.

Note:  The interfering power of a radio system having a different bandwidth can be estimated by the aforementioned algorithms. This calculation is only required for the interference due to unwanted emissions but not for blocking and intermodulation.

(fVLR-fILT)+bVLR2

(fVLR-fILT)-bVLR2

(fVLR-fILT)

Victim Receiver

Unwanted Emission Mask

Figure 377: Integration of the unwanted emissions in the victim link receiver band

The total interfering power relative to carrier  can be calculated by integration over the receiver bandwidth from  to

(Eq. 113)

with  denoting the normalized user-defined mask in dBc/Hz.

This mask is expressed as an array of N+1 points and assumed linear between these points.

(Eq. 114)

This leads to:

(Eq. 115)

where:

(Eq. 116)

(Eq. 117)

Intermediate calculation

(Eq. 118)

(Eq. 119)

Eventually:

(Eq. 120)

Unwanted Emission noise floor

The aforementioned formulas are also applicable to absolute emission floor emission_floorILT (dBm) . This emission floor mask can be described by a triplet (frequency offset (MHz), emission floor (dBm), reference bandwidth (MHz)).

Figure 378: illustration of the emission mask and the unwanted emission floor mask

The real emission is bounded by the emission floor by the following equation and illustrated in Figure 379.

(Eq. 121)

Note that the value of the emission_floorit is scaled to the VLR bandwidth. This means that assuming a VLR bandwidth of 200 KHz and the value of the Emission mask and Unwanted Emission floor as presented in Figure 378, the following emisssionit are calculated:

for a fILT = 900.3 MHz and fVLR = 900.0 MHz:

emissionILT = max (-30 + 5 + 0 , -10)

emissionILT = -10 dBm;

for a fILT = 900.0 MHz and fVLR = 900.0 MHz:

emissionILT = max (0 + 5 + 0 , 13)

emissionILT = 13 dBm;

Figure 379: Resulting power level for the calculation of iRSS

Examples of dBc emission mask and relation between normalised mask and ref. BW mask

The following range of attenuation is considered, for a system of 30 dBm using a 20 kHz emission bandwidth. The reference bandwidth for the attenuation is 10 kHz. Within the emission bandwidth the reference bandwidth is taken equal to the emission bandwidth.

Table 64: Emission mask value as illustrated in Figure 380

| Frequency offset | Attenuation in dBc |
| --- | --- |
| 0 to 10 kHz | 0 |
| 10 to less than 20 kHz | Min (p(dBW)+30, 36) |
| 20 to less than 30 kHz | Min (p(dBW) + 40, 36) |
| 30 to less than 40 kHz | Min (p(dBW) + 45, 40) |
| 40 to less than 50 kHz | 50 |

Figure 380, shows the “upper” part of the mask derived from Table 63, the whole mask is symmetric. The frequency offset is defined in MHz.

Figure 380: Example of “Upper part” of an Emission Mask in dBc

If the mask is symmetric, the whole mask may be obtained by using the “Sym” function in SEAMCAT as shown in Figure 381. N1 and N2 correspond to the same level of power and correspond to attenuation defined in dBc given in reference bandwidth (kHz) and normalised bandwidth (1 MHz) respectively.

N1 (dBm/Bref) = P (dBm) + Att1 (dBc/Bref)	(Eq. 122)

N2 (dBm/1 MHz) = P (dBm) + Att2 (dBc/1 MHz)

where P is the Power within the emission bandwidth.

N1 (dBm/Bref) and N2 (dBm/1 MHz) represent the same level of power (Pi):

Pi (dBm) = N1 (dBm/Bref) + 10 log(Bref)

Pi (dBm) = N2 (dBm/1 MHz) + 10 log(1 MHz)

Therefore the relationship between the attenuations in dBc defined in reference bandwidth and in 1 MHz is given by:

Delta = N1 (dBm/1 MHz) - N2 (dBm/Bref)

Delta = Pi (dBm) - 10 log(Bref) - (Pi (dBm) - 10 log(1 MHz))

Delta = 10 log( (1 MHz)/(Bref) )

Note:

- If the reference bandwidth is larger than the emission bandwidth then the attenuation must be defined with positive sign;

- If the reference bandwidth is lower than the emission bandwidth then the attenuation must be defined with negative sign;

- If the reference bandwidth is equal to the emission bandwidth then the attenuation should be set-up at zero.

Figure 381: Link between the normalised mask and the mask given in reference bandwidth

Examples of calculation using dBc

The ITU-R Recommendation ITU-R SM.329 ‎[4] on Unwanted emissions in the spurious domain provides a definition of dBc unit, which is defined as “Decibels relative to the unmodulated carrier power of the emission. In the cases which do not have a carrier, for example in some digital modulation schemes where the carrier is not accessible for measurement, the reference level equivalent to dBc is decibels relative to the mean power P. From this recommendation the following example is presented:

A land mobile transmitter, with any value of emission bandwidth, must meet an attenuation of 43 + 10 logP, or 70 dBc, whichever is less stringent. To measure the emissions the use of a reference bandwidth of 100 kHz is recommended for spurious emissions within the frequency range 30 to 1000 MHz.

With a measured total mean power of 10 W:

Attenuation relative to total mean power = 43 + 10 log(10) = 53 dB

The 53 dBc is less stringent than 70 dBc, so the 53 dBc value is used. Therefore, emissions must not exceed 53 dBc in a 100 kHz reference bandwidth, or converting to an absolute level: 10 dBW – 53 dBc = – 43 dBW in a 100 kHz reference bandwidth.

With a measured total mean power of 1 000 W:

Attenuation relative to total mean power = 43 + 10 log(1000 ) = 73 dB

The 73 dBc is more stringent than 70 dBc limit, so the 70 dBc value is used. Therefore, emissions must not exceed 70 dBc in a 100 kHz reference bandwidth, or converting to an absolute level: 30 dBW - 70 dBc = -40 dBW in a 100 kHz reference bandwidth.

Defining a CDMA  unwanted emission mask in SEAMCAT (Interfering Link)

BS example

As an example of a BS spectrum emission masks (i.e. CDMA DL as interferer), TS 25.104 ‎[1] have shown that different values may need to be considered depending on the BS maximum output power Table 65 presents Spectrum emission mask values, BS maximum output power P  43 dBm.

Table 65: Spectrum emission mask values, BS maximum output power P  43 dBm (from ‎[1])

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement Band I, II, III, IV, V, VII, VIII, X | Additional requirements Band II, IV, V, X | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | -14 dBm | -15 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | -15 dBm | 30 kHz |
| (see note 3) | 3.515MHz f_offset < 4.0MHz | -26 dBm | NA | 30 kHz |
| 3.5 MHz f fmax | 4.0MHz f_offset < f_offsetmax | -13 dBm | -13 dBm 4 | 1 MHz |

Assuming a BS Tx power of 43 dBm, the equation of Table 66 gives the SEAMCAT input as presented in XXX and as illustrated in Figure 382.

Note that for this example, we add the limit from Recommendation ITU-R SM.329 for frequency offset above 12.5 MHz ‎[4].

Table 66: Example of derivation of 3GPP spectrum emission mask into a SEAMCAT input format (with addition of the ITU-R Rec. SM.329 limit)

| Frequency offset of measurement filter -3dB point, f | Minimum requirement Band VIII | SEAMCAT (in ref/measurement bandwidth) | Measurement bandwidth |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | -14 dBm | 43 – (-14) = 57 | 30 kHz |
| 2.7 MHz f < 3.5 MHz |  | Linear | 30 kHz |
| (see note 3) | -26 dBm | 43 – (-26) = 69 | 30 kHz |
| 3.5 MHz f fmax | -13 dBm | 43 – (-13) = 56 | 1 MHz |
| Above 12.5 MHz (ITU-R SM.329) | -36 dBm | 43 – (-36) = 79 | 100 kHz |

Figure 382: Illustration of the spectrum emission mask at the BS used in SEAMCAT and its derivation from the 3GPP specification and ITU-R SM.329 (for frequency offset larger than 12.5 MHz within the spurious emission domain in the frequency range 30 to 1000 MHz).

UE example

As an example of a UE spectrum emission masks (i.e. CDMA UL as interferer), TS 25.101 ‎[5] indicates that the power of any UE emission shall not exceed the levels specified in Table 67.

Table 67: Spectrum Emission Mask Requirement

| Δf in MHz (Note 1) | Minimum requirement |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | Relative requirement | Absolute requirement |  |
| 2.5 - 3.5 |  | -71.1 dBm | 30 kHz (Note 2) |
| 3.5 - 7.5 |  | -55.8 dBm | 1 MHz (Note 3) |
| 7.5 - 8.5 |  | -55.8 dBm | 1 MHz (Note 3) |
| 8.5 - 12.5 MHz | -49 dBc | -55.8 dBm | 1 MHz (Note 3) |
| Note 1: f is the separation between the carrier frequency and the centre of the measurement bandwidth. Note 2: The first and last measurement position with a 30 kHz filter is at f equals to 2.515 MHz and 3.485 MHz. Note 3: The first and last measurement position with a 1 MHz filter is at f equals to 4 MHz and 12 MHz. |  |  |  |

Table 67 provides directly the values in dBc which are also the format for the SEAMCAT input. Therefore the values from Table 67 translate into SEAMCAT as illustrated in Figure 383.

Figure 383: Illustration of the spectrum emission mask at the UE used in SEAMCAT and its derivation from the 3GPP specification and ITU-R SM.329 (for frequency offset larger than 12.5 MHz within the spurious emission domain in the frequency range 30 to 1000 MHz)

Receiver selectivity and blocking attenuation

Basic concept

The receiver captures capturing some unwanted signals outside its own channel bandwidth because its filter is not ideal.

Figure 384: Illustration of the non-ideal receiver filter with respect to an ideal 
interfering link transmitter

For generic systems, SEAMCAT provides three modes of considering the impact due to blocking (see Section 4.2):

- User-defined:the values of the blocking mask are expressed directly as a Blocking response (filter selectivity) in relative positive values (dB) (see Figure 12);

Protection ratio:
the values of the blocking mask are expressed in relative positive values (dB); 
SEAMCAT considers the underlying protection criterion applied by the measurement procedure of used in typical standards when calculating the resulting blocking response

Sensitivity:
the values of the blocking mask are expressed in absolute power levels (dBm); 
SEAMCAT considers the underlying protection criterion applied by the measurement procedure used in typical standards when calculating the resulting blocking response

For cellular systems, if blocking is spsecified as ACS (Adjacent Channel Selectivity), then it is effectively the same as the User-defined mode for generic systems.

In all cases SEAMCAT perceives receiver blocking as a mask, which is a function of frequency separation between interfering and desired signals Δf=(fILT-fVLR), however you may also specify a constant value of blocking across the whole frequency range.

Rx Bandwidth

Tx Bandwidth

Victim Link Receiver Blocking Mask

Interfering Link Transmitter Emission Mask

fVLR

fILT

Blocking

Figure 385: Blocking

When calculating the value iRSSblocking, SEAMCAT applies the obtained receiver attenuation value aVLR to the full power of the interfering signal and will consider the resulting power (interfering signal at receiver input less blocking attenuation) as in-channel noise floor increase of the VLR.

In practice, SEAMCAT integrates the interfering signal captured by the receiver, excluding the part of the interfering signal falling into the necessary bandwith of the receiver). For this integration, both the bandwidth of the interferer and the bandwidth of the victim are taken into account.

Schematic description of the iRSS_blocking algorithm

This section describes the algorithm to be used to calculate iRSS_blocking depending on the overlap between the bandwidths of VLR and ILT. There are in total 5 main cases (see sections ‎A8.2.1 to ‎A8.2.5). For the details on the implementation of the integral, see section ‎A8.2.6.

Case 1: No overlap

Figure 386:  iRSS_blocking case 1: no overlap

blocking= i=loweruppermaskValuei∙dF

where:

lower:	offset - BWILT/2 – emission mask asymmetry

upper:	lower + BWILT

dF:	step size, e.g. min{0.1;( BWILT)/20}

Case 2: Partial overlap with fILT > fVLR

Figure 387: iRSS_blocking case 2: partial overlap with fILT > fVLR

Note: the blue coloured range is covered by the iRSS_unwanted calculation and therefore not taken into account in the iRSS_blocking calculation.

blocking= i=loweruppermaskValuei∙dF

where:

lower:	BWVLR/2

upper:	offset + BWILT/2 – emission mask asymmetry

dF:	step size, e.g. min{0.1;( BWILT)/20}

Case 3: Partial overlap with fILT < fVLR

Figure 388: iRSS_blocking case 3: partial overlap with fILT < fVLR

Note: the blue coloured range is covered by the iRSS_unwanted calculation and therefore not taken into account in the iRSS_blocking calculation.

blocking= i=loweruppermaskValuei∙dF

where:

upper:	- BWVLR/2

lower:	– offset - BWILT/2 – emission mask asymmetry

dF:	step size, e.g. min{0.1;( BWILT)/20}

Case 4: Total overlap with BWVLR < BWILT

Figure 389: iRSS_blocking case 4: Total overlap with BWVLR < BWILT

Note: the blue coloured range is covered by the iRSS_unwanted calculation and therefore not taken into account in the iRSS_blocking calculation.

blocking= i=lowerBWVLR2<i<-BWVLR2uppermaskValuei∙dF

where:

lower:	offset - BWILT/2 – emission mask asymmetry

upper:	lower + BWILT

dF:	step size, e.g. min{0.1;BWVLR/5}  1)

excluding the range fVLR ± BWVLR/2

Note that at least 3 samples are required for the range fVLR ± BWVLR.

Case 5: total overlap with BWVLR > BWILT

Figure 390: iRSS_blocking case 5: total overlap with BWVLR > BWILT

In this case, there is no blocking as the interference is completely covered by the unwanted calculation. The blocking attenuation is therefore set to 1000 dB.

Implementation of the integral for iRSS_blocking

The integral requires positive values of the blocking mask, i.e. the blocking mode has to be considered first.

ablocking= -10*log10f=lowern∙dF10-maskValuef10BWILT∙dF

where:

n=rangedF

and

range=upper-lower

The parameter maskValue(f) corresponds to the following equations (see section ‎1.4.6).

For generic systems in user-defined mode, it corresponds to the mask specified in the receiver settings;

- maskValue(f) = BlockUD (dB).

For generic systems defined with blocking modes different to user-defined (i.e. sensitivity and protection ratio);

Protection Ratio (dB):

- maskValue(f) = BlockPR (dB) + C/(N+I) (dB) + (N+I)/N (dB) - I/N (dB)

Sensitivity Mode (dBm):

- maskValue(f)  = BlockSens (dBm) – SensitivityVLR (dBm) + C/(N+I) (dB) – I/N (dB)

For cellular systems:

If the mask is defined in positive values (ACS), then maskValue(f) corresponds to the mask specified in the receiver settings;

If the mask is defined in negative values, then  maskValue(f) corresponds to the calculated positive mask using the input parameters Standard Desensitisation and I/N_target (see section ‎A8.2.8).

Considerations on the bandwidths used in the integral

Calculation of BW for GENERIC systems

The algorithm is based on the mask entries and takes the values as relative to the carrier (dBc) at the zero offset.
It consists of two loops, one for the upper part of the mask, the second for the lower part.

The loops stop if the relative power of the actual offset is less than the limit .

The lower and upper bound are saved as they are required by the calculation algorithm so that the blocking average takes account of symmetrical emission masks.

Calculation of BW for OFDMA

For a Base Station: the total number of resource blocks (max. RBs per BS) multiplied by the bandwidth of one resource block.

For a User Equipment: the bandwidth is: the number of RBs per MS (UE) multiplied by the bandwidth or one resource block.

Calculation of BW for CDMA

The algorithm uses the reference bandwidth of the system, which is the same for either BS or UE (user defined in CDMA general settings). .

Conversion of negative blocking mask values in cellular receivers (OFDMA and CDMA)

In case of receivers of cellular systems (OFDMA and CDMA), if the mask has negative values, it needs to be converted  using the following input parameters in the receiver settings:

Standard desensitisation (for which the blocking mask values where derived);

Target I/N  (which depends on the scenario considered and can be extracted from the CEPT/ECC or ITU-R deliverables ).

- If the mask is entered as positive values then these parameters are not used.

Figure 391: Input parameters for cellular receivers blocking masks

Default values of standard desensitisation and Target I/N are specified in the table below.

Table 68: Default values of standard desensitisation and Target I/N

|  | Standard Desensitisation (dB) | Target I/N (dB) |
| --- | --- | --- |
| OFDMA UL | +6 | -6 |
| OFDMA DL | +6 | -6 |
| CDMA UL | +6 | -6 |
| CDMA DL | +3 | -6 |

The algorithm to convert negative blocking mask values is as follows:

Blocking Response = IOOB_target – Noise floor – I/Ntarget

where:

I/Ntarget is always understood as ‘target I/N of the victim’, which is calculated for D_target (target desensitisation)

Noise floor = 10*log(kTB) + F.

K= Boltzmann Constant

T = Noise temperature (Kelvin)

B = Receiver bandwidth in MHz

F = Noise Figure (dB)

IOOB_target  is calculated as follows :

IOOB-TARGET = IOOB-STANDARD – DSTANDARD + DTARGET

where:

IOOB-STANDARD = is the original blocking mask;

DSTANDARD = is the standard desensitisation for which the blocking mask values (IOOB-STANDARD) were derived. It is an input of cellular receiver settings (see Figure 391).

DTARGET = 10*log10(10^(I/N_target/10)+1) ; where I/N_target is an input of cellular receiver settings.

Finally we obtain:

Blocking Response = IOOB-TARGET – Noise floor– I/Ntarget, which means:

Blocking Response = IOOB-STANDARD – DSTANDARD + DTARGET – kTBF – I/Ntarget

User-defined mode

In this case, the receiver attenuation is a direct input with positive values in dB. Therefore SEAMCAT will take the receiver attenuation aVLR(f) values directly from the function (might be also constant value) that you entered.

aVLR(f) = blockUse defined (f) (dB) 				(Eq. 123)

where f = (fILT - fVLR)

fv

Receiver Mask

fI

Rx bandwidth

Rejection

of the receiver

Blocking Response = filtering (user defined mode)

Blocking:

0 dB

Attenuation (dB)

Figure 392: Illustration of blocking and the associated Blocking Response in a user defined mode

Figure 393: Setting up the blocking response in the user-defined mode

Protection ratio

In this case, SEAMCAT takes the positive values of the blocking mask and considers the underlying measurement procedure (see for instance ETSI EN 300 113 (section 9 therein)) by this calculation:

aVLR∆f=blockprotectionratio∆f+CN+I+N+IN-I/N			(Eq. 124)

where 
		blockprotectionratio 		the positive values in dB from the mask (2)
		C/(N+I), (N+I)/N and I/N 		the input parameters of the VLR

Figure 394: Setting up the blocking response in the Protection ratio mode

Sensitivity

Similar to the Protection ratio mode, SEAMCAT calculates the resulting blocking response by

aVLR∆f=blocksensitivity∆f-sensitivityVLR+CN+I-I/N		(Eq. 125)

where 
	blocksensitivity 		the absolute power limit in (mostly negative) dBm from the mask (2)
	sensitivityVLR 		the input parameters of the VLR (3)
	C/(N+I) and I/N 		the input parameters of the VLR.

Figure 395: Setting up the blocking response in the sensitivity mode

Blocking level measurements

To calculate the receiver attenuation in blocking response modes, the fundamental relationships between receiver sensitivity, wanted signal, unwanted signal (i.e. the interfering signal) and noise floor need to be established, based on the principle of measuring blocking as a level of adjacent interfering signal which results in reduction of output power from modulated useful signal by a certain limit.

This limit of acceptable wanted signal degradation is assumed to be 3 dB, based on suggestion in ITU-R SM.332-4 and the blocking measurement procedure described in ETSI EN 300 113. The latter procedure is briefly summarised as follows (see Figure 396):

The wanted signal (Generator A) is switched on, tuned at receiver frequency and its level is adjusted to the level of maximum useable sensitivety of the receiver, which corresponds to BER=10-2 at the receiver output. The level of wanted signal is then further increased by 3 dB;

The unwanted signal is added by switching on Generator B, with given frequency shift from receiver frequency, and its output level is first increased until a BER=10-1 or worse is obtained; then decreased again until the BER=10-2 is re-attained at the receiver output;

The corresponding level of unwanted signal is noted. The “blocking” ratio is then expressed as ratio, in dB, of the level of unwanted signal to the level of wanted signal;

Test is repeated for other frequency shifts f.

Figure 396: Blocking measurement arrangement (from ETSI EN 300 113)

Receiver attenuation as element of blocking response

Based on the definition and measurement procedure of blocking, it is possible to obtain the receiver attenuation parameter aVLR(f) from the following fundamental relationships between wanted and unwanted signals, receiver sensitivity and noise, as observed when blocking event is triggered:

Figure 397: Relationship between signal levels when blocking occurs 
(Protection Ratio and Sensitivity modes)

Note: the coloured arrows show the relationship of the input parameters used for the calculation of the resulting blocking response (aVLR(∆f)) and described in the sections above.

Examples:

VLR 	noise floor:		-114 dBm

sensitivity:		-98 dBm

C/(N+I):			16 dB

I/N:			-6 dB

(N+I)/N			1 dB

a)	blocking mode "Sensitivity"
	limit:			-45 dBm

aVLR∆f=limit-sensitivity+CN+I-IN

aVLR∆f=-45 dBm--98dBm+16 dB--6dB

aVLR∆f=75 dB

b)	blocking mode "Protection ratio"

limit:			52 dB

aVLR∆f=limit+N+IN+CN+I-IN

aVLR∆f=52 dB+1 dB+16 dB-(-6)dB

aVLR∆f=75 dB

CDMA / OFDMA simulation: multiple ACS

Editors note: this section may be reviewed due to future updates in the implementation

As pointed out on the introduction to this Annex, the implemented algorithm for the calculation of the impact due to blocking supposes a constant (flat) shape of the blocking mask over the complete bandwidth of the interfering system. This approach might cause inaccuracies, in particular in case of larger bandwidths of the ILT.

When simulating cellular environment like LTE, the 3GPP standard provides different ACS values depending on the bandwidth of the interferer. For instance, it is known that LTE can support flexible channel bandwidth, 1.4MHz, 3MHz, 5MHz, 10MHz, 15MHz and 20MHz. Therefore, in order to be able to deal with more LTE co-existence scenarios in the future, it is unavoidable to support scenarios with different bandwidth interferers.

SEAMCAT is basically able to handle such a case. You may define scenario specific blocking masks, that means for each combination of VLR and ILT frequency allocations separate masks. Some guidance is given by e.g. ETSI TR 136 942 V12.0.0 (2014-10) on section 12.1.3.2. You should exercise caution and not mixing the ACIR (Adjacent Channel Interference Ratio) the standard is talking about with the blocking values required for the SEAMCAT masks.

ACS mask definition for interfers with different bandwidths

Let’s assume the following example of two interferers with two distinct bandwidths (10MHz and 5 MHz) which results in two ACS values. For this you would need to introduce an ACS “mask” so that it is defined at the specific interference frequencies for various interfering bandwidth and not to a same bandwidth. You need to define the ACS to the left for 10 MHz using the approximation of 3GPP TR36.942 (p75) and you can define the ACS for 5 MHz in the right hand side.

Figure 398: ACS mask examples for two interferers with different bandwidth

In the case where two interferers are geographically separated but transmitting at the same frequency, you can trick SEAMCAT by slightly shifting one of the frequency (a few kHz is enough) so that the value for a specific frequency is used in the simulation (see below).

Figure 399: ACS mask definition (Case where two interferers are geographically separated but transmitting at the same frequency)

The only situation, where it would be difficult (but still feasible) is when the interfering frequency is distributed over a large bandwidth, then you would have to define the “other” ACS value elsewhere in the frequency domain and you may have 1‐2 dB difference due to the propagation models (exact value to be assessed of course).

Calculating the average ACS value

According to 3GPP TR36.942 p75, there are some example calculations of average ACS.

1100.1×ACSaverage=140×5100.1×ACS1+5100.1×ACS2+30100.1×ACS3 		(Eq. 126)

Figure 400: Calculating the average ACS value

: Examples of deriving parameter values from radio standards

This annex provides some examples of how certain parameter values may be derived from relevant parameter values for use in SEAMCAT simulations can be derived from relevant 3GPP and ETSI standards. The examples are not exhaustive and consider different parameters.

Blocking levels in 3GPP and ETSI specifications

SINR, ACS, blocking levels or intermodulation thresholds are defined by 3GPP/ETSI for a given desensitisation. These values are used for compliance testing to the specifications.

As an example, in GSM, tests related to blocking are performed with a desensitisation of 20 dB for the two first adjacent channels, and 3 dB for the following channels. In UMTS and LTE, the following table applies.

|  | 1st adjacent 5 MHz block | Following adjacent 5 MHz blocks | Adjacent narrowband interferer |
| --- | --- | --- | --- |
| UMTS BS | 6 dB | 6 dB | 6 dB |
| LTE BS | 6 dB | 6 dB | 6 dB |
| UMTS UE | 14 dB | 3 dB | 10 dB |
| LTE UE | 14 dB | 6 to 9 dB | 13 to 16 dB8 |

Table 69: Desensitisation considered for testing ACS and blocking levels

It can be noted that the desensitisation considered for testing varies with the frequency offset, while the network is designed for a specific value. In section 7.5.1 of 3GPP TS 25 101 ‎[5] and TS 36 101 ‎[1] related to the UE, the desensitisation value is explicitly mentioned as a testing parameter.

Blocking level determination methodology

The blocking parameters of a standard are specified for specific values used for compliance testing, which do not always correspond to typical operational values. In particular, blocking values are specified for desensitisation values which may not correspond to the desired operational value for specific studies (e.g. 1 dB for BS and 3 dB for UE). It is therefore sometimes necessary to translate the Blocking Level IOOB-STANDARD defined by the standard for a desensitisation DSTANDARD into the corresponding Blocking Level IOOB-TARGET for another desensitisation DTARGET.

Coexistence analysis are conducted for a standard that provides a maximum power (Blocking Level) of an interfering signal received outside of the in-band, for a given frequency offset between the wanted signal and the interfering signal. This absolute blocking level IOOB-STANDARD is specified in the standard for a specific desensitisation DSTANDARD. This is the case for the blocking specifications in UMTS and LTE standards (3GPP TS 25 101 ‎[5], TS 25 104 ‎[3], TS 36 101 ‎[1] and TS 36 104 ‎[2]). It is also the case for the blocking specifications for the 3rd adjacent channel and the following channels in GSM (3GPP TS 45 005 ‎[21]).

For studies considering C/(N+I) protection ratio, an example of blocking specifications for the first 2 adjacent channels can be found in the GSM standard (3GPP TS 45 005 ‎[21]).

Derivation of blocking response from a noise floor rise

When an interfering signal IOOB is applied to the RF input of a receiver outside of the receiver’s bandwidth, the receiver will be interfered with due to the imperfect selectivity of the receiver’s filter. The receiver’s filter attenuates the interfering signal IOOB into an ‘equivalent in-band interfering signal’ IIB.

The receiver Blocking Response is defined as the receiver filter attenuation of signals outside of receiver’s channel/band (in dB):

Blocking Response = IIB - IOOB

The present section derives the receiver Blocking Response from the Blocking Level specified in the standard.

Starting from:

DSTANDARD = 10.log10[(iIB-STANDARD + n)/n]				(Eq. 127)

IIB-STANDARD can be derived from the following equation in dB:

IIB-STANDARD = N + 10.log10[10^(DSTANDARD/10) - 1]				(Eq. 128)

noting that iIB / n = [(iIB + n) / n] - 1 = 10D/10 - 1

The blocking response is then derived by the following equation:

Blocking Response = IIB-STANDARD - IOOB-STANDARD 	(Eq. 129)

= N + 10.log10[10^(DSTANDARD/10) - 1] - IOOB-STANDARD		(Eq. 130)

From the Blocking Response, it is possible to derive the receiver Blocking Level for another value of desensitisation.

In order to derive this new receiver Blocking Level, we assume that the Blocking Response is constant and fully linear over the complete range of desensitisation from 0 to DSTANDARD.

IIB-TARGET can be derived from the following equation:

IIB-TARGET = N + 10.log10[10^(DTARGET/10) - 1]				(Eq. 131)

IOOB-TARGET can be derived from the following equation:

IOOB-TARGET = IIB-TARGET - Blocking Response						(Eq. 132)

IOOB-TARGET = 10.log10[10^(DTARGET/10) - 1] - 10.log10[10^(DSTANDARD/10) - 1] + IOOB-STANDARD	(Eq. 133)

IOOB-TARGET = I/NTARGET – I/NSTANDARD + IOOB-STANDARD					(Eq. 134)

Derivation from a protection ratio

This is the case for GSM, with regards to the 2 first adjacent channels.

In 3GPP TS 45 005 ‎[21], Table 6.3-1 in section 6.3, protection ratios are specified in dB for a desensitisation DSTANDARD of 20 dB (see section 6.1).

- Co-channel, [C/Ic]dB = C - Ic = 9 dB

- First adjacent channel, [C/Ia1]dB = C - Ia1 = -9 dB

- Second adjacent channel, [C/Ia2]dB = C - Ia2 = -41 dB

These are the “blocking protection ratios”.

The Blocking Response is given by the following equation:

Blocking Response	= IIB-STANDARD - IOOB-STANDARD						(Eq. 135)

= (C - IOOB-STANDARD) - (C - IIB-STANDARD) 						(Eq. 136)

= (C - Ia) - (C - Ic)

IOOB-TARGET can then be derived from the following equation:

IOOB-TARGET = N + 10.log10[10^(DTARGET/10) - 1] + (C - Ic) - (C - Ia)					(Eq. 137)

IOOB-TARGET = N + I/NTARGET + (C - Ic) - (C - Ia) 							(Eq. 138)

Fixed service parameters

Recommendation ITU-R F.758 ‎[22] provides fixed service charactersitics for sharing with aircraft-mounted FSS Earth stations in the 27.5-29.5 GHz band.. As specified in the Recommendation (Annex 2, Section 4), the system parameters providedare not representative of any actual FS system, but represent an averaging or an expected range of values suitable for generic sharing/compatibility studies.

Stations FS5 and FS6 in the Table 70 Error! Reference source not found.below have parameters in line with the Rec. ITU-R F.758 ‎[22]. . FS1 and FS3 stations are representative of typically deployed PP FS stations. FS2 is representative of a longer than typical link operating mountainous terrain and hence an elevation angle of 5 degrees is appropriate, . FS4 is representative of a P-MP Hub station. P-MP subscriber stations are expected to have parameters similar to those valid for PP stations.

Antenna radiation patterns for FS-PP are usually assumed in sharing studies using the formulas in Recommendation ITU-R F.699 ‎[23] (when a single FS system is considered as victim/interferer) or Recommendation ITU-R F.1245 ‎[24] (when averaging among a number of FS systems as victim/interferer). For P-MP it is recommended to use Recommendation ITU-R F.1336 ‎[25].

Table 70: Recommended FS characteristics for FS vs FSS studies

| Parameter | FS1 (PP) | FS2 (PP) | FS3 (PP) | FS4 (P-MP) | FS5 (PP based on Rec. ITU-R 758-5) | FS6 (P-MP based on Rec. ITU-R 758-5) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RX antenna height | 20 m | 20m | 20m | 20m | Not specified, 20m is suggested | Not specified, 20m is suggested |  |
| RX antenna pattern | ITU-R Rec. F.1245 | ITU-R Rec. F.1245 | ITU-R Rec. F. 1245 | ITU-R Rec. F.1336 | ITU-R Rec. F. 1245 | ITU-R Rec. F.1336 | Source: Rec F.758-5 (doc 5/BL/24) |
| Receiver noise figure, F | 6 dB | 6 dB | 6 dB | 6 dB | 8 dB | 8 dB | See note |
| RX frequency | 28 GHz | 28 GHz | 28 GHz | 28 GHz | 24.25 – 29.50 GHz | 24.25 – 29.50 GHz |  |
| RX elevation angle | 0° | 5° | 10° | 0° | Not specified, 10° is suggested | Not specified, 0° is suggested |  |
| RX peak gain | 45 dBi | 43 dBi | 35 dBi | 18 dBi |  | 6.5 dBi | See note |

Table 71: Recommended FS characteristics for FS vs FSS studies

The Spectrum Mask for P-P and P-MP can be found in ETSI EN 302 217-2-2 ‎[26] Annex A (table A4) and EN 302 326-2 (Section 5.3.4.1) respectively. Blocking information is available from EN 301 390 (Section 7).

With respect to antenna height for P-P, the BS and TS minimum antenna height are 20 m and for P-MP, the BS minimum antenna height is 20 m and the TS minimum antenna height is 1.5 m.

For the antenna elevation, some countries have reported an elevation range of 3 to 10 degrees.  In rare cases a range of maximum elevation angle from 15 to 30 degrees has been reported.

ECC Report 100 uses an I/N ≤ 10 dB; I/N ≤ 6 dB is mentioned only for the protection of ENG/OB. FS systems (P-P and P-MP) commonly adopt I/N ≤ 10 dB in line with Rec. ITU-R F.758 ‎[22]

Short range devices parameters

This section presents a non exhaustive list of the main standards for short range devices where technical parameters may be found.

EN 300 220 ‎[28]: Short Range Devices (SRD); Radio equipment to be used in the 25 MHz to 1 000 MHz; frequency range with power levels ranging up to 500 mW;

EN 300 422 ‎[29]: Wireless microphones in the 25 MHz to 3 GHz frequency range; (Assistive Listing Devices (ALD) standard)

EN 300 328 ‎[30]: Wideband transmission systems; Data transmission equipment operating in the 2,4 GHz ISM band and using wide band modulation techniques; (WLAN standard)

EN 300 440 ‎[31]: Short range devices; Radio equipment to be used in the 1 GHz to 40 GHz frequency range; Part 1: Technical characteristics and test methods

Distribution

Figure 401: Distribution dialog box

The trials performed on all of the SEAMCAT distributions are based upon using internal Java pseudo-random number generator. All the below distributions are presented with their equations, working range, parameters and ouput result. They can all be tested using the “test distribution” feature (see Section ‎2.18).

Constant distribution

In this case a trial performed on this variable always returns the same constant value (may be integer or floating point)

User-defined distribution

It is a continuous distribution defined by its cumulative distribution function (a distribution with values associated with probability), entered as pairs (x, y=F(x)):

(Eq. 139)

The definition range of the input Prob. is between 0 and 1.

The example of Figure 402 returns frequency that will operate at 901.031 MHz, for another snapshot it will be 902.636MHz, for another one 905.893 MHz etc…

Figure 402: User defined distribution: parameters (left) PDF (right)

Uniform distribution

It represents a continuous uniform distribution, with given min and max values, and all intermediate values having equal probability:

(Eq. 140)

This example returns frequencies that will operate at 900.937 MHz, for another snapshot it will be 904.916 MHz, etc…

Figure 403: Uniform distribution: parameters (left) PDF (right)

The continuous nature of this function results in that the trial returns double floating point number within the range (min, max) with the same probability (0,1). This means that this function is not suitable e.g. to define frequency hopping pattern, since the latter would usually occur within a set of (pre-defined) discrete channels. Such cases can be modelled using Discrete uniform function described below.

Gaussian distribution

It is an ordinary Gaussian (Normal) distribution defined by mean m and standard deviation σ values:

(Eq. 141)

Figure 404: Gaussian distribution: parameters (left) PDF (right)

Rayleigh distribution

It is an ordinary Rayleigh distribution defined via its min and standard deviation σ values:

(Eq. 142)

Figure 405: Rayleigh distribution: parameters (left) PDF (right)

Uniform polar distance distribution

Uniform polar distance is a distribution function designed to define a random positioning of transmitter along the radius of coverage cell R (i.e. all the points are distributed over a circular area defined by max distance with the same probability), to achieve a random uniform distribution of transmitters within a circular area centred around a given zero-point.

This function has one parameter - max distance - and probability of distribution along that distance is defined as:

(Eq. 143)

The uniform polar distance distribution is typically used for deriving distance factor used in calculation of the relative locations of transceivers within a link and between victim and interfering links. The result of the trial on such a distribution, the distance factor, is then multiplied by a coverage radius or simulation radius. Hence the default maximum value of R is set to 1, meaning that after multiplication of this random factor with the radius value, the resulting distance will be distributed uniformly along the entire coverage/simulation radius.

Note that this is equivalent to defining a uniform density of transmitters on a circular area together with uniform polar angle (with 360 deg) or uniform distribution in the angle domain (0,360 deg).

Figure 406: Uniform polar distance distribution: parameters (left) PDF (right)

Uniform polar angle distribution

The uniform polar angle is to be used along with “Uniform polar distance”, this function is designed to describe a uniform distribution of transmitters within a circular area centred around a given zero-point. But whereas “Uniform polar distance” describes random distance to centre point, the “Uniform polar angle” function defines random angle (azimuth) of transmitter with regards to centre point.

This function has one input parameter - maximum angle Amax - and the generated random values will be placed with equal probability (uniform distrbution function) within the range -Amax...Amax.

This distribution is similar to uniform distribution but in the angle domain.

Figure 407: Uniform polar angle distribution: parameters (left) PDF (right)

User-defined (stair) distribution

The user-defined (stair) is the discrete alternative of continuous User-defined function described above. The Stair function is defined through a discrete set of values Xi, i = 1…N and associated probabilities S(Xi) . This is the pair of set (Xi, S(Xi)) where Xi represents all possible values that might be assigned to the variable, whereas S(Xi) represents their cumulative probabilities:

(Eq. 144)

Such a distribution will be entered in form of a list of couples (Xi, S(Xi)). Entering such a distribution in cumulated form allow to control that the sum of the probabilities is equal to 1.

The definition range of the input Cum. Prob. (S(Xi)) is (0, 1)

This example returns frequencies that will operate at 904 MHz, for another snapshot it will be 902 MHz, etc…

Figure 408: User defined (stair): parameters (left) PDF (right)

Discrete uniform distribution

The discrete uniform is the discrete alternative of the Uniform distribution described above. The Discrete uniform distribution is defined by the following parameters:

Lower bound Xmin of the distribution;

Upper bound Xmax of the distribution;

Step S (e.g. channel spacing in the case of frequency distributions);

Step shift Ss (e.g. to set the step values to the centre frequency of the channel).

Figure 409: Discrete uniform distribution: parameters (left) CDF (right)

As a result, the generated discrete random parameter will be taking the following values:

(Eq. 145)

each value being assigned the same probability:

(Eq. 146)

with i=1...N

(Eq. 147)

The following examples, with a step shift of 0 and a step of 0.2, returns frequencies that will operate at 1000 MHz, for another snapshot it will be 1000.2 MHz, etc… the maximum value will be 1001 MHz

Figure 410: Discrete Uniform (with step 0)

Adding a step shift of 0.1 and keeping a step of 0.2, returns frequencies that will operate at 1000.1 MHz, 1000.3 MHz etc… with a maximum value of 1000.9 MHz.

Figure 411: Discrete Uniform (with step 0.1)

: Antenna

Graphical interfaces

For generic systems, the antenna modelling consists of two elements: pointing and pattern definition.. The “Antenna Pointing” panel defines the antenna height,  and the pointing in azimuth and in elevation. The “Antenna Patterns definition” panel as illustrated in Figure 412 is where the pattern across the full range of angles is defined.

The antenna pointing azimuth and elevation angles determine the ultimate antenna gain applied in calculation of the link budget, but not the mutual physical positioning of thetransmitter and receiver pair. The latter is determined by the “Path azimuth angle” parameter (see ‎ANNEX 12:).

  - (b)

Figure 412: Antenna pointing (a) and antenna patterns definition (b) 
defining the antenna parameters in a generic system

For cellular systems, the antenna is defined in the mobile or base station panel as shown in Figure 413.

  - (b)

Figure 413: Mobile station panel (a) and base station panel (b)
 defining the antenna parameters in a cellular system

Antenna height

The reference point for 'height of the antenna’ should correspond to its definition in the selected propagation model for that link (e.g. it might be the height above ground, effective height or height above local clutter).

antenna pointing azimuth reference  and antenna azimuth (within a link)

For the generic system, an antenna can be set so it is pointing or not at its corresponding transmitter or receiver in the azimuth plane, plus an additional angle offset:

The antenna pointing azimuth reference: There are two options to set the reference of the antenna pointing (). The antennas can either be “pointing”, i.e. the reference direction of the main beam is towards the Rx or the Tx, or fixed, i.e. the reference direction of main beam is by default towards EAST. The control of the pointing of the antenna is contained within the link (transmitter/receiver). The principle is illustrated for the victim system in the graphic of Figure 415 and also applies to the interfering system.

The antenna azimuth  ( ).: This distribution allows the antenna to be steered to a certain angle with respect to the antenna pointing azimuth referenceNote that an offset can be applied whether the pointing option is selected or not – the pointing only determines the reference (0 degree)point.

A positive value is equivalent to counter-clockwise to the 0 degree reference

Figure 414: Azimuth angle convention

(Eq. 148)

Where p is the pointing option.

Unless intentional fixed pointing is foreseen in a simulation scenario, it is recommended to always select antenna pointing and set the azimuth angle to const=0, meaning that the main beam of the antenna is directed towards the corresponding link element (transmitter or receiver).

| Pointing | Fixed |
| --- | --- |
| In this example, the VLR antenna is pointing towards the VLT by default (red arrow). You can then set a specific aziumuth angle in order to steer the antenna away from the default reference. It is set to 0 in the illustration. This parameter may be used either to introduce random pointing of antennas (e.g. in a mobile environment), or to show a deliberate pointing offset (e.g. choosing a particular gain point on the antenna directivity pattern). In this case the antenna are pointing at each other. | In this configuration, the VLR "antenna azimuth" is set to 10 degrees, which results in the main beam to be shifted from the EAST direction accordingly. In this case the antennas are not pointing at each other - and the VLR is pointing 10 degrees from EAST. |

Figure 415: Illustration of pointing options in the azimuth domain, from the VLR

Note that differences in antenna height can have an impact on the effective azimuth angle, and thus an additional correction factor is necessary, as outlined in the following section.

antenna pointing elevation reference  and antenna elevation (within a link)

For the generic system, an antenna can be set so that it is pointing or not at its corresponding transmitter or receiver ,plus an additional  a tilt angle:

- The antenna pointing elevation reference () follows the same concept as for "antenna pointing azimuth". When selected, the antennas are pointing toward the corresponding link element (transmitter or receiver). If it is not selected, it means that the antenna reference is fixed to the horizontal plane.

- The antenna elevation ( ).: This distribution allows you to steer the antenna to a certain angle with respect to the antenna pointing elevation reference. Adding the antenna elevation component allows simulation of a real life antenna down-tilt or up-tilt This feature is useful to simulate beamforming..

A positive value is equivalent to counter-clockwise to the 0 degree reference, that is to say: positive = up-tilt to the 0 degree reference (i.e. towards the sky, such as an aeronautical or satellite link) and negative = down-tilt to the 0 degree reference (i.e. towards the ground, such as in a mobile base station).

Figure 416: Elevation angle convention

(Eq. 149)

(Eq. 150)

There is also a resulting impact on the azimuth due to the change in elevation:

(Eq. 151)

Unless intentional fixed pointing is foreseen in a simulation scenario, it is recommended to always select antenna pointing and set the elevation angle to const=0, meaning that the main beam of the antenna is directed towards the corresponding link element (transmitter or receiver).

| Pointing | Fixed |
| --- | --- |
| In this case the antennas are pointing (i.e. tilted) at each other. | In this case the antennas are not tilted at each other, i.e. their 0 degree reference is the horizontal plane. |

Figure 417: Illustration of the pointing options in the elevation domain

For the cellular system the antenna tilt is defined at the base station as described in Section ‎7.5.5 and illustrated in Figure 418. It is equivalent to a physical tilt of an antenna on a mast, (-) sign is a downtilt, (+) sign is an uptilt.

Figure 418: Illustration of the antenna tilt cellular system

Antenna azimuths and elevations (ILT - VLR path)

For the Interfering Transmitter (ILT) - Victim Receiver (VLR) path, mutual angles and subsequent antenna discrimination angles/azimuths in the horizontal plane and tilts/elevations in the vertical plane are pre-determined by calculations already performed for the relative azimuth / elevation on the victim (VLT-VLR) and interferer (ILT-ILR) paths and the the relative positioning of the VLR with ILT.

For all angle calculations, the azimuth reference is the ILT-VLR path and the elevation reference is the horizontal plane.

For the case of tilted antennas on either the ILT or VLR, additional corrections to the azimuth and elevation are necessary, as outlined below.

Calculation of antenna azimuths for untilted antennas (ILT - VLR path)

The antenna azimuth (bearing, or discrimination angle) in the horizontal plane from the VLR in the direction of the ILT for the case of no tilt () is derived from:

- direction of the victim link (i.e. the relative angle between VLR and VLT),

- direction of the VLR-ILT path. This direction may also be defined by XVLR, YVLR, XILT, YILT, absolute coordinates of the VLR and ILT.

- azimuth of the VLR antenna in the direction of the VLT,

through the following relationship:

(Eq. 152)

(Eq. 153)

Similarly, the azimuth (in the horizontal plane) from the ILT in the direction of the VLR for the case of no tilt () is derived from:

- direction of the interfering link (i.e. the relative angle between ILT and ILR),

- direction of the VLR-ILT path. This direction may also be defined by Xvlr, Yvlr, Xilt, Yilt, absolute coordinates of the VLR and ILT,

- azimuth of the ILT antenna in the direction of the ILR,

through the following relationship:

(Eq. 154)

(Eq. 155)

Calculation of antenna elevations for untilted antennas (ILT - VLR path)

The elevation of the VLR antenna in the direction of the ILT antenna for the case of no tilt () is derived as follows:

(Eq. 156)

where:

φ is the elevation angle of the ILT antenna as seen from the VLR antenna.

Similarly, the elevation angle of the ILT antenna in the direction of the VLR antenna for the case of no tilt () is derived as follows:

(Eq. 157)

where:

φ is the angle to the VLR antenna as seen from the ILT antenna.

Calculation of antenna azimuths and elevations for tilted antennas (ILT - VLR path)

The application of mechanical tilt to either antenna results in an additional correction to both the elevation and azimuth angles as calculated above for the untilted case as follows.

VLR to ILT path:

(Eq. 158)

(Eq. 159)

ILT to VLR path:

(Eq. 160)

(Eq. 161)

These corrections are based on the geometrical derivation in Annex 5 of Recommendation ITU-R F.1336-4 ‎[25], and apply for the general case of any mechanically tilted antenna.

Antenna Patterns definition

- Figure 419: Antenna pattern identication dialog box

Table 72: Description of the Antenna pattern user interface in SEAMCAT

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Library | - | Library | - | Allows importing/exporting of antennas from/to the library to/from the workspace |
| Name: name of the Antenna |  |  |  |  |
| Description: comments on the antenna |  |  |  |  |
| Antenna peak gain | gmax | Scalar | dBi | Describes peak antenna gain, i.e. gain in the direction of maximum radiation (main lobe) |
| Horizontal patterns: Horizontal normalized antenna pattern | gH () | Function (X,Y) | dB/deg | Defined by pairs (φ, gH(φ)), where φ (deg, range 0...+360) describes the angular offset in horizontal plane with relation to the direction of maximum radiation, and gH(φ) describes the relative gain, related to the antenna maximum peak gain. Note: gH(φ) values should be negative. Note: The antenna pattern range definition for CDMA has been modified to match the antenna patterns specified by 3GPP (deg, range -180...+180) |
| Vertical patterns: Vertical normalized antenna pattern | gV (φ) | Function (X,Y) | dB/deg | Input angle values between –90O and 90O. For the gain, only input negative values relative to the Antenna peak gain. |
| Spherical patterns: Spherical normalized antenna pattern |  |  |  | It describes the antenna spherical radiation pattern, defined by pairs (Φ, gS(Φ)), where Φ (deg, range 0...+180) describes the spherical angle, and gS(Φ) describes the relative gain, related to the antenna maximum peak gain. gS(Φ) values are negative and relative to the Antenna peak gain: cosϕ=cosθcosφ |

To define the horizontal, vertical or spherical antenna patterns, the dialogue window is similar to those used to enter the function dialogue windows. However an important difference is that antenna pattern dialogue windows do not allow setting Constant values, and they have two displays to present the entered data - the usual XY plane, and the Polar display. These are illustrated by two respective pictures below.

Figure 420: Antenna pattern dialog box

Figure 421: Defining antenna pattern, using XY coordinates system

Figure 422: Defining antenna pattern, using Polar coordinates system

By default all antennas have 0 dBi gain and are non-directional, i.e. no associated directivity pattern. The antenna pattern can be defined by modifying the antenna gain and activating one or more of radiation patterns depending on the requirements of the scenario. For example,  it may be sufficient to activate only the horizontal pattern, but if the vertical discrimination is important in the scenario, both vertical and horizontal patterns could be activated.

Figure 423: Horizontal, vertical and spehrical antenna pattern convention

Antenna Gain

Most of the built-in antennas assume that the frequency has no impact on the antenna efficiency/gain, therefore antenna gains are calculated based on pure peak gain combined with necessary angular discrimination in case of directional antennas (i.e. antennas with defined radiation patterns).

If no spherical antenna pattern is defined:

The calculation of the antenna gain for a path between trx1 and trx2 is performed as follows:

Gain in horizontal plane: if a horizontal antenna pattern is defined, then

(Eq. 162)

Gain in vertical plane: if a vertical antenna pattern is defined, then:

(Eq. 163)

When the two antenna patterns (horizontal and vertical) are selected, this means that the horizontal and vertical planes are combined and SEAMCAT calculates the resulting antenna gain as a pseudo 3-D antenna gain based on multiplication in linear domain of the horizontal and vertical gains:

(Eq. 164)

(Eq. 165)

If a spherical antenna pattern is defined:

The spherical-angle gain is calculated as follows:

(Eq. 166)

The total gain is then calculated as follows:

(Eq. 167)

Antenna library

See Section ‎13.5.1

: Relative positioning

Overview

The relative position between two points (ILT/ILR/VLT/VLR) is described by its Distance, Azimuth and Delta X,Y when an uncorrelated mode is selected , or only by Delta X, Y when the correlated mode is selected. The selection of this mode and input parameters are described in Section ‎5.4.1 (VLT-VLR or ILT-ILR) and Section ‎10.3.1 (Interfering link to victim link path).

Within a generic system, the position of the receiver is always specified relative to the transmitter. For the relation between the interfering link and victim link, in a generic system the user may select the reference component as either the ILT or ILR, relative to either the VLT or VLR. This is illustrated below:

Figure 424: Relative positioning combinations between 2 generic systems

For a cellular system as victim or interferer the base station of the reference cell is always used to specify the relative positioning, as illustrated below:

Figure 425: Relative positioning combinations between a cellular interferer and a generic victim

Figure 426: Relative positioning combinations between a generic interferer and a cellular victim

The notation in this annex refers to positioning between a transmitter and receiver (Tx and Rx), but the same equations apply for any of the other positioning combinations outlined above.

  - Calculation of position for correlated mode

When a correlated mode is selected, the distance between 2 elements is determined purely through the Delta X and Delta Y parameters:

XRx=XTx+∆X						(Eq. 168)

YRx=YTx+∆Y						(Eq. 169)

where:	(XTx, YTx) is the trialled Cartesian position of the Tx
	∆X is the trialled Delta X value
	∆Y is the trialled Delta Y value
	(XRx, YRx) is the resulting Cartesian position of the Rx

  - Calculation of position for uncorrelated modeS

When an uncorrelated mode is selected, the distance between 2 elements is determined by distance and azimuth parameters, in addition to Delta X and Delta Y:

XRx=XTx+∆X+DpathRmaxcos⁡(∝)			(Eq. 170)

YRx=YTx+∆Y+DpathRmaxsin⁡(∝)			(Eq. 171)

where:	Dpathis the trialled path distance factor
	Rmaxis the coverage radius in system links, or the simulaton radius in the interference path
	∝ is the trialled path azimuth

Reference point for the VLTto VLR and ILT to ILR position

For the path azimuths on the VLT->VLR and ILT->ILR links, the reference point is a transmitter (the VLT or ILT) and the angle is from the positive direction of the X axis to the path towards a corresponding receiver (VLR or ILR, respectively) in a link. As an example if you select a uniform path azimuth distribution between 0 and 90 degrees (and a path distance factor set to Uniform polar dist. with a max. distance of 1), you will get the following distributions:

| Victim Link System: | Interfering Link System |
| --- | --- |
| VLT VLRi VLRj | ILT ILRi ILRj |
| (a) | (b) |

Figure 427: Illustration of the reference point for the VLT-VLR (a) and ILT-ILR (b)

Example 1: “Pacman”

| Parameters | Scenario outline results |
| --- | --- |
| Path Distance factor: Uniform Polar distance distribution with Max. distance: 1 Path azimuth for ILT - ILR: Uniform distribution with Min: -135 deg and Max: 115 deg | Figure 428: A “Pacman” interfering link |

Example 2: Hexagon:

You are also able to select a polygon shape as an alternative to the default circle. A various selection of polygon is available (see Section ‎5.4.1). You are able to rotate counter-clock wise (ccw) the polygon shape.

| Parameters | Scenario outline results |
| --- | --- |
| Path Distance factor: Uniform Polar distance distribution with Max. distance: 1 Path azimuth for ILT - ILR: Uniform. distribution Min: 0 ; Max: 360 Use a polygon : True. Shape of the polygon: Hexagon Turn CCW: 0 deg | Figure 429: Interfering link system with hexagonal shape |

Reference point for the interfering link to Victim link position

For the path azimuth used in the relative victim link to interfering link positioning, the reference component can be set by the user to be the ILT or the ILR, and the relative component (“Position relative to” parameter) can be set to the VLT or VLR The angle between them  is calculated between the positive direction of X axis and the path towards the reference compeont.

Example 1:

In this case the ILTis positioned with respect to the VLT with a constant path azimuth of 45° and a simulation radius of 1 km we should expect the following distribution (the path distance factor is set to 1, and there is no delta offset, so the distance is equal to 1 km (path distance factor x simulation radius = 1)):

Figure 430: Positioning of the ILT with respect to the VLT

| Parameters | Scenario outline results |
| --- | --- |

Figure 431: Input parameter settings and results 
for Example 1

Example 2

In Example 1, the Δx and Δy were  set to zero. If we select an offset (Δx, Δy), the relative  position of the Interfering Link with respect to the Victim Link will shift the ILT  (red square) by Δx horizontally and Δy vertically. In the case that the ILT distribution is not a single point, an offset (Δx, Δy) will shift the centre of the ILT distribution accordingly.

Figure 432: Positioning of the ILT with respect to the VLT with a Δx and Δy offset

| Parameters | Scenario outline results |
| --- | --- |

Figure 433: Input parameter settings and results for Example 2

Example 3

In this case the interfering link is positioned with respect to the VLR with a constant path azimuth of 45 degrees and a constant path distance factor of 1, the following distribution results:

Figure 434: Positioning of the ILT with respect with VLR

| Parameters | Scenario outline results |
| --- | --- |

Table 73: Input parameter settings and SEAMCAT results 
for Example 3

Coverage and simulation radius

Coverage radius

Figure 435: Representation of the coverage radius

The parameter coverage radius  is used to describe the operational area of the victim and/or interfering systems. SEAMCAT allows three modes for defining the coverage radius.

- User Defined radius (constant value)

- Noise-limited network  (calculated at the start of events generation)

- Traffic-limited network  (calculated at the start of events generation)

The coverage radius is implemented as plugin, which means that it can be customised by the user.

User-defined coverage radius

This option allows entering the maximum radius directly where the origin of the coverage radius is the Victim Link Transmitter.

Noise-limited network

Assuming that the received power is equal to the sensitivity of the victim link receiver, then the radius Rmax can be determined for the wanted radio path by the following equation.

(Eq. 172)

where the path loss is defined by a median loss plus an additional term representing the distribution

(Eq. 173)

where:

Fmedian: propagation loss not including slow fading, i.e. path loss without variations option;

Fslowfading(X%): slow fading margin for X% coverage loss;

The distribution of  the path loss  can be expressed in a general way by the following equation:

where  is the cumulative distribution for Rmax and the resulting mean path loss  and an additional path loss  due to availability or coverage  . The availability y of the system is linked to the coverage loss through the simple relation y = 1 – x. Assuming that slow fading can be approximated by a log-normal distribution, i.e. median  mean, the relation  can be introduced, where  stands for a multiple of the standard deviation .

Here are some examples for illustration:

for 68 % coverage, b = 1;

for 95 % coverage,  = 1.96;

for 95.5 % coverage,  = 2;

for 99 % coverage,  = 2.58;

for 99.9 % coverage,  =3.29;

- The exact values can be easily determined by using the inverse Gaussian function.

(Eq. 174)

The determination of the zero of function v, is made through a recursive method such as regula-falsi used in logarithmic scale which should yield a better precision. The solution of such a method provides the following equation:

(Eq. 175)

In this case, formulas given for  need to be inverted.

Note 1: The inverse of the normalised Gaussian cumulative distribution is implemented through a piece approximation.

Note 2: Ro is to be set to 1 m (0.001 km)

Note 3: If after running a simulation it appears that the resulting coverage radius is equal or very close to the minimum distance or the maximum distance used in the calculation of the coverage radius, it is likely that there is a mistake in the values provided. This can be solved by reducing the minimum distance or increasing the minimum distance used in the calculation, so that the algorithm can calculate the corresponding coverage radius.

Note 4: When setting the Rmin, Rmax values, please observe the validity range for the selected propagation model. Otherwise SEAMCAT will produce a warning message when the simulation starts.

Traffic-limited network

The coverage radius RmaxILT (Interfering link) or  RmaxVLT  (Victim link) is determined from the following equation:

(Eq. 176)

hence:

(Eq. 177)

Coverage radius library

See Section ‎13.5.2.

Simulation radius

The parameter simulation radius Rsimu is used to describe the radius of the area where the interefering link transmitters are distributed around the victim link receiver. It is used in the different positioning modes as described in the following sections.

Figure 436: Representation of the Simulation radius

None

In the mode none, Rsimu defines the circular area where the ILTs are spread. In this case, the relative location of ILT-VLR is calculated as follows:

Trial of the path distance factor according to the scenario settings for the ILT-VLR- path;

Calculation of the absolute distance as a product of the user-defined simulation radius (constant value defined in the VLR-ILT path settings) and the above path distance factor;

Trial of the path azimuth angle according to the relevant distribution set in the scenario for ILT-VLR path. Based on that relative angle-distance information, the absolute positions could be established with reference to a known position, usually with respect to the VLR.

Uniform density mode

In the mode uniform density, Rsimu defines the circular area where the ILTs are spread. The Rsimu in the ILT-VLR path  is calculated at the very beginning of a simulation and applied in all events. It is defined as:

(Eq. 178)

densitactive  is the density of active transmitters and is defined as follows:

(Eq. 179)

where:

nactive - number of active transmitters set in the scenario for IT-VR path (note htat nactive should be sufficiently large so that the (n+1)th interferer would contribute with a negligible additional interfering power);

densit - density of transmitters, 1/km2, user-defined parameter in the table Interferers density of ILT-VLR path;

Pit - probability of transmission, user-defined parameter;

activityit(time) - user-defined temporal activity function (i.e. look-up table). The time value used for calculation is specified in the parameter time (see below).

time - parameter defining which of the values in the above activity function should be used in calculations (e.g. if the activity(time) function contains three pairs: (1;0.5), (2;0.7), (3; 0.9) and the time parameter is set to 2, then the activity(time) function will return a value 0.7).

In case a fixed simulation radius with a defined density of active users is provided, the number the nactive  users can be evaluated by using the following equation, which is derived from the above equation.

(Eq. 180)

When setting up your simulation, if the relevant input parameter is the simulation radius, it is recommended to use the mode “None” instead of the “Uniform density” mode. In that case, the activity of the transmitter may be simulated using  e.g. the parameter duty cycle, which defines a distribution of the transmit power.

If the relevant input parameter is the user density (i.e. without setting the simulation radius as input), it is recommended to use the “Uniform density” mode.

For this mode, the path distance factor is fixed to a uniform polar distance distribution with a maximum distance of 1 and is not editable.

Protection distance

If a minimum protection distance between the ILT and the VLR is introduced, then Rsimu based on the uniform density mode results  in the following equation:

(Eq. 181)

Each trial with  will be rejected and repeated for another trial producing .

Closest interferer

In this mode, the ILT is randomly placed in a circular area, the simulation radius is automatically derived from the density and activity of transmitters. The influence of the closest interferer can be estimated by having a distance dILTVLR following a Rayleigh distribution  as defined in ‎ANNEX 10: and where the parameter  is related to the density of transmitters

(Eq. 182)

where:

(Eq. 183)

For this mode, the path distance factor is fixed to a Rayleigh distribution and is not editable.

: Power control function (for non-CDMA only)

Power control in the interfering link

A power control feature is implemented within SEAMCAT for the interfering link (see example in Section ‎4.9). When this feature is activated, this introduces a power control gain for the interefering link transmitter, defined as:

gILTPC=fPCPILToutput,GILT→ILR,GILR→ILT,PLILT→ILR,PCthreshold,PCdyn,PCstep		(Eq. 184)

where:

PILToutput	: power supplied to the ILT antenna (before power control);

GILT→ILR	: ILT antenna gain in the direction of the ILR;

GILR→ILT	: ILR antenna gain in the direction of the ILT;

PLILT→ILR 	: path loss between the ILT and the ILR;

PCthreshold 	: power control receiver threshold, if the received power is lower than this threshold, then no power control takes place (dBm);

PCdyn 	: power control dynamic range (dB);

PCstep 	: steps of power control (dB).

The power received in the interfering link receiver results in:

p=PILToutput+GILT→ILR+GILR→ILT-PLILT→ILR			(Eq. 185)

It distinguished between 3 cases:

Case 1 - (Received power below power threshold):	p≤PCthreshold

gILTPC=0

Case (i+1) - (within power control limits):		PCthreshold+i-1×PCstep≤p<PCthreshold+i×PCstep

gILTPC=-i-1×PCstep

where i is an integer ranging from 1 to nstep=PCdynPCstep

Case (n_step +2) - (beyond power control limits):	p≥PCthreshold+PCdyn

gILTPC=-PCdyn

Received power dynamic range in the victim link

The received power dynamic range in the victim link is a simplified power control mechanism which ensures only that a certain dRSS is not exceeded in the victim link receiver.

The “receive power dynamic range” input parameter (denoted Pcmax in Table 12) is the maximum range of the receive power that a victim link receiver can accept, in terms of the maximum receive power over the VLR’s sensitivity threshold. This means that if the trialled dRSS value exceeds (sens+Pcmax), the dRSSmax is set to the “sensitivity” + “Receive power dynamic range”. Figure 437 (a) and (b) present an example on how to set the PC off and on respectively. Figure 438 (a) and (b) illustrates the effect of the power control in the victim link for PC off and on respectively.

| (a) | (b) |
| --- | --- |

Figure 437: Setting up the received power dynamic range for the victim link (a) PC off (b) PC on

| (a) | (b) |
| --- | --- |

Figure 438: illustration of the received power dynamic range effect on the victim link

        - PC off (b) PC on with a limit of -98 dBm = -103 dBm (sens) + 5 dB (Pcmax)

: Cellular algorithms

Cellular Network detailed information

When a scenario contains one or more CDMA networks SEAMCAT preserves the status of the simulated systems for the last event or for a specific event when using the play/replay feature (See section ‎2.16).

Cellular network

When an item is selected in the main plot area its details are shown in the details table on the right handside of the “cellular structure” tab. The values shown depend on the type of element. When no element is selected the system itself is considered to be selected and the table is as shown in Figure 439.

Figure 439: Downlink system details – see Table 74

Table 74: Downlink system details

| Name | Description |
| --- | --- |
| Link direction | Link direction of the CDMA network |
| Frequency | Carrier frequency of the CDMA network |
| Bandwidth | Bandwidth of the system |
| Cell Radius | The cell radius of the CDMA network is based on 3GPP2 and is equivalent to R in the graphic below. |
| Bitrate | Bit rate of the system. It is used to calculate the system processing gain |
| Link Level Data | Used to map the link level simulation (bit level) to the system level simulation (SIR level) |
| Total non interfered capacity | Same value as shown in the Snapshot summary. |
| Non interfered capacity per cell | Number of active UEs when the CDMA network is balanced. Note: This is the result of the optimal capacity finding simulation – and the value to extract and input to reuse the result. |
| Number of trials during capacity test | The number of trials input parameter |
| Obtained success rate | The result of optimal capacity finding for the capacity used. This can be different than the target (0.8 for downlink and target noise rise for uplink) if the algorithm has detected narrow oscillation (back and forth between the same values). Please refer to the CDMA specification for more details on this algorithm. |
| Number of ignored users (due to LLD) | Link level values are extrapolated from the values input and there is a build in hard coded stop if a UE is requiring more than half of received signal to be the wanted signal. Any UEs requiring more than this limit is automatically dropped. This number counts the number of UEs in this snapshot dropped due to this limit. |
| Total dropped users | Total number of dropped UEs (both due internal and external interference) |
| Users dropped before interference | Number of UEs dropped by power balance before the introduction of external interference. Along with the total number of dropped UEs this gives insight into the impact of external interference compared to the internal interference. |
| Number of ignored users (due to LLD) | Duplicate – will be removed. |
| Propagation model | The name of the selected propagation model for the system. |
| Maximum traffic channel proportion | This is the maximum transmit power for a single traffic channel (UE) – deducted from the maximum transmit power for the base stations and the specified maximum traffic channel fraction. |
| Noise floor | Calculated value of the Noise Floor |
| Percentage of active users in soft-handover | Linear percentage of voice active UEs who are in soft-handover. This is very dependent on the selected propagation model. |
| Percentage of dropped users in soft-handover | Linear percentage of dropped UEs in soft-handover. |

Details of voice user

When a voice active UE is clicked, the details shown in Figure 441 (DL) and Figure 442 (UL) and Table 75Table 74 is shown.

Figure 440: Details of voice active user – downlink

| Figure 441: Details of voice active user - downlink | Figure 442: Details of voice active user - uplink |
| --- | --- |

Table 75: Parameters of voice active user details Details of inactive link

| Name | Description |
| --- | --- |
| Userid | UE ID in the CDMA network |
| position | (x,y) information in km |
| status | The UE can be one of connected / dropped / Not allowed to connect. |
| Speed | Speed of the selected UE (from input distribution parameter) |
| Geometry | Calculated geometry. Please refer to CDMA specification for details. |
| Achieved Eb/No | Please refer to CDMA specification for details. |
| Required Eb/No | Extrapolated value from Link Level Data. Please refer to CDMA specification for details. |
| External Interference | The level of external interference experienced by this UE |
| Is in soft-handover | True or false |
| Total power received from inactive list | Sum of inner system interference. This is the sum of received power from all base stations to which the selected UE is NOT connected. |
| Total power received from active list | Sum of received power from connected base stations. This is includes the pilot power. |
| Traffic channel power | The sum of received traffic power from connected base stations. This is the wanted signal. |
| Distance to first cell in active set | Metric distance to the base station with the shortest distance in dB (pathloss) |
| Angle to first cell in active set | The Cartesian angle. Used when the base station antenna has a defined horizontal pattern |
| Elevation angle to first cell in active list | Only calculated and used when base station has a defined vertical pattern |
| Connected sector of first cell in active list | If tri-sector layout is used this indicates which sector the UE is connected to |
| Antenna gain from first BS in active list | Actual antenna gain to base station. |
| Multipath | Randomly selected value of 1 or 2 – used to determine Link Level Data. |
| Achieved CI | Please refer to CDMA specification for details. |
| Required Eb/No | Extrapolated value from Link Level Data. Please refer to CDMA specification for details. |
| Transmit power | The actual transmit power of this UE |

When the active UE is selected (Figure 443), CTRL and select any BS, to display the details of the link between the two (Figure 444).

| DL case | UL case |
| --- | --- |

Figure 443: inactive link

Figure 444: Details of selected link

Table 76: Parameters of selected link details

| Name | Description |
| --- | --- |
| Userid | UE ID of selected UE |
| In user active list | False or True |
| Cell id | ID of the selected base station |
| Cell location id | In tri-sector scenarios the cell id is not always equal to the cell id. The cell locations range from 1 – 19. |
| Connected sector of cell location | The connected sector of the selected base-station |
| User position | (X,Y) in km |
| Cell Position | (X,Y) in km |
| Using wrap-around | Is the link using wrap-around? Most often “long” connection are using wrap-around – but it really depends on the selected propagation model. |
| distance | The BS to UE distance |
| BS antenna gain | Calculated antenna gain for the base station |
| UE antenna gain | Antenna gain for the UE – input parameter |
| Path loss | Calculated pathloss in dB for the selected link. |
| Effective pathloss | Actual value of pathloss after adjustment for MCL |
| Horizontal angle | Angle from UE to BS |
| Vertical angle | Angle from UE to BS |
| Total transmit power | Transmit power of either base-station (downlink) or UE (uplink) |
| Total received power | Power received on this link by either base-station (uplink) or UE (downlink) |

Details of a cell

When a BS is selected, the following information is shown (Figure 445 and Figure 446 for UL and Figure 447 for DL).

Figure 445: BS selected

Figure 446: Downlink BS selected

Figure 447: Uplink BS selected

Table 77: Parameters of selected base-station

| Name | Description | Link direction |
| --- | --- | --- |
| Cell ID | ID of selected base-station | General |
| Transmit power | Transmit power of the BS | Downlink |
| Total interference | Sum of internal and external interference at the BS | Uplink |
| Number of served users | Sum of voice active and voice inactive UEs | General |
| Antenna Height | Value from the input distribution | General |
| Number of dropped users | UEs dropped from the base-station | General |
| Outage percentage | Fraction of connected UEs that were dropped | Downlink |
| Noise rise over Noise Floor | The noise rise experienced at the selected BS | Uplink |
| Position | (X,Y) in km | General |
| Pilot channel Power | Power for the pilot channel power as deducted from the pilot channel fraction and the maximum transmit power | Downlink |
| Overhead channel power | Power for the overhead channel as deducted from the overhead channel fraction and the maximum transmit power | Downlink |
| Number of active users | Number of connected UEs who has this BS as their primary link. | Downlink |
| Intersystem interference | Sum of interference from all UE’s not connected to this BS | Uplink |
| External interference, Unwanted | Unwanted part of external interference | Uplink |
| External interference, Selectivity | Selectivity part of external interference | Uplink |
| Active connections | Number users connected to this BS (primary and secondary links) | General |
| Is Reference cell | When TRUE, then it is the reference cell | General |

Example on DL CDMA “optimal capacity”

Figure 448: Non interfered capacity finding – Downlink

- Users (UEs) per cell = 20

- Number of trial = 20

- Delta number of users (UEs) per cell = 20

- Tolerance of initial outage: 5%

- Number of base stations = 19

- Run [number of trials] with 20 UEs per cell - Each “trial” consist of the following high-level steps: - Generate CDMA system as specified in scenario - Add [UE per cell] x [number of base stations] = 380 UEs - Run power balancing without adding any external interference - After power balancing note if the percentage of dropped UEs is below 5%. - Count the number of successful trials and see if it is still possible to reach the 80% target. - After 17 trials SEAMCAT concludes it is not possible to reach 80% and 20 UEs per cell is less than the specified systems optimal capacity - Adjust [UE per cell] by adding [delta UE per cell] (20 + 20 = 40) and go to step 2. - Run [number of trials] with 40 UE per cell - Same as step (a) above – but now the system is filled with 40 x 19 = 760 UE and allowed outage is 5% of 760 (38 UEs) - After running 5 trials of which none have been successful SEAMCAT determines that the system is filled with too many users - Adjust [delta UEs per cell] by dividing with 2 (20/2 = 10) - Adjust [UEs per cell] by subtracting [delta UEs per cell] (40 – 10 = 30) and go to step 3 - Run [number of trials] with 30 UEs per cell - System is filled with (30 x 19 = 570) and tolerance is 28 UEs - After running 5 trials of which none have been successful SEAMCAT determines that the system is filled with too many UEs - Adjust [delta UEs per cell] by dividing with 2 (10/2 = 5) - Adjust [UEs per cell] by subtracting [delta UEs per cell] (30 – 5 = 25) and go to step 4 - Run [number of trials] with 25 UEs per cell - System is filled with (25 x 19 = 475) and tolerance is 23 UEs - After running 10 trials 5 of them have been unsuccessful so SEAMCAT determines that the system is filled with too many UEs - Adjust [delta UEs per cell] by dividing with 2 (5/2 ~ 3) - Adjust [UEs per cell] by subtracting [delta UEs per cell] (25 – 3 = 22) and go to step 5 - Run of [number of trials] with 22 UEs per cell - System is filled with (22 x 19 = 418) and tolerance is 20 UEs - After 17 trials SEAMCAT concludes it is not possible to reach 80% and 20 UEs per cell is less than the specified systems optimal capacity - Adjust [delta UEs per cell] by dividing with 2 (3/2 ~ 2) - Adjust [UEs per cell] by adding [delta UEs per cell] (22 + 2 = 24) and go to step 6 - Run of [number of trials] with 24 UEs per cell (let’s assume that it is the end of Figure 448) - For the purpose of this example we assume that 24 UE per cell is the optimal capacity for the tested system. - After 20 trials we assume that 16 trials were successful – so optimal capacity finding is terminated and SEAMCAT starts the actual simulation.

CDMA UL – enhancement to allow cell selection

Flow chart

Figure 449: Flow chart of the CDMA UL “Cell selection” implementation (in red) 
with respect to the initial CDMA UL (in black)

Algorithm

Convention: i is the index of the cells

- Balance the power of the UE (i.e. set their Tx power) (internalPowerBalance()), the reference network capacity is calculated with the average noise rise reaching the defined threshold (i.e. targetNetworkNoiseRise input to SEAMCAT, usually 5.5 dB or 6dB) without external interference (i.e. cellNoiseRiseInitial[i] ) - Calculate the average noise rise (with external interference) in the whole network and the noise rise in each cell (i.e. cellNoiseRiseInterferer[i]) - Arrange the active users in a list so that they are sorted based on their Tx power (i.e. activeUsers) - Calculate the relative cell noise rise which is the difference between noise rise with and without external interference for that cell. (i.e. relativeCellNoiseRise[i] = - cellNoiseRiseInitial[i] + cellNoiseRiseInterferer[i]) - If the network average noise rise is above the value of noise rise averaged in a network calculated in step 1 then start the process of dropping users. - Selection of the algorithm between ‘average noise rise in a network’ or ‘ noise rise per cell’ (choice made by the SEAMCAT user) - ‘Average noise rise in a network’ selected While ‘network average noise rise’ > ‘initial network average noise rise (step1)’ - Remove user(s) from ‘activeUsers list’ (start with users having highest power level) - Re-optimize network (re-balance the power of the UEs) to achieve a new ‘network average noise rise’ value When ‘while loop’ finished, continue with step 12. - ‘Noise rise per cell selected’ Loop to identify the affected cells based on relativeCellNoiseRise[i] If - cellNoiseRiseInitial[i] + cellNoiseRiseInterferer[i] > TargetCellNoiseRise then Cell[i] is considered affected. The TargetCellNoiseRise (in dB) is an input to SEAMCAT set to a default value of 0.1 dB. You have the freedom of choosing a different value. The algorithm will select cells fulfilling above criteria and assess those cells whether to drop users from those cells or not. The default value has been chosen to ensure that the analysis does not disregard any cases of interfered cells, since users may also be dropped as the consequence of a low noise rise. A list of affected cells is created (i.e. selectedCell). - Sort the list selectedCell so that the cell with the highest relative noise rise is the first element - Start loop over the sorted list selectedCells (starting from the first element) - Extract the active users of selectedCell[i] from activeUsers into a list selectedCellActiveUsers (one list per cell) - Calculate the current new noise rise for selectedCell[i] (i.e. currentCellNoiseRise[i]). - Start removing (i.e. dropping) the active users from selectedCellActiveUsers (by default the ActiveUsers list is reduced). The UE with strongest Tx power is removed first. - Proceed with internalPowerBalance() over the remaining users for the whole network - Calculate the new noise rise for selectedCell[i] (i.e. currentCellNoiseRise[i]). If currentCellNoiseRise[i] >cellNoiseRiseInitial[i], continue until there is no more users or the cell noise rise is below or equal to cellNoiseRiseInitial[i]. Calculate the average network noise rise for output processing. If average noise rise below the targetNetworkNoiseRise exit the loop. - Outside the loop - Note that for each event the optimized number of UE is being re-calculated. - Produce: a) Save CDF of number of users in reference cell and whole network for initial optimized network (without external interference) and final optimized network b) The CDF of the capacity loss in the reference cell and in the whole network (for each event, calculate the capacity loss in %). c) The CDF of the capacity loss in the worst cellper event (the first strongest cell: selectedCell[1]). Only if ‘Noise rise per cell selected’. The cell-ID can be different from event to event but the capacity loss is to be extracted). d) The number of cells affected per event. Only if ‘Noise rise per cell selected’. e) Get vector of average network noise rise (initial optimized network (without external interference), initial optimized network (with external interference) and final optimized network)

OFDMA DL algorithm

The following describe the OFDMA DL algorithm presented in Section ‎9.7.

For i=1:# of snapshots - Distribute sufficiently many UEs randomly throughout the system area such that to each cell within the HO margin of 3 dB the same number of users is allocated as active UEs. Calculate the pathloss from each UE to all cells and find the smallest pathloss Link the UE randomly to a cell to which the pathloss is within the smallest pathloss plus the HO margin of 3 dB Select K UEs randomly from all the UEs linked to one cell as active UEs. These active UEs will be scheduled during this snapshot. Note: a full load system is assumed, namely, all available resource blocks (RBs) will be allocated to active UEs. And each UE is scheduled with the same number of RBs. Thus, the BS transmit power per UE is fixed. Let denotes the maximum transmit power of BS is the number of all available RBs in each cell is the transmit power from BS to the active UE, and . Calculate DL C/I for all active UEs in all cells. Loop over all cells from to (the number of cells in the system area e.g. 57 for 19 sites with tri-sector antennas) Loop over all active UEs from to For the -th active UE in the -th cell (i.e.) its C/I is denoted by , Determine the throughput for each UE with its C/I according to the link-to-system level mapping. Collect statistics.

OFDMA UL algorithm

The following describe the OFDMA UL algorithm presented in Section ‎9.8.

For i=1:# of snapshots - Distribute sufficiently many UEs randomly throughout the system area such that to each cell within the HO margin of 3 dB the same number of users is allocated as active UEs Calculate the pathloss from each UE to all cells and find the smallest pathloss Link the UE randomly to a cell to which the pathloss is within the smallest pathloss plus the HO margin of 3 dB Select K UEs randomly from all the UEs linked to one cell as active UEs. These active UEs will be scheduled during this snapshot Note: a full load system is assumed by default, namely, all available RBs will be allocated to active UEs. And each UE is scheduled with the same number of RBs. This behaviour can be overridden (see ‎9.3.6) and the system can be partialy loaded, each UE is still scheduled with the same number of RBs. Perform UL power control Calculate UL C/I for all active UEs in all cells. Loop over all cells from to (the number of cells in the system area e.g. 57 for 19 sites with tri-sector antennas) Loop over all active UEs from to For the -th active UE in the -th cell (i.e.) its C/I is denoted by . Determine the throughput for each UE with its C/I according to the link-to-system level mapping. Collect statistics.

Implementation of the OFDMA UL PC

Introduction

There is no power control (PC) in OFDMA DL. The only PC is in the OFDMA UL and it is implemented following the description from TR36.942 ‎[12].

Figure 450: Extract from the 3GPP TR 36.942 ‎[12]

The coupling loss (CL) is the effective path loss (Section ‎7.6.1) and is proportional to the cell size. Figure 451 illustrates an OFDMA UL for a single cell:

Figure 451: Illustration of the CL for a single cell

For one event, SEAMCAT calculates the CL for all the active UEs which are uniformly distributed over cells and the network. SEAMCAT gathers all the CL value and generate a CDF of it. The CL_xile value is taken out of this CDF for a certain percentile (xile = “power scaling threshold”) and then fed into the equation. The power scaling threshold can be interpreted as the percentage of UEs Transmitting with less than the maximum power.

Figure 452 illustrates the CDF of the CL and the meaning of the X-ile with some illustrative numbers.

Figure 452: Illustration of the CDF of the CL and the meaning of the X-ile (with illustrative numbers)

The default value of 90% was taken while calibrating the tool with respect to the TR36.942 ‎[12] example.

What does it take for a UE to transmit full power ?

Assuming gamma = 1, for the UE to transmit full power, this means that Pt = Pmax

To achieve that you need to have min (1,max(Rmin,(CL/CL-xile))) = 1

in this case max(Rmin,(CL/CL-xile)) >1

with Pmax = 24 dBm and Pmin = -30 dBm, Rmin = 3.9810717055349725E-6 (linear value), therefore (CL/CL-xile) should be higher than  1.

For (CL/CL-xile) to be higher than 1, this means CL > CL-xile

So if the CL-xile =0.5 (50 %) then you are likely to have half of the UE transmitting full power.

Increasing the CL-xile, reduces the number of occurrence (i.e. number of UE) to have UEs transmitting full power. This is equivalent to reducing the red box in Figure 453.

Figure 453: Illustration that if a UE has a CL higher than CL-xile, it transmit full power

How can I check the distribution of the UEs’ Tx power?

SEAMCAT allows you to check the OFDMA PC behaviour by extracting the Tx power of the UE for the last event.

Figure 454: OFDMA TX power output vector

ACS, ACLR and ACIR approach in SEAMCAT

SEAMCAT does not use the ACIR methodology in its calculation since it has the flexibility to evaluates the unwanted and blocking either separatey or jointly.

The ACLR as a single input is never used in any of the algorithm of SEAMCAT. The ACLR is calculated from the spectrum emission mask and only displayed as information when setting the spectrum mask of the interferer as;

The ACS is considered in the victim link receiver blocking attenuation input field.

ACIR methodology

Where an interferer transmits at a frequency that lies outside the nominal pass-band of the wanted signal, the level of interference experienced is a function of a) the interferer’s spectral leakage, as defined by its emission power spectral density, and b) the frequency response of the filtering at the receiver. These two effects can be characterised by the interferer’s adjacent-channel leakage ratio (ACLR) and the receiver’s adjacent channel selectivity (ACS) respectively, as illustrated in Figure 455.

The combination of these two parameters, in the form of

(Eq. 186)

represents the fraction of the received interferer power which is experienced as interference by the receiver, and is referred to as the adjacent-channel interference ratio (ACIR).

The ACLR of a signal is defined as the ratio of the signal’s power (nominally equal to the power over the signal’s pass-band) divided by the power of the signal when measured at the output of a (nominally rectangular) receiver filter centred on an adjacent frequency channel. By setting up the unwanted emission mask in SEAMCAT, it is possible to display directly the ACLR (facility only available in the OFDMA module version).

The ACS of a receiver is defined as the ratio of the receiver’s filter attenuation over its pass-band divided by the receiver’s filter attenuation over an adjacent frequency channel. It can be readily shown that ACIR1 = ACLR1+ ACS1. The victim link receiver blocking attenuation is used in SEAMCAT to input the ACS.

The ACIR is defined as the ratio of the power of an adjacent-channel interferer as received at the victim, divided by the interference power experienced by the victim link receiver as a result of both transmitter and receiver imperfections.

In other words, for a received interferer power at a frequency offset from the wanted signal, and for an ACIR (function of the frequency offset), the experienced interference power is reduced by the  ACIR as sketched in Figure 455.

Figure 455: Interference as a result of limited ACLR and ACS

Note: This kind of description of the adjacent frequency bands parameters for compatibility issues is widely used in the 3GPP standardization.

ACS (ACIR in DL)

In DL ACIR-1=ACLRBS-1+ACSUE-1, and the dominant part of ACIR is due to the UE frequency selectivity (ACS) (i.e. ACLRBS is very large compare to ACSUE and ACIR ≈ ACSUE. By varying the ACS of the victim link receiver, it is therefore possible to compare 3GPP methodology with SEAMCAT results.

Figure 456: Illustration of the selection of the ACS

ACLR (ACIR in UL)

The ACLR calculation is similar to the unwanted calculation BUT note that in 3GPP it is the integration of the interfering power in the adjacent channel where the bandwidth equal to the interfering emission bandwidth while the unwanted uses the victim bandwidth (See illustration from 3GPP TR36.942 ‎[12]).

Figure 457: Illustration of the ACLR for a 20 MHz E-UTRA UE aggressor ro 5 MHz E-UTRA UE victims (TR36.942 ‎[12])

In UL ACIR-1=ACLRUE-1+ACSBS-1, and the dominant part of ACIR is due to the UE adjacent channel leakage (ACLR) (i.e. ACSBS is very large compare to ACLRUE and ACIR ≈ ACLRUE. When the victim system is CDMA or OFDMA only, the spectrum mask at the interferer (whether it is a non-CDMA/OFDMA or CDMA/OFDMA module) will automatically display the ACLR.

This feature allows you to have an idea of the ACLR value with respect to the spectrum mask that you enter. This will help you in comparing external results that have been generated based on an ACIR methodology.

Figure 458: ACLR interface – default value

When the ACLR display is activated, the Interefering link transmitter Emissions Mask dialog box is composed of 3 important parts

Part #1 corresponds the usual set of the spectrum mask.

Part #2 corresponds to setting the ACLR parameters:

Adjacent channel (MHz) (default value = 1 MHz);

Emission bandwidth (MHz) (default value = 1.25 MHz);

Update display button;

Part #3 corresponds to the display of the spectrum mask and the ACLR.

When you enter the mask, the ACLR is automaticaly calculated with the default ACLR setting parameter values.

Figure 459: ACLR interface – with a spectrum mask – Step 1

For this example, assuming that the first adjacent channel starts at 4.5 MHz and the interfering emission bandwidth of the system is 9 MHz, you have to set the parameters as shown in Figure 460. The green bars represents for information the various ACLR depending on each of the adjacent channels (see Figure 461 for a zoom of the display). On the right hand side the axis represent the normalized ACLR in dB.

Figure 460: ACLR interface – with a spectrum mask – Step 2

Figure 461: Zoom of the display

ACLR for OFDMA UL as interferer

Note that for an OFDMA UL interfering system of 10 MHz, where there is 5 active users to simulate, the spectrum mask to set in SEAMCAT is defined over 2 MHz (10MHz / 5), but the ACLR is computed for the full system emission bandwidth. Therefore you would need to set:

Adjacent channel = +1 MHz (if the victim frequency is higher than the interferer) or -1 MHz (if the victim frequency is lower than the interferer).

Interfering Emission Bandwidth = 10 MHz

Usage requirement

SEAMCAT allows you to define the spectrum as part of a spectrum range which can be strictly above or below the centre offset as shown in Figure 462.

Figure 462: Portion of spectrum mask definition (i.e. excluding the center offset)

This flexibility creates a restriction in the ACLR display. It is because the ACLR display algorithm is normalized to the in-band power. Therefore, in order to generate the ACLR display it is necessary to have the spectrum mask being defined within the range of the system emission bandwidth (centered to the zero offset).

By default, in the case where a user would set the mask so that the ACLR would fail to compute, SEAMCAT will automatically disable the ACLR display functionality as shown in Figure 463.

Figure 463: ACLR display functionality disabled due to the specific definition of the spectrum

: Spectrum sensing algorithm

Overall Algorithm

Assuming ideal case, the basic proposal of a CR is as follows:

if sRSS > Detection threshold -> WSDOFF = no transmission = no interference

if sRSS < Detection threshold -> WSDON = interference calculation is possible

STEP 1: Identification of the frequencies to be tested.

It is assumed that the frequency of the interfering WSD is dependent on the frequency range defined for the victim. This means that when the CR module is activated, the interfering frequency function dialogue box is  deactivated.

M is the number of possible channels that the WSD will operates in.

One can consider following possible ways to define operating frequency for the victim device:

Victim frequency = constant

interferer frequency = Victim frequency;

M = 1

victim frequencies are discrete;

the possible WSD frequencies that you defined;

M equals the number of discrete inputs victim frequencies are distributed between fmin and fmax;

The possible WSD frequencies are from fmin+(M-1)*victim bandwidth  until  fmax (i.e. assuming that f is the center frequency). Note that (fmin+(M-1)*victim) ≤ fmax;

Extract M WSD channels.

STEP 2: Assigned the WSD with a frequency

for each events{

Note: the victim operates at a single frequency fv. Several interferers may be operating in various frequencies

if (dRSS > sensitivity){

for each WSDs  {

for channel m = 1 to M+1(i.e. over all the WSD channels){

STEP 2.1: Calculate the sRSS

Calculate the sRSS (considering the unwanted mask of the DTT) at the channel m  (i.e. WSD at fm and victim at fv) (see ‎ANNEX 6:)

STEP 2.2: Identification of “available” channels

Detect if a WSD is allowed to transmit (i.e. compare to threshold)

Store the channels that are accessible as vector available_channel

Store the channels that are not accessible as vector non_available_channel

}

STEP 3: Probability of Failure  (if activated)

Failure status (either TRUE or FALSE) of this WSD is extracted as an event from a distribution. i.e. depending of the pfailure

If this WSD has failure = TRUE, then it means that it will transmit in the victim frequency without power constraint

Update the vectors available_channel and non_available_channel so that non_available channel= 0 and available_channel=1 and is the victim frequency.

If this WSD has failure = FALSE, proceed without changing the vectors available_channel and non_available_channel

STEP 4: Select an available channel / determination of the WSD channel

if (non_available_channel < 1) (i.e. all are available){

No frequency is blocked, the max e.i.r.p. of the WSD is considered (i.e. e.i.r.p. deduced from the Tx and antenna gain of the WSD which are inputs to SEAMCAT.)

The WSD is placed randomly among the available_channel

}

else (i.e. one or more channels are blocked (i.e. non-available)){

if (all channels are blocked){

try another WSD  back to STEP 2.1 and store the number of inactive WSD

}

else (not all the channels are blocked){

if (single frequency case) (i.e. frequency = constant){

No frequency is blocked, the max e.i.r.p of the WSD is considered (i.e. e.i.r.p  deduced from the Tx and antenna gain of the WSD which are inputs to SEAMCAT.)

}

else {

Step 4.1: Apply the Table of Constraints

For each of the non_available_channel, read the associated predefined table for which the “co-channel” row is synchronised to the non-available channel. Create a new e.i.r.p table with the dimension (number of rows: number of non_available_channel, number of columns: number of available_channel).

for each of the available_channel{

- Extract the lowest  e.i.r.p from the table. This gives the vector (available channel, e.i.r.p).

Step 4.2: Determination of the WSD channel /e.i.r.p

From this vector extract the couple (fi, “max e.i.r.p”) the vector (available channel, e.i.r.p)

if several channels are associated with the same “max e.i.r.p” ,  the WSD is placed randomly among the channels.

}

}

}

}

} (end of the WSD loop)

Note: Now the active WSDs are assigned a frequency and a e.i.r.p..max.

STEP 5: Cumulate the frequency/ e.i.r.p  of all the active WSDs for each event into a single vector.

Note: The below calculation is as normal to SEAMCAT

for each active WSDs {

Extract the Tx power = e.i.r.p max - GmaxIt→VLR

Calculate iRSSunwanted (i.e. victim at fv )

Calculate iRSSblocking (i.e. victim at fv )

Calculate iRSSoverlaoding from the interfering WSD to the at the DTT victim link receiver frequency.

}

Calculate the 				(Eq. 187)

Calculate the 				(Eq. 188)

Note: If none of the WSDs are active iRSScomp = -1000 dB (by default) to allow the interference calculation.

}

else{

Skip this event

}

}

Determination of the E.I.R.Pmax in-block limit

The following presents the results of the SEAMCAT logfile, when the E.I.R.Pmax in-block limit is to be determined. For this example the following E.I.R.Pmax in-block limit is set in SEAMCAT

Figure 464: e.i.r.p.max in-block limit

The following log file is extracted and it shows the channel availability for one event and for one WSD (It no.1, WSD no. 0). In this case, this WSD would be allowed to transmit at 1000.1 and 1000.3 MHz, but it is not allowed to transmit in the 1000.5, 1000.7 and 1000.9 MHz. The Victim is placed at 1000.9 MHz.

Figure 465: Extract of the logfile

Figure 466: Illustration of the Selection process

: Propagation models

Introduction

Note on using propagation models

SEAMCAT has the means to use built-in models but also the means of programming user-defined propagation models. The plugin concept was chosen to avoid the inherent format limitations of the earlier versions of SEAMCAT interface and make the plugin model to work as fast as any built-in model.

To aid in the selection process, Table 78 provides an indication of the application ranges and typical areas of suitability of the built-in propagation models in SEAMCAT:

Table 78: Application ranges and typical areas for the various propagation models 
implemented in SEAMCAT

| Model | Annex | Frequency range | Distance range | Typical application area |
| --- | --- | --- | --- | --- |
| Free space loss | ‎A17.2 |  | LoS-limited | Fixed links and other systems/paths where direct-LoS could be assumed |
| Extended Hata | ‎A17.3 | 30 MHz - 3 GHz | Up to 40 km | Mobile services and other services working in non-LoS/cluttered environment . Note that in theory, the model can go up to 100 km since the curvature of the earth is included, but in practice it is recommended to use it up to 40 km. |
| Extended Hata-SRD | ‎A17.4 | 30 MHz - 3 GHz | Up to 300 m | Short range links under direct-LoS assumption, important: antenna heights up to 3 m |
| Spherical diffraction (ITU-R P.452) | ‎A17.5 | Above 3 GHz | Up to and beyond radio horizon | Interference on terrestrial paths in predominantly open (e.g. rural) areas |
| ITU-R P.452: ITU-R P.452-16 ITU-R P.452-14 (superseded) | ‎A17.7 ‎A17.6 (superseded) | about 0.1 GHz to 50 GHz | up to a distance limit of 10 000 km | Prediction method for the evaluation of interference between stations on the surface of the Earth at frequencies above about 0.1 GHz, accounting for clear-air interference mechanisms. |
| ITU-R P.1546: ITU-R P.1546-5 ITU-R P.1546-4 (superseded) ITU-R P.1546-1 (obsolete) | ‎A17.10 ‎A17.9 (superseded) ‎A17.7 (obsolete) | 30 MHz – 3 GHz | <1 000 km | Broadcasting and other terrestrial services, typically considered in cases with high mounted transmitter antenna (e.g. above 50-60 m). Note that ITU-R P.1546-5 is also valid for short horizontal distances smaller than 1 km. Note that Rec. ITU-R P.1546-4 proposes a solution where values of distances less than 1 km may occur and where short-range propagation model is not practicable and a simple means is required to converge towards free-space field strengths for arbitrarily short distances (see section 14 of ITU-R Rec. P.1546-4). |
| JTG5-6 | ‎A17.10 | 600 MHz - 2 GHz | >0 – 1 000 km | similar to ITU-R P.1546-4 but covering the distances below 1 km |
| Longley Rice (ITM) | ‎A17.12 | 20 MHz – 40 GHz | 1 – 2 000 km | Antenna height from 50 cm to 3 km and surface refraction from 250 to 400 N |
| IEEE 802.11 Model C | ‎A17.13 |  |  |  |
| ITU-R Rec. P. 1411 | ‎A17.14 | 300 MHz – 3 GHz | < 3 km | § 4.3 of the recommendation; for propagation between terminals located from below roof-top height to near street level |
| ITU-R Rec. P. 528 | ‎A17.15 | 125 MHz – 15.5 GHz | > 0 – 1 800 km | ground-air, ground-satellite, air-air, air-satellite, and satellite-satellite links |
| Propagation plug-in | ‎A17.1.4 | model specific | model specific | model specific |

Principal propagation mechanisms

Recommendation ITU-R P.452 presents in its overall description a summary of the principal propagation mechanisms. It explains that interference may arise through a range of propagation mechanisms whose individual dominance depends on climate, radio frequency, time percentage of interest, distance and path topography. At any one time a single mechanism or more than one may be present. The principal interference propagation mechanisms are as follows:

Line-of-sight (Figure 467): The most straightforward interference propagation situation is when a line-of-sight (LoS) transmission path exists under normal (i.e. well-mixed) atmospheric conditions. However, an additional complexity can come into play when subpath diffraction causes a slight increase in signal level above that normally expected. Also, on all but the shortest paths (i.e. paths longer than about 5 km) signal levels can often be significantly enhanced for short periods of time by multipath and focusing effects resulting from atmospheric stratification (see Figure 468).

Diffraction (Figure 467): Beyond LoS and under normal conditions, diffraction effects generally dominate wherever significant signal levels are to be found. For services where anomalous short-term problems are not important, the accuracy to which diffraction can be modelled generally determines the density of systems that can be achieved. The diffraction prediction capability must have sufficient utility to cover smooth-earth, discrete obstacle and irregular (unstructured) terrain situations.

Tropospheric scatter (Figure 467): This mechanism defines the “background” interference level for longer paths (e.g. more than 100-150 km) where the diffraction field becomes very weak. However, except for a few special cases involving sensitive receivers or very high power interferers (e.g. radar systems), interference via troposcatter will be at too low a level to be significant.

Figure 467: Long-term interference propagation mechanisms (Source: ITU-R P.452)

Surface ducting (Figure 468): This is the most important short-term propagation mechanism that can cause interference over water and in flat coastal land areas, and can give rise to high signal levels over long distances (more than 500 km over the sea). Such signals can exceed the equivalent “free-space” level under certain conditions.

Figure 468: Anomalous (short-term) interference propagation mechanisms (Source: ITU-R P.452)

Elevated layer reflection and refraction (Figure 468): The treatment of reflection and/or refraction from layers at heights up to a few hundred metres is of major importance as these mechanisms enable signals to overcome the diffraction loss of the terrain very effectively under favourable path geometry situations. Again the impact can be significant over quite long distances (up to 250-300 km).

Hydrometeor scatter (Figure 468): Hydrometeor scatter can be a potential source of interference between terrestrial link transmitters and earth stations because it may act virtually omnidirectionally, and can therefore have an impact off the great-circle interference path. However, the interfering signal levels are quite low and do not usually represent a significant problem.

Using the Extended Hata vs. P.1546 models

The most usually used SEAMCAT built-in propagation models in typical SEAMCAT scenarios are the Extended Hata model and the Rec.ITU-R P.1546 model. Therefore, when choosing between those two models, one should be aware of their similarities and differences:

The Hata model (also referred to as Okumura-Hata) was originally developed for non-Line-of-Sight (Non-LOS) paths in urban environments typical of mobile services - that is with low-height mobile terminals moving in cluttered environment.

The ITU-R P.1546 model was originally developed for broadcasting environment (at that time being the Rec. ITU-R P.370), however later, it was modified into a universal terrestrial propagation model, by approximating between the original P.370 curves and curves of other models, including those of Okumura-Hata model.

Therefore the P.1546 and Hata models may yield comparable results at distances of some 10-40 km (the typical range of Hata model) in certain range of conditions and for certain combinations of transmitter and antenna heights.

Regarding the antenna heights it is important to keep in mind that the Extended Hata and P.1546 models use different definitions of antenna heights.

The Extended Hata model assumes simply the height of transmitting antenna above ground.

The P.1546 model assumes that the height of transmitting antenna (h1, ht) is the effective height above the average height of terrain over the propagation path (taken between 3-15 km). In that regard it should be noted that SEAMCAT by itself does not have any knowledge about the actual terrain, therefore user should carefully consider the appropriate value of transmitting antenna height, following guidance in Rec. ITU-R P.1546.

On the other hand, it should be noted that the P.1546 model does not have a distinction between urban and rural propagation conditions except when calculating correction for receiving antenna height. Thus, the setting of URBAN, SUBURBAN or RURAL environment when selecting P.1546 model simply leads to an assumption of certain height of local clutter, surrounding the receiver antenna. This is then compared with the height of receiver antenna and depending on their relationship, the appropriate correction will be made. For example, if the receiver antenna height is less than the assumed height of local clutter, then the propagation loss will increase.

By default SEAMCAT assumes the height of local clutter to be 10 m for both RURAL and SUBURBAN environment and 20 m for URBAN environment. In SEAMCAT, the user is given the option to define their own local clutter height, which may be then different from the above default values. Therefore, when using P.1546 model, it is very important to consider the height of receiver antenna with regards to the assumed height of local clutter.

The figure below shows an example of how the median propagation loss calculated with P.1546 model would change depending on the height of receive antenna (h2), and how it compares with the corresponding Extended Hata model results (the curves are for 600 MHz, 50% time, 50% locations, default local clutter setting in P.1546).

Figure 469: Comparison between P1546 and Extended Hata model.

When the height of receiver antenna h2 in P.1546 is set to 1.5 m, which is much below the assumed height of local clutter (the default value of 10 dB was used in these simulations), then the propagation loss is much higher (by almost 20 dB) than the one generated with P.1546 for h2=10 m (i.e. when receiver antenna height was set equal to the height of local clutter). In the latter case, the P.1546 model predicts losses that are quite close to the results produced with the Extended Hata model, especially for shorter distances.

Propagation model plugin

The user can define very complex propagation modelling with propagation model plugin using standard Java programming language.

The propagation model plugin is intended to allow the user to create their own propagation models through a script describing the path loss calculation. This script consists of a sequence of formulas conforming to a well-defined syntax and may be edited by means of standard text editor. Further details are found in Section ‎14.4.

Free space

This model describes the theoretical minimum propagation path loss achievable in free space conditions. This model is appropriate to use on paths were unobstructed direct-Line-of-Sight propagation could be expected (e.g. point-to-point fixed service links, links over short distances in open areas, etc).

Figure 470: SEAMCAT Interface to the free space propagation model

For free space model the path loss is defined by:

(Eq. 189)

The variations correspond to the standard deviation variations.

Note: The Free space model allows the possibility to include some variations in the similar manner as it is done for all other built-in models. If user wants to use just the median theoretical value, the variations option should be disabled.

Figure 471 illustrates when the variation is disabled (i.e. no std).

Figure 471: Example of the vector events of the Free Space when Variation is disabled 
(i.e. equivalent to σ = 0 dB) active (i.e. mean = 91.52 dB)

Figure 472 presents the Free Space when Variation is activated (std = σ = 2.5 dB). The value presented in Figure 471 and Figure 472 are extracted with the following parameters: f = 900 MHz, d =  1 km, htx = hrx = 10 m.

Figure 472: Example of Free Space results when the Variation (i.e. σ = 2.5 dB) is active 
(i.e. mean = 91.52 dB)

Extended Hata model

The Hata model (also referred to as Okumura-Hata) was originally developed for non-Line-of-Sight (Non-LoS) paths in urban environments typical of mobile services - that is with low-height mobile terminals moving in cluttered environment. In the SEAMCAT propagation library, the user will find it under the name “Extended Hata”.

The Hata-SRD variation was developed in CEPT within the Project Team SE24 for studies of Short Range Devices (SRD). The basis for modification was an assumption that although SRD devices are usually operated at low antenna heights (typically person-carried devices, i.e. with antenna height of ca. 1.5 m), but the interference would usually occur at relatively short distances (up to 100 m or so) when direct-Line-of-Sight (LoS) or near-direct-LoS might be assumed. Therefore the Hata-SRD model includes adjusted expression of the parameter b(Hb), which in the standard Hata model would introduce significant additional path loss when transmitter antenna height is much less than 30 m. It was found that this additional loss is not justified in the considered SRD scenarios of short range & direct-LoS communication/coupling. In the SEAMCAT propagation library, the user will find the model under the name  “Extend Hata - SRD”

Figure 473: SEAMCAT Interface to the Extended Hata propagation model (general model on the left) and its adapted version for SRDs (on the right)

Table 79: Description of the Extended Hata and Extended Hata (SRD) models

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation |  |  |  | Variation in path loss takes into account the uncertainty of building design, furniture, room size, etc. |
| General environment |  |  |  | Environment of the propagation: urban, rural, suburban |
| Propagation environment |  |  |  | Environment of the propagation: Below roof, Above roof (used for standard deviation calculations) ONLY USED IF VARIATION OPTION IS CHECKED |
| Wall loss(indoor - indoor) |  | Scalar | dB |  |
| Wall loss std dev (indoor - indoor) |  | Scalar | dB |  |
| Loss between adjacent floor |  | Scalar | dB |  |
| Empirical parameters: | b |  |  |  |
| Size of the room | droom | Scalar | m |  |
| Height of each floor | hfloor | Scalar | m |  |

Outdoor-outdoor propagation

The Extended Hata model implemented in SEAMCAT calculates propagation loss between transmitter and receiver as:

(Eq. 190)

where:

f 	: frequency (MHz)

h1 	: transmitter antenna height (m, above ground)

h2 	: receiver antenna height (m, above ground)

d 	: distance between transmitter and receiver (km)

env 	: general environment attenuation (dB)

L 	: median path loss (dB)

: Variation in path loss (dB)

The validity ranges identified by COST 231 based on the original work of Okumura and Hata should be applied when using the Extended Hata model in SEAMCAT, i.e. Hm: 1 to 10m and Hb: 30 to 200m (Note that the Hb is assumed to be above roof top) where:

Hm = min (h1, h2)

Hb = max (h1, h2)

If Hm is below 1m, a value of 1m should be used instead. If Hb is above 200m, this might also lead to significant errors. This gives the possibility to use this model reciprocally (DL and UL) as described in Report ITU-R SM.2028-1.

Median path loss

Table 80: Description of the median path loss L depending on the distance

| Dist. Range (km) | Env. | Frequency Range (MHz) | Median path Loss (dB) |  |
| --- | --- | --- | --- | --- |
| d < 0,04 |  |  |  | (Eq. 191) |
| d 0.1 | Urban | 30 < f 150 |  | (Eq. 192) |
|  |  | 150 <f 1500 |  | (Eq. 193) |
|  |  | 1500 <f 2000 |  | (Eq. 194) |
|  |  | 2000 < f 3000 |  | (Eq. 195) |
|  | suburban |  |  | (Eq. 196) |
|  | open area |  |  | (Eq. 197) |
| 0.04 d < 0.1 |  |  |  | (Eq. 198) |

where:

(Eq. 199)

(Eq. 200)

(Eq. 201)

when L is below the free space attenuation for the same distance, the free space attenuation is used instead.

Variation in path loss

The variation in path loss is achieved by applying the log-normal distribution (slow-fading), i.e. a normal (Gaussian) distribution of logarithmic values. The relative standard deviation is given by the following equations:

Table 81: Variation in path loss

| Dist. Range (km) | Propagation mode | Standard Deviation (dB) |  |
| --- | --- | --- | --- |
|  | above roof |  | (Eq. 202) |
|  | below roof |  | (Eq. 203) |
|  | above roof |  |  |
|  | below roof |  |  |
|  | above roof |  | (Eq. 204) |
|  | below roof |  | (Eq. 205) |

Indoor-outdoor propagation

The use of the Extended Hata model for indoor-outdoor propagation introduces the following additional terms

Median path loss

(Eq. 206)

where  Lew is the attenuation due to external walls. In the case the Rx is indoor then the Lew is the wall loss defined at the Rx and when the Tx is indoor the wall loss defined at the Tx is considered.

Variation in path loss

Uncertainty on materials and relative location in the building increases the standard deviation of the log-normal distribution :

(Eq. 207)

when the Rx is indoor and Tx outdoor or

(Eq. 208)

when the Tx is indoor and Rx outdoor where and are the wall loss standard deviation defined at the Rx and Tx respectively.

Indoor-indoor propagation

The use of the Extended Hata model for indoor-indoor propagation introduces the following adjustments according to the relative location of the pair of transmitter and receiver.

Same-building condition

The first step is to estimate whether the transmitter and the receiver are located in the same building. This is done trough a statistic trial. Let us denote P the probability that the transmitter and the receiver are located in the same building. P is calculated according to the following scheme:

Table 82: Same-building probability

| Dist. Range (km) | Same Building Probability |
| --- | --- |
|  | P = 1 |
|  | P = 0 |

Transmitter and Receiver in different buildings

When transmitter and receiver are located in different buildings, the calculation mode is similar to the indoor-outdoor propagation mode but with doubled additional values.

###### Median path loss

(Eq. 209)

where

(Eq. 210)

Where  and are the attenuation due to external walls as defined in the Rx and Tx side respectively.

Variation in path loss

(Eq. 211)

Transmitter and Receiver in same building

In this latter case a specific propagation model is used:

Median path Loss

The corresponding median path loss is given by the following formula :

(Eq. 212)

with

(Eq. 213)

where :

Lwi 	: loss of wall (default 5 dB)

Lf 	: loss between adjacent floor (default 18.3 dB)

b 	: empirical parameter (default 0.46)

droom 	: size of the room (default 4 m)

hfloor 	: height of each floor (default 3 m)

h2	: height of the receiver (m)

h1	: height of the transmitter (m)

d	: the distance between the transmitter and receiver (m)

Variation in path loss

Variation in path loss is modelled as an additional log-normal distribution, in order to take into account the uncertainty of building design, furniture of the rooms, etc. Typically it is set to 10 dB.

Extended Hata (SRD)

This model is a modified version of the Extended Hata model which was developed in CEPT for studies of Short Range Devices (SRD). The basis for modification was an assumption, that although SRD devices are usually operated at low antenna heights (typically person-carried devices, i.e. with antenna height of about 1.5 m), but the interference would usually occur at relatively short distances (up to 100 m or so) when direct- or near- LoS might be assumed.

The expression of b(Hb) parameter in the Extended Hata model, giving large extra losses for transmitter antenna heights below 30 m, was considered to be unnecessarily severe. Therefore, the only difference between Extended Hata-SRD and Extended Hata model lies in the new expression for the antenna gain factor b(Hb), which for Extended Hata model is expressed as

b = min(0, 20log(Hb/30))							(Eq. 214)

to be replaced in Hata-SRD model by :

b = ( 1.1 log( f ) - 0.7 ) * min(10, Hb ) - ( 1.56 log(f ) - 0.8 ) + max( 0, 20 log( Hb / 10 ) );	(Eq. 215)

Note: This expression assumes that antenna heights should not exceed 1.5-3 m.

Spherical diffraction

A spherical propagation model based on various ITU-R Recs 452, 676 and 526 is applied in SEAMCAT for larger distances and higher frequencies

Figure 474: SEAMCAT Interface to the Spherical diffraction propagation model

Table 83: Spherical diffraction propagation model

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation |  |  |  | Variation in path loss takes into account the uncertainty of building design, furniture, room size, etc. |
| Wall loss(indoor - indoor) |  | Scalar | dB |  |
| Wall loss std dev (indoor -indoor) |  | Scalar | dB |  |
| Wall loss(indoor - outdoor) |  | Scalar | dB |  |
| Wall loss std dev (indoor - outdoor) |  | Scalar | dB |  |
| Loss between adjacent floor |  | Scalar | dB |  |
| Empirical parameters: | b |  |  |  |
| Size of the room | droom | Scalar | m |  |
| Height of each floor | hfloor | Scalar | m |  |
| Water concentration |  | Scalar | g/m2 |  |
| Earth surface admittance |  |  |  |  |
| Refraction Index gradient |  |  |  |  |
| Refraction layer probability |  |  |  |  |
| Time percentage (min) |  |  |  |  |
| Time percentage (max) |  |  |  |  |

Calculation Algorithm

According to ITU-R Rec. P.452-5 the median path loss between transmitter and receiver is given by the following equation:

(Eq. 216)

where:

is the basic loss in dB as function of the time percentage  in %

is the frequency in GHz

is the distance in km

is the diffraction loss in dB as function of the time percentage  in %

is the attenuation due to atmospheric gas and water in dB

Attenuation due to atmosphere

The attenuation due to atmosphere is given by

(Eq. 217)

where:

linear attenuation due to dry air (oxygen) in dB/km

linear attenuation in dB/km due to water as function 
of the water concentration  in g/m³, default value: 3 g/m³

Both terms can be approximated by the following equations according to ITU-R Rec. P.676-1:

- Attenuation due to water :

(Eq. 218)

for  < 350 GHz

- Attenuation due to oxygen :

57 GHz		(Eq. 219)

57 < 60 GHz

60 < 63 GHz

for > 63 GHz

Note: For simplification a linear interpolation between 57 and 63 GHz is used. The maximum is 15 dB/km for 60 GHz.

Attenuation due to diffraction

According to ITU-R Rec. P.526-2, the diffraction loss  can be derived by the received field strength  referred to the free space  :

(Eq. 220)

with:

is the normalized radio path between transmitter and receiver

is the normalized antenna height of the transmitter

is the normalized antenna height of the receiver

where:

is a parameter derived from the earth admittance factor : =1 for > 20 MHz.

is the frequency in MHz

is the equivalent earth radius in km (definition see below)

is the distance in km

is the antenna height above ground in m with =1 or 2 for the transmitter or receiver, respectively

The distance-dependent term  is given by the semi-empirical formula:

(Eq. 221)

The antenna height gain  is given by the formula set:

for >2				(Eq. 222)

for

for

for

where is the normalized earth surface admittance factor (see ITU-R Rec. P.526), default value:

All frequencies used attenuation due to diffraction have the unit MHz in contrast to the attenuation due to atmosphere where GHz is applied.

Equivalent Earth radius

The equivalent Earth radius  is considered to be dependent on the time percentage according to :

with the Earth radius factor  expressed as:

for

for

and

where:

is the mean gradient of the radio refraction profile over a 1 km layer of the atmosphere from the surface. The default value is 40 unit/km for Europe (standard atmosphere). This yields  and = 8500 km.

Note : 	The mean gradient is positive.

is the existence probability (in %) of the super-refractive layer ( 100 unit/km) in the low atmosphere. Default value: 1 % for Europe.

Note : 	The probabilities  and  are denoted in %, i.e. a range of variety: 0...100 %.

Note : 	p is set to  50%, i.e. the median with respect to time is computed. Later versions may allow to choose a certain time percentage.

Range of application

The following restrictions of the aforementioned spherical diffraction model are to be considered:

When used for outdoor-outdoor environment the model presents no variations. On the contrary for indoor-outdoor or indoor-indoor configurations, checking or un-checking the variations checkbox adds or suppresses the variable component of the corresponding corrections;

The frequency range should be larger than 3 GHz, with caution lower frequencies may be used but not below 300 MHz due to the surface admittance and polarisation effects;

The model was developed for open (rural) area. Therefore, additional attenuation due to obstacles like buildings found in suburban or urban environment is not included;

The loss due to rain is not covered;

This model is applicable only for terrestrial radio paths.

Indoor-outdoor propagation

The use of the spherical diffraction model for indoor-outdoor propagation introduces the following additional terms

Median path loss

The median path loss is increased by the attenuation due to external walls (Lew):

(Eq. 223)

Variation in path loss

Variation in path loss is already taken into account in the SD model, through the calculation of the equivalent earth radius. Thus additional variation due to uncertainty concerning materials and relative location of the transmitter and receiver is reflected by an additional lognormal distribution trial using the above defined add.

(Eq. 224)

Indoor-indoor propagation

The use of the Spherical Diffraction model for indoor-indoor propagation introduces different additional terms regard to the relative location of the transmitter and receiver.

Same-building condition

The Same-Building condition is determined in the same way as for the Extended Hata model (see section ‎A17.3).

Transmitter and Receiver in different buildings

When transmitter and receiver are located in different buildings, the calculation mode is similar to the indoor-outdoor propagation mode but with doubled additional values for both median path loss and variation path loss.

Median path loss

The median path loss is increased by the attenuation due to external walls Lew.

(Eq. 225)

Variation in path loss

(Eq. 226)

Transmitter and Receiver in same building

The calculation is the same as for the Extended Hata Model (see section ‎A17.3).

ITU-R  P.452-14

This propagation model is kept in SEAMCAT for backward compatibility of former studies, although it is formally superseded by Recommendation ITU-R P.452-16.

Description of the model

Recommendation ITU-R P.452-14 contains an interference prediction procedure for the evaluation of the available propagation loss over unwanted signal paths between stations on the surface of the Earth for frequencies above about 0.1 GHz, with losses not exceeded for time percentages over the range 0.001 ≤ p ≤ 50% and up to a distance limit of 10 000 km.

The models contained within Recommendation ITU-R P.452 work from the assumption that the interfering transmitter and the interfered-with receiver both operate within the surface layer of atmosphere. The procedure includes a complementary set of propagation models which ensure that the predictions embrace all the significant interference propagation mechanisms that can arise. Methods for analysing the radio-meteorological and topographical features of the path are provided so that predictions can be prepared for any practical interference path.

The clutter losses are designated as Aht (dB) and Ahr (dB) for the interferer and interfered-with stations respectively. The additional protection available is height dependent, and is therefore modelled by a height-gain function normalized to the nominal height of the clutter. Appropriate nominal heights are available for a range of clutter types.

The correction applies to all clear-air predictions in this Recommendation, i.e. for all propagation modes and time percentages.

A basic problem in interference prediction (which is indeed common to all tropospheric prediction procedures) is the difficulty of providing a unified consistent set of practical methods covering a wide range of distances and time percentages; i.e. for the real atmosphere in which the statistics of dominance by one mechanism merge gradually into another as meteorological and/or path conditions change. Especially in these transitional regions, a given level of signal may occur for a total time percentage which is the sum of those in different mechanisms. The approach in this procedure has been to define completely separate methods for clear-air and hydrometeor-scatter interference prediction, as described in §§ 4 and 5 respectively of this recommendation.

The clear-air method consists of separate models for diffraction, ducting/layer-reflection, and troposcatter. All three are applied for every case, irrespective of whether a path is LoS or transhorizon. The results are then combined into an overall prediction using a blending technique that ensures for any given path distance and time percentage that the signal enhancement in the equivalent notional line-of-sight model is the highest attainable.

Input parameters

Figure 475: SEAMCAT Interface to the ITU-R Rec. P.452-14 model

Table 84: ITU-R  P.452-14 propagation model

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation | - | Boolean | - | Variation in path loss. |
| Diffraction | - | Boolean | - | Select the diffraction component of the model |
| Tropospheric scatter | - | Boolean | - | Select the tropospheric scatter component of the model |
| Ducting/layer reflection | - | Boolean | - | Select the ducting/layer reflection component of the model |
| Water concentration | ρ | Scalar | g/m3 | water vapour density for tropospheric study only |
| Surface pressure | P | Scalar | hPa | Surface pressure (default 1013.25 hPa) |
| Refraction index gradient | ΔN | Scalar | 1/km | It is the average radio-refractive index lapse-rate through the lowest 1 km of the atmosphere, provides the data upon which the appropriate effective Earth radius can be calculated. Note that ΔN is a positive quantity. |
| Surface temperature | T | Scalar | Degree Celsius | Surface temperature (default 15 degree Celsius) |
| Latitude | φt, φr | Scalar | Degree | Latitude of station, used in the path centre location calculation |
| Additional clutter loss at the Tx | Aht | Scalar | dB | Additional clutter loss at the Tx |
| Additional clutter loss at the Rx | Ahr | Scalar | dB | Additional clutter loss at the Rx |
| Antenna gain at the Tx | Gt | Scalar | dB | Antenna gain of the Tx in the direction of the horizon along the great-circle interference path (dBi) |
| Antenna gain at the Rx | Gr | Scalar | dB | Antenna gain of the Rx in the direction of the horizon along the great-circle interference path (dBi) |
| Sea level surface refractivity | N0 | Scalar | - | The sea-level surface refractivity, is used only by the troposcatter model as a measure of location variability of the troposcatter scatter mechanism. As the scatter path calculation is based on a path geometry determined by annual or worst-month values of ΔN, there is no additional need for worst-month values of N0. The correct values of ΔN and N0 are given by the path-centre values as derived from the appropriate maps. |
| Time Percentage |  | Distribution | % | Percentage time variability for which the calculated basic transmission loss is not exceeded |

The longitude parameter of the Recommendation ITU-R P.452-14 is used for the computation of the position based on a grid point data file. In SEAMCAT, this parameter is not used as x and y coordinates are sufficient. The latitude, however is used in SEAMCAT in evaluation of the point incidence of anomalous propagation, β0 (%), for the path centre location where β0 (%) is the time percentage for which refractive index lapse-rates exceeding 100 N units/km can be expected in the first 100 m of the lower atmosphere. β0 (%) is used to estimate the relative incidence of fully developed anomalous propagation at the latitude under consideration.

ITU-R P.452-16

Similarly to Recommendation ITU-R P.452-14, Recommendation ITU-R P.452-16 is implemented in SEAMCAT for clear-air portion of propagation over flat (zero-height) land-only terrain profiles with clutter at the Tx and Rx sides. No coastal or sea radio-climatic zones are accounted for. The major difference between the two versions of Recommendation lies in the definition of diffraction model. Recommendation ITU-R P.452-14 uses the Deygout diffraction model that can be a discontinuous function of the receiver antenna heights. The diffraction loss in Recommendation ITU-R P.452-16 is calculated by the combination of a method based on the Bullington construction and spherical-Earth diffraction (“delta-Bullington” model) that does not have an inherent discontinuity of the Deygout model. Additionally,  Recommendation ITU-R P.676-11 (Annex 1) has been implemented for gaseous absorption calculations

Description of the model

Recommendation ITU-R P.452-16 contains an interference-prediction procedure for the evaluation of the available propagation loss over unwanted signal paths between stations on the surface of the Earth for frequencies above about 0.1 GHz to 50 GHz, with losses not exceeded for time percentages over the range 0.001 ≤ p ≤ 50% and up to a distance limit of 10 000 km.

Recommendation ITU-R P.452 considers different propagation mechanisms, such as line-of-sight, diffraction, tropospheric scatter, surface ducting, elevated layer reflection and refraction and combines them into an overall prediction in which the statistics of dominance by one mechanism merge gradually into another. SEAMCAT P.452-16 implementation allows the user to choose to compute the diffraction-only portion of the propagation loss.

SEAMCAT propagation model P.452-16 implements also clutter loss following the method of application as described in Recommendation ITU-R P.452-16: (i) the basic propagation loss is first computed for the portion of the path between the positions of Tx and Rx clutter, considering nominal clutter heights as antenna heights; (ii) the additional loss due to protection from local clutter is then added to the previously computed basic propagation loss. There is a predefined set of clutter (ground-cover) categories that the user can choose from: Suburban, Dense suburban, Urban, Dense urban, High-rise urban, and Industrial zone. The user can also specify other clutter conditions by introducing the corresponding Tx/Rx clutter nominal heights as well as the distances from Tx/Rx antennas.

Input parameters

Figure 476: SEAMCAT Interface to the ITU-R P.452-16 model

Table 85 : ITU-R P.452-16 propagation model

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation | - | Boolean | - | Variation in path loss. |
| Diffraction only | - | Boolean | - | Select to compute only the diffraction component of the propagation model |
| Water concentration | ρ | Scalar | g/m3 | Water vapour density (default 3 g/m3) |
| Surface pressure | P | Scalar | hPa | Surface pressure (default 1013.25 hPa) |
| Refraction index gradient | ΔN | Scalar | 1/km | The average radio-refractive index lapse-rate through the lowest 1 km of the atmosphere, provides the data upon which the appropriate effective Earth radius can be calculated. Note that ΔN is a positive quantity. |
| Surface temperature | T | Scalar | Degree Celsius | Surface temperature (default 15 degree Celsius) |
| Latitude |  | Scalar | Degree | Latitude of the path-centre location between Tx and Rx |
| Antenna gain at the Tx | Gt | Scalar | dB | Antenna gain of the Tx in the direction of the horizon along the great-circle interference path (dBi) |
| Antenna gain at the Rx | Gr | Scalar | dB | Antenna gain of the Rx in the direction of the horizon along the great-circle interference path (dBi) |
| Sea level surface refractivity | N0 | Scalar | - | The sea-level surface refractivity used by the troposcatter model as a measure of location variability of the troposcatter scatter mechanism. The correct values of ΔN and N0 are given by the path-centre values as derived from the appropriate maps. |
| Time Percentage | p | Distribution | % | Percentage time variability for which the basic transmission loss is not exceeded. |
| Local clutter conditions at Tx/Rx |  |  |  | Predefined set of clutter (ground-cover) categories that the user can choose from: Suburban, Dense suburban, Urban, Dense urban, High-rise urban, and Industrial zone. |
| User specified clutter nominal heights at Tx/Rx | ha_t, ha_r | Scalar | m | Clutter nominal heights specified by user if different from the predefined conditions |
| User specified clutter nominal distance from Tx/Rx | dk_t, dk_r | Scalar | m | Nominal clutter distances from Tx/Rx antenna, specified by user if different from the predefined conditions |

ITU-R P.1546-1

This propagation model is kept in SEAMCAT for backward compatibility of former studies, although it is formally superseded.

Description of the model

This is a point-to-area propagation loss model. There are three implementations of this recommendation in SEAMCAT (Recommendation ITU-R P.1546-1, ITU-R P.1546-4, and ITU-R P.1546-5 (see the next two sections)).

This recommendation replaced the original broadcasting propagation prediction curves given in Recommendation ITU-R P.370. The adjusted P.1546 model is suitable for modelling propagation path loss in the broadcasting, land mobile and certain fixed services (e.g. those employing point-to-multipoint systems) in the frequency range 30 to 3 000 MHz and for the distance range 1 km to 1 000 km.

A few simplifications have been introduced for SEAMCAT implementation of the model:

Restriction to propagation over land paths only;

Flat terrain;

No negative antenna heights.

Figure 477: SEAMCAT Interface to the ITU-R P.1546-1 propagation model

Table 86: ITU-R Recommendation P.1546-1 propagation model

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation |  |  |  | Variation in path loss |
| General environment |  |  |  | Environment of the propagation: urban, rural, suburban |
| System |  |  |  | Digital (BW<1MHz), Digital (BW>1MHz), Analogue |
| Time Percentage (min) |  |  |  | Minimum time variability from which the field strengths value are exceeded |
| Time percentage (max) |  |  |  | Maximum time variability from which the field strengths value are exceeded |
| use user specified local clutter height |  |  |  | User selectable clutter height value which overwrite any predefined value when selected |

This propagation model is described in Recommendation ITU-R P.1546-1 Annex 8, which was developed based on the prior model in Recommendation ITU-R P.370. This is a propagation model for point-to-area prediction of field strength for the broadcasting, land mobile, maritime mobile and certain fixed services (e.g. those employing point-to-multipoint systems) in the frequency range 30 to 3000 MHz and for the distance range 1 km to 1000 km. The model provided in Recommendation ITU-R P.1546-1 should be used in the frequency range (30 to 3 000 MHz) and the same distance range (1 km to 1 000 km). The Land curves are implemented only. The sea curves or the mixed sea-land path are not implemented.

Input parameters

Parameters of this propagation model are listed below:

Path dependant parameters (constant during a simulation for a given path) are:

Time percentage: 	pt (%)

Transmitter system : analogue/digital

Transmitter bandwidth : Bt

Global environment : rural, suburban, urban

Variable parameters (which vary for each event of a simulation):

Effective height of transmitter antenna (see the Rec. P.1546-1 for definition): ht (m)

Receiver antenna height (above ground, see height of local clutter in section (6) below): hr (m)

Frequency  f (MHz)

Distance d (km)

Algorithm

For calculation of the path loss according to Recommendation ITU-R P.1546-1 Annex 8 the following procedure is followed:

- Check range of application of the propagation model regarding time percentage, frequency, distance, and antenna height:

Time percentage : 			1%  pt  50 %

Frequency : 			30 MHz  f  3 000 MHz

Distance : 				0.001 km   d  1000 km

Transmitter antenna height : 	0 m  ht  3000 m

Receiver antenna height :	 	1 m  hr  3000 m

Determination of lower and higher nominal percentages ptinf and ptsup:

If t < 10 then ptinf = 1 % and ptsup= 10 % else ptinf = 10 % and ptsup= 50 %

Determination of the lower and higher nominal frequencies :

If f < 600 MHz then finf = 100 MHz and fsup= 600 MHz else finf = 600 MHz and fsup= 2000 MHz

If ht  10 m: calculate field strength E(f= f , d, ht, hr, pt) :

4.1) Calculation of the four following field strengths:

- E(f= finf , d, ht, hr, ptinf)

- E(f= fsup , d, ht, hr, ptinf)

- E(f= finf , d, ht, hr, ptsup)

- E(f= fsup , d, ht, hr, ptsup)

according to the procedure described in steps 4.1.1. to 4.1.4.

4.1.1) Calculate the dimensionless parameter k, function of the required transmitter height, ht:

(Eq. 227)

4.1.2) Determine from the following table the set of parameters a0 to a3, b0 to b7, c0 to c6 and d0 to d1 to be used according to nominal values of frequencies and time percentages:

Table 87: Parameters a0 to a3, b0 to b7, c0 to c6 and d0 to d1

| Frequency | 100 MHz |  |  | 600 MHz |  |  | 2 000 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pt % | 50 | 10 | 1 | 50 | 10 | 1 | 50 | 10 | 1 |
| a0 | 0.0814 | 0.0814 | 0.0776 | 0.0946 | 0.0913 | 0.0870 | 0.0946 | 0.0941 | 0.0918 |
| a1 | 0.761 | 0.761 | 0.726 | 0.8849 | 0.8539 | 0.8141 | 0.8849 | 0.8805 | 0.8584 |
| a2 | 30.444 | 30.444 | 29.028 | 35.399 | 34.160 | 32.567 | 35.399 | 35.222 | 34.337 |
| a3 | 90.226 | 90.226 | 90.226 | 92.778 | 92.778 | 92.778 | 94.493 | 94.493 | 94.493 |
| b0 | 33.6238 | 40.4554 | 45.577 | 51.6386 | 35.3453 | 36.8836 | 30.0051 | 25.0641 | 31.3878 |
| b1 | 10.8917 | 12.8206 | 14.6752 | 10.9877 | 15.7595 | 13.8843 | 15.4202 | 22.1011 | 15.6683 |
| b2 | 2.3311 | 2.2048 | 2.2333 | 2.2113 | 2.2252 | 2.3469 | 2.2978 | 2.3183 | 2.3941 |
| b3 | 0.4427 | 0.4761 | 0.5439 | 0.5384 | 0.5285 | 0.5246 | 0.4971 | 0.5636 | 0.5633 |
| b4 | 1.256E-7 | 7.788E-7 | 1.050E-6 | 4.323E-6 | 1.704E-7 | 5.169E-7 | 1.677E-7 | 3.126E-8 | 1.439E-7 |
| b5 | 1.775 | 1.68 | 1.65 | 1.52 | 1.76 | 1.69 | 1.762 | 1.86 | 1.77 |
| b6 | 49.39 | 41.78 | 38.02 | 49.52 | 49.06 | 46.5 | 55.21 | 54.39 | 49.18 |
| b7 | 103.01 | 94.3 | 91.77 | 97.28 | 98.93 | 101.59 | 101.89 | 101.39 | 100.39 |
| c0 | 5.4419 | 5.4877 | 4.7697 | 6.4701 | 5.8636 | 4.7453 | 6.9657 | 6.5809 | 6.0398 |
| c1 | 3.7364 | 2.4673 | 2.7487 | 2.9820 | 3.0122 | 2.9581 | 3.6532 | 3.547 | 2.5951 |
| c2 | 1.9457 | 1.7566 | 1.6797 | 1.7604 | 1.7335 | 1.9286 | 1.7658 | 1.7750 | 1.9153 |
| c3 | 1.845 | 1.9104 | 1.8793 | 1.7508 | 1.7452 | 1.7378 | 1.6268 | 1.7321 | 1.6542 |
| c4 | 415.91 | 510.08 | 343.24 | 198.33 | 216.91 | 247.68 | 114.39 | 219.54 | 186.67 |
| c5 | 0.1128 | 0.1622 | 0.2642 | 0.1432 | 0.1690 | 0.1842 | 0.1309 | 0.1704 | 0.1019 |
| c6 | 2.3538 | 2.1963 | 1.9549 | 2.2690 | 2.1985 | 2.0873 | 2.3286 | 2.1977 | 2.3954 |
| d0 | 10 | 5.5 | 3 | 5 | 5 | 8 | 8 | 8 | 8 |
| d1 | 1 | 1 | 2 | 1.2 | 1.2 | 0 | 0 | 0 | 0 |

4.1.3) Calculate the unblended to maximum value field strength, Eu, at the distance, d, and transmitting height, ht, as follows:

(Eq. 228)

where:

and:

and:

where:

where:

and:

4.1.4) Calculate the blended to the free space value of field strength, Eb, at the distance, d, and transmitting height, ht, as follows:

(Eq. 229)

where:

Efs is the free-space field strength

Efs = 106.9  20 log(d)	dB(V/m)

pbb is a blend coefficient set to value 8.

4.2) Calculation of the field strength E(f, d, ht, hr, ptinf) using log-linear interpolation in frequency range:

E = Einf + (Esup  Einf) log(f/finf)/log(fsup/finf)     (dB(V/m))		(Eq. 230)

where:

Einf:	E(f= finf , d, ht, hr, ptinf)

Esup:	E(f= fsup , d, ht, hr, ptinf)

4.3) Dual calculation for the field strength E(f, d, ht, hr, ptsup) using log-linear interpolation in frequency range:

E = Einf + (Esup  Einf) log(f/finf)/log(fsup/finf)     (dB(V/m))		(Eq. 231)

where:

Einf:	E(f= finf , d, ht, hr, ptsup)

Esup:	E(f= fsup , d, ht, hr, ptsup)

4.4) Calculation of the field strength E(f, d, ht, hr, pt) using log-linear interpolation formula in time percentage range :

E = Esup (Qinf  Qt)/(Qinf  Qsup) + Einf (Qt  Qsup)/(Qinf  Qsup)     (dB(μV/m))	(Eq. 232)

Where: (Qi (x) being the inverse complementary cumulative normal distribution function):

Qt	= Qi (pt/100)

Qinf	= Qi (ptinf/100)

Qsup	= Qi (ptsup /100)

Einf 	= E(f, d, ht, hr, ptinf)

Esup	= E(f, d, ht, hr, ptsup)

For a transmitting/base antenna height ht less than 10 m, determine the field strength for the required height and distance using following method

The procedure for extrapolating field strength at a required distance d km for values of ht in the range 0 m to 10 m is based on smooth-Earth horizon distances in km written as dH(h) = 4.1h, where h is the required value of transmitting/base antenna height ht in metres.

For d < dH(ht) the field strength is given by the 10 m height curve at its horizon distance, plus E, where E is the difference in field strengths on the 10 m height curve at distances d and the ht horizon distance.

For d  dH(ht) the field strength is given by the 10 m height curve at distance d beyond its horizon distance, where d is the difference between d and the ht horizon distance.

This may be expressed in the following formulae where E10 (d) is the field strength in dB(µV/m) calculated for transmitter antenna 10 m and for a distance d (km) according to the procedure described in step 4:

E	=	E10(dH(10)) + E10(d)  E10(dH(ht))	dB(µV/m)   d < dH(ht)

=	E10(dH(10) + d  dH(ht))	dB(µV/m)   d  dH(ht)

If in the latter equation dH(10) + d  dH(ht) exceeds 1 000 km, even though d  1 000 km, E10 may be found from linear extrapolation for log(distance) of the curve, given by:

E10	=	Einf + (Esup  Einf) log(d/Dinf)/log(Dsup/Dinf)	dB(µV/m)

where:

Dinf:		penultimate tabulation distance (km)

Dsup:		final tabulation distance (km)

Einf:		field strength at penultimate tabulation distance (dB(µV/m))

Esup:		field strength at final tabulation distance (dBµV/m))

Note: this recommendation is not valid for distances greater than 1 000 km. This method should be used only for extrapolating for ht < 10 m.

If the receiving antenna height hr is not equal to the height of representative clutter at its location (denoted R), correct the field strength as follows:

The field-strength values given by the land curves and associated tabulations in this recommendation are for a reference receiving antenna at a height, R (m), representative of the height of the ground cover surrounding the receiving/mobile antenna, subject to a minimum height value of 10 m.

SEAMCAT always assumes the height of local clutter R , depending on the propagation environment set in the model selection window:

Table 88: Default clutter height in the ITU-R P.1546-1 model (when clutter height option not activated)

| Selected environment | Assumed height of local clutter (m) |
| --- | --- |
| Rural | 10 |
| Sub-urban | 10 |
| Urban | 20 |

If the receiving antenna height, hr (m), is different from the assumed R value, as shown in Table 88 , a correction shall be added to the field strength taken from the curve.

Where the receiving antenna is adjacent to land account should first be taken of the elevation angle of the arriving ray by calculating a modified representative clutter height R' (m), given by:

R'	=	R	(m)	        for ht  6.5d + R		(Eq. 233)

= 	(1 000 d R – 15 ht)/(1 000 d  15 )	 (m) for ht > 6.5d + R		(Eq. 234)

where ht is in metres and distance d is in km.

The value of R' must be limited if necessary such that it is not less than 1 m.

When the receiving antenna is in an urban environment the correction is then given by:

Correction 	= (6.03)  J()	dB	for hr < R

= K hr log(hr /R') 	dB	for hr  R	(Eq. 235)

where J() is given by :

(Eq. 236)

where:

=	Knu hdiffθclutt

hdif	=	R'  hr	(m)

clut	=	arctan (hdif /27) (degree)

K hr	=	3.2 + 6.2 log(f)

Knu	=	0.0108 f

f	=	frequency (MHz)

Where the receiving antenna is adjacent to land in a rural environment the correction is given by the above equation (3) for all values of hr.

If the required distance is equal to or greater than d10, then again the correction for the required value of h2 should be calculated using above equation (2) with R' set to 10 m.

If the required distance is less than d10, then the correction to be added to the field strength E should be calculated using:

Correction 	= 0.0 				dB	d  d (hr)			(Eq. 237)

= (C10) log(d/dhr)/log(d10/dhr)	dB	dhr < d < d10

where:

C10:	correction for the required value of hr at distance d10 using equation (2) with R' set to 10 m,

d10:	distance at which the path just has 0.6 Fresnel clearance for hr = 10 m
calculated as D06(f, ht, 10) as given in note 2

dhr:	distance at which the path just has 0.6 Fresnel clearance for the required value of hr calculated as D06(f, ht, hr) as given in note 2

This recommendation is not valid for receiving antenna heights, hr, less than 1 m.

Add a log-normal term G(L) corresponding to the variability in the percentage of locations :

Values of standard deviation for digital systems having a bandwidth less than 1 MHz and for analogue systems are given as a function of frequency by:

L = K + 1.6 log(f)	dB		(Eq. 238)

where :

K 	= 	2.1 - for mobile systems in urban locations;

3.8 - for mobile systems in suburban locations or amongst rolling hills;

5.1 - for analogue broadcasting systems.

For digital systems having a bandwidth of 1 MHz or greater, a standard deviation of 5.5 dB should be used at all frequencies.

If necessary, limit the resulting field strength to the maximum value calculated as follows:

The field strength must not exceed a maximum value Emax given by:

EMax = Efs	dB(V/m)	for land paths

where Efs is the free space field strength for 1 kW e.r.p. given by:

Efs = 106.9  20 log(d)     (dB(V/m))	(Eq. 239)

9) Convert field strength to path loss using following formula:

Lb = 77.2 – E 20 log f     (dB)		(Eq. 240)

where:

Lb:	 basic transmission loss (dB)

E:	 field strength in dB(V/m) measured with a transmitting power of 1 W e.i.r.p.

f:	 frequency (MHz).

Note 1: The following approximation to the inverse complementary cumulative normal distribution function, Qi(x), is valid for 0.01  x  0.99 :

Qi(x) = T(x) –(x) 	if x  0.5

Qi(x) = -{T(1-x) – (1-x)}	if x > 0.5

where:

C0 = 2.515517

C1 = 0.802853

C2 = 0.010328

D1 = 1.432788

D2 = 0.189269

D3 = 0.001308

Note 2: the path length which just achieves a clearance of 0.6 of the first Fresnel zone over a smooth curved Earth, for a given frequency and antenna heights ht and hr, is given approximately by:

where:

Df = 		(km)	(frequency-dependent term)

= 		km

Dh = 		             (km)	(asymptotic term defined by horizon distances)

= 		km

f:	frequency (MHz)

ht, hr:	antenna heights above smooth Earth (m)

In the above equations, the value of ht must be limited, if necessary, such that it is not less than zero. Moreover, the resulting values of D06 must be limited, if necessary, such that it is not less than 0.001 km.

Note 3: the case ht is less than zero described in the recommendation is not handled.

Note 4: no correction due to terrain clearance angle is implemented.

ITU-R P.1546-4

Input parameters

Figure 478: SEAMCAT Interface to the ITU-R P.1546-4 propagation model

Table 89: ITU-R Recommendation P.1546-4 propagation model

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation |  |  |  | Variation in path loss |
| General environment |  |  |  | Environment of the propagation: urban, rural, suburban, dense urban |
| System |  |  |  | Mobile, broadcasting digital and broadcasting analogue |
| Time Percentage (min) |  |  |  | Minimum time variability from which the field strength values are exceeded |
| Time percentage (max) |  |  |  | Maximum time variability from which the field strength values are exceeded |
| use user specified local clutter height |  |  |  | User selectable clutter height value which overwrite any predefined value when selected |
| use user specified standard deviation |  |  |  | User selectable standard deviation value which overwrite any predefined value when selected |
| Area of location variability |  |  |  | 500x500 m, < 2 km radius, < 50 km radius |
| Terminal designations |  |  |  | When selected the algorithm will consider the option a), b) and c) of ITU-R. 1546-4 Annex 5 §1.1. By default it is selected. |

This propagation model is fully described in Recommendation ITU-R P.1546-4. Only the land path is implemented. The sea path and the mixed sea/land path are not implemented. The ranges of application are the same as for the P1546-1.

Algorithm

The terminal designations

This propagation model is not reciprocal with respect to designations of the transmitting/base station and the receiver/mobile station/terminal. It is primarily intended for use with broadcasting and mobile services where the transmitter/base antenna is above the level of local clutter.

When it is used to calculate the coverage of, or for the coordination of, broadcasting and/or base-to-mobile stations, then the actual transmitting/base station should be treated as the “transmitting/base”. For this the SEAMCAT user has to deselect the “Terminal designation” switch.

In other cases, whch is the default setup in SEAMCAT, where there is no a priori reason to consider either terminal as the transmitting/base, then the selection of which terminal to designate as the transmitting/base station for the purposes of this propagation model can be as follows:

- if both terminals are below the levels of clutter in their respective vicinities, then this model will not give accurate predictions to the problem at hand. Users should seek guidance from other, more appropriate, ITU-R Recommendations. Note that SEAMCAT will prompt you with a warning message;

- if one terminal is in an open location or above the surrounding clutter, whereas the other terminal is below the level of clutter, then the open/uncluttered terminal should be treated as the transmitting/base station for the purposes of actual identity as the transmitter/base or receiver/mobile;

- if both terminals are open/uncluttered, then the terminal with the greater effective height should be treated as the transmitting/base station for the purposes of this Recommendation.

As noted above, this model is not reciprocal. However, in some special cases, it may be treated as such. For these particular cases, for example, which may be encountered for the monitoring and prediction of receiver/mobile-to-transmitter/base coverage and/or interference, it may be useful, under items b) and c) of the previous discussion, to designate that terminal in the open location, which should also always have the higher effective height, as the transmitter/base as the “high” antenna, whilst designating the terminal in the cluttered location as the receiver/mobile as the “low” antenna, without regard to which terminal is actually the transmitter/base and receiver/mobile. Users of this model should note, for the purposes of calculation, that if these “high” and “low” designations are to be used, the high antenna termination will always be synonymous with (and equivalent to) the transmitter/base station’s effective height, h1, whilst the low antenna termination will always be synonymous with (and equivalent to) the receiver/mobile’s height, h2, with the same qualifier, in the subsequent calculation of field strength or basic transmission loss.

Maximum field-strength values

The field strength must not exceed a maximum value Emax as described in ITU-R P.1546-1. In principle any correction which increases a field strength must not be allowed to produce values greater than these limits for the family of curves and distance concerned. However, limitation to maximum values should be applied only where indicated in Annex 6 of the ITU-R P.1546-4.

Determination of transmitting/base antenna height, h1

The transmitting/base antenna height, h1, to be used in the calculation depends on the type and length of the path and on various items of height information, which may not all be available.

For land paths, the effective height of the transmitting/base antenna, heff, is defined as its height in metres over the average level of the ground between distances of 3 and 15 km from the transmitting/base antenna in the direction of the receiving/mobile antenna. Where the value of effective transmitting/base antenna height, heff, is not known it should be estimated from general geographic information. This model is not valid when the transmitting/base antenna is below the height of surrounding clutter.

The value of h1 to be used in calculation should be obtained using the method given in § 3.1, 3.2 or in § 3.3 of P1546-4 as appropriate.

Land paths shorter than 15 km

For land paths less than 15 km, the following method is used:

Terrain information not available

In SEAMCAT, no terrain information is available. Therefore, the value of h1 is calculated according to path length, d, as follows:

m for d  3 km		(Eq. 241)

m for 3 km < d < 15 km

where ha is the antenna height above ground (e.g. height of the mast).

Land paths of 15 km or longer

For these paths:

m				(Eq. 242)

Transmitting/base antenna height, h1, in the range 10 m to 3 000 m

If the value of h1 coincides with one of the eight heights for which curves are provided, namely 10, 20, 37.5, 75, 150, 300, 600 or 1 200 m, the required field strength may be obtained directly from the plotted curves or the associated tabulations. Otherwise the required field strength should be interpolated or extrapolated from field strengths obtained from two curves using:

dB(μV/m)	(Eq. 243)

where:

hinf :	600 m if h1 > 1200 m, otherwise the nearest nominal effective height below h1

hsup :	1200 m if h1 > 1200 m, otherwise the nearest nominal effective height above h1

Einf :	field-strength value for hinf at the required distance

Esup :	field-strength value for hsup at the required distance.

The field strength resulting from extrapolation for h1 > 1200 m should be limited if necessary such that it does not exceed the maximum defined in § 2 of Recommendation ITU-R P.1546-4.

This model is not valid for h1 > 3000 m.

Transmitting/base antenna height, h1, in the range 0 m to 10 m

For a land path, the field strength at the required distance d km for 0  h1 < 10 m is calculated using:

dB(V/m)	(Eq. XXX)

where:

dB(V/m)	(Eq. YYYa)

dB	(Eq. YYYb)

Ch1neg10:	the correction Ch1 in dB calculated using equation (12) in § 4.3 of ITU-R P.1546-4 at the required distance for h1 = 10 m

E10 and E20 :	the field strengths in dB(V/m) calculated according to § 4.1 of ITU-R P.1546-4 at the required distance for h1 = 10 m and h1 = 20 m respectively..

Note that the corrections C1020 and Ch1neg10 should both evaluate to negative quantities.

Interpolation of field strength as a function of distance

Figures 1 to 24 of Recommendation ITU-R P.1546-4 show field strength plotted against distance, d, the range 1 km to 1 000 km. No interpolation for distance is needed if field strengths are read directly from these graphs. For greater precision, and for computer implementation, field strengths should be obtained from the associated tabulations (see Annex 1 of ITU-R P.1546-4, § 3). In this case, unless d coincides with one of the tabulation distances given in Table 1, the field strength, E (dB(V/m)), should be linearly interpolated for the logarithm of the distance using:

dB(V/m)	 (Eq. 244)

where:

d :	distance for which the prediction is required

dinf :	nearest tabulation distance less than d

dsup :	nearest tabulation distance greater than d

Einf :	field-strength value for dinf

Esup :	field-strength value for dsup.

This model is not valid for values of d less than 1 km or greater than 1 000 km.

Interpolation and extrapolation of field strength as a function of frequency

Field-strength values for the required frequency should be obtained by interpolating between the values for the nominal frequency values of 100, 600 and 2000 MHz. In the case of frequencies below 100 MHz or above 2 000 MHz, the interpolation must be replaced by an extrapolation from the two nearer nominal frequency values. For most paths interpolation or extrapolation for log (frequency) can be used, but for some sea paths when the required frequency is less than 100 MHz it is necessary to use an alternative method.

For land paths, and for sea paths where the required frequency is greater than 100 MHz, the required field strength, E, should be calculated using:

dB(V/m)	(Eq. 245)

where:

f :	frequency for which the prediction is required (MHz)

finf :	lower nominal frequency (100 MHz if f < 600 MHz, 600 MHz otherwise)

fsup :	higher nominal frequency (600 MHz if f < 600 MHz, 2 000 MHz otherwise)

Einf :	field-strength value for finf

Esup :	field-strength value for fsup.

The field strength resulting from extrapolation for frequency above 2 000 MHz should be limited if necessary such that it does not exceed the maximum value given in § 2 of Recommendation ITU-R P.1546-4.

For sea paths where the required frequency is less than 100 MHz an alternative method should be used, based upon the path lengths at which 0.6 of the first Fresnel zone is just clear of obstruction by the sea surface. An approximate method for calculating this distance is given in § 17 of Recommendation ITU-R P.1546-4.

Interpolation of field strength as a function of percentage time

Field-strength values for a given percentage of time between 1% and 50% time should be calculated by interpolation between the nominal values 1% and 10% or between the nominal values 10% and 50% of time using:

dB(μV/m)	 (Eq. 246)

where:

t :	percentage time for which the prediction is required

tinf :	lower nominal percentage time

tsup :	upper nominal percentage time

Qt =	Qi (t/100)

Qinf  =	Qi (tinf /100)

Qsup =	Qi (tsup /100)

Einf :	field-strength value for time percentage tinf

Esup :	field-strength value for time percentage tsup

where Qi (x) is the inverse complementary cumulative normal distribution function.

This model is valid for field strengths exceeded for percentage times in the range 1% to 50% only. Extrapolation outside the range 1% to 50% time is not valid.

A method for the calculation of Qi (x) is given in Annex 5, § 15 of Recommendation ITU-R P.1546-4.

Correction for receiving/mobile antenna height

The field-strength values given by the land curves and associated tabulations in this model are for a reference receiving/mobile antenna at a height, R (m), representative of the height of the ground cover surrounding the receiving/mobile antenna, subject to a minimum height value of 10 m. Examples of reference heights are 20 m for an urban area, 30 m for a dense urban area and 10 m for a suburban area.

Table 90: Default clutter height in ITU-R P.1546-4 model (when clutter height option not activated)

| Selected environment | Assumed height of local clutter m |
| --- | --- |
| Rural | 10 |
| Sub-urban | 10 |
| Urban | 20 |
| Dense urban | 30 |

Where the receiving/mobile antenna is on land account should first be taken of the elevation angle of the arriving ray by calculating a modified representative clutter height R' (m), given by:

m	(Eq. 247)

where h1 and R (m) and distance d (km). Note that for h1 < 6.5d + R, R′ ≈ R. The value of R' must be limited if necessary such that it is not less than 1 m.

when the receiving/mobile antenna is in an urban environment the correction is then given by:

Correction                dB	for h2 < R′		(Eq. 248)

dB	for h2 ≥ R′

where J() is given by equation (12a), and:

=

=	                   m

= 	                  degrees

=

=

f: 	frequency (MHz).

In cases in an urban environment where R' is less than 10 m, the correction should be reduced by Kh2 log(10/R′). Where the receiving/mobile antenna is on land in a rural or open environment the correction is given by equation  for all values of h2 with R' set to 10 m.

Correction for short urban/suburban paths

If a path of length less than 15 km covers buildings of uniform height over flat terrain, a correction representing the reduction of field strength due to building clutter should be added to the field strength. The correction is given by:

Correction = 	(Eq. 249)

where ha is the antenna height above ground (m) (i.e. height of the mast) and R is representative of the height of the ground cover surrounding the receiving/mobile antenna as defined in § 9 of Recommendation ITU-R P.1546-4, which also represents the height of ground cover surrounding the transmitting/base antenna. This correction only applies when d is less than 15 km and h1 − R is less than 150 m.

Variation in path loss

Values of standard deviation are dependent on frequency and environment, and empirical studies have shown a considerable spread. Representative values for areas of 500 m  500 m are given by the following expression (In SEAMCAT it is understood that the below equation only applies to the Mobile system and not to the Broadcasting systems):

dB	(Eq. 250)

where:

K =	1.2, for receivers with antennas below clutter height in urban or suburban environments for mobile systems with omnidirectional antennas at car-roof height

K =	1.0, for receivers with rooftop antennas near the clutter height

K =	0.5, for receivers in rural areas

f : 	required frequency (MHz).

In SEAMCAT depending on the selected system the standard deviation is defined as follow:

Mobile: when “< 2 km radius” and “< 50 km radius” is selected, 4 dB and 8 dB are added to the calculated standard deviation respectively. This responds to the note of the Recommendation stating that if the area over which the variability is to apply is greater than 500 m  500 m, or if the variability is to relate to all areas at a given range, rather than the variation across individual areas, the value of L will be greater. Empirical studies have suggested that location variability is increased (with respect to the small area values) by up to 4 dB for a 2 km radius and up to 8 dB for a 50 km radius.

Broadcasting, digital: the standard deviation is set to a constant value of 5.5 dB. No correction is added.

Broadcasting, analogue: The standard deviation is defined as follow and no correction is added:

dB	(Eq. 251)

In addition, SEAMCAT allows you to select a standard deviation value which overwrites any predefined value when selecting the “use user specified standard deviation” switch.

ITU-R P.1546-5

This propagation model is fully described in Recommendation ITU-R P.1546-5. Similarly to ITU-R P.1546-4, only the land terrain profiles are implemented in SEAMCAT. The ranges of application are the same as for the ITU-R P.1546-4, except for the Tx-Rx horizontal distances that can be smaller than 1 km.

Input parameters

Figure 479: SEAMCAT Interface to the ITU-R P.1546-5 propagation model

Table 91: ITU-R P.1546-5 propagation model

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation |  |  |  | Variation in path loss |
| User specified standard deviation |  | Scalar | dB | User-specified standard deviation value which overwrite any predefined value when selected |
| Time Percentage | p | Distribution | % | Time variability for which the field strength value are exceeded: 1% - 50% |
| General environment around the Rx |  |  |  | Environment of the propagation around the Rx: rural, suburban, urban, dense urban |
| Representative clutter height around the Rx | R2 | Scalar | m | User-specified clutter height at the Rx which overwrites any predefined value when selected |
| Location probability | q | Distribution | % | Location variability: 1% - 99% (default value: 50%) |
| Area of location variability |  |  |  | 500x500 m, < 2 km radius, < 50 km radius |
| System |  |  |  | Mobile, broadcasting digital and broadcasting analogue |
| General environment around the Tx |  |  |  | Environment of the propagation around the Tx: rural, suburban, urban, dense urban |
| Representative clutter height around the Tx | R1 | Scalar | m | User-specified clutter height at the Tx which overwrites any predefined value when selected |
| Average profile height around the Rx |  | Scalar | m | The average level of the terrain height between distances of 3 and 15 km from the Rx antenna in the direction of the Tx antenna. If Tx-Rx distance d is less than 15 km, then the terrain height is averaged between distances 0.2 d and d km. |
| Average profile height around the Tx |  | Scalar | m | The average level of the terrain height between distances of 3 and 15 km from the Tx antenna in the direction of the Rx antenna. If Tx-Rx distance d is less than 15 km, then the terrain height is averaged between distances 0.2 d and d km. |
| Terrain clearance angle (at the Rx) | tca | Scalar | deg | The h2 terminal's clearance angle as calculated in Section 11, noting that this is the elevation angle relative to the local horizontal |
| Terrain clearance angle (at the Tx) | eff1 | Scalar | deg | The h1 terminal’s terrain clearance angle calculated using the method in Paragraph 4.3 case a), whether or not h1 is negative |
| Terrain height above sea level at the Rx |  | Scalar | m | Terrain height in meters above sea level at the receiver/mobile |
| Terrain height above sea level at the Tx |  | Scalar | m | Terrain height in meters above sea level at the transmitter/base |
| Terminal designations |  |  |  | When selected the algorithm will consider the option a), b) and c) of ITU-R. 1546-5 Annex 5 §1.1. By default it is selected. |

Algorithm

In this section only the parts of Recommendation ITU-R P.1546-5 that are different from Recommendation ITU-R P.1546-4 are described.

The terminal designations

This Recommendation is not reciprocal with respect to designations of the transmitting/base station and the receiver/mobile station/terminal. When this Recommendation is used to calculate the coverage of, or for the coordination of, broadcasting and/or base-to-mobile stations, then the actual transmitting/base station should be treated as the “transmitting/base”. In other cases where there is no a priori reason to consider either terminal as the transmitting/base, then the selection of which terminal to designate as the transmitting/base station for the purposes of this Recommendation can be as follows:

- if both terminals are at or below the levels of clutter in their respective vicinities, then the terminal with the greater height above ground should be treated as the transmitting/base station;

- if one terminal is in an open location or above the surrounding clutter, whereas the other terminal is at or below the level of clutter, then the open/uncluttered terminal should be treated as the transmitting/base station;

- if both terminals are open/uncluttered, then the terminal with the greater effective height should be treated as the transmitting/base station.

Effect of clutter shielding Transmitting/base antenna

If the transmitting/base antenna is over or adjacent to land on which there is clutter, the correction given in Annex 5, § 10 of Recommendation ITU-R P.1546-5 should be applied, irrespective of the transmitting/base antenna height above ground. The correction should be used in all such cases, including when the antenna is above the clutter height. The correction is zero when the terminal is higher than a frequency-dependent clearance height above the clutter.

Correction                 dB	(XXa)

where J() is given by equation (12a) or (12b),

and:

= 	for R1  ha	(XXb)

=	otherwise	(XXc)

hdif1 = 	m	(XXd)

= 	degrees	(XXe)

=  		(XXf)

f: frequency (MHz).

And R1 is the height of clutter, m above ground level, in the vicinity of the transmitting/base terminal.

Correction for antenna height difference

Annex 5, § 14 of Recommendation ITU-R P.1546-5 gives a correction to account for the difference between the two antenna heights above ground. This correction is calculated as follows.

dB	(YY)

where d is the horizontal distance and the slope distance, dslope, is given as follows.

Where terrain information is available, use:

km	(YYa)

Where terrain information is not available, use:

km	(YYb)

and htter and hrter are the terrain heights in metres above sea level at the transmitter/base and receiving/mobile terminals respectively.

The hypotenuse geometry implied by equation (YYa) is unrealistic for paths long enough for Earth curvature to be significant, but for such long paths the associated error is negligible. Although the correction given by equation (YY) is very small except for short paths and high values of h1, it is recommended that it is used in all cases to avoid making an arbitrary decision as to precision.

Horizontal distances less than 1 km

The field strength curves cover horizontal distances from 1 km to 1 000 km. Annex 5, § 15 of Recommendation ITU-R P.1546-5 describes the method for horizontal distances less than 1 km. For paths less than 1 km the model is extended to arbitrarily short horizontal distances as follows:

If the horizontal distance is less than or equal to 0.04 km the field strength, E, is given by:

dB(V/m)	(ZZa)

Otherwise

dB(V/m)        (ZZb)

where:

dslope:	slope distance given by equation (YYa) or (YYb) for the horizontal distance d

dinf:	slope distance given by equation (YYa) or (YYb) for d = 0.04 km

dsup:	slope distance given by equation (YYa) or (YYb) for d = 1 km

Einf:	106.9 – 20 log (dinf)

Esup:	field strength given by §§ 1 to 14 for d = 1 km.

This extension to arbitrarily short horizontal distance is based on the assumption that as a path decreases in length below 1 km there is an increasing probability that a lower-loss path will exist passing around obstacles rather than over them. For paths of 0.04 km horizontal distance or shorter, it is assumed that line-of-sight with full Fresnel clearance exists between the terminals, and the field strength is calculated as the free-space value based on the slope distance.

If these assumptions do not fit the required short-range scenario, appropriate adjustments should be made to account for effects such as street-canyon propagation, building entry, indoor sections of path, or body effects.

This extension to short distances can allow the path to have a steep inclination, or even be vertical if ha > h2. It is important to note that the predicted field strength does not take account of the vertical radiation pattern of the transmitting/base antenna. The field strength corresponds to 1 kW e.r.p. in the direction of radiation.

JTG5-6

Description of the model

A propagation model, as developed in  JTG 5-6 (http://www.itu.int/md/R07-JTG5.6-C/en) has been implemented in SEAMCAT which combines the ITU-R P.1546, Extended Hata and Free space depending on the distance.

Interpolation ambiguity

In parallel to the development of the JTG 5-6 plugins for CPG PTD, similar activities in SE42 (CEPT project team under WGSE closed in 2010) were on-going and in the course of the development of the plugins an ambiguity was found with respect to the interpretation of the interpolation between 100 m and 1 km and the 40 m to 100 m (see details in Doc. ECC/CPG11 PT-D(09)024).

Therefore, there was a need to develop two plugins to be made available based on the following figure:

Figure 480: Interpolation ambiguity (red: original WP3K/JTG 5-6/54, green: SE42 modification)

JTG56 (based on the WP3K/JTG 5-6/54 document) where the interpolation is between the local Hata and P1546_1km and where the loss is max of (interpolated value, free space)). This is equivalent to the red curve of the illustrative figure.

JTG56 (based on SE42 modification) is the same as above but with a modification to the interpolation as agreed in SE42 where the interpolation is made between max(hata,free space) and P1546_1km and loss is directly the interpolated value. Finally the loss is max (loss, free space). This is equivalent to the green curve of the illustrative figure above.

The selection between the two implementations is presented in the GUI of the JTG5-6 below:

Figure 481: GUI of the JTG5-6

Table 92: JTG 5-6 propagation model

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation |  |  |  | Variation in path loss |
| Allow reciprocity in the antenna height |  |  |  |  |
| Use the SE42 modification (Extended Hata instead of Free space for d 0.1 km) |  |  |  |  |
| General environment |  |  |  | Environment of the propagation: rural, suburban, urban. |
| Time probability [1% or 50%] |  |  |  | time probability: either 50 % or 1% for the interfering path (using P.1546 model) |
| Cut off distance [<100m] |  |  |  |  |
| User defined local clutter height |  |  |  |  |

Assumptions on the Plugin to CPG PTD (-> JTG 5-6):

Outdoor to outdoor - Frequency : implemented to vary from 600 MHz to 2000 MHz as indicated in 3K/54.

Tx Antenna heights (H_Tx): 30 m to 200 m

Rx antenna heights (H_Rx): 1.5 m to 10 m

standard deviation (stddev): hard coded value of 5.5 dB for all distances if H_Rx > clutter height. If H_Rx < clutter height and if H_Tx < Clutter height, the stddev = 7 dB (for all the distances). The use of hard coded value means that the shadowing effect is always activated.

Note on assumptions

This plugin considers a limitation to free space in all cases (i.e. both Hata (Report ITU-R SM.2028-1) and P.1546-3 (Recommendation ITU-R P.1546-3))

In the current plugin, the correction factor in Eq. 29 of section 10 of Annex 5 of Rec ITU-R P.1546-3 has been removed. This equation is not discussed in 3K/54.

Since in Annex 7 to Document 5-6/53 the environment (i.e. Urban, Suburban and Rural) are within square brackets, it was decided to be generic in the implementation of the plugins and to include the all cases by using the same frequency correction factor as the already existing built-in SEAMCAT Extended Hata model.

The models that are used in the plugin are only for outdoor – outdoor scenarios.

.

Longley Rice (ITM)

Description of the model

ITM propagation model also known as Longley-Rice was developed to estimate radio transmission loss over irregular terrain for VHM, UHF, and SHF frequency bands. Computing of signal loss is based on electromagnetic theory and on statistical analysis. It has been complemented by empirical dependences, obtained during tests and measurements.

ITM model has two modes of working. Point-to-Point mode needs details of terrain profile. Area prediction mode  - which is described below - uses estimation of parameters based on empirical medians.

The basic input parameters such as: antennas height, frequency, and terrain irregularity determine the computing method. The algorithm can be divided into three areas (determined by a function of the distance): A) Line-of-sight range (understood as uninterrupted visibility of the bulge of the Earth Plane, but which may be obstructed by obstacles on the route), B) Diffraction range, C) Scatter range. In the first range the path loss is computed using Geometric-optics rays theory. In the second range Fresnel-Kirchoff knife-edge diffraction theory is used. In the third range, the Forward scatter theory is applied.

The Longley-Rice (Area prediction mode) propagation model was originally implemented for SEAMCAT in the  National Telecommunications Institute (Poland) in co-operation with the  Wrocław University of Technology. The task was performed under Research Project of the Polish Ministry of Science and Higher Education entitled: Next generation teleinformatics services - technology, application and market aspects (no. PBZ-MNiSW-02/II/2007).

The SEAMCAT implementation of the Longley-Rice (Irregular Terrain Model) assumes a propagation model in area prediction mode (i.e. no need for terrain profile details to make a path loss calculation).The ITM model returns predicted median attenuation of a radio signal, regarding not only statistical terrain, system, climate parameters but also variability of the signal in time and in space, assuming a given level of confidence. Longley-Rice computes path loss into three regions:

Line-of-sight range;

diffraction range;

- and

scatter range (for distances between 1 – 2000 km).

The ITM model is based on electromagnetic theory and on statistical analysis, the original model was developed in 1960’s in the USA.

At the beginning necessary geometry parameters are calculated, based on input parameters. Then, depending on the range in which the transceiver and receiver are, ITM model computes a reference attenuation Aref – the median attenuation relative to a free space. The reference attenuation is a function of distance d, what is illustrated below. The cumulative attenuation comprises reference attenuation and attenuation of quantiles A(qT, qL, qs) – which are calculated based on normal distribution, as variability of time, localization, and reliability.

Figure 482: Reference attenuation as function of distance

Working conditions

Longley-Rice model works correct in given frequency ranges, antenna heights, distance, surface refraction, as shown in Table 89.

Table 93: Range of ITM model correctly working

| Parameter | Range |
| --- | --- |
| Frequency | 20 MHz – 40 GHz |
| Antenna height | 50 cm – 3 km |
| Distance | 1 – 2 000 km |
| Surface refraction | 250 -400 N |

Input Parameters

In order to make correctly calculations using the Longley-Rice propagation, model should take into account a number of input parameters describing the weather phenomena, system parameters, sitting criteria, statistical criteria which was shown in the table below.

Figure 483: GUI of the Longley Rice

Table 94: Longley Rice propagation model

| Description | Symbol | Type | Unit | Comments |  |
| --- | --- | --- | --- | --- | --- |
| Variation | - | Boolean | - | Variation in path loss takes into account the uncertainty of building design, furniture, room size, etc. |  |
| Mean surface refractivity (Ground refractivity) | - | Scalar | Ns | Equatorial Continental Subtropical Maritime Subtropical Desert Continental Temperate Maritime Temperate over land Maritime Temperate over sea | 360 320 370 280 301 320 350 |
| Terrain irregularity parameter | - | Scalar | M | Flat Plains Hills Mountains Rugged mountains | 0 30 90 200 500 |
| Electrical ground Conductivity | - | Scalar | S/m | Average ground Poor ground Good ground Fresh water Sea water | 0.005 0.001 0.020 0.010 5 |
| Relative permittivity | - | Scalar |  | Average ground Poor ground Good ground Fresh water Sea water | 15 4 25 81 81 |
| Polarisation | - | List | - | 0 – horizontal, 1- vertical |  |
| Site Criteria | - | List | - | Random, Careful, Very Careful |  |
| Radio climate | - | List | - | Equatorial Continental Subtropical Maritime Subtropical Desert Continental Temperate Maritime Temperate over land Maritime Temperate over sea |  |
| Time percentage | - | Distribution or Scalar | % | 1 –99 |  |
| Location percentage | - | Distribution or Scalar | % | 1 –99 |  |
| Confidence level | - | Distribution or Scalar | % | 1 – 99 |  |
| Mode of variability | - | List | - | Single, individual, broadcast |  |
| Standard deviation | Std. dev. | Scalar | dB |  |  |

IEEE 802.11 Model C (modified)

The IEEE 802.11 (Model C) [1] was used in the development of the ECC Report 131Annex 2 [2] to characterise all TS-TS (terminal station) radio propagation in the Monte Carlo studies.

Note: the “modified” term to the plugin refers to the fact that the algorithm has been implemented to ensure that the user sets most of the input parameters.

Assumptions

Measurements results reported in the literature indicate that the presence of people across the propagation link between a transmitter and a receiver could cause additional loss (of up to 20 to 30dB), as a result of body loss or multi-path interference due to body scattering.

In circumstances where the spatial density of TSs is high (as in the geometries investigated in ECC Report 131), the probability of TS-TS path blocking is also high, and hence the TS-TS path can no longer be treated as line-of-sight. Consequently, a path loss model with a greater exponent is more suitable than the free space path loss model for the characterisation of TS-TS links in dense hotspots.

In ECC Report 131, the mean path loss is characterised by a dual-slope model with a break point at 5 m. In addition, the log-normal distributed shadowing with standard deviations of 3 dB and 4 dB is assumed in ECC Report 131 for separations of less than 5 m and greater than 5 m, respectively.

Where the calculated path loss is less than free space attenuation for the same distance, the free space attenuation is used instead.

Equation

The mean path loss is characterised by a dual-slope model with a break point dBP, an exponent of 2 for all distances less than dBP, and an exponent of 3.5 otherwise. In short, the mean path loss, L, in dB is

(Eq. 252)

where d is the separation between the transmitter and receiver in kilometres, dBP = 0.005 is the break-point in km (i.e. 5 m), and LFS is free space path loss:

(Eq. 253)

where hTx and hRx are the height of the transmitter (Tx) and receiver (Rx) respectively and are expressed in m. d is the distance between the Tx and Rx and is expressed in km. f is the frequency and is expressed in MHz.

The log-normal distributed shadowing with the corresponding standard deviation is applied to the pathloss calculated.

This propagation model is used to calculate terminal-terminal interference and takes account of shadowing losses due to objects between the two terminals, but does not explicitly account for any loss from near-field objects, such as the person carrying the equipment.

Figure 484: GUI of the IEEE 802.11 Model C (modified)

Table 95: IEEE 802.11 Model C (modified) propagation model

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation | - | Boolean | - | Variation in path loss takes into account the uncertainty of building design, furniture, room size, etc. |
| Distance to break point | BP | Scalar | m |  |
| Log-Normal distribution before BP | - | Scalar | dB |  |
| Log-Normal distribution after BP | - | Scalar | dB |  |

ITU-R Rec. P. 1411

Description of the model

Recommendation ITU-R P.1411 ‎[15], in §4.3, proposes a model for Tx and Rx antenna heights between 1.9 and 3 m in the UHF band (from 300 MHz to 3 GHz) and distances up to 3 000 m. This model allows SEAMCAT to investigate scenarios in urban environment when both transmitter and receiver antennas are low height antennas i.e. located near to the ground (below roof-top height to near street level).

. The model includes both LoS and NLoS regions, and predicts the rapid decrease in signal level occuring at the corner between the LoS and NLoS regions. The model includes the statistics of location variability in the LoS and NLoS regions, and provides a statistical model for the corner distance between the LoS and NLoS regions. Figure 485 illustrates the LoS, NLoS and corner regions, and the statistical variability predicted by the model.

Working conditions

This model is recommended for propagation between low-height terminals where both terminal antenna heights are near street level well below roof-top height, but are otherwise unspecified. It is reciprocal with respect to transmitter and receiver and is valid for frequencies in the range 300-3 000 MHz. The model is based on measurements made in the UHF band with antenna heights between 1.9 and 3.0 m above ground, and transmitter-receiver distances up to 3 000 m.

Figure 485: Curves of basic transmission loss not exceeded for 1, 10, 50, 90 and 99% of locations

(frequency = 400 MHz, suburban)

Algorithm

The parameters required are the frequency f (MHz) and the distance between the terminals d (m).

Step 1:	Calculate the median value of the line-of-sight loss:

(Eq. 254)

Step 2:	For the required location percentage, p (%), calculate the LoS location correction:

with  = 7 dB		(Eq. 255)

Alternatively, values of the LoS correction for p = 1, 10, 50, 90 and 99% are given in Table 96

Step 3:	Add the LoS location correction to the median value of LoS loss:

(Eq. 256)

Step 4:	Calculate the median value of the NLoS loss:

(Eq. 257)

Lurban depends on the urban category and is 0 dB for suburban, 6.8 dB for urban and 2.3 dB for dense urban/high-rise.

Step 5:	For the required location percentage, p (%), add the NLoS location correction:

with  = 7 dB	(Eq. 258)

is the inverse normal cumulative distribution function. An approximation to this function, good for p between 1 and 99% is given by the location variability function Qi(x) of Recommendation ITU-R P.1546. Alternatively, values of the NLoS location correction for p = 1, 10, 50, 90 and 99% are given in Table 96.

Table 96: Table of LoS and NLoS location variability corrections

| p (%) | LLoS (dB) | LNLoS (dB) | dLoS (m) |
| --- | --- | --- | --- |
| 1 | –11.3 | –16.3 | 976 |
| 10 | –7.9 | –9.0 | 276 |
| 50 | 0.0 | 0.0 | 44 |
| 90 | 10.6 | 9.0 | 16 |
| 99 | 20.3 | 16.3 | 10 |

Step 6:	Add the NLoS location correction to the median value of NLoS loss:

(Eq. 259)

Step 7:	For the required location percentage, p (%), calculate the distance dLoS for which the LoS fraction FLoS equals p:

(Eq. 260)

Values of dLoS for p = 1, 10, 50, 90 and 99% are given in Table 96. This model has not been tested for p < 0.1%. The statistics were obtained from two cities in the United Kingdom and may be different in other countries. Alternatively, if the corner distance is known in a particular case, set dLoS(p) to this distance.

Step 8: 	The path loss at the distance d is then given as:

a)	If d < dLoS, then L(d, p) = LLoS(d, p)

b)	If d > dLoS + w, then L(d, p) = LNLoS(d, p)

c)	Otherwise linearly interpolate between the values LLoS(dLoS, p) and LNLoS(dLoS + w, p):

Input parameters

The input parameters for this model are as follow:

Figure 486: GUI of the ITU-R P.1411 §4.3

Table 97: ITU-R P.1411 §4.3 propagation model

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation | - | B | - | Variation in path loss takes into account the uncertainty of building design, furniture, room size, etc. |
| General environment | - | - | - | Environment of the propagation: urban, suburban, Dense Urban/High Rise Dense Urban/High-Rise means high buildings, thus buildings are spaced. “Dense” is to be interpreted in terms of people grouping, not buildings grouping. This explains the reason why attenuation is less in Dense Urban/High Rise than in Urban) |
| Percentage of locations | - | S | % | Percentage of locations : from 1 to 99 |
| Width for transition region | - | S | m | Average street width : typical value is 15 m |
| Variations std. dev. | - | S | dB |  |

ITU-R Rec. P. 528

Description of the model

Recommendation ITU-R P.528 contains a method for calculating path loss in the frequency range from 125 to 15500 MHz for ground-air, ground-satellite, air-air, air-satellite, and satellite-satellite links.  It provides an interpolation method from a set of curves and a set of curves.

Working conditions

The implementation in SEAMCAT contains a consistency check of input parameters. It produces error messages if input parameters are not in the range specified by Recommendation ITU-R P.528 i.e.:

Minimum (ground) antenna height :	[1.5-1 000] m;

Maximum (aero) antenna height: 	[1 000-20 000] m;

Frequency: 	[125-15 500] MHz;

Time percentage: 	[1-95] %;

Distance: 	[0-1800] km.

Algorithm

Recommendation ITU-R P.528  Annex 2 describes separate stages of the calculation. A step-by-step description of the overall interpolation method is given in § 8 of that Annex. Section 2 gives the calculation for field strength from the transmission loss value(s) taken from the curves in Annex 3. Sections 3 to 8 of this Annex describe how to interpolate for distance, h1, h2, frequency and percentage time.

Input parameters

Input parameters are the heights of the antennas above mean sea level, and the frequency which is obtained from SEAMCAT internal calculation. The user needs to set the time percentage on the model GUI.

Figure 487: GUI of the ITU-R P.528

Table 98: ITU-R P.528 propagation model

| Description | Symbol | Type | Unit | Comments |
| --- | --- | --- | --- | --- |
| Variation | - | Boolean | - | Variation in path loss |
| Time percentage | - | Scalar | % | Time percentage for which prediction is required (1% - 95 %) |
| Variations std. dev. | - | Scalar | dB |  |

Plugin: Compilation and debuging

Java and Maven

Java

Go to Sun or Oracle website and search for downloading the Java Development kit (JDK).  You will get to download a .zip file. Note that the version of the JDK are changing quite regularly, therefore the version presented in this manual may not be up-to-date when you read it, but the spirit is the same. The JDK (Java development kit) includes the JRE (Java Runtime Environment).

Installing a compiler: Example with Apache maven

You can download Apache Maven at http://maven.apache.org/download.cgi and save it under your program files (if you work under Microsoft OS).

Figure 488: Set Maven in your poject. Select File -> Settings

Figure 489: Click on override for the “Maven home directory field”

Select “apache-maven-3 xx” (or above) from your file system.

Associate seamcat-model-5.0.0.jar to your project

SEAMCAT supports custom written plugins. You need to develop your own plugin using a Java development environment (IDE) for instance IDEA Intellij.

When creating a plugin you need to have the library seamcat-model-5.0.0.jar associated to your project. This file is needed if you want to be able to see the SEAMCAT model interface.

The Java dependencies (.jar file) needed for you development can be downloaded from the official download page under “Plugin development”.

Figure 490: Go to “Open Module Settings”

Figure 491: Click the green cross on the right handside and select “Jar or directories”

Go and fetch the .jar file from your file system (you need to download it from the official SEAMCAT web page to get the latest version)

Figure 492: list of dependencies

You can also see it under the “External Libraries”, just below your project

Figure 493: External libraries

Develop a Plugin

We want to develop a PMP that calculates the path loss as an increasing function starting from 0 (when the distance is 0) and growing towards infinity at the square of the distance times a distribution given as input by the user.  In pseudo code something like the following:

path loss = dist(tx, rx)^2 * distribution.trial()

Note: This is just an example and not a real propagation model.

The following prerequisites are assumed:

basic understanding of Java;

intellij IDEA is installed and used for development;

maven 3 (3.0.0 or above) is installed;

seamcat-model-5.0.0.jar is downloaded.

Setting up the project

Setting up the project is done using maven and uses the template project found in the plugin-template.zip.

Figure 494: Example in setting up a project

Unzip this file and name the main directory "pm-square". The contents of the unzipped folder is an empty Java project with maven configuration for the project dependencies and instructions on how to create a .jar file.

After importingthe intellij project, it is important that the local maven repository knows about seamcat-model-5.0.0.jar dependency. Follow the instruction of section ‎A18.2. The local maven repository will now contain the needed dependency and with intellij the project can now be imported. Do this by opening intellij and choose File<New<Project from Existing Sources-t.

Figure 495: Select the pom.xml file in the 'pm-square' folder.

Go through the various windows

Development

We first create a package named 'com.example.pmsquare' for the plugin classes. This is done by right clicking the 'src/main/java' folder and selecting new->package.

Figure 496: create a package named 'com.example.pmsquare' for the plugin classes

Figure 497: Write com.example.pmsquare in the dialog that appears

Figure 498: Remember to have the “Compact Empty Middle Packages” unchecked in the view settings  , so that you can see the tree structure

Figure 499: On the PMSsquare folder now right click again to create a java class called PMSquare

Figure 500: create a java class called PMSquare

Figure 501: Make PMSquare class implement PropagationModelPlugin 
(and auto implement the needed methods)

Figure 502: Press ALT + enter (on PC)

Note: <Empty> is an empty interface contained in the seamcat-model.jar package

Figure 503: Implement  methods

Figure 504: Select “implement methods” to auto implement the interface

Figure 505: Results of the auto implementation

Building & Installing

The project is packaged into a .jar file by running the maven command 'install'. This can be done either from the intellij maven tab (on the far right there is a vertical tab called Maven - unfold and double click on “install”). Or it can be done from a command terminal by standing in the folder where the pom.xml file is located and write mvn clean install.

Figure 506: The successful run of the 'install' command will produce a target folder containing 
the resulting .jar file

You may have warning complaining of a wrong “Maven Home directory” configuration. This is because your maven home directory is not set properly. Click on the “configuration dialog” link, which will lead you to the Maven setting panel.

Figure 507: The successful run of the 'install' command will produce a target folder containing 
the resulting .jar file

When your “Maven Home directory” is set properly, double click on “Install” again, you can see that the compilation starts

If you have uploaded the latest version of the SEAMCAT interface for plugin, then IntellijIdea will by default use it (even though it has been generated from another project).

If the compile works without failure you should get a “BUILD SUCCESS”

Figure 508: Compilation status

IntellijIdea/maven created a Target folder where you can see the .jar that has been created. You can then upload that .jar into SEAMCAT.

Figure 509: Target folder

You are now ready to install your .jar plugin into SEAMCAT.

How to debug your plugin

You need to have the following installed on your machine

- Intellij (version 12 and forward) with SEAMCAT setup and running in debug mode;

- The source code of the plugin project;

- The plugin .jar file installed in the running SEAMCAT application.

To debug the plugin code the jar file of the plugin and the source code must be associated with the SEAMCAT project. This is done by doing the following steps:

- From the SEAMCAT intellij project open the project structure  (menu "File -> Project Structure ...");

From the "Project Settings" choose "Libraries";

Choose "Attach files or Directories" (by clicking the green plus sign on the right);

First attach the .jar file of the plugin (this will attach the binary code of the plugin to your SEAMCAT project);

Second attach the root directory of the plugin project (this will attach the source code of the plugin).

Figure 510: How to associate a plugin .jar to its source code

You should now be able to open the plugin class from the SEAMCAT project. Press Ctrl-n and type the name of the plugin class you want to debug (note that the auto-complete may not work, so you need to write the full filename). This should open the source file of the plugin class and breakpoints should be possible to set normally.

Notice that if you make changes to this file it will be out of sync with the binary file running from within SEAMCAT. So eventual bugs to be fixed will have to be done inside the plugin project AND then subsequentially built and installed in SEAMCAT to be tested.

Example of PMP

This simple example of a Propagation Model Plugin applies Free Space propagation loss with its default distance factor of 2, in case the antenna heights of TX and RX are greater than the limit given by the input annotation 'Height of antenna', otherwise it applies the factor given by the input annotation 'Distance coefficient'. If 'variation' is selected it uses the standard deviation given by the input annotation 'Standard deviation'.

It aims to show:

the basic structure of a propagation plugin which is creating its own Input interface

how to set up basic consistency checks

how to define input annotations including tool tip information

Note that for simplicity Runtime Exceptions have not been included

The structure details are:

- calculation of the resulting path loss

consistency check

  - the validity of the input parameter 'Distance coefficient';

  - whether the value of the input parameter 'antenna height' conflicts with the antenna heights set on the system if the PMP is used for the system link or

description

- unique name of the plugin – must not be empty;

- optional but recommended description of the plugin characteristics;

Input annotations
note that the value of 'variations' is either selecting or deselecting the variations by default.

Figure 511: PMP example source code

You may notice that for the generation of the random Gaussian distribution value of the standard deviation the built-in Factory.distributionFactory() has been used.

Figure 512: PMP example configuration panel

Figure 513: PMP example consistency check warnings

Example of AP

This simple example of an Antenna plugin modifies within a given beam width the resulting antenna gain proportional to the ratio of the actual distance and the distance 'of adaption'.

It aims to show

the basic structure of an antenna plugin which is creating its own Input interface;

how to set up basic consistency checks;

how to define input annotations including tool tip information.

Note that for simplicity Runtime Exceptions have not been included.

The structure details are:

- calculation of the resulting antenna gain

consistency check

- the validity of the input parameter 'beam width';

- whether the value of the input parameter 'adaptive distance' conflicts with the coverage radius, taking account of the path distance factor;

- the valid system type applying this antenna.

description

- unique name of the plugin – must not be empty;

- optional but recommended description of the plugin characteristics;

Input annotations
note that the value 'peakGain' is setting the default value of the plugin.

Figure 514: Antenna Plugin example source code

Figure 515: Antenna Plugin configuration panel

Figure 516: Consistency check warning messages

Example of EPP

The basics of the structure of an EPP and its components are shown on Section 8.6, therefore these details are not repeated on this section.

The example wants to show

how to collect simulated data

how to generate own results based on the collected data

how to provide additional information using a custom UI assigned to the EPP (see Section A.19.3.1)

how to apply user defined buttons onto the custom UI which might be used also for a 'user defined consistency check' of the custom UI (see Section A.19.3.2)

The following capabilities are implemented onto the example:

collect data required for the calculation of the distribution of C/(N+I) for each interference link separately

generate a vector group containing the distribution vectors

do a user defined consistency check on the custom UI

let the custom UI calculate the probability of exceeding a given limit for the C/(N+I) criterion and let the UI show the result on its panel

On the main part of collecting data the following is done:

Note the annotation on top binding the custom UI to this EPP

- in case the consistency warning regarding the not supported cellular systems has been ignored, the EPP stops and puts onto the result panel the appropriate message;
this could be understood as "Runtime exception"

takes the required values of the victim

collects the interference power of each active interfering transmitter;
it has to be pointed out that for a real plugin the bandwidth of the victim receiver alone wouldn't be sufficient to determine whether co-channel is to consider or not

puts the calculated distribution of C/(N+I) for each interference link as vector group onto the result panel

Figure 517: EPP example source code (without other implemented methods)

Figure 518: EPP example source code (consistency check)

Figure 519: EPP example source code (selecting the map entry)

Example of extracting generic system intermediary results

The example shows the main core of the EPP Demo 8 which purpose is to collect intermediary distance results from a generic system.

You can see in yellow highlighted code (line 32) the mechanism to extract the distance between the VLT and the VLR.

You can see in violet highlighted code (line 53) the mechanism to extract the distance between the ILT and the VLR.

Figure 520: EPP example of extraction of distance for the victim and the interfering link

Example of a EPP with custom UI

The main part of the custom UI (as a reminder: it is a result type of the EPP) is quite simple:

it gets [1] the panels which are defined in [2] and specified in [3] and puts these together to the custom UI as horizontally split pane. There is no further action on that part.

Figure 521: Post processing UI source code (without other implemented methods)

The right panel [a] is unlike the left panel not an annotation interface, its construction is described on Section ‎A18.7.4

Post Processing buttons on the UI

User defined post processing buttons are generated through the annotation @PostProcessing as shown on Section ‎14.7.3.3. How to apply a 'consistency check' feature to the post processing is basically shown on Section ‎14.7.3.4.

This example uses one post processing as (stand alone) consistency check which is inherently used by the other one which performs the calculation of the probability of exceeding the criterion limit.

Figure 522: Post processing UI source code (consistency check)

The post processing "calculate probability"

- takes the distribution vectors generated by the EPP and

derives the probability of exceeding the given limit and

puts the result onto a text field of the right panel.

For simplicity the functionality 'summation vector' has not been implemented.

Figure 523: Post processing UI source code (calculation)

User defined panel (right panel)

Figure 524: Post processing UI source code (user defined panel construction)

Figure 525: Post processing UI (with an example of a constancy warning)

Example of intermediary results

When you develop your own plugin, you are able to auto-complete the list of available temporary parameters. For instance in an IntelJIdea development editor, if you press CTRL+SPACE an auto complete of the victimResult is availbale.

First, of course, you will need to assign what victimResult is in such a manner:

LinkResult victimResult = linkResult.getVictimSystemLinks().get(0);

Figure 526 presents an example of the content of an auto-complete of a variable of the type LinkResult

Figure 526: Example of content of the LinkResult interface 
(auto-complete “CTRL+SPACE” in IntelJidea)

The EPP allows you to access interemediary results. The following is a non-exhaustive list of the LinkResult interface:

AntennaResult rxAntenna():		Receiver antenna results in the direction of the transmitter

AntennaResult txAntenna():		Transmitter antenna results in the direction of the receiver

double getTxRxAngle():			Angle between transmitter and receiver

double getTxRxDistance():		Distance between transmitter and receiver.

double getTxRxPathLoss(): 		Path loss between transmitter-receiver path.

double getEffectiveTxRxPathLoss():	Effective Path loss between transmitter-receiver path.

double getBlockingAttenuation():	get the blocking attenuation;

double getTxPower():			get the transmitter power

double getFrequency():			get the frequency of the system

boolean isTxRxInSameBuilding():	indicates if the Tx or Rx is in the same building (from propagation model)

double getRxNoiseFloor()		get the receiver noise floor

Similarly, if you look one level down in the RxAntenna() that is follows an AntennaResult interface, you will see further parameters listed:

Figure 527: Example of content of the AntennaResult interface 
(auto-complete “CTRL+SPACE” in IntelJidea)

And likewise, if you look into the getposition() that is follows a Point2D interface, you will have the choice to select either getX() or get(Y) for the X and Y position of the received antenna of the victim.

Figure 528: Example of content of the Point2D interface 
(auto-complete “CTRL+SPACE” in IntelJidea)

: Consistency check (examples)

Frequency out of range consistency check

When you set-up the operating frequency of the ILT and the VLR so far apart that the ILT emission mask and the VLR mask do not overlap as illustrated in Figure 529, the consistency warning  message of Figure 530 is prompted by SEAMCAT for you to correct it.

899.8

fv=900 MHz

Victim link receiver Mask

Interferer Transmitter emission mask

fi=901 MHz

901.5

900.5

900.2

400 kHz

freq.

1 MHz

No overlap

in the masks

Figure 529: Illustration of the warning displayed in Figure 530

Figure 530: Frequency out of range consistency check warning

Solution: modify the operating frequencies or the size of the spectrum masks so that an overlap is occurring.

Horizontal pattern out of range consistency check

You erroneously specified the horizontal pattern for instance in the range of 0 ... 270 degrees:

Figure 531: Antenna pattern range consistency check warning

Solution: set the antenna pattern correctly or choose a valid antenna.

Extended Hata propagation model out of range consistency check:

You tried to apply the Extended Hata model to the interference link of a scenario which uses frequencies below 30 MHz:

Figure 532: Propagation model range consistency check warning

Solution: apply for instance the Free Space propagation model to the interference link.

: Debug information

SEAMCAT configuration

The SEAMCAT configuration panel is accessible via general menu option <File/Configuration> () also using <CTRL+G>, which opens the following dialog box:

Figure 533: Access to the SEAMCAT configuration window

On the General tab the following options are available for controlling the logging function:

SEAMCAT home: This option allows defining the SEAMCAT home directory where log file, report folder etc.. are placed (see Section ‎1.7.2);

Log level: choose from among the log level options DEBUG, INFO, WARN, ERROR, FATAL or OFF;

Format string: Format pattern of the log statements. Select or edit a pattern from the list. Newly edited patterns are added to the list;

Select the application look and feel: select from various display themes of SEAMCAT (Nimbus theme is the default one);

The checkbox option < Show the “welcome to SEAMCAT” screen at startup > activates a dialog box which opens each time SEAMCAT is started (see Section ‎2.2).

Debug information format

In debug mode only, it is recommended to select the Log Level: DEBUG and Format String: {(%F:%L[%M]) - %m%n} which would result in the most deep level of reporting where:

%F	represents the name of the Java class;

%L	the location inside this class (line number);

%M	the name of the method;

%m	the explaining message;

%n	the evaluated number(s) – if any.

On the SEAMCAT configuration panel there is the option of choosing one of the below format settings:

Figure 534: Options of format strings for the debug information

where (additionally):

%-5p	represents the first 5 digits of the information type (here DEBUG) at the leading left

%5p	the first 5 digits of the information type (here DEBUG) at the actual position

%d	the default time format (yyyy-mm-dd HH:mm:SS,sss)

%d (..)	a specified time format

%t	the running thread

Notes:

- This format is also applied to the header of the information recorded by the seamact.log file (see Annex 19).

In addition to the predefined selection, the format string field is also editable and can be modified the Format String according to your demands.

Debug information format settings:

Figure 535: Examples of format strings for the debug information

: Reference material and workspaces ECC/CEPT reports

The following table summarises studies where SEAMCAT has been used and to give examples on how different technologies have been implemented.

The scenarios and parameters contained in these reports should only be used as an initial guide when you are starting to develop your simulation.

The following reports can be found at the ECO web site www.erodocdb.dk

|  | Technology |  |  |  |  |  |  |  |  |  |  |  |  | Power flux density calculation | Interference criteria |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UMTS | UWB | DVB-T | FS | GSM | GSM-R | PL | PMR | PWMS | RAS | RNS | Satellite | TETRA |  | C/I | C/(I+N) | (N+I)/N | I/N | Capacity loss |
| ECC Report 076 |  |  |  | X |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| ECC Report 093 CEPT Report 16 | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  | X |  | X |  |
| ECC Report 096 | X |  |  |  |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |
| ECC Report 099 |  |  |  |  |  |  |  | X |  |  |  |  | X |  | X |  |  |  |  |
| ECC Report 104 |  |  | X |  |  |  |  | X |  |  |  |  |  |  | X |  |  |  |  |
| ECC Report 121 |  |  |  | X |  |  |  |  | X |  |  |  |  |  |  |  |  | X |  |
| ECC Report 122 CEPT Report 28 | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  | X |  | X | X |
| ECC Report 123 |  | X |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  | X |  |
| ECC Report 128 |  |  |  |  |  |  | X |  |  |  | X | X |  |  |  |  |  | X |  |

Table 99: List of ECC Report/CEPT Report containing SEAMCAT workspaces

: dB conversion table

Table 100: Conversion table

| Linear | dB |
| --- | --- |
| 2 | 3 dB |
| 4 | 6 dB |
| 8 | 9 dB |
| 16 | 12 dB |
| 10 | 10 dB |
| 20 | 13 dB |
| 100 | 20 dB |
| 1000 | 30 dB |
| 25 | 100/4= 20 dB - 6 dB= 14 dB |
| 2000 | 1000*2 = 30 dB + 3 dB = 33 dB |
| 50 | 100/2=20 dB – 3 dB = 17 dB |

How to make the most of the SEAMCAT figures

Tip 1: To know the properties, change the titles name or resize the axis of a graphic, right click on the graph and the following windows will appear:

Figure 536: Illustration on how to get the properties dialog box of a graph

Figure 537: Illustration on changing the chart properties

Tip 2: To Zoom in: right (or left), click on the mouse and select the area that you want to zoom on.

Tip 3: To Zoom out: right (or left), click on the mouse move toward the upper left corner.

Tip 4: In case that is difficult to see some colors like yellow in the figure below, it is possible to change the background color. For that, right-click, select properties / plot / appearance and then background paint.

Figure 538: Illustration of default white background color

Figure 539: Illustration of light grey background color

: Example - TETRA MS interferers the BS of the analogue FM PMR

Scenario overview

This section provides an example of sharing studies including an analysis of a TETRA MS interfering with an FM BS, based on the information provided in the ERC Report 104 on “Adjacent Band Compatibility Of 400 MHz TETRA And Analogue FM PMR –  An Analysis Completed Using MONTE CARLO Based Simulation Tool”. This scenario involves a population of TETRA mobile stations interfering with a victim FM base station as shown in Figure 540. In all of the simulations the victim FM system is assumed to have a 7.8 km cell radius which provides a 90 % area availability.

Victim Link

Transmitter

(VLT)

FM Mobile Station

Victim Link

Receiver

(VLR)

FM Base Station

dRSS

Victim link

Interfering Link

Transmitter

(ILT)

TETRA Mobile Station

Interfering Link

Receiver

(ILR)

TETRA Base Station

iRSS

Interfering link

Figure 540: Scenario of the simulation

Figure 541 illustrates the band plan assumed for this investigation. 2 MHz of spectrum has been allocated to the up-link of TETRA and directly adjacent to this, 2 MHz has been allocated to FM.

Figure 541: Band allocations of TETRA MS and FM BS systems

The purpose of this example of simulation is to use SEAMCAT to obtain the probability of interference resulting from unwanted emissions of the TETRA Mobile Stations falling at the frequency where the FM Base Station is operating.

Characteristics of the systems

Victim link characteristics – FM

The victim FM system is assumed to have a 7.8 km cell radius which provides a 90% area availability. The reception bandwidth of the FM BS victim link receiver is equal to 8 kHz. Based on information provided in ERC Report 104 for FM system based on 12.5 kHz channel spacing, the following characteristics are used to describe the FM equipment:

Table 101: Parameters assumed for 12.5 kHz FM systems

| Parameter | Mobile Station | Base Station |
| --- | --- | --- |
| Channel Spacing | 12.5 kHz | 12.5 kHz |
| Transmit Power | 37 dBm | 44 dBm |
| Receiver Bandwidth | 8 kHz | 8 kHz |
| Antenna Height | 1.5 m | 30 m |
| Antenna Gain | 0 dBi | 9 dBi |
| Active Interferer Density Range | Variable | Variable |
| Receiver Sensitivity | - 107 dBm | - 110 dBm |
| Receiver Protection Criterion (C/I) | 21 dB | 21 dB |
| Power Control Characteristic | Not used | Not used |

Note 1: This exercise assumes that the noise floor is small compared to the interfering signal (I).

Note 2: The Noise Floor can be obtained using the following equation (see ERC Report 68 ‎[9]):

Noise Floor = Sensitivity – Receiver Protection Ratio

Table 102: Receiver blocking for 12.5 kHz FM systems

| Frequency Offset | Base Station |
| --- | --- |
| Any frequency | - 23 dBm |

Interfering link characteristics – TETRA

The interfering TETRA system is assumed to have a 18 kHz emission bandwidth and a channel spacing which is equal to 25 kHz. The distribution in terms of frequency use over the 2 MHz band is taken as a uniform distribution, where each channel is being assigned the same probability. Based on information provided in ERC Report 104 for TETRA system based on 25 kHz channel spacing, the following characteristics are used to describe the TETRA equipments:

Table 103: Parameters used to model the TETRA systems

| Parameter | Mobile Station | Base Station |
| --- | --- | --- |
| Channel Spacing | 25 kHz | 25 kHz |
| Transmit Power | 30 dBm | 40 dBm |
| Receiver Bandwidth | 18 kHz | 18 kHz |
| Antenna Height | 1.5 m | 30 m |
| Antenna Gain | 0 dBi | 11 dBi |
| Active Interferer Density Range | Variable | Variable |
| Receiver Sensitivity | - 103 dBm | - 106 dBm |
| Receiver Protection Criterion (C/I) | 19 dB | 19 dB |
| Power Control Characteristic | 5 dB steps to a maximum of 15 dB. Threshold = - 86 dBm | Not used |

Table 104: Unwanted emissions for TETRA systems (measurement bandwidth of 18 kHz) 
(interefering link transmitter)

| Frequency Offset | 30 dBm Mobile Station | Attenuation in dBc |
| --- | --- | --- |
| 25 kHz | - 30 dBm | 60 |
| 50 kHz | - 36 dBm | 66 |
| 75 kHz | - 36 dBm | 66 |
| 100 - 250 kHz | - 45 dBm | 75 |
| 250 - 500 kHz | - 50 dBm | 80 |
| > 500 kHz | - 70 dBm | 100 |

Assumptions

Starting from the characteristics provided above, some “reference” hypothesis might be used to obtain a first set of results.

Distribution of the Mobile Stations (MS) and propagation model

It is assumed that all mobile stations are located uniformly distributed in the areas around their base stations. For worst case consideration a rural area may be assumed.

TETRA network

A 2 MHz band is available for TETRA systems of 25 kHz channel spacing, therefore a total of 80 channels are available. It is supposed that only one TETRA MS terminal may use the same frequency at a given time.

The frequency cluster is taken equal to 9, this leads to have 80/9 = 8.88 frequency per cell available. Then up to 9 terminals may be deployed in a TETRA cell.

If the TETRA cell is a traffic limited network and the density of terminals is equal to 5 terminals per km², then the coverage radius is given by the following formula (see ERC Report 68 or User Documentation):

(Eq. 261)

The radius of the cell of the interfering system is given by

R = sqrt( (80*1) / (3.14*5*9) ) = 0.7523 km.

Figure 542 describes a TETRA network based on the above assumptions.

Figure 542: TETRA network based on these assumptions

Interference situation

As a worst case assumption, we may suppose that, the FM BS station will be based in the TETRA cell where the channel (411.9875 MHz) operating close to the edge of the TETRA band is used. The same probability is assigned to each channel available in the TETRA cell. Then the frequency plan in this cell starts from 410.0125 MHz up to 411.9875  MHz with a step of 25 kHz x 9. A discrete uniform distribution for frequency can be selected, e.g., between 410.0125 MHz and 411.9875 MHz having a step of 25 kHz (see ‎ANNEX 10: for a description of the discrete uniform distribution).

If there is no spatial correlation between the elements of the victim system and the elements of the interfering system then the simulation radius of the simulation is given by the formula:

= radius of the area where interferers are uniformly spread

(Eq. 262)

We have to consider 9 active interferers with a density of 5 terminals/km². Then the radius of simulation area results in:

R = sqrt( 9 / (3.14 x 5) ) = 0.7569 km.

The FM Base Station, which is the victim link receiver, is supposed to be operating at 412.00625 MHz, that is the centre of the first channel, i.e. no guard interval is assumed.

Emission mask

The total emission of the interefering link transmitter consists of

The wanted signal within the defined bandwidth corresponding to a frequency offset of ± 50 % of the emission bandwidth ItBW;

The out-of-band emission caused by the modulation within, e.g. ± 50...250 % of ItBW;

The spurious emission outside these ranges.

This means the mask to be defined has to cover this whole range. This allows co-channel consideration as well.

It should be noted that the TETRA MS unwanted emissions mask is not defined for frequency offsets below 12.5 kHz. Considering the channel spacing of 25 kHz and assuming 0 dBc/18 kHz in that range, then in the undefined range between 12.5 and 25 kHz, a linear interpolation between 0 and 60 dBc attenuation will be performed. It should also be taken into account that the reference bandwidth (18 kHz) used for the mask is identical to the bandwidth of the interfering system (see ‎ANNEX 6:).

Probability of interference

Based on the above assumptions the probability of interference (C/I criteria and unwanted emissions + blocking are taken into account) is about 1.1 % when the PC is “on” and about 8.1 % when the PC is “off”.

Workspace setting

In this chapter, only the necessary inputs are mentioned. All other parameters are not required.

- SYSTEMS:

1.1 FM as Victim Link

- Name:				FM

- Frequency:  			Constant 412.00625 MHz  (channel spacing)

    - Transmitter: FM  MS (VLT)

Transmitter Identification

- Name:				FM-MS

Antenna pointing

- Antenna height:			______________

- Antenna azimuth:		0...360 degrees, uniform

- Antenna elevation:		0 degrees, constant	(horizontal direction)

Antenna Pattern Identification

- Description:			Omni-directional

- Antenna peak gain:		______________

Emission Characteristics

- Power:				______________

1.1.2 Receiver: FM  BS (VLR)

Receiver Identification

- Name:				FM-BS

Antenna pointing

- Antenna height:			______________

- Antenna azimuth:		0...360 degrees, uniform

- Antenna elevation:		0 degrees, constant	(horizontal direction)

Antenna Pattern Identification

- Description:			Omni-directional

- Antenna peak gain:		______________

Reception Characteristics

- Noise floor:			______________ (Noise Floor =Sensitivity – C/I)

- Blocking mode:			Sensitivity

- Blocking mask:			-23 dB, constant

- Intermodulation rejection: 	0 dB, constant

- Sensitivity:			______________

- Reception bandwidth:    		8 kHz

- Overloading: 			NO

Interfering Criteria

- C/I:			21 dB (if only C/I, then C/(N+I) and (I+N)/N are not required)

- C/(I+N): 		21dB (C/(N+I) required only for selected Blocking-Mode)

    - Transmitter to Receiver path (VLT - > VLR)

Relative location

- Correlation: 			None

- Path azimuth VLR: 		Uniform distribution 0 to 360 degrees

- Path distance factor:  		Uniform polar, 1*cell radius

Coverage radius

- Cell radius mode:		User-defined radius

- Fixed radius:			______________

Local environment:

- Receiver: 			100% outdoor

- Transmitter: 			100% outdoor

Propagation model

- Model:				Extended Hata

- Variation:			Yes

- General environment: 		______________

- Propagation environment:	ABOVE ROOF (with a 30 m FM BS-Antenna height)

  - TETRA as Interfering Link

- Name:				TETRA

- Frequency:  			410.1875…411.9875 MHz, uniform

    - Transmitter: TETRA  MS (ÍLT)

Transmitter Identification

- Name:				TETRA-MS

Antenna pointing

- Antenna height:			______________

- Antenna azimuth:		0...360 degrees, uniform

- Antenna elevation:		0 degrees, constant	(horizontal direction)

Antenna Pattern Identification

- Description:			Omni-directional

- Antenna peak gain:		______________

Emission Characteristics

- Power:				______________

- Emission Mask:

Unwanted mask: Offset in MHz, power in dBc referred to Reference bandwidth

-10.0000 	-100.0000	18.0000

-0.5010 	-100.0000	18.0000

-0.5000 	-80.0000		18.0000

-0.2510 	-80.0000		18.0000

-0.2500 	-75.0000		18.0000

-0.1000		-75.0000		18.0000

-0.0750		-66.0000		18.0000

-0.0500		-66.0000		18.0000

-0.0250 	-60.0000		18.0000

-0.0125		0.0000		18.0000

0.0125		0.0000		18.0000

0.0250		-60.0000		18.0000

0.0500 	-66.0000		18.0000

0.0750 	-66.0000		18.0000

0.1000 	-75.0000		18.0000

0.2500 	-75.0000		18.0000

0.2510 	-80.0000		18.0000

0.5000 	-80.0000		18.0000

0.5010 	-100.0000	18.0000

10.0000	-100.0000	18.0000

- Transmitting bandwidth: 	18 kHz (Bandwidth < channel)

- Reference bandwidth:		18 kHz

- Unwanted emission floor: 	NO

- Power control:

- Power Control step size: 	5 dB

- Min Threshold:		-86 dBm

- Dynamic Range:		15 dB

1.2.2 	Receiver: TETRA  BS (ILR)

Receiver Identification

- Name:				TETRA-BS

Antenna pointing

- Antenna height:			______________

- Antenna azimuth:		0...360 degrees, uniform

- Antenna elevation:		0 degrees, constant (horizontal direction)

Antenna Pattern Identification

- Description:			Omni-directional

- Antenna peak gain:		______________

Reception Characteristics

- Noise floor:			______________ (Noise Floor = Sensitivity – C/I)

- Blocking mode:			User defined

- Blocking mask:			0 dB, constant

- Intermodulation rejection: 	0 dB, constant

- Sensitivity:			______________

- Reception bandwidth:    		______________

- Overloading: 			NO

Interfering Criteria

- C/I:			19 dB (if only C/I, then C/(N+I) and (I+N)/N are not required)

- C/(I+N): 		16 dB (C/(N+I) required only for selected Blocking-Mode)

- (N+I)/N:			3 dB

- I/N:			0 dB

    - Transmitter to Receiver path (ILT - > ILR)

Relative location

- Correlation: 			None

- Path azimuth VLR: 		Uniform distribution 0 to 360 degrees

- Path distance factor:  		Uniform polar 1*cell radius

Coverage radius

- Cell radius mode:		Traffic limited network

- Density (terminals per Km2):	______________

- Nr. of channels:  		______________

- Nr. of users per channel: 	1

- Frequency cluster: 		______________

Local environment:

- Receiver: 			100% outdoor

- Transmitter: 			100% outdoor

Propagation model

- Model:				Extended Hata

- Variation:			Yes

- General environment: 		______________

- Propagation environment:	ABOVE ROOF (with a 30 m TETRA BS-Antenna height)

- 2. SCENARIO

- 2.1. VICTIM SYTEM

- Victim System:	 	FM

- Frequency: 		Constant, 412.00625

2.2. SIMULATION CONTROL

- Number of events: 	20 000

2.3. INTERFERING SYSTEM LINKS

- Link1: 			TETRA

- Frequency:  		410.1875…411.9875 MHz, uniform

2.4 ILT -> VLR path

Relative positioning of the interfering link

- Mode:					Uniform density

- Position relative to:				Victim Link Receiver

- Delta X:					Constant, 0

- Delta Y:					Constant, 0

- Set ILR at the centre of the distribution: 	NO

- Path azimuth:				Uniform distribution, 0 to 360 degrees

- Number of active transmitters:		9

- Minimum coupling loss:			Constant, 0

- Protection distance:				Constant, 0

Interferers density

- Density of Tx: 				______________ (*1/Km2)

- Probability of transmission:			1

- Activity:					Constant, 1

- Time:					1 hour

Propagation Model

- Model:				Extended Hata

- Variation:				Yes

- General environment: 		______________

- Propagation environment:		ABOVE ROOF (with a 30 m FM BS-Antenna height)

- 3. RESULTS

Test of the simulation

Calculated radius

- Coverage radius (wanted system): 		7.8 km

- Coverage radius (interfering system 1): 		0.7522 km

- Simulation radius 1:				0.7569 km

Generated Signals

PC “on”

- dRSS: 		mean:  ______________(-69.9 dBm),	std:  ______________(11.68 dB)

- iRSSunwanted: 		mean:  ______________(-127.5 dBm), 	std:  ______________(10.86 dB)

- iRSSblocking: 		mean:  ______________(-144.6 dBm), 	std:  ______________(7.13 dB)

PC “off”

- dRSS: 		mean:  ______________(-69.8 dBm),	std:  ______________(11.78 dB)

- iRSSunwanted: 		mean:  ______________(-112.6 dBm), 	std:  ______________(10.84 dB)

- iRSSblocking: 		mean:  ______________(-129.6 dBm), 	std:  ______________(7.19 dB)

Interference calculation/ICE calculation

- Interference Criteria:			C/I

- Compatibility mode:			Yes (Probability of interference)

- Unwanted emissions:			Yes

- Blocking:				Yes

Result

PC “on”

- Probability of interference:  	______________ (~1 %)

PC “off”

- Probability of interference: 	______________ (~8.4 %)

: Answers to the Tetra vs FM PMR example

Note: See the questions of this exercise in section ‎ANNEX 24:.

SEAMCAT parametrisation of simulation scenario TETRA versus FM.

- 1. SYSTEMS:

1.1 FM as Victim Link

- Name:				FM

- Frequency:  			Constant 412.00625 MHz  (channel spacing)

    - Transmitter: FM  MS (VLT)

Transmitter Identification

- Name:				FM-MS

Antenna pointing

- Antenna height:			1.5 m, constant

- Antenna azimuth:		0...360 degrees, uniform

- Antenna elevation:		0 degrees, constant	(horizontal direction)

Antenna Pattern Identification

- Description:			Omni-directional

- Maximum gain:			0 dBi

Emission Characteristics

- Power:				37 dBm, constant

1.1.2 Receiver: FM  BS (VLR)

Receiver Identification

- Name:				FM-BS

Antenna pointing

- Antenna height:			30 m, constant

- Antenna azimuth:		0...360 degrees, uniform

- Antenna elevation:		0 degrees, constant	(horizontal direction)

Antenna Pattern Identification

- Description:			Omni-directional

- Maximum gain:			9 dBi

Reception Characteristics

- Noise floor:			-131 dBm, constant(=Sensitivity – C/I)

- Blocking mode:			Sensitivity

- Blocking mask:			-23 dB, constant

- Intermodulation rejection: 	0 dB, constant

- Sensitivity:			-110 dBm

- Reception bandwidth:    		8 kHz

- Overloading: 			NO

Interfering Criteria

- C/I:			21 dB (if only C/I, then C/(N+I) and (I+N)/N are not required)

- C/(I+N): 		21dB (C/(N+I) required only for selected Blocking-Mode)

    - Transmitter to Receiver path (VLT - > VLR)

Relative location

- Correlation: 			None

- Path azimuth VLR: 		Uniform distribution 0 to 360 degrees

- Path distance factor:  		Uniform polar, 1*cell radius

Coverage radius

- Cell radius mode:		User-defined radius

- Fixed radius:			7.8 km

Local environment:

- Receiver: 			100% outdoor

- Transmitter: 			100% outdoor

Propagation model

- Model:				Extended Hata

- Variation:			Yes

- General environment: 		RURAL (results in lowest path attenuation)

- Propagation environment:	ABOVE ROOF (with a 30 m FM BS-Antenna height)

  - TETRA as Interfering Link

- Name:				TETRA

- Frequency:  			410.1875…411.9875 MHz, uniform

    - Transmitter: TETRA  MS (ÍLT)

Transmitter Identification

- Name:				TETRA-MS

Antenna pointing

- Antenna height:			1.5 m, constant

- Antenna azimuth:		0...360 degrees, uniform

- Antenna elevation:		0 degrees, constant	(horizontal direction)

Antenna Pattern Identification

- Description:			Omni-directional

- Maximum gain:			0 dBi

Emission Characteristics

- Power:				30 dBm, constant

- Emission Mask:

Unwanted mask: Offset in MHz, power in dBc referred to Reference bandwidth

-10.0000 	-100.0000	18.0000

-0.5010 	-100.0000	18.0000

-0.5000 	-80.0000	18.0000

-0.2510 	-80.0000	18.0000

-0.2500 	-75.0000	18.0000

-0.1000	-75.0000	18.0000

-0.0750	-66.0000	18.0000

-0.0500	-66.0000	18.0000

-0.0250 	-60.0000	18.0000

-0.0125	   0.0000	18.0000

0.0125	  0.0000	18.0000

0.0250	-60.0000	18.0000

0.0500 	-66.0000	18.0000

0.0750 	-66.0000	18.0000

0.1000 	-75.0000	18.0000

0.2500 	-75.0000	18.0000

0.2510 	-80.0000	18.0000

0.5000 	-80.0000	18.0000

0.5010 	-100.0000	18.0000

10.0000	-100.0000	18.0000

- Transmitting bandwidth: 	18 kHz (Bandwidth < channel)

- Reference bandwidth:		18 kHz

- Unwanted emission floor: 	NO

- Power control:

- Power Control step size: 	5 dB

- Min Threshold:		-86 dBm

- Dynamic Range:		15 dB

1.2.2 	Receiver: TETRA  BS (ILR)

Receiver Identification

- Name:				TETRA-BS

Antenna pointing

- Antenna height:			30 m, constant

- Antenna azimuth:		0...360 degrees, uniform

- Antenna elevation:		0 degrees, constant (horizontal direction)

Antenna Pattern Identification

- Description:			Omni-directional

- Maximum gain:			11 dBi

Reception Characteristics

- Noise floor:			-125 dBm, constant(=Sensitivity – C/I)

- Blocking mode:			User defined

- Blocking mask:			0 dB, constant

- Intermodulation rejection: 	0 dB, constant

- Sensitivity:			-106 dBm

- Reception bandwidth:    		18 kHz

- Overloading: 			NO

Interfering Criteria

- C/I:			19 dB (if only C/I, then C/(N+I) and (I+N)/N are not required)

- C/(I+N): 		16 dB (C/(N+I) required only for selected Blocking-Mode)

- (N+I)/N:			3 dB

- I/N:			0 dB

    - Transmitter to Receiver path (ILT - > ILR)

Relative location

- Correlation: 			None

- Path azimuth VLR: 		Uniform distribution 0 to 360 degrees

- Path distance factor:  		Uniform polar 1*cell radius

Coverage radius

- Cell radius mode:		Traffic limited network

- Density:			5

- Nr. of channels:  		80

- Nr. of users per channel: 	1

- Frequency cluster: 		9

Local environment:

- Receiver: 			100% outdoor

- Transmitter: 			100% outdoor

Propagation model

- Model:				Extended Hata

- Variation:			Yes

- General environment: 		RURAL(results in lowest path attenuation)

- Propagation environment:	ABOVE ROOF (with a 30 m TETRA BS-Antenna height)

2. SCENARIO

2.1. VICTIM SYTEM

- Victim System:	 	FM

- Frequency: 		Constant, 412.00625

2.2. SIMULATION CONTROL

- Number of events: 	20 000

2.3. INTERFERING SYSTEM LINKS

- Link1: 			TETRA

- Frequency:  		410.1875…411.9875 MHz, uniform

2.4 ILT -> VLR path

Relative positioning of the interfering link

- Mode:					Uniform density

- Position relative to:			Victim Link Receiver

- Delta X:					Constant, 0

- Delta Y:					Constant, 0

- Set ILR at the centre of the distribution: 	NO

- Path azimuth:				Uniform distribution, 0 to 360 degrees

- Number of active transmitters:		9

- Minimum coupling loss:			Constant, 0

- Protection distance:			Constant, 0

Interferers density

- Density of Tx: 			5 * 1/Km2

- Probability of transmission:		1

- Activity:				Constant, 1

- Time:				1 hour

Propagation Model

- Model:				Extended Hata

- Variation:				Yes

- General environment: 		RURAL(results in lowest path attenuation)

- Propagation environment:		ABOVE ROOF (with a 30 m FM BS-Antenna height)

- RESULTS

Test of the simulation

Calculated radius

- Coverage radius (wanted system): 		7.8 km

- Coverage radius (interfering system 1): 		0.7522 km

- Simulation radius 1:				0.7569 km

Generated Signals

PC “on”

- dRSS: 		mean:	-69.9 dBm,	std:  11.68 dB

- iRSSunwanted: 		mean:  -127.5 dBm, 	std:  10.86 dB

- iRSSblocking: 		mean:  -144.6 dBm, 	std:  7.13 dB

PC “off”

- dRSS: 		mean:	-69.8 dBm,	std:  11.78 dB

- iRSSunwanted: 		mean:  -112.6 dBm, 	std:  10.84 dB

- iRSSblocking: 		mean:  -129.6 dBm, 	std:  7.19 dB

Interference calculation/ICE calculation

- Interference Criteria:			C/I

- Compatibility mode:			Yes (Probability of interference)

- Unwanted emissions:			Yes

- Blocking:				Yes

Results

PC “on”

- Probability of interference:  	1 %

PC “off”

- Probability of interference: 	8.4 %

: List of references

- 3GPP TS 36.101, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception

- 3GPP 36.104, Evolved Universal

- 3GPP TS 25.104 (Release 7), Base Station (BS) radio transmission and reception (FDD), 2005-12

- Recommendation ITU-R SM.329: Unwanted emissions in the spurious domain

- 3GPP TS 25.101 (Release 7), User Equipment (UE) radio transmission and reception (FDD), 2005-12

- ECC Report 82: Compatibility study for UMTS operating within the GSM 900 and GSM 1800 frequency bands

- ECC Report 96: Compatibility between UMTS 900/1800 and systems operating in adjacent bands

- WCDMA for UMTS by Holma and Toskala (Wiley)

- ERC Report 68: Compatibility studies in the band 5725-5875 MHz between Fixed Wireless Access (FWA) systems and other systems

- Radio Regulation RR.1.146

- Nicholas Metropolis (1987), "The beginning of the Monte Carlo method", Los Alamos Science (1987 Special Issue dedicated to Stanislaw Ulam): 125–130, http://library.lanl.gov/la-pubs/00326866.pdf

- 3GPP - TR 36.942: LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios.

- ITU-R Report SM. 2028-1: Monte Carlo simulation methodology for the use in sharing and compatibility studies between different radio services or systems.

- “Other-Cell Interference in Cellular Power-Controlled CDMA”, Andrew J. Viterbi, Audrey M. Viterbi, and Ephraim Zehavi, IEEE Transactions on Communications, vol.42, No. 2/3/4,  p1501-1504, 1994.

- Recommendation ITU-R P.1411-7, Propagation data and prediction methods for the planning of short-range outdoor radiocommunication systems and radio local area networks in the frequency range 300 MHz to 100 GHz

- ECC Report 159, Technical and operational requirements for the possible operation of cognitive radio systems in the ‘white spaces’ of the frequency band 470-790 MHz

- ECC Report 138, Measurements on the performance of DVB-T receivers in the presence of interference from the mobile service (especially from UMTS).

- EN 300 113-1 (2011-11), Electromagnetic compatibility and Radio spectrum Matters (ERM); Land mobile service; Radio equipment intended for the transmission of data (and/or speech) using constant or non-constant envelope modulation and having an antenna connector; Part 1: Technical characteristics and methods of measurement

- ERC Report 101, A comparison of the minimum coupling loss method, enhanced minimum coupling loss method, and the Monte-Carlo simulation

- ECC Recommendation (02)05: Unwanted emissions

- 3GPP TS 45.005, GSM/EDGE Radio transmission and reception

- Recommendation ITU-R F.758, System parameters and considerations in the development of criteria for sharing or compatibility between digital fixed wireless systems in the fixed service and systems in other services and other sources of interference

- Recommendation ITU-R F.699, Reference radiation patterns for fixed wireless system antennas for use in coordination studies and interference assessment in the frequency range from 100 MHz to about 70 GHz

- Recommendation ITU-R F.1245, Mathematical model of average and related radiation patterns for line-of-sight point-to-point fixed wireless system antennas for use in certain coordination studies and interference assessment in the frequency range from 1 GHz to about 70 GHz

- Recommendation the ITU-R F.1336, Reference radiation patterns of omnidirectional, sectoral and other antennas for the fixed and mobile service for use in sharing studies in the frequency range from 400 MHz to about 70 GHz

- ETSI EN 302 217-2-2, Fixed Radio Systems; Characteristics and requirements for point-to-point equipment and antennas; Part 2-2: Digital systems operating in frequency bands where frequency co-ordination is applied

- ECC Report 100, Compatibility studies in the band 3400- 3800 MHz between Broadband Wireless Access (BWA) systems and other services

- EN 300 220: Short Range Devices (SRD); Radio equipment to be used in the 25 MHz to 1000 MHz frequency range with power levels ranging up to 500 mW;

- EN 300 422: Wireless microphones in the 25 MHz to 3 GHz frequency range;

- EN 300 328: Wideband transmission systems; Data transmission equipment operating in the 2.4 GHz ISM band and using wide band modulation techniques

- EN 300 440: Short range devices; Radio equipment to be used in the 1 GHz to 40 GHz frequency range; Part 1: Technical characteristics and test methods

: Glossary

Terminology used in the software and in this manual is:

| 3GPP | The 3rd Generation Partnership Project prepares, approves and maintains globally applicable technical specifications and technical reports for the evolved 3rd generation and beyond mobile system known as UMTS and LTE/E-UTRA. 3GPP also maintains the technical specifications and technical reports for GSM, including GPRS and EDGE. |
| --- | --- |
| 3GPP2 | The 3rd Generation Partnership Project 2 for the North American and Asian interest. |
| ACIR | Adjacent Channel Interference Ratio is defined as the ratio of the power of an adjacent-channel interferer, to the power measured after a receive filter in the adjacent channel and is a result of both transmitter and receiver imperfections. |
| ACLR | Adjacent Channel Leakage (Power) Ratio. The ACLR of a signal is defined as the ratio of the signal’s power to the power of the signal when measured at the output of a (nominally rectangular) receiver filter centred on an adjacent frequency channel. |
| ACS | Adjacent Channel Selectivity (ACS) is a measurement of a receiver's ability to process a desired signal while rejecting a strong signal in an adjacent frequency channel. ACS is defined as the ratio of external interference to the interference appearing at a receiver input. |
| Active list (CDMA module) | Different for each UE. When a UE is dropped in the network, it connects to all the BS (i.e. 19 or 57). The connect procedure involves the calculation of the path loss. The UE sort all the connections by path loss so that the first item of the list is the one with the shortest path loss. In the CDMA module only two BSs which are within the handover margin can be added to the active list. In the OFDMA module, there is no limitation on the number of BSs |
| C | Desired (wanted) signal received by the Victim Link Receiver (VLR) |
| BS | Base Station of a cellular system. |
| CDMA | Code Division Multiple Access (CDMA) is a multiple access scheme for digital radio, to send voice, data, and signalling data between mobile phones and cell sites. CDMA channels are defined with codes and permit many simultaneous transmitters on the same frequency channel. |
| Coverage Radius | Radius of the coverage where receivers are distributed around the transmitter of the same link |
| e.i.r.p. | Effective isotropic radiated power is the amount of power that would have to be emitted by an isotropic antenna (that evenly distributes power in all directions and is a theoretical construct) to produce the peak power density observed in the direction of maximum antenna gain. |
| Interfering link: | Link that interferes with the Victim Link Receiver (VLR). It is composed by Interfering Link Transmitters (ILT) and Interfering Link Receivers (ILR) |
| Interfering link receiver (ILR) | Receiver within the considered Interfering Link. |
| Interefering link transmitter (ILT) | Transmitter within the considered Interfering Link |
| I | Interfering signal received by the Victim link receiver |
| LTE | Long Term Evolution is the name given to a project within the Third Generation Partnership Project (3GPP) to enhance the Universal Mobile Telecommunications System (UMTS) mobile phone standard. The resulting E-UTRA (Evolved Universal Terrestrial Radio Access) radio interface is based on orthogonal frequency division multiple access (OFDMA) for the downlink and single carrier frequency division multiple access (SC-FDMA) for the uplink. |
| N | Noise floor |
| OFDM | Orthogonal Frequency-Division Multiplexing (OFDM) is a digital multi-carrier modulation scheme, which uses a large number of closely-spaced orthogonal sub-carriers. Each sub-carrier is modulated with a conventional modulation scheme (such as quadrature amplitude modulation) at a low symbol rate, maintaining data rates similar to conventional single-carrier modulation schemes in the same bandwidth. |
| Optimal Capacity (CDMA module) | The number of UEs per cell that a given CDMA network is able to serve under “ideal” conditions without any external interference |
| Simulation radius | Radius of the area where Interefering link transmitters (ILTs) are distributed around the Victim link receiver (VLR). |
| Victim link | Link containing transmitters (VLT) and receivers (VLR) of the system which is interfered with by Interfering Link Transmitters (ILT) |
| Victim link receiver (VLR) | Receiver within the considered Victim Link |
| Victim link transmitter (VLT) | Transmitter within the considered Victim Link |

# Index

A

ACIR	171, 360

ACLR	360

ACS	360

Antenna

Gain	108, 332

Pattern	330

B

bandwidth	38

bandwidth correction factor	30

Blocking	14

iRSSblocking	38, 86

Response	83

C

C/I	36

CDMA

Algorithm	353

Capacity finding	161, 162

Emission mask	299

Non-interfered capacity	161, 353

Outage	209

Power control	131

Reference Cell	137

Closest interferer	344

Compatibility calculation mode	39, 82

Consistency check	80, 84, 114, 281

Coverage radius	72, 92, 341

D

Debug mode	56

Delta X/Y	119

Desensitisation	37

dRSS	30

Calculation	72, 283

User defined	283

E

Emission

Bandwidth	75

Mask	85, 292

Unwanted	37

F

Filtering	360

Floor

Uwanted emission floor	295

Frequency cluster/re-use	92

I

interference criteria	36

Interfering transmitter	75

Intermodulation	288

Response	113

iRSS	30

Calculation	78, 285

iRSSintermodulation	288

iRSSunwanted	85

M

Monte Carlo	14

N

Noise floor	34, 88, 113, 450

Noise-limited network	341

O

OFDMA

C/I calculation	170

Look-up table	170

LTE	164

P

Path azimuth	336

Path distance factor	100, 119, 183, 188

Postioning of the Vr vs It	77

power control	105, 117, 283, 285, 286, 288, 289, 292, 345, 450

Propagation model

Extended Hata	376

Extended Hata - SRD	381

Free space	374

ITU-R Rec. P.1546-1	391

Plug-in	374

Protection distance	101, 184, 185, 186

Protection ratio	88

S

Sensitivity	34, 39, 87

sensitivity mode	87

Simulation report	27

T

Traffic-limited network	341

Translation calculation mode	39, 83

U

user defined mode	86

W

Wanted signal	33

Wanted signal margin	34

History

| Document history |  |  |
| --- | --- | --- |
| Edition 1 | January 2010 | Published and printed |
| Edition 2 | April 2016 | Published |

|  | Nyropsgade 37, 4th floor 1602 Copenhagen, Denmark | Tel: + 45 33 89 63 00 | E-mail: seamcat@eco.cept.org Website: www.cept.org/eco |
| --- | --- | --- | --- |

Event, snapshot and simulation trial have the same meaning in this report.

It should be noted that certain scenarios involving aeronautical or satellite systems can also be modelled if suitable propagation models are used.

Cellular systems with multiple receivers (downlink) or multiple transmitters (uplink) are handled differently

Available only if simulated

the order is given by the sum of the absolute values of the coefficients, here 2 + |1| = 3

This algorithm was introduced in SEAMCAT 5.0.1

requires the bounds of  the bandwidth

Depending on the LTE channel width

The System noise power density shall be obtained from the following equation:

N0 (dBW/Hz)= F(dB) + 10*log10(kT0) 
where : T0 = 290 K,  F is the noise figure and k is the Boltzmann constant. This is equivalent to:
N0 (dBW/Hz) = F(dB)  204 (dBW/Hz) or N0(dBm/MHz) = F(dB)  114 (dBm/MHz)

The difference between the peak gain values of stations FS1 and FS2 comes from the fact that a typical PP FS station with high elevation angle (10°) is usually employed for short-range links near hilly areas.

Tilt can either be specified by the user (e.g. a mobile base station) or result from the selection of antenna pointing in elevation on one of the links (e.g. a fixed link with different Tx and Rx heights)

Within a system link, uncorrelated means the “Correlated distance” tickbox is not selected. For the interfering link to victim link path, the “None”, “Uniform” and Closest” are uncorrelated.

With 380 UEs in the system and a tolerance of 5%– no more than 19 UEs can be dropped during power balancing if trial is to be considered successful

As more than 80% is already successful there is no need to simulate more trials

The level of unwanted emissions is supposed to be constant for frequency offset higher than 500kHz