Doc. FM(16)150 Annex 1

ECC Report XYZ

Survey about the Technical and Regulatory Aspects and the Needs for Spectrum Regulation for UAS

Approved Month 2017

# Executive summary

Recent years have seen massive development in UAS technology and the market for civil UAS is showing exponential growth and as with all other significant new technologies, there are a number of challenges in fully realising the potential for growth that UAS bring with them. One of these challenges are the spectrum requirements for drones. Frequencies are used for remote control and identification of drones and also for payload transmissions (e.g. onboard cameras sending information to the ground).

ECC conducted in 2015 a survey on the civil use of UAS pilotless aircrafts. The purpose of this questionnaire was to collect available information from CEPT administrations. The questionnaire was also made available to Organisations which have an MOU/LOU with the ECC including ICAO, EASA and other competent organisations.

Other activities in this field included ADCO (Group of Administrative Co-operation under the R&TTE Directive) who conducted a campaign on remotely piloted aircraft systems (RPAS).

In order to get proper information the questionnaire was split into 2 parts:

Part 1: Spectrum regulation part: this part is focusing on current and planned spectrum regulation as well as existing problems and interference cases and is intended to be filled out by CEPT administrations.

Part 2: Aeronautical regulation part: this part is focusing on the aeronautical definitions, regulations, requirements and operational scenarios of such UAS and is intended to be filled out by stakeholders including air traffic management organisations, users, industry, etc.

This ECC Report provides therefore also relevant information from the aeronautical side.

An assessment of the responses to the questionnaire on drones has been established.

It was questioned whether the development of a new ECC Recommendation on frequency bands for RPAS would be necessary. This ECC Report provides guidance to CEPT administrations and industry on frequency bands which would be suitable for drones, both control/command and payload, and that such a Recommendation could be supportive in this respect. However, before this can be considered in detail, the ECC Report should be available, at least as a stable draft.

In total, 58 responses were received at the European Communications Office (ECO), 30 from CEPT administrations and 28 from stakeholders.

Note: ETSI SRDoc under preparation

Possible subjects:

5030-5091MHz, according to note 5.443C of Radio Regulation(WRC 2012)

WRC12 designated the 5030-5091 MHz band for UAS radio applications. As the national usage of this band by microwave landing systems (MLS) is only in the UK, it can be envisaged to open the 5030-5091 MHz band for professional UAS use/ mission critical UAS use.  There is a need of harmonisation at least at CEPT level for the frequency band (primarily the 5030-5091 MHz band) and coordination of RloS (radio line-of-sight) equipment in conjunction with a harmonized ETSI standard.

5.443C: The use of the frequency band 5030-5091 MHz by the aeronautical mobile (R) service is limited to internationally standardized aeronautical systems. Unwanted emissions from the aeronautical mobile (R) service in the frequency band 5030-5091 MHz shall be limited to protect RNSS system downlinks in the adjacent 5010-5030 MHz band. Until such time that an appropriate value is established in a relevant ITU-R Recommendation, the e.i.r.p. density limit of −75 dBW/MHz in the frequency band 5010-5030 MHz for any AM(R)S station unwanted emission should be used. (WRC-12)

ICAO is in progress of standardising/harmonising the use for aeronautical systems such as freight/cargo/ passenger airplanes. Hence, it could be difficult for drones.

Also: See Report from Denmark: 5150-5250 may be an opportunity.

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	6

2	General UAS description	7

2.1	Types of radiocommunications links	7

3	who responded	8

4	Spectrum Regulation Part	9

4.1	Existing national regulatory framework	9

4.2	Spectrum harmonisation measures	17

4.3	Spectrum use (categorisation, differentiation)	21

4.4	Existing problems such as interference	31

4.5	Additional information	35

5	Aeronautical regulation part	37

5.1	Existing regulatory framework	37

5.2	Categorisation of UAS	47

5.3	Existing problems during the operation/test of UAS	54

5.4	Additional information	59

6	EU legislation in progress	61

7	ADCO Market surveillance campaign	62

8	New COMMISSION REGULATION (EU) laying down rules as regards unmanned Aircraft operation	63

8.1	Terminology used	63

8.1.1	EU, EASA and ICAO	63

8.1.2	ITU	64

8.1.3	Classification of air spaces	64

8.2	New key principles	65

8.3	European Aviation Safety Agency	65

8.4	Market forecast	67

8.5	Proposed new regulation	67

A. Requirements for Unmanned Aircraft	67

B. Compliance of Unmanned Aircraft	68

C.  Market Surveillance Mechanisms	68

D.  Delegated Acts	68

8.6	Third Party Liability Issues and Security	68

8.7	Privacy Issues	69

8.8	On-going process towards the new regulation	69

9	Frequency considerations for professional drones	71

10	Conclusions	75

ANNEX 1: List of reference	76

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| ADCO |  |
| CEPT |  |
| EASA |  |
| EC |  |
| ECC |  |
| ECO |  |
| ERM |  |
| ETSI |  |
| ICAO |  |
| LOU |  |
| MOU |  |
| RED |  |
| RPAS |  |
| R&TTE |  |
| TCAM |  |
| UAS |  |

# Introduction

Recent years have seen massive development in UAS technology and the market for civil UAS is showing exponential growth and as with all other significant new technologies, there are a number of challenges in fully realising the potential for growth that UAS bring with them. One of these challenges are the spectrum requirements for drones. Frequencies are used for remote control and identification of drones and also for payload transmissions (e.g. onboard cameras sending information to the ground).

The focus of the present Report is on the civil use of UAS pilotless aircrafts.

# Other activities in this field include ADCO (Group of Administrative Co-operation under the R&TTE Directive) who is having a campaign on remotely piloted aircraft systems (RPAS). Several ECC administrations confirmed interest for such a campaign.

# General UAS description

# Report ITU-R M.2171 (12/2009).

Deployment of unmanned aircraft systems (UAS) will require access to both terrestrial and satellite spectrum.

# Communications are key in UAS systems due to the remote nature of human presence. Safety-of-flight is the driving factor when the seamless flight of UAS within civilian air traffic is at stake. In the end, safe operation of UAS relies on communications which represents a critical step in enabling UAS operations in non-segregated airspaces.

# This report focusses on UAS at typical heights above ground up to 762 m (comes from the segmentation of the air space).

## Types of radiocommunications links

The Report ITU-R M.2171 defines for safe operations of UA under LoS and BLoS conditions, three types of radiocommunications between the UA and the UACS are required, which are as follows:

- radiocommunications in conjunction with air traffic control relay (depending on the area of operation) ;

- radiocommunications for UA command and control;

- radiocommunications in support of the sense and avoid function.

It is left to the UA system designer to combine two or more of these three radiocommunications into a common physical link.

Command and control is the typical link between the UACS and the UA. The following two ways of communications are:

•	The uplink: To send telecommands to the UA for flight and navigation equipment control.

•	The downlink: To send telemetry (e.g. flight status) from the UA to the UACS. It is anticipated that in some flight conditions or in specific airspaces it could be necessary to downlink video streams. Such a requirement could lead to data rates of several hundreds of kbit/s per UA.

# In areas under the responsibility of the aeronautical authorities, it is expected that the command and control communications will have to be compliant with ICAO standards to be further specified on this function. Nevertheless, in the periods where the UA will follow a full autonomous flight, the up and down links could have very low data rates.

# Sense and avoid

# who responded

Responses (total of 58):

CEPT administrations (30):

Albania, Austria, Belgium, Bosnia Herzegovina, Croatia, Cyprus, Czech Republic, Estonia, Finland, France, Georgia, Germany, Greece, Hungary, Ireland, Italy, Latvia, Lithuania, Montenegro, Netherlands, Norway, Portugal, Russian Federation, Serbia, Slovakia, Slovenia, Spain, Sweden, Switzerland, United Kingdom

Stakeholders (28):

Note: for the aeronautical regulations part, some national authorities have been included by CEPT administrations in their answers.

Aerodromo Municipal de Portimão (POR), Agencia Estatal de Seguridad Aérea (E), Airfilms Productions B.V. (NED), Belgian Civil Aviation Authority –BCAA, Bundeskommission Modellflug im DAeC -EMIG-RC, Chairman- (D), Civil Aviation Authority (CZE), Civil Aviation Agency (LVA), Dep. of Civil Aviation (CYP), DFS Deutsche Flugsicherung GmbH (D), Direction Generale de l'Armement –DGA (F), European Aviation Safety Agency – EASA, Euro USC International (G), Switzerland(DETEC), Federal Supervisory Authority for Air Navigation Services (D), Finavia Corporation (FI), The Finnish Border Guard (FI), Air navigation services Guardia Nacional Republicana (POR), Kantonspolizei Bern (SUI),  IPQ - CT190 - Individual Expert (POR), Robonic Ltd Oy (FI), Selex (I), Slovenia Control LTD, Skeye BV (NED), Swiss Federation of Civil Drones – SFCD, Telespazio (I), Trimble (BEL),  Unidade Especial de Polícia - Núcleo de Meios Técnicos e Audiovisoais (POR), VTO Technologies Ltd (G),

To note:

A national report from Denmark on the future regulation of civil drones March 2015 is added at the end of part 1 of this summary.

# Spectrum Regulation Part

## Existing national regulatory framework

- Question 1: Is there already a national regulatory framework 
(or reports available on civil UAS)?

| Albania | Yes |
| --- | --- |
| Austria | Yes |
| Belgium | Yes |
| Bosnia Herzegovina | No |
| Croatia | No |
| Cyprus | No |
| Czech Republic | No |
| Estonia | No |
| Finland | No |
| France | No |
| Georgia | No |
| Germany | No |
| Greece | Yes |
| Hungary | No |
| Ireland | No |
| Italy | No |
| Latvia | Yes |
| Lithuania | No |
| Montenegro | No |
| Netherlands | Yes |
| Norway | No |
| Portugal | No |
| Russian Federation | No |
| Serbia | No |
| Slovakia | No |
| Slovenia | No |
| Spain | No |
| Sweden | No |
| Switzerland | Yes |
| United Kingdom | Yes |

Assessment of WGFM CG Drones:

Several national regulatory frameworks already exist. Some administrations under ‘No’ indicate to be at drafting stage of a national regulatory framework. There are differences amongst them. See also answers to question 10 (part 2).

EASA proposes to create common rules for the operation of drones in Europe.

TCAM subgroup: what is under RE-D and what is under EASA regulation?

As for drones applying the strict interpretation – other interpretations are also possible due to several defined exclusions in both directives - would mean that drones with a weight above 150 kg would fall outside the RED and be covered by the Regulation (EC) No. 216/2008 (only). Drones with a weigh of 150 kg and below are covered by the RED (only). Lowering this weight limit (as this is under discussion, some proposals even go down to 0 kg) would therefore have an effect on the application of the two directives. . The outcome of such discussions could have some impact on the way how frequencies for drones are authorised.

A lot of information delivered in the responses to the questionnaire is not related to spectrum issues.

- Question 1.1: If so, have you identified any challenges within the existing national 
regulatory framework? 
(Please provide additional information including links/documents available)

| Austria | Only for the aeronautical part; https://www.austrocontrol.at/jart/prj3/austro_control/main.jart?rel=en&content-id=1380112440527 (partly in German) |
| --- | --- |
| Albania | http://www.akep.al/en/legislation/regulation Regulation for conditions for use of frequencies and technical requirements for radio equipment exempted from individual authorization |
| Greece | The existing regulatory framework consists of the implementation of the provisions within the EC Decision (2006/771/EC as amended by 2013/752/EU) and the ERC RECOMMENDATION (70-03) with respect to the frequency ranges for model control in the framework of short range device regulation No challenges have been identified |
| Latvia | Existing national regulatory framework covers only recreational and sport model aircraft activities. Link to existing national regulation: http://likumi.lv/doc.php?id=141998 Latvia is developing new UAS regulation. |
| Netherlands | The challenge is finding broadband spectrum for the payload (Video) |
| Switzerland | - Flying model control under national regulation RIR 1007-02 - Common UAS models operate in the ISM bands |
| United Kingdom | Bands allocated exclusively to the Aeronautical service are managed by the civil Aviation Authority whose policies on spectrum use are laid out in chapter 5 of CAP722 (http://www.caa.co.uk/application.aspx?catid=33&pagetype=65&appid=11&mode=detail&id=415 ). Other bands are managed by Ofcom. Authorisation in any band allocated to the Aeronautical Service (exclusively or otherwise) is co-ordinated with the CAA. Only Ofcom may authorise the civil use of the radio spectrum. Any authorisation would be consistent the Authorisations Framework. As far as challenges are concerned the following have been identified so far in order ensure safe operations of UAV:- • Finding sufficient appropriately protected spectrum • Compatibility with incumbent systems • Defining the requirement • Developing regionally/globally agreed planning criteria in order to optimise spectrum usage Additional issues may be present for payload but those uses are likely to be met in bands that are not used for command and control purposes. Those bands are not managed by the aviation regulator. Ofcom provides information on frequencies and technical restrictions that apply to the operation of radio-controlled models in OfW311 http://stakeholders.ofcom.org.uk/spectrum/information/licence-exempt-radio-use/licence-exempt-devices/ofw311 |

Assessment of WGFM CG Drones:

NL: some experience with illegal use. Sees also demand for spectrum outside of 2.4 GHz / 5 GHz. NL commissioned a study by research entity about the impact on other users in 2.4 GHz 
(results by May 2016).

Latvia is developing new UAS regulation.

Analyse in more detail links from Austria, Sweden and UK.

- Question 1.2: ’NO’ Remarks

| Czech Republic | There is no special document dealing with frequencis in case of UAS (the only aeronautical regulation is available - see Question 7) |
| --- | --- |
| Estonia | Ministry of Economic Affairs and Communications is drafting new performance passed regulation for UAS |
| Finland | There is no specific regulation, except that frequencies have been indentified; see Question 2 |
| France | No regulatory framework for UAS from a spectrum regulation point of view currently exists |
| Georgia | For the time being no |
| Germany | No |
| Hungary | Hungary has not a national regulatory framework on civil UAS yet. NMHH published a prospectus on frequency usage and licensing aspects of UAS in 2014 (http://nmhh.hu/dokumentum/164476/uav_tajekoztato_v51.pdf ). In order to help users to select the appropriate frequency we summarized the usable frequencies for SRD utilization in table format extracting from our NTFA. |
| Ireland | Not Related to Spectrum Matters. |
| Italy | The Italian administration has not received any request of frequency for drones yet |
| Montenegro | Under study |
| Norway | No, there is no regulatory framework in place for RPAS, related to spectrum use. However, we have the following comment; First of all it is necessary to have a clear understanding of what is covered by EASA and what is not covered by EASA. We believe radio products not covered by EASA should be covered by the Radio Equipment Directive (RED). TCAM is currently looking into some issues regarding the Radio Equipment Directive and the text that we believe is written for exclusion of products that are within the scope of EASA. In particular we believe that products that are exempted from the EASA requirements should be in the scope of the RED. And we believe it will be a good idea to make sure that even products that are under the scope of the EASA requirement should comply with the same set of minimum requirements as other radio products. The reason is that if the aeronautical systems comply with at least the same set of minimum requirements as other radio systems, then it will be easier to perform good compatibility studies when frequency use changes. Based on input from some stakeholders we believe that in some cases professional stakeholders may want to use traditional aeronautical radio systems like for example transponder (for anti-collision/identification) or VHF either on board the drones or from the ground when operating drones. (A drone operator who wishes to communicate to other users of the same airspace may find that the aeronautical communication radio is virtually the only way to warn another airspace user about a potentially dangerous situation.) Our current national regulation on radio equipment essentially states that radio equipment compliant with the EASA requirements may be used on board aircraft. EASA has several mechanisms that a manufacturer may use to demonstrate compliance. The concept of ETSO approval is probably the most convenient method for national authorities and other stakeholders in many cases (Our experience from the traditional aircraft sector is that it is very easy to communicate to people that they must be sure to buy ETSO-approved equipment). However, in many cases the existing/traditional ETSOs are not appropriate for drones since there is no pilot on board the drone that can operate radio equipment on board the drone in the same way as a pilot does in a traditional aircraft. So as a radio administration we wish for a standard or specification that equipment manufacturers can demonstrate compliance with in the near future. We believe European Standards Organizations will not create such standards unless they are asked to do so since they currently believe that this is within the scope of EASA. Therefore, unless EASA has resources to do this, it would be nice if EASA could request the standards organizations to do this work for EASA. If this is not in the scope of EASA, it would be nice if EASA can make it clear for the stakeholders that this is the view of EASA. Modern electronics can have low weight. It would be nice if EASA could point out how to deal with cases where drones/drone operators are using traditional aeronautical radio such as transponders and aeronautical communication radio such as VHF. A transponder in a low weight drone may affect the surveillance system for traditional aircraft. But since the use of transponders in drones have some obvious benefits, we don’t think that it should be forbidden to use transponders in drones. Perhaps the use of such radio products in drones can be dealt with as “specific risk operations”, ref. EASA A-NPA 2015-10? |
| Portugal | No, there is no national regulatory framework for UAS in Portugal. |
| Russian Federation | Only for radio control models |
| Slovakia | No |
| Spain | There is no regulatory framework |

Assessment of WGFM CG Drones:

It will be a good idea to make sure that even products that are under the scope of the EASA requirement should comply with the same set of minimum requirements as other radio products. The reason is that if the aeronautical systems comply with at least the same set of minimum requirements as other radio systems, then it will be easier to perform good compatibility studies when frequency use changes. This would also ensure that there are not any gaps in the regulations, i.e. cases where the defined exclusions leave it open about which regulation should apply. Disadvantage: double application of Directives. Note that EASA regulation from their perspective does not try to ensure the protection of other radio services.

Many CEPT administrations have no national framework or very limited framework. These may benefit from the guidance from CEPT.

- Question 2: Are there specific frequencies (incl. requirements) identified for UAS in your 
NTFA?

| Albania | Yes |
| --- | --- |
| Austria | No |
| Belgium | Yes |
| Bosnia Herzegovina | No |
| Croatia | No |
| Cyprus | No |
| Czech Republic | No |
| Finland | Yes |
| France | No |
| Georgia | No |
| Germany | No |
| Greece | Yes |
| Hungary | No |
| Ireland | Yes |
| Italy | Yes |
| Latvia | Yes |
| Lithuania | No |
| Lithuania | No |
| Montenegro | No |
| Netherlands | Yes |
| Norway | No |
| Portugal | No |
| Russian Federation | Yes |
| Serbia | No |
| Slovakia | No |
| Slovenia | No |
| Spain | No |
| Sweden | No |
| Switzerland | Yes |
| United Kingdom | No |

Assessment of WGFM CG Drones:

Answers mostly informed only about ‘SRD’ regulations used by drones and also 5030-5091MHz, according to note 5.443C of Radio Regulation (WRC 2012).

Some differences noted, e.g. Denmark in 5 GHz w.r.t. DFS and indoor/outdoor 5150-5250 MHz. In general, sometimes, an administration allows more (i.e. has a ‘positive restriction’). With drones, this could potentially lead to interference problems.

- Question 2.1: If so, please provide information in the field below

| Albania | http://www.akep.al/en/legislation/regulation |
| --- | --- |
| Finland | There are frequencies for telecommand equipment for use with scale model aircraft in the frequency band 34.995–35.225 MHz. In addition, collective licence-exempt frequencies are available for wide-band data transmission equipment (WAS/RLAN) in 2,4 GHz and 5,8 GHz. None of these frequency bands are specifically identified as UAS frequencies. Also in the National Frequency Table of Allocations we have the frequency band 5031-5090 MHz allocated for the control and non-payload communications for aeronautical route traffic (passenger / freight). |
| Greece | The following frequency ranges are available for model control (including flying models) and there aren’t any special provisions in place for other types of UAS (eg governmental use): 26.995 MHz, 27.045 MHz, 27.095 MHz, 27.145 MHz, 27.195 MHz (according to 2013/752/EU and ERC REC 70-03) 40.665 MHz, 40.675 MHz, 40.685 MHz, 40.695 MHz (according to ERC/DEC/(01)12 and ERC REC 70-03) 34.995–35.225 MHz (only for flying models, according to ERC/DEC/(01)11 and ERC REC 70-03) |
| Ireland | Insofar as model aircraft are concerned, ComReg is aligned with ECC Recommendation 70-03, which provides for unlicensed operation of model aircraft using specific frequencies and associated technical parameters. Currently, within Ireland, UAS operate largely in harmonised licence exempt spectrum (i.e., 2.4 GHz and 5.8 GHz) Apart from the above allowances, no frequencies have been yet identified for UAS. |
| Italy | 5030-5091MHz, according to note 5.443C of Radio Regulation(WRC 2012) |
| Latvia | 34,995-35,225 MHz 100mW e.r.p. dedicated for aircraft model use only. |
| Netherlands | - 5030 – 5091 MHz - 2300 – 2495 MHz And license exempt spectrum (2,4 and 5 GHz |
| Russian Federation | The Decision of the State Radio Frequency Commission №07-20-03-001 from 07.05.2007 «Allocation of frequency bands for Short Range Devices» harmonization to use by radiocontrol flying models (toys) the following frequency bands: 26.957-27.283 MHz, 28.0-28.2 MHz, 40.66-40.7 MHz and 2400-2483.5 MHz |
| Switzerland | - 34,995 – 35,225 MHz - Band ISM 2,4 GHz and Band ISM 5 GHz |

- Question 2.2: ’NO’ Remarks

| Austria | Currently, they are using SRD and RLAN Equipment. There are specific frequencies planed for the CNPC-Link depending on harmonization measures. |
| --- | --- |
| Czech Republic | There are no frequencies (terms and definitions, (national) footnotes etc.) identified for UAS in the Czech NTFA (Decree No. 105/2010 Coll.) |
| France | No specific frequencies currently identified for UAS. Specific frequencies have been identified for flying models related to EC Decision 2006/771/CE amended as well as Decisions ERC/DEC/(01)11 and ERC/DEC/(01)12 and implemented nationally through French frequency Regulator ARCEP decision n°2014-1263. |
| Georgia | No specific frequencies yet. |
| Germany | No |
| Hungary | There are no specific frequencies for UAS, yet. The study has been started at NMHH about the possible new regulation on usage of frequencies for UAS purposes. |
| Montenegro | Under study |
| Norway | No, there are presently no defined frequency areas for RPAS/UAS in the Norwegian NTFA. |
| Portugal | No, the Portuguese NTFA does not designate spectrum specifically intended for the use of UAS. |
| Slovakia | In the NTFA (2015-2016) of the Slovak Republic we have no such frequencies or requirements identified for UAS applications. |
| Spain | There are not specific frequencies in Spain for UAS |
| Sweden | There are no specific frequencies. UAS are assigned frequencies in the aeronautical bands for ATC. Otherwise UAS use licence exempted bands, e.g. 2.4 GHz and 5 GHz bands |
| United Kingdom | Not specifically but bands are listed along with potential use in the aforementioned CAP722. Additionally bands for the usage of UAS would be reflected in the UK Frequency Allocation Table (http://stakeholders.ofcom.org.uk/spectrum/information/uk-fat/ ) Ofcom provides minimum requirements for the establishment, installation and use of licence exempt short range devices in document IR2030/23 http://stakeholders.ofcom.org.uk/binaries/spectrum/spectrum-policy-area/spectrum-management/research-guidelines-tech-info/interface-requirements/IR_2030-june2014.pdf |

## Spectrum harmonisation measures

- Question 3: Is there a requirement for harmonisation measures needed?

| Albania | No |
| --- | --- |
| Austria | Yes |
| Belgium | Yes |
| Bosnia Herzegovina | No |
| Croatia | No |
| Czech Republic | Yes |
| Dominik Meyer | Yes |
| Finland | Yes |
| France | No |
| Georgia | No |
| Germany | No |
| Greece | Yes |
| Hungary | Yes |
| Ireland | Yes |
| Italy | No |
| Latvia | Yes |
| Lithuania | Yes |
| Montenegro | No |
| Netherlands | Yes |
| Norway | Yes |
| Portugal | Yes |
| Russian Federation | No |
| Serbia | No |
| Slovakia | No |
| Spain | Yes |
| Sweden | Yes |
| Switzerland | Yes |
| United Kingdom | Yes |

Assessment of WGFM CG Drones:

Answers differ very much from each other. It is not clear which specific technical and regulatory harmonised conditions are concerned.

A collection of all the material and helping to identify guidelines/guidance as well as problems with current approaches may be helpful.

Questions raised: should one indicate preferred shared bands for drones? This implies to restrict them from using other frequencies (similar to the ‘ISM-concept’). Higher frequencies above 6 GHz could also be considered in this context (though line-of-sight will be required normally at such frequencies). Relates also to risks involved (shorter or greater operating range, potential to interfere in a given frequency range). One may distinguish between CNPC and payload communications.

- Question 3.1: ’YES-answers’ - Please explain

|  | If so, please indicate which ECC deliverable |
| --- | --- |
| Austria | ECC Decision would be preferred, but ECC Recommendation is also possible. |
| Czech Republic | ECC Recommendation (?) |
| Finland | There is a lot of interest for different kind of applications using UAS. At the moment, there are no harmonized frequencies (other than those that are for SRDs, WLANs etc) that would provide longer link distances between the controller (remote pilot) and UAS. Therefore, we believe that European-wide harmonization for frequencies for UAS is highly required and work towards an ECC Decision or ECC Recommendation should start quickly. |
| Greece | Yes, we believe there should be harmonization measures across CEPT under an ECC Decision for critical (e.g. civil governmental) applications |
| Hungary | WRC12 designated the 5030-5091 MHz band for UAS radio applications. As the national usage of this band by microwave landing systems (MLS) is negligible, the NMHH intends to open the 5030-5091 MHz band for professional UAS use. Nevertheless the Authority does not experience considerable radio equipment manufacturing activity for this band in Hungary. There is a need of harmonization at least CEPT level for frequency band (primarily the 5030-5091 MHz band) and coordination of RloS (radio line-of-sight) equipment in conjunction with a harmonized ETSI standard by the opinion of NMHH. |
| Ireland | There might be a need for harmonization measures. Given the nature of operation of UAS, ComReg is of the opinion that the potential for interference with other systems harmonize these frequencies may be higher than when compared with similar systems which are operational on a ground-based platform only. Therefore, further consideration should be given to implementation of additional harmonization measures – this may serve to mitigate the potential for UAS to cause harmful interference to other systems using similar frequencies. Further, it is noted that many UAS currently operate using licence exempt spectrum. In light of this fact, and given the accelerating rate of growth of RPAS for professional services, ComReg believes that harmonization efforts may require careful consideration in the near future. The nature of the service which harmonizes the UAS, height & range of operation, associated transmit power levels, and coexistence with other systems are all examples of factors which may need to be considered when assessing any need for harmonization. |
| Latvia | Waiting for common technical regulations in EC. |
| Lithuania | ECC recommendations would be helpful at this stage. |
| Netherlands | Yes, harmonization of frequency bands and technical parameters. - A ECC report that describes the radio applications (and spectrum requirements) used by drones; - ECC Recommendation that addresses the identification of (preferred) frequency bands that can be used by drones; - ECC Recommendation, aimed at harmonization implementation measures for Drones in the (preferred) identified frequency bands Note: including the possibility of LSA in frequency bands that are identified for MFCN. |
| Norway | Yes, due to common boarders with neighbouring countries, we consider there to be a need for harmonization of spectrum related to frequency use for RPAS. We would prefer an ECC Recommendation. |
|  | ECC recommendations would be helpful at this stage. |
| Portugal | Yes. The Portuguese Administration considers that a harmonization measure at CEPT level facilitates the regulation for the use of frequencies by UAS. As a first step an ECC Report on UAS would identify, among other subjects, UAS frequency usage, type of application (e.g. control and non-payload communications (CNPC), categorization of the UAS (e.g. by concept of operation, weight, etc.). This ECC Report would give a clear picture of the UAS environment, focusing mainly in the frequency bands, usage scenarios and the corresponding conditions for the use of the spectrum. As a second step, the development of an ECC Decision would identify spectrum and conditions for the use of the spectrum by UAS. |
| Spain | We need harmonized bands below 2ghz for uas. Spain would support to develop a new ecc regulation on this issue |
| Switzerland | Yes, there is a potential requirement for civil UAS with specific technical and regulatory harmonized conditions. |
| United Kingdom | Given that the aircraft will operate globally and in some cases travel between countries or regions there is a need for harmonisation measures. Additionally it has been assumed that global harmonisation will minimise the amount of spectrum required to ensure the safe operation of UAVs. |

Assessment of WGFM CG Drones:

Application of 216/2008 and RE-D (and other directives) for drones still under discussion or can be subject to changes.

Those in favour of a harmonisation deliverable like to see the frequencies identified for drones in a central document and also related technical conditions as well as information about other regulation covered by the document.

14 administrations indicated support for a harmonisation deliverable. An appropriate way forward could be suggested in creating first an ECC Report which outlines the needs, pros and cons of a harmonisation deliverable and then, to be decided at a later stage,  a harmonisation deliverable (ECC/DEC or ECC/REC). The ECC Report should also give an overview of the application scenarios, planned fields of operations.

From the answers to part 2, one can also see some motivations (demand) in favour of a harmonisation deliverables (need for clear rules, common market, ensure air traffic safety, spectrum compatibility, etc.)

NLOS communications may need from the operational viewpoint some harmonisation, if not covered elsewhere (e.g. RR).

Several administrations also report using the AMS(R)S (aeronautical safety) allocated 5030-5091 MHz frequency band.

- Question 3.2: ‘NO-answers’ - Please indicate the reasons

| Albania | There is no requirements about the harmonisation measures because the usage of this frequencies does not need Individual Authorisation. |
| --- | --- |
| France | There are currently no requests from UAS manufacturers for harmonization measures. The current questionnaire is aimed at collecting the manufacturers’ forecast for UAS frequency needs in the near future. |
| Georgia | For harmonisation measures relevant ECC decisions and recommendations are to be implemented. |
| Germany | Not yet. First we need to check how the UAS will be handled by the international aviation law. |
| Italy | The Italian administration has not received any request of frequency for drones yet. |
| Montenegro | Since we still do not have national regulatory framework set up, no requirement for harmonisation are yet needed. |
| Russian Federation | Not yet, due to very limited usage of civil UAS at the current stage |
| Serbia | Requirements will be provided by adopting regulations for UAS |
| Slovakia | We have received no such requirement for harmonization of the measures. |

## Spectrum use (categorisation, differentiation)

- Question 4: Do you have / foresee different types of regulation for different radio applications?

| Albania | any other |
| --- | --- |
| Austria | control and non-payload communications (CNPC) payload (incl. video) |
| Belgium | control and non-payload communications (CNPC) payload (incl. video) |
| Bosnia Herzegovina | any other |
| Croatia | control and non-payload communications (CNPC) payload (incl. video) any other |
| Czech Republic | control and non-payload communications (CNPC) payload (incl. video) |
| Finland | control and non-payload communications (CNPC) payload (incl. video) |
| France | any other |
| Georgia | any other |
| Germany | any other |
| Greece | control and non-payload communications (CNPC) payload (incl. video) |
| Hungary | control and non-payload communications (CNPC) payload (incl. video) any other |
| Ireland | any other |
| Latvia | control and non-payload communications (CNPC) payload (incl. video) any other |
| Lithuania | any other |
| Netherlands | any other |
| Norway | any other |
| Portugal | control and non-payload communications (CNPC) payload (incl. video) any other |
| Russian Federation | any other |
| Serbia | any other |
| Slovakia | any other |
| Slovenia | any other |
| Spain | any other |
| Sweden | any other |
| Switzerland | control and non-payload communications (CNPC) payload (incl. video) any other |
| United Kingdom | control and non-payload communications (CNPC) payload (incl. video) |

Assessment of WGFM CG Drones:

- Question 4.1: ’Control and Non-Payload’ - Please explain

| Austria | This should be harmonised within CEPT |
| --- | --- |
| Czech Republic | There is a need to use licensed (dedicated) frequencies due to the safety reasons (CNPC). |
| Finland | CNPC communications will require more protected radio link than the payload communications, since it is seen as safety related communication. |
| Greece | Currently there are no specific provisions for different type of applications within the UAS scope of applications. General provisions (such as those for WAS/RLANs and non-specific short range devices) apply according the above mentioned EC Decision and ERC Recommendation. |
| Hungary | The Hungarian regulation currently does not distinguish the CNPC, payload and other radio devices in the SRD bands. As more and more UASs will operate there may be a need to separate the payload and the CNPC radio links, and a backup (safety) control channel. According to expert level plan of NMHH the RLoS CNPC radio links may operate in the 5030-5091 MHz band by modified regulation. The payload radio links can operate in the PMSE bands if the usage is defined as PMSE by our understanding. In order to avoid the in-band interference for CNCP a separate band should be found for the backup (safety) control channel. |
| Latvia | Non-specific short range device spectrum could be used for various applications. |
| Portugal | There is no regulation for different radio applications for UAS applications in Portugal. This needs to be further investigated. |
| Switzerland | Objective to identify a dedicated spectrum for CNPC |
| United Kingdom | Given that this integral to the safe operation of the aircraft and the public’s perception of that safe operation spectrum and the systems used for this application will have to meet the requirements of the aviation regulatory authorities which on a local scale would be the Civil Aviation Authority and on a global scale ICAO. As UAS activity increases in the hobby, commercial and professional areas, some exclusive frequencies with co-ordination (license) would be useful for professional UAS operations carrying high value or critical payloads. This could be for broadcasters wanting to use a UAS to cover a large event which would require guaranteed communications for both control (safety) and payload (content). |

Assessment of WGFM CG Drones:

Several replies: there is a need to use licensed (dedicated) frequencies due to the safety reasons (CNPC - control and non-payload communications (CNPC).

Given that this integral to the safe operation of the aircraft and the public’s perception of that safe operation spectrum and the systems used for this application will have to meet the requirements of the aviation regulatory authorities which on a local scale would be the Civil Aviation Authority and on a global scale ICAO.

As UAS activity increases in the hobby, commercial and professional areas, some exclusive frequencies with co-ordination (license) would be useful for professional UAS operations carrying high value or critical payloads. This could be for broadcasters wanting to use a UAS to cover a large event which would require guaranteed communications for both control (safety) and payload (content).

Payload (content): not linked to safety aspects. Some administrations mentioned PMSE frequencies in this context. Look to Answers in part 2.

Several administrations have a national framework with various weight categories (list them in a table..)

Compare whether certain categorisations are included either in the spectrum regulation or the aeronautical regulation on national level (e.g. flight height restrictions, exclusion zones (may depend on whether based on air traffic safety or spectrum compatibility)).

Needs more investigation.

- Question 4.2: ’Payload’ - Please explain

| Austria | They may use those frequencies, which are already licenced or foreseen for the relevant user groups (Police Forces, firefighters, TV production). |
| --- | --- |
| Belgium | The control-command link and the payload link are designed in such a way that never ever a failure in the payload link can end up in a failure of the control command link and thus in a crash of the aircraft. It can be done by separate circuits or by other technical solutions. |
| Czech Republic | Unlicensed frequencies (General Authorisation) could be used in specific cases (payload). |
| Finland | CNPC communications will require more protected radio link than the payload communications, since it is seen as safety related communication. |
| Greece | Difference on regulation between payload and non-payload radio communications should be assessed, especially for critical UAS applications, given the fact that payload communication transmissions will take place from high altitude. |
| Hungary | See the answer to question 4.1. |
| Latvia | Non-specific short range device spectrum could be used for various applications. |
| Portugal | There is no regulation for different radio applications for UAS applications in Portugal. This needs to be further investigated. |
| Switzerland | Objective to identify a dedicated spectrum for payload communications |
| United Kingdom | As noted: payload is not linked to safe flight and does not need the same consideration as that spectrum used in support of the control of the UAS itself. Here, to a certain extent, the physical limitations of the UAS may impact its ability to carry a variety bands in support of its payload requirements. As UAS activity increases in the hobby, commercial and professional areas, some exclusive frequencies with co-ordination (license) would be useful for professional UAS operations carrying high value or critical payloads. This could be for broadcasters wanting to use a UAS to cover a large event which would require guaranteed communications for both control (safety) and payload (content). |

- Question 4.3: ’Any other’ - Please specify

| Albania | No |
| --- | --- |
| Bosnia Herzegovina | All depends on requests we receive. So far, we had none |
| France | There are currently no such provisions, as long as the different UAS stations comply with the national frequency regulation. In the future, some frequency bands may be identified for Command and Control functions, but only for operations implying a certain level of aeronautical safety to be defined by the national aeronautical authority. |
| Georgia | We each radio applications should be considered. For the time being, with this regard no regulation exists but in future ECC appropriate deliverable should be used for harmonisation purposes. |
| Germany | Not yet, (see Q2) |
| Hungary | See the answer to question 4.1 |
| Ireland | No regulations currently exist within Ireland for UAS. However, ComReg will continue to monitor national and international developments in this area, and may review the need for regulatory requirements should spectrum usage needs within this field evolve in the future. |
| Latvia | Non-specific short range device spectrum could be used for various applications. |
| Lithuania | Under consideration. |
| Netherlands | Due to the technical neutral spectrum regime and convergence our policy is to have as few types of regulation as possible in our national frequency table. |
| Norway | Today the use of frequencies for the different applications is mainly regulated through the regulation “Regulations concerning general authorizations for the use of radio frequencies”(License exempt frequencies), but nothing is RPAS-specific. We also have RPAS operators who apply for a license to be able to use frequencies exclusively for RPAS operations/tests. However, there has only been issued a very limited number of licenses for this purpose(in VHF-/UHF-bands). But the demand is increasing. We have had several enquiries regarding the use of frequencies reserved for amateur radio use. The enquiries mainly contain questions regarding amateur license holders and the possibility of using these frequencies for RPAS control. |
| Portugal | There is no regulation for different radio applications for UAS applications in Portugal. This needs to be further investigated. |
| Russian Federation | No. The question is under consideration. |
| Serbia | Currently there are no regulations for UAS. The regulation shall be in accordance with the ITU and CEPT regulations for all types of applications. |
| Slovakia | No, we have not special frequency spectrum regulation for UAS in Slovak Republic. Into the future we will keep implementations of the CEPT/ECC common regulation for UAS. So far, we have only one general authorization No. VPR-15/2012 on use frequencies for the operation by radio equipment dedicated for remote control models purpose in the air, on land or over or under the water surface - in line with Annex 8 of the ERCREC 70-03. For different types radio applications we plan same type of regulation. |
| Slovenia | No, NTFA can be used |
| Spain | No |
| Switzerland | Not yet, discussions have just started. Focus is on non-line of sight flights. |

Assessment of WGFM CG Drones:

- Question 5: Do you have / foresee different types of regulation for different concepts of 
operation? Please explain

| Albania | any other |
| --- | --- |
| Belgium | visual line-of-sight / non visual line of sight safety aspects user groups range of operation |
| Bosnia Herzegovina | any other |
| Croatia | weight and/or dimensions visual line-of-sight / non visual line of sight safety aspects user groups range of operation any other |
| Czech Republic | visual line-of-sight / non visual line of sight user groups range of operation |
| Finland | weight and/or dimensions visual line-of-sight / non visual line of sight user groups |
| France | any other |
| Georgia | safety aspects range of operation |
| Germany | any other |
| Greece | any other |
| Hungary | safety aspects any other |
| Ireland | weight and/or dimensions visual line-of-sight / non visual line of sight safety aspects user groups range of operation any other |
| Latvia | weight and/or dimensions visual line-of-sight / non visual line of sight safety aspects user groups range of operation any other |
| Lithuania | any other |
| Netherlands | any other |
| Norway | weight and/or dimensions visual line-of-sight / non visual line of sight safety aspects user groups range of operation |
| Portugal | weight and/or dimensions visual line-of-sight / non visual line of sight safety aspects user groups range of operation any other |
| Russian Federation | any other |
| Serbia | any other |
| Slovakia | any other |
| Slovenia | any other |
| Spain | any other |
| Switzerland | any other |
| United Kingdom | weight and/or dimensions visual line-of-sight / non visual line of sight safety aspects user groups |

- Question 5.1: ’weight and/or dimensions’ Please explain

| Finland | There are different types of use of UAS and therefore there may also be a need to regulate the use by different regulations. |
| --- | --- |
| Ireland | See Answer to 5.6. |
| Latvia | Under consideration. |
| Norway | When RPA’s exceeds a certain weight and dimension, there are safety issues to be considered when contemplating which frequency band will be the most appropriate for control of the RPAS. For bigger RPA’s, it will be more crucial to be able to sustain the control communication with the RPAS to avoid larger RPA’s of significant size and dimension, falling down or malfunctioning in other ways in areas where they might do serious damage. |
| Portugal | There is no regulation for different concepts of operation for UAS applications in Portugal. This needs to be further investigated. |
| United Kingdom | We do envisage different regulation for spectrum which will be based on the safety of other aircraft and the public in general however that will involve other aspects of operation. The primary driver will be safety and ensuring that a CNPC link is suitable to meet the aeronautical safety requirements that will be based on the threat posed to other aircraft and the public in general that in turn is dependent on the potential kinetic energy of a UAV and its proximity to another aircraft or the public. |

- Question 5.2: ’Visual LOS/ NLOS’ - Please explain

| Czech Republic | The only visual line-of-sight mode is alloved for civil UAS in the Czech Republic at present |
| --- | --- |
| Finland | There are different types of use of UAS and therefore there may also be a need to regulate the use by different regulations. |
| Ireland | See Answer to 5.6. |
| Latvia | Under consideration. |
| Norway | Related to difference in spectrum use, which frequency bands will be used for VLOS and BVLOS is dependent on the defined distances and altitudes the BVLOS and VLOS RPAS will be able to travel. |
| Portugal | There is no regulation for different concepts of operation for UAS applications in Portugal. This needs to be further investigated. |
| United Kingdom | It is likely that the safety requirements will be the same for UAVs with the same operational function that are operating within line of sight or beyond line of sight but may well operate in different frequency bands due to the nature of operation. |

- Question 5.3: ’Safety-aspects’ - Please explain

| Belgium | Belgian Civil Aviation Authority (BCAA) uses a risk based, operational centric approach. This means that BCAA doesn’t use a categorization of aircraft based on weight or dimensions apart from the 150kg limiting its competence in RPAS business. The national regulation tries to make clear requirements for different type of users like toys players, model aircraft users (non-professional use for recreation and sports) and professional use of aircraft. BCAA handles both visual and beyond visual line of sight operations. |
| --- | --- |
| Georgia | We think first of all , questions 5.3 and 5.5 have to be taken into account. |
| Hungary | As for safety aspects, there is a need for a backup (security) control channel which should operate in a different frequency band from CNPC by opinion of NMHH. |
| Ireland | See Answer to 5.6. |
| Latvia | Under consideration. |
| Norway | The use of widely used frequency bands with very low level of protection such as 2.4 GHz for control of drones is not desirable. We also refer to question 5.1. |
| Portugal | There is no regulation for different concepts of operation for UAS applications in Portugal. This needs to be further investigated. |
| United Kingdom | It is likely that the safety requirements will be the same for UAVs with the same operational function that are operating within line of sight or beyond line of sight but may well operate in different frequency bands due to the nature of operation. |

- Question 5.4: ’User groups’ - Please explain

| Czech Republic | There is a need to use licensed (dedicated) frequencies for commercial use (unlicensed frequencies (General Authorisation) could be used for recreational purposes) |
| --- | --- |
| Finland | There are different types of use of UAS and therefore there may also be a need to regulate the use by different regulations. |
| Ireland | See Answer to 5.6. |
| Latvia | Under consideration. |
| Norway | We do not have regulations in place targeting specific RPAS user groups, but our primary focus would be commercial users, due to recreational users being able to make use of the license exempt frequency bands given by the document “Regulations concerning general authorizations for the use of radio frequencies” for recreational-/hobby purposes. However, given the signs of exponential growth within the RPAS industry, the spectrum needs of non-commercial users will also have to be taken into consideration in the near future, to avoid saturation within the license exempt frequency bands used by the non-commercial users, as they are already heavily used (there are no frequency bands within the license exempt frequency bands that are specifically dedicated to RPAS). |
| Portugal | There is no regulation for different concepts of operation for UAS applications in Portugal. This needs to be further investigated. |
| United Kingdom | It would be challenging and resource heavy for administrations to make a distinction between users group which in turn would influence which spectrum band could or could not be used. Some states may well make a distinction for military usage, which would be out of scope for an ECC deliverable. |

- Question 5.5: ’Range of operation’ - Please explain

| Czech Republic | Range of operation corresponds with weight and dimensions of UAS and its endurance (and vice versa) Weight and/or dimensions: Model aircraft < 20 kg maximum take-off mass (MTOM) and/or UAS < 20 kg MTOM, used solely for recreational purposes are out of CAA (Czech Republic) scope (out of registration, regulation) |
| --- | --- |
| Georgia | We think first of all , questions 5.3 and 5.5 have to be taken into account. |
| Ireland | See Answer to 5.6. |
| Latvia | Under consideration. |
| Norway | No, but for BVLOS operations (and especially long distance operations), satellite communication might be considered a solution, depending on the distances and altitudes the BVLOS RPAS would be able to travel. |
| Portugal | There is no regulation for different concepts of operation for UAS applications in Portugal. This needs to be further investigated. |

- Question 5.6: ’Any other’ - Please specify

| Albania | No |
| --- | --- |
| France | There are currently no such provisions. In the future, there may be some frequency bands identified for Command and Control functions, but only for operations implying a certain level of aeronautical safety. This level will be fixed by the civil aviation authority and not by the frequency regulator. |
| Germany | Not yet, (see Q2) |
| Greece | We foresee different types of regulation for critical (e.g. civil governmental) and non-critical operations For non-critical applications the existing framework (ERC RECOMMENDATION (70-03)) should be sufficient, although studies might be needed to verify this conclusion, given the fact that all relevant systems/ bands included in ERC REC 70-03 had been originally studied under the assumption that frequencies were to be used at ground level. Moreover, for identified critical applications, where the risk of interference to the UAS cannot be negligible, the development of harmonisation measures seems to be necessary. In these cases the use of a dedicated frequency range, used on a shared access basis exclusively for UAS applications, could be considered, preferably by an ECC Decision taking into account potential cross border coordination issues. |
| Hungary | Weight and/or dimensions and the visual line-of-site and non-visual line-of-site control rules of UAS basically belong to Aviation Authority, and are not in focus of spectrum management. Until the national regulation differs from the classification of ITU, we can apply ITU UAS categories can be found in report ITU-R M.2171. |
| Ireland | From a spectrum management viewpoint, ComReg is of the opinion that it is reasonable to anticipate that different types of regulation may be required for different concepts of operation in the future - this may be contingent upon a number of factors including those suggested above, such as the nature of service utilising the UAS, range of operation, height restrictions, and associated power requirements. |
| Latvia | Under consideration. |
| Lithuania | Under consideration. |
| Netherlands | Some concepts of operation require exclusive spectrum rights and some concepts of operation can share the available spectrum. The operator of the UAS has to decide which kind of license he will apply for. |
| Portugal | There is no regulation for different concepts of operation for UAS applications in Portugal. This needs to be further investigated. |
| Russian Federation | No. The question is under consideration. |
| Serbia | Currently there are no regulations for UAS. The regulation shall be in accordance with the ITU and CEPT regulations for all types of applications. |
| Slovakia | For different concepts of operation we have not frequency spectrum regulation for UAS in Slovak Republic. Into the future we will keep implementations of the CEPT/ECC common regulation for UAS. For different concepts of operation we plan same type of regulation – one or more particular general authorizations. General authorization format is well known and proven way (on national level) for implementation decisions and recommendations in the area European frequency spectrum regulations. |
| Slovenia | No |
| Spain | No |
| Switzerland | Not yet, but is foreseen in accordance with international developments. |

Assessment of WGFM CG Drones:

## Existing problems such as interference

- Question 6: Are there existing problems such as interference cases or illegal spectrum use by 
UAS?

| Albania | No |
| --- | --- |
| Austria | No |
| Belgium | Yes |
| Bosnia Herzegovina | No |
| Croatia | No |
| Cyprus | No |
| Czech Republic | No |
| Finland | Yes |
| France | No |
| Georgia | No |
| Germany | No |
| Greece | No |
| Hungary | Yes |
| Ireland | No |
| Italy | No |
| Lithuania | Yes |
| Montenegro | No |
| Netherlands | Yes |
| Norway | No |
| Portugal | No |
| Russian Federation | No |
| Serbia | No |
| Slovakia | No |
| Slovenia | No |
| Spain | No |
| Switzerland | Yes |
| United Kingdom | Yes |

Assessment of WGFM CG Drones:

7 replies said yes;

Illegal use of frequencies;

Over powered transmitters;

Not may reported cases of electromagnetic interference;

See ADCO RED campaign results.

- Question 6.1: YES answer - Please explain the problems(s)

| Belgium | There are cases where the RPAS user tried an illegal use of frequency. Our national CEPT stopped the user before things could go wrong |
| --- | --- |
| Finland | Our Market Surveillance Team performed a measurement campaign where several radio controlled multicopters where tested. The tests are a part of the common European market surveillance campaign. The results were alarming: the tested equipment didn’t pass (exceedings in transmitting power and incorrect operating frequencies). The results of the common campaign will be published by R&TTE ADCO. |
| Hungary | There is no official information about interference cases, but according to the experiences of UAS users the SRD power limits are frequently violated in 433 MHz, 2.4 GHz and 5 GHz bands. |
| Latvia | There have been cases of non EU market video transmitter mounted on UAS, resulting in interference with aviation radar and instrumental landing systems. |
| Lithuania | Interferences from illegal equipment with illegal spectral settings exist. |
| Netherlands | Yes. Illegal use of spectrum. See attached document in question 1. The use of frequencies by drones can have a greater impact on other frequency users. Because the frequencies are used in the air, the interference areas will also be larger |
| Switzerland | Crash of an UAS behind the airfield due to adverse electromagnetic interference. The problems were not known before; the electromagnetic interference must have acted by the UAS as a similar opposite command that led to its destruction. There was only material loss. Electromagnetic fields in a city, a region or underground electric power supplies are very common, therefore this point has to be considered properly. In the meantime, the problem has been solved through software (frequencies used in this case were in the 2.4 GHz) |
| United Kindom | Almost certainly over powered FPV transmitters. The UAS manufacturers seem confused about the 5.8GHz band, the UK acceptable frequencies and power. You can purchase FPV transmitters that cover Bands A-F covering a frequency range of 5645-5945MHz at high power (up to 2W) when the UK allows 25mW e.i.r.p. from 5725-5875MHz. Harmonising this might persuade the manufacturers to limit the transmit frequency range. Due to a lack of harmonisation UAVs, to-date, have been allocated frequencies on an ad-hoc basis by the administration in which the UAV was built and first flown, when that UAV is exported or used in another country the frequencies originally assigned may not be suitable. Should the UAV be then operated it can and has caused interference issues. |

- Question 6.2: ’NO’ Remarks

| Albania | Till now AKEP has not identified any interference cases or illegal spectrum use by UAS. |
| --- | --- |
| Austria | Up to now no interference cases have been reported. |
| Cyprus | Not reported |
| Czech Republic | No interference problems (complaints) indicated by Czech Telecommunication Office. Illegal (spectrum) use assessment is difficult due to the lack of information provided by Civil Aviation Authority. |
| France | No interference cases involving UAS have been reported to the ANFR to this day. |
| Georgia | We haven't yet meet such kind of problems. |
| Germany | There are no reported Problems that can be traced back to the use of UAS. |
| Greece | There are no reported cases of interference |
| Ireland | No Interference cases reported to date. |
| Italy | We have no cases of interference or illegal spectrum, in fact we are not aware of drones working in our country. |
| Norway | No official reports. But there have been enquiries regarding the frequencies given by the document “Regulations concerning general authorizations for the use of radio frequencies”(license exempt frequencies), where users report a lot of activity on the frequencies available for RPAS (the frequencies are not specifically dedicated to RPAS use, but more generalized use). This has also led to interruption of the control function. There have also been verbal reports on illegal use, however, no reports on any communication being disrupted by this alleged illegal use of frequencies. You can find more details on the document “Regulations concerning general authorizations for the use of radio frequencies” in the link below. |
| Portugal | No interference has been reported do far. However, in order to prevent the use of UAS in 2.4 GHz or other bands using more power than the authorised, it is beneficial to define an appropriate framework. |
| Russian Federation | Not yet, due to very limited usage of civil UAS at the current stage |
| Slovakia | We received no one report to eliminate existing problems with regard to interference cases or illegal spectrum use by UAS. |
| Spain | At this moment, we haven´t detected any interference. however, we may have them in the future |

## Additional information

- Question 7: Any further relevant information? If so, please add here.

| Czech Republic | Air Navigation Services of the Czech Republic (ANS) (http://www.rlp.cz/en/Pages/homepage.aspx ) – has issued a supplement X (Unmanned systems) to (ICAO Annex) L2 regulation recently, which “sets binding national requirements for the design, construction, maintenance, modification and operation of unmanned systems that meet the criteria of Annex II to Regulation of the European Parliament and Council Regulation (EC) no. 216/2008, as amended, and is the recommended procedure for the operation of model aircraft with maximum take-off mass not exceeding 20 kg” (not available in English – see here http://lis.rlp.cz/predpisy/predpisy/index.htm and click “L2” and then “Doplněk X” in the 2nd level column right). - Civil Aviation Authority Czech Republic (CAA) (http://www.caa.cz/letadla-bez-pilota-na-palube/unmanned-aircraft) – there is ‘Unmanned Aircraft Department” as a part of this office dealing with authorization (see the application form and instructions – there are items No. 25 and No. 27 concerning frequencies). Although part of the process is a checking of device and verification of the ability of the pilot to control the UAS, no one in the approval process is involved into the verification of the frequencies used. CTO is interested in entering into this process. |
| --- | --- |
| Denmark |  |
| Estonia | We can not decide our plans with regard to the future use of frequencies for UAS before the decisions or recommendations in ECC, ITU and ICAO level are made. |
| France | The protection of privacy should be taken into account when recording data onboard UAS. http://www.cnil.fr/linstitution/actualite/article/article/usages-des-drones-et-protection-des-donnees-personnelles/ |
| Georgia | UAS application should be reflected in our national NTFA. |
| Greece | ΕΕΤΤ Telecommunication Equipment Dept. participated in relevant RTTE ADCO campaign for RPAS Drones. Five (5) types of equipment have been evaluated for administrative and technical compliance. Based on our market survey results most of the products available in Greek market operate at 2.4GHz. Furthermore, it is believed that the proliferation of UAS applications operating within the framework of SRDs (ERC RECOMMENDATION (70-03)) will require technical review on the usage of the respective frequency bands, given the fact that the footprint of spectrum usage will be altered by the UAS applications, manly due to the increased altitude from which these applications operate. Special attention should be given to the effect that UAS applications will have on existing SRD applications sharing the same band. |
| Ireland | Ireland intends to make a contribution to the current ADCO R&TTE Market Surveillance Campaign on Remotely Piloted Aircraft Systems (RPAS), which seeks to determine the compliance level of RPAS available on the European market. |
| Netherlands | The use of public or private LTE-networks. See also: http://www.telecompaper.com/news/radioaccess-demos-drone-over-private-lte-network--1090181 |
| Norway | http://eng.nkom.no/laws-and-rules/regulations . It is not uncommon to experience WIFI-problems when using computers with 2.4 GHz in crowded areas. Since some drone systems also use such widely used frequencies with low protection requirements, it is likely that there have been some incidents related to the use of these frequencies for control of drones, even if we are not aware of any confirmed cases in Norway. |
| Portugal | Beyond spectrum issues, subjects like safety (including state safety), personal data protection and privacy as well as civil responsibility should be addressed by the relevant/competent bodies. Additionally, we would like to inform that in Portugal there are some frequencies available on a license exempt basis for model control (as in Annex 8 of Recommendations 70-03). |
| United Kingdom | Limited tests carried out in the UK for the seventh joint cross-border market surveillance campaign showed that 4 out of the 5 units under test were non-compliant in some way. Full details of the tests are contained in reports submitted during the summer as part of that campaign. Due to the nature of UAS operation and elevated platform, any unwanted/spurious emissions have the potential to radiate over a large area. |

# Aeronautical regulation part

## Existing regulatory framework

- Question 8: Is there already a national regulatory framework or reports available on civil UAS?

| Aerodromo Municipal de Portimão (POR) | No |
| --- | --- |
| AESA (E) | Yes |
| Airfilms Production (NED) | Yes |
| Austria | Yes |
| Belgian Civil Aviation Authority (BCAA) | Yes |
| Bundeskommission Modelflug (D) | Yes |
| Civil Aviation Authority (CZE) | Yes |
| Civil Aviation Agency (LVA) | Yes |
| Croatia | Yes |
| Dep. of Civil Aviation (CYP) | No |
| Switzerland | Yes |
| DFS Deutsche Flugsicherung (D) | Yes |
| DGA (F) | Yes |
| EASA | Yes |
| EuroUSC International (G) | Yes |
| Federal Supervisory Authority for Air Navigation Services (D) | Yes |
| Finavia | No |
| Finland | Yes |
| France | Yes |
| Guardia Nacional Republicana (POR) | No |
| Hungary | Yes |
| IPQ (POR) | Yes |
| Kantonspolizei Bern (SUI) | Yes |
| Lithuania | Yes |
| Montenegro | No |
| Norway | Yes |
| Robonic (FI) | No |
| Selex (I) | Yes |
| Skeye (NED) | Yes |
| Slovenia Control | No |
| Sweden | Yes |
| Swiss Federation of Civil Drones | No |
| Telespazio (I) | Yes |
| The Finnish Border Guard | Yes |
| Trimble (BEL) | Yes |
| Unidade Especial de Polícia - Núcleo de Meios Técnicos e Audiovisoais (POR) | No |
| United Kingdom | Yes |
| VTO Technologies (G) | Yes |

- Question 8.1: If so, have you identified any challenges within the existing national regulatory framework? (Please include links/documents if possible)

| AESA (E) | You can find more information in this link http://www.seguridadaerea.gob.es/lang_en/cias_empresas/trabajos/rpas/marco/default.aspx |
| --- | --- |
| Civil Aviation Agency (LVA) | Existing national regulatory framework covers only recreational and sport model aircraft activities. Link to national regulation: http://likumi.lv/doc.php?id=141998 Latvia is developing new UAS regulation. Planed to be in force by the end of 2015. And this regulation will define requirements for today’s identified users operations. |
| Civil Aviation Authority (CZE) | No specific frequencies nor maximum output power are mentioned in the RPAS regulation. It can be downloaded in English here: https://www4.icao.int/rpas/Documents/Czech%20UAS%20regulatory%20framework%202013-05-30%20-%20ENGLISH.pdf |
| Croatia | http://narodne-novine.nn.hr/clanci/sluzbeni/2015_05_49_974.html |
| DGA (F) | Arrête of april 11th, 2012. Security when UAV fail and fall. Terrorism activity |
| DFS Deutsche Flugsicherung (D) | National regulation is available for UAS below 25kg. The national regulations can found in the attached documents: |
| EASA | Currently, EASA is only responsible for drones with a maximum take of weight in excess of 150 kg, except for military, customs, police, search and rescue, firefighting, coastguard operations as well as similar activities and services. EASA has published an intermediate policy: https://www.easa.europa.eu/document-library/policy-statements/ey013-01. This policy addresses the airworthiness-certification aspects of the drones. The unique characteristics of drones, including the command and control link, are addressed with special conditions. This means that the certification basis is established individually per project. The policy foresees high level safety objectives for the command and control link: 7.2 Command and Control Link Consideration of the following airworthiness factors should be included in the UAS type certification basis: a) The UAS flight crew should be provided with a continuous indication of the command and control link signal strength together with the maximum link range. b) Any single failure in the command and control system (uplink or downlink) should not affect normal control of the unmanned aircraft. c) Uplinks/downlinks are sensitive to electromagnetic interference (EMI). The command and control link, in addition to operating in appropriate frequency band(s), should be adequately protected from this hazard. d) Contingencies for failures or interruptions of the command and control link must be defined by the applicant and evaluated as part of the airworthiness certification. For example: lapse times, intermittent failures, alternate modes of command and control and total loss of command and control link. Currently, below 150 kg, national regulations are applicable. Such regulations can be accessed here on EASA Web site: http://www.easa.europa.eu/unmanned-aircraft-systems-uas-and-remotely-piloted-aircraft-systems-rpas . A summary of worldwide drone regulations can be accessed here: http://drones.newamerica.org . In March 2015, EASA presented its new regulatory approach for safely operating drones. This new approach is called “concept of operations”. This “concept of operations” foresees regulations proportionate to the operational risks (http://www.easa.europa.eu/newsroom-and-events/general-publications/concept-operations-drones ). Such regulations have to be developed and endorsed. |
| EuroUSC International (G) | There is no element of airworthiness assessment of the Remotely Piloted Aircraft Systems (RPAS). This allows sub standard ‘hobby’ type systems to be used in a commercial environment posing significant safety risks. Record keeping is very poor. EuroUSC operate a comprehensive database of registered RPAS along with a safety database of reported occurrences. Operators systems are unchecked with regards to transmission power output by the regulating body left to EuroUSC to advise its candidates. Frequency allocation and transmission is simply not covered. http://www.caa.co.uk/docs/33/CAP%20722%20Sixth%20Edition%20March%202015.pdf Currently, requirements are varied across countries. For example, the UK have no airworthiness requirement where as the Netherlands have rigid requirements including an independent airworthiness assessment of each and every RPAS intended for commercial operations. Transmission power output limits are varied across the countries. The UK allow 2.4GHz @ 100mW whereas France have limited these 2.4GHz transmissions to 10mW. This means that an operator is often forced to make major modifications to their RPAS each time they operate in a different country. |
| Federal Supervisory Authority for Air Navigation Services (D) | A national regulatory framework for civil UAS is the LuftVG and the LuftVO, which implements regulations and definitions for UAS in a very general and high level manner. Special national regulations for spectrum management are envisaged to be included into the Frequenzverordnung, which determines national spectrum use based on radio regulations from ITU. For that regulatory framework, no current experiences |
| Finland | Not yet as our regulatory framework consists of a number of sections in the Aviation Act (864/2014) and a new regulation to be issued soon after 1 October 2015. |
| France | The whole framework is available at the following web link: http://www.developpement-durable.gouv.fr/Quelle-place-pour-les-drones-dans,45924.html Moreover the following administrative orders describe the usage and conception rules for UAS: - Arrêté du 17 décembre 2015 relatif à l'utilisation de l'espace aérien par les aéronefs qui circulent sans personne à bord https://www.legifrance.gouv.fr/eli/arrete/2015/12/17/DEVA1528469A/jo/texte - Arrêté du 17 décembre 2015 relatif à la conception des aéronefs civils qui circulent sans personne à bord, aux conditions de leur emploi et aux capacités requises des personnes qui les utilisent https://www.legifrance.gouv.fr/eli/arrete/2015/12/17/DEVA1528542A/jo/texte |
| Hungary | The Hungarian Aviation Act (ACT XCVII. Of 1997 on Air Traffic) and other decrees contain the fundamental requirements related to UAS civil operation. The detailed national regulatory framework of civil operation is under preparation. The main challenge of the UAS business is to certify any unmanned aircraft system as air vehicle, ground control station and communication link as well as the operator. |
| IPQ (POR) | Just for sports, but only permitted in Certified Airfields, exclusively for aero models. |
| Kantonspolizei Bern (SUI) | http://www.bazl.admin.ch/dienstleistungen/02658/index.html?lang=de |
| Lithuania | Regulation on the Unmanned Aircraft Operations, approved by Order No. 4R-17 of Director of Civil Aviation Administration, 23 January 2014. http://caa.lt/index.php?-1659137123 |
| Norway | The RPAS industry has evolved past the initial regulatory framework (prepared by the Norwegian CAA), and so the existing regulatory framework for RPAS operational issues is being revised and updated, and is currently out on public hearing. The finalized version will be made public during the fall of 2015. The Norwegian radio regulation for radio licenses for Norwegian aircraft relies heavily on the EASA approval concepts, but for the time being we lack the exact requirements for radio products for drones from EASA. Since the general view is that airborne products are covered by EASA, the European Standardization Organisations do not seem particularly interested in developing standards for this purpose since they must see a future for such standards to be willing to use their resources on this task. Therefore we see that radio technologies that were not developed for control of drones are used for control of drones. We are about to revise this regulation, but for the mentioned reasons for the time being we feel that we lack a good solution for drones. The regulatory framework of the Norwegian CAA existing today is considered a minimum-requirement regulatory framework, which will have to adapt to the progress of the RPAS technology and industry continuously. And there has also been made some decisions in conjunction with the development of this regulatory framework, which will have to be revised and compared to the future European regulatory framework, which is under construction. |
| Selex (I) | Radio systems for Identification/Surveillance required by new ENAC rule for small RPAS in low level operations requires dedicated studies. A systems for electronic identification which allows real-time transmission of data concerning the RPAS and the owner / operator and the essential data of flight based on existing ADS-B 1090 MHz ES (Extended Squitter) could be evaluated, but the impact on existing/planned ADS-B Surveillance system for larger aircrafts/RPAS operations requires careful evaluations, as this spectrum is already overloaded in certain airspaces and will become even more in the future. Dedicated impact studies and technical/operational limitations to overcome these should be performed. |
| Telespazio (I) | Existing ENAC regulation issued on 16th March 2015. ENAC regulation on RPAS (MTOW below 150kg) is available at the following links: • https://www.enac.gov.it/La_Normativa/Normativa_Enac/Regolamenti/Regolamenti_ad_hoc/info-122671512.html • https://www.enac.gov.it/La_Normativa/Normativa_Enac/Disposizioni/info593565219.html Concerning RPAS with MTOW> 150kg, Telespazio is working in collaboration with JARUS on the set-up of RCP target parameters for the use of satcom links for RPAS BRLOS Operations, in the context of ESA / EDA funded DeSIRE 2 Project (see attached presentation). Documentation set by the JARUs Group on RCP for C2 CNPC link in BRLOS operations can be found through the following link: http://jarus-rpas.org/index.php/deliverable/category/12-external-consultation-on-jarus-c2-rcp See document attached |
| The Finnish Border Guard | Aviation act and aviation regulations are most important framework for UAS regulation. National RPAS regulation is on draft phase and it is expected that it will be published during 2015. Existing frequency regulations, available frequencies and power limits have been planned mostly for the UAS which are used inside visual line of sight (VLOS). However security authorities have needs to fly beyond visual line of sight (BVLOS). In connection with that there should be available suitable frequency windows and higher power limits for BVLOS activities in order to send high quality command information from ground to air and receive high quality payload and control information from air to ground. Suitable frequencies and powers would decrease aviation risks. Using of those frequencies and powers should be possible also on state border areas. |
| Trimble (BEL) | Yes. The regulations (likely released by the end of 2015) contain limits (e.g. max height limit of 90 m) and operator requirements (logging, licensing) that are more strict than the current EASA proposal. |
| Switzerland | The regulation with regard to unmanned system is the DETEC Ordinance on Special Category Aircraft (OSCA). For the time being, it is not possible to issue a “Type Certificate” for UAS, but that will change in the future. |
| United Kingdom | Yes in CAP722 (see the answer to Question 1 of part 1 for the link) |
| VTO Technologies (G) | Securing BVLOS RPAS frequencies |

- Question 8.2: ’NO’ Remarks

| Dep. of Civil Aviation (CYP) | Draft national regulatory framework exists |
| --- | --- |
| Guardia Nacional Republicana (POR) | There is no regulation available in Portugal for the operation of civil RPAS. |
| Montenegro | Under study. |
| Robonic (FI) | Regulations are under development and to be finished, will be published during autumn 2015 by Finnish CAA (Trafi). They will be concentrated on VLOS operations. |
| Slovenia Control | There is no national regulatory framework |
| Swiss Federation of Civil Drones | Not for frequencies. |

Assessment of WGFM CG Drones:

A national regulatory framework or reports available on civil UAS is available in a considerable number of countries.

Further assessment needed to analyse in more detail what kind of UAS is covered by existing national regulations since some are limited to only certain UAS (e.g. weight limits, only models). Structure the national conditions expressed in these regulations:

1. EASA airworthiness requirements and their applicability (under change process at this moment)

2. Civil/ Non-civil use (definitions, may not be the same for all countries, e.g. in relation to some security applications)

2. control link control and max. operating range limits, also LoS and NLoS

3. contingencies/safe modes in case of interference or any interruption of the control link

4. weight limits

5. exclusion zones (or alternatively, permit to use only at dedicated zones/ airfields)

6. flight height restrictions

7. operator requirements (need to pass an examination before operating certain drones??)

8. Day/night restrictions

9. Restrictions to support enforcement (may include record keeping, databases, electronic identification, other restrictions)

10. Satellite communication links  to/from drones.

- Question 9: Are there specific frequencies mentioned within these regulations?

| Aerodromo Municipal de Portimão (POR) | No |
| --- | --- |
| AESA (E) | No |
| Airfilms Production (NED) | No |
| Austria | No |
| Belgian Civil Aviation Authority (BCAA) | Yes |
| Bundeskommission Modelflug (D) | No |
| Civil Aviation Authority (CZE) | No |
| Civil Aviation Agency (LVA) | No |
| Croatia | No |
| Dep. of Civil Aviation (CYP) | No |
| Switzerland | No |
| DFS Deutsche Flugsicherung (D) | Yes |
| DGA (F) | Yes |
| EASA | No |
| EuroUSC International (G) | No |
| Federal Supervisory Authority for Air Navigation Services (D) | No |
| Finavia | No |
| Finland | No |
| France | No |
| Guardia Nacional Republicana (POR) | No |
| Hungary | No |
| IPQ (POR) | Yes |
| Kantonspolizei Bern (SUI) | No |
| Lithuania | No |
| Montenegro | No |
| Norway | No |
| Robonic (FI) | No |
| Selex (I) | No |
| Skeye (NED) | No |
| Slovenia Control | No |
| Sweden | No |
| Swiss Federation of Civil Drones | No |
| The Finnish Border Guard | Yes |
| Telespazio (I) | Yes |
| Trimble (BEL) | No |
| Unidade Especial de Polícia - Núcleo de Meios Técnicos e Audiovisoais (POR) | No |
| United Kingdom | Yes |
| VTO Technologies (G) | Yes |

- Question 9.1: ’YES-answer’ - Please indicate

| Belgian Civil Aviation Authority (BCAA) | Belgian Civil Aviation Authority states clearly in the regulation that only frequencies accepted by the national CEPT (IBPT-BIPT) may be used by RPAS operators. They need to ask for permission or positive advice before they can use the RPAS for professional use. The frequencies used for toys and model aircraft are covered by the regulatory framework for entering the market place and are in compliance with European Directive for toys and CE markings requirements for products and services put on the European market. |
| --- | --- |
| DFS Deutsche Flugsicherung (D) | Only radio equipment (telemetry equipment) that complies with the provisions governing such equipment may be used. The provisions and orders issued by the Federal Network Agency governing this equipment shall be observed. In case of sustained or repeated (radio) interferences, the Federal Network Agency and the aeronautical authorities are to be informed. |
| DGA (F) | 2.4 GHz |
| IPQ (POR) | The assigned for aero models. |
| Telespazio (I) | ENAC regulations make specific reference to the obligation for the RPAS Data Link to make use of authorized frequencies, without specifying the frequencies range. The JARUS document does not make any specific reference to frequencies. |
| The Finnish Border Guard | According to aviation regulations radio licenses are requested from Communications Regulatory Authority (CRA) but National Civil Aviation Authority (NCAA) conducts radio equipment surveillance in connection with inspections of aircrafts. Also VHF radio rules have to take in to account if aviation radio is used during UAS flights. According to regulations of CRA command and control transmitters can be used on frequency area 5030-5091 Mhz. Other frequencies are considered case by case. License to use possible transmitters of payload have to request separately. According to aviation regulations it is recommended that aviators of aircrafts listen to international emergency frequency 121.5 Mhz but in practice RPAS pilots have rarely possibility to do that. |
| United Kingdom | See chapter 5 for a full explanation but the following frequency bands are mentioned 255 - 526.5 kHz Radionavigation 108 – 137 MHz Radionavigation/Radiocommunications 328.6 – 335.4 MHz Radionavigation 960 – 1 350 MHz Radionavigation/Radar 2 700 – 3 100 MHz Radar 4 200 – 4 400 MHz Radionavigation 5 000 – 5 150 MHz Radionavigation 9 000 – 9 200 MHz Radar 9 300 – 9 500 MHz Radar Note: Radionavigation/radiolocation bands: this does not reflect any specific provisions for UAS yet As well as 35 MHz, 2.4 GHz and 5.8 GHz |
| VTO Technologies (G) | 5030-5091 MHz Note: The national document only reflects the WRC-12 decision. |

- Question 9.2: ’NO’ Remarks

| Civil Aviation Agency (LVA) | In the current regulation it is defined that during radio controlled model flight can be used radio frequencies which are defined in the regulations about radio frequency spectrum for model aircraft flight operations. |
| --- | --- |
| Civil Aviation Authority (CZE) | Usually 2,4 (uplink) and 5,8 (downlink) GHz are used (98%). |
| EASA | EASA intermediate policy (https://www.easa.europa.eu/document-library/policy-statements/ey013-01 ) does not mention specific frequencies and states that “It is reminded that approval for all frequencies used in UAS operations must be obtained from national authorities. This is not part of an airworthiness approval”. |
| EuroUSC International (G) | There are no frequencies currently allocated specifically for commercial RPAS but there is a strong requirement. Many systems utiilise a wide range of frequencies including 2.4GHz, 5.8GHz, 433MHz. The operators of these RPAS often express difficulties in identifying legal limits for power outputs. A simple document is required stating legal frequency and power output limits (mW) for airborne RPAS. The current stakeholder document is not easily interpreted. http://stakeholders.ofcom.org.uk/binaries/spectrum/spectrum-information/UKFAT_2013.pdf 433 – 459MHz frequencies tend to have better range qualities and could be utilised for extended and beyond line of sight flight operations |
| Federal Supervisory Authority for Air Navigation Services (D) | Currently there are no special frequencies within the Frequenzverordung for use of UAS scheduled. The general spectrum for these kinds of frequencies will be a very important topic for future use of UAS in German airspace. For that, control and non-payload communication of UAS in non-segregated airspace is one item on the agenda (Agenda Item 1.5) of the next world radio conference in November 2015 (WRC-15) and also a substantial part of the ICAO-Position for this WRC-15. |
| Guardia Nacional Republicana (POR) | There is no specific frequencies available in Portugal for the operation of civil RPAS. |
| Hungary | There are no specific frequencies in these regulations. The UAS purpose frequencies will be provided within the spectrum regulation framework (see the answers to Questions 1 and 2 in the Spectrum regulation part). |
| Kantonspolizei Bern (SUI) | The BAKOM has to define the use of frequencies. |
| Montenegro | Under study. |
| Robonic (FI) | Regulations are under development and to be finished, will be published during autumn 2015 by Finnish CAA (Trafi). They will be concentrated on VLOS operations. |
| Selex (I) | Only generic requirements on the use of authorized frequencies and chosen in order to minimize voluntary and non-voluntary interferences which could compromise safety (“Il data link deve utilizzare frequenze autorizzate e scelte opportunamente in modo da minimizzare la possibilità di interferenze involontarie e volontarie che possano compromettere la sicurezza delle operazioni”) |
| Slovenia Control | Frequencies are not specified |
| Swiss Federation of Civil Drones | Civil UAS use band in 2.4 and 5.8GHz, often where WLAN is. |
| Trimble (BEL) | They refer to the BIPT |

Assessment of WGFM CG Drones:

The vast majority of the existing national aeronautical frameworks for UAS do not define any specific frequency use for UAS. These documents often simply refer to the national regulatory framework for frequency use, and in particular to the possibilities for UAS operation inside of the existing provisions for generally authorised frequency use.

## Categorisation of UAS

- Question 10: Explain the categorisation of UAS (e.g. by concept of operation, range of operation, weight, visual line-of-sight/ non visual line of sight, safety aspects, user groups, etc.) in use in your country, including important technical requirements

|  | Please explain here and include link/documentation pertinent to your country, if possible |
| --- | --- |
| AESA (E) | There are different types of drones depending on weight (more or less than 150 Kg, 25 Kg,…) |
| Airfilms Production (NED) | Any other Weight and/or dimensions, visual line-of-sight / non visual line of sight http://www.ilent.nl/onderwerpen/transport/luchtvaart/dronevliegers/ |
| Austria | http://www.austrocontrol.at/jart/prj3/austro_control/main.jart?rel=en&content-id=1380112440527 |
| Belgian Civil Aviation Authority (BCAA) | Belgian Civil Aviation Authority (BCAA) uses a risk based, operational centric approach. This means that BCAA doesn’t use a categorization of aircraft based on weight or dimensions apart from the 150kg limiting its competence in RPAS business. The national regulation tries to make clear requirements for different type of users like toys players, model aircraft users (non-professional use for recreation and sports) and professional use of aircraft. BCAA handles both visual and beyond visual line of sight operations. • Safety aspects : BCAA only accepts RPAS operations after approved safety analysis report looking into emergency scenarios and the preventive and corrective actions taken. • The minimal technical requirements are : Art. 46. §1. Every RPAS has to be equipped with : 1° fail-safe system and/or other procedure to stop the flight safely, when needed 2° a tracking system to find back the RPA after landing ; 3° navigation lights ; 4° navigation-instruments adapted to the type of flight and executed in compliance wiht the applicable legislation ; 5° anti-collision lights when the RPAS operates in a zone where other aircraft operate at the same time. §2. A RPAS is developed so that : 1° the circuits for command and control shall never be disturbed by interference or other problem coming from the payload circuit ; 2° the execution of an emergency scenario shall never lead to an uncontrollable crash of the RPA ; 3° if the case, the reaction time of the communication shall never jeopardize the safety of the operations. |
| Bundeskommission Modelflug (D) | There is a primary categorization, the purpose of the mission. It is divided in commercial use and use for sports and recreation (aeromodelling). For both groups only VLOS-operation is permitted. There are no common technical requirements. Commercial use always needs a permission for operation while the aeromodelling area can operate without permission if the mass of the drone (or model) is below 5 kg. From 5 – 25 kg a permission is required but it is linked to the airfield, not to the model or person. Above 25 kg a certification of the model is required. In the area of aero-modelling drones are models like i.e. fixed wing models or helicopters. |
| Civil Aviation Agency (LVA) | In the new UAS regulation there is foreseen categorization by weight: - up to 1.5 kg - more than 1.5. kg up to 25 kg - more than 25 kg |
| Civil Aviation Authority (CZE) | Regulation can be downloaded here: https://www4.icao.int/rpas/Documents/Czech%20UAS%20regulatory%20framework%202013-05-30%20-%20ENGLISH.pdf All aspects are used: VLOS/BVLOS, area of operations (away from people, close to people, >5,5 km from aerodrome or closer), below 100 m AGL in CTR, below 300 m AGL elsewhere, MTOM of 0,91 kg, 7 kg, 20 kg and many other aspects. Kinetic energy is not used directly but is used "behind the scenes" during assessment of safety distances. |
| Croatia | There are 3 different class of UAVs: up to 5 kg, from 5 to 25 kg, and to (including) 150kg |
| Dep. of Civil Aviation (CYP) | Risk based categorisation |
| Switzerland | Swiss FOCA follows a risk-based approach. Operation with low risk are proceeded with no or little regulation. Basically there are two categories: “Open” & “Specific” “Open” = max. 30 kg, 100m outside of crowds, VLOS (Visual line of sight). Open does not require an authorisation from Swiss FOCA. A crowd is defined as a minimum of 24 people standing close together, as on a bus station during rush hours or an open air concert. The rationale is that one cannot escape if surrounded by many people around. It is therefore possible in Switzerland to fly in cities, as long as the 100m distance from crowds is respected. “Specific” = everything else. An authorisation from FOCA is required. A Total Hazard and Risk Assessment is required to evaluate the safety conditions required for the intended operation. These safety requirements can be technical or operational. From the authorisation side there are specific category authorisations for: VLOS closer than 100m to crowds but not direct over the crowd (SIDE approval) VLOS direct over the crowd (OVER approval) VLOS tethered BLOS |
| DFS Deutsche Flugsicherung (D) | See document attached |
| DGA (F) | ANFR Registration is working on this affair. Control and non-payload communcations (CNPC) Both, frequencies must be different. Control UAV is only control UAV and mustn't be jamed by video frequency. Weight and/or dimensions In France we have "Arrête april 11th, 2012" micro < 2 kg ; mini 2 to 20 kg ; >20kg = class 1 150 à 600 kg = class 2 < 600 kg = class 3 |
| EASA | The current European regulation only foresees drones over 150 kg and below 150 kg. The proposed EASA concept of operations anticipates a risk-based approach based on operations. There would be 3 operational categories: • open category without involvement of aviation authorities for low risk operations. This category would have operational limitations: visual line of sight, maximum altitude, minimum distance for airport and sensitive zones. It is likely that this category will be defined by a maximum kinetic energy translated into simple parameters for straightforward interpretation (probably with weight and other factors). • Specific category with an approval from the national aviation authorities possibly supported by qualified entities and/or qualified operator for an increased risk. A safety risk assessment would be required. The operation would be approved on the basis of an operations manual. The airworthiness of the drone and the competence of the staff operating the drone would be based on the outcome of the risk assessment. • Certified category: this category is similar to manned aviation. The limit between “specific” and “certified” categorie and control could receive an independent approval from the certification of the drone. A summary of the current national categories can be accessed here: http://drones.newamerica.org |
| EuroUSC International (G) | The CAA categorise operations by mass (see page 31) http://www.caa.co.uk/docs/33/CAP%20722%20Sixth%20Edition%20March%202015.pdf EuroUSC have their own internal categorisation system which they are happy to share which is based on the concept of operation and pilot competence. 'tablet' programmed, waypoint operated RPAS fit into a different class and carry different pilot skills to a manually operated RPAS. Mass also fits into the equation There are currently International standards in planning which may well create various classes of RPAS split by size and weight of RPAS and types of operation. Small systems tend to be flown at close ranges. Therefore, small hobby typr multi-rotor systems which rarely tend to operate in excess of 150 metres could easily operate on lower power output such as 2.4GHz @ 20mW. This measure may help to alleviate interference for operators of larger systems with higher frequency power outputs. |
| Finavia | - Weight >25 kg - Weight <25 kg - Controlled visually - Controlled non visually |
| Finland | The most important division line is the one between VLOS and BVLOS operations. The type of the operating area (densely populated areas/open-air crowds of persons/vicinity of airports/other) is also very important, as well as the mass of the UA (max 7 kg over densely populated areas and open-air crowds of persons; max 25 kg generally). |
| France | The administrative order « arrêté du 11 avril 2012 relatif à l’utilisation de l’espace aérien par les aéronefs qui circulent sans personne à bord » mentioned above describes a categorization of UAS in categories and operational scenarios according to the mass of the UAS as well as altitude and distance of flight, and population density in the area. http://www.legifrance.gouv.fr/affichTexte.do?cidTexte=JORFTEXT000025834986 |
| Guardia Nacional Republicana (POR) | There is no regulation available in Portugal for the operation of civil RPAS. The national regulatory initiative belongs to the ANAC (Portuguese National Civil Aviation Authority). ANAC held two events (seminars) in 2015 in order to promote national debate on the use of airspace and gather opinions from its stakeholders. Following these initiatives ANAC drafted a legislative proposal which included a chapter on communications and radio frequencies. This chapter of this project has been left to the care of Portuguese authority for communications. ANAC's legislative project for the use of airspace by RPAS was presented to the Portuguese Government and to date has not been approved. GNR, as the most important low-enforcement institution in Portugal, with public safety responsibility in over 94% of the national territory, and also as state aircraft user and major stakeholder, proposed to the Portuguese aeronautic authority to adopt the following categorization for civil RPAS legislation: Category¦Civil Classification¦MTOW (Kg)¦Range (Km)*¦Typical flight altitude (m) ¦Autonomy (Hours) Indicative Values I Micro < 5 < 10 < 250 < 1 II Mini < 25 < 10 < 300 < 2 III Medium < 150 < 70 ** < 3.000 > 3 * The range can be limited by the communications capacity. ** Distance limited by terrestrial communications. In case equipped with satellite communication system, range can be higher. |
| Hungary | - Categorization of Maximum Take Off Weight (MTOW): MTOW < 0,5 kg MTOW = 0,5 < 7 kg MTOW = 7 < 25 Kg MTOW = 25 < 150 kg - Categorization of civil operation: HOBBY operation : only recreational flying PROFESSIONAL operation (by companies) |
| IPQ (POR) | NIL |
| Kantonspolizei Bern (SUI) | Nato classification is usually used: http://www.google.ch/url?sa=t&rct=j&q=&esrc=s&source=web&cd=2&ved=0CCkQFjABahUKEwjup8qR29rGAhXGoYAKHV0UBS4&url=http%3A%2F%2Fuvs-info.com%2Fphocadownload%2F05_3b_2010%2FP061-062_NATO_Dave-Ehredt.pdf&ei=gwylVa7aIsbDggTdqJTwAg&usg=AFQjCNEKrRp0VEao4Q8MdWMoToJN-LE2KA&bvm=bv.97653015,d.eXY http://www.bazl.admin.ch/dienstleistungen/02658/index.html?lang=en |
| Lithuania | By weight: up to 300 g, 300 g – 25 kg, above 25 kg. By range of operation: normal use in visual line-of-sight (but in any case not further than 1000 m from the operator’s location). Individual authorizations for exceptional use above visual line-of-sight also determinate. Safety aspects: normal use - keeping the minimum distance not less than 50 m from vehicles, buildings and other persons. Individual authorizations for exceptional use also determinate. http://caa.lt/index.php?-1659137123 |
| Montenegro | Under study. |
| Norway | The regulatory framework is being revised and will be finalized during the fall of 2015. But to give an example, the following (simplified) is what is suggested: RO 1 can have a starting total weight of up til 2.5 kg and a maximum velocity of 60 knots RO 2 can have a starting total weight of up til 25 kg and a maximum velocity of 80 knots RO 3 can have a starting total weight of 25 kg or more and a maximum velocity of 80 knots or more, or be driven by a turbine motor, in which will require an operator permit (of the sort RO 3, jf.§34). Operator permit (RO 3) is also mandatory for flying BVLOS. |
| Robonic (FI) | Regulations are under development and to be finished, will be published during autumn 2015 by Finnish CAA (Trafi). They will be concentrated on VLOS operations. |
| Selex (I) | According to ENAC Regulation RPAS which are under ENAC responsibility are distinguished in terms of MTOW and Operations as: • RPAS with MTOW below 0.3 kg • RPAS with MTOW between 0.3 kg and 2 kg • RPAS with MTOW between 2 kg and 25 kg • RPAS with MTOW between 25 kg and 150 kg Operations can be conducted in: • Visual Line of Sight (LOS) • Extended Visual Line of Sight (EVLOS) • Beyond Line of Sight (BLOS) |
| Skeye (NED) | All VLOS |
| Slovenia Control | NO CATEGORISATION OF UAS IS SPECIFIED |
| Sweden | http://www.transportstyrelsen.se/sv/luftfart/Luftfartyg-och-luftvardighet/Obemannade-luftfartyg-UAS/ Permission from the Transport Agency is required for unmanned aircraft used or designed for: • Testing and research • Commercial purposes, therefore any kind of activity in which remuneration is received for work performed • Mission flight and the like, which are not regarded as pleasure or recreation • Be flown out of sight of the pilot. All unmanned aerial vehicles, which falling within the above points is called by the Transport Agency as a UAS, and get a special registration mark. 1.1 UAS divided into the following categories of permit 1.1.1 Category 1A Unmanned aircraft with a maximum take off weight of less than or equal to 1.5 kg, which develops a maximum kinetic energy of 150 J and is flown only within sight of the pilot. Learn more about Category 1A UAS http://www.transportstyrelsen.se/sv/luftfart/Luftfartyg-och-luftvardighet/Obemannade-luftfartyg-UAS/Sok-tillstand-for-UAS/Kategori-1A/ and how to apply for permission. 1.1.2 Category 1B Unmanned aircraft with a maximum takeoff weight of more than 1.5 kg but less than or equal to 7 kg, which develops a maximum kinetic energy of up to 1000 J and operated solely within view of the pilot. Read more Category 1B http://www.transportstyrelsen.se/sv/luftfart/Luftfartyg-och-luftvardighet/Obemannade-luftfartyg-UAS/Sok-tillstand-for-UAS/Kategori-1B/ UAS and how to apply for permission. 1.1.3 Category 2 Unmanned aircraft with a maximum takeoff weight of more than 7 kg which only operated within sight of the pilot. Learn more about Category 2 http://www.transportstyrelsen.se/sv/luftfart/Luftfartyg-och-luftvardighet/Obemannade-luftfartyg-UAS/Sok-tillstand-for-UAS/Kategori-2/ UAS and how to apply for permission. 1.1.4 Category 3 Unmanned aircraft certified to be flown and checked out of sight of the pilot. Learn more about Category 3 http://www.transportstyrelsen.se/sv/luftfart/Luftfartyg-och-luftvardighet/Obemannade-luftfartyg-UAS/Sok-tillstand-for-UAS/Kategori-3/ UAS and how to apply for permission. |
| Swiss Federation of Civil Drones | By environment: - fix and temporary populated areas (cities, villages, events) - airfields and controlled airspace - any other By operation type: VLOS - Visual Line of Sight of the Pilot EVLOS - Enhanced VLOS (VLOS for Obserfer, FPV (First Person View) for Pilot) BVLOS - Beyond VLOS (FPV for Pilot only) EVLOS and BVLOS will be typically more than 500m from the control station. |
| Telespazio (I) | According to our experience, UAS categorization depends mainly on: • RLOS /BRLOS Operations • MTOW (above or below 150 kg) • Area of operations overflown (depending on population density). As prime contractor for DeSIRE 2 project, our main interest is on large RPA Platforms employed in BRLOS Operations over low populated /maritime areas for specific institutional applications. The use of satcom link for CNPC data is key for BRLOS Operations and it is our main target in the project. |
| The Finnish Border Guard | Aviation act and aviation regulations are most important framework for UAS regulation. National RPAS regulation is on draft phase and it is expected that it will be published during 2015. It is estimated that Drafted RPAS regulation is or will be one of the most liberal RPAS regulation in the Europe. Right now and in practice VLOS activities with less than 25 kg MTOW under 150 meters altitude are allowed without special permission. Using of more than 25 kg MTOW UAS is usually subjected to license. BVLOS activities are allowed only inside segregated airspace. The Finnish Border Guard has been planned to start VLOS and BVLOS test flights on state border and maritime areas with fixed wing UAS during next year and with heli-/multi-copter type UAS during next five years. |
| Trimble (BEL) | They are divided in groups by concept of operation. |
| Unidade Especial de Polícia - Núcleo de Meios Técnicos e Audiovisoais (POR) | The category of UAV is Quadcopter type. Used in traffic control and masses of people.It can be used in cartography, searching for missing persons. The weight of the machines are 2 to 3 klg |
| United Kingdom | See chapter 1 Section 2 of CAP722 but basically they are currently divided into 3 categories by weight Small UAS 0-20 kg responsibility of national aviation authority Light UAS 20-150 kg responsibility of national aviation authority UAS >150 kg responsibility of European Aviation Safety Agency |
| VTO Technologies (G) | European harmonisation of BVLOS (Beyond Visual Line of Sight) RPAS C2C frequencies, essential for BVLOS RPAS operations regulatory compliance. Not essential for BVLOS RPAS operations regulatory compliance. Control and non-payload communications (CNPC), payload (incl. video). Visual line-of-sight / non visual line of sight, range of operation. Range and distances involved. Long range RPAS frequencies require stronger signals and frequency bands. |

Assessment of WGFM CG Drones:

A number of different concepts for the categorisation of UAS exists:

1. No categorisation at all

2. Environmental based categorisation (metropolitan areas, villages, event-based);

3. By operation type (VLOS, EVLOS, FPV (First Person View) for Pilot), BVLOS - Beyond VLOS 
    (FPV for Pilot only);

4. Risk based categorisation (requires an analysis for each UAS type and its type of mission, may 
    differentiate according to the type of user (hobby or professional), operating distance and its conditions, 
    weight and dimensions of the UAS, safety-features (e.g. anti-collision features, safe modes). The risk 
    based categorisation approach can also include elements of the environmental-based and operating 
    type-based categorisation approach as outlined under 2 and 3.

## Existing problems during the operation/test of UAS

- Question 11: Have there been any problems in respect to radio (e.g. Interferences, lost connections, fail safe features, etc.) during the operation and/or test of a UAS?

| Aerodromo Municipal de Portimão (POR) | No |
| --- | --- |
| AESA (E) | No |
| Airfilms Production (NED) | Yes |
| Austria | No |
| Belgian Civil Aviation Authority (BCAA) | Yes |
| Bosnia Herzegovina | No |
| Bundeskommission Modelflug (D) | No |
| Civil Aviation Agency (LVA) | No |
| Civil Aviation Authority (CZE) | Yes |
| Croatia | No |
| Dep. of Civil Aviation (CYP) | No |
| DFS Deutsche Flugsicherung (D) | Yes |
| DGA (F) | Yes |
| SWITZERLAND | Yes |
| EASA | No |
| EuroUSC International (G) | Yes |
| Federal Supervisory Authority for Air Navigation Services (D) | No |
| Finavia | No |
| Finland | No |
| Guardia Nacional Republicana (POR) | Yes |
| Hungary | No |
| IPQ (POR) | No |
| Lithuania | Yes |
| Montenegro | No |
| Norway | No |
| Robonic (FI) | No |
| Selex (I) | No |
| Skeye (NED) | No |
| Slovenia Control | No |
| Swiss Federation of Civil Drones | Yes |
| Telespazio (I) | No |
| The Finnish Border Guard | Yes |
| Trimble (BEL) | No |
| Unidade Especial de Polícia - Núcleo de Meios Técnicos e Audiovisoais (POR) | No |
| United Kingdom | Yes |
| VTO Technologies (G) | No |

- Question 11.1: Please explain the problem(s)

| Belgian Civil Aviation Authority (BCAA) | BCAA had some cases of loss of radio control link and also one case of trial to use illegal frequency band. |
| --- | --- |
| Civil Aviation Authority (CZE) | C2 datalink fails time to time but it is not easy to determine the reason (might be hardware failure, software failure or interference or out of reach reasons). Several older types operated on 35 MHz were very vulnerable to interference, 2,4 GHz seems quite good, but is sensitive to unobstructed direct connection. |
| Switzerland | Crash of an UAS behind the airfield due to adverse electromagnetic interference. The problems were not known before; the electromagnetic interference must have acted by the UAS as a similar opposite command that led to its destruction. There was only material loss. Electromagnetic fields in a city, a region or underground electric power supplies are very common, therefore this point has to be considered properly. In the meantime, the problem has been solved through software (frequencies used in this case were in the 2.4 GHz) |
| DFS Deutsche Flugsicherung (D) | It has been observed that sometimes frequencies are used that are forbidden by national and international regulations (e.g. 1090 MHz). Also datalink loss will be occurring sometimes, causing unplanned use of the UA. |
| DGA (F) | With medical operating and domestic application. In our country there are a lot of things who work with remote control |
| EuroUSC International (G) | Range issues are frequently experienced when operating in areas of intense 2.4GHz transmissions. Operators often experience poor reception of camera images when using 5.8GHz at the legal transmission power prompting them to fit illegal, high power transmitter modules. Many users are unaware of the effects of solar flares on the reception of GPS reception and poor piloting skills often leads to situations where operators find themselves operating with a GPS capture failure and without sufficient skills to operate manually. EuroUSC test for this scenario as part of their flight operations examination. This should not cause interference with the command and control frequency and specific frequencies and power outputs be listed with an obvious route allowing an operator to operate worldwide without constant changing of transmitter modules. Commercial operators should have their own protected frequencies requiring a licence which would only be available to registered commercial RPAS operators helping to prevent ‘cowboy’ hobby type inherently unsafe operators. 2.4GHz is widely used, especially in congested areas where wi-fi routers etc fill the spectrum. This causes high background interference reducing the operating range of commercial operations. Many hobby type aircraft and toys use a power output far in excess of requirements. A reduced power output and therefore range help to keep toys close to operators helping safety and helping to reduce interference. Perhaps a specific frequency such as 1.2GHz could be used for commercial RPAS operations. Many hobby ‘First Person View’ operators are illegally broadcasting far in excess of the legal limits without any fear of prosecution and often unaware of any legal requirements. |
| Guardia Nacional Republicana (POR) | For GNR, the possibility to open up segregated airspace to civil unmanned aircraft flight in Portugal creates a very complex security problem materialized through the possibility of civil RPAS flying over areas (Class A, C and D), assigned to airfields and airports, overflight people and urban spaces. For safety & security reasons it is desirable to interdict any possibility of flying civil RPAS on (less than 150 kg) in urban areas which makes the lack of national legislation a serious problem. The best solution is to only allow RPAS flights in “G Class” space, which represent also the need of law enforcement and compliance over the use of radio frequencies by unauthorized civilian’s users. |
| Lithuania | Cases of lost connections (and lost equipment) are known. |
| Swiss Federation of Civil Drones | If using legal power: lost connections Mostly: Use of illegal power to prevent connection loss |
| The Finnish Border Guard | Due to low flying altitudes, forests and shape of ground the radio horizon is achieved quite fast. Sometimes it has been losses of command and control connections due to unsuitable antennas or placement of the antennas on the drone or on the ground. It has also been noticed that the manufactures do not want to make suitable transmitters for BVLOS activities due to possibilities to fly BVLOS separates a lot between different countries. It makes BVLOS suitable equipment more expensive than those really should be. In order to make economical BVLOS activities possible for security authorities, it is necessary to establish higher segregated areas for UAS activities and ensure releasing of suitable frequency windows and using of more powerful transmitters on security authorities duties. It is also worth to notice that many security authorities conducts their duties outside of mobile phone connection areas so command and control and payload information connections cannot be based on mobile phone nets. It is assumed that in the future security authorities need to establish link network or use satellite connections in order to fly long distance controlled BVLOS flights. |
| United Kingdom | See the answer to question 4 of part 1 |

- Question 11.2: ’NO’ Remarks

| Aerodromo Municipal de Portimão (POR) | Non. |
| --- | --- |
| AESA (E) | WE HAVE NO ANY INFORMATION |
| Austria | Not yet official reported |
| Bosnia Herzegovina | No reported use of UAS |
| Bundeskommission Modelflug (D) | In the area of aeromodelling using 2.4 GHz equipment interference is very hard to identify and there are no proven events. Lost connections are quite normal due to exceeding the range of the R/C-equipment. For sophisticated drones above the toy-area this is covered by automatic means like a GPS-based return-to-home function. |
| Civil Aviation Agency (LVA) | Not reported. |
| Croatia | Not to our knowledge |
| Dep. Of Civil Aviation (CYP) | No Info |
| EASA | EASA does not currently have occurrence reports from drones’ incidents and accidents. This is due to the fact that no drone has been certified by EASA so far. On the other hand, EASA follows the incidents/accidents which are reported by the military. Example: MQ-1B, T/N 00-3068, 27 June 2014, United States Air Force Aircraft Accident Investigation Board Report Another accident was recently reported: |
| Federal Supervisory Authority for Air Navigation Services (D) | Non of such problems with civil UAS known. This question should also be directed to the federal states in there responsibility for civil UAS, the federal armed forces for military UAS and to DFS with reference to the ferry flight of the military UAS EuroHawk in 2011. |
| Finland | No reported problems so far, but the regulation to be issued very soon contains reporting requirements. |
| Hungary | There have not been any problems with the approved frequencies. |
| IPQ (POR) | Not to my knowledge |
| Montenegro | The more precise information will be provided when the test is completed. |
| Norway | There have not been any official reports in respect to this. However, we have been informed of lost connections when using the license exempt frequencies for control. |
| Robonic (FI) | There has not been any problems with Robonic’s test site flights |
| Selex (I) | No interferences have been recorded by Selex ES during RPAS operations and flight tests, although these have been always conducted in segregated airspaces. |
| Slovenia Control | NO OPERATIONAL PROBLEMS ARE KNOWN |
| Telespazio (I) | N/A to DeSIRE 2 project, as these aspects have not yet been addressed. |
| Trimble (BEL) | No remarks |
| Unidade Especial de Polícia – Núcleo de Meios Técnicos e Audiovisoais (POR) | Anything |

Assessment of WGFM CG Drones:

Reported problems include:

1. Illegal use of frequencies;

2. Use of greater emission levels to enhance the operating distance or to make the drone s operation safer
    against interference and loss of the control link;

3. Problem that UAS dominantly use 2.4 GHz (or other ‘unlicensed’ bands), fully under ‘no protection’ 
    conditions and where interference is always a possibility; a fact which may not be understood by all UAS 
    users;

4. Lack of frequency harmonisation for:
    a. Professional UAS’ with more mission-critical type of mission (the idea is to have a frequency opportunity 
        which is more dedicated to UAS than a generally authorised ‘SRD-band;

b. BVLoS UAS (could be individually licensed).

## Additional information

- Question 12: Any further relevant information? If so, please add here.

| AESA (E) | No |
| --- | --- |
| Airfilms Production (NED) | The Netherlands is currently killing the RPAS market with the introduced regulations. The playfield for professional operators has become complicated, non-flexible and limited to a situation that results in a very small perspective. They made rules like exams for companies, aircrafts & pilots, but there a no certified / accredited companies (manufacturing and pilot schools). |
| Bundeskommission Modelflug (D) | There is an information-flyer for customers under preparation by the DAeC (German Aero Club). The draft is attached. Also there are ongoing activities for the integration of drones into the air space. The complete A-NPA is available here: https://easa.europa.eu/system/files/dfu/A-NPA%202015-10.pdf An executive summary is available here: http://easa.europa.eu/document-library/notices-of-proposed-amendment/npa-2015-10 |
| Switzerland | Swiss FOCA has authorised several BLOS operations and VLOS tethered. |
| DFS Deutsche Flugsicherung (D) | Operation of drones may cause possible radio-frequency interference with other services which may lead to misconduct of the drone or the other service, respectively. This aspect should be considered by the regulatory measures. |
| DGA (F) | It's time for us in Europe to have a real frame work to control frequency and we will be happy to contribute of this registration by my knowledge and my experience. |
| EuroUSC International (G) | It is strongly believed that an International, protected and licensed RPAS operating frequency is required. Transmissions from commercial and hobby RPAS should be periodically and randomly checked and illegal users should be prosecuted. This should be an enforced, encoded, channel hopping protocol as currently widely utilised by the hobby fraternity utilising 2.4GHz. The power output should be sufficient to ensure reception above and beyond the maximum permitted flight envelope range. A specific, protected frequency range should be made available for commercial RPAS. Perhaps with a licence requirement where only registered commercial operators can apply and obtain. |
| Guardia Nacional Republicana (POR) | Most European countries are still striving with problem of unauthorized civil RPAS use. Legislation left to the responsibility of national authorities is urgent. The GNR expectation is that the future European regulatory framework and its transposition into law will shape the adoption of more efficient security measures. |
| IPQ (POR) | Very low knowledge of Rules of Air, and air navigation by the operators of UAV`s. |
| Norway | New and updated regulatory framework is to be published soon (late fall 2015). |
| Swiss Federation of Civil Drones | Transmission of an image is not only payload! If the image is used to give the pilot the in-air-view to observe the sky for traffic, this link will be part of the cockpit and be part of the control link. More from the practice: There exist solutions for digital image transmission. As the control of the UAS as well as the telemetry is not a huge data load, the same link will be used for control and telemetry as well. Lost of one (image, control or telemetry) results anyway in a emergency situation. So there is no need to split into several links (with a higher possibility of interferences). - If you have control and telemetry but no image, you can not observe the sky for traffic --> emergency - If you have no control but telemetry and image, autopilot may continue, but you can not tell him that there is traffic --> emergency - If you have control and image but no telemetry, you have no idea about the system status (power consumption, speed, position) --> emergency So, image transmission may also be part of the control link. It is "only" payload, if you have a spare sensor for ground observing (no matter if infrared or normal picture or any other sensor) ------------- A special frequency to UAS is very welcome. But it must be powerful enough to fly BVLOS and to transmit an imagequality of FullHD at least (lower resolution prevent of observing the sky for traffic) with a latency of less than 150ms (if lower, a pilot can not control the UAS without the support of an autopilot). |
| Telespazio (I) | The use of dual-link satcom concept is a key for BRLOS applications, and it is main target of the project to demonstrate its suitability for RPA CNPC Data for future integration of RPAS into non segregated, civil airspace. |
| Trimble (BEL) | No further info |
| Unidade Especial de Polícia - Núcleo de Meios Técnicos e Audiovisoais (POR) | Anything |

Assessment of WGFM CG Drones:

- When defining dedicated solutions for UAS frequency use, it needs to define which type of communications
  may use it (only control/telecommand or also telemetry which may include some images, or even a video 
  link or feedback for telecommand/control). In this regard, some scenarios may need to be described and to,
  e.g. in a second step, if necessary, study on possible frecuencies for payload links (video and images);
- Some concern about interference from drones expressed (2.4 GHz, or 5.8 GHz);
- For BVLoS UAS: need to consider satcom concepts as well as terrestrial spectrum use;
- A harmonised spectrum use possibility for UAS should reduce the unauthorised frequency use.

# EU legislation in progress

Include here some information about the EU legislation in progress

Just to share references / links with regard to the on-going process about ‘EU legislation in progress’ changing the EASA regulation.

http://www.europarl.europa.eu/thinktank/en/document.html?reference=EPRS_BRI(2016)573933

It includes amongst others some stronger requirements on drones.

There is also a briefing on the whole process: http://www.europarl.europa.eu/RegData/etudes/BRIE/2016/573933/EPRS_BRI(2016)573933_EN.pdf

Briefing on 'Civil drones in the European Union', which gives an overview of opportunities and challenges related to drones. It explains the current regulation of drones and presents the initial views of the EU institutions and stakeholders on future EU drone rules.

http://www.europarl.europa.eu/RegData/etudes/BRIE/2015/571305/EPRS_BRI(2015)571305_EN.pdf

ETSI reported to the last ECC meeting that their working assumption is for small UAS/UAVs:   “Small” UAV are covered by Machinery Directive 2006/42/EC and RED, and not EASA.

I heard that the current consideration may be about decreasing the weight limit from 150 kg to 25 kg (like USA) but did not find a draft of the future legislation. Other proposals are still in the discussions.

EASAs technical opinion based on the A-NPA2015-10 proposes rules for unmannd aircraft down to an MTOM of 0 gramms.

That means that UAVs / RPAS or however you call it will be taken out of the national regulations and will fall under the SERA rules.

The TechOp of EASA can be found here: https://easa.europa.eu/document-library/opinions/opinion-technical-nature

In the EASA proposal for new rules chapter 2.3:

With reference to mass-produced unmanned aircraft which pose a low risk, it is proposed to use existing market surveillance mechanisms, as governed by Regulation (EC) No 765/20084 and Decision No 768/2008/EC5, which are specifically devised for the production and marketing of such type of products.

My interpretation is that the RED and ETSI-standards will still be applicable tot these mass-produced unmanned aircraft.

# ADCO Market surveillance campaign

ADCO R&TTE 51 has endorsed the final report on the common market surveillance campaign carried out by the national market surveillance authorities’ members of the ADCO R&TTE. This campaign was focused on RPAS (Remotly Piloted Aircraft Systems). As this report may be interesting for ECC, I have the pleasure to send you officially a copy of the report itself. The report will be presented officially in TCAM WG beginning next year.

The report is also available on the commission’s website: http://ec.europa.eu/growth/sectors/electrical-engineering/rtte-directive/ under the topic “Market Surveillance reports”.

Here the direct link on the report: http://ec.europa.eu/DocsRoom/documents/13343/attachments/1/translations/en/renditions/native.

79 products were checked in 1st half of 2015, control links mostly use 2.4 GHz (84%) while some products have video cameras which use also 5.8 GHz (about 25%). Most products are manufactured in the Far East (92%). Many products are not marked CE (37%); missing or incomplete DoC (45%), documentation compliance only around 21%; technical compliance: 50%, almost independent from price, non-compliant for spurious emissions (23 products) and too high power/power density emissions (14), mostly remote control part, overall non-compliance is 92%. Some equipment CE marked while obviously not intended for the European market -> compliance with EN 300 440 claimed but higher US emission levels referenced. Auto landing functions implemented by about 30% to prevent from uncontrollable falling down.

These results are supported by feedback from ADCO regarding the 7th R&TTE Market Surveillance Campaign on Remotely Piloted Aircraft System where most sampled products used the 2.4 GHz band for command and control purposes.

The ADCO Report highlights, among others, the following conclusions for the frequency bands of the tested Remotely Piloted Aircraft System:

•	Four out of five (82%) products had administrative non-compliances within the meaning of R&TTE Directive; this non-compliance level is extremely high.

•	Half (51%) of all assessed RPAS were found to be non-compliant in relation to the effective use of spectrum.

•	Due to the low compliance with administrative requirements, the overall non-compliance is approximately 92%.

•	Spurious emissions (70%) and radiated power/power density (23%) are the main reasons for non-compliance.

ADCO recommends that economic operators should be identified to find possible solutions, civil aviation authorities will be informed, customs will be informed.

# New COMMISSION REGULATION (EU) laying down rules as regards unmanned Aircraft operation

Currently, the European Union (EU) does not regulate the civilian use of remotely piloted aircraft (RPA) with a mass of 150 kg or less. Such aircraft are governed by national rules adopted by the EU Member States.  RPA above the threshold of 150 kg fall within the mandate of the European Aviation Safety Agency (EASA).  Since 2014, the European Commission has been engaged in promoting the integration of remotely piloted aircraft systems (RPAS) into the European civil aviation airspace.  Following the EASA’s Technical Opinion adopted in 2015 that recommended a risk-based regulatory approach to govern the operation of drones, the Commission introduced a proposal to replace the current regulation governing drones. The proposal is designed to integrate all drones, regardless of their size, into the EU aviation safety framework.  A key objective of the proposal is to ensure that the design, production, maintenance, and operation of unmanned aircraft complies with the essential requirements of manned aircraft.  The European Parliament and other EU bodies strictly regulate the processing of personal data and the right to private life.  Operators of drones will be subject to tougher standards and requirements contained in the Data Protection Regulation, adopted by the European Parliament in April 2016, which will take effect in the summer of 2016.  Once the proposal on drones is approved by the Parliament and the Council of the EU, it will contribute towards the integration of drones into the European aviation airspace and provide the Commission with the legal authority to adopt delegated acts in compliance with the EASA’s standards.

The EU does currently not regulate drones whose mass is 150 kg or less, because the current governing regulation, Regulation 216/2008 on Common Rules in the Field of Civil Aviation, only covers aircraft whose mass is above that size.  Such large drones fall within the competence of the EASA.  Drones whose mass is less than 150 km may be regulated at the Member State level.  Indeed, several countries, including Austria, the Czech Republic, Denmark, Germany, France, Italy, Poland, Spain, Sweden, and the United Kingdom, have already adopted national rules.  Because of differing national rules on criteria and conditions for the operation of drones and related safety issues, operators must apply for a separate authorisation in each country.  In the Commission’s view, the current fragmented regulatory framework inhibits the further proliferation of drones and overall growth of the EU market in drones.

Note that the EASA Regulation does not address spectrum management aspects with regard to other radio services and applications than those set out by ICAO to ensure the operations of aeronautical services.

## Terminology used

### EU, EASA and ICAO

At the European Union (EU) level, no uniform terminology is used to denote what is commonly known as drones. The European Parliament uses the term “civil drones” to differentiate civilian drones from those intended for military purposes.  The European Commission uses the term “remotely piloted aircraft systems” (RPAS).  The European Aviation Safety Agency (EASA), an EU body established in 2002 with the mandate to issue implementing rules and approve airworthiness standards, defines drones as “unmanned aircraft . . . which includes any aircraft operated or designed to be operated without a pilot on board.”  This term also includes machines that are normally not perceived by the general public as aircraft, such as flying toys, small tethered balloons, or kites.  The EASA uses the term “drones” in all its communications to the general public.  The EASA’s definition of a drone is in line with the definition of “unmanned aerial vehicle” (UAV) provided by the International Civil Aviation Organization (ICAO), which is in charge of implementing the 1944 Chicago Convention on International Civil Aviation.  The ICAO defines a UAV as “a pilotless aircraft, in the sense of Article 8 of the Convention on International Civil Aviation, which is flown without a pilot-in-command on-board and is either remotely and fully controlled from another place (ground, another aircraft, space) or programmed and fully autonomous.”  UAVs are further divided into two categories: (1) those that are remotely piloted by a human and hence are designated as RPAS; and (2) those that are “autonomous,” meaning those that are controlled by a computer without pilot intervention after take-off.  This second category is outside the scope of the EU’s regulation.

The new Draft EU regulation has introduced two definitions: one for unmanned aircraft and one for equipment used to remotely control the unmanned aircraft.  The reason  was  to  avoid  that  the  equipment  to  remotely  control  the  unmanned  aircraft  be  systematically  part  of  the “certification” of the unmanned aircraft. Therefore, unmanned aircraft covers only the flying element (the aircraft).  As  the  equipment  to  remotely  control  the  unmanned  aircraft  is  a  key  element  of  its operations, this equipment must also be regulated by the prototype regulation [see below).

A writing convention uses Unmanned Aircraft  System  (UAS)  to  cover  both  the  unmanned  aircraft  and  the  equipment  to  remotely  control  it.  Unmanned Aircraft System is an internationally recognised definition and its acronym (UAS) well known.

### ITU

Report ITU-R M.2171 (12/2009) describes the following terminology:

Unmanned Aircraft (UA): Designates all types of aircraft remotely controlled.

Unmanned Aircraft Control Station (UACS): Facilities from which a UA is controlled remotely.

Control Link subsystem: Communication link between the UA and the UACS carrying telecommands (from the pilot to the UA) and telemetry (from the UA to the pilot).

Control and non-payload communications (CNPC): The radio links, used to exchange information between the UA and UACS, that ensure safe, reliable, and effective UA flight operation. The functions of CNPC can be related to different types of information such as: telecommand messages, non-payload telemetry data, support for navigation aids, air traffic control voice relay, air traffic services data relay, target track data, airborne weather radar downlink data, non-payload video downlink data.

Sense and avoid (S&A): S&A corresponds to the piloting principle “see and avoid” used in all air space volumes where the pilot is responsible for ensuring separation from nearby aircraft, terrain and obstacles.

Unmanned Aircraft System (UAS): Consists of the following subsystems:

•	Unmanned aircraft (UA) subsystem (i.e. the aircraft itself);

•	Unmanned aircraft control station (UACS) subsystem;

•	Air traffic control (ATC) communications subsystem (not necessarily relayed through the UA);

•	Sense and avoid (S&A) subsystem;

•	Payload subsystem (e.g. video camera …).

Radio line-of-sight (LoS): is defined as the direct radio line of sight radiocommunication between the UA and UACS.

Beyond radio line-of-sight (BLoS): is defined as the indirect radio communication between the UA and a UACS using satellite communication services.

Handover operations: is the transfer:

•	of a direct (LoS) RF communication from one dedicated UACS to another (LoS) dedicated UACS;

•	of a direct (LoS) to an indirect (BLoS) RF communication link or vice versa.

### Classification of air spaces

The aim of the WRC-12 Agenda item 1.3 is to study the spectrum requirements and possible regulatory actions needed to support the safe operation of all kinds of UA in non-segregated airspaces.

Segregated Airspace is restricted airspace of defined dimensions for the exclusive use of specific users.

Non-segregated Airspace is airspace other than those designated as segregated airspace.

The category of airspace has a pronounced impact on the data rate required for ATC communications, command and control, and particularly regarding sense and avoid.

## New key principles

The Riga Declaration on Remotely Piloted Aircraft, “Framing the Future of Aviation,” [ref] which was adopted on March 6, 2015, by Commission representatives, civil aviation officials, data protection national authorities, and representatives from the manufacturing industry, recognized the following key guiding principles to be taken under consideration in the future regulation of drones:

Drones must be dealt with as a new type of aircraft and any safety rules imposed must be proportional to the risk of each operation.

There is a critical need for the EU to establish safety rules immediately and also to lay down technologies and standards for the integration of drones within civil aviation.

The protection of privacy of individuals will lead to greater public acceptance.

The operator of a drone bears responsibility for its use.

In connection with the last principle, the Declaration raised the issue of insurance, third-party liability, and compensation schemes for victims, all of which fall within the domain of the individual EU Members.

In September 2015, the European Parliament’s Transport and Tourism Committee adopted, on its own initiative, the Report on Safe Use of Remotely Piloted Aircraft Systems (RPAS), Commonly Known as Unmanned Aerial Vehicles (UAVs), in the Field of Civil Aviation [ref].  In the Report, the Committee endorsed the key principles agreed to in the Riga Declaration and the Commission’s intention to remove the 150-kg threshold and replace it with a comprehensive EU regulatory framework.  The Committee also approved the EASA’s new competence to regulate drones and urged the EASA to budget funds for drone-related activities.

## European Aviation Safety Agency

The EASA provides opinions and formulates technical rules relating to the construction, design, and operational aspects of aircraft, and is also responsible for assisting the Commission by providing technical, administrative, and scientific support.

In May 2015, the EASA adopted a document titled Concept of Operations for Drones: A Risk Based Approach to Regulation of Unmanned Aircraft [ref], which urged regulation of the operation of drones in a manner proportionate to the risk of the specific operation, and also proposed to establish three categories of drone operations: Open, Specific, and Certified, with associated regulatory regimes. To mitigate privacy concerns, the EASA suggested the installation of chips/SIM cards in drones.  Other suggestions included the self-registration of drone operations in a Web-based application maintained by the local authorities.

At the request of the Commission, the EASA on December 18, 2015, issued a Technical Opinion on Introduction of a Regulatory Framework for the Operation of Unmanned Aircraft.  The Opinion contains 27 specific proposals for a regulatory framework and for low-risk operations of all unmanned aircraft irrespective of their size.  The Technical Opinion divides drones into three categories depending on risk:

- Open (low risk): Safety is ensured through compliance with operational limitations, mass limitations as a proxy of energy, product safety requirements, and a minimum set of operational rules. The ‘open’ category does not require any pre-approval as safety is ensured notably by a combination of measures including requirements and limitations on the operation, the unmanned aircraft system and the involved personnel and organisations. These general measures are complemented by conditions to access to airspace determined by the Member States ;

Specific (medium risk): Authorisation is given by a national aviation authority (NAA), possibly assisted by a qualified entity (QE), following a risk assessment performed by the operator.  A manual of operations lists the risk mitigation measures. The ‘specific’ category requires operators to obtain an authorisation given by the competent authority based on a risk assessment performed by the operator. As this could be burdensome for authorities and operators, a concept of standard scenario covering certain types of operations or flights has been developed. As operations with different risk levels are envisaged, the standard scenario will identify the cases where in lieu of the authorisation, a simple declaration by the operator will be sufficient to start the operation. These standard scenarios will be included in Certification Specifications (CS);

Certified (higher risk): The requirements applicable to this category are comparable to those for manned aviation.  Oversight is provided by the NAA (issue of licenses and approval of maintenance, operations, training, Air Traffic Management/Air Navigation Services (ATM/ANS), and airfield organizations) and EASA (design and approval of foreign organizations).

This operation centric approach is risk based. Intellectually a legislation only based on risk assessment could work but would lead to a significant burden for operators and competent authorities (e.g. operators to produce risk assessments, authorities to approve risk assessment). In addition for complex operations conducted with complex unmanned aircraft, there could be benefit to adopt a certification approach as it would avoid re-doing compliance demonstration and as it would give confidence to the public.

The balance between these considerations has led to create the three categories. In addition, some of these unmanned aircraft can be considered as toys, which are regulated by the product legislation.

The European Aviation Safety Agency (EASA) has also published a first draft of  Commission implementing rules (‘Prototype’ Commission Regulation on Unmanned Aircraft Operations and its Explanatory Note) to provide a clearer idea of what could be a European regulation and further engage with stakeholders. These 'prototype' rules are building on the Technical Opinion on the operation of drones published in December 2015, the related public consultation, based on a concept of operations for drones and a proposal to create common rules for operating drones in Europe issued earlier in 2015.

The safety of the ‘open’ category relies in particular on a set of limitations: unmanned aircraft maximum take-off mass must be below 25kg as a proxy to a limitation in energy, flight are limited to height below 150 m above ground or sea level and the unmanned aircraft must remain in visual line of sight (VLOS) of the remote pilot in order to reduce the risk of collision with other unmanned aircraft. The 25 kg limit was chosen because it is quite frequently the limit for ‘model aircraft’ to fly without an approval of its design. It is also the limit adopted by the FAA, ANAC Brazil and Transport Canada. Of course with such a maximum take-off mass, there is a need to have sub-categories in order to have rules proportionate to the risk. The limitations to 150m and VLOS are very important to mitigate the risk of collision with other aircraft.

Preliminary sub-categories of operations have been created to ensure that the rules remain proportionate. They are characterised as follows:

- a class of unmanned aircraft system. The essential requirements for the class of UAS may call for installation of geo-fencing functionality and electronic identification;

a set of limitations (maximum height; distance from uninvolved persons; VLOS) as appropriate;

requirement for pilot competence as appropriate

The combination of these three factors ensures the safety of the sub-category. Four preliminary categories have been defined from A0 to A3, ranging from the less complex to the more complex one.

The Council of the EU, and specifically the transport, telecommunications, and energy ministers in charge of the aviation market, advocate a harmonised EU approach to civil drone use while emphasising the need to take into consideration the experience gained in this field by the Member States, according to their comments at a public hearing.  Most of the ministers opinioned that the EASA was the entity best suited to develop technical and safety standards, licenses, and certificates, and agreed on the gradual and progressive integration of drones into civil aviation.

It was agreed with Member States that the Commission and EASA would develop a roadmap to  provide  more  clarity  on  what  are  the  plans  to  roll  out  the  operation  centric  concept.  The  roadmap  includes information  on  rulemaking  tasks,  development  of  standards,  research,  cooperation  with  international  organisations and FAA. It was developed during three workshops with Member States (March, April and May 2016) and presented to Industry at a workshop in June. However, this roadmap did not fully clarify all issues and EASA decided to produce a prototype regulation for ‘open’ and ‘specific’ categories by the end of the summer. This prototype regulation proposes actual rules providing the necessary clarity, notably on what are the responsibilities of the Member States and what is the flexibility offered to them. It has been called ‘prototype’ to reflect the fact that they should help preparing the formal rulemaking process that will follow.

Indeed, the intention is to publish this ‘prototype’ regulation and gather reactions which will be used to develop the necessary Notice of Proposed Amendments later in 2016. Reactions will be collected using a dedicated mailbox and dedicated workshops. In  addition,  as  this  prototype  regulation  will  be  available  at  the  start  of  the  negotiations  between  the  European Commission,  the  Council  and  the  Parliament,  they  may  facilitate  debates  and  avoid  that  the  Basic  Regulation  text becomes too specific. The detailed cover regulation should become a key element in these discussions.

Some of the provisions of the prototype rules will contribute to the application of other legislations such as security, privacy, data protection and environment. For example the requirements for geo-fencing together with the possibility for Member States to define zones where the activity of drones is prohibited or limited, contributes to security and privacy.  This  provides  an  effective  means  to  adapt  unmanned  aircraft  operations  to  the  specific  context  of  each Member State. Another significant flexibility is relative to the register of unmanned aircraft operators: Member States have quite an amount of flexibility in its implementation provided it includes the information required by the rule.

## Market forecast

According to a report by the European Parliament’s Committee on Transport and Tourism, the EU holds a leading edge in the civilian sector, with 2,500 operators (400 in the UK, 300 in Germany, 1,500 in France, 250 in Sweden, etc.) compared to 2,342 operators in the rest of the world [ref].  It is estimated that within the next ten years the drone industry could be worth 10% of the aviation market, or €15 billion per year.  The Aerospace and Defense Industries Association of Europe forecast that about 150,000 drone-related jobs will be created in Europe by 2050, excluding employment generated through operator services.

In September 2015, the European Parliament’s Transport and Tourism Committee adopted, on its own initiative, the Report on Safe Use of Remotely Piloted Aircraft Systems (RPAS), Commonly Known as Unmanned Aerial Vehicles (UAVs), in the Field of Civil Aviation [ref].  In the Report, the Committee endorsed the key principles agreed to in the Riga Declaration and the Commission’s intention to remove the 150-kg threshold and replace it with a comprehensive EU regulatory framework.  The Committee also approved the EASA’s new competence to regulate drones and urged the EASA to budget funds for drone-related activities.

## Proposed new regulation

In December 2015, the Commission introduced a proposal to adopt EU rules on drones and to repeal Regulation 216/2008 [ref].  The main objective of the proposed Regulation is to establish and maintain the same civil aviation safety standards for manned and unmanned aviation throughout the EU, and at the same time to ensure a high and uniform level of environmental protection.  It also seeks to expand the EASA’s competence to include RPAS with a mass below 150 kg.  The proposed Regulation would apply, inter alia, to the design, production, maintenance and operation of unmanned aircraft, their engines, propellers, parts and non-installed equipment, as well as the equipment to control unmanned aircraft remotely, where such aircraft are operated within the Single European Sky airspace by an operator established or residing within the territory to which the Treaties apply.

#### A. Requirements for Unmanned Aircraft

Pursuant to article 45 of the proposed Regulation, the design, production, maintenance, and operation of unmanned aircraft and their engines, propellers, parts, non-installed equipment, and equipment to control them remotely would need to comply with the essential requirements set out in Annex IX.

#### B. Compliance of Unmanned Aircraft

The Commission would be given the authority to adopt delegated acts concerning the specifications for the design, production, maintenance, and operation of unmanned aircraft.  Drones would be subject to certifications and declarations that they comply with such specifications.  A drone’s certificate would specify its safety-related limitations, operating conditions, and privileges.

#### C.  Market Surveillance Mechanisms

Mass-produced unmanned aircraft that pose a very low risk would be subject to the existing market surveillance mechanisms provided in Regulation 765/2008 and Decision No. 768/2008.  The national aviation authorities would remain indirectly involved, as the operational capability limitations that would be imposed (e.g., that the unmanned aircraft should not fly higher than, for instance, 50 meters to minimise risks) would have to stem directly from traditional aviation requirements.  The market surveillance mechanism would rely on justified complaints filed from citizens or undertakings to detect noncompliant products.  Findings of noncompliance in one particular Member State would then be communicated throughout the single EU market.  The EASA, which would assume additional responsibilities, would not be responsible for the oversight of the market surveillance mechanisms.  The Commission, in exercising its authority as the EU body in charge of implementation, would be authorized to verify if the Member States were fulfilling their responsibilities.

#### D.  Delegated Acts

The proposed Regulation does not set forth specifications for the design, production, maintenance, and operation of unmanned aircraft.  Such specifications would be promulgated by the Commission in delegated acts, pursuant to article 47 of the proposed Regulation.  When the Commission adopts such acts, it would be required to immediately notify the Parliament and the Council simultaneously.  This authority would be granted to the Commission for an indefinite period of time and would enter into force only if no objection was expressed by the European Parliament or the Council within two months after they received notification of that act or if, before the expiration of that period, the European Parliament and the Council had both informed the Commission that they did not object.  Until the delegated acts are adopted, the relevant provisions of Regulation (EC) 216/2008 would continue to apply.

In delegated acts the Commission would determine the

- conditions and procedures for issuing, maintaining, amending, suspending, or revoking the certificates for the design, production, maintenance, and operation of unmanned aircraft;

conditions for situations in which, with a view to achieving the objectives of the Regulation and while taking account the nature and risk of the particular activity concerned, such certificates must be required or declarations must be permitted;

conditions and procedures under which an operator of an unmanned aircraft must rely on the certificates or declarations issued in accordance with airworthiness and environmental standards, and other essential requirements;

conditions under which the requirements concerning the design, production, and maintenance of unmanned aircraft and their engines, propellers, parts, non-installed equipment, and equipment to control them remotely shall not need to meet certain other specifications in the Regulation;

marking and identification of unmanned aircraft; and

conditions under which operations of unmanned aircraft must be prohibited, limited, or subject to certain conditions in the interest of safety.

## Third Party Liability Issues and Security

Currently, all insurance obligations for aircraft operations are governed by Regulation 785/2004, which requires all commercial operators of aircraft to purchase third-party liability insurance.  Regulation 785/2004 contains limits for the minimum amount of third-party liability insurance based on the mass of aircraft during take-off.  For drones that weigh less than 500 kg, the minimum cover required is €660,000.  Drones that weigh less than 20 kg are not subject to insurance requirements.

The prototype regulation does not directly address these important issues because they are regulated at European or National level. They will however contribute to implementing them as follows:

- operators must register except if they operate only unmanned aircraft (UA) of the simpler sub-categories;

the Member State may define zones or airspace areas where UA operations are prohibited or restricted: these can be created, for instance, for security reasons;

obligation for the operator to comply with security requirements (Operator: natural or legal person that operate the drone);

the pilot of a UA must not fly close to emergency response efforts:

the learning objectives for the pilot involved in flying the most complex sub-categories of the ‘open’ category envisage  the  knowledge  of  flight  restrictions  (e.g. security)  and  the  understanding  of  ethical  airmanship.  The same applies for the competence of a pilot for the ‘specific’ category;

the risk assessment of the ‘specific’ category must take into account  areas  with  special  limitations  (e.g.  for Security or privacy reasons);

geo-fencing and electronic  identification  will be  required in standards of some sub-categories  in the ‘open’ category.

## Privacy Issues

The EU and its Member States have adopted strict privacy and personal data rules, contained in the 1995 Data Protection Directive [ref] and based on articles 7 and 8 of the binding 2009 Charter of Fundamental Rights of the European Union, and on article 16 of the Treaty on the Functioning of the European Union.  EU Members are also bound by article 8 of the Council of Europe Convention on Human Rights.  In addition, Members have their own constitutional and statutory rules on privacy, and domestic legislation implementing EU legislation.

Commercial users of drones appear to fall under the EU legislation on personal data protection.  Thus, commercial operators of drones have to comply with the applicable data protection principles, such as those concerning purpose limitations, data minimization, and proportionality, as well as the transparency principle, which requires individuals to be informed of any processing carried out during the operation of a drone.  On the other hand, private users of drones for hobby and leisure purposes may be exempt from the scope of the Data Protection Directive based on the household exemption.[51]  Other exemptions contained in the Directive concern processing for journalistic purposes and for law enforcement purposes.[52]  Possible criminal uses of civil drones would fall within the competence of EU Member States, since they are allowed to not apply the data protection rules on grounds of public safety, public security, and public order.

Drones normally carry video cameras to enable pilots to fly them or have other technological installations to record and store data that can eventually be uploaded on the Internet.  Consequently, the private life and property of individuals may be interfered with and violated when drones capture images of people in their houses or gardens.  Also, surveillance equipment installed on drones would make possible the gathering and processing of personal data and thus interfere with and potentially violate the right to privacy and data protection of individuals. The future regulation of the manufacturing and trade of drones, including the production, selling, buying, internal and international trade, and notice for buyers on risks and hazards, be designed in a manner to minimize any risks to citizens and their rights.

In his opinion on the Commission’s Communication A New Era for Aviation [ref] the European Data Protection Supervisor clarified that RPAS, being aircraft systems, do not on their own process personal data, but once they are equipped with other technologies may give rise to very diverse commercial, professional, law enforcement, intelligence, and private uses.  The Data Protection Supervisor reached a number of  conclusions with regard to the use of drones that involve the processing of personal data [ref].

## On-going process towards the new regulation

Some issues will be taken into further consideration, especially where relevant impacts are foreseen, and a detailed analysis could be conducted, also depending on the feedback provided by stakeholders in the on-going process:.

- Geo-fencing systems, e.g.: categories that should install these systems; definition of geographical data format and reference; way information should be provided to the operator and uploaded in the UA;

Identification, e.g.: interoperability with other manned aircraft; ways enforcement authorities could identify UA.

Registration and authorisation, e.g.: minimum threshold; factors to take into consideration (such as privacy, UAS carrying a HD camera); possibility of declaration as alternative to registration

Pilot Competence, e.g.: categories for which license is needed; use of online tutorial for less risky sub-categories;

Procedure for authorities, e.g.: different authorities to identify: aviation, market surveillance and enforcement;

monitoring and enforcement activities; how to check that the flight limitations are respected; training courses;

role  of  Competent  Authorities  with  regards  to  oversight,  registration,  designation  and  certification;  flow  of information of authorities across Member States; resources needed (e.g. for examining a document, testing or inspecting  the  UAS,  issuing  certificates,  authorisations  and  approvals;  maintain  register  of  UA  operators, declarations and authorisations and certificates);

-Categorisation,  e.g.:  sub-categorisation of the ‘open’ category and criteria according to which this should be done (e.g. weight, risk elements); model aircraft;

Occurrences reporting, e.g.: category and damage to be reported; how this should be done; by whom (e.g. self-reporting of the operator);

Fragmentation  of  rules,  e.g.:  currently  rules  quite  fragmented  at  national  level  and  the  need  to  foster harmonisation

# Frequency considerations for professional drones

Frequencies for more professional drones under individual license – for discussion (preliminary considerations)

Most common channel bandwidth for telecommand and control is 1 MHz or between 300 kHz and 3 MHz, mostly spread-spectrum, duty cycled, shared spectrum use, systems must be robust, shared licensed access. Typical heights above ground up to 762 m (comes from the segmentation of the air space).

Target: CEPT harmonization, harmonised technical conditions for the ‘drones’ market.

ITU-R1 and ECA Table:

| Frequency band | Allocations (ITU) | Allocations (ECA) | Applications (ECA) |
| --- | --- | --- | --- |
| 5030 MHz - 5091 MHz (5.444) | - AERONAUTICAL RADIONAVIGATION - AERONAUTICAL MOBILE-SATELLITE (R) (5.443D) - AERONAUTICAL MOBILE (R) (5.443C) |  | - Radiodetermination applications - MLS |
| 5091 MHz - 5150 MHz (5.444) | - FIXED-SATELLITE (EARTH-TO-SPACE) (5.444A) - AERONAUTICAL MOBILE-SATELLITE (R) (5.443AA) - AERONAUTICAL RADIONAVIGATION - AERONAUTICAL MOBILE (5.444B) missing in ECA | - FIXED-SATELLITE (EARTH-TO-SPACE) (5.444A) - AERONAUTICAL MOBILE-SATELLITE (R) (5.443AA) - AERONAUTICAL RADIONAVIGATION | - Radiodetermination applications |
| 5150 MHz - 5250 MHz (5.446) (5.446C) (5.447) (5.447B) (5.447C) | - MOBILE EXCEPT AERONAUTICAL MOBILE (5.446A) (5.446B) - FIXED-SATELLITE (EARTH-TO-SPACE) (5.447A) - AERONAUTICAL RADIONAVIGATION missing in ECA | - MOBILE EXCEPT AERONAUTICAL MOBILE (5.446A) (5.446B) - FIXED-SATELLITE (EARTH-TO-SPACE) (5.447A) | - BBDR - Radiodetermination applications - Feeder links - Radio LANs - Aeronautical telemetry |

Both missing allocations have never been in the ECA Table. Many CEPT administrations (actually the majority) has included them in their NTFAs. Proposal: it looks like a long-standing error not to include these allocations. They should be added as soon as possible.

CNPC (Control non-payload communication):

5030-5091 MHz

5.443C : The use of the frequency band 5030-5091 MHz by the aeronautical mobile (R) service is limited to internationally standardized aeronautical systems. Unwanted emissions from the aeronautical mobile (R) service in the frequency band 5030-5091 MHz shall be limited to protect RNSS system downlinks in the adjacent 5010-5030 MHz band. Until such time that an appropriate value is established in a relevant ITU-R Recommendation, the e.i.r.p. density limit of −75 dBW/MHz in the frequency band 5010-5030 MHz for any AM(R)S station unwanted emission should be used. (WRC-12)

MLS (UK) authorized for 3 sites in the UK

Possibility for telecommand uplink (ground to drone)? (Aeronautical Mobile)

5091-5150 MHz

5.444B	The use of the band 5 091-5 150 MHz by the aeronautical mobile service is limited to:

  - systems operating in the aeronautical mobile (R) service and in accordance with international aeronautical standards, limited to surface applications at 	airports. Such use shall be in accordance with Resolution 748 (Rev. WRC 15);

  - aeronautical telemetry transmissions from aircraft stations (see No. 1.83) in accordance with Resolution 418  (Rev.WRC-15).   (WRC 15);

Possibility: aircraft station, RES 418, for the telemetry downlink from the drone to the ground? (Aeronautical Mobile)

USA also discussing this for professional drones telecommand & control. This has potential for wider than European harmonization, if possible after studies.

Payload option:

5150-5250 MHz

Possibility for the payload to the ground? (Unprotected)

Original indoor Restriction for RLANs in 5150-5250 MHz in the USA (taken as a ‘representative similar case’):

Problem. Aggregated Interference into MSS satellite front end, see Globalstar

https://ecfsapi.fcc.gov/file/7520959857.pdf

However, the FCC changed the rules in 2014 and withdrew the indoor restriction.

U-NII Low (U-NII-1): 5.150-5.250 GHz. Originally limited to indoor use only. Regulations required use of an integrated antenna, with power limited to 50 mW. Rules changed in 2014 to permit outdoor operation, maximum fixed power 1 watt, maximum fixed EIRP 4 watts (+36 dBm) point-to-multipoint, 200 watts (+53 dBm) point-to-point. However, strict out-of-band emission rules limit practical point-to-point power to lower levels.

See FCC:

https://transition.fcc.gov/bureaus/oet/ea/presentations/files/oct14/51-New-Rules-for-UNII-Bands,-Oct-2014-TN.pdf

- No fixed outdoor installations and strict elevation angle related limits? Should be investigated by WGSE?

Option for drones:

Some of the FSS Band could be used, but there still are some open issues, which should be solved during the current study cycle at ITU and finalized at WRC-19.

FSS bands (Information/ more clarity would be needed from ICAO  - this is for international UAS traffic which could possibly be used also on national basis for further UAS use, but typically at higher heights above ground and in segregated air space)

AI 1.5 (WG 4A)  to consider the use of frequency bands allocated to the fixed-satellite service not subject to Appendices 30, 30A and 30B for the control and non-payload communications (CNPC) of unmanned aircraft systems (UAS) in non-segregated airspaces, in accordance with Resolution 153 (WRC-12);

Consensus has reached at the last day of the Conference for the CNPC usage for UAS. The WRC-15 made a provisional footnote allocation in number of FSS bands. ICAO is invited to start developing the necessary standard and recommended practices. Provisional allocation will come into force after the WRC-19

approval of technical conditions of CNPC links usage of those  bands.

Resolution 155

1 that assignments to stations of geostationary FSS satellite networks operating in the

frequency bands 10.95-11.2 GHz (space-to-Earth), 11.45-11.7 GHz (space-to-Earth), 11.7-

12.2 GHz (space-to-Earth) in Region 2, 12.2-12.5 GHz (space-to-Earth) in Region 3, 12.5-

12.75 GHz (space-to-Earth) in Regions 1 and 3 and 19.7-20.2 GHz (space-to-Earth), and in the

frequency bands 14-14.47 GHz (Earth-to-space) and 29.5-30.0 GHz (Earth-to-space), may be used

for UAS CNPC links in non-segregated airspace*, provided that the conditions specified in resolves

below are met;

1900-1920 MHz:

This band was studied for BB-DA2GC and use for DECT/SRDs. Alternatively, it could be studied for use by professional drones as new alternative primary usage and shared with DECT/SRDs (i.e. preffessional drones instead of BB-DA2GC).

ECA:

Proposals to WGFM:

From the questionnaire:

14 administrations support the harmonisation of preferred frequencies for drones. The main reasons behind their proposal are that:

  - using unlicensed bands shared by various types of applications is likely to increase the risk of interference to and from drones;

  - using unlicensed bands would limit the range of operation;

  - harmonisation would allow users to operate drones close to a border or in cross-border scenarios;

  - harmonisation would reduce the global footprint of drones on the spectrum resources.

WGFM CG Drones considers that a new opportunity for professional drones use should be found. This would also support the new aeronautical regulation under development for drones. This regulation foresees different categories for drones – according to the risks involved – from toys on one end to very professional drone applications where visual contact may not always be possible (radio line of sight), on the other end.

- Include the ‘missing allocations in 5091-5150 MHz and 5150-5250 MHz

- Terminology: include in annex 2 of ECC Decision (01)03 – application terminology – the term ‘Unmanned Aircraft’, to be discussed in EFIS/MG for its best location (parent term(s), layer 3 seems sufficient. The term is also used by ITU in Report ITU-R M.2171 and in line with EASA.

- Invite CEPT administrations to make proposals to WGFM CG Drones about which frequencies should be discussed for professional drones applications. The current assumptions and considerations are:

    - Most common channel bandwidth for telecommand and control is typically 1 MHz or between 300 kHz and 3 MHz, mostly spread-spectrum, duty cycled, shared spectrum use, systems must be robust, shared licensed access for CNPC. Typical heights above ground up to 762 m (comes from the segmentation of the air space);

    - Preliminary discussions focus on possibilities in 5030-5250 MHz, and possibly 1900-1920 MHz (, for both telecommand and control as well as payload may be possible, also with regard to technology support.

    - It was noted from Report ITU-R M.2171 that the aggregated demand for terrestrial UA systems could be up to 34 MHz for CNPC (all types, high usage density of 1 per km2, also in segregated air space),  4.6 MHz for ground to UA, 29.4 for UA to ground;

    - Target: CEPT harmonisation, harmonised technical conditions for the ‘Unmanned aircraft’ market;

    - Note the latest draft ECC Report with regard to information on the new aeronautical legislation and prototype regulations for ‘Unmanned aircraft’

- Inform ETSI about the considerations and requesting support for the initiative, e.g. use existing work item for TR 103 373 - frequency use for professional UAS in Europe, and contribute to the process.

- After feedback from CEPT administrations, discuss whether studies should be started in one or more particular bands.

# Conclusions

List of reference

- …

There are also questions about the applicability of the toy directive and the machinery directive