| Radiocommunication Study Groups |  |
| --- | --- |
| Received: Date 20xx Subject: WRC-27 agenda item 1.10 | PTB(25)029 ANNEX VI-04 Document XX/-E |
|  | Date 20xx |
|  | Original: English |
| The United Kingdom of Great Britain and Northern Ireland |  |
| Assesment of candidate pfd masks in studies under WRC-27 agenda item 1.10. |  |

Resolution 775 (Rev. CMR-23) “resolves to invite the ITU Radiocommunication Sector to complete in time for the 2027 world radiocommunication conference the appropriate studies to determine power flux-density (pfd) and equivalent isotropically radiated power (e.i.r.p.) limits to be included in Article 21 for satellite services (fixed-satellite service (FSS), mobile-satellite service (MSS) and broadcasting-satellite service (BSS)) to protect the current and planned fixed and mobile services in the frequency bands 71-76 GHz and 81-86 GHz”.

In Article 21, pfd limits are defined as pfd masks on the surface of the earth assuming free space propagation conditions, i.e. hard pfd limits that depend solely on the angle of arrival of the interfering signal, measured above the horizon. As such, these levels are not influenced by propagation conditions or clutter. Assessment of the level of interference depends only on geometric calculations based on the positions of satellites over time.

This document provides initial elements for further discussion within WP 5C regarding assessment of candidate pdf masks, some considerations on parameters for sensitivity analysis, and consequential proposals regarding the choice of parameters to retain for stations of the fixed service.

WP 5C is invited to include the attached elements in the Working document on sharing studies under agenda item 1.10 (Annex 2.4 to Document 5C/152).

Attachment:	1

Attachment

PROPOSED MODIFICATIONS TO THE WORKING DOCUMENT ON SHARING STUDIES UNDER AGENDA ITEM 1.10

Revision of Annex 2.4 to Working Party 5C Chair’s Report

# Introduction

WRC-23 approved WRC-27 agenda item 1.10 by Resolution 813 (WRC-23), in which point 1.10 is:

1.10	to consider developing power flux density and equivalent isotropically radiated power limits for inclusion in Article 21 of the Radio Regulations for the fixed-satellite, mobile-satellite and broadcasting-satellite services to protect the fixed and mobile services in the frequency bands 71-76 GHz and 81-86 GHz, in accordance with Resolution 775 (Rev.WRC-23);

In Resolution 775 (WRC-23), the World Radiocommunication Conference (Dubai, 2023),

resolves to invite the ITU Radiocommunication Sector to complete in time for the 2027 world radiocommunication conference

the appropriate studies to determine power flux-density (pfd) and equivalent isotropically radiated power (e.i.r.p.) limits to be included in Article 21 for satellite services (fixed-satellite service (FSS), mobile-satellite service (MSS) and broadcasting-satellite service (BSS)) to protect the current and planned fixed and mobile services in the frequency bands 71-76 GHz and 81-86 GHz, …

This document contains the result of the studies under WRC-27 agenda item 1.10, in response to Resolution 775 (Rev.WRC-23).

# Provisions of the Radio Regulations

The extracts from Article 5 of the Radio Regulations (RR), edition 2024, is presented in Tables 1 and 2 for the frequency ranges 71-76 GHz and 81-86 GHz, respectively.

Table 1

Extract from Article 5 of Radio Regulations for 71-76 GHz band

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 71-74 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) |  |  |
| 74-76 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) 5.561 |  |  |

Table 2

Extract from Article 5 of Radio Regulations for 81-86 GHz band

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 81-84 FIXED 5.338A FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) 5.149 5.561A |  |  |
| 84-86 FIXED 5.338A FIXED-SATELLITE (Earth-to-space) 5.561B MOBILE RADIO ASTRONOMY 5.149 |  |  |

Based on the information provided above, the appropriate studies could be carried out taking into account the nature of frequency bands and the service allocation in these frequency bands, to determine related pfd and e.i.r.p. limits, as shown below:

–	pfd limit for fixed-satellite service (space-to-Earth) to protect mobile service and fixed service in 71-76 GHz;

–	pfd limit for mobile-satellite service (space-to-Earth) to protect mobile service and Fixed Service in 71-74 GHz;

–	pfd limit for broadcasting-satellite service to protect mobile service and fixed service in 74-76 GHz;

–	e.i.r.p. limit for fixed-satellite service (Earth-to-space) to protect mobile service and fixed service in 81-86 GHz;

–	e.i.r.p. limit for mobile-satellite service (Earth-to-space) to protect mobile service and fixed service in 81-84 GHz.

# Propagation conditions

Document 5C/74 from WPs 3J and 3M contains references to the following recommendations:

Recommendation ITU-R P.452 is applicable for sharing studies between stations on the surface of the Earth. While the models have been tested up to 50 GHz, it is considered that it can be used for frequencies up to 100 GHz with the following caveats:

1	The free space loss component is applicable for all frequencies without limit.

2	The gaseous attenuation component, based on Recommendation ITU-R P.676, is applicable to 1 000 GHz.

3	The prediction method for diffraction loss is applicable for frequencies to at least 100 GHz although the input data required, such as terrain profiles, may not be detailed enough at the higher frequencies. However, the loss due to diffraction at these frequencies is quite large, so once a path is fully transhorizon, the only significant propagation mechanism is troposcatter.

4	While the troposcatter and anomalous propagation methods are based on measurements up to 50 GHz and have not been tested at higher frequencies, there is nothing intrinsic in the methods that would prevent their use between 50 and 86 GHz.

Recommendation ITU-R P.619 is applicable for sharing studies between stations in space and stations on the Earth’s surface.

Recommendation ITU-R P.2108 – Prediction of clutter loss.

Recommendation ITU-R P.2109 – Prediction of building entry loss.

Notwithstanding the elements received from WPs 3J and 3M, in Section V of RR Article 21, the limit of power flux-density from space-stations relates to the power flux-density which would be obtained under assumed free-space propagation conditions.

# Characteristics and protection criteria of FS stations

The following ITU-R Recommendations contain relevant technical and operational characteristics as well as protection criteria for on FS systems:

–	Recommendation ITU-R F.758-8 contains the principles for the development of sharing criteria of digital systems in the FS. It also contains information on representative technical characteristics of digital fixed wireless systems (FWS) in the FS for use in sharing studies above about 30 MHz. For agenda item (AI) 1.10, the following table abstracted from Table 12 contains the system parameters for PP FS systems in allocated bands from 71-76 GHz and 81-86 GHz.

Table 3

Typical values for FS point-point system parameters in the frequency band 71-76 and 81-86 GHz

| Frequency range (GHz) | 71-76/81-86 |  |
| --- | --- | --- |
| Reference ITU-R Recommendation | F.2006 |  |
| Modulation | QPSK | 64-QAM |
| Channel spacing and receiver noise bandwidth (MHz) | 250, 500, 750, 1 000, 1 250, 1 500, 1 750, 2 000, 2 250 | 500, 700, 1 000 |
| Tx output power range (dBW) | –10 | –20 |
| Tx output power density range (dBW/MHz)(1) | –41 | –47…-50 |
| Feeder/multiplexer loss range (dB) | 0 | 0 |
| Antenna gain range (dBi) | 54 | 44…50 |
| e.i.r.p. range (dBW) | 44 | 24…30 |
| e.i.r.p. density range (dBW/MHz)(1) | 13 | –6…3 |
| Receiver noise figure typical (dB) | 10 | 8 |
| Receiver noise power density typical (=NRX) (dBW/MHz) | –134 | –136 |
| Normalized Rx input level for 1 × 10–6 BER (dBW/MHz) | –120.5 | –94…-91 |
| Nominal long-term interference power density (dBW/MHz)(2) | –134 + I/N | –136 + I/N |

–	Recommendation ITU-R F.699-8 provides reference radiation patterns for, and information on, point-to-point FWS antennas in the frequency range from 100 MHz to 86 GHz. This information may be used in single-entry analyses and interference assessments when information concerning the FWS antenna is not available.

–	Recommendation ITU-R F.1245-3 provides average sidelobes and related reference radiation patterns for point-to-point FWS antennas in the frequency range from 1 GHz to 86 GHz. This information may be used for aggregate coordination and interference assessment studies when information concerning the FWS antenna is not available.

Typical FS station parameters are provided in the following table, to facilitate the sharing study.

Considering the above ITU-R recommendations, the following parameters are selected to develop studies.

Table 4

Typical values for FS point-point system parameters
in the frequency band 71-76 and 81-86 GHz

| System parameters | Typical Value |
| --- | --- |
| Channel spacing and receiver noise bandwidth (MHz) | 500 |
| Modulation | 64-QAM |
| Feeder/multiplexer loss (dB) | 0 |
| Antenna gain (dBi) | 41.5, 45 or 51 |
| Antenna size (m) | 0.2, 0.3 or 0.6 |
| Receiver noise figure (dB) | 8 |
| Antenna height(m) | 30 |
| Antenna RPE | F.699-8 and F.1245-3 |
| Link length (km) | 0.4 - 3 |
| Elevation angle (degree) | -4 to 4 |
| Nominal long-term interference power density (dBW/MHz) | –136 + I/N |

The following protection criteria used in study of AI 1.10 are:

‒	for the long-term, the I/N at the input of the FS receiver should not exceed –10 dB for more than 20% of the time;

‒	for the short-term, the I/N at the input of the FS receiver should not exceed +11 dB for more than 0.00128% of the time.	 The derivation methodology is in Annex 1, from different source.

# Characteristics and protection criteria of MS stations

TBD

# Characteristics of FSS & BSS stations

Tables in Annex 2 summarize system characteristics, sent by WP 4A in Document 5C/142, that could be used in sharing studies within the context of WRC-27 AI 1.10. Satellite systems and networks in BR IFIC (Space services) with frequency assignments in the 71-76 GHz and 81-86 GHz bands could also be used for sharing studies.

With respect to the question on how to model multiple systems to perform aggregate interference calculations, WP 4A recommends conducting simulations with the systems listed in Annex 2  noting that (1) additional satellite systems and networks in BR IFIC (Space services) could be included in aggregate interference calculations and (2) multiple co-frequency GSOs can operate over the same area from different longitudes on the GSO arc.

Moreover, WP 4A notes that non-GSO systems may implement mitigation measures among themselves. Normally, these measures, if applied, are confidential, but one possible technique to model these measures is to use avoidance angle between co-frequency beams. That is, for instance, on a given frequency, System A’s beam will need to have at least 1.5° angular separation with System B’s beam if they are serving the same area on Earth. It is not feasible to serve the same spot on the same frequency from satellites that are in close proximity of each other.

When considering aggregate interference from GSO satellites, a minimum longitude difference between satellites of 1° could be used.

Table 2 and 3 of Annex 2, as well as BR IFIC (Space Service) contains characteristics of earth stations to be considered by WP 5C to determine appropriate e.i.r.p. limits.

Files attached to the liaison statement (5C/142) of WP 4A are placed on the SharePoint (link). BR has also provided information on characteristics of FSS and MSS networks and systems in Document 5C/148.

# Characteristics of MSS stations

Upon request of WP 4C (see Document 5C/143), BR reviewed the satellite filings to gather characteristics of MSS networks or systems in the frequency bands 71-74 GHz and 81-84 GHz and sent them directly to WP 5C. The dataset can be found in shared folder (link). It includes orbital parameters of satellite systems published by administration in BR IFIC (Space Services). BR has also provided information on characteristics of FSS and MSS networks and systems in Document 5C/148.

Annex 3 in this working document presents 4 systems extracted from that dataset, to be used as possible examples of MSS systems for studies under this agenda item.

# Protection of the fixed service

## Methodology for assessment of candidate pfd masks

### Existing pfd masks in RR article 21

Article 21, and more specifically Table 21-4, provides pfd limits that shall not be exceeded by emissions from a space station, for all conditions and for all methods of modulation.

Table 5

Extract of RR Table 21-4 in the frequency range 42-42.5 GHz

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| 40-40.5 GHz | Fixed-satellite Mobile-satellite | −115 | −115 + 0.5( − 5) |  | −105 | 1 MHz |
| 42-42.5 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | −120 11, 21 | 5°-25° |  | −105 11, 21 | 1 MHz |
|  |  |  | −120 + 0.75( − 5) 11, 21 |  |  |  |
| 42-42.5 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | −127 21 | 5°-20° | 20°-25° | −105 21 | 1 MHz |
|  |  |  | −127 + (4/3) ( − 5) 21 | −107 0.4 ( − 20) 21 |  |  |

The pfd is the power flux density produced on earth’s surface.

These pfd limits are defined as a function of the angle of arrival above the horizontal plane and are, therefore, purely geometrically defined. These pfd mask could be starting point for studies under this agenda item

### Calculation of the angle of arrival and separation angle

The angle of arrival above the horizon can be calculated with the position of a victim station and the position of a space station.

With orbital characteristics collected by WP 5C from data provided by WP 4A (5C/142), WP 4C (5C/143) and the radiocommunication bureau (5C/148), it is possible to know the position of satellites at any given moment in time. Coordinates are then projected in a common east, north, up (ENU) reference frame, centred on the terrestrial station, in order to calculate:

‒	the angle of arrival θelev above the horizon, and

‒	the separation angle φsep, between the direction of the antenna of the terrestrial station and the direction of the space station.

This calculation makes no distinction between satellites above or below minimum elevation angle.

### Calculation of the power of the interfering signal

Knowing the separation angle φsep, Rec. ITU-R F.1245-3 can be used to calculate the antenna gain in the direction of the space station G1245φsep. The level of interference can then be evaluated with the effective antenna aperture Ae:

|  | Ae=G1245φsep+10log10λ24π | (dB m2) | (1) |
| --- | --- | --- | --- |

where  is the wavelength (m).

|  | I=pfdcandidate maskθelev+Ae | (dB W/MHz) | (2) |
| --- | --- | --- | --- |

The formula above doesn’t take into account atmospheric losses, as the pfd is given at the level of the ground in accordance with Article 21.16 which provides that “The power flux-density at the Earth’s surface produced by emissions from a space station, including emissions from a reflecting satellite, for all conditions and for all methods of modulation, shall not exceed the limit given in Table 21-4. The limit relates to the power flux-density which would be obtained under assumed free-space propagation conditions and applies to emissions by a space station of the service indicated where the frequency bands are shared with equal rights with the fixed or mobile service, unless otherwise stated.”. Further consideration may be required regarding atmospheric losses to assess whether these need further consideration or not.

### Schematic diagram

FIGURE 1

Calculation of the power of interference

### Repeated calculation over time and for various hypothesis

The above calculation can be repeated for different moments in time over a given period. The period should be long enough to take into account all possible geometries for the non-GSO system(s) under consideration and have enough data to produce stable statistics.

Due to the high antenna discrimination of 30 cm and 60 cm antenna dishes around 80 GHz, in order not to miss an event where a satellite is crossing the main beam of the terrestrial station antenna, the minimum step of calculation should be somewhere in a range between 1 and 5 seconds.

In order to compare results to low percentages of times, like 0.00128%. The dataset should be around 100/0.00128*100 ≈ 78.125×105 events, equivalent to ~30 days with steps of 0.33 s.

To minimise time taken for simulations, 3, 10 and 31 days were used.

### Sensitivity analysis

In application of the methodology described above, this section provides calculation examples to investigate the influence of some parameters of the fixed service on calculation results. Therefore, these results are subject to further discussions and refinement.

      - Common input data

##### Satellite systems under consideration

Information on systems to be considered in studies are grouped in Annex 2 and 3.

In this section, systems I, J, K, N and Q are used. This selection of system is arbitrary and simulation results may be updated at a later stage, to take into account developments for example, aggregation of interference from all visible satellites.

##### Pfd masks

In this document, pfd masks of the following table are studied as a staring point.

Table 6

Pfd masks used in this document

| Mask | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
| Pfd mask 01 | 0°-5° | 5°-25° |  | 25°-90° | 1 MHz |
|  | −115 | −115 + 0.5(d − 5) |  | −105 |  |
| Pfd mask 02 | 0°-5° | 5°-20° | 20°-25° | 25°-90° | 1 MHz |
|  | −127 | −127 + (4/3) ( − 5) | −107 0.4 ( − 20) | −105 |  |
| Pfd mask 03 | 0°-5° | 5°-25° |  | 25°-90° | 1 MHz |
|  | −120 | −120 + 0.75 (d − 5) |  | −105 |  |

#### Effect of FS station latitude

Table 7

Extract of RR Table 21-4 in the frequency range 42-42.5 GHz

| Parameter | Value | Unit |
| --- | --- | --- |
| Number of victim stations | 9 | - |
| Longitude | 0 | ° |
| Latitude | 0 - 80 | ° |
| Azimuth | 0, 45, 90, 135, 180 | ° |
| Up tilt | 1 | ° |
| Antenna diameter | 60 | ° |
| Satellite systems | I, J, K and N | - |
| pfd mask | 01 | - |

The following figures show 9 cumulative distribution functions (CDF) corresponding to stations of the fixed service with latitudes from 0 to 80°. Other parameters are described in the above table.

Each curve represents the probability that an interference power is exceeded for more than a given percentage of time.

##### Azimuth 0°

{3 days at 0.33 seconds per steps}

FIGURE 2

CDF for 9 stations of the FS with different latitudes, azimuth 0°

FIGURE 3

CDF for 9 stations of the FS with different latitudes, azimuth 0° (zoom short-term)

FIGURE 4

CDF for 9 stations of the FS with different latitudes, azimuth 0° (zoom long-term)

##### Azimuth 45°

{3 days at 0.33 seconds per steps}

FIGURE 5

CDF for 9 stations of the FS with different latitudes, azimuth 45°

FIGURE 6

CDF for 9 stations of the FS with different latitudes, azimuth 45° (zoom short-term)

FIGURE 7

CDF for 9 stations of the FS with different latitudes, azimuth 45° (zoom long-term)

##### Azimuth 90°

{3 days at 0.33 seconds per steps}

FIGURE 8

CDF for 9 stations of the FS with different latitudes, azimuth 90°

FIGURE 9

CDF for 9 stations of the FS with different latitudes, azimuth 90° (zoom short-term)

FIGURE 10

CDF for 9 stations of the FS with different latitudes, azimuth 90° (zoom long-term)

Because system I has an orbital plan in the equatorial plan, a station at latitude 0° with an antenna pointing towards East (i.e. 90° of azimuth) will receive higher levels of long-term interference.

##### Azimuth 135°

{3 days at 0.33 seconds per steps}

FIGURE 11

CDF for 9 stations of the FS with different latitudes, azimuth 135°

FIGURE 12

CDF for 9 stations of the FS with different latitudes, azimuth 135° (zoom short-term)

FIGURE 13

CDF for 9 stations of the FS with different latitudes, azimuth 135° (zoom long-term)

##### Azimuth 180°

{3 days at 0.33 seconds per steps}

FIGURE 14

CDF for 9 stations of the FS with different latitudes, azimuth 180°

FIGURE 15

CDF for 9 stations of the FS with different latitudes, azimuth 180° (zoom short-term)

FIGURE 16

CDF for 9 stations of the FS with different latitudes, azimuth 180° (zoom long-term)

##### Observations

The above CDFs show that the worst latitude varies with the FS antenna azimuth, and that for a given FS antenna azimuth, the worst latitude for the short-term interference is not necessarily the same as the worst latitude for the long-term interference.

#### Effect of FS antenna diameter

Table 8

Parameters

| Parameter | Value | Unit |
| --- | --- | --- |
| Number of victim stations | 3 | - |
| Longitude | 0 | ° |
| Latitude | 48 | ° |
| Azimuth | 90 | ° |
| Up tilt | 1 | ° |
| Antenna diameter | 20, 30 or 60 | ° |
| Satellite systems | I, J, K and N | - |
| pfd mask | 01 | - |

The following figure shows 3 cumulative distribution functions (CDF) corresponding to stations of the fixed service with antennas of 30 or 60 cm diameters. These CDFs represent the percentage of time a given interference power is exceeded.

{31 days at 0.33 seconds per steps}

FIGURE 17

CDF for antennas of diameters 30 and 60 cm

For low percentages of time (when considering the short-term protection criterion), 60 cm antennas will be more prone to interference because their maximum gain is around 6 dB higher than the maximum gain of 30 cm antennas.

For higher percentages of time, 20 and 30 cm antennas have less selectivity with higher side lobes compared to 60 cm antennas. Nonetheless, for higher numbers of satellites, the probability that a satellite crosses the main antenna beam may exceed 20%.

Therefore, when considering compliance with the long-term protection criterion, it may be useful to study antennas of various diameters: from 20 to 60 cm.

#### Effect of FS up tilt

Table 9

Extract of RR Table 21-4 in the frequency range 42-42.5 GHz

| Parameter | Value | Unit |
| --- | --- | --- |
| Number of victim stations | 6 | - |
| Longitude | 0 | ° |
| Latitude | 48 | ° |
| Azimuth | 90 | ° |
| Up tilt | 0 to 5 | ° |
| Antenna diameter | 60 | ° |
| Satellite systems | I, J, K and N | - |
| pfd mask | 02 | - |

The following figure shows 6 CDFs, giving the percentage of time a given level of interference is exceeded.

{31 days at 0.33 seconds per steps}

FIGURE 18

CDFs for antenna tilt in the range 0 to 5°

FIGURE 19

CDFs for antenna tilt in the range 0 to 5°, zoom long-term

For the short-term interference assessment, even with thousands of satellites, no definitive conclusion can be drawn.

Still, 1 or 2° of up tilt may be an appropriate choice, as the main beam of the antenna would stay in the lower part of the pfd mask.

#### Effect of the number of satellites

The parameters of the following table are used in this section.

Table 10

| Parameter | Value | Unit |
| --- | --- | --- |
| Number of victim stations | 3 | - |
| Longitude | 0 | ° |
| Latitude | 48 | ° |
| Azimuth | 90 | ° |
| Up tilt | 2 | ° |
| Antenna diameter | 60 | cm |
| pfd mask | 01 | - |
| Satellite systems | J, K & Q | - |

##### System J (864 satellites)

The following CDFs represent the percentage of time a given level of interference is exceeded, for various FS antennas azimuths.

{10 days at 0.33 seconds per steps}

FIGURE 20

CDFs for azimuths 90, 135 and 180°

FIGURE 21

CDFs for azimuths 90, 135 and 180° (zoom long-term)

##### System K (5 952 satellites)

The following CDFs represent the percentage of time a given level of interference is exceeded, for various FS antennas azimuths.

{10 days at 0.33 seconds per steps}

FIGURE 22

CDFs for azimuths 90, 135 and 180°

##### System Q (67 744 satellites)

The following CDFs represent the percentage of time a given level of interference is exceeded, for various FS antennas azimuths.

{10 days at 0.33 seconds per steps}

FIGURE 23

CDFs for azimuths 90, 135 and 180°

##### Observations

At a latitude of 47°, an antenna at 90° of azimuth will receive higher level of interference compared to 135 and 180° of azimuth.

The size of the system influences the long-term interference.

But its influence on the short-term interference may be further investigated.

### Observations

The pfd value provided by a pfd mask in this document is independent of propagation conditions and clutter.

These preliminary study results show the influence of several parameters on the level of short-term and long-term interference, without any conclusions about the protection of FS in the 71-76 GHz

These results show that studies should consider a range of parameters for sensitivity analysis for the fixed service, including:

- Latitude in the range 0 to +/-80°

- Elevation in the range 0 to 4 or 5°

- Azimuths without limitations

- Antenna diameters: 20 to 60 cm

# Studies for FSS & BSS stations

TBD

# Studies for MSS stations

TBD

# Summary of the results of studies

TBD

Derivation of short-term protection criteria for agenda item 1.10

Introduction

The derivation of short-term protection criteria is according to Recommendations ITU-R F.1606-0 and ITU-R F.1495-2.

The methodology presented in this Annex is based on the assumption that fading in the 71-76 GHz and 81-86 GHz (E-band) is dominated by rain and that, therefore, even if long-term interference has an effect on the performance of the link, the main way to have an outage of the FS link is to have an interference level higher than the fade margin of the link, whatever the propagation conditions may be.

On this basis, the following apportionment of the effect of interference on the degradation of the link (and on the EPOs) has been assumed:

–	20% of FS link degradation due to long-term interference;

–	80% of FS link degradation due to short-term interference.

EPOs

The allowable degradation in performance of real FWS due to interference from other services sharing the same frequency bands on a primary basis are expressed as a permissible fraction (10%) of the total EPOs and are defined in Recommendation ITU-R F.1565.

Fixed satellite systems are currently used in the E-band for the backhaul links of wireless access networks for point-to-point (P-P) applications. The service transmitted via E-band FS systems is packet service only.

The corresponding EPO values are given in the following table, and correspond to the following assumptions:

–	Access network section (Table 10 of Recommendation ITU-R F.1565-1, as shown in Fig. 1, for convenience);

–	Rate from 160 to 3500 Mbit/s in Tables 10 of Recommendation ITU-R F.1565-1;

–	C  8%.

Table 11

|  | EPO (fraction of any month) based on the application of Recommendation ITU-R F.1565-1 |  |
| --- | --- | --- |
|  | Total allowable to interference | Short-term interference (80%) |
| Errored second ratio | For further study | N/A |
| Severely errored second ratio | 1.6  10–5 | 1.28  10–5 |

FIGURE 25

Table 10 of Recommendation ITU-R F.1565-1

Please note, EPO based on ESR is more stringent and is more appropriate to protect FS. However, consider there is no ESR value for high capacity in Table 10, EPO at E-band based on only SESR is provided in this contribution. Update of EPO at E-band should be considered if ESR value for high capacity is supplemented in the future.

Short-term criteria

Methodology

As explained in § 1, the main way to have an outage of the FS link, considering short-term interference, is to have an interference level higher than the fade margin of the link, whatever the propagation conditions may be. That means, the permissible I/N for short-term protection is the largest value that will not cause SES in the absence of fading.

The definition of short-term criteria is then linked to both values of fade margin (or net fade margin considering ATPC1) and EPO allocated to short-term interference as defined in Table 1, considering that the fade margin is allocated to the short-term criteria.

Fixed service fade margins

In the E-band, since link lengths are likely to be short and AM is normally enabled to high modulation, and also BCA applications would decrease the fade margin, an FS fade margin of 10 dB was considered representative of conventional links.

As the EPO are referenced to ITU-T Recommendations G.826 definitions, it is necessary to extrapolate the fade margin corresponding to SES levels. On the basis of agreed assumptions (for detailed derivation, please refer to Recommendation ITU-R F.1606-0), the fade margin for SES is 1 dB higher than the fade margin referenced to the BER 1 ×10–6 level. Table 2 summarizes these different values of fade margins and, associated with the correspondent EPO ratios, allows to define short-term criteria for the FS.

Table 12

|  | Fade margin (dB) | EPO ratio |
| --- | --- | --- |
| BER 1 ×10–6 | 10 |  |
| SES | 11 | 1.28 × 10–5 |

Thus, the short-term criteria proposed to be used in E-band have been defined associating the SES EPO (see Table 2) and the corresponding I/N as defined in Table 3.

Table 13

|  | I/N (dB) | Percentage of time not to be exceeded (%) |
| --- | --- | --- |
| Criterion | 11 | 0.00128 |

Proposed examples of FSS satellite systems to be considered
for studies under WRC-27 agenda item 1.10

Table 14

Orbit configuration

| Parameter | System A | System B | System C | System D | System E | System F | System G | System H | System I | System J | System K | System L | System M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Height (km) | 525, 530, 535 | 590, 610, 630 | 35786 | 1050 | 1414 | 450 to 900 | 340 to 614 | 600, 1200 | 8 062 | 1 175 | 355, 347 | 500, 500, 600, 600, 700, 700, 800, 800, 900, 900, 1000, 1000, 1100, 1100, 1200, 1200, 1300, 1300, 1400, 1400, 8100, 8100, 8100, 8100, 8100, 8100, 8100, 8100, 12000, 12000, 12000, 12000, 12000, 12000, 12000, 12000, 16000, 16000, 16000, 16000, 16000, 16000, 16000, 16000, 20000, 20000, 20000, 20000, 20000, 20000, 20000, 20000, 23222, 23222, | 340, 345, 350, 360, 525, 530, 535, 604, 614 |
| Number of planes | 28, 28, (24,4) | 28, 36, 34 | 1 | 12 | 8 | 81 | 794 | 132 | 1, 4, 6 | 18 | 24, 24 | 36, 36, 36, 36, 34, 34, 30, 30, 28, 28, 24, 24, 24, 24, 22, 24, 20, 20, 18, 18, 1, 12, 12, 12, 12, 12, 12, 12, 1, 12, 12, 12, 12, 12, 12, 12, 1, 12, 12, 12, 12, 12, 12, 12, 1, 12, 12, 12, 12, 12, 12, 12, 1, 12 | 12, 18, 48, 48, 48, 30, 28, 28, 28 |
| Satellites per plane | 120, 120, (28,27) | 28, 36, 34 | 1 | 28 | 6 | 1 to 8 | 12 to 120 | 36 to 72 | 32, 16, 12 | 48 | 124 | 36, 36, 32, 32, 32, 32, 32, 32, 30, 30, 24, 24, 24, 24, 24, 24, 24, 24, 20, 20, 96, 10, 10, 10, 10, 10, 10, 10, 96, 10, 10, 10, 10, 10, 10, 10, 96, 10, 10, 10, 10, 10, 10, 10, 96, 10, 10, 10, 10, 10, 10, 10, 96, 9 | 110, 110, 110, 120, 120, 120, 120, 12, 18 |
| Inclination angle (deg) | 53, 43, 33 | 33, 42, 51.9 | 0 | 89 | 52 | 0 to 98.9 | 33 to 148 | 40 to 87.9 | 0, 90, 45 | 86.5 | 50, 50.2 | 50, 85, 50, 85, 50, 85, 50, 85, 50, 85, 50, 85, 50, 85, 50, 89, 50, 85, 50, 85, 0, 15, 45, 60, 65, 70, 75, 80, 0, 15, 45, 60, 65, 70, 75, 80, 0, 15, 45, 60, 65, 70, 75, 80, 0, 15, 45, 60, 65, 70, 75, 80, 0, 56 | 53, 46, 38, 97, 53, 43, 33, 148, 116 |
| RAAN | Equally spaced | Equally spaced |  | Equally spaced | Equally spaced | Equally spaced | Equally spaced | Equally spaced | Equally spaced | Equally spaced | Equally spaced | Equally spaced | Equally spaced |

Table 15

Other characteristics for systems A to C

| Parameter | System A | System A | System B | System B | System C | System C |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency (GHz) | 71-76 | 81-86 | 71-76 | 81-86 | 71-76 | 81-86 |
| Peak antenna gain (dBi) | 52 | 60.9 (D: 1.85 m) | 41.9 or 48 | 53.6 (D: 1 m, 59.6 (D: 2 m, ), 64.5 (D: 3.5 m) | 50 | 50 (D:0.6 m) |
| Antenna Pattern | Rec S.1528 Recommend 1.2 for main beam Recommend 1.4 for side lobes (beyond 15°) | Recommendation ITU-R S.580-6 | Rec. S.1528 Recommend 1.2 for main beam Recommend 1.4 for side lobes (beyond 15°) | Recommendation ITU-R S.580-6 | For satellite: Appendix 7 Annex 3 Section 3 G1= ‒13 dB Beamwidth = 0.42 deg | For earth station: S.580 |
| Input power density (dBW/Hz) | ‒103 to ‒83.57 Max power only used while compensating for low elevation angles or rain fade attenuation | ‒93 to ‒80.8 Max power only used while compensating for low elevation angles or rain fade attenuation | ‒106.2 to ‒86.2 Max power only used while compensating for low elevation angles or rain fade attenuation | ‒97 to ‒77 Max power only used while compensating for low elevation angles or rain fade attenuation | ‒77.8 | ‒77.8 |
| Minimum Elevation Angle (degrees) | 15 | 15 | 20 | 20 | 3 | 3 |
| Bandwidth (MHz) | 1 250 | 1 250 | 100 | 100 | 180 | 180 |
| Out of band emission mask | SM.1541-6 | SM.1541-6 | SM.1541-6 | SM.1541-6 | SM.1541-6 | SM.1541-6 |
| Number of co-frequency beams (N_co) | 32 | 32 | 32 | 32 | 1 | 1 |
| Max Power Flux Density on the ground dBW/m2/MHz | ‒106 | N/A | ‒104 | N/A | ‒129.85 | N/A |
| Worst Earth station density per 2 000 000 km2 | N/A | 76 | N/A | 5 | N/A | 25 |

Table 16

Other characteristics for systems D to H

| System | System D |  | System E |  | System F |  | System G |  | System H |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency (GHz) | 71-76 | 81-86 | 71-76 | 81-86 | 71-76 | 81-86 | 71-76 | 81-86 | 71-76 | 81-86 |
| Peak Antenna Gain (dBi) | 34.1 | 40.9 to 61.3 | 25 to 45 | 35.4 to 72 | 59 | 30 to 70.5 | 35 to 60 | 35 to 71 | 39.4 to 55.9 | 41.9 to 68.3 |
| Antenna pattern | Rec S.1528 | Rec 580-6 | Rec S.1528 | AP8 | Rec S.1528 | Rec 580-6 | Rec S.1528 | AP8 | Rec S.1528 | AP8 |
| Beamwidth |  | 0.13 to 1.65 |  | 0.04 to 2.79 |  | 0.05 to 5.6 |  | 0.05 to 3 |  | 0.07 to 1.41 |
| Input Power Density (dBW/Hz) | −36.1 to −30.1 |  | −82.1 to −45.9 |  | −83 to −74 |  | −80 to −55 |  | −94.6 to −72.3 |  |

Proposed examples of MSS satellite systems to be considered
for studies under WRC-27 agenda item 1.10

Table 13

Orbit configuration

| Parameter | System N | System O | System P | System Q |
| --- | --- | --- | --- | --- |
| Perigee (km) | 21000 | 3800, 3000, 1776, 1215, 6400 | 540, 21028.6, 23136.8, 25245, 25245, 27353.2, 29461.4, 30726.3, 35786.1 | 340, 345, 350, 360, 525, 528, 530, 535, 540, 550, 560, 560, 570, 604, 614 |
| Apogee (km) | 21000 | 9000, 9800, 11024, 11585, 6400 | 540, 50543.5, 48435.2, 46327, 46327.1, 44218.9, 42110.6, 40845.7, 35786.1 | 340, 345, 350, 360, 525, 528, 530, 535, 540, 550, 560, 560, 570, 604, 614 |
| Number of planes | 6 | 31, 1, 9, 9, 9 | 9, 256, 256, 256, 1, 255, 512, 512, 128, 256, 128, 128 | 96, 96, 96, 60, 56, 28, 56, 56, 72, 72, 4, 6, 36, 24, 36 |
| Satellites per plane | 6 | 7, 7, 21, 21, 21 | 23, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1 | 110, 110, 110, 120, 120, 120, 120, 120, 22, 22, 43, 58, 20, 12, 18 |
| Inclination angle (deg) | 55 | 63, 64, 45, 50, 52 | 98, 63, 41, 46, 46, 46, 63, 63, 38, 42, 20, 25 | 53, 46, 38, 97, 53, 43, 43, 33, 53, 53, 98, 98, 70, 148, 116 |
| RAAN | Equally spaced | Equally spaced | Equally spaced | Equally spaced |

______________

____________________

This document has been developed and agreed within the framework of CEPT CPG PTB.

1 	For an FS link using ATPC, the net fade margin  total fade margin – ATPC range.

Altitude 535 km, Inclination 33 deg, 24 planes with 28 satellites per plane and 4 planes with 27 satellites per plane.

Systems A and M are variations of the same system and as such, they shouldn’t be aggregated in studies.

This is an average input power spectral density meaning there could be higher and lower power spectral densities employed by the System C satellite.

The worst density is provided in the table. To scale for larger area, the density should be considered together with a factor of 0.65 to account for the fact that the worst density isn’t feasible on a wider scale. For instance, density of 10 in 2 million km2, if scaled to 10 million km2 is: 10 × 10 000 000/2 000 000 × 0.65 = 32.5

Systems A, M and Q are variations of the same system and as such, they shouldn’t be aggregated in studies.