Guidance on the use of aerial UE in border areas operating in the bands 703-733 MHz, 832-862 MHz, 880-915 MHz, 1710-1785 MHz, 1920-1980 MHz, 2500-2570 MHz and 2570-2620 MHz

ECC Report <No>

approved DD Month YYYY

# Executive summary

TABLE OF CONTENTS

0	Executive summary	2

1	Introduction	5

2	Definitions	6

3	Aerial UE in border areas	7

3.1	Aerial UE usage and applications	7

4	Agreements in place at the border	8

4.1	Between administrations	8

4.2	Between mobile operators	8

4.3	Roaming agreements between mobile operators	8

5	Possible case of interference issues in border areas	9

6	Possible mitigation techniques applicable in border areas	10

6.1	Impact of non-transmission zones in border areas	10

7	Conclusions	11

ANNEX 1: Simulation scenario and assumption for UAS operation in border areas	12

A1.1 Reference case in border area	12

A1.2 Simulation scenario in border area	12

A1.3 Simulation assumption	13

A1.4 Method for determining the no transmission zone	16

A1.5 Simulation results	16

ANNEX 2: List of References	17

LIST OF ABBREVIATIONS

| Abbreviation | Explanation |
| --- | --- |
| CEPT | European Conference of Postal and Telecommunications Administrations |
| ECC | Electronic Communications Committee |

# Introduction

# Definitions

| Term | Definition |
| --- | --- |

# Aerial UE in border areas

## Aerial UE usage and applications

# Agreements in place at the border

## Between administrations

## Between mobile operators

## Roaming agreements between mobile operators

# Possible case of interference issues in border areas

Use case Bâle Mulhouse international airport

Proposal from Latvia to perform practical measurements to assess possible impact of Aerial UE on RAS

# Possible mitigation techniques applicable in border areas

See ECC Report 309

## Impact of non-transmission zones in border areas

# Conclusions

Assessment of a need for a toolbox for administrations

Simulation scenario and assumption for UAS operation in border areas

Reference case in border area

In the border area, before the authorisation of UAS operation, the reference case is the cross-border coordination rule from ECC Recommendation(s). The reference case scenario is illustrated in Figure 1.

Figure 1: Reference case scenario

Calculate the distance D0 with the field strength value from the ECC cross-border coordination recommendation.

Simulation scenario in border area

The simulation scenario for UAS operation in the border area is illustrated in Figure 2.

Figure 2: Simulation scenario for UAS operation in border area

ECC Report 309 has concluded that the adjacent channel interference from UAS to MFCN UL is not a problem. The simulation from UAS operation in the network A in Country A to the network B in Country B will be carried out only for co-channel case. As shown in Figure 2, the simulation is to be done following the steps below:

1 UAS UE and 2 ground UEs in the reference cell of the network A, UE traffic represents 33% of the cell capacity, 3GPP Report xxx said aerial UE traffic should be less than 30%;

Aerial UE flying height is randomly between 30 and 200 meters;

The separation distance D1 between the network A reference cell BS and the network B reference cell BS is determined with the criterion that the network B reference cell uplink throughput loss is less than 5%.

Simulation assumption

Simulation assumptions are taken from the Table 28 in the Annex 5 of the ECC Report 309 [1] and are provide in Table 1.

Table 1: Simulation assumptions for drones operation in 700, 800 and 2600 MHz bands in urban and rural areas

| Parameter | Network Part A | Network Part B | Reference |
| --- | --- | --- | --- |
| System | LTE | LTE |  |
| Duplex mode | FDD | FDD | 3GPP TS36.104 [2] |
| Channel bandwidth (MHz) | 10 MHz (UL: 703-713 MHz) (UL: 842-852 MHz) (UL: 2500-2510 MHz) | 10 MHz (UL: 703-713 MHz) (UL: 842-852 MHz) (UL: 2500-2510 MHz) | LTE 10 MHz channel |
| Centre frequency (MHz) | UL: 708 UL: 2505 | UL: 708 UL: 2505 |  |
| MIMO | 2x2 | 2x2 |  |
| BS Tx Power | 43 dBm/10 MHz per MIMO branch 46 dBm/10 MHz per cell | 43 dBm/10 MHz per MIMO branch 46 dBm/10 MHz per cell | Report ITU-R M.2292 [3] |
| Antenna | Recommendation ITU-R F.1336 (recommends 3.1) [4] ka = 0.7 kp = 0.7 kh = 0.7 kv = 0.3 Horizontal 3 dB beamwidth: 65 degrees Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336 [4]. | Recommendation ITU-R F.1336 (recommends 3.1) [4] ka = 0.7 kp = 0.7 kh = 0.7 kv = 0.3 Horizontal 3 dB beamwidth: 65 degrees Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336 [4] | Report ITU-R M.2292 [3] |
| Antenna height (m) | 700-800 MHz: 30 (Urban) 40 (Rural) 2600 MHz: 20 (Urban) 30 (Rural) | 700-800 MHz: 30 (Urban) 40 (Rural) 2600 MHz: 20 (Urban) 30 (Rural) | Report ITU-R M.2292 [3] and Doc WP5D#416 [5] |
| BS Antenna gain (dBi) | 700-800 MHz 15 (Urban) 18 (Rural) 2600 MHz 16 (Urban) 18 (Rural) | 700-800 MHz 15 (Urban) 18 (Rural) 2600 MHz 16 (Urban) 18 (Rural) | ECC Report 82 [6] for 700 MHz with similarity to 800 MHz ITU-R Report M.2292 [3] for 2600 MHz |
| Feeder loss (dB) | 3 | 3 | Report ITU-R M.2292 [3] |
| Downtilt | 700-800 MHz 6° (Urban) 3° (Rural) 2600 MHz 10° (Urban) 3° (Rural) | 700-800 MHz 6° (Urban) 3° (Rural) 2600 MHz 10° (Urban) 3° (Rural) | Report ITU-R M.2292 [3] Based on national data for 700-800 MHz band urban |
| Maximum e.i.r.p. (dBm) | 58 dBm 61 dBm | 58 dBm 61 dBm | Report ITU-R M.2292 [3] |
| BS Noise figure | 5 | 5 | Report ITU-R M.2292 [3] |
| Site type | Tri-sector | Tri-sector |  |
| Cell Range (m) | 700-800 MHz 600 (Urban) 8000 (Rural) 2600 MHz 400 (Urban) 4000 (Rural) | 700-800 MHz 600 (Urban) 8000 (Rural) 2600 MHz 400 (Urban) 4000 (Rural) | ECC PT1(10)128 [7] Report ITU-R M.2292 [3] |
| Handover Margin (dB) | 1 | 1 | ECC PT1(10)128 [7] |
| SINR Minimum (dB) | -10 | -10 | ECC PT1(10)128 [7] and 3GPP TR 36.942 [8] |
| UE Tx Power (dBm) | 23 | 23 | 3GPP TS36.101 [9] |
| UE Antenna height (m) | For normal UE in urban (Note 1): 1.5, 4.5, 7.5, 10.5, 13,5, 16.5, 19.5 (Note 2) For normal UE in rural: 1.5 m For drone: 30, 50, 100, 150, 200, 250, 300 | For normal UE in urban: 1.5, 4.5, 7.5, 10.5, 13.5, 16.5, 19.52 For normal UE in rural: 1.5 m | 6-7 floors are considered in urban area |
| UE Antenna gain (dBi) | -3 | -3 |  |
| Indoor penetration loss (dB) | 20 dB (Urban) 10 dB (Rural) | 20 dB (Urban) 10 dB (Rural) |  |
| Body loss (dB) | 0 for drone 1 dB for normal UE | 1 |  |
| Minimum Tx Power (dBm) | -40 | -40 | ECC PT1(10)128 [7] |
| UE noise figure (dB) | 9 | 9 | Report ITU-R M.2292 [3] |
| Number of UEs/Cell | 1 | 1 |  |
| BS to UE propagation model | Free Space (UEs being drones) | Recommendation ITU-R P.1546-5 Urban/Rural for H<30 m | Recommendation ITU-R P.1546-5 [10] |
| Standard deviation (dB) | 6 (Urban) 8 (Rural) | 6 (Urban) 8 (Rural) |  |
| UEs distribution per floor | ground floor (h=1.5 m): 25%/30% (Note 3) 1st floor (h=4.5 m): 25%/30% 2nd floor (h=7.5 m): 10% 3rd floor (h=10.5 m): 10% 4th floor (h=13.5 m): 10% 5th floor (h=16.5 m): 10% 6th floor (h=19.5 m): 10% (Note 4) |  | ECC Report 309 [1] section A5.3.4 |  |
| Propagation model for interference link from Drone to the victim network Part B BS | Free Space for H>= 30 m With 5 dB standard deviation |  |  |  |
| Standard deviation for the interference link | 5 dB for Free Space |  |  |  |
| Separation distance between reference cell of Part A and Part B | Cell Range *(3/2) corresponding to the BS Inter-Site distance 700-800 MHz: 900 m in urban,12 km in rural 2600 MHz: 600 m in urban, 6 km in rural |  |  |  |
| Indoor/outdoor UEs | Urban: 70% indoor, 30% outdoor Rural: 50% indoor, 50% outdoor |  |  |  |
| UE and drone transmission power scheme | Power control algorithm over -40.23 dBm output power range | Power control algorithm over -40.23 dBm output power range | 3GPP TR 36.942 [8] (Section 12.1.4) and Recommendation ITU-R M.2101-0 [11] |
| Note 1: Based on 3 m height of floor. Note 2: 19.5 m is only valid for 700 MHz because the typical height of a building for 2600 MHz is 20 m. Note 3: 25% for 700 MHz Band, 30% for 2600 MHz because the typical size of building (in urban environment) is lower when deploying antenna in higher frequency. Note 4: Only for the 700 MHz band case. |  |  |  |

Scenario 1 where the BS antenna is pointing to the borderline

Method for determining the no transmission zone

For each frequency band: 700 MHz, 800 MHz, 900 MHz, 1800 MHz, 2100 MHz and 2600 MHz, follow the steps below:

Calculate the distance D0 as described in the section A1.1;

Determine the distance D1 using the simulation scenario and method described in section A1.2 and the simulation assumptions in the section A1.3;

The no transmission zone is the distance to the borderline: D2 = D1 – D0.

Figure 3 Simulation scenario for UAS operation in border area first method

Simulation results

Simulations results first method for 800 band in rural area

Table 4: Simulation results for 800 band in rural area for the first scenario

| D1 inter system distance km | Bitrate loss ref cell % | Victim BS iRSS unwanted Median dBm | D2 no transmission zone km |
| --- | --- | --- | --- |
| 21.5 | 0.9 | -120.1 | 0 |

Those results show that the drones can fly up to the border line without a need of no-transmission zone.

SCENARIO 2 WHERE THE BS ANTENNA IS NOT POINTING TO THE BORDERLINE

Method for determining the no transmission zone

In this method we keep the same parameters as the one given in ECC Report 309 see (2.2 and 2.3 section) except we use a single sector system for network B, and interfering sector from network A which is not pointing to the border.

The simulation scenario for UAS operation in the border area is illustrated in Figure 5.

Figure 5. Simulation scenario for UAS operation in border area second method

We keep also the steps for determining D0 and D2 as in scenario 1.

Results of simulations are detailed below:

Table 5: Simulation results for reference case

| D0 distance to border (km) | FS value at 3m at the border line (dBm) |
| --- | --- |
| 3.15 | -79.1 |

For 800 MHz is rural environment in scenario 2, distance to border D0 calculated to meet the closest the field strength value 59 dB(µV/m)/5 MHz is equal to 3.15 km.

Simulations results second method for 800 band in rural area

As the D0 distance to the border of 3.15 km calculated above is lower than the cell radius used for rural environment, two cases were simulated:

One where inter-site distance D1 is 6.3 km with a cell radius of 3.15 km so that the drone in system A cannot fly above the border.

The second D1 is 8 km and cell radius is set to 4 km as defined in the previous parameters sets.

Table 6: Simulation results for 800 band in rural area second scenario

| D1 inter system distance km | Cell Radius km | Bitrate loss ref cell % | Victim BS iRSS unwanted Median dBm | D2 no transmission zone km |
| --- | --- | --- | --- | --- |
| 6.3 | 3.15 | 2.33 | -118.38 | 0 |
| 8 | 4 | 2.2 | -118.59 | 0 |

The simulation results show that network B reference cell uplink throughput loss is around 2% when drone flies within a sector not pointing at the border area.

Conclusions

The simulation results show that there is no need to define a no-transmission zone for drones’ operation in 800 MHz band in border of rural area for both scenarios.

List of References

ECC Report 309: "Analysis of the usage of aerial UE for communication in current MFCN harmonised bands", approved 3 July 2020

3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

Report ITU-R M.2292: "Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses”

Recommendation ITU-R F.1336-4 (02/2014): "Reference radiation patterns of omnidirectional, sectoral and other antennas for the fixed and mobile service for use in sharing studies in the frequency range from 400 MHz to about 70 GHz"

Document ITU-R WP5D 416: "Use of IMT for audio-visual distribution and production"

ECC Report 82: "Compatibility study for UMTS operating within the GSM 900 and GSM 1800 frequency bands", approved June 2006

ECC PT1(10)128: “LS to STG on LTE parameters for SEAMCAT”

3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios"

3GPP TS 36.101 V15.4.0: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

Recommendation ITU-R P.1546-5 (09/2013): "Method for point-to-area predictions for terrestrial services in the frequency range 30 MHz to 4 000 MHz"

Recommendation ITU-R M.2101-0 (02/2017): "Modelling and simulation of IMT networks and systems for use in sharing and compatibility studies"