From:	ETSI Secretariat

Subject:	Report of ETSI recent activities

# Summary

This report summarises the recent activities of ETSI in the field of Spectrum & Equipment Regulation.  The status of Harmonised Standards and System Reference documents in draft and recently published is given in annexes with links to the ETSI Work Programme.

# Progress on ongoing issues

## Recently published Harmonised standards under RED article 3.2 (M/536)

The last listing of Harmonised Standards in the OJEU was on 10 October 2020, which listed eight  Harmonised Standards. In total in 2020 eighteen Harmonised Standards were cited.

ETSI expresses growing concern over the time between the ETSI publication and the citation of Harmonised Standards in the OJ.  In broad terms, this mean time has doubled between 2020 and 2018, and tripled between 2020 and 2016.  This time lapse includes a number of factors including the time for Commission’s internal processes and the time for the treatment of Commission communications post-publication by ETSI and by third parties that the Commission has decided to consult.  It also includes one case where the Commission has chosen to cite an old version of a Harmonised Standard with an additional restriction.  Details of the standards cited in 2020 and those published in 2020 and awaiting citation are included in annexes 1 and 2.

## Cochlear Implants

The Commission had advised in EG-RE#5 (2 March 2020) that cochlear implants should be covered by the RED (although they had been excluded from the RTTED).  The Commission had commented that “harmonised standards should proportionately address the risks (e.g. of interfering and of being interfered) of this equipment” (see EG RE(06)03, Minutes of EG RE 05).

Following concerns raised by the European Commission that the existing EMC standards for cochlear implants should not be used to address the article 3(2), and at the request of the European Commission, ETSI has approached the CEPT to identify any radio services that share spectrum with cochlear implants and which may present the above risks so that a technical sharing conditions could be developed as a basis for these Harmonised Standards.

ETSI TC ERM TG17 is expected to contact the CEPT Short-Range Devices Maintenance Group to progress this issue.

## Short-Range Devices: Inductive applications operating below 9 kHz

At the request of the Commission, ETSI has requested CEPT to provide information on radio services operating below 9 kHz, which can be used to derive sharing conditions that can be respected in Harmonised Standards.  CEPT has extended EFIS to permit Administrations to record spectrum use in this band and send a circular letter requesting Administrations to provide the necessary information.

ETSI requests CEPT Administrations to provide this information as soon as possible to enable spectrum-sharing conditions to be identified and documented that would be respected by Harmonised Standards.

## WAS/RLANs at 5 GHz and 6 GHz

ETSI BRAN has contributed to the CEPT/ECC work under mandate to the RSC to review the spectrum-sharing conditions for WAS/RLAN at 5 GHz in the light of discussions at WRC-19, and to develop spectrum-sharing conditions for the 6 GHz band.  Harmonised Standards EN 301 893 (5 GHz) is being modified, and EN 303 687 (6 GHz) is under development.  The CEPT/ECC studies have not indicated any change in the existing conditions for reception by WAS/RLANs

## Possible invocation of Article 3(3) & 4 of RED

## 3(3)(a). “Common charger”

EN 301 489-34 (External Power Supplies (EPS) for mobile phones), the version cited under the EMC Directive (V1.4.1, 2013-05) has been superseded by V2.1.1 (2019-04). Because of EC interventions on EN 301 489-1 V.2.1.1, which is a normative reference, this version will not be submitted to the Commission for citation. ETSI may revise EN 301 489-34 V2.1.1 once the Commission’s position regarding the “common charger” has stabilised. The Commission has informed the Experts Group on Radio Equipment of progress on this issue.

## 3(3)(d,e,f). “Cybersecurity”

Preliminary versions of a Delegated Act invoking these articles and an associated draft Standardisation Request have been discussed to the Expert Group on Radio Equipment in February 2021.  ETSI has made initial points of principle, notably that Standardisation Request needs to be clear, unambiguous and feasible within its timescale.  In order to avoid the problems experienced under the current mandate M/536, ETSI has is identified that the criteria for acceptance of Harmonised Standards need to be clearly documented and objectively verifiable to establish a predictable regulatory framework.

## Articles 3(3)(i) & 4 “Software”.

Invocation of these articles has been discussed to ensure that a compliant radio product remains compliant after the installation of a software update.  ETSI has discussed principles of how compliance could be demonstrated with Harmonised Standards. It is anticipated that the Commission will begin drafting of the necessary acts to invoke the essential requirement and also the associated standardization request when the work on Cybersecurity has progressed furher.

# Long-running issues causing disruption to the work programme under RED

## Selection of (particularly receiver) parameters

This remains the principal issue preventing HS from being cited, particularly in innovative areas where “classical” parameters are not appropriate as they assume a classical receiver architecture.  ETSI ERM TG UWB is revising TS 103 567 v1.1.1 (2019-09) and EN 303 883 v 1.1.1 (2016-09) into a 2-part EN (on National Vote until 21 February 2021) on “signal interference handling” which is based on presumed interferers and is independent of the receiver technology.

These ENs are not Harmonised Standards in themselves but are intended to be cited as Normative Reference in Harmonised Standards for UWB and other short-range devices.  This is a technology-neutral method which should improve the consistency of selection of parameters across HS, as requested by the European Commission in May 2018.

In addition, the Commission has requested ETSI to consult CEPT before concluding on this topic.  This is being discussed in the context of work item SE21_24, which is scheduled to terminate at the end of 2022. CEPT has also published ECC Report 310 (2019-12), taking note of the parameters that the CEPT considers necessary for management of the radio spectrum.  It concludes that “reference sensitivity” (a term used principally in 3GPP standards) is important in the definition of blocking tests, which need to be carefully defined, although this is not a parameter necessary for spectrum management in its own right.  Other HS use similar concepts to “reference sensitivity” but with other terminology.

## Assessment of (draft) Harmonised Standards

Under Standardisation Requests, the ETSI TC has entire responsibility to develop the technical provisions that it considers necessary to form the basis of presumption of conformity.  The Commission’s consultants (HASTAC) provide their assessment as an assistance to the TC during drafting, and the consultants will ultimately advise the Commission, but they do not take away the powers or responsibilities of either the TC or the Commission.

If the TC disagrees with the consultant’s opinion, this does not formally prevent the TC proceeding to ENAP and the eventual adoption of the Harmonised Standard, nor does it prevent ETSI from submitting the adopted HS to the Commission to be cited in the OJEU.  If the TB disagrees with the assessment of the HASTAC, ETSI can always dispute the assessment and inform the Commission of their reasons.

The final decision whether to cite an HS in the OJEU (after publication by ETSI and delivery to the EC) lies entirely with the Commission. It is outside the control of ETSI and it is not determined by the Commission’s consultant (although the Commission can be expected to give the consultant’s  recommendation due consideration, of course).

Although the TCs formally have the possibility to progress a HS in cases of disagreement with the Commission’s consultants, some TCs do not have confidence that their arguments will be given a fair hearing.  In these cases we observe a growing tendency for TCs to produce alternative documents that represent the state of the art and which can be used as the basis of assessment by Notified Bodies without embarking on a citation process which has become long and unpredictable.

This tendency is increased by a growing number of inconsistencies observed:

- between successive assessments by HASTACs,

- between the HASTACs assessments and the assessments made by Commission staff

- between decisions by the Commission staff on similar standards.

In particular a number of cases have been observed recently where the Commission had criticised draft Harmonised Standards because they refer to other Harmonised Standards which are already cited in the OJEU.

Ongoing discussions with the Commission (via its consultants) of selection of technical parameters has also delayed the approval of standards for satellite earth stations and Digital Mobile Radio, where the Commission has asked for specific written justification if its choices are not followed.

ETSI remains ready to work with the Commission to establish a clear, unambiguous and objectively verifiable requirements for Harmonised Standards to be cited in the OJEU.  statement of the Commission’s requirements that the Committees can use as a reliable basis to develop Harmonised Standards with confidence that they will be cited.

## Commission proposals to cite previous versions of HS with restrictions

Following a request from a Member State, ETSI has been informed (Ares(2021)1343504, 18 February 2021) of Commission plans to cite some old versions of Harmonised Standards with a comment in the OJEU adapting certain provisions to provisions in spectrum regulations.

ETSI encourages Member States to comment into the ETSI process as members.  If that is not possible, they should at least contribute during the Public approval process (ENAP) so that their comments can be considered in a timely and transparent manner.  Administrations may comment to the Commission when HS are being considered for citation, but this to exceptional cases if errors have occurred (in principle it should never be needed). Comments post adoption require a complete ENAP to take account of their views in a new version, and undermines the transparency & openness of the NLF.

| Version proposed to be cited with remark | More recent versions |
| --- | --- |
| EN 302 208 v 3.1.1 (2016-10) | v 3.3.1 (2020-08) Published |
| EN 302 567 v 1.2.1 (2012-01) | v 2.1.1 (2017-07) Published v2.2.0 (2020-12) on ENAP |
| EN 301 444 V2.1.2 (2016-11) | v2.2.1 (2021-02) on National Vote procedure |
| EN 300 487 V2.1.2 (2016-11) | v2.1.5 (2020-11) in draft (reviewing HASTAC assessment) |
| EN 301 426 V2.1.2 (2016-11) | v2.1.5 (2020-11) in draft (reviewing HASTAC assessment) |
| EN 301 473 V2.1.2 (2016-11) | v2.1.5 (2020-11) in draft (awaiting HASTAC assessment) |
| EN 301 681 V2.1.2 (2016-11) | v2.1.4 (2020-11) in draft (reviewing HASTAC assessment) |

A detailed reply to this Commission’s letter is under preparation.

## Satellite Earth Stations

A fruitful dialogue has been established with HAS consultant (for the EC) in view of citation in the OJ of several TC SES’s ENs under RED

ETSI reported in EG-RE#07 that it had received HAS comments for the following 3 ENs:

- EN 303 981 (on National Vote until 6 April 2021)

- EN 303 699 (published 2021-02) and

- EN 302 186 (on ENAP until 26 April 2021)

A number of Harmonised Standards for the “L-band” (1,5/1,6 GHz) had been modified at the request of CEPT/ECC/PT1 to take account of the use of the band below 1518 MHz by MFCN.  Progress on these HS had been delayed by discussions with the Commission (via the Consultants) which required the Committee to produce specific justifications when the Commission’s selection of technical parameters is not followed.  EN 301 444, currently on National Vote until 12 April 2021, is being used to assess whether the solution which has been found to satisfy the Commission’s consultants also satisfies the Commission staff. If the Commission’s assessments are consistent, this should allow a number if HS addressing the L-band to be cited.

## Digital Mobile Radio (DMR)

ETSI has published EN 300 113 v3.1.1 (2020-06).  It contains technical revisions in particular concerning receiver parameters (adjacent/alternate channels & co-channel  and editorial modifications to v2.2.1.  This version is not intended to be submitted for listing in OJEU although this is not excluded at a later date.

Work items on another ten Private Mobile Radio standards had been stopped in ERMTGDMR following disagreement with the Commission representatives on selection of technical parameters.  It is hoped that the experience gained in the use of this new version of EN 300 113 will allow confidence to be regained and work to recommence on the others

## Electromagnetic Compatibility (EMC)

Although the Commission had confirmed that it had no technical concern with EN 301 489-1, it had also informed ETSI that it will no longer cite Harmonised Standards with a “generic” or “non-specific” scope.  This EN 301 489-1 reflects the “generally-accepted state of the art” and can be used as a Normative Reference for other parts of the EN 301 489 series or as the basis of a technical file to be assessed by a Notified Body.

ETSI has since received Commission comments on EN 301 489 parts 17 and 22 that criticise CENELEC Harmonised Standards which have been cited in the OJEU that are used as a Normative Reference in the ETSI HS. ETSI is in discussion with CENELEC TC210 to resolve these issues.

## Over-the Air (OTA) receiver requirements

Background: Standardisation Request M/536 under the RED specified that “Receiver performance is also of particular importance for mobile terminals, in particular antenna performance…”.  TC MSG has held a series of meetings of an ad-hoc group to develop such requirements for LTE User equipment, which are intended to be incorporated in a future version of EN 301 908-13.

ETSI MSG has completed a study on requirements for LTE UE, see TR 103 803 (v1.1.1, 2020-12)).  A new work item has been adopted to revise EN 301 908-13 to include Over the air (OTA) antenna performance requirements in terms of Receiver Total Radiated Sensitivity (TRS) and Total Radiated Power (TRP) for E-UTRA UEs in speech mode and Beside Head and Hand position

## Other Standardisation Requests

## E112 – Smartphones to provide position information via Galileo / WAS during emergency calls (Standardisation Request M/568)

The Commission has invoked article 3(3)(g) (“access to emergency services”) for smartphones via Commission Implementing Decision 2019/320.  Equipment needs to comply by 17 March 2022. ETSI had been consulted on a draft Standardisation Request M/568 for Harmonised Standards, and had identified significant shortcomings that rendered the request unfeasible. The version finally addressed to the EC Committee on Standards had addressed some of the shortcomings, but the substantial issues that remained unresolved were not signalled to the Committee. As the Commission confirmed that conditional acceptance of the SR was not possible, ETSI regretfully could not accept the SR as drafted.

ETSI remains committed to supporting the Commission in developing a revised Standardisation Request that would both be feasible and would meet the regulatory objective.

## ETSI deliverables produced under other Standardisation Requests

ETSI has recently accepted the following Standardisation requests on other fields:

- M/570: in support of Directive (EU) 2016/797 for the Future Rail Mobile Communication system (‘FRMCS’).

- M/573: in support of Regulation (EU) 2019/424 laying down ecodesign requirements for servers and data storage products

ETSI has delivered the following under standardisation requests in 2020:

- M/390 (European Air Traffic Management Network): 6 “Community Specifications”

- M/441 (Smart Meters): 8 Technical Specifications

- M/453 (Intelligent Transport Systems): 8 deliverables (1 TR, 5 TS, 2 EN)

- M/460 (Electronic Signatures): 4 deliverables (2 TS, 2 EN)

- M/462 (Energy efficiency): 4 deliverables (2 TS, 2 ES)

- M/496 (Space): 5 Technical Specifications

- M/530 (Privacy and personal data protection): 1 Technical Specification

- M/536 (Radio Equipment Directive): 27 HS (18 cited in OJEU)

- M/546 (Intelligent Transport Systems): 3 deliverables (2 TS, 1 EN)

- M/554 (Accessibility): 1 EN

## Draft SR on On-board Weighing equipment

ETSI has also been consulted on a draft Standardisation Request regarding on-board weighing equipment in support of Regulation (EU) 2019/1213 (https://ec.europa.eu/docsroom/documents/44771).  ETSI has submitted comments (developed in TC ITS) to the Commission which should be incorporated before the draft is submitted to Committee on Standards for advisory opinion.

# Issues from previous meetings if the Expert Group on Radio Equipment that are now resolved

## User Access Restrictions

Background: An issue had arisen in HASTAC comments on Harmonised Standards for Wireless Industrial Automation (EN 303 258) and EN 301 598 in TV White Space technology. 5 GHz Radio LANs (EN 301 893) v 2.1.1 is currently cited in the OJEU with requirements on user access restrictions with similar text.  Administrations in TC BRAN had expressed different views on whether these requirements should continue to be included in Harmonised Standards.

Following discussions in EG-RE(07), the Commission had reported the following conclusion to the ETSI Board meeting#129, 22 to 24 September 2021.

- The clause “user access restrictions” should remain in EN 301 893;

- The clause “user access restrictions” should be reintroduced in EN 301 598 V2.1.1 in the ongoing revision of the harmonised standard;

- A suitable test method for the equipment covered by that standard should be inserted, at least for the (sub)categories for which it is possible to insert it.

## Equipment used in locally-managed spectrum (e.g. Radio Microphones, Broadcast Transmitters, Assistive Listening Devices, Vision Links, Private Mobile Radio (PMR), Fixed Radio Links)

Background: Certain equipment (e.g. PMSE) needs to be adjustable by the user to meet spectrum-use conditions that are not known at the point of placing on the market, but may be set, for example, for a special event or for long-term use at a specific location.  These devices typically use a “tuning range” to take advantage of Member States’ allocation of spectrum on a geographical, site-specific or National basis.  The user needs to be aware of the local regulatory conditions of use when operating the equipment and adjust the equipment to respect them.  These conditions of use are typically enforced by National spectrum-management authorities, not market-surveillance authorities.

This issue had initially arisen with PMSE equipment; however, it affects all equipment which is used in non-Harmonised spectrum whose requirements may be set Nationally and may be defined after the equipment has been placed on the market.  Systems affected include PMSE, radio & TV broadcast transmitters, Private Mobile Radio (PMR) and fixed radio links that are individually licenced

In the 71st meeting of the RSC, the Commission confirmed that “Such devices typically use a “tuning range” to take advantage of Member States’ allocations on a geographical basis; the responsibility lying on the user to adjust the equipment within the tuning range in line with the technical conditions of the individual authorisation unless advanced automatic selection techniques are used, the responsibility lying on the manufacturer that the device respects these selected technical conditions”.

## Work items for ENs that cover equipment requirements within the scope of RED that are requested to be included under M/536

The Commission also confirmed to the Board meeting#129 that ETSI can continue producing related harmonised standards under this "old" standardisation request until the new standardisation request has been issued, as long as these standards concern the currently applicable essential requirements

# Annex 1: Harmonised Standards cited in the OJEU under RED in 2020

The last listing of Harmonised Standards in the OJEU under the RED was on 27 October 2020 bringing the total cited in 2020 to 18.

|  |  |  | Cover date | Citation date | Days for Final Citation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN | 300 328 | 2.2.2 | 2019-07 | 05-Feb-20 | 196 | HS Wideband transmission systems 2,4 GHz | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=51206 |
| EN | 300 674-2-2 | 2.2.1 | 2019-06 | 05-Feb-20 | 229 | HS DSRC OBU 5,8 GHz | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=52895 |
| EN | 300 698 | 2.3.1 | 2018-11 | 05-Feb-20 | 442 | Maritime VHF for use on inland waterways | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=53986 |
| EN | 301 908-1 | 13.1.1 | 2019-11 | 22-Apr-20 | 166 | Part 1 Update to 3GPP Release 13 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=53611 |
| EN | 301 908-13 | 13.1.1 | 2019-11 | 27-Oct-20 | 334 | E-UTRA UE v13.1.1 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50585 |
| EN | 301 908-14 | 13.1.1 | 2019-09 | 22-Apr-20 | 205 | FDD BS v13.1.1 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=49569 |
| EN | 301 908-18 | 13.1.1 | 2019-09 | 22-Apr-20 | 205 | FDD MSR BS v13.1.1 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=49574 |
| EN | 301 908-2 | 13.1.1 | 2020-06 | 27-Oct-20 | 133 | FDD UE v13.1.1 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50586 |
| EN | 301 908-3 | 13.1.1 | 2019-09 | 22-Apr-20 | 205 | FDD BS 13.1.1 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=49568 |
| EN | 302 065-3 | 2.1.1 | 2016-11 | 06-Feb-20 | 1178 | HEN for UWB for ground based vehicular applications | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=46666 |
| EN | 302 217-2 | 3.2.2 | 2020-02 | 27-Oct-20 | 249 | HS Point-to-Point 1 GHz to 86 GHz | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50981 |
| EN | 303 098 | 2.2.1 | 2019-02 | 05-Feb-20 | 356 | Maritime survival locating devices - AIS | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=51215 |
| EN | 303 213-5-1 | 1.1.1 | 2020-03 | 27-Oct-20 | 223 | Receivers and interrogators for MLAT | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=46589 |
| EN | 303 213-6-1 | 3.1.1 | 2019-07 | 27-Oct-20 | 476 | X-band sensors for A-SMGCS | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=51030 |
| EN | 303 345-2 | 1.1.1 | 2020-02 | 27-Oct-20 | 263 | HS AM broadcast sound receivers | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=52951 |
| EN | 303 345-5 | 1.1.1 | 2020-02 | 27-Oct-20 | 263 | HS DRM broadcast sound receivers | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=52954 |
| EN | 303 364-3 | 1.1.1 | 2019-07 | 27-Oct-20 | 476 | PSR in X band (8500-10000 MHz) | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=49549 |
| EN | 303 520 | 1.2.1 | 2019-06 | 05-Feb-20 | 230 | HS for ULP-Capsule Camera | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56335 |

# Annex 2: Harmonised Standards under RED published and submitted to EC since 1 Jan 2020, not cited in the OJEU

|  |  |  | Cover date | Sent to EC |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| EN | 301 489-17 | 3.2.4 | 2020-09 | 10-Sep-2020 | EMC for broadband data transmission systems | EC request for clarification (Ares(2020)6044446, 28 Oct 2020), Status in ERMWGEMC: Addressed in resolution meeting 26 Nov 2020; As issues refer to references to cited CENELEC HS, a reply is being drafted with CENELEC TC210 and is expected to be sent shortly. |
| EN | 301 489-22 | 2.1.1 | 2020-10 | 28-Oct-2020 | EMC for Ground-based aeronautical | EC request for clarification (Ares(2020)7712495, 17 Dec 2020), ERMWGEMC discussed at its meeting, 27 to 29 Jan 2021. Most issues are covered by discussions under part 17. The response is expected to cover both standards. |
| EN | 302 066 | 2.2.1 | 2020-06 | 03-Jul-2020 | Ground- and Wall- Probing Radio determination (GPR/WPR) devices; | EC questions for clarification (Ares(2020)4212971 (11 August); responded 25 August; response from Commission (Ares(2020)7253463, 1 Dec 2020) confirming negative assessment or possible citation with restriction. ETSI reply 18 Dec 2020. Awaiting EC decision to cite Further EC reply Ares(2021)305026, 14 Jan: EC considering possible text for restriction. |
| EN | 302 208 | 3.3.1 | 2020-08 | 26-Aug-2020 |  | EC questions for clarification received (Ares(2020)5221310, 26 Aug 2020), response 5 Nov 2020. Revision WI expected. Awaiting EC decision to cite (possibly with a restriction)2 |
| EN | 302 609 | 2.2.1 | 2020-10 | 25-Nov-2020 | Radio equipment for Euroloop railway systems. | EC request for clarification (Ares(2020)7728632, 18 December 2020); being reviewed by ERM TG28. Two obvious minor errors to be corrected; New Work Item under approval. Reply being drafted by ERM TG28. Awaiting EC decision whether to cite (possibly with a restriction)2 |
| EN | 303 135 | 2.2.1 | 2020-08 | 25-Nov-2020 | CS/VTS/HR Radars | EC request for justification re: receiver radio-frequency intermodulation (Ares(2021)111100, 6 Jan 2021). Reply sent 15 January. Awaiting EC decision to cite2 |
| EN | 303 258 | 1.1.1 | 2020-04 | 20-Apr-2020 | HS WIA 5,8 GHz | EC questions for clarification (Ares(2020)420600, 11 Aug 2020); responded by ERM TG41 11 Dec 2020; Further EC comments (Ares(2021)160316, 8 January 2021). Reply sent 22 January Awaiting EC decision to cite1 |
| EN | 303 340 | 1.2.1 | 2020-09 | 21-Oct-2020 | Digital Terrestrial TV Broadcast Receivers | EC requests for clarification (Ares(2020)5762790, 22 Oct 2020). Response sent 17 Feb 2021. Awaiting EC decision to cite. |

# Annex 3: Draft Harmonised Standards under RED: status changed since 1 Jan 2020

| EN |  | MWSSC |  | TB adoption of WI | 13-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=62032 |
| --- | --- | --- | --- | --- | --- | --- |
| EN |  | mmW SSC |  | TB adoption of WI | 13-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=62031 |
| EN | 300 220-2 | SRDs (25 MHz to 1 GHz)-Part 2 | 0.0.5 | Stable draft | 11-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58934 |
| EN | 300 224 | Paging service |  | Start of work | 20-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58902 |
| EN | 300 386 | Revision of EN300386 | 2.1.23 | Draft Review after PE | 29-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=53551 |
| EN | 300 422-1 | Wireless microphones, Audio PMSE up to 3 GHz Part 1 | 0.0.13 | Stable draft | 29-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=53883 |
| EN | 300 422-4 | Wireless microphones, Audio PMSE up to 3 GHz Part 4 ALD | 0.0.3 | Stable draft | 21-Sep-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=53882 |
| EN | 300 440-1 | Radiocommunication equipment in 1-40 GHz | 0.0.5 | Early draft | 25-Jun-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=57422 |
| EN | 300 440-3 | Intrusion detection |  | Start of work | 06-May-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58936 |
| EN | 300 487 | Receive-Only Mobile Earth Stations (ROMES) 1,5 GHz | 2.1.5 | EC Assessment Received | 09-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50117 |
| EN | 300 674-2-1 | HS DSRC RSU 5,8 GHz | 2.2.6 | EC Assessment Received | 13-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=52870 |
| EN | 300 718-1 | Avalanche beacon equipment for buried people | 0.0.11 | Draft receipt by ETSI Secretariat | 19-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=57408 |
| EN | 301 025 | VHF Class D DSC | 2.2.2 | Start of TB approval process | 27-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=57503 |
| EN | 301 091-1 | Harmonised standard 76 GHz vehicular radars. |  | Start of work | 10-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59542 |
| EN | 301 091-2 | Harmonised standard 76 GHz infrastructure radars |  | Start of work | 10-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59543 |
| EN | 301 091-3 | Harmonised standard 76 GHz railway |  | Start of work | 10-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59541 |
| EN | 301 406-1 | HS for DECT, DECT-evolution and DECT-ULE operating in the 1 880 MHz - 1 900 MHz band |  | Start of work | 01-Nov-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=61428 |
| EN | 301 406-2 | HS for DECT-2020 NR operating in the 1 880 MHz - 1 900 MHz band |  | Start of work | 01-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=61429 |
| EN | 301 426 | Proposed revision to EN 301 426 to include the extended L-band. | 2.1.5 | EC Assessment Received | 01-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50121 |
| EN | 301 444 | LMES operating in the 1,5 GHz and 1,6 GHz L- and extended L-band. | 2.2.1 | Start of Vote | 11-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50120 |
| EN | 301 473 | AES below 3 GHz AMSS/MSS and/or AMS(R)S/MSS | 2.1.5 | Mature draft sent to EC and recog. Org. | 19-Nov-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50119 |
| EN | 301 489-12 |  | 0.0.4 | WG approval | 01-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58100 |
| EN | 301 489-13 | EMC for CB equipment | 0.0.5 | Early draft | 31-Aug-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=53844 |
| EN | 301 489-19 | EMC for Receive Only Mobile Earth Stations (ROMES) and GNSS receivers | 0.0.7 | Draft Review after PE | 29-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58112 |
| EN | 301 489-20 | EMC for Earth Stations in the Mobile Satellite Service | 0.0.4 | WG approval | 01-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58103 |
| EN | 301 489-28 | EMC for digital wireless video links between 1.3 GHz & 78 GHz. | 0.0.7 | EC Assessment Received | 24-Nov-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=53904 |
| EN | 301 489-3 | EMC for Short Range Devices | 0.0.6 | WG approval | 01-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58095 |
| EN | 301 489-50 | EMC for Cellular Communication BS, repeater and ancillary equipment | 2.2.2 | Start of EN Approval Procedure | 07-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58114 |
| EN | 301 489-52 | EMC for mobile phones | 1.1.2 | Start of EN Approval Procedure | 15-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=47939 |
| EN | 301 489-54 | Aeronautical and meteo EMC | 0.0.10 | Stable draft | 09-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=57413 |
| EN | 301 598 | HS WAS 470 MHz to 790 MHz | 2.1.7 | Final draft for approval | 04-Mar-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54529 |
| EN | 301 681 | GEO Mobile Earth Stations 1,5/1,6 GHz (MSS) | 2.1.4 | EC Assessment Received | 07-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50118 |
| EN | 301 893 | HS for 5 GHz RLANs | 2.1.41 | Stable draft | 11-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=53378 |
| EN | 301 908-1 | Part 1 Update to 3GPP Release 15 | 13.0.10 | EC Assessment Received | 25-Jan-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=57418 |
| EN | 301 908-10 | HS for IMT-2000, DECT | 4.2.15 | Mature draft sent to EC and recog. Org. | 21-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50941 |
| EN | 301 908-13 | E-UTRA UE v13.2.1 | 0.0.6 | Stable draft | 22-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=61364 |
| EN | 301 908-14 | E-UTRA BS v15.1.1 | 13.0.11 | End of pre-processing | 04-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=57417 |
| EN | 301 908-18 | 3GPP Rel-15 update including NR | 13.0.17 | EC Assessment Received | 30-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56368 |
| EN | 301 908-23 | AAS BS V15.1.1 | 0.0.6 | Stable draft | 23-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50990 |
| EN | 301 908-24 | NR BS V15.1.1 | 0.0.16 | Final draft for approval | 28-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54728 |
| EN | 301 908-25 | Part 25 NR UE V15.1.1 | 0.0.3 | Stable draft | 16-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54786 |
| EN | 302 064 | HS WVL operating in the 1,3 GHz to 50 GHz frequency band | 0.0.11 | Stable draft | 03-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54468 |
| EN | 302 065-3-1 | UWB keyless entry | 0.1.8 | Stable draft | 09-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=55073 |
| EN | 302 065-4-1 | BMA | 0.1.12 | Final draft for approval | 11-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=55074 |
| EN | 302 065-4-3 | ground sensor | 0.0.1 | Early draft | 21-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58195 |
| EN | 302 065-4-4 | UWB vehicular sensors | 0.0.5 | Stable draft | 09-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58462 |
| EN | 302 065-4-5 | parking lot sensor |  | Start of work | 29-Apr-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58927 |
| EN | 302 077 | HS DAB transmitters VHF band III | 0.0.3 | EC Assessment Received | 03-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56352 |
| EN | 302 186 | Ku-band AES 11/12/14 GHz | 2.2.0 | Start of EN Approval Procedure | 26-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=51021 |
| EN | 302 194-1 | Magnetron based Navigation radar used on inland waterways | 0.0.6 | Early draft | 02-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=51395 |
| EN | 302 195 | ULP-AMI and ULP-AMI-P in the 9 kHz to 315 kHz range | 2.1.4 | Early draft | 03-Jun-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54438 |
| EN | 302 217-2 | HS Point-to-Point 1 GHz to 86 GHz | 0.1.2 | Final draft for approval | 22-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59275 |
| EN | 302 245 | HS DRM transmitters | 0.0.3 | EC Assessment Received | 09-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56353 |
| EN | 302 248-1 | non-SOLAS magnetron radars | 0.0.5 | Early draft | 07-Feb-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=47942 |
| EN | 302 296 | HS Digital Terrestrial TV Transmitters | 0.0.5 | Draft receipt by ETSI Secretariat | 02-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54470 |
| EN | 302 326-2 | HS Digital Multipoint Radio Equipment | 2.1.0 | EC assessment before WNV received | 15-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=46680 |
| EN | 302 480 | MCOBA / NCU HS | 2.1.12 | Final draft for approval | 09-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=52988 |
| EN | 302 567 | HS 60 GHz Multiple-Gigabit/s equipment | 2.2.0 | Start of EN Approval Procedure | 18-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54005 |
| EN | 302 571 | C-ITS HS | 0.0.15 | Stable draft | 21-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=51356 |
| EN | 302 608 | Radio equipment for Eurobalise railway systems |  | Start of work | 06-May-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58435 |
| EN | 302 858 | Harmonised standard 24 GHz narrowband vehicular radar |  | Start of work | 30-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=61445 |
| EN | 303 132 | Maritime survival locating device - DSC | 0.0.2 | Early draft | 17-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56835 |
| EN | 303 204 | Networked SRDs 870 – 876 MHz | 3.1.0 | Start of Vote | 22-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56355 |
| EN | 303 213-5-2 | Reference and vehicle transmitters for MLAT | 0.0.14 | Stable draft | 17-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=46588 |
| EN | 303 345-3 | HS FM broadcast sound receivers | 0.0.15 | Draft Review after PE | 15-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=52952 |
| EN | 303 345-4 | HS DAB broadcast sound receivers | 0.0.12 | Draft Review after PE | 15-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=52953 |
| EN | 303 347-1 | S-Band Meteorological radars | 1.1.4 | Waiting - see "Remarks" | 14-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50827 |
| EN | 303 347-2 | C-Band Meteorological radars | 1.1.4 | Waiting - see "Remarks" | 14-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=47114 |
| EN | 303 347-3 | X-Band Meteorological radars | 1.1.4 | Waiting - see "Remarks" | 14-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50828 |
| EN | 303 348 | HS Inductive loop for hearing impaired in 10 Hz to 9 kHz | 0.0.7 | TB approval | 08-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54462 |
| EN | 303 354 | HS Amplifiers and active antennas for broadcast reception in domestic premises | 1.1.5 | Stable draft | 14-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54738 |
| EN | 303 363-1 | SSR Interrogator | 1.0.1 | Start of EN Approval Procedure | 15-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=47109 |
| EN | 303 363-2 |  | 0.1.10 | Early draft | 17-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54081 |
| EN | 303 364-1-1 | PSR in L band (1215-1400 MHz) | 0.0.9 | Stable draft | 26-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=49550 |
| EN | 303 364-1-2 | Phased array L band PSR | 0.0.2 | Early draft | 12-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=61459 |
| EN | 303 364-2 | PSR in S band (2700-3100 MHz) | 1.0.1 | Start of EN Approval Procedure | 20-Nov-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=47110 |
| EN | 303 372-1 | HEN Broadcast receivers Part-1: Outdoor unit receiving in the 10.7 to 12.75 GHz | 0.0.8 | Final draft for approval | 08-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=52868 |
| EN | 303 372-2 | HEN satellite broadcast receivers indoor unit | 0.0.6 | Final draft for approval | 14-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=52869 |
| EN | 303 413 | GNSS receivers harmonised standard | 1.2.1 | Start of Vote | 11-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54047 |
| EN | 303 447 | Robotic Mowers | 1.2.0 | Start of TB review after AP comments | 05-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54100 |
| EN | 303 489 | Wide Area Multilateration (WAM) Harmonised Standard | 0.0.5 | Early draft | 11-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50826 |
| EN | 303 623 | AeroMACS | 0.0.2 | Early draft | 11-Mar-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=54479 |
| EN | 303 659 | SRD in Data Networks in 865-868 MHz/915-919.4MHz | 0.0.5 | Early draft | 22-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=57420 |
| EN | 303 661 | Ground Based Synthetic Aperture Radar | 0.0.8 | Early draft | 28-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=57436 |
| EN | 303 687 | HS for 6 GHz RLANs | 0.0.11 | Stable draft | 10-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58036 |
| EN | 303 722 | WDTS for Fixed Networks in the 57 - 71 GHz band | 0.0.6 | Stable draft | 11-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58483 |
| EN | 303 734 | Inductive glueing systems |  | Start of work | 06-May-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58937 |
| EN | 303 753 | WDTS for Mobile and Fixed Equipment in the 57 - 71 GHz band | 0.0.2 | Early draft | 15-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59482 |
| EN | 303 758 | HEN for TETRA | 0.0.13 | Final draft for approval | 20-Jan-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59457 |
| EN | 303 978 | GEO ESOMP 27,5 GHz to 30,0 GHz | 2.1.3 | Waiting - see "Remarks" | 16-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=50122 |
| EN | 303 980 | EN for Ku-band NGSO terminals | 1.2.0 | Start of EN Approval Procedure | 05-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58933 |
| EN | 303 981 | Non-GEO WBES 11 to 14GHz | 1.2.0 | Start of Vote | 05-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56941 |
| EN | 304 220-1 |  | 0.0.4 | Stable draft | 29-Sep-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56361 |
| EN | 304 220-2 |  | 0.0.3 | Stable draft | 12-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56362 |
| EN | 305 550-5 | USRCD mmW communication |  | Start of work | 14-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59468 |

# Annex 4: Other supporting documents in preparation & published (selection)

## In preparation

| ERM | TR |  | ETSI activities for possible EC delegated acts for RED articles |  | TB adoption of WI | 27-Jul-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59493 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ERM TGUWB | TS | 103 567 | TS on signal interferer handling |  | Start of work | 14-Oct-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=60659 |
| RT JTFIR | TR | 103 580 | Urban Rail ITS & Road ITS in 5,9GHz |  | TB adoption of WI | 07-Oct-2019 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58440 |
| ERM TG37 | TR | 103 667 | TR on an overall framework for shared use of 5.9 GHz spectrum by ITS-G5 & LTE-V2X | 0.0.7 | Stable draft | 08-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=57437 |
| BRAN | TR | 103 721 | WAS/RLAN Mitigation techniques in the 5725 - 5850 MHz | 0.0.3 | Early draft | 24-Sep-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=58200 |
| RT JTFIR | TS | 103 745 | Urban Rail ITS/Road ITS shared use of spectrum |  | TB adoption of WI | 21-Apr-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59449 |
| BRAN | TS | 103 754 | BRAN MAP Performance testing | 0.0.3 | Early draft | 08-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59516 |
| ERM TG37 | TR | 103 766 | TR on co-channel co-existence between ITS-G5 and LTE-V2X | 0.0.16 | Early draft | 14-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56944 |
| ERM WGRM | TR | 103 773 | Reference documents for methods of measurements | 0.0.4 | Early draft | 17-Feb-2021 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59519 |
| ERM TGUWB | EN | 303 883-1 | UWB TX measurement EN | 1.2.1 | Start of Vote | 22-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=55058 |
| ERM TGUWB | EN | 303 883-2 | UWB rx measurementEN | 1.2.1 | Start of Vote | 22-Dec-2020 | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=55059 |

## Published

| TS | 102 933-1 | 2.1.1 | 2015-06 | GSM-R improved receiver parameters | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=44509 |
| --- | --- | --- | --- | --- | --- |
| TS | 102 933-2 | 2.1.1 | 2015-06 | GSM-R improved receiver parameters | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=44510 |
| TS | 103 567 | 1.1.1 | 2019-09 | TS on signal interferer handling | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=52899 |
| TR | 103 580 | 1.1.1 | 2019-08 |  | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=53989 |
| TR | 103 631 | 1.1.1 | 2019-03 | Characteristics of WAS/RLANs in 6 725 to 7 125 MHz | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=56310 |
| TR | 103 803 | 1.1.1 | 2020-12 |  | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=59924 |
| EN | 303 645 | 2.1.1 | 2020-06 | Securing Consumer IoT | http://webapp.etsi.org/workProgram/Report_WorkItem.asp?wki_id=57991 |

In the case of EN 302 065-3 this does not refer to the original citation of the standard, but to the later insertion of a note in the OJEU to reflect evolved spectrum-sharing conditions.

Ares(2021)230358: 11 January 2021