FM60(24)035Rev1 – NOR & UK

FM60(24)036Rev1 – France

Frequency arrangement for the 3800-4200 MHz FREQUENCY BAND based on TDD

The frequency arrangement is a TDD arrangement, based on a block size of 5 MHz starting at the lower edge of the band at 3800 MHz. Multiple adjacent blocks of 5 MHz can be combined to obtain wider channels. To protect MFCN operating below 3800 MHz, a restricted block 3800-3860 MHz is defined for only WBB low power BS.

- Figure 1: 3800-4200 MHz frequency arrangement

LEAST RESTRICTIVE TECHNICAL CONDITIONS SUITABLE FOR LMP WBB IN THE FREQUENCY BAND 3800-4200 MHZ

The technical conditions presented in this annex have been developed assuming an authorisation regime where the base station locations of transmitters and receivers are known.

The least restrictive technical conditions defined in this annex are in the form of a block-edge mask (BEM) applicable to LMP WBB base stations. The BEM includes “baseline” power limits for the protection of MFCN below 3800 MHz and aeronautical radio altimeters above 4200 MHz.

The term block edge refers to the frequency boundary of spectrum licensed to a mobile communication network.

These technical conditions allow unsynchronised operations to perform local vertical market needs.

Non-AAS (non-active antenna systems) refers to WBB LMP base station transmitters which are manufactured and supplied separately to antenna systems.

AAS refers to a base station and antenna system where the amplitude and/or phase between antenna elements is continually adjusted resulting in an antenna pattern that varies in response to short term changes in the radio environment. This is intended to exclude long term beam shaping such as fixed electrical down tilt.

Technical conditions for base stations

Table 1: In-block power limit

|  | Max e.i.r.p. per cell/sector |
| --- | --- |
| Low power BS | ≤ 24 dBm/channel for BW ≤ 10 MHz Otherwise: ≤ 31 dBm/channel ≤ 18 dBm/5MHz |
| Medium power BS above 3860 MHz | ≤ 51 dBm/channel ≤ 38 dBm/5MHz |

Table 2 – BS Minimum Adjacent Channel Leakage power Ratio in 3800-4240 MHz band

| BS e.i.r.p. | Min ACLR |
| --- | --- |
| ≤ 24 dBm/channel | 30 dB |
| > 24 dBm/channel | 45 dB |

Table 3 – Baseline requirement for BS to protect MFCN BS in 3.4-3.8 GHz

|  | Max power per cell/sector in 3.4-3.8 GHz |
| --- | --- |
| Low power BS | -25 dBm/5MHz e.i.r.p. |
| Medium power BS non AAS | -33 dBm/5MHz e.i.r.p. |
| Medium power BS AAS | -38 dBm/5 MHz t.r.p. |

Table 4 – Baseline requirement for BS to protect radio altimeters in 4.2-4.4 GHz

| Frequency range | Max power per cell/sector |
| --- | --- |
| 4200-4240 MHz | Table 2 |
| 4240-4400 MHz | -30 dBm/MHz t.r.p. as per ERC/REC 74-01 |

Administrations wishing to avoid coordination between neighbouring WBB low power BS shall implement the following requirement:

Table 4 – Max field strength requirements

|  | Max field strength at licensed area border |
| --- | --- |
| Low power BS | 32 dBµV/m/5MHz at 3m |

BS receiver requirement

Table 5 – Requirements on receiver characteristics for BS

| Parameter | Value |
| --- | --- |
| Level of the wanted signal level | RefSens + 6 dB |
| Maximum 5 MHz NR interfering signal in 3.4-3.8 GHz | -15 dBm |
| The antenna connector of the BS receiver is the reference point. The reference sensitivity RefSens is the minimum mean power received at the antenna connector for non AAS BS or TAB connector for AAS BS at which a specified minimum performance shall be met for a specified reference measurement channel. These requirements cover both blocking and third-order intermodulation. |  |

UE In-block requirement

UE in-block power requirements:

Table 7 – UE in-block power requirements

|  | Max power |
| --- | --- |
| Mobile terminals | 28 dBm t.r.p. |
| Fixed or nomadic terminals | 28 dBm e.i.r.p. |

In addition, the following parameters apply:

Minimum ACLR: 30 dB

Uplink power control is mandatory and shall be activated

List of references

3GPP power class 2 + a 2 dB tolerance