ECC Recommendation (11)05

Cross-border Coordination for Mobile/Fixed Communications Networks (MFCN) in the frequency band 2500-2690 MHz

approved 26 May 2011

latest corrected 8 March 2024

# introduction

This Recommendation contains provisions for cross-border coordination between Mobile/Fixed Communications Networks (MFCN) in border areas in the frequency band 2500-2690 MHz.

In this Recommendation, MFCN includes LTE and New Radio (NR) non-AAS and AAS base stations.

FDD and TDD modes of operation are considered.

# ECC recommendation (11)05 of 26 May 2011 on Cross-border coordination for Mobile/Fixed Communications Networks (MFCN) in the frequency band 2500-2690 MHz, Amended 3 February 2017, updated 18 November 2022, corrected 8 march 2024, amended DD MM YYYY

“The European Conference of Postal and Telecommunications Administrations,

considering

that the Radio Regulations [1] identify the frequency band 2500-2690 MHz for terrestrial IMT (RR 5.384A);

that “mobile/fixed communications networks” for the purpose of this Recommendation includes IMT and other communications networks in the mobile and fixed services;

that ECC Decision (05)05 [2] provides the harmonised conditions for mobile/fixed communications networks (MFCN) operating in the band 2500-2690 MHz;

that EC Decision of 13 June 2008 on the harmonisation of the 2500-2690 MHz frequency band for terrestrial systems capable of providing electronic communications services in the Community (2008/477/EC) [3] is mandatory for EU and EFTA countries;

that in many CEPT member countries there may be multiple operators for MFCN systems;

that different administrations may wish to adopt different approaches to cross border co-ordination;

that CEPT Report 19 [4] proposes one set of trigger values for cross-border coordination of MFCN systems at 2500-2690 MHz;

that administrations may diverge from the technical parameters, propagation models and procedures described in this Recommendation subject to bilateral / multilateral agreements;

that coordination is necessary between countries operating different technologies and different channel bandwidths in the same frequency band;

that ECC Recommendation (15)01, annex 5 [5] provide guidance for cross-border coordination of synchronised and unsynchronised operation between LTE and NR systems;

that Physical-layer Cell Identity (PCI) coordination is necessary for LTE/NR systems to avoid unnecessary signalling load and handover failures;

that other radio parameters may need to be coordinated for LTE/NR systems on a bi/multi-lateral basis.

recommends

- that cross-border coordination between MFCN systems in border areas should be based on bilateral or multilateral agreements between administrations;

- that cross-border coordination between MFCN systems and other services in neighbouring countries should be based on bilateral / multilateral agreements;

- that bilateral / multilateral agreements should define coordination methods which encompass all MFCN radio interfaces present on each side of the border;

- that cross-border co-ordination between FDD MFCN systems should be based on the principles and the field strength limits provided in ANNEX 1;

- that cross-border co-ordination between TDD MFCN systems should use the principles and the field strength limits provided in ANNEX 2;

- that if the levels in ANNEX 1 and / or ANNEX 2 are exceeded coordination is required and the procedure detailed in ANNEX 4 should be used;

- that the coordination of LTE/NR systems in border areas should be based on the PCIs provided in ANNEX 5 when synchronisation signal centre frequencies are aligned;

- that interference field strength predictions should be made by using one of the propagation models defined in ANNEX 3;

- that administrations should encourage and facilitate the establishment of arrangements between operators of neighbouring countries with the aim to enhance the efficient use of the spectrum and the coverage in the border areas;

- that coordination in coastal areas is based on prediction of field strength levels at the coastline of the neighbouring country. Other principles for co-ordination in coastal areas may be agreed between the administrations concerned;

- that administrations may diverge from the technical parameters and procedures described in this Recommendation subject to bilateral / multilateral agreements;

- that this Recommendation should be reviewed within 5 years of its adoption in the light of practical experience of its application and of the operation of MFCN systems.”

Note:

Please check the ECO Documentation Database https://docdb.cept.org/ for the up to date position on the implementation of this and other ECC Recommendations.

FIELD STRENGTH LEVELS FOR THE CROSS-BORDER OPERATION WITHOUT COORDINATION BETWEEN FDD MFCN SYSTEMS IN THE FREQUENCY BAND 2500-2690 MHz

In this annex, field strength values are given for cross-border scenarios between MFCN FDD systems for non-AAS and AAS base stations on both sides of the borderline.

Preferential and non-preferential PCIs for LTE and NR are given in ANNEX 5.

Administrations/operators may agree on other values of field strength and preferential frequency usage based on bilateral/multilateral agreements/arrangements.

Synchronisation signal means Synchronisation Signal Block (SSB) for 5G NR and Primary/Secondary Synchronisation Signal (PSS/SSS) for LTE.

For non-AAS base stations the field strength values are given for 5 MHz frequency block size. In cases of other frequency block sizes other than 5 MHz, a factor of 10 × log10 (channel bandwidth/(5 MHz)) should be added to the field strength values. The data and control channels are fixed and therefore the same field strength values in dB(µV/m)/5 MHz apply to both channel types. The mean value is with respect to e.g. changing traffic load and downlink transmission time. The field strength values are upper bounds of each cell produced by the base station.

For AAS base stations the upper bound field strength for each cell of a base station is given as the median for a random beamforming data channel with 5 MHz frequency block size and for a static SSB field with SCS 30 kHz. In the case of a SCS other than 30 kHz, a correction factor of 10 × log10 (SCS in kHz/(30 kHz)) dB, should be added to the SSB field strength value.

Field strength levels for MFCN FDD systems in the frequency band 2500-2690 MHz

- Non-AAS and AAS base stations of MFCN FDD systems on both sides of the borderline in the frequency band 2500-2570 MHz with synchronisation signal centre frequencies not aligned for all PCIs or with synchronisation signal centre frequencies aligned and for preferential PCIs may be used without coordination with a neighbouring country if the field strength of each cell produced by base station does not exceed the values of:

65 dBµV/m/(5 MHz) at a height of 3 m above ground at the borderline between countries and

49 dBµV/m/(5 MHz) at a height of 3 m above ground at a distance of 6 km inside the neighbouring country.

- Non-AAS and AAS base stations of MFCN FDD systems on both sides of the borderline in the frequency band 2500-2570 MHz with synchronisation signal centre frequencies aligned and for non-preferential PCIs may be used without coordination with a neighbouring country if the field strength of each cell produced by base station does not exceed the value of:

49 dBµV/m/(5 MHz) at a height of 3 m above ground at the borderline between neighbouring countries.

Table 1 gives an overview of the field strength values for non-AAS and AAS base stations.

Table 1: Field strength values at a height of 3 m above ground for MFCN FDD systems

| Non-Preferential frequency usage |  |  |
| --- | --- | --- |
| Synchronisation signal centre frequencies aligned |  | Synchronisation signal centre frequencies not aligned |
| Preferential PCIs | Non-preferential PCIs | All PCIs |
| 65 dBµV/m/(5 MHz) @ 0 km And 49 dBµV/m/(5 MHz) @ 6 km | 49 dBµV/m/(5 MHz) @ 0 km | 65 dBµV/m/(5 MHz) @ 0 km and 49 dBµV/m/(5 MHz) @ 6 km |
| @ stands for “at a distance inside the neighbouring country” For NR base station the mean values in this table correspond to the following SSB values: 65 dBμV/m/(5 MHz) corresponds to 43 dBµV/m/(30 kHz) 49 dBμV/m/(5 MHz) corresponds to 27 dBµV/m/(30 kHz) |  |  |

FIELD STRENGTH LEVELS FOR THE CROSS-BORDER OPERATION WITHOUT COORDINATION BETWEEN TDD MFCN SYSTEMS IN THE FREQUENCY BAND 2500-2690 MHz

In this annex, field strength values are given for cross-border scenarios between MFCN TDD systems for non-AAS and AAS base stations on both sides of the borderline.

Preferential and non-preferential PCIs for LTE and NR are given in ANNEX 5.

Administrations/operators may agree on other values of field strength and preferential frequency usage based on bilateral/multilateral agreements/arrangements.

Synchronisation signal means Synchronisation Signal Block (SSB) for 5G NR and Primary/Secondary Synchronisation Signal (PSS/SSS) for LTE.

For non-AAS base stations the field strength values are given for 5 MHz frequency block size. In cases of other frequency block sizes other than 5 MHz, a factor of 10 × log10 (channel bandwidth/(5 MHz)) should be added to the field strength values. The data and control channels are fixed and therefore the same field strength values in dBµV/m/(5 MHz) apply to both channel types. The mean value is with respect to e.g. changing traffic load and downlink transmission time. The field strength values are upper bounds of each cell produced by the base station.

For AAS base stations the upper bound field strength for each cell of a base station is given as the median for a random beamforming data channel with 5 MHz frequency block size and for a static SSB field with SCS 30 kHz. In the case of a SCS other than 30 kHz, a correction factor of 10 × log10 (SCS in kHz/(30 kHz)) dB, should be added to the SSB field strength value.

Field strength levels for MFCN TDD systems in the frequency band 2500-2690 MHz

Synchronised operation

Base stations of synchronised MFCN TDD systems on both sides of the borderline with

synchronisation signal centre frequencies not aligned, for all PCIs or with

synchronisation signal centre frequencies aligned and for preferential PCIs

may be used without coordination with a neighbouring country if the field strength of each cell produced by the base station does not exceed the field strength values as listed below:

For non-AAS base stations field strength value

65 dBµV/m/(5 MHz) at a height of 3 m above ground at the borderline between countries and

49 dBµV/m/(5 MHz) at a height of 3 m above ground at a distance of 6 km inside the neighbouring country.

For AAS base stations median field strength value

77 dBµV/m/(5 MHz) at a height of 3 m above ground at the borderline between countries and

59 dBµV/m/(5 MHz) at a height of 3 m above ground at a distance of 6 km inside the neighbouring country.

Base stations of synchronised MFCN TDD systems on both sides of the borderline with

synchronisation signal centre frequencies aligned and for non-preferential PCIs

may be used without coordination with a neighbouring country if the field strength of each cell produced by the base station for data channel does not exceed the values as listed below:

For non-AAS base stations field strength value:

49 dBµV/m/(5 MHz) at a height of 3 m above ground at the borderline between countries;

For AAS base stations median field strength value:

59 dBµV/m/(5 MHz) at a height of 3 m above ground at the borderline between countries

Table 2 and Table 3 give an overview of the field strength values for non-AAS and AAS base stations respectively.

Table 2: Field strength values at a height of 3 m above ground for synchronised operation for non-AAS base stations

| Synchronised operation non-AAS base stations |  |  |
| --- | --- | --- |
| Synchronisation signal centre frequencies aligned |  | Synchronisation signal centre frequencies not aligned |
| Preferential PCIs | Non-preferential PCIs | All PCIs |
| 65 dBμV/m/(5 MHz) @ 0 km and 49 dBμV/m/(5 MHz) @ 6 km | 49 dBμV/m/(5 MHz) @ 0 km | 65 dBμV/m/(5 MHz) @ 0 km and 49 dBμV/m/(5 MHz) @ 6 km |
| @ stands for “at a distance from the borderline into the neighbouring country”. For NR base station the mean values in this table correspond to the following SSB values: 65 dBμV/m/(5 MHz) corresponds to 43 dBµV/m/(30 kHz) 49 dBμV/m/(5 MHz) corresponds to 27 dBµV/m/(30 kHz) |  |  |

Table 3: Field strength values at a height of 3 m above ground for synchronised operation for AAS base stations with median data channel value from beamforming and SSB value

| Synchronised operation AAS base stations |  |  |
| --- | --- | --- |
| Synchronisation signal centre frequencies aligned |  | Synchronisation signal centre frequencies not aligned |
| Preferential PCIs | Non-preferential PCIs | All PCIs |
| 77 dBμV/m/(5 MHz) @ 0 km and 59 dBμV/m/(5 MHz) @ 6 km | 59 dBμV/m/(5 MHz) @ 0 km | 77 dBμV/m/(5 MHz) @ 0 km and 59 dBμV/m/(5 MHz) @ 6 km |
| @ stands for “at a distance from the borderline into the neighbouring country”. For NR base station the mean values in this table correspond to the following SSB values: 77 dBμV/m/(5 MHz) corresponds to a) 67 dBµV/m/(30 kHz) for single-beam antenna pattern, b) 74 dBµV/m/(30 kHz) for multi-beam antenna pattern 59 dBμV/m/(5 MHz) corresponds to a) 49 dBµV/m/(30 kHz) for single-beam antenna pattern, b) 56 dBµV/m/(30 kHz) for multi-beam antenna pattern |  |  |

Unsynchronised operation

Base stations of unsynchronised MFCN TDD systems on both sides of the borderline with

synchronisation signal centre frequencies aligned and not aligned, for all PCIs

may be used without coordination with a neighbouring country if the field strength of each cell produced by base station does not exceed the values as listed below:

For non-AAS base stations field strength value

30 dBµV/m/(5 MHz) at a height of 3 m above ground at the borderline between neighbouring countries.

For AAS base stations median field strength value

13 dBµV/m/(5 MHz) at a height of 3 m above ground at the borderline between neighbouring countries.

Table 4 and Table 5 give an overview of the field strength values for non-AAS and AAS base stations respectively.

Table 4: Field strength values at a height of 3 m above ground for unsynchronised operation with non-preferential frequency blocks for non-AAS base stations

Unsynchronised operation with non-preferential frequency blocks non-AAS base stations

All PCIs

30 dBμV/m/(5 MHz) @ 0 km

@ stands for “at a distance from the borderline into the neighbouring country”. For NR base station the mean values in this table correspond to the following SSB values: 30 dBμV/m/(5 MHz) corresponds to 8 dBµV/m/(30 kHz)

Table 5: Field strength values at a height of 3 m above ground for unsynchronised operation with non-preferential frequency blocks for AAS base stations

Unsynchronised operation with non-preferential frequency blocks AAS base stations

All PCIs

13 dBμV/m/(5 MHz) @ 0 km

@ stands for “at a distance from the borderline into the neighbouring country”. For NR base station the mean values in this table correspond to the following SSB values: 13 dBμV/m/(5 MHz) corresponds to a) 1 dBµV/m/(30 kHz) for single-beam antenna pattern, b) 5 dBµV/m/(30 kHz) for multi-beam antenna pattern

PROPAGATION MODEL

The following methods are proposed for assessment of anticipated interference inside neighbouring country based on established trigger values. Due to complexity of radio wave propagation nature different methods are proposed to be considered by administrations and are included here for guidance purposes only.

It should be noted that following methods provide theoretical predictions based on available terrain knowledge. It is practically impossible to recreate these methods with measurement procedures in the field. Therefore only some approximation of measurements could be used to check compliance with those methods based on practical measurement procedures. The details of such approximation are not included in this recommendation and should be negotiated between countries based on their radio monitoring practices.

Path specific model

Where appropriate detailed terrain data is available, the propagation model for interference field strength prediction is the latest version of Recommendation ITU-R P.452 [6], For the relevant transmitting terminal, predictions of path loss would be made at x km steps along radials of y km at z degree intervals. The values for those receiver locations within the neighbouring country would be used to construct a histogram of path loss – and if 10% of predicted values exceed the threshold the station should be required to be coordinated.

Values for x, y, z and path specific field strength levels are to be agreed between the administrations concerned.

Site General model

If it is not desirable to utilise detailed terrain height data for the propagation modelling in the border area, the basic model to be used to trigger coordination between administrations and to decide, if co-ordination is necessary, is Recommendation ITU-R P.1546 [7]. This model is to be employed for 50% locations, 10% time and using a receiver height of 3 m.

For specific reception areas where terrain roughness adjustments for improved accuracy of field strength prediction are needed, administrations may use correction factors according to terrain irregularity and/or an averaged value of the TCA parameter in order to describe the roughness of the area on and around the coordination line.

Administrations and/or operators concerned may agree to deviate from the aforementioned model by mutual consent.

Area calculations

In the case where greater accuracy is required, administrations and operators may use the area calculation below.

For calculations, all the pixels of a given geographical area to be agreed between the administrations concerned in a neighbouring country are taken into consideration.

For the relevant base station, predictions of path loss should be made for all the pixels of a given geographical area from a base station and at a receiver antenna height of 3 m above ground.

For evaluation,

only 10 percent of the number of geographical area between the borderline (including also the borderline) and the 6 km line itself inside the neighbouring country may be interfered by higher field strength than the trigger field strength value given for the borderline in ANNEX 1 and ANNEX 2 at a height of 3 m above ground;

- only 10 percent of the number of geographical area between the 6 km (including also 6km line) and 12 km line inside the neighbouring country may be interfered by higher field strength than the trigger field strength value given for the 6 km line in ANNEX 1 and ANNEX 2 at a height of 3 m above ground.

It is recommended that during area calculations not only detailed terrain data but also clutter data be taken into account. Use of correction factors for clutter is crucial in particular where the border area is ‘open’ or ‘quasi-open’ from the point of view of clutter or where the interfering base station is just a few kilometres from a borderline.

If the distance between a base station and a terrain point of a borderline is closer than or equal to 1 km, free space propagation model needs to be applied. Furthermore, if there is no terrain obstacle within the 1st Fresnel zone, also the free space propagation model should be applied.

If clutter data is not available, it is proposed to extend the usage of free space propagation model to a few kilometres, depending on the clutter situation in border areas.

For area type interference calculations, propagation models with path specific terrain correction factors are recommended (e.g. Recommendation ITU-R P.1546 [7] with the terrain clearance angle correction factor TCA, HCM method with the terrain clearance angle correction factor or Recommendation ITU-R P.1812 [8]).

As to correction factors for clutters ‘open area’ and ‘quasi-open area’, 20 dB and 15 dB should be used respectively. Recommendation ITU-R P.1406 [9] should be used if a finer selection of clutter is required.

It must be noted that terrain irregularity factor Δh is not recommended to be used in area calculations. Administrations and/or operators concerned may agree to deviate from the aforementioned models by mutual consent.

COORDINATION PROCEDURE

When requesting coordination the relevant characteristics of the base station should be submitted to the administration affected. All of the following characteristics should be included:

- centre frequency (MHz);

synchronisation signal centre frequency (MHz), if differs from centre frequency;

channel bandwidth (MHz);

subcarrier spacing (kHz) (only for NR);

name of transmitter station;

country of location of transmitter station;

geographical coordinates (W/E, N; WGS84);

effective antenna height (m);

antenna polarisation;

antenna azimuth (deg);

directivity in antenna systems or antenna gain (dBi);

effective radiated power (dBW);

expected coverage zone or cell range (km);

date of entry into service (month, year);

PCI numbers used (only for LTE and NR);

antenna tilt (electrical and mechanical ) (deg);

antenna pattern or envelop;

SSB antenna patterns;

AAS antenna configuration (number of arrays and number of column).

For synchronised LTE and NR TDD networks in the 2500-2690 MHz band, the following characteristics should be included:

Frame structure including the special slot “S” configuration (the format at symbol level for slots between downlink and uplink slots);

Clock phase, frequency and time synchronisation;

Global Synchronisation Channel Number (GSCN) in case of NR.

The administration affected should evaluate the request for coordination and should within 30 days notify the result of the evaluation to the administration requesting coordination.

In the course of the coordination procedure an administration may request additional information.

If no reply is received by the administration requesting coordination within 30 days, it may send a reminder to the administration affected. An administration not having responded within 30 days following communication of the reminder should be deemed to have given its consent and the station may be put into use with the characteristics given in the request for coordination.

The periods mentioned above may be extended by common consent.

PHYSICAL-LAYER CELL IDENTITIES (PCI) FOR LTE and nr

ETSI TS 36.211 [10] defines 168 “unique physical-layer cell-identity groups” in §6.11, numbered 0…167, hereafter called “PCI groups” for LTE. Within each PCI group there are three separate PCIs giving 504 PCIs in total.

For NR in ETSI TS 138 211 [11] (§7.4.2) the number of physical-layer cell-identity groups (different cell IDs) have been increased to 336, numbered 0..335.

Administrations should apply sharing of PCIs in border areas, an equitable distribution of these PCIs when synchronisation signal centre frequencies are aligned as provided in Table 6.

Sharing of PCIs between operators of neighbouring countries should only be applied where synchronisation signal centre frequencies used in the neighbouring countries are aligned independent of the channel bandwidth or where it is not known whether or not the synchronisation signal centre frequencies used in the neighbouring countries are aligned, or where there is no network in operation in the neighbouring country unless otherwise stated in ANNEX 1 or administration/operator agreements/arrangements.

As shown in the Table 6, the PCIs for LTE and NR are divided into 6 sub-sets containing each one sixth of the available PCIs. Each country is allocated three sets (half of the PCIs) in a bilateral case, and two sets (one third of the PCIs) in a trilateral case, therefore dividing the PCI groups or PCIs is equivalent.

The preferential PCIs of a two country PCI sharing should be applied for a base station if the level of field strength relating to non-preferential PCIs could be exceeded at the borderline of only one neighbouring country. The preferential PCIs of a three country PCI sharing should be applied for a base station if the level of field strength related to non-preferential PCIs could be exceeded at the borderline of only two neighbouring countries.

Four types of countries are defined in a way such that no country will use the same code set as any one of its neighbours. The following lists describe the distribution of European countries:

Type country 1:	AZE, BEL, CVA, CYP, CZE, DNK, E, FIN, GRC, IRL, ISL, LTU, MCO, SMR, SRB, SUI, SVN and UKR;.

Type country 2:	AND, BIH, BUL, D, EST, G, GEO, HNG, I, and MDA;.

Type country 3:	ALB, AUT, F, HOL, HRV, MLT, POL, POR, ROU and S;.

Type country 4:	LIE, LUX, LVA, MKD, MNE, NOR, SVK and TUR.

For each type of country, the Table 6 and Figure 1 describe the sharing of the PCIs with its neighbouring countries, with the following conventions of writing:

|  | Preferential PCI |
| --- | --- |
|  | non-preferential PCI |

The 504/1008 physical-layer cell-identities should be divided into the following 6 sub-sets when the synchronisation signal centre frequencies are aligned in border areas:

Table 6: PCI sub-sets for LTE and NR for use in border areas when the synchronisation signal centre frequencies are aligned

| PCI | Set A | Set B | Set C | Set D | Set E | Set F |  | PCI | Set A | Set B | Set C | Set D | Set E | Set F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Country 1 LTE | 0..83 | 84..167 | 168..251 | 252..335 | 336..419 | 420..503 |  | Country 2 LTE | 0..83 | 84..167 | 168..251 | 252..335 | 336..419 | 420..503 |
| Country 1 NR | 0..83 504-587 | 84..167 588..671 | 168..251 672..755 | 252..335 756..839 | 336..419 840..923 | 420..503 924..1007 |  | Country 2 NR | 0..83 504-587 | 84..167 588..671 | 168..251 672..755 | 252..335 756..839 | 336..419 840..923 | 420..503 924..1007 |
| Border 1-2 |  |  |  |  |  |  |  | Border 2-1 |  |  |  |  |  |  |
| Zone 1-2-3 |  |  |  |  |  |  |  | Zone 2-3-1 |  |  |  |  |  |  |
| Border 1-3 |  |  |  |  |  |  |  | Border 2-3 |  |  |  |  |  |  |
| Zone 1-2-4 |  |  |  |  |  |  |  | Zone 2-1-4 |  |  |  |  |  |  |
| Border 1-4 |  |  |  |  |  |  |  | Border 2-4 |  |  |  |  |  |  |
| Zone 1-3-4 |  |  |  |  |  |  |  | Zone 2-3-4 |  |  |  |  |  |  |
| PCI | Set A | Set B | Set C | Set D | Set E | Set F |  | PCI | Set A | Set B | Set C | Set D | Set E | Set F |
| Country 3 LTE | 0..83 | 84..167 | 168..251 | 252..335 | 336..419 | 420..503 |  | Country 4 LTE | 0..83 | 84..167 | 168..251 | 252..335 | 336..419 | 420..503 |
| Country 3 NR | 0..83 504-587 | 84..167 588..671 | 168..251 672..755 | 252..335 756..839 | 336..419 840..923 | 420..503 924..1007 |  | Country 4 NR | 0..83 504-587 | 84..167 588..671 | 168..251 672..755 | 252..335 756..839 | 336..419 840..923 | 420..503 924..1007 |
| Border 3-2 |  |  |  |  |  |  |  | Border 4-1 |  |  |  |  |  |  |
| Zone 3-1-2 |  |  |  |  |  |  |  | Zone 4-1-2 |  |  |  |  |  |  |
| Border 3-1 |  |  |  |  |  |  |  | Border 4-2 |  |  |  |  |  |  |
| Zone 3-1-4 |  |  |  |  |  |  |  | Zone 4-2-3 |  |  |  |  |  |  |
| Border 3-4 |  |  |  |  |  |  |  | Border 4-3 |  |  |  |  |  |  |
| Zone 3-2-4 |  |  |  |  |  |  |  | Zone 4-3-1 |  |  |  |  |  |  |

Note: In certain specific cases (e.g. AUT/HRV) where the distance between two countries of the same type number is very small 
(< few 10s km), it may be necessary to address the situation in bi/multilateral coordination agreements as necessary, and may include further subdivision of the allocated PCIs in certain areas.

Figure 1: Country type map

list of references

ITU Radio Regulations, Edition of 2024

ECC Decision (05)05: “Harmonised utilisation of spectrum for Mobile/Fixed Communications Networks (MFCN) operating within the band 2500-2690 MHz”, latest corrected 4 March 2022

EC Decision 2008/477/EC: “Commission Decision of 13 June 2008 on the hHarmonisation of the 2500-2690 MHz frequency band for terrestrial systems capable of providing electronic communications services in the Community”

CEPT Report 19: “Report from CEPT to the European Commission in response to EC Mandate to develop least restrictive technical conditions for frequency bands addressed in the context of WAPECS”, approved October 2008

ECC Recommendation (15)01: “Cross-border coordination for Mobile/Fixed Communications Networks (MFCN) in the frequency bands: 694-790 MHz, 1427-1518 MHz and 3400-3800 MHz”, approved on 13 February 2015, latest amended on 10 June 2022

Recommendation ITU-R P.452: “Prediction procedure for the evaluation of interference between stations on the surface of the Earth at frequencies above about 100 MHz”

Recommendation ITU-R P.1546: “Method for point to area predictions for terrestrial services in the frequency range 30 MHz to 4 000 MHz”

Recommendation ITU-R P.1812: “A path-specific propagation prediction method for point-to-area terrestrial services in the frequency range 30 MHz to 6 GHz”

Recommendation ITU-R P.1406: “Propagation effects relating to terrestrial land mobile and broadcasting services in the VHF and UHF bands”

ETSI TS 136 211: “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”

ETSI TS 138 211 V16.2.0 (2020-07): “5G; NR; Physical channels and modulation”

not occupied bandwidth

not occupied bandwidth

e.g. as used by members of the HCM-Agreement