# 26 GHz band simulation assumptions

## System parameters

Table 1: BS parameters

| Parameter | Macrocell (New ECC Rec.) | Smallcell (New ECC Rec.) | Outdoor BS (ECC Report 307) |
| --- | --- | --- | --- |
| Duplex mode | TDD |  | TDD |
| Centre frequency (GHz) | 26 |  | 26 |
| Channel bandwidth (MHz) | 200 MHz |  | 200 MHz |
| BS loading (%) | 100 % |  | 100 % |
| BS antenna height (m) | 30 | 6 | 6 m or 15 m (above ground level) |
| BS Tx power (dBm) TRP | 30.6 | 30.6 | 25 dBm/200 MHz |
| AAS antenna configuration | 16x16 | 8x8 | 8x8 |
| AAS antenna model | Rec. ITU-R M.2101 |  | Rec. ITU-R M.2101 |
| Element gain (dBi) | 7.3 |  | 5 dBi is used with normalization factor |
| H/V element spacing | 0.5 l /0.5 l |  | 0.5 of wavelength for both Horizontal/vertical |
| Front-to-Back Ratio (dB) | 30 |  | 25 for both Horizontal/vertical |
| Sidelobe limit (dB) | 30 |  |  |
| Cell range (mobile deployment) (m) | 250 | 150 |  |
| BS antenna downtilt (°) | 10 | 3 | 10 or 15 degrees |
| Ohmic loss (dB) | 0 |  | 3 (Array Ohmic loss) |
| ACLR (dB) | 28 |  | 28 |
| ACS (dB) | 23.5 |  | 23.5 |
| Noise figure (dB) | 10 | 14 | 10 |
| Data rate vs SINR mapping table | Bitrate mapping table in SEAMCAT | Bitrate mapping table in SEAMCAT | ECC Report 307 section 4.1.4 |

Table 2: UE parameters

| Parameter | New ECC Rec. | ECC Report 307 |
| --- | --- | --- |
| UE antenna height (m) | 1.5 | 1.5 |
| Outdoor UE (%) | 100% |  |
| UE antenna AAS | 4x4 | 4 x 4 |
| Element gain (dBi) | 5 | 5 |
| Element H/V spacing | 0.5l/0.5l | 0.5 of wavelength for both Horizontal/vertical |
| Front to back ratio (dB) | 25 |  |
| Sidelobe limit (dB) | 25 |  |
| AAS antenna model | Rec. ITU-R M.2101 | Rec. ITU-R M.2101 |
| Body loss (dB) | 4 | 4 |
| Ohmic loss (dB) | 0 | 3 (Array Ohmic loss) |
| UE Tx power (dBm) (TRP) | 22 | 22 |
| UE Rx noise figure (dB) | 10 | 10 |
| UE PC range (dB) | 63 | 63 |
| UE ACLR (dB) | 17 | 17 |
| UE ACS (dB) | 22.5 | 22.5 |

## Propagation model

Table 3: Propagation model used in simulations (New ECC Rec.)

|  | Macrocell | Smallcell |
| --- | --- | --- |
| BS to UE | 3GPP 38.901 (UMa, LoS Probabilities) | 3GPP 38.901 (UMi, LoS Probabilities) |
| Macrocell BS to Macrocell BS | Recommendation ITU-R P.525 (Free space) |  |
| Smallcell BS to Smallcell BS | 3GPP 38.901 (UMi LoS Probabilities) |  |
| Macrocell BS to Smallcell BS | 3GPP 38.901 (UMa LoS Probabilities) |  |
| Smallcell BS to Macrocell BS | 3GPP 38.901 (UMi LoS Probabilities) |  |
| Note: scenarios according to 3GPP TR 38.901 path loss model: Urban Macro Cell (UMa), Urban Micro Cell (UMi), Rural Macro Cell (RMa); Line of Sight conditions: LoS, LoS, LoS Probabilities |  |  |

Table 4: Propagation model used in simulations (ECC Report 307)

| Parameter | For 6 m antenna height (urban/suburban hotspot) | For 15 m antenna height (suburban open space) |
| --- | --- | --- |
| BS-BS propagation model | Report ITU-R M.2412-0 | Report ITU-R M.2412-0 or Recommendation ITU-R P.525 (free-space path loss) |
| BS-UE | Report ITU-R M.2412-0 |  |